������������������������������������������������������Ŀ

�  Nintendo Entertainment System Documentation v0.31   �

�            by Y0SHi (yoshi@parodius.com)             �

��������������������������������������������������������

�����������������������������Ŀ

� READ ME   READ ME   READ ME �

�������������������������������

I'd like to dedicate this document to Alex Krasivsky (Landy on IRC) for

all of his help and moral support throughout the past year regarding the

NES, and issues outside of the console realm. During the good times, and

the bad times, Alex was there. Spasibo, Alex; umnyj russki...

�����������������������������Ŀ

� READ ME   READ ME   READ ME �

�������������������������������

There are bound to be contradictory statements in this document. There

are errors, and many problems regarding terminology (VROM vs. CHR-RAM,

etc.). It is *FAR* from perfect. I have not incorporated all the terminology

I would like to incorporate. Give it time and support: it'll get there.

You will notice severe similarities between Marat Fayzullin's NES Docu-

mentation and mine; I originally based my work on his. Without Marat's

document, I would have never had the desire to create this one.

Finally, please, give credit where credit is due. All of the people in

Section #14 helped contribute to this document. Be sure to thank them,

as without their help, this document wouldn't be much more than a waste

of bandwidth.

���������������������Ŀ

� 1 �   Description   �

�����������������������

The NES consists of a custom 6502 CPU, and a PPU (Picture Processing

Unit) used for graphics. Programs can communicate with the NES via

registers (I/O ports, so-to-speak), which allow different things to

happen internally to the NES.

There are multiple areas of memory on the NES, and they should not be

confused. Terms marked with an asterisk ('*') will be the terms used

from here on.

������������������������������������������������������������������������Ŀ

� Term.    � Description                                                 �

������������������������������������������������������������������������Ĵ

�* PRG-ROM � Nintendo of Japan's term for the actual program code area.  �

�          �                                                             �

�          � It stands for PRoGram ROM.                                  �

������������������������������������������������������������������������Ĵ

�* CHR-RAM � Nintendo of Japan's term for the Pattern Table of the PPU's �

�          � internal RAM.                                               �

�          �                                                             �

�          � It stands for CHaRacter RAM.                                �

������������������������������������������������������������������������Ĵ

�* VRAM    � The RAM internal to the PPU (Picture Processing Unit).      �

�          � There is 16K of internal VRAM.                              �

������������������������������������������������������������������������Ĵ

�* WRAM    � This is the memory which is most commonly used for games    �

�          � which support saving (usually RPGs), such as Zelda 1 & 2,   �

�          � Crystalis, the Final Fantasy series, and other games.       �

�          �                                                             �

�          � It stands for Writeable RAM.                                �

������������������������������������������������������������������������Ĵ

�* MMC     � The microcontrollers used in carts to address memory past   �

�          � the standard 6502 64K boundary. They can also be used to    �

�          � address extra VRAM, and may also be used for "special       �

�          � effects."                                                   �

�          �                                                             �

�          � MMC stands for Memory Management Controller.                �

������������������������������������������������������������������������Ĵ

� VROM     � The Pattern Table data kept external to the PPU itself;     �

�          � standard CHR-RAM data, which is swapped in and out of the   �

�          � PPU via an MMC.                                             �

�          �                                                             �

�          � VROM stands for Visual (Virtual?) ROM.                      �

������������������������������������������������������������������������Ĵ

� CHR      � Synonymous with CHR-RAM.                                    �

������������������������������������������������������������������������Ĵ

� PRG      � Synonymous with PRG-ROM.                                    �

������������������������������������������������������������������������Ĵ

� SRAM     � Synonymous with WRAM.                                       �

��������������������������������������������������������������������������

������������������������Ŀ

� 2 �   CPU Memory Map   �

��������������������������

�������������������������������������������������������������Ŀ

� Address � Size  � Description                               �

�������������������������������������������������������������Ĵ

� $0000   � $800  � RAM                                       �

� $0800   � $800  � RAM (mirrored from $0000)                 �

� $1000   � $800  � RAM (mirrored from $0000)                 �

� $1800   � $800  � RAM (mirrored from $0000)                 �

� $2000   � $3000 � Registers                                 �

� $5000   � $1000 � Expansion Modules                         �

� $6000   � $2000 � WRAM                                      �

� $8000   � $4000 � PRG-ROM (Lower)                           �

� $C000   � $4000 � PRG-ROM (Upper)                           �

���������������������������������������������������������������

Note the two seperate PRG-ROM sections; they are congruent, but they

also play seperate roles, depending upon the size of the cartridge itself.

Some games only hold one (1) 16K bank of PRG-ROM, which should be loaded

into $C000 (Upper), not $8000 (Lower). Some games which do this are:

Battle City, Mario Brothers, Millipede, Nuts & Milk, and Tennis. There

are others as well.

Most games load themselves into $8000 (Lower PRG-ROM), using 32K of

PRG-ROM space. The first game (to my knowledge) to use the entire PRG-ROM

space is Super Mario Brothers. However, all games with more than one (1)

16K bank of PRG-ROM load themselves into $8000 as well. These games use

MMCs (see Section #10) to address PRG-ROM past the 32K boundary, and

to access more than 8K of CHR-RAM simultaneously.

���������������������������������Ŀ

� 3 �   PPU & Sprite Memory Map   �

�����������������������������������

PPU Memory Map

�������������������������������������������������������������Ŀ

� Address � Size  � Description                               �

�������������������������������������������������������������Ĵ

� $0000   � $1000 � Pattern Table #0     (possibly CHR-RAM)   �

� $1000   � $1000 � Pattern Table #1     (possibly CHR-RAM)   �

� $2000   � $3C0  � Name Table #0                             �

� $23C0   � $40   � Attribute Table #0                        �

� $2400   � $3C0  � Name Table #1                             �

� $27C0   � $40   � Attribute Table #1                        �

� $2800   � $3C0  � Name Table #2        (based on mirroring) �

� $2BC0   � $40   � Attribute Table #2   (based on mirroring) �

� $2C00   � $3C0  � Name Table #3        (based on mirroring) �

� $2FC0   � $40   � Attribute Table #3   (based on mirroring) �

� $3000   � $F00  � [---EMPTY---]                             �

� $3F00   � $10   � Image Palette                             �

� $3F10   � $10   � Sprite Palette                            �

� $3F20   � $E0   � [---EMPTY---]                             �

���������������������������������������������������������������

Sprite RAM

�������������������������������������������������������������Ŀ

� Address � Size  � Description                               �

�������������������������������������������������������������Ĵ

� $0000   � $100  � Sprite RAM                                �

���������������������������������������������������������������

For more information about Sprite RAM, see Sections #6 and #9.

VRAM is quite unorganized, as shown above. To my knowledge, VRAM is

addressed via 14-bit address, and therefore should wrap back around to

$0000 if the memory boundary passes the end of memory ($3FFF).

There is only enough VRAM memory to support two (2) Name Tables and

two (2) Attribute Tables. Therefore, Name/Attribute Tables #2 and #3

are mirrors of Name/Attribute Tables #0 and #1. Which Tables are mirrored

depends upon the mirroring bit set inside of the cartridge header (see

Section #11 for more information).

In a real NES, reading/writing VRAM should only be attempted during

VBlank. Many smaller ROMs have CHR-RAM for the Pattern Tables. In this

case, you won't be able to write into this memory.

Writing to VRAM

���������������

1) Write upper address byte into $2006

2) Write lower address byte into $2006

3) Write data into $2007. After each write, the address will auto-

increment by 1, or 32 (if Bit #2 of $2000 is 1).

Reading from VRAM

�����������������

1) Write upper address byte into $2006

2) Write lower address byte into $2006

3) Read data from $2007. The first read from $2007 is invalid, and

therefore should be taken care of before actual data is read.

4) Read data from $2007. From here on, after each read, the address will

auto-increment by 1, or 32 (if Bit #2 of $2000 is 1).

The Name Table holds "tile index values." Tiles themselves are 8x8

pixels. The entire Name Table itself is 32x30 tiles (256x240 pixels).

However, due to NTSC and PAL frequencies, the actual displayed resolution

is different.

For NTSC, the upper and lower 16 pixels are not displayed, therefore the

resolution is 256x224. For PAL the upper and lower 8 pixels are not dis-

played, therefore the resolution is 256x240. The Name Table itself keeps

a static size of 32x30 tiles (256x240 pixels).

These values (re: "tile index values") are usually put into the Name

Table via writes to the $2006 register (see Section #6). These values are

used to reference the information stored in the Pattern Table. For you

emulator authors out there, the formulae is quite simple:

(VALUE * 16) + ScreenPatternTableAddress

Where VALUE is the value which is written to register $2006, and

ScreenPatternTableAddress is the Screen Pattern Table Address defined in

Bit #2 in register $2000 (see Section #6).

The NES can only display 16 colours on the screen simultaneously.

The Pattern Table contains tiles in the following format:

Contents of                       Colour

Pattern Table                      Result

���������������                    ��������

%00010000 = $10 �Ŀ                ...1....  Periods are used to rep-

%00000000 = $00   �                ..2.2...  resent colour 0. Numbers

%01000100 = $44   �                .3...3..  shown represent colour #.

%00000000 = $00   ��� Bit 0        2.....2.

%11111110 = $FE   �                1111111.

%00000000 = $00   �                2.....2.

%10000010 = $82   �                3.....3.

%00000000 = $00 ���                ........

%00000000 = $00 �Ŀ

%00101000 = $28   �

%01000100 = $44   �

%10000010 = $82   ��� Bit 1

%00000000 = $00   �

%10000010 = $82   �

%10000010 = $82   �

%00000000 = $00 ���

The result of the above Pattern Table is the character 'A', as shown

in the "Colour Result" section in the upper right.

Only two (2) bits for each pixel of a tile are stored in the Pattern

Table. The other two (2) bits are taken from the Attribute Table. Four (4)

bits make up the entire possible colours on the NES, which therefore shows

that the NES can display 16 colours on the screen simultaneously.

Each byte in an Attribute Table represents a 4x4 group of tiles on the

screen. There's multiple ways to describe what the function of one (1)

byte in the Attribute Table does:

* Holds the upper two (2) bits of a 32x32 pixel grid, per 16x16 pixels.

* Holds the upper two (2) bits of sixteen (16) 8x8 tiles.

* Holds the upper two (2) bits of four (4) 4x4 tile grids.

It's confusing as hell, to be honest. A few graphical diagrams may help

those who are confused:

������������������������Ŀ

�  Square 0  �  Square 1 �  #0-F represents an 8x8 tile

�   #0  #1   �   #4  #5  �

�   #2  #3   �   #6  #7  �  Square [x] represents four (4) 8x8 tiles

������������������������Ĵ   (i.e. a 16x16 pixel grid)

�  Square 2  �  Square 3 �

�   #8  #9   �   #C  #D  �

�   #A  #B   �   #E  #F  �

��������������������������

Attribute Byte

����������������

%00000000

������ Upper two (2) colour bits for Square 0 (Tiles #0/1/2/3)

� � ���� Upper two (2) colour bits for Square 1 (Tiles #4/5/6/7)

� ������ Upper two (2) colour bits for Square 2 (Tiles #8/9/A/B)

�������� Upper two (2) colour bits for Square 3 (Tiles #C/D/E/F)

Putting all of this information together is a cinch, assuming you can

overlay bit patterns in your head. A similar method is used in the tweaked

graphics modes known as "MODE-X" modes on the PC. Most DOS-oriented emu-

lators use what's known as "MODE-Q", since it's resolution is 256x256x256.

For more information, research "chained graphics modes."

Two (2) palettes exist; the Image Palette and the Sprite Palette. These

palettes are more of a "lookup table" than an actual palette, since they

do not hold physical RGB values.

The NES has pre-set RGB value for both the Image and Sprite Palettes.

The actual 256 RGB palette can be obtained from Loopy (see Section #13).

Mirroring also occurs between the Image Palette and the Sprite Palette.

Any data which is written to $3F00 is mirrored to $3F04, $3F08, and $3F0C.

Any data which is written to $3F10 is mirrored to $3F14, $3F18, and $3F1C.

Colour #0 in both Palettes defines transparency.

������������������������������Ŀ

� 4 �   Background Scrolling   �

��������������������������������

It seems there's quite a few questions regarding just how to go about

using Register $2005 (see Section #6) to pan the background. I myself am

still having problems understanding just exactly how this register works.

But, thanks to Pat Mccomack, there is a small explanation which I'll

post here so that those who're having problems can hopefully make use of

this information:

"This stuff is used in smb1 (horizontal scrolling) but it's probably the

same way with other games. Scrolling seems to be scanline based, ie during

refresh the game can write different values to the scroll register (usually

they do this when they detect the sprite 0 hit via reg $2002).  The scroll-

ing screen layout also can change.  With horizontal scrolling, the name

table at the first screen (0-255) is the one specified in reg $2000, and

the name table for the second screen (256-512) is the opposite of the first

one, based on mirroring I guess.  I'll let you know more stuff as I find it,

if you want me to that is :)."

If someone out there knows more about the BGSCROL register, please con-

tact me, as the implementations of this register differentiate between

NESticle, NESA, and iNES.

��������������������Ŀ

� 5 �   Interrupts   �

����������������������

The 6502 has four interrupts: ABORT, IRQ/BRK, NMI, and RESET.

Each interrupt has it's own vector, usually pointing to some code

to be executed. A vector is a 16-bit address which specifies a location

to "jump to" when the interrupt is triggered.

ABORT is not controllable via software, and is a hardware interrupt

which cannot be modified. It has no vector. If you're an emulator author,

don't worry about emulating ABORT, as it's a pin on the 6502 chip itself.

IRQ/BRK is triggered when the 6502 executes the BRK instruction. BRK

can be used for many things, but is *RARELY* used, since it generally

puts the machine into a state of unhappiness. However, for you emulator

authors out there, be sure to emulate BRK correctly, as some games do

use BRK to force the jump at the IRQ/BRK vector. Super Nintendo (SNES)

games do this as well, but that's a different issue alltogether.

NMI stands for Non-Maskable Interrupt, and is generated by each

refresh (VBlank/VBL), which occurs at different intervals depending upon

the system used. The PAL version of the NES runs at a 50Hz (50 times/sec)

interval. The NTSC version runs at a 60Hz (60 times/sec) interval.

RESET is triggered on power-up. The ROM is loaded into memory, and

the 6502 jumps to the address specified in the RESET vector.

Back to the issue of NMI. NMI is only executed 50 times/sec. when

Bit #7 in $2000 (see Section #6) is set to 1. This tells the NES to

generate interrupts (or more specifically, execute NMI) everytime a

refresh occurs. Interrupt latency on the 6502 is seven (7) cycles; this

means it takes seven (7) cycles to move in and out of an interrupt.

Most interrupts should return using the RTI instruction. Some NES carts

do not use this method, such as Final Fantasy 1. These carts return from

interrupts in a very odd fashion: by manipulating the stack by hand, and

then doing an RTS. This is technically valid, but morally is "bad." If

you're an emulator author, just be sure you implement all your opcodes

right and you should be fine. The game code will take care of the rest.

The following interrupts have the following vector-points in ROM:

��������������������Ŀ

� Vector � Interrupt �

��������������������Ĵ

� $FFFA  � NMI       �

� $FFFC  � RESET     �

� $FFFE  � IRQ/BRK   �

����������������������

Here are the correct interrupt priority orders for the 6502:

����������������������Ŀ

� Priority � Interrupt �

����������������������Ĵ

� Highest  � RESET     �

�          � NMI       �

� Lowest   � IRQ/BRK   �

������������������������

�������������������Ŀ

� 6 �   Registers   �

���������������������

Address: The 16-bit address of the register (in ROM)

Stats: Each register has different statistics and "aspects" to it.

These stats are defined as follows:

R = Register is readable.

W = Register is writeable.

2 = A "double-write" register.

? = Statistics are unknown, or are possibly wrong.

Bits: Most registers have bits you can toggle on/off which do

different things to the NES itself.

[Label]: Labels I have assigned to each register (for programmers).

��������������������������������������������������������������������������Ŀ

� Address � Stats � Bits     � Description                       [Label]   �

��������������������������������������������������������������������������Ĵ

� $2000   �  W    � vhzcpwNN � PPU Control Register #1           [PPUCNT1] �

�         �       �          �                                             �

�         �       �          �  v = Execute NMI on VBlank                  �

�         �       �          �         0 = Disabled                        �

�         �       �          �         1 = Enabled                         �

�         �       �          �  h = Execute NMI on Sprite Hit              �

�         �       �          �         0 = Disabled                        �

�         �       �          �         1 = Enabled                         �

�         �       �          �  z = Sprite Size                            �

�         �       �          �         0 = 8x8                             �

�         �       �          �         1 = 8x16                            �

�         �       �          �  c = Screen Pattern Table Address           �

�         �       �          �         0 = $0000 (VRAM)                    �

�         �       �          �         1 = $1000 (VRAM)                    �

�         �       �          �  p = Sprite Pattern Table Address           �

�         �       �          �         0 = $0000 (VRAM)                    �

�         �       �          �         1 = $1000 (VRAM)                    �

�         �       �          �  w = PPU Address Read/Write Increment       �

�         �       �          �         0 = Increment by 1                  �

�         �       �          �         1 = Increment by 32                 �

�         �       �          �  N = Name Table Select                      �

�         �       �          �        00 = $2000 (VRAM)                    �

�         �       �          �        01 = $2400 (VRAM)                    �

�         �       �          �        10 = $2800 (VRAM)                    �

�         �       �          �        11 = $2C00 (VRAM)                    �

��������������������������������������������������������������������������Ĵ

� $2001   �  W    � fffpcSIt � PPU Control Register #2           [PPUCNT2] �

�         �       �          �                                             �

�         �       �          �  f = Full Background Colour                 �

�         �       �          �       000 = None   \                        �

�         �       �          �       001 = Red     \ Select one only       �

�         �       �          �       010 = Green   /                       �

�         �       �          �       100 = Blue   /                        �

�         �       �          �  p = Sprite Display                         �

�         �       �          �         0 = Hide sprites                    �

�         �       �          �         1 = Show sprites                    �

�         �       �          �  c = Screen Display                         �

�         �       �          �         0 = Off (screen off)                �

�         �       �          �         1 = On  (screen on)                 �

�         �       �          �  S = Sprite Clip                            �

�         �       �          �         0 = Don't show sprites in the left  �

�         �       �          �             8-pixel column                  �

�         �       �          �         1 = Show sprites everywhere         �

�         �       �          �  I = Image Clip                             �

�         �       �          �         0 = Don't show the left 8 pixels of �

�         �       �          �             the screen                      �

�         �       �          �         1 = Show the left 8 pixels          �

�         �       �          �  t = Colour Display                         �

�         �       �          �         0 = Mono-tone display               �

�         �       �          �         1 = Colour display                  �

��������������������������������������������������������������������������Ĵ

� $2002   � R     � vhs00000 � PPU Status Register               [PPUSTAT] �

�         �       �          �                                             �

�         �       �          �  v = VBlank Occurance Flag                  �

�         �       �          �         0 = No VBlank                       �

�         �       �          �         1 = VBlank                          �

�         �       �          �  h = Hit Occurance Flag                     �

�         �       �          �         0 = No hit                          �

�         �       �          �         1 = Refresh has hit Sprite #0       �

�         �       �          �  s = Sprite Count Max                       �

�         �       �          �         0 = Less than 8 sprites on the      �

�         �       �          �             current scanline                �

�         �       �          �         1 = More than 8 sprites on the      �

�         �       �          �             current scanline                �

�         �       �          �                                             �

�         �       �          � NOTE: Reading this register resets Bit 7,   �

�         �       �          �       also also resets the Background       �

�         �       �          �       Scroll Register bits as well.         �

�         �       �          � NOTE: Bit 6 is reset to 0 at the beginning  �

�         �       �          �       of the next refresh.                  �

�         �       �          � NOTE: Bit 6 is not set until the first      �

�         �       �          �       actual pixel (i.e. non-transparent)   �

�         �       �          �       is drawn. Therefore, if you have a    �

�         �       �          �       sprite (8x8) which has it's first 4   �

�         �       �          �       pixels as transparent, and it's 5th   �

�         �       �          �       as a non-transparent value, Bit 6     �

�         �       �          �       will be set after the 5th pixel is    �

�         �       �          �       found & drawn.                        �

�         �       �          � NOTE: If Bit 5 is set, the PPU will NOT let �

�         �       �          �       you write to VRAM.                    �

��������������������������������������������������������������������������Ĵ

� $2003   �  W    � aaaaaaaa � Sprite Memory Address             [SPRADDR] �

�         �       �          �                                             �

�         �       �          �  Specifies the address in Sprite RAM to     �

�         �       �          �  access via $2004 (see Section #9).         �

��������������������������������������������������������������������������Ĵ

� $2004   �  W    � dddddddd � Sprite I/O Register                 [SPRIO] �

�         �       �          �                                             �

�         �       �          �  Used to read/write to the address spec-    �

�         �       �          �  ified via $2003 in Sprite RAM.             �

��������������������������������������������������������������������������Ĵ

� $2005   �  W2   � dddddddd � Background Scroll Register        [BGSCROL] �

�         �       �          �                                             �

�         �       �          �  Used to scroll the screen vertically and   �

�         �       �          �  horizontally. This is a double-write       �

�         �       �          �  register.                                  �

�         �       �          �                                             �

�         �       �          �  BYTE 1: Horizontal Scroll                  �

�         �       �          �  BYTE 2: Vertical Scroll                    �

�         �       �          �                                             �

�         �       �          �  The scrolled data will span across multip- �

�         �       �          �  le Name Tables. The layout is as follows:  �

�         �       �          �                                             �

�         �       �          �      �������������������������Ŀ            �

�         �       �          �      � #2 ($2800) � #3 ($2C00) �            �

�         �       �          �      �������������������������Ĵ            �

�         �       �          �      � #0 ($2000) � #1 ($2400) �            �

�         �       �          �      ���������������������������            �

�         �       �          �                                             �

�         �       �          � NOTE: If the Vertical Scroll value is >239, �

�         �       �          �       it will be ignored. Some emulators    �

�         �       �          �       write 0 to the Vertical Scroll if     �

�         �       �          �       the value is >239.                    �

�         �       �          � NOTE: Remember, there is only enough VRAM   �

�         �       �          �       for two (2) Name Tables.              �

�         �       �          � NOTE: After a VBL occurs, the next write    �

�         �       �          �       will control the low-byte of the      �

�         �       �          �       Horizontal Scroll.                    �

��������������������������������������������������������������������������Ĵ

� $2006   �  W2   � aaaaaaaa � PPU Memory Address                [PPUADDR] �

�         �       �          �                                             �

�         �       �          �  Specifies the address in VRAM in which     �

�         �       �          �  data should be read from or written to.    �

�         �       �          �  This is a double-write register. The high- �

�         �       �          �  byte of the 16-bit address is written      �

�         �       �          �  first, then the low-byte.                  �

��������������������������������������������������������������������������Ĵ

� $2007   � RW    � dddddddd � PPU I/O Register                    [PPUIO] �

�         �       �          �                                             �

�         �       �          �  Used to read/write to the address spec-    �

�         �       �          �  ified via $2006 in VRAM.                   �

��������������������������������������������������������������������������Ĵ

� $4000   � RW    � CChessss � Square Wave Control Register #1             �

�         �       �          �                                             �

�         �       �          �   C = Duty Cycle (Positive vs. Negative)    �

�         �       �          �        00 = 87.5%                           �

�         �       �          �        01 = 75.0%                           �

�         �       �          �        10 = 58.0%                           �

�         �       �          �        11 = 25.0%                           �

�         �       �          �   h = Hold Note                             �

�         �       �          �         0 = Don't hold note                 �

�         �       �          �         1 = Hold note                       �

�         �       �          �   e = Envelope Select                       �

�         �       �          �         0 = Envelope Vary                   �

�         �       �          �         1 = Envelope Fixed                  �

�         �       �          �   s = Playback Rate                         �

��������������������������������������������������������������������������Ĵ

� $4001   � RW    � fsssHrrr � Square Wave Control Register #2             �

�         �       �          �                                             �

�         �       �          �   f = Frequency Fixed/Variable Select       �

�         �       �          �          0 = Fixed  (bits 0-6 disabled)     �

�         �       �          �          1 = Variable (bits 0-6 enabled)    �

�         �       �          �   s = Frequency Change Speed                �

�         �       �          �   H = Low/High Frequency Select             �

�         �       �          �          0 = Low -> High                    �

�         �       �          �          1 = High -> Low                    �

�         �       �          �   r = Frequency Range (0=Min, 7=Max)        �

��������������������������������������������������������������������������Ĵ

� $4002   � RW    � dddddddd � Square Wave Frequency Value Register #1     �

�         �       �          �                                             �

�         �       �          �   d = Frequency Value Data (lower 8-bits)   �

��������������������������������������������������������������������������Ĵ

� $4003   � RW    � tttttddd � Square Wave Frequency Value Register #2     �

�         �       �          �                                             �

�         �       �          �   d = Frequency Value Data (upper 3-bits)   �

�         �       �          �   t = Active Time Length                    �

�         �       �          �                                             �

�         �       �          � NOTE: The Frequency Value is a full 11-bits �

�         �       �          �       in size; be aware you will need to    �

�         �       �          �       write the upper 3-bits to $4003.      �

��������������������������������������������������������������������������Ĵ

� $4004   � RW    � CChessss � Square Wave Control Register #1             �

�         �       �          �                                             �

�         �       �          �   C = Duty Cycle (Positive vs. Negative)    �

�         �       �          �        00 = 87.5%                           �

�         �       �          �        01 = 75.0%                           �

�         �       �          �        10 = 58.0%                           �

�         �       �          �        11 = 25.0%                           �

�         �       �          �   h = Hold Note                             �

�         �       �          �         0 = Don't hold note                 �

�         �       �          �         1 = Hold note                       �

�         �       �          �   e = Envelope Select                       �

�         �       �          �         0 = Envelope Vary                   �

�         �       �          �         1 = Envelope Fixed                  �

�         �       �          �   s = Playback Rate                         �

��������������������������������������������������������������������������Ĵ

� $4005   � RW    � fsssHrrr � Square Wave Control Register #2             �

�         �       �          �                                             �

�         �       �          �   f = Frequency Fixed/Variable Select       �

�         �       �          �          0 = Fixed  (bits 0-6 disabled)     �

�         �       �          �          1 = Variable (bits 0-6 enabled)    �

�         �       �          �   s = Frequency Change Speed                �

�         �       �          �   H = Low/High Frequency Select             �

�         �       �          �          0 = Low -> High                    �

�         �       �          �          1 = High -> Low                    �

�         �       �          �   r = Frequency Range (0=Min, 7=Max)        �

��������������������������������������������������������������������������Ĵ

� $4006   � RW    � dddddddd � Square Wave Frequency Value Register #1     �

�         �       �          �                                             �

�         �       �          �   d = Frequency Value Data (lower 8-bits)   �

��������������������������������������������������������������������������Ĵ

� $4007   � RW    � tttttddd � Square Wave Frequency Value Register #2     �

�         �       �          �                                             �

�         �       �          �   d = Frequency Value Data (upper 3-bits)   �

�         �       �          �   t = Active Time Length                    �

�         �       �          �                                             �

�         �       �          � NOTE: The Frequency Value is a full 11-bits �

�         �       �          �       in size; be aware you will need to    �

�         �       �          �       write the upper 3-bits to $4007.      �

��������������������������������������������������������������������������Ĵ

� $4008   � RW    � CChessss � Triangle Wave Control Register #1           �

�         �       �          �                                             �

�         �       �          �   C = Duty Cycle (Positive vs. Negative)    �

�         �       �          �        00 = 87.5%                           �

�         �       �          �        01 = 75.0%                           �

�         �       �          �        10 = 58.0%                           �

�         �       �          �        11 = 25.0%                           �

�         �       �          �   h = Hold Note                             �

�         �       �          �         0 = Don't hold note                 �

�         �       �          �         1 = Hold note                       �

�         �       �          �   e = Envelope Select                       �

�         �       �          �         0 = Envelope Vary                   �

�         �       �          �         1 = Envelope Fixed                  �

�         �       �          �   s = Playback Rate                         �

��������������������������������������������������������������������������Ĵ

� $4009   � RW    � fsssHrrr � Triangle Wave Control Register #2           �

�         �       �          �                                             �

�         �       �          �   f = Frequency Fixed/Variable Select       �

�         �       �          �          0 = Fixed  (bits 0-6 disabled)     �

�         �       �          �          1 = Variable (bits 0-6 enabled)    �

�         �       �          �   s = Frequency Change Speed                �

�         �       �          �   H = Low/High Frequency Select             �

�         �       �          �          0 = Low -> High                    �

�         �       �          �          1 = High -> Low                    �

�         �       �          �   r = Frequency Range (0=Min, 7=Max)        �

��������������������������������������������������������������������������Ĵ

� $400A   � RW    � dddddddd � Triangle Wave Frequency Value Register #1   �

�         �       �          �                                             �

�         �       �          �   d = Frequency Value Data (lower 8-bits)   �

��������������������������������������������������������������������������Ĵ

� $400B   � RW    � tttttddd � Triangle Wave Frequency Value Register #2   �

�         �       �          �                                             �

�         �       �          �   d = Frequency Value Data (upper 3-bits)   �

�         �       �          �   t = Active Time Length                    �

�         �       �          �                                             �

�         �       �          � NOTE: The Frequency Value is a full 11-bits �

�         �       �          �       in size; be aware you will need to    �

�         �       �          �       write the upper 3-bits to $400B.      �

��������������������������������������������������������������������������Ĵ

� $400C   � RW    � CChessss � Noise Control Register #1                   �

�         �       �          �                                             �

�         �       �          �   C = Duty Cycle (Positive vs. Negative)    �

�         �       �          �        00 = 87.5%                           �

�         �       �          �        01 = 75.0%                           �

�         �       �          �        10 = 58.0%                           �

�         �       �          �        11 = 25.0%                           �

�         �       �          �   h = Hold Note                             �

�         �       �          �         0 = Don't hold note                 �

�         �       �          �         1 = Hold note                       �

�         �       �          �   e = Envelope Select                       �

�         �       �          �         0 = Envelope Vary                   �

�         �       �          �         1 = Envelope Fixed                  �

�         �       �          �   s = Playback Rate                         �

��������������������������������������������������������������������������Ĵ

� $400D   � RW    � fsssHrrr � Noise Control Register #2                   �

�         �       �          �                                             �

�         �       �          �   f = Frequency Fixed/Variable Select       �

�         �       �          �          0 = Fixed  (bits 0-6 disabled)     �

�         �       �          �          1 = Variable (bits 0-6 enabled)    �

�         �       �          �   s = Frequency Change Speed                �

�         �       �          �   H = Low/High Frequency Select             �

�         �       �          �          0 = Low -> High                    �

�         �       �          �          1 = High -> Low                    �

�         �       �          �   r = Frequency Range (0=Min, 7=Max)        �

��������������������������������������������������������������������������Ĵ

� $400E   � RW    � dddddddd � Frequency Value Register #1                 �

�         �       �          �                                             �

�         �       �          �   d = Frequency Value Data (lower 8-bits)   �

��������������������������������������������������������������������������Ĵ

� $400F   � RW    � tttttddd � Frequency Value Register #2                 �

�         �       �          �                                             �

�         �       �          �   d = Frequency Value Data (upper 3-bits)   �

�         �       �          �   t = Active Time Length                    �

�         �       �          �                                             �

�         �       �          � NOTE: The Frequency Value is a full 11-bits �

�         �       �          �       in size; be aware you will need to    �

�         �       �          �       write the upper 3-bits to $400F.      �

��������������������������������������������������������������������������Ĵ

� $4010   � RW    � CChessss � PCM Control Register #1                     �

�         �       �          �                                             �

�         �       �          �   C = Duty Cycle (Positive vs. Negative)    �

�         �       �          �        00 = 87.5%                           �

�         �       �          �        01 = 75.0%                           �

�         �       �          �        10 = 58.0%                           �

�         �       �          �        11 = 25.0%                           �

�         �       �          �   h = Hold Note                             �

�         �       �          �         0 = Don't hold note                 �

�         �       �          �         1 = Hold note                       �

�         �       �          �   e = Envelope Select                       �

�         �       �          �         0 = Envelope Vary                   �

�         �       �          �         1 = Envelope Fixed                  �

�         �       �          �   s = Playback Rate                         �

��������������������������������������������������������������������������Ĵ

� $4011   � RW    � vvvvvvvv � PCM Volume Control Register                 �

�         �       �          �                                             �

�         �       �          �   v = Volume                                �

��������������������������������������������������������������������������Ĵ

� $4012   � RW    � aaaaaaaa � PCM Address Register                        �

�         �       �          �                                             �

�         �       �          �   a = Address                               �

��������������������������������������������������������������������������Ĵ

� $4013   � RW    � LLLLLLLL � PCM Data Length Register                    �

�         �       �          �                                             �

�         �       �          �   L = Data Size/Length                      �

��������������������������������������������������������������������������Ĵ

� $4014   �  W    �          � Sprite DMA                        [SPRDMA]  �

�         �       �          �                                             �

�         �       �          �  Transfers 256 bytes of memory at address   �

�         �       �          �  $100*N, where N is the value written to    �

�         �       �          �  this register, into Sprite RAM.            �

��������������������������������������������������������������������������Ĵ

� $4015   � RW    � 000abcde � Sound Control Register            [SNDCNT]  �

�         �       �          �                                             �

�         �       �          �  a = Channel 1  (0=Disable, 1=Enable)       �

�         �       �          �  b = Channel 2  (0=Disable, 1=Enable)       �

�         �       �          �  c = Channel 3  (0=Disable, 1=Enable)       �

�         �       �          �  d = Channel 4  (0=Disable, 1=Enable)       �

�         �       �          �  e = Channel 5  (0=Disable, 1=Enable)       �

��������������������������������������������������������������������������Ĵ

� $4016   � RW    � ???STeed � Joypad #1                         [SPECIO1] �

�         �       �          � [READING]                                   �

�         �       �          �                                             �

�         �       �          �  S = Zapper sprite detection                �

�         �       �          �         0 = Sprite not detected             �

�         �       �          �         1 = Sprite detected in front of     �

�         �       �          �             crosshair                       �

�         �       �          �  T = Zapper trigger                         �

�         �       �          �         0 = Pressed                         �

�         �       �          �         1 = Not pressed                     �

�         �       �          �  e = Expansion Port Data                    �

�         �       �          �  d = Joypad Data (see Section #8)           �

�         �       �          ���������������������������������������������Ĵ

�         �       � ?????eej � [WRITING]                                   �

�         �       �          �                                             �

�         �       �          �  j = Joypad Strobe                          �

�         �       �          �         0 = Clear joypad strobe             �

�         �       �          �         1 = Reset joypad strobe             �

�         �       �          �                                             �

�         �       �          �  e = Expansion Port Data                    �

��������������������������������������������������������������������������Ĵ

� $4017   � R     � ?????eed � Joypad #2                         [SPECIO2] �

�         �       �          � [READING]                                   �

�         �       �          �                                             �

�         �       �          �  e = Expansion Port Data                    �

�         �       �          �  d = Joypad Data (see Section #8)           �

����������������������������������������������������������������������������

����������������������Ŀ

� 7 �   VBL/Hit Bits   �

������������������������

The VBlank flag is contained in the 7th bit of read-only location $2002.

It indicates whether PPU is scanning the screen, or generating a vertical

blanking impulse. It is set in the end of each frame (scanline 232), and

stays on until the next screen refresh starts from scanline 8. The pro-

gram can reset this bit prematurely by reading $2002.

The Hit Flag is contained in the 6th bit of read-only location $2002.

It goes to 1 when PPU starts refreshing the first scanline where sprite#0

is located. For example, if sprite#0's Y coordinate is 34, the Hit flag

will be set in scanline 34. The Hit flag is reset when vertical blanking

impulse starts. The program can reset this bit prematurely by reading from

$2002. There is a total of 262 lines, but only 239 are shown (or 224 for

NTSC).

�����������������������Ŀ

� 8 �   Joypad/Zapper   �

�������������������������

There are two joysticks which are accessed via locations $4016 and

$4017. To reset joysticks, write first 1, then 0 into $4016. This way, you

will generate a strobe in the joysticks' circuitry. Then, read either from

$4016 (for joystick 0) or from $4017 (for joystick 1).  Each read will

give you the status of a single button in the 0th bit (1 if pressed, 0

otherwise):

����������������������������������������������������������������Ŀ

� Read # �  1  �  2  �   3    �   4   � 5  �  6   �  7   �   8   �

����������������������������������������������������������������Ĵ

�  A  �  B  � SELECT � START � UP � DOWN � LEFT � RIGHT �

���������������������������������������������������������

Bit 1 indicates whether joystick is connected to the port or not. It is

set to 0 if the joystick is connected, 1 if not. Bits 6 and 7 of $4016

and $4017 also seem to have some significance, which is not clear yet.

The rest of bits is set to zeroes. Some games expect to get *exactly* $41

from $4016/$4017, if a button is pressed, which has to be taken into

account.

For Zapper (Light Gun) information, see Section #6.

�����������������Ŀ

� 9 �   Sprites   �

�������������������

There are 64 sprites, which can be either 8x8 or 8x16 pixels. Sprites

patterns are stored in on of the Pattern Tables in the PPU Memory. Sprite

attributes are stored in the Sprite RAM of 256 bytes, which is not a

part of neither CPU nor PPU address space. The entire contents of Sprite

Memory can be written via DMA (see Section #6). Sprite RAM can also be

accessed byte-by-byte by putting the starting address into $2003 and then

writing/reading $2004 (the address will auto-increment). The format of

Sprite RAM is as follows:

�������������������������������������������������������Ŀ

� Sprite #0 � Sprite #1 � ... � Sprite #62 � Sprite #63 �

���������������������������������������������������������

�    �������������������������������������������������������������Ŀ

����Ĵ Byte # � Bits     � Description                             �

�������������������������������������������������������������Ĵ

�   0    � YYYYYYYY � Sprite Y coordinate - 1. Consider the   �

�        �          � coordinate the upper-left corner of     �

�        �          � the sprite itself.                      �

�   1    � IIIIIIII � Sprite Tile Index #                     �

�   2    � vhp???cc � Colour/Attributes                       �

�        �          �  v = Vertical Flip   (1=Flip)           �

�        �          �  h = Horizontal Flip (1=Flip)           �

�        �          �  p = Sprite Priority Bit                �

�        �          �         0 = Sprite on top of background �

�        �          �         1 = Sprite behind background    �

�        �          �  c = Upper two (2) bits of colour       �

�   3    � XXXXXXXX � Sprite X coordinate (upper-left corner) �

���������������������������������������������������������������

The Sprite Tile Index # is obtained the same way as Tile Index #s are

in regards to the Name Table (background) picture.

For 8x16 sprites, the Pattern Table is at $0000 in VRAM, containing 256

8x16 tiles. Bit #3 of $2001 (see Section #6) has no effect on 8x16 sprites.

The Sprite Tile Index # in the Sprite Attribute RAM is rotated right

one (1) bit by the PPU, when drawing. Therefore, a Tile Index # $01 will

draw tile #128, $02 would draw tile #1, $03 would draw tile #129, etc..

The Sprite Priority bit works as follows: Sprites with a lower Tile

Index # will have a higher priority (e.g. #0 will be drawn over #1).

���������������Ŀ

� 10 �   MMCs   �

�����������������

Each reference number below (e.g. "3)" or "1)") is the iNES Mapper

number. The actual MMC title is printed after, if one is available. Do not

get these confused.

1) Nintendo MMC1

This mapper is commonly used in 256K carts, such as Bomberman 2,

Destiny Of The Emperor, Megaman 2, Airwolf, Operation Wolf, Castlevania 2,

SilkWorm, and others. It may be used to switch PRG-ROM and CHR-RAM.  This

MMC also supports WRAM.

MMC1 has four (4) 8 bit registers, which are accessed via following

addresses:

�����������������������������������������������������������������������Ŀ

� Reg # � Range � Bits     � Description                                �

�����������������������������������������������������������������������Ĵ

�   0   � $8000 � ???s???m � Mapper Control Register #1                 �

�       �  ...  �          �                                            �

�       � $9FFF �          �  s = Size Select                           �

�       �       �          �         0 = Select 8K CHR-RAM at $0000     �

�       �       �          �         1 = Swap 4K at $0000 and $1000     �

�       �       �          �  m = Mirror Select                         �

�       �       �          �         0 = Horizontal Mirroring           �

�       �       �          �         1 = Vertical Mirroring             �

�����������������������������������������������������������������������Ĵ

�   1   � $A000 � ssssssss � CHR-RAM Page Selection Register            �

�       �  ...  �          �                                            �

�       � $BFFF �          �  Sets the CHR-RAM page at $0000, with the  �

�       �       �          �  size selected via Register #0.            �

�����������������������������������������������������������������������Ĵ

�   2   � $C000 � ssssssss � CHR-RAM 4K Page Selection Register         �

�       �  ...  �          �                                            �

�       � $DFFF �          �  Sets the 4K CHR-RAM page at $1000, but    �

�       �       �          �  only if 4K CHR-RAM pages are selected via �

�       �       �          �  Register #0 (otherwise ignored).          �

�����������������������������������������������������������������������Ĵ

�   3   � $E000 � ssssssss � PRG-ROM 16K Page Selection Register        �

�       �  ...  �          �                                            �

�       � $FFFF �          �  Sets the 16K ROM page at $8000. The page  �

�       �       �          �  at $C000 is hardwired to the last ROM     �

�       �       �          �  page in the cart. Page 0 starts at $8000. �

�������������������������������������������������������������������������

Before writing to a mapper register for the first time, you need to reset

it by setting bit #7 in each address range base ($8000, $A000, $C000, and

$E000) before writing data to each address range itself. Once you've

done this, you may write the value bit by bit to the appropriate address

range. For example, the following code will write $0C into register 3:

lda  #%10000000

sta  $8000      ; Resetting range #0

sta  $A000      ; Resetting range #1

sta  $C000      ; Resetting range #2

sta  $E000      ; Resetting range #3

lda  #$0C       ; This is our value

sta  $EFD9      ; Writing bit 0

lsr  a          ; Shifting

sta  $EFD9      ; Writing bit 1

lsr  a          ; Shifting

sta  $EFD9      ; Writing bit 2

lsr  a          ; Shifting

sta  $EFD9      ; Writing bit 3

lsr  a          ; Shifting

sta  $EFD9      ; Writing bit 4

2) 74HC161/74HC32 Mapper

This is a quite simple mapper used in most Konami (Life Force,

Castlevania, Metal Gear) and some other cartridges (Ghosts & Goblins). It

only switches the ROM. All cartridges with this mapper have 8kB CHR-RAM

at $0000. The mapper has a single 8 bit register which can be written via

locations $8000-$FFFF. It contains a number of 16K PRG-ROM page at $8000.

The page at $C000 is always hardwired to the last ROM page in the cart.

The cartridge starts with page 0 at $8000.

3) VROM Switch

This mapper is used in the Goonies series and many Japanese-only games.

It only allows you to switch 8kB pages of CHR-RAM. The PRG-ROM is either

16K or 32K and is not paged. The mapper has a single 8-bit register which

can be written via locations $8000-$FFFF. It contains a number of the 8K

CHR-RAM page at $0000.

4) Nintendo MMC3

This MMC is used in many recent cartridges, such as: Batman Returns,

Super Contra, Vindicators, Silver Surfer, Crystalis, Legacy of the Wizard,

and others. This MMC is able to generate it's own interrupts via the IRQ

line, and has a set of commands to switch PRG-ROM and CHR-RAM. CHR-RAM

pages are 1K, PRG-ROM are 8K.

NOTE: The ROM pages at $C000 and $E000 are hardwired to the last pages

of the ROM, and are not switchable. However, they can be swapped via $E000

though.

��������������������������������������������������������������������������Ŀ

� Address � Stats � Bits     � Description                                 �

��������������������������������������������������������������������������Ĵ

� $8000   �  W ?  � as???ddd � Function Select Register                    �

�         �       �          �                                             �

�         �       �          �    a = CHR-RAM Base Address Select          �

�         �       �          �         0 = $0000                           �

�         �       �          �         1 = $1000                           �

�         �       �          �    s = CHR-RAM Page Base Select (CMDs 6/7)  �

�         �       �          �         0 = Select $8000 and $A000          �

�         �       �          �         1 = Select $A000 and $C000          �

�         �       �          �  ddd = Function # (0-7)                     �

�         �       �          �                                             �

�         �       �          � NOTE: Writing to this register resets       �

�         �       �          �       changes made to the $E000 register.   �

��������������������������������������������������������������������������Ĵ

� $8001   �  W    � dddddddd � Page Number Select Register                 �

�         �       �          �                                             �

�         �       �          �  This register selects the page # to be     �

�         �       �          �  read from (or written to).                 �

�         �       �          �                                             �

�         �       �          � NOTE: When using CMD 0, Bit #0 of this reg- �

�         �       �          �       ister is ignored. Therefore, a value  �

�         �       �          �       of 5 will select pages 4 & 5).        �

��������������������������������������������������������������������������Ĵ

� $A000   �  W    � ???????m � Shadow Select Register                      �

�         �       �          �                                             �

�         �       �          �  m = Shadowing                              �

�         �       �          �         0 = Vertical                        �

�         �       �          �         1 = Horizontal                      �

��������������������������������������������������������������������������Ĵ

� $A001   �  W ?  � p??????? � Pattern Table Control Register              �

�         �       �          �                                             �

�         �       �          �  p = Pattern Table Enable/Disable           �

�         �       �          �         0 = Disable use of $0000-$1FFF in   �

�         �       �          �             VRAM                            �

�         �       �          �         1 = Enable use of $0000-$1FFF in    �

�         �       �          �             VRAM                            �

�         �       �          �                                             �

�         �       �          � NOTE: Information here could possibly be    �

�         �       �          �       incorrect.                            �

��������������������������������������������������������������������������Ĵ

� $C000   �       � dddddddd � IRQ Decrement Register                      �

�         �       �          �                                             �

�         �       �          �  A value (1-255) is written here. The chip  �

�         �       �          �  auto-decrements this value at every VBL.   �

�         �       �          �  Once 0 is hit, IRQ/BRK is executed. If 0   �

�         �       �          �  is stored, then this feature is disabled.  �

��������������������������������������������������������������������������Ĵ

� $C001   �  W    �          � Temporary Latch Register                    �

�         �       �          �                                             �

��������������������������������������������������������������������������Ĵ

� $E000   �  W    �          � IRQ Control Register #1                     �

�         �       �          �                                             �

�         �       �          �  Any writes which occur to this register    �

�         �       �          �  will cause the value written to $C001 to   �

�         �       �          �  be copied into $C000. IRQs are also dis-   �

�         �       �          �  abled as well.                             �

��������������������������������������������������������������������������Ĵ

� $E001   �  W    �          � IRQ Control Register #2                     �

�         �       �          �                                             �

�         �       �          �  Any writes to this register enable IRQs.   �

����������������������������������������������������������������������������

In order to use the mapper, you should first write a command number

into $8000, and then a value (page number) into $8001. Following commands

exist:

Cmd  Function

���������������������������������������������������������������������������

0   Select two (2) 1K CHR-RAM pages at the following address:

CHR-RAM Base Address + $0000

See Register $8001 for more details regarding page selection.

1   Select two (2) 1K CHR-RAM pages at the following address:

CHR-ROM Base Address + $0800

See Register $8001 for more details regarding page selection.

2   Select a 1K CHR-RAM page at CHR-RAM Base Address + $1000

3   Select a 1K CHR-RAM page at CHR-RAM Base Address + $1400

4   Select a 1K CHR-RAM page at CHR-RAM Base Address + $1800

5   Select a 1K CHR-RAM page at CHR-RAM Base Address + $1C00

6   Select a 8K PRG-ROM page at CHR-RAM Page Base Select (see Reg. $8000).

The initial value seems to be 0.

7   Select a 8K PRG-ROM page at CHR-ROM Page Base Select (see Reg. $8000).

The initial value seems to be 1.

���������������������������������������������������������������������������

5) Nintendo MMC5

This mapper is used in Castlevania III. It supports a quad-table layout,

allowing four (4) physical Name Tables to be used.

According to Nintendo Power Issue #20, this mapper allows you to extend

the colour capabilities of the NES, from 2 bits per attribute byte to 4

bits per attribute byte. It also is stated that this mapper allows vertical

split-screening; how either of these are possible I do not know.

6) FFE F4xxx Mapper

This mapper allows swapping of PRG-ROM, and supports four (4) Pattern

Tables. This mapper was invented by FFE (Front Far East), and seems to

only be used on F4xxx-style games, such as the famous Wai Wai World

("Konami World"), and others. It also supports IRQ and counter registers,

resembling that of Nintendo's MMC3.

Two styles of data can be written here, but most of the time, the format

of the more commonly used data (used in "Wai Wai World") is:

��������������������������������������������������������������������������Ŀ

� Address � Stats � Bits     � Description                                 �

��������������������������������������������������������������������������Ĵ

� $8000   � RW    � 00ppppCC � PRG-ROM/CHR-RAM Control Register            �

� $A000   �       �          �                                             �

� $C000   �       �          �    p = 16K PRG-ROM Select                   �

� $E000   �       �          �         Selects 16K PRG-ROM bank at $8000   �

�         �       �          �   CC = Pattern Table Select                 �

�         �       �          �         Selects Pattern Table #0-3          �

����������������������������������������������������������������������������

The more "uncommonly used" format is:

��������������������������������������������������������������������������Ŀ

� Address � Stats � Bits     � Description                                 �

��������������������������������������������������������������������������Ĵ

� $8000   � RW    � 00CCpppp � PRG-ROM/CHR-RAM Control Register            �

� $A000   �       �          �                                             �

� $C000   �       �          �    p = 16K PRG-ROM Select                   �

� $E000   �       �          �         Selects 16K PRG-ROM bank at $8000   �

�         �       �          �   CC = Pattern Table Select                 �

�         �       �          �         Selects Pattern Table #0-3          �

����������������������������������������������������������������������������

NOTE: It seems most these images are purely PRG-ROM; i.e. no CHR-RAM. All

the CHR-RAM is kept inside the PRG-ROM, which is swapped in and out during

runtime via the MMC.

7) ROM Switch

This mapper is used in Wizards & Warriors (1 & 2) and Deadly Towers.

Writes to $8000-$FFFF select the 32K PRG-ROM bank at $8000. Bit #4

controls which Name Table and Attribute Table to use; the other Name Tables

and Attribute Tables mirror the table selected using Bit #4.

8) FFE F3xxx Mapper

No information is currently available.

9) Nintendo MMC2

This MMC is used in the "Mike Tyson's Punch-Out!" cart. It supports

switching of PRG-ROM and CHR-RAM.

All PRG-ROM banks are 8K (vs. 16K) in size, with a base ROM address of

$8000. CHR-RAM banks appear to be 4K in size,

��������������������������������������������������������������������������Ŀ

� Address � Stats � Bits     � Description                                 �

��������������������������������������������������������������������������Ĵ

� $A000   �  W    � dddddddd � PRG-ROM 8K Page Select Register             �

�  ...    �       �          �                                             �

� $AFFF   �       �          �    d = 8K PRG-ROM Select                    �

�         �       �          �         Selects 8K PRG-ROM bank at $8000.   �

�         �       �          �                                             �

� $B000   �  W    � dddddddd � CHR-RAM 4K Page Select Register #1          �

� $C000   �       �          �                                             �

�         �       �          �    d = 4K CHR-RAM Select                    �

�         �       �          �         Selects 4K CHR-RAM bank at VRAM     �

�         �       �          �         address $0000.                      �

�         �       �          �                                             �

� $D000   �  W    � dddddddd � CHR-RAM 4K Page Select Register #2          �

� $E000   �       �          �                                             �

�         �       �          �    d = 4K CHR-RAM Select                    �

�         �       �          �         Selects 4K CHR-RAM bank at VRAM     �

�         �       �          �         address $1000.                      �

����������������������������������������������������������������������������

The last three (3) PRG-ROM banks in the cart itself are hardwired to the

last three (3) 8K sections of ROM. For instance, in the case of "Mike

Tyson's Punch-Out!" which has sixteen (16) 8K PRG-ROM banks, PRG-ROM

bank #13, #14, and #15 would be mapped to $A000, $C000, and $E000, respec-

tively.

NOTE: The CHR-RAM 4K Page Select Register seem to work in congruency;

by this I mean, when data is written to $B000, $D000 will function, but

$E000 will not (and therefore, the same rules apply to $C000/$E000 where

$D000 will not work).

10) [Unused]

11) Colour Dreams Mapper

This mapper is commonly used in Colour Dreams games, but not all of

them will function correctly when using this mapper.

PRG-ROM banks are 32K in size, and CHR-RAM banks are 8K.

��������������������������������������������������������������������������Ŀ

� Address � Stats � Bits     � Description                                 �

��������������������������������������������������������������������������Ĵ

� $8000   �  W    � CCCCpppp � PRG-ROM/CHR-RAM Page Select Register        �

�  ...    �       �          �                                             �

� $FFFF   �       �          �    C = 8K CHR-RAM Select                    �

�         �       �          �         Selects 8K CHR-RAM bank at VRAM     �

�         �       �          �         address $0000.                      �

�         �       �          �    p = 32K PRG-ROM Select                   �

�         �       �          �         Selects 32K PRG-ROM bank at $8000.  �

����������������������������������������������������������������������������

12) [Unused]

13) [Unused]

14) [Unused]

15) 100-in-1 Mapper

NOTE: Assume ROM16k[] is an array of 16K PRG-ROM Banks.

NOTE: Unlike other MMCs, the 16K of PRG-ROM loaded into $C000 on startup

is the second 16K PRG-ROM page, not the last 16K PRG-ROM page.

��������������������������������������������������������������������������Ŀ

� Address � Stats � Bits     � Description                                 �

��������������������������������������������������������������������������Ĵ

� $8000   �  W ?  � smNNNNNN � PRG-ROM Control Register #1                 �

�         �       �          �                                             �

�         �       �          �    s = Swap 8K Pages                        �

�         �       �          �         Swaps 8K at $8000 and $A000         �

�         �       �          �         Swaps 8K at $C000 and $E000         �

�         �       �          �    m = Mirroring Control                    �

�         �       �          �         0 = Vertical                        �

�         �       �          �         1 = Horizontal                      �

�         �       �          �    N = PRG-ROM Page Select                  �

�         �       �          �         $8000 now holds ROM16k[N]           �

�         �       �          �         $C000 now holds ROM16k[N+1]         �

��������������������������������������������������������������������������Ĵ

� $8001   �  W    � s?NNNNNN � PRG-ROM Control Register #2                 �

�         �       �          �                                             �

�         �       �          �    s = Swap 8K Pages                        �

�         �       �          �         Swaps 8K at $C000 and $E000         �

�         �       �          �    N = PRG-ROM Page Select                  �

�         �       �          �         $C000 now holds ROM16k[N]           �

��������������������������������������������������������������������������Ĵ

� $8002   �  W    � S?NNNNNN � PRG-ROM Control Register #3                 �

�         �       �          �                                             �

�         �       �          �    S = Upper/Lower 8K Select                �

�         �       �          �         0 = Select Lower 8K of 16K segment  �

�         �       �          �         1 = Select Upper 8K of 16K segment  �

�         �       �          �    N = 8K PRG-ROM Page Select               �

�         �       �          �         $8000 now holds ROM16k[N]           �

�         �       �          �         $A000 now holds ROM16k[N]           �

�         �       �          �         $C000 now holds ROM16k[N]           �

�         �       �          �         $E000 now holds ROM16k[N]           �

�         �       �          �                                             �

�         �       �          � NOTE: Bit 7 handles only Bits 0-5 of this   �

�         �       �          �       register, and does not affect any     �

�         �       �          �       other registers.                      �

�         �       �          �                                             �

�         �       �          � NOTE: Bits 0-5 base their 8K Selection on   �

�         �       �          �       Bit 7. Keep this in mind.             �

��������������������������������������������������������������������������Ĵ

� $8003   �  W    � smNNNNNN � PRG-ROM Control Register #4                 �

�         �       �          �                                             �

�         �       �          �    s = Swap 8K Pages                        �

�         �       �          �         Swaps 8K at $C000 and $E000         �

�         �       �          �    m = Mirroring Control                    �

�         �       �          �         0 = Vertical                        �

�         �       �          �         1 = Horizontal                      �

�         �       �          �    N = ROM Page Select                      �

�         �       �          �         $C000 now holds ROM16k[N]           �

����������������������������������������������������������������������������

���������������������������Ŀ

� 11 �   .NES File Format   �

�����������������������������

This is the .NES File Format, created by Marat Fayzullin (author of

iNES).

����������������������������������������������������������������������Ŀ

� Offset � Size � Content(s)                                           �

����������������������������������������������������������������������Ĵ

�   0    �  3   � 'NES'                                                �

�   3    �  1   � $1A                                                  �

�   4    �  1   � Number of 16K PRG-ROM banks                          �

�   5    �  1   � Number of 8K CHR-RAM banks                           �

�   6    �  1   � ROM Control Byte                                     �

�        �      �   %00000000                                          �

�        �      �    ���ٳ���� 0=Horizontal Mirroring                  �

�        �      �      � ���   1=Vertical Mirroring                    �

�        �      �      � ����� 1=WRAM located at $6000-$7FFF           �

�        �      �      � ����� 1=512-byte trainer at $7000-$71FF       �

�        �      �      � ����� 1=Four-screen VRAM layout               �

�        �      �      �                                               �

�        �      �      ������� iNES Mapper #                           �

�        �      �                                                      �

�  7-15  �  9   � [Reserved for expansion, should be $00]              �

� 16-..  �      � PRG-ROM banks (in ascending order). A trainer pre-   �

�        �      � cedes the first bank, if a trainer exists.           �

� ..-EOF �      � CHR-RAM banks (in ascending order).                  �

������������������������������������������������������������������������

������������������������������������������������������Ŀ

� 12 �   Famicom Disk System Format (.DKA/.DKB/.500)   �

��������������������������������������������������������

This is the file format for Famicom Disk System images; the only

emulator at this time to support them is Pasowing, which is the

emulator which these images are based off of.

The format for the .DKA/.DKB (DisK side-A and DisK side-B) files

is somewhat undocumented (as shown). .DKA and .DKB files are 64K in

size (65536 bytes). The format for the file is as follows:

����������������������������������������������������������������������Ŀ

� Offset � Size � Content(s)                                           �

����������������������������������������������������������������������Ĵ

� [IMAGE HEADER]                                                       �

�                                                                      �

�   0    �  1   � [Unknown]                                            �

�   1    �  3   � Disk ID                                              �

�   6    �  1   � Disk #                                               �

�   38   �  1   � Disk # (???)                                         �

�   63   �  1   � Amount of data blocks                                �

�                                                                      �

� [DATA BLOCK HEADER]                                                  �

�                                                                      �

�   64   �  1   � $03                                                  �

�   65   �  1   � Block #                                              �

�   66   �  8   � Name                                                 �

�   73   �  2   � Destination                                          �

�   75   �  2   � Size of data                                         �

�   77   �  1   � Data Type                                            �

�        �      � ���������                                            �

�        �      �     �                                                �

�        �      �     ������� $00 = PRG-ROM                            �

�        �      �             $01 = CHR-RAM                            �

�        �      �             $02 = [Unknown]                          �

� 78-..  �      � Data                                                 �

�        �      �                                                      �

� ..-EOF �      � Repeat loading DATA BLOCKS HEADERS as shown above;   �

�        �      � continue loading until EOF is reached.               �

������������������������������������������������������������������������

������������������������������������Ŀ

� 13 �   Notes to emulator authors   �

��������������������������������������

The NES is a 6502 (NMOS) CPU. It is not a 65C02 (CMOS) CPU as rumoured.

Ignore opcodes which are "bad" (not in the 6502 instruction set); a lot

of ROMs available are flawed, due to bad backup units or readers. I've

noticed this in the "Twinbee" and "Adventure of Lolo" ROMs floating around.

If a trainer is present in the .NES image, your emulator should load the

trainer into it's respective memory range, as well as PRG-ROM and CHR-RAM.

Begin executing code at $7000, not at the RESET vector.

The Four-screen VRAM layout is used within other mappers, not just iNES

Mapper #5 (MMC5). I have seen it used in iNES Mapper #15 and others.

A reminder: MMC != iNES Mapper.

�����������������Ŀ

� 14 �   Thanks   �

�������������������

Thanks to the following people for helping make this document what it

has become today. In alphabetical order:

Acey                (d0p@sofi.ah.dk)

Alex Krasivsky      (bcat@lapkin.rosprint.ru)

Avatar Z            (swahlen@nfinity.com)

Bloodlust Software  (bldlust@southwind.net)

CiXeL

D                   (d@animal.blarg.net)

Dan Boris           (dan.boris@coat.com)

FanWen              (yangfanw@ms4.hinet.net)

Kevin Horton        (khorton@iquest.net)

Loopy               (loopy@itsnet.com)

Marat Fayzullin     (fms@freeflight.com)

Mike Perry          (mj-perry@uiuc.edu)

MindRape            (mindrape@goodnet.com)

Neill Corlett       (corlett@elwha.nrrc.ncsu.edu)

Pat Mccomack        (splat@primenet.com)

Paul Robson         (AutismUK@aol.com)

star69              (mr6v@andrew.cmu.edu)

Stumble             (stumble@alpha.pulsar.net)

Typhoon Z           (typhoonz@parodius.com)

��������������������Ŀ

� 15 �   Revisions   �

����������������������

v0.31: [#6] MMC2 added.

[#6] Mapper #11 added.

[#6] Mapper #6 updated.

v0.30: [--] First public release.