PART 103--ULTRALIGHT VEHICLES

Special Federal Aviation Regulation No. 45-1 [Note]

Subpart A--General

Sec.

103.1  Applicability.

103.3  Inspection requirements.

103.5  Waivers.

103.7  Certification and registration.

Subpart B--Operating Rules

103.9  Hazardous operations.

103.11  Daylight operations.

103.13  Operation near aircraft; right-of-way rules.

103.15  Operations over congested areas.

103.17  Operations in certain airspace.

103.19  Operations in prohibited or restricted areas.

103.20  Flight restrictions in the proximity of certain areas designated by

notice to airmen.

103.21  Visual reference with the surface.

103.23  Flight visibility and cloud clearance requirements.

Authority: 49 U.S.C. app. 1348, 1354(a), 1421(a), 1422, and 1423; 49

U.S.C. app. 1655(c).

Source: Docket No. 21631, 47 FR 38776, Sept. 2, 1982, unless otherwise

noted.

Special Federal Aviation Regulation No. 45-1

Editorial Note: For the text of SFAR No. 45-1, see Part 71 of this chapter.

Subpart A--General

Sec. 103.1  Applicability.

This part prescribes rules governing the operation of ultralight vehicles

in the United States. For the purposes of this part, an ultralight vehicle is

a vehicle that:

(a) Is used or intended to be used for manned operation in the air by a

single occupant;

(b) Is used or intended to be used for recreation or sport purposes only;

(c) Does not have any U.S. or foreign airworthiness certificate; and

(d) If unpowered, weighs less than 155 pounds; or

(e) If powered:

(1) Weighs less than 254 pounds empty weight, excluding floats and safety

devices which are intended for deployment in a potentially catastrophic

situation;

(2) Has a fuel capacity not exceeding 5 U.S. gallons;

(3) Is not capable of more than 55 knots calibrated airspeed at full power

in level flight; and

(4) Has a power-off stall speed which does not exceed 24 knots calibrated

airspeed.

Sec. 103.3  Inspection requirements.

(a) Any person operating an ultralight vehicle under this part shall, upon

request, allow the Administrator, or his designee, to inspect the vehicle to

determine the applicability of this part.

(b) The pilot or operator of an ultralight vehicle must, upon request of

the Administrator, furnish satisfactory evidence that the vehicle is subject

only to the provisions of this part.

Sec. 103.5  Waivers.

No person may conduct operations that require a deviation from this part

except under a written waiver issued by the Administrator.

Sec. 103.7  Certification and registration.

(a) Notwithstanding any other section pertaining to certification of

aircraft or their parts or equipment, ultralight vehicles and their component

parts and equipment are not required to meet the airworthiness certification

standards specified for aircraft or to have certificates of airworthiness.

(b) Notwithstanding any other section pertaining to airman certification,

operators of ultralight vehicles are not required to meet any aeronautical

knowledge, age, or experience requirements to operate those vehicles or to

have airman or medical certificates.

(c) Notwithstanding any other section pertaining to registration and

marking of aircraft, ultralight vehicles are not required to be registered or

to bear markings of any type.

Subpart B--Operating Rules

Sec. 103.9  Hazardous operations.

(a) No person may operate any ultralight vehicle in a manner that creates a

hazard to other persons or property.

(b) No person may allow an object to be dropped from an ultralight vehicle

if such action creates a hazard to other persons or property.

Sec. 103.11  Daylight operations.

(a) No person may operate an ultralight vehicle except between the hours of

sunrise and sunset.

(b) Notwithstanding paragraph (a) of this section, ultralight vehicles may

be operated during the twilight periods 30 minutes before official sunrise

and 30 minutes after official sunset or, in Alaska, during the period of

civil twilight as defined in the Air Almanac, if:

(1) The vehicle is equipped with an operating anticollision light visible

for at least 3 statute miles; and

(2) All operations are conducted in uncontrolled airspace.

Sec. 103.13  Operation near aircraft; right-of-way rules.

(a) Each person operating an ultralight vehicle shall maintain vigilance so

as to see and avoid aircraft and shall yield the right-of-way to all

aircraft.

(b) No person may operate an ultralight vehicle in a manner that creates a

collision hazard with respect to any aircraft.

(c) Powered ultralights shall yield the right-of-way to unpowered

ultralights.

Sec. 103.15  Operations over congested areas.

No person may operate an ultralight vehicle over any congested area of a

city, town, or settlement, or over any open air assembly of persons.

Sec. 103.17   Operations in certain airspace.

No person may operate an ultralight vehicle within Class A, Class B, Class

C, or Class D airspace or within the lateral boundaries of the surface area

of Class E airspace designated for an airport unless that person has prior

authorization from the ATC facility having jurisdiction over that airspace.

EFFECTIVE DATE NOTE: Amdt. 103-4, 56 FR 65662, Dec. 17, 1991, revised

Sec. 103.17 effective September 16, 1993. The text of Sec. 130.17 in effect

until September 16, 1993 reads as follows:

Sec. 103.17  Operations in certain airspace.

No person may operate an ultralight vehicle within an airport traffic area,

control zone, airport radar service area, terminal control area, or positive

control area unless that person has prior authorization from the air traffic

control facility having jurisdiction over that airspace.

[Doc. No. 23708, 50 FR 9259, Mar. 6, 1985]

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Sec. 103.19  Operations in prohibited or restricted areas.

No person may operate an ultralight vehicle in prohibited or restricted

areas unless that person has permission from the using or controlling agency,

as appropriate.

Sec. 103.20  Flight restrictions in the proximity of certain areas designated

by notice to airmen.

No person may operate an ultralight vehicle in areas designated in a Notice

to Airmen under Sec. 91.143 or Sec. 91.141 of this chapter, unless authorized

by ATC.

[Doc. No. 24454, 50 FR 4969, Feb. 5, 1985, as amended by Amdt. 103-3, 54 FR

34331, Aug. 18, 1989]

Effective Date Note: At 54 FR 34331, August 18, 1989, Sec. 103.20 was

amended by changing the cross reference "Sec. 91.102 or Sec. 91.104" to read

"Sec. 91.143 or Sec. 91.141", effective August 18, 1990.

Sec. 103.21  Visual reference with the surface.

No person may operate an ultralight vehicle except by visual reference with

the surface.

Sec. 103.23   Flight visibility and cloud clearance requirements.

No person may operate an ultralight vehicle when the flight visibility or

distance from clouds is less than that in the table found below. All

operations in Class A, Class B, Class C, and Class D airspace or Class E

airspace designated for an airport must receive prior ATC authorization as

required in Sec. 103.17 of this part.

Flight

Airspace                visibility        Distance from clouds

Class A                           Not applicable   Not Applicable.

Class B                           3 statute miles  Clear of Clouds.

Class C                           3 statute miles  500 feet below.

1,000 feet above.

2,000 feet horizontal.

Class D                           3 statute miles  500 feet below.

1,000 feet above.

2,000 feet horizontal.

Class E:

Less than 10,000 feet MSL        3 statute miles  500 feet below.

1,000 feet above.

2,000 feet horizontal.

At or above 10,000 feet MSL      5 statute miles  1,000 feet below.

1,000 feet above.

1 statute mile horizontal.

Class G:

1,200 feet or less above the

surface (regardless of MSL

altitude)                       1 statute mile   Clear of clouds.

More than 1,200 feet above the

surface but less than 10,000

feet MSL                        1 statute mile   500 feet below.

1,000 feet above.

2,000 feet horizontal.

More than 1,200 feet above the

surface and at or above 10,000

feet MSL                        5 statute miles  1,000 feet below.

1,000 feet above.

1 statute mile horizontal.

EFFECTIVE DATE NOTE: Amdt. 103-4, 56 FR 65662, Dec. 17, 1991, revised

Sec. 103.23 effective September 16, 1993. The text of Sec. 103.23 in effect

until September 16, 1993 reads as follows:

Sec. 103.23  Flight visibility and cloud clearance requirements.

No person may operate an ultralight vehicle when the flight visibility or

distance from clouds is less than that in the following table, as

appropriate:

Minimum

flight

visibility

Flight altitudes             /1/       Minimum distance from clouds

1,200 feet or less above the

surface regardless of MSL

altitude:

(1) Within controlled airspace           3  500 feet below, 1,000 feet

above, 2,000 feet horizontal.

(2) Outside controlled                   1  Clear of clouds.

airspace

More than 1,200 feet above the

surface but less than 10,000

feet MSL:

(1) Within controlled airspace           3  500 feet below, 1,000 feet

above, 2,000 feet horizontal.

(2) Outside controlled                   1  500 feet below, 1,000 feet

airspace                                    above, 2,000 feet horizontal.

More than 1,200 feet above the            5  1,000 feet below, 1,000 feet

surface and at or above 10,000               above, 1 statute mile

feet MSL                                     horizontal.

/1/ Statute miles.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************