PART 105--PARACHUTE JUMPING

Special Federal Aviation Regulation No. 45-1 [Note]

Subpart A--General

Sec.

105.1  Applicability.

Subpart B--Operating Rules

105.11  Applicability.

105.13  General.

105.14  Radio equipment and use requirements.

105.15  Jumps over or into congested areas or open air assembly of persons.

105.17  Jumps over or onto airports.

105.19  Jumps in or into control zones with functioning control towers

operated by the United States.

105.20  Jumps in or into airport radar service areas.

105.21  Jumps into or within positive control areas and terminal control

areas.

105.23  Jumps in or into other airspace.

105.25  Information required, and notice of cancellation or postponement of

jump.

105.27  Jumps over or within restricted or prohibited areas.

105.29  Flight visibility and clearance from clouds requirements.

105.33  Parachute jumps between sunset and sunrise.

105.35  Liquor and drugs.

105.37  Inspections.

Subpart C--Parachute Equipment

105.41  Applicability.

105.43  Parachute equipment and packing requirements.

Authority: 49 U.S.C. app. 1348, 1354, and 1421; 49 U.S.C. 106(g).

Source: Docket No. 1491, 27 FR 11636 Nov. 27, 1962, unless otherwise noted.

EFFECTIVE DATE NOTE: Amdt. 105-10, 56 FR 65663, Dec. 17, 1991, revised Sec.

105.19 and removed and reserved Sec.s 105.20 and 105.21 effective

September 16, 1993. For the convenience of the user, the revised and reserved

items are set forth as follows:

PART 105--PARACHUTE JUMPING

*     *     *     *     *

Subpart B--Operating Rules

*     *     *     *     *

105.19  Jumps in or into Class A, Class B, Class C, and Class D airspace.

105.20  [Reserved]

105.21  [Reserved]

Special Federal Aviation Regulation No. 45-1

Editorial Note: For the text of SFAR No. 45-1, see Part 71 of this chapter.

Subpart A--General

Sec. 105.1   Applicability.

(a) This part prescribes rules governing parachute jumps made in the United

States except parachute jumps necessary because of an inflight emergency.

(b) For the purposes of this part, a "parachute jump" means the descent of

a person, to the surface from an aircraft in flight, when he intends to use,

or uses, a parachute during all or part of that descent.

Subpart B--Operating Rules

Sec. 105.11   Applicability.

(a) Except as provided in paragraphs (b) and (c) of this section, this

subpart prescribes operating rules governing parachute jumps to which this

part applies.

(b) This subpart does not apply to a parachute jump necessary to meet an

emergency on the surface, when it is made at the direction, or with the

approval, of an agency of the United States, or of a State, Puerto Rico, the

District of Columbia, or a possession of the United States, or of a political

subdivision of any of them.

(c) Sections 105.13 through 105.17 and Secs. 105.27 through 105.37 of this

subpart do not apply to a parachute jump made by a member of an Armed Force:

(1) Over or within a restricted area when that area is under the control of

an Armed Force; or

(2) In military operations in uncontrolled airspace.

(d) Section 105.23 does not apply to a parachute jump made by a member of

an Armed Force within a restricted area that extends upward from the surface

when that area is under the control of an Armed Force.

[Doc. No. 1491, 27 FR 11636, Nov. 27, 1962, as amended by Amdt. 105-4, 33 FR

11901, Aug. 22, 1968]

Sec. 105.13   General.

No person may make a parachute jump, and no pilot in command of an aircraft

may allow a parachute jump to be made from that aircraft, if that jump

creates a hazard to air traffic or to persons or property on the surface.

Sec. 105.14   Radio equipment and use requirements.

(a) Except when otherwise authorized by ATC--

(1) No person may make a parachute jump, and no pilot in command of an

aircraft may allow a parachute jump to be made from that aircraft, in or into

controlled airspace unless, during that flight--

(i) The aircraft is equipped with a functioning two-way radio

communications system appropriate to the ATC facilities to be used;

(ii) Radio communications have been established between the aircraft and

the nearest FAA air traffic control facility or FAA flight service station at

least 5 minutes before the jumping activity is to begin, for the purpose of

receiving information in the aircraft about known air traffic in the vicinity

of the jumping activity; and

(iii) The information described in paragraph (a)(1)(ii) of this section has

been received by the pilot in command and the jumpers in that flight; and

(2) The pilot in command of an aircraft used for any jumping activity in or

into controlled airspace shall, during each flight--

(i) Maintain or have maintained a continuous watch on the appropriate

frequency of the aircraft's radio communications system from the time radio

communications are first established between the aircraft and ATC, until he

advises ATC that the jumping activity is ended from that flight; and

(ii) Advise ATC that the jumping activity is ended for that flight when the

last parachute jumper from the aircraft reaches the ground.

(b) If, during any flight, the required radio communications system is or

becomes inoperative, any jumping activity from the aircraft in or into

controlled airspace shall be abandoned. However, if the communications system

becomes inoperative in flight after receipt of a required ATC authorization,

the jumping activity from that flight may be continued.

[Amdt. 105-2, 31 FR 16612, Dec. 29, 1966]

Sec. 105.15   Jumps over or into congested areas or open air assembly of

persons.

(a) No person may make a parachute jump, and no pilot in command of an

aircraft may allow a parachute jump to be made from that aircraft, over or

into a congested area of a city, town, or settlement, or an open air assembly

of person unless a certificate of authorization for that jump has been issued

under this section. However, a parachutist may drift over that congested area

or open air assembly with a fully deployed and properly functioning parachute

if he is at a sufficient altitude to avoid creating a hazard to persons and

property on the ground.

(b) An application for a certificate of authorization issued under this

section is made in a form and in a manner prescribed by the Administrator and

must be submitted to the FAA Flight Standards District Office having

jurisdiction over the area in which the parachute jump is to be made, at

least 4 days before the day of that jump.

(c) Each holder of a certificate of authorization issued under this section

shall present that certificate for inspection upon the request of the

Administrator, or any Federal, State, or local official.

[Doc. No. 1491, 27 FR 11636, Nov. 27, 1962, as amended by Doc. No. 4057,

Amdt. 105-1, 29 FR 14920, Nov. 4, 1964; Amdt. 105-7, 43 FR 22641, May 25,

1978]

Sec. 105.17   Jumps over or onto airports.

Unless prior approval has been given by the airport management, no person

may make a parachute jump, and no pilot in command of an aircraft may allow a

parachute jump to be made from that aircraft--

(a) Over an airport that does not have a functioning control tower operated

by the United States; or

(b) Onto any airport.

However, a parachutist may drift over that airport with a fully deployed and

properly functioning parachute if he is at least 2,000 feet above that

airport's traffic pattern, and avoids creating a hazard to air traffic or to

persons and property on the ground.

[Doc. No. 4057, Amdt. 105-1, 29 FR 14920, Nov. 4, 1964]

Sec. 105.19   Jumps in or into Class A, Class B, Class C, and Class D

airspace.

(a) No person may make a parachute jump, and no pilot in command may allow

a parachute jump to be made from that aircraft, in or into Class A, Class B,

Class C, and Class D airspace without, or in violation of, the terms of an

ATC authorization issued under this section.

(b) Each request for an authorization under this section must be submitted

to the nearest FAA air traffic control facility or FAA flight service station

and must include the information prescribed by Sec. 105.25(a).

EFFECTIVE DATE NOTE: Amdt. 105-10, 56 FR 65663, Dec. 17, 1991, revised

Sec. 105.19 effective September 16, 1993. The text of Sec. 105.19 in effect

until September 16, 1993 reads as follows:

Sec. 105.19   Jumps in or into control zones with functioning control towers

operated by the United States.

(a) No person may make a parachute jump, and no pilot in command may allow

a parachute jump to be made from that aircraft, in or into a control zone in

which there is a functioning control tower operated by the United States

without, or in violation of the terms of, an authorization issued under this

section.

(b) Each request for an authorization under this section must be submitted

t@the control tower having jurisdiction over the control zone concerned and

must include the information prescribed in Sec. 105.25.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Sec. 105.20  [Reserved. Amdt. 105-10, 56 FR 65663, Dec. 17, 1991]

EFFECTIVE DATE NOTE: Amdt. 105-10, 56 FR 65663, Dec. 17, 1991, removed and

reserved Sec. 105.20 effective September 16, 1993. The text of Sec. 105.20 in

effect until September 16, 1993 reads as follows:

Sec. 105.20  Jumps in or into airport radar service areas.

(a) No person may make a parachute jump and no pilot in command may allow a

parachute jump to be made from that aircraft in or into an airport radar

service area without, or in violation of, the terms of an ATC authorization

issued under this section.

(b) Each request for an authorization under this section must be submitted

to the control tower at the airport for which the airport radar service area

is designated.

[Doc. No. 23708, 50 FR 9259, Mar. 6, 1985]

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Sec. 105.21  [Reserved. Amdt. 105-10, 56 FR 65663, Dec. 17, 1991]

EFFECTIVE DATE NOTE: Amdt. 105-10, 56 FR 65663, Dec. 17, 1991, removed and

reserved Sec. 105.21 effective September 16, 1993. The text of Sec. 105.21 in

effect until September 16, 1993 reads as follows:

Sec. 105.21  Jumps into or within positive control areas and terminal

control areas.

(a) No person may make a parachute jump, and no pilot in command of an

aircraft may allow a parachute jump to be made from that aircraft, in or into

a positive control area or terminal control area without, or in violation of,

an authorization issued under this section.

(b) Each request for an authorization issued under this section must be

submitted to the nearest FAA air traffic control facility or FAA flight

service station and must include the information prescribed by Sec.

105.25(a).

[Amdt. 105-2, 31 FR 16612, Dec. 29, 1966, as amended by Amdt. 105-9, 51 FR

21907, June 17, 1986]

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Sec. 105.23   Jumps in or into other airspace.

(a) No person may make a parachute jump, and no pilot in command of an

aircraft may allow a parachute jump to be made from that aircraft, in or into

airspace unless the nearest FAA air traffic control facility or FAA flight

service station was notified of that jump at least 1 hour before the jump is

to be made, but not more than 24 hours before the jumping is to be completed,

and the notice contained the information prescribed in Sec. 105.25(a).

(b) Notwithstanding paragraph (a) of this section, ATC may accept from a

parachute jumping organization a written notification of a scheduled series

of jumps to be made over a stated period of time not longer than 12 calendar

months. The notification must contain the information prescribed by Sec.

105.25(a), identify the responsible persons associated with that jumping

activity, and be submitted at least 15 days, but not more than 30 days,

before the jumping is to begin. ATC may revoke the acceptance of the

notification for any failure of the jumping organization to comply with its

terms.

(c) This section does not apply to parachute jumps in or into any airspace

or place described in Sec. 105.15, Sec. 105.19, or Sec. 105.21.

[Amdt. 105-2, 31 FR 16612, Dec. 29, 1966]

Sec. 105.25   Information required, and notice of cancellation or

postponement of jump.

(a) Each person requesting an authorization under Sec. 105.19 or Sec.

105.21, and each person submitting a notice under Sec. 105.23, must include

the following information (on an individual or group basis) in that request

or notice:

(1) The date and time jumping will begin.

(2) The size of the jump zone expressed in nautical mile radius around the

target.

(3) The location of the center of the jump zone in relation to--

(i) The nearest VOR facility in terms of the VOR radial on which it is

located, and its distance in nautical miles from the VOR facility when that

facility is 30 nautical miles or less from the drop zone target; or

(ii) The nearest airport, town, or city depicted on the appropriate Coast

and Geodetic Survey WAC or Sectional Aeronautical chart, when the nearest VOR

facility is more than 30 nautical miles from the drop zone target.

(4) The altitudes above mean sea level at which jumping will take place.

(5) The duration of the intended jump.

(6) The name, address, and telephone number of the person requesting the

authorization or giving notice.

(7) The identification of the aircraft to be used.

(8) The radio frequencies, if any, available in the aircraft.

(b) Each person requesting an authorization under Sec. 105.19 or Sec.

105.21, and each person submitting a notice under Sec. 105.23, must promptly

notify the FAA air traffic control facility or FAA flight service station

from which it requested authorization or which it notified, if the proposed

or scheduled jumping activity is canceled or postponed.

[Amdt. 105-2, 31 FR 16612, Dec. 29, 1966, as amended by Amdt. 105-6, 41 FR

47229, Oct. 28, 1976; Amdt. 105-9, 51 FR 21907, June 17, 1986]

Sec. 105.27   Jumps over or within restricted or prohibited areas.

No person may make a parachute jump, and no pilot in command may allow a

parachute jump to be made from that aircraft, over or within a restricted

area or prohibited area unless the controlling agency of the area concerned

has authorized that jump.

Sec. 105.29   Flight visibility and clearance from clouds requirements.

No person may make a parachute jump, and no pilot in command of an aircraft

may allow a parachute jump to be made from that aircraft--

(a) Into or through a cloud; or

(b) When the flight visibility is less, or at a distance from clouds that

is less, than that prescribed in the following table:

Flight

visibility

(statute

Altitude                miles)         Distance from clouds

(1) 1,200 feet or less above              3  500 feet below. 1,000 feet

the surface regardless of the                above. 2,000 feet horizontal.

MSL altitude

(2) More than 1,200 feet above            3  500 feet below. 1,000 feet

the surface but less than                    above. 2,000 feet horizontal.

10,000 feet MSL

(3) More than 1,200 feet above            5  1,000 feet below. 1,000 feet

the surface and at or above                  above. 1 mile horizontal.

10,000 feet MSL

[Amdt. 105-5, 36 FR 8775, May 13, 1971]

Sec. 105.33  Parachute jumps between sunset and sunrise.

(a) No person may make a parachute jump, and no pilot in command of an

aircraft may allow any person to make a parachute jump from that aircraft,

between sunset and sunrise, unless that person is equipped with a means of

producing a light visible for at least 3 statute miles.

(b) Each person making a parachute jump between sunset and sunrise shall

display the light required by paragraph (a) of this section from the time

that person exits the aircraft until that person reaches the surface.

[Amdt. 105-7, 43 FR 22641, May 25, 1978]

Sec. 105.35   Liquor and drugs.

No person may make a parachute jump while, and no pilot in command of an

aircraft may allow a person to make a parachute jump from that aircraft if

that person appears to be:

(a) Under the influence of intoxicating liquor; or

(b) Using any drug that affects his faculties in any way contrary to

safety.

Sec. 105.37   Inspections.

The Administrator may inspect (including inspections at the jump site), any

parachute jump operation to which this part applies, to determine compliance

with the regulations of this part.

Subpart C--Parachute Equipment

Sec. 105.41   Applicability.

(a) Except as provided in paragraph (b) of this section, this subpart

prescribes rules governing parachute equipment used in parachute jumps to

which this part applies.

(b) This subpart does not apply to a parachute jump made by a member of an

Armed Force using parachute equipment of an Armed Force.

Sec. 105.43   Parachute equipment and packing requirements.

(a) No person may make a parachute jump, and no pilot in command of an

aircraft may allow any person to make a parachute jump from that aircraft,

unless that person is wearing a single harness dual parachute pack, having at

least one main parachute and one approved auxiliary parachute that are packed

as follows:

(1) The main parachute must have been packed by a certificated parachute

rigger, or by the person making the jump, within 120 days before the date of

its use.

(2) The auxiliary must have been packed by a certificated and appropriately

rated parachute rigger:

(i) Within 120 days before the date of use, if its canopy, shroud, and

harness are composed exclusively of nylon, rayon, or other similar synthetic

fiber or material that is substantially resistant to damage from mold,

mildew, or other fungi and other rotting agents propagated in a moist

environment; or

(ii) Within 60 days before the date of use, if it is composed in any amount

of silk, pongee, or other natural fiber, or material not specified in

paragraph (a)(2)(i) of this section.

(b) No person may make a parachute jump using a static line attached to the

aircraft and the main parachute unless an assist device, described and

attached as follows, is used to aid the pilot chute in performing its

function, or, if no pilot chute is used, to aid in the direct deployment of

the main parachute canopy.

(1) The assist device must be long enough to allow the container to open

before a load is placed on the device.

(2) The assist device must have a static load strength of--

(i) At least 28 pounds but not more than 160 pounds, if it is used to aid

the pilot chute in performing its function; or

(ii) At least 56 pounds but not more than 320 pounds, if it is used to aid

in the direct deployment of the main parachute canopy.

(3) The assist device must be attached--

(i) At one end, to the static line above the static line pins, or, if

static pins are not used, above the static line ties to the parachute cone;

and

(ii) At the other end, to the pilot chute apex, bridle cord or bridle loop,

or, if no pilot chute is used, to the main parachute canopy.

(c) No person may attach an assist device required by paragraph (b) of this

section to any main parachute unless he has a current parachute rigger

certificate issued under Part 65 of this chapter or is the person who makes

the jump with that parachute.

(d) For the purpose of this section, an "approved" parachute is:

(1) A parachute manufactured under a type certificate or a technical

standard order (C-23 series); or

(2) A personnel-carrying military parachute (other than a high altitude,

high-speed, or ejection kind) identified by an NAF, AAF, or AN drawing

number, an AAF order number, or any other military designation or

specification number.

[Doc. No. 1491, 27 FR 11636, Nov. 27, 1962 as amended by Amdt. 105-3, 33 FR

8481 June 8, 1968; Amdt. 105-7, 43 FR 22641, May 25, 1978]