PART 139--CERTIFICATION AND OPERATIONS: LAND AIRPORTS SERVING CERTAIN AIR

CARRIERS

Subpart A--General

Sec.

139.1Applicability.

139.3Definitions.

139.5Standards and procedures for compliance with the certification and

operations requirements of this part.

Subpart B--Certification

139.101Certification requirements: General.

139.103Application for certificate.

139.105Inspection authority.

139.107Issuance of certificate.

139.109Duration of certificate.

139.111Exemptions.

139.113Deviations.

Subpart C--Airport Certification Manual and Airport Certification

Specifications

139.201Airport operating certificate: Airport certification manual.

139.203Preparation of airport certification manual.

139.205Contents of airport certification manual.

139.207Maintenance of airport certification manual.

139.209Limited airport operating certificate: Airport certification

specifications.

139.211Preparation of airport certification specifications.

139.213Contents of airport certification specifications.

139.215Maintenance of airport certification specifications.

139.217Amendment of airport certification manual or airport certification

specifications.

Subpart D--Operations

139.301Inspection authority.

139.303Personnel.

139.305Paved areas.

139.307Unpaved areas.

139.309Safety areas.

139.311Marking and lighting.

139.313Snow and ice control.

139.315Aircraft rescue and firefighting: Index determination.

139.317Aircraft rescue and firefighting: Equipment and agents.

139.319Aircraft rescue and firefighting: Operational requirements.

139.321Handling and storing of hazardous substances and materials.

139.323Traffic and wind direction indicators.

139.325Airport emergency plan.

139.327Self-inspection program.

139.329Ground vehicles.

139.331Obstructions.

139.333Protection of navaids.

139.335Public protection.

139.337Wildlife hazard management.

139.339Airport condition reporting.

139.341Identifying, marking, and reporting construction and other

unserviceable areas.

139.343Noncomplying conditions.

Authority: 49 U.S.C. app. 1354(a) and 1432; 49 U.S.C. 106(g).

Source: Docket No. 24812, 52 FR 44282, Nov. 18, 1987, unless otherwise

noted.

*****************************************************************************

56 FR 66968, No. 249, Dec. 27, 1991

SUMMARY: This notice informs all certificated airport owners and operators

subject to FAR part 139 that they must have an approved airport signage plan

by November 1, 1992.

SUPPLEMENTARY INFORMATION: Federal Aviation Regulations, 14 CFR part 139,

Sec. 139.205(b)(12) and Sec. 139.213(b)(10) contain the requirement to

include in the Airport Certification Manual or the Airport Certification

Specifications, as applicable, the following: "A description of, and

procedures for maintaining, the marking and lighting systems as required by

Sec. 139.311." Additionally, Secs. 139.205(b)(26) and Sec. 139.213(b)(16)

state that the airport certification manual and airport certification

specifications must contain any other item which the Administrator of the

Federal Aviation Administration (FAA) finds is necessary in the public

interest.

The purpose of this notice is to inform all certificated airport owners and

operators that the FAA finds that the inclusion of the airport signage plan

in the airport certification manual or the airport certification

specifications is in the public interest and requires that airports subject

to Federal Aviation Regulation part 139 prepare a signage plan for FAA

approval. This plan will detail the type and the location of each sign

required by Sec. 139.311.(a) (3), (4), and (5). Any airport subject to FAR

part 139 must have an approved airport signage plan no later than November 1,

1992. Additionally, any airport subject to FAR part 139 must have an approved

sign plan in order to be eligible for Airport Improvement Program funds for

such signing.

Advisory Circular No. 150/5340-18C was adopted July 31, 1991 and contains

acceptable standards to satisfy these signage plan requirements.

Authority: 49 U.S.C. App. 1354(a) and 1432; 49 U.S.C. section 106(g)

(Revised, Pub. L. 97-449, January 12, 1983).

*****************************************************************************

Subpart A--General

Sec. 139.1Applicability.

This part prescribes rules governing the certification and operation of

land airports which serve any scheduled or unscheduled passenger operation of

an air carrier that is conducted with an aircraft having a seating capacity

of more than 30 passengers. This part does not apply to airports at which air

carrier passenger operations are conducted only by reason of the airport

being designated as an alternate airport.

Sec. 139.3Definitions.

The following are definitions of terms as used in this part:

AFFF means aqueous film forming foam agent.

Air carrier means a person who holds or who is required to hold an air

carrier operating certificate issued under this chapter while operating

aircraft having a seating capacity of more than 30 passengers.

Air carrier aircraft means an aircraft with a seating capacity of more than

30 passengers which is being operated by an air carrier.

Air carrier operation means the takeoff or landing of an air carrier

aircraft and includes the period of time from 15 minutes before and until 15

minutes after the takeoff or landing.

Airport means an area of land or other hard surface, excluding water, that

is used or intended to be used for the landing and takeoff of aircraft, and

includes its buildings and facilities, if any.

Airport operating certificate means a certificate, issued under this part,

for operation of an airport serving scheduled operations of air carriers.

Average daily departures means the average number of scheduled departures

per day of air carrier aircraft computed on the basis of the busiest 3

consecutive months of the immediately preceding 12 calendar months; except

that if the average daily departures are expected to increase, then "average

daily departures" may be determined by planned rather than current activity

in a manner acceptable to the Administrator.

Certificate holder means the holder of an airport operating certificate or

a limited airport operating certificate, except that as used in Subpart D

"certificate holder" does not mean the holder of a limited airport operating

certificate if its airport certification specifications, or this part, do not

require compliance with the section in which it is used.

Heliport means an airport or an area of an airport used or intended to be

used for the landing and takeoff of helicopters.

Index means an airport ranking according to the type and quantity of

aircraft rescue and firefighting equipment and agent required, determined by

the length and frequency of air carrier aircraft served by the airport, as

provided in Subpart D of this part.

Limited airport operating certificate means a certificate, issued under

this part, for the operation of an airport serving unscheduled operations of

air carriers.

Movement area means the runways, taxiways, and other areas of an airport

which are used for taxiing or hover taxiing, air taxiing, takeoff, and

landing of aircraft, exclusive of loading ramps and aircraft parking areas.

Regional Airports Division Manager means the airports division manager for

the FAA region in which the airport is located.

Safety area means a designated area abutting the edges of a runway or

taxiway intended to reduce the risk of damage to an aircraft inadvertently

leaving the runway or taxiway.

Wildlife hazard means a potential for a damaging aircraft collision with

wildlife on or near an airport. As used in this part, "wildlife" includes

domestic animals while out of the control of their owners.

[Doc. No. 24812, 52 FR 44282, Nov. 18, 1987; 53 FR 4258, Feb. 12, 1988, as

amended by Amdt. 139-16, 54 FR 39295, Sept. 25, 1989]

Sec. 139.5Standards and procedures for compliance with the certification

and operations requirements of this part.

Certain requirements prescribed by Subparts C and D of this part must be

complied with in a manner acceptable to the Administrator. FAA Advisory

Circulars contain standards and procedures that are acceptable to the

Administrator for compliance with Subparts C and D. Some of these advisory

circulars are referenced in specific sections of this part. The standards and

procedures in them, or other standards and procedures approved by the

Administrator, may be used to comply with those sections.

Subpart B--Certification

Sec. 139.101Certification requirements: general.

(a) No person may operate a land airport in any State of the United States,

the District of Columbia, or any territory or possession of the United

States, serving any scheduled passenger operation of an air carrier operating

an aircraft having a seating capacity of more than 30 passengers without an

airport operating certificate, or in violation of that certificate, the

applicable provisions of this part, or the approved airport certification

manual for that airport.

(b) Unless otherwise authorized by the Administrator, no person may operate

a land airport in any State of the United States, the District of Columbia,

or any territory or possession of the United States, serving any unscheduled

passenger operation of an air carrier operating an aircraft having a seating

capacity of more than 30 passengers without a limited airport operating

certificate, or in violation of that certificate, the applicable provisions

of this part, or the approved airport specifications for that airport.

[Amdt. No. 139-7, 55 FR 48214, Nov. 19, 1990]

*****************************************************************************

55 FR 48212, No. 223, Nov. 19, 1990

SUMMARY: This final rule makes two changes to the certification and

operations regulations of land airports serving air carriers. The first

change revises the certification requirements to provide that a person

operating an uncertificated airport may serve, when authorized by the

Administrator, unscheduled air carrier operations with aircraft having a

seating capacity of more than 30 passengers. As revised, airport

certification requirements and the regulations applicable to air carrier

operations are consistent in this regard. The second change clarifies

responsibility for the establishment of and compliance with rules for airport

ground vehicle operations by tenants, contractors, and employees. This change

is necessary to address the responsibility of certificate holders with regard

to ground vehicle operations.

EFFECTIVE DATE: December 19, 1990.

*****************************************************************************

Sec. 139.103Application for certificate.

(a) Each applicant for an airport operating certificate or a limited

airport operating certificate must submit an application, in a form and in

the manner prescribed by the Administrator, to the Regional Airports Division

Manager.

(b) The application must be accompanied by two copies of an airport

certification manual or airport certification specifications, as appropriate,

prepared in accordance with Subpart C of this part.

[Doc. No. 24812, 52 FR 44282, Nov. 18, 1987, as amended by Amdt. 139-16, 54

FR 39295, Sept. 25, 1989]

Sec. 139.105Inspection authority.

Each applicant for an airport operating certificate or a limited airport

operating certificate must allow the Administrator to make any inspections,

including unannounced inspections, or tests to determine compliance with--

(a) The Federal Aviation Act of 1958, as amended; and

(b) The requirements of this part.

Sec. 139.107Issuance of certificate.

(a) An applicant for an airport operating certificate is entitled to a

certificate if--

(1) The provisions of Sec. 139.103 of this subpart are met;

(2) The Administrator, after investigation, finds that the applicant is

properly and adequately equipped and able to provide a safe airport operating

environment in accordance with--

(i) Subpart D of this part, and

(ii) Any limitations which the Administrator finds necessary in the public

interest; and

(3) The Administrator approves the airport certification manual.

(b) An applicant for a limited airport operating certificate is entitled to

a certificate if--

(1) The provisions of Sec. 139.103 of this subpart are met;

(2) The Administrator, after investigation, finds that the applicant is

properly and adequately equipped and able to provide a safe airport operating

environment in accordance with--

(i) The provisions of Subpart D listed in Sec. 139.213(a) of this part, and

(ii) Any other provisions of this part and any limitations which the

Administrator finds necessary in the public interest; and

(3) The Administrator approves the airport certification specifications.

Sec. 139.109 Duration of certificate.

An airport operating certificate or a limited airport operating certificate

issued under this part is effective until it is surrendered by the

certificate holder or is suspended or revoked by the Administrator.

Sec. 139.111 Exemptions.

(a) An applicant or a certificate holder may petition the Administrator

under Sec. 11.25, Petitions for Rule Making or Exemptions, of this chapter

for an exemption from any requirement of this part.

(b) An applicant or a certificate holder, enplaning annually less than one-

quarter of 1 percent of the total number of passengers enplaned at all air

carrier airports, may petition the Administrator under Sec. 11.25, Petitions

for Rule Making or Exemptions, of this chapter for an exemption from all or

part of the rescue and firefighting equipment requirements of this part on

the grounds that compliance with those requirements is, or would be,

unreasonably costly, burdensome, or impractical.

(c) Each petition filed under this section must be submitted in duplicate

to the Regional Airports Division Manager.

[Doc. No. 24812, 52 FR 44282, Nov. 18, 1987, as amended by Amdt. 139-16, 54

FR 39295, Sept. 25, 1989]

Sec. 139.113 Deviations.

In emergency conditions requiring immediate action for the protection of

life or property, involving the transportation of persons by air carriers,

the certificate holder may deviate from any requirement of Subpart D of this

part to the extent required to meet that emergency. Each certificate holder

who deviates from a requirement under this paragraph shall, as soon as

practicable, but not later than 14 days after the emergency, report in

writing to the Regional Airports Division Manager stating the nature, extent,

and duration of the deviation.

[Doc. No. 24812, 52 FR 44282, Nov. 18, 1987, as amended by Amdt. 139-16, 54

FR 39295, Sept. 25, 1989]

Subpart C--Airport Certification Manual and Airport Certification

Specifications

Sec. 139.201 Airport operating certificate: Airport certification manual.

(a) An applicant for an airport operating certificate must prepare, and

submit with an application, an airport certification manual for approval by

the Administrator. Only those items addressing subjects required for

certification under this part shall be included in the airport certification

manual.

(b) Except as provided in paragraph (c) of this section, each certificate

holder shall comply with an approved airport certification manual that meets

the requirements of Secs. 139.203 and 139.205.

(c) A certificate holder with an approved airport operations manual on

December 31, 1987, may use the manual in lieu of the manual required by

paragraph (b) of this section until December 31, 1988. Until the certificate

holder has an approved airport certification manual, it shall comply with

Sec. 139.207 as if that section applied to its airport operations manual.

[Docket No. 24812, 52 FR 44282, Nov. 18, 1987; 53 FR 4119, Feb. 12, 1988]

Sec. 139.203 Preparation of airport certification manual.

(a) Each airport certification manual required by this part shall--

(1) Be typewritten and signed by the airport operator;

(2) Be in a form that is easy to revise;

(3) Have the date of initial approval or approval of the latest revision on

each page or item in the manual and include a page revision log; and

(4) Be organized in a manner helpful to the preparation, review, and

approval processes.

(b) FAA Advisory Circulars in the 139 series contain standards and

procedures for the development of airport certification manuals which are

acceptable to the Administrator.

Sec. 139.205 Contents of airport certification manual.

(a) Each airport certification manual required by this part shall include

operating procedures, facilities and equipment descriptions, responsibility

assignments, and any other information needed by personnel concerned with

operating the airport in order to comply with--

(1) The provisions of Subpart D of this part; and

(2) Any limitations which the Administrator finds necessary in the public

interest.

(b) In complying with paragraph (a) of this section, the airport

certification manual must include at least the following elements:

(1) Lines of succession of airport operational responsibility.

(2) Each current exemption issued to the airport from the requirements of

this part.

(3) Any limitations imposed by the Administrator.

(4) A grid map or other means of identifying locations and terrain features

on and around the airport which are significant to emergency operations.

(5) The system of runway and taxiway identification.

(6) The location of each obstruction required to be lighted or marked

within the airport's area of authority.

(7) A description of each movement area available for air carriers and its

safety areas and each road described in Sec. 139.319(k) that serves it.

(8) Procedures for avoidance of interruption or failure during construction

work of utilities serving facilities or navaids which support air carrier

operations.

(9) Procedures for maintaining the paved areas as required by Sec. 139.305.

(10) Procedures for maintaining the unpaved areas as required by Sec.

139.307.

(11) Procedures for maintaining the safety areas as required by Sec.

139.309.

(12) A description of, and procedures for maintaining, the marking and

lighting systems as required by Sec. 139.311.

(13) A snow and ice control plan as required by Sec. 139.313.

(14) A description of the facilities, equipment, personnel, and procedures

for meeting the rescue and firefighting requirements in Secs. 139.317 and

139.319.

(15) Procedures for complying with the requirements of Sec. 139.321

relating to hazardous substances and materials.

(16) A description of, and procedures for maintaining, the traffic and wind

direction indicators required by Sec. 139.323.

(17) An emergency plan as required by Sec. 139.325.

(18) Procedures for conducting the self-inspection program as required by

Sec. 139.327.

(19) Procedures for controlling ground vehicles as required by Sec.

139.329.

(20) Procedures for obstruction removal, marking, or lighting as required

by Sec. 139.331.

(21) Procedures for protection of navaids as required by Sec. 139.333.

(22) A description of public protection as required by Sec. 139.335.

(23) A wildlife hazard management plan as required by Sec. 139.337.

(24) Procedures for airport condition reporting as required by Sec.

139.339.

(25) Procedures for identifying, marking, and reporting construction and

other unserviceable areas as required by Sec. 139.341.

(26) Any other item which the Administrator finds is necessary in the

public interest.

[Docket No. 24812, 52 FR 44282, Nov. 18, 1987; 53 FR 4119 and 4258, Feb. 12,

1988]

Sec. 139.207 Maintenance of airport certification manual.

Each holder of an airport operating certificate shall--

(a) Keep its airport certification manual current at all times;

(b) Maintain at least one complete and current copy of its approved airport

certification manual on the airport;

(c) Furnish the applicable portions of the approved airport certification

manual to the airport personnel responsible for their implementation;

(d) Make the copy required by paragraph (b) of this section available for

inspection by the Administrator upon request; and

(e) Provide the Administrator with one complete and current copy required

by paragraph (b) of this section.

Sec. 139.209 Limited airport operating certificate: Airport certification

specifications.

(a) An applicant for a limited airport operating certificate must prepare,

and submit with an application, airport certification specifications for

approval by the Administrator. Only those items addressing subjects required

for certification under this part shall be included in the airport

certification specifications.

(b) Except as provided in paragraph (c) of this section, each certificate

holder shall comply with the approved airport certification specifications

that meet the requirements of Secs. 139.211 and 139.213.

(c) A certificate holder with an approved airport operations specification

on December 31, 1987, may use those specifications in lieu of the

specifications required by paragraph (b) of this section until December 31,

1988. Until the certificate holder has approved airport certification

specifications, it shall comply with Sec. 139.215 as if that section applied

to its airport operations specifications.

[Docket No. 24812, 52 FR 44282, Nov. 18, 1987; 53 FR 4119, Feb. 12, 1988]

Sec. 139.211 Preparation of airport certification specifications.

(a) Each airport certification specifications required by this part shall--

(1) Be typewritten and signed by the airport operator;

(2) Be in a form that is easy to revise;

(3) Have the date of initial approval or approval of the latest revision on

each page or item in the specifications and include a page revision log; and

(4) Be organized in a manner helpful to the preparation, review, and

approval processes.

(b) FAA Advisory Circulars in the 139 series contain standards and

procedures for the development of airport certification specifications which

are acceptable to the Administrator.

Sec. 139.213 Contents of airport certification specifications.

(a) The airport certification specifications required by this part shall

include operating procedures, facilities and equipment descriptions,

responsibility assignments, and any other information needed by personnel

concerned with operating the airport in order to comply with--

(1) The following provisions of Subpart D of this part:

(i) Section 139.301 Inspection authority.

(ii) Section 139.303 Personnel.

(iii) Section 139.305 Paved areas.

(iv) Section 139.307 Unpaved areas.

(v) Section 139.309 Safety areas.

(vi) Section 139.311 Marking and lighting.

(vii) Section 139.339 Airport condition reporting.

(2) Any other provisions of Subpart D of this part, and any limitations,

which the Administrator finds necessary in the public interest.

(b) In complying with paragraph (a) of this section, the airport

certification specifications shall include at least the following elements:

(1) Lines of succession of airport operational responsibility.

(2) Each current exemption issued to the airport from the requirements of

this part.

(3) Any limitations imposed by the Administrator.

(4) The system of runway and taxiway identification.

(5) The location of each obstruction required to be lighted or marked

within the airport's area of authority.

(6) A description of each movement area available for air carriers and its

safety areas.

(7) Procedures for maintaining the paved areas as required by Sec. 139.305.

(8) Procedures for maintaining the unpaved areas as required by Sec.

139.307.

(9) Procedures for maintaining the safety areas as required by Sec.

139.309.

(10) A description of, and procedures for maintaining, the marking and

lighting systems as required by Sec. 139.311.

(11) A description of the facilities, equipment, personnel, and procedures

for emergency response to aircraft rescue and firefighting needs.

(12) Procedures for safety in storing and handling of hazardous substances

and materials.

(13) A description of, and procedures for maintaining, any traffic and wind

direction indicators on the airport.

(14) A description of the procedures used for conducting self-inspections

of the airport.

(15) Procedures and responsibilities for airport condition reporting as

required by Sec. 139.339.

(16) Procedures for compliance with any other provisions of Subpart D of

this part, and any limitations, which the Administrator finds necessary in

the public interest.

Sec. 139.215 Maintenance of airport certification specifications.

Each holder of a limited airport operating certificate shall--

(a) Keep its airport certification specifications current at all times;

(b) Maintain at least one complete and current copy of its approved airport

certification specifications on the airport;

(c) Furnish the applicable portions of the approved airport certification

specifications to the airport personnel responsible for their implementation;

(d) Make the copy required by paragraph (b) of this section available for

inspection by the Administrator upon request; and

(e) Provide the Administrator with one complete and current copy required

by paragraph (b) of this section.

Sec. 139.217 Amendment of airport certification manual or airport

certification specifications.

(a) The Regional Airports Division Manager may amend any airport

certification manual or any airport certification specifications approved

under this part, either--

(1) Upon application by the certification holder; or

(2) On the Regional Airports Division Manager's own initiative if the

Regional Airports Division Manager determines that safety in air

transportation or air commerce and the public interest require the amendment.

(b) An applicant for an amendment to its airport certification manual or

its airport certification specifications shall file its application with the

Regional Airports Division Manager at least 30 days before the proposed

effective date of the amendment, unless a shorter filing period is allowed by

that office.

(c) At any time within 30 days after receiving a notice of refusal to

approve the application for amendment, the certificate holder may petition

the Administrator to reconsider the refusal to amend.

(d) In the case of amendments initiated by the Regional Airports Division

Manager, the office notifies the certificate holder of the proposed

amendment, in writing, fixing a reasonable period (but not less than 7 days)

within which the certificate holder may submit written information, views,

and arguments on the amendment. After considering all relevant material

presented, the Regional Airports Division Manager notifies the certificate

holder of any amendment adopted or rescinds the notice. The amendment becomes

effective not less than 30 days after the certificate holder receives notice

of it, except that prior to the effective date the certificate holder may

petition the Administrator to reconsider the amendment, in which case its

effective date is stayed pending a decision by the Administrator.

(e) Notwithstanding the provisions of paragraph (d) of this section, if the

Regional Airports Division Manager finds that there is an emergency requiring

immediate action with respect to safety in air transportation or air commerce

that makes the procedures in this paragraph impractical or contrary to the

public interest, the Regional Airports Division Manager may issue an

amendment, effective without stay on the date the certificate holder receives

notice of it. In such a case, the Regional Airports Division Manager

incorporates the finding of the emergency, and a brief statement of the

reasons for the finding, in the notice of the amendment. Within 30 days after

the issuance of such an emergency amendment, the certificate holder may

petition the Administrator to reconsider either the finding of an emergency

or the amendment itself or both. This petition does not automatically stay

the effectiveness of the emergency amendment.

[Doc. No. 24812, 52 FR 44282, Nov. 18, 1987, as amended by Amdt. 139-16, 54

FR 39295, Sept. 25, 1989]

Subpart D--Operations

Sec. 139.301 Inspection authority.

Each certificate holder shall allow the Administrator to make any

inspections, including unannounced inspections, or tests to determine

compliance with this part.

Sec. 139.303 Personnel.

Each certificate holder shall maintain sufficient qualified personnel to

comply with the requirements of its airport certification manual or airport

certification specifications and the applicable rules of this part.

Sec. 139.305 Paved areas.

(a) Each certificate holder shall maintain, and promptly repair the

pavement of, each runway, taxiway, loading ramp, and parking area on the

airport which is available for air carrier use as follows:

(1) The pavement edges shall not exceed 3 inches difference in elevation

between abutting pavement sections and between full strength pavement and

abutting shoulders.

(2) The pavement shall have no hole exceeding 3 inches in depth nor any

hole the slope of which from any point in the hole to the nearest point at

the lip of the hole is 45 degrees or greater as measured from the pavement

surface plane, unless, in either case, the entire area of the hole can be

covered by a 5-inch diameter circle.

(3) The pavement shall be free of cracks and surface variations which could

impair directional control of air carrier aircraft.

(4) Except as provided in paragraph (b) of this section, mud, dirt, sand,

loose aggregate, debris, foreign objects, rubber deposits, and other

contaminants shall be removed promptly and as completely as practicable.

(5) Except as provided in paragraph (b) of this section, any chemical

solvent that is used to clean any pavement area shall be removed as soon as

possible, consistent with the instructions of the manufacturer of the

solvent.

(6) The pavement shall be sufficiently drained and free of depressions to

prevent ponding that obscures markings or impairs safe aircraft operations.

(b) Paragraphs (a)(4) and (a)(5) of this section do not apply to snow and

ice accumulations and their control, including the associated use of

materials such as sand and deicing solutions.

(c) FAA Advisory Circulars in the 150 series contain standards and

procedures for the maintenance and configuration of paved areas which are

acceptable to the Administrator.

Sec. 139.307 Unpaved areas.

(a) Each certificate holder shall maintain and promptly repair the surface

of each gravel, turf, or other unpaved runway, taxiway, or loading ramp and

parking area on the airport which is available for air carrier use as

follows:

(1) No slope from the edge of the full-strength surfaces downward to the

existing terrain shall be steeper than 2:1.

(2) The full-strength surfaces shall have adequate crown or grade to assure

sufficient drainage to prevent ponding.

(3) The full-strength surfaces shall be adequately compacted and

sufficiently stable to prevent rutting by aircraft, or the loosening or

buildup of surface material which could impair directional control of

aircraft or drainage.

(4) The full-strength surfaces must have no holes or depressions which

exceed 3 inches in depth and are of a breadth capable of impairing

directional control or causing damage to an aircraft.

(5) Debris and foreign objects shall be promptly removed from the surface.

(b) Standards and procedures for the maintenance and configuration of

unpaved full-strength surfaces shall be included in the airport certification

manual or the airport certification specifications, as appropriate, for

compliance with this section.

Sec. 139.309 Safety areas.

(a) To the extent practicable, each certificate holder shall provide and

maintain for each runway and taxiway which is available for air carrier use--

(1) If the runway or taxiway had a safety area on December 31, 1987, and if

no reconstruction or significant expansion of the runway or taxiway was begun

on or after January 1, 1988, a safety area of at least the dimensions that

existed on December 31, 1987; or

(2) If construction, reconstruction, or significant expansion of the runway

or taxiway began on or after January 1, 1988, a safety area which conforms to

the dimensions acceptable to the Administrator at the time construction,

reconstruction, or expansion began.

(b) Each certificate holder shall maintain its safety areas as follows:

(1) Each safety area shall be cleared and graded, and have no potentially

hazardous ruts, humps, depressions, or other surface variations.

(2) Each safety area shall be drained by grading or storm sewers to prevent

water accumulation.

(3) Each safety area shall be capable under dry conditions of supporting

snow removal equipment, and aircraft rescue and firefighting equipment, and

supporting the occasional passage of aircraft without causing major damage to

the aircraft.

(4) No object may be located in any safety area, except for objects that

need to be located in a safety area because of their function. These objects

shall be constructed, to the extent practical, on frangibly mounted

structures of the lowest practical height with the frangible point no higher

than 3 inches above grade.

(c) FAA Advisory Circulars in the 150 series contain standards and

procedures for the configuration and maintenance of safety areas acceptable

to the Administrator.

Sec. 139.311 Marking and lighting.

(a) Each certificate holder shall provide and maintain at least the

following marking systems for air carrier operations on the airport:

(1) Runway markings meeting the specifications for the approach with the

lowest minimums authorized for each runway.

(2) Taxiway centerline and edge markings.

(3) Signs identifying taxiing routes on the movement area.

(4) Runway holding position markings and signs.

(5) ILS critical area markings and signs.

(b) Each certificate holder shall provide and maintain, when the airport is

open during hours of darkness or during conditions below VFR minimums, at

least the following lighting systems for air carrier operations on the

airport:

(1) Runway lighting meeting the specifications for the approach with the

lowest minimums authorized for each runway.

(2) One of the following taxiway lighting systems:

(i) Centerline lights.

(ii) Centerline reflectors.

(iii) Edge lights.

(iv) Edge reflectors.

(3) An airport beacon.

(4) Approach lighting meeting the specifications for the approach with the

lowest minimums authorized for each runway, unless otherwise provided and

maintained by the FAA or another agency.

(5) Obstruction marking and lighting, as appropriate, on each object within

its authority which constitutes an obstruction under Part 77 of this chapter.

However, this lighting and marking is not required if it is determined to be

unnecessary by an FAA aeronautical study.

(c) Each certificate holder shall properly maintain each marking or

lighting system installed on the airport which is owned by the certificate

holder. As used in this section, to "properly maintain" includes: To clean,

replace, or repair any faded, missing, or nonfunctional item of lighting; to

keep each item unobscured and clearly visible; and to ensure that each item

provides an accurate reference to the user.

(d) Each certificate holder shall ensure that all lighting on the airport,

including that for aprons, vehicle parking areas, roadways, fuel storage

areas, and buildings, is adequately adjusted or shielded to prevent

interference with air traffic control and aircraft operations.

(e) FAA Advisory Circulars in the 150 series contain standards and

procedures for equipment, material, installation, and maintenance of light

systems and marking listed in this section which are acceptable to the

Administrator.

(f) Notwithstanding paragraph (a) of this section, a certificate holder is

not required to provide the identified signs in paragraph (a)(3) of this

section until January 1, 1995. Each certificate holder shall maintain each

marking system that meets paragraph (a)(3) of this section.

[Doc. No. 24812, 52 FR 44282, Nov. 18, 1987, as amended by Amdt. 139-15, 53

FR 40843, Oct. 18, 1988; Amdt. 139-19, 57 FR 15164, Apr. 24, 1992; Amdt.

139-20, 59 FR 7120, Feb. 14, 1994]

*****************************************************************************

59 FR 7118, No. 30, Feb. 14, 1994

SUMMARY: This final rule amends a final rule issued without notice based

upon comments received in response to that final rule. This final rule amends

the compliance date for certain sign requirements required under the Federal

Aviation Regulations (FAR) for airports certificated under 14 CFR part 139.

The compliance date for these sign requirements expired on January 1, 1994.

This amendment will provide the time necessary for industry to manufacture

and airport operators to install the required signs.

EFFECTIVE DATE: February 14, 1994.

*****************************************************************************

Sec. 139.313 Snow and ice control.

(a) Each certificate holder whose airport is located where snow and icing

conditions regularly occur shall prepare, maintain, and carry out a snow and

ice control plan.

(b) The snow and ice control plan required by this section shall include

instructions and procedures for--

(1) Prompt removal or control, as completely as practical, of snow, ice,

and slush on each movement area;

(2) Positioning snow off of movement area surfaces so that all air crarrier

aircraft propellers, engine pods, rotors, and wingtips will clear any

snowdrift and snowbank as the aircraft's landing gear traverses any full

strength portion of the movement area;

(3) Selection and application of approved materials for snow and ice

control to ensure that they adhere to snow and ice sufficiently to minimize

engine ingestion;

(4) Timely commencement of snow and ice control operations; and

(5) Prompt notification, in accordance with Sec. 139.339, of all air

carriers using the airport when any portion of the movement area normally

available to them is less than satisfactorily cleared for safe operation by

their aircraft.

(c) FAA Advisory Circulars in the 150 series contain standards for snow and

ice control equipment, materials, and procedures for snow and ice control

which are acceptable to the Administrator.

[Docket No. 24812, 52 FR 44282, Nov. 18, 1987; 53 FR 4258, Feb. 12, 1988]

Sec. 139.315 Aircraft rescue and firefighting: Index determination.

(a) An Index is required by paragraph (c) of this section for each

certificate holder. The Index is determined by a combination of--

(1) The length of air carrier aircraft expressed in groups; and

(2) Average daily departures of air carrier aircraft.

(b) For the purpose of Index determination, air carrier aircraft lengths

are grouped as follows:

(1) Index A includes aircraft less than 90 feet in length.

(2) Index B includes aircraft at least 90 feet but less than 126 feet in

length.

(3) Index C includes aircraft at least 126 feet but less than 159 feet in

length.

(4) Index D includes aircraft at least 159 feet but less than 200 feet in

length.

(5) Index E includes aircraft at least 200 feet in length.

(c) Except as provided in Sec. 139.319(c), the Index required by Sec.

139.319 is determined as follows:

(1) If there are five or more average daily departures of air carrier

aircraft in a single Index group serving that airport, the longest Index

group with an average of 5 or more daily departures is the Index required for

the airport.

(2) If there are less than five average daily departures of air carrier

aircraft in a single Index group serving that airport, the next lower Index

from the longest Index group with air carrier aircraft in it is the Index

required for the airport. The minimum designated Index shall be Index A.

Sec. 139.317Aircraft rescue and firefighting: Equipment and agents.

The following rescue and firefighting equipment and agents are the minimum

required for the Indexes referred to in Sec. 139.315:

(a) Index A: One vehicle carrying at least--

(1) 500 pounds of sodium-based dry chemical or halon 1211; or

(2) 450 pounds of potassium-based dry chemical and water with a

commensurate quantity of AFFF to total 100 gallons, for simultaneous dry

chemical and AFFF foam application.

(b) Index B: Either of the following:

(1) One vehicle carrying at least 500 pounds of sodium-based dry chemical

or halon 1211, and 1,500 gallons of water, and the commensurate quantity of

AFFF for foam production.

(2) Two vehicles--

(i) One vehicle carrying the extinguishing agents as specified in paragraph

(a)(1) or (2) of this section; and

(ii) One vehicle carrying an amount of water and the commensurate quantity

of AFFF so that the total quantity of water for foam production carried by

both vehicles is at least 1,500 gallons.

(c) Index C: Either of the following:

(1) Three vehicles--

(i) One vehicle carrying the extinguishing agents as specified in paragraph

(a)(1) or (2) of this section; and

(ii) Two vehicles carrying an amount of water and the commensurate quantity

of AFFF so that the total quantity of water for foam production carried by

all three vehicles is at least 3,000 gallons.

(2) Two vehicles--

(i) One vehicle carrying the extinguishing agents as specified in paragraph

(b)(1) of this section; and

(ii) One vehicle carrying water and the commensurate quantity of AFFF so

that the total quantity of water for foam production carried by both vehicles

is at least 3,000 gallons.

(d) Index D: Three vehicles--

(1) One vehicle carrying the extinguishing agents as specified in paragraph

(a)(1) or (2) of this section; and

(2) Two vehicles carrying an amount of water and the commensurate quantity

of AFFF so that the total quantity of water for foam production carried by

all three vehicles is at least 4,000 gallons.

(e) Index E: Three vehicles--

(1) One vehicle carrying the extinguishing agents as specified in paragraph

(a)(1) or (2) of this section; and

(2) Two vehicles carrying an amount of water and the commensurate quantity

of AFFF so that the total quantity of water for foam production carried by

all three vehicles is at least 6,000 gallons.

(f) Notwithstanding the provisions of paragraphs (a) through (e) of this

section, any certificate holder whose vehicles met the requirements of this

part for quantity and type of extinguishing agent on December 31, 1987, may

comply with the Index requirements of this section by carrying extinguishing

agents to the full capacity of those vehicles. Whenever any of those vehicles

is replaced or rehabilitated, the capacity of the replacement or

rehabilitated vehicle shall be sufficient to comply with the requirements of

the required Index.

(g) Foam discharge capacity. Each aircraft rescue and firefighting vehicle

used to comply with Index B, C, D, or E requirements with a capacity of at

least 500 gallons of water for foam production shall be equipped with a

turret. Vehicle turret discharge capacity shall be as follows:

(1) Each vehicle with a minimum rated vehicle water tank capacity of at

least 500 gallons but less than 2,000 gallons shall have a turret discharge

rate of at least 500 gallons per minute but not more than 1,000 gallons per

minute.

(2) Each vehicle with a minimum rated vehicle water tank capacity of at

least 2,000 gallons shall have a turret discharge rate of at least 600

gallons per minute but not more than 1,200 gallons per minute.

(3) Notwithstanding the requirements of paragraph (g) of this section, any

certificate holder whose aircraft rescue and firefighting vehicles are not

equipped with turrets or do not have the discharge capacity required in this

section, but otherwise met the requirements of this part on December 31,

1987, need not comply with paragraph (g) of this section for a particular

vehicle until that vehicle is replaced or rehabilitated.

(h) Dry chemical and halon 1211 discharge capacity. Each aircraft rescue

and firefighting vehicle which is required to carry dry chemical or halon

1211 for compliance with the index requirements of this section must meet one

of the following minimum discharge rates for the equipment installed:

(1) Dry chemical or halon 1211 through a hand line, 5 pounds per second.

(2) Dry chemical or halon 1211 through a turret, 16 pounds per second.

(i) Extinguishing agent substitutions. The following extinguishing agent

substitutions may be made:

(1) Protein or fluoroprotein foam concentrates may be substituted for AFFF.

When either of these substitutions is selected, the volume of water to be

carried for the substitute foam production shall be calculated by multiplying

the volume of water required for AFFF by the factor 1.5.

(2) Sodium- or potassium-based dry chemical or halon 1211 may be

substituted for AFFF. Up to 30 percent of the amount of water specified for

AFFF production may be replaced by dry chemical or halon 1211, except that

for airports where such extreme climatic conditions exist that water is

either unmanageable or unobtainable, as in arctic or desert regions, up to

100 percent of the required water may be replaced by dry chemical or halon

1211. When this substitution is selected, 12.7 pounds of dry chemical or

halon 1211 shall be substituted for each gallon of water used for AFFF foam

production.

(3) Sodium- or potassium-based dry chemical or halon 1211 may be

substituted for protein or fluoroprotein foam. When this substitution is

selected, 8.4 pounds of dry chemical or halon 1211 shall be substituted for

one gallon of water for protein or fluoroprotein foam production.

(4) AFFF may be substituted for dry chemical or halon 1211. For airports

where meteorological conditions, such as consistently high winds and

precipitation, would frequently prevent the effective use of dry chemical or

halon 1211, up to 50 percent of these agents may be replaced by water for

AFFF production. When this substitution is selected, one gallon of water for

foam production with the commensurate quantity of AFFF shall be substituted

for 12.7 pounds of dry chemical or halon 1211.

(5) Potassium-based dry chemical may be substituted for sodium-based dry

chemical. Where 500 pounds of sodium-based dry chemical is specified, 450

pounds of potassium-based dry chemical may be substituted.

(6) Other extinguishing agent substitutions acceptable to the Administrator

may be made in amounts that provide equivalent firefighting capability.

(j) In addition to the quantity of water required, each vehicle required to

carry AFFF shall carry AFFF in an appropriate amount to mix with twice the

water required to be carried by the vehicle.

(k) FAA Advisory Circulars in the 150 series contain standards and

procedures for AFFF equipment and agents which are acceptable to the

Administrator.

[52 FR 44282, Nov. 18, 1987; 53 FR 4120 and 4258, Feb. 12, 1988]

Sec. 139.319Aircraft rescue and firefighting: Operational requirements.

(a) Except as provided in paragraph (c) of this section, each certificate

holder shall provide on the airport, during air carrier operations at the

airport, at least the rescue and firefighting capability specified for the

Index required by Sec. 139.317.

(b) Increase in Index. Except as provided in paragraph (c) of this section,

if an increase in the average daily departures or the length of air carrier

aircraft results in an increase in the Index required by paragraph (a) of

this section, the certificate holder shall comply with the increased

requirements.

(c) Reduction in rescue and firefighting. During air carrier operations

with only aircraft shorter than the Index aircraft group required by

paragraph (a) of this section, the certificate holder may reduce the rescue

and firefighting to a lower level corresponding to the Index group of the

longest air carrier aircraft being operated.

(d) Any reduction in the rescue and firefighting capability from the Index

required by paragraph (a) of this section in accordance with paragraph (c) of

this section shall be subject to the following conditions:

(1) Procedures for, and the persons having the authority to implement, the

reductions must be included in the airport certification manual.

(2) A system and procedures for recall of the full aircraft rescue and

firefighting capability must be included in the airport certification manual.

(3) The reductions may not be implemented unless notification to air

carriers is provided in the Airport/Facility Directory or Notices to Airmen

(NOTAM), as appropriate, and by direct notification of local air carriers.

(e) Vehicle communications. Each vehicle required under Sec. 139.317 shall

be equipped with two-way voice radio communications which provides for

contact with at least--

(1) Each other required emergency vehicle;

(2) The air traffic control tower, if it is located on the airport; and

(3) Other stations, as specified in the airport emergency plan.

(f) Vehicle marking and lighting. Each vehicle required under Sec. 139.317

shall--

(1) Have a flashing or rotating beacon; and

(2) Be painted or marked in colors to enhance contrast with the background

environment and optimize daytime and nighttime visibility and identification.

(g) FAA Advisory Circulars in the 150 series contain standards for

painting, marking and lighting vehicles used on airports which are acceptable

to the Administrator.

(h) Vehicle readiness. Each vehicle required under Sec. 139.317 shall be

maintained as follows:

(1) The vehicle and its systems shall be maintained so as to be

operationally capable of performing the functions required by this subpart

during all air carrier operations.

(2) If the airport is located in a geographical area subject to prolonged

temperatures below 33 degrees Fahrenheit, the vehicles shall be provided with

cover or other means to ensure equipment operation and discharge under

freezing conditions.

(3) Any required vehicle which becomes inoperative to the extent that it

cannot perform as required by Sec. 139.319(h)(1) shall be replaced

immediately with equipment having at least equal capabilities. If replacement

equipment is not available immediately, the certificate holder shall so

notify the Regional Airports Division Manager and each air carrier using the

airport in accordance with Sec. 139.339. If the required Index level of

capability is not restored within 48 hours, the airport operator, unless

otherwise authorized by the Administrator, shall limit air carrier operations

on the airport to those compatible with the Index corresponding to the

remaining operative rescue and firefighting equipment.

(i) Response requirements. (1) Each certificate holder, with the airport

rescue and firefighting equipment required under this part and the number of

trained personnel which will assure an effective operation, shall--

(i) Respond to each emergency during periods of air carrier operations; and

(ii) When requested by the Administrator, demonstrate compliance with the

response requirements specified in this section.

(2) The response required by paragraph (i)(1)(ii) of this section shall

achieve the following performance:

(i) Within 3 minutes from the time of the alarm, at least one required

airport rescue and firefighting vehicle shall reach the midpoint of the

farthest runway serving air carrier aircraft from its assigned post, or reach

any other specified point of comparable distance on the movement area which

is available to air carriers, and begin application of foam, dry chemical, or

halon 1211.

(ii) Within 4 minutes from the time of alarm, all other required vehicles

shall reach the point specified in paragraph (i)(2)(i) of this section from

their assigned post and begin application of foam, dry chemical, or halon

1211.

(j) Personnel. Each certificate holder shall ensure the following:

(1) All rescue and firefighting personnel are equipped in a manner

acceptable to the Administrator with protective clothing and equipment needed

to perform their duties.

(2) All rescue and firefighting personnel are properly trained to perform

their duties in a manner acceptable to the Administrator. The training

curriculum shall include initial and recurrent instruction in at least the

following areas:

(i) Airport familiarization.

(ii) Aircraft familiarization.

(iii) Rescue and firefighting personnel safety.

(iv) Emergency communications systems on the airport, including fire

alarms.

(v) Use of the fire hoses, nozzles, turrets, and other appliances required

for compliance with this part.

(vi) Application of the types of extinguishing agents required for

compliance with this part.

(vii) Emergency aircraft evacuation assistance.

(viii) Firefighting operations.

(ix) Adapting and using structural rescue and firefighting equipment for

aircraft rescue and firefighting.

(x) Aircraft cargo hazards.

(xi) Familiarization with firefighters' duties under the airport emergency

plan.

(3) All rescue and firefighting personnel participate in at least one live-

fire drill every 12 months.

(4) After January 1, 1989, at least one of the required personnel on duty

during air carrier operations has been trained and is current in basic

emergency medical care. This training shall include 40 hours covering at

least the following areas:

(i) Bleeding.

(ii) Cardiopulmonary resuscitation.

(iii) Shock.

(iv) Primary patient survey.

(v) Injuries to the skull, spine, chest, and extremities.

(vi) Internal injuries.

(vii) Moving patients.

(viii) Burns.

(ix) Triage.

(5) Sufficient rescue and firefighting personnel are available during all

air carrier operations to operate the vehicles, meet the response times, and

meet the miminum agent discharge rates required by this part;

(6) Procedures and equipment are established and maintained for alerting

rescue and firefighting personnel by siren, alarm, or other means acceptable

to the Administrator, to any existing or impending emergency requiring their

assistance.

(k) Emergency access roads. Each certificate holder shall ensure that roads

which are designated for use as emergency access roads for aircraft rescue

and firefighting vehicles are maintained in a condition that will support

those vehicles during all-weather conditions.

[Docket No. 24812, 52 FR 44282, Nov. 18, 1987; 53 FR 4258, Feb. 12, 1988, as

amended by Amdt. 139-15, 53 FR 40843, Oct. 18, 1988; Amdt. 139-16, 54 FR

39295, Sept. 25, 1989]

Sec. 139.321 Handling and storing of hazardous substances and materials.

(a) Each certificate holder which acts as a cargo handling agent shall

establish and maintain procedures for the protection of persons and property

on the airport during the handling and storing of any material regulated by

the Hazardous Materials Regulations (49 CFR Part 171, et seq.), that is, or

is intended to be, transported by air. These procedures shall provide for at

least the following:

(1) Designated personnel to receive and handle hazardous substances and

materials.

(2) Assurance from the shipper that the cargo can be handled safely,

including any special handling procedures required for safety.

(3) Special areas for storage of hazardous materials while on the airport.

(b) Each certificate holder shall establish and maintain standards

acceptable to the Administrator for protecting against fire and explosions in

storing, dispensing, and otherwise handling fuel, lubricants, and oxygen

(other than articles and materials that are, or are intended to be, aircraft

cargo) on the airport. These standards shall cover facilities, procedures,

and personnel training and shall address at least the following:

(1) Grounding and bonding.

(2) Public protection.

(3) Control of access to storage areas.

(4) Fire safety in fuel farm and storage areas.

(5) Fire safety in mobile fuelers, fueling pits, and fueling cabinets.

(6) After January 1, 1989, training of fueling personnel in fire safety in

accordance with paragraph (e) of this section.

(7) The fire code of the public body having jurisdiction over the airport.

(c) Each certificate holder shall, as a fueling agent, comply with and,

except as provided in paragraph (h) of this section, require all other

fueling agents operating on the airport to comply with the standards

established under paragraph (b) of this section and shall perform reasonable

surveillance of all fueling activities on the airport with respect to those

standards.

(d) Each certificate holder shall inspect the physical facilities of each

airport tenant fueling agent at least once every 3 months for compliance with

paragraph (b) of this section and maintain a record of that inspection for at

least 12 months. The certificate holder may use an independent organization

to perform this inspection if--

(1) It is acceptable by the Administrator; and

(2) It prepares a record of its inspection sufficiently detailed to assure

the certificate holder and the FAA that the inspection is adequate.

(e) The training required in paragraph (b)(6) of this section shall include

at least the following:

(1) At least one supervisor with each fueling agent shall have completed an

aviation fuel training course in fire safety which is acceptable to the

Administrator.

(2) All other employees who fuel aircraft, accept fuel shipments, or

otherwise handle fuel shall receive at least on-the-job training in fire

safety from the supervisor trained in accordance with paragraph (e)(1) of

this section.

(f) Each certificate holder shall obtain certification once a year from

each airport tenant fueling agent that the training required by paragraph (e)

of this section has been accomplished.

(g) Unless otherwise authorized by the Administrator, each certificate

holder shall require each tenant fueling agent to take immediate corrective

action whenever the certificate holder becomes aware of noncompliance with a

standard required by paragraph (b) of this section. The certificate holder

shall notify the appropriate FAA Regional Airports Division Manager

immediately when noncompliance is discovered and corrective action cannot be

accomplished within a reasonable period of time.

(h) A certificate holder need not require an air carrier operating under

Part 121 or Part 135 of this chapter to comply with the standards required by

this section.

(i) FAA Advisory Circulars in the 150 Series contain standards and

procedures for the handling and storage of hazardous substances and materials

which are acceptable to the Administrator.

[52 FR 44282, Nov. 18, 1987; 53 FR 4120, Feb. 12, 1988, as amended by Amdt.

139-15, 53 FR 40843, Oct. 18, 1988; Amdt. 139-16, 54 FR 39295, Sept. 25,

1989]

Sec. 139.323Traffic and wind direction indicators.

Each certificate holder shall provide the following on its airport:

(a) A wind cone that provides surface wind direction information visually

to pilots. For each airport in a terminal control area, supplemental wind

cones shall be installed at each runway end or at least at one point visible

to the pilot while on final approach and prior to takeoff. If the airport is

open for air carrier operations during hours of darkness, the wind direction

indicators must be lighted.

(b) For airports serving any air carrier operation when there is no control

tower operating, a segmented circle around one wind cone and a landing strip

and traffic pattern indicator for each runway with a right-hand traffic

pattern.

EFFECTIVE DATE NOTE: Amdt. 139-18, 56 FR 65664, Dec. 17, 1991, revised

paragraph (a) of Sec. 139.323 effective September 16, 1993. For the

convenience of the user, the revised text is set forth as follows:

Sec. 139.323 Traffic and wind direction indicators.

* * * * *

(a) A wind cone that provides surface wind direction information visually

to pilots. For each airport in a Class B airspace area, supplemental wind

cones must be installed at each runway end or at least at one point visible

to the pilot while on final approach and prior to takeoff. If the airport is

open for air carrier operations during hours of darkness, the wind direction

indicators must be lighted.

*****************************************************************************

56 FR 65638, No. 242, Dec. 17, 1991

SUMMARY: This final rule amends the Federal Aviation Regulations (FAR) to

adopt certain recommendations of the National Airspace Review (NAR)

concerning changes to regulations and procedures in regard to airspace

classifications. These changes are intended to: (1) Simplify airspace

designations; (2) achieve international commonality of airspace designations;

(3) increase standardization of equipment requirements for operations in

various classifications of airspace; (4) describe appropriate pilot

certificate requirements, visual flight rules (VFR) visibility and distance

from cloud rules, and air traffic services offered in each class of airspace;

and (5) satisfy the responsibilities of the United States as a member of the

International Civil Aviation Organization (ICAO). The final rule also amends

the requirement for minimum distance from clouds in certain airspace areas

and the requirements for communications with air traffic control (ATC) in

certain airspace areas; eliminates airport radar service areas (ARSAs),

control zones, and terminal control areas (TCAs) as airspace classifications;

and eliminates the term "airport traffic area." The FAA believes simplified

airspace classifications will reduce existing airspace complexity and thereby

enhance safety.

EFFECTIVE DATE: These regulations become effective September 16, 1993,

except that Secs. 11.61(c), 91.215(d), 71.601, 71.603, 71.605, 71.607, and

71.609 and Part 75 become effective December 12, 1991, and except that

amendatory instruction number 20, Sec. 71.1, is effective as of December 17,

1991 through September 15, 1993, and that Secs. 71.11 and 71.19 become

effective October 15, 1992. The incorporation by reference of FAA Order

7400.7 in Sec. 71.1 (amendatory instruction number 20) is approved by the

Director of the Federal Register as of December 17, 1991, through September

15, 1993. The incorporation by reference of FAA Order 7400.9 in Sec. 71.1

(amendatory instruction number 24) is approved by the Director of the

Federal Register as of September 16, 1993 through September 15, 1994.

*****************************************************************************

Sec. 139.325 Airport emergency plan.

(a) Each certificate holder shall develop and maintain an airport emergency

plan designed to minimize the possibility and extent of personal injury and

property damage on the airport in an emergency. The plan must include--

(1) Procedures for prompt response to all of the emergencies listed in

paragraph (b) of this section, including a communications network; and

(2) Sufficient detail to provide adequate guidance to each person who must

implement it.

(b) The plan required by this section must contain instructions for

response to--

(1) Aircraft incidents and accidents;

(2) Bomb incidents, including designated parking areas for the aircraft

involved;

(3) Structural fires;

(4) Natural disaster;

(5) Radiological incidents;

(6) Sabotage, hijack incidents, and other unlawful interference with

operations;

(7) Failure of power for movement area lighting; and

(8) Water rescue situations.

(c) The plan required by this section must address or include--

(1) To the extent practicable, provisions for medical services including

transportation and medical assistance for the maximum number of persons that

can be carried on the largest air carrier aircraft that the airport

reasonably can be expected to serve;

(2) The name, location, telephone number, and emergency capability of each

hospital and other medical facility, and the business address and telephone

number of medical personnel on the airport or in the communities it serves,

agreeing to provide medical assistance or transportation;

(3) The name, location, and telephone number of each rescue squad,

ambulance service, military installation, and government agency on the

airport or in the communities it serves, that agrees to provide medical

assistance or transportation;

(4) An inventory of surface vehicles and aircraft that the facilities,

agencies, and personnel included in the plan under paragraphs (c)(2) and

(c)(3) of this section will provide to transport injured and deceased persons

to locations on the airport and in the communities it serves;

(5) Each hangar or other building on the airport or in the communities it

serves that will be used to accommodate uninjured, injured, and deceased

persons;

(6) Crowd control, specifying the name and location of each safety or

security agency that agrees to provide assistance for the control of crowds

in the event of an emergency on the airport; and

(7) The removal of disabled aircraft including to the extent practical the

name, location and telephone numbers of agencies with aircraft removal

responsibilities or capabilities.

(d) The plan required by this section must provide for--

(1) The marshalling, transportation, and care of ambulatory injured and

uninjured accident survivors;

(2) The removal of disabled aircraft;

(3) Emergency alarm systems; and

(4) Coordination of airport and control tower functions relating to

emergency actions.

(e) The plan required by this section shall contain procedures for

notifying the facilities, agencies, and personnel who have responsibilities

under the plan of the location of an aircraft accident, the number of persons

involved in that accident, or any other information necessary to carry out

their responsibilities, as soon as that information is available.

(f) The plan required by this section shall contain provisions, to the

extent practicable, for the rescue of aircraft accident victims from

significant bodies of water or marsh lands adjacent to the airport which are

crossed by the approach and departure flight paths of air carriers. A body of

water or marsh land is significant if the area exceeds one-quarter square

mile and cannot be traversed by conventional land rescue vehicles. To the

extent practicable, the plan shall provide for rescue vehicles with a

combined capacity for handling the maximum number of persons that can be

carried on board the largest air carrier aircraft that the airport reasonably

can be expected to serve.

(g) Each certificate holder shall--

(1) Coordinate its plan with law enforcement agencies, rescue and fire

fighting agencies, medical personnel and organizations, the principal tenants

at the airport, and all other persons who have responsibilities under the

plan;

(2) To the extent practicable, provide for participation by all facilities,

agencies, and personnel specified in paragraph (g)(1) of this section in the

development of the plan;

(3) Ensure that all airport personnel having duties and responsibilities

under the plan are familiar with their assignments and are properly trained;

(4) At least once every 12 months, review the plan with all of the parties

with whom the plan is coordinated as specified in paragraph (g)(1) of this

section, to ensure that all parties know their responsibilities and that all

of the information in the plan is current; and

(5) Hold a full-scale airport emergency plan exercise at least once every 3

years.

(h) FAA Advisory Circulars in the 150 Series contain standards and

procedures for the development of an airport emergency plan which are

acceptable to the Administrator.

[52 FR 44282, Nov. 18, 1987; 53 FR 4258, Feb. 12, 1988]

Sec. 139.327Self-inspection program.

(a) Each certificate holder shall inspect the airport to assure compliance

with this subpart--

(1) Daily, except as otherwise required by the airport certification manual

or airport certification specifications;

(2) When required by any unusual condition such as construction activities

or meteorological conditions that may affect safe air carrier operations; and

(3) Immediately after an accident or incident.

(b) Each certificate holder shall provide the following:

(1) Equipment for use in conducting safety inspections of the airport;

(2) Procedures, facilities, and equipment for reliable and rapid

dissemination of information between airport personnel and its air carriers;

(3) Procedures to ensure that qualified inspection personnel perform the

inspections; and

(4) A reporting system to ensure prompt correction of unsafe airport

conditions noted during the inspection.

(c) Each certificate holder shall prepare and keep for at least 6 months,

and make available for inspection by the Administrator on request, a record

of each inspection prescribed by this section, showing the conditions found

and all corrective actions taken.

(d) FAA Advisory Circulars in the 150 series contain standards and

procedures for the conduct of airport self-inspections which are acceptable

to the Administrator.

[52 FR 44282, Nov. 18, 1987; 53 FR 4120, Feb. 12, 1988]

Sec. 139.329 Ground vehicles.

Each certificate holder shall--

(a) Limit access to movement areas and safety areas only to those ground

vehicles necessary for airport operations;

(b) Establish and implement procedures for the safe and orderly access to,

and operation on, the movement area and safety areas by ground vehicles,

including provisions identifying the consequences of noncompliance with the

procedures by an employee, tenant, or contractor;

(c) When an air traffic control tower is in operation, ensure that each

ground vehicle operating on the movement area is controlled by one of the

following:

(1) Two-way radio communications between each vehicle and the tower,

(2) An escort vehicle with two-way radio communications with the tower to

accompany any vehicle without a radio, or

(3) Measures acceptable to the Administrator for controlling vehicles, such

as signs, signals, or guards, when it is not operationally practical to have

two-way radio communications with the vehicle or an escort vehicle;

(d) When an air traffic control tower is not in operation, provide adequate

procedures to control ground vehicles on the movement area through

prearranged signs or signals;

(e) Ensure that each employee, tenant, or contractor who operates a ground

vehicle on any portion of the airport that has access to the movement area is

familiar with the airport's procedures for the operation of ground vehicles

and the consequences of noncompliance; and

(f) On request by the Administrator, make available for inspection any

record of accidents or incidents on the movement areas involving air carrier

aircraft and/or ground vehicles.

[Docket No. 24812, 52 FR 44282, Nov. 18, 1987, as amended by Amdt. No.

139-17, 55 FR 48214, Nov. 19, 1990]

*****************************************************************************

55 FR 48212, No. 223, Nov. 19, 1990

SUMMARY: This final rule makes two changes to the certification and

operations regulations of land airports serving air carriers. The first

change revises the certification requirements to provide that a person

operating an uncertificated airport may serve, when authorized by the

Administrator, unscheduled air carrier operations with aircraft having a

seating capacity of more than 30 passengers. As revised, airport

certification requirements and the regulations applicable to air carrier

operations are consistent in this regard. The second change clarifies

responsibility for the establishment of and compliance with rules for airport

ground vehicle operations by tenants, contractors, and employees. This change

is necessary to address the responsibility of certificate holders with regard

to ground vehicle operations.

EFFECTIVE DATE: December 19, 1990.

*****************************************************************************

Sec. 139.331 Obstructions.

Each certificate holder shall ensure that each object in each area within

its authority which exceeds any of the heights or penetrates the imaginary

surfaces described in Part 77 of this chapter is either removed, marked, or

lighted. However, removal, marking, and lighting is not required if it is

determined to be unnecessary by an FAA aeronautical study.

Sec. 139.333 Protection of navaids.

Each certificate holder shall--

(a) Prevent the construction of facilities on its airport that, as

determined by the Administrator, would derogate the operation of an

electronic or visual navaid and air traffic control facilities on the

airport;

(b) Protect, or if the owner is other than the certificate holder, assist

in protecting, all navaids on its airport against vandalism and theft; and

(c) Prevent, insofar as it is within the airport's authority, interruption

of visual and electronic signals of navaids.

Sec. 139.335 Public protection.

(a) Each certificate holder shall provide--

(1) Safeguards acceptable to the Administrator to prevent inadvertent entry

to the movement area by unauthorized persons or vehicles; and

(2) Reasonable protection of persons and property from aircraft blast.

(b) Fencing meeting the requirements of Part 107 of this chapter in areas

subject to that part is acceptable for meeting the requirements of paragraph

(a)(1) of this section.

Sec. 139.337 Wildlife hazard management.

(a) Each certificate holder shall provide for the conduct of an ecological

study, acceptable to the Administrator, when any of the following events

occurs on or near the airport:

(1) An air carrier aircraft experiences a multiple bird strike or engine

ingestion.

(2) An air carrier aircraft experiences a damaging collision with wildlife

other than birds.

(3) Wildlife of a size or in numbers capable of causing an event described

in paragraph (a) (1) or (2) of this section is observed to have access to any

airport flight pattern or movement area.

(b) The study required in paragraph (a) of this section shall contain at

least the following:

(1) Analysis of the event which prompted the study.

(2) Identification of the species, numbers, locations, local movements, and

daily and seasonal occurrences of wildlife observed.

(3) Identification and location of features on and near the airport that

attract wildlife.

(4) Description of the wildlife hazard to air carrier operations.

(c) The study required by paragraph (a) of this section shall be submitted

to the Administrator, who determines whether or not there is a need for a

wildlife hazard management plan. In reaching this determination, the

Administrator considers--

(1) The ecological study;

(2) The aeronautical activity at the airport;

(3) The views of the certificate holder;

(4) The views of the airport users; and

(5) Any other factors bearing on the matter of which the Administrator is

aware.

(d) When the Administrator determines that a wildlife hazard management

plan is needed, the certificate holder shall formulate and implement a plan

using the ecological study as a basis. The plan shall--

(1) Be submitted to, and approved by, the Administrator prior to

implementation; and

(2) Provide measures to alleviate or eliminate wildlife hazards to air

carrier operations.

(e) The plan shall include at least the following:

(1) The persons who have authority and responsibility for implementing the

plan.

(2) Priorities for needed habitat modification and changes in land use

identified in the ecological study, with target dates for completion.

(3) Requirements for and, where applicable, copies of local, state, and

Federal wildlife control permits.

(4) Identification of resources to be provided by the certificate holder

for implementation of the plan.

(5) Procedures to be followed during air carrier operations, including at

least--

(i) Assignment of personnel responsibilities for implementing the

procedures;

(ii) Conduct of physical inspections of the movement area and other areas

critical to wildlife hazard management sufficiently in advance of air carrier

operations to allow time for wildlife controls to be effective;

(iii) Wildlife control measures; and

(iv) Communication between the wildlife control personnel and any air

traffic control tower in operation at the airport.

(6) Periodic evaluation and review of the wildlife hazard management plan

for--

(i) Effectiveness in dealing with the wildlife hazard; and

(ii) Indications that the existence of the wildlife hazard, as previously

described in the ecological study, should be reevaluated.

(7) A training program to provide airport personnel with the knowledge and

skills needed to carry out the wildlife hazard management plan required by

paragraph (d) of this section.

(f) Notwithstanding the other requirements of this section, each

certificate holder shall take immediate measures to alleviate wildlife

hazards whenever they are detected.

(g) FAA Advisory Circulars in the 150 series contain standards and

procedures for wildlife hazard management at airports which are acceptable to

the Administrator.

Sec. 139.339Airport condition reporting.

(a) Each certificate holder shall provide for the collection and

dissemination of airport condition information to air carriers.

(b) In complying with paragraph (a) of this section, the certificate holder

shall utilize the NOTAM system and, as appropriate, other systems and

procedures acceptable to the Administrator.

(c) In complying with paragraph (a) of this section, the certificate holder

shall provide information on the following airport conditions which may

affect the safe operations of air carriers:

(1) Construction or maintenance activity on movement areas, safety areas,

or loading ramps and parking areas.

(2) Surface irregularities on movement areas or loading ramps and parking

areas.

(3) Snow, ice, slush, or water on the movement area or loading ramps and

parking areas.

(4) Snow piled or drifted on or near movement areas contrary to Sec.

139.313.

(5) Objects on the movement area or safety areas contrary to Sec. 139.309.

(6) Malfunction of any lighting system required by Sec. 139.311.

(7) Unresolved wildlife hazards as identified in accordance with Sec.

139.337.

(8) Nonavailability of any rescue and firefighting capability required in

Secs. 139.317 and 139.319.

(9) Any other condition as specified in the airport certification manual or

airport certification specifications, or which may otherwise adversely affect

the safe operations of air carriers.

(d) FAA Advisory Circulars in the 150 series contain standards and

procedures for using the NOTAM system for dissemination of airport

information which are acceptable to the Administrator.

[52 FR 44282, Nov. 18, 1987; 53 FR 4258, Feb. 12, 1988]

Sec. 139.341 Identifying, marking, and reporting construction and other

unserviceable areas.

(a) Each certificate holder shall--

(1) Mark and, if appropriate, light in a manner acceptable to the

Administrator--

(i) Each construction area and unserviceable area which is on or adjacent

to any movement area or any other area of the airport on which air carrier

aircraft may be operated;

(ii) Each item of construction equipment and each construction roadway,

which may affect the safe movement of aircraft on the airport; and

(iii) Any area adjacent to a navaid that, if traversed, could cause

derogation of the signal or the failure of the navaid, and

(2) Provide procedures, such as a review of all appropriate utility plans

prior to construction, for avoiding damage to existing utilities, cables,

wires, conduits, pipelines, or other underground facilities.

(b) FAA Advisory Circulars in the 150 series contain standards and

procedures for identifying and marking construction areas which are

acceptable to the Administrator.

Sec. 139.343 Noncomplying conditions.

Unless otherwise authorized by the Administrator, whenever the requirements

of Subpart D of this part cannot be met to the extent that uncorrected unsafe

conditions exist on the airport, the certificate holder shall limit air

carrier operations to those portions of the airport not rendered unsafe by

those conditions.