Title 14--Aeronautics and Space

CHAPTER I--FEDERAL AVIATION ADMINISTRATION, DEPARTMENT OF TRANSPORTATION

SUBCHAPTER C--AIRCRAFT

PART 25--AIRWORTHINESS STANDARDS: TRANSPORT CATEGORY AIRPLANES

Special Federal Aviation Regulation No. 13

Subpart A--General

Sec. 25.1 Applicability.

Sec. 25.2 Special retroactive requirements.

Subpart B--Flight

General

Sec. 25.21 Proof of compliance.

Sec. 25.23 Load distribution limits.

Sec. 25.25 Weight limits.

Sec. 25.27 Center of gravity limits.

Sec. 25.29 Empty weight and corresponding center of gravity.

Sec. 25.31 Removable ballast.

Sec. 25.33 Propeller speed and pitch limits.

Performance

Sec. 25.101 General.

Sec. 25.103 Stalling speed.

Sec. 25.105 Takeoff.

Sec. 25.107 Takeoff speeds.

Sec. 25.109 Accelerate-stop distance.

Sec. 25.111 Takeoff path.

Sec. 25.113 Takeoff distance and takeoff run.

Sec. 25.115 Takeoff flight path.

Sec. 25.117 Climb: general.

Sec. 25.119 Landing climb: All-engine-operating.

Sec. 25.121 Climb: One-engine-inoperative.

Sec. 25.123 En route flight paths.

Sec. 25.125 Landing.

Controllability and Maneuverability

Sec. 25.143 General.

Sec. 25.145 Longitudinal control.

Sec. 25.147 Directional and lateral control.

Sec. 25.149 Minimum control speed.

Trim

Sec. 25.161 Trim.

Stability

Sec. 25.171 General.

Sec. 25.173 Static longitudinal stability.

Sec. 25.175 Demonstration of static longitudinal stability.

Sec. 25.177 Static lateral-directional stability.

Sec. 25.181 Dynamic stability.

Stalls

Sec. 25.201 Stall demonstration.

Sec. 25.203 Stall characteristics.

Sec. 25.205 [Removed. Amdt. 25-72, 55 FR 29775, July 20, 1990]

Sec. 25.207 Stall warning.

Ground and Water Handling Characteristics

Sec. 25.231 Longitudinal stability and control.

Sec. 25.233 Directional stability and control.

Sec. 25.235 Taxiing condition.

Sec. 25.237 Wind velocities.

Sec. 25.239 Spray characteristics, control, and stability on

water.

Miscellaneous Flight Requirements

Sec. 25.251 Vibration and buffeting.

Sec. 25.253 High-speed characteristics.

Sec. 25.255 Out-of-trim characteristics.

Subpart C--Structure

General

Sec. 25.301 Loads.

Sec. 25.303 Factor of safety.

Sec. 25.305 Strength and deformation.

Sec. 25.307 Proof of structure.

Flight Loads

Sec. 25.321 General.

Flight Maneuver and Gust Conditions

Sec. 25.331 General.

Sec. 25.333 Flight envelope.

Sec. 25.335 Design airspeeds.

Sec. 25.337 Limit maneuvering load factors.

Sec. 25.341 Gust loads.

Sec. 25.343 Design fuel and oil loads.

Sec. 25.345 High lift devices.

Sec. 25.349 Rolling conditions.

Sec. 25.351 Yawing conditions.

Supplementary Conditions

Sec. 25.361 Engine torque.

Sec. 25.363 Side load on engine mount.

Sec. 25.365 Pressurized cabin loads.

Sec. 25.367 Unsymmetrical loads due to engine failure.

Sec. 25.371 Gyroscopic loads.

Sec. 25.373 Speed control devices.

Control Surface and System Loads

Sec. 25.391 Control surface loads: general.

Sec. 25.393 Loads parallel to hinge line.

Sec. 25.395 Control system.

Sec. 25.397 Control system loads.

Sec. 25.399 Dual control system.

Sec. 25.405 Secondary control system.

Sec. 25.407 Trim tab effects.

Sec. 25.409 Tabs.

Sec. 25.415 Ground gust conditions.

Sec. 25.427 Unsymmetrical loads.

Sec. 25.445 Outboard fins.

Sec. 25.457 Wing flaps.

Sec. 25.459 Special devices.

Ground Loads

Sec. 25.471 General.

Sec. 25.473 Ground load conditions and assumptions.

Sec. 25.477 Landing gear arrangement.

Sec. 25.479 Level landing conditions.

Sec. 25.481 Tail-down landing conditions.

Sec. 25.483 One-wheel landing conditions.

Sec. 25.485 Side load conditions.

Sec. 25.487 Rebound landing condition.

Sec. 25.489 Ground handling conditions.

Sec. 25.491 Takeoff run.

Sec. 25.493 Braked roll conditions.

Sec. 25.495 Turning.

Sec. 25.497 Tail-wheel yawing.

Sec. 25.499 Nose-wheel yaw.

Sec. 25.503 Pivoting.

Sec. 25.507 Reversed braking.

Sec. 25.509 Towing loads.

Sec. 25.511 Ground load: unsymmetrical loads on multiple-wheel

units.

Water Loads

Sec. 25.521 General.

Sec. 25.523 Design weights and center of gravity positions.

Sec. 25.525 Application of loads.

Sec. 25.527 Hull and main float load factors.

Sec. 25.529 Hull and main float landing conditions.

Sec. 25.531 Hull and main float takeoff condition.

Sec. 25.533 Hull and main float bottom pressures.

Sec. 25.535 Auxiliary float loads.

Sec. 25.537 Seawing loads.

Emergency Landing Conditions

Sec. 25.561 General.

Sec. 25.562 Emergency landing dynamic conditions.

Sec. 25.563 Structural ditching provisions.

Fatigue Evaluation

Sec. 25.571 Damage--tolerance and fatigue evaluation of

structure.

Lightning Protection

Sec. 25.581 Lightning protection.

Subpart D--Design and Construction

General

Sec. 25.601 General.

Sec. 25.603 Materials.

Sec. 25.605 Fabrication methods.

Sec. 25.607 Fasteners.

Sec. 25.609 Protection of structure.

Sec. 25.611 Accessibility provisions.

Sec. 25.613 Material strength properties and design values.

Sec. 25.615 [Removed. Amdt. 25-72, 55 FR 29776, July 20, 1990]

Sec. 25.619 Special factors.

Sec. 25.621 Casting factors.

Sec. 25.623 Bearing factors.

Sec. 25.625 Fitting factors.

Sec. 25.629 Aeroelastic stability requirements.

Sec. 25.631 Bird strike damage.

Control Surfaces

Sec. 25.651 Proof of strength.

Sec. 25.655 Installation.

Sec. 25.657 Hinges.

Control Systems

Sec. 25.671 General.

Sec. 25.672 Stability augmentation and automatic and power-

operated systems.

Sec. 25.673 [Removed. Amdt. 25-72, 55 FR 29777, July 20, 1990]

Sec. 25.675 Stops.

Sec. 25.677 Trim systems.

Sec. 25.679 Control system gust locks.

Sec. 25.681 Limit load static tests.

Sec. 25.683 Operation tests.

Sec. 25.685 Control system details.

Sec. 25.689 Cable systems.

Sec. 25.693 Joints.

Sec. 25.697 Lift and drag devices, controls.

Sec. 25.699 Lift and drag device indicator.

Sec. 25.701 Flap and slat interconnection.

Sec. 25.703 Takeoff warning system.

Landing Gear

Sec. 25.721 General.

Sec. 25.723 Shock absorption tests.

Sec. 25.725 Limit drop tests.

Sec. 25.727 Reserve energy absorption drop tests.

Sec. 25.729 Retracting mechanism.

Sec. 25.731 Wheels.

Sec. 25.733 Tires.

Sec. 25.735 Brakes.

Sec. 25.737 Skis.

Floats and Hulls

Sec. 25.751 Main float buoyancy.

Sec. 25.753 Main float design.

Sec. 25.755 Hulls.

Personnel and Cargo Accommodations

Sec. 25.771 Pilot compartment.

Sec. 25.772 Pilot compartment doors.

Sec. 25.773 Pilot compartment view.

Sec. 25.775 Windshields and windows.

Sec. 25.777 Cockpit controls.

Sec. 25.779 Motion and effect of cockpit controls.

Sec. 25.781 Cockpit control knob shape.

Sec. 25.783 Doors.

Sec. 25.785 Seats, berths, safety belts, and harnesses.

Sec. 25.787 Stowage compartments.

Sec. 25.789 Retention of items of mass in passenger and crew

compartments and galleys.

Sec. 25.791 Passenger information signs and placards.

Sec. 25.793 Floor surfaces.

Emergency Provisions

Sec. 25.801 Ditching.

Sec. 25.803 Emergency evacuation.

Sec. 25.805 [Removed. 55 FR 29781, July 20, 1990]

Sec. 25.807 Emergency exits.

Sec. 25.809 Emergency exit arrangement.

Sec. 25.810 Emergency egress assist means and escape routes.

Sec. 25.811 Emergency exit marking.

Sec. 25.812 Emergency lighting.

Sec. 25.813 Emergency exit access.

Sec. 25.815 Width of aisle.

Sec. 25.817 Maximum number of seats abreast.

Sec. 25.819 Lower deck service compartments (including

galleys).

Ventilation and Heating

Sec. 25.831 Ventilation.

Sec. 25.832 Cabin ozone concentration.

Sec. 25.833 Combustion heating systems.

Pressurization

Sec. 25.841 Pressurized cabins.

Sec. 25.843 Tests for pressurized cabins.

Fire Protection

Sec. 25.851 Fire extinguishers.

Sec. 25.853 Compartment interiors.

Sec. 25.854 Lavatory fire protection.

Sec. 25.855 Cargo or baggage compartments.

Sec. 25.857 Cargo compartment classification.

Sec. 25.858 Cargo compartment fire detection systems.

Sec. 25.859 Combustion heater fire protection.

Sec. 25.863 Flammable fluid fire protection.

Sec. 25.865 Fire protection of flight controls, engine mounts,

and other flight structure.

Sec. 25.867 Fire protection: other components.

Sec. 25.869 Fire protection: systems.

Miscellaneous

Sec. 25.871 Leveling means.

Sec. 25.875 Reinforcement near propellers.

Subpart E--Powerplant

General

Sec. 25.901 Installation.

Sec. 25.903 Engines.

Sec. 25.904 Automatic takeoff thrust control system (ATTCS).

Sec. 25.905 Propellers.

Sec. 25.907 Propeller vibration.

Sec. 25.925 Propeller clearance.

Sec. 25.929 Propeller deicing.

Sec. 25.933 Reversing systems.

Sec. 25.934 Turbojet engine thrust reverser system tests.

Sec. 25.937 Turbopropeller-drag limiting systems.

Sec. 25.939 Turbine engine operating characteristics.

Sec. 25.941 Inlet, engine, and exhaust compatibility.

Sec. 25.943 Negative acceleration.

Sec. 25.945 Thrust or power augmentation system.

Fuel System

Sec. 25.951 General.

Sec. 25.952 Fuel system analysis and test.

Sec. 25.953 Fuel system independence.

Sec. 25.954 Fuel system lightning protection.

Sec. 25.955 Fuel flow.

Sec. 25.957 Flow between interconnected tanks.

Sec. 25.959 Unusable fuel supply.

Sec. 25.961 Fuel system hot weather operation.

Sec. 25.963 Fuel tanks: general.

Sec. 25.965 Fuel tank tests.

Sec. 25.967 Fuel tank installations.

Sec. 25.969 Fuel tank expansion space.

Sec. 25.971 Fuel tank sump.

Sec. 25.973 Fuel tank filler connection.

Sec. 25.975 Fuel tank vents and carburetor vapor vents.

Sec. 25.977 Fuel tank outlet.

Sec. 25.979 Pressure fueling system.

Sec. 25.981 Fuel tank temperature.

Fuel System Components

Sec. 25.991 Fuel pumps.

Sec. 25.993 Fuel system lines and fittings.

Sec. 25.994 Fuel system components.

Sec. 25.995 Fuel valves.

Sec. 25.997 Fuel strainer or filter.

Sec. 25.999 Fuel system drains.

Sec. 25.1001 Fuel jettisoning system.

Oil System

Sec. 25.1011 General.

Sec. 25.1013 Oil tanks.

Sec. 25.1015 Oil tank tests.

Sec. 25.1017 Oil lines and fittings.

Sec. 25.1019 Oil strainer or filter.

Sec. 25.1021 Oil system drains.

Sec. 25.1023 Oil radiators.

Sec. 25.1025 Oil valves.

Sec. 25.1027 Propeller feathering system.

Cooling

Sec. 25.1041 General.

Sec. 25.1043 Cooling tests.

Sec. 25.1045 Cooling test procedures.

Induction System

Sec. 25.1091 Air induction.

Sec. 25.1093 Induction system icing protection.

Sec. 25.1101 Carburetor air preheater design.

Sec. 25.1103 Induction system ducts and air duct systems.

Sec. 25.1105 Induction system screens.

Sec. 25.1107 Inter-coolers and after-coolers.

Exhaust System

Sec. 25.1121 General.

Sec. 25.1123 Exhaust piping.

Sec. 25.1125 Exhaust heat exchangers.

Sec. 25.1127 Exhaust driven turbo-superchargers.

Powerplant Controls and Accessories

Sec. 25.1141 Powerplant controls: general.

Sec. 25.1142 Auxiliary power unit controls.

Sec. 25.1143 Engine controls.

Sec. 25.1145 Ignition switches.

Sec. 25.1147 Mixture controls.

Sec. 25.1149 Propeller speed and pitch controls.

Sec. 25.1153 Propeller feathering controls.

Sec. 25.1155 Reverse thrust and propeller pitch settings below

the flight regime.

Sec. 25.1157 Carburetor air temperature controls.

Sec. 25.1159 Supercharger controls.

Sec. 25.1161 Fuel jettisoning system controls.

Sec. 25.1163 Powerplant accessories.

Sec. 25.1165 Engine ignition systems.

Sec. 25.1167 Accessory gearboxes.

Powerplant Fire Protection

Sec. 25.1181 Designated fire zones; regions included.

Sec. 25.1182 Nacelle areas behind firewalls, and engine pod

attaching structures containing flammable fluid lines.

Sec. 25.1183 Flammable fluid-carrying components.

Sec. 25.1185 Flammable fluids.

Sec. 25.1187 Drainage and ventilation of fire zones.

Sec. 25.1189 Shutoff means.

Sec. 25.1191 Firewalls.

Sec. 25.1192 Engine accessory section diaphragm.

Sec. 25.1193 Cowling and nacelle skin.

Sec. 25.1195 Fire extinguishing systems.

Sec. 25.1197 Fire extinguishing agents.

Sec. 25.1199 Extinguishing agent containers.

Sec. 25.1201 Fire extinguishing system materials.

Sec. 25.1203 Fire detector system.

Sec. 25.1207 Compliance.

Subpart F--Equipment

General

Sec. 25.1301 Function and installation.

Sec. 25.1303 Flight and navigation instruments.

Sec. 25.1305 Powerplant instruments.

Sec. 25.1307 Miscellaneous equipment.

Sec. 25.1309 Equipment, systems, and installations.

Instruments: Installation

Sec. 25.1321 Arrangement and visibility.

Sec. 25.1322 Warning, caution, and advisory lights.

Sec. 25.1323 Airspeed indicating system.

Sec. 25.1325 Static pressure systems.

Sec. 25.1326 Pitot heat indication systems.

Sec. 25.1327 Magnetic direction indicator.

Sec. 25.1329 Automatic pilot system.

Sec. 25.1331 Instruments using a power supply.

Sec. 25.1333 Instrument systems.

Sec. 25.1335 Flight director systems.

Sec. 25.1337 Powerplant instruments.

Electrical Systems and Equipment

Sec. 25.1351 General.

Sec. 25.1353 Electrical equipment and installations.

Sec. 25.1355 Distribution system.

Sec. 25.1357 Circuit protective devices.

Sec. 25.1359 [Removed. 55 FR 29785, July 20, 1990]

Sec. 25.1363 Electrical system tests.

Lights

Sec. 25.1381 Instrument lights.

Sec. 25.1383 Landing lights.

Sec. 25.1385 Position light system installation.

Sec. 25.1387 Position light system dihedral angles.

Sec. 25.1389 Position light distribution and intensities.

Sec. 25.1391 Minimum intensities in the horizontal plane of

forward and rear position lights.

Sec. 25.1393 Minimum intensities in any vertical plane of

forward and rear position lights.

Sec. 25.1395 Maximum intensities in overlapping beams of

forward and rear position lights.

Sec. 25.1397 Color specifications.

Sec. 25.1399 Riding light.

Sec. 25.1401 Anticollision light system.

Sec. 25.1403 Wing icing detection lights.

Safety Equipment

Sec. 25.1411 General.

Sec. 25.1413 [Removed. 55 FR 29785, July 20, 1990]

Sec. 25.1415 Ditching equipment.@

Sec. 25.1416 [Removed. 55 FR 29785, July 20, 1985]

Sec. 25.1419 Ice protection.

Sec. 25.1421 Megaphones.

Miscellaneous Equipment

Sec. 25.1423 Public address system.

Sec. 25.1431 Electronic equipment.

Sec. 25.1433 Vacuum systems.

Sec. 25.1435 Hydraulic systems.

Sec. 25.1438 Pressurization and pneumatic systems.

Sec. 25.1439 Protective breathing equipment.

Sec. 25.1441 Oxygen equipment and supply.

Sec. 25.1443 Minimum mass flow of supplemental oxygen.

Sec. 25.1445 Equipment standards for the oxygen distributing

system.

Sec. 25.1447 Equipment standards for oxygen dispensing units.

Sec. 25.1449 Means for determining use of oxygen.

Sec. 25.1450 Chemical oxygen generators.

Sec. 25.1451 [Removed. 55 FR 29786, July 20, 1990]

Sec. 25.1453 Protection of oxygen equipment from rupture.

Sec. 25.1455 Draining of fluids subject to freezing.

Sec. 25.1457 Cockpit voice recorders.

Sec. 25.1459 Flight recorders.

Sec. 25.1461 Equipment containing high energy rotors.

Subpart G--Operating Limitations and Information

Sec. 25.1501 General.

Operating Limitations

Sec. 25.1503 Airspeed limitations: general.

Sec. 25.1505 Maximum operating limit speed.

Sec. 25.1507 Maneuvering speed.

Sec. 25.1511 Flap extended speed.

Sec. 25.1513 Minimum control speed.

Sec. 25.1515 Landing gear speeds.

Sec. 25.1519 Weight, center of gravity, and weight

distribution.

Sec. 25.1521 Powerplant limitations.

Sec. 25.1522 Auxiliary power unit limitations.

Sec. 25.1523 Minimum flight crew.

Sec. 25.1525 Kinds of operation.

Sec. 25.1527 Maximum operating altitude.

Sec. 25.1529 Instructions for Continued Airworthiness.

Sec. 25.1531 Maneuvering flight load factors.

Sec. 25.1533 Additional operating limitations.

Markings and Placards

Sec. 25.1541 General.

Sec. 25.1543 Instrument markings: general.

Sec. 25.1545 Airspeed limitation information.

Sec. 25.1547 Magnetic direction indicator.

Sec. 25.1549 Powerplant and auxiliary power unit instruments.

Sec. 25.1551 Oil quantity indication.

Sec. 25.1553 Fuel quantity indicator.

Sec. 25.1555 Control markings.

Sec. 25.1557 Miscellaneous markings and placards.

Sec. 25.1561 Safety equipment.

Sec. 25.1563 Airspeed placard.

Airplane Flight Manual

Sec. 25.1581 General.

Sec. 25.1583 Operating limitations.

Sec. 25.1585 Operating procedures.

Sec. 25.1587 Performance information.

Appendix A

Appendix B

Appendix C to Part 25

Appendix D to Part 25

Appendix E to Part 25

Appendix F to Part 25

Appendix G to Part 25--Continuous Gust Design Criteria

Appendix H to Part 25--Instructions for Continued Airworthiness

Appendix I to Part 25--Installation of an Automatic Takeoff Thrust

Control System (ATTCS)

Appendix J to Part 25--Emergency Demonstration

Special Federal Aviation Regulation No. 13

1. Applicability. Contrary provisions of the Civil Air Regulations

regarding certification notwithstanding,1 this regulation shall provide the

basis for approval by the Administrator of modifications of individual

Douglas DC-3 and Lockheed L-18 airplanes subsequent to the effective date of

this regulation.

NOTE 1 It is not intended to waive compliance with such airworthiness

requirements as are included in the operating parts of the Civil Air

Regulations for specific types of operation.

2. General modifications. Except as modified in sections 3 and 4 of this

regulation, an applicant for approval of modifications to a DC-3 or L-18

airplane which result in changes in design or in changes to approved

limitations shall show that the modifications were accomplished in accordance

with the rules of either Part 4a or Part 4b in effect on September 1, 1953,

which are applicable to the modification being made: Provided, That an

applicant may elect to accomplish a modification in accordance with the rules

of Part 4b in effect on the date of application for the modification in lieu

of Part 4a or Part 4b as in effect on September 1, 1953: And provided

further, That each specific modification must be accomplished in accordance

with all of the provisions contained in the elected rules relating to the

particular modification.

3. Specific conditions for approval. An applicant for any approval of the

following specific changes shall comply with section 2 of this regulation as

modified by the applicable provisions of this section.

(a) Increase in take-off power limitation--1,200 to 1,350 horsepower. The

engine take-off power limitation for the airplane may be increased to more

than 1,200 horsepower but not to more than 1,350 horsepower per engine if the

increase in power does not adversely affect the flight characteristics of the

airplane.

(b) Increase in take-off power limitation to more than 1,350 horsepower.

The engine take-off power limitation for the airplane may be increased to

more than 1,350 horsepower per engine if compliance is shown with the flight

characteristics and ground handling requirements of Part 4b.

(c) Installation of engines of not more than 1,830 cubic inches

displacement and not having a certificated take-off rating of more than 1,350

horsepower. Engines of not more than 1,830 cubic inches displacement and not

having a certificated take-off rating of more than 1,350 horsepower which

necessitate a major modification of redesign of the engine installation may

be installed, if the engine fire prevention and fire protection are

equivalent to that on the prior engine installation.

(d) Installation of engines of more than 1,830 cubic inches displacement or

having certificated take-off rating of more than 1,350 horsepower. Engines of

more than 1,830 cubic inches displacement or having certificated take-off

rating of more than 1,350 horsepower may be installed if compliance is shown

with the engine installation requirements of Part 4b: Provided, That where

literal compliance with the engine installation requirements of Part 4b is

extremely difficult to accomplish and would not contribute materially to the

objective sought, and the Administrator finds that the experience with the

DC-3 or L-18 airplanes justifies it, he is authorized to accept such measures

of compliance as he finds will effectively accomplish the basic objective.

4. Establishment of new maximum certificated weights. An applicant for

approval of new maximum certificated weights shall apply for an amendment of

the airworthiness certificate of the airplane and shall show that the weights

sought have been established, and the appropriate manual material obtained,

as provided in this section.

Note: Transport category performance requirements result in the

establishment of maximum certificated weights for various altitudes.

(a) Weights-25,200 to 26,900 for the DC-3 and 18,500 to 19,500 for the L-

18. New maximum certificated weights of more than 25,200 but not more than

26,900 pounds for DC-3 and more than 18,500 but not more than 19,500 pounds

for L-18 airplanes may be established in accordance with the transport

category performance requirements of either Part 4a or Part 4b, if the

airplane at the new maximum weights can meet the structural requirements of

the elected part.

(b) Weights of more than 26,900 for the DC-3 and 19,500 for the L-18. New

maximum certificated weights of more than 26,900 pounds for DC-3 and 19,500

pounds for L-18 airplanes shall be established in accordance with the

structural performance, flight characteristics, and ground handling

requirements of Part 4b: Provided,  That where literal compliance with the

structural requirements of Part 4b is extremely difficult to accomplish and

would not contribute materially to the objective sought, and the

Administrator finds that the experience with the DC-3 or L-18 airplanes

justifies it, he is authorized to accept such measures of compliance as he

finds will effectively accomplish the basic objective.

(c) Airplane flight manual-performance operating information. An approved

airplane flight manual shall be provided for each DC-3 and L-18 airplane

which has had new maximum certificated weights established under this

section. The airplane flight manual shall contain the applicable performance

information prescribed in that part of the regulations under which the new

certificated weights were established and such additional information as may

be necessary to enable the application of the take-off, en route, and landing

limitations prescribed for transport category airplanes in the operating

parts of the Civil Air Regulations.

(d) Performance operating limitations. Each airplane for which new maximum

certificated weights are established in accordance with paragraphs (a) or (b)

of this section shall be considered a transport category airplane for the

purpose of complying with the performance operating limitations applicable to

the operations in which it is utilized.

5. Reference. Unless otherwise provided, all references in this regulation

to Part 4a and Part 4b are those parts of the Civil Air Regulations in effect

on September 1, 1953.

This regulation supersedes Special Civil Air Regulation SR-398 and shall

remain effective until superseded or rescinded by the Board.

[19 FR 5039, Aug. 11, 1954. Redesignated at 29 FR 19099, Dec. 30, 1964]

Subpart A--General

Sec. 25.1  Applicability.

(a) This part prescribes airworthiness standards for the issue of type

certificates, and changes to those certificates, for transport category

airplanes.

(b) Each person who applies under Part 21 for such a certificate or change

must show compliance with the applicable requirements in this part.

Sec. 25.2  Special retroactive requirements.

The following special retroactive requirements are applicable to an

airplane for which the regulations referenced in the type certificate predate

the sections specified below--

(a) Irrespective of the date of application, each applicant for a

supplemental type certificate (or an amendment to a type certificate)

involving an increase in passenger seating capacity to a total greater than

that for which the airplane has been type certificated must show that the

airplane concerned meets the requirements of:

(1) Sections 25.721(d), 25.783(g), 25.785(c), 25.803(c) (2) through (9),

25.803 (d) and (e), 25.807 (a), (c), and (d), 25.809 (f) and (h), 25.811,

25.812, 25.813 (a), (b), and (c), 25.815, 25.817, 25.853 (a) and (b),

25.855(a), 25.993(f), and 25.1359(c) in effect on October 24, 1967, and

(2) Sections 25.803(b) and 25.803(c)(1) in effect on April 23, 1969.

(b) Irrespective of the date of application, each applicant for a

supplemental type certificate (or an amendment to a type certificate) for an

airplane manufactured after October 16, 1987, must show that the airplane

meets the requirements of Sec. 25.807(c)(7) in effect on July 24, 1989.

(c) Compliance with subsequent revisions to the sections specified in

paragraph (a) or (b) above may be elected in accordance with Sec.

21.101(a)(2) of this chapter or may be required in accordance with Sec.

21.101(b) of this chapter.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29773, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Subpart B--Flight

General

Sec. 25.21  Proof of compliance.

(a) Each requirement of this subpart must be met at each appropriate

combination of weight and center of gravity within the range of loading

conditions for which certification is requested. This must be shown--

(1) By tests upon an airplane of the type for which certification is

requested, or by calculations based on, and equal in accuracy to, the results

of testing; and

(2) By systematic investigation of each probable combination of weight and

center of gravity, if compliance cannot be reasonably inferred from

combinations investigated.

(b) [Reserved]

(c) The controllability, stability, trim, and stalling characteristics of

the airplane must be shown for each altitude up to the maximum expected in

operation.

(d) Parameters critical for the test being conducted, such as weight,

loading (center of gravity and inertia), airspeed, power, and wind, must be

maintained within acceptable tolerances of the critical values during flight

testing.

(e) If compliance with the flight characteristics requirements is dependent

upon a stability augmentation system or upon any other automatic or power-

operated system, compliance must be shown with Secs. 25.671 and 25.672.

(f) In meeting the requirements of Secs. 25.105(d), 25.125, 25.233, and

25.237, the wind velocity must be measured at a height of 10 meters above the

surface, or corrected for the difference between the height at which the wind

velocity is measured and the 10-meter height.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5671, Apr. 8, 1970; Amdt. 25-42, 43 FR 2320, Jan. 16, 1978; Amdt. 25-72, 55

FR 29774, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.23  Load distribution limits.

(a) Ranges of weights and centers of gravity within which the airplane may

be safely operated must be established. If a weight and center of gravity

combination is allowable only within certain load distribution limits (such

as spanwise) that could be inadvertently exceeded, these limits and the

corresponding weight and center of gravity combinations must be established.

(b) The load distribution limits may not exceed--

(1) The selected limits;

(2) The limits at which the structure is proven; or

(3) The limits at which compliance with each applicable flight requirement

of this subpart is shown.

Sec. 25.25  Weight limits.

(a) Maximum weights. Maximum weights corresponding to the airplane

operating conditions (such as ramp, ground or water taxi, takeoff, en route,

and landing), environmental conditions (such as altitude and temperature),

and loading conditions (such as zero fuel weight, center of gravity position

and weight distribution) must be established so that they are not more than--

(1) The highest weight selected by the applicant for the particular

conditions; or

(2) The highest weight at which compliance with each applicable structural

loading and flight requirement is shown, except that for airplanes equipped

with standby power rocket engines the maximum weight must not be more than

the highest weight established in accordance with Appendix E of this part; or

(3) The highest weight at which compliance is shown with the certification

requirements of Part 36 of this chapter.

(b) Minimum weight. The minimum weight (the lowest weight at which

compliance with each applicable requirement of this part is shown) must be

established so that it is not less than--

(1) The lowest weight selected by the applicant;

(2) The design minimum weight (the lowest weight at which compliance with

each structural loading condition of this part is shown); or

(3) The lowest weight at which compliance with each applicable flight

requirement is shown.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5671, Apr. 8, 1970; Amdt. 25-63, 53 FR 16365, May 6, 1988]

Sec. 25.27  Center of gravity limits.

The extreme forward and the extreme aft center of gravity limitations must

be established for each practicably separable operating condition. No such

limit may lie beyond--

(a) The extremes selected by the applicant;

(b) The extremes within which the structure is proven; or

(c) The extremes within which compliance with each applicable flight

requirement is shown.

Sec. 25.29  Empty weight and corresponding center of gravity.

(a) The empty weight and corresponding center of gravity must be determined

by weighing the airplane with--

(1) Fixed ballast;

(2) Unusable fuel determined under Sec. 25.959; and

(3) Full operating fluids, including--

(i) Oil;

(ii) Hydraulic fluid; and

(iii) Other fluids required for normal operation of airplane systems,

except potable water, lavatory precharge water, and fluids intended for

injection in the engine.

(b) The condition of the airplane at the time of determining empty weight

must be one that is well defined and can be easily repeated.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-42, 43 FR

2320, Jan. 16, 1978; Amdt. 25-72, 55 FR 29774, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.31  Removable ballast.

Removable ballast may be used on showing compliance with the flight

requirements of this subpart.

Sec. 25.33  Propeller speed and pitch limits.

(a) The propeller speed and pitch must be limited to values that will

ensure-

(1) Safe operation under normal operating conditions; and

(2) Compliance with the performance requirements of Secs. 25.101 through

25.125.

(b) There must be a propeller speed limiting means at the governor. It must

limit the maximum possible governed engine speed to a value not exceeding the

maximum allowable r.p.m.

(c) The means used to limit the low pitch position of the propeller blades

must be set so that the engine does not exceed 103 percent of the maximum

allowable engine rpm or 99 percent of an approved maximum overspeed,

whichever is greater, with--

(1) The propeller blades at the low pitch limit and governor inoperative;

(2) The airplane stationary under standard atmospheric conditions with no

wind; and

(3) The engines operating at the takeoff manifold pressure limit for

reciprocating engine powered airplanes or the maximum takeoff torque limit

for turbopropeller engine-powered airplanes.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-57, 49 FR

6848, Feb. 23, 1984; Amdt. 25-72, 55 FR 29774, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Performance

Sec. 25.101  General.

(a) Unless otherwise prescribed, airplanes must meet the applicable

performance requirements of this subpart for ambient atmospheric conditions

and still air.

(b) The performance, as affected by engine power or thrust, must be based

on the following relative humidities;

(1) For turbine engine powered airplanes, a relative humidity of--

(i) 80 percent, at and below standard temperatures; and

(ii) 34 percent, at and above standard temperatures plus 50 deg. F.

Between these two temperatures, the relative humidity must vary linearly.

(2) For reciprocating engine powered airplanes, a relative humidity of 80

percent in a standard atmosphere. Engine power corrections for vapor pressure

must be made in accordance with the following table:

Specific

humidity

Vapor     w (Lb.

pressure  moisture

Altitude   e (In.   per lb.        Density ratio

H (ft.)     Hg.)    dry air)  <rho>/<sigma>=0.0023769

0     0.403   0.00849                  0.99508

1,000      .354    .00773                   .96672

2,000      .311    .00703                   .93895

3,000      .272    .00638                   .91178

4,000      .238    .00578                   .88514

5,000      .207    .00523                   .85910

6,000     .1805    .00472                   .83361

7,000     .1566    .00425                   .80870

8,000     .1356    .00382                   .78434

9,000     .1172    .00343                   .76053

10,000     .1010    .00307                   .73722

15,000     .0463   .001710                   .62868

20,000    .01978   .000896                   .53263

25,000    .00778   .000436                   .44806

(c) The performance must correspond to the propulsive thrust available

under the particular ambient atmospheric conditions, the particular flight

condition, and t@ relative humidity specified in paragraph (b) of this

section. The available propulsive thrust must correspond to engine power or

thrust, not exceeding the approved power or thrust less--

(1) Installation losses; and

(2) The power or equivalent thrust absorbed by the accessories and services

appropriate to the particular ambient atmospheric conditions and the

particular flight condition.

(d) Unless otherwise prescribed, the applicant must select the takeoff, en

route, approach, and landing configurations for the airplane.

(e) The airplane configurations may vary with weight, altitude, and

temperature, to the extent they are compatible with the operating procedures

required by paragraph (f) of this section.

(f) Unless otherwise prescribed, in determining the accelerate-stop

distances, takeoff flight paths, takeoff distances, and landing distances,

changes in the airplane's configuration, speed, power, and thrust, must be

made in accordance with procedures established by the applicant for operation

in service.

(g) Procedures for the execution of balked landings and missed approaches

associated with the conditions prescribed in Secs. 25.119 and 25.121(d) must

be established.

(h) The procedures established under paragraphs (f) and (g) of this section

must--

(1) Be able to be consistently executed in service by crews of average

skill;

(2) Use methods or devices that are safe and reliable; and

(3) Include allowance for any time delays, in the execution of the

procedures, that may reasonably be expected in service.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55466, Dec. 20, 1976]

Sec. 25.103  Stalling speed.

(a) VS is the calibrated stalling speed, or the minimum steady flight

speed, in knots, at which the airplane is controllable, with--

(1) Zero thrust at the stalling speed, or, if the resultant thrust has no

appreciable effect on the stalling speed, with engines idling and throttles

closed;

(2) Propeller pitch controls (if applicable) in the position necessary for

compliance with paragraph (a)(1) of this section and the airplane in other

respects (such as flaps and landing gear) in the condition existing in the

test in which VS is being used;

(3) The weight used when VS is being used as a factor to determine

compliance with a required performance standard; and

(4) The most unfavorable center of gravity allowable.

(b) The stalling speed VS is the minimum speed obtained as follows:

(1) Trim the airplane for straight flight at any speed not less than 1.2 VS

or more than 1.4 VS At a speed sufficiently above the stall speed to ensure

steady conditions, apply the elevator control at a rate so that the airplane

speed reduction does not exceed one knot per second.

(2) Meet the flight characteristics provisions of Sec. 25.203.

Sec. 25.105  Takeoff.

(a) The takeoff speeds described in Sec. 25.107, the accelerate-stop

distance described in Sec. 25.109, the takeoff path described in Sec. 25.111,

and the takeoff distance and takeoff run described in Sec. 25.113, must be

determined--

(1) At each weight, altitude, and ambient temperature within the

operational limits selected by the applicant; and

(2) In the selected configuration for takeoff.

(b) No takeoff made to determine the data required by this section may

require exceptional piloting skill or alertness.

(c) The takeoff data must be based on--

(1) A smooth, dry, hard-surfaced runway, in the case of land planes and

amphibians;

(2) Smooth water, in the case of seaplanes and amphibians; and

(3) Smooth, dry snow, in the case of skiplanes.

(d) The takeoff data must include, within the established operational

limits of the airplane, the following operational correction factors:

(1) Not more than 50 percent of nominal wind components along the takeoff

path opposite to the direction of takeoff, and not less than 150 percent of

nominal wind components along the takeoff path in the direction of takeoff.

(2) Effective runway gradients.

Sec. 25.107  Takeoff speeds.

(a) V1 must be established in relation to VEF as follows:

(1) VEF is the calibrated airspeed at which the critical engine is assumed

to fail. VEF must be selected by the applicant, but may not be less than VmcG

determined under Sec. 25.149(e).

(2) V1, in terms of calibrated airspeed, is the takeoff decision speed

selected by the applicant; however, V1 may not be less than VEF plus the

speed gained with the critical engine inoperative during the time interval

between the instant at which the critical engine is failed, and the instant

at which the pilot recognizes and reacts to the engine failure, as indicated

by the pilot's application of the first retarding means during accelerate-

stop tests.

(b) V2MIN, in terms of calibrated airspeed, may not be less than--

(1) 1.2 VS for--

(i) Two-engine and three-engine turbopropeller and reciprocating engine

powered airplanes; and

(ii) Turbojet powered airplanes without provisions for obtaining a

significant reduction in the one-engine-inoperative power-on stalling speed;

(2) 1.15 VS for--

(i) Turbopropeller and reciprocating engine powered airplanes with more

than three engines; and

(ii) Turbojet powered airplanes with provisions for obtaining a significant

reduction in the one-engine-inoperative power-on stalling speed; and

(3) 1.10 times VMC established under Sec. 25.149.

(c) V2, in terms of calibrated airspeed, must be selected by the applicant

to provide at least the gradient of climb required by Sec. 25.121(b) but may

not be less than--

(1) V2MIN, and

(2) VR plus the speed increment attained (in accordance with Sec. 25.111

(c)(2)) before reaching a height of 35 feet above the takeoff surface.

(d) VMU is the calibrated airspeed at and above which the airplane can

safely lift off the ground, and continue the takeoff. VMU speeds must be

selected by the applicant throughout the range of thrust-to-weight ratios to

be certificated. These speeds may be established from free air data if these

data are verified by ground takeoff tests.

(e) VR, in terms of calibrated airspeed, must be selected in accordance

with the conditions of paragraphs (e) (1) through (4) of this section:

(1) VR may not be less than--

(i) V1;

(ii) 105 percent of VMC;

(iii) The speed (determined in accordance with Sec. 25.111(c)(2)) that

allows reaching V2 before reaching a height of 35 feet above the takeoff

surface; or

(iv) A speed that, if the airplane is rotated at its maximum practicable

rate, will result in a VLOF of not less than 110 percent of VMU in the all-

engines-operating condition and not less than 105 percent of VMU determined

at the thrust-to-weight ratio corresponding to the one-engine-inoperative

condition.

(2) For any given set of conditions (such as weight, configuration, and

temperature), a single value of VR, obtained in accordance with this

paragraph, must be used to show compliance with both the one-engine-

inoperative and the all-engines-operating takeoff provisions.

(3) It must be shown that the one-engine-inoperative takeoff distance,

using a rotation speed of 5 knots less than VR established in accordance with

paragraphs (e)(1) and (2) of this section, does not exceed the corresponding

one-engine-inoperative takeoff distance using the established VR. The takeoff

distances must be determined in accordance with Sec. 25.113(a)(1).

(4) Reasonably expected variations in service from the established takeoff

procedures for the operation of the airplane (such as over-rotation of the

airplane and out-of-trim conditions) may not result in unsafe flight

characteristics or in marked increases in the scheduled takeoff distances

established in accordance with Sec. 25.113(a).

(f) VLOF is the calibrated airspeed at which the airplane first becomes

airborne.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55466, Dec. 20, 1976; Amdt. 25-42, 43 FR 2320, Jan. 16, 1978]

Sec. 25.109  Accelerate-stop distance.

(a) The accelerate-stop distance is the greater of the following distances:

(1) The sum of the distances necessary to--

(i) Accelerate the airplane from a standing start to VEF with all engines

operating;

(ii) Accelerate the airplane from VEF to V1 and continue the acceleration

for 2.0 seconds after V1 is reached, assuming the critical engine fails at

VEF; and

(iii) Come to a full stop from the point reached at the end of the

acceleration period prescribed in paragraph (a)(1)(ii) of this section,

assuming that the pilot does not apply any means of retarding the airplane

until that point is reached and that the critical engine is still

inoperative.

(2) The sum of the distances necessary to--

(i) Accelerate the airplane from a standing start to V1 and continue the

acceleration for 2.0 seconds after V1 is reached with all engines operating;

and

(ii) Come to a full stop from the point reached at the end of the

acceleration period prescribed in paragraph (a)(2)(i) of this section,

assuming that the pilot does not apply any means of retarding the airplane

until that point is reached and that all engines are still operating.

(b) Means other than wheel brakes may be used to determine the accelerate-

stop distance if that means--

(1) Is safe and reliable;

(2) Is used so that consistent results can be expected under normal

operating conditions; and

(3) Is such that exceptional skill is not required to control the airplane.

(c) The landing gear must remain extended throughout the accelerate-stop

distance.

(d) If the accelerate-stop distance includes a stopway with surface

characteristics substantially different from those of a smooth hard-surfaced

runway, the takeoff data must include operational correction factors for the

accelerate-stop distance. The correction factors must account for the

particular surface characteristics of the stopway and the variations in these

characteristics with seasonal weather conditions (such as temperature, rain,

snow, and ice) within the established operational limits.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-42, 43 FR

2321, Jan. 16, 1978]

Sec. 25.111  Takeoff path.

(a) The takeoff path extends from a standing start to a point in the

takeoff at which the airplane is 1,500 feet above the takeoff surface, or at

which the transition from the takeoff to the en route configuration is

completed and a speed is reached at which compliance with Sec. 25.121(c) is

shown, whichever point is higher. In addition--

(1) The takeoff path must be based on the procedures prescribed in Sec.

25.101(f);

(2) The airplane must be accelerated on the ground to VEF, at which point

the critical engine must be made inoperative and remain inoperative for the

rest of the takeoff; and

(3) After reaching VEF, the airplane must be accelerated to V2.

(b) During the acceleration to speed V2, the nose gear may be raised off

the ground at a speed not less than VR. However, landing gear retraction may

not be begun until the airplane is airborne.

(c) During the takeoff path determination in accordance with paragraphs (a)

and (b) of this section--

(1) The slope of the airborne part of the takeoff path must be positive at

each point;

(2) The airplane must reach V2 before it is 35 feet above the takeoff

surface and must continue at a speed as close as practical to, but not less

than V2, until it is 400 feet above the takeoff surface;

(3) At each point along the takeoff path, starting at the point at which

the airplane reaches 400 feet above the takeoff surface, the available

gradient of climb may not be less than--

(i) 1.2 percent for two-engine airplanes;

(ii) 1.5 percent for three-engine airplanes; and

(iii) 1.7 percent for four-engine airplanes; and

(4) Except for gear retraction and propeller feathering, the airplane

configuration may not be changed, and no change in power or thrust that

requires action by the pilot may be made, until the airplane is 400 feet

above the takeoff surface.

(d) The takeoff path must be determined by a continuous demonstrated

takeoff or by synthesis from segments. If the takeoff path is determined by

the segmental method--

(1) The segments must be clearly defined and must be related to the

distinct changes in the configuration, power or thrust, and speed;

(2) The weight of the airplane, the configuration, and the power or thrust

must be constant throughout each segment and must correspond to the most

critical condition prevailing in the segment;

(3) The flight path must be based on the airplane's performance without

ground effect; and

(4) The takeoff path data must be checked by continuous demonstrated

takeoffs up to the point at which the airplane is out of ground effect and

its speed is stabilized, to ensure that the path is conservative relative to

the continous path.

The airplane is considered to be out of the ground effect when it reaches a

height equal to its wing span.

(e) For airplanes equipped with standby power rocket engines, the takeoff

path may be determined in accordance with section II of Appendix E.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-6, 30 FR

8468, July 2, 1965; Amdt. 25-42, 43 FR 2321, Jan. 16, 1978; Amdt. 25-54, 45

FR 60172, Sept. 11, 1980; Amdt. 25-72, 55 FR 29774, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.113  Takeoff distance and takeoff run.

(a) Takeoff distance is the greater of--

(1) The horizontal distance along the takeoff path from the start of the

takeoff to the point at which the airplane is 35 feet above the takeoff

surface, determined under Sec. 25.111; or

(2) 115 percent of the horizontal distance along the takeoff path, with all

engines operating, from the start of the takeoff to the point at which the

airplane is 35 feet above the takeoff surface, as determined by a procedure

consistent with Sec. 25.111.

(b) If the takeoff distance includes a clearway, the takeoff run is the

greater of--

(1) The horizontal distance along the takeoff path from the start of the

takeoff to a point equidistant between the point at which VLOF is reached and

the point at which the airplane is 35 feet above the takeoff surface, as

determined under Sec. 25.111; or

(2) 115 percent of the horizontal distance along the takeoff path, with all

engines operating, from the start of the takeoff to a point equidistant

between the point at which VLOF is reached and the point at which the

airplane is 35 feet above the takeoff surface, determined by a procedure

consistent with Sec. 25.111.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5671, Apr. 8, 1970]

Sec. 25.115  Takeoff flight path.

(a) The takeoff flight path begins 35 feet above the takeoff surface at the

end of the takeoff distance determined in accordance with Sec. 25.113(a).

(b) The net takeoff flight path data must be determined so that they

represent the actual takeoff flight paths (determined in accordance with Sec.

25.111 and with paragraph (a) of this section) reduced at each point by a

gradient of climb equal to--

(1) 0.8 percent for two-engine airplanes;

(2) 0.9 percent for three-engine airplanes; and

(3) 1.0 percent for four-engine airplanes.

(c) The prescribed reduction in climb gradient may be applied as an

equivalent reduction in acceleration along that part of the takeoff flight

path at which the airplane is accelerated in level flight.

Sec. 25.117  Climb: general.

Compliance with the requirements of Secs. 25.119 and 25.121 must be shown

at each weight, altitude, and ambient temperature within the operational

limits established for the airplane and with the most unfavorable center of

gravity for each configuration.

Sec. 25.119  Landing climb: All-engine-operating.

In the landing configuration, the steady gradient of climb may not be less

than 3.2 percent, with--

(a) The engines at the power or thrust that is available eight seconds

after initiation of movement of the power or thrust controls from the minimum

flight idle to the takeoff position; and

(b) A climb speed of not more than 1.3 VS.

Sec. 25.121  Climb: One-engine-inoperative.

(a) Takeoff; landing gear extended.  In the critical takeoff configuration

existing along the flight path (between the points at which the airplane

reaches VLOF and at which the landing gear is fully retracted) and in the

configuration used in Sec. 25.111 but without ground effect, the steady

gradient of climb must be positive for two-engine airplanes, and not less

than 0.3 percent for three-engine airplanes or 0.5 percent for four-engine

airplanes, at VLOF and with--

(1) The critical engine inoperative and the remaining engines at the power

or thrust available when retraction of the landing gear is begun in

accordance with Sec. 25.111 unless there is a more critical power operating

condition existing later along the flight path but before the point at which

the landing gear is fully retracted; and

(2) The weight equal to the weight existing when retraction of the landing

gear is begun, determined under Sec. 25.111.

(b) Takeoff; landing gear retracted.  In the takeoff configuration existing

at the point of the flight path at which the landing gear is fully retracted,

and in the configuration used in Sec. 25.111 but without ground effect, the

steady gradient of climb may not be less than 2.4 percent for two-engine

airplanes, 2.7 percent for three-engine airplanes, and 3.0 percent for four-

engine airplanes, at V2 and with--

(1) The critical engine inoperative, the remaining engines at the takeoff

power or thrust available at the time the landing gear is fully retracted,

determined under Sec. 25.111, unless there is a more critical power operating

condition existing later along the flight path but before the point where the

airplane reaches a height of 400 feet above the takeoff surface; and

(2) The weight equal to the weight existing when the airplane's landing

gear is fully retracted, determined under Sec. 25.111.

(c) Final takeoff. In the en route configuration at the end of the takeoff

path determined in accordance with Sec. 25.111, the steady gradient of climb

may not be less than 1.2 percent for two-engine airplanes, 1.5 percent for

three-engine airplanes, and 1.7 percent for four-engine airplanes, at not

less than 1.25 VS and with--

(1) The critical engine inoperative and the remaining engines at the

available maximum continuous power or thrust; and

(2) The weight equal to the weight existing at the end of the takeoff path,

determined under Sec. 25.111.

(d) Approach. In the approach configuration corresponding to the normal

all-engines-operating procedure in which VS for this configuration does not

exceed 110 percent of the VS for the related landing configuration, the

steady gradient of climb may not be less than 2.1 percent for two-engine

airplanes, 2.4 percent for three-engine airplanes, and 2.7 percent for four-

engine airplanes, with--

(1) The critical engine inoperative, the remaining engines at the available

takeoff power or thrust;

(2) The maximum landing weight; and

(3) A climb speed established in connection with normal landing procedures,

but not exceeding 1.5 VS.

Sec. 25.123  En route flight paths.

(a) For the en route configuration, the flight paths prescribed in

paragraphs (b) and (c) of this section must be determined at each weight,

altitude, and ambient temperature, within the operating limits established

for the airplane. The variation of weight along the flight path, accounting

for the progressive consumption of fuel and oil by the operating engines, may

be included in the computation. The flight paths must be determined at any

selected speed, with--

(1) The most unfavorable center of gravity;

(2) The critical engines inoperative;

(3) The remaining engines at the available maximum continuous power or

thrust; and

(4) The means for controlling the engine-cooling air supply in the position

that provides adequate cooling in the hot-day condition.

(b) The one-engine-inoperative net flight path data must represent the

actual climb performance diminished by a gradient of climb of 1.1 percent for

two-engine airplanes, 1.4 percent for three-engine airplanes, and 1.6 percent

for four-engine airplanes.

(c) For three- or four-engine airplanes, the two-engine-inoperative net

flight path data must represent the actual climb performance diminished by a

gradient of climb of 0.3 percent for three-engine airplanes and 0.5 percent

for four-engine airplanes.

Sec. 25.125  Landing.

(a) The horizontal distance necessary to land and to come to a complete

stop (or to a speed of approximately 3 knots for water landings) from a point

50 feet above the landing surface must be determined (for standard

temperatures, at each weight, altitude, and wind within the operational

limits established by the applicant for the airplane) as follows:

(1) The airplane must be in the landing configuration.

(2) A stabilized approach, with a calibrated airspeed of not less than

1.3 VS,  must be maintained down to the 50 foot height.

(3) Changes in configuration, power or thrust, and speed, must be made in

accordance with the established procedures for service operation.

(4) The landing must be made without excessive vertical acceleration,

tendency to bounce, nose over, ground loop, porpoise, or water loop.

(5) The landings may not require exceptional piloting skill or alertness.

(b) For landplanes and amphibians, the landing distance on land must be

determined on a level, smooth, dry, hard-surfaced runway. In addition--

(1) The pressures on the wheel braking systems may not exceed those

specified by the brake manufacturer;

(2) The brakes may not be used so as to cause excessive wear of brakes or

tires; and

(3) Means other than wheel brakes may be used if that means--

(i) Is safe and reliable;

(ii) Is used so that consistent results can be expected in service; and

(iii) Is such that exceptional skill is not required to control the

airplane.

(c) For seaplanes and amphibians, the landing distance on water must be

determined on smooth water.

(d) For skiplanes, the landing distance on snow must be determined on

smooth, dry, snow.

(e) The landing distance data must include correction factors for not more

than 50 percent of the nominal wind components along the landing path

opposite to the direction of landing, and not less than 150 percent of the

nominal wind components along the landing path in the direction of landing.

(f) If any device is used that depends on the operation of any engine, and

if the landing distance would be noticeably increased when a landing is made

with that engine inoperative, the landing distance must be determined with

that engine inoperative unless the use of compensating means will result in a

landing distance not more than that with each engine operating.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55 FR

29774, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Controllability and Maneuverability

Sec. 25.143  General.

(a) The airplane must be safely controllable and maneuverable during--

(1) Takeoff;

(2) Climb;

(3) Level flight;

(4) Descent; and

(5) Landing.

(b) It must be possible to make a smooth transition from one flight

condition to any other flight condition without exceptional piloting skill,

alertness, or strength, and without danger of exceeding the airplane limit-

load factor under any probable operating conditions, including--

(1) The sudden failure of the critical engine;

(2) For airplanes with three or more engines, the sudden failure of the

second critical engine when the airplane is in the en route, approach, or

landing configuration and is trimmed with the critical engine inoperative;

and

(3) Configuration changes, including deployment or retraction of

deceleration devices.

(c) If, during the testing required by paragraphs (a) and (b) of this

section, marginal conditions exist with regard to required pilot strength,

the "strength of pilots" limits may not exceed the limits prescribed in the

following table:

Values in pound of force

as applied to the control

wheel or rudder pedals    Pitch  Roll  Yaw

For temporary application     75    60  150

For prolonged application     10     5   20

(d) In showing the temporary control force limitations of paragraph (c) of

this section, approved operating procedures or conventional operating

practices must be followed (including being as nearly trimmed as possible at

the next preceding steady flight condition, except that, in the case of

takeoff, the airplane must be trimmed in accordance with approved operating

procedures).

(e) For the purpose of complying with the prolonged control force

limitations of paragraph (c) of this section, the airplane must be as nearly

trimmed as possible.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-42, 43 FR

2321, Jan. 16, 1978]

Sec. 25.145  Longitudinal control.

(a) It must be possible at any speed between the trim speed prescribed in

Sec. 25.103(b)(1) and Vs, to pitch the nose downward so that the acceleration

to this selected trim speed is prompt with--

(1) The airplane trimmed at the trim speed prescribed in Sec. 25.103(b)(1).

(2) The landing gear extended;

(3) The wing flaps (i) retracted and (ii) extended; and

(4) Power (i) off and (ii) at maximum continuous power on the engines.

(b) With the landing gear extended, no change in trim control, or exertion

of more than 50 pounds control force (representative of the maximum temporary

force that readily can be applied by one hand) may be required for the

following maneuvers:

(1) With power off, flaps retracted, and the airplane trimmed at 1.4 VS1,

extend the flaps as rapidly as possible while maintaining the airspeed at

approximately 40 percent above the stalling speed existing at each instant

throughout the maneuver.

(2) Repeat paragraph (b)(1) except initially extend the flaps and then

retract them as rapidly as possible.

(3) Repeat paragraph (b)(2) except with takeoff power.

(4) With power off, flaps retracted, and the airplane trimmed at 1.4 VS1,

apply takeoff power rapidly while maintaining the same airspeed.

(5) Repeat paragraph (b)(4) except with flaps extended.

(6) With power off, flaps extended, and the airplane trimmed at 1.4 VS1,

obtain and maintain airspeeds between 1.1 VS1,  and either 1.7 VS1,  or VFE,

whichever is lower.

(c) Is must be possible, without exceptional piloting skill, to prevent

loss of altitude when complete retraction of the high lift devices from any

position is begun during steady, straight, level flight at 1.1 VS1 for

propeller powered airplanes, or 1.2 VS1 for turbojet powered airplanes,

with--

(1) Simultaneous application of not more than takeoff power taking into

account the critical engine operating conditions;

(2) The landing gear extended; and

(3) The critical combinations of landing weights and altitudes.

If gated high-lift device control positions are provided, retraction must be

shown from any position from the maximum landing position to the first gated

position, between gated positions, and from the last gated position to the

full retraction position. In addition, the first gated control position from

the landing position must correspond with the high-lift devices configuration

used to establish the go-around procedure from the landing configuration.

Each gated control position must require a separate and distinct motion of

the control to pass through the gated position and must have features to

prevent inadvertent movement of the control through the gated position.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5671, Apr. 8, 1970; Amdt. 25-72, 55 FR 29774, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.147  Directional and lateral control.

(a) Directional control; general. It must be possible, with the wings

level, to yaw into the operative engine and to safely make a reasonably

sudden change in heading of up to 15 degrees in the direction of the critical

inoperative engine. This must be shown at 1.4Vs1 for heading changes up to 15

degrees (except that the heading change at which the rudder pedal force is

150 pounds need not be exceeded), and with--

(1) The critical engine inoperative and its propeller in the minimum drag

position;

(2) The power required for level flight at 1.4 VS1,  but not more than

maximum continuous power;

(3) The most unfavorable center of gravity;

(4) Landing gear retracted;

(5) Flaps in the approach position; and

(6) Maximum landing weight.

(b) Directional control; airplanes with four or more engines. Airplanes

with four or more engines must meet the requirements of paragraph (a) of this

section except that--

(1) The two critical engines must be inoperative with their propellers (if

applicable) in the minimum drag position;

(2) [Reserved]

(3) The flaps must be in the most favorable climb position.

(c) Lateral control; general. It must be possible to make 20 deg. banked

turns, with and against the inoperative engine, from steady flight at a speed

equal to 1.4 VS1,  with--

(1) The critical engine inoperative and its propeller (if applicable) in

the minimum drag position;

(2) The remaining engines at maximum continuous power;

(3) The most unfavorable center of gravity;

(4) Landing gear (i) retracted and (ii) extended;

(5) Flaps in the most favorable climb position; and

(6) Maximum takeoff weight.

(d) Lateral control; airplanes with four or more engines. Airplanes with

four or more engines must be able to make 20 deg. banked turns, with and

against the inoperative engines, from steady flight at a speed equal to 1.4

VS1,  with maximum continuous power, and with the airplane in the

configuration prescribed by paragraph (b) of this section.

(e) Lateral control; all engines operating.  With the engines operating,

roll response must allow normal maneuvers (such as recovery from upsets

produced by gusts and the initiation of evasive maneuvers). There must be

enough excess lateral control in sideslips (up to sideslip angles that might

be required in normal operation), to allow a limited amount of maneuvering

and to correct for gusts. Lateral control must be enough at any speed up to

VFC/MFC to provide a peak roll rate necessary for safety, without excessive

control forces or travel.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-42, 43 FR

2321, Jan. 16, 1978; Amdt. 25-72, 55 FR 29774, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.149  Minimum control speed.

(a) In establishing the minimum control speeds required by this section,

the method used to simulate critical engine failure must represent the most

critical mode of powerplant failure with respect to controllability expected

in service.

(b) VMC is the calibrated airspeed at which, when the critical engine is

suddenly made inoperative, it is possible to maintain control of the airplane

with that engine still inoperative and maintain straight flight with an angle

of bank of not more than 5 degrees.

(c) VMC may not exceed 1.2 VS with--

(1) Maximum available takeoff power or thrust on the engines;

(2) The most unfavorable center of gravity;

(3) The airplane trimmed for takeoff;

(4) The maximum sea level takeoff weight (or any lesser weight necessary to

show VMC);

(5) The airplane in the most critical takeoff configuration existing along

the flight path after the airplane becomes airborne, except with the landing

gear retracted;

(6) The airplane airborne and the ground effect negligible; and

(7) If applicable, the propeller of the inoperative engine--

(i) Windmilling;

(ii) In the most probable position for the specific design of the propeller

control; or

(iii) Feathered, if the airplane has an automatic feathering device

acceptable for showing compliance with the climb requirements of Sec. 25.121.

(d) The rudder forces required to maintain control at VMC may not exceed

150 pounds nor may it be necessary to reduce power or thrust of the operative

engines. During recovery, the airplane may not assume any dangerous attitude

or require exceptional piloting skill, alertness, or strength to prevent a

heading change of more than 20 degrees.

(e) VMCG, the minimum control speed on the ground, is the calibrated

airspeed during the takeoff run at which, when the critical engine is

suddenly made inoperative, it is possible to maintain control of the airplane

using the rudder control alone (without the use of nosewheel steering), as

limited by 150 pounds of force, and the lateral control to the extent of

keeping the wings level to enable the takeoff to be safely continued using

normal piloting skill. In the determination of VMCG, assuming that the path

of the airplane accelerating with all engines operating is along the

centerline of the runway, its path from the point at which the critical

engine is made inoperative to the point at which recovery to a direction

parallel to the centerline is completed may not deviate more than 30 feet

laterally from the centerline at any point. VMCG must be established with--

(1) The airplane in each takeoff configuration or, at the option of the

applicant, in the most critical takeoff configuration;

(2) Maximum available takeoff power or thrust on the operating engines;

(3) The most unfavorable center of gravity;

(4) The airplane trimmed for takeoff; and

(5) The most unfavorable weight in the range of takeoff weights.

(f) VMCL, the minimum control speed during landing approach with all

engines operating, is the calibrated airspeed at which, when the critical

engine is suddenly made inoperative, it is possible to maintain control of

the airplane with that engine still inoperative and maintain straight flight

with an angle of bank of not more than 5 degrees. VMCL must be established

with--

(1) The airplane in the most critical configuration for approach with all

engines operating;

(2) The most unfavorable center of gravity;

(3) The airplane trimmed for approach with all engines operating;

(4) The maximum sea level landing weight (or any lesser weight necessary to

show VMCL); and

(5) Maximum available takeoff power or thrust on the operating engines.

(g) For airplanes with three or more engines, VMCL-2, the minimum control

speed during landing approach with one critical engine inoperative, is the

calibrated airspeed at which, when a second critical engine is suddenly made

inoperative, it is possible to maintain control of the airplane with both

engines still inoperative and maintain straight flight with an angle of bank

of not more than 5 degrees. VMCL-2 must be established with--

(1) The airplane in the most critical configuration for approach with the

critical engine inoperative;

(2) The most unfavorable center of gravity;

(3) The airplane trimmed for approach with the critical engine inoperative;

(4) The maximum sea level landing weight (or any lesser weight necessary to

show VMCL-2);

(5) The power or thrust on the operating engines required to maintain an

approach path angle of 3 degrees when one critical engine is inoperative; and

(6) The power or thrust on the operating engines rapidly changed,

immediately after the second critical engine is made inoperative, from the

power or thrust prescribed in paragraph (g)(5) of this section to--

(i) Minimum available power or thrust; and

(ii) Maximum available takeoff power or thrust.

(h) The rudder control forces required to maintain control at VMCL and

VMCL-2 may not exceed 150 pounds, nor may it be necessary to reduce the power

or thrust of the operating engines. In addition, the airplane may not assume

any dangerous attitudes or require exceptional piloting skill, alertness, or

strength to prevent a divergence in the approach flight path that would

jeopardize continued safe approach when--

(1) The critical engine is suddenly made inoperative; and

(2) For the determination of VMCL-2, the power or thrust on the operating

engines is changed in accordance with paragraph (g)(6) of this section.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-42, 43 FR

2321, Jan. 16, 1978; Amdt. 25-72, 55 FR 29774, July 20, 1990; 55 FR 37607,

Sept. 12, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Trim

Sec. 25.161  Trim.

(a) General. Each airplane must meet the trim requirements of this section

after being trimmed, and without further pressure upon, or movement of,

either the primary controls or their corresponding trim controls by the pilot

or the automatic pilot.

(b) Lateral and directional trim. The airplane must maintain lateral and

directional trim with the most adverse lateral displacement of the center of

gravity within the relevant operating limitations, during normally expected

conditions of operation (including operation at any speed from 1.4 VS1 to

VMO/MMO).

(c) Longitudinal trim. The airplane must maintain longitudinal trim

during--

(1) A climb with maximum continuous power at a speed not more than 1.4 VS1,

with the landing gear retracted, and the flaps (i) retracted and (ii) in the

takeoff position;

(2) A glide with power off at a speed not more than 1.4 VS1,  with the

landing gear extended, the wing flaps (i) retracted and (ii) extended, the

most unfavorable center of gravity position approved for landing with the

maximum landing weight, and with the most unfavorable center of gravity

position approved for landing regardless of weight; and

(3) Level flight at any speed from 1.4 VS1, to VMO/MMO,  with the landing

gear and flaps retracted, and from 1.4 VS1 to VLE with the landing gear

extended.

(d) Longitudinal, directional, and lateral trim. The airplane must maintain

longitudinal, directional, and lateral trim (and for the lateral trim, the

angle of bank may not exceed five degrees) at 1.4 VS1 during climbing flight

with--

(1) The critical engine inoperative;

(2) The remaining engines at maximum continuous power; and

(3) The landing gear and flaps retracted.

(e) Airplanes with four or more engines. Each airplane with four or more

engines must maintain trim in rectilinear flight--

(1) At the climb speed, configuration, and power required by Sec. 25.123(a)

for the purpose of establishing the rate of climb;

(2) With the most unfavorable center of gravity position; and

(3) At the weight at which the two-engine-inoperative climb is equal to at

least 0.013 VS02 at an altitude of 5,000 feet.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5671, Apr. 8, 1970; Amdt. 25-38, 41 FR 55466, Dec. 20, 1976]

Stability

Sec. 25.171  General.

The airplane must be longitudinally, directionally, and laterally stable in

accordance with the provisions of Secs. 25.173 through 25.177. In addition,

suitable stability and control feel (static stability) is required in any

condition normally encountered in service, if flight tests show it is

necessary for safe operation.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-7, 30 FR

13117, Oct. 15, 1965]

Sec. 25.173  Static longitudinal stability.

Under the conditions specified in Sec. 25.175, the characteristics of the

elevator control forces (including friction) must be as follows:

(a) A pull must be required to obtain and maintain speeds below the

specified trim speed, and a push must be required to obtain and maintain

speeds above the specified trim speed. This must be shown at any speed that

can be obtained except speeds higher than the landing gear or wing flap

operating limit speeds or VFC/MFC,  whichever is appropriate, or lower than

the minimum speed for steady unstalled flight.

(b) The airspeed must return to within 10 percent of the original trim

speed for the climb, approach, and landing conditions specified in Sec.

25.175 (a), (c), and (d), and must return to within 7.5 percent of the

original trim speed for the cruising condition specified in Sec. 25.175(b),

when the control force is slowly released from any speed within the range

specified in paragraph (a) of this section.

(c) The average gradient of the stable slope of the stick force versus

speed curve may not be less than 1 pound for each 6 knots.

(d) Within the free return speed range specified in paragraph (b) of this

section, it is permissible for the airplane, without control forces, to

stabilize on speeds above or below the desired trim speeds if exceptional

attention on the part of the pilot is not required to return to and maintain

the desired trim speed and altitude.

[Amdt. 25-7, 30 FR 13117, Oct. 15, 1965]

Sec. 25.175  Demonstration of static longitudinal stability.

Static longitudinal stability must be shown as follows:

(a) Climb. The stick force curve must have a stable slope at speeds between

85 and 115 percent of the speed at which the airplane--

(1) Is trimmed, with--

(i) Wing flaps retracted;

(ii) Landing gear retracted;

(iii) Maximum takeoff weight; and

(iv) 75 percent of maximum continuous power for reciprocating engines or

the maximum power or thrust selected by the applicant as an operating

limitation for use during climb for turbine engines; and

(2) Is trimmed at the speed for best rate-of-climb except that the speed

need not be less than 1.4 VS1.

(b) Cruise. Static longitudinal stability must be shown in the cruise

condition as follows:

(1) With the landing gear retracted at high speed, the stick force curve

must have a stable slope at all speeds within a range which is the greater of

15 percent of the trim speed plus the resulting free return speed range, or

50 knots plus the resulting free return speed range, above and below the trim

speed (except that the speed range need not include speeds less than 1.4 VS1,

nor speeds greater than VFC/MFC,   nor speeds that require a stick force of

more than 50 pounds), with--

(i) The wing flaps retracted;

(ii) The center of gravity in the most adverse position (see Sec. 25.27);

(iii) The most critical weight between the maximum takeoff and maximum

landing weights;

(iv) 75 percent of maximum continuous power for reciprocating engines or

for turbine engines, the maximum cruising power selected by the applicant as

an operating limitation (see Sec. 25.1521), except that the power need not

exceed that required at VMO/MMO; and

(v) The airplane trimmed for level flight with the power required in

paragraph (b)(1)(iv) of this section.

(2) With the landing gear retracted at low speed, the stick force curve

must have a stable slope at all speeds within a range which is the greater of

15 percent of the trim speed plus the resulting free return speed range, or

50 knots plus the resulting free return speed range, above and below the trim

speed (except that the speed range need not include speeds less than 1.4 VS1,

nor speeds greater than the minimum speed of the applicable speed range

prescribed in paragraph (b)(1), nor speeds that require a stick force of more

than 50 pounds), with--

(i) Wing flaps, center of gravity position, and weight as specified in

paragraph (b)(1) of this section;

(ii) Power required for level flight at a speed equal to VMO + 1.4 VS1/2;

and

(iii) The airplane trimmed for level flight with the power required in

paragraph (b)(2)(ii) of this section.

(3) With the landing gear extended, the stick force curve must have a

stable slope at all speeds within a range which is the greater of 15 percent

of the trim speed plus the resulting free return speed range, or 50 knots

plus the resulting free return speed range, above and below the trim speed

(except that the speed range need not include speeds less than 1.4 VS1,  nor

speeds greater than VLE, nor speeds that require a stick force of more than

50 pounds), with--

(i) Wing flap, center of gravity position, and weight as specified in

paragraph (b)(1) of this section;

(ii) 75 percent of maximum continuous power for reciprocating engines or,

for turbine engines, the maximum cruising power selected by the applicant as

an operating limitation, except that the power need not exceed that required

for level flight at VLE; and

(iii) The aircraft trimmed for level flight with the power required in

paragraph (b)(3)(ii) of this section.

(c) Approach. The stick force curve must have a stable slope at speeds

between 1.1 VS1 and 1.8 VS1,  with--

(1) Wing flaps in the approach position;

(2) Landing gear retracted;

(3) Maximum landing weight; and

(4) The airplane trimmed at 1.4 VS1 with enough power to maintain level

flight at this speed.

(d) Landing. The stick force curve must have a stable slope, and the stick

force may not exceed 80 pounds, at speeds between 1.1 VS0 and 1.3 VS0 with--

(1) Wing flaps in the landing position;

(2) Landing gear extended;

(3) Maximum landing weight;

(4) Power or thrust off on the engines; and

(5) The airplane trimmed at 1.4 VS0 with power or thrust off.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-7, 30 FR

13117, Oct. 15, 1965]

Sec. 25.177  Static lateral-directional stability.

(a) [Reserved]

(b) [Reserved]

(c) In straight, steady sideslips, the aileron and rudder control movements

and forces must be substantially proportional to the angle of sideslip in a

stable sense; and the factor of proportionality must lie between limits found

necessary for safe operation throughout the range of sideslip angles

appropriate to the operation of the airplane. At greater angles, up to the

angle at which full rudder is used or a rudder force of 180 pounds is

obtained, the rudder pedal forces may not reverse; and increased rudder

deflection must be needed for increased angles of sideslip. Compliance with

this paragraph must be demonstrated for all landing gear and flap positions

and symmetrical power conditions at speeds from 1.2 VS1 to VFE, VLE, or VFC/

MFC, as appropriate.

(d) The rudder gradi@ts must meet the requirements of paragraph (c) at

speeds between VMO/MMO and VFC/MFC except that the dihedral effect (aileron

deflection opposite the corresponding rudder input) may be negative provided

the divergence is gradual, easily recognized, and easily controlled by the

pilot.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29774, July 20, 1990; 55 FR 37607,

Sept. 12, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.181  Dynamic stability.

(a) Any short period oscillation, not including combined lateral-

directional oscillations, occurring between 1.2 VS and maximum allowable

speed appropriate to the configuration of the airplane must be heavily

damped with the primary controls--

(1) Free; and

(2) In a fixed position.

(b) Any combined lateral-directional oscillations ("Dutch roll") occurring

between 1.2 VS and maximum allowable speed appropriate to the configuration

of the airplane must be positively damped with controls free, and must be

controllable with normal use of the primary controls without requiring

exceptional pilot skill.

[Amdt. 25-42, 43 FR 2322, Jan. 16, 1978, as amended by Amdt. 25-72, 55 FR

29775, July 20, 1990; 55 FR 37607, Sept. 12, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Stalls

Sec. 25.201  Stall demonstration.

(a) Stalls must be shown in straight flight and in 30 degree banked turns

with--

(1) Power off; and

(2) The power necessary to maintain level flight at 1.6 VS1 (where VS1

corresponds to the stalling speed with flaps in the approach position, the

landing gear retracted, and maximum landing weight).

(b) In either condition required by paragraph (a) of this section, it must

be possible to meet the applicable requirements of Sec. 25.203 with--

(1) Flaps and landing gear in any likely combination of positions;

(2) Representative weights within the range for which certification is

requested; and

(3) The most adverse center of gravity for recovery.

(c) The following procedure must be used to show compliance with Sec.

25.203:

(1) With the airplane trimmed for straight flight at the speed prescribed

in Sec. 25.103(b)(1), reduce the speed with the elevator control until it is

steady at slightly above stalling speed. Apply elevator control so that the

speed reduction does not exceed one knot per second until (i) the airplane is

stalled, or (ii) the control reaches the stop.

(2) As soon as the airplane is stalled, recover by normal recovery

techniques.

(d) Occurrence of stall is defined as follows:

(1) The airplane may be considered stalled when, at an angle of attack

measurably greater than that for maximum lift, the inherent flight

characteristics give a clear and distinctive indication to the pilot that the

airplane is stalled. Typical indications of a stall, occurring either

individually or in combination, are--

(i) A nose-down pitch that cannot be readily arrested;

(ii) A roll that cannot be readily arrested; or

(iii) If clear enough, a loss of control effectiveness, an abrupt change in

control force or motion, or a distinctive shaking of the pilot's controls.

(2) For any configuration in which the airplane demonstrates an

unmistakable inherent aerodynamic warning of a magnitude and severity that is

a strong and effective deterrent to further speed reduction, the airplane may

be considered stalled when it reaches the speed at which the effective

deterrent is clearly manifested.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55466, Dec. 20, 1976; Amdt. 25-42, 43 FR 2322, Jan. 16, 1978]

Sec. 25.203  Stall characteristics.

(a) It must be possible to produce and to correct roll and yaw by

unreversed use of the aileron and rudder controls, up to the time the

airplane is stalled. No abnormal nose-up pitching may occur. The longitudinal

control force must be positive up to and throughout the stall. In addition,

it must be possible to promptly prevent stalling and to recover from a stall

by normal use of the controls.

(b) For level wing stalls, the roll occurring between the stall and the

completion of the recovery may not exceed approximately 20 degrees.

(c) For turning flight stalls, the action of the airplane after the stall

may not be so violent or extreme as to make it difficult, with normal

piloting skill, to effect a prompt recovery and to regain control of the

airplane.

Sec. 25.205  [Removed.  Amdt. 25-72, 55 FR 29775, July 20, 1990]

EDITORIAL NOTE:  For the convenience of the user, the removed text is

set out below.

Sec. 25.205  Stalls: Critical engine inoperative.

(a) It must be possible to safely recover from a stall with the critical

engine inoperative--

(1) Without applying power to the inoperative engine;

(2) With flaps and landing gear retracted; and

(3) With the remaining engines at up to 75 percent of maximum continuous

power, or up to the power at which the wings can be held level with the use

of maximum control travel, whichever is less.

(b) The operating engines may be throttled back during stall recovery from

stalls with the critical engine inoperative.

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.207  Stall warning.

(a) Stall warning with sufficient margin to prevent inadvertent stalling

with the flaps and landing gear in any normal position must be clear and

distinctive to the pilot in straight and turning flight.

(b) The warning may be furnished either through the inherent aerodynamic

qualities of the airplane or by a device that will give clearly

distinguishable indications under expected conditions of flight. However, a

visual stall warning device that requires the attention of the crew within

the cockpit is not acceptable by itself. If a warning device is used, it must

provide a warning in each of the airplane configuations prescribed in

paragraph (a) of this section at the speed prescribed in paragraph (c) of

this section.

(c) The stall warning must begin at a speed exceeding the stalling speed

(i.e., the speed at which the airplane stalls or the minimum speed

demonstrated, whichever is applicable under the provisions of Sec. 25.201(d))

by seven percent or at any lesser margin if the stall warning has enough

clarity, duration, distinctiveness, or similar properties.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-7, 30 FR

13118, Oct. 15, 1965; Amdt. 25-42, 43 FR 2322, Jan. 16, 1978]

Ground and Water Handling Characteristics

Sec. 25.231  Longitudinal stability and control.

(a) Landplanes may have no uncontrollable tendency to nose over in any

reasonably expected operating condition or when rebound occurs during landing

or takeoff. In addition--

(1) Wheel brakes must operate smoothly and may not cause any undue tendency

to nose over; and

(2) If a tail-wheel landing gear is used, it must be possible, during the

takeoff ground run on concrete, to maintain any altitude up to thrust line

level, at 80 percent of VS1.

(b) For seaplanes and amphibians, the most adverse water conditions safe

for takeoff, taxiing, and landing, must be established.

Sec. 25.233  Directional stability and control.

(a) There may be no uncontrollable ground-looping tendency in 90 deg. cross

winds, up to a wind velocity of 20 knots or 0.2 VS0, whichever is greater,

except that the wind velocity need not exceed 25 knots. At any speed at which

the airplane may be expected to be operated on the ground. This may be shown

while establishing the 90 deg. cross component of wind velocity required by

Sec. 25.237.

(b) Landplanes must be satisfactorily controllable, without exceptional

piloting skill or alertness, in power-off landings at normal landing speed,

without using brakes or engine power to maintain a straight path. This may be

shown during power-off landings made in conjunction with other tests.

(c) The airplane must have adequate directional control during taxiing.

This may be shown during taxiing prior to takeoffs made in conjunction with

other tests.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5671, Apr. 8, 1970; Amdt. 25-42, 43 FR 2322, Jan. 16, 1978]

Sec. 25.235  Taxiing condition.

The shock absorbing mechanism may not damage the structure of the airplane

when the airplane is taxied on the roughest ground that may reasonably be

expected in normal operation.

Sec. 25.237  Wind velocities.

(a) For landplanes and amphibians, a 90-degree cross component of wind

velocity, demonstrated to be safe for takeoff and landing, must be

established for dry runways and must be at least 20 knots or 0.2 VS0,

whichever is greater, except that it need not exceed 25 knots.

(b) For seaplanes and amphibians, the following applies:

(1) A 90-degree cross component of wind velocity, up to which takeoff and

landing is safe under all water conditions that may reasonably be expected in

normal operation, must be established and must be at least 20 knots or 0.2

Vs0, whichever is greater, except that it need not exceed 25 knots.

(2) A wind velocity, for which taxiing is safe in any direction under all

water conditions that may reasonably be expected in normal operation, must be

established and must be at least 20 knots or 0.2 VS0, whichever is greater,

except that it need not exceed 25 knots.

[Amdt. 25-42, 43 FR 2322, Jan. 16, 1978]

Sec. 25.239  Spray characteristics, control, and stability on water.

(a) For seaplanes and amphibians, during takeoff, taxiing, and landing, and

in the conditions set forth in paragraph (b) of this section, there may be

no--

(1) Spray characteristics that would impair the pilot's view, cause damage,

or result in the taking in of an undue quantity of water;

(2) Dangerously uncontrollable porpoising, bounding, or swinging tendency;

or

(3) Immersion of auxiliary floats or sponsons, wing tips, propeller blades,

or other parts not designed to withstand the resulting water loads.

(b) Compliance with the requirements of paragraph (a) of this section must

be shown--

(1) In water conditions, from smooth to the most adverse condition

established in accordance with Sec. 25.231;

(2) In wind and cross-wind velocities, water currents, and associated waves

and swells that may reasonably be expected in operation on water;

(3) At speeds that may reasonably be expected in operation on water;

(4) With sudden failure of the critical engine at any time while on water;

and

(5) At each weight and center of gravity position, relevant to each

operating condition, within the range of loading conditions for which

certification is requested.

(c) In the water conditions of paragraph (b) of this section, and in the

corresponding wind conditions, the seaplane or amphibian must be able to

drift for five minutes with engines inoperative, aided, if necessary, by a

sea anchor.

Miscellaneous Flight Requirements

Sec. 25.251  Vibration and buffeting.

(a) The airplane must be demonstrated in flight to be free from any

vibration and buffeting that would prevent continued safe flight in any

likely operating condition.

(b) Each part of the airplane must be demonstrated in flight to be free

from excessive vibration under any appropriate speed and power conditions up

to VDF/MDF. The maximum speeds shown must be used in establishing the

operating limitations of the airplane in accordance with Sec. 25.1505.

(c) Except as provided in paragraph (d) of this section, there may be no

buffeting condition, in normal flight, including configuration changes during

cruise, severe enough to interfere with the control of the airplane, to cause

excessive fatigue to the crew, or to cause structural damage. Stall warning

buffeting within these limits is allowable.

(d) There may be no perceptible buffeting condition in the cruise

configuration in straight flight at any speed up to VMO/MMO, except that

stall warning buffeting is allowable.

(e) For an airplane with MD greater than .6 or with a maximum operating

altitude greater than 25,000 feet, the positive maneuvering load factors at

which the onset of perceptible buffeting occurs must be determined with the

airplane in the cruise configuration for the ranges of airspeed or Mach

number, weight, and altitude for which the airplane is to be certificated.

The envelopes of load factor, speed, altitude, and weight must provide a

sufficient range of speeds and load factors for normal operations. Probable

inadvertent excursions beyond the boundaries of the buffet onset envelopes

may not result in unsafe conditions.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5671, Apr. 8, 1970; Amdt. 25-72, 55 FR 29775, July 20, 1990; Amdt. 25-77, 57

FR 28949, June 29, 1992]

*****************************************************************************

57 FR 28946, No. 125, June 29, 1992

SUMMARY: This amendment revises the airworthiness standards of the Federal

Aviation Regulations (FAR) for type certification of transport category

airplanes concerning vibration, buffet, flutter and divergence. It clarifies

the requirement to consider flutter and divergence when treating certain

damage and failure conditions required by other sections of the FAR and

adjusts the safety margins related to aeroelastic stabiity to make them more

appropriate for the conditions to which they apply. These changes are made to

provide consistency with other sections of the FAR and to take into account

advances in technology and the evolution of the design of transport

airplanes.

EFFECTIVE DATE: July 29, 1992.

*****************************************************************************

Sec. 25.253  High-speed characteristics.

(a) Speed increase and recovery characteristics.  The following speed

increase and recovery characteristics must be met:

(1) Operating conditions and characteristics likely to cause inadvertent

speed increases (including upsets in pitch and roll) must be simulated with

the airplane trimmed at any likely cruise speed up to VMO/MMO. These

conditions and characteristics include gust upsets, inadvertent control

movements, low stick force gradient in relation to control friction,

passenger movement, leveling off from climb, and descent from Mach to

airspeed limit altitudes.

(2) Allowing for pilot reaction time after effective inherent or artificial

speed warning occurs, it must be shown that the airplane can be recovered to

a normal attitude and its speed reduced to VMO/MMO, without-

(i) Exceptional piloting strength or skill;

(ii) Exceeding VD/MD, VDF/MDF, or the structural limitations; and

(iii) Buffeting that would impair the pilot's ability to read the

instruments or control the airplane for recovery.

(3) With the airplane trimmed at any speed up to VMO /MMO, there must be no

reversal of the response to control input about any axis at any speed up to

VDF/MDF. Any tendency to pitch, roll, or yaw must be mild and readily

controllable, using normal piloting techniques. When the airplane is trimmed

at VMO/MMO, the slope of the elevator control force versus speed curve need

not be stable at speeds greater than VFC/MFC, but there must be a push force

at all speeds up to VDF/MDF and there must be no sudden or excessive

reduction of elevator control force as VDF/MDF is reached.

(b) Maximum speed for stability characteristics, VFC/MFC. VFC/MFC is the

maximum speed at which the requirements of Secs. 25.147(e), 25.175(b)(1),

25.177, and 25.181 must be met with flaps and landing gear retracted. It may

not be less than a speed midway between VMO/MMO and VDF/MDF, except that, for

altitudes where Mach number is the limiting factor, MFC need not exceed the

Mach number at which effective speed warning occurs.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5671, Apr. 8, 1970; Amdt. 25-54, 45 FR 60172, Sept. 11, 1980; Amdt. 25-72, 55

FR 29775, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.255  Out-of-trim characteristics.

(a) From an initial condition with the airplane trimmed at cruise speeds up

to VMO/MMO, the airplane must have satisfactory maneuvering stability and

controllability with the degree of out-of-trim in both the airplane nose-up

and nose-down directions, which results from the greater of--

(1) A three-second movement of the longitudinal trim sy@em at its normal

rate for the particular flight condition with no aerodynamic load (or an

equivalent degree of trim for airplanes that do not have a power-operated

trim system), except as limited by stops in the trim system, including those

required by Sec. 25.655(b) for adjustable stabilizers; or

(2) The maximum mistrim that can be sustained by the autopilot while

maintaining level flight in the high speed cruising condition.

(b) In the out-of-trim condition specified in paragraph (a) of this

section, when the normal acceleration is varied from +1 g to the positive and

negative values specified in paragraph (c) of this section--

(1) The stick force vs. g curve must have a positive slope at any speed up

to and including VFC/MFC; and

(2) At speeds between VFC/MFC and VDF/MDF the direction of the primary

longitudinal control force may not reverse.

(c) Except as provided in paragraphs (d) and (e) of this section,

compliance with the provisions of paragraph (a) of this section must be

demonstrated in flight over the acceleration range--

(1) -1 g to +2.5 g; or

(2) 0 g to 2.0 g, and extrapolating by an acceptable method to -1 g and

+2.5 g.

(d) If the procedure set forth in paragraph (c)(2) of this section is used

to demonstrate compliance and marginal conditions exist during flight test

with regard to reversal of primary longitudinal control force, flight tests

must be accomplished from the normal acceleration at which a marginal

condition is found to exist to the applicable limit specified in paragraph

(b)(1) of this section.

(e) During flight tests required by paragraph (a) of this section, the

limit maneuvering load factors prescribed in Secs. 25.333(b) and 25.337, and

the maneuvering load factors associated with probable inadvertent excursions

beyond the boundaries of the buffet onset envelopes determined under Sec.

25.251(e), need not be exceeded. In addition, the entry speeds for flight

test demonstrations at normal acceleration values less than 1 g must be

limited to the extent necessary to accomplish a recovery without exceeding

VDF/MDF.

(f) In the out-of-trim condition specified in paragraph (a) of this

section, it must be possible from an overspeed condition at VDF/MDF to

produce at least 1.5 g for recovery by applying not more than 125 pounds of

longitudinal control force using either the primary longitudinal control

alone or the primary longitudinal control and the longitudinal trim system.

If the longitudinal trim is used to assist in producing the required load

factor, it must be shown at VDF/MDF that the longitudinal trim can be

actuated in the airplane nose-up direction with the primary surface loaded to

correspond to the least of the following airplane nose-up control forces:

(1) The maximum control forces expected in service as specified in Secs.

25.301 and 25.397.

(2) The control force required to produce 1.5 g.

(3) The control force corresponding to buffeting or other phenomena of such

intensity that it is a strong deterrent to further application of primary

longitudinal control force.

[Amdt. No. 25-42, 43 FR 2322, Jan. 16, 1978]

Subpart C--Structure

General

Sec. 25.301  Loads.

(a) Strength requirements are specified in terms of limit loads (the

maximum loads to be expected in service) and ultimate loads (limit loads

multiplied by prescribed factors of safety). Unless otherwise provided,

prescribed loads are limit loads.

(b) Unless otherwise provided, the specified air, ground, and water loads

must be placed in equilibrium with inertia forces, considering each item of

mass in the airplane. These loads must be distributed to conservatively

approximate or closely represent actual conditions. Methods used to determine

load intensities and distribution must be validated by flight load

measurement unless the methods used for determining those loading conditions

are shown to be reliable.

(c) If deflections under load would significantly change the distribution

of external or internal loads, this redistribution must be taken into

account.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970]

Sec. 25.303  Factor of safety.

Unless otherwise specified, a factor of safety of 1.5 must be applied to

the prescribed limit load which are considered external loads on the

structure. When a loading condition is prescribed in terms of ultimate loads,

a factor of safety need not be applied unless otherwise specified.

[Amdt. 25-23, 35 FR 5672, Apr. 8, 1970]

Sec. 25.305  Strength and deformation.

(a) The structure must be able to support limit loads without detrimental

permanent deformation. At any load up to limit loads, the deformation may not

interfere with safe operation.

(b) The structure must be able to support ultimate loads without failure

for at least 3 seconds. However, when proof of strength is shown by dynamic

tests simulating actual load conditions, the 3-second limit does not apply.

Static tests conducted to ultimate load must include the ultimate deflections

and ultimate deformation induced by the loading. When analytical methods are

used to show compliance with the ultimate load strength requirements, it must

be shown that--

(1) The effects of deformation are not significant;

(2) The deformations involved are fully accounted for in the analysis; or

(3) The methods and assumptions used are sufficient to cover the effects of

these deformations.

(c) Where structural flexibility is such that any rate of load application

likely to occur in the operating conditions might produce transient stresses

appreciably higher than those corresponding to static loads, the effects of

this rate of application must be considered.

(d) The dynamic response of the airplane to vertical and lateral continuous

turbulence must be taken into account. The continuous gust design criteria of

Appendix G of this part must be used to establish the dynamic response unless

more rational criteria are shown.

(e) The airplane must be designed to withstand any vibration and buffeting

that might occur in any likely operating condition up to VD/MD, including

stall and probable inadvertent excursions beyond the boundaries of the buffet

onset envelope. This must be shown by analysis, flight tests, or other tests

found necessary by the Administrator.

(f) Unless shown to be extremely improbable, the airplane must be designed

to withstand any forced structural vibration resulting from any failure,

malfunction or adverse condition in the flight control system. These must be

considered limit loads and must be investigated at airspeeds up to VC/MC.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970; Amdt. 25-54, 45 FR 60172, Sept. 11, 1980; Amdt. 25-77, 57

FR 28949, June 29, 1992]

*****************************************************************************

57 FR 28946, No. 125, June 29, 1992

SUMMARY: This amendment revises the airworthiness standards of the Federal

Aviation Regulations (FAR) for type certification of transport category

airplanes concerning vibration, buffet, flutter and divergence. It clarifies

the requirement to consider flutter and divergence when treating certain

damage and failure conditions required by other sections of the FAR and

adjusts the safety margins related to aeroelastic stabiity to make them more

appropriate for the conditions to which they apply. These changes are made to

provide consistency with other sections of the FAR and to take into account

advances in technology and the evolution of the design of transport

airplanes.

EFFECTIVE DATE: July 29, 1992.

*****************************************************************************

Sec. 25.307   Proof of structure.

(a) Compliance with the strength and deformation requirements of this

subpart must be shown for each critical loading condition. Structural

analysis may be used only if the structure conforms to that for which

experience has shown this method to be reliable. The Administrator may

require ultimate load tests in cases where limit load tests may be

inadequate.

(b) [Reserved]

(c) [Reserved]

(d) When static or dynamic tests are used to show compliance with the

requirements of Sec. 25.305(b) for flight structures, appropriate material

correction factors must be applied to the test results, unless the structure,

or part thereof, being tested has features such that a number of elements

contribute to the total strength of the structure and the failure of one

element results in the redistribution of the load through alternate load

paths.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970; Amdt. 25-54, 45 FR 60172, Sept. 11, 1980; Amdt. 25-72, 55

FR 29775, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Flight Loads

Sec. 25.321  General.

(a) Flight load factors represent the ratio of the aerodynamic force

component (acting normal to the assumed longitudinal axis of the airplane) to

the weight of the airplane. A positive load factor is one in which the

aerodynamic force acts upward with respect to the airplane.

(b) Considering compressibility effects at each speed, compliance with the

flight load requirements of this subpart must be shown--

(1) At each critical altitude within the range of altitudes selected by the

applicant;

(2) At each weight from the design minimum weight to the design maximum

weight appropriate to each particular flight load condition; and

(3) For each required altitude and weight, for any practicable distribution

of disposable load within the operating limitations recorded in the Airplane

Flight Manual.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970]

Flight Maneuver and Gust Conditions

Sec. 25.331  General.

(a) Procedure. The analysis of symmetrical flight must include at least the

conditions specified in paragraphs (b) through (d) of this section. The

following procedure must be used:

(1) Enough points on the maneuvering and gust envelopes must be

investigated to ensure that the maximum load for each part of the airplane

structure is obtained. A conservative combined envelope may be used.

(2) The significant forces acting on the airplane must be placed in

equilibrium in a rational or conservative manner. The linear inertia forces

must be considered in equilibrium with thrust and all aerodynamic loads,

while the angular (pitching) inertia forces must be considered in equilibrium

with thrust and all aerodynamic moments, including moments due to loads on

components such as tail surfaces and nacelles. Critical thrust values in the

range from zero to maximum continuous thrust must be considered.

(3) Where sudden displacement of a control is specified, the assumed rate

of control surface displacement may not be less than the rate that could be

applied by the pilot through the control system.

(4) In determining elevator angles and chordwise load distribution (in the

maneuvering conditions of paragraphs (b) and (c) of this section) in turns

and pullups, the effect of corresponding pitching velocities must be taken

into account. The in-trim and out-of-trim flight conditions specified in Sec.

25.255 must be considered.

(b) Maneuvering balanced conditions.  Assuming the airplane to be in

equilibrium with zero pitching acceleration, the maneuvering conditions A

through I on the maneuvering envelope in Sec. 25.333(b) must be investigated.

(c) Maneuvering pitching conditions.  The following conditions involving

pitching acceleration must be investigated:

(1) Maximum elevator displacement at VA. The airplane is assumed to be

flying in steady level flight (point A1, Sec. 25.333(b)) and, except as

limited by pilot effort in accordance with Sec. 25.397(b), the pitching

control is suddenly moved to obtain extreme positive pitching acceleration

(nose up). The dynamic response or, at the option of the applicant, the

transient rigid body response of the airplane, must be taken into account in

determining the tail load. Airplane loads which occur subsequent to the

normal acceleration at the center of gravity exceeding the maximum positive

limit maneuvering load factor, n, need not be considered.

(2) Specified control displacement. A checked maneuver, based on a rational

pitching control motion vs. time profile, must be established in which the

design limit load factor specified in Sec. 25.337 will not be exceeded.

Unless lesser values cannot be exceeded, the airplane response must result in

pitching accelerations not less than the following:

(i) A positive pitching acceleration (nose up) is assumed to be reached

concurrently with the airplane load factor of 1.0 (Points A1 to D1, Sec.

25.333(b)). The positive acceleration must be equal to at least

39n

----    (n-1.5), (Radians/sec./2/ )

v

where--

n is the positive load factor at the speed under consideration, and V is the

airplane equivalent speed in knots.

(ii) A negative pitching acceleration (nose down) is assumed to be reached

concurrently with the positive maneuvering load factor (Points A2 to D2,

Sec. 25.333(b)). This negative pitching acceleration must be equal to at

least

-26n

------    (n-1.5), (Radians/sec./2/ )

v

where--

n is the positive load factor at the speed under consideration; and V is the

airplane equivalent speed in knots.

(d) Gust conditions. The gust conditions B' through J' Sec. 25.333(c), must

be investigated. The following provisions apply:

(1) The air load increment due to a specified gust must be added to the

initial balancing tail load corresponding to steady level flight.

(2) The alleviating effect of wing down-wash and of the airplane's motion

in response to the gust may be included in computing the tail gust load

increment.

(3) Instead of a rational investigation of the airplane response, the gust

alleviation factor Kg may be applied to the specified gust intensity for the

horizontal tail.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970; Amdt. 25-46, 43 FR 50594, Oct. 30, 1978; 43 FR 52495,

Nov. 13, 1978; 43 FR 54082, Nov. 20, 1978; Amdt. 25-72, 55 FR 29775, July 20,

1990; 55 FR 37607, Sept. 12, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.333  Flight envelope.

(a) General. The strength requirements must be met at each combination of

airspeed and load factor on and within the boundaries of the representative

maneuvering and gust envelopes (V-n  diagrams) of paragraphs (b) and (c) of

this section. These envelopes must also be used in determining the airplane

structural operating limitations as specified in Sec. 25.1501.

(b) Maneuvering envelope.

[ ...Illustration appears here... ]

(c) Gust envelope.

[ ...Illustration appears here... ]

Sec. 25.335  Design airspeeds.

The selected design airspeeds are equivalent airspeeds (EAS). Estimated

values of VS0 and VS1 must be conservative.

(a) Design cruising speed, VC. For VC, the following apply:

(1) The minimum value of VC must be sufficiently greater than VB to provide

for inadvertent speed increases likely to occur as a result of severe

atmospheric turbulence.

(2) In the absence of a rational investigation substantiating the use of

other values, VC may not be less than VB+43 knots. However, it need not

exceed the maximum speed in level flight at maximum continuous power for the

corresponding altitude.

(3) At altitudes where VD is limited by Mach number, VC may be limited to a

selected Mach number.

(b) Design dive speed, VD. VD must be selected so that VC/MC is not greater

than 0.8 VD/MD, or so that the minimum speed margin between VC/MC and VD/MD

is the greater of the following values:

(1) From an initial condition of stabilized flight at VC/MC, the airplane

is upset, flown for 20 seconds along a flight path 7.5 deg. below the initial

path, and then pulled up at a load factor of 1.5 g (0.5 g  acceleration

increment). The speed increase occurring in this maneuver may be calculated

if reliable or conservative aerodynamic data is used. Power as specified in

Sec. 25.175(b)(1)(iv) is assumed until the pullup is initiated, at which time

power reduction and the use of pilot controlled drag devices may be assumed;

(2) The minimum speed margin must be enough to provide for atmospheric

variations (such as horizontal gusts, and penetration of jet streams and cold

fronts) and for instrument errors and airframe production variations. These

factors may be considered on a probability basis. However, the margin at

altitude where MC is limited by compressibility effects may not be less than

0.05 M.

(c) Design maneuvering speed VA. For VA, the following apply:

(1) VA may not be less than VS1 <radical>n where--

(i) n is the limit positive maneuvering load factor at VC; and

(ii) VS1 is the stalling speed with flaps retracted.

(2) VA and VS must be evaluated at the design weight and altitude under

consideration.

(3) VA need not be more than VC or the speed at which the positive CN max

curve intersects the positive maneuver load factor line, whichever is less.

(d) Design speed for maximum gust intensity, VB. For VB, the following

apply:

(1) VB may not be less than the speed determined by the intersection of the

line representing the maximum position lift CN max and the line representing

the rough air gust velocity on the gust V-n diagram, or (<radical>ng) VS1,

whichever is less, where--

(i) ng is the positive airplane gust load factor due to gust, at speed VC

(in accordance with Sec. 25.341), and at the particular weight under

consideration; and

(ii) VS1 is the stalling speed with the flaps retracted at the particular

weight under consideration.

(2) VB need not be greater than VC.

(e) Design flap speeds, VF. For VF, the following apply:

(1) The design flap speed for each flap position (established in accordance

with Sec. 25.697(a)) must be sufficiently greater than the operating speed

recommended for the corresponding stage of flight (including balked landings)

to allow for probable variations in control of airspeed and for transition

from one flap position to another.

(2) If an automatic flap positioning or load limiting device is used, the

speeds and corresponding flap positions programmed or allowed by the device

may be used.

(3) VF may not be less than--

(i) 1.6 VS1 with the flaps in takeoff position at maximum takeoff weight;

(ii) 1.8 VS1 with the flaps in approach position at maximum landing weight,

and

(iii) 1.8 VS0 with the flaps in landing position at maximum landing weight.

(f) Design drag device speeds, VDD. The selected design speed for each drag

device must be sufficiently greater than the speed recommended for the

operation of the device to allow for probable variations in speed control.

For drag devices intended for use in high speed descents, VDD may not be less

than VD. When an automatic drag device positioning or load limiting means is

used, the speeds and corresponding drag device positions programmed or

allowed by the automatic means must be used for design.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970]

Sec. 25.337  Limit maneuvering load factors.

(a) Except where limited by maximum (static) lift coefficients, the

airplane is assumed to be subjected to symmetrical maneuvers resulting in the

limit maneuvering load factors prescribed in this section. Pitching

velocities appropriate to the corresponding pull-up and steady turn maneuvers

must be taken into account.

(b) The positive limit maneuvering load factor "n" for any speed up to Vn

may not be less than 2.1+24,000/ (W +10,000) except that "n" may not be less

than 2.5 and need not be greater than 3.8--where "W" is the design maximum

takeoff weight.

(c) The negative limit maneuvering load factor--

(1) May not be less than -1.0 at speeds up to VC; and

(2) Must vary linearly with speed from the value at VC to zero at VD.

(d) Maneuvering load factors lower than those specified in this section may

be used if the airplane has design features that make it impossible to exceed

these values in flight.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970]

Sec. 25.341  Gust loads.

(a) The airplane is assumed to be subjected to symmetrical vertical gusts

in level flight. The resulting limit load factors must correspond to the

conditions determined as follows:

(1) Positive (up) and negative (down) rough air gusts of 66 fps at VB must

be considered at altitudes between sea level and 20,000 feet. The gust

velocity may be reduced linearly from 66 fps at 20,000 feet to 38 fps at

50,000 feet.

(2) Positive and negative gusts of 50 fps at VC must be considered at

altitudes between sea level and 20,000 feet. The gust velocity may be reduced

linearly from 50 fps at 20,000 feet to 25 fps at 50,000 feet.

(3) Positive and negative gusts of 25 fps at VD must be considered at

altitudes between sea level and 20,000 feet. The gust velocity may be reduced

linearly from 25 fps at 20,000 feet to 12.5 fps at 50,000 feet.

(b) The following assumptions must be made:

(1) The shape of the gust is

Ude        2<pi>s

U = --- (1-cos ------ )

2           25C

where--

s=distance penetrated into gust (ft);

C=mean geometric chord of wing (ft); and

Ude=derived gust velocity referred to in paragraph (a) (fps).

(2) Gust load factors vary linearly between the specified conditions B'

through G', as shown on the gust envelope in Sec. 25.333(c).

(c) In the absence of a more rational analysis, the gust load factors must

be computed as follows:

KgUdeVa

n=1 + ---------

498 (W/S)

where--

0.88<mu>g

Kg = ----------- = gust alleviation factor;

5.3+<mu>g

2(W/S)

<mu>g = ------ = airplane mass ratio:

rCag

Ude=derived gust velocities referred to in paragraph (a) (fps);

r=density of air (slugs cu. ft.);

W/S=wing loading (psf);

C=mean geometric chord (ft);

g=acceleration due to gravity (ft/sec**2);

V=airplane equivalent speed (knots); and

a=slope of the airplane normal force coefficient curve CNA per radian if the

gust loads are applied to the wings and horizontal method. The wing lift

curve slope CAL per radian may be used when the gust load is applied to

the wings only and the horizontal tail gust loads are treated as a

separate condition.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55

FR 29775, July 20, 1990; 55 FR 37607, Sept. 12, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.343  Design fuel and oil loads.

(a) The disposable load combinations must include each fuel and oil load in

the range from zero fuel and oil to the selected maximum fuel and oil load. A

structural reserve fuel condition, not exceeding 45 minutes of fuel under the

operating conditions in Sec. 25.1001(e) and (f), as applicable, may be

selected.

(b) If a structural reserve fuel condition is selected, it must be used as

the minimum fuel weight condition for showing compliance with the flight load

requirements as prescribed in this subpart. In addition--

(1) The structure must be designed for a condition of zero fuel and oil in

the wing at limit loads corresponding to--

(i) A maneuvering load factor of +2.25; and

(ii) Gust intensities equal to 85 percent of the values prescribed in Sec.

25.341; and

(2) Fatigue evaluation of the structure must account for any increase in

operating stresses resulting from the design condition of paragraph (b)(1) of

this section; and

(3) The flutter, deformation, and vibration requirements must also be met

with zero fuel.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-18, 33 FR

12226, Aug. 30, 1968; Amdt. 25-72, 55 FR 29775, July 20, 1990; 55 FR 37607,

Sept. 12, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.345  High lift devices.

(a) If flaps are to be used during takeoff, approach, or landing, at the

design flap speeds established for these stages of flight under Sec.

25.335(e) and with the flaps in the corresponding positions, the airplane is

assumed to be subjected to symmetrical maneuvers and gusts within the range

determined by--

(1) Maneuvering to a positive limit load factor of 2.0; and

(2) Positive and negative 25 fps derived gusts acting normal to the flight

path in level flight.

(b) The airplane must be designed for the conditions prescribed in

paragraph (a) of this section, except that the airplane load factor need not

exceed 1.0, taking into account, as separate conditions, the effects of--

(1) Propeller slipstream corresponding to maximum continuous power at the

design flap speeds VF, and with takeoff power at not less than 1.4 times the

stalling speed for the particular flap position and associated maximum

weight; and

(2) A head-on gust of 25 feet per second velocity (EAS).

(c) If flaps or similar high lift devices are to be used in en route

conditions, and with flaps in the appropriate position at speeds up to the

flap design speed chosen for these conditions, the airplane is assumed to be

subjected to symmetrical maneuvers and gusts within the range determined by--

(1) Maneuvering to a positive limit load factor as prescribed in Sec.

25.337(b); and

(2) Positive and negative derived gusts as prescribed in Sec. 25.341 acting

normal to the flight path in level flight.

(d) The airplane must be designed for landing at the maximum takeoff weight

with a maneuvering load factor of 1.5g and the flaps and similar high lift

devices in the landing configuration.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-46, 43 FR

50595, Oct. 30, 1978; Amdt. 25-72, 55 FR 29775, July 20, 1990; 55 FR 37607,

Sept. 12, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.349  Rolling conditions.

The airplane must be designed for rolling loads resulting from the

conditions specified in paragraphs (a) and (b) of this section. Unbalanced

aerodynamic moments about the center of gravity must be reacted in a rational

or conservative manner, considering the principal masses furnishing the

reacting inertia forces.

(a) Maneuvering. The following conditions, speeds, and aileron deflections

(except as the deflections may be limited by pilot effort) must be considered

in combination with an airplane load factor of zero and of two-thirds of the

positive maneuvering factor used in design. In determining the required

aileron deflections, the torsional flexibility of the wing must be considered

in accordance with Sec. 25.301(b):

(1) Conditions corresponding to steady rolling velocities must be

investigated. In addition, conditions corresponding to maximum angular

acceleration must be investigated for airplanes with engines or other weight

concentrations outboard of the fuselage. For the angular acceleration

conditions, zero rolling velocity may be assumed in the absence of a rational

time history investigation of the maneuver.

(2) At VA,  a sudden deflection of the aileron to the stop is assumed.

(3) At VC,  the aileron deflection must be that required to produce a rate

of roll not less than that obtained in paragraph (a)(2) of this section.

(4) At VD,  the aileron deflection must be that required to produce a rate

of roll not less than one-third of that in paragraph (a)(2) of this section.

(b) Unsymmetrical gusts. The condition of unsymmetrical gusts must be

considered by modifying the symmetrical flight conditions B' or C' (in Sec.

25.333(c)) whichever produces the critical load. It is assumed that 100

percent of the wing air load acts on one side of the airplane and 80 percent

acts on the other side.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970]

Sec. 25.351  Yawing conditions.

The airplane must be designed for loads resulting from the conditions

specified in paragraphs (a) and (b) of this section. Unbalanced aerodynamic

moments about the center of gravity must be reacted in a rational or

conservative manner considering the principal masses furnishing the reacting

inertia forces:

(a) Maneuvering. At speeds from VMC to VD, the following maneuvers must be

considered. In computing the tail loads, the yawing velocity may be assumed

to be zero:

(1) With the airplane in unaccelerated flight at zero yaw, it is assumed

that the rudder control is suddenly displaced to the maximum deflection, as

limited by the control surface stops, or by a 300-pound rudder pedal force,

whichever is less.

(2) With the rudder deflected as specified in paragraph (a)(1) of this

section, it is assumed that the airplane yaws to the resulting sideslip

angle.

(3) With the airplane yawed to the static sideslip angle corresponding to

the rudder deflection specified in paragraph (a)(1) of this section, it is

assumed that the rudder is returned to neutral.

(b) Lateral gusts. The airplane is assumed to encounter derived gusts

normal to the plane of symmetry while in unaccelerated flight. The derived

gusts and airplane speeds corresponding to conditions B' through J' (in Sec.

25.333(c)) (as determined by Secs. 25.341 and 25.345(a)(2) or Sec.

25.345(c)(2)) must be investigated. The shape of the gust must be as

specified in Sec. 25.341. In the absence of a rational investigation of the

airplane's response to a gust, the gust loading on the vertical tail surfaces

must be computed as follows:

KgtUdeVatSt

Lt = -----------

498

where--

Lt=vertical tail load (lbs.);

0.88<mu>gt

Kgt = ------------ = gust alleviation factor;

5.3+<mu>gt

2W         K

<mu>gt = -------- ( ----- )**2 =lateral mass ratio;

pCtgatSt    lt

Ude=derived gust velocity (fps);

p=air density (slugs/cu. ft.);

W=airplane weight (lbs.);

St=area of vertical tail (ft.**2);

Ct=mean geometric chord of vertical surface (ft.);

at=lift curve slope of vertical tail (per radian);

K=radius of gyration in yaw (ft).;

lt=distance from airplane c.g., to lift center of vertical surface (ft.);

g=acceleration due to gravity (ft./sec.**2); and

V=airplane equivalent speed (knots).

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970; Amdt. 25-46, 43 FR 50595, Oct. 30, 1978; Amdt. 25-72, 55

FR 29775, July 20, 1990; 55 FR 37608, Sept. 12, 1990; 55 FR 41415, Oct. 11,

1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Supplementary Conditions

Sec. 25.361  Engine torque.

(a) Each engine mount and its supporting structure must be designed for the

effects of--

(1) A limit engine torque corresponding to takeoff power and propeller

speed acting simultaneously with 75 percent of the limit loads from flight

condition A of Sec. 25.333(b);

(2) A limit torque corresponding to the maximum continuous power and

propeller speed, acting simultaneously with the limit loads from flight

condition A of Sec. 25.333(b); and

(3) For turbopropeller installations, in addition to the conditions

specified in paragraphs (a)(1) and (2) of this section, a limit engine torque

corresponding to takeoff power and propeller speed, multiplied by a factor

accounting for propeller control system malfunction, including quick

feathering, acting simultaneously with 1g level flight loads. In the absence

of a rational analysis, a factor of 1.6 must be used.

(b) For turbine engine installations, the engine mounts and supporting

structure must be designed to withstand each of the following:

(1) A limit engine torque load imposed by sudden engine stoppage due to

malfunction or structural failure (such as compressor jamming).

(2) A limit engine torque load imposed by the maximum acceleration of the

engine.

(c) The limit engine torque to be considered under paragraph (a) of this

section must be obtained by multiplying mean torque for the specified power

and speed by a factor of--

(1) 1.25 for turbopropeller installations;

(2) 1.33 for reciprocating engines with five or more cylinders; or

(3) Two, three, or four, for engines with four, three, or two cylinders,

respectively.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970; Amdt. 25-46, 43 FR 50595, Oct. 30, 1978; Amdt. 25-72, 55

FR 29776, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.363  Side load on engine mount.

(a) Each engine mount and its supporting structure must be designed for a

limit load factor in a lateral direction, for the side load on the engine

mount, at least equal to the maximum load factor obtained in the yawing

conditions but not less than--

(1) 1.33; or

(2) One-third of the limit load factor for flight condition A as prescribed

in Sec. 25.333(b).

(b) The side load prescribed in paragraph (a) of this section may be

assumed to be independent of other flight conditions.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970]

Sec. 25.365  Pressurized cabin loads.

For airplanes with one or more pressurized compartments the following

apply:

(a) The airplane structure must be strong enough to withstand the flight

loads combined with pressure differential loads from zero up to the maximum

relief valve setting.

(b) The external pressure distribution in flight, and stress concentrations

and fatigue effects must be accounted for.

(c) If landings may be made with the compartment pressurized, landing loads

must be combined with pressure differential loads from zero up to the maximum

allowed during landing.

(d) The airplane structure must be strong enough to withstand the pressure

differential loads corresponding to the maximum relief valve setting

multiplied by a factor of 1.33, omitting other loads.

(e) Any structure, component or part, inside or outside a pressurized

compartment, the failure of which could interfere with continued safe flight

and landing, must be designed to withstand the effects of a sudden release of

pressure through an opening in any compartment at any operating altitude

resulting from each of the following conditions:

(1) The penetration of the compartment by a portion of an engine following

an engine disintegration;

(2) Any opening in any pressurized compartment up to the size Ho in square

feet; however, small compartments may be combined with an adjacent

pressurized compartment and both considered as a single compartment for

openings that cannot reasonably be expected to be confined to the small

compartment. The size Ho must be computed by the following formula:

Ho=PAs

where,

Ho=Maximum opening in square feet, need not exceed 20 square feet.

As

P = ---- +.024

6240

As=Maximum cross-sectional area of the pressurized shell normal to the

longitudinal axis, in square feet; and

(3) The maximum opening caused by airplane or equipment failures not shown

to be extremely improbable.

(f) In complying with paragraph (e) of this section, the fail-safe features

of the design may be considered in determining the probability of failure or

penetration and probable size of openings, provided that possible improper

operation of closure devices and inadvertent door openings are also

considered. Furthermore, the resulting differential pressure loads must be

combined in a rational and conservative manner with 1-g level flight loads

and any loads arising from emergency depressurization conditions. These loads

may be considered as ultimate conditions; however, any deformations

associated with these conditions must not interfere with continued safe

flight and landing. The pressure relief provided by intercompartment venting

may also be considered.

(g) Bulkheads, floors, and partitions in pressurized compartments for

occupants must be designed to withstand the conditions specified in paragraph

(e) of this section. In addition, reasonable design precautions must be taken

to minimize the probability of parts becoming detached and injuring occupants

while in their seats.

EDITORIAL NOTE: At 55 FR 29776, July 20, 1990, Sec. 25.365 was amended by

removing the words "for occupants" from the introductory sentence, but could

not be executed because of an intervening amendment.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-54, 45 FR

60172, Sept. 11, 1980; Amdt. 25-71, 55 FR 13477, Apr. 10, 1990; Amdt. 25-72,

55 FR 29776, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.367  Unsymmetrical loads due to engine failure.

(a) The airplane must be designed for the unsymmetrical loads resulting

from the failure of the critical engine. Turbopropeller airplanes must be

designed for the following conditions in combination with a single

malfunction of the propeller drag limiting system, considering the probable

pilot corrective action on the flight controls:

(1) At speeds between VMC and VD, the loads resulting from power failure

because of fuel flow interruption are considered to be limit loads.

(2) At speeds between VMC and VC, the loads resulting from the

disconnection of the engine compressor from the turbine or from loss of the

turbine blades are considered to be ultimate loads.

(3) The time history of the thrust decay and drag build-up occurring as a

result of the prescribed engine failures must be substantiated by test or

other data applicable to the particular engine-propeller combination.

(4) The timing and magnitude of the probable pilot corrective action must

be conservatively estimated, considering the characteristics of the

particular engine-propeller-airplane combination.

(b) Pilot corrective action may be assumed to be initiated at the time

maximum yawing velocity is reached, but not earlier than two seconds after

the engine failure. The magnitude of the corrective action may be based on

the control forces specified in Sec. 25.397(b) except that lower forces may

be assumed where it is shown by anaylsis or test that these forces can

control the yaw and roll resulting from the prescribed engine failure

conditions.

Sec. 25.371  Gyroscopic loads.

The structure supporting the engines must be designed for gyroscopic loads

associated with the conditions specified in Secs. 25.331, 25.349, and 25.351,

with the engines at maximum continuous r.p.m.

Sec. 25.373  Speed control devices.

If speed control devices (such as spoilers and drag flaps) are installed

for use in en route conditions--

(a) The airplane must be designed for the symmetrical maneuvers and gusts

prescribed in Secs. 25.333, 25.337, and 25.341, and the yawing maneuvers and

lateral gusts in Sec. 25.351, at each setting and the maximum speed

associated with that setting; and

(b) If the device has automatic operating or load limiting features, the

airplane must be designed for the maneuver and gust conditions prescribed in

paragraph (a) of this section, at the speeds and corresponding device

positions that the mechanism allows.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55 FR

29776, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Control Surface and System Loads

Sec. 25.391  Control surface loads: general.

The control surfaces must be designed for the limit loads resulting from

the flight conditions in Secs. 25.331, 25.349, and 25.351 and the ground gust

conditions in Sec. 25.415, considering the requirements for--

(a) Loads parallel to hinge line, in Sec. 25.393;

(b) Pilot effort effects, in Sec. 25.397;

(c) Trim tab effects, in Sec. 25.407;

(d) Unsymmetrical loads, in Sec. 25.427; and

(e) Outboard fins, in Sec. 25.445.

Sec. 25.393  Loads parallel to hinge line.

(a) Control surfaces and supporting hinge brackets must be designed for

inertia loads acting parallel to the hinge line.

(b) In the absence of more rational data, the inertia loads may be assumed

to be equal to KW, where--

(1) K=24 for vertical surfaces;

(2) K=12 for horizontal surfaces; and

(3) W=weight of the movable surfaces.

Sec. 25.395  Control system.

(a) Longitudinal, lateral, directional, and drag control system and their

supporting structures must be designed for loads corresponding to 125 percent

of the computed hinge moments of the movable control surface in the

conditions prescribed in Sec. 25.391.

(b) The system limit loads, except the loads resulting from ground gusts,

need not exceed the loads that can be produced by the pilot (or pilots) and

by automatic or power devices operating the controls.

(c) The loads must not be less than those resulting from application of the

minimum forces prescribed in Sec. 25.397(c).

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5672, Apr. 8, 1970; Amdt. 25-72, 55 FR 29776, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.397  Control system loads.

(a) General. The maximum and minimum pilot forces, specified in paragraph

(c) of this section, are assumed to act at the appropriate control grips or

pads (in a manner simulating flight conditions) and to be reacted at the

attachment of the control system to the control surface horn.

(b) Pilot effort effects. In the control surface flight loading condition,

the air loads on movable surfaces and the corresponding deflections need not

exceed those that would result in flight from the application of any pilot

force within the ranges specified in paragraph (c) of this section. Two-

thirds of the maximum values specified for the aileron and elevator may be

used if control surface hinge moments are based on reliable data. In applying

this criterion, the effects of servo mechanisms, tabs, and automatic pilot

systems, must be considered.

(c) Limit pilot forces and torques. The limit pilot forces and torques are

as follows:

Minimum

Maximum forces     forces or

Control               or torques        torques

Aileron:

Stick                      100 lbs           40 lbs.

Wheel /1/                  80 D in.-lbs /2/  40 D in.-lbs.

Elevator:

Stick                      250 lbs           100 lbs.

Wheel (symmetrical)        300 lbs           100 lbs.

Wheel (unsymmetrical) /3/                    100 lbs.

Rudder                      300 lbs           130 lbs.

/1/ The critical parts of the aileron control system must

be designed for a single tangential force with a limit

value equal to 1.25 times the couple force determined from

these criteria.

/2/ D= wheel diameter (inches).

/3/ The unsymmetrical forces must be applied at one of the

normal handgrip points on the periphery of the control

wheel.

[Doc. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55466, Dec. 20, 1976; Amdt. 25-72, 55 FR 29776, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.399  Dual control system.

(a) Each dual control system must be designed for the pilots operating in

opposition, using individual pilot forces not less than--

(1) 0.75 times those obtained under Sec. 25.395; or

(2) The minimum forces specified in Sec. 25.397(c).

(b) The control system must be designed for pilot forces applied in the

same direction, using individual pilot forces not less than 0.75 times those

obtained under Sec. 25.395.

Sec. 25.405  Secondary control system.

Secondary controls, such as wheel brake, spoiler, and tab controls, must be

designed for the maximum forces that a pilot is likely to apply to those

controls. The following values may be used:

Pilot Control Force Limits (Secondary Controls)

Control                           Limit pilot forces

Miscellaneous:      1+R

*Crank, wheel, or   (------) x 50 lbs., but

lever.              3

not less than 50 lbs. nor more than 150 lbs. (R=radius).

(Applicable to any angle within 20 deg. of plane of

control).

Twist               133 in.-lbs.

Push-pull           To be chosen by applicant.

*Limited to flap, tab, stabilizer, spoiler, and landing gear operation

controls.

Sec. 25.407  Trim tab effects.

The effects of trim tabs on the control surface design conditions must be

accounted for only where the surface loads are limited by maximum pilot

effort. In these cases, the tabs are considered to be deflected in the

direction that would assist the pilot, and the deflections are--

(a) For elevator trim tabs, those required to trim the airplane at any

point within the positive portion of the pertinent flight envelope in Sec.

25.333(b), except as limited by the stops; and

(b) For aileron and rudder trim tabs, those required to trim the airplane

in the critical unsymmetrical power and loading conditions, with appropriate

allowance for rigging tolerances.

Sec. 25.409  Tabs.

(a) Trim tabs. Trim tabs must be designed to withstand loads arising from

all likely combinations of tab setting, primary control position, and

airplane speed (obtainable without exceeding the flight load conditions

prescribed for the airplane as a whole), when the effect of the tab is

opposed by pilot effort forces up to those specified in Sec. 25.397(b).

(b) Balancing tabs. Balancing tabs must be designed for deflections

consistent with the primary control surface loading conditions.

(c) Servo tabs. Servo tabs must be designed for deflections consistent with

the primary control surface loading conditions obtainable within the pilot

maneuvering effort, considering possible opposition from the trim tabs.

Sec. 25.415  Ground gust conditions.

(a) The control system must be designed as follows for control surface

loads due to ground gusts and taxiing downwind:

(1) The control system between the stops nearest the surfaces and the

cockpit controls must be designed for loads corresponding to the limit hinge

moments H of paragraph (a)(2) of this section. These loads need not exceed--

(i) The loads corresponding to the maximum pilot loads in Sec. 25.397(c)

for each pilot alone; or

(ii) 0.75 times these maximum loads for each pilot when the pilot forces

are applied in the same direction.

(2) The control system stops nearest the surfaces, the control system

locks, and the parts of the systems (if any) between these stops and locks

and the control surface horns, must be designed for limit hinge moments H

obtained from the formula, H=KcSsq, where--

H=limit hinge moment (ft. lbs.);

c=mean chord of the control surface aft of the hinge line (ft.);

Ss=area of the control surface aft of the hinge line (sq. ft.);

q=dynamic pressure (p.s.f.) based on a design speed not less than 14.6(W/S)

1/2 +14.6 (f.p.s.), except that the design speed need not exceed 88

f.p.s. (W/S is wing loading based on maximum airplane weight and wing

area); and

K=limit hinge moment factor for ground gusts derived in paragraph (b) of this

section.

(b) The limit hinge moment factor K for ground gusts must be derived as

follows:

Surface           K                      Position of controls

(a) Aileron            0.75  Control column locked or lashed in mid-position.

(b) ......do  /1/ 1 +/-0.50  Ailerons at full throw.

(c) Elevator  /1/ 1 +/-0.75  (c) Elevator full down.

(d) ......do  /1/ 1 +/-0.75  (d) Elevator full up.

(e) Rudder             0.75  (e) Rudder in neutral.

(f) ......do           0.75  (f) Rudder at full throw.

/1/ A positive value of K indicates a moment tending to depress the surface,

while a negative value of K indicates a moment tending to raise the surface.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55 FR

29776, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.427  Unsymmetrical loads.

(a) Horizontal tail surfaces and their supporting structure must be

designed for unsymmetrical loads arising from yawing and slipstream effects,

in combination with the prescribed flight conditions.

(b) In the absence of more rational data, the following apply:

(1) For airplanes that are conventional in regard to location of

propellers, wings, tail surfaces, and fuselage shape--

(i) 100 percent of the maximum loading from the symmetrical flight

conditions may be assumed to act on the surface on one side of the plane of

symmetry; and

(ii) 80 percent of this loading may be assumed to act on the other side.

(2) For empennage arrangements where the horizontal tail surfaces have

appreciable dihedral or are supported by the vertical tail surfaces, the

surfaces and supporting structure must be designed for the combined vertical

and horizontal surface loads resulting from each prescribed flight load

condition considered separately.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.445  Outboard fins.

(a) If outboard fins are on the horizontal tail surface, the tail surfaces

must be designed for the maximum horizontal surface load in combination with

the corresponding loads induced on the vertical surfaces by endplate effects.

These induced effects need not be combined with other vertical surface loads.

(b) To provide for unsymmetrical loading when outboard fins extend above

and below the horizontal surface, the critical vertical surface loading (load

per unit area) determined under Sec. 25.391 must also be applied as follows:

(1) 100 percent to the area of the vertical surfaces above (or below) the

horizontal surface.

(2) 80 percent to the area below (or above) the horizontal surface.

Sec. 25.457  Wing flaps.

Wing flaps, their operating mechanisms, and their supporting structures

must be designed for critical loads occurring in the conditions prescribed in

Sec. 25.345, accounting for the loads occurring during transition from one

flap position and airspeed to another.

Sec. 25.459  Special devices.

The loading for special devices using aerodynamic surfaces (such as slots,

slats, and spoilers) must be determined from test data.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55 FR

29776, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Ground Loads

Sec. 25.471  General.

(a) Loads and equilibrium. For limit ground loads--

(1) Limit ground loads obtained under this subpart are considered to be

external forces applied to the airplane structure; and

(2) In each specified ground load condition, the external loads must be

placed in equilibrium with the linear and angular inertia loads in a rational

or conservative manner.

(b) Critical centers of gravity. The critical centers of gravity within the

range for which certification is requested must be selected so that the

maximum design loads are obtained in each landing gear element. Fore and aft,

vertical, and lateral airplane centers of gravity must be considered. Lateral

displacements of the c.g. from the airplane centerline which would result in

main gear loads not greater than 103 percent of the critical design load for

symmetrical loading conditions may be selected without considering the

effects of these lateral c.g. displacements on the loading of the main gear

elements, or on the airplane structure provided--

(1) The lateral displacement of the c.g. results from random passenger or

cargo disposition within the fuselage or from random unsymmetrical fuel

loading or fuel usage; and

(2) Appropriate loading instructions for random disposable loads are

included under the provisions of Sec. 25.1583(c)(1) to ensure that the

lateral displacement of the center of gravity is maintained within these

limits.

(c) Landing gear dimension data. Figure 1 of Appendix A contains the basic

landing gear dimension data.

[Amdt. 25-23, 35 FR 5673, Apr. 8, 1970]

Sec. 25.473  Ground load conditions and assumptions.

(a) For the landing conditions specified in Secs. 25.479 through 25.485,

the following apply:

(1) The selected limit vertical inertia load factors at the center of

gravity of the airplane may not be less than the values that would be

obtained--

(i) In the attitude and subject to the drag loads associated with the

particular landing condition;

(ii) With a limit descent velocity of 10 f.p.s. at the design landing

weight (the maximum weight for landing conditions at the maximum descent

velocity); and

(iii) With a limit descent velocity of 6 f.p.s. at the design takeoff

weight (the maximum weight for landing conditions at a reduced descent

velocity).

(2) Airplane lift, not exceeding the airplane weight, may be assumed to

exist throughout the landing impact and to act through the center of gravity

of the airplane.

(b) The prescribed descent velocities may be modified if it is shown that

the airplane has design features that make it impossible to develop these

velocities.

(c) The minimum limit inertia load factors corresponding to the required

limit descent velocities must be determined in accordance with Sec.

25.723(a).

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.477  Landing gear arrangement.

Sections 25.479 through 25.485 apply to airplanes with conventional

arrangements of main and nose gears, or main and tail gears, when normal

operating techniques are used.

Sec. 25.479  Level landing conditions.

(a) In the level attitude, the airplane is assumed to contact the ground at

forward velocity components, ranging from VL1 to 1.25 VL2 parallel to the

ground, and to be subjected to the load factors prescribed in Sec.

25.473(a)(1) with--

(1) VL1 equal to VS0 (TAS) at the appropriate landing weight and in

standard sea level conditions; and

(2) VL2 equal to VS0 (TAS) at the appropriate landing weight and altitudes

in a hot day temperature of 41 degrees F. above standard.

(b) The effects of increased contact speeds must be investigated if

approval of downwind landings exceeding 10 knots is desired.

(c) Assuming that the following combinations of vertical and drag

components act at the axle centerline, the following apply:

(1) For the condition of maximum wheel spin-up load, drag components

simulating the forces required to accelerate the wheel rolling assembly up to

the specified ground speed must be combined with the vertical ground

reactions existing at the instant of peak drag loads. The coefficient of

friction between the tires and the ground may be established by considering

the effects of skidding velocity and tire pressure. However, this coefficient

of friction need not be more than 0.8. This condition must be applied to the

landing gear, directly affected attaching structure, and large mass items

such as external fuel tanks and nacelles.

(2) For the condition of maximum wheel vertical load, an aft acting drag

component of not less than 25 percent of the maximum vertical ground reaction

must be combined with the maximum ground reaction of Sec. 25.473.

(3) For the condition of maximum springback load, forward-acting horizontal

loads resulting from a rapid reduction of the spin-up drag loads must be

combined with the vertical ground reactions at the instant of the peak

forward load. This condition must be applied to the landing gear, directly

affected attaching structure, and large mass items such as external fuel

tanks and nacelles.

(d) For the level landing attitude for airplanes with tail wheels, the

conditions specified in paragraphs (a) through (c) of this section must be

investigated with the airplane horizontal reference line horizontal in

accordance with figure 2 of Appendix A.

(e) For the level landing attitude for airplanes with nose wheels, shown in

figure 2 of Appendix A, the conditions specified in paragraphs (a) through

(c) of this section must be investigated, assuming the following attitudes:

(1) An attitude in which the main wheels are assumed to contact the ground

with the nose wheel just clear of the ground.

(2) If reasonably attainable at the specified descent and forward

velocities, an attitude in which the nose and main wheels are assumed to

contact the ground simultaneously. For this attitude--

(i) The nose and main gear may be separately investigated under the

conditions in paragraph (c) (1) and (3) of this section; and

(ii) The pitching moment is assumed, under the condition in paragraph

(c)(2) of this section, to be resisted by the nose gear.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.481  Tail-down landing conditions.

(a) In the tail-down attitude, the airplane is assumed to contact the

ground at forward velocity components, ranging from VL1 to VL2,  parallel to

the ground, and is subjected to the load factors prescribed in Sec.

25.473(a)(1) with--

(1) VL1 equal to VS0 (TAS) at the appropriate landing weight and in

standard sea level conditions; and

(2) VL2 equal to VS0 (TAS) at the appropriate landing weight and altitudes

in a hot day temperature of 41 degrees F. above standard.

The combination of vertical and drag components specified in Sec. 25.479(c)

(1) and (3) is considered to be acting at the main wheel axle centerline.

(b) For the tail-down landing condition for airplanes with tail wheels, the

main and tail wheels are assumed to contact the ground simultaneously, in

accordance with figure 3 of Appendix A. Ground reaction conditions on the

tail wheel are assumed to act--

(1) Vertically; and

(2) Up and aft through the axle at 45 degrees to the ground line.

(c) For the tail-down landing condition for airplanes with nose wheels, the

airplane is assumed to be at an attitude corresponding to either the stalling

angle or the maximum angle allowing clearance with the ground by each part of

the airplane other than the main wheels, in accordance with figure 3 of

Appendix A, whichever is less.

Sec. 25.483  One-wheel landing conditions.

For the one-wheel landing condition, the airplane is assumed to be in the

level attitude and to contact the ground on one side of the main landing

gear, in accordance with Figure 4 of Appendix A. In this attitude--

(a) The ground reactions must be the same as those obtained on that side

under Sec. 25.479(c)(2); and

(b) Each unbalanced external load must be reacted by airplane inertia in a

rational or conservative manner.

Sec. 25.485  Side load conditions.

(a) For the side load condition, the airplane is assumed to be in the level

attitude with only the main wheels contacting the ground, in accordance with

figure 5 of Appendix A.

(b) Side loads of 0.8 of the vertical reaction (on one side) acting inward

and 0.6 of the vertical reaction (on the other side) acting outward must be

combined with one-half of the maximum vertical ground reactions obtained in

the level landing conditions. These loads are assumed to be applied at the

ground contact point and to be resisted by the inertia of the airplane. The

drag loads may be assumed to be zero.

Sec. 25.487  Rebound landing condition.

(a) The landing gear and its supporting structure must be investigated for

the loads occurring during rebound of the airplane from the landing surface.

(b) With the landing gear fully extended and not in contact with the

ground, a load factor of 20.0 must act on the unsprung weights of the landing

gear. This load factor must act in the direction of motion of the unsprung

weights as they reach their limiting positions in extending with relation to

the sprung parts of the landing gear.

Sec. 25.489  Ground handling conditions.

Unless otherwise prescribed, the landing gear and airplane structure must

be investigated for the conditions in Secs. 25.491 through 25.509 with the

airplane at the design ramp weight (the maximum weight for ground handling

conditions). No wing lift may be considered. The shock absorbers and tires

may be assumed to be in their static position.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.491  Takeoff run.

The landing gear and the airplane structure are assumed to be subjected to

loads not less than those obtained under conditions described in Sec. 25.235.

Sec. 25.493  Braked roll conditions.

(a) An airplane with a tail wheel is assumed to be in the level attitude

with the load on the main wheels, in accordance with figure 6 of Appendix A.

The limit vertical load factor is 1.2 at the design landing weight and 1.0 at

the design ramp weight. A drag reaction equal to the vertical reaction

multiplied by a coefficient of friction of 0.8, must be combined with the

vertical ground reaction and applied at the ground contact point.

(b) For an airplane with a nose wheel the limit vertical load factor is 1.2

at the design landing weight, and 1.0 at the design ramp weight. A drag

reaction equal to the vertical reaction, multiplied by a coefficient of

friction of 0.8, must be combined with the vertical reaction and applied at

the ground contact point of each wheel with brakes. The following two

attitudes, in accordance with figure 6 of Appendix A, must be considered:

(1) The level attitude with the wheels contacting the ground and the loads

distributed between the main and nose gear. Zero pitching acceleration is

assumed.

(2) The level attitude with only the main gear contacting the ground and

with the pitching moment resisted by angular acceleration.

(c) A drag reaction lower than that prescribed in paragraphs (a) and (b) of

this section may be used if it is substantiated that an effective drag force

of 0.8 times the vertical reaction cannot be attained under any likely

loading condition.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.495  Turning.

In the static position, in accordance with figure 7 of Appendix A, the

airplane is assumed to execute a steady turn by nose gear steering, or by

application of sufficient differential power, so that the limit load factors

applied at the center of gravity are 1.0 vertically and 0.5 laterally. The

side ground reaction of each wheel must be 0.5 of the vertical reaction.

Sec. 25.497  Tail-wheel yawing.

(a) A vertical ground reaction equal to the static load on the tail wheel,

in combination with a side component of equal magnitude, is assumed.

(b) If there is a swivel, the tail wheel is assumed to be swiveled 90 deg.

to the airplane longitudinal axis with the resultant load passing through the

axle.

(c) If there is a lock, steering device, or shimmy damper the tail wheel is

also assumed to be in the trailing position with the side load acting at the

ground contact point.

Sec. 25.499  Nose-wheel yaw.

(a) A vertical load factor of 1.0 at the airplane center of gravity, and a

side component at the nose wheel ground contact equal to 0.8 of the vertical

ground reaction at that point are assumed.

(b) With the airplane assumed to be in static equilibrium with the loads

resulting from the use of brakes on one side of the main landing gear, the

nose gear, its attaching structure, and the fuselage structure forward of the

center of gravity must be designed for the following loads:

(1) A vertical load factor at the center of gravity of 1.0.

(2) A forward acting load at the airplane center of gravity of 0.8 times

the vertical load on one main gear.

(3) Side and vertical loads at the ground contact point on the nose gear

that are required for static equilibrium.

(4) A side load factor at the airplane center of gravity of zero.

(c) If the loads prescribed in paragraph (b) of this section result in a

nose gear side load higher than 0.8 times the vertical nose gear load, the

design nose gear side load @y be limited to 0.8 times the vertical load,

with unbalanced yawing moments assumed to be resisted by airplane inertia

forces.

(d) For other than the nose gear, its attaching structure, and the forward

fuselage structure, the loading conditions are those prescribed in paragraph

(b) of this s@tion, except that--

(1) A lower drag reaction may be used if an effective drag force of 0.8

times the vertical reaction cannot be reached under any likely loading

condition; and

(2) The forward acting load at the center of gravity need not exceed the

maximum drag reaction on one main gear, determined in accordance with Sec.

25.493(b).

(e) With the airplane at design ramp weight, and the nose gear in any

steerable position, the combined application of full normal steering torque

and a vertical force equal to the maximum static reaction on the nose gear

must be considered in designing the nose gear, its attaching structure, and

the forward fuselage structure.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970; Amdt. 25-46, 43 FR 50595, Oct. 30, 1978]

Sec. 25.503  Pivoting.

(a) The airplane is assumed to pivot about one side of the main gear with

the brakes on that side locked. The limit vertical load factor must be 1.0

and the coefficient of friction 0.8.

(b) The airplane is assumed to be in static equilibrium, with the loads

being applied at the ground contact points, in accordance with figure 8 of

Appendix A.

Sec. 25.507  Reversed braking.

(a) The airplane must be in a three point static ground attitude.

Horizontal reactions parallel to the ground and directed forward must be

applied at the ground contact point of each wheel with brakes. The limit

loads must be equal to 0.55 times the vertical load at each wheel or to the

load developed by 1.2 times the nominal maximum static brake torque,

whichever is less.

(b) For airplanes with nose wheels, the pitching moment must be balanced by

rotational inertia.

(c) For airplanes with tail wheels, the resultant of the ground reactions

must pass through the center of gravity of the airplane.

Sec. 25.509  Towing loads.

(a) The towing loads specified in paragraph (d) of this section must be

considered separately. These loads must be applied at the towing fittings and

must act parallel to the ground. In addition--

(1) A vertical load factor equal to 1.0 must be considered acting at the

center of gravity;

(2) The shock struts and tires must be in their static positions; and

(3) With WT as the design ramp weight, the towing load, FTOW, is--

(i) 0.3 WT for WT less than 30,000 pounds;

(ii) (6WT+450,000)/7 for WT between 30,000 and 100,000 pounds; and

(iii) 0.15 WT for WT over 100,000 pounds.

(b) For towing points not on the landing gear but near the plane of

symmetry of the airplane, the drag and side tow load components specified for

the auxiliary gear apply. For towing points located outboard of the main

gear, the drag and side tow load components specified for the main gear

apply. Where the specified angle of swivel cannot be reached, the maximum

obtainable angle must be used.

(c) The towing loads specified in paragraph (d) of this section must be

reacted as follows:

(1) The side component of the towing load at the main gear must be reacted

by a side force at the static ground line of the wheel to which the load is

applied.

(2) The towing loads at the auxiliary gear and the drag components of the

towing loads at the main gear must be reacted as follows:

(i) A reaction with a maximum value equal to the vertical reaction must be

applied at the axle of the wheel to which the load is applied. Enough

airplane inertia to achieve equilibrium must be applied.

(ii) The loads must be reacted by airplane inertia.

(d) The prescribed towing loads are as follows:

Load

Tow point      Position       Magnitude         No.         Direction

Main gear                     0.75 FTOW per  1              Forward,

main gear      2              parallel to

unit           3              drag axis.

4              Forward, at

30 deg. to

drag axis.

Aft,

parallel to

drag axis.

Aft, at 30

deg. to drag

axis.

Auxiliary      Swiveled       1.0 FTOW       5              Forward.

gear           forward                       6              Aft.

Swiveled aft   ......do       7              Forward.

8              Aft.

Swiveled 45    0.5 FTOW       9              Forward, in

deg. from                     10             plane of

forward                                      wheel.

Aft, in

plane of

wheel.

Swiveled 45    ......do       11             Forward, in

deg. from                     12             plane of

aft                                          wheel.

Aft, in

plane of

wheel.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.511  Ground load: unsymmetrical loads on multiple-wheel units.

(a) General. Multiple-wheel landing gear units are assumed to be subjected

to the limit ground loads prescribed in this subpart under paragraphs (b)

through (f) of this section. In addition--

(1) A tandem strut gear arrangement is a multiple-wheel unit; and

(2) In determining the total load on a gear unit with respect to the

provisions of paragraphs (b) through (f) of this section, the transverse

shift in the load centroid, due to unsymmetrical load distribution on the

wheels, may be neglected.

(b) Distribution of limit loads to wheels; tires inflated. The distribution

of the limit loads among the wheels of the landing gear must be established

for each landing, taxiing, and ground handling condition, taking into account

the effects of the following factors:

(1) The number of wheels and their physical arrangements. For truck type

landing gear units, the effects of any seesaw motion of the truck during the

landing impact must be considered in determining the maximum design loads for

the fore and aft wheel pairs.

(2) Any differentials in tire diameters resulting from a combination of

manufacturing tolerances, tire growth, and tire wear. A maximum tire-diameter

differential equal to 2/3  of the most unfavorable combination of diameter

variations that is obtained when taking into account manufacturing

tolerances, tire growth, and tire wear, may be assumed.

(3) Any unequal tire inflation pressure, assuming the maximum variation to

be +/-5 percent of the nominal tire inflation pressure.

(4) A runway crown of zero and a runway crown having a convex upward shape

that may be approximated by a slope of 1 1/2  percent with the horizontal.

Runway crown effects must be considered with the nose gear unit on either

slope of the crown.

(5) The airplane attitude.

(6) Any structural deflections.

(c) Deflated tires. The effect of deflated tires on the structure must be

considered with respect to the loading conditions specified in paragraphs (d)

through (f) of this section, taking into account the physical arrangement of

the gear components. In addition--

(1) The deflation of any one tire for each multiple wheel landing gear

unit, and the deflation of any two critical tires for each landing gear unit

using four or more wheels per unit, must be considered; and

(2) The ground reactions must be applied to the wheels with inflated tires

except that, for multiple-wheel gear units with more than one shock strut, a

rational distribution of the ground reactions between the deflated and

inflated tires, accounting for the differences in shock strut extensions

resulting from a deflated tire, may be used.

(d) Landing conditions. For one and for two deflated tires, the applied

load to each gear unit is assumed to be 60 percent and 50 percent,

respectively, of the limit load applied to each gear for each of the

prescribed landing conditions. However, for the drift landing condition of

Sec. 25.485, 100 percent of the vertical load must be applied.

(e) Taxiing and ground handling conditions.  For one and for two deflated

tires--

(1) The applied side or drag load factor, or both factors, at the center of

gravity must be the most critical value up to 50 percent and 40 percent,

respectively, of the limit side or drag load factors, or both factors,

corresponding to the most severe condition resulting from consideration of

the prescribed taxiing and ground handling conditions;

(2) For the braked roll conditions of Sec. 25.493 (a) and (b)(2), the drag

loads on each inflated tire may not be less than those at each tire for the

symmetrical load distribution with no deflated tires;

(3) The vertical load factor at the center of gravity must be 60 percent

and 50 percent, respectively, of the factor with no deflated tires, except

that it may not be less than 1g; and

(4) Pivoting need not be considered.

(f) Towing conditions. For one and for two deflated tires, the towing load,

FTOW, must be 60 percent and 50 percent, respectively, of the load

prescribed.

Water Loads

Sec. 25.521  General.

(a) Seaplanes must be designed for the water loads developed during takeoff

and landing, with the seaplane in any attitude likely to occur in normal

operation, and at the appropriate forward and sinking velocities under the

most severe sea conditions likely to be encountered.

(b) Unless a more rational analysis of the water loads is made, or the

standards in ANC-3 are used, Secs. 25.523 through 25.537 apply.

(c) The requirements of this section and Secs. 25.523 through 25.537 apply

also to amphibians.

Sec. 25.523  Design weights and center of gravity positions.

(a) Design weights. The water load requirements must be met at each

operating weight up to the design landing weight except that, for the takeoff

condition prescribed in Sec. 25.531, the design water takeoff weight (the

maximum weight for water taxi and takeoff run) must be used.

(b) Center of gravity positions. The critical centers of gravity within the

limits for which certification is requested must be considered to reach

maximum design loads for each part of the seaplane structure.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.525  Application of loads.

(a) Unless otherwise prescribed, the seaplane as a whole is assumed to be

subjected to the loads corresponding to the load factors specified in Sec.

25.527.

(b) In applying the loads resulting from the load factors prescribed in

Sec. 25.527, the loads may be distributed over the hull or main float bottom

(in order to avoid excessive local shear loads and bending moments at the

location of water load application) using pressures not less than those

prescribed in Sec. 25.533(b).

(c) For twin float seaplanes, each float must be treated as an equivalent

hull on a fictitious seaplane with a weight equal to one-half the weight of

the twin float seaplane.

(d) Except in the takeoff condition of Sec. 25.531, the aerodynamic lift on

the seaplane during the impact is assumed to be 2/3  of the weight of the

seaplane.

Sec. 25.527  Hull and main float load factors.

(a) Water reaction load factors nW must be computed in the following

manner:

(1) For the step landing case

C1VS02

nw =  ----------------

(Tan2/3 b)W1/3

(2) For the bow and stern landing cases

C1VS02               K1

nw =  ----------------  x  ----------

(Tan2/3 b)W1/3       (1+rx2)2/3

(b) The following values are used:

(1) nW=water reaction load factor (that is, the water reaction divided by

seaplane weight).

(2) C1=empirical seaplane operations factor equal to 0.012 (except that

this factor may not be less than that necessary to obtain the minimum value

of step load factor of 2.33).

(3) VS0=seaplane stalling speed in knots with flaps extended in the

appropriate landing position and with no slipstream effect.

(4) <beta>=angle of dead rise at the longitudinal station at which the load

factor is being determined in accordance with figure 1 of Appendix B.

(5) W=seaplane design landing weight in pounds.

(6) K1=empirical hull station weighing factor, in accordance with figure 2

of Appendix B.

(7) rx=ratio of distance, measured parallel to hull reference axis, from

the center of gravity of the seaplane to the hull longitudinal station at

which the load factor is being computed to the radius of gyration in pitch of

the seaplane, the hull reference axis being a straight line, in the plane of

symmetry, tangential to the keel at the main step.

(c) For a twin float seaplane, because of the effect of flexibility of the

attachment of the floats to the seaplane, the factor K1 may be reduced at the

bow and stern to 0.8 of the value shown in figure 2 of Appendix B. This

reduction applies only to the design of the carrythrough and seaplane

structure.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.529  Hull and main float landing conditions.

(a) Symmetrical step, bow, and stern landing. For symmetrical step, bow,

and stern landings, the limit water reaction load factors are those computed

under Sec. 25.527. In addition--

(1) For symmetrical step landings, the resultant water load must be applied

at the keel, through the center of gravity, and must be directed

perpendicularly to the keel line;

(2) For symmetrical bow landings, the resultant water load must be applied

at the keel, one-fifth of the longitudinal distance from the bow to the step,

and must be directed perpendicularly to the keel line; and

(3) For symmetrical stern landings, the resultant water load must be

applied at the keel, at a point 85 percent of the longitudinal distance from

the step to the stern post, and must be directed perpendicularly to the keel

line.

(b) Unsymmetrical landing for hull and single float seaplanes.

Unsymmetrical step, bow, and stern landing conditions must be investigated.

In addition--

(1) The loading for each condition consists of an upward component and a

side component equal, respectively, to 0.75 and 0.25 tan <beta> times the

resultant load in the corresponding symmetrical landing condition; and

(2) The point of application and direction of the upward component of the

load is the same as that in the symmetrical condition, and the point of

application of the side component is at the same longitudinal station as the

upward component but is directed inward perpendicularly to the plane of

symmetry at a point midway between the keel and chine lines.

(c) Unsymmetrical landing; twin float seaplanes. The unsymmetrical loading

consists of an upward load at the step of each float of 0.75 and a side load

of 0.25 tan <beta> at one float times the step landing load reached under

Sec. 25.527. The side load is directed inboard, perpendicularly to the plane

of symmetry midway between the keel and chine lines of the float, at the same

longitudinal station as the upward load.

Sec. 25.531  Hull and main float takeoff condition.

For the wing and its attachment to the hull or main float--

(a) The aerodynamic wing lift is assumed to be zero; and

(b) A downward inertia load, corresponding to a load factor computed from

the following formula, must be applied:

CTOVS12

n =  ----------------

(tan2/3 b)W1/3

where--

n= inertia load factor;

CTO=empirical seaplane operations factor equal to 0.004;

VS1=seaplane stalling speed (knots) at the design takeoff weight with the

flaps extended in the appropriate takeoff position;

<beta>=angle of dead rise at the main step (degrees); and

W=design water takeoff weight in pounds.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.533  Hull and main float bottom pressures.

(a) General. The hull and main float structure, including frames and

bulkheads, stringers, and bottom plating, must be designed under this

section.

(b) Local pressures. For the design of the bottom plating and stringers and

their attachments to the supporting structure, the following pressure

distributions must be applied:

(1) For an unflared bottom, the pressure at the chine is 0.75 times the

pressure at the keel, and the pressures between the keel and chine vary

linearly, in accordance with figure 3 of Appendix B. The pressure at the keel

(psi) is computed as follows:

K2VS12

Pk = C2 x  ------

tan bk

where--

Pk=pressure (p.s.i.) at the keel;

C2=0.00213;

K2=hull station weighing factor, in accordance with figure 2 of Appendix B;

VS1= seaplane stalling speed (Knots) at the design water takeoff weight with

flaps extended in the appropriate takeoff position; and

<beta>k=angle of dead rise at keel, in accordance with figure 1 of Appendix

B.

(2) For a flared bottom, the pressure at the beginning of the flare is the

same as that for an unflared bottom, and the pressure between the chine and

the beginning of the flare varies linearly, in accordance with figure 3 of

Appendix B. The pressure distribution is the same as that prescribed in

paragraph (b)(1) of this section for an unflared bottom except that the

pressure at the chine is computed as follows:

K2VS12

Pch = C3 x  ------

tan b

where--

Pch=pressure (p.s.i.) at the chine;

C3=0.0016;

K2=hull station weighing factor, in accordance with figure 2 of Appendix B;

VS1= seaplane stalling speed at the design water takeoff weight with flaps

extended in the appropriate takeoff position; and

<beta>= angle of dead rise at appropriate station.

The area over which these pressures are applied must simulate pressures

occurring during high localized impacts on the hull or float, but need not

extend over an area that would induce critical stresses in the frames or in

the overall structure.

(c) Distributed pressures. For the design of the frames, keel, and chine

structure, the following pressure distributions apply:

(1) Symmetrical pressures are computed as follows:

K2VS02

P = C4 x  ------

tan b

where--

P =pressure (p.s.i.);

C4 =0.078 C1 (with C1 computed under Sec. 25.527);

K2 =hull station weighing factor, determined in accordance with figure 2 of

Appendix B;

VS0 =seaplane stalling speed (Knots) with landing flaps extended in the

appropriate position and with no slipstream effect; and

VS0 =seaplane stalling speed with landing flaps extended in the appropriate

position and with no slipstream effect; and <beta>=angle of dead rise at

appropriate station.

(2) The unsymmetrical pressure distribution consists of the pressures

prescribed in paragraph (c)(1) of this section on one side of the hull or

main float centerline and one-half of that pressure on the other side of the

hull or main float centerline, in accordance with figure 3 of Appendix B.

These pressures are uniform and must be applied simultaneously over the

entire hull or main float bottom. The loads obtained must be carried into the

sidewall structure of the hull proper, but need not be transmitted in a fore

and aft direction as shear and bending loads.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.535  Auxiliary float loads.

(a) General. Auxiliary floats and their attachments and supporting

structures must be designed for the conditions prescribed in this section. In

the cases specified in paragraphs (b) through (e) of this section, the

prescribed water loads may be distributed over the float bottom to avoid

excessive local loads, using bottom pressures not less than those prescribed

in paragraph (g) of this section.

(b) Step loading. The resultant water load must be applied in the plane of

symmetry of the float at a point three-fourths of the distance from the bow

to the step and must be perpendicular to the keel. The resultant limit load

is computed as follows, except that the value of L need not exceed three

times the weight of the displaced water when the float is completely

submerged:

C5   VSO**2  W 2/3

L= -----------------------------------

tan 2/3 <beta>S (1 + ry**2) **2/3

where--

L =limit load (lbs.);

C5 =0.0053;

VS0 =seaplane stalling speed (knots) with landing flaps extended in the

appropriate position and with no slipstream effect;

W =seaplane design landing weight in pounds;

<beta>S=angle of dead rise at a station 3/4  of the distance from the bow to

the step, but need not be less than 15 degrees; and

ry=ratio of the lateral distance between the center of gravity and the plane

of symmetry of the float to the radius of gyration in roll.

(c) Bow loading. The resultant limit load must be applied in the plane of

symmetry of the float at a point one-fourth of the distance from the bow to

the step and must be perpendicular to the tangent to the keel line at that

point. The magnitude of the resultant load is that specified in paragraph (b)

of this section.

(d) Unsymmetrical step loading. The resultant water load consists of a

component equal to 0.75 times the load specified in paragraph (a) of this

section and a side component equal to 3.25 tan <beta> times the load

specified in paragraph (b) of this section. The side load must be applied

perpendicularly to the plane of symmetry of the float at a point midway

between the keel and the chine.

(e) Unsymmetrical bow loading. The resultant water load consists of a

component equal to 0.75 times the load specified in paragraph (b) of this

section and a side component equal to 0.25 tan <beta> times the load

specified in paragraph (c) of this section. The side load must be applied

perpendicularly to the plane of symmetry at a point midway between the keel

and the chine.

(f) Immersed float condition. The resultant load must be applied at the

centroid of the cross section of the float at a point one-third of the

distance from the bow to the step. The limit load components are as follows:

vertical = <rho>gVSO

aft = Cx2 <rho>V**2/3 (KVSO)**2.

side = Cy2 <rho>V**2/3 (KVSO)**2.

where--

<rho>=mass density of water (slugs/ft.2);

V =volume of float (ft.2);

Cx =coefficient of drag force, equal to 0.133;

Cy =coefficient of side force, equal to 0.106;

K =0.8, except that lower values may be used if it is shown that the floats

are incapable of submerging at a speed of 0.8 VS0 in normal operations;

VS0 =seaplane stalling speed (knots) with landing flaps extended in the

appropriate position and with no slipstream effect; and

g =acceleration due to gravity (ft./sec.2).

(g) Float bottom pressures. The float bottom pressures must be established

under Sec. 25.533, except that the value of K2 in the formulae may be taken

as 1.0. The angle of dead rise to be used in determining the float bottom

pressures is set forth in paragraph (b) of this section.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970]

Sec. 25.537  Seawing loads.

Seawing design loads must be based on applicable test data.

Emergency Landing Conditions

Sec. 25.561  General.

(a) The airplane, although it may be damaged in emergency landing

conditions on land or water, must be designed as prescribed in this section

to protect each occupant under those conditions.

(b) The structure must be designed to give each occupant every reasonable

chance of escaping serious injury in a minor crash landing when--

(1) Proper use is made of seats, belts, and all other safety design

provisions;

(2) The wheels are retracted (where applicable); and

(3) The occupant experiences the following ultimate inertia forces acting

separately relative to the surrounding structure:

(i) Upward, 3.0g

(ii) Forward, 9.0g

(iii) Sideward, 3.0g on the airframe; and 4.0g on the seats and their

attachments.

(iv) Downward, 6.0g

(v) Rearward, 1.5g

(c) The supporting structure must be designed to restrain, under all loads

up to those specified in paragraph (b)(3) of this section, each item of mass

that could injure an occupant if it came loose in a minor crash landing.

(d) Seats and items of mass (and their supporting structure) must not

deform under any loads up to those specified in paragraph (b)(3) of this

section in any manner that would impede subsequent rapid evacuation of

occupants.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5673, Apr. 8, 1970; Amdt. 25-64, 53 FR 17646, May 17, 1988]

Sec. 25.562  Emergency landing dynamic conditions.

(a) The seat and restraint system in the airplane must be designed as

prescribed in this section to protect each occupant during an emergency

landing condition when--

(1) Proper use is made of seats, safety belts, and shoulder harnesses

provided for in the design; and

(2) The occupant is exposed to loads resulting from the conditions

prescribed in this section.

(b) Each seat type design approved for crew or passenger occupancy during

takeoff and landing must successfully complete dynamic tests or be

demonstrated by rational analysis based on dynamic tests of a similar type

seat, in accordance with each of the following emergency landing conditions.

The tests must be conducted with an occupant simulated by a 170-pound

anthropomorphic test dummy, as defined by 49 CFR Part 572, Subpart B, or its

equivalent, sitting in the normal upright position.

(1) A change in downward vertical velocity (Dv) of not less than 35 feet

per second, with the airplane's longitudinal axis canted downward 30 degrees

with respect to the horizontal plane and with the wings level. Peak floor

deceleration must occur in not more than 0.08 seconds after impact and must

reach a minimum of 14g.

(2) A change in forward longitudinal velocity (Dv) of not less than 44 feet

per second, with the airplane's longitudinal axis horizontal and yawed 10

degrees either right or left, whichever would cause the greatest likelihood

of the upper torso restraint system (where installed) moving off the

occupant's shoulder, and with the wings level. Peak floor deceleration must

occur in not more than 0.09 seconds after impact and must reach a minimum of

16g. Where floor rails or floor fittings are used to attach the seating

devices to the test fixture, the rails or fittings must be misaligned with

respect to the adjacent set of rails or fittings by at least 10 degrees

vertically (i.e., out of Parallel) with one rolled 10 degrees.

(c) The following performance measures must not be exceeded during the

dynamic tests conducted in accordance with paragraph (b) of this section:

(1) Where upper torso straps are used for crewmembers, tension loads in

individual straps must not exceed 1,750 pounds. If dual straps are used for

restraining the upper torso, the total strap tension loads must not exceed

2,000 pounds.

(2) The maximum compressive load measured between the pelvis and the lumbar

column of the anthropomorphic dummy must not exceed 1,500 pounds.

(3) The upper torso restraint straps (where installed) must remain on the

occupant's shoulder during the impact.

(4) The lap safety belt must remain on the occupant's pelvis during the

impact.

(5) Each occupant must be protected from serious head injury under the

conditions prescribed in paragraph (b) of this section. Where head contact

with seats or other structure can occur, protection must be provided so that

the head impact does not exceed a Head Injury Criterion (HIC) of 1,000 units.

The level of HIC is defined by the equation:

1         t22.5

HCI =     {  [  (t2-t1) [--------  I        a(t)dt ]  }

(t2-t1)     t1               max

Where:

t1 is the initial integration time,

t2 is the final integration time, and

a(t) is the total acceleration vs. time curve for the head strike, and where

(t) is in seconds, and (a) is in units of gravity (g).

(6) Where leg injuries may result from contact with seats or other

structure, protection must be provided to prevent axially compressive loads

exceeding 2,250 pounds in each femur.

(7) The seat must remain attached at all points of attachment, although the

structure may have yielded.

(8) Seats must not yield under the tests specified in paragraphs (b)(1) and

(b)(2) of this section to the extent they would impede rapid evacuation of

the airplane occupants.

[Amdt. 25-64, 53 FR 17646, May 17, 1988]

Sec. 25.563  Structural ditching provisions.

Structural strength considerations of ditching provisions must be in

accordance with Sec. 25.801(e).

Fatigue Evaluation

Sec. 25.571  Damage--tolerance and fatigue evaluation of structure.

(a) General. An evaluation of the strength, detail design, and fabrication

must show that catastrophic failure due to fatigue, corrosion, or accidental

damage, will be avoided throughout the operational life of the airplane. This

evaluation must be conducted in accordance with the provisions of paragraphs

(b) and (e) of this section, except as specified in paragraph (c) of this

section, for each part of the structure which could contribute to a

catastrophic failure (such as wing, empennage, control surfaces and their

systems, the fuselage, engine mounting, landing gear, and their related

primary attachments). Advisory Circular AC No. 25.571-1 contains guidance

information relating to the requirements of this section (copies of the

advisory circular may be obtained from the U.S. Department of Transportation,

Publications Section M443.1, Washington, D.C. 20590). For turbojet powered

airplanes, those parts which could contribute to a catastrophic failure must

also be evaluated under paragraph (d) of this section. In addition, the

following apply:

(1) Each evaluation required by this section must include--

(i) The typical loading spectra, temperatures, and humidities expected in

service;

(ii) The identification of principal structural elements and detail design

points, the failure of which could cause catastrophic failure of the

airplane; and

(iii) An analysis, supported by test evidence, of the principal structural

elements and detail design points identified in paragraph (a)(1)(ii) of this

section.

(2) The service history of airplanes of similar structural design, taking

due account of differences in operating conditions and procedures, may be

used in the evaluations required by this section.

(3) Based on the evaluations required by this section, inspections or other

procedures must be established as necessary to prevent catastrophic failure,

and must be included in the Airworthiness Limitations section of the

Instruction for Continued Airworthiness required by Sec. 25.1529.

(b) Damage-tolerance evaluation. The evaluation must include a

determination of the probable locations and modes of damage due to fatigue,

corrosion, or accidental damage. The determination must be by analysis

supported by test evidence and (if available) service experience. Damage at

multiple sites due to prior fatigue exposure must be included where the

design is such that this type of damage can be expected to occur. The

evaluation must incorporate repeated load and static analyses supported by

test evidence. The extent of damage for residual strength evaluation at any

time within the operational life must be consistent with the initial

detectability and subsequent growth under repeated loads. The residual

strength evaluation must show that the remaining structure is able to

withstand loads (considered as static ultimate loads) corresponding to the

following conditions:

(1) The limit symmetrical maneuvering conditions specified in Sec. 25.337

at VC and in Sec. 25.345.

(2) The limit gust condition specified in Secs. 25.305(d), 25.341, and

25.351(b) at the specified speeds up to Vc, and in Sec. 25.345.

(3) The limit rolling conditions specified in Sec. 25.349 and the limit

unsymmetrical conditions specified in Secs. 25.367 and 25.427, at speeds up

to VC.

(4) The limit yaw maneuvering conditions specified in Sec. 25.351(a) at the

specified speeds up to VC.

(5) For pressurized cabins, the following conditions:

(i) The normal operating differential pressure combined with the expected

external aerodynamic pressures applied simultaneously with the flight loading

conditions specified in paragraphs (b) (1) through (4) of this section, if

they have a significant effect.

(ii) The expected external aerodynamic pressures in 1 g flight combined

with a cabin differential pressure equal to 1.1 times the normal operating

differential pressure without any other load.

(6) For landing gear and directly-affected airframe structure, the limit

ground loading conditions specified in Secs. 25.473, 25.491, and 25.493.

If significant changes in structural stiffness or geometry, or both, follow

from a structural failure, or partial failure, the effect on damage tolerance

must be further investigated.

(c) Fatigue (safe-life) evaluation. Compliance with the damage-tolerance

requirements of paragraph (b) of this section is not required if the

applicant establishes that their application for particular structure is

impractical. This structure must be shown by analysis, supported by test

evidence, to be able to withstand the repeated loads of variable magnitude

expected during its service life without detectable cracks. Appropriate safe-

life scatter factors must be applied.

(d) Sonic fatigue strength. It must be shown by analysis, supported by test

evidence, or by the service history of airplanes of similar structural design

and sonic excitation environment, that--

(1) Sonic fatigue cracks are not probable in any part of the flight

structure subject to sonic excitation; or

(2) Catastrophic failure caused by sonic cracks is not probable assuming

that the loads prescribed in paragraph (b) of this section are applied to all

areas affected by those cracks.

(e) Damage-tolerance (discrete source) evaluation. The airplane must be

capable of successfully completing a flight during which likely structural

damage occurs as a result of--

(1) Impact with a 4-pound bird at Vc at sea level to 8,000 feet;

(2) Uncontained fan blade impact;

(3) Uncontained engine failure; or

(4) Uncontained high energy rotating machinery failure.

The damaged structure must be able to withstand the static loads (considered

as ultimate loads) which are reasonably expected to occur on the flight.

Dynamic effects on these static loads need not be considered. Corrective

action to be taken by the pilot following the incident, such as limiting

maneuvers, avoiding turbulence, and reducing speed, must be considered. If

significant changes in structural stiffness or geometry, or both, follow from

a structural failure or partial failure, the effect on damage tolerance must

be further investigated.

[Amdt. 25-45, 43 FR 46242, Oct. 5, 1978, as amended by Amdt. 25-54, 45 FR

60173, Sept. 11, 1980; Amdt. 25-72, 55 FR 29776, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Lightning Protection

Sec. 25.581  Lightning protection.

(a) The airplane must be protected against catastrophic effects from

lightning.

(b) For metallic components, compliance with paragraph (a) of this section

may be shown by--

(1) Bonding the components properly to the airframe; or

(2) Designing the components so that a strike will not endanger the

airplane.

(c) For nonmetallic components, compliance with paragraph (a) of this

section may be shown by--

(1) Designing the components to minimize the effect of a strike; or

(2) Incorporating acceptable means of diverting the resulting electrical

current so as not to endanger the airplane.

[Amdt. 25-23, 35 FR 5674, Apr. 8, 1970]

Subpart D--Design and Construction

General

Sec. 25.601  General.

The airplane may not have design features or details that experience has

shown to be hazardous or unreliable. The suitability of each questionable

design detail and part must be established by tests.

Sec. 25.603  Materials.

The suitability and durability of materials used for parts, the failure of

which could adversely affect safety, must--

(a) Be established on the basis of experience or tests;

(b) Conform to approved specifications (such as industry or military

specifications, or Technical Standard Orders) that ensure their having the

strength and other properties assumed in the design data; and

(c) Take into account the effects of environmental conditions, such as

temperature and humidity, expected in service.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55466, Dec. 20 1976; Amdt. 25-46, 43 FR 50595, Oct. 30, 1978]

Sec. 25.605  Fabrication methods.

(a) The methods of fabrication used must produce a consistently sound

structure. If a fabrication process (such as gluing, spot welding, or heat

treating) requires close control to reach this objective, the process must be

performed under an approved process specification.

(b) Each new aircraft fabrication method must be substantiated by a test

program.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-46, 43 FR

50595, Oct. 30, 1978]

Sec. 25.607  Fasteners.

(a) Each removable bolt, screw, nut, pin, or other removable fastener must

incorporate two separate locking devices if--

(1) Its loss could preclude continued flight and landing within the design

limitations of the airplane using normal pilot skill and strength; or

(2) Its loss could result in reduction in pitch, yaw, or roll control

capability or response below that required by Subpart B of this chapter.

(b) The fasteners specified in paragraph (a) of this section and their

locking devices may not be adversely affected by the environmental conditions

associated with the particular installation.

(c) No self-locking nut may be used on any bolt subject to rotation in

operation unless a nonfriction locking device is used in addition to the

self-locking device.

[Amdt. 25-23, 35 FR 5674, Apr. 8, 1970]

Sec. 25.609  Protection of structure.

Each part of the structure must--

(a) Be suitably protected against deterioration or loss of strength in

service due to any cause, including--

(1) Weathering;

(2) Corrosion; and

(3) Abrasion; and

(b) Have provisions for ventilation and drainage where necessary for

protection.

Sec. 25.611  Accessibility provisions.

Means must be provided to allow inspection (including inspection of

principal structural elements and control systems), replacement of parts

normally requiring replacement, adjustment, and lubrication as necessary for

continued airworthiness. The inspection means for each item must be

practicable for the inspection interval for the item. Nondestructive

inspection aids may be used to inspect structural elements where it is

impracticable to provide means for direct visual inspection if it is shown

that the inspection is effective and the inspection procedures are specified

in the maintenance manual required by Sec. 25.1529.

[Amdt. 25-23, 35 FR 5674, Apr. 8, 1970]

Sec. 25.613  Material strength properties and design values.

(a) Material strength properties must be based on enough tests of material

meeting approved specifications to establish design values on a statistical

basis.

(b) Design values must be chosen to minimize the probability of structural

failures due to material variability. Except as provided in paragraph (e) of

this section, compliance with this paragraph must be shown by selecting

design values which assure material strength with the following probability:

(1) Where applied loads are eventually distributed through a single member

within an assembly, the failure of which would result in loss of structural

integrity of the component, 99 percent probability with 95 percent

confidence.

(2) For redundant structure, in which the failure of individual elements

would result in applied loads being safely distributed to other load carrying

members, 90 percent probability with 95 percent confidence.

(c) The effects of temperature on allowable stresses used for design in an

essential component or structure must be considered where thermal effects are

significant under normal operating conditions.

(d) The strength, detail design, and fabrication of the structure must

minimize the probability of disastrous fatigue failure, particularly at

points of stress concentration.

(e) Greater design values may be used if a "premium selection" of the

material is made in which a specimen of each individual item is tested before

use to determine that the actual strength properties of that particular item

will equal or exceed those used in design.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-46, 43 FR

50595, Oct. 30, 1978; Amdt. 25-72, 55 FR 29776, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.615  [Removed.  Amdt. 25-72, 55 FR 29776, July 20, 1990]

EDITORIAL NOTE: For the convenience of the user, the removed text is

set out below.

Sec. 25.615  Design properties.

(a) Design properties outlined in MIL-HDBK-5 may be used subject to the

following conditions:

(1) Where applied loads are eventually distributed through a single member

within an assembly, the failure of which would result in the loss of the

structural integrity of the component involved, the guaranteed minimum design

mechanical properties ("A" values) when listed in MIL-HDBK-5 must be met.

(2) Redundant structures, in which the failure of individual elements would

result in applied loads being safely distributed to other load-carrying

members, may be designed on the basis of the "90 percent probability ("B"

values)" when listed in MIL-HDBK-5.

(b) Design values greater than the guaranteed minimums required by

paragraph (a) of this section may be used where only guaranteed minimum

values are normally allowed if a "premium selection" of the material is made

in which a specimen of each individual item is tested before use to determine

that the actual strength properties of that particular item will equal or

exceed those used in design.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5674. Apr. 8, 1970]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.619  Special factors.

The factor of safety prescribed in Sec. 25.303 must be multiplied by the

highest pertinent special factor of safety prescribed in Secs. 25.621 through

25.625 for each part of the structure whose strength is--

(a) Uncertain;

(b) Likely to deteriorate in service before normal replacement; or

(c) Subject to appreciable variability because of uncertainties in

manufacturing processes or inspection methods.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5674, Apr. 8, 1970]

Sec. 25.621  Casting factors.

(a) General. The factors, tests, and inspections specified in paragraphs

(b) through (d) of this section must be applied in addition to those

necessary to establish foundry quality control. The inspections must meet

approved specifications. Paragraphs (c) and (d) of this section apply to any

structural castings except castings that are pressure tested as parts of

hydraulic or other fluid systems and do not support structural loads.

(b) Bearing stresses and surfaces. The casting factors specified in

paragraphs (c) and (d) of this section--

(1) Need not exceed 1.25 with respect to bearing stresses regardless of the

method of inspection used; and

(2) Need not be used with respect to the bearing surfaces of a part whose

bearing factor is larger than the applicable casting factor.

(c) Critical castings. For each casting whose failure would preclude

continued safe flight and landing of the airplane or result in serious injury

to occupants, the following apply:

(1) Each critical casting must--

(i) Have a casting factor of not less than 1.25; and

(ii) Receive 100 percent inspection by visual, radiographic, and magnetic

particle or penetrant inspection methods or approved equivalent

nondestructive inspection methods.

(2) For each critical casting with a casting factor less than 1.50, three

sample castings must be static tested and shown to meet--

(i) The strength requirements of Sec. 25.305 at an ultimate load

corresponding to a casting factor of 1.25; and

(ii) The deformation requirements of Sec. 25.305 at a load of 1.15 times

the limit load.

(3) Examples of these castings are structural attachment fittings, parts of

flight control systems, control surface hinges and balance weight

attachments, seat, berth, safety belt, and fuel and oil tank supports and

attachments, and cabin pressure valves.

(d) Noncritical castings. For each casting other than those specified in

paragraph (c) of this section, the following apply:

(1) Except as provided in paragraphs (d) (2) and (3) of this section, the

casting factors and corresponding inspections must meet the following table:

Casting factor                            Inspection

2.0 or more                      100 percent visual.

Less than 2.0 but more than 1.5  100 percent visual, and magnetic particle or

penetrant or equivalent nondestructive

inspection methods.

1.25 through 1.50                100 percent visual, magnetic particle or

penetrant, and radiographic, or approved

equivalent nondestructive inspection

methods.

(2) The percentage of castings inspected by nonvisual methods may be

reduced below that specified in paragraph (d)(1) of this section when an

approved quality control procedure is established.

(3) For castings procured to a specification that guarantees the mechanical

properties of the material in the casting and provides for demonstration of

these properties by test of coupons cut from the castings on a sampling

basis--

(i) A casting factor of 1.0 may be used; and

(ii) The castings must be inspected as provided in paragraph (d)(1) of this

section for casting factors of "1.25 through 1.50" and tested under paragraph

(c)(2) of this section.

Sec. 25.623  Bearing factors.

(a) Except as provided in paragraph (b) of this section, each part that has

clearance (free fit), and that is subject to pounding or vibration, must have

a bearing factor large enough to provide for the effects of normal relative

motion.

(b) No bearing factor need be used for a part for which any larger special

factor is prescribed.

Sec. 25.625  Fitting factors.

For each fitting (a part or terminal used to join one structural member to

another), the following apply:

(a) For each fitting whose strength is not proven by limit and ultimate

load tests in which actual stress conditions are simulated in the fitting and

surrounding structures, a fitting factor of at least 1.15 must be applied to

each part of--

(1) The fitting;

(2) The means of attachment; and

(3) The bearing on the joined members.

(b) No fitting factor need be used--

(1) For joints made under approved practices and based on comprehensive

test data (such as continuous joints in metal plating, welded joints, and

scarf joints in wood); or

(2) With respect to any bearing surface for which a larger special factor

is used.

(c) For each integral fitting, the part must be treated as a fitting up to

the point at which the section properties become typical of the member.

(d) For each seat, berth, safety belt, and harness, the fitting factor

specified in Sec. 25.785(f)(3) applies.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5674, Apr. 8, 1970; Amdt. 25-72, 55 FR 29776, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.629   Aeroelastic stability requirements.

(a) General. The aeroelastic stability evaluations required under this

section include flutter, divergence, control reversal and any undue loss of

stability and control as a result of structural deformation. The aeroelastic

evaluation must include whirl modes associated with any propeller or rotating

device that contributes significant dynamic forces. Compliance with this

section must be shown by analyses, wind tunnel tests, ground vibration tests,

flight tests, or other means found necessary by the Administrator.

(b) Aeroelastic stability envelopes. The airplane must be designed to be

free from aeroelastic instability for all configurations and design

conditions within the aeroelastic stability envelopes as follows:

(1) For normal conditions without failures, malfunctions, or adverse

conditions, all combinations of altitudes and speeds encompassed by the VD/MD

versus altitude envelope enlarged at all points by an increase of 15 percent

in equivalent airspeed at both constant Mach number and constant altitude. In

addition, a proper margin of stability must exist at all speeds up to VD/MD

and, there must be no large and rapid reduction in stability as VD/MD is

approached. The enlarged envelope may be limited to Mach 1.0 when MD is less

than 1.0 at all design altitudes, and

(2) For the conditions described in Sec. 25.629(d) below, for all approved

altitudes, any airspeed up to the greater airspeed defined by;

(i) The VD/MD envelope determined by Sec. 25.335(b); or,

(ii) An altitude-airspeed envelope defined by a 15 percent increase in

equivalent airspeed above VC at constant altitude, from sea level to the

altitude of the intersection of 1.15 VC with the extension of the constant

cruise Mach number line, MC, then a linear variation in equivalent airspeed

to MC+.05 at the altitude of the lowest VC/MC intersection; then, at higher

altitudes, up to the maximum flight altitude, the boundary defined by a .05

Mach increase in MC at constant altitude.

(c) Balance weights. If concentrated balance weights are used, their

effectiveness and strength, including supporting structure, must be

substantiated.

(d) Failures, malfunctions, and adverse conditions. The failures,

malfunctions, and adverse conditions which must be considered in showing

compliance with this section are:

(1) Any critical fuel loading conditions, not shown to be extremely

improbable, which may result from mismanagement of fuel.

(2) Any single failure in any flutter damper system.

(3) For airplanes not approved for operation in icing conditions, the

maximum likely ice accumulation expected as a result of an inadvertent

encounter.

(4) Failure of any single element of the structure supporting any engine,

independently mounted propeller shaft, large auxiliary power unit, or large

externally mounted aerodynamic body (such as an external fuel tank).

(5) For airplanes with engines that have propellers or large rotating

devices capable of significant dynamic forces, any single failure of the

engine structure that would reduce the rigidity of the rotational axis.

(6) The absence of aerodynamic or gyroscopic forces resulting from the most

adverse combination of feathered propellers or other rotating devices capable

of significant dynamic forces. In addition, the effect of a single feathered

propeller or rotating device must be coupled with the failures of paragraphs

(d)(4) and (d)(5) of this section.

(7) Any single propeller or rotating device capable of significant dynamic

forces rotating at the highest likely overspeed.

(8) Any damage or failure condition, required or selected for investigation

by Sec. 25.571. The single structural failures described in paragraphs (d)(4)

and (d)(5) of this section need not be considered in showing compliance with

this section if;

(i) The structural element could not fail due to discrete source damage

resulting from the conditions described in Sec. 25.571(e), and

(ii) A damage tolerance investigation in accordance with Sec. 25.571(b)

shows that the maximum extent of damage assumed for the purpose of residual

strength evaluation does not involve complete failure of the structural

element.

(9) Any damage, failure, or malfunction considered under Secs. 25.631,

25.671, 25.672, and 25.1309.

(10) Any other combination of failures, malfunctions, or adverse conditions

not shown to be extremely improbable.

(e) Flight flutter testing. Full scale flight flutter tests at speeds up to

VDF/MDF must be conducted for new type designs and for modifications to a

type design unless the modifications have been shown to have an insignificant

effect on the aeroelastic stability. These tests must demonstrate that the

airplane has a proper margin of damping at all speeds up to VDF/MDF, and that

there is no large and rapid reduction in damping as VDF/MDF, is approached.

If a failure, malfunction, or adverse condition is simulated during flight

test in showing compliance with paragraph (d) of this section, the maximum

speed investigated need not exceed VFC/MFC if it is shown, by correlation of

the flight test data with other test data or analyses, that the airplane is

free from any aeroelastic instability at all speeds within the altitude-

airspeed envelope described in paragraph (b)(2) of this section.

[57 FR 28949, June 29, 1992]

*****************************************************************************

57 FR 28946, No. 125, June 29, 1992

SUMMARY: This amendment revises the airworthiness standards of the Federal

Aviation Regulations (FAR) for type certification of transport category

airplanes concerning vibration, buffet, flutter and divergence. It clarifies

the requirement to consider flutter and divergence when treating certain

damage and failure conditions required by other sections of the FAR and

adjusts the safety margins related to aeroelastic stabiity to make them more

appropriate for the conditions to which they apply. These changes are made to

provide consistency with other sections of the FAR and to take into account

advances in technology and the evolution of the design of transport

airplanes.

EFFECTIVE DATE: July 29, 1992.

*****************************************************************************

Sec. 25.631  Bird strike damage.

The empennage structure must be designed to assure capability of continued

safe flight and landing of the airplane after impact with an 8-pound bird

when the velocity of the airplane (relative to the bird along the airplane's

flight path) is equal to VC at sea level, selected under Sec. 25.335(a).

Compliance with this section by provision of redundant structure and

protected location of control system elements or protective devices such as

splitter plates or energy absorbing material is acceptable. Where compliance

is shown by analysis, tests, or both, use of data on airplanes having similar

structural design is acceptable.

[Amdt. 25-23, 35 FR 5674, Apr. 8, 1970]

Control Surfaces

Sec. 25.651  Proof of strength.

(a) Limit load tests of control surfaces are required. These tests must

include the horn or fitting to which the control system is attached.

(b) Compliance with the special factors requirements of Secs. 25.619

through 25.625 and 25.657 for control surface hinges must be shown by

analysis or individual load tests.

Sec. 25.655  Installation.

(a) Movable tail surfaces must be installed so that there is no

interference between any surfaces when one is held in its extreme position

and the others are operated through their full angular movement.

(b) If an adjustable stabilizer is used, it must have stops that will limit

its range of travel to the maximum for which the airplane is shown to meet

the trim requirements of Sec. 25.161.

Sec. 25.657  Hinges.

(a) For control surface hinges, including ball, roller, and self-lubricated

bearing hinges, the approved rating of the bearing may not be exceeded. For

nonstandard bearing hinge configurations, the rating must be established on

the basis of experience or tests and, in the absence of a rational

investigation, a factor of safety of not less than 6.67 must be used with

respect to the ultimate bearing strength of the softest material used as a

bearing.

(b) Hinges must have enough strength and rigidity for loads parallel to the

hinge line.

[Amdt. 25-23, 35 FR 5674, Apr. 8, 1970]

Control Systems

Sec. 25.671  General.

(a) Each control and control system must operate with the ease, smoothness,

and positiveness appropriate to its function.

(b) Each element of each flight control system must be designed, or

distinctively and permanently marked, to minimize the probability of

incorrect assembly that could result in the malfunctioning of the system.

(c) The airplane must be shown by analysis, tests, or both, to be capable

of continued safe flight and landing after any of the following failures or

jamming in the flight control system and surfaces (including trim, lift,

drag, and feel systems), within the normal flight envelope, without requiring

exceptional piloting skill or strength. Probable malfunctions must have only

minor effects on control system operation and must be capable of being

readily counteracted by the pilot.

(1) Any single failure, excluding jamming (for example, disconnection or

failure of mechanical elements, or structural failure of hydraulic

components, such as actuators, control spool housing, and valves).

(2) Any combination of failures not shown to be extremely improbable,

excluding jamming (for example, dual electrical or hydraulic system failures,

or any single failure in combination with any probable hydraulic or

electrical failure).

(3) Any jam in a control position normally encountered during takeoff,

climb, cruise, normal turns, descent, and landing unless the jam is shown to

be extremely improbable, or can be alleviated. A runaway of a flight control

to an adverse position and jam must be accounted for if such runaway and

subsequent jamming is not extremely improbable.

(d) The airplane must be designed so that it is controllable if all engines

fail. Compliance with this requirement may be shown by analysis where that

method has been shown to be reliable.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5674, Apr. 8, 1970]

Sec. 25.672  Stability augmentation and automatic and power-operated systems.

If the functioning of stability augmentation or other automatic or power-

operated systems is necessary to show compliance with the flight

characteristics requirements of this part, such systems must comply with Sec.

25.671 and the following:

(a) A warning which is clearly distinguishable to the pilot under expected

flight conditions without requiring his attention must be provided for any

failure in the stability augmentation system or in any other automatic or

power-operated system which could result in an unsafe condition if the pilot

were not aware of the failure. Warning systems must not activate the control

systems.

(b) The design of the stability augmentation system or of any other

automatic or power-operated system must permit initial counteraction of

failures of the type specified in Sec. 25.671(c) without requiring

exceptional pilot skill or strength, by either the deactivation of the

system, or a failed portion thereof, or by overriding the failure by movement

of the flight controls in the normal sense.

(c) It must be shown that after any single failure of the stability

augmentation system or any other automatic or power-operated system--

(1) The airplane is safely controllable when the failure or malfunction

occurs at any speed or altitude within the approved operating limitations

that is critical for the type of failure being considered;

(2) The controllability and maneuverability requirements of this part are

met within a practical operational flight envelope (for example, speed,

altitude, normal acceleration, and airplane configurations) which is

described in the Airplane Flight Manual; and

(3) The trim, stability, and stall characteristics are not impaired below a

level needed to permit continued safe flight and landing.

[Amdt. 25-23, 35 FR 5675 Apr. 8, 1970]

Sec. 25.673  [Removed.  Amdt. 25-72, 55 FR 29777, July 20, 1990]

EDITORIAL NOTE: For the convenience of the user, the removed text is set

out below.

Sec. 25.673  Two-control airplanes.

Two-control airplanes must be able to continue safely in flight and landing

if any one connecting element in the directional-lateral flight control

system fails.

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.675  Stops.

(a) Each control system must have stops that positively limit the range of

motion of each movable aerodynamic surface controlled by the system.

(b) Each stop must be located so that wear, slackness, or take-up

adjustments will not adversely affect the control characteristics of the

airplane because of a change in the range of surface travel.

(c) Each stop must be able to withstand any loads corresponding to the

design conditions for the control system.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55466, Dec. 20, 1976]

Sec. 25.677  Trim systems.

(a) Trim controls must be designed to prevent inadvertent or abrupt

operation and to operate in the plane, and with the sense of motion, of the

airplane.

(b) There must be means adjacent to the trim control to indicate the

direction of the control movement relative to the airplane motion. In

addition, there must be clearly visible means to indicate the position of the

trim device with respect to the range of adjustment.

(c) Trim control systems must be designed to prevent creeping in flight.

Trim tab controls must be irreversible unless the tab is appropriately

balanced and shown to be free from flutter.

(d) If an irreversible tab control system is used, the part from the tab to

the attachment of the irreversible unit to the airplane structure must

consist of a rigid connection.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5675, Apr. 8, 1970]

Sec. 25.679  Control system gust locks.

(a) There must be a device to prevent damage to the control surfaces

(including tabs), and to the control system, from gusts striking the airplane

while it is on the ground or water. If the device, when engaged, prevents

normal operation of the control surfaces by the pilot, it must--

(1) Automatically disengage when the pilot operates the primary flight

controls in a normal manner; or

(2) Limit the operation of the airplane so that the pilot receives

unmistakable warning at the start of takeoff.

(b) The device must have means to preclude the possibility of it becoming

inadvertently engaged in flight.

Sec. 25.681  Limit load static tests.

(a) Compliance with the limit load requirements of this Part must be shown

by tests in which--

(1) The direction of the test loads produces the most severe loading in the

control system; and

(2) Each fitting, pulley, and bracket used in attaching the system to the

main structure is included.

(b) Compliance must be shown (by analyses or individual load tests) with

the special factor requirements for control system joints subject to angular

motion.

Sec. 25.683  Operation tests.

It must be shown by operation tests that when portions of the control

system subject to pilot effort loads are loaded to 80 percent of the limit

load specified for the system and the powered portions of the control system

are loaded to the maximum load expected in normal operation, the system is

free from--

(a) Jamming;

(b) Excessive friction; and

(c) Excessive deflection.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5675, Apr. 8, 1970]

Sec. 25.685  Control system details.

(a) Each detail of each control system must be designed and installed to

prevent jamming, chafing, and interference from cargo, passengers, loose

objects, or the freezing of moisture.

(b) There must be means in the cockpit to prevent the entry of foreign

objects into places where they would jam the system.

(c) There must be means to prevent the slapping of cables or tubes against

other parts.

(d) Sections 25.689 and 25.693 apply to cable systems and joints.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55466, Dec. 20, 1976]

Sec. 25.689  Cable systems.

(a) Each cable, cable fitting, turnbuckle, splice, and pulley must be

approved. In addition--

(1) No cable smaller than 1/8  inch in diameter may be used in the aileron,

elevator, or rudder systems; and

(2) Each cable system must be designed so that there will be no hazardous

change in cable tension throughout the range of travel under operating

conditions and temperature variations.

(b) Each kind and size of pulley must correspond to the cable with which it

is used. Pulleys and sprockets must have closely fitted guards to prevent the

cables and chains from being displaced or fouled. Each pulley must lie in the

plane passing through the cable so that the cable does not rub against the

pulley flange.

(c) Fairleads must be installed so that they do not cause a change in cable

direction of more than three degrees.

(d) Clevis pins subject to load or motion and retained only by cotter pins

may not be used in the control system.

(e) Turnbuckles must be attached to parts having angular motion in a manner

that will positively prevent binding throughout the range of travel.

(f) There must be provisions for visual inspection of fairleads, pulleys,

terminals, and turnbuckles.

Sec. 25.693  Joints.

Control system joints (in push-pull systems) that are subject to angular

motion, except those in ball and roller bearing systems, must have a special

factor of safety of not less than 3.33 with respect to the ultimate bearing

strength of the softest material used as a bearing. This factor may be

reduced to 2.0 for joints in cable control systems. For ball or roller

bearings, the approved ratings may not be exceeded.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29777, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.697  Lift and drag devices, controls.

(a) Each lift device control must be designed so that the pilots can place

the device in any takeoff, en route, approach, or landing position

established under Sec. 25.101(d). Lift and drag devices must maintain the

selected positions, except for movement produced by an automatic positioning

or load limiting device, without further attention by the pilots.

(b) Each lift and drag device control must be designed and located to make

inadvertent operation improbable. Lift and drag devices intended for ground

operation only must have means to prevent the inadvertant operation of their

controls in flight if that operation could be hazardous.

(c) The rate of motion of the surfaces in response to the operation of the

control and the characteristics of the automatic positioning or load limiting

device must give satisfactory flight and performance characteristics under

steady or changing conditions of airspeed, engine power, and airplane

attitude.

(d) The lift device control must be designed to retract the surfaces from

the fully extended position, during steady flight at maximum continuous

engine power at any speed below VF +9.0 (knots).

[Amdt. 25-23, 35 FR 5675, Apr. 8, 1970, as amended by Amdt. 25-46, 43 FR

50595, Oct. 30, 1978; Amdt. 25-57, 49 FR 6848, Feb. 23, 1984]

Sec. 25.699  Lift and drag device indicator.

(a) There must be means to indicate to the pilots the position of each lift

or drag device having a separate control in the cockpit to adjust its

position. In addition, an indication of unsymmetrical operation or other

malfunction in the lift or drag device systems must be provided when such

indication is necessary to enable the pilots to prevent or counteract an

unsafe flight or ground condition, considering the effects on flight

characteristics and performance.

(b) There must be means to indicate to the pilots the takeoff, en route,

approach, and landing lift device positions.

(c) If any extension of the lift and drag devices beyond the landing

position is possible, the controls must be clearly marked to identify this

range of extension.

[Amdt. 25-23, 35 FR 5675, Apr. 8, 1970]

Sec. 25.701  Flap and slat interconnection.

(a) Unless the airplane has safe flight characteristics with the flaps or

slats retracted on one side and extended on the other, the motion of flaps or

slats on opposite sides of the plane of symmetry must be synchronized by a

mechanical interconnection or approved equivalent means.

(b) If a wing flap or slat interconnection or equivalent means is used, it

must be designed to account for the applicable unsymmetrical loads, including

those resulting from flight with the engines on one side of the plane of

symmetry inoperative and the remaining engines at takeoff power.

(c) For airplanes with flaps or slats that are not subjected to slipstream

conditions, the structure must be designed for the loads imposed when the

wing flaps or slats on one side are carrying the most severe load occurring

in the prescribed symmetrical conditions and those on the other side are

carrying not more than 80 percent of that load.

(d) The interconnection must be designed for the loads resulting when

interconnected flap or slat surfaces on one side of the plane of symmetry are

jammed and immovable while the surfaces on the other side are free to move

and the full power of the surface actuating system is applied.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29777, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.703  Takeoff warning system.

A takeoff warning system must be installed and must meet the following

requirements:

(a) The system must provide to the pilots an aural warning that is

automatically activated during the initial portion of the takeoff roll if the

airplane is in a configuration, including any of the following, that would

not allow a safe takeoff:

(1) The wing flaps or leading edge devices are not within the approved

range of takeoff positions.

(2) Wing spoilers (except lateral control spoilers meeting the requirements

of Sec. 25.671), speed brakes, or longitudinal trim devices are in a position

that would not allow a safe takeoff.

(b) The warning required by paragraph (a) of this section must continue

until--

(1) The configuration is changed to allow a safe takeoff;

(2) Action is taken by the pilot to terminate the takeoff roll;

(3)  The  airplane  is  rotated  for takeoff; or

(4) The warning is manually deactivated by the pilot.

(c) The means used to activate the system must function properly throughout

the ranges of takeoff weights, altitudes, and temperatures for which

certification is requested.

[Amdt. 25-42, 43 FR 2323, Jan. 16, 1978]

Landing Gear

Sec. 25.721  General.

(a) The main landing gear system must be designed so that if it fails due

to overloads during takeoff and landing (assuming the overloads to act in the

upward and aft directions), the failure mode is not likely to cause--

(1) For airplanes that have passenger seating configuration, excluding

pilots seats, of nine seats or less, the spillage of enough fuel from any

fuel system in the fuselage to constitute a fire hazard; and

(2) For airplanes that have a passenger seating configuration, excluding

pilots seats, of 10 seats or more, the spillage of enough fuel from any part

of the fuel system to constitute a fire hazard.

(b) Each airplane that has a passenger seating configuration excluding

pilots seats, of 10 seats or more must be designed so that with the airplane

under control it can be landed on a paved runway with any one or more landing

gear legs not extended without sustaining a structural component failure that

is likely to cause the spillage of enough fuel to constitute a fire hazard.

(c) Compliance with the provisions of this section may be shown by analysis

or tests, or both.

[Amdt. 25-32, 37 FR 3969, Feb. 24, 1972]

Sec. 25.723  Shock absorption tests.

(a) It must be shown that the limit load factors selected for design in

accordance with Sec. 25.473 for takeoff and landing weights, respectively,

will not be exceeded. This must be shown by energy absorption tests except

that analyses based on earlier tests conducted on the same basic landing gear

system which has similar energy absorption characteristics may be used for

increases in previously approved takeoff and landing weights.

(b) The landing gear may not fail in a test, demonstrating its reserve

energy absorption capacity, simulating a descent velocity of 12 f.p.s. at

design landing weight, assuming airplane lift not greater than the airplane

weight acting during the landing impact.

[Amdt. 25-23, 35 FR 5675, Apr. 8, 1970, as amended by Amdt. 25-46, 43 FR

50595, Oct. 30, 1978; Amdt. 25-72, 55 FR 29777, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.725  Limit drop tests.

(a) If compliance with Sec. 25.723(a) is shown by free drop tests, these

tests must be made on the complete airplane, or on units consisting of a

wheel, tire, and shock absorber, in their proper positions, from free drop

heights not less than--

(1) 18.7 inches for the design landing weight conditions; and

(2) 6.7 inches for the design takeoff weight conditions.

(b) If airplane lift is simulated by air cylinders or by other mechanical

means, the weight used for the drop must be equal to W. If the effect of

airplane lift is represented in free drop tests by an equivalent reduced

mass, the landing gear must be dropped with an effective mass equal to

h+(1-L)d

We = W x  ------------

h+d

where--

We= the effective weight to be used in the drop test (lbs.);

h =specified free drop height (inches);

d =deflection under impact of the tire (at the approved inflation pressure)

plus the vertical component of the axle travel relative to the drop mass

(inches);

W=WM for main gear units (lbs.), equal to the static weight on that unit with

the airplane in the level attitude (with the nose wheel clear in the case

of nose wheel type airplanes);

W=WT for tail gear units (lbs.), equal to the static weight on the tail unit

with the airplane in the tail-down attitude;

W=WN for nose wheel units (lbs.), equal to the vertical component of the

static reaction that would exist at the nose wheel, assuming that the

mass of the airplane acts at the center of gravity and exerts a force of

1.0 g downward and 0.25 g forward; and

L = ratio of the assumed airplane lift to the airplane weight, but not more

than 1.0.

(c) The drop test attitude of the landing gear unit and the application of

appropriate drag loads during the test must simulate the airplane landing

conditions in a manner consistent with the development of a rational or

conservative limit load factor value.

(d) The value of d used in the computation of We in paragraph (b) of this

section may not exceed the value actually obtained in the drop test.

(e) The limit inertia load factor n must be determined from the free drop

test in paragraph (b) of this section according to the following formula:

We

n = nj  ----  + L

W

where--

nj =the load factor developed in the drop test (that is, the acceleration dv/

dt in g's recorded in the drop test) plus 1.0; and

We,  W, and L are the same as in the drop test computation.

(f) The value of n determined in paragraph (e) of this section may not be

more than the limit inertia load factor used in the landing conditions in

Sec. 25.473.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5675, Apr. 8, 1970]

Sec. 25.727  Reserve energy absorption drop tests.

(a) If compliance with the reserve energy absorption condition specified in

Sec. 25.723(b) is shown by free drop tests, the drop height may not be less

than 27 inches.

(b) If airplane lift is simulated by air cylinders or by other mechanical

means, the weight used for the drop must be equal to W. If the effect of

airplane lift is represented in free drop tests by an equivalent reduced

mass, the landing gear must be dropped with an effective mass,

Wh

We =  ------

h+d

where the symbols and other details are the same as in Sec. 25.725(b).

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5675, Apr. 8, 1970]

Sec. 25.729  Retracting mechanism.

(a) General. For airplanes with retractable landing gear, the following

apply:

(1) The landing gear retracting mechanism, wheel well doors, and supporting

structure, must be designed for--

(i) The loads occurring in the flight conditions when the gear is in the

retracted position,

(ii) The combination of friction loads, inertia loads, brake torque loads,

air loads, and gyroscopic loads resulting from the wheels rotating at a

peripheral speed equal to 1.3 Vs (with the flaps in takeoff position at

design takeoff weight), occurring during retraction and extension at any

airspeed up to 1.6 Vs1 (with the flaps in the approach position at design

landing weight), and

(iii) Any load factor up to those specified in Sec. 25.345(a) for the flaps

extended condition.

(2) Unless there are other means to decelerate the airplane in flight at

this speed, the landing gear, the retracting mechanism, and the airplane

structure (including wheel well doors) must be designed to withstand the

flight loads occurring with the landing gear in the extended position at any

speed up to 0.67 VC.

(3) Landing gear doors, their operating mechanism, and their supporting

structures must be designed for the yawing maneuvers prescribed for the

airplane in addition to the conditions of airspeed and load factor prescribed

in paragraphs (a) (1) and (2) of this section.

(b) Landing gear lock. There must be positive means to keep the landing

gear extended, in flight and on the ground.

(c) Emergency operation. There must be an emergency means for extending the

landing gear in the event of--

(1) Any reasonably probable failure in the normal retraction system; or

(2) The failure of any single source of hydraulic, electric, or equivalent

energy supply.

(d) Operation test. The proper functioning of the retracting mechanism must

be shown by operation tests.

(e) Position indicator and warning device. If a retractable landing gear is

used, there must be a landing gear position indicator (as well as necessary

switches to actuate the indicator) or other means to inform the pilot that

the gear is secured in the extended (or retracted) position. This means must

be designed as follows:

(1) If switches are used, they must be located and coupled to the landing

gear mechanical systems in a manner that prevents an erroneous indication of

"down and locked" if the landing gear is not in a fully extended position, or

of "up and locked" if the landing gear is not in the fully retracted

position. The switches may be located where they are operated by the actual

landing gear locking latch or device.

(2) The flightcrew must be given an aural warning that functions

continuously, or is periodically repeated, if a landing is attempted when the

landing gear is not locked down.

(3) The warning must be given in sufficient time to allow the landing gear

to be locked down or a go-around to be made.

(4) There must not be a manual shut-off means readily available to the

flightcrew for the warning required by paragraph (e)(2) of this section such

that it could be operated instinctively, inadvertently, or by habitual

reflexive action.

(5) The system used to generate the aural warning must be designed to

eliminate false or inappropriate alerts.

(6) Failures of systems used to inhibit the landing gear aural warning,

that would prevent the warning system from operating, must be improbable.

(f) Protection of equipment in wheel wells. Equipment that is essential to

safe operation of the airplane and that is located in wheel wells must be

protected from the damaging effects of--

(1) A bursting tire, unless it is shown that a tire cannot burst from

overheat; and

(2) A loose tire tread, unless it is shown that a loose tire tread cannot

cause damage.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5676, Apr. 8, 1970; Amdt. 25-42, 43 FR 2323, Jan. 16, 1978; Amdt. 25-72, 55

FR 29777, July 20, 1990; Amdt. 25-75, 56 FR 63762, Dec. 5, 1991]

*****************************************************************************

56 FR 63760, No. 234, Dec. 5, 1991

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the airworthiness standards for landing gear aural warning systems in

transport category airplanes to reflect current design practices. They

require that if a landing is attempted when the landing gear is not locked

down, the flightcrew must be given an aural warning in sufficient time to

allow the landing gear to be locked down or a go-around to be made. These

amendments state the intent of the current regulations in more objective

terms to eliminate nuisance warnings and to simplify the certification

process.

EFFECTIVE DATE: January 6, 1992.

*****************************************************************************

Sec. 25.731  Wheels.

(a) Each main and nose wheel must be approved.

(b) The maximum static load rating of each wheel may not be less than the

corresponding static ground reaction with--

(1) Design maximum weight; and

(2) Critical center of gravity.

(c) The maximum limit load rating of each wheel must equal or exceed the

maximum radial limit load determined under the applicable ground load

requirements of this part.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55

FR 29777, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.733  Tires.

(a) When a landing gear axle is fitted with a single wheel and tire

assembly, the wheel must be fitted with a suitable tire of proper fit with a

speed rating approved by the Administrator that is not exceeded under

critical conditions and with a load rating approved by the Administrator that

is not exceeded under--

(1) The loads on the main wheel tire, corresponding to the most critical

combination of airplane weight (up to maximum weight) and center of gravity

position, and

(2) The loads corresponding to the ground reactions in paragraph (b) of

this section, on the nose wheel tire, except as provided in paragraphs (b)(2)

and (b)(3) of this section.

(b) The applicable ground reactions for nose wheel tires are as follows:

(1) The static ground reaction for the tire corresponding to the most

critical combination of airplane weight (up to maximum ramp weight) and

center of gravity position with a force of 1.0g acting downward at the center

of gravity. This load may not exceed the load rating of the tire.

(2) The ground reaction of the tire corresponding to the most critical

combination of airplane weight (up to maximum landing weight) and center of

gravity position combined with forces of 1.0g downward and 0.31g forward

acting at the center of gravity. The reactions in this case must be

distributed to the nose and main wheels by the principles of statics with a

drag reaction equal to 0.31 times the vertical load at each wheel with brakes

capable of producing this ground reaction. This nose tire load may not exceed

1.5 times the load rating of the tire.

(3) The ground reaction of the tire corresponding to the most critical

combination of airplane weight (up to maximum ramp weight) and center of

gravity position combined with forces of 1.0g downward and 0.20g forward

acting at the center of gravity. The reactions in this case must be

distributed to the nose and main wheels by the principles of statics with a

drag reaction equal to 0.20 times the vertical load at each wheel with brakes

capable of producing this ground reaction. This nose tire load may not exceed

1.5 times the load rating of the tire.

(c) When a landing gear axle is fitted with more than one wheel and tire

assembly, such as dual or dual-tandem, each wheel must be fitted with a

suitable tire of proper fit with a speed rating approved by the Administrator

that is not exceeded under critical conditions, and with a load rating

approved by the Administrator that is not exceeded by--

(1) The loads on each main wheel tire, corresponding to the most critical

combination of airplane weight (up to maximum weight) and center of gravity

position, when multiplied by a factor of 1.07; and

(2) Loads specified in paragraphs (a)(2), (b)(1), (b)(2), and (b)(3) of

this section on each nose wheel tire.

(d) Each tire installed on a retractable landing gear system must, at the

maximum size of the tire type expected in service, have a clearance to

surrounding structure and systems that is adequate to prevent unintended

contact between the tire and any part of the structure or systems.

(e) For an airplane with a maximum certificated takeoff weight of more than

75,000 pounds, tires mounted on braked wheels must be inflated with dry

nitrogen or other gases shown to be inert so that the gas mixture in the tire

does not contain oxygen in excess of 5 percent by volume, unless it can be

shown that the tire liner material will not produce a volatile gas when

heated or that means are provided to prevent tire temperatures from reaching

unsafe levels.

[Amdt. 25-48, 44 FR 68752, Nov. 29, 1979, as amended by Amdt. 25-72, 55 FR

29777, July 20, 1990; Amdt. 25-78, 58 FR 11781, Feb. 26, 1993]

*****************************************************************************

58 FR 11778, No. 37, Feb. 26, 1993

SUMMARY: This amendment to the Federal Aviation Regulations (FAR) requires

that an inert gas, such as nitrogen, be used in lieu of air, for inflation of

tires on certain transport category airplanes. This action is prompted by at

least three cases in which the oxygen in air-filled tires combined with

volatile gases given off by a severely overheated tire and exploded upon

reaching autoignition temperature. The use of an inert gas for tire inflation

will eliminate the possibility of a tire explosion.

EFFECTIVE DATE: March 29, 1993.

*****************************************************************************

Sec. 25.735  Brakes.

(a) Each brake must be approved.

(b) The brake system and associated systems must be designed and

constructed so that if any electrical, pneumatic, hydraulic, or mechanical

connecting or transmitting element (excluding the operating pedal or handle)

fails, or if any single source of hydraulic or other brake operating energy

supply is lost, it is possible to bring the airplane to rest under conditions

specified in Sec. 25.125, with a mean deceleration during the landing roll of

at least 50 percent of that obtained in determining the landing distance as

prescribed in that section. Subcomponents within the brake assembly, such as

brake drum, shoes, and actuators (or their equivalents), shall be considered

as connecting or transmitting elements, unless it is shown that leakage of

hydraulic fluid resulting from failure of the sealing elements in these

subcomponents within the brake assembly would not reduce the braking

effectiveness below that specified in this paragraph.

(c) Brake controls may not require excessive control force in their

operation.

(d) The airplane must have a parking control that, when set by the pilot,

will without further attention, prevent the airplane from rolling on a paved,

level runway with takeoff power on the critical engine.

(e) If antiskid devices are installed, the devices and associated systems

must be designed so that no single probable malfunction will result in a

hazardous loss of braking ability or directional control of the airplane.

(f) The brake kinetic energy capacity rating of each main wheel-brake

assembly may not be less than the kinetic energy absorption requirements

determined under either of the following methods:

(1) The brake kinetic energy absorption requirements must be based on a

rational analysis of the sequence of events expected during operational

landings at maximum landing weight. This analysis must include conservative

values of airplane speed at which the brakes are applied, braking coefficient

of friction between tires and runway, aerodynamic drag, propeller drag or

power-plant forward thrust, and (if more critical) the most adverse single

engine or propeller malfunction.

(2) Instead of a rational analysis, the kinetic energy absorption

requirements for each main wheel brake assembly may be derived from the

following formula, which assumes an equal distribution of braking between

main wheels:

WV 2

KE = 0.0443  ----

N

where--

KE=Kinetic energy per wheel (ft.-lb.);

W=Design landing weight (lb.);

V=Airplane speed in knots. V must be not less than VS 0, the poweroff

stalling speed of the airplane at sea level, at the design landing

weight, and in the landing configuration; and

N=Number of main wheels with brakes.

The formula must be modified in cases of unequal braking distribution.

(g) The minimum stalling speed rating of each main wheel-brake assembly

(that is, the initial speed used in the dynamometer tests) may not be more

than the V used in the determination of kinetic energy in accordance with

paragraph (f) of this section, assuming that the test procedures for wheel-

brake assemblies involve a specified rate of deceleration, and, therefore,

for the same amount of kinetic energy, the rate of energy absorption (the

power absorbing ability of the brake) varies inversely with the initial

speed.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5676, Apr. 8, 1970; Amdt. 25-48, 44 FR 68742, Nov. 29, 1979; Amdt. 25-72, 55

FR 29777, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.737  Skis.

Each ski must be approved. The maximum limit load rating of each ski must

equal or exceed the maximum limit load determined under the applicable ground

load requirements of this part.

Floats and Hulls

Sec. 25.751  Main float buoyancy.

Each main float must have--

(a) A buoyancy of 80 percent in excess of that required to support the

maximum weight of the seaplane or amphibian in fresh water; and

(b) Not less than five watertight compartments approximately equal in

volume.

Sec. 25.753  Main float design.

Each main float must be approved and must meet the requirements of Sec.

25.521.

Sec. 25.755  Hulls.

(a) Each hull must have enough watertight compartments so that, with any

two adjacent compartments flooded, the buoyancy of the hull and auxiliary

floats (and wheel tires, if used) provides a margin of positive stability

great enough to minimize the probability of capsizing in rough, fresh water.

(b) Bulkheads with watertight doors may be used for communication between

compartments.

Personnel and Cargo Accommodations

Sec. 25.771  Pilot compartment.

(a) Each pilot compartment and its equipment must allow the minimum flight

crew (established under Sec. 25.1523) to perform their duties without

unreasonable concentration or fatigue.

(b) The primary controls listed in Sec. 25.779(a), excluding cables and

control rods, must be located with respect to the propellers so that no

member of the minimum flight crew (established under Sec. 25.1523), or part

of the controls, lies in the region between the plane of rotation of any

inboard propeller and the surface generated by a line passing through the

center of the propeller hub making an angle of five degrees forward or aft of

the plane of rotation of the propeller.

(c) If provision is made for a second pilot, the airplane must be

controllable with equal safety from either pilot seat.

(d) The pilot compartment must be constructed so that, when flying in rain

or snow, it will not leak in a manner that will distract the crew or harm the

structure.

(e) Vibration and noise characteristics of cockpit equipment may not

interfere with safe operation of the airplane.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-4, 30 FR

6113, Apr. 30, 1965]

Sec. 25.772  Pilot compartment doors.

For an airplane that has a maximum passenger seating configuration of more

than 20 seats and that has a lockable door installed between the pilot

compartment and the passenger compartment:

(a) The emergency exit configuration must be designed so that neither

crewmembers nor passengers need use that door in order to reach the emergency

exits provided for them; and

(b) Means must be provided to enable flight crewmembers to directly enter

the passenger compartment from the pilot compartment if the cockpit door

becomes jammed.

[Doc. No. 24344, Admt. 25-72, 55 FR 29777, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.773  Pilot compartment view.

(a) Nonprecipitation conditions. For nonprecipitation conditions, the

following apply:

(1) Each pilot compartment must be arranged to give the pilots a

sufficiently extensive, clear, and undistorted view, to enable them to safely

perform any maneuvers within the operating limitations of the airplane,

including taxiing takeoff, approach, and landing.

(2) Each pilot compartment must be free of glare and reflection that could

interfere with the normal duties of the minimum flight crew (established

under Sec. 25.1523). This must be shown in day and night flight tests under

nonprecipitation conditions.

(b) Precipitation conditions. For precipitation conditions, the following

apply:

(1) The airplane must have a means to maintain a clear portion of the

windshield, during precipitation conditions, sufficient for both pilots to

have a sufficiently extensive view along the flight path in normal flight

attitudes of the airplane. This means must be designed to function, without

continuous attention on the part of the crew, in--

(i) Heavy rain at speeds up to 1.6 Vs1 with lift and drag devices

retracted; and

(ii) The icing conditions specified in Sec. 25.1419 if certification with

ice protection provisions is requested.

(2) The first pilot must have--

(i) A window that is openable under the conditions prescribed in paragraph

(b)(1) of this section when the cabin is not pressurized, provides the view

specified in that paragraph, and gives sufficient protection from the

elements against impairment of the pilot's vision; or

(ii) An alternate means to maintain a clear view under the conditions

specified in paragraph (b)(1) of this section, considering the probable

damage due to a severe hail encounter.

(c) Internal windshield and window fogging. The airplane must have a means

to prevent fogging of the internal portions of the windshield and window

panels over an area which would provide the visibility specified in paragraph

(a) of this section under all internal and external ambient conditions,

including precipitation conditions, in which the airplane is intended to be

operated.

(d) Fixed markers or other guides must be installed at each pilot station

to enable the pilots to position themselves in their seats for an optimum

combination of outside visibility and instrument scan. If lighted markers or

guides are used they must comply with the requirements specified in Sec.

25.1381.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5676, Apr. 8, 1970; Amdt. 25-46, 43 FR 50595, Oct. 30, 1978; Amdt. 25-72, 55

FR 29778, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.775  Windshields and windows.

(a) Internal panes must be made of nonsplintering material.

(b) Windshield panes directly in front of the pilots in the normal conduct

of their duties, and the supporting structures for these panes, must

withstand, without penetration, the impact of a four-pound bird when the

velocity of the airplane (relative to the bird along the airplane's flight

path) is equal to the value of VC, at sea level, selected under Sec.

25.335(a).

(c) Unless it can be shown by analysis or tests that the probability of

occurrence of a critical windshield fragmentation condition is of a low

order, the airplane must have a means to minimize the danger to the pilots

from flying windshield fragments due to bird impact. This must be shown for

each transparent pane in the cockpit that--

(1) Appears in the front view of the airplane;

(2) Is inclined 15 degrees or more to the longitudinal axis of the

airplane; and

(3) Has any part of the pane located where its fragmentation will

constitute a hazard to the pilots.

(d) The design of windshields and windows in pressurized airplanes must be

based on factors peculiar to high altitude operation, including the effects

of continuous and cyclic pressurization loadings, the inherent

characteristics of the material used, and the effects of temperatures and

temperature differentials. The windshield and window panels must be capable

of withstanding the maximum cabin pressure differential loads combined with

critical aerodynamic pressure and temperature effects after any single

failure in the installation or associated systems. It may be assumed that,

after a single failure that is obvious to the flight crew (established under

Sec. 25.1523), the cabin pressure differential is reduced from the maximum,

in accordance with appropriate operating limitations, to allow continued safe

flight of the airplane with a cabin pressure altitude of not more than 15,000

feet.

(e) The windshield panels in front of the pilots must be arranged so that,

assuming the loss of vision through any one panel, one or more panels remain

available for use by a pilot seated at a pilot station to permit continued

safe flight and landing.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5676, Apr. 8, 1970; Amdt. 25-38, 41 FR 55466, Dec. 20, 1976]

Sec. 25.777  Cockpit controls.

(a) Each cockpit control must be located to provide convenient operation

and to prevent confusion and inadvertent operation.

(b) The direction of movement of cockpit controls must meet the

requirements of Sec. 25.779. Wherever practicable, the sense of motion

involved in the operation of other controls must correspond to the sense of

the effect of the operation upon the airplane or upon the part operated.

Controls of a variable nature using a rotary motion must move clockwise from

the off position, through an increasing range, to the full on position.

(c) The controls must be located and arranged, with respect to the pilots'

seats, so that there is full and unrestricted movement of each control

without interference from the cockpit structure or the clothing of the

minimum flight crew (established under Sec. 25.1523) when any member of this

flight crew, from 5'2'' to 6'3'' in height, is seated with the seat belt and

shoulder harness (if provided) fastened.

(d) Identical powerplant controls for each engine must be located to

prevent confusion as to the engines they control.

(e) Wing flap controls and other auxiliary lift device controls must be

located on top of the pedestal, aft of the throttles, centrally or to the

right of the pedestal centerline, and not less than 10 inches aft of the

landing gear control.

(f) The landing gear control must be located forward of the throttles and

must be operable by each pilot when seated with seat belt and shoulder

harness (if provided) fastened.

(g) Control knobs must be shaped in accordance with Sec. 25.781. In

addition, the knobs must be of the same color, and this color must contrast

with the color of control knobs for other purposes and the surrounding

cockpit.

(h) If a flight engineer is required as part of the minimum flight crew

(established under Sec. 25.1523), the airplane must have a flight engineer

station located and arranged so that the flight crewmembers can perform their

functions efficiently and without interfering with each other.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-46, 43 FR

50596, Oct. 30, 1978]

Sec. 25.779  Motion and effect of cockpit controls.

Cockpit controls must be designed so that they operate in accordance with

the following movement and actuation:

(a) Aerodynamic controls:

(1) Primary.

Controls            Motion and effect

Aileron   Right (clockwise) for right wing down.

Elevator  Rearward for nose up.

Rudder    Right pedal forward for nose right.

(2) Secondary.

Controls                           Motion and effect

Flaps (or auxiliary lift devices)  Forward for flaps up; rearward for flaps

down.

Trim tabs (or equivalent)          Rotate to produce similar rotation of the

airplane about an axis parallel to the

axis of the control.

(b) Powerplant and auxiliary controls:

(1) Powerplant.

Controls                           Motion and effect

Power or thrust      Forward to increase forward thrust and rearward to

increase rearward thrust.

Propellers           Forward to increase rpm.

Mixture              Forward or upward for rich.

Carburetor air heat  Forward or upward for cold.

Supercharger         Forward or upward for low blower. For

turbosuperchargers, forward, upward, or clockwise, to

increase pressure.

(2) Auxiliary.

Motion and

Controls        effect

Landing gear  Down to extend.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55

FR 29778, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.781  Cockpit control knob shape.

Cockpit control knobs must conform to the general shapes (but not

necessarily the exact sizes or specific proportions) in the following figure:

[ ...Illustration appears here... ]

Flap Control Knob

[ ...Illustration appears here... ]

Landing Gear Control Knob

[ ...Illustration appears here... ]

Mixture Control Knob

[ ...Illustration appears here... ]

Supercharger Control Knob

[ ...Illustration appears here... ]

Power or Thrust Knob

[ ...Illustration appears here... ]

Propeller Control Knob

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55 FR

29778, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.783  Doors.

(a) Each cabin must have at least one easily accessible external door.

(b) There must be a means to lock and safeguard each external door against

opening in flight (either inadvertently by persons or as a result of

mechanical failure or failure of a single structural element either during or

after closure). Each external door must be openable from both the inside and

the outside, even though persons may be crowded against the door on the

inside of the airplane. Inward opening doors may be used if there are means

to prevent occupants from crowding against the door to an extent that would

interfere with the opening of the door. The means of opening must be simple

and obvious and must be arranged and marked so that it can be readily located

and operated, even in darkness. Auxiliary locking devices may be used.

(c) Each external door must be reasonably free from jamming as a result of

fuselage deformation in a minor crash.

(d) Each external door must be located where persons using them will not be

endangered by the propellers when appropriate operating procedures are used.

(e) There must be a provision for direct visual inspection of the locking

mechanism to determine if external doors, for which the initial opening

movement is not inward (including passenger, crew, service, and cargo doors),

are fully closed and locked. The provision must be discernible under

operational lighting conditions by appropriate crewmembers using a flashlight

or equivalent lighting source. In addition, there must be a visual warning

means to signal the appropriate flight crewmembers if any external door is

not fully closed and locked. The means must be designed such that any failure

or combination of failures that would result in an erroneous closed and

locked indication is improbable for doors for which the initial opening

movement is not inward.

(f) External doors must have provisions to prevent the initiation of

pressurization of the airplane to an unsafe level if the door is not fully

closed and locked. In addition, it must be shown by safety analysis that

inadvertent opening is extemely improbable.

(g) Cargo and service doors not suitable for use as emergency exits need

only meet paragraphs (e) and (f) of this section and be safeguarded against

opening in flight as a result of mechanical failure or failure of a single

structural element.

(h) Each passenger entry door in the side of the fuselage must qualify as a

Type A, Type I, or Type II passenger emergency exit and must meet the

requirements of Secs. 25.807 through 25.813 that apply to that type of

passenger emergency exit.

(i) If an integral stair is installed in a passenger entry door that is

qualified as a passenger emergency exit, the stair must be designed so that

under the following conditions the effectiveness of passenger emergency

egress will not be impaired:

(1) The door, integral stair, and operating mechanism have been subjected

to the inertia forces specified in Sec. 25.561(b)(3), acting separately

relative to the surrounding structure.

(2) The airplane is in the normal ground attitude and in each of the

attitudes corresponding to collapse of one or more legs of the landing gear.

(j) All lavatory doors must be designed to preclude anyone from becoming

trapped inside the lavatory, and if a locking mechanism is installed, it be

capable of being unlocked from the outside without the aid of special tools.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-15, 32 FR

13262, Sept. 20, 1967; Amdt. 25-23, 35 FR 5676, Apr. 8, 1970; Amdt. 25-54, 45

FR 60173, Sept. 11, 1980; Amdt. 25-72, 55 FR 29780, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.785  Seats, berths, safety belts, and harnesses.

(a) A seat (or berth for a nonambulant person) must be provided for each

occupant who has reached his or her second birthday.

(b) Each seat, berth, safety belt, harness, and adjacent part of the

airplane at each station designated as occupiable during takeoff and landing

must be designed so that a person making proper use of these facilities will

not suffer serious injury in an emergency landing as a result of the inertia

forces specified in Secs. 25.561 and 25.562.

(c) Each seat or berth must be approved.

(d) Each occupant of a seat that makes more than an 18-degree angle with

the vertical plane containing the airplane centerline must be protected from

head injury by a safety belt and an energy absorbing rest that will support

the arms, shoulders, head, and spine, or by a safety belt and shoulder

harness that will prevent the head from contacting any injurious object. Each

occupant of any other seat must be protected from head injury by a safety

belt and, as appropriate to the type, location, and angle of facing of each

seat, by one or more of the following:

(1) A shoulder harness that will prevent the head from contacting any

injurious object.

(2) The elimination of any injurious object within striking radius of the

head.

(3) An energy absorbing rest that will support the arms, shoulders, head,

and spine.

(e) Each berth must be designed so that the forward part has a padded end

board, canvas diaphragm, or equivalent means, that can withstand the static

load reaction of the occupant when subjected to the forward inertia force

specified in Sec. 25.561. Berths must be free from corners and protuberances

likely to cause injury to a person occupying the berth during emergency

conditions.

(f) Each seat or berth, and its supporting structure, and each safety belt

or harness and its anchorage must be designed for an occupant weight of 170

pounds, considering the maximum load factors, inertia forces, and reactions

among the occupant, seat, safety belt, and harness for each relevant flight

and ground load condition (including the emergency landing conditions

prescribed in Sec. 25.561). In addition--

(1) The structural analysis and testing of the seats, berths, and their

supporting structures may be determined by assuming that the critical load in

the forward, sideward, downward, upward, and rearward directions (as

determined from the prescribed flight, ground, and emergency landing

conditions) acts separately or using selected combinations of loads if the

required strength in each specified direction is substantiated. The forward

load factor need not be applied to safety belts for berths.

(2) Each pilot seat must be designed for the reactions resulting from the

application of the pilot forces prescribed in Sec. 25.395.

(3) The inertia forces specified in Sec. 25.561 must be multiplied by a

factor of 1.33 (instead of the fitting factor prescribed in Sec. 25.625) in

determining the strength of the attachment of each seat to the structure and

each belt or harness to the seat or structure.

(g) Each seat at a flight deck station must have a restraint system

consisting of a combined safety belt and shoulder harness with a single-point

release that permits the flight deck occupant, when seated with the restraint

system fastened, to perform all of the occupant's necessary flight deck

functions. There must be a means to secure each combined restraint system

when not in use to prevent interference with the operation of the airplane

and with rapid egress in an emergency.

(h) Each seat located in the passenger compartment and designated for use

during takeoff and landing by a flight attendant required by the operating

rules of this chapter must be:

(1) Near a required floor level emergency exit, except that another

location is acceptable if the emergency egress of passengers would be

enhanced with that location. A flight attendant seat must be located adjacent

to each Type A emergency exit. Other flight attendant seats must be evenly

distributed among the required floor level emergency exits to the extent

feasible.

(2) To the extent possible, without compromising proximity to a required

floor level emergency exit, located to provide a direct view of the cabin

area for which the flight attendant is responsible.

(3) Positioned so that the seat will not interfere with the use of a

passageway or exit when the seat is not in use.

(4) Located to minimize the probability that occupants would suffer injury

by being struck by items dislodged from service areas, stowage compartments,

or service equipment.

(5) Either forward or rearward facing with an energy absorbing rest that is

designed to support the arms, shoulders, head, and spine.

(6) Equipped with a restraint system consisting of a combined safety belt

and shoulder harness unit with a single point release. There must be means to

secure each restraint system when not in use to prevent interference with

rapid egress in an emergency.

(i) Each safety belt must be equipped with a metal to metal latching

device.

(j) If the seat backs do not provide a firm handhold, there must be a

handgrip or rail along each aisle to enable persons to steady themselves

while using the aisles in moderately rough air.

(k) Each projecting object that would injure persons seated or moving about

the airplane in normal flight must be padded.

(l) Each forward observer's seat required by the operating rules must be

shown to be suitable for use in conducting the necessary enroute inspection.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29780, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.787  Stowage compartments.

(a) Each compartment for the stowage of cargo, baggage, carry-on articles,

and equipment (such as life rafts), and any other stowage compartment must be

designed for its placarded maximum weight of contents and for the critical

load distribution at the appropriate maximum load factors corresponding to

the specified flight and ground load conditions, and to the emergency landing

conditions of Sec. 25.561(b), except that the forces specified in the

emergency landing conditions need not be applied to compartments located

below, or forward, of all occupants in the airplane. If the airplane has a

passenger seating configuration, excluding pilots seats, of 10 seats or more,

each stowage compartment in the passenger cabin, except for underseat and

overhead compartments for passenger convenience, must be completely enclosed.

(b) There must be a means to prevent the contents in the compartments from

becoming a hazard by shifting, under the loads specified in paragraph (a) of

this section. For stowage compartments in the passenger and crew cabin, if

the means used is a latched door, the design must take into consideration the

wear and deterioration expected in service.

(c) If cargo compartment lamps are installed, each lamp must be installed

so as to prevent contact between lamp bulb and cargo.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-32, 37 FR

3969, Feb. 24, 1972; Amdt. 25-38, 41 FR 55466, Dec. 20, 1976; Amdt. 25-51, 45

FR 7755, Feb. 4, 1980]

Sec. 25.789  Retention of items of mass in passenger and crew compartments

and galleys.

(a) Means must be provided to prevent each item of mass (that is part of

the airplane type design) in a passenger or crew compartment or galley from

becoming a hazard by shifting under the appropriate maximum load factors

corresponding to the specified flight and ground load conditions, and to the

emergency landing conditions of Sec. 25.561(b).

(b) Each interphone restraint system must be designed so that when

subjected to the load factors specified in Sec. 25.561(b)(3), the interphone

will remain in its stowed position.

[Amdt. 25-32, 37 FR 3969, Feb. 24, 1972, as amended by Amdt. 25-46, 43 FR

50596, Oct. 30, 1978]

Sec. 25.791  Passenger information signs and placards.

(a) If smoking is to be prohibited, there must be at least one placard so

stating that is legible to each person seated in the cabin. If smoking is to

be allowed, and if the crew compartment is separated from the passenger

compartment, there must be at least one sign notifying when smoking is

prohibited. Signs which notify when smoking is prohibited must be operable by

a member of the flightcrew and, when illuminated, must be legible under all

probable conditions of cabin illumination to each person seated in the cabin.

(b) Signs that notify when seat belts should be fastened and that are

installed to comply with the operating rules of this chapter must be operable

by a member of the flightcrew and, when illuminated, must be legible under

all probable conditions of cabin illumination to each person seated in the

cabin.

(c) A placard must be located on or adjacent to the door of each receptacle

used for the disposal of flammable waste materials to indicate that use of

the receptacle for disposal of cigarettes, etc., is prohibited.

(d) Lavatories must have "No Smoking" or "No Smoking in Lavatory" placards

conspicuously located on or adjacent to each side of the entry door.

(e) Symbols that clearly express the intent of the sign or placard may be

used in lieu of letters.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29780, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.793  Floor surfaces.

The floor surface of all areas which are likely to become wet in service

must have slip resistant properties.

[Amdt. 25-51, 45 FR 7755, Feb. 4, 1980]

Emergency Provisions

Sec. 25.801  Ditching.

(a) If certification with ditching provisions is requested, the airplane

must meet the requirements of this section and Secs. 25.807(e), 25.1411, and

25.1415(a).

(b) Each practicable design measure, compatible with the general

characteristics of the airplane, must be taken to minimize the probability

that in an emergency landing on water, the behavior of the airplane would

cause immediate injury to the occupants or would make it impossible for them

to escape.

(c) The probable behavior of the airplane in a water landing must be

investigated by model tests or by comparison with airplanes of similar

configuration for which the ditching characteristics are known. Scoops,

flaps, projections, and any other factor likely to affect the hydrodynamic

characteristics of the airplane, must be considered.

(d) It must be shown that, under reasonably probable water conditions, the

flotation time and trim of the airplane will allow the occupants to leave the

airplane and enter the liferafts required by Sec. 25.1415. If compliance with

this provision is shown by buoyancy and trim computations, appropriate

allowances must be made for probable structural damage and leakage. If the

airplane has fuel tanks (with fuel jettisoning provisions) that can

reasonably be expected to withstand a ditching without leakage, the

jettisonable volume of fuel may be considered as buoyancy volume.

(e) Unless the effects of the collapse of external doors and windows are

accounted for in the investigation of the probable behavior of the airplane

in a water landing (as prescribed in paragraphs (c) and (d) of this section),

the external doors and windows must be designed to withstand the probable

maximum local pressures.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55 FR

29781, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.803  Emergency evacuation.

(a) Each crew and passenger area must have emergency means to allow rapid

evacuation in crash landings, with the landing gear extended as well as with

the landing gear retracted, considering the possibility of the airplane being

on fire.

(b) [Reserved]

(c) For airplanes having a seating capacity of more than 44 passengers, it

must be shown that the maximum seating capacity, including the number of

crewmembers required by the operating rules for which certification is

requested, can be evacuated from the airplane to the ground under simulated

emergency conditions within 90 seconds. Compliance with this requirement must

be shown by actual demonstration using the test criteria outlined in appendix

J of this part unless the Administrator finds that a combination of analysis

and testing will provide data equivalent to that which would be obtained by

actual demonstration.

(d) [Reserved]

(e) [Reserved]

[Doc. No. 5066, 29 FR 18291 Dec. 24, 1964, as amended by Amdt. 25-15, 32 FR

13262, Sept. 20, 1967; Amdt. 25-20, 34 FR 5544, Mar. 22, 1969; Amdt. 25-32,

37 FR 3969, Feb. 24, 1972; Amdt. 25-46, 43 FR 50596, Oct. 30, 1978; Amdt.

25-72, 55 FR 29781, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.805  [Removed.  55 FR 29781, July 20, 1990]

EDITORIAL NOTE: For the convenience of the user, the removed text is

set out below.

Sec. 25.805  Flight crew emergency exits.

Except for airplanes with a passenger capacity of 20 or less in which the

proximity of passenger emergency exits to the flight crew area offers a

convenient and readily accessible means of evacuation for the flight crew,

the following apply:

(a) There must be either one exit on each side of the airplane or a top

hatch, in the flight crew area.

(b) Each exit must be of sufficient size and must be located so as to allow

rapid evacuation of the crew. An exit size and shape of other than at least

19 by 20 inches unobstructed rectangular opening may be used only if exit

utility is satisfactorily shown, by a typical flight crewmember, to the

Administrator.

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.807   Emergency exits.

(a) Type. For the purpose of this part, the types of exits are defined as

follows:

(1) Type I. This type is a floor level exit with a rectangular opening of

not less than 24 inches wide by 48 inches high, with corner radii not greater

than one-third the width of the exit.

(2) Type II. This type is a rectangular opening of not less than 20 inches

wide by 44 inches high, with corner radii not greater than one-third the

width of the exit. Type II exits must be floor level exits unless located

over the wing, in which case they may not have a step-up inside the airplane

of more than 10 inches nor a step-down outside the airplane of more than 17

inches.

(3) Type III. This type is a rectangular opening of not less than 20 inches

wide by 36 inches high, with corner radii not greater than one-third the

width of the exit, and with a step-up inside the airplane of not more than 20

inches. If the exit is located over the wing, the step-down outside the

airplane may not exceed 27 inches.

(4) Type IV. This type is a rectangular opening of not less than 19 inches

wide by 26 inches high, with corner radii not greater than one-third the

width of the exit, located over the wing, with a step-up inside the airplane

of not more than 29 inches and a step-down outside the airplane of not more

than 36 inches.

(5) Ventral. This type is an exit from the passenger compartment through

the pressure shell and the bottom fuselage skin. The dimensions and physical

configuration of this type of exit must allow at least the same rate of

egress as a Type I exit with the airplane in the normal ground attitude, with

landing gear extended.

(6) Tail cone. This type is an aft exit from the passenger compartment

through the pressure shell and through an openable cone of the fuselage aft

of the pressure shell. The means of opening the tailcone must be simple and

obvious and must employ a single operation.

(7) Type A. This type is a floor level exit with a rectangular opening of

not less than 42 inches wide by 72 inches high with corner radii not greater

than one-sixth of the width of the exit.

(b) Step down distance. Step down distance, as used in this section, means

the actual distance between the bottom of the required opening and a usable

foot hold, extending out from the fuselage, that is large enough to be

effective without searching by sight or feel.

(c) Over-sized exits. Openings larger than those specified in this section,

whether or not of rectangular shape, may be used if the specified rectangular

opening can be inscribed within the opening and the base of the inscribed

rectangular opening meets the specified step-up and step-down heights.

(d) Passenger emergency exits. Except as provided in paragraphs (d) (3)

through (7) of this section, the minimum number and type of passenger

emergency exits is as follows:

(1) For passenger seating configurations of 1 through 299 seats:

Emergency exits for

each side of the

fuselage

Passenger

seating

configuration

(crewmember

seats not     Type  Type  Type  Type

included)      I     II   III    IV

1 through 9                           1

10 through 19                   1

20 through 39             1     1

40 through 79       1           1

80 through 109      1           2

110 through 139     2           1

140 through 179     2           2

Additional exits are required for passenger seating configurations greater

than 179 seats in accordance with the following table:

Additional

emergency    Increase in

exits       passenger

(each side     seating

of      configuration

fuselage)      allowed

Type A                110

Type I                 45

Type II                40

Type III               35

(2) For passenger seating configurations greater than 299 seats, each

emergency exit in the side of the fuselage must be either a Type A or Type I.

A passenger seating configuration of 110 seats is allowed for each pair of

Type A exits and a passenger seating configuration of 45 seats is allowed for

each pair of Type I exits.

(3) If a passenger ventral or tail cone exit is installed and that exit

provides at least the same rate of egress as a Type III exit with the

airplane in the most adverse exit opening condition that would result from

the collapse of one or more legs of the landing gear, an increase in the

passenger seating configuration beyond the limits specified in paragraph (d)

(1) or (2) of this section may be allowed as follows:

(i) For a ventral exit, 12 additional passenger seats.

(ii) For a tail cone exit incorporating a floor level opening of not less

than 20 inches wide by 60 inches high, with corner radii not greater than

one-third the width of the exit, in the pressure shell and incorporating an

approved assist means in accordance with Sec. 25.809(h), 25 additional

passenger seats.

(iii) For a tail cone exit incorporating an opening in the pressure shell

which is at least equivalent to a Type III emergency exit with respect to

dimensions, step-up and step-down distance, and with the top of the opening

not less than 56 inches from the passenger compartment floor, 15 additional

passenger seats.

(4) For airplanes on which the vertical location of the wing does not allow

the installation of overwing exits, an exit of at least the dimensions of a

Type III exit must be installed instead of each Type IV exit required by

subparagraph (1) of this paragraph.

(5) An alternate emergency exit configuration may be approved in lieu of

that specified in paragraph (d) (1) or (2) of this section provided the

overall evacuation capability is shown to be equal to or greater than that of

the specified emergency exit configuration.

(6) The following must also meet the applicable emergency exit requirements

of Secs. 25.809 through 25.813:

(i) Each emergency exit in the passenger compartment in excess of the

minimum number of required emergency exits.

(ii) Any other floor level door or exit that is accessible from the

passenger compartment and is as large or larger than a Type II exit, but less

than 46 inches wide.

(iii) Any other passenger ventral or tail cone exit.

(7) For an airplane that is required to have more than one passenger

emergency exit for each side of the fuselage, no passenger emergency exit

shall be more than 60 feet from any adjacent passenger emergency exit on the

same side of the same deck of the fuselage, as measured parallel to the

airplane's longitudinal axis between the nearest exit edges.

(e) Ditching emergency exits for passengers. Ditching emergency exits must

be provided in accordance with the following requirements whether or not

certification with ditching provisions is requested:

(1) For airplanes that have a passenger seating configuration of nine seats

or less, excluding pilots seats, one exit above the waterline in each side of

the airplane, meeting at least the dimensions of a Type IV exit.

(2) For airplanes that have a passenger seating configuration of 10 seats

or more, excluding pilots seats, one exit above the waterline in a side of

the airplane, meeting at least the dimensions of a Type III exit for each

unit (or part of a unit) of 35 passenger seats, but no less than two such

exits in the passenger cabin, with one on each side of the airplane. The

passenger seat/exit ratio may be increased through the use of larger exits,

or other means, provided it is shown that the evacuation capability during

ditching has been improved accordingly.

(3) If it is impractical to locate side exits above the waterline, the side

exits must be replaced by an equal number of readily accessible overhead

hatches of not less than the dimensions of a Type III exit, except that for

airplanes with a passenger configuration of 35 seats or less, excluding

pilots seats, the two required Type III side exits need be replaced by only

one overhead hatch.

(f) Flightcrew emergency exits. For airplanes in which the proximity of

passenger emergency exits to the flightcrew area does not offer a convenient

and readily accessible means of evacuation of the flightcrew, and for all

airplanes having a passenger seating capacity greater than 20, flightcrew

exits shall be located in the flightcrew area. Such exits shall be of

sufficient size and so located as to permit rapid evacuation by the crew. One

exit shall be provided on each side of the airplane; or, alternatively, a top

hatch shall be provided. Each exit must encompass an unobstructed rectangular

opening of at least 19 by 20 inches unless satisfactory exit utility can be

demonstrated by a typical crewmember.

[Doc. No. 24344, Admt. 25-72, 55 FR 29781, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.809  Emergency exit arrangement.

(a) Each emergency exit, including a flight crew emergency exit, must be a

movable door or hatch in the external walls of the fuselage, allowing

unobstructed opening to the outside.

(b) Each emergency exit must be openable from the inside and the outside

except that sliding window emergency exits in the flight crew area need not

be openable from the outside if other approved exits are convenient and

readily accessible to the flight crew area. Each emergency exit must be

capable of being opened, when there is no fuselage deformation--

(1) With the airplane in the normal ground attitude and in each of the

attitudes corresponding to collapse of one or more legs of the landing gear;

and

(2) Within 10 seconds measured from the time when the opening means is

actuated to the time when the exit is fully opened.

(c) The means of opening emergency exits must be simple and obvious and may

not require exceptional effort. Internal exit-opening means involving

sequence operations (such as operation of two handles or latches or the

release of safety catches) may be used for flight crew emergency exits if it

can be reasonably established that these means are simple and obvious to

crewmembers trained in their use.

(d) If a single power-boost or single power-operated system is the primary

system for operating more than one exit in an emergency, each exit must be

capable of meeting the requirements of paragraph (b) of this section in the

event of failure of the primary system. Manual operation of the exit (after

failure of the primary system) is acceptable.

(e) Each emergency exit must be shown by tests, or by a combination of

analysis and tests, to meet the requirements of paragraphs (b) and (c) of

this section.

(f) There must be a means to lock each emergency exit and to safeguard

against its opening in flight, either inadvertently by persons or as a result

of mechanical failure. In addition, there must be a means for direct visual

inspection of the locking mechanism by crewmembers to determine that each

emergency exit, for which the initial opening movement is outward, is fully

locked.

(g) There must be provisions to minimize the probability of jamming of the

emergency exits resulting from fuselage deformation in a minor crash landing.

(h) When required by the operating rules for any large passenger-carrying

turbojet-powered airplane, each ventral exit and tailcone exit must be--

(1) Designed and constructed so that it cannot be opened during flight; and

(2) Marked with a placard readable from a distance of 30 inches and

installed at a conspicuous location near the means of opening the exit,

stating that the exit has been designed and constructed so that it cannot be

opened during flight.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-15, 32 FR

13264, Sept. 20, 1967; Amdt. 25-32, 37 FR 3970, Feb. 24, 1972; Amdt. 25-34,

37 FR 25355, Nov. 30, 1972; Amdt. 25-46, 43 FR 50597, Oct. 30, 1978; Amdt.

25-47, 44 FR 61325, Oct. 25, 1979; Amdt. 25-72, 55 FR 29782, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.810  Emergency egress assist means and escape routes.

(a) Each nonoverwing landplane emergency exit more than 6 feet from the

ground with the airplane on the ground and the landing gear extended and each

nonoverwing Type A exit must have an approved means to assist the occupants

in descending to the ground.

(1) The assisting means for each passenger emergency exit must be a self-

supporting slide or equivalent; and, in the case of a Type A exit, it must be

capable of carrying simultaneously two parallel lines of evacuees. In

addition, the assisting means must be designed to meet the following

requirements:

(i) It must be automatically deployed and deployment must begin during the

interval between the time the exit opening means is actuated from inside the

airplane and the time the exit is fully opened. However, each passenger

emergency exit which is also a passenger entrance door or a service door must

be provided with means to prevent deployment of the assisting means when it

is opened from either the inside or the outside under nonemergency conditions

for normal use.

(ii) It must be automatically erected within 10 seconds after deployment is

begun.

(iii) It must be of such length after full deployment that the lower end is

self-supporting on the ground and provides safe evacuation of occupants to

the ground after collapse of one or more legs of the landing gear.

(iv) It must have the capability, in 25-knot winds directed from the most

critical angle, to deploy and, with the assistance of only one person, to

remain usable after full deployment to evacuate occupants safely to the

ground.

(v) For each system installation (mockup or airplane installed), five

consecutive deployment and inflation tests must be conducted (per exit)

without failure, and at least three tests of each such five-test series must

be conducted using a single representative sample of the device. The sample

devices must be deployed and inflated by the system's primary means after

being subjected to the inertia forces specified in Sec. 25.561(b). If any

part of the system fails or does not function properly during the required

tests, the cause of the failure or malfunction must be corrected by positive

means and after that, the full series of five consecutive deployment and

inflation tests must be conducted without failure.

(2) The assisting means for flightcrew emergency exits may be a rope or any

other means demonstrated to be suitable for the purpose. If the assisting

means is a rope, or an approved device equivalent to a rope, it must be--

(i) Attached to the fuselage structure at or above the top of the emergency

exit opening, or, for a device at a pilot's emergency exit window, at another

approved location if the stowed device, or its attachment, would reduce the

pilot's view in flight;

(ii) Able (with its attachment) to withstand a 400-pound static load.

(b) Assist means from the cabin to the wing are required for each Type A

exit located above the wing and having a stepdown unless the exit without an

assist means can be shown to have a rate of passenger egress at least equal

to that of the same type of nonoverwing exit. If an assist means is required,

it must be automatically deployed and automatically erected, concurrent with

the opening of the exit and self-supporting within 10 seconds.

(c) An escape route must be established from each overwing emergency exit,

and (except for flap surfaces suitable as slides) covered with a slip

resistant surface. Except where a means for channeling the flow of evacuees

is provided--

(1) The escape route must be at least 42 inches wide at Type A passenger

emergency exits and must be at least 2 feet wide at all other passenger

emergency exits, and

(2) The escape route surface must have a reflectance of at least 80

percent, and must be defined by markings with a surface-to-marking contrast

ratio of at least 5:1.

(d) If the place on the airplane structure at which the escape route

required in paragraph (c) of this section terminates, is more than 6 feet

from the ground with the airplane on the ground and the landing gear

extended, means to reach the ground must be provided to assist evacuees who

have used the escape route. If the escape route is over a flap, the height of

the terminal edge must be measured with the flap in the takeoff or landing

position, whichever is higher from the ground. The assisting means must be

usable and self-supporting with one or more landing gear legs collapsed and

under a 25-knot wind directed from the most critical angle. The assisting

means provided for each escape route leading from a Type A emergency exit

must be capable of carrying simultaneously two parallel lines of evacuees.

For other than Type A exits, the assist means must be capable of carrying

simultaneously as many parallel lines of evacuees as there are required

escape routes.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29782, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.811  Emergency exit marking.

(a) Each passenger emergency exit, its means of access, and its means of

opening must be conspicuously marked.

(b) The identity and location of each passenger emergency exit must be

recognizable from a distance equal to the width of the cabin.

(c) Means must be provided to assist the occupants in locating the exits in

conditions of dense smoke.

(d) The location of each passenger emergency exit must be indicated by a

sign visible to occupants approaching along the main passenger aisle (or

aisles). There must be--

(1) A passenger emergency exit locator sign above the aisle (or aisles)

near each passenger emergency exit, or at another overhead location if it is

more practical because of low headroom, except that one sign may serve more

than one exit if each exit can be seen readily from the sign;

(2) A passenger emergency exit marking sign next to each passenger

emergency exit, except that one sign may serve two such exits if they both

can be seen readily from the sign; and

(3) A sign on each bulkhead or divider that prevents fore and aft vision

along the passenger cabin to indicate emergency exits beyond and obscured by

the bulkhead or divider, except that if this is not possible the sign may be

placed at another appropriate location.

(e) The location of the operating handle and instructions for opening exits

from the inside of the airplane must be shown in the following manner:

Each Type III passenger emergency exit operating handle must be self-

illuminated with an initial brightness of at least 160 microlamberts. If the

operating handle is covered, self-illuminated cover removal instructions

having an initial brightness of at least 160 microlamberts must also be

provided.

(1) Each passenger emergency exit must have, on or near the exit, a marking

that is readable from a distance of 30 inches.

(2) Each passenger emergency exit operating handle and the cover removal

instructions, if the operating handle is covered, must--

(i) Be self-illuminated with an initial brightness of at least 160

microlamberts; or

(ii) Be conspicuously located and well illuminated by the emergency

lighting even in conditions of occupant crowding at the exit.

(3) [Reserved]

(4) Each Type A, Type I, and Type II passenger emergency exit with a

locking mechanism released by rotary motion of the handle must be marked--

(i) With a red arrow, with a shaft at least three-fourths of an inch wide

and a head twice the width of the shaft, extending along at least 70 degrees

of arc at a radius approximately equal to three-fourths of the handle length.

(ii) So that the centerline of the exit handle is within +/-1 inch of the

projected point of the arrow when the handle has reached full travel and has

released the locking mechanism, and

(iii) With the word "open" in red letters 1 inch high, placed horizontally

near the head of the arrow.

(f) Each emergency exit that is required to be openable from the outside,

and its means of opening, must be marked on the outside of the airplane. In

addition, the following apply:

(1) The outside marking for each passenger emergency exit in the side of

the fuselage must include a 2-inch colored band outlining the exit.

(2) Each outside marking including the band, must have color contrast to be

readily distinguishable from the surrounding fuselage surface. The contrast

must be such that if the reflectance of the darker color is 15 percent or

less, the reflectance of the lighter color must be at least 45 percent.

"Reflectance" is the ratio of the luminous flux reflected by a body to the

luminous flux it receives. When the reflectance of the darker color is

greater than 15 percent, at least a 30-percent difference between its

reflectance and the reflectance of the lighter color must be provided.

(3) In the case of exists other than those in the side of the fuselage,

such as ventral or tail cone exists, the external means of opening, including

instructions if applicable, must be conspicuously marked in red, or bright

chrome yellow if the background color is such that red is inconspicuous. When

the opening means is located on only one side of the fuselage, a conspicuous

marking to that effect must be provided on the other side.

(g) Each sign required by paragraph (d) of this section may use the word

"exit" in its legend in place of the term "emergency exit".

[Amdt. 25-15, 32 FR 13264, Sept. 20, 1967, as amended by Amdt. 25-32, 37 FR

3970, Feb. 24, 1972; Amdt. 25-46, 43 FR 50597, Oct. 30, 1978; 43 FR 52495,

Nov. 13, 1978; Amdt. 25-79, 58 FR 45229, Aug. 26, 1993]

*****************************************************************************

58 FR 45224, No. 164, Aug. 26, 1993

SUMMARY: These amendments to the airworthiness standards for transport

category airplanes and the operating rules for air carrier operators of such

airplanes modify the procedures for conducting an emergency evacuation

demonstration. These include a requirement that the flightcrew take no active

role in the demonstration, and a change to the age/sex distribution

requirement for demonstration participants. In addition, the airworthiness

standards are amended to standardize the illumination requirements for the

handles of the various types of passenger emergency exits, and to add a

requirement to prevent the inadvertent disabling of the public address system

because of an unstowed microphone. These amendments are intended to enhance

the provisions for egress of occupants of transport category airplanes under

emergency conditions.

EFFECTIVE DATE: September 27, 1993.

*****************************************************************************

Sec. 25.812  Emergency lighting.

(a) An emergency lighting system, independent of the main lighting system,

must be installed. However, the sources of general cabin illumination may be

common to both the emergency and the main lighting systems if the power

supply to the emergency lighting system is independent of the power supply to

the main lighting system. The emergency lighting system must include:

(1) Illuminated emergency exit marking and locating signs, sources of

general cabin illumination, interior lighting in emergency exit areas, and

floor proximity escape path marking.

(2) Exterior emergency lighting.

(b) Emergency exit signs--

(1) For airplanes that have a passenger seating configuration, excluding

pilot seats, of 10 seats or more must meet the following requirements:

(i) Each passenger emergency exit locator sign required by Sec.

25.811(d)(1) and each passenger emergency exit marking sign required by Sec.

25.811(d)(2) must have red letters at least 1 1/2  inches high on an

illuminated white background, and must have an area of at least 21 square

inches excluding the letters. The lighted background-to-letter contrast must

be at least 10 : 1. The letter height to stroke-width ratio may not be more

than 7 : 1 nor less than 6 : 1. These signs must be internally electrically

illuminated with a background brightness of at least 25 foot-lamberts and a

high-to-low background contrast no greater than 3 : 1.

(ii) Each passenger emergency exit sign required by Sec. 25.811(d)(3) must

have red letters at least 1 1/2  inches high on a white background having an

area of at least 21 square inches excluding the letters. These signs must be

internally electrically illuminated or self-illuminated by other than

electrical means and must have an initial brightness of at least 400

microlamberts. The colors may be reversed in the case of a sign that is self-

illuminated by other than electrical means.

(2) For airplanes that have a passenger seating configuration, excluding

pilot seats, of nine seats or less, that are required by Sec. 25.811(d) (1),

(2), and (3) must have red letters at least 1 inch high on a white background

at least 2 inches high. These signs may be internally electrically

illuminated, or self-illuminated by other than electrical means, with an

initial brightness of at least 160 microlamberts. The colors may be reversed

in the case of a sign that is self-illuminated by other than electrical

means.

(c) General illumination in the passenger cabin must be provided so that

when measured along the centerline of main passenger aisle(s), and cross

aisle(s) between main aisles, at seat arm-rest height and at 40-inch

intervals, the average illumination is not less than 0.05 foot-candle and the

illumination at each 40-inch interval is not less than 0.01 foot-candle. A

main passenger aisle(s) is considered to extend along the fuselage from the

most forward passenger emergency exit or cabin occupant seat, whichever is

farther forward, to the most rearward passenger emergency exit or cabin

occupant seat, whichever is farther aft.

(d) The floor of the passageway leading to each floor-level passenger

emergency exit, between the main aisles and the exit openings, must be

provided with illumination that is not less than 0.02 foot-candle measured

along a line that is within 6 inches of and parallel to the floor and is

centered on the passenger evacuation path.

(e) Floor proximity emergency escape path marking must provide emergency

evacuation guidance for passengers when all sources of illumination more than

4 feet above the cabin aisle floor are totally obscured. In the dark of the

night, the floor proximity emergency escape path marking must enable each

passenger to--

(1) After leaving the passenger seat, visually identify the emergency

escape path along the cabin aisle floor to the first exits or pair of exits

forward and aft of the seat; and

(2) Readily identify each exit from the emergency escape path by reference

only to markings and visual features not more than 4 feet above the cabin

floor.

(f) Except for subsystems provided in accordance with paragraph (h) of this

section that serve no more than one assist means, are independent of the

airplane's main emergency lighting system, and are automatically activated

when the assist means is erected, the emergency lighting system must be

designed as follows.

(1) The lights must be operable manually from the flight crew station and

from a point in the passenger compartment that is readily accessible to a

normal flight attendant seat.

(2) There must be a flight crew warning light which illuminates when power

is on in the airplane and the emergency lighting control device is not armed.

(3) The cockpit control device must have an "on," "off," and "armed"

position so that when armed in the cockpit or turned on at either the cockpit

or flight attendant station the lights will either light or remain lighted

upon interruption (except an interruption caused by a transverse vertical

separation of the fuselage during crash landing) of the airplane's normal

electric power. There must be a means to safeguard against inadvertent

operation of the control device from the "armed" or "on" positions.

(g) Exterior emergency lighting must be provided as follows:

(1) At each overwing emergency exit the illumination must be--

(i) Not less than 0.03 foot-candle (measured normal to the direction of the

incident light) on a 2-square-foot area where an evacuee is likely to make

his first step outside the cabin;

(ii) Not less than 0.05 foot-candle (measured normal to the direction of

the incident light) for a minimum width of 42 inches for a Type A overwing

emergency exit and of 2 feet for all other overwing emergency exits along the

30 percent of the slip-resistant portion of the escape route required in Sec.

25.803(e) that is farthest from the exit; and

(iii) Not less than 0.03 foot-candle on the ground surface with the landing

gear extended (measured normal to the direction of the incident light) where

an evacuee using the established escape route would normally make first

contact with the ground.

(2) At each non-overwing emergency exit not required by Sec. 25.809(f) to

have descent assist means the illumination must be not less than 0.03 foot-

candle (measured normal to the direction of the incident light) on the ground

surface with the landing gear extended where an evacuee is likely to make his

first contact with the ground outside the cabin.

(h) The means required in Sec. 25.809 (f)(1) and (h) to assist the

occupants in descending to the ground must be illuminated so that the erected

assist means is visible from the airplane.

(1) If the assist means is illuminated by exterior emergency lighting, it

must provide illumination of not less than 0.03 foot-candle (measured normal

to the direction of the incident light) at the ground end of the erected

assist means where an evacuee using the established escape route would

normally make first contact with the ground, with the airplane in each of the

attitudes corresponding to the collapse of one or more legs of the landing

gear.

(2) If the emergency lighting subsystem illuminating the assist means

serves no other assist means, is independent of the airplane's main emergency

lighting system, and is automatically activated when the assist means is

erected, the lighting provisions--

(i) May not be adversely affected by stowage; and

(ii) Must provide illumination of not less than 0.03 foot-candle (measured

normal to the direction of incident light) at the ground and of the erected

assist means where an evacuee would normally make first contact with the

ground, with the airplane in each of the attitudes corresponding to the

collapse of one or more legs of the landing gear.

(i) The energy supply to each emergency lighting unit must provide the

required level of illumination for at least 10 minutes at the critical

ambient conditions after emergency landing.

(j) If storage batteries are used as the energy supply for the emergency

lighting system, they may be recharged from the airplane's main electric

power system: Provided, That, the charging circuit is designed to preclude

inadvertent battery discharge into charging circuit faults.

(k) Components of the emergency lighting system, including batteries,

wiring relays, lamps, and switches must be capable of normal operation after

having been subjected to the inertia forces listed in Sec. 25.561(b).

(l) The emergency lighting system must be designed so that after any single

transverse vertical separation of the fuselage during crash landing--

(1) Not more than 25 percent of all electrically illuminated emergency

lights required by this section are rendered inoperative, in addition to the

lights that are directly damaged by the separation;

(2) Each electrically illuminated exit sign required under Sec.

25.811(d)(2) remains operative exclusive of those that are directly damaged

by the separation; and

(3) At least one required exterior emergency light for each side of the

airplane remains operative exclusive of those that are directly damaged by

the separation.

[Amdt. 25-15, 32 FR 13265, Sept. 20, 1967, as amended by Amdt. 25-28, 36 FR

16899, Aug. 26, 1971; Amdt. 25-32, 37 FR 3971, Feb. 24, 1972; Amdt. 25-46, 43

FR 50597, Oct. 30, 1978; Amdt. 25-58, 49 FR 43186, Oct. 26, 1984]

Sec. 25.813  Emergency exit access.

Each required emergency exit must be accessible to the passengers and

located where it will afford an effective means of evacuation. Emergency exit

distribution must be as uniform as practical, taking passenger distribution

into account; however, the size and location of exits on both sides of the

cabin need not be symmetrical. If only one floor level exit per side is

prescribed, and the airplane does not have a tail cone or ventral emergency

exit, the floor level exit must be in the rearward part of the passenger

compartment, unless another location affords a more effective means of

passenger evacuation. Where more than one floor level exit per side is

prescribed, at least one floor level exit per side must be located near each

end of the cabin, except that this provision does not apply to combination

cargo/passenger configurations. In addition--

(a) There must be a passageway leading from the nearest main aisle to each

Type I, Type II, or Type A emergency exit and between individual passenger

areas. Each passageway leading to a Type A exit must be unobstructed and at

least 36 inches wide. Passageways between individual passenger areas and

those leading to Type I and Type II emergency exits must be unobstructed and

at least 20 inches wide. Unless there are two or more main aisles, each Type

A exit must be located so that there is passenger flow along the main aisle

to that exit from both the forward and aft directions. If two or more main

aisles are provided, there must be unobstructed cross-aisles at least 20

inches wide between main aisles. There must be--

(1) A cross-aisle which leads directly to each passageway between the

nearest main aisle and a Type A exit; and

(2) A cross-aisle which leads to the immediate vicinity of each passageway

between the nearest main aisle and a Type 1, Type II, or Type III exit;

except that when two Type III exits are located within three passenger rows

of each other, a single cross-aisle may be used if it leads to the vicinity

between the passageways from the nearest main aisle to each exit.

(b) Adequate space to allow crewmember(s) to assist in the evacuation of

passengers must be provided as follows:

(1) The assist space must not reduce the unobstructed width of the

passageway below that required for the exit.

(2) For each Type A exit, assist space must be provided at each side of the

exit regardless of whether the exit is covered by Sec. 25.810(a).

(3) For any other type exit that is covered by Sec. 25.810(a), space must

at least be provided at one side of the passageway.

(c) The following must be provided for each Type III or Type IV exit--(1)

There must be access from the nearest aisle to each exit. In addition, for

each Type III exit in an airplane that has a passenger seating configuration

of 60 or more--

(i) Except as provided in paragraph (c)(1)(ii), the access must be provided

by an unobstructed passageway that is at least 10 inches in width for

interior arrangements in which the adjacent seat rows on the exit side of the

aisle contain no more than two seats, or 20 inches in width for interior

arrangements in which those rows contain three seats. The width of the

passageway must be measured with adjacent seats adjusted to their most

adverse position. The centerline of the required passageway width must not be

displaced more than 5 inches horizontally from that of the exit.

(ii) In lieu of one 10- or 20-inch passageway, there may be two

passageways, between seat rows only, that must be at least 6 inches in width

and lead to an unobstructed space adjacent to each exit. (Adjacent exits must

not share a common passageway.) The width of the passageways must be measured

with adjacent seats adjusted to their most adverse position. The unobstructed

space adjacent to the exit must extend vertically from the floor to the

ceiling (or bottom of sidewall stowage bins), inboard from the exit for a

distance not less than the width of the narrowest passenger seat installed on

the airplane, and from the forward edge of the forward passageway to the aft

edge of the aft passageway. The exit opening must be totally within the fore

and aft bounds of the unobstructed space.

(2) In addition to the access--

(i) For airplanes that have a passenger seating configuration of 20 or

more, the projected opening of the exit provided must not be obstructed and

there must be no interference in opening the exit by seats, berths, or other

protrusions (including any seatback in the most adverse position) for a

distance from that exit not less than the width of the narrowest passenger

seat installed on the airplane.

(ii) For airplanes that have a passenger seating configuration of 19 or

fewer, there may be minor obstructions in this region, if there are

compensating factors to maintain the effectiveness of the exit.

(3) For each Type III exit, regardless of the passenger capacity of the

airplane in which it is installed, there must be placards that--

(i) Are readable by all persons seated adjacent to and facing a passageway

to the exit;

(ii) Accurately state or illustrate the proper method of opening the exit,

including the use of handholds; and

(iii) If the exit is a removable hatch, state the weight of the hatch and

indicate an appropriate location to place the hatch after removal.

(d) If it is necessary to pass through a passageway between passenger

compartments to reach any required emergency exit from any seat in the

passenger cabin, the passageway must be unobstructed. However, curtains may

be used if they allow free entry through the passageway.

(e) No door may be installed in any partition between passenger

compartments.

(f) If it is necessary to pass through a doorway separating the passenger

cabin from other areas to reach any required emergency exit from any

passenger seat, the door must have a means to latch it in open position. The

latching means must be able to withstand the loads imposed upon it when the

door is subjected to the ultimate inertia forces, relative to the surrounding

structure, listed in Sec. 25.561(b).

[Amdt. 25-1, 30 FR 3204, Mar. 9, 1965, as amended by Amdt. 25-15, 32 FR

13265, Sept. 20, 1967; Amdt. 25-32, 37 FR 3971, Feb. 24, 1972; Amdt. 25-46,

43 FR 50597, Oct. 30, 1978; Amdt. 25-72, 55 FR 29783, July 20, 1990; Amdt.

25-76, 57 FR 19244, May 4, 1992; 57 FR 29120, June 30, 1992]

*****************************************************************************

57 FR 19220, No. 86, May 4, 1992

Corrected. 57 FR 29120, No. 126, June 30, 1992

SUMMARY: This amendment revises the Federal Aviation Regulations (FAR) to

require improved access to the Type III emergency exits (typically smaller

over-wing exits) in transport category airplanes with 60 or more passenger

seats. These changes are the result of tests that were conducted at the

FAA's Civil Aeromedical Institute (CAMI), and are intended to improve the

ability of occupants to evacuate an airplane under emergency conditions.

They affect air carriers and commercial operators of transport category

airplanes as well as the manufacturers of such airplanes.

EFFECTIVE DATE: June 3, 1992.

*****************************************************************************

Sec. 25.815  Width of aisle.

The passenger aisle width at any point between seats must equal or exceed

the values in the following table:

Minimum

passenger

aisle width

(inches)

25

Less    in.

than    and

Passenger    25 in.  more

seating      from   from

capacity     floor   floor

10 or less     /1/ 12     15

11 through 19      12     20

20 or more         15     20

/1/ A narrower width not less than 9

inches may be approved when

substantiated by tests found necessary

by the Administrator.

[Amdt. 25-15, 32 FR 13265, Sept. 20, 1967, as amended by Amdt. 25-38, 41 FR

55466, Dec. 20, 1976]

Sec. 25.817  Maximum number of seats abreast.

On airplanes having only one passenger aisle, no more than three seats

abreast may be placed on each side of the aisle in any one row.

[Amdt. 25-15, 32 FR 13265, Sept. 20, 1967]

Sec. 25.819  Lower deck service compartments (including galleys).

For airplanes with a service compartment located below the main deck, which

may be occupied during taxi or flight but not during takeoff or landing, the

following apply:

(a) There must be at least two emergency evacuation routes, one at each end

of each lower deck service compartment or two having sufficient separation

within each compartment, which could be used by each occupant of the lower

deck service compartment to rapidly evacuate to the main deck under normal

and emergency lighting conditions. The routes must provide for the evacuation

of incapacitated persons, with assistance. The use of the evacuation routes

may not be dependent on any powered device. The routes must be designed to

minimize the possibility of blockage which might result from fire, mechanical

or structural failure, or persons standing on top of or against the escape

routes. In the event the airplane's main power system or compartment main

lighting system should fail, emergency illumination for each lower deck

service compartment must be automatically provided.

(b) There must be a means for two-way voice communication between the

flight deck and each lower deck service compartment.

(c) There must be an aural emergency alarm system, audible during normal

and emergency conditions, to enable crewmembers on the flight deck and at

each required floor level emergency exit to alert occupants of each lower

deck service compartment of an emergency situation.

(d) There must be a means, readily detectable by occupants of each lower

deck service compartment, that indicates when seat belts should be fastened.

(e) If a public address system is installed in the airplane, speakers must

be provided in each lower deck service compartment.

(f) For each occupant permitted in a lower deck service compartment, there

must be a forward or aft facing seat which meets the requirements of Sec.

25.785(c) and must be able to withstand maximum flight loads when occupied.

(g) For each powered lift system installed between a lower deck service

compartment and the main deck for the carriage of persons or equipment, or

both, the system must meet the following requirements:

(1) Each lift control switch outside the lift, except emergency stop

buttons, must be designed to prevent the activation of the life if the lift

door, or the hatch required by paragraph (g)(3) of this section, or both are

open.

(2) An emergency stop button, that when activated will immediately stop the

lift, must be installed within the lift and at each entrance to the lift.

(3) There must be a hatch capable of being used for evacuating persons from

the lift that is openable from inside and outside the lift without tools,

with the lift in any position.

[Amdt. 25-53, 45 FR 41593, June 19, 1980; 45 FR 43154, June 26, 1980]

Ventilation and Heating

Sec. 25.831  Ventilation.

(a) Each passenger and crew compartment must be ventilated, and each crew

compartment must have enough fresh air (but not less than 10 cu. ft. per

minute per crewmember) to enable crewmembers to perform their duties without

undue discomfort or fatigue.

(b) Crew and passenger compartment air must be free from harmful or

hazardous concentrations of gases or vapors. In meeting this requirement, the

following apply:

(1) Carbon monoxide concentrations in excess of 1 part in 20,000 parts of

air are considered hazardous. For test purposes, any acceptable carbon

monoxide detection method may be used.

(2) Carbon dioxide in excess of three percent by volume (sea level

equivalent) is considered hazardous in the case of crewmembers. Higher

concentrations of carbon dioxide may be allowed in crew compartments if

appropriate protective breathing equipment is available.

(c) There must be provisions made to ensure that the conditions prescribed

in paragraph (b) of this section are met after reasonably probable failures

or malfunctioning of the ventilating, heating, pressurization, or other

systems and equipment.

(d) If accumulation of hazardous quantities of smoke in the cockpit area is

reasonably probable, smoke evacuation must be readily accomplished, starting

with full pressurization and without depressurizing beyond safe limits.

(e) Except as provided in paragraph (f) of this section, means must be

provided to enable the occupants of the following compartments and areas to

control the temperature and quantity of ventilating air supplied to their

compartment or area independently of the temperature and quantity of air

supplied to other compartments and areas:

(1) The flight crew compartment.

(2) Crewmember compartments and areas other than the flight crew

compartment unless the crewmember compartment or area is ventilated by air

interchange with other compartments or areas under all operating conditions.

(f) Means to enable the flight crew to control the temperature and quantity

of ventilating air supplied to the flight crew compartment independently of

the temperature and quantity of ventilating air supplied to other

compartments are not required if all of the following conditions are met:

(1) The total volume of the flight crew and passenger compartments is 800

cubic feet or less.

(2) The air inlets and passages for air to flow between flight crew and

passenger compartments are arranged to provide compartment temperatures

within 5 degrees F. of each other and adequate ventilation to occupants in

both compartments.

(3) The temperature and ventilation controls are accessible to the flight

crew.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-41, 42 FR

36970, July 18, 1977]

Sec. 25.832  Cabin ozone concentration.

(a) The airplane cabin ozone concentration during flight must be shown now

to exceed--

(1) 0.25 parts per million by volume, sea level equivalent, at any time

above flight level 320; and

(2) 0.01 parts per million by volume, sea level equivalent, time-weighted

average during any 3-hour interval above flight level 270.

(b) For the purpose of this section, "sea level equivalent" refers to

conditions of 25 deg. C and 760 millimeters of mercury pressure.

(c) Compliance with this section must be shown by analysis or tests based

on airplane operational procedures and performance limitations, that

demonstrate that either--

(1) The airplane cannot be operated at an altitude which would result in

cabin ozone concentrations exceeding the limits prescribed by paragraph (a)

of this section; or

(2) The airplane ventilation system, including any ozone control equipment,

will maintain cabin ozone concentrations at or below the limits prescribed by

paragraph (a) of this section.

[Amdt. 25-50, 45 FR 3883, Jan. 1, 1980, as amended by Amdt. 25-56, 47 FR

58489, Dec. 30, 1982]

Sec. 25.833  Combustion heating systems.

Combustion heaters must be approved.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29783, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Pressurization

Sec. 25.841  Pressurized cabins.

(a) Pressurized cabins and compartments to be occupied must be equipped to

provide a cabin pressure altitude of not more than 8,000 feet at the maximum

operating altitude of the airplane under normal operating conditions. If

certification for operation over 25,000 feet is requested, the airplane must

be able to maintain a cabin pressure altitude of not more than 15,000 feet in

the event of any reasonably probable failure or malfunction in the

pressurization system.

(b) Pressurized cabins must have at least the following valves, controls,

and indicators for controlling cabin pressure:

(1) Two pressure relief valves to automatically limit the positive pressure

differential to a predetermined value at the maximum rate of flow delivered

by the pressure source. The combined capacity of the relief valves must be

large enough so that the failure of any one valve would not cause an

appreciable rise in the pressure differential. The pressure differential is

positive when the internal pressure is greater than the external.

(2) Two reverse pressure differential relief valves (or their equivalents)

to automatically prevent a negative pressure differential that would damage

the structure. One valve is enough, however, if it is of a design that

reasonably precludes its malfunctioning.

(3) A means by which the pressure differential can be rapidly equalized.

(4) An automatic or manual regulator for controlling the intake or exhaust

airflow, or both, for maintaining the required internal pressures and airflow

rates.

(5) Instruments at the pilot or flight engineer station to show the

pressure differential, the cabin pressure altitude, and the rate of change of

the cabin pressure altitude.

(6) Warning indication at the pilot or flight engineer station to indicate

when the safe or preset pressure differential and cabin pressure altitude

limits are exceeded. Appropriate warning markings on the cabin pressure

differential indicator meet the warning requirement for pressure differential

limits and an aural or visual signal (in addition to cabin altitude

indicating means) meets the warning requirement for cabin pressure altitude

limits if it warns the flight crew when the cabin pressure altitude exceeds

10,000 feet.

(7) A warning placard at the pilot or flight engineer station if the

structure is not designed for pressure differentials up to the maximum relief

valve setting in combination with landing loads.

(8) The pressure sensors necessary to meet the requirements of paragraphs

(b)(5) and (b)(6) of this section and Sec. 25.1447(c), must be located and

the sensing system designed so that, in the event of loss of cabin pressure

in any passenger or crew compartment (including upper and lower lobe

galleys), the warning and automatic presentation devices, required by those

provisions, will be actuated without any delay that would significantly

increase the hazards resulting from decompression.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55466, Dec. 20, 1976]

Sec. 25.843  Tests for pressurized cabins.

(a) Strength test. The complete pressurized cabin, including doors,

windows, and valves, must be tested as a pressure vessel for the pressure

differential specified in Sec. 25.365(d).

(b) Functional tests. The following functional tests must be performed:

(1) Tests of the functioning and capacity of the positive and negative

pressure differential valves, and of the emergency release valve, to

stimulate the effects of closed regulator valves.

(2) Tests of the pressurization system to show proper functioning under

each possible condition of pressure, temperature, and moisture, up to the

maximum altitude for which certification is requested.

(3) Flight tests, to show the performance of the pressure supply, pressure

and flow regulators, indicators, and warning signals, in steady and stepped

climbs and descents at rates corresponding to the maximum attainable within

the operating limitations of the airplane, up to the maximum altitude for

which certification is requested.

(4) Tests of each door and emergency exit, to show that they operate

properly after being subjected to the flight tests prescribed in paragraph

(b)(3) of this section.

Fire Protection

Sec. 25.851   Fire extinguishers.

(a) Hand fire extinguishers. (1) The following minimum number of hand fire

extinguishers must be conveniently located and evenly distributed in

passenger compartments:

Passenger        No. of

capacity      extinguishers

7 through 30                1

31 through 60               2

61 through 200              3

201 through 300              4

301 through 400              5

401 through 500              6

501 through 600              7

601 through 700              8

(2) At least one hand fire extinguisher must be conveniently located in the

pilot compartment.

(3) At least one readily accessible hand fire extinguisher must be

available for use in each Class A or Class B cargo or baggage compartment and

in each Class E cargo or baggage compartment that is accessible to

crewmembers in flight.

(4) At least one hand fire extinguisher must be located in, or readily

accessible for use in, each galley located above or below the passenger

compartment.

(5) Each hand fire extinguisher must be approved.

(6) At least one of the required fire extinguishers located in the

passenger compartment of an airplane with a passenger capacity of at least 31

and not more than 60, and at least two of the fire extinguishers located in

the passenger compartment of an airplane with a passenger capacity of 61 or

more must contain Halon 1211 (bromochlorodifluoromethane CBrC1F2), or

equivalent, as the extinguishing agent. The type of extinguishing agent used

in any other extinguisher required by this section must be appropriate for

the kinds of fires likely to occur where used.

(7) The quantity of extinguishing agent used in each extinguisher required

by this section must be appropriate for the kinds of fires likely to occur

where used.

(8) Each extinguisher intended for use in a personnel compartment must be

designed to minimize the hazard of toxic gas concentration.

(b) Built-in fire extinguishers. If a built-in fire extinguisher is

provided--

(1) Each built-in fire extinguishing system must be installed so that--

(i) No extinguishing agent likely to enter personnel compartments will be

hazardous to the occupants; and

(ii) No discharge of the extinguisher can cause structural damage.

(2) The capacity of each required built-in fire extinguishing system must

be adequate for any fire likely to occur in the compartment where used,

considering the volume of the compartment and the ventilation rate.

[56 FR 15456, April 16, 1991]

*****************************************************************************

56 FR 15450, No. 73, Apr. 16, 1991

SUMMARY: This amendment provides improved cabin fire protection for

transport category airplanes by requiring: (1) Each lavatory in an airplane

with a passenger seating capacity of 20 or more to be equipped with a smoke

detector system that provides warning to the cockpit or to the passenger

cabin crew; (2) each lavatory trash receptacle in an airplane with a seating

capacity of 20 or more to be equipped with a fire extinguisher that

discharges automatically upon the occurrence of a fire within the

receptacle; (3) the number of hand fire extinguishers in the cabins of

airplanes with passenger seating capacities greater than 200 to be

increased; (4) a specified number of the hand fire extinguishers in the

cabin to contain Halon 1211 or equivalent as the extinguishing agent; and

(5) one hand fire extinguisher in each galley that is located above or below

the passenger compartment. In addition, one hand fire extinguisher would be

required for certain all-cargo airplanes. These safety protections against

possible inflight fires are currently required for operation of airplanes

used in air carrier or commercial service. This amendment adopts these

requirements as design standards for transport category airplanes.

EFFECTIVE DATE: May 16, 1991.

*****************************************************************************

Sec. 25.853  Compartment interiors.

For each compartment occupied by the crew or passengers, the following

apply:

(a) Materials (including finishes or decorative surfaces applied to the

materials) must meet the applicable test criteria prescribed in part I of

appendix F of this part or other approved equivalent methods.

(b) In addition to meeting the requirements of paragraph (a), seat

cushions, except those on flight crewmember seats, must meet the test

requirements of part II of appendix F of this part, or equivalent.

(c) For airplanes with passenger capacities of 20 or more, interior ceiling

and wall panels (other than lighting lenses), partitions, and the outer

surfaces of galleys, large cabinets and stowage compartments (other than

underseat stowage compartments and compartments for stowing small items, such

as magazines and maps) must also meet the test requirements of parts IV and V

of appendix F of this part, or other approved equivalent method, in addition

to the flammability requirements prescribed in paragraph (a) of this section.

(d) Smoking is not to be allowed in lavatories. If smoking is to be allowed

in any compartment occupied by the crew or passengers, an adequate number of

self-contained, removable ashtrays must be provided for all seated occupants,

and

(e) Regardless of whether smoking is allowed in any other part of the

airplane, lavatories must have self-contained removable ashtrays located

conspicuously on or near the entry side of each lavatory door, except that

one ashtray may serve more than one lavatory door if the ashtray can be seen

readily from the cabin side of each lavatory served.

(f) Each receptacle used for the disposal of flammable waste material must

be fully enclosed, constructed of at least fire resistant materials, and must

contain fires likely to occur in it under normal use. The ability of the

receptacle to contain those fires under all probable conditions of wear,

misalignment, and ventilation expected in service must be demonstrated by

test.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29783, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.854   Lavatory fire protection.

For airplanes with a passenger capacity of 20 or more:

(a) Each lavatory must be equipped with a smoke detector system or

equivalent that provides a warning light in the cockpit, or provides a

warning light or audible warning in the passenger cabin that would be readily

detected by a flight attendant; and

(b) Each lavatory must be equipped with a built-in fire extinguisher for

each disposal receptacle for towels, paper, or waste, located within the

lavatory. The extinguisher must be designed to discharge automatically into

each disposal receptacle upon occurrence of a fire in that receptacle.

[56 FR 15456, April 16, 1991]

*****************************************************************************

56 FR 15450, No. 73, Apr. 16, 1991

SUMMARY: This amendment provides improved cabin fire protection for

transport category airplanes by requiring: (1) Each lavatory in an airplane

with a passenger seating capacity of 20 or more to be equipped with a smoke

detector system that provides warning to the cockpit or to the passenger

cabin crew; (2) each lavatory trash receptacle in an airplane with a seating

capacity of 20 or more to be equipped with a fire extinguisher that

discharges automatically upon the occurrence of a fire within the

receptacle; (3) the number of hand fire extinguishers in the cabins of

airplanes with passenger seating capacities greater than 200 to be

increased; (4) a specified number of the hand fire extinguishers in the

cabin to contain Halon 1211 or equivalent as the extinguishing agent; and

(5) one hand fire extinguisher in each galley that is located above or below

the passenger compartment. In addition, one hand fire extinguisher would be

required for certain all-cargo airplanes. These safety protections against

possible inflight fires are currently required for operation of airplanes

used in air carrier or commercial service. This amendment adopts these

requirements as design standards for transport category airplanes.

EFFECTIVE DATE: May 16, 1991.

*****************************************************************************

Sec. 25.855  Cargo or baggage compartments.

For each cargo and baggage compartment not occupied by crew or passengers,

the following apply:

(a) The compartment must meet one of the class requirements of Sec. 25.857.

(b) Class B through Class E cargo or baggage compartments, as defined in

Sec. 25.857, must have a liner, and the liner must be separate from (but may

be attached to) the airplane structure.

(c) Ceiling and sidewall liner panels of Class C and D compartments must

meet the test requirements of part III of appendix F of this part or other

approved equivalent methods.

(d) All other materials used in the construction of the cargo or baggage

compartment must meet the applicable test criteria prescribed in part I of

appendix F of this part or other approved equivalent methods.

(e) No compartment may contain any controls, wiring, lines, equipment, or

accessories whose damage or failure would affect safe operation, unless those

items are protected so that--

(1) They cannot be damaged by the movement of cargo in the compartment, and

(2) Their breakage or failure will not create a fire hazard.

(f) There must be means to prevent cargo or baggage from interfering with

the functioning of the fire protective features of the compartment.

(g) Sources of heat within the compartment must be shielded and insulated

to prevent igniting the cargo or baggage.

(h) Flight tests must be conducted to show compliance with the provisions

of Sec. 25.857 concerning--

(1) Compartment accessibility,

(2) The entries of hazardous quantities of smoke or extinguishing agent

into compartments occupied by the crew or passengers, and

(3) The dissipation of the extinguishing agent in Class C compartments.

(i) During the above tests, it must be shown that no inadvertent operation

of smoke or fire detectors in any compartment would occur as a result of fire

contained in any other compartment, either during or after extinguishment,

unless the extinguishing system floods each such compartment simultaneously.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29784, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.857  Cargo compartment classification.

(a) Class A; A Class A cargo or baggage compartment is one in which--

(1) The presence of a fire would be easily discovered by a crewmember while

at his station; and

(2) Each part of the compartment is easily accessible in flight.

(b) Class B. A Class B cargo or baggage compartment is one in which--

(1) There is sufficient access in flight to enable a crewmember to

effectively reach any part of the compartment with the contents of a hand

fire extinguisher;

(2) When the access provisions are being used, no hazardous quantity of

smoke, flames, or extinguishing agent, will enter any compartment occupied by

the crew or passengers;

(3) There is a separate approved smoke detector or fire detector system to

give warning at the pilot or flight engineer station.

(c) Class C. A Class C cargo or baggage compartment is one not meeting the

requirements for either a Class A or B compartment but in which--

(1) There is a separate approved smoke detector or fire detector system to

give warning at the pilot or flight engineer station;

(2) There is an approved built-in fire-extinguishing system controllable

from the pilot or flight engineer stations;

(3) There are means to exclude hazardous quantities of smoke, flames, or

extinguishing agent, from any compartment occupied by the crew or passengers;

(4) There are means to control ventilation and drafts within the

compartment so that the extinguishing agent used can control any fire that

may start within the compartment.

(d) Class D. A Class D cargo or baggage compartment is one in which--

(1) A fire occurring in it will be completely confined without endangering

the safety of the airplane or the occupants;

(2) There are means to exclude hazardous quantities of smoke, flames, or

other noxious gases, from any compartment occupied by the crew or passengers;

(3) Ventilation and drafts are controlled within each compartment so that

any fire likely to occur in the compartment will not progress beyond safe

limits; and

(4) [Reserved]

(5) Consideration is given to the effect of heat within the compartment on

adjacent critical parts of the airplane. For compartments of 500 cu. ft. or

less, an airflow of 1500 cu. ft. per hour is acceptable.

(6) The compartment volume does not exceed 1,000 cubic feet.

(e) Class E. A Class E cargo compartment is one on airplanes used only for

the carriage of cargo and in which--

(1) [Reserved]

(2) There is a separate approved smoke or fire detector system to give

warning at the pilot or flight engineer station;

(3) There are means to shut off the ventilating airflow to, or within, the

compartment, and the controls for these means are accessible to the flight

crew in the crew compartment;

(4) There are means to exclude hazardous quantities of smoke, flames, or

noxious gases, from the flight crew compartment; and

(5) The required crew emergency exits are accessible under any cargo

loading condition.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-32, 37 FR

3972, Feb. 24, 1972; Amdt. 25-60, 51 FR 18243, May 16, 1986]

Sec. 25.858   Cargo compartment fire detection systems.

If certification with cargo compartment fire detection provisions is

requested, the following must be met for each cargo compartment with those

provisions:

(a) The detection system must provide a visual indication to the flight

crew within one minute after the start of a fire.

(b) The system must be capable of detecting a fire at a temperature

significantly below that at which the structural integrity of the airplane is

substantially decreased.

(c) There must be means to allow the crew to check in flight, the

functioning of each fire detector circuit.

(d) The effectiveness of the detection system must be shown for all

approved operating configurations and conditions.

[Amdt. 25-54, 45 FR 60173, Sept. 11, 1980]

Sec. 25.859  Combustion heater fire protection.

(a) Combustion heater fire zones. The following combustion heater fire

zones must be protected from fire in accordance with the applicable

provisions of Secs. 25.1181 through 25.1191 and Secs. 25.1195 through

25.1203;

(1) The region surrounding the heater, if this region contains any

flammable fluid system components (excluding the heater fuel system), that

could--

(i) Be damaged by heater malfunctioning; or

(ii) Allow flammable fluids or vapors to reach the heater in case of

leakage.

(2) The region surrounding the heater, if the heater fuel system has

fittings that, if they leaked, would allow fuel or vapors to enter this

region.

(3) The part of the ventilating air passage that surrounds the combustion

chamber. However, no fire extinguishment is required in cabin ventilating air

passages.

(b) Ventilating air ducts. Each ventilating air duct passing through any

fire zone must be fireproof. In addition--

(1) Unless isolation is provided by fireproof valves or by equally

effective means, the ventilating air duct downstream of each heater must be

fireproof for a distance great enough to ensure that any fire originating in

the heater can be contained in the duct; and

(2) Each part of any ventilating duct passing through any region having a

flammable fluid system must be constructed or isolated from that system so

that the malfunctioning of any component of that system cannot introduce

flammable fluids or vapors into the ventilating airstream.

(c) Combustion air ducts. Each combustion air duct must be fireproof for a

distance great enough to prevent damage from backfiring or reverse flame

propagation. In addition--

(1) No combustion air duct may have a common opening with the ventilating

airstream unless flames from backfires or reverse burning cannot enter the

ventilating airstream under any operating condition, including reverse flow

or malfunctioning of the heater or its associated components; and

(2) No combustion air duct may restrict the prompt relief of any backfire

that, if so restricted, could cause heater failure.

(d) Heater controls; general. Provision must be made to prevent the

hazardous accumulation of water or ice on or in any heater control component,

control system tubing, or safety control.

(e) Heater safety controls. For each combustion heater there must be the

following safety control means:

(1) Means independent of the components provided for the normal continuous

control of air temperature, airflow, and fuel flow must be provided, for each

heater, to automatically shut off the ignition and fuel supply to that heater

at a point remote from that heater when any of the following occurs:

(i) The heat exchanger temperature exceeds safe limits.

(ii) The ventilating air temperature exceeds safe limits.

(iii) The combustion airflow becomes inadequate for safe operation.

(iv) The ventilating airflow becomes inadequate for safe operation.

(2) The means of complying with paragraph (e)(1) of this section for any

individual heater must--

(i) Be independent of components serving any other heater whose heat output

is essential for safe operation; and

(ii) Keep the heater off until restarted by the crew.

(3) There must be means to warn the crew when any heater whose heat output

is essential for safe operation has been shut off by the automatic means

prescribed in paragraph (e)(1) of this section.

(f) Air intakes. Each combustion and ventilating air intake must be located

so that no flammable fluids or vapors can enter the heater system under any

operating condition--

(1) During normal operation; or

(2) As a result of the malfunctioning of any other component.

(g) Heater exhaust. Heater exhaust systems must meet the provisions of

Secs. 25.1121 and 25.1123. In addition, there must be provisions in the

design of the heater exhaust system to safely expel the products of

combustion to prevent the occurrence of--

(1) Fuel leakage from the exhaust to surrounding compartments;

(2) Exhaust gas impingement on surrounding equipment or structure;

(3) Ignition of flammable fluids by the exhaust, if the exhaust is in a

compartment containing flammable fluid lines; and

(4) Restriction by the exhaust of the prompt relief of backfires that, if

so restricted, could cause heater failure.

(h) Heater fuel systems. Each heater fuel system must meet each powerplant

fuel system requirement affecting safe heater operation. Each heater fuel

system component within the ventilating airstream must be protected by

shrouds so that no leakage from those components can enter the ventilating

airstream.

(i) Drains. There must be means to safely drain fuel that might accumulate

within the combustion chamber or the heat exchanger. In addition--

(1) Each part of any drain that operates at high temperatures must be

protected in the same manner as heater exhausts; and

(2) Each drain must be protected from hazardous ice accumulation under any

operating condition.

[Doc. No. 5066, 29 FR 18291, Dec. 24 1964, as amended by Amdt. 25-11, 32 FR

6912, May 5, 1967; Amdt. 25-23, 35 FR 5676, Apr. 8, 1970]

Sec. 25.863  Flammable fluid fire protection.

(a) In each area where flammable fluids or vapors might escape by leakage

of a fluid system, there must be means to minimize the probability of

ignition of the fluids and vapors, and the resultant hazards if ignition does

occur.

(b) Compliance with paragraph (a) of this section must be shown by analysis

or tests, and the following factors must be considered:

(1) Possible sources and paths of fluid leakage, and means of detecting

leakage.

(2) Flammability characteristics of fluids, including effects of any

combustible or absorbing materials.

(3) Possible ignition sources, including electrical faults, overheating of

equipment, and malfunctioning of protective devices.

(4) Means available for controlling or extinguishing a fire, such as

stopping flow of fluids, shutting down equipment, fireproof containment, or

use of extinguishing agents.

(5) Ability of airplane components that are critical to safety of flight to

withstand fire and heat.

(c) If action by the flight crew is required to prevent or counteract a

fluid fire (e.g., equipment shutdown or actuation of a fire extinguisher)

quick acting means must be provided to alert the crew.

(d) Each area where flammable fluids or vapors might escape by leakage of a

fluid system must be identified and defined.

[Amdt. 25-23, 35 FR 5676, Apr. 8, 1970, as amended by Amdt. 25-46, 43 FR

50597, Oct. 30, 1978]

Sec. 25.865  Fire protection of flight controls, engine mounts, and other

flight structure.

Essential flight controls, engine mounts, and other flight structures

located in designated fire zones or in adjacent areas which would be

subjected to the effects of fire in the fire zone must be constructed of

fireproof material or shielded so that they are capable of withstanding the

effects of fire.

[Amdt. 25-23, 35 FR 5676, Apr. 8, 1970]

Sec. 25.867  Fire protection: other components.

(a) Surfaces to the rear of the nacelles, within one nacelle diameter of

the nacelle centerline, must be at least fire-resistant.

(b) Paragraph (a) of this section does not apply to tail surfaces to the

rear of the nacelles that could not be readily affected by heat, flames, or

sparks coming from a designated fire zone or engine compartment of any

nacelle.

[Amdt. 25-23, 35 FR 5676, Apr. 8, 1970]

Sec. 25.869  Fire protection: systems.

(a) Electrical system components:

(1) Components of the electrical system must meet the applicable fire and

smoke protection requirements of Secs. 25.831(c) and 25.863.

(2) Electrical cables, terminals, and equipment in designated fire zones,

that are used during emergency procedures, must be at least fire resistant.

(3) Main power cables (including generator cables) in the fuselage must be

designed to allow a reasonable degree of deformation and stretching without

failure and must be--

(i) Isolated from flammable fluid lines; or

(ii) Shrouded by means of electrically insulated, flexible conduit, or

equivalent, which is in addition to the normal cable insulation.

(4) Insulation on electrical wire and electrical cable installed in any

area of the fuselage must be self-extinguishing when tested in accordance

with the applicable portions of part I, appendix F of this part.

(b) Each vacuum air system line and fitting on the discharge side of the

pump that might contain flammable vapors or fluids must meet the requirements

of Sec. 25.1183 if the line or fitting is in a designated fire zone. Other

vacuum air systems components in designated fire zones must be at least fire

resistant.

(c) Oxygen equipment and lines must--

(1) Not be located in any designated fire zone,

(2) Be protected from heat that may be generated in, or escape from, any

designated fire zone, and

(3) Be installed so that escaping oxygen cannot cause ignition of grease,

fluid, or vapor accumulations that are present in normal operation or as a

result of failure or malfunction of any system.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29784, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Miscellaneous

Sec. 25.871  Leveling means.

There must be means for determining when the airplane is in a level

position on the ground.

[Amdt. 25-23, 35 FR 5676, Apr. 8, 1970]

Sec. 25.875  Reinforcement near propellers.

(a) Each part of the airplane near the propeller tips must be strong and

stiff enough to withstand the effects of the induced vibration and of ice

thrown from the propeller.

(b) No window may be near the propeller tips unless it can withstand the

most severe ice impact likely to occur.

General

Sec. 25.901  Installation.

(a) For the purpose of this part, the airplane powerplant installation

includes each component that--

(1) Is necessary for propulsion;

(2) Affects the control of the major propulsive units; or

(3) Affects the safety of the major propulsive units between normal

inspections or overhauls.

(b) For each powerplant--

(1) The installation must comply with--

(i) The installation instructions provided under Sec. 33.5 of this chapter;

and

(ii) The applicable provisions of this subpart;

(2) The components of the installation must be constructed, arranged, and

installed so as to ensure their continued safe operation between normal

inspections or overhauls;

(3) The installation must be accessible for necessary inspections and

maintenance; and

(4) The major components of the installation must be electrically bonded to

the other parts of the airplane.

(c) For each powerplant and auxiliary power unit installation, it must be

established that no single failure or malfunction or probable combination of

failures will jeopardize the safe operation of the airplane except that the

failure of structural elements need not be considered if the probability of

such failure is extremely remote.

(d) Each auxiliary power unit installation must meet the applicable

provisions of this subpart.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5676, Apr. 8, 1970; Amdt. 25-40, 42 FR 15042, Mar. 17, 1977; Amdt. 25-46, 43

FR 50597, Oct. 30, 1978]

Sec. 25.903  Engines.

(a) Engine type certificate.

(1) Each engine must have a type certificate and must meet the applicable

requirements of part 34 of this chapter.

(2) Each turbine engine must either--

(i) Comply with Sec. 33.77 of this chapter in effect on October 31, 1974,

or as subsequently amended; or

(ii) Be shown to have a foreign object ingestion service history in similar

installation locations which has not resulted in any unsafe condition.

(b) Engine isolation. The powerplants must be arranged and isolated from

each other to allow operation, in at least one configuration, so that the

failure or malfunction of any engine, or of any system that can affect the

engine, will not--

(1) Prevent the continued safe operation of the remaining engines; or

(2) Require immediate action by any crewmember for continued safe

operation.

(c) Control of engine rotation. There must be means for stopping the

rotation of any engine individually in flight, except that, for turbine

engine installations, the means for stopping the rotation of any engine need

be provided only where continued rotation could jeopardize the safety of the

airplane. Each component of the stopping and restarting system on the engine

side of the firewall that might be exposed to fire must be at least fire-

resistant. If hydraulic propeller feathering systems are used for this

purpose, the feathering lines must be at least fire resistant under the

operating conditions that may be expected to exist during feathering.

(d) Turbine engine installations. For turbine engine installations--

(1) Design precautions must be taken to minimize the hazards to the

airplane in the event of an engine rotor failure or of a fire originating

within the engine which burns through the engine case.

(2) The powerplant systems associated with engine control devices, systems,

and instrumentation, must be designed to give reasonable assurance that those

engine operating limitations that adversely affect turbine rotor structural

integrity will not be exceeded in service.

(e) Restart capability. (1) Means to restart any engine in flight must be

provided.

(2) An altitude and airspeed envelope must be established for in-flight

engine restarting, and each engine must have a restart capability within that

envelope.

(3) For turbine engine powered airplanes, if the minimum windmilling speed

of the engines, following the inflight shutdown of all engines, is

insufficient to provide the necessary electrical power for engine ignition, a

power source independent of the engine-driven electrical power generating

system must be provided to permit in-flight engine ignition for restarting.

(f) Auxiliary Power Unit. Each auxiliary power unit must be approved or

meet the requirements of the category for its intended use.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5676, Apr. 8, 1970; Amdt. 25-40, 42 FR 15042, Mar. 17, 1977; Amdt. 25-57, 49

FR 6848, Feb. 23, 1984; Amdt. 25-72, 55 FR 29784, July 20, 1990; Amdt. 25-73,

55 FR 32861, Aug. 10, 1990; 55 FR 35139, Aug. 28, 1990]

EFFECTIVE DATE NOTE: At Amdt. 25-72, 55 FR 29784, July 20, 1990, Sec.

25.903 was amended by adding paragraph (f) effective Aug. 20, 1990.

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

Sec. 25.904  Automatic takeoff thrust control system (ATTCS).

Each applicant seeking approval for installation of an engine power control

system that automatically resets the power or thrust on the operating

engine(s) when any engine fails during the takeoff must comply with the

requirements of Appendix I of this part.

[Amdt. 25-62, 52 FR 43156, Nov. 9, 1987]

Sec. 25.905  Propellers.

(a) Each propeller must have a type certificate.

(b) Engine power and propeller shaft rotational speed may not exceed the

limits for which the propeller is certificated.

(c) Each component of the propeller blade pitch control system must meet

the requirements of Sec. 35.42 of this chapter.

(d) Design precautions must be taken to minimize the hazards to the

airplane in the event a propeller blade fails or is released by a hub

failure. The hazards which must be considered include damage to structure and

vital systems due to impact of a failed or released blade and the unbalance

created by such failure or release.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-54, 45 FR

60173, Sept. 11, 1980; Amdt. 25-57, 49 FR 6848, Feb. 23, 1984; Amdt 25-72,

55 FR 29784, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.907  Propeller vibration.

(a) The magnitude of the propeller blade vibration stresses under any

normal condition of operation must be determined by actual measurement or by

comparison with similar installations for which these measurements have been

made.

(b) The determined vibration stresses may not exceed values that have been

shown to be safe for continuous operation.

Sec. 25.925  Propeller clearance.

Unless smaller clearances are substantiated, propeller clearances with the

airplane at maximum weight, with the most adverse center of gravity, and with

the propeller in the most adverse pitch position, may not be less than the

following:

(a) Ground clearance. There must be a clearance of at least seven inches

(for each airplane with nose wheel landing gear) or nine inches (for each

airplane with tail wheel landing gear) between each propeller and the ground

with the landing gear statically deflected and in the level takeoff, or

taxiing attitude, whichever is most critical. In addition, there must be

positive clearance between the propeller and the ground when in the level

takeoff attitude with the critical tire(s) completely deflated and the

corresponding landing gear strut bottomed.

(b) Water clearance. There must be a clearance of at least 18 inches

between each propeller and the water, unless compliance with Sec. 25.239(a)

can be shown with a lesser clearance.

(c) Structural clearance. There must be--

(1) At least one inch radial clearance between the blade tips and the

airplane structure, plus any additional radial clearance necessary to prevent

harmful vibration;

(2) At least one-half inch longitudinal clearance between the propeller

blades or cuffs and stationary parts of the airplane; and

(3) Positive clearance between other rotating parts of the propeller or

spinner and stationary parts of the airplane.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55

FR 29784, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.929  Propeller deicing.

(a) For airplanes intended for use where icing may be expected, there must

be a means to prevent or remove hazardous ice accumulation on propellers or

on accessories where ice accumulation would jeopardize engine performance.

(b) If combustible fluid is used for propeller deicing, Secs. 25.1181

through 25.1185 and 25.1189 apply.

Sec. 25.933  Reversing systems.

(a) For turbojet reversing systems--

(1) Each system intended for ground operation only must be designed so that

during any reversal in flight the engine will produce no more than flight

idle thrust. In addition, it must be shown by analysis or test, or both,

that--

(i) Each operable reverser can be restored to the forward thrust position;

and

(ii) The airplane is capable of continued safe flight and landing under any

possible position of the thrust reverser.

(2) Each system intended for inflight use must be designed so that no

unsafe condition will result during normal operation of the system, or from

any failure (or reasonably likely combination of failures) of the reversing

system, under any anticipated condition of operation of the airplane

including ground operation. Failure of structural elements need not be

considered if the probability of this kind of failure is extremely remote.

(3) Each system must have means to prevent the engine from producing more

than idle thrust when the reversing system malfunctions, except that it may

produce any greater forward thrust that is shown to allow directional control

to be maintained, with aerodynamic means alone, under the most critical

reversing condition expected in operation.

(b) For propeller reversing systems--

(1) Each system intended for ground operation only must be designed so that

no single failure (or reasonably likely combination of failures) or

malfunction of the system will result in unwanted reverse thrust under any

expected operating condition. Failure of structural elements need not be

considered if this kind of failure is extremely remote.

(2) Compliance with this section may be shown by failure analysis or

testing, or both, for propeller systems that allow propeller blades to move

from the flight low-pitch position to a position that is substantially less

than that at the normal flight low-pitch position. The analysis may include

or be supported by the analysis made to show compliance with the requirements

of Sec. 35.21 of this chapter for the propeller and associated installation

components.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29784, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.934  Turbojet engine thrust reverser system tests.

Thrust reversers installed on turbojet engines must meet the requirements

of Sec. 33.97 of this chapter.

[Amdt. 25-23, 35 FR 5677, Apr. 8, 1970]

Sec. 25.937  Turbopropeller-drag limiting systems.

Turbopropeller power airplane propeller-drag limiting systems must be

designed so that no single failure or malfunction of any of the systems

during normal or emergency operation results in propeller drag in excess of

that for which the airplane was designed under Sec. 25.367. Failure of

structural elements of the drag limiting systems need not be considered if

the probability of this kind of failure is extremely remote.

Sec. 25.939  Turbine engine operating characteristics.

(a) Turbine engine operating characteristics must be investigated in flight

to determine that no adverse characteristics (such as stall, surge, or

flameout) are present, to a hazardous degree, during normal and emergency

operation within the range of operating limitations of the airplane and of

the engine.

(b) [Reserved]

(c) The turbine engine air inlet system may not, as a result of air flow

distortion during normal operation, cause vibration harmful to the engine.

[Amdt. 25-11, 32 FR 6912, May 5, 1967, as amended by Amdt. 25-40, 42 FR

15043, Mar. 17, 1977]

Sec. 25.941  Inlet, engine, and exhaust compatibility.

For airplanes using variable inlet or exhaust system geometry, or both--

(a) The system comprised of the inlet, engine (including thrust

augmentation systems, if incorporated), and exhaust must be shown to function

properly under all operating conditions for which approval is sought,

including all engine rotating speeds and power settings, and engine inlet and

exhaust configurations;

(b) The dynamic effects of the operation of these (including consideration

of probable malfunctions) upon the aerodynamic control of the airplane may

not result in any condition that would require exceptional skill, alertness,

or strength on the part of the pilot to avoid exceeding an operational or

structural limitation of the airplane; and

(c) In showing compliance with paragraph (b) of this section, the pilot

strength required may not exceed the limits set forth in Sec. 25.143(c),

subject to the conditions set forth in paragraphs (d) and (e) of Sec. 25.143.

[Amdt. 25-38, 41 FR 55467, Dec. 20, 1976]

Sec. 25.943  Negative acceleration.

No hazardous malfunction of an engine, an auxiliary power unit approved for

use in flight, or any component or system associated with the powerplant or

auxiliary power unit may occur when the airplane is operated at the negative

accelerations within the flight envelopes prescribed in Sec. 25.333. This

must be shown for the greatest duration expected for the acceleration.

[Amdt. 25-40, 42 FR 15043, Mar. 17, 1977]

Sec. 25.945  Thrust or power augmentation system.

(a) General. Each fluid injection system must provide a flow of fluid at

the rate and pressure established for proper engine functioning under each

intended operating condition. If the fluid can freeze, fluid freezing may not

damage the airplane or adversely affect airplane performance.

(b) Fluid tanks. Each augmentation system fluid tank must meet the

following requirements:

(1) Each tank must be able to withstand without failure the vibration,

inertia, fluid, and structural loads that is may be subject to in operation.

(2) The tanks as mounted in the airplane must be able to withstand without

failure or leakage an internal pressure 1.5 times the maximum operating

pressure.

(3) If a vent is provided, the venting must be effective under all normal

flight conditions.

(4) [Reserved]

(c) Augmentation system drains must be designed and located in accordance

with Sec. 25.1455 if--

(1) The augmentation system fluid is subject to freezing; and

(2) The fluid may be drained in flight or during ground operation.

(d) The augmentation liquid tank capacity available for the use of each

engine must be large enough to allow operation of the airplane under the

approved procedures for the use of liquid-augmented power. The computation of

liquid consumption must be based on the maximum approved rate appropriate for

the desired engine output and must include the effect of temperature on

engine performance as well as any other factors that might vary the amount of

liquid required.

(e) This section does not apply to fuel injection systems.

[Amdt. 25-40, 42 FR 15043, Mar. 17, 1977, as amended by Amdt. 25-72, 55

FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Fuel System

Sec. 25.951  General.

(a) Each fuel system must be constructed and arranged to ensure a flow of

fuel at a rate and pressure established for proper engine and auxiliary power

unit functioning under each likely operating condition, including any

maneuver for which certification is requested and during which the engine or

auxiliary power unit is permitted to be in operation.

(b) Each fuel system must be arranged so that any air which is introduced

into the system will not result in--

(1) Power interruption for more than 20 seconds for reciprocating engines;

or

(2) Flameout for turbine engines.

(c) Each fuel system for a turbine engine must be capable of sustained

operation throughout its flow and pressure range with fuel initially

saturated with water at 80 deg. F and having 0.75cc of free water per gallon

added and cooled to the most critical condition for icing likely to be

encountered in operation.

(d) Each fuel system for a turbine engine powered airplane must meet the

applicable fuel venting requirements of part 34 of this chapter.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5677, Apr. 8, 1970; Amdt. 25-36, 39 FR 35460, Oct. 1, 1974; Amdt. 25-38, 41

FR 55467, Dec. 20, 1976; Amdt. 25-73, 55 FR 32861, Aug. 10, 1990; 55 FR

35139, Aug. 28, 1990]

*****************************************************************************

55 FR 32856, No. 155, Aug. 10, 1990

SUMMARY: This final rule codifies as new part 34 all of the applicable

aircraft engine fuel venting and exhaust emission requirements of Special

Federal Aviation Regulation (SFAR) 27-5, and the test procedures specified

under the regulations implementing the Clean Air Act. This rule consolidates

all of the requirements and test procedures into this part, and inserts into

other affected parts the requirements to comply with new part 34. New part 34

does not alter any of the requirements specified under SFAR 27-5 or the

regulations implementing the Clean Air Act.

EFFECTIVE DATE: September 10, 1990.

*****************************************************************************

Sec. 25.952  Fuel system analysis and test.

(a) Proper fuel system functioning under all probable operating conditions

must be shown by analysis and those tests found necessary by the

Administrator. Tests, if required, must be made using the airplane fuel

system or a test article that reproduces the operating characteristics of the

portion of the fuel system to be tested.

(b) The likely failure of any heat exchanger using fuel as one of its

fluids may not result in a hazardous condition.

[Amdt. 25-40, 42 FR 15043, Mar. 17, 1977]

Sec. 25.953  Fuel system independence.

Each fuel system must meet the requirements of Sec. 25.903(b) by--

(a) Allowing the supply of fuel to each engine through a system independent

of each part of the system supplying fuel to any other engine; or

(b) Any other acceptable method.

Sec. 25.954  Fuel system lightning protection.

The fuel system must be designed and arranged to prevent the ignition of

fuel vapor within the system by--

(a) Direct lightning strikes to areas having a high probability of stroke

attachment;

(b) Swept lightning strokes to areas where swept strokes are highly

probable; and

(c) Corona and streamering at fuel vent outlets.

[Amdt. 25-14, 32 FR 11629, Aug. 11, 1967]

Sec. 25.955  Fuel flow.

(a) Each fuel system must provide at least 100 percent of the fuel flow

required under each intended operating condition and maneuver. Compliance

must be shown as follows:

(1) Fuel must be delivered to each engine at a pressure within the limits

specified in the engine type certificate.

(2) The quantity of fuel in the tank may not exceed the amount established

as the unusable fuel supply for that tank under the requirements of Sec.

25.959 plus that necessary to show compliance with this section.

(3) Each main pump must be used that is necessary for each operating

condition and attitude for which compliance with this section is shown, and

the appropriate emergency pump must be substituted for each main pump so

used.

(4) If there is a fuel flowmeter, it must be blocked and the fuel must flow

through the meter or its bypass.

(b) If an engine can be supplied with fuel from more than one tank, the

fuel system must--

(1) For each reciprocating engine, supply the full fuel pressure to that

engine in not more than 20 seconds after switching to any other fuel tank

containing usable fuel when engine malfunctioning becomes apparent due to the

depletion of the fuel supply in any tank from which the engine can be fed;

and

(2) For each turbine engine, in addition to having appropriate manual

switching capability, be designed to prevent interruption of fuel flow to

that engine, without attention by the flight crew, when any tank supplying

fuel to that engine is depleted of usable fuel during normal operation, and

any other tank, that normally supplies fuel to that engine alone, contains

usable fuel.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-11, 32 FR

6912, May 5, 1967]

Sec. 25.957  Flow between interconnected tanks.

If fuel can be pumped from one tank to another in flight, the fuel tank

vents and the fuel transfer system must be designed so that no structural

damage to the tanks can occur because of overfilling.

Sec. 25.959  Unusable fuel supply.

The unusable fuel quantity for each fuel tank and its fuel system

components must be established at not less than the quantity at which the

first evidence of engine malfunction occurs under the most adverse fuel feed

condition for all intended operations and flight maneuvers involving fuel

feeding from that tank. Fuel system component failures need not be

considered.

[Amdt. 25-23, 35 FR 5677, Apr. 8, 1970, as amended by Amdt. 25-40, 42 FR

15043, Mar. 17, 1977]

Sec. 25.961  Fuel system hot weather operation.

(a) The fuel system must perform satisfactorily in hot weather operation.

This must be shown by showing that the fuel system from the tank outlets to

each engine is pressurized, under all intended operations, so as to prevent

vapor formation, or must be shown by climbing from the altitude of the

airport elected by the applicant to the maximum altitude established as an

operating limitation under Sec. 25.1527. If a climb test is elected, there

may be no evidence of vapor lock or other malfunctioning during the climb

test conducted under the following conditions:

(1) For reciprocating engine powered airplanes, the engines must operate at

maximum continuous power, except that takeoff power must be used for the

altitudes from 1,000 feet below the critical altitude through the critical

altitude. The time interval during which takeoff power is used may not be

less than the takeoff time limitation.

(2) For turbine engine powered airplanes, the engines must operate at

takeoff power for the time interval selected for showing the takeoff flight

path, and at maximum continuous power for the rest of the climb.

(3) The weight of the airplane must be the weight with full fuel tanks,

minimum crew, and the ballast necessary to maintain the center of gravity

within allowable limits.

(4) The climb airspeed may not exceed--

(i) For reciprocating engine powered airplanes, the maximum airspeed

established for climbing from takeoff to the maximum operating altitude with

the airplane in the following configuration:

(A) Landing gear retracted.

(B) Wing flaps in the most favorable position.

(C) Cowl flaps (or other means of controlling the engine cooling supply) in

the position that provides adequate cooling in the hot-day condition.

(D) Engine operating within the maximum continuous power limitations.

(E) Maximum takeoff weight; and

(ii) For turbine engine powered airplanes, the maximum airspeed established

for climbing from takeoff to the maximum operating altitude.

(5) The fuel temperature must be at least 110 deg. F.

(b) The test prescribed in paragraph (a) of this section may be performed

in flight or on the ground under closely simulated flight conditions. If a

flight test is performed in weather cold enough to interfere with the proper

conduct of the test, the fuel tank surfaces, fuel lines, and other fuel

system parts subject to cold air must be insulated to simulate, insofar as

practicable, flight in hot weather.

[Amdt. 25-11, 32 FR 6912, May 5, 1967, as amended by Amdt. 25-57, 49 FR 6848,

Feb. 23, 1984]

Sec. 25.963  Fuel tanks: general.

(a) Each fuel tank must be able to withstand, without failure, the

vibration, inertia, fluid, and structural loads that it may be subjected to

in operation.

(b) Flexible fuel tank liners must be approved or must be shown to be

suitable for the particular application.

(c) Integral fuel tanks must have facilities for interior inspection and

repair.

(d) Fuel tanks within the fuselage contour must be able to resist rupture

and to retain fuel, under the inertia forces prescribed for the emergency

landing conditions in Sec. 25.561. In addition, these tanks must be in a

protected position so that exposure of the tanks to scraping action with the

ground is unlikely.

(e) Fuel tank access covers must comply with the following criteria in

order to avoid loss of hazardous quantities of fuel:

(1) All covers located in an area where experience or analysis indicates a

strike is likely must be shown by analysis or tests to minimize penetration

and deformation by tire fragments, low energy engine debris, or other likely

debris.

(2) All covers must be fire resistant as defined in part 1 of this chapter.

(f) For pressurized fuel tanks, a means with fail-safe features must be

provided to prevent the buildup of an excessive pressure difference between

the inside and the outside of the tank.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-40, 42 FR

15043, Mar. 17, 1977; Amdt. 25-69, 54 FR 40352, Sept. 29, 1989]

Sec. 25.965  Fuel tank tests.

(a) It must be shown by tests that the fuel tanks, as mounted in the

airplane, can withstand, without failure or leakage, the more critical of the

pressures resulting from the conditions specified in paragraphs (a)(1) and

(2) of this section. In addition, it must be shown by either analysis or

tests, that tank surfaces subjected to more critical pressures resulting from

the condition of paragraphs (a)(3) and (4) of this section, are able to

withstand the following pressures:

(1) An internal pressure of 3.5 psi.

(2) 125 percent of the maximum air pressure developed in the tank from ram

effect.

(3) Fluid pressures developed during maximum limit accelerations, and

deflections, of the airplane with a full tank.

(4) Fluid pressures developed during the most adverse combination of

airplane roll and fuel load.

(b) Each metallic tank with large unsupported or unstiffened flat surfaces,

whose failure or deformation could cause fuel leakage, must be able to

withstand the following test, or its equivalent, without leakage or excessive

deformation of the tank walls:

(1) Each complete tank assembly and its supports must be vibration tested

while mounted to simulate the actual installation.

(2) Except as specified in paragraph (b)(4) of this section, the tank

assembly must be vibrated for 25 hours at an amplitude of not less than 1/32

of an inch (unless another amplitude is substantiated) while 2/3  filled with

water or other suitable test fluid.

(3) The test frequency of vibration must be as follows:

(i) If no frequency of vibration resulting from any r.p.m. within the

normal operating range of engine speeds is critical, the test frequency of

vibration must be 2,000 cycles per minute.

(ii) If only one frequency of vibration resulting from any r.p.m. within

the normal operating range of engine speeds is critical, that frequency of

vibration must be the test frequency.

(iii) If more than one frequency of vibration resulting from any r.p.m.

within the normal operating range of engine speeds is critical, the most

critical of these frequencies must be the test frequency.

(4) Under paragraphs (b)(3) (ii) and (iii) of this section, the time of

test must be adjusted to accomplish the same number of vibration cycles that

would be accomplished in 25 hours at the frequency specified in paragraph

(b)(3)(i) of this section.

(5) During the test, the tank assembly must be rocked at the rate of 16 to

20 complete cycles per minute, through an angle of 15 deg. on both sides of

the horizontal (30 deg. total), about the most critical axis, for 25 hours.

If motion about more than one axis is likely to be critical, the tank must be

rocked about each critical axis for 12 1/2  hours.

(c) Except where satisfactory operating experience with a similar tank in a

similar installation is shown, nonmetallic tanks must withstand the test

specified in paragraph (b)(5) of this section, with fuel at a temperature of

110 deg. F. During this test, a representative specimen of the tank must be

installed in a supporting structure simulating the installation in the

airplane.

(d) For pressurized fuel tanks, it must be shown by analysis or tests that

the fuel tanks can withstand the maximum pressure likely to occur on the

ground or in flight.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-11, 32 FR

6913, May 5, 1967; Amdt. 25-40, 42 FR 15043, Mar. 17, 1977]

Sec. 25.967  Fuel tank installations.

(a) Each fuel tank must be supported so that tank loads (resulting from the

weight of the fuel in the tanks) are not concentrated on unsupported tank

surfaces. In addition--

(1) There must be pads, if necessary, to prevent chafing between the tank

and its supports;

(2) Padding must be nonabsorbent or treated to prevent the absorption of

fluids;

(3) If a flexible tank liner is used, it must be supported so that it is

not required to withstand fluid loads; and

(4) Each interior surface of the tank compartment must be smooth and free

of projections that could cause wear of the liner unless--

(i) Provisions are made for protection of the liner at these points; or

(ii) The construction of the liner itself provides that protection.

(b) Spaces adjacent to tank surfaces must be ventilated to avoid fume

accumulation due to minor leakage. If the tank is in a sealed compartment,

ventilation may be limited to drain holes large enough to prevent excessive

pressure resulting from altitude changes.

(c) The location of each tank must meet the requirements of Sec.

25.1185(a).

(d) No engine nacelle skin immediately behind a major air outlet from the

engine compartment may act as the wall of an integral tank.

(e) Each fuel tank must be isolated from personnel compartments by a

fumeproof and fuelproof enclosure.

Sec. 25.969  Fuel tank expansion space.

Each fuel tank must have an expansion space of not less than 2 percent of

the tank capacity. It must be impossible to fill the expansion space

inadvertently with the airplane in the normal ground attitude. For pressure

fueling systems, compliance with this section may be shown with the means

provided to comply with Sec. 25.979(b).

[Amdt. 25-11, 32 FR 6913, May 5, 1967]

Sec. 25.971  Fuel tank sump.

(a) Each fuel tank must have a sump with an effective capacity, in the

normal ground attitude, of not less than the greater of 0.10 percent of the

tank capacity or one-sixteenth of a gallon unless operating limitations are

established to ensure that the accumulation of water in service will not

exceed the sump capacity.

(b) Each fuel tank must allow drainage of any hazardous quantity of water

from any part of the tank to its sump with the airplane in the ground

attitude.

(c) Each fuel tank sump must have an accessible drain that--

(1) Allows complete drainage of the sump on the ground;

(2) Discharges clear of each part of the airplane; and

(3) Has manual or automatic means for positive locking in the closed

position.

Sec. 25.973  Fuel tank filler connection.

Each fuel tank filler connection must prevent the entrance of fuel into any

part of the airplane other than the tank itself. In addition--

(a) [Reserved]

(b) Each recessed filler connection that can retain any appreciable

quantity of fuel must have a drain that discharges clear of each part of the

airplane;

(c) Each filler cap must provide a fuel-tight seal; and

(d) Each fuel filling point, except pressure fueling connection points,

must have a provision for electrically bonding the airplane to ground fueling

equipment.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-40, 42 FR

15043, Mar. 17, 1977; Amdt. 25-72, 55 FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.975  Fuel tank vents and carburetor vapor vents.

(a) Fuel tank vents. Each fuel tank must be vented from the top part of the

expansion space so that venting is effective under any normal flight

condition. In addition--

(1) Each vent must be arranged to avoid stoppage by dirt or ice formation;

(2) The vent arrangement must prevent siphoning of fuel during normal

operation;

(3) The venting capacity and vent pressure levels must maintain acceptable

differences of pressure between the interior and exterior of the tank,

during--

(i) Normal flight operation;

(ii) Maximum rate of ascent and descent; and

(iii) Refueling and defueling (where applicable);

(4) Airspaces of tanks with interconnected outlets must be interconnected;

(5) There may be no point in any vent line where moisture can accumulate

with the airplane in the ground attitude or the level flight attitude, unless

drainage is provided; and

(6) No vent or drainage provision may end at any point--

(i) Where the discharge of fuel from the vent outlet would constitute a

fire hazard; or

(ii) From which fumes could enter personnel compartments.

(b) Carburetor vapor vents. Each carburetor with vapor elimination

connections must have a vent line to lead vapors back to one of the fuel

tanks. In addition--

(1) Each vent system must have means to avoid stoppage by ice; and

(2) If there is more than one fuel tank, and it is necessary to use the

tanks in a definite sequence, each vapor vent return line must lead back to

the fuel tank used for takeoff and landing.

Sec. 25.977  Fuel tank outlet.

(a) There must be a fuel strainer for the fuel tank outlet or for the

booster pump. This strainer must--

(1) For reciprocating engine powered airplanes, have 8 to 16 meshes per

inch; and

(2) For turbine engine powered airplanes, prevent the passage of any object

that could restrict fuel flow or damage any fuel system component.

(b) [Reserved]

(c) The clear area of each fuel tank outlet strainer must be at least five

times the area of the outlet line.

(d) The diameter of each strainer must be at least that of the fuel tank

outlet.

(e) Each finger strainer must be accessible for inspection and cleaning.

[Amdt. 25-11, 32 FR 6913, May 5, 1967, as amended by Amdt. 25-36, 39 FR

35460, Oct. 1, 1974]

Sec. 25.979  Pressure fueling system.

For pressure fueling systems, the following apply:

(a) Each pressure fueling system fuel manifold connection must have means

to prevent the escape of hazardous quantities of fuel from the system if the

fuel entry valve fails.

(b) An automatic shutoff means must be provided to prevent the quantity of

fuel in each tank from exceeding the maximum quantity approved for that tank.

This means must--

(1) Allow checking for proper shutoff operation before each fueling of the

tank; and

(2) Provide indication at each fueling station of failure of the shutoff

means to stop the fuel flow at the maximum quantity approved for that tank.

(c) A means must be provided to prevent damage to the fuel system in the

event of failure of the automatic shutoff means prescribed in paragraph (b)

of this section.

(d) The airplane pressure fueling system (not including fuel tanks and fuel

tank vents) must withstand an ultimate load that is 2.0 times the load

arising from the maximum pressures, including surge, that is likely to occur

during fueling. The maximum surge pressure must be established with any

combination of tank valves being either intentionally or inadvertently

closed.

(e) The airplane defueling system (not including fuel tanks and fuel tank

vents) must withstand an ultimate load that is 2.0 times the load arising

from the maximum permissible defueling pressure (positive or negative) at the

airplane fueling connection.

[Amdt. 25-11, 32 FR 6913, May 5, 1967, as amended by Amdt. 25-38, 41 FR

55467, Dec. 20, 1976; Amdt. 25-72, 55 FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.981  Fuel tank temperature.

(a) The highest temperature allowing a safe margin below the lowest

expected auto ignition temperature of the fuel in the fuel tanks must be

determined.

(b) No temperature at any place inside any fuel tank where fuel ignition is

possible may exceed the temperature determined under paragraph (a) of this

section. This must be shown under all probable operating, failure, and

malfunction conditions of any component whose operation, failure, or

malfunction could increase the temperature inside the tank.

[Amdt. 25-11, 32 FR 6913, May 5, 1967]

Fuel System Components

Sec. 25.991  Fuel pumps.

(a) Main pumps. Each fuel pump required for proper engine operation, or

required to meet the fuel system requirements of this subpart (other than

those in paragraph (b) of this section, is a main pump. For each main pump,

provision must be made to allow the bypass of each positive displacement fuel

pump other than a fuel injection pump (a pump that supplies the proper flow

and pressure for fuel injection when the injection is not accomplished in a

carburetor) approved as part of the engine.

(b) Emergency pumps. There must be emergency pumps or another main pump to

feed each engine immediately after failure of any main pump (other than a

fuel injection pump approved as part of the engine).

Sec. 25.993  Fuel system lines and fittings.

(a) Each fuel line must be installed and supported to prevent excessive

vibration and to withstand loads due to fuel pressure and accelerated flight

conditions.

(b) Each fuel line connected to components of the airplane between which

relative motion could exist must have provisions for flexibility.

(c) Each flexible connection in fuel lines that may be under pressure and

subjected to axial loading must use flexible hose assemblies.

(d) Flexible hose must be approved or must be shown to be suitable for the

particular application.

(e) No flexible hose that might be adversely affected by exposure to high

temperatures may be used where excessive temperatures will exist during

operation or after engine shut-down.

(f) Each fuel line within the fuselage must be designed and installed to

allow a reasonable degree of deformation and stretching without leakage.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-15, 32 FR

13266, Sept. 20, 1967]

Sec. 25.994  Fuel system components.

Fuel system components in an engine nacelle or in the fuselage must be

protected from damage which could result in spillage of enough fuel to

constitute a fire hazard as a result of a wheels-up landing on a paved

runway.

[Amdt. 25-57, 49 FR 6848, Feb. 23, 1984]

Sec. 25.995  Fuel valves.

In addition to the requirements of Sec. 25.1189 for shutoff means, each

fuel valve must--

(a) [Reserved]

(b) Be supported so that no loads resulting from their operation or from

accelerated flight conditions are transmitted to the lines attached to the

valve.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-40, 42 FR

15043, Mar. 17, 1977]

Sec. 25.997  Fuel strainer or filter.

There must be a fuel strainer or filter between the fuel tank outlet and

the inlet of either the fuel metering device or an engine driven positive

displacement pump, whichever is nearer the fuel tank outlet. This fuel

strainer or filter must--

(a) Be accessible for draining and cleaning and must incorporate a screen

or element which is easily removable;

(b) Have a sediment trap and drain except that it need not have a drain if

the strainer or filter is easily removable for drain purposes;

(c) Be mounted so that its weight is not supported by the connecting lines

or by the inlet or outlet connections of the strainer or filter itself,

unless adequate strength margins under all loading conditions are provided in

the lines and connections; and

(d) Have the capacity (with respect to operating limitations established

for the engine) to ensure that engine fuel system functioning is not

impaired, with the fuel contaminated to a degree (with respect to particle

size and density) that is greater than that established for the engine in

Part 33 of this chapter.

[Amdt. No. 25-36, 39 FR 35460, Oct. 1, 1974, as amended by Amdt. 25-57, 49 FR

6848, Feb. 23, 1984]

Sec. 25.999  Fuel system drains.

(a) Drainage of the fuel system must be accomplished by the use of fuel

strainer and fuel tank sump drains.

(b) Each drain required by paragraph (a) of this section must--

(1) Discharge clear of all parts of the airplane;

(2) Have manual or automatic means for positive locking in the closed

position; and

(3) Have a drain valve--

(i) That is readily accessible and which can be easily opened and closed;

and

(ii) That is either located or protected to prevent fuel spillage in the

event of a landing with landing gear retracted.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55467, Dec. 20, 1976]

Sec. 25.1001  Fuel jettisoning system.

(a) A fuel jettisoning system must be installed on each airplane unless it

is shown that the airplane meets the climb requirements of Secs. 25.119 and

25.121(d) at maximum takeoff weight, less the actual or computed weight of

fuel necessary for a 15-minute flight comprised of a takeoff, go-around, and

landing at the airport of departure with the airplane configuration, speed,

power, and thrust the same as that used in meeting the applicable takeoff,

approach, and landing climb performance requirements of this part.

(b) If a fuel jettisoning system is required it must be capable of

jettisoning enough fuel within 15 minutes, starting with the weight given in

paragraph (a) of this section, to enable the airplane to meet the climb

requirements of Secs. 25.119 and 25.121(d), assuming that the fuel is

jettisoned under the conditions, except weight, found least favorable during

the flight tests prescribed in paragraph (c) of this section.

(c) Fuel jettisoning must be demonstrated beginning at maximum takeoff

weight with flaps and landing gear up and in--

(1) A power-off glide at 1.4 Vs1;

(2) A climb at the one-engine inoperative best rate-of-climb speed, with

the critical engine inoperative and the remaining engines at maximum

continuous power; and

(3) Level flight at 1.4 Vs1; if the results of the tests in the conditions

specified in paragraphs (c) (1) and (2) of this section show that this

condition could be critical.

(d) During the flight tests prescribed in paragraph (c) of this section, it

must be shown that--

(1) The fuel jettisoning system and its operation are free from fire

hazard;

(2) The fuel discharges clear of any part of the airplane;

(3) Fuel or fumes do not enter any parts of the airplane; and

(4) The jettisoning operation does not adversely affect the controllability

of the airplane.

(e) For reciprocating engine powered airplanes, means must be provided to

prevent jettisoning the fuel in the tanks used for takeoff and landing below

the level allowing 45 minutes flight at 75 percent maximum continuous power.

However, if there is an auxiliary control independent of the main jettisoning

control, the system may be designed to jettison the remaining fuel by means

of the auxiliary jettisoning control.

(f) For turbine engine powered airplanes, means must be provided to prevent

jettisoning the fuel in the tanks used for takeoff and landing below the

level allowing climb from sea level to 10,000 feet and thereafter allowing 45

minutes cruise at a speed for maximum range. However, if there is an

auxiliary control independent of the main jettisoning control, the system may

be designed to jettison the remaining fuel by means of the auxiliary

jettisoning control.

(g) The fuel jettisoning valve must be designed to allow flight personnel

to close the valve during any part of the jettisoning operation.

(h) Unless it is shown that using any means (including flaps, slots, and

slats) for changing the airflow across or around the wings does not adversely

affect fuel jettisoning, there must be a placard, adjacent to the jettisoning

control, to warn flight crewmembers against jettisoning fuel while the means

that change the airflow are being used.

(i) The fuel jettisoning system must be designed so that any reasonably

probable single malfunction in the system will not result in a hazardous

condition due to unsymmetrical jettisoning of, or inability to jettison,

fuel.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-18, 33 FR

12226, Aug. 30, 1968; Amdt. 25-57, 49 FR 6848, Feb. 23, 1984]

Oil System

Sec. 25.1011  General.

(a) Each engine must have an independent oil system that can supply it with

an appropriate quantity of oil at a temperature not above that safe for

continuous operation.

(b) The usable oil capacity may not be less than the product of the

endurance of the airplane under critical operating conditions and the

approved maximum allowable oil consumption of the engine under the same

conditions, plus a suitable margin to ensure system circulation. Instead of a

rational analysis of airplane range for the purpose of computing oil

requirements for reciprocating engine powered airplanes, the following fuel/

oil ratios may be used:

(1) For airplanes without a reserve oil or oil transfer system, a fuel/oil

ratio of 30:1 by volume.

(2) For airplanes with either a reserve oil or oil transfer system, a fuel/

oil ratio of 40:1 by volume.

(c) Fuel/oil ratios higher than those prescribed in paragraphs (b) (1) and

(2) of this section may be used if substantiated by data on actual engine oil

consumption.

Sec. 25.1013  Oil tanks.

(a) Installation. Each oil tank installation must meet the requirements of

Sec. 25.967.

(b) Expansion space. Oil tank expansion space must be provided as follows:

(1) Each oil tank used with a reciprocating engine must have an expansion

space of not less than the greater of 10 percent of the tank capacity or 0.5

gallon, and each oil tank used with a turbine engine must have an expansion

space of not less than 10 percent of the tank capacity.

(2) Each reserve oil tank not directly connected to any engine may have an

expansion space of not less than two percent of the tank capacity.

(3) It must be impossible to fill the expansion space inadvertently with

the airplane in the normal ground attitude.

(c) Filler connection. Each recessed oil tank filler connection that can

retain any appreciable quantity of oil must have a drain that discharges

clear of each part of the airplane. In addition, each oil tank filler cap

must provide an oil-tight seal.

(d) Vent. Oil tanks must be vented as follows:

(1) Each oil tank must be vented from the top part of the expansion space

so that venting is effective under any normal flight condition.

(2) Oil tank vents must be arranged so that condensed water vapor that

might freeze and obstruct the line cannot accumulate at any point.

(e) Outlet. There must be means to prevent entrance into the tank itself,

or into the tank outlet, of any object that might obstruct the flow of oil

through the system. No oil tank outlet may be enclosed by any screen or guard

that would reduce the flow of oil below a safe value at any operating

temperature. There must be a shutoff valve at the outlet of each oil tank

used with a turbine engine, unless the external portion of the oil system

(including the oil tank supports) is fireproof.

(f) Flexible oil tank liners. Each flexible oil tank liner must be approved

or must be shown to be suitable for the particular application.

[Doc. No. 5066, 29 FR 18291, Dec. 24, as amended by Amdt. 25-19, 33 FR 15410,

Oct. 17, 1968; Amdt. 25-23, 35 FR 5677, Apr. 8, 1970; Amdt. 25-36, 39 FR

35460, Oct. 1, 1974; Amdt. 25-57, 49 FR 6848, Feb. 23, 1984; Amdt. 25-72, 55

FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1015  Oil tank tests.

Each oil tank must be designed and installed so that--

(a) It can withstand, without failure, each vibration, inertia, and fluid

load that it may be subjected to in operation; and

(b) It meets the provisions of Sec. 25.965, except--

(1) The test pressure--

(i) For pressurized tanks used with a turbine engine, may not be less than

5 p.s.i. plus the maximum operating pressure of the tank instead of the

pressure specified in Sec. 25.965(a); and

(ii) For all other tanks may not be less than 5 p.s.i. instead of the

pressure specified in Sec. 25.965(a); and

(2) The test fluid must be oil at 250 deg. F. instead of the fluid

specified in Sec. 25.965(c).

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-36, 39 FR

35461, Oct. 1, 1974]

Sec. 25.1017  Oil lines and fittings.

(a) Each oil line must meet the requirements of Sec. 25.993 and each oil

line and fitting in any designated fire zone must meet the requirements of

Sec. 25.1183.

(b) Breather lines must be arranged so that--

(1) Condensed water vapor that might freeze and obstruct the line cannot

accumulate at any point;

(2) The breather discharge does not constitute a fire hazard if foaming

occurs or causes emitted oil to strike the pilot's windshield; and

(3) The breather does not discharge into the engine air induction system.

Sec. 25.1019  Oil strainer or filter.

(a) Each turbine engine installation must incorporate an oil strainer or

filter through which all of the engine oil flows and which meets the

following requirements:

(1) Each oil strainer or filter that has a bypass must be constructed and

installed so that oil will flow at the normal rate through the rest of the

system with the strainer or filter completely blocked.

(2) The oil strainer or filter must have the capacity (with respect to

operating limitations established for the engine) to ensure that engine oil

system functioning is not impaired when the oil is contaminated to a degree

(with respect to particle size and density) that is greater than that

established for the engine under Part 33 of this chapter.

(3) The oil strainer or filter, unless it is installed at an oil tank

outlet, must incorporate an indicator that will indicate contamination before

it reaches the capacity established in accordance with paragraph (a)(2) of

this section.

(4) The bypass of a strainer or filter must be constructed and installed so

that the release of collected contaminants is minimized by appropriate

location of the bypass to ensure that collected contaminants are not in the

bypass flow path.

(5) An oil strainer or filter that has no bypass, except one that is

installed at an oil tank outlet, must have a means to connect it to the

warning system required in Sec. 25.1305(c)(7).

(b) Each oil strainer or filter in a powerplant installation using

reciprocating engines must be constructed and installed so that oil will flow

at the normal rate through the rest of the system with the strainer or filter

element completely blocked.

[Amdt. 25-36, 39 FR 35461, Oct. 1, 1974, as amended by Amdt. 25-57, 49 FR

6848, Feb. 23, 1984]

Sec. 25.1021  Oil system drains.

A drain (or drains) must be provided to allow safe drainage of the oil

system. Each drain must--

(a) Be accessible; and

(b) Have manual or automatic means for positive locking in the closed

position.

[Amdt. 25-57, 49 FR 6848, Feb. 23, 1984]

Sec. 25.1023  Oil radiators.

(a) Each oil radiator must be able to withstand, without failure, any

vibration, inertia, and oil pressure load to which it would be subjected in

operation.

(b) Each oil radiator air duct must be located so that, in case of fire,

flames coming from normal openings of the engine nacelle cannot impinge

directly upon the radiator.

Sec. 25.1025  Oil valves.

(a) Each oil shutoff must meet the requirements of Sec. 25.1189.

(b) The closing of oil shutoff means may not prevent propeller feathering.

(c) Each oil valve must have positive stops or suitable index provisions in

the "on" and "off" positions and must be supported so that no loads resulting

from its operation or from accelerated flight conditions are transmitted to

the lines attached to the valve.

Sec. 25.1027  Propeller feathering system.

(a) If the propeller feathering system depends on engine oil, there must be

means to trap an amount of oil in the tank if the supply becomes depleted due

to failure of any part of the lubricating system other than the tank itself.

(b) The amount of trapped oil must be enough to accomplish the feathering

operation and must be available only to the feathering pump.

(c) The ability of the system to accomplish feathering with the trapped oil

must be shown. This may be done on the ground using an auxiliary source of

oil for lubricating the engine during operation.

(d) Provision must be made to prevent sludge or other foreign matter from

affecting the safe operation of the propeller feathering system.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55467, Dec. 20, 1976]

Cooling

Sec. 25.1041  General.

The powerplant and auxiliary power unit cooling provisions must be able to

maintain the temperatures of powerplant components, engine fluids, and

auxiliary power unit components and fluids within the temperature limits

established for these components and fluids, under ground, water, and flight

operating conditions, and after normal engine or auxiliary power unit

shutdown, or both.

[Amdt. 25-38, 41 FR 55467, Dec. 20, 1976]

Sec. 25.1043  Cooling tests.

(a) General. Compliance with Sec. 25.1041 must be shown by tests, under

critical ground, water, and flight operating conditions. For these tests, the

following apply:

(1) If the tests are conducted under conditions deviating from the maximum

ambient atmospheric temperature, the recorded powerplant temperatures must be

corrected under paragraphs (c) and (d) of this section.

(2) No corrected temperatures determined under paragraph (a)(1) of this

section may exceed established limits.

(3) For reciprocating engines, the fuel used during the cooling tests must

be the minimum grade approved for the engines, and the mixture settings must

be those normally used in the flight stages for which the cooling tests are

conducted. The test procedures must be as prescribed in Sec. 25.1045.

(b) Maximum ambient atmospheric temperature. A maximum ambient atmospheric

temperature corresponding to sea level conditions of at least 100 degrees F

must be established. The assumed temperature lapse rate is 3.6 degrees F per

thousand feet of altitude above sea level until a temperature of -69.7

degrees F is reached, above which altitude the temperature is considered

constant at -69.7 degrees F. However, for winterization installations, the

applicant may select a maximum ambient atmospheric temperature corresponding

to sea level conditions of less than 100 degrees F.

(c) Correction factor (except cylinder barrels). Unless a more rational

correction applies, temperatures of engine fluids and powerplant components

(except cylinder barrels) for which temperature limits are established, must

be corrected by adding to them the difference between the maximum ambient

atmospheric temperature and the temperature of the ambient air at the time of

the first occurrence of the maximum component or fluid temperature recorded

during the cooling test.

(d) Correction factor for cylinder barrel temperatures. Unless a more

rational correction applies, cylinder barrel temperatures must be corrected

by adding to them 0.7 times the difference between the maximum ambient

atmospheric temperature and the temperature of the ambient air at the time of

the first occurrence of the maximum cylinder barrel temperature recorded

during the cooling test.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-42, 43 FR

2323, Jan. 16, 1978]

Sec. 25.1045  Cooling test procedures.

(a) Compliance with Sec. 25.1041 must be shown for the takeoff, climb, en

route, and landing stages of flight that correspond to the applicable

performance requirements. The cooling tests must be conducted with the

airplane in the configuration, and operating under the conditions, that are

critical relative to cooling during each stage of flight. For the cooling

tests, a temperature is "stabilized" when its rate of change is less than two

degrees F. per minute.

(b) Temperatures must be stabilized under the conditions from which entry

is made into each stage of flight being investigated, unless the entry

condition normally is not one during which component and the engine fluid

temperatures would stabilize (in which case, operation through the full entry

condition must be conducted before entry into the stage of flight being

investigated in order to allow temperatures to reach their natural levels at

the time of entry). The takeoff cooling test must be preceded by a period

during which the powerplant component and engine fluid temperatures are

stabilized with the engines at ground idle.

(c) Cooling tests for each stage of flight must be continued until--

(1) The component and engine fluid temperatures stabilize;

(2) The stage of flight is completed; or

(3) An operating limitation is reached.

(d) For reciprocating engine powered airplanes, it may be assumed, for

cooling test purposes, that the takeoff stage of flight is complete when the

airplane reaches an altitude of 1,500 feet above the takeoff surface or

reaches a point in the takeoff where the transition from the takeoff to the

en route configuration is completed and a speed is reached at which

compliance with Sec. 25.121(c) is shown, whichever point is at a higher

altitude. The airplane must be in the following configuration:

(1) Landing gear retracted.

(2) Wing flaps in the most favorable position.

(3) Cowl flaps (or other means of controlling the engine cooling supply) in

the position that provides adequate cooling in the hot-day condition.

(4) Critical engine inoperative and its propeller stopped.

(5) Remaining engines at the maximum continuous power available for the

altitude.

(e) For hull seaplanes and amphibians, cooling must be shown during taxiing

downwind for 10 minutes, at five knots above step speed.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-57, 49 FR

6848, Feb. 23, 1984]

Induction System

Sec. 25.1091  Air induction.

(a) The air induction system for each engine and auxiliary power unit must

supply--

(1) The air required by that engine and auxiliary power unit under each

operating condition for which certification is requested; and

(2) The air for proper fuel metering and mixture distribution with the

induction system valves in any position.

(b) Each reciprocating engine must have an alternate air source that

prevents the entry of rain, ice, or any other foreign matter.

(c) Air intakes may not open within the cowling, unless--

(1) That part of the cowling is isolated from the engine accessory section

by means of a fireproof diaphragm; or

(2) For reciprocating engines, there are means to prevent the emergence of

backfire flames.

(d) For turbine engine powered airplanes and airplanes incorporating

auxiliary power units--

(1) There must be means to prevent hazardous quantities of fuel leakage or

overflow from drains, vents, or other components of flammable fluid systems

from entering the engine or auxiliary power unit intake system; and

(2) The airplane must be designed to prevent water or slush on the runway,

taxiway, or other airport operating surfaces from being directed into the

engine or auxiliary power unit air inlet ducts in hazardous quantities, and

the air inlet ducts must be located or protected so as to minimize the

ingestion of foreign matter during takeoff, landing, and taxiing.

(e) If the engine induction system contains parts or components that could

be damaged by foreign objects entering the air inlet, it must be shown by

tests or, if appropriate, by analysis that the induction system design can

withstand the foreign object ingestion test conditions of Sec. 33.77 of this

chapter without failure of parts or components that could create a hazard.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55467, Dec. 20, 1976; Amdt. 25-40, 42 FR 15043, Mar. 17, 1977; Amdt. 25-57,

49 FR 6849, Feb. 23, 1984]

Sec. 25.1093  Induction system icing protection.

(a) Reciprocating engines. Each reciprocating engine air induction system

must have means to prevent and eliminate icing. Unless this is done by other

means, it must be shown that, in air free of visible moisture at a

temperature of 30 deg. F., each airplane with altitude engines using--

(1) Conventional venturi carburetors have a preheater that can provide a

heat rise of 120 deg. F. with the engine at 60 percent of maximum continuous

power; or

(2) Carburetors tending to reduce the probability of ice formation has a

preheater that can provide a heat rise of 100 deg. F. with the engine at 60

percent of maximum continuous power.

(b) Turbine engines. (1) Each turbine engine must operate throughout the

flight power range of the engine (including idling), without the accumulation

of ice on the engine, inlet system components, or airframe components that

would adversely affect engine operation or cause a serious loss of power or

thrust--

(i) Under the icing conditions specified in appendix C, and

(ii) In falling and blowing snow within the limitations established for the

airplane for such operation.

(2) Each turbine engine must idle for 30 minutes on the ground, with the

air bleed available for engine icing protection at its critical condition,

without adverse effect, in an atmosphere that is at a temperature between 15

deg. and 30 deg. F (between -9 deg. and -1 deg. C) and has a liquid water

content not less than 0.3 grams per cubic meter in the form of drops having a

mean effective diameter not less than 20 microns, followed by momentary

operation at takeoff power or thrust. During the 30 minutes of idle

operation, the engine may be run up periodically to a moderate power or

thrust setting in a manner acceptable to the Administrator.

(c) Supercharged reciprocating engines. For each engine having a

supercharger to pressurize the air before it enters the carburetor, the heat

rise in the air caused by that supercharging at any altitude may be utilized

in determining compliance with paragraph (a) of this section if the heat rise

utilized is that which will be available, automatically, for the applicable

altitude and operating condition because of supercharging.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55467, Dec. 20, 1976; Amdt. 25-40, 42 FR 15043, Mar. 17, 1977; Amdt. 25-57,

49 FR 6849, Feb. 23, 1984; Amdt. 25-72, 55 FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1101  Carburetor air preheater design.

Each carburetor air preheater must be designed and constructed to--

(a) Ensure ventilation of the preheater when the engine is operated in cold

air;

(b) Allow inspection of the exhaust manifold parts that it surrounds; and

(c) Allow inspection of critical parts of the preheater itself.

Sec. 25.1103  Induction system ducts and air duct systems.

(a) Each induction system duct upstream of the first stage of the engine

supercharger and of the auxiliary power unit compressor must have a drain to

prevent the hazardous accumulation of fuel and moisture in the ground

attitude. No drain may discharge where it might cause a fire hazard.

(b) Each induction system duct must be--

(1) Strong enough to prevent induction system failures resulting from

normal backfire conditions; and

(2) Fire-resistant if it is in any fire zone for which a fire-extinguishing

system is required, except that ducts for auxiliary power units must be

fireproof within the auxiliary power unit fire zone.

(c) Each duct connected to components between which relative motion could

exist must have means for flexibility.

(d) For turbine engine and auxiliary power unit bleed air duct systems, no

hazard may result if a duct failure occurs at any point between the air duct

source and the airplane unit served by the air.

(e) Each auxiliary power unit induction system duct must be fireproof for a

sufficient distance upstream of the auxiliary power unit compartment to

prevent hot gas reverse flow from burning through auxiliary power unit ducts

and entering any other compartment or area of the airplane in which a hazard

would be created resulting from the entry of hot gases. The materials used to

form the remainder of the induction system duct and plenum chamber of the

auxiliary power unit must be capable of resisting the maximum heat conditions

likely to occur.

(f) Each auxiliary power unit induction system duct must be constructed of

materials that will not absorb or trap hazardous quantities of flammable

fluids that could be ignited in the event of a surge or reverse flow

condition.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-46, 43 FR

50597, Oct. 30, 1978]

Sec. 25.1105  Induction system screens.

If induction system screens are used--

(a) Each screen must be upstream of the carburetor;

(b) No screen may be in any part of the induction system that is the only

passage through which air can reach the engine, unless it can be deiced by

heated air;

(c) No screen may be deiced by alcohol alone; and

(d) It must be impossible for fuel to strike any screen.

Sec. 25.1107  Inter-coolers and after-coolers.

Each inter-cooler and after-cooler must be able to withstand any vibration,

inertia, and air pressure load to which it would be subjected in operation.

Exhaust System

Sec. 25.1121  General.

For powerplant and auxiliary power unit installations the following apply:

(a) Each exhaust system must ensure safe disposal of exhaust gases without

fire hazard or carbon monoxide contamination in any personnel compartment.

For test purposes, any acceptable carbon monoxide detection method may be

used to show the absence of carbon monoxide.

(b) Each exhaust system part with a surface hot enough to ignite flammable

fluids or vapors must be located or shielded so that leakage from any system

carrying flammable fluids or vapors will not result in a fire caused by

impingement of the fluids or vapors on any part of the exhaust system

including shields for the exhaust system.

(c) Each component that hot exhaust gases could strike, or that could be

subjected to high temperatures from exhaust system parts, must be fireproof.

All exhaust system components must be separated by fireproof shields from

adjacent parts of the airplane that are outside the engine and auxiliary

power unit compartments.

(d) No exhaust gases may discharge so as to cause a fire hazard with

respect to any flammable fluid vent or drain.

(e) No exhaust gases may discharge where they will cause a glare seriously

affecting pilot vision at night.

(f) Each exhaust system component must be ventilated to prevent points of

excessively high temperature.

(g) Each exhaust shroud must be ventilated or insulated to avoid, during

normal operation, a temperature high enough to ignite any flammable fluids or

vapors external to the shroud.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-40, 42 FR

15043, Mar. 17, 1977]

Sec. 25.1123  Exhaust piping.

For powerplant and auxiliary power unit installations, the following apply:

(a) Exhaust piping must be heat and corrosion resistant, and must have

provisions to prevent failure due to expansion by operating temperatures.

(b) Piping must be supported to withstand any vibration and inertia loads

to which it would be subjected in operation; and

(c) Piping connected to components between which relative motion could

exist must have means for flexibility.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-40, 42 FR

15044, Mar. 17, 1977]

Sec. 25.1125  Exhaust heat exchangers.

For reciprocating engine powered airplanes, the following apply:

(a) Each exhaust heat exchanger must be constructed and installed to

withstand each vibration, inertia, and other load to which it would be

subjected in operation. In addition--

(1) Each exchanger must be suitable for continued operation at high

temperatures and resistant to corrosion from exhaust gases;

(2) There must be means for the inspection of the critical parts of each

exchanger;

(3) Each exchanger must have cooling provisions wherever it is subject to

contact with exhaust gases; and

(4) No exhaust heat exchanger or muff may have any stagnant areas or liquid

traps that would increase the probability of ignition of flammable fluids or

vapors that might be present in case of the failure or malfunction of

components carrying flammable fluids.

(b) If an exhaust heat exchanger is used for heating ventilating air--

(1) There must be a secondary heat exchanger between the primary exhaust

gas heat exchanger and the ventilating air system; or

(2) Other means must be used to preclude the harmful contamination of the

ventilating air.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55467, Dec. 20, 1976]

Sec. 25.1127  Exhaust driven turbo-superchargers.

(a) Each exhaust driven turbo-supercharger must be approved or shown to be

suitable for the particular application. It must be installed and supported

to ensure safe operation between normal inspections and overhauls. In

addition, there must be provisions for expansion and flexibility between

exhaust conduits and the turbine.

(b) There must be provisions for lubricating the turbine and for cooling

turbine parts where temperatures are critical.

(c) If the normal turbo-supercharger control system malfunctions, the

turbine speed may not exceed its maximum allowable value. Except for the

waste gate operating components, the components provided for meeting this

requirement must be independent of the normal turbo-supercharger controls.

Powerplant Controls and Accessories

Sec. 25.1141  Powerplant controls: general.

Each powerplant control must be located, arranged, and designed under Secs.

25.777 through 25.781 and marked under Sec. 25.1555. In addition, it must

meet the following requirements:

(a) Each control must be located so that it cannot be inadvertently

operated by persons entering, leaving, or moving normally in, the cockpit.

(b) Each flexible control must be approved or must be shown to be suitable

for the particular application.

(c) Each control must have sufficient strength and rigidity to withstand

operating loads without failure and without excessive deflection.

(d) Each control must be able to maintain any set position without constant

attention by flight crewmembers and without creep due to control loads or

vibration.

(e) The portion of each powerplant control located in a designated fire

zone that is required to be operated in the event of fire must be at least

fire resistant.

(f) Powerplant valve controls located in the cockpit must have--

(1) For manual valves, positive stops or in the case of fuel valves

suitable index provisions, in the open and closed position; and

(2) For power-assisted valves, a means to indicate to the flight crew when

the valve--

(i) Is in the fully open or fully closed position; or

(ii) Is moving between the fully open and fully closed position.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-40, 42 FR

15044, Mar. 17, 1977; Amdt. 25-72, 55 FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1142  Auxiliary power unit controls.

Means must be provided on the flight deck for starting, stopping, and

emergency shutdown of each installed auxiliary power unit.

[Amdt. 25-46, 43 FR 50598, Oct. 30, 1978]

Sec. 25.1143  Engine controls.

(a) There must be a separate power or thrust control for each engine.

(b) Power and thrust controls must be arranged to allow--

(1) Separate control of each engine; and

(2) Simultaneous control of all engines.

(c) Each power and thrust control must provide a positive and immediately

responsive means of controlling its engine.

(d) For each fluid injection (other than fuel) system and its controls not

provided and approved as part of the engine, the applicant must show that the

flow of the injection fluid is adequately controlled.

(e) If a power or thrust control incorporates a fuel shutoff feature, the

control must have a means to prevent the inadvertent movement of the control

into the shutoff position. The means must--

(1) Have a positive lock or stop at the idle position; and

(2) Require a separate and distinct operation to place the control in the

shutoff position.

[Amdt. 25-23, 35 FR 5677, Apr. 8, 1970, as amended by Amdt. 25-38, 41 FR

55467, Dec. 20, 1976; Amdt. 25-57, 49 FR 6849, Feb. 23, 1984]

Sec. 25.1145  Ignition switches.

(a) Ignition switches must control each engine ignition circuit on each

engine.

(b) There must be means to quickly shut off all ignition by the grouping of

switches or by a master ignition control.

(c) Each group of ignition switches, except ignition switches for turbine

engines for which continuous ignition is not required, and each master

ignition control must have a means to prevent its inadvertent operation.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-40, 42 FR

15044 Mar. 17, 1977]

Sec. 25.1147  Mixture controls.

(a) If there are mixture controls, each engine must have a separate

control. The controls must be grouped and arranged to allow--

(1) Separate control of each engine; and

(2) Simultaneous control of all engines.

(b) Each intermediate position of the mixture controls that corresponds to

a normal operating setting must be identifiable by feel and sight.

(c) The mixture controls must be accessible to both pilots. However, if

there is a separate flight engineer station with a control panel, the

controls need be accessible only to the flight engineer.

Sec. 25.1149  Propeller speed and pitch controls.

(a) There must be a separate propeller speed and pitch control for each

propeller.

(b) The controls must be grouped and arranged to allow--

(1) Separate control of each propeller; and

(2) Simultaneous control of all propellers.

(c) The controls must allow synchronization of all propellers.

(d) The propeller speed and pitch controls must be to the right of, and at

least one inch below, the pilot's throttle controls.

Sec. 25.1153  Propeller feathering controls.

(a) There must be a separate propeller feathering control for each

propeller. The control must have means to prevent its inadvertent operation.

(b) If feathering is accomplished by movement of the propeller pitch or

speed control lever, there must be means to prevent the inadvertent movement

of this lever to the feathering position during normal operation.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-11, 32 FR

6913, May 5, 1967]

Sec. 25.1155  Reverse thrust and propeller pitch settings below the flight

regime.

Each control for reverse thrust and for propeller pitch settings below the

flight regime must have means to prevent its inadvertent operation. The means

must have a positive lock or stop at the flight idle position and must

require a separate and distinct operation by the crew to displace the control

from the flight regime (forward thrust regime for turbojet powered

airplanes).

[Amdt. 25-11, 32 FR 6913, May 5, 1967]

Sec. 25.1157  Carburetor air temperature controls.

There must be a separate carburetor air temperature control for each

engine.

Sec. 25.1159  Supercharger controls.

Each supercharger control must be accessible to the pilots or, if there is

a separate flight engineer station with a control panel, to the flight

engineer.

Sec. 25.1161  Fuel jettisoning system controls.

Each fuel jettisoning system control must have guards to prevent

inadvertent operation. No control may be near any fire extinguisher control

or other control used to combat fire.

Sec. 25.1163  Powerplant accessories.

(a) Each engine mounted accessory must--

(1) Be approved for mounting on the engine involved;

(2) Use the provisions on the engine for mounting; and

(3) Be sealed to prevent contamination of the engine oil system and the

accessory system.

(b) Electrical equipment subject to arcing or sparking must be installed to

minimize the probability of contact with any flammable fluids or vapors that

might be present in a free state.

(c) If continued rotation of an engine-driven cabin supercharger or of any

remote accessory driven by the engine is hazardous if malfunctioning occurs,

there must be means to prevent rotation without interfering with the

continued operation of the engine.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-57, 49 FR

6849, Feb. 23, 1984]

Sec. 25.1165  Engine ignition systems.

(a) Each battery ignition system must be supplemented by a generator that

is automatically available as an alternate source of electrical energy to

allow continued engine operation if any battery becomes depleted.

(b) The capacity of batteries and generators must be large enough to meet

the simultaneous demands of the engine ignition system and the greatest

demands of any electrical system components that draw electrical energy from

the same source.

(c) The design of the engine ignition system must account for--

(1) The condition of an inoperative generator;

(2) The condition of a completely depleted battery with the generator

running at its normal operating speed; and

(3) The condition of a completely depleted battery with the generator

operating at idling speed, if there is only one battery.

(d) Magneto ground wiring (for separate ignition circuits) that lies on the

engine side of the fire wall, must be installed, located, or protected, to

minimize the probability of simultaneous failure of two or more wires as a

result of mechanical damage, electrical faults, or other cause.

(e) No ground wire for any engine may be routed through a fire zone of

another engine unless each part of that wire within that zone is fireproof.

(f) Each ignition system must be independent of any electrical circuit, not

used for assisting, controlling, or analyzing the operation of that system.

(g) There must be means to warn appropriate flight crewmembers if the

malfunctioning of any part of the electrical system is causing the continuous

discharge of any battery necessary for engine ignition.

(h) Each engine ignition system of a turbine powered airplane must be

considered an essential electrical load.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5677, Apr. 8, 1970; Amdt. 25-72, 55 FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1167  Accessory gearboxes.

For airplanes equipped with an accessory gearbox that is not certificated

as part of an engine--

(a) The engine with gearbox and connecting transmissions and shafts

attached must be subjected to the tests specified in Sec. 33.49 or Sec. 33.87

of this chapter, as applicable;

(b) The accessory gearbox must meet the requirements of Secs. 33.25 and

33.53 or 33.91 of this chapter, as applicable; and

(c) Possible misalignments and torsional loadings of the gearbox,

transmission, and shaft system, expected to result under normal operating

conditions must be evaluated.

[Amdt. 25-38, 41 FR 55467, Dec. 20, 1976]

Powerplant Fire Protection

Sec. 25.1181  Designated fire zones; regions included.

(a) Designated fire zones are--

(1) The engine power section;

(2) The engine accessory section;

(3) Except for reciprocating engines, any complete powerplant compartment

in which no isolation is provided between the engine power section and the

engine accessory section;

(4) Any auxiliary power unit compartment;

(5) Any fuel-burning heater and other combustion equipment installation

described in Sec. 25.859;

(6) The compressor and accessory sections of turbine engines; and

(7) Combustor, turbine, and tailpipe sections of turbine engine

installations that contain lines or components carrying flammable fluids or

gases.

(b) Each designated fire zone must meet the requirements of Secs. 25.867,

and 25.1185 through 25.1203.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-11, 32 FR

6913, May 5, 1967; Amdt. 25-23, 35 FR 5677, Apr. 8, 1970; Amdt. 25-72, 55 FR

29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1182  Nacelle areas behind firewalls, and engine pod attaching

structures containing flammable fluid lines.

(a) Each nacelle area immediately behind the firewall, and each portion of

any engine pod attaching structure containing flammable fluid lines, must

meet each requirement of Secs. 25.1103(b), 25.1165 (d) and (e), 25.1183,

25.1185(c), 25.1187, 25.1189, and 25.1195 through 25.1203, including those

concerning designated fire zones. However, engine pod attaching structures

need not contain fire detection or extinguishing means.

(b) For each area covered by paragraph (a) of this section that contains a

retractable landing gear, compliance with that paragraph need only be shown

with the landing gear retracted.

[Amdt. 25-11, 32 FR 6913, May 5, 1967]

Sec. 25.1183  Flammable fluid-carrying components.

(a) Except as provided in paragraph (b) of this section, each line,

fitting, and other component carrying flammable fluid in any area subject to

engine fire conditions, and each component which conveys or contains

flammable fluid in a designated fire zone must be fire resistant, except that

flammable fluid tanks and supports in a designated fire zone must be

fireproof or be enclosed by a fireproof shield unless damage by fire to any

non-fireproof part will not cause leakage or spillage of flammable fluid.

Components must be shielded or located to safeguard against the ignition of

leaking flammable fluid. An integral oil sump of less than 25-quart capacity

on a reciprocating engine need not be fireproof nor be enclosed by a

fireproof shield.

(b) Paragraph (a) of this section does not apply to--

(1) Lines, fittings, and components which are already approved as part of a

type certificated engine; and

(2) Vent and drain lines, and their fittings, whose failure will not result

in, or add to, a fire hazard.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-11, 32 FR

6913, May 5, 1967; Amdt. 25-36, 39 FR 35461, Oct. 1, 1974; Amdt. 25-57, 49 FR

6849, Feb. 23, 1984]

Sec. 25.1185  Flammable fluids.

(a) Except for the integral oil sumps specified in Sec. 25.1013 (a), no

tank or reservoir that is a part of a system containing flammable fluids or

gases may be in a designated fire zone unless the fluid contained, the design

of the system, the materials used in the tank, the shut-off means, and all

connections, lines, and control provide a degree of safety equal to that

which would exist if the tank or reservoir were outside such a zone.

(b) There must be at least one-half inch of clear airspace between each

tank or reservoir and each firewall or shroud isolating a designated fire

zone.

(c) Absorbent materials close to flammable fluid system components that

might leak must be covered or treated to prevent the absorption of hazardous

quantities of fluids.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964 as amended by Amdt. 25-19, 33 FR

15410, Oct. 17, 1968]

Sec. 25.1187  Drainage and ventilation of fire zones.

(a) There must be complete drainage of each part of each designated fire

zone to minimize the hazards resulting from failure or malfunctioning of any

component containing flammable fluids. The drainage means must be--

(1) Effective under conditions expected to prevail when drainage is needed;

and

(2) Arranged so that no discharged fluid will cause an additional fire

hazard.

(b) Each designated fire zone must be ventilated to prevent the

accumulation of flammable vapors.

(c) No ventilation opening may be where it would allow the entry of

flammable fluids, vapors, or flame from other zones.

(d) Each ventilation means must be arranged so that no discharged vapors

will cause an additional fire hazard.

(e) Unless the extinguishing agent capacity and rate of discharge are based

on maximum air flow through a zone, there must be means to allow the crew to

shut off sources of forced ventilation to any fire zone except the engine

power section of the nacelle and the combustion heater ventilating air ducts.

Sec. 25.1189  Shutoff means.

(a) Each engine installation and each fire zone specified in Sec.

25.1181(a) (4) and (5) must have a means to shut off or otherwise prevent

hazardous quantities of fuel, oil, deicer, and other flammable fluids, from

flowing into, within, or through any designated fire zone, except that

shutoff means are not required for--

(1) Lines, fittings, and components forming an integral part of an engine;

and

(2) Oil systems for turbine engine installations in which all components of

the system in a designated fire zone, including oil tanks, are fireproof or

located in areas not subject to engine fire conditions.

(b) The closing of any fuel shutoff valve for any engine may not make fuel

unavailable to the remaining engines.

(c) Operation of any shutoff may not interfere with the later emergency

operation of other equipment, such as the means for feathering the propeller.

(d) Each flammable fluid shutoff means and control must be fireproof or

must be located and protected so that any fire in a fire zone will not affect

its operation.

(e) No hazardous quantity of flammable fluid may drain into any designated

fire zone after shutoff.

(f) There must be means to guard against inadvertent operation of the

shutoff means and to make it possible for the crew to reopen the shutoff

means in flight after it has been closed.

(g) Each tank-to-engine shutoff valve must be located so that the operation

of the valve will not be affected by powerplant or engine mount structural

failure.

(h) Each shutoff valve must have a means to relieve excessive pressure

accumulation unless a means for pressure relief is otherwise provided in the

system.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5677, Apr. 8, 1970; Amdt. 25-57, 49 FR 6849, Feb. 23, 1984]

Sec. 25.1191  Firewalls.

(a) Each engine, auxiliary power unit, fuel-burning heater, other

combustion equipment intended for operation in flight, and the combustion,

turbine, and tailpipe sections of turbine engines, must be isolated from the

rest of the airplane by firewalls, shrouds, or equivalent means.

(b) Each firewall and shroud must be--

(1) Fireproof;

(2) Constructed so that no hazardous quantity of air, fluid, or flame can

pass from the compartment to other parts of the airplane;

(3) Constructed so that each opening is sealed with close fitting fireproof

grommets, bushings, or firewall fittings; and

(4) Protected against corrosion.

Sec. 25.1192  Engine accessory section diaphragm.

For reciprocating engines, the engine power section and all portions of the

exhaust system must be isolated from the engine accessory compartment by a

diaphragm that complies with the firewall requirements of Sec. 25.1191.

[Amdt. 25-23, 35 FR 5678, Apr. 8, 1970]

Sec. 25.1193  Cowling and nacelle skin.

(a) Each cowling must be constructed and supported so that it can resist

any vibration, inertia, and air load to which it may be subjected in

operation.

(b) Cowling must meet the drainage and ventilation requirements of Sec.

25.1187.

(c) On airplanes with a diaphragm isolating the engine power section from

the engine accessory section, each part of the accessory section cowling

subject to flame in case of fire in the engine power section of the

powerplant must--

(1) Be fireproof; and

(2) Meet the requirements of Sec. 25.1191.

(d) Each part of the cowling subject to high temperatures due to its

nearness to exhaust system parts or exhaust gas impingement must be

fireproof.

(e) Each airplane must--

(1) Be designed and constructed so that no fire originating in any fire

zone can enter, either through openings or by burning through external skin,

any other zone or region where it would create additional hazards;

(2) Meet paragraph (e)(1) of this section with the landing gear retracted

(if applicable); and

(3) Have fireproof skin in areas subject to flame if a fire starts in the

engine power or accessory sections.

Sec. 25.1195  Fire extinguishing systems.

(a) Except for combustor, turbine, and tail pipe sections of turbine engine

installations that contain lines or components carrying flammable fluids or

gases for which it is shown that a fire originating in these sections can be

controlled, there must be a fire extinguisher system serving each designated

fire zone.

(b) The fire extinguishing system, the quantity of the extinguishing agent,

the rate of discharge, and the discharge distribution must be adequate to

extinguish fires. It must be shown by either actual or simulated flights

tests that under critical airflow conditions in flight the discharge of the

extinguishing agent in each designated fire zone specified in paragraph (a)

of this section will provide an agent concentration capable of extinguishing

fires in that zone and of minimizing the probability of reignition. An

individual "one-shot" system may be used for auxiliary power units, fuel

burning heaters, and other combustion equipment. For each other designated

fire zone, two discharges must be provided each of which produces adequate

agent concentration.

(c) The fire extinguishing system for a nacelle must be able to

simultaneously protect each zone of the nacelle for which protection is

provided.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-46, 43 FR

50598, Oct. 30, 1978]

Sec. 25.1197  Fire extinguishing agents.

(a) Fire extinguishing agents must--

(1) Be capable of extinguishing flames emanating from any burning of fluids

or other combustible materials in the area protected by the fire

extinguishing system; and

(2) Have thermal stability over the temperature range likely to be

experienced in the compartment in which they are stored.

(b) If any toxic extinguishing agent is used, provisions must be made to

prevent harmful concentrations of fluid or fluid vapors (from leakage during

normal operation of the airplane or as a result of discharging the fire

extinguisher on the ground or in flight) from entering any personnel

compartment, even though a defect may exist in the extinguishing system. This

must be shown by test except for built-in carbon dioxide fuselage compartment

fire extinguishing systems for which--

(1) Five pounds or less of carbon dioxide will be discharged, under

established fire control procedures, into any fuselage compartment; or

(2) There is protective breathing equipment for each flight crewmember on

flight deck duty.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55467, Dec. 20, 1976; Amdt. 25-40, 42 FR 15044, Mar. 17, 1977]

Sec. 25.1199  Extinguishing agent containers.

(a) Each extinguishing agent container must have a pressure relief to

prevent bursting of the container by excessive internal pressures.

(b) The discharge end of each discharge line from a pressure relief

connection must be located so that discharge of the fire extinguishing agent

would not damage the airplane. The line must also be located or protected to

prevent clogging caused by ice or other foreign matter.

(c) There must be a means for each fire extinguishing agent container to

indicate that the container has discharged or that the charging pressure is

below the established minimum necessary for proper functioning.

(d) The temperature of each container must be maintained, under intended

operating conditions, to prevent the pressure in the container from--

(1) Falling below that necessary to provide an adequate rate of discharge;

or

(2) Rising high enough to cause premature discharge.

(e) If a pyrotechnic capsule is used to discharge the extinguishing agent,

each container must be installed so that temperature conditions will not

cause hazardous deterioration of the pyrotechnic capsule.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5678, Apr. 8, 1970; Amdt. 25-40, 42 FR 15044, Mar. 17, 1977]

Sec. 25.1201  Fire extinguishing system materials.

(a) No material in any fire extinguishing system may react chemically with

any extinguishing agent so as to create a hazard.

(b) Each system component in an engine compartment must be fireproof.

Sec. 25.1203  Fire detector system.

(a) There must be approved, quick acting fire or overheat detectors in each

designated fire zone, and in the combustion, turbine, and tailpipe sections

of turbine engine installations, in numbers and locations ensuring prompt

detection of fire in those zones.

(b) Each fire detector system must be constructed and installed so that--

(1) It will withstand the vibration, inertia, and other loads to which it

may be subjected in operation;

(2) There is a means to warn the crew in the event that the sensor or

associated wiring within a designated fire zone is severed at one point,

unless the system continues to function as a satisfactory detection system

after the severing; and

(3) There is a means to warn the crew in the event of a short circuit in

the sensor or associated wiring within a designated fire zone, unless the

system continues to function as a satisfactory detection system after the

short circuit.

(c) No fire or overheat detector may be affected by any oil, water, other

fluids or fumes that might be present.

(d) There must be means to allow the crew to check, in flight, the

functioning of each fire or overheat detector electric circuit.

(e) Wiring and other components of each fire or overheat detector system in

a fire zone must be at least fire-resistant.

(f) No fire or overheat detector system component for any fire zone may

pass through another fire zone, unless--

(1) It is protected against the possibility of false warnings resulting

from fires in zones through which it passes; or

(2) Each zone involved is simultaneously protected by the same detector and

extinguishing system.

(g) Each fire detector system must be constructed so that when it is in the

configuration for installation it will not exceed the alarm activation time

approved for the detectors using the response time criteria specified in the

appropriate Technical Standard Order for the detector.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5678, Apr. 8, 1970; Amdt. 25-26, 36 FR 5493, Mar. 24, 1971]

Sec. 25.1207  Compliance.

Unless otherwise specified, compliance with the requirements of Secs.

25.1181 through 25.1203 must be shown by a full scale fire test or by one or

more of the following methods:

(a) Tests of similar powerplant configurations;

(b) Tests of components;

(c) Service experience of aircraft with similar powerplant configurations;

(d) Analysis.

[Amdt. 25-46, 43 FR 50598, Oct. 30, 1978]

Subpart F--Equipment

General

Sec. 25.1301  Function and installation.

Each item of installed equipment must--

(a) Be of a kind and design appropriate to its intended function;

(b) Be labeled as to its identification, function, or operating

limitations, or any applicable combination of these factors;

(c) Be installed according to limitations specified for that equipment; and

(d) Function properly when installed.

Sec. 25.1303  Flight and navigation instruments.

(a) The following flight and navigation instruments must be installed so

that the instrument is visible from each pilot station:

(1) A free air temperature indicator or an air-temperature indicator which

provides indications that are convertible to free-air temperature.

(2) A clock displaying hours, minutes, and seconds with a sweep-second

pointer or digital presentation.

(3) A direction indicator (nonstabilized magnetic compass).

(b) The following flight and navigation instruments must be installed at

each pilot station:

(1) An airspeed indicator. If airspeed limitations vary with altitude, the

indicator must have a maximum allowable airspeed indicator showing the

variation of VMO with altitude.

(2) An altimeter (sensitive).

(3) A rate-of-climb indicator (vertical speed).

(4) A gyroscopic rate-of-turn indicator combined with an integral slip-skid

indicator (turn-and-bank indicator) except that only a slip-skid indicator is

required on large airplanes with a third attitude instrument system useable

through flight attitudes of 360 deg. of pitch and roll and installed in

accordance with Sec. 121.305(j) of this title.

(5) A bank and pitch indicator (gyroscopically stabilized).

(6) A direction indicator (gyroscopically stabilized, magnetic or

nonmagnetic).

(c) The following flight and navigation instruments are required as

prescribed in this paragraph:

(1) A speed warning device is required for turbine engine powered airplanes

and for airplanes with VMO/MMO greater than 0.8 VDF/MDF or 0.8 V D/MD. The

speed warning device must give effective aural warning (differing

distinctively from aural warnings used for other purposes) to the pilots,

whenever the speed exceeds VMO plus 6 knots or MMO  +0.01. The upper limit of

the production tolerance for the warning device may not exceed the prescribed

warning speed.

(2) A machmeter is required at each pilot station for airplanes with

compressibility limitations not otherwise indicated to the pilot by the

airspeed indicating system required under paragraph (b)(1) of this section.

[Amdt. 25-23, 35 FR 5678, Apr. 8, 1970, as amended by Amdt. 25-24, 35 FR

7108, May 6, 1970; Amdt. 25-38, 41 FR 55467, Dec. 20, 1976]

Sec. 25.1305  Powerplant instruments.

The following are required powerplant instruments:

(a) For all airplanes. (1) A fuel pressure warning means for each engine,

or a master warning means for all engines with provision for isolating the

individual warning means from the master warning means.

(2) A fuel quantity indicator for each fuel tank.

(3) An oil quantity indicator for each oil tank.

(4) An oil pressure indicator for each independent pressure oil system of

each engine.

(5) An oil pressure warning means for each engine, or a master warning

means for all engines with provision for isolating the individual warning

means from the master warning means.

(6) An oil temperature indicator for each engine.

(7) Fire-warning indicators.

(8) An augmentation liquid quantity indicator (appropriate for the manner

in which the liquid is to be used in operation) for each tank.

(b) For reciprocating engine-powered airplanes. In addition to the

powerplant instruments required by paragraph (a) of this section, the

following powerplant instruments are required:

(1) A carburetor air temperature indicator for each engine.

(2) A cylinder head temperature indicator for each air-cooled engine.

(3) A manifold pressure indicator for each engine.

(4) A fuel pressure indicator (to indicate the pressure at which the fuel

is supplied) for each engine.

(5) A fuel flowmeter, or fuel mixture indicator, for each engine without an

automatic altitude mixture control.

(6) A tachometer for each engine.

(7) A device that indicates, to the flight crew (during flight), any change

in the power output, for each engine with--

(i) An automatic propeller feathering system, whose operation is initiated

by a power output measuring system; or

(ii) A total engine piston displacement of 2,000 cubic inches or more.

(8) A means to indicate to the pilot when the propeller is in reverse

pitch, for each reversing propeller.

(c) For turbine engine-powered airplanes.  In addition to the powerplant

instruments required by paragraph (a) of this section, the following

powerplant instruments are required:

(1) A gas temperature indicator for each engine.

(2) A fuel flowmeter indicator for each engine.

(3) A tachometer (to indicate the speed of the rotors with established

limiting speeds) for each engine.

(4) A means to indicate, to the flight crew, the operation of each engine

starter that can be operated continuously but that is neither designed for

continuous operation nor designed to prevent hazard if it failed.

(5) An indicator to indicate the functioning of the powerplant ice

protection system for each engine.

(6) An indicator for the fuel strainer or filter required by Sec. 25.997 to

indicate the occurrence of contamination of the strainer or filter before it

reaches the capacity established in accordance with Sec. 25.997(d).

(7) A warning means for the oil strainer or filter required by Sec.

25.1019, if it has no bypass, to warn the pilot of the occurrence of

contamination of the strainer or filter screen before it reaches the capacity

established in accordance with Sec. 25.1019(a)(2).

(8) An indicator to indicate the proper functioning of any heater used to

prevent ice clogging of fuel system components.

(d) For turbojet engine powered airplanes.  In addition to the powerplant

instruments required by paragraphs (a) and (c) of this section, the following

powerplant instruments are required:

(1) An indicator to indicate thrust, or a parameter that is directly

related to thrust, to the pilot. The indication must be based on the direct

measurement of thrust or of parameters that are directly related to thrust.

The indicator must indicate a change in thrust resulting from any engine

malfunction, damage, or deterioration.

(2) A position indicating means to indicate to the flight crew when the

thrust reversing device is in the reverse thrust position, for each engine

using a thrust reversing device.

(3) An indicator to indicate rotor system unbalance.

(e) For turbopropeller-powered airplanes.  In addition to the powerplant

instruments required by paragraphs (a) and (c) of this section, the following

powerplant instruments are required:

(1) A torque indicator for each engine.

(2) Position indicating means to indicate to the flight crew when the

propeller blade angle is below the flight low pitch position, for each

propeller.

(f) For airplanes equipped with fluid systems (other than fuel) for thrust

or power augmentation, an approved means must be provided to indicate the

proper functioning of that system to the flight crew.

[Amdt. 25-23, 35 FR 5678, Apr. 8, 1970, as amended by Amdt. 25-35, 39 FR

1831, Jan. 15, 1974; Amdt. 25-36, 39 FR 35461, Oct. 1, 1974; Amdt. 25-38, 41

FR 55467, Dec. 20, 1976; Amdt. 25-54, 45 FR 60173, Sept. 11, 1980; Amdt.

25-72, 55 FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1307  Miscellaneous equipment.

The following is required miscellaneous equipment:

(a) [Reserved]

(b) Two or more independent sources of electrical energy.

(c) Electrical protective devices, as prescribed in this part.

(d) Two systems for two-way radio communications, with controls for each

accessible from each pilot station, designed and installed so that failure of

one system will not preclude operation of the other system. The use of a

common antenna system is acceptable if adequate reliability is shown.

(e) Two systems for radio navigation, with controls for each accessible

from each pilot station, designed and installed so that failure of one system

will not preclude operation of the other system. The use of a common antenna

system is acceptable if adequate reliability is shown.

[Amdt. 25-23, 35 FR 5678, Apr. 8, 1970, as amended by Amdt. 25-46, 43 FR

50598, Oct. 30, 1978; Amdt. 25-54, 45 FR 60173, Sept. 11, 1980; Amdt. 25-72,

55 FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1309  Equipment, systems, and installations.

(a) The equipment, systems, and installations whose functioning is required

by this subchapter, must be designed to ensure that they perform their

intended functions under any foreseeable operating condition.

(b) The airplane systems and associated components, considered separately

and in relation to other systems, must be designed so that--

(1) The occurrence of any failure condition which would prevent the

continued safe flight and landing of the airplane is extremely improbable,

and

(2) The occurrence of any other failure conditions which would reduce the

capability of the airplane or the ability of the crew to cope with adverse

operating conditions is improbable.

(c) Warning information must be provided to alert the crew to unsafe system

operating conditions, and to enable them to take appropriate corrective

action. Systems, controls, and associated monitoring and warning means must

be designed to minimize crew errors which could create additional hazards.

(d) Compliance with the requirements of paragraph (b) of this section must

be shown by analysis, and where necessary, by appropriate ground, flight, or

simulator tests. The analysis must consider--

(1) Possible modes of failure, including malfunctions and damage from

external sources.

(2) The probability of multiple failures and undetected failures.

(3) The resulting effects on the airplane and occupants, considering the

stage of flight and operating conditions, and

(4) The crew warning cues, corrective action required, and the capability

of detecting faults.

(e) Each installation whose functioning is required by this subchapter, and

that requires a power supply, is an "essential load" on the power supply. The

power sources and the system must be able to supply the following power loads

in probable operating combinations and for probable durations:

(1) Loads connected to the system with the system functioning normally.

(2) Essential loads, after failure of any one prime mover, power converter,

or energy storage device.

(3) Essential loads after failure of--

(i) Any one engine on two-engine airplanes; and

(ii) Any two engines on three-or-more-engine airplanes.

(4) Essential loads for which an alternate source of power is required by

this chapter, after any failure or malfunction in any one power supply

system, distribution system, or other utilization system.

(f) In determining compliance with paragraphs (e) (2) and (3) of this

section, the power loads may be assumed to be reduced under a monitoring

procedure consistent with safety in the kinds of operation authorized. Loads

not required in controlled flight need not be considered for the two-engine-

inoperative condition on airplanes with three or more engines.

(g) In showing compliance with paragraphs (a) and (b) of this section with

regard to the electrical system and equipment design and installation,

critical environmental conditions must be considered. For electrical

generation, distribution, and utilization equipment required by or used in

complying with this chapter, except equipment covered by Technical Standard

Orders containing environmental test procedures, the ability to provide

continuous, safe service under foreseeable environmental conditions may be

shown by environmental tests, design analysis, or reference to previous

comparable service experience on other aircraft.

[Amdt. 25-23, 35 FR 5679, Apr. 8, 1970, as amended by Amdt. 25-38, 41 FR

55467, Dec. 20, 1976; Amdt. 25-41, 42 FR 36970, July 18, 1977]

Instruments: Installation

Sec. 25.1321  Arrangement and visibility.

(a) Each flight, navigation, and powerplant instrument for use by any pilot

must be plainly visible to him from his station with the minimum practicable

deviation from his normal position and line of vision when he is looking

forward along the flight path.

(b) The flight instruments required by Sec. 25.1303 must be grouped on the

instrument panel and centered as nearly as practicable about the vertical

plane of the pilot's forward vision. In addition--

(1) The instrument that most effectively indicates attitude must be on the

panel in the top center position;

(2) The instrument that most effectively indicates airspeed must be

adjacent to and directly to the left of the instrument in the top center

position:

(3) The instrument that most effectively indicates altitude must be

adjacent to and directly to the right of the instrument in the top center

position; and

(4) The instrument that most effectively indicates direction of flight must

be adjacent to and directly below the instrument in the top center position.

(c) Required powerplant instruments must be closely grouped on the

instrument panel. In addition--

(1) The location of identical powerplant instruments for the engines must

prevent confusion as to which engine each instrument relates; and

(2) Powerplant instruments vital to the safe operation of the airplane must

be plainly visible to the appropriate crewmembers.

(d) Instrument panel vibration may not damage or impair the accuracy of any

instrument.

(e) If a visual indicator is provided to indicate malfunction of an

instrument, it must be effective under all probable cockpit lighting

conditions.

[Amdt. 25-23, 35 FR 5679, Apr. 8, 1970, as amended by Amdt. 25-41, 42 FR

36970, July 18, 1977]

Sec. 25.1322  Warning, caution, and advisory lights.

If warning, caution or advisory lights are installed in the cockpit, they

must, unless otherwise approved by the Administrator, be--

(a) Red, for warning lights (lights indicating a hazard which may require

immediate corrective action);

(b) Amber, for caution lights (lights indicating the possible need for

future corrective action);

(c) Green, for safe operation lights; and

(d) Any other color, including white, for lights not described in

paragraphs (a) through (c) of this section, provided the color differs

sufficiently from the colors prescribed in paragraphs (a) through (c) of this

section to avoid possible confusion.

[Amdt. 25-38, 41 FR 55467, Dec. 20, 1976]

Sec. 25.1323  Airspeed indicating system.

For each airspeed indicating system, the following apply:

(a) Each airspeed indicating instrument must be approved and must be

calibrated to indicate true airspeed (at sea level with a standard

atmosphere) with a minimum practicable instrument calibration error when the

corresponding pitot and static pressures are applied.

(b) Each system must be calibrated to determine the system error (that is,

the relation between IAS and CAS) in flight and during the accelerated

takeoff ground run. The ground run calibration must be determined--

(1) From 0.8 of the minimum value of V1 to the maximum value of V2,

considering the approved ranges of altitude and weight; and

(2) With the flaps and power settings corresponding to the values

determined in the establishment of the takeoff path under Sec. 25.111

assuming that the critical engine fails at the minimum value of V1.

(c) The airspeed error of the installation, excluding the airspeed

indicator instrument calibration error, may not exceed three percent or five

knots, whichever is greater, throughout the speed range, from--

(1) VMO to 1.3 VS1, with flaps retracted; and

(2) 1.3 VS0 to VFE with flaps in the landing position.

(d) Each system must be arranged, so far as practicable, to prevent

malfunction or serious error due to the entry of moisture, dirt, or other

substances.

(e) Each system must have a heated pitot tube or an equivalent means of

preventing malfunction due to icing.

(f) Where duplicate airspeed indicators are required, their respective

pitot tubes must be far enough apart to avoid damage to both tubes in a

collision with a bird.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-57, 49 FR

6849, Feb. 23, 1984]

Sec. 25.1325  Static pressure systems.

(a) Each instrument with static air case connections must be vented to the

outside atmosphere through an appropriate piping system.

(b) Each static port must be designed and located in such manner that the

static pressure system performance is least affected by airflow variation, or

by moisture or other foreign matter, and that the correlation between air

pressure in the static pressure system and true ambient atmospheric static

pressure is not changed when the airplane is exposed to the continuous and

intermittent maximum icing conditions defined in Appendix C of this part.

(c) The design and installation of the static pressure system must be such

that--

(1) Positive drainage of moisture is provided; chafing of the tubing and

excessive distortion or restriction at bends in the tubing is avoided; and

the materials used are durable, suitable for the purpose intended, and

protected against corrosion; and

(2) It is airtight except for the port into the atmosphere. A proof test

must be conducted to demonstrate the integrity of the static pressure system

in the following manner:

(i) Unpressurized airplanes. Evacuate the static pressure system to a

pressure differential of approximately 1 inch of mercury or to a reading on

the altimeter, 1,000 feet above the airplane elevation at the time of the

test. Without additional pumping for a period of 1 minute, the loss of

indicated altitude must not exceed 100 feet on the altimeter.

(ii) Pressurized airplanes. Evacuate the static pressure system until a

pressure differential equivalent to the maximum cabin pressure differential

for which the airplane is type certificated is achieved. Without additional

pumping for a period of 1 minute, the loss of indicated altitude must not

exceed 2 percent of the equivalent altitude of the maximum cabin differential

pressure or 100 feet, whichever is greater.

(d) Each pressure altimeter must be approved and must be calibrated to

indicate pressure altitude in a standard atmosphere, with a minimum

practicable calibration error when the corresponding static pressures are

applied.

(e) Each system must be designed and installed so that the error in

indicated pressure altitude, at sea level, with a standard atmosphere,

excluding instrument calibration error, does not result in an error of more

than +/-30 feet per 100 knots speed for the appropriate configuration in the

speed range between 1.3 VS0 with flaps extended and 1.8 VS1 with flaps

retracted. However, the error need not be less than +/-30 feet.

(f) If an altimeter system is fitted with a device that provides

corrections to the altimeter indication, the device must be designed and

installed in such manner that it can be bypassed when it malfunctions, unless

an alternate altimeter system is provided. Each correction device must be

fitted with a means for indicating the occurrence of reasonably probable

malfunctions, including power failure, to the flight crew. The indicating

means must be effective for any cockpit lighting condition likely to occur.

(g) Except as provided in paragraph (h) of this section, if the static

pressure system incorporates both a primary and an alternate static pressure

source, the means for selecting one or the other source must be designed so

that--

(1) When either source is selected, the other is blocked off; and

(2) Both sources cannot be blocked off simultaneously.

(h) For unpressurized airplanes, paragraph (g)(1) of this section does not

apply if it can be demonstrated that the static pressure system calibration,

when either static pressure source is selected, is not changed by the other

static pressure source being open or blocked.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-5, 30 FR

8261, June 29, 1965; Amdt. 25-12, 32 FR 7587, May 24, 1967; Amdt. 25-41, 42

FR 36970, July 18, 1977]

Sec. 25.1326  Pitot heat indication systems.

If a flight instrument pitot heating system is installed, an indication

system must be provided to indicate to the flight crew when that pitot

heating system is not operating. The indication system must comply with the

following requirements:

(a) The indication provided must incorporate an amber light that is in

clear view of a flight crewmember.

(b) The indication provided must be designed to alert the flight crew if

either of the following conditions exist:

(1) The pitot heating system is switched "off".

(2) The pitot heating system is switched "on" and any pitot tube heating

element is inoperative.

[Amdt. 25-43, 43 FR 10339, Mar. 13, 1978]

Sec. 25.1327  Magnetic direction indicator.

(a) Each magnetic direction indicator must be installed so that its

accuracy is not excessively affected by the airplane's vibration or magnetic

fields.

(b) The compensated installation may not have a deviation, in level flight,

greater than 10 degrees on any heading.

Sec. 25.1329  Automatic pilot system.

(a) Each automatic pilot system must be approved and must be designed so

that the automatic pilot can be quickly and positively disengaged by the

pilots to prevent it from interfering with their control of the airplane.

(b) Unless there is automatic synchronization, each system must have a

means to readily indicate to the pilot the alignment of the actuating device

in relation to the control system it operates.

(c) Each manually operated control for the system must be readily

accessible to the pilots.

(d) Quick release (emergency) controls must be on both control wheels, on

the side of each wheel opposite the throttles.

(e) Attitude controls must operate in the plane and sense of motion

specified in Secs. 25.777(b) and 25.779(a) for cockpit controls. The

direction of motion must be plainly indicated on, or adjacent to, each

control.

(f) The system must be designed and adjusted so that, within the range of

adjustment available to the human pilot, it cannot produce hazardous loads on

the airplane, or create hazardous deviations in the flight path, under any

condition of flight appropriate to its use, either during normal operation or

in the event of a malfunction, assuming that corrective action begins within

a reasonable period of time.

(g) If the automatic pilot integrates signals from auxiliary controls or

furnishes signals for operation of other equipment, there must be positive

interlocks and sequencing of engagement to prevent improper operation.

Protection against adverse interaction of integrated components, resulting

from a malfunction, is also required.

(h) If the automatic pilot system can be coupled to airborne navigation

equipment, means must be provided to indicate to the flight crew the current

mode of operation. Selector switch position is not acceptable as a means of

indication.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-46, 43 FR

50598, Oct. 30, 1978]

Sec. 25.1331  Instruments using a power supply.

(a) For each instrument required by Sec. 25.1303(b) that uses a power

supply, the following apply:

(1) Each instrument must have a visual means integral with, the instrument,

to indicate when power adequate to sustain proper instrument performance is

not being supplied. The power must be measured at or near the point where it

enters the instruments. For electric instruments, the power is considered to

be adequate when the voltage is within approved limits.

(2) Each instrument must, in the event of the failure of one power source,

be supplied by another power source. This may be accomplished automatically

or by manual means.

(3) If an instrument presenting navigation data receives information from

sources external to that instrument and loss of that information would render

the presented data unreliable, the instrument must incorporate a visual means

to warn the crew, when such loss of information occurs, that the presented

data should not be relied upon.

(b) As used in this section, "instrument" includes devices that are

physically contained in one unit, and devices that are composed of two or

more physically separate units or components connected together (such as a

remote indicating gyroscopic direction indicator that includes a magnetic

sensing element, a gyroscopic unit, an amplifier and an indicator connected

together).

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-41, 42 FR

36970, July 18, 1977]

Sec. 25.1333  Instrument systems.

For systems that operate the instruments required by Sec. 25.1303(b) which

are located at each pilot's station--

(a) Means must be provided to connect the required instruments at the first

pilot's station to operating systems which are independent of the operating

systems at other flight crew stations, or other equipment;

(b) The equipment, systems, and installations must be designed so that one

display of the information essential to the safety of flight which is

provided by the instruments, including attitude, direction, airspeed, and

altitude will remain available to the pilots, without additional crewmember

action, after any single failure or combination of failures that is not shown

to be extremely improbable; and

(c) Additional instruments, systems, or equipment may not be connected to

the operating systems for the required instruments, unless provisions are

made to ensure the continued normal functioning of the required instruments

in the event of any malfunction of the additional instruments, systems, or

equipment which is not shown to be extremely improbable.

[Amdt. 25-23, 35 FR 5679, Apr. 8, 1970, as amended by Amdt. 25-41, 42 FR

36970, July 18, 1977]

Sec. 25.1335  Flight director systems.

If a flight director system is installed, means must be provided to

indicate to the flight crew its current mode of operation. Selector switch

position is not acceptable as a means of indication.

[Amdt. 25-41, 42 FR 36970, July 18, 1977]

Sec. 25.1337  Powerplant instruments.

(a) Instruments and instrument lines.

(1) Each powerplant and auxiliary power unit instrument line must meet the

requirements of Secs. 25.993 and 25.1183.

(2) Each line carrying flammable fluids under pressure must--

(i) Have restricting orifices or other safety devices at the source of

pressure to prevent the escape of excessive fluid if the line fails; and

(ii) Be installed and located so that the escape of fluids would not create

a hazard.

(3) Each powerplant and auxiliary power unit instrument that utilizes

flammable fluids must be installed and located so that the escape of fluid

would not create a hazard.

(b) Fuel quantity indicator. There must be means to indicate to the flight

crewmembers, the quantity, in gallons or equivalent units, of usable fuel in

each tank during flight. In addition--

(1) Each fuel quantity indicator must be calibrated to read "zero" during

level flight when the quantity of fuel remaining in the tank is equal to the

unusable fuel supply determined under Sec. 25.959;

(2) Tanks with interconnected outlets and airspaces may be treated as one

tank and need not have separate indicators; and

(3) Each exposed sight gauge, used as a fuel quantity indicator, must be

protected against damage.

(c) Fuel flowmeter system. If a fuel flowmeter system is installed, each

metering component must have a means for bypassing the fuel supply if

malfunction of that component severely restricts fuel flow.

(d) Oil quantity indicator. There must be a stick gauge or equivalent means

to indicate the quantity of oil in each tank. If an oil transfer or reserve

oil supply system is installed, there must be a means to indicate to the

flight crew, in flight, the quantity of oil in each tank.

(e) Turbopropeller blade position indicator. Required turbopropeller blade

position indicators must begin indicating before the blade moves more than

eight degrees below the flight low pitch stop. The source of indication must

directly sense the blade position.

(f) Fuel pressure indicator. There must be means to measure fuel pressure,

in each system supplying reciprocating engines, at a point downstream of any

fuel pump except fuel injection pumps. In addition--

(1) If necessary for the maintenance of proper fuel delivery pressure,

there must be a connection to transmit the carburetor air intake static

pressure to the proper pump relief valve connection; and

(2) If a connection is required under paragraph (f)(1) of this section, the

gauge balance lines must be independently connected to the carburetor inlet

pressure to avoid erroneous readings.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-40, 42 FR

15044, Mar. 17, 1977]

Electrical Systems and Equipment

Sec. 25.1351  General.

(a) Electrical system capacity. The required generating capacity, and

number and kinds of power sources must--

(1) Be determined by an electrical load analysis; and

(2) Meet the requirements of Sec. 25.1309.

(b) Generating system. The generating system includes electrical power

sources, main power busses, transmission cables, and associated control,

regulation, and protective devices. It must be designed so that--

(1) Power sources function properly when independent and when connected in

combination;

(2) No failure or malfunction of any power source can create a hazard or

impair the ability of remaining sources to supply essential loads;

(3) The system voltage and frequency (as applicable) at the terminals of

all essential load equipment can be maintained within the limits for which

the equipment is designed, during any probable operating condition; and

(4) System transients due to switching, fault clearing, or other causes do

not make essential loads inoperative, and do not cause a smoke or fire

hazard.

(5) There are means accessible, in flight, to appropriate crewmembers for

the individual and collective disconnection of the electrical power sources

from the system.

(6) There are means to indicate to appropriate crewmembers the generating

system quantities essential for the safe operation of the system, such as the

voltage and current supplied by each generator.

(c) External power. If provisions are made for connecting external power to

the airplane, and that external power can be electrically connected to

equipment other than that used for engine starting, means must be provided to

ensure that no external power supply having a reverse polarity, or a reverse

phase sequence, can supply power to the airplane's electrical system.

(d) Operation without normal electrical power. It must be shown by

analysis, tests, or both, that the airplane can be operated safely in VFR

conditions, for a period of not less than five minutes, with the normal

electrical power (electrical power sources excluding the battery)

inoperative, with critical type fuel (from the standpoint of flameout and

restart capability), and with the airplane initially at the maximum

certificated altitude. Parts of the electrical system may remain on if--

(1) A single malfunction, including a wire bundle or junction box fire,

cannot result in loss of both the part turned off and the part turned on; and

(2) The parts turned on are electrically and mechanically isolated from the

parts turned off.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-41, 42 FR

36970, July 18, 1977; Amdt. 25-72, 55 FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1353  Electrical equipment and installations.

(a) Electrical equipment, controls, and wiring must be installed so that

operation of any one unit or system of units will not adversely affect the

simultaneous operation of any other electrical unit or system essential to

the safe operation.

(b) Cables must be grouped, routed, and spaced so that damage to essential

circuits will be minimized if there are faults in heavy current-carrying

cables.

(c) Storage batteries must be designed and installed as follows:

(1) Safe cell temperatures and pressures must be maintained during any

probable charging or discharging condition. No uncontrolled increase in cell

temperature may result when the battery is recharged (after previous complete

discharge)--

(i) At maximum regulated voltage or power;

(ii) During a flight of maximum duration; and

(iii) Under the most adverse cooling condition likely to occur in service.

(2) Compliance with paragraph (c)(1) of this section must be shown by test

unless experience with similar batteries and installations has shown that

maintaining safe cell temperatures and pressures presents no problem.

(3) No explosive or toxic gases emitted by any battery in normal operation,

or as the result of any probable malfunction in the charging system or

battery installation, may accumulate in hazardous quantities within the

airplane.

(4) No corrosive fluids or gases that may escape from the battery may

damage surrounding airplane structures or adjacent essential equipment.

(5) Each nickel cadmium battery installation capable of being used to start

an engine or auxiliary power unit must have provisions to prevent any

hazardous effect on structure or essential systems that may be caused by the

maximum amount of heat the battery can generate during a short circuit of the

battery or of its individual cells.

(6) Nickel cadmium battery installations capable of being used to start an

engine or auxiliary power unit must have--

(i) A system to control the charging rate of the battery automatically so

as to prevent battery overheating;

(ii) A battery temperature sensing and over-temperature warning system with

a means for disconnecting the battery from its charging source in the event

of an over-temperature condition; or

(iii) A battery failure sensing and warning system with a means for

disconnecting the battery from its charging source in the event of battery

failure.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-41, 42 FR

36970, July 18, 1977; Amdt. 25-42, 43 FR 2323, Jan. 16, 1978]

Sec. 25.1355  Distribution system.

(a) The distribution system includes the distribution busses, their

associated feeders, and each control and protective device.

(b) [Reserved]

(c) If two independent sources of electrical power for particular equipment

or systems are required by this chapter, in the event of the failure of one

power source for such equipment or system, another power source (including

its separate feeder) must be automatically provided or be manually selectable

to maintain equipment or system operation.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-23, 35 FR

5679, Apr. 8, 1970; Amdt. 25-38, 41 FR 55468, Dec. 20, 1976]

Sec. 25.1357  Circuit protective devices.

(a) Automatic protective devices must be used to minimize distress to the

electrical system and hazard to the airplane in the event of wiring faults or

serious malfunction of the system or connected equipment.

(b) The protective and control devices in the generating system must be

designed to de-energize and disconnect faulty power sources and power

transmission equipment from their associated busses with sufficient rapidity

to provide protection from hazardous over-voltage and other malfunctioning.

(c) Each resettable circuit protective device must be designed so that,

when an overload or circuit fault exists, it will open the circuit

irrespective of the position of the operating control.

(d) If the ability to reset a circuit breaker or replace a fuse is

essential to safety in flight, that circuit breaker or fuse must be located

and identified so that it can be readily reset or replaced in flight.

(e) Each circuit for essential loads must have individual circuit

protection. However, individual protection for each circuit in an essential

load system (such as each position light circuit in a system) is not

required.

(f) If fuses are used, there must be spare fuses for use in flight equal to

at least 50 percent of the number of fuses of each rating required for

complete circuit protection.

(g) Automatic reset circuit breakers may be used as integral protectors for

electrical equipment (such as thermal cut-outs) if there is circuit

protection to protect the cable to the equipment.

Sec. 25.1359  [Removed. 55 FR 29785, July 20, 1990]

EDITORIAL NOTE: For the convenience of the user, the removed text is

set out below.

Sec. 25.1359  Electrical system fire and smoke protection.

(a) Components of the electrical system must meet the applicable fire and

smoke protection requirements of Secs. 25.831(c), 25.863, and 25.867.

(b) Electrical cables, terminals, and equipment in designated fire zones,

that are used during emergency procedures, must be at least fire-resistant.

(c) Main power cables (including generator cables) in the fuselage must be

designed to allow a reasonable degree of deformation and stretching without

failure and must--

(1) Be isolated from flammable fluid lines; or

(2) Be shrouded by means of electrically insulated flexible conduit, or

equivalent, which is in addition to the normal cable insulation.

(d) Insulation on electrical wire and electrical cable installed in any

area of the fuselage must be self-extinguishing when tested at an angle of 60

deg. in accordance with the applicable portions of Appendix F of this part,

or other approved equivalent methods. The average burn length may not exceed

3 inches and the average flame time after removal of the flame source may not

exceed 30 seconds. Drippings from the test specimen may not continue to flame

for more than an average of 3 seconds after falling.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-17, 33 FR

9066, June 20, 1968; Amdt. 25-32, 37 FR 3972, Feb. 24, 1972; Amdt. 25-57, 49

FR 6849, Feb. 23, 1984]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1363  Electrical system tests.

(a) When laboratory tests of the electrical system are conducted--

(1) The tests must be performed on a mock-up using the same generating

equipment used in the airplane;

(2) The equipment must simulate the electrical characteristics of the

distribution wiring and connected loads to the extent necessary for valid

test results; and

(3) Laboratory generator drives must simulate the actual prime movers on

the airplane with respect to their reaction to generator loading, including

loading due to faults.

(b) For each flight condition that cannot be simulated adequately in the

laboratory or by ground tests on the airplane, flight tests must be made.

Lights

Sec. 25.1381  Instrument lights.

(a) The instrument lights must--

(1) Provide sufficient illumination to make each instrument, switch and

other device necessary for safe operation easily readable unless sufficient

illumination is available from another source; and

(2) Be installed so that--

(i) Their direct rays are shielded from the pilot's eyes; and

(ii) No objectionable reflections are visible to the pilot.

(b) Unless undimmed instrument lights are satisfactory under each expected

flight condition, there must be a means to control the intensity of

illumination.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55 FR

29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1383  Landing lights.

(a) Each landing light must be approved, and must be installed so that--

(1) No objectionable glare is visible to the pilot;

(2) The pilot is not adversely affected by halation; and

(3) It provides enough light for night landing.

(b) Except when one switch is used for the lights of a multiple light

installation at one location, there must be a separate switch for each light.

(c) There must be a means to indicate to the pilots when the landing lights

are extended.

Sec. 25.1385  Position light system installation.

(a) General. Each part of each position light system must meet the

applicable requirements of this section and each system as a whole must meet

the requirements of Secs. 25.1387 through 25.1397.

(b) Forward position lights. Forward position lights must consist of a red

and a green light spaced laterally as far apart as practicable and installed

forward on the airplane so that, with the airplane in the normal flying

position, the red light is on the left side and the green light is on the

right side. Each light must be approved.

(c) Rear position light. The rear position light must be a white light

mounted as far aft as practicable on the tail or on each wing tip, and must

be approved.

(d) Light covers and color filters.  Each light cover or color filter must

be at least flame resistant and may not change color or shape or lose any

appreciable light transmission during normal use.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55468, Dec. 20, 1976]

Sec. 25.1387  Position light system dihedral angles.

(a) Except as provided in paragraph (e) of this section, each forward and

rear position light must, as installed, show unbroken light within the

dihedral angles described in this section.

(b) Dihedral angle L (left) is formed by two intersecting vertical planes,

the first parallel to the longitudinal axis of the airplane, and the other at

110 degrees to the left of the first, as viewed when looking forward along

the longitudinal axis.

(c) Dihedral angle R (right) is formed by two intersecting vertical planes,

the first parallel to the longitudinal axis of the airplane, and the other at

110 degrees to the right of the first, as viewed when looking forward along

the longitudinal axis.

(d) Dihedral angle A (aft) is formed by two intersecting vertical planes

making angles of 70 degrees to the right and to the left, respectively, to a

vertical plane passing through the longitudinal axis, as viewed when looking

aft along the longitudinal axis.

(e) If the rear position light, when mounted as far aft as practicable in

accordance with Sec. 25.1385(c), cannot show unbroken light within dihedral

angle A (as defined in paragraph (d) of this section), a solid angle or

angles of obstructed visibility totaling not more than 0.04 steradians is

allowable within that dihedral angle, if such solid angle is within a cone

whose apex is at the rear position light and whose elements make an angle of

30 deg. with a vertical line passing through the rear position light.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-30, 36 FR

21278, Nov. 5, 1971]

Sec. 25.1389  Position light distribution and intensities.

(a) General. The intensities prescribed in this section must be provided by

new equipment with light covers and color filters in place. Intensities must

be@determined with the light source operating at a steady value equal to the

average luminous output of the source at the normal operating voltage of the

airplane. The light distribution and intensity of each position light must

meet the requirements of paragraph (b) of this section.

(b) Forward and rear position lights. The light distribution and

intensities of forward and rear position lights must be expressed in terms of

minimum intensities in the horizontal plane, minimum intensities in any

vertical plane, and maximum intensities in overlapping beams, within dihedral

angles L, R, and A, and must meet the following requirements:

(1) Intensities in the horizontal plane. Each intensity in the horizontal

plane (the plane containing the longitudinal axis of the airplane and

perpendicular to the plane of symmetry of the airplane) must equal or exceed

the values in Sec. 25.1391.

(2) Intensities in any vertical plane.  Each intensity in any vertical

plane (the plane perpendicular to the horizontal plane) must equal or exceed

the appropriate value in Sec. 25.1393, where I is the minimum intensity

prescribed in Sec. 25.1391 for the corresponding angles in the horizon@l

plane.

(3) Intensities in overlaps between adjacent signals. No intensity in any

overlap between adjacent signals may exceed the values given in Sec. 25.1395,

except that higher intensities in overlaps may be used with main beam

intensities substantially greater than the minima specified in Secs. 25.1391

and 25.1393 if the overlap intensities in relation to the main beam

intensities do not adversely affect signal clarity. When the peak intensity

of the forward position lights is more than 100 candles, the maximum overlap

intensities between them may exceed the values given in Sec. 25.1395 if the

overlap intensity in Area A is not more than 10 percent of peak position

light intensity and the overlap intensity in Area B is not greater than 2.5

percent of peak position light intensity.

Sec. 25.1391  Minimum intensities in the horizontal plane of forward and rear

position lights.

Each position light intensity must equal or exceed the applicable values in

the following table:

Angle from right or left

Dihedral angle (light     of longitudinal axis,

included)          measured from dead ahead    Intensity (candles)

L and R (forward red and  0 deg. to 10 deg.         40

green)                    10 deg. to 20 deg.        30

20 deg. to 110 deg.       5

A (rear white)            110 deg. to 180 deg.      20

Sec. 25.1393  Minimum intensities in any vertical plane of forward and rear

position lights.

Each position light intensity must equal or exceed the applicable values in

the following table:

Angle above or

below the       Intensity,

horizontal plane       l

0 deg.                    1.00

0 deg. to 5 deg.          0.90

5 deg. to 10 deg.         0.80

10 deg. to 15 deg.        0.70

15 deg. to 20 deg.        0.50

20 deg. to 30 deg.        0.30

30 deg. to 40 deg.        0.10

40 deg. to 90 deg.        0.05

Sec. 25.1395  Maximum intensities in overlapping beams of forward and rear

position lights.

No position light intensity may exceed the applicable values in the

following table, except as provided in Sec. 25.1389(b)(3).

Maximum intensity

Area A     Area B

Overlaps             (candles)  (candles)

Green in dihedral angle L              10          1

Red in dihedral angle R                10          1

Green in dihedral angle A               5          1

Red in dihedral angle A                 5          1

Rear white in dihedral angle L          5          1

Rear white in dihedral angle R          5          1

Where--

(a) Area A includes all directions in the adjacent dihedral angle that pass

through the light source and intersect the common boundary plane at more than

10 degrees but less than 20 degrees; and

(b) Area B includes all directions in the adjacent dihedral angle that pass

through the light source and intersect the common boundary plane at more than

20 degrees.

Sec. 25.1397  Color specifications.

Each position light color must have the applicable International Commission

on Illumination chromaticity coordinates as follows:

(a) Aviation red--

"y" is not greater than 0.335; and

"z" is not greater than 0.002.

(b) Aviation green--

"x" is not greater than 0.440-0.320 y ;

"x" is not greater than y --0.170; and

"y" is not less than 0.390-0.170 x.

(c) Aviation white--

"x" is not less than 0.300 and not greater than 0.540;

"y" is not less than "x --0.040" or "y0--0.010", whichever is the smaller;

and

"y" is not greater than "x+0.020" nor "0.636-0.400 x";

Where "y0" is the "y" coordinate of the Planckian radiator for the value of

"x"  considered.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-27, 36 FR

12972, July 10, 1971]

Sec. 25.1399  Riding light.

(a) Each riding (anchor) light required for a seaplane or amphibian must be

installed so that it can--

(1) Show a white light for at least 2 nautical miles at night under clear

atmospheric conditions; and

(2) Show the maximum unbroken light practicable when the airplane is moored

or drifting on the water.

(b) Externally hung lights may be used.

Sec. 25.1401  Anticollision light system.

(a) General. The airplane must have an anticollision light system that--

(1) Consists of one or more approved anticollision lights located so that

their light will not impair the crew's vision or detract from the conspicuity

of the position lights; and

(2) Meets the requirements of paragraphs (b) through (f) of this section.

(b) Field of coverage. The system must consist of enough lights to

illuminate the vital areas around the airplane considering the physical

configuration and flight characteristics of the airplane. The field of

coverage must extend in each direction within at least 75 degrees above and

75 degrees below the horizontal plane of the airplane, except that a solid

angle or angles of obstructed visibility totaling not more than 0.03

steradians is allowable within a solid angle equal to 0.15 steradians

centered about the longitudinal axis in the rearward direction.

(c) Flashing characteristics. The arrangement of the system, that is, the

number of light sources, beam width, speed of rotation, and other

characteristics, must give an effective flash frequency of not less than 40,

nor more than 100 cycles per minute. The effective flash frequency is the

frequency at which the airplane's complete anticollision light system is

observed from a distance, and applies to each sector of light including any

overlaps that exist when the system consists of more than one light source.

In overlaps, flash frequencies may exceed 100, but not 180 cycles per minute.

(d) Color. Each anticollision light must be either aviation red or aviation

white and must meet the applicable requirements of Sec. 25.1397.

(e) Light intensity. The minimum light intensities in all vertical planes,

measured with the red filter (if used) and expressed in terms of "effective"

intensities, must meet the requirements of paragraph (f) of this section. The

following relation must be assumed:

t2

INTEGRAL       I(t)dt

t1

Ie =

--------------

0.2+(t2-t1)

where:

Ie=effective intensity (candles).

I(t)=instantaneous intensity as a function of time.

t2--t1=flash time interval (seconds).

Normally, the maximum value of effective intensity is obtained when t2 and t1

are chosen so that the effective intensity is equal to the instantaneous

intensity at t2 and t1.

(f) Minimum effective intensities for anticollision lights. Each

anticollision light effective intensity must equal or exceed the applicable

values in the following table.

Angle above or    Effective

below the       intensity

horizontal plane   (candles)

0 deg. to 5 deg.          400

5 deg. to 10 deg.         240

10 deg. to 20 deg.         80

20 deg. to 30 deg.         40

30 deg. to 75 deg.         20

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-27, 36 FR

12972, July 10, 1971; Amdt. 25-41, 42 FR 36970, July 18, 1977]

Sec. 25.1403  Wing icing detection lights.

Unless operations at night in known or forecast icing conditions are

prohibited by an operating limitation, a means must be provided for

illuminating or otherwise determining the formation of ice on the parts of

the wings that are critical from the standpoint of ice accumulation. Any

illumination that is used must be of a type that will not cause glare or

reflection that would handicap crewmembers in the performance of their

duties.

[Amdt. 25-38, 41 FR 55468, Dec. 20, 1976]

Safety Equipment

Sec. 25.1411  General.

(a) Accessibility. Required safety equipment to be used by the crew in an

emergency must be readily accessible.

(b) Stowage provisions. Stowage provisions for required emergency equipment

must be furnished and must--

(1) Be arranged so that the equipment is directly accessible and its

location is obvious; and

(2) Protect the safety equipment from inadvertent damage.

(c) Emergency exit descent device. The stowage provisions for the emergency

exit descent device required by Sec. 25.809(f) must be at the exits for which

they are intended.

(d) Liferafts. (1) The stowage provisions for the liferafts described in

Sec. 25.1415 must accommodate enough rafts for the maximum number of

occupants for which certification for ditching is requested.

(2) Liferafts must be stowed near exits through which the rafts can be

launched during an unplanned ditching.

(3) Rafts automatically or remotely released outside the airplane must be

attached to the airplane by means of the static line prescribed in Sec.

25.1415.

(4) The stowage provisions for each portable liferaft must allow rapid

detachment and removal of the raft for use at other than the intended exits.

(e) Long-range signaling device. The stowage provisions for the long-range

signaling device required by Sec. 25.1415 must be near an exit available

during an unplanned ditching.

(f) Life preserver stowage provisions. The stowage provisions for life

preservers described in Sec. 25.1415 must accommodate one life preserver for

each occupant for which certification for ditching is requested. Each life

preserver must be within easy reach of each seated occupant.

(g) Life line stowage provisions. If certification for ditching under Sec.

25.801 is requested, there must be provisions to store life lines. These

provisions must--

(1) Allow one life line to be attached to each side of the fuselage; and

(2) Be arranged to allow the life lines to be used to enable the occupants

to stay on the wing after ditching.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-32, 37 FR

3972, Feb. 24, 1972; Amdt. 25-46, 43 FR 50598, Oct. 30, 1978; Amdt. 25-53, 45

FR 41593, June 19, 1980; Amdt. 25-70, 54 FR 43925, Oct. 27, 1989; Amdt.

25-79, 58 FR 45229, Aug. 26, 1993]

*****************************************************************************

58 FR 45224, No. 164, Aug. 26, 1993

SUMMARY: These amendments to the airworthiness standards for transport

category airplanes and the operating rules for air carrier operators of such

airplanes modify the procedures for conducting an emergency evacuation

demonstration. These include a requirement that the flightcrew take no active

role in the demonstration, and a change to the age/sex distribution

requirement for demonstration participants. In addition, the airworthiness

standards are amended to standardize the illumination requirements for the

handles of the various types of passenger emergency exits, and to add a

requirement to prevent the inadvertent disabling of the public address system

because of an unstowed microphone. These amendments are intended to enhance

the provisions for egress of occupants of transport category airplanes under

emergency conditions.

EFFECTIVE DATE: September 27, 1993.

*****************************************************************************

Sec. 25.1413  [Removed. 55 FR 29785, July 20, 1990]

EDITORIAL NOTE: For the convenience of the user, the removed text is

set out below.

Sec. 25.1413  Safety belts.

(a) If there are means to indicate to the passengers when safety belts

should be fastened, they must be installed to be operated from either pilot

seat.

(b) The rated strength of safety belts may not be less than that required

to withstand the ultimate load factors specified in Sec. 25.561, considering

the dimensional characteristics of the belt installation for the specific

seat or berth arrangement.

(c) Each belt and shoulder harness must be attached so that no part of the

anchorage can fail at a load lower than that which would result from the

application of ultimate load factors equal to those specified in Sec. 25.561,

multiplied by a factor of 1.33. This factor must be used instead of the

fitting factor prescribed in Sec. 25.625. The forward load factor need not be

applied to safety belts for berths.

(d) Each safety belt must be equipped with a metal to metal latching

device.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-44, 43 FR

46233, Oct. 5, 1978; Amdt. 25-51, 45 FR 7755, Feb. 4, 1980]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1415  Ditching equipment.

(a) Ditching equipment used in airplanes to be certificated for ditching

under Sec. 25.801, and required by the operating rules of this chapter, must

meet the requirements of this section.

(b) Each liferaft and each life preserver must be approved. In addition--

(1) Unless excess rafts of enough capacity are provided, the buoyancy and

seating capacity beyond the rated capacity of the rafts must accommodate all

occupants of the airplane in the event of a loss of one raft of the largest

rated capacity; and

(2) Each raft must have a trailing line, and must have a static line

designed to hold the raft near the airplane but to release it if the airplane

becomes totally submerged.

(c) Approved survival equipment must be attached to each liferaft.

(d) There must be a survival type emergency locator transmitter that meets

the applicable requirements of TSO-C91 for use in one liferaft.

(e) For airplanes not certificated for ditching under Sec. 25.801 and not

having approved life preservers, there must be an approved flotation means

for each occupant. This means must be within easy reach of each seated

occupant and must be readily removable from the airplane.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-29, 36 FR

18722, Sept. 21, 1971; Amdt 25-50, 45 FR 38348, June 9, 1980; Amdt. 25-72, 55

FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1416  [Removed. 55 FR 29785, July 20, 1985]

EDITORIAL NOTE: For the convenience of the user, the removed text is

set out below.

Sec. 25.1416  Pneumatic de-icer boot system.

If certification with ice protection provisions is desired and a pnuematic

de-icer boot system is installed--

(a) The system must meet the requirements specified in Sec. 25.1419,

(b) The system and its components must be designed to perform their

intended function under any normal system operating temperature or pressure,

and

(c) Means to indicate to the flight crew that the pneumatic de-icer boot

system is receiving adequate pressure and is functioning normally must be

provided.

[Amdt. 25-46, 43 FR 50598, Oct. 30, 1978]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1419  Ice protection.

If certification with ice protection provisions is desired, the airplane

must be able to safely operate in the continuous maximum and intermittent

maximum icing conditions of appendix C. To establish that the airplane can

operate within the continuous maximum and intermittent maximum conditions of

appendix C:

(a) An analysis must be performed to establish that the ice protection for

the various components of the airplane is adequate, taking into account the

various airplane operational configurations; and

(b) To verify the ice protection analysis, to check for icing anomalies,

and to demonstrate that the ice protection system and its components are

effective, the airplane or its components must be flight tested in the

various operational configurations, in measured natural atmospheric icing

conditions and, as found necessary, by one or more of the following means:

(1) Laboratory dry air or simulated icing tests, or a combination of both,

of the components or models of the components.

(2) Flight dry air tests of the ice protection system as a whole, or of its

individual components.

(3) Flight tests of the airplane or its components in measured simulated

icing conditions.

(c) Caution information, such as an amber caution light or equivalent, must

be provided to alert the flightcrew when the anti-ice or de-ice system is not

functioning normally.

(d) For turbine engine powered airplanes, the ice protection provisions of

this section are considered to be applicable primarily to the airframe. For

the powerplant installation, certain additional provisions of subpart E of

this part may be found applicable.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1421  Megaphones.

If a megaphone is installed, a restraining means must be provided that is

capable of restraining the megaphone when it is subjected to the ultimate

inertia forces specified in Sec. 25.561(b)(3).

[Amdt. 25-41, 42 FR 36970, July 18, 1977]

Miscellaneous Equipment

Sec. 25.1423  Public address system.

A public address system required by this chapter must--

(a) Be powerable when the aircraft is in flight or stopped on the ground,

after the shutdown or failure of all engines and auxiliary power units, or

the disconnection or failure of all power sources dependent on their

continued operation, for--

(1) A time duration of at least 10 minutes, including an aggregate time

duration of at least 5 minutes of announcements made by flight and cabin

crewmembers, considering all other loads which may remain powered by the same

source when all other power sources are inoperative; and

(2) An additional time duration in its standby state appropriate or

required for any other loads that are powered by the same source and that are

essential to safety of flight or required during emergency conditions.

(b) Be capable of operation within 10 seconds by a flight attendant at

those stations in the passenger compartment from which the system is

accessible.

(c) Be intelligible at all passenger seats, lavatories, and flight

attendant seats and work stations.

(d) Be designed so that no unused, unstowed microphone will render the

system inoperative.

(e) Be capable of functioning independently of any required crewmember

interphone system.

(f) Be accessible for immediate use from each of two flight crewmember

stations in the pilot compartment.

(g) For each required floor-level passenger emergency exit which has an

adjacent flight attendant seat, have a microphone which is readily accessible

to the seated flight attendant, except that one microphone may serve more

than one exit, provided the proximity of the exits allows unassisted verbal

communication between seated flight attendants.

[Amdt. 25-79, 58 FR 45229, Aug. 26, 1993]

*****************************************************************************

58 FR 45224, No. 164, Aug. 26, 1993

SUMMARY: These amendments to the airworthiness standards for transport

category airplanes and the operating rules for air carrier operators of such

airplanes modify the procedures for conducting an emergency evacuation

demonstration. These include a requirement that the flightcrew take no active

role in the demonstration, and a change to the age/sex distribution

requirement for demonstration participants. In addition, the airworthiness

standards are amended to standardize the illumination requirements for the

handles of the various types of passenger emergency exits, and to add a

requirement to prevent the inadvertent disabling of the public address system

because of an unstowed microphone. These amendments are intended to enhance

the provisions for egress of occupants of transport category airplanes under

emergency conditions.

EFFECTIVE DATE: September 27, 1993.

*****************************************************************************

Sec. 25.1431  Electronic equipment.

(a) In showing compliance with Sec. 25.1309 (a) and (b) with respect to

radio and electronic equipment and their installations, critical

environmental conditions must be considered.

(b) Radio and electronic equipment must be supplied with power under the

requirements of Sec. 25.1355(c).

(c) Radio and electronic equipment, controls, and wiring must be installed

so that operation of any one unit or system of units will not adversely

affect the simultaneous operation of any other radio or electronic unit, or

system of units, required by this chapter.

Sec. 25.1433  Vacuum systems.

There must be means, in addition to the normal pressure relief, to

automatically relieve the pressure in the discharge lines from the vacuum air

pump when the delivery temperature of the air becomes unsafe.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55 FR

29785, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1435  Hydraulic systems.

(a) Design. (1) Each element of the hydraulic system must be designed to

withstand, without deformation that would prevent it from performing its

intended function, the design operating pressure loads in combination with

limit structural loads which may be imposed.

(2) Each element of the hydraulic system must be able to withstand, without

rupture, the design operating pressure loads multiplied by a factor of 1.5 in

combination with ultimate structural loads that can reasonably occur

simultaneously. Design operating pressure is maximum normal operating

pressure, excluding transient pressure.

(b) Tests and analysis. (1) A complete hydraulic system must be static

tested to show that it can withstand 1.5 times the design operating pressure

without a deformation of any part of the system that would prevent it from

performing its intended function. Clearance between structural members and

hydraulic system elements must be adequate and there must be no permanent

detrimental deformation. For the purpose of this test, the pressure relief

valve may be made inoperable to permit application of the required pressure.

(2) Compliance with Sec. 25.1309 for hydraulic systems must be shown by

functional tests, endurance tests, and analyses. The entire system, or

appropriate subsystems, must be tested in an airplane or in a mock-up

installation to determine proper performance and proper relation to other

aircraft systems. The functional tests must include simulation of hydraulic

system failure conditions. Endurance tests must simulate the repeated

complete flights that could be expected to occur in service. Elements which

fail during the tests must be modified in order to have the design deficiency

corrected and, where necessary, must be sufficiently retested. Simulation of

operating and environmental conditions must be completed on elements and

appropriate portions of the hydraulic system to the extent necessary to

evaluate the environmental effects. Compliance with Sec. 25.1309 must take

into account the following:

(i) Static and dynamic loads including flight, ground, pilot, hydrostatic,

inertial and thermally induced loads, and combinations thereof.

(ii) Motion, vibration, pressure transients, and fatigue.

(iii) Abrasion, corrosion, and erosion.

(iv) Fluid and material co@patibility.

(v) Leakage and wear.

(c) Fire protection. Each hydraulic system using flammable hydraulic fluid

must meet the applicable requirements of Secs. 25.863, 25.1183, 25.1185, and

25.1189.

[Amdt. 25-13, 32 FR 9154, June 28, 1967, as amended by Amdt. 25-41, 42 FR

36971, July 18, 1977; Amdt. 25-72, 55 FR 29786, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1438  Pressurization and pneumatic systems.

(a) Pressurization system elements must be burst pressure tested to 2.0

times, and proof pressure tested to 1.5 times, the maximum normal operating

pressure.

(b) Pneumatic system elements must be burst pressure tested to 3.0 times,

and proof pressure tested to 1.5 times, the maximum normal operating

pressure.

(c) An analysis, or a combination of analysis and test, may be substituted

for any test required by paragraph (a) or (b) of this section if the

Administrator finds it equivalent to the required test.

[Amdt. 25-41, 42 FR 36971, July 18, 1977]

Sec. 25.1439  Protective breathing equipment.

(a) If there is a class A, B, or E cargo compartment, protective breathing

equipment must be installed for the use of appropriate crewmembers. In

addition, protective breathing equipment must be installed in each isolated

separate compartment in the airplane, including upper and lower lobe galleys,

in which crewmember occupancy is permitted during flight for the maximum

number of crewmembers expected to be in the area during any operation.

(b) For protective breathing equipment required by paragraph (a) of this

section or by any operating rule of this chapter, the following apply:

(1) The equipment must be designed to protect the flight crew from smoke,

carbon dioxide, and other harmful gases while on flight deck duty and while

combating fires in cargo compartments.

(2) The equipment must include--

(i) Masks covering the eyes, nose, and mouth; or

(ii) Masks covering the nose and mouth, plus accessory equipment to cover

the eyes.

(3) The equipment, while in use, must allow the flight crew to use the

radio equipment and to communicate with each other, while at their assigned

duty stations.

(4) The part of the equipment protecting the eyes may not cause any

appreciable adverse effect on vision and must allow corrective glasses to be

worn.

(5) The equipment must supply protective oxygen of 15 minutes duration per

crewmember at a pressure altitude of 8,000 feet with a respiratory minute

volume of 30 liters per minute BTPD. If a demand oxygen system is used, a

supply of 300 liters of free oxygen at 70 deg. F. and 760 mm. Hg. pressure is

considered to be of 15-minute duration at the prescribed altitude and minute

volume. If a continuous flow protective breathing system is used (including a

mask with a standard rebreather bag) a flow rate of 60 liters per minute at

8,000 feet (45 liters per minute at sea level) and a supply of 600 liters of

free oxygen at 70 deg. F. and 760 mm. Hg. pressure is considered to be of 15-

minute duration at the prescribed altitude and minute volume. BTPD refers to

body temperature conditions (that is, 37 deg. C., at ambient pressure, dry).

(6) The equipment must meet the requirements of paragraphs (b) and (c) of

Sec. 25.1441.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55468, Dec. 20, 1976]

Sec. 25.1441  Oxygen equipment and supply.

(a) If certification with supplemental oxygen equipment is requested, the

equipment must meet the requirements of this section and Secs. 25.1443

through 25.1453.

(b) The oxygen system must be free from hazards in itself, in its method of

operation, and in its effect upon other components.

(c) There must be a means to allow the crew to readily determine, during

flight, the quantity of oxygen available in each source of supply.

(d) The oxygen flow rate and the oxygen equipment for airplanes for which

certification for operation above 40,000 feet is requested must be approved.

Sec. 25.1443  Minimum mass flow of supplemental oxygen.

(a) If continuous flow equipment is installed for use by flight

crewmembers, the minimum mass flow of supplemental oxygen required for each

crewmember may not be less than the flow required to maintain, during

inspiration, a mean tracheal oxygen partial pressure of 149 mm. Hg. when

breathing 15 liters per minute, BTPS, and with a maximum tidal volume of 700

cc. with a constant time interval between respirations.

(b) If demand equipment is installed for use by flight crewmembers, the

minimum mass flow of supplemental oxygen required for each crewmember may not

be less than the flow required to maintain, during inspiration, a mean

tracheal oxygen partial pressure of 122 mm. Hg., up to and including a cabin

pressure altitude of 35,000 feet, and 95 percent oxygen between cabin

pressure altitudes of 35,000 and 40,000 feet, when breathing 20 liters per

minute BTPS. In addition, there must be means to allow the crew to use

undiluted oxygen at their discretion.

(c) For passengers and cabin attendants, the minimum mass flow of

supplemental oxygen required for each person at various cabin pressure

altitudes may not be less than the flow required to maintain, during

inspiration and while using the oxygen equipment (including masks) provided,

the following mean tracheal oxygen partial pressures:

(1) At cabin pressure altitudes above 10,000 feet up to and including

18,500 feet, a mean tracheal oxygen partial pressure of 100 mm. Hg. when

breathing 15 liters per minute, BTPS, and with a tidal volume of 700 cc. with

a constant time interval between respirations.

(2) At cabin pressure altitudes above 18,500 feet up to and including

40,000 feet, a mean tracheal oxygen partial pressure of 83.8 mm. Hg. when

breathing 30 liters per minute, BTPS, and with a tidal volume of 1,100 cc.

with a constant time interval between respirations.

(d) If first-aid oxygen equipment is installed, the minimum mass flow of

oxygen to each user may not be less than four liters per minute, STPD.

However, there may be a means to decrease this flow to not less than two

liters per minute, STPD, at any cabin altitude. The quantity of oxygen

required is based upon an average flow rate of three liters per minute per

person for whom first-aid oxygen is required.

(e) If portable oxygen equipment is installed for use by crewmembers, the

minimum mass flow of supplemental oxygen is the same as specified in

paragraph (a) or (b) of this section, whichever is applicable.

Sec. 25.1445  Equipment standards for the oxygen distributing system.

(a) When oxygen is supplied to both crew and passengers, the distribution

system must be designed for either--

(1) A source of supply for the flight crew on duty and a separate source

for the passengers and other crewmembers; or

(2) A common source of supply with means to separately reserve the minimum

supply required by the flight crew on duty.

(b) Portable walk-around oxygen units of the continuous flow, diluter-

demand, and straight demand kinds may be used to meet the crew or passenger

breathing requirements.

Sec. 25.1447  Equipment standards for oxygen dispensing units.

If oxygen dispensing units are installed, the following apply:

(a) There must be an individual dispensing unit for each occupant for whom

supplemental oxygen is to be supplied. Units must be designed to cover the

nose and mouth and must be equipped with a suitable means to retain the unit

in position on the face. Flight crew masks for supplemental oxygen must have

provisions for the use of communication equipment.

(b) If certification for operation up to and including 25,000 feet is

requested, an oxygen supply terminal and unit of oxygen dispensing equipment

for the immediate use of oxygen by each crewmember must be within easy reach

of that crewmember. For any other occupants, the supply terminals and

dispensing equipment must be located to allow the use of oxygen as required

by the operating rules in this chapter.

(c) If certification for operation above 25,000 feet is requested, there

must be oxygen dispensing equipment meeting the following requirements:

(1) There must be an oxygen dispensing unit connected to oxygen supply

terminals immediately available to each occupant, wherever seated. If

certification for operation above 30,000 feet is requested, the dispensing

units providing the required oxygen flow must be automatically presented to

the occupants before the cabin pressure altitude exceeds 15,000 feet and the

crew must be provided with a manual means to make the dispensing units

immediately available in the event of failure of the automatic system. The

total number of dispensing units and outlets must exceed the number of seats

by at least 10 percent. The extra units must be as uniformly distributed

throughout the cabin as practicable.

(2) Each flight crewmember on flight deck duty must be provided with demand

equipment. In addition, each flight crewmember must be provided with a quick-

donning type of oxygen dispensing unit, connected to an oxygen supply

terminal, that is immediately available to him when seated at his station,

and that is designed and installed so that it--

(i) Can be placed on the face from its ready position, properly secured,

sealed, and supply oxygen upon demand, with one hand within five seconds and

without disturbing eyeglasses or causing delay in proceeding with emergency

duties; and

(ii) Allows, while in place, the performance of normal communication

functions.

(3) There must be at least two outlets and units of dispensing equipment of

a type similar to that required by paragraph (c)(1) of this section in--

(i) Each washroom; and

(ii) Each lavatory, if separate from the washroom.

(4) Portable oxygen equipment must be immediately available for each cabin

attendant.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-41, 42 FR

36971, July 18, 1977]

Sec. 25.1449  Means for determining use of oxygen.

There must be a means to allow the crew to determine whether oxygen is

being delivered to the dispensing equipment.

Sec. 25.1450  Chemical oxygen generators.

(a) For the purpose of this section, a chemical oxygen generator is defined

as a device which produces oxygen by chemical reaction.

(b) Each chemical oxygen generator must be designed and installed in

accordance with the following requirements:

(1) Surface temperature developed by the generator during operation may not

create a hazard to the airplane or to its occupants.

(2) Means must be provided to relieve any internal pressure that may be

hazardous.

(c) In addition to meeting the requirements in paragraph (b) of this

section, each portable chemical oxygen generator that is capable of sustained

operation by successive replacement of a generator element must be placarded

to show--

(1) The rate of oxygen flow, in liters per minute;

(2) The duration of oxygen flow, in minutes, for the replaceable generator

element; and

(3) A warning that the replaceable generator element may be hot, unless the

element construction is such that the surface temperature cannot exceed 100

degrees F.

[Amdt. 25-41, 42 FR 36971, July 18, 1977]

Sec. 25.1451  [Removed.  55 FR 29786, July 20, 1990]

EDITORIAL NOTE: For the convenience of the user, the removed text is

set out below.

Sec. 25.1451  Fire protection for oxygen equipment.

(a) Oxygen equipment and lines may not be in any designated fire zone.

(b) Oxygen equipment and lines must be protected from heat that may be

generated in, or escape from, any designated fire zone.

(c) Oxygen equipment and lines must be installed so that escaping oxygen

cannot cause ignition of grease, fluid, or vapor accumulations that are

present in normal operation or as a result of failure or malfunction of any

system.

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1453  Protection of oxygen equipment from rupture.

Oxygen pressure tanks, and lines between tanks and the shutoff means, must

be--

(a) Protected from unsafe temperatures; and

(b) Located where the probability and hazards of rupture in a crash landing

are minimized.

Sec. 25.1455  Draining of fluids subject to freezing.

If fluids subject to freezing may be drained overboard in flight or during

ground operation, the drains must be designed and located to prevent the

formation of hazardous quantities of ice on the airplane as a result of the

drainage.

[Amdt. 25-23, 35 FR 5680, Apr. 8, 1970]

Sec. 25.1457  Cockpit voice recorders.

(a) Each cockpit voice recorder required by the operating rules of this

chapter must be approved and must be installed so that it will record the

following:

(1) Voice communications transmitted from or received in the airplane by

radio.

(2) Voice communications of flight crewmembers on the flight deck.

(3) Voice communications of flight crewmembers on the flight deck, using

the airplane's interphone system.

(4) Voice or audio signals identifying navigation or approach aids

introduced into a headset or speaker.

(5) Voice communications of flight crewmembers using the passenger

loudspeaker system, if there is such a system and if the fourth channel is

available in accordance with the requirements of paragraph (c)(4)(ii) of this

section.

(b) The recording requirements of paragraph (a)(2) of this section must be

met by installing a cockpit-mounted area microphone, located in the best

position for recording voice communications originating at the first and

second pilot stations and voice communications of other crewmembers on the

flight deck when directed to those stations. The microphone must be so

located and, if necessary, the preamplifiers and filters of the recorder must

be so adjusted or supplemented, that the intelligibility of the recorded

communications is as high as practicable when recorded under flight cockpit

noise conditions and played back. Repeated aural or visual playback of the

record may be used in evaluating intelligibility.

(c) Each cockpit voice recorder must be installed so that the part of the

communication or audio signals specified in paragraph (a) of this section

obtained from each of the following sources is recorded on a separate

channel:

(1) For the first channel, from each boom, mask, or hand-held microphone,

headset, or speaker used at the first pilot station.

(2) For the second channel from each boom, mask, or hand-held microphone,

headset, or speaker used at the second pilot station.

(3) For the third channel--from the cockpit-mounted area microphone.

(4) For the fourth channel, from--

(i) Each boom, mask, or hand-held microphone, headset, or speaker used at

the station for the third and fourth crew members; or

(ii) If the stations specified in paragraph (c)(4)(i) of this section are

not required or if the signal at such a station is picked up by another

channel, each microphone on the flight deck that is used with the passenger

loudspeaker system, if its signals are not picked up by another channel.

(5) As far as is practicable all sounds received by the microphone listed

in paragraphs (c) (1), (2), and (4) of this section must be recorded without

interruption irrespective of the position of the interphone-transmitter key

switch. The design shall ensure that sidetone for the flight crew is produced

only when the interphone, public address system, or radio transmitters are in

use.

(d) Each cockpit voice recorder must be installed so that--

(1) It receives its electric power from the bus that provides the maximum

reliability for operation of the cockpit voice recorder without jeopardizing

service to essential or emergency loads;

(2) There is an automatic means to simultaneously stop the recorder and

prevent each erasure feature from functioning, within 10 minutes after crash

impact; and

(3) There is an aural or visual means for preflight checking of the

recorder for proper operation.

(e) The record container must be located and mounted to minimize the

probability of rupture of the container as a result of crash impact and

consequent heat damage to the record from fire. In meeting this requirement,

the record container must be as far aft as practicable, but may not be where

aft mounted engines may crush the container during impact. However, it need

not be outside of the pressurized compartment.

(f) If the cockpit voice recorder has a bulk erasure device, the

installation must be designed to minimize the probability of inadvertent

operation and actuation of the device during crash impact.

(g) Each recorder container must--

(1) Be either bright orange or bright yellow;

(2) Have reflective tape affixed to its external surface to facilitate its

location under water; and

(3) Have an underwater locating device, when required by the operating

rules of this chapter, on or adjacent to the container which is secured in

such manner that they are not likely to be separated during crash impact.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-2, 30 FR

3932, Mar. 26, 1965; Amdt. 25-16, 32 FR 13914, Oct. 6, 1967; Amdt. 25-41, 42

FR 36971, July 18, 1977; Amdt. 25-65, 53 FR 26143, July 11, 1988]

Sec. 25.1459  Flight recorders.

(a) Each flight recorder required by the operating rules of this chapter

must be installed so that--

(1) It is supplied with airspeed, altitude, and directional data obtained

from sources that meet the accuracy requirements of Secs. 25.1323, 25.1325,

and 25.1327, as appropriate;

(2) The vertical acceleration sensor is rigidly attached, and located

longitudinally either within the approved center of gravity limits of the

airplane, or at a distance forward or aft of these limits that does not

exceed 25 percent of the airplane's mean aerodynamic chord;

(3) It receives its electrical power from the bus that provides the maximum

reliability for operation of the flight recorder without jeopardizing service

to essential or emergency loads;

(4) There is an aural or visual means for preflight checking of the

recorder for proper recording of data in the storage medium.

(5) Except for recorders powered solely by the engine-driven electrical

generator system, there is an automatic means to simultaneously stop a

recorder that has a data erasure feature and prevent each erasure feature

from functioning, within 10 minutes after crash impact; and

(6) There is a means to record data from which the time of each radio

transmission either to or from ATC can be determined.

(b) Each nonejectable record container must be located and mounted so as to

minimize the probability of container rupture resulting from crash impact and

subsequent damage to the record from fire. In meeting this requirement the

record container must be located as far aft as practicable, but need not be

aft of the pressurized compartment, and may not be where aft-mounted engines

may crush the container upon impact.

(c) A correlation must be established between the flight recorder readings

of airspeed, altitude, and heading and the corresponding readings (taking

into account correction factors) of the first pilot's instruments. The

correlation must cover the airspeed range over which the airplane is to be

operated, the range of altitude to which the airplane is limited, and 360

degrees of heading. Correlation may be established on the ground as

appropriate.

(d) Each recorder container must--

(1) Be either bright orange or bright yellow;

(2) Have reflective tape affixed to its external surface to facilitate its

location under water; and

(3) Have an underwater locating device, when required by the operating

rules of this chapter, on or adjacent to the container which is secured in

such a manner that they are not likely to be separated during crash impact.

(e) Any novel or unique design or operational characteristics of the

aircraft shall be evaluated to determine if any dedicated parameters must be

recorded on flight recorders in addition to or in place of existing

requirements.

[Amdt. 25-8, 31 FR 127, Jan. 6, 1966, as amended by Amdt. 25-25, 35 FR 13192,

Aug. 19, 1970; Amdt. 25-37, 40 FR 2577, Jan. 14, 1975; Amdt. 25-41, 42 FR

36971, July 18, 1977; Amdt. 25-65, 53 FR 26144, July 11, 1988]

Sec. 25.1461  Equipment containing high energy rotors.

(a) Equipment containing high energy rotors must meet paragraph (b), (c),

or (d) of this section.

(b) High energy rotors contained in equipment must be able to withstand

damage caused by malfunctions, vibration, abnormal speeds, and abnormal

temperatures. In addition--

(1) Auxiliary rotor cases must be able to contain damage caused by the

failure of high energy rotor blades; and

(2) Equipment control devices, systems, and instrumentation must reasonably

ensure that no operating limitations affecting the integrity of high energy

rotors will be exceeded in service.

(c) It must be shown by test that equipment containing high energy rotors

can contain any failure of a high energy rotor that occurs at the highest

speed obtainable with the normal speed control devices inoperative.

(d) Equipment containing high energy rotors must be located where rotor

failure will neither endanger the occupants nor adversely affect continued

safe flight.

[Amdt. 25-41, 42 FR 36971, July 18, 1977]

Subpart G--Operating Limitations and Information

Sec. 25.1501  General.

(a) Each operating limitation specified in Secs. 25.1503 through 25.1533

and other limitations and information necessary for safe operation must be

established.

(b) The operating limitations and other information necessary for safe

operation must be made available to the crewmembers as prescribed in Secs.

25.1541 through 25.1587.

[Amdt. 25-42, 43 FR 2323, Jan. 16, 1978]

Operating Limitations

Sec. 25.1503  Airspeed limitations: general.

When airspeed limitations are a function of weight, weight distribution,

altitude, or Mach number, limitations corresponding to each critical

combination of these factors must be established.

Sec. 25.1505  Maximum operating limit speed.

The maximum operating limit speed (VMO/MMO airspeed or Mach Number,

whichever is critical at a particular altitude) is a speed that may not be

deliberately exceeded in any regime of flight (climb, cruise, or descent),

unless a higher speed is authorized for flight test or pilot training

operations. VMO/MMO must be established so that it is not greater than the

design cruising speed VC and so that it is sufficiently below VD/MD or VDF/

MDF, to make it highly improbable that the latter speeds will be

inadvertently exceeded in operations. The speed margin between VMO/MMO and

VD/MD or VDFM/DF may not be less than that determined under Sec. 25.335(b) or

found necessary during the flight tests conducted under Sec. 25.253.

[Amdt. 25-23, 35 FR 5680, Apr. 8, 1970]

Sec. 25.1507  Maneuvering speed.

The maneuvering speed must be established so that it does not exceed the

design maneuvering speed VA determined under Sec. 25.335(c).

Sec. 25.1511  Flap extended speed.

The established flap extended speed VFE must be established so that it does

not exceed the design flap speed VF chosen under Secs. 25.335(e) and 25.345,

for the corresponding flap positions and engine powers.

Sec. 25.1513  Minimum control speed.

The minimum control speed VMC determined under Sec. 25.149 must be

established as an operating limitation.

Sec. 25.1515  Landing gear speeds.

(a) The established landing gear operating speed or speeds, VLO, may not

exceed the speed at which it is safe both to extend and to retract the

landing gear, as determined under Sec. 25.729 or by flight characteristics.

If the extension speed is not the same as the retraction speed, the two

speeds must be designated as VLO(EXT) and VLO(RET), respectively.

(b) The established landing gear extended speed VLE may not exceed the

speed at which it is safe to fly with the landing gear secured in the fully

extended position, and that determined under Sec. 25.729.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55468, Dec. 20, 1976]

Sec. 25.1519  Weight, center of gravity, and weight distribution.

The airplane weight, center of gravity, and weight distribution limitations

determined under Secs. 25.23 through 25.27 must be established as operating

limitations.

Sec. 25.1521  Powerplant limitations.

(a) General. The powerplant limitations prescribed in this section must be

established so that they do not exceed the corresponding limits for which the

engines or propellers are type certificated and do not exceed the values on

which compliance with any other requirement of this part is based.

(b) Reciprocating engine installations. Operating limitations relating to

the following must be established for reciprocating engine installations:

(1) Horsepower or torque, r.p.m., manifold pressure, and time at critical

pressure altitude and sea level pressure altitude for--

(i) Maximum continuous power (relating to unsupercharged operation or to

operation in each supercharger mode as applicable); and

(ii) Takeoff power (relating to unsupercharged operation or to operation in

each supercharger mode as applicable).

(2) Fuel grade or specification.

(3) Cylinder head and oil temperatures.

(4) Any other parameter for which a limitation has been established as part

of the engine type certificate except that a limitation need not be

established for a parameter that cannot be exceeded during normal operation

due to the design of the installation or to another established limitation.

(c) Turbine engine installations. Operating limitations relating to the

following must be established for turbine engine installations:

(1) Horsepower, torque or thrust, r.p.m., gas temperature, and time for--

(i) Maximum continuous power or thrust (relating to augmented or

unaugmented operation as applicable).

(ii) Takeoff power or thrust (relating to augmented or unaugmented

operation as applicable).

(2) Fuel designation or specification.

(3) Any other parameter for which a limitation has been established as part

of the engine type certificate except that a limitation need not be

established for a parameter that cannot be exceeded during normal operation

due to the design of the installation or to another established limitation.

(d) Ambient temperature. An ambient temperature limitation (including

limitations for winterization installations, if applicable) must be

established as the maximum ambient atmospheric temperature established in

accordance with Sec. 25.1043(b).

[Doc. No. 24344, Amdt. 25-72, 55 FR 29786, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1522  Auxiliary power unit limitations.

If an auxiliary power unit is installed in the airplane, limitations

established for the auxiliary power unit, including categories of operation,

must be specified as operating limitations for the airplane.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29786, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1523  Minimum flight crew.

The minimum flight crew must be established so that it is sufficient for

safe operation, considering--

(a) The workload on individual crewmembers;

(b) The accessibility and ease of operation of necessary controls by the

appropriate crewmember; and

(c) The kind of operation authorized under Sec. 25.1525.

The criteria used in making the determinations required by this section are

set forth in Appendix D.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-3, 30 FR

6067, Apr. 29, 1965]

Sec. 25.1525  Kinds of operation.

The kinds of operation to which the airplane is limited are established by

the category in which it is eligible for certification and by the installed

equipment.

Sec. 25.1527  Maximum operating altitude.

The maximum altitude up to which operation is allowed, as limited by

flight, structural, powerplant, functional, or equipment characteristics,

must be established.

Sec. 25.1529  Instructions for Continued Airworthiness.

The applicant must prepare Instructions for Continued Airworthiness in

accordance with Appendix H to this part that are acceptable to the

Administrator. The instructions may be incomplete at type certification if a

program exists to ensure their completion prior to delivery of the first

airplane or issuance of a standard certificate of airworthiness, whichever

occurs later.

[Amdt. 25-54, 45 FR 60173, Sept. 11, 1980]

Sec. 25.1531  Maneuvering flight load factors.

Load factor limitations, not exceeding the positive limit load factors

determined from the maneuvering diagram in Sec. 25.333(b), must be

established.

Sec. 25.1533  Additional operating limitations.

(a) Additional operating limitations must be established as follows:

(1) The maximum takeoff weights must be established as the weights at which

compliance is shown with the applicable provisions of this part (including

the takeoff climb provisions of Sec. 25.121(a) through (c), for altitudes and

ambient temperatures).

(2) The maximum landing weights must be established as the weights at which

compliance is shown with the applicable provisions of this part (including

the landing and approach climb provisions of Secs. 25.119 and 25.121(d) for

altitudes and ambient temperatures).

(3) The minimum takeoff distances must be established as the distances at

which compliance is shown with the applicable provisions of this part

(including the provisions of Secs. 25.103 and 25.113, for weights, altitudes,

temperatures, wind components, and runway gradients).

(b) The extremes for variable factors (such as altitude, temperature, wind,

and runway gradients) are those at which compliance with the applicable

provisions of this part is shown.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55468, Dec. 20, 1976; Amdt. 25-72, 55 FR 29786, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Markings and Placards

Sec. 25.1541  General.

(a) The airplane must contain--

(1) The specified markings and placards; and

(2) Any additional information, instrument markings, and placards required

for the safe operation if there are unusual design, operating, or handling

characteristics.

(b) Each marking and placard prescribed in paragraph (a) of this section--

(1) Must be displayed in a conspicuous place; and

(2) May not be easily erased, disfigured, or obscured.

Sec. 25.1543  Instrument markings: general.

For each instrument--

(a) When markings are on the cover glass of the instrument, there must be

means to maintain the correct alignment of the glass cover with the face of

the dial; and

(b) Each instrument marking must be clearly visible to the appropriate

crewmember.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-72, 55

FR 29786, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1545  Airspeed limitation information.

The airspeed limitations required by Sec. 25.1583 (a) must be easily read

and understood by the flight crew.

Sec. 25.1547  Magnetic direction indicator.

(a) A placard meeting the requirements of this section must be installed

on, or near, the magnetic direction indicator.

(b) The placard must show the calibration of the instrument in level flight

with the engines operating.

(c) The placard must state whether the calibration was made with radio

receivers on or off.

(d) Each calibration reading must be in terms of magnetic heading in not

more than 45 degree increments.

Sec. 25.1549  Powerplant and auxiliary power unit instruments.

For each required powerplant and auxiliary power unit instrument, as

appropriate to the type of instrument--

(a) Each maximum and, if applicable, minimum safe operating limit must be

marked with a red radial or a red line;

(b) Each normal operating range must be marked with a green arc or green

line, not extending beyond the maximum and minimum safe limits;

(c) Each takeoff and precautionary range must be marked with a yellow arc

or a yellow line; and

(d) Each engine, auxiliary power unit, or propeller speed range that is

restricted because of excessive vibration stresses must be marked with red

arcs or red lines.

[Amdt. 25-40, 42 FR 15044, Mar. 17, 1977]

Sec. 25.1551  Oil quantity indication.

Each oil quantity indicating means must be marked to indicate the quantity

of oil readily and accurately.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29786, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1553  Fuel quantity indicator.

If the unusable fuel supply for any tank exceeds one gallon, or five

percent of the tank capacity, whichever is greater, a red arc must be marked

on its indicator extending from the calibrated zero reading to the lowest

reading obtainable in level flight.

Sec. 25.1555  Control markings.

(a) Each cockpit control, other than primary flight controls and controls

whose function is obvious, must be plainly marked as to its function and

method of operation.

(b) Each aerodynamic control must be marked under the requirements of Secs.

25.677 and 25.699.

(c) For powerplant fuel controls--

(1) Each fuel tank selector control must be marked to indicate the position

corresponding to each tank and to each existing cross feed position;

(2) If safe operation requires the use of any tanks in a specific sequence,

that sequence must be marked on, or adjacent to, the selector for those

tanks; and

(3) Each valve control for each engine must be marked to indicate the

position corresponding to each engine controlled.

(d) For accessory, auxiliary, and emergency controls--

(1) Each emergency control (including each fuel jettisoning and fluid

shutoff must be colored red; and

(2) Each visual indicator required by Sec. 25.729(e) must be marked so that

the pilot can determine at any time when the wheels are locked in either

extreme position, if retractable landing gear is used.

Sec. 25.1557  Miscellaneous markings and placards.

(a) Baggage and cargo compartments and ballast location. Each baggage and

cargo compartment, and each ballast location must have a placard stating any

limitations on contents, including weight, that are necessary under the

loading requirements. However, underseat compartments designed for the

storage of carry-on articles weighing not more than 20 pounds need not have a

loading limitation placard.

(b) Powerplant fluid filler openings. The following apply:

(1) Fuel filler openings must be marked at or near the filler cover with--

(i) The word "fuel";

(ii) For reciprocating engine powered airplanes, the minimum fuel grade;

(iii) For turbine engine powered airplanes, the permissible fuel

designations; and

(iv) For pressure fueling systems, the maximum permissible fueling supply

pressure and the maximum permissible defueling pressure.

(2) Oil filler openings must be marked at or near the filler cover with the

word "oil".

(3) Augmentation fluid filler openings must be marked at or near the filler

cover to identify the required fluid.

(c) Emergency exit placards. Each emergency exit placard must meet the

requirements of Sec. 25.811.

(d) Doors. Each door that must be used in order to reach any required

emergency exit must have a suitable placard stating that the door is to be

latched in the open position during takeoff and landing.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-32, 37 FR

3972, Feb. 24, 1972; Amdt. 25-38, 41 FR 55468, Dec. 20, 1976; Amdt. 25-72, 55

FR 29786, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1561  Safety equipment.

(a) Each safety equipment control to be operated by the crew in emergency,

such as controls for automatic liferaft releases, must be plainly marked as

to its method of operation.

(b) Each location, such as a locker or compartment, that carries any fire

extinguishing, signaling, or other life saving equipment must be marked

accordingly.

(c) Stowage provisions for required emergency equipment must be

conspicuously marked to identify the contents and facilitate the easy removal

of the equipment.

(d) Each liferaft must have obviously marked operating instructions.

(e) Approved survival equipment must be marked for identification and

method of operation.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-46, 43 FR

50598, Oct. 30, 1978]

Sec. 25.1563  Airspeed placard.

A placard showing the maximum airspeeds for flap extension for the takeoff,

approach, and landing positions must be installed in clear view of each

pilot.

Airplane Flight Manual

Sec. 25.1581  General.

(a) Furnishing information. An Airplane Flight Manual must be furnished

with each airplane, and it must contain the following:

(1) Information required by Secs. 25.1583 through 25.1587.

(2) Other information that is necessary for safe operation because of

design, operating, or handling characteristics.

(3) Any limitation, procedure, or other information established as a

condition of compliance with the applicable noise standards of part 36 of

this chapter.

(b) Approved information. Each part of the manual listed in Secs. 25.1583

through 25.1587, that is appropriate to the airplane, must be furnished,

verified, and approved, and must be segregated, identified, and clearly

distinguished from each unapproved part of that manual.

(c) [Reserved]

(d) Each Airplane Flight Manual must include a table of contents if the

complexity of the manual indicates a need for it.

[Amdt. 25-42, 43 FR 2323, Jan. 16, 1978, as amended by Amdt. 25-72, 55 FR

29786, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1583  Operating limitations.

(a) Airspeed limitations. The following airspeed limitations and any other

airspeed limitations necessary for safe operation must be furnished:

(1) The maximum operating limit speed VMO/MMO and a statement that this

speed limit may not be deliberately exceeded in any regime of flight (climb,

cruise, or descent) unless a higher speed is authorized for flight test or

pilot training.

(2) If an airspeed limitation is based upon compressibility effects, a

statement to this effect and information as to any symptoms, the probable

behavior of the airplane, and the recommended recovery procedures.

(3) The maneuvering speed VA and a statement that full application of

rudder and aileron controls, as well as maneuvers that involve angles of

attack near the stall, should be confined to speeds below this value.

(4) The flap extended speed VFE and the pertinent flap positions and engine

powers.

(5) The landing gear operating speed or speeds, and a statement explaining

the speeds as defined in Sec. 25.1515(a).

(6) The landing gear extended speed VLE, if greater than VLO, and a

statement that this is the maximum speed at which the airplane can be safely

flown with the landing gear extended.

(b) Powerplant limitations. The following information must be furnished:

(1) Limitations required by Sec. 25.1521 and Sec. 25.1522.

(2) Explanation of the limitations, when appropriate.

(3) Information necessary for marking the instruments required by Secs.

25.1549 through 25.1553.

(c) Weight and loading distribution. The weight and center of gravity

limits required by Secs. 25.25 and 25.27 must be furnished in the Airplane

Flight Manual. All of the following information must be presented either in

the Airplane Flight Manual or in a separate weight and balance control and

loading document which is incorporated by reference in the Airplane Flight

Manual:

(1) The condition of the airplane and the items included in the empty

weight as defined in accordance with Sec. 25.29.

(2) Loading instructions necessary to ensure loading of the airplane within

the weight and center of gravity limits, and to maintain the loading within

these limits in flight.

(3) If certification for more than one center of gravity range is

requested, the appropriate limitations, with regard to weight and loading

procedures, for each separate center of gravity range.

(d) Flight crew. The number and functions of the minimum flight crew

determined under Sec. 25.1523 must be furnished.

(e) Kinds of operation. The kinds of operation approved under Sec. 25.1525

must be furnished.

(f) Altitudes. The altitude established under Sec. 25.1527.

(g) [Reserved]

(h) Additional operating limitations. The operating limitations established

under Sec.25.1533 must be furnished.

(i) Maneuvering flight load factors. The positive maneuvering limit load

factors for which the structure is proven, described in terms of

accelerations, must be furnished.

[Doc. No. 5066, 29 FR 1891, Dec. 24, 1964, as amended by Amdt. 25-38, 41 FR

55468, Dec, 20, 1976; Amdt. 25-42, 43 FR 2323, Jan. 16, 1978; Amdt. 25-46, 43

FR 50598, Oct. 30, 1978; Amdt. 25-72, 55 FR 29787, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Sec. 25.1585  Operating procedures.

(a) Information and instructions regarding the peculiarities of normal

operations (including starting and warming the engines, taxiing, operation of

wing flaps, landing gear, and the automatic pilot) must be furnished,

together with recommended procedures for--

(1) Engine failure (including minimum speeds, trim, operation of the

remaining engines, and operation of flaps);

(2) Stopping the rotation of propellers in flight;

(3) Restarting turbine engines in flight (including the effects of

altitude);

(4) Fire, decompression, and similar emergencies;

(5) Ditching (including the procedures based on the requirements of Secs.

25.801, 25.807(d), 25.1411, and 25.1415 (a) through (e));

(6) Use of ice protection equipment;

(7) Use of fuel jettisoning equipment, including any operating precautions

relevant to the use of the system;

(8) Operation in turbulence for turbine powered airplanes (including

recommended turbulence penetration airspeeds, flight peculiarities, and

special control instructions);

(9) Restoring a deployed thrust reverser intended for ground operation only

to the forward thrust position in fight or continuing fight and landing with

the thrust reverser in any position except forward thrust; and

(10) Disconnecting the battery from its charging source, if compliance is

shown with Sec. 25.1353 (c)(6)(ii) or (c)(6)(iii).

(b) Information identifying each operating condition in which the fuel

system independence prescribed in Sec. 25.953 is necessary for safety must be

furnished, together with instructions for placing the fuel system in a

configuration used to show compliance with that section.

(c) The buffet onset envelopes determined under Sec. 25.251 must be

furnished. The buffet onset envelopes presented may reflect the center of

gravity at which the airplane is normally loaded during cruise if corrections

for the effect of different center of gravity locations are furnished.

(d) Information must be furnished which indicates that when the fuel

quantity indicator reads "zero" in level flight, any fuel remaining in the

fuel tank cannot be used safely in flight.

(e) Information on the total quantity of usable fuel for each fuel tank

must be furnished.

[Doc. No. 5066, 29 FR 18291, Dec. 24, 1964, as amended by Amdt. 25-11, 32 FR

6913, May 5, 1967; Amdt. 25-23, 35 FR 5680, Apr. 8, 1970; Amdt. 25-40, 42 FR

15044, Mar. 17, 1977; Amdt. 25-42, 43 FR 2323, Jan 16, 1978; Amdt. 25-46, 43

FR 50598, Oct. 30, 1978]

Sec. 25.1587  Performance information.

(a) Each Airplane Flight Manual must contain information to permit

conversion of the indicated temperature to free air temperature if other than

a free air temperature indicator is used to comply with the requirements of

Sec. 25.1303(a)(1).

(b) Each Airplane Flight Manual must contain the performance information

computed under the applicable provisions of this part for the weights,

altitudes, temperatures, wind components, and runway gradients, as

applicable, within the operational limits of the airplane, and must contain

the following:

(1) The conditions under which the performance information was obtained,

including the speeds associated with the performance information.

(2) Vs determined in accordance with Sec. 25.103.

(3) The following performance information (determined by extrapolation and

computed for the range of weights between the maximum landing and maximum

takeoff weights):

(i) Climb in the landing configuration.

(ii) Climb in the approach configuration.

(iii) Landing distance.

(4) Procedures established under Sec. 25.101 (f), (g), and (h) that are

related to the limitations and information required by Sec. 25.1533 and by

this paragraph. These procedures must be in the form of guidance material,

including any relevant limitations or information.

(5) An explanation of significant or unusual flight or ground handling

characteristics of the airplane.

[Amdt. 25-42, 43 FR 2324, Jan. 16, 1978, as amended by Amdt. 25-72, 55 FR

29787, July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Appendix A

Figure 1 -- Basic Landing Gear Dimension Data

[ ...Illustration appears here... ]

Figure 2 --Level Landing

[ ...Illustration appears here... ]

Figure 3 -- Tail-Down Landing

[ ...Illustration appears here... ]

Figure 5 -- Lateral Drift Landing

[ ...Illustration appears here... ]

Figure 6 -- Braked Roll

[ ...Illustration appears here... ]

Figure 7 -- Ground Turning

[ ...Illustration appears here... ]

Figure 8 -- Pivoting, Nose or Tail Wheel Type

[ ...Illustration appears here... ]

Appendix B

Figure 1 -- Pictorial Definition of Angles, Dimensions, and

Directions On a Seaplane

[ ...Illustration appears here... ]

Figure 2 -- Hull Station Weighing Factor

[ ...Illustration appears here... ]

Figure 3 -- Transverse Pressure Distributions

[ ...Illustration appears here...]

Appendix C to Part 25

(a) Continuous maximum icing. The maximum continuous intensity of

atmospheric icing conditions (continuous maximum icing) is defined by the

variables of the cloud liquid water content, the mean effective diameter of

the cloud droplets, the ambient air temperature, and the interrelationship of

these three variables as shown in figure 1 of this appendix. The limiting

icing envelope in terms of altitude and temperature is given in figure 2 of

this appendix. The inter-relationship of cloud liquid water content with drop

diameter and altitude is determined from figures 1 and 2. The cloud liquid

water content for continuous maximum icing conditions of a horizontal extent,

other than 17.4 nautical miles, is determined by the value of liquid water

content of figure 1, multiplied by the appropriate factor from figure 3 of

this appendix.

(b) Intermittent maximum icing. The intermittent maximum intensity of

atmospheric icing conditions (intermittent maximum icing) is defined by the

variables of the cloud liquid water content, the mean effective diameter of

the cloud droplets, the ambient air temperature, and the interrelationship of

these three variables as shown in figure 4 of this appendix. The limiting

icing envelope in terms of altitude and temperature is given in figure 5 of

this appendix. The inter-relationship of cloud liquid water content with drop

diameter and altitude is determined from figures 4 and 5. The cloud liquid

water content for intermittent maximum icing conditions of a horizontal

extent, other than 2.6 nautical miles, is determined by the value of cloud

liquid water content of figure 4 multiplied by the appropriate factor in

figure 6 of this appendix.

Figures 1-3-- Continuous Maximum (Stratiform Clouds)

[ illustrations appear below ]

Figures 4-6-- Intermittent Maximum (Cumuliform Clouds)

[ illustrations appear below ]

Appendix D to Part 25

Criteria for determining minimum flight crew. The following are considered

by the Agency in determining the minimum flight crew under Sec. 25.1523:

(a) Basic workload functions. The following basic workload functions are

considered:

(1) Flight path control.

(2) Collision avoidance.

(3) Navigation.

(4) Communications.

(5) Operation and monitoring of aircraft engines and systems.

(6) Command decisions.

(b) Workload factors. The following workload factors are considered

significant when analyzing and demonstrating workload for minimum flight crew

determination:

(1) The accessibility, ease, and simplicity of operation of all necessary

flight, power, and equipment controls, including emergency fuel shutoff

valves, electrical controls, electronic controls, pressurization system

controls, and engine controls.

(2) The accessibility and conspicuity of all necessary instruments and

failure warning devices such as fire warning, electrical system malfunction,

and other failure or caution indicators. The extent to which such instruments

or devices direct the proper corrective action is also considered.

(3) The number, urgency, and complexity of operating procedures with

particular consideration given to the specific fuel management schedule

imposed by center of gravity, structural or other considerations of an

airworthiness nature, and to the ability of each engine to operate at all

times from a single tank or source which is automatically replenished if fuel

is also stored in other tanks.

(4) The degree and duration of concentrated mental and physical effort

involved in normal operation and in diagnosing and coping with malfunctions

and emergencies.

(5) The extent of required monitoring of the fuel, hydraulic,

pressurization, electrical, electronic, deicing, and other systems while en

route.

(6) The actions requiring a crewmember to be unavailable at his assigned

duty station, including: observation of systems, emergency operation of any

control, and emergencies in any compartment.

(7) The degree of automation provided in the aircraft systems to afford

(after failures or malfunctions) automatic crossover or isolation of

difficulties to minimize the need for flight crew action to guard against

loss of hydraulic or electric power to flight controls or to other essential

systems.

(8) The communications and navigation workload.

(9) The possibility of increased workload associated with any emergency

that may lead to other emergencies.

(10) Incapacitation of a flight crewmember whenever the applicable

operating rule requires a minimum flight crew of at least two pilots.

(c) Kind of operation authorized. The determination of the kind of

operation authorized requires consideration of the operating rules under

which the airplane will be operated. Unless an applicant desires approval for

a more limited kind of operation. It is assumed that each airplane

certificated under this Part will operate under IFR conditions.

[Amdt. 25-3, 30 FR 6067, Apr. 29, 1965]

Appendix E to Part 25

I--Limited Weight Credit For Airplanes Equipped With Standby Power

(a) Each applicant for an increase in the maximum certificated takeoff and

landing weights of an airplane equipped with a type-certificated standby

power rocket engine may obtain an increase as specified in paragraph (b) if--

(1) The installation of the rocket engine has been approved and it has been

established by flight test that the rocket engine and its controls can be

operated safely and reliably at the increase in maximum weight; and

(2) The Airplane Flight Manual, or the placard, markings or manuals

required in place thereof, set forth in addition to any other operating

limitations the Administrator may require, the increased weight approved

under this regulation and a prohibition against the operation of the airplane

at the approved increased weight when--

(i) The installed standby power rocket engines have been stored or

installed in excess of the time limit established by the manufacturer of the

rocket engine (usually stenciled on the engine casing); or

(ii) The rocket engine fuel has been expended or discharged.

(b) The currently approved maximum takeoff and landing weights at which an

airplane is certificated without a standby power rocket engine installation

may be increased by an amount that does not exceed any of the following:

(1) An amount equal in pounds to 0.014 IN, where I is the maximum usable

impulse in pounds-seconds available from each standby power rocket engine and

N is the number of rocket engines installed.

(2) An amount equal to 5 percent of the maximum certificated weight

approved in accordance with the applicable airworthiness regulations without

standby power rocket engines installed.

(3) An amount equal to the weight of the rocket engine installation.

(4) An amount that, together with the currently approved maximum weight,

would equal the maximum structural weight established for the airplane

without standby rocket engines installed.

II--Performance Credit for Transport Category Airplanes Equipped With Standby

Power

The Administrator may grant performance credit for the use of standby power

on transport category airplanes. However, the performance credit applies only

to the maximum certificated takeoff and landing weights, the takeoff

distance, and the takeoff paths, and may not exceed that found by the

Administrator to result in an overall level of safety in the takeoff,

approach, and landing regimes of flight equivalent to that prescribed in the

regulations under which the airplane was originally certificated without

standby power. For the purposes of this Appendix, "standby power" is power or

thrust, or both, obtained from rocket engines for a relatively short period

and actuated only in cases of emergency. The following provisions apply:

(1) Takeoff; general. The takeoff data prescribed in paragraphs (2) and (3)

of this Appendix must be determined at all weights and altitudes, and at

ambient temperatures if applicable, at which performance credit is to be

applied.

(2) Takeoff path.

(a) The one-engine-inoperative takeoff path with standby power in use must

be determined in accordance with the performance requirements of the

applicable airworthiness regulations.

(b) The one-engine-inoperative takeoff path (excluding that part where the

airplane is on or just above the takeoff surface) determined in accordance

with paragraph (a) of this section must lie above the one-engine-inoperative

takeoff path without standby power at the maximum takeoff weight at which all

of the applicable air-worthiness requirements are met. For the purpose of

this comparison, the flight path is considered to extend to at least a height

of 400 feet above the takeoff surface.

(c) The takeoff path with all engines operating, but without the use of

standby power, must reflect a conservatively greater overall level of

performance than the one-engine-inoperative takeoff path established in

accordance with paragraph (a) of this section. The margin must be established

by the Administrator to insure safe day-to-day operations, but in no case may

it be less than 15 percent. The all-engines-operating takeoff path must be

determined by a procedure consistent with that established in complying with

paragraph (a) of this section.

(d) For reciprocating-engine-powered airplanes, the takeoff path to be

scheduled in the Airplane Flight Manual must represent the one-engine-

operative takeoff path determined in accordance with paragraph (a) of this

section and modified to reflect the procedure (see paragraph (6)) established

by the applicant for flap retraction and attainment of the en route speed.

The scheduled takeoff path must have a positive slope at all points of the

airborne portion and at no point must it lie above the takeoff path specified

in paragraph (a) of this section.

(3) Takeoff distance. The takeoff distance must be the horizontal distance

along the one-engine-inoperative take off path determined in accordance with

paragraph (2)(a) from the start of the takeoff to the point where the

airplane attains a height of 50 feet above the takeoff surface for

reciprocating-engine-powered airplanes and a height of 35 feet above the

takeoff surface for turbine-powered airplanes.

(4) Maximum certificated takeoff weights. The maximum certificated takeoff

weights must be determined at all altitudes, and at ambient temperatures, if

applicable, at which performance credit is to be applied and may not exceed

the weights established in compliance with paragraphs (a) and (b) of this

section.

(a) The conditions of paragraphs (2) (b) through (d) must be met at the

maximum certificated takeoff weight.

(b) Without the use of standby power, the airplane must meet all of the en

route requirements of the applicable airworthiness regulations under which

the airplane was originally certificated. In addition, turbine-powered

airplanes without the use of standby power must meet the final takeoff climb

requirements prescribed in the applicable airworthiness regulations.

(5) Maximum certificated landing weights.

(a) The maximum certificated landing weights (one-engine-inoperative

approach and all-engine-operating landing climb) must be determined at all

altitudes, and at ambient temperatures if applicable, at which performance

credit is to be applied and must not exceed that established in compliance

with paragraph (b) of this section.

(b) The flight path, with the engines operating at the power or thrust, or

both, appropriate to the airplane configuration and with standby power in

use, must lie above the flight path without standby power in use at the

maximum weight at which all of the applicable airworthiness requirements are

met. In addition, the flight paths must comply with subparagraphs (i) and

(ii) of this paragraph.

(i) The flight paths must be established without changing the appropriate

airplane configuration.

(ii) The flight paths must be carried out for a minimum height of 400 feet

above the point where standby power is actuated.

(6) Airplane configuration, speed, and power and thrust; general. Any

change in the airplane's configuration, speed, and power or thrust, or both,

must be made in accordance with the procedures established by the applicant

for the operation of the airplane in service and must comply with paragraphs

(a) through (c) of this section. In addition, procedures must be established

for the execution of balked landings and missed approaches.

(a) The Administrator must find that the procedure can be consistently

executed in service by crews of average skill.

(b) The procedure may not involve methods or the use of devices which have

not been proven to be safe and reliable.

(c) Allowances must be made for such time delays in the execution of the

procedures as may be reasonably expected to occur during service.

(7) Installation and operation; standby power. The standby power unit and

its installation must comply with paragraphs (a) and (b) of this section.

(a) The standby power unit and its installation must not adversely affect

the safety of the airplane.

(b) The operation of the standby power unit and its control must have

proven to be safe and reliable.

[Amdt. 25-6, 30 FR 8468, July 2, 1965]

Appendix F to Part 25

Part I--Test Criteria and Procedures for Showing Compliance with Sec. 25.853,

or 25.855.

(a) Material test criteria--(1) Interior compartments occupied by crew or

passengers. (i) Interior ceiling panels, interior wall panels, partitions,

galley structure, large cabinet walls, structural flooring, and materials

used in the construction of stowage compartments (other than underseat

stowage compartments and compartments for stowing small items such as

magazines and maps) must be self-extinguishing when tested vertically in

accordance with the applicable portions of part I of this appendix. The

average burn length may not exceed 6 inches and the average flame time after

removal of the flame source may not exceed 15 seconds. Drippings from the

test specimen may not continue to flame for more than an average of 3 seconds

after falling.

(ii) Floor covering, textiles (including draperies and upholstery), seat

cushions, padding, decorative and nondecorative coated fabrics, leather,

trays and galley furnishings, electrical conduit, thermal and acoustical

insulation and insulation covering, air ducting, joint and edge covering,

liners of Class B and E cargo or baggage compartments, floor panels of Class

B, C, D, or E cargo or baggage compartments, insulation blankets, cargo

covers and transparencies, molded and thermoformed parts, air ducting joints,

and trim strips (decorative and chafing), that are constructed of materials

not covered in subparagraph (iv) below, must be self-extinguishing when

tested vertically in accordance with the applicable portions of part I of

this appendix or other approved equivalent means. The average burn length may

not exceed 8 inches, and the average flame time after removal of the flame

source may not exceed 15 seconds. Drippings from the test specimen may not

continue to flame for more than an average of 5 seconds after falling.

(iii) Motion picture film must be safety film meeting the Standard

Specifications for Safety Photographic Film PHI.25 (available from the

American National Standards Institute, 1430 Broadway, New York, NY 10018). If

the film travels through ducts, the ducts must meet the requirements of

subparagraph (ii) of this paragraph.

(iv) Clear plastic windows and signs, parts constructed in whole or in part

of elastomeric materials, edge lighted instrument assemblies consisting of

two or more instruments in a common housing, seat belts, shoulder harnesses,

and cargo and baggage tiedown equipment, including containers, bins, pallets,

etc., used in passenger or crew compartments, may not have an average burn

rate greater than 2.5 inches per minute when tested horizontally in

accordance with the applicable portions of this appendix.

(v) Except for small parts (such as knobs, handles, rollers, fasteners,

clips, grommets, rub strips, pulleys, and small electrical parts) that would

not contribute significantly to the propagation of a fire and for electrical

wire and cable insulation, materials in items not specified in paragraphs

(a)(1) (i), (ii), (iii), or (iv) of part I of this appendix may not have a

burn rate greater than 4.0 inches per minute when tested horizontally in

accordance with the applicable portions of this appendix.

(2) Cargo and baggage compartments not occupied by crew or passengers.

(i) Thermal and acoustic insulation (including coverings) used in each

cargo and baggage compartment must be constructed of materials that meet the

requirements set forth in paragraph (a)(1)(ii) of part I of this appendix.

(ii) A cargo or baggage compartment defined in Sec. 25.857 as Class B or E

must have a liner constructed of materials that meet the requirements of

paragraph (a)(1)(ii) of part I of this appendix and separated from the

airplane structure (except for attachments). In addition, such liners must be

subjected to the 45 degree angle test. The flame may not penetrate (pass

through) the material during application of the flame or subsequent to its

removal. The average flame time after removal of the flame source may not

exceed 15 seconds, and the average glow time may not exceed 10 seconds.

(iii) A cargo or baggage compartment defined in Sec. 25.857 as Class B, C,

D, or E must have floor panels constructed of materials which meet the

requirements of paragraph (a)(1)(ii) of part I of this appendix and which are

separated from the airplane structure (except for attachments). Such panels

must be subjected to the 45 degree angle test. The flame may not penetrate

(pass through) the material during application of the flame or subsequent to

its removal. The average flame time after removal of the flame source may not

exceed 15 seconds, and the average glow time may not exceed 10 seconds.

(iv) Insulation blankets and covers used to protect cargo must be

constructed of materials that meet the requirements of paragraph (a)(1)(ii)

of part I of this appendix. Tiedown equipment (including containers, bins,

and pallets) used in each cargo and baggage compartment must be constructed

of materials that meet the requirements of paragraph (a)(1)(v) of part I of

this appendix.

(3) Electrical system components. Insulation on electrical wire or cable

installed in any area of the fuselage must be self-extinguishing when

subjected to the 60 degree test specified in part I of this appendix. The

average burn length may not exceed 3 inches, and the average flame time after

removal of the flame source may not exceed 30 seconds. Drippings from the

test specimen may not continue to flame for more than an average of 3 seconds

after falling.

(b) Test Procedures--(1) Conditioning. Specimens must be conditioned to

70+/-5 F., and at 50 percent +/-5 percent relative humidity until moisture

equilibrium is reached or for 24 hours. Each specimen must remain in the

conditioning environment until it is subjected to the flame.

(2) Specimen configuration. Except for small parts and electrical wire and

cable insulation, materials must be tested either as section cut from a

fabricated part as installed in the airplane or as a specimen simulating a

cut section, such as a specimen cut from a flat sheet of the material or a

model of the fabricated part. The specimen may be cut from any location in a

fabricated part; however, fabricated units, such as sandwich panels, may not

be separated for test. Except as noted below, the specimen thickness must be

no thicker than the minimum thickness to be qualified for use in the

airplane. Test specimens of thick foam parts, such as seat cushions, must be

1/2 -inch in thickness. Test specimens of materials that must meet the

requirements of paragraph (a)(1)(v) of part I of this appendix must be no

more than 1/8 -inch in thickness. Electrical wire and cable specimens must be

the same size as used in the airplane. In the case of fabrics, both the warp

and fill direction of the weave must be tested to determine the most critical

flammability condition. Specimens must be mounted in a metal frame so that

the two long edges and the upper edge are held securely during the vertical

test prescribed in subparagraph (4) of this paragraph and the two long edges

and the edge away from the flame are held securely during the horizontal test

prescribed in subparagraph (5) of this paragraph. The exposed area of the

specimen must be at least 2 inches wide and 12 inches long, unless the actual

size used in the airplane is smaller. The edge to which the burner flame is

applied must not consist of the finished or protected edge of the specimen

but must be representative of the actual cross-section of the material or

part as installed in the airplane. The specimen must be mounted in a metal

frame so that all four edges are held securely and the exposed area of the

specimen is at least 8 inches by 8 inches during the 45 deg. test prescribed

in subparagraph (6) of this paragraph.

(3) Apparatus. Except as provided in subparagraph (7) of this paragraph,

tests must be conducted in a draft-free cabinet in accordance with Federal

Test Method Standard 191 Model 5903 (revised Method 5902) for the vertical

test, or Method 5906 for horizontal test (available from the General Services

Administration, Business Service Center, Region 3, Seventh & D Streets SW.,

Washington, DC 20407). Specimens which are too large for the cabinet must be

tested in similar draft-free conditions.

(4) Vertical test. A minimum of three specimens must be tested and results

averaged. For fabrics, the direction of weave corresponding to the most

critical flammability conditions must be parallel to the longest dimension.

Each specimen must be supported vertically. The specimen must be exposed to a

Bunsen or Tirrill burner with a nominal 3/8 -inch I.D. tube adjusted to give

a flame of 1 1/2  inches in height. The minimum flame temperature measured by

a calibrated thermocouple pyrometer in the center of the flame must be 1550

deg.F. The lower edge of the specimen must be 3/4 -inch above the top edge of

the burner. The flame must be applied to the center line of the lower edge of

the specimen. For materials covered by paragraph (a)(1)(i) of part I of this

appendix, the flame must be applied for 60 seconds and then removed. For

materials covered by paragraph (a)(1)(ii) of part I of this appendix, the

flame must be applied for 12 seconds and then removed. Flame time, burn

length, and flaming time of drippings, if any, may be recorded. The burn

length determined in accordance with subparagraph (7) of this paragraph must

be measured to the nearest tenth of an inch.

(5) Horizontal test. A minimum of three specimens must be tested and the

results averaged. Each specimen must be supported horizontally. The exposed

surface, when installed in the aircraft, must be face down for the test. The

specimen must be exposed to a Bunsen or Tirrill burner with a nominal 3/8 -

inch I.D. tube adjusted to give a flame of 1 1/2  inches in height. The

minimum flame temperature measured by a calibrated thermocouple pyrometer in

the center of the flame must be 1550 deg.F. The specimen must be positioned

so that the edge being tested is centered 3/4 -inch above the top of the

burner. The flame must be applied for 15 seconds and then removed. A minimum

of 10 inches of specimen must be used for timing purposes, approximately 1

1/2  inches must burn before the burning front reaches the timing zone, and

the average burn rate must be recorded.

(6) Forty-five degree test. A minimum of three specimens must be tested and

the results averaged. The specimens must be supported at an angle of 45 deg.

to a horizontal surface. The exposed surface when installed in the aircraft

must be face down for the test. The specimens must be exposed to a Bunsen or

Tirrill burner with a nominal 3/8 -inch I.D. tube adjusted to give a flame of

1 1/2  inches in height. The minimum flame temperature measured by a

calibrated thermocouple pyrometer in the center of the flame must be 1550

deg.F. Suitable precautions must be taken to avoid drafts. The flame must be

applied for 30 seconds with one-third contacting the material at the center

of the specimen and then removed. Flame time, glow time, and whether the

flame penetrates (passes through) the specimen must be recorded.

(7) Sixty degree test. A minimum of three specimens of each wire

specification (make and size) must be tested. The specimen of wire or cable

(including insulation) must be placed at an angle of 60 deg. with the

horizontal in the cabinet specified in subparagraph (3) of this paragraph

with the cabinet door open during the test, or must be placed within a

chamber approximately 2 feet high by 1 foot by 1 foot, open at the top and at

one vertical side (front), and which allows sufficient flow of air for

complete combustion, but which is free from drafts. The specimen must be

parallel to and approximately 6 inches from the front of the chamber. The

lower end of the specimen must be held rigidly clamped. The upper end of the

specimen must pass over a pulley or rod and must have an appropriate weight

attached to it so that the specimen is held tautly throughout the

flammability test. The test specimen span between lower clamp and upper

pulley or rod must be 24 inches and must be marked 8 inches from the lower

end to indicate the central point for flame application. A flame from a

Bunsen or Tirrill burner must be applied for 30 seconds at the test mark. The

burner must be mounted underneath the test mark on the specimen,

perpendicular to the specimen and at an angle of 30 deg. to the vertical

plane of the specimen. The burner must have a nominal bore of 3/8 -inch and

be adjusted to provide a 3-inch high flame with an inner cone approximately

one-third of the flame height. The minimum temperature of the hottest portion

of the flame, as measured with a calibrated thermocouple pyrometer, may not

be less than 1750 deg.F. The burner must be positioned so that the hottest

portion of the flame is applied to the test mark on the wire. Flame time,

burn length, and flaming time of drippings, if any, must be recorded. The

burn length determined in accordance with paragraph (8) of this paragraph

must be measured to the nearest tenth of an inch. Breaking of the wire

specimens is not considered a failure.

(8) Burn length. Burn length is the distance from the original edge to the

farthest evidence of damage to the test specimen due to flame impingement,

including areas of partial or complete consumption, charring, or

embrittlement, but not including areas sooted, stained, warped, or

discolored, nor areas where material has shrunk or melted away from the heat

source.

Part II--Flammability of Seat Cushions

(a) Criteria for Acceptance. Each seat cushion must meet the following

criteria:

(1) At least three sets of seat bottom and seat back cushion specimens must

be tested.

(2) If the cushion is constructed with a fire blocking material, the fire

blocking material must completely enclose the cushion foam core material.

(3) Each specimen tested must be fabricated using the principal components

(i.e., foam core, flotation material, fire blocking material, if used, and

dress covering) and assembly processes (representative seams and closures)

intended for use in the production articles. If a different material

combination is used for the back cushion than for the bottom cushion, both

material combinations must be tested as complete specimen sets, each set

consisting of a back cushion specimen and a bottom cushion specimen. If a

cushion, including outer dress covering, is demonstrated to meet the

requirements of this appendix using the oil burner test, the dress covering

of that cushion may be replaced with a similar dress covering provided the

burn length of the replacement covering, as determined by the test specified

in Sec. 25.853(b), does not exceed the corresponding burn length of the dress

covering used on the cushion subjected to the oil burner test.

(4) For at least two-thirds of the total number of specimen sets tested,

the burn length from the burner must not reach the side of the cushion

opposite the burner. The burn length must not exceed 17 inches. Burn length

is the perpendicular distance from the inside edge of the seat frame closest

to the burner to the farthest evidence of damage to the test specimen due to

flame impingement, including areas of partial or complete consumption,

charring, or embrittlement, but not including areas sooted, stained, warped,

or discolored, or areas where material has shrunk or melted away from the

heat source.

(5) The average percentage weight loss must not exceed 10 percent. Also, at

least two-thirds of the total number of specimen sets tested must not exceed

10 percent weight loss. All droppings falling from the cushions and mounting

stand are to be discarded before the after-test weight is determined. The

percentage weight loss for a specimen set is the weight of the specimen set

before testing less the weight of the specimen set after testing expressed as

the percentage of the weight before testing.

(b) Test Conditions. Vertical air velocity should average 25 fpm+/-10 fpm

at the top of the back seat cushion. Horizontal air velocity should be below

10 fpm just above the bottom seat cushion. Air velocities should be measured

with the ventilation hood operating and the burner motor off.

(c) Test Specimens. (1) For each test, one set of cushion specimens

representing a seat bottom and seat back cushion must be used.

(2) The seat bottom cushion specimen must be 18+/- 1/8  inches (457+/-3 mm)

wide by 20+/- 1/8  inches (508+/-3 mm) deep by 4+/- 1/8  inches (102+/-3 mm)

thick, exclusive of fabric closures and seam overlap.

(3) The seat back cushion specimen must be 18+/- 1/8  inches (432+/-3 mm)

wide by 25+/- 1/8  inches (635+/-3 mm) high by 2+/- 1/8  inches (51+/-3 mm)

thick, exclusive of fabric closures and seam overlap.

(4) The specimens must be conditioned at 70+/-5  deg.F (21+/-2  deg.C)

55%+/-10% relative humidity for at least 24 hours before testing.

(d) Test Apparatus. The arrangement of the test apparatus is shown in

Figures 1 through 5 and must include the components described in this

section. Minor details of the apparatus may vary, depending on the model

burner used.

(1) Specimen Mounting Stand. The mounting stand for the test specimens

consists of steel angles, as shown in Figure 1. The length of the mounting

stand legs is 12+/- 1/8  inches (305+/-3 mm). The mounting stand must be used

for mounting the test specimen seat bottom and seat back, as shown in Figure

2. The mounting stand should also include a suitable drip pan lined with

aluminum foil, dull side up.

(2) Test Burner. The burner to be used in testing must--

(i) Be a modified gun type;

(ii) Have an 80-degree spray angle nozzle nominally rated for 2.25 gallons/

hour at 100 psi;

(iii) Have a 12-inch (305 mm) burner cone installed at the end of the draft

tube, with an opening 6 inches (152 mm) high and 11 inches (280 mm) wide, as

shown in Figure 3; and

(iv) Have a burner fuel pressure regulator that is adjusted to deliver a

nominal 2.0 gallon/hour of NZ 2 Grade kerosene or equivalent required for the

test.

Burner models which have been used successfully in testing are the Lennox

Model OB-32, Carlin Model 200 CRD, and Park Model DPL 3400. FAA published

reports pertinent to this type of burner are: (1) Powerplant Enginering

Report No. 3A, Standard Fire Test Apparatus and Procedure for Flexible Hose

Assemblies, dated March 1978; and (2) Report No. DOT/FAA/RD/76/213,

Reevaluation of Burner Characteristics for Fire Resistance Tests, dated

January 1977.

(3) Calorimeter.

(i) The calorimeter to be used in testing must be a (0-15.0 BTU/ft2-sec. 0-

17.0 w/cm**2) calorimeter, accurate +/-3%, mounted in a 6-inch by 12-inch

(152 by 305 mm) by 3/4 -inch (19 mm) thick calcium silicate insulating board

which is attached to a steel angle bracket for placement in the test stand

during burner calibration, as shown in Figure 4.

(ii) Because crumbling of the insulating board with service can result in

misalignment of the calorimeter, the calorimeter must be monitored and the

mounting shimmed, as necessary, to ensure that the calorimeter face is flush

with the exposed plane of the insulating board in a plane parallel to the

exit of the test burner cone.

(4) Thermocouples. The seven thermocouples to be used for testing must be

1/16 - to 1/8 -inch metal sheathed, ceramic packed, type K, grounded

thermocouples with a nominal 22 to 30 American wire gage (AWG)-size

conductor. The seven thermocouples must be attached to a steel angle bracket

to form a thermocouple rake for placement in the test stand during burner

calibration, as shown in Figure 5.

(5) Apparatus Arrangement. The test burner must be mounted on a suitable

stand to position the exit of the burner cone a distance of 4+/- 1/8  inches

(102+/-3 mm) from one side of the specimen mounting stand. The burner stand

should have the capability of allowing the burner to be swung away from the

specimen mounting stand during warmup periods.

(6) Data Recording. A recording potentiometer or other suitable calibrated

instrument with an appropriate range must be used to measure and record the

outputs of the calorimeter and the thermocouples.

(7) Weight Scale. Weighing Device--A device must be used that with proper

procedures may determine the before and after test weights of each set of

seat cushion specimens within 0.02 pound (9 grams). A continuous weighing

system is preferred.

(8) Timing Device. A stopwatch or other device (calibrated to +/-1 second)

must be used to measure the time of application of the burner flame and self-

extinguishing time or test duration.

(e) Preparation of Apparatus. Before calibration, all equipment must be

turned on and the burner fuel must be adjusted as specified in paragraph

(d)(2).

(f) Calibration. To ensure the proper thermal output of the burner, the

following test must be made:

(1) Place the calorimeter on the test stand as shown in Figure 4 at a

distance of 4+/- 1/8  inches (102+/-3 mm) from the exit of the burner cone.

(2) Turn on the burner, allow it to run for 2 minutes for warmup, and

adjust the burner air intake damper to produce a reading of 10.5+/-0.5 BTU/

ft2-sec. (11.9+/-0.6 w/cm**2) on the calorimeter to ensure steady state

conditions have been achieved. Turn off the burner.

(3) Replace the calorimeter with the thermocouple rake (Figure 5).

(4) Turn on the burner and ensure that the thermocouples are reading

1900+/-100  deg.F (1038+/-38  deg.C) to ensure steady state conditions have

been achieved.

(5) If the calorimeter and thermocouples do not read within range, repeat

steps in paragraphs 1 through 4 and adjust the burner air intake damper until

the proper readings are obtained. The thermocouple rake and the calorimeter

should be used frequently to maintain and record calibrated test parameters.

Until the specific apparatus has demonstrated consistency, each test should

be calibrated. After consistency has been confirmed, several tests may be

conducted with the pre-test calibration before and a calibration check after

the series.

(g) Test Procedure. The flammability of each set of specimens must be

tested as follows:

(1) Record the weight of each set of seat bottom and seat back cushion

specimens to be tested to the nearest 0.02 pound (9 grams).

(2) Mount the seat bottom and seat back cushion test specimens on the test

stand as shown in Figure 2, securing the seat back cushion specimen to the

test stand at the top.

(3) Swing the burner into position and ensure that the distance from the

exit of the burner cone to the side of the seat bottom cushion specimen is

4+/- 1/8  inches (102+/-3 mm).

(4) Swing the burner away from the test position. Turn on the burner and

allow it to run for 2 minutes to provide adequate warmup of the burner cone

and flame stabilization.

(5) To begin the test, swing the burner into the test position and

simultaneously start the timing device.

(6) Expose the seat bottom cushion specimen to the burner flame for 2

minutes and then turn off the burner. Immediately swing the burner away from

the test position. Terminate test 7 minutes after initiating cushion exposure

to the flame by use of a gaseous extinguishing agent (i.e., Halon or CO2).

(7) Determine the weight of the remains of the seat cushion specimen set

left on the mounting stand to the nearest 0.02 pound (9 grams) excluding all

droppings.

(h) Test Report. With respect to all specimen sets tested for a particular

seat cushion for which testing of compliance is performed, the following

information must be recorded:

(1) An identification and description of the specimens being tested.

(2) The number of specimen sets tested.

(3) The initial weight and residual weight of each set, the calculated

percentage weight loss of each set, and the calculated average percentage

weight loss for the total number of sets tested.

(4) The burn length for each set tested.

Part III--Test Method to Determine Flame Penetration Resistance of Cargo

Compartment Liners.

(a) Criteria for Acceptance. (1) At least three specimens of cargo

compartment sidewall or ceiling liner panels must be tested.

(2) Each specimen tested must simulate the cargo compartment sidewall or

ceiling liner panel, including any design features, such as joints, lamp

assemblies, etc., the failure of which would affect the capability of the

liner to safey contain a fire.

(3) There must be no flame penetration of any specimen within 5 minutes

after application of the flame source, and the peak temperature measured at 4

inches above the upper surface of the horizontal test sample must not exceed

400  deg.F.

(b) Summary of Method. This method provides a laboratory test procedure for

measuring the capability of cargo compartment lining materials to resist

flame penetration with a 2 gallon per hour (GPH) NZ2 Grade kerosene or

equivalent burner fire source. Ceiling and sidewall liner panels may be

tested individually provided a baffle is used to simulate the missing panel.

Any specimen that passes the test as a ceiling liner panel may be used as a

sidewall liner panel.

(c) Test Specimens. (1) The specimen to be tested must measure 16+/- 1/8

inches (406+/-3 mm) by 24+1/8  inches (610+/-3 mm).

(2) The specimens must be conditioned at 70  deg.F.+/-5  deg.F. (21

deg.C.+/-2  deg.C.) and 55%+/-5% humidity for at least 24 hours before

testing.

(d) Test Apparatus. The arrangement of the test apparatus, which is shown

in Figure 3 of Part II and Figures 1 through 3 of this part of Appendix F,

must include the components described in this section. Minor details of the

apparatus may vary, depending on the model of the burner used.

(1) Specimen Mounting Stand. The mounting stand for the test specimens

consists of steel angles as shown in Figure 1.

(2) Test Burner. The burner to be used in tesing must--

(i) Be a modified gun type.

(ii) Use a suitable nozzle and maintain fuel pressure to yield a 2 GPH fuel

flow. For example: an 80 degree nozzle nominally rated at 2.25 GPH and

operated at 85 pounds per square inch (PSI) gage to deliver 2.03 GPH.

(iii) Have a 12 inch (305 mm) burner extension installed at the end of the

draft tube with an opening 6 inches (152 mm) high and 11 inches (280 mm) wide

as shown in Figure 3 of Part II of this appendix.

(iv) Have a burner fuel pressure regulator that is adjusted to deliver a

nominal 2.0 GPH of NZ2 Grade kerosene or equivalent.

Burner models which have been used successfully in testing are the Lenox

Model OB-32, Carlin Model 200 CRD and Park Model DPL. The basic burner is

described in FAA Powerplant Engineering Report No. 3A, Standard Fire Test

Apparatus and Procedure for Flexible Hose Assemblies, dated March 1978;

however, the test settings specified in this appendix differ in some

instances from those specified in the report.

(3) Calorimeter. (i) The calorimeter to be used in testing must be a total

heat flux Foil Type Gardon Gage of an appropriate range (approximately 0 to

15.0 British thermal unit (BTU) per ft.2 sec., 0-17.0 watts/cm**2). The

calorimeter must be mounted in a 6 inch by 12 inch (152 by 305 mm) by 3/4

inch (19 mm) thick insulating block which is attached to a steel angle

bracket for placement in the test stand during burner calibration as shown in

Figure 2 of this part of this appendix.

(ii) The insulating block must be monitored for deterioration and the

mounting shimmed as necessary to ensure that the calorimeter face is parallel

to the exit plane of the test burner cone.

(4) Thermocouples. The seven thermocouples to be used for testing must be

1/16  inch ceramic sheathed, type K, grounded thermocouples with a nominal 30

American wire gage (AWG) size conductor. The seven thermocouples must be

attached to a steel angle bracket to form a thermocouple rake for placement

in the test stand during burner calibration as shown in Figure 3 of this part

of this appendix.

(5) Apparatus Arrangement. The test burner must be mounted on a suitable

stand to position the exit of the burner cone a distance of 8 inches from the

ceiling liner panel and 2 inches from the sidewall liner panel. The burner

stand should have the capability of allowing the burner to be swung away from

the test specimen during warm-up periods.

(6) Instrumentation. A recording potentiometer or other suitable instrument

with an appropriate range must be used to measure and record the outputs of

the calorimeter and the thermocouples.

(7) Timing Device. A stopwatch or other device must be used to measure the

time of flame application and the time of flame penetration, if it occurs.

(e) Preparation of Apparatus. Before calibration, all equipment must be

turned on and allowed to stabilize, and the burner fuel flow must be adjusted

as specified in paragraph (d)(2).

(f) Calibration. To ensure the proper thermal output of the burner the

following test must be made:

(1) Remove the burner extension from the end of the draft tube. Turn on the

blower portion of the burner without turning the fuel or igniters on. Measure

the air velocity using a hot wire anemometer in the center of the draft tube

across the face of the opening. Adjust the damper such that the air velocity

is in the range of 1550 to 1800 ft./min. If tabs are being used at the exit

of the draft tube, they must be removed prior to this measurement. Reinstall

the draft tube extension cone.

(2) Place the calorimeter on the test stand as shown in Figure 2 at a

distance of 8 inches (203 mm) from the exit of the burner cone to simulate

the position of the horizontal test specimen.

(3) Turn on the burner, allow it to run for 2 minutes for warm-up, and

adjust the damper to produce a calorimeter reading of 8.0+/-0.5 BTU per ft.2

sec. (9.1+/-0.6 Watts/cm**2).

(4) Replace the calorimeter with the thermocouple rake (see Figure 3).

(5) Turn on the burner and ensure that each of the seven thermocouples

reads 1700  deg.F. +/-100  deg.F. (927  deg.C. +/-38  deg.C.) to ensure

steady state conditions have been achieved. If the temperature is out of this

range, repeat steps 2 through 5 until proper readings are obtained.

(6) Turn off the burner and remove the thermocouple rake.

(7) Repeat (1) to ensure that the burner is in the correct range.

(g) Test Procedure. (1) Mount a thermocouple of the same type as that used

for calibration at a distance of 4 inches (102 mm) above the horizontal

(ceiling) test specimen. The thermocouple should be centered over the burner

cone.

(2) Mount the test specimen on the test stand shown in Figure 1 in either

the horizontal or vertical position. Mount the insulating material in the

other position.

(3) Position the burner so that flames will not impinge on the specimen,

turn the burner on, and allow it to run for 2 minutes. Rotate the burner to

apply the flame to the specimen and simultaneously start the timing device.

(4) Expose the test specimen tto the flame for 5 minutes and then turn off

the burner. The test may be terminated earlier if flame penetration is

observed.

(5) When testing ceiling liner panels, record the peak temperature measured

4 inches above the sample.

(6) Record the time at which flame penetration occurs if applicable.

(h) Test Report. The test report must include the following:

(1) A complete description of the materials tested including type,

manufacturer, thickness, and other appropriate data.

(2) Observations of the behavior of the test specimens during flame

exposure such as delamination, resin ignition, smoke, ect., including the

time of such occurrence.

(3) The time at which flame penetration occurs, if applicable, for each of

the three specimens tested.

(4) Panel orientation (ceiling or sidewall).

[ ...Illustration appears here... ]

Figure 1

[ ...Illustration appears here... ]

Figure 2

[ ...Illustration appears here... ]

Figure 3

[ ...Illustration appears here... ]

Figure 4

[ ...Illustration appears here... ]

Figure 5

[ ...Illustration appears here... ]

Figure 1. Test Apparatus for Horizontal and Vertical Mounting

[ ...Illustration appears here... ]

Figure 2. Calorimeter Bracket

[ ...Illustration appears here... ]

Figure 3. Thermocouple Rake Bracket

Part IV--Test Method to Determine the Heat Release Rate From Cabin Materials

Exposed to Radiant Heat.

(a) Summary of Method. The specimen to be tested is injected into an

environmental chamber through which a constant flow of air passes. The

specimen's exposure is determined by a radiant heat source adjusted to

produce the desired total heat flux on the specimen of 3.5 W/cm/2/, using a

calibrated calorimeter. The specimen is tested so that the exposed surface is

vertical. Combustion is initiated by piloted ignition. The combustion

products leaving the chamber are monitored in order to calculate the release

rate of heat.

(b) Apparatus. The Ohio State University (OSU) rate of heat release

apparatus, as described below, is used. This is a modified version of the

rate of heat release apparatus standardized by the American Society of

Testing and Materials (ASTM), ASTM E-906.

(1) This apparatus is shown in Figure 1. All exterior surfaces of the

apparatus, except the holding chamber, shall be insulated with 25 mm thick,

low density, high-temperature, fiberglass board insulation. A gasketed door

through which the sample injection rod slides forms an airtight closure on

the specimen hold chamber.

(2) Thermopile. The temperature difference between the air entering the

environmental chamber and that leaving is monitored by a thermopile having

five hot and five cold, 24-gauge Chromel-Alumel junctions. The hot junctions

are spaced across the top of the exhaust stack, 10 mm below the top of the

chimney. One thermocouple is located in the geometric center, with the other

four located 30 mm from the center along the diagonal toward each of the

corners (Figure 5). The cold junctions are located in the pan below the lower

air distribution plate (see paragraph (b)(4)). Thermopile hot junctions must

be cleared of soot deposits as needed to maintain the calibrated sensitivity.

(3) Radiation Source. A radiant heat source for generating a flux up to 100

kW/m/2/, using four silicon carbide elements, Type LL, 20 inches (50.8 cm)

long by 5/8  inch (1.54 cm) O.D., nominal resistance 1.4 ohms, is shown in

Figures 2A and 2B. The silicon carbide elements are mounted in the stainless

steel panel box by inserting them through 15.9-mm holes in 0.8 mm thick

ceramic fiber board. Location of the holes in the pads and stainless steel

cover plates are shown in Figure 2B. The diamond shaped mask of 24-gauge

stainless steel is added to provide uniform heat flux over the area occupied

by the 150- by 150-mm vertical sample.

(4) Air Distribution System. The air entering the environmental chamber is

distributed by a 6.3 mm thick aluminum plate having eight, No. 4 drill holes,

51 mm from sides on 102 mm centers, mounted at the base of the environmental

chamber. A second plate of 18 gauge steel having 120, evenly spaced, No. 28

drill holes is mounted 150 mm above the aluminum plate. A well-regulated air

supply is required. The air supply manifold at the base of the pyramidal

section has 48, evenly spaced, No. 26 drill holes located 10 mm from the

inner edge of the manifold so that 0.03 m3/second of air flows between the

pyramidal sections and 0.01 m3/second flows through the environmental chamber

when total air flow to apparatus is controlled at 0.04 m3/second.

(5) Exhaust Stack. An exhaust stack, 133 mm by 70 mm in cross section, and

254 mm long, fabricated from 28 gauge stainless steel, is mounted on the

outlet of the pyramidal section. A 25 mm by 76 mm plate of 31 gauge stainless

steel is centered inside the stack, perpendicular to the air flow, 75 mm

above the base of the stack.

(6) Specimen Holders. The 150-mm x 150-mm specimen is tested in a vertical

orientation. The holder (Figure 3) is provided with a specimen holder frame,

which touches the specimen (which is wrapped with aluminum foil as required

by paragraph (d)(3) of this Part) along only the 6-mm perimeter, and a "V"

shaped spring to hold the assembly together. A detachable 12-mm x 12-mm x

150-mm drip pan and two .020-inch stainless steel wires (as shown in Figure

3) should be used for testing of materials prone to melting and dripping. The

positioning of the spring and frame may be changed to accommodate different

specimen thicknesses by inserting the retaining rod in different holes on the

specimen holder.

Since the radiation shield described in ASTM E-906 is not used, a guide pin

is added to the injection mechanism. This fits into a slotted metal plate on

the injection mechanism outside of the holding chamber and can be used to

provide accurate positioning of the specimen face after injection. The front

surface of the specimen shall be 100 mm from the closed radiation doors after

injection.

The specimen holder clips onto the mounted bracket (Figure 3). The mounting

bracket is attached to the injection rod by three screws which pass through a

wide area washer welded onto a 1/2 -inch nut. The end of the injection rod is

threaded to screw into the nut and a .020 inch thick wide area washer is held

between two 1/2 -inch nuts which are adjusted to tightly cover the hole in

the radiation doors through which the injection rod or calibration

calorimeter pass.

(7) Calorimeter. A total-flux type calorimeter must be mounted in the

center of a 1/2 -inch Kaowool "M" board inserted in the sample holder to

measure the total heat flux. The calorimeter must have a view angle of 180

degrees and be calibrated for incident flux. The calorimeter calibration must

be acceptable to the Administrator.

(8) Pilot-Flame Positions. Pilot ignition of the specimen must be

accomplished by simultaneously exposing the specimen to a lower pilot burner

and an upper pilot burner, as described in paragraphs (b)(8)(i) and

(b)(8)(ii), respectively. The pilot burners must remain lighted for the

entire 5-minute duration of the test.

(i) Lower Pilot Burner. The pilot-flame tubing must be 6.3 mm O.D., 0.8 mm

wall, stainless steel tubing. A mixture of 120 cm/3//min. of methane and 850

cm/3//min. of air must be fed to the lower pilot flame burner. The normal

position of the end of the pilot burner tubing is 10 mm from and

perpendicular to the exposed vertical surface of the specimen. The centerline

at the outlet of the burner tubing must intersect the vertical centerline of

the sample at a point 5 mm above the lower exposed edge of the specimen.

(ii) Upper Pilot Burner. The pilot burner must be a straight length of 6.3

mm O.D., 0.8 mm wall, stainless steel tubing that is 360 mm long. One end of

the tubing shall be closed, and three No. 40 drill holes shall be drilled

into the tubing, 60 mm apart, for gas ports, all radiating in the same

direction. The first hole must be 5 mm from the closed end of the tubing. The

tube is inserted into the environmental chamber through a 6.6 mm hole drilled

10 mm above the upper edge of the window frame. The tube is supported and

positioned by an adjustable "Z" shaped support mounted outside the

environmental chamber, above the viewing window. The tube is positioned above

and 20 mm behind the exposed upper edge of the specimen. The middle hole must

be in the vertical plane perpendicular to the exposed surface of the specimen

which passes through its vertical centerline and must be pointed toward the

radiation source. The gas supplied to the burner must be methane adjusted to

produce flame lengths of 25 mm.

(c) Calibration of Equipment. (1) Heat Release Rate. A burner as shown in

Figure 4 must be placed over the end of the lower pilot flame tubing using a

gas tight connection. The flow of gas to the pilot flame must be at least 99

percent methane and must be accurately metered. Prior to usage, the wet test

meter is properly leveled and filled with distilled water to the tip of the

internal pointer while no gas is flowing. Ambient temperature and pressure of

the water are based on the internal wet test meter temperature. A baseline

flow rate of approximately 1 liter/min is set and increased to higher preset

flows of 4, 6, 8, 6, and 4 liters/min. The rate is determined by using a

stopwatch to time a complete revolution of the wet test meter for both the

baseline and higher flow, with the flow returned to baseline before changing

to the next higher flow. The thermopile baseline voltage is measured. The gas

flow to the burner must be increased to the higher preset flow and allowed to

burn for 2.0 minutes, and the thermopile voltage must be measured. The

sequence is repeated until all five values have been determined. The average

of the five values must be used as the calibration factor. The procedure must

be repeated if the percent relative standard deviation is greater than 5

percent. Calculations are shown in paragraph (f).

(2) Flux Uniformity. Uniformity of flux over the specimen must be checked

periodically and after each heating element change to determine if it is

within acceptable limits of plus or minus 5 percent.

(d) Sample Preparation.

(1) The standard size for vertically mounted specimens is 150 x 150 mm with

thicknesses up to 45 mm.

(2) Conditioning. Specimens must be conditioned as described in Part 1 of

this appendix.

(3) Mounting. Only one surface of a specimen will be exposed during a test.

A single layer of 0.025 mm aluminum foil is wrapped tightly on all unexposed

sides.

(e) Procedure. (1) The power supply to the radiant panel is set to produce

a radiant flux of 3.5 W/cm/2/. The flux is measured at the point which the

center of the specimen surface will occupy when positioned for test. The

radiant flux is measured after the air flow through the equipment is adjusted

to the desired rate. The sample should be tested in its end use thickness.

(2) The pilot flames are lighted and their position, as described in

paragraph (b)(8), is checked.

(3) The air flow to the equipment is set at 0.04 plus or minus 0.001 m/3//s

at atmospheric pressure. Proper air flow may be set and monitored by either:

(1) An orfice meter designed to produce a pressure drop of at least 200 mm of

the manometric fluid, or by (2) a rotometer (varable orfice meter) with a

scale capable of being read to plus or minus 0.0004 m/3//s. The stop on the

vertical specimen holder rod is adjusted so that the exposed surface of the

specimen is positioned 100 mm from the entrance when injected into the

environmental chamber.

(4) The specimen is placed in the hold chamber with the radiation doors

closed. The airtight outer door is secured, and the recording devices are

started. The specimen must be retained in the hold chamber for 60 seconds,

plus or minus 10 seconds, before injection. The thermopile "zero" value is

determined during the last 20 seconds of the hold period.

(5) When the specimen is to be injected, the radiation doors are opened,

the specimen is injected into the environmental chamber, and the radiation

doors are closed behind the specimen.

(6) [Reserved]

(7) Injection of the specimen and closure of the inner door marks time

zero. A record of the thermopile output with at least one data point per

second must be made during the time the specimen is in the environmental

chamber.

(8) The test duration time is five minutes.

(9) A minimum of three specimens must be tested.

(f) Calculations. (1) The calibration factor is calculated as follows:

[ ...Illustration appears here... ]

Formula for Calibration Factor

F0=flow of methane at baseline (1pm)

F1=higher preset flow of methane (1pm)

V0=thermopile voltage at baseline (mv)

V1=thermopile voltage at higher flow (mv)

Ta=Ambient temperature (K)

P=Ambient pressure (mm Hg)

Pv=Water vapor pressure (mm Hg)

(2) Heat release rates may be calculated from the reading of the thermopile

output voltage at any instant of time as

VmxKh

HRR = ----------

.02323m**2

HRR=Heat release Rate kw/m**2

Vm=measured thermopile voltage (mv)

Kh=Calibration Factor (Kw/mv)

(3) The integral of the heat release rate is the total heat release as a

function of time and is calculated by multiplying the rate by the data

sampling frequency in minutes and summing the time from zero to two minutes.

(g) Criteria. The total positive heat release over the first two minutes of

exposure for each of the three or more samples tested must be averaged, and

the peak heat release rate for each of the samples must be averaged. The

average total heat release must not exceed 65 kilowatt-minutes per square

meter, and the average peak heat release rate must not exceed 65 kilowatts

per square meter.

(h) Report. The test report must include the following for each specimen

tested:

(1) Description of the specimen.

(2) Radiant heat flux to the specimen, expressed in W/cm**2.

(3) Data giving release rates of heat (in kW/m**2 ) as a function of time,

either graphically or tabulated at intervals no greater than 10 seconds. The

calibration factor (kn) must be recorded.

(4) If melting, sagging, delaminating, or other behavior that affects the

exposed surface area or the mode of burning occurs, these behaviors must be

reported, together with the time at which such behaviors were observed.

(5) The peak heat release and the 2-minute integrated heat release rate

must be reported.

Part V. Test Method to Determine the Smoke Emission Characteristics of Cabin

Materials

(a) Summary of Method. The specimens must be constructed, conditioned, and

tested in the flaming mode in accordance with American Society of Testing and

Materials (ASTM) Standard Test Method ASTM F814-83.

(b) Acceptance Criteria. The specific optical smoke density (Ds), which is

obtained by averaging the reading obtained after 4 minutes with each of the

three specimens, shall not exceed 200.

[ ...Illustration appears here... ]

Figure 1. Release Rate Apparatus

[ ...Illustration appears here... ]

Figure 2A. "Globar" Radiant Panel

[ ...Illustration appears here... ]

Figure 2B. "Globar" Radiant Panel

[ ...Illustration appears here... ]

Figure 3.

[ ...Illustration appears here... ]

Figure 4.

[ ...Illustration appears here... ]

Figure 5. Thermocouple Position

[ ...Illustration appears here... ]

[Amdt. 25-32, 37 FR 3972, Feb. 24, 1972; 37 FR 5284, Mar. 14, 1972, as

amended by Amdt. 25-55, 47 FR 13315, Mar. 29, 1982; Amdt. 25-59, 49 FR 43193,

Oct. 26, 1984; Amdt. 25-60, 51 FR 18243, May 16, 1986; Amdt. 25-61, 51 FR

26214, July 21, 1986; 51 FR 28322, Aug. 7, 1986; Amdt. 25-66, 53 FR 32573,

Aug. 25, 1988; 53 FR 37542, 37671, Sept. 27, 1988; Amdt. 25-72, 55 FR 29787,

July 20, 1990]

*****************************************************************************

55 FR 29756, No. 140, July 20, 1990

SUMMARY: These amendments to the Federal Aviation Regulations (FAR) update

the standards for type certification of transport category airplanes for

clarity and accuracy, and ensure that the standards are appropriate and

practicable for the smaller transport category airplanes common to regional

air carrier operation.

EFFECTIVE DATE: August 20, 1990.

*****************************************************************************

Appendix G to Part 25--Continuous Gust Design Criteria

The continuous gust design criteria in this appendix must be used in

establishing the dynamic response of the airplane to vertical and lateral

continuous turbulence unless a more rational criteria is used. The following

gust load requirements apply to mission analysis and design envelope

analysis:

(a) The limit gust loads utilizing the continuous turbulence concept must

be determined in accordance with the provisions of either paragraph (b) or

paragraphs (c) and (d) of this appendix.

(b) Design envelope analysis. The limit loads must be determined in

accordance with the following:

(1) All critical altitudes, weights, and weight distributions, as specified

in Sec. 25.321(b), and all critical speeds within the ranges indicated in

paragraph (b)(3) of this appendix must be considered.

(2) Values of A (ratio of root-mean-square incremental load root-mean-

square gust velocity) must be determined by dynamic analysis. The power

spectral density of the atmospheric turbulence must be as given by the

equation--

1+8/3 (1.339 LV)2

f(V) = s2L/(Pi)  --------------------

[1+(1.339 LV)2]11/6

where:

f=power-spectral density (ft./sec.) 2/rad./ft.

s=root-mean-square gust velocity, ft./sec.

V=reduced frequency, radians per foot.

L=2,500 ft.

(3) The limit loads must be obtained by multiplying the A values determined

by the dynamic analysis by the following values of the gust velocity

U<sigma>:

(i) At speed Vc: U<sigma>=85 fps true gust velocity in the interval 0 to

30,000 ft. altitude and is linearly decreased to 30 fps true gust velocity at

80,000 ft. altitude. Where the Administrator finds that a design is

comparable to a similar design with extensive satisfactory service

experience, it will be acceptable to select U<sigma> at Vc less than 85 fps,

but not less than 75 fps, with linear decrease from that value at 20,000 feet

to 30 fps at 80,000 feet. The following factors will be taken into account

when assessing comparability to a similar design:

(1) The transfer function of the new design should exhibit no unusual

characteristics as compared to the similar design which will significantly

affect response to turbulence; e.g., coalescence of modal response in the

frequency regime which can result in a significant increase of loads.

(2) The typical mission of the new airplane is substantially equivalent to

that of the similar design.

(3) The similar design should demonstrate the adequacy of the U<sigma>

selected.

(ii) At speed VB: U<sigma> is equal to 1.32 times the values obtained under

paragraph (b)(3)(i) of this appendix.

(iii) At speed VD: U<sigma> is equal to 1/2  the values obtained under

paragraph (b)(3)(i) of this appendix.

(iv) At speeds between VB and Vc and between Vc and VD: U<sigma> is equal

to a value obtained by linear interpolation.

(4) When a stability augmentation system is included in the analysis, the

effect of system nonlinearities on loads at the limit load level must be

realistically or conservatively accounted for.

(c) Mission analysis. Limit loads must be determined in accordance with the

following:

(1) The expected utilization of the airplane must be represented by one or

more flight profiles in which the load distribution and the variation with

time of speed, altitude, gross weight, and center of gravity position are

defined. These profiles must be divided into mission segments or blocks, for

analysis, and average or effective values of the pertinent parameters defined

for each segment.

(2) For each of the mission segments defined under paragraph (c)(1) of this

appendix, values of A and No must be determined by analysis. A is defined as

the ratio of root-mean-square incremental load to root-mean-square gust

velocity and No is the radius of gyration of the load power spectral density

function about zero frequency. The power spectral density of the atmospheric

turbulence must be given by the equation set forth in paragraph (b)(2) of

this appendix.

(3) For each of the load and stress quantities selected, the frequency of

exceedance must be determined as a function of load level by means of the

equation--

|Y-Yone=g|

N(y) = SUM tNo [ P1 exp ( - ---------------------)

b1A

|Y-Yone=g|

+ P2 exp (- ------------)]

b2A

where--

t=selected time interval.

y=net value of the load or stress.

Yone=g=value of the load or stress in one-g level flight.

N(y)=average number of exceedances of the indicated value of the load or

stress in unit time.

SUM =symbol denoting summation over all mission segments.

No, A=parameters determined by dynamic analysis as defined in paragraph

(c)(2) of this appendix.

P1, P2, b1, b2=parameters defining the probability distributions of root-

mean-square gust velocity, to be read from Figures 1 and 2 of this

appendix.

The limit gust loads must be read from the frequency of exceedance curves at

a frequency of exceedance of 2x10-5 exceedances per hour. Both positive and

negative load directions must be considered in determining the limit loads.

(4) If a stability augmentation system is utilized to reduce the gust

loads, consideration must be given to the fraction of flight time that the

system may be inoperative. The flight profiles of paragraph (c)(1) of this

appendix must include flight with the system inoperative for this fraction of

the flight time. When a stability augmentation system is included in the

analysis, the effect of system nonlinearities on loads at the limit load

level must be conservatively accounted for.

(d) Supplementary design envelope analysis. In addition to the limit loads

defined by paragraph (c) of this appendix, limit loads must also be

determined in accordance with paragraph (b) of this appendix, except that--

(1) In paragraph (b)(3)(i) of this appendix, the value of U<sigma>=85 fps

true gust velocity is replaced by U<sigma>=60 fps true gust velocity on the

interval 0 to 30,000 ft. altitude, and is linearly decreased to 25 fps true

gust velocity at 80,000 ft. altitude; and

(2) In paragraph (b) of this appendix, the reference to paragraphs

(b)(3)(i) through (b)(3)(iii) of this appendix is to be understood as

referring to the paragraph as modified by paragraph (d)(1).

[ ...Illustration appears here... ]

Figure 1 (graph)

[ ...Illustration appears here... ]

Figure 2 (graph)

[Amdt. 25-54, 45 FR 60173, Sept. 11, 1980]

Appendix H to Part 25--Instructions for Continued Airworthiness

H25.1  General.

(a) This appendix specifies requirements for the preparation of

Instructions for Continued Airworthiness as required by Sec. 25.1529.

(b) The Instructions for Continued Airworthiness for each airplane must

include the Instructions for Continued Airworthiness for each engine and

propeller (hereinafter designated "products" ), for each appliance required

by this chapter, and any required information relating to the interface of

those appliances and products with the airplane. If Instructions for

Continued Airworthiness are not supplied by the manufacturer of an appliance

or product installed in the airplane, the Instructions for Continued

Airworthiness for the airplane must include the information essential to the

continued airworthiness of the airplane.

(c) The applicant must submit to the FAA a program to show how changes to

the Instructions for Continued Airworthiness made by the applicant or by the

manufacturers or products and appliances installed in the airplane will be

distributed.

H25.2  Format.

(a) The Instructions for Continued Airworthiness must be in the form of a

manual or manuals as appropriate for the quantity of data to be provided.

(b) The format of the manual or manuals must provide for a practical

arrangement.

H25.3  Content.

The contents of the manual or manuals must be prepared in the English

language. The Instructions for Continued Airworthiness must contain the

following manuals or sections, as appropriate, and information:

(a) Airplane maintenance manual or section. (1) Introduction information

that includes an explanation of the airplane's features and data to the

extent necessary for maintenance or preventive maintenance.

(2) A description of the airplane and its systems and installations

including its engines, propellers, and appliances.

(3) Basic control and operation information describing how the airplane

components and systems are controlled and how they operate, including any

special procedures and limitations that apply.

(4) Servicing information that covers details regarding servicing points,

capacities of tanks, reservoirs, types of fluids to be used, pressures

applicable to the various systems, location of access panels for inspection

and servicing, locations of lubrication points, lubricants to be used,

equipment required for servicing, tow instructions and limitations, mooring,

jacking, and leveling information.

(b) Maintenance instructions. (1) Scheduling information for each part of

the airplane and its engines, auxiliary power units, propellers, accessories,

instruments, and equipment that provides the recommended periods at which

they should be cleaned, inspected, adjusted, tested, and lubricated, and the

degree of inspection, the applicable wear tolerances, and work recommended at

these periods. However, the applicant may refer to an accessory, instrument,

or equipment manufacturer as the source of this information if the applicant

shows that the item has an exceptionally high degree of complexity requiring

specialized maintenance techniques, test equipment, or expertise. The

recommended overhaul periods and necessary cross references to the

Airworthiness Limitations section of the manual must also be included. In

addition, the applicant must include an inspection program that includes the

frequency and extent of the inspections necessary to provide for the

continued airworthiness of the airplane.

(2) Troubleshooting information describing probable malfunctions, how to

recognize those malfunctions, and the remedial action for those malfunctions.

(3) Information describing the order and method of removing and replacing

products and parts with any necessary precautions to be taken.

(4) Other general procedural instructions including procedures for system

testing during ground running, symmetry checks, weighing and determining the

center of gravity, lifting and shoring, and storage limitations.

(c) Diagrams of structural access plates and information needed to gain

access for inspections when access plates are not provided.

(d) Details for the application of special inspection techniques including

radiographic and ultrasonic testing where such processes are specified.

(e) Information needed to apply protective treatments to the structure

after inspection.

(f) All data relative to structural fasteners such as identification,

discard recommendations, and torque values.

(g) A list of special tools needed.

H25.4  Airworthiness Limitations section.

The Instructions for Continued Airworthiness must contain a section titled

Airworthiness Limitations that is segregated and clearly distinguishable from

the rest of the document. This section must set forth each mandatory

replacement time, structural inspection interval, and related structural

inspection procedure approved under Sec. 25.571. If the Instructions for

Continued Airworthiness consist of multiple documents, the section required

by this paragraph must be included in the principal manual. This section must

contain a legible statement in a prominent location that reads: "The

Airworthiness Limitations section is FAA approved and specifies maintenance

required under Secs. 43.16 and 91.403 of the Federal Aviation Regulations

unless an alternative program has been FAA approved."

[Amdt. 25-54, 45 FR 60177, Sept. 11, 1980, as amended by Amdt. 25-68, 54 FR

34329, Aug. 18, 1989]

Effective Date Note: At 54 FR 34329, Aug. 18, 1989, Sec. H25.4 in Appendix

H, Part 25 was amended by changing the cross reference "Sec. 91.163" to "Sec.

91.403", effective August 18, 1990.

Appendix I to Part 25--Installation of an Automatic Takeoff Thrust Control

System (ATTCS)

I25.1  General.

(a) This appendix specifies additional requirements for installation of an

engine power control system that automatically resets thrust or power on

operating engine(s) in the event of any one engine failure during takeoff.

(b) With the ATTCS and associated systems functioning normally as designed,

all applicable requirements of Part 25, except as provided in this appendix,

must be met without requiring any action by the crew to increase thrust or

power.

I25.2  Definitions.

(a) Automatic Takeoff Thrust Control System (ATTCS). An ATTCS is defined as

the entire automatic system used on takeoff, including all devices, both

mechanical and electrical, that sense engine failure, transmit signals,

actuate fuel controls or power levers or increase engine power by other means

on operating engines to achieve scheduled thrust or power increases, and

furnish cockpit information on system operation.

(b) Critical Time Interval. When conducting an ATTCS takeoff, the critical

time interval is between V1 minus 1 second and a point on the minimum

performance, all-engine flight path where, assuming a simultaneous occurrence

of an engine and ATTCS failure, the resulting minimum flight path thereafter

intersects the Part 25 required actual flight path at no less than 400 feet

above the takeoff surface. This time interval is shown in the following

illustration:

[ ...Illustration appears here... ]

I25.3  Performance and System Reliability Requirements.

The applicant must comply with the performance and ATTCS reliability

requirements as follows:

(a) An ATTCS failure or a combination of failures in the ATTCS during the

critical time interval:

(1) Shall not prevent the insertion of the maximum approved takeoff thrust

or power, or must be shown to be an improbable event.

(2) Shall not result in a significant loss or reduction in thrust or power,

or must be shown to be an extremely improbable event.

(b) The concurrent existence of an ATTCS failure and an engine failure

during the critical time interval must be shown to be extremely improbable.

(c) All applicable performance requirements of Part 25 must be met with an

engine failure occurring at the most critical point during takeoff with the

ATTCS system functioning.

I25.4  Thrust Setting.

The initial takeoff thrust or power setting on each engine at the

beginning of the takeoff roll may not be less than any of the following:

(a) Ninety (90) percent of the thrust or power set by the ATTCS (the

maximum takeoff thrust or power approved for the airplane under existing

ambient conditions);

(b) That required to permit normal operation of all safety-related systems

and equipment dependent upon engine thrust or power lever position; or

(c) That shown to be free of hazardous engine response characteristics when

thrust or power is advanced from the initial takeoff thrust or power to the

maximum approved takeoff thrust or power.

I25.5  Powerplant Controls.

(a) In addition to the requirements of Sec. 25.1141, no single failure or

malfunction, or probable combination thereof, of the ATTCS, including

associated systems, may cause the failure of any powerplant function

necessary for safety.

(b) The ATTCS must be designed to:

(1) Apply thrust or power on the operating engine(s), following any one

engine failure during takeoff, to achieve the maximum approved takeoff thrust

or power without exceeding engine operating limits;

(2) Permit manual decrease or increase in thrust or power up to the maximum

takeoff thrust or power approved for the airplane under existing conditions

through the use of the power lever. For airplanes equipped with limiters that

automatically prevent engine operating limits from being exceeded under

existing ambient conditions, other means may be used to increase the thrust

or power in the event of an ATTCS failure provided the means is located on or

forward of the power levers; is easily identified and operated under all

operating conditions by a single action of either pilot with the hand that is

normally used to actuate the power levers; and meets the requirements of Sec.

25.777 (a), (b), and (c);

(3) Provide a means to verify to the flightcrew before takeoff that the

ATTCS is in a condition to operate; and

(4) Provide a means for the flightcrew to deactivate the automatic

function. This means must be designed to prevent inadvertent deactivation.

I25.6  Powerplant Instruments.

In addition to the requirements of Sec. 25.1305:

(a) A means must be provided to indicate when the ATTCS is in the armed or

ready condition; and

(b) If the inherent flight characteristics of the airplane do not provide

adequate warning that an engine has failed, a warning system that is

independent of the ATTCS must be provided to give the pilot a clear warning

of any engine failure during takeoff.

[Amdt. 25-62, 52 FR 43156, Nov. 9, 1987]

Appendix J to Part 25--Emergency Demonstration

The following test criteria and procedures must be used for showing

compliance with Sec. 25.803:

(a) The emergency evacuation must be conducted either during the dark of

the night or during daylight with the dark of night simulated. If the

demonstration is conducted indoors during daylight hours, it must be

conducted with each window covered and each door closed to minimize the

daylight effect. Illumination on the floor or ground may be used, but it must

be kept low and shielded against shining into the airplane's windows or

doors.

(b) The airplane must be in a normal attitude with landing gear extended.

(c) Unless the airplane is equipped with an off-wing descent means, stands

or ramps may be used for descent from the wing to the ground. Safety

equipment such as mats or inverted life rafts may be placed on the floor or

ground to protect participants. No other equipment that is not part of the

emergency evacuation equipment of the airplane may be used to aid the

participants in reaching the ground.

(d) Except as provided in paragraph (a) of this Appendix, only the

airplane's emergency lighting system may provide illumination.

(e) All emergency equipment required for the planned operation of the

airplane must be installed.

(f) Each external door and exit, and each internal door or curtain, must be

in the takeoff configuration.

(g) Each crewmember must be seated in the normally assigned seat for

takeoff and must remain in the seat until receiving the signal for

commencement of the demonstration. Each crewmember must be a person having

knowledge of the operation of exits and emergency equipment and, if

compliance with Sec. 121.291 is also being demonstrated, each flight

attendant must be a member of a regularly scheduled line crew.

(h) A representative passenger load of persons in normal health must be

used as follows:

(1) At least 40 percent of the passenger load must be female.

(2) At least 35 percent of the passenger load must be over 50 years of age.

(3) At least 15 percent of the passenger load must be female and over 50

years of age.

(4) Three life-size dolls, not included as part of the total passenger

load, must be carried by passengers to simulate live infants 2 years old or

younger.

(5) Crewmembers, mechanics, and training personnel, who maintain or operate

the airplane in the normal course of their duties, may not be used as

passengers.

(i) No passenger may be assigned a specific seat except as the

Administrator may require. Except as required by subparagraph (g) of this

paragraph, no employee of the applicant may be seated next to an emergency

exit.

(j) Seat belts and shoulder harnesses (as required) must be fastened.

(k) Before the start of the demonstration, approximately one-half of the

total average amount of carry-on baggage, blankets, pillows, and other

similar articles must be distributed at several locations in aisles and

emergency exit access ways to create minor obstructions.

(l) No prior indication may be given to any crewmember or passenger of the

particular exits to be used in the demonstration.

(m) The applicant may not practice, rehearse, or describe the demonstration

for the participants nor may any participant have taken part in this type of

demonstration within the preceding 6 months.

(n) The pretakeoff passenger briefing required by Sec. 121.571 may be

given. The passengers may also be advised to follow directions of crewmembers

but not be instructed on the procedures to be followed in the demonstration.

(o) If safety equipment as allowed by paragraph (c) of this appendix is

provided, either all passenger and cockpit windows must be blacked out or all

of the emergency exits must have safety equipment in order to prevent

disclosure of the available emergency exits.

(p) Not more than 50 percent of the emergency exits in the sides of the

fuselage of an airplane that meets all of the requirements applicable to the

required emergency exits for that airplane may be used for the demonstration.

Exits that are not to be used in the demonstration must have the exit handle

deactivated or must be indicated by red lights, red tape, or other acceptable

means placed outside the exits to indicate fire or other reason why they are

unusable. The exits to be used must be representative of all of the emergency

exits on the airplane and must be designated by the applicant, subject to

approval by the Administrator. At least one floor level exit must be used.

(q) Except as provided in paragraph (c) of this section, all evacuees must

leave the airplane by a means provided as part of the airplane's equipment.

(r) The applicant's approved procedures must be fully utilized, except the

flightcrew must take no active role in assisting others inside the cabin

during the demonstration.

(s) The evacuation time period is completed when the last occupant has

evacuated the airplane and is on the ground. Provided that the acceptance

rate of the stand or ramp is no greater than the acceptance rate of the means

available on the airplane for descent from the wing during an actual crash

situation, evacuees using stands or ramps allowed by paragraph (c) of this

Appendix are considered to be on the ground when they are on the stand or

ramp.

[Doc. No. 24344, Amdt. 25-72, 55 FR 29788, July 20, 1990, as amended by Amdt.

25-79, 58 FR 45229, Aug. 26, 1993]

*****************************************************************************

58 FR 45224, No. 164, Aug. 26, 1993

SUMMARY: These amendments to the airworthiness standards for transport

category airplanes and the operating rules for air carrier operators of such

airplanes modify the procedures for conducting an emergency evacuation

demonstration. These include a requirement that the flightcrew take no active

role in the demonstration, and a change to the age/sex distribution

requirement for demonstration participants. In addition, the airworthiness

standards are amended to standardize the illumination requirements for the

handles of the various types of passenger emergency exits, and to add a

requirement to prevent the inadvertent disabling of the public address system

because of an unstowed microphone. These amendments are intended to enhance

the provisions for egress of occupants of transport category airplanes under

emergency conditions.

EFFECTIVE DATE: September 27, 1993.

*****************************************************************************