PART 133--ROTORCRAFT EXTERNAL-LOAD OPERATIONS

Subpart A--Applicability

Sec.

133.1  Applicability.

Subpart B--Certification Rules

133.11  Certificate required.

133.13  Duration of certificate.

133.14  Carriage of narcotic drugs, marihuana, and depressant or stimulant

drugs or substances.

133.15  Application for certificate issuance or renewal.

133.17  Requirements for issuance of a rotorcraft external-load operator

certificate.

133.19  Rotorcraft.

133.21  Personnel.

133.23  Knowledge and skill.

133.25  Amendment of certificate.

133.27  Availability, transfer, and surrender of certificate.

Subpart C--Operating Rules and Related Requirements

133.31  Emergency operations.

133.33  Operating rules.

133.35  Carriage of persons.

133.37  Crewmember training, currency, and testing requirements.

133.39  Inspection authority.

Subpart D--Airworthiness Requirements

133.41  Flight characteristics requirements.

133.43  Structures and design.

133.45  Operating limitations.

133.47  Rotorcraft-load combination flight manual.

133.49  Markings and placards.

133.51  Airworthiness certification.

Authority: 49 U.S.C. 1348, 1354(a), 1421, and 1427; 49 U.S.C. 106(g)

(Revised Pub. L. 97-449, January 12, 1983).

Source: Docket No. 1529, 29 FR 603, Jan. 24, 1964, unless otherwise noted.

Editorial Note: For miscellaneous amendments to cross references in this

Part 133, see Amdt. 133-2, 31 FR 9211, July 6, 1966.

Subpart A--Applicability

Sec. 133.1   Applicability.

This part prescribes--

(a) Airworthiness certification rules for rotorcraft used in; and

(b) Operating and certification rules governing the conduct of rotorcraft

external-load operations in the United States by any person.

(c) The certification rules of this part do not apply to--

(1) Rotorcraft manufacturers when developing external-load attaching means;

(2) Rotorcraft manufacturers demonstrating compliance of equipment utilized

under this part or appropriate portions of Part 27 or 29 of this chapter;

(3) Operations conducted by a person demonstrating compliance for the

issuance of a certificate or authorization under this part;

(4) Training flights conducted in preparation for the demonstration of

compliance with this part; or

(5) A Federal, State, or local government conducting operations with public

aircraft.

(d) For the purpose of this part, a person other than a crewmember or a

person who is essential and directly connected with the external-load

operation may be carried only in approved Class D rotorcraft-load

combinations.

[Doc. No. 15176, Amdt. 133-6, 42 FR 24198, May 12, 1977, as amended by Amdt.

133-9, 51 FR 40707, Nov. 7, 1986]

Subpart B--Certification Rules

Sec. 133.11   Certificate required.

(a) No person subject to this part may conduct rotorcraft external-load

operations within the United States without, or in violation of the terms of,

a Rotorcraft External-Load Operator Certificate issued by the Administrator

under Sec. 133.17.

(b) No person holding a Rotorcraft External-Load Operator Certificate may

conduct rotorcraft external-load operations subject to this part under a

business name that is not on that certificate.

[Doc. No. 15176, Amdt. 133-6, 42 FR 24198, May 12, 1977, as amended by Amdt.

133-7, 42 FR 32531, June 27, 1977; Amdt. 133-9, 51 FR 40707, Nov. 7, 1986]

Sec. 133.13   Duration of certificate.

Unless sooner surrendered, suspended, or revoked, a Rotorcraft External-

Load Operator Certificate expires at the end of the twenty-fourth month after

the month in which it is issued or renewed.

[Doc. No. 15176, Amdt. 133-6, 42 FR 24198, May 12, 1977, as amended by Amdt.

133-7, 42 FR 32531, June 27, 1977; Amdt. 133-9, 51 FR 40707, Nov. 7, 1986]

Sec. 133.14   Carriage of narcotic drugs, marihuana, and depressant or

stimulant drugs or substances.

If the holder of a certificate issued under this part permits any aircraft

owned or leased by that holder to be engaged in any operation that the

certificate holder knows to be in violation of Sec. 91.19(a) of this chapter,

that operation is a basis for suspending or revoking the certificate.

[Doc. No. 12035, Amdt. 133-4, 38 FR 17493, July 2, 1973, as amended by Amdt.

133-10, 54 FR 34332, Aug. 18, 1989]

Effective Date Note: At 54 FR 34331, August 18, 1989, Sec. 133.14 was

amended by changing the cross reference "Sec. 91.12(a)" to read "Sec.

91.19(a)", effective August 18, 1990.

Sec. 133.15  Application for certificate issuance or renewal.

Application for an original certificate or renewal of a certificate issued

under this part is made on a form, and in a manner, prescribed by the

Administrator. The form may be obtained from an FAA Flight Standards District

Office. The completed application is sent to the district office that has

jurisdiction over the area in which the applicant's home base of operation is

located.

[Doc. No. 15176, Amdt. 133-6, 42 FR 24198, May 12, 1977, as amended by Amdt.

133-11, 54 FR 39294, Sept. 25, 1989]

Sec. 133.17   Requirements for issuance of a rotorcraft external-load

operator certificate.

If an applicant shows that he complies with Secs. 133.19, 133.21, and

133.23, the Administrator issues a Rotorcraft External-Load Operator

Certificate to him with an authorization to operate specified rotorcraft with

those classes of rotorcraft-load combinations for which he complies with the

applicable provisions of Subpart D of this part.

Sec. 133.19   Rotorcraft.

(a) The applicant must have the exclusive use of at least one rotorcraft

that--

(1) Was type certificated under, and meets the requirements of, Part 27 or

29 of this chapter (but not necessarily with external-load-carrying attaching

means installed) or of Sec. 21.25 of this chapter for the special purpose of

rotorcraft external-load operations;

(2) Complies with the certification provisions in Subpart D of this part

that apply to the rotorcraft-load combinations for which authorization is

requested; and

(3) Has a valid standard or restricted category airworthiness certificate.

(b) For the purposes of paragraph (a) of this section, a person has

exclusive use of a rotorcraft if he has the sole possession, control, and use

of it for flight, as owner, or has a written agreement (including

arrangements for the performance of required maintenance) giving him that

possession, control, and use for at least six consecutive months.

[Doc. No. 15176, Amdt. 133-6, 42 FR 24198, May 12, 1977]

Sec. 133.21  Personnel.

(a) The applicant must hold, or have available the services of at least one

person who holds, a current commercial or airline transport pilot

certificate, with a rating appropriate for the rotorcraft prescribed in Sec.

133.19, issued by the Administrator.

(b) The applicant must designate one pilot, who may be the applicant, as

chief pilot for rotorcraft external-load operations. The applicant also may

designate qualified pilots as assistant chief pilots to perform the functions

of the chief pilot when the chief pilot is not readily available. The chief

pilot and assistant chief pilots must be acceptable to the Administrator and

each must hold a current Commercial or Airline Transport Pilot Certificate,

with a rating appropriate for the rotorcraft prescribed in Sec. 133.19.

(c) The holder of a Rotorcraft External-Load Operator Certificate shall

report any change in designation of chief pilot or assistant chief pilot

immediately to the FAA certificate-holding office. The new chief pilot must

be designated and must comply with Sec. 133.23 within 30 days or the operator

may not conduct further operations under the Rotorcraft External-Load

Operator Certificate unless otherwise authorized by the FAA certificate-

holding office.

[Doc. No. 1529, 29 FR 603, Jan. 24, 1964, as amended by Amdt. 133-9, 51 FR

40707, Nov. 7, 1986]

Sec. 133.23   Knowledge and skill.

(a) Except as provided in paragraph (d) of this section, the applicant, or

the chief pilot designated in accordance with Sec. 133.21(b), must

demonstrate to the Administrator satisfactory knowledge and skill regarding

rotorcraft external-load operations as set forth in paragraphs (b) and (c) of

this section.

(b) The test of knowledge (which may be oral or written, at the option of

the applicant) covers the following subjects:

(1) Steps to be taken before starting operations, including a survey of the

flight area.

(2) Proper method of loading, rigging, or attaching the external load.

(3) Performance capabilities, under approved operating procedures and

limitations, of the rotorcraft to be used.

(4) Proper instructions of flight crew and ground workers.

(5) Appropriate rotorcraft-load combination flight manual.

(c) The test of skill requires appropriate maneuvers for each class

requested. The appropriate maneuvers for each load class must be demonstrated

in the rotorcraft prescribed in Sec. 133.19.

(1) Takeoffs and landings.

(2) Demonstration of directional control while hovering.

(3) Acceleration from a hover.

(4) Flight at operational airspeeds.

(5) Approaches to landing or working area.

(6) Maneuvering the external load into the release position.

(7) Demonstration of winch operation, if a winch is installed to hoist the

external load.

(d) Compliance with paragraphs (b) and (c) of this section need not be

shown if the Administrator finds, on the basis of the applicant's (or his

designated chief pilot's) previous experience and safety record in rotorcraft

external-load operations, that his knowledge and skill are adequate.

[Doc. No. 1529, 29 FR 603, Jan. 24, 1964, as amended by Amdt. 133-9, 51 FR

40707, Nov. 7, 1986]

Sec. 133.25   Amendment of certificate.

(a) The holder of a Rotorcraft External-Load Certificate may apply to the

FAA Flight Standards District Office having jurisdiction over the area in

which the applicant's home base of operation is located, or to the Flight

Standards District Office nearest the area in which operations are to be

conducted, for an amendment of the applicant's certificate, to add or delete

a rotorcraft-load combination authorization, by executing the appropriate

portion of the form used in applying for a Rotorcraft External-Load Operator

Certificate. If the applicant for the amendment shows compliance with Secs.

133.19, and 133.49, the Flight Standards District Office issues an amended

Rotorcraft External-Load Operator Certificate to the applicant with

authorization to operate with those classes of rotorcraft-load combinations

for which the applicant complies with the applicable provisions of Subpart D

of this part.

(b) The holder of a rotorcraft external-load certificate may apply for an

amendment to add or delete a rotorcraft authorization by submitting to the

certificate-holding FAA Flight Standards District Office a new list of

rotorcraft, by registration number, with the classes of rotorcraft-load

combinations for which authorization is requested.

[Amdt. 133-8, 43 FR 52206, Nov. 9, 1978, as amended by Amdt. 133-9, 51 FR

40707, Nov. 7, 1986; Amdt. 133-11, 54 FR 39294, Sept. 25, 1989]

Sec. 133.27   Availability, transfer, and surrender of certificate.

(a) Each holder of a rotorcraft external-load operator certificate shall

keep that certificate and a list of authorized rotorcraft at the home base of

operations and shall make it available for inspection by the Administrator

upon request.

(b) Each person conducting a rotorcraft external-load operation shall carry

a facsimile of the Rotorcraft External-Load Operator Certificate in each

rotorcraft used in the operation.

(c) If the Administrator suspends or revokes a Rotorcraft External-Load

Operator Certificate, the holder of that certificate shall return it to the

Administrator. If the certificate holder, for any other reason, discontinues

operations under his certificate, and does not resume operations within two

years, he shall return the certificate to the FAA Flight Standards District

Office having jurisdiction over the area in which his home base of operations

is located.

[Doc. No. 1529, 29 FR 603, Jan. 24, 1964, as amended by Amdt. 133-9, 51 FR

40708, Nov. 7, 1986; Amdt. 133-11, 54 FR 39294, Sept. 25, 1989]

Subpart C--Operating Rules and Related Requirements

Sec. 133.31  Emergency operations.

(a) In an emergency involving the safety of persons or property, the

certificate holder may deviate from the rules of this part to the extent

required to meet that emergency.

(b) Each person who, under the authority of this section, deviates from a

rule of this part shall notify the Administrator within 10 days after the

deviation. Upon the request of the Administrator, that person shall provide

the certificate-holding FAA Flight Standards District Office a complete

report of the aircraft operation involved, including a description of the

deviation and reasons for it.

[Doc. No. 24550, Amdt. 133-9, 51 FR 40708, Nov. 7, 1986, as amended by Amdt.

133-11, 54 FR 39294, Sept. 25, 1989]

Sec. 133.33  Operating rules.

(a) No person may conduct a rotorcraft external-load operation without, or

contrary to, the Rotorcraft-Load Combination Flight Manual prescribed in Sec.

133.47.

(b) No person may conduct a rotorcraft external-load operation unless--

(1) The rotorcraft complies with Sec. 133.19; and

(2) The rotorcraft and rotorcraft-load combination is authorized under the

Rotorcraft External-Load Operator Certificate.

(c) Before a person may operate a rotorcraft with an external-load

configuration that differs substantially from any that person has previously

carried with that type of rotorcraft (whether or not the rotorcraft-load

combination is of the same class), that person must conduct, in a manner that

will not endanger persons or property on the surface, such of the following

flight-operational checks as the Administrator determines are appropriate to

the rotorcraft-load combination:

(1) A determination that the weight of the rotorcraft-load combination and

the location of its center of gravity are within approved limits, that the

external load is securely fastened, and that the external load does not

interfere with devices provided for its emergency release.

(2) Make an initial liftoff and verify that controllability is

satisfactory.

(3) While hovering, verify that directional control is adequate.

(4) Accelerate into forward flight to verify that no attitude (whether of

the rotorcraft or of the external load) is encountered in which the

rotorcraft is uncontrollable or which is otherwise hazardous.

(5) In forward flight, check for hazardous oscillations of the external

load, but if the external load is not visible to the pilot, other crewmembers

or ground personnel may make this check and signal the pilot.

(6) Increase the forward airspeed and determine an operational airspeed at

which no hazardous oscillation or hazardous aerodynamic turbulence is

encountered.

(d) Notwithstanding the provisions of Part 91 of this chapter, the holder

of a Rotorcraft External-Load Operator Certificate may conduct (in rotorcraft

type certificated under and meeting the requirements of Part 27 or 29 of this

chapter, including the external-load attaching means) rotorcraft external-

load operations over congested areas if those operations are conducted

without hazard to persons or property on the surface and comply with the

following:

(1) The operator must develop a plan for each complete operation,

coordinate this plan with the FAA Flight Standards District Office having

jurisdiction over the area in which the operation will be conducted, and

obtain approval for the operation from that district office. The plan must

include an agreement with the appropriate political subdivision that local

officials will exclude unauthorized persons from the area in which the

operation will be conducted, coordination with air traffic control, if

necessary, and a detailed chart depicting the flight routes and altitudes.

(2) Each flight must be conducted at an altitude, and on a route, that will

allow a jettisonable external load to be released, and the rotorcraft landed,

in an emergency without hazard to persons or property on the surface.

(e) Notwithstanding the provisions of Part 91 of this chapter, and except

as provided in Sec. 133.45(d), the holder of a Rotorcraft External-Load

Operator Certificate may conduct external-load operations, including

approaches, departures, and load positioning maneuvers necessary for the

operation, below 500 feet above the surface and closer than 500 feet to

persons, vessels, vehicles, and structures, if the operations are conducted

without creating a hazard to persons or property on the surface.

(f) No person may conduct rotorcraft external-load operations under IFR

unless specifically approved by the Administrator. However, under no

circumstances may a person be carried as part of the external-load under IFR.

[Doc. No. 24550, Amdt. No. 133-9, 51 FR 40708, Nov. 7, 1986, as amended by

Amdt. 133-11, 54 FR 39294, Sept. 25, 1989]

Sec. 133.35  Carriage of persons.

(a) No certificate holder may allow a person to be carried during

rotorcraft external-load operations unless that person--

(1) Is a flight crewmember;

(2) Is a flight crewmember trainee;

(3) Performs an essential function in connection with the external-load

operation; or

(4) Is necessary to accomplish the work activity directly associated with

that operation.

(b) The pilot in command shall ensure that all persons are briefed before

takeoff on all pertinent procedures to be followed (including normal,

abnormal, and emergency procedures) and equipment to be used during the

external-load operation.

[Doc. No. 24550, Amdt. No. 133-9, 51 FR 40708, Nov. 7, 1986]

Sec. 133.37  Crewmember training, currency, and testing requirements.

(a) No certificate holder may use, nor may any person serve, as a pilot in

operations conducted under this part unless that person--

(1) Has successfully demonstrated,to the Administrator knowledge and skill

with respect to the rotorcraft-load combination in accordance with Sec.

133.23 (in the case of a pilot other than the chief pilot or an assistant

chief pilot who has been designated in accordance with Sec. 133.21(b), this

demonstration may be made to the chief pilot or assistant chief pilot); and

(2) Has in his or her personal possession a letter of competency or an

appropriate logbook entry indicating compliance with paragraph (a)(1) of this

section.

(b) No certificate holder may use, nor may any person serve as, a

crewmember or other operations personnel in Class D operations conducted

under this part unless, within the preceding 12 calendar months, that person

has successfully completed either an approved initial or a recurrent training

program.

(c) Notwithstanding the provisions of paragraph (b) of this section, a

person who has performed a rotorcraft external-load operation of the same

class and in an aircraft of the same type within the past 12 calendar months

need not undergo recurrent training.

[Doc. No. 24550, Amdt. No. 133-9, 51 FR 40708, Nov. 7, 1986]

Sec. 133.39   Inspection authority.

Each person conducting an operation under this part shall allow the

Administrator to make any inspections or tests that he considers necessary to

determine compliance with the Federal Aviation Regulations and the Rotorcraft

External-Load Operator Certificate.

[Doc. No. 1529, 29 FR 603, Jan. 24, 1964. Redesignated by Amdt. 133-9, 51 FR

40708, Nov. 7, 1986]

Subpart D--Airworthiness Requirements

Sec. 133.41   Flight characteristics requirements.

(a) The applicant must demonstrate to the Administrator, by performing the

operational flight checks prescribed in paragraphs (b), (c), and (d) of this

section, as applicable, that the rotorcraft-load combination has satisfactory

flight characteristics, unless these operational flight checks have been

demonstrated previously and the rotorcraft-load combination flight

characteristics were satisfactory. For the purposes of this demonstration,

the external-load weight (including the external-load attaching means) is the

maximum weight for which authorization is requested.

(b) Class A rotorcraft-load combinations: The operational flight check must

consist of at least the following maneuvers:

(1) Take off and landing.

(2) Demonstration of adequate directional control while hovering.

(3) Acceleration from a hover.

(4) Horizontal flight at airspeeds up to the maximum airspeed for which

authorization is requested.

(c) Class B and D rotorcraft-load combinations: The operational flight

check must consist of at least the following maneuvers:

(1) Pickup of the external load.

(2) Demonstration of adequate directional control while hovering.

(3) Acceleration from a hover.

(4) Horizontal flight at airspeeds up to the maximum airspeed for which

authorization is requested.

(5) Demonstrating appropriate lifting device operation.

(6) Maneuvering of the external load into release position and its release,

under probable flight operation conditions, by means of each of the quick-

release controls installed on the rotorcraft.

(d) Class C rotorcraft-load combinations: For Class C rotorcraft-load

combinations used in wire-stringing, cable-laying, or similar operations, the

operational flight check must consist of the maneuvers, as applicable,

prescribed in paragraph (c) of this section.

[Doc. No. 1529, 29 FR 603, Jan. 24, 1964, as amended by Amdt. 133-5, 41 FR

55475, Dec. 20, 1976; Amdt. 133-9, 51 FR 40709, Nov. 7, 1986]

Sec. 133.43   Structures and design.

(a) External-load attaching means. Each external-load attaching means must

have been approved under--

(1) Part 8 of the Civil Air Regulations on or before January 17, 1964;

(2) Part 133, before February 1, 1977;

(3) Part 27 or 29 of this chapter, as applicable, irrespective of the date

of approval; or

(4) Section 21.25 of this chapter.

(b) Quick release devices. Each quick release device must have been

approved under--

(1) Part 27 or 29 of this chapter, as applicable;

(2) Part 133, before February 1, 1977; or

(3) Section 21.25 of this chapter, except the device must comply with Secs.

27.865(b) and 29.865(b), as applicable, of this chapter.

(c) Weight and center of gravity--

(1) Weight. The total weight of the rotorcraft-load combination must not

exceed the total weight approved for the rotorcraft during its type

certification.

(2) Center of gravity. The location of the center of gravity must, for all

loading conditions, be within the range established for the rotorcraft during

its type certification. For Class C rotorcraft-load combinations, the

magnitude and direction of the loading force must be established at those

values for which the effective location of the center of gravity remains

within its established range.

[Amdt. 133-5, 41 FR 55475, Dec. 20, 1976, as amended at Amdt. 133-12, 55 FR

8005, Mar. 6, 1990]

*****************************************************************************

55 FR 7992, No. 44, Mar. 6, 1990

SUMMARY: This rule adopts new and revised airworthiness standards for

certification of airframe and related equipment on both normal and transport

category rotorcraft. In addition, one amendment changes an operating rule

affecting external load operators. These amendments grew out of a rotorcraft

regulatory review program and the recognition by both government and industry

that updated safety standards are needed. These amendments provide a high

level of safety in design requirements, while removing certain unnecessary

existing burdens and better utilizing the unique characteristics and

capabilities of rotorcraft.

EFFECTIVE DATE: April 5, 1990.

*****************************************************************************

Sec. 133.45   Operating limitations.

In addition to the operating limitations set forth in the approved

Rotorcraft Flight Manual, and to any other limitations the Administrator may

prescribe, the operator shall establish at least the following limitations

and set them forth in the Rotorcraft-Load Combination Flight Manual for

rotorcraft-load combination operations:

(a) The rotorcraft-load combination may be operated only within the weight

and center of gravity limitations established in accordance with Sec.

133.43(c).

(b) The rotorcraft-load combination may not be operated with an external

load weight exceeding that used in showing compliance with Secs. 133.41 and

133.43.

(c) The rotorcraft-load combination may not be operated at airspeeds

greater than those established in accordance with Sec. 133.41 (b), (c), and

(d).

(d) No person may conduct an external-load operation under this part with a

rotorcraft type certificated in the restricted category under Sec. 21.25 of

this chapter over a densely populated area, in a congested airway, or near a

busy airport where passenger transport operations are conducted.

(e) The rotorcraft-load combination of Class D may be conducted only in

accordance with the following:

(1) The rotorcraft to be used must have been type certificated under

transport Category A for the operating weight and provide hover capability

with one engine inoperative at that operating weight and altitude.

(2) The rotorcraft must be equipped to allow direct radio

intercommunication among required crewmembers.

(3) The personnel lifting device must be FAA approved.

(4) The lifting device must have an emergency release requiring two

distinct actions.

[Doc. No. 1529, 29 FR 603, Jan. 24, 1964, as amended by Amdt. 133-1, 30 FR

883, Jan. 28, 1965; Amdt. 133-5, 41 FR 55476, Dec. 20, 1976; Amdt. 133-6, 42

FR 24198, May 12, 1977; Amdt. 133-9, 51 FR 40709, Nov. 7, 1986]

Sec. 133.47   Rotorcraft-load combination flight manual.

The applicant must prepare a Rotorcraft-Load Combination Flight Manual and

submit it for approval by the Administrator. The manual must be prepared in

accordance with the rotorcraft flight manual provisions of Subpart G of Part

27 or 29 of this chapter, whichever is applicable. The limiting height-speed

envelope data need not be listed as operating limitations. The manual must

set forth--

(a) Operating limitations, procedures (normal and emergency), performance,

and other information established under this subpart;

(b) The class of rotorcraft-load combinations for which the airworthiness

of the rotorcraft has been demonstrated in accordance with Secs. 133.41 and

133.43; and

(c) In the information section of the Rotorcraft-Load Combination Flight

Manual--

(1) Information on any peculiarities discovered when operating particular

rotorcraft-load combinations;

(2) Precautionary advice regarding static electricity discharges for Class

B, Class C, and Class D rotorcraft-load combinations; and

(3) Any other information essential for safe operation with external loads.

[Doc. No. 1529, 29 FR 603, Jan. 24, 1964, as amended by Amdt. 133-9, 51 FR

40709, Nov. 7, 1986]

Sec. 133.49   Markings and placards.

The following markings and placards must be displayed conspicuously and

must be such that they cannot be easily erased, disfigured, or obscured:

(a) A placard (displayed in the cockpit or cabin) stating the class of

rotorcraft-load combination for which the rotorcraft has been approved and

the occupancy limitation prescribed in Sec. 133.45(a).

(b) A placard, marking, or instruction (displayed next to the external-load

attaching means) stating the maximum external load prescribed as an operating

limitation in Sec. 133.45(c).

Sec. 133.51  Airworthiness certification.

A Rotorcraft External-Load Operator Certificate is a current and valid

airworthiness certificate for each rotorcraft type certificated under Part 27

or 29 of this chapter (or their predecessor parts) and listed by registration

number on a list attached to the certificate, when the rotorcraft is being

used in operations conducted under this part.

[Doc. No. 24550, Amdt. No. 133-9, 51 FR 40709, Nov. 7, 1986]