Title 14--Aeronautics and Space

CHAPTER I--FEDERAL AVIATION ADMINISTRATION, DEPARTMENT OF TRANSPORTATION

SUBCHAPTER F--AIR TRAFFIC AND GENERAL OPERATING RULES

PART 91--GENERAL OPERATING AND FLIGHT RULES

Special Federal Aviation Regulations

SFAR No. 29-4 -- Limited IFR Operations of Rotorcraft

SFAR No. 41   -- Amended or Supplemental Type Certificate for Maximum

Takeoff Weight in Excess of 12,500 Pounds

SFAR No. 50-2 -- Special Flight Rules in the Vicinity of the Grand

Canyon National Park, AZ.

SFAR No. 51-1 -- Special Flight Rules in the Vicinity of Los Angeles

International Airport

SFAR No. 60   -- Air Traffic Control System Emergency Operation

SFAR No. 64   -- Special Flight Authorizations for Noise Restricted Aircraft

SFAR No. 66   -- Prohibition Against Certain Flights Between the United States and

Yugoslavia

SFAR No. 67   -- Prohibition Against Certain Flights Within the Territory and

Airspace of Afghanistan

SFAR No. 71--Special Operating Rules for Air Tour Operators in the State of

Hawaii

Subpart A-General

Sec.

91.1 Applicability.

91.3 Responsibility and authority of the pilot in command.

91.5 Pilot in command of aircraft requiring more than one required pilot.

91.7 Civil aircraft airworthiness.

91.9 Civil aircraft flight manual, marking, and placard requirements.

91.11 Prohibition against interference with crewmembers.

91.13 Careless or reckless operation.

91.15 Dropping objects.

91.17 Alcohol or drugs.

91.19 Carriage of narcotic drugs, marihuana, and depressant or stimulant

drugs or substances.

91.21 Portable electronic devices.

91.23 Truth-in-leasing clause requirement in leases and conditional sales

contracts.

91.25 Aviation Safety Reporting Program: Prohibition against use of

reports for enforcement purposes.

91.27 - 91.99 [Reserved]

Subpart B-Flight Rules

General

91.101 Applicability.

91.103 Preflight action.

91.105 Flight crewmembers at stations.

91.107 Use of safety belts.

91.109 Flight instruction; Simulated instrument flight and certain flight

tests.

91.111 Operating near other aircraft.

91.113 Right-of-way rules: Except water operations.

91.115 Right-of-way rules: Water operations.

91.117 Aircraft speed.

91.119 Minimum safe altitudes: General.

91.121 Altimeter settings.

91.123 Compliance with ATC clearances and instructions.

91.125 ATC light signals.

91.126 Operating on or in the vicinity of an airport in Class G airspace.

91.127 Operating on or in the vicinity of an airport in Class E airspace.

91.129 Operations in Class D airspace.

91.130 Operations in Class C airspace.

91.131 Operations in Class B airspace.

91.133 Restricted and prohibited areas.

91.135 Operations in Class A airspace.

91.137 Temporary flight restrictions.

91.138 Temporary flight restrictions in national disaster areas in the State

of Hawaii.

91.139 Emergency air traffic rules.

91.141 Flight restrictions in the proximity of the Presidential and other

parties.

91.143 Flight limitation in the proximity of space flight operations.

91.144  Temporary Restriction on Flight Operations During Abnormally High

Barometric Pressure Conditions.

91.145 - 91.149 [Reserved]

Visual Flight Rules

91.151 Fuel requirements for flight in VFR conditions.

91.153 VFR flight plan: Information required.

91.155 Basic VFR weather minimums.

91.157 Special VFR weather minimums.

91.159 VFR cruising altitude or flight level.

91.161-91.165 [Reserved]

Instrument Flight Rules

91.167 Fuel requirements for flight in IFR conditions.

91.169 IFR flight plan: Information required.

91.171 VOR equipment check for IFR operations.

91.173 ATC clearance and flight plan required.

91.175 Takeoff and landing under IFR.

91.177 Minimum altitudes for IFR operations.

91.179 IFR cruising altitude or flight level.

91.181 Course to be flown.

91.183 IFR radio communications.

91.185 IFR operations: Two-way radio communications failure.

91.187 Operation under IFR in controlled airspace: Malfunction reports.

91.189 Category II and III operations: General operating rules.

91.191 Category II manual.

91.193 Certificate of authorization for certain Category II operations.

91.195-91.199 [Reserved]

Subpart C-Equipment, Instrument, and Certificate Requirements

91.201 [Reserved]

91.203 Civil aircraft: Certifications required.

91.205 Powered civil aircraft with standard category U.S. airworthiness

certificates: Instrument and equipment requirements.

91.207 Emergency locator transmitters.

91.209 Aircraft lights.

91.211 Supplemental oxygen.

91.213 Inoperative instruments and equipment.

91.215 ATC transponder and altitude reporting equipment and use.

91.217 Data correspondence between automatically reported pressure

altitude data and the pilot's altitude reference.

91.219 Altitude alerting system or device: Turbojet-powered civil

airplanes.

91.221 Traffic alert and collision avoidance system equipment and use.

91.223 - 91.299 [Reserved]

Subpart D-Special Flight Operations

91.301 [Reserved]

91.303 Aerobatic flight.

91.305 Flight test areas.

91.307 Parachutes and parachuting.

91.309 Towing: Gliders.

91.311 Towing: Other than under 91.309.

91.313 Restricted category civil aircraft: Operating limitations.

91.315 Limited category civil aircraft: Operating limitations.

91.317 Provisionally certificated civil aircraft: Operating limitations.

91.319 Aircraft having experimental certificates: Operating limitations.

91.321 Carriage of candidates in Federal elections.

91.323 Increased maximum certificated weights for certain airplanes

operated in Alaska.

91.325 Primary Category Aircraft: Operating limitations.

91.326-91.399 [Reserved]

Subpart E-Maintenance, Preventive Maintenance, and Alterations

91.401 Applicability.

91.403 General.

91.405 Maintenance required.

91.407 Operation after maintenance, preventive maintenance, rebuilding,

or alteration.

91.409 Inspections.

91.411 Altimeter system and altitude reporting equipment tests and

inspections.

91.413 ATC transponder tests and inspections.

91.415 Changes to aircraft inspection programs.

91.417 Maintenance records.

91.419 Transfer of maintenance records.

91.421 Rebuilt engine maintenance records.

91.423-91.499 [Reserved]

Subpart F-Large and Turbine-Powered Multiengine Airplanes

91.501 Applicability.

91.503 Flying equipment and operating information.

91.505 Familiarity with operating limitations and emergency equipment.

91.507 Equipment requirements: Over-the-top or night VFR operations.

91.509 Survival equipment for overwater operations.

91.511 Radio equipment for overwater operations.

91.513 Emergency equipment.

91.515 Flight altitude rules.

91.517 Smoking and safety belt signs.

91.519 Passenger briefing.

91.521 Shoulder harness.

91.523 Carry-on baggage.

91.525 Carriage of cargo.

91.527 Operating in icing conditions.

91.529 Flight engineer requirements.

91.531 Second in command requirements.

91.533 Flight attendant requirements.

91.535 Stowage of food, beverage, and passenger service equipment during

aircraft movement on the surface, takeoff, and landing.

91.537-91.599 [Reserved]

Subpart G-Additional Equipment and Operating Requirements for Large and

Transport Category Aircraft

91.601 Applicability.

91.603 Aural speed warning device.

91.605 Transport category civil airplane weight limitations.

91.607 Emergency exits for airplanes carrying passengers for hire.

91.609 Flight recorders and cockpit voice recorders.

91.611 Authorization for ferry flight with one engine inoperative.

91.613 Materials for compartment interiors.

91.615 - 91.699 [Reserved]

Subpart H-Foreign Aircraft Operations and Operations of U.S.-Registered

Civil Aircraft Outside of the United States

91.701 Applicability.

91.703 Operations of civil aircraft of U.S. registry outside of the United

States.

91.705 Operations within the North Atlantic Minimum Navigation Performance

Specifications Airspace.

91.707 Flights between Mexico or Canada and the United States.

91.709 Operations to Cuba.

91.711 Special rules for foreign civil aircraft.

91.713 Operation of civil aircraft of Cuban registry.

91.715 Special flight authorizations for foreign civil aircraft.

91.717 - 91.799 [Reserved]

Subpart I-Operating Noise Limits

91.801 Applicability: Relation to part 36.

91.803 Part 125 operators: Designation of applicable regulations.

91.805 Final compliance: Subsonic airplanes.

91.807 Phased compliance under parts 121, 125, and 135: Subsonic

airplanes.

91.809 Replacement airplanes.

91.811 Service to small communi.

91.813 Compliance plans and status: U.S. operations of subsonic airplanes.

91.815 Agricultural and fire fighting airplanes: Noise operating

limitations.

91.817 Civil aircraft sonic boom.

91.819 Civil supersonic airplanes that do not comply with part 36.

91.821 Civil supersonic airplanes: Noise limits.

91.823-91.849 [Reserved]

91.853  Final compliance:  civil subsonic airplanes.

91.855  Entry and nonaddition rule

91.857  Airplanes imported to points outside the contiguous United

States.

91.859  Modification to meet Stage 3 noise levels.

91.861  Base level.

91.863  Transfers of Stage 2 airplanes with base level.

91.865  Phased compliance for operators with base level.

91.867  Phased compliance for new entrants.

91.869  Carry-forward compliance.

91.871  Waivers from interim compliance requirements.

91.873  Waivers from final compliance.

91.875  Annual progress reports.

Subpart J-Waivers

91.901 [Reserved]

91.903 Policy and procedures.

91.905 List of rules subject to waivers.

91.907 - 91.999 [Reserved]

Appendix A-Category II Operations: Manual, Instruments, Equipment, and

Maintenance

Appendix B-Authorizations to Exceed Mach 1 ( 91.817)

Appendix C-Operations in the North Atlantic (NAT) Minimum Navigation

Performance Specifications (MNPS) Airspace

Appendix D-Airports/Locations: Special Operating Restrictions.

Appendix E-Airplane Flight Recorder Specifications

Appendix F-Helicopter Flight Recorder Specificationsbookma

SFAR No. 29-4--Limited IFR Operations of Rotorcraft

1. Contrary provisions of Parts 21, 27, and 29 of the Federal Aviation

Regulations notwithstanding, an operator of a rotorcraft that is not

otherwise certificated for IFR operations may conduct an approved limited IFR

operation in the rotorcraft when--

(a) FAA approval for the operation has been issued under paragraph 2 of

this SFAR;

(b) The operator complies with all conditions and limitations established

by this SFAR and the approval; and

(c) A copy of the approval and this SFAR are set forth as a supplement to

the Rotorcraft Flight Manual.

2. FAA approval for the operation of a rotorcraft in limited IFR operations

may be issued when the following conditions are met:

(a) The operation is approved as part of the FAA study of limited

rotorcraft IFR operations.

(b) Specific FAA approval has been obtained for the following:

(i) The rotorcraft (make, model, and serial number).

(ii) The flightcrew.

(iii) The procedures to be followed in the operation of the rotorcraft

under IFR and the equipment that must be operable during such operations.

(c) The conditions and limitations necessary for the safe operation of the

rotorcraft in limited IFR operations have been established, approved, and

incorporated into the operating limitations section of the Rotorcraft Flight

Manual.

3. An approval issued under paragraph 2 of this Special Federal Aviation

Regulation and the change to the Rotorcraft Flight Manual specified in

paragraph 2(c) of this Special Federal Aviation Regulation constitute a

supplemental type certificate for each rotorcraft approved under paragraph 2

of this SFAR. The supplemental type certificate will remain in effect until

the approval to operate issued under the Special Federal Aviation Regulation

is surrendered, revoked, or otherwise terminated.

4. Notwithstanding Sec. 91.167(a)(3) of the Federal Aviation Regulations, a

person may operate a rotorcraft in a limited IFR operation approved under

paragraph 2(a) of the Special Federal Aviation Regulation with enough fuel to

fly, after reaching the alternate airport, for not less than 30 minutes, when

that period of time has been approved.

5. Expiration.

(a) New applications for limited IFR rotorcraft operations under SFAR No.

29 may be submitted for approval until, but not including, the effective date

of Amendment No. 1 of the Rotorcraft Regulatory Review Program. On and after

the effective date of Amendment No. 1 of the Rotorcraft Regulatory Review

Program, all applicants for certification of IFR rotorcraft operations must

comply with the applicable provisions of the Federal Aviation Regulations.

(b) This Special Federal Aviation Regulation will terminate when all

approvals issued under Special Federal Aviation Regulation No. 29 are

surrendered, revoked, or otherwise terminated.

SFAR No. 41

1. Applicability.

(a) Contrary provisions of Parts 21 and 23 of the Federal Aviation

Regulations notwithstanding, an applicant is entitled to an amended or

supplemental type certificate in the normal category for a reciprocating or

turbopropeller-powered multiengine small airplane originally type

certificated prior to October 17, 1979, in accordance with Part 23 of the

Federal Aviation Regulations in effect on March 13, 1971, or later, that is

to be certificated with a passenger seating configuration, excluding pilot

seats, of 10 seats or more (but not more than 19 seats) at a maximum

certificated takeoff weight of 12,500 pounds or less, if the applicant

complies with--

(1) The regulations incorporated in the type certificate; and

(2) The requirements of Appendix A of Part 135 of the Federal Aviation

Regulations in effect on September 26, 1978, except that the landing distance

must be determined for standard atmosphere at each weight, altitude, and wind

within the operating limits established by the applicant in accordance with

Sec. 23.75(a) of this chapter in effect on September 26, 1978. Instead of a

gliding approach specified in Sec. 23.75(a), the landing may be preceded by a

steady approach down to the 50-foot height at a gradient of descent not

greater than 5.2 percent (3 deg. ) at a calibrated airspeed not less than

1.3V.

(b) Contrary provisions of Parts 1, 21, 23, 91, 121, and 135 of the Federal

Aviation Regulations notwithstanding, an applicant is entitled to an amended

or supplemental type certificate in the normal category for a reciprocating

or turbopropeller powered multiengine airplane originally type certificated

prior to October 17, 1979 that is to be certificated with a maximum takeoff

weight in excess of 12,500 pounds, a specified maximum zero fuel weight to be

established by the applicant, and, where requested by the applicant, an

increase in passenger seating configuration, (but not more than 19 passenger

seats) if the applicant complies with--

(1) The regulations incorporated in the type certificate;

(2) The requirements of Appendix A of Part 135 of the Federal Aviation

Regulations in effect on September 26, 1978 with the exceptions specified in

section 5 of this Special Federal Aviation Regulation; and

(3) The additional requirements specified in sections 7 through 14 of this

Special Federal Aviation Regulation applicable to takeoff weights in excess

of 12,500 pounds.

(c) Contrary provisions of Part 1 of the Federal Aviation Regulations

notwithstanding, an airplane certificated under paragraph (b) of this section

is considered to be a small airplane for purposes of Parts 21, 23, 36, 121,

135, and 139 of the Federal Aviation Regulations, and a large airplane for

purposes of Parts 61 and 91. Compliance with the small airplane provisions of

Part 36 of the Federal Aviation Regulations must be shown at the maximum

certificated takeoff weight approved under this Special Federal Aviation

Regulation.

2. Eligibility. Any person may apply for a supplemental type certificate

(or an amended type certificate in the case of a type certificate holder)

under this Special Federal Aviation Regulation.

3. Production limitation. An amended or supplemental type certificate

issued pursuant to section 1.(b) of this Special Federal Aviation Regulation

is effective for the purpose of obtaining an original or an amended

airworthiness certificate, until October 17, 1991 unless the type certificate

is sooner surrendered, suspended, revoked, or terminated.

4. Restrictions. For airplanes certificated under section 1.(b) of this

Special Federal Aviation Regulation--

(a) The maximum zero fuel weight of the airplane must be established as an

operating limitation; and

(b) Except as provided in paragraph (c) of this section, the airworthiness

certificate shall be endorsed "This airplane at weights in excess of 5,700 kg

does not meet the airworthiness requirements of ICAO, as prescribed by Annex

8 of the Convention on International Civil Aviation."

(c) An applicant is entitle to type certificate amendment or a supplement

type certificate that shows compliance with ICAO Annex 8 if the airplane

meets SFAR and the following requirements prescribed by the Administrator in

effect on October 17, 1979:

(1) At each weight, altitude and temperature within the operational limits

selected by the applicant--

(i) For approach climb performance, comply with Secs. 25.121(d) and

25.1533(a)(2).

(ii) For takeoff performance, comply with Secs. 25.105(d), 25,11,

25.113(a), and 25.115.

(2) For gust loads design at rough air gust speed VB, comply with Secs.

25.335(d), 25.341(a)(1), and 25.351(b).

(3) For smoke evacuation design, comply with Sec. 25.831(d).

(4) For engine rotation and restarting design, comply with Sec. 25.903(c)

and (e).

(5) For engine cooling design, comply with Sec. 25.1521(e).

5. Exceptions. For purposes of obtaining an amended or supplemental type

certificate under section 1.(b) of this Special Federal Aviation Regulation,

the following exceptions apply. All references in this section to specific

sections of Parts 23 and 25 of this chapter are to those in effect on

September 26, 1978 if no other date is given:

(a) Compliance with section 1 of Appendix A of Part 135 of the Federal

Aviation Regulations is not required.

(b) Compliance may be shown with the applicable regulations incorporated in

the type certificate in lieu of the requirements of Appendix A of Part 135 of

the Federal Aviation Regulations for takeoff weights of 12,500 pounds or

less, if the airplane was type certificated--

(1) Under FAR Part 23 in effect prior to Amendment 23-10 and the airplane

is to be used only in FAR Part 91 operations;

(2) Before July 19, 1970, in the normal category with a passenger seating

configuration, excluding any pilot seat, of 10 seats or more, (but not to

exceed 19 passenger seats) and meets special conditions issued by the

Administrator for airplanes intended for use in operations under FAR Part

135; or

(3) Before July 19, 1970, in the normal category with a passenger seating

configuration, excluding any pilot seat, of 10 seats or more, (but not to

exceed 19 passenger seats) and meets the additional airworthiness standards

in Special Federal Aviation Regulation No. 23.

(c) In lieu of compliance with sections 7., and 19.(c) of Appendix A of

Part 135 of the Federal Aviation Regulations, comply with the following at

takeoff weights in excess of 12,500 pounds:

Landing. (a) The landing distance must be determined for standard

atmosphere at each weight, altitude, and wind within the operational limits

established by the applicant in accordance with Sec. 23.75(a) of this

chapter. Instead of a gliding approach specified in Sec. 23.75(a), the

landing may be preceded by a steady approach down to the 50-foot height at a

gradient of descent not greater than 5.2 percent (3 deg.) at a calibrated

airspeed not less than 1.3 VS1.

(b) The landing distance data must include correction factors for not more

than 50 percent of the nominal wind components along the landing path

opposite to the direction of landing, and not less than 150 percent of the

nominal wind components along the landing path in the direction of landing.

(d) In lieu of compliance with section 28 of Appendix A of Part 135 of the

Federal Aviation Regulations, comply with the following:

Fatigue evaluation of flight structure. Unless it is shown that the

structure, operating stress levels, materials, and expected use are

comparable from a fatigue standpoint to a similar design which has had

substantial satifactory service experience, the strength, detail design, and

the fabrication of those parts of the wing, wing carrythrough, vertical fin,

horizortal stabilizer, and attaching structure whose failure would be

catastrophic must be evaluted under either--

(a) A fatigue strength investigation in which the structure is shown by

analysis, tests, or both, to be able to withstand the repeated loads of

variable magnitude expected in service. Analysis alone is acceptable only

when it is conservative and applied to simple structures; or

(b) A fail-safe strength investigation in which it is shown by analysis,

tests, or both, that catastrophic failure of the structure is not probable

after fatigue, or obvious partial failure, of a principal structural element,

and that the remaining structure is able to withstand a static ultimate load

factor of 75 percent of the critical limit load factor at VC. These loads

must be multiplied by a factor of 1.15 unless the dynamic effects of failure

under static load are otherwise considered.

(e) In lieu of compliance with section 32 of Appendix A of Part 135 of the

Federal Aviation Regulations, comply with the following:

Doors and exits. The airplane must meet the requirements of Secs.  23.783

and 23.807 (a)(3), (b), and (c) of this chapter, and in addition the

following requirements:

(a) Each cabin must have at least one easily accessible external door.

(b) There must be a means to lock and safeguard each external door against

opening in flight (either inadvertently by persons or as a result of

mechanical failure or failure of a single structural element). Each external

door must be operable from both the inside and the outside, even though

persons may be crowded against the door on the inside of the airplane. Inward

opening doors may be used if there are means to prevent occupants from

crowding against the door to an extent that would interfere with the opening

of the door. The means of opening must be simple and obvious and must be

arranged and marked so that it can be readily located and operated, even in

darkness. Auxiliary locking devices may be used.

(c) Each external door must be reasonably free from jamming as a result of

fuselage deformation in a minor crash.

(d) Each external door must be located where persons using it will not be

endangered by the propellers when appropriate operating procedures are used.

(e) There must be a provision for direct visual inspection of the locking

mechanism by crewmembers to determine whether external doors, for which the

initial opening movement is outward (including passenger, crew, service, and

cargo doors), are fully locked. In addition, there must be a visual means to

signal to appropriate crewmembers when normally used external doors are

closed and fully locked.

(f) Cargo and service doors not suitable for use as exits in an emergency

need only meet paragraph (e) of section 5(e) of this regulation and be

safeguarded against opening in flight as a result of mechanical failure or

failure of a single structural element.

(g) The passenger entrance door must qualify as a floor level emergency

exit. If an integral stair is installed at such a passenger entry door, the

stair must be designed so that when subjected to the inertia forces specified

in Sec. 23.561 of this chapter, and following the collapse of one or more

legs of the landing gear, it will not interfere to an extent that will reduce

the effectiveness of emergency egress through the passenger entry door. Each

additional required emergency exit except floor level exits must be located

over the wing or must be provided with acceptable means to assist the

occupants in descending to the ground. In addition to the passenger entrance

door--

(1) For a total passenger seating capacity of 15 or less, an emergency

exit, as defined in Sec. 23.807(b) of this chapter, is required on each side

of the cabin; and

(2) For a total passenger seating capacity of 16 through 19, three

emergency exits, as defined in Sec. 23.807(b) of this chapter, are required

with one on the same side as the door and two on the side opposite the door.

(h) An evacuation demonstration must be conducted utilizing the maximum

number of occupants for which certification is desired. It must be conducted

under simulated night conditions utilizing only the emergency exits on the

most critical side of the aircraft. The participants must be representative

of average airline passengers with no prior practice or rehearsal for the

demonstration. Evacuation must be completed within 90 seconds.

(i) Each emergency exit must be marked with the word "Exit" by a sign which

has white letters 1 inch high on a red background 2 inches high, be self-

illuminated or independently internally electrically illuminated, and have a

minimum luminescence (brightness) of at least 160 microlamberts. The colors

may be reversed if the passenger compartment illumination is essentially the

same.

(j) Access to window type emergency exits may not be obstructed by seats or

seat backs.

(k) The width of the main passenger aisle at any point between seats must

equal or exceed the values in the following table:

Minimum main

passenger aisle

width--

Less

than 25

Number of     inches   25 inches

passenger      from     and more

seats       floor    from floor

10 through 19  9 inches  15 inches.

(f) In lieu of compliance with Section 45 of Appendix A of Part 135 of the

Federal Aviation Regulations, comply with Sec. 23.954 of this chapter.

(g) In lieu of compliance with Section 56 of Appendix A of Part 135 of the

Federal Aviation Regulations, comply with the following:

Cowlings. The airplane must be designed and constructed so that no fire

originating in any engine compartment can enter, either through openings or

by burn through, any other region where it would create additional hazards.

(h) In lieu of compliance with Section 57 of Appendix A of Part 135 of the

Federal Aviation Regulations, comply with Sec. 25.863 of this chapter.

6. Additional requirements--general. The additional requirements specified

in sections 7 through 14 apply to the certification of airplanes pursuant to

section 1.(b) of this Special Federal Aviation Regulation.

7. Compartment interiors.

(a) If smoking is to be prohibited, there must be a placard so stating, and

if smoking is to be allowed--

(1) There must be an adequate number of self-contained removable ashtrays;

and

(2) Where the crew compartment is separated from the passenger compartment,

there must be at least one sign (using either letters or symbols) notifying

all passengers when smoking is prohibited. Signs which notify when smoking is

prohibited must--

(i) Be legible to each passenger seated in the passenger cabin under all

probable lighting conditions; and

(ii) When illuminated, be so constructed that the crew can turn them on and

off.

(b) Each disposal receptacle for towels, paper, or waste must be fully

enclosed and constructed of at least fire resistant materials, and must

contain fires likely to occur in it under normal use. The ability of the

disposal receptacle to contain those fires under all probable conditions of

wear, misalignment, and ventilation expected in service must be demonstrated

by test. A placard containing the legible words "No Cigarette Disposal" must

be located on or near each disposal receptacle door.

(c) Lavatories must have "No Smoking" or "No Smoking in Lavatory" placards

located conspicuously on each side of the entry door, and self-contained

removable ashtrays located conspicuously on or near the entry side of each

lavatory door, except that one ashtray may serve more than one lavatory door

if it can be seen from the cabin side of each lavatory door served. The

placards must have red letters at least one-half inch high on a white

background at least one inch high. (A "No smoking" symbol may be included on

the placard).

(d) There must be at least one hand fire extinguisher conventiently located

in the pilot compartment.

(e) There must be at least one hand fire extinguisher conventiently located

in the passenger compartment.

8. Landing gear. Comply with Sec. 25.721(a)(2), (b), and (c) of this

chapter in effect on September 26, 1978.

9. Fuel system components crashworthiness. Comply with Secs. 25.963(d) and

25.994 of this chapter in effect on September 26, 1978.

10. Shutoff means. Comply with Sec. 23.1189 of this chapter in effect on

September 26, 1978.

11. Fire detector and extinguishing systems.

(a) Fire detector systems.

(1) There must be a means which ensures the prompt detection of a fire in

an engine compartment.

(2) Each fire detector must be constructed and installed to withstand the

vibration, inertia, and other loads to which it may be subjected in

operation.

(3) No fire detector may be affected by any oil, water, other fluids, or

fumes that might be present.

(4) There must be means to allow the crew to check, in flight, the function

of each fire detector electric circuit.

(5) Wiring and other components of each fire detector system in an engine

compartment must be at least fire resistant.

(b) Fire extinguishing systems.

(1) Except for combustor, turbine, and tail pipe sections of turbine engine

installations that contain lines or components carrying flammable fluids or

gases for which it is shown that a fire originating in these sections can be

controlled, there must be a fire extinguisher system serving each engine

compartment.

(2) The fire extinguishing system, the quantity of the extinguishing agent,

the rate of discharge, and the discharge distribution must be adequate to

extinguish fires. An individual "one shot" system may be used.

(3) The fire-extinguishing system for a nacelle must be able to

simultaneously protect each compartment of the nacelle for which protection

is provided.

12. Fire extinguishing agents. Comply with Sec. 25.1197 of this chapter in

effect on September 26, 1978.

13. Extinguishing agent containers. Comply with Sec. 25.1199 of this

chapter in effect on September 26, 1978.

14. Fire extinguishing system materials. Comply with Sec. 25.1201 of this

chapter in effect on September 26, 1978.

15. Expiration. This Special Federal Aviation Regulation terminates on

September 13, 1983, unless sooner rescinded or superseded.

SFAR No. 50-2

Special Flight Rules in the Vicinity of the Grand Canyon  National Park, AZ

Section 1. Applicability. This rule prescribes special operating rules for

all persons operating aircraft in the following airspace, designated as the

Grand Canyon National Park Special Flight Rules Area:

That airspace extending upward from the surface up to but not including

14,500 feet MSL within an area bounded by a line beginning at lat. 36

deg.09'30'' N., long. 114 deg.03'00'' W.; northeast to lat. 36 deg.14'00''

N., long. 113 deg.09'50'' W.; thence northeast along the boundary of the

Grand Canyon National Park to lat. 36 deg.24'47'' N., long. 112 deg.52'00''

W.; to lat. 36 deg.30'30'' N., long. 112 deg.36'15'' W.; to lat. 36

deg.21'30'' N., long. 112 deg.00'00'' W.; to lat. 36 deg.35'30'' N., long.

111 deg.53'10'' W.; to lat. 36 deg.53'00'' N., long. 111 deg.36'45'' W.; to

lat. 36 deg.53'00'' N., long. 111 deg.33'00'' W.; to lat. 36 deg.19'00'' N.,

long. 111 deg.50'50'' W.; to lat. 36 deg.17'00'' N., long. 111 deg.42'00''

W.; to lat. 35 deg.59'30'' N., long. 111 deg.42'00'' W.; to lat. 35

deg.57'30'' N., long. 112 deg.03'55'' W.; thence counterclockwise via the

5-statute mile radius of the Grand Canyon Airport reference point (lat. 35

deg.57'09'' N., long 112 deg.08'47'' W.) to lat. 35 deg.57'30'' N., long.

112 deg.14'00'' W.; to lat 35 deg.57'30'' N., long. 113 deg.11'00'' W.; to

lat. 35 deg.42'30'' N., long. 113 deg.11'00'' W.; to lat. 35 deg.38'30'' N.,

long. 113 deg.27'30'' W.; thence counterclockwise via the 5-statute mile

radius of the Peach Springs VORTAC to lat. 35 deg.41'20'' N., long. 113

deg.36'00'' W.; to lat. 35 deg.55'25'' N., long 113 deg.49'10'' W.; to lat.

35 deg.57'45'' N., long. 113 deg.45'20'' W.; thence northwest along the park

boundary to lat. 36 deg.02'20'' N., long. 113 deg.50'15'' W.; to lat. 36

deg.00'10'' N., long., 113 deg.53'45'' W.; thence to the point of beginning.

Sec. 2. Definitions. For the purposes of this special regulation:

"Flight Standards District Office" means the FAA Flight Standards District

Office with jurisdiction for the geographical area containing the Grand

Canyon.

"Park" means the Grand Canyon National Park.

"Special Flight Rules Area" means the Grand Canyon National Park Special

Flight Rules Area.

Sec. 3. Aircraft operations: general. Except in an emergency, no person may

operate an aircraft in the Special Flight Rules Area under VFR on or after

September 22, 1988, or under IFR on or after April 6, 1989, unless the

operation--

(a) Is conducted in accordance with the following procedures:

Note: THE FOLLOWING PROCEDURES DO NOT RELIEVE THE PILOT FROM SEE-AND-AVOID

RESPONSIBILITY OR COMPLIANCE WITH FAR 91.119.

(1) Unless necessary to maintain a safe distance from other aircraft or

terrain--

(i) Remain clear of the areas described in Section 4; and

(ii) Remain at or above the following altitudes in each sector of the

canyon:

Eastern section from Lees Ferry to North Canyon and North Canyon to

Boundary Ridge: as prescribed in Section 5.

Boundary Ridge to Supai Point (Yumtheska Point): 10,000 feet MSL.

Supai Point to Diamond Creek: 9,000 feet MSL.

Western section from Diamond Creek to the Grand Wash Cliffs: 8,000 feet

MSL.

(2) Proceed through the four flight corridors described in Section 4 at the

following altitudes unless otherwise authorized in writing by the Flight

Standards District Office:

Northbound

11,500 or

13,500 feet MSL

Southbound

10,500 or

12,500 feet MSL.

(b) Is authorized in writing by the Flight Standards District Office and is

conducted in compliance with the conditions contained in that authorization.

Normally authorization will be granted for operation in the areas described

in Section 4 or below the altitudes listed in Section 5 only for operations

of aircraft necessary for law enforcement, firefighting, emergency medical

treatment/evacuation of persons in the vicinity of the Park; for support of

Park maintenance or activities; or for aerial access to and maintenance of

other property located within the Special Flight Rules Area. Authorization

may be issued on a continuing basis.

(c)(1) Prior to November 1, 1988, is conducted in accordance with a

specific authorization to operate in that airspace incorporated in the

operator's Part 135 operations specifications in accordance with the

provisions of SFAR 50-1, notwithstanding the provisions of Sections 4 and 5;

and

(2) On or after November 1, 1988, is conducted in accordance with a

specific authorization to operate in that airspace incorporated in the

operator's Part 135 operations specifications and approved by the Flight

Standards District Office in accordance with the provisions of SFAR 50-2.

(d) Is a search and rescue mission directed by the U.S. Air Force Rescue

Coordination Center.

(e) Is conducted within 3 nautical miles of Whitmore Airstrip, Pearce Ferry

Airstrip, North Rim Airstrip, Cliff Dwellers Airstrip, or Marble Canyon

Airstrip at an altitude less than 3,000 feet above airport elevation, for the

purpose of landing at or taking off from that facility. Or

(f) Is conducted under an IFR clearance and the pilot is acting in

accordance with ATC instructions. An IFR flight plan may not be filed on a

route or at an altitude that would require operation in an area described in

Section 4.

Section 4. Flight-free zones. Except in an emergency or if otherwise

necessary for safety of flight, or unless otherwise authorized by the Flight

Standards District Office for a purpose listed in section 3(b), no person may

operate an aircraft in the Special Flight Rules Area within the following

areas:

(a) Desert View Flight-Free Zone. Within an area bounded by a line

beginning at Lat. 35 deg.59'30'' N., Long. 111 deg.46'20'' W. to 35

deg.59'30'' N., Long. 111 deg.52'45'' W.; to Lat. 36 deg.04'50'' N., Long.

111 deg.52'00'' W.; to Lat. 36 deg.06'00'' N., Long. 111 deg.46'20'' W.; to

the point of origin; but not including the airspace at and above 10,500 feet

MSL within 1 mile of the western boundary of the zone. The area between the

Desert View and Bright Angel Flight-Free Zones is designated the "Zuni Point

Corridor."

(b) Bright Angel Flight-Free Zone. Within an area bounded by a line

beginning at Lat. 35 deg.59'30'' N., Long. 111 deg.55'30'' W.; to Lat. 35

deg.59'30'' N., Long. 112 deg.04'00'' W.; thence counterclockwise via the 5-

statute-mile radius of the Grand Canyon Airport point (Lat. 35 deg.57'09''

N., Long. 112 deg.08'47'' W.) to Lat. 36 deg.01'30'' N., Long. 112

deg.11'00'' W.; to Lat. 36 deg.06'15'' N., Long. 112 deg.12'50'' W.; to Lat.

36 deg.14'40'' N., Long. 112 deg.08'50'' W.; to Lat. 36 deg.14'40'' N., Long.

111 deg.57'30'' W.; to Lat. 36 deg.12'30'' N., Long. 111 deg.53'50'' W.; to

the point of origin; but not including the airspace at and above 10,500 feet

MSL within 1 mile of the eastern boundary between the southern boundary and

Lat. 36 deg.04'50'' N. or the airspace at and above 10,500 feet MSL within 2

miles of the northwest boundary. The area bounded by the Bright Angel and

Shinumo Flight-Free Zones is designated the "Dragon Corridor."

(c) Shinumo Flight-Free Zone. Within an area bounded by a line beginning at

lat. 36 deg.04'00'' N., long. 112 deg.16'40'' W.; northwest along the park

boundary to a point at lat. 36 deg.12'47'' N., long. 112 deg.30'53'' W.; to

lat. 36 deg.21'15'' N., long. 112 deg.20'20'' W.; east along the park

boundary to lat. 36 deg.21'15'' N., long. 112 deg.13'55'' W.; to lat. 36

deg.14'40'' N., long. 112 deg.11'25'' W.; to the point of origin. The area

between the Thunder River/Toroweap and Shinumo Flight-Free Zones is

designated the "Fossil Canyon Corridor."

(d) Toroweap/Thunder River Flight-Free Zone. Within an area bounded by a

line beginning at lat. 36 deg.22'45'' N., long. 112 deg.20'35'' W.; thence

northwest along the boundary of the Grand Canyon National Park to lat. 36

deg.17'48'' N., long. 113 deg.03'15'' W.; to lat. 36 deg.15'00'' N., long.

113 deg.07'10'' W.; to lat. 36 deg.10'30'' N., long. 113 deg.07'10'' W.;

thence east along the Colorado River to the confluence of Havasu Canyon (lat.

36 deg.18'40'' N., long. 112 deg.45'45'' W.;) including that area within a

1.5-nautical-mile radius of Toroweap Overlook (lat. 36 deg.12'45'' N., long.

113 deg.03'30'' W.) to the point of origin; but not including the following

airspace designated as the "Tuckup Corridor": at or above 10,500 feet MSL

within 2 nautical miles either side of a line extending between lat. 36

deg.24'47'' N., long. 112 deg.48'50'' W.; and lat. 36 deg.17'10'' N., long.

112 deg.48'50'' W.; to the point of origin.

Sec. 5. Minimum flight altitudes. Except in an emergency or if otherwise

necessary for safety of flight, or unless otherwise authorized by the Flight

Standards District Office for a purpose listed in Section 3(b), no person may

operate an aircraft in the Special Flight Rules Area at an altitude lower

than the following:

(a) Eastern section from Lees Ferry to North Canyon: 5,000 feet MSL.

(b) Eastern section from North Canyon to Boundary Ridge: 6,000 feet MSL.

(c) Boundary Ridge to Supai (Yumtheska) Point: 7,500 feet MSL.

(d) Supai Point to Diamond Creek: 6,500 feet MSL.

(e) Western section from Diamond Creek to the Grand Wash Cliffs: 5,000 feet

MSL.

Sec. 6. Commercial sightseeing flights.

(a) Notwithstanding the provisions of Federal Aviation Regulations Sec.

135.1(b)(2), nonstop sightseeing flights that begin and end at the same

airport, are conducted within a 25-statute-mile radius of that airport, and

operate in or through the Special Flight Rules Area during any portion of the

flight are governed by the provisions of Part 135.

(b) No person holding or required to hold an operating certificate under

Part 135 may operate an aircraft in the Special Flight Rules Area except as

authorized by operations specifications issued under that part.

Sec. 7. Minimum terrain clearance. Except in an emergency, when necessary

for takeoff or landing, or unless authorized by the Flight Standards District

Office for a purpose listed in Section 3(b), no person may operate an

aircraft within 500 feet of any terrain or structure located between the

north and south rims of the Grand Canyon.

Sec. 8. Communications. Except when in contact with the Grand Canyon

National Park Airport Traffic Control Tower during arrival or departure or on

a search and rescue mission directed by the U.S. Air Force Rescue

Coordination Center, no person may operate an aircraft in the Special Flight

Rules Area unless he monitors the appropriate frequency continuously while in

that airspace.

Sec. 9. Termination date. This Special Federal Aviation Regulation expires

on June 15, 1995.

[ ...Illustration appears here... ]

Grand Canyon National Park (Flight Map)

SFAR No. 51-1--Special Flight Rules in the Vicinity of Los Angeles

International Airport

Section 1. Applicability: This rule establishes a special operating area

for persons operating aircraft under visual flight rules (VFR) in the

following airspace of the Los Angeles Class B airspace area designated as the

Los Angeles Special Flight Rules Area:

That part of Area A of the Los Angeles TCA between 3,500 feet above mean

sea level (MSL) and 4,500 feet MSL, inclusive, bounded on the north by

Ballona Creek, on the east by the San Diego Freeway, on the south by Imperial

Highway, and on the west by the Pacific Ocean shoreline.

Section 2. Aircraft operations, general. Unless otherwise authorized by the

Administrator, no person may operate an aircraft in the airspace described in

Section 1 unless the operation is conducted under the following rules.

a. The flight must be conducted under VFR and only when operation may be

conducted in compliance with Sec. 91.155(a).

b. The aircraft must be equipped as specified in FAR 91.215(b) replying on

Code 1201 prior to entering and while operating in this area.

c. The pilot shall have a current Los Angeles Terminal Area Chart in the

aircraft.

d. The pilot shall operate on the Santa Monica very high frequency omni-

directional radio range (VOR) 132 deg. radial.

e. Operations in a southeasterly direction shall be in level flight at

3,500 feet MSL.

f. Operations in a northwesterly direction shall be in level flight at

4,500 feet MSL.

g. Indicated airspeed shall not exceed 140 knots.

h. Anticollision lights and aircraft position/navigation lights shall be

on. Use of landing lights is recommended.

i. Turbojet aircraft are prohibited from VFR operations in this area

Section 3. Notwithstanding the provisions of Sec. 91.131(a), an air traffic

control authorization is not required in the Los Angeles Special Flight Rules

Area for operations in compliance with section 2 of this SFAR. All other

provisions of Sec. 91.131 apply to operate in the Special Flight Rules Area.

Special Federal Aviation Regulation No. 60--Air Traffic Control System

Emergency Operation

1. Each person shall, before conducting any operation under the Federal

Aviation Regulations (14 CFR chapter I), be familiar with all available

information concerning that operation, including Notices to Airmen issued

under Sec. 91.139 and, when activated, the provisions of the National Air

Traffic Reduced Complement Operations Plan available for inspection at

operating air traffic facilities and Regional air traffic division offices,

and the General Aviation Reservation Program. No operator may change the

designated airport of intended operation for any flight contained in the

October 1, 1990, OAG.

2. Notwithstanding any provision of the Federal Aviation Regulations to the

contrary, no person may operate an aircraft in the Air Traffic Control

System:

a. Contrary to any restriction, prohibition, procedure or other action

taken by the Director of the Office of Air Traffic Systems Management

(Director) pursuant to paragraph 3 of this regulation and announced in a

Notice to Airmen pursuant to Sec. 91.139 of the Federal Aviation Regulations.

b. When the National Air Traffic Reduced Complement Operations Plan is

activated pursuant to paragraph 4 of this regulation, except in accordance

with the pertinent provisions of the National Air Traffic Reduced Complement

Operations Plan.

3. Prior to or in connection with the implementation of the RCOP, and as

conditions warrant, the Director is authorized to:

a. Restrict, prohibit, or permit VFR and/or IFR operations at any airport,

Class B airspace area, Class C airspace area, or other class of controlled

airspace.

b. Give priority at any airport to flights that are of military necessity,

or are medical emergency flights, Presidential flights, and flights

transporting critical Government employees.

c. Implement, at any airport, traffic management procedures, that may

include reduction of flight operations. Reduction of flight operations will

be accomplished, to the extent practical, on a pro rata basis among and

between air carrier, commercial operator, and general aviation operations.

Flights cancelled under this SFAR at a high density traffic airport will be

considered to have been operated for purposes of part 93 of the Federal

Aviation Regulations.

4. The Director may activate the National Air Traffic Reduced Complement

Operations Plan at any time he finds that it is necessary for the safety and

efficiency of the National Airspace System. Upon activation of the RCOP and

notwithstanding any provision of the FAR to the contrary, the Director is

authorized to suspend or modify any airspace designation.

5. Notice of restrictions, prohibitions, procedures and other actions taken

by the Director under this regulation with respect to the operation of the

Air Traffic Control system will be announced in Notices to Airmen issued

pursuant to Sec. 91.139 of the Federal Aviation Regulations.

6. The Director may delegate his authority under this regulation to the

extent he considers necessary for the safe and efficient operation of the

National Air Traffic Control System.

SFAR No. 64--Special Flight Authorizations for Noise Restricted Aircraft

1. Contrary provisions of part 91, subpart I notwithstanding, an operator

of a civil subsonic turbojet airplane with maximum weight of more than 75,000

pounds may conduct an approved limited nonrevenue operation of that airplane

to or from a U.S. airport when such operation has been authorized by the FAA

under paragraph 2 of this SFAR; and

(a) The operator complies with all conditions and limitations established

by this SFAR and the authorization;

(b) A copy of the authorization is carried aboard the airplane during all

operations to or from a U.S. airport;

(c) The airplane carries an appropriate airworthiness certificate issued by

the country of registration and meets the registration and identification

requirements of that country; and

(d) Whenever the application is for operation to a location at which FAA-

approved noise abatement retrofit equipment is to be installed to make the

aircraft comply with Stage 2 or Stage 3 noise levels as defined in part 36 of

this chapter, the applicant must have a valid contract for such equipment.

2. Authorization for the operation of a Stage 1 or Stage 2 civil turbojet

airplane to or from a U.S. airport may be issued by the FAA for the following

purposes:

Stage 1 Airplanes

(a) For a Stage 1 airplane owned by a U.S. owner/applicant on and since

November 4, 1990:

(i) Obtaining modifications necessary to meet Stage 2 noise levels as

defined in part 36 of this chapter;

(ii) Obtaining modifications necessary to meet Stage 3 noise levels as

defined in part 36 of this chapter; or

(iii) Scrapping the airplane, as deemed necessary by the FAA, to obtain

spare parts to support U.S. programs for the national defense or safety.

(b) For a Stage 1 airplane owned by a non-U.S. owner/applicant:

(i) Obtaining modifications necessary to meet Stage 2 noise levels as

defined in part 36 of this chapter;

(ii) Obtaining modifications necessary to meet Stage 3 noise levels as

defined in part 36 of this chapter; or

(iii) Scrapping the airplane, as deemed necessary by the FAA, to obtain

spare parts to support U.S. programs for the national defense or safety.

(c) For a Stage 1 airplane purchased by a U.S. owner/applicant on or after

November 5, 1990:

(i) Obtaining modifications necessary to meet Stage 2 noise levels as

defined in part 36 of this chapter, provided that the airplane does not

subsequently operate in the contiguous United States;

(ii) Obtaining modifications necessary to meet Stage 3 noise levels as

defined in part 36 of this chapter; or

(iii) Scrapping the airplane, as deemed necessary by the FAA, to obtain

spare parts to support U.S. programs for the national defense or safety.

Stage 2 Airplanes

(d) For a Stage 2 airplane purchased by a U.S. owner/applicant on or after

November 5, 1990, obtaining modification to meet Stage 3 noise levels as

defined in part 36 of this chapter.

(e) For Stage 2 airplanes that were U.S.-owned on and since November 4,

1990, and that have been removed from service to achieve compliance with Sec.

91.865 or Sec. 91.867 of this part:

(i) Obtaining modifications to meet Stage 3 noise levels as defined in part

36 of this chapter;

(ii) Prior to January 1, 2000, exporting an airplane, including flying the

airplane to or from any airport in the contiguous United States necessary for

the exportation of that airplane; or

(iii) Prior to January 1, 2000, operating the airplane as deemed necessary

by the FAA for the sale, lease, storage, or scrapping of the airplane.

3. An application for a special flight authorization under this Special

Federal Aviation Regulation shall be submitted to the FAA, Director of the

Office of Environment and Energy, received no less than five days prior to

the requested flight, and include the following:

(a) The applicant's name and telephone number;

(b) The name of the airplane operator;

(c) The make, model, registration number, and serial number of the

airplane;

(d) The reason why such authorization is necessary;

(e) The purpose of the flight;

(f) Each U.S. airport at which the flight will be operated and the number

of takeoffs and landings at each;

(g) The approximate dates of the flights;

(h) The number of people on board the airplane and the function of each

person;

(i) Whether a special flight permit under FAR part 21.199 or a special

flight authorization under FAR part 91.715 is required for the flight;

(j) A copy of the contract for noise abatement retrofit equipment, if

appropriate; and

(k) Any other information or documentation requested by the Director,

Office of Environment and Energy, as necessary to determine whether the

application should be approved.

4. The Special Federal Aviation Regulation terminates on December 31, 1999,

unless sooner rescinded or superseded.

Special Federal Aviation Regulation No. 66

Prohibition Against Certain Flights Between the United States and Yugoslavia

1. Applicability. Except as provided in paragraphs 3 and 4 of this Special

Federal Aviation Regulation, this rule applies to all aircraft operations

originating from, destined to land in, or overflying the territory of the

United States.

2. Special flight restrictions. Except as provided in paragraph 3 of this

SFAR--

(a) No person shall operate an aircraft or initiate a flight from any point

in the United States to any point in the Federal Republic of Yugoslavia

(Serbia and Montenegro) (hereinafter "Yugoslavia"), or to any intermediate

destination on a flight the ultimate destination of which is in Yugoslavia or

which includes a landing at any point in Yugoslavia in its intended

itinerary;

(b) No person shall operate an aircraft to any point in the United States

from any point in Yugoslavia, or from any intermediate point of departure on

a flight the origin of which is in Yugoslavia, or which includes a departure

from any point in Yugoslavia in its intended itinerary; and

(c) No person shall operate an aircraft over the territory of the United

States if that aircraft's flight itinerary includes any landing at or

departure from any point in Yugoslavia.

3. Permitted operations. This SFAR shall not prohibit the takeoff or

landing of an aircraft, the initiation of a flight, or the overflight of

United States territory by an aircraft authorized to conduct such operations

by the United States Government in consultation with the United Nations

Security Council Committee established by UN Security Council Resolution 757

(1992).

4. Emergency situations. In an emergency that requires immediate decision

and action for the safety of the flight, the pilot in command of an aircraft

may deviate from this SFAR to the extent required by that emergency. Any

deviation required by an emergency shall be reported to the Air Traffic

Control Facility having jurisdiction as soon as possible.

5. Expiration. This Special Federal Aviation Regulation expires August 26,

1994.

Special Federal Aviation Regulation (SFAR) No. 67

Special Federal Aviation Regulation No. 67--Prohibition Against Certain

Flights Within the Territory and Airspace of Afghanistan

1. Applicability. This rule applies to all U.S. air carriers and commercial

operators, all persons exercising the privileges of an airman certificate

issued by the FAA, and all operators using aircraft registered in the United

States except where the operator of such aircraft is a foreign air carrier.

2. Flight prohibition. Except as provided in paragraph 3 and 4 of this

SFAR, no person described in paragraph 1 may conduct flight operations within

the territory and airspace of Afghanistan.

3. Permitted operations. This SFAR does not prohibit persons described in

paragraph 1 from conducting flight operations within the territory and

airspace of Afghanistan where such operations are authorized either by

exemption issued by the Administrator or by another agency of the United

States Government with the approval of the FAA.

4. Emergency situations. In an emergency that requires immediate decision

and action for the safety of the flight, the pilot in command of an aircraft

may deviate from this SFAR to the extent required by that emergency. Except

for U.S. air carriers and commercial operators that are subject to the

requirements of 14 CFR 121.557, 121.559, or 135.19, each person who deviates

from this rule shall, within ten (10) days of the deviation, excluding

Saturdays, Sundays, and Federal holidays, submit to the nearest FAA Flight

Standards District Office a complete report of the operations of the aircraft

involved in the deviation, including a description of the deviation and the

reasons therefor.

5. Expiration. This Special Federal Aviation Regulation expires May 10,

1995.

SFAR No. 71--Special Operating Rules for Air Tour Operators in the State of

Hawaii

Section 1. Applicability. This Special Federal Aviation Regulation

prescribes operating rules for airplane and helicopter visual flight rules

air tour flights conducted in the State of Hawaii under parts 91 and 135 of

the Federal Aviation Regulations. This rule does not apply to flights

conducted in gliders or hot air balloons.

Section 2. Definitions. For the purposes of this SFAR:

"Air tour" means any sightseeing flight conducted under visual flight rules

in an airplane or helicopter for compensation or hire.

"Air tour operator" means any person who conducts an air tour.

Section 3. Helicopter flotation equipment. No person may conduct an air

tour in Hawaii in a single-engine helicopter beyond the shore of any island,

regardless of whether the helicopter is within gliding distance of the shore,

unless:

(a) The helicopter is amphibious or is equipped with floats adequate to

accomplish a safe emergency ditching and approved flotation gear is easily

accessible for each occupant; or

(b) Each person on board the helicopter is wearing approved flotation gear.

Section 4. Helicopter performance plan. Each operator must complete a

performance plan before each helicopter air tour flight. The performance plan

must be based on the information in the Rotorcraft Flight Manual (RFM),

considering the maximum density altitude for which the operation is planned

for the flight to determine the following:

(a) Maximum gross weight and center of gravity (CG) limitations for

hovering in ground effect;

(b) Maximum gross weight and CG limitations for hovering out of ground

effect; and,

(c) Maximum combination of weight, altitude, and temperature for which

height-velocity information in the RFM. is valid.

The pilot in command (PIC) must comply with the performance plan.

Section 5. Helicopter operating limitations. Except for approach to and

transition from a hover, the PIC shall operate the helicopter at a

combination of height and forward speed (including hover) that would permit a

safe landing in event of engine power loss, in accordance with the height-

speed envelope for that helicopter under current weight and aircraft

altitude.

Section 6. Minimum flight altitudes. Except when necessary for takeoff and

landing, or operating in compliance with an air traffic control clearance, or

as otherwise authorized by the Administrator, no person may conduct an air

tour in Hawaii:

(a) Below an altitude of 1,500 feet above the surface over all areas of the

State of Hawaii, and,

(b) Closer than 1,500 feet to any person or property; or,

(c) Below any altitude prescribed by federal statute or regulation.

Section 7. Passenger briefing. Before takeoff, each PIC of an air tour

flight of Hawaii with a flight segment beyond the ocean shore of any island

shall ensure that each passenger has been briefed on the following, in

addition to requirements set forth in Sec. 91.107 or 135.117:

(a) Water ditching procedures;

(b) Use of required flotation equipment; and

(c) Emergency egress from the aircraft in event of a water landing.'

Section 8. Termination date. This Special Federal Aviation Regulation

expires on October 26, 1997.

Issued in Washington, DC, on September 22, 1994.

Subpart A--General

Sec. 91.1  Applicability.

(a) Except as provided in paragraph (b) of this section and Sec. 91.703,

this part prescribes rules governing the operation of aircraft (other than

moored balloons, kites, unmanned rockets, and unmanned free balloons, which

are governed by part 101 of this chapter, and ultralight vehicles operated in

accordance with part 103 of this chapter) within the United States, including

the waters within 3 nautical miles of the U.S. coast.

(b) Each person operating an aircraft in the airspace overlying the waters

between 3 and 12 nautical miles from the coast of the United States shall

comply with Secs. 91.1 through 91.21; Secs. 91.101 through 91.143; Secs.

91.151 through 91.159; Secs. 91.167 through 91.193; Sec. 91.203; Sec. 91.205;

Secs. 91.209 through 91.217; Sec. 91.221; Secs. 91.303 through 91.319; Sec.

91.323; Sec. 91.605; Sec. 91.609; Secs. 91.703 through 91.715; and 91.903.

Sec. 91.3  Responsibility and authority of the pilot in command.

(a) The pilot in command of an aircraft is directly responsible for, and is

the final authority as to, the operation of that aircraft.

(b) In an in-flight emergency requiring immediate action, the pilot in

command may deviate from any rule of this part to the extent required to meet

that emergency.

(c) Each pilot in command who deviates from a rule under paragraph (b) of

this section shall, upon the request of the Administrator, send a written

report of that deviation to the Administrator.

Sec. 91.5  Pilot in command of aircraft requiring more than one required

pilot.

No person may operate an aircraft that is type certificated for more than

one required pilot flight crewmember unless the pilot in command meets the

requirements of Sec. 61.58 of this chapter.

Sec. 91.7  Civil aircraft airworthiness.

(a) No person may operate a civil aircraft unless it is in an airworthy

condition.

(b) The pilot in command of a civil aircraft is responsible for determining

whether that aircraft is in condition for safe flight. The pilot in command

shall discontinue the flight when unairworthy mechanical, electrical, or

structural conditions occur.

Sec. 91.9  Civil aircraft flight manual, marking, and placard requirements.

(a) Except as provided in paragraph (d) of this section, no person may

operate a civil aircraft without complying with the operating limitations

specified in the approved Airplane or Rotorcraft Flight Manual, markings, and

placards, or as otherwise prescribed by the certificating authority of the

country of registry.

(b) No person may operate a U.S.-registered civil aircraft--

(1) For which an Airplane or Rotorcraft Flight Manual is required by Sec.

21.5 of this chapter unless there is available in the aircraft a current,

approved Airplane or Rotorcraft Flight Manual or the manual provided for in

Sec. 121.141(b); and

(2) For which an Airplane or Rotorcraft Flight Manual is not required by

Sec. 21.5 of this chapter, unless there is available in the aircraft a

current approved Airplane or Rotorcraft Flight Manual, approved manual

material, markings, and placards, or any combination thereof.

(c) No person may operate a U.S.-registered civil aircraft unless that

aircraft is identified in accordance with part 45 of this chapter.

(d) Any person taking off or landing a helicopter certificated under part

29 of this chapter at a heliport constructed over water may make such

momentary flight as is necessary for takeoff or landing through the

prohibited range of the limiting height-speed envelope established for the

helicopter if that flight through the prohibited range takes place over water

on which a safe ditching can be accomplished and if the helicopter is

amphibious or is equipped with floats or other emergency flotation gear

adequate to accomplish a safe emergency ditching on open water.

Sec. 91.11  Prohibition against interference with crewmembers.

No person may assault, threaten, intimidate, or interfere with a crewmember

in the performance of the crewmember's duties aboard an aircraft being

operated.

Sec. 91.13  Careless or reckless operation.

(a) Aircraft operations for the purpose of air navigation. No person may

operate an aircraft in a careless or reckless manner so as to endanger the

life or property of another.

(b) Aircraft operations other than for the purpose of air navigation. No

person may operate an aircraft, other than for the purpose of air navigation,

on any part of the surface of an airport used by aircraft for air commerce

(including areas used by those aircraft for receiving or discharging persons

or cargo), in a careless or reckless manner so as to endanger the life or

property of another.

Sec. 91.15  Dropping objects.

No pilot in command of a civil aircraft may allow any object to be dropped

from that aircraft in flight that creates a hazard to persons or property.

However, this section does not prohibit the dropping of any object if

reasonable precautions are taken to avoid injury or damage to persons or

property.

Sec. 91.17  Alcohol or drugs.

(a) No person may act or attempt to act as a crewmember of a civil

aircraft--

(1) Within 8 hours after the consumption of any alcoholic beverage;

(2) While under the influence of alcohol;

(3) While using any drug that affects the person's faculties in any way

contrary to safety; or

(4) While having .04 percent by weight or more alcohol in the blood.

(b) Except in an emergency, no pilot of a civil aircraft may allow a person

who appears to be intoxicated or who demonstrates by manner or physical

indications that the individual is under the influence of drugs (except a

medical patient under proper care) to be carried in that aircraft.

(c) A crewmember shall do the following:

(1) On request of a law enforcement officer, submit to a test to indicate

the percentage by weight of alcohol in the blood, when--

(i) The law enforcement officer is authorized under State or local law to

conduct the test or to have the test conducted; and

(ii) The law enforcement officer is requesting submission to the test to

investigate a suspected violation of State or local law governing the same or

substantially similar conduct prohibited by paragraph (a)(1), (a)(2), or

(a)(4) of this section.

(2) Whenever the Administrator has a reasonable basis to believe that a

person may have violated paragraph (a)(1), (a)(2), or (a)(4) of this section,

that person shall, upon request by the Administrator, furnish the

Administrator, or authorize any clinic, hospital, doctor, or other person to

release to the Administrator, the results of each test taken within 4 hours

after acting or attempting to act as a crewmember that indicates percentage

by weight of alcohol in the blood.

(d) Whenever the Administrator has a reasonable basis to believe that a

person may have violated paragraph (a)(3) of this section, that person shall,

upon request by the Administrator, furnish the Administrator, or authorize

any clinic, hospital, doctor, or other person to release to the

Administrator, the results of each test taken within 4 hours after acting or

attempting to act as a crewmember that indicates the presence of any drugs in

the body.

(e) Any test information obtained by the Administrator under paragraph (c)

or (d) of this section may be evaluated in determining a person's

qualifications for any airman certificate or possible violations of this

chapter and may be used as evidence in any legal proceeding under section

602, 609, or 901 of the Federal Aviation Act of 1958.

Sec. 91.19  Carriage of narcotic drugs, marihuana, and depressant or

stimulant drugs or substances.

(a) Except as provided in paragraph (b) of this section, no person may

operate a civil aircraft within the United States with knowledge that

narcotic drugs, marihuana, and depressant or stimulant drugs or substances as

defined in Federal or State statutes are carried in the aircraft.

(b) Paragraph (a) of this section does not apply to any carriage of

narcotic drugs, marihuana, and depressant or stimulant drugs or substances

authorized by or under any Federal or State statute or by any Federal or

State agency.

Sec. 91.21  Portable electronic devices.

(a) Except as provided in paragraph (b) of this section, no person may

operate, nor may any operator or pilot in command of an aircraft allow the

operation of, any portable electronic device on any of the following U.S.-

registered civil aircraft:

(1) Aircraft operated by a holder of an air carrier operating certificate

or an operating certificate; or

(2) Any other aircraft while it is operated under IFR.

(b) Paragraph (a) of this section does not apply to--

(1) Portable voice recorders;

(2) Hearing aids;

(3) Heart pacemakers;

(4) Electric shavers; or

(5) Any other portable electronic device that the operator of the aircraft

has determined will not cause interference with the navigation or

communication system of the aircraft on which it is to be used.

(c) In the case of an aircraft operated by a holder of an air carrier

operating certificate or an operating certificate, the determination required

by paragraph (b)(5) of this section shall be made by that operator of the

aircraft on which the particular device is to be used. In the case of other

aircraft, the determination may be made by the pilot in command or other

operator of the aircraft.

Sec. 91.23  Truth-in-leasing clause requirement in leases and conditional

sales contracts.

(a) Except as provided in paragraph (b) of this section, the parties to a

lease or contract of conditional sale involving a U.S.-registered large civil

aircraft and entered into after January 2, 1973, shall execute a written

lease or contract and include therein a written truth-in-leasing clause as a

concluding paragraph in large print, immediately preceding the space for the

signature of the parties, which contains the following with respect to each

such aircraft:

(1) Identification of the Federal Aviation Regulations under which the

aircraft has been maintained and inspected during the 12 months preceding the

execution of the lease or contract of conditional sale, and certification by

the parties thereto regarding the aircraft's status of compliance with

applicable maintenance and inspection requirements in this part for the

operation to be conducted under the lease or contract of conditional sale.

(2) The name and address (printed or typed) and the signature of the person

responsible for operational control of the aircraft under the lease or

contract of conditional sale, and certification that each person understands

that person's responsibilities for compliance with applicable Federal

Aviation Regulations.

(3) A statement that an explanation of factors bearing on operational

control and pertinent Federal Aviation Regulations can be obtained from the

nearest FAA Flight Standards district office.

(b) The requirements of paragraph (a) of this section do not apply--

(1) To a lease or contract of conditional sale when--

(i) The party to whom the aircraft is furnished is a foreign air carrier or

certificate holder under part 121, 125, 127, 135, or 141 of this chapter, or

(ii) The party furnishing the aircraft is a foreign air carrier,

certificate holder under part 121, 125, 127, or 141 of this chapter, or a

certificate holder under part 135 of this chapter having appropriate

authority to engage in air taxi operations with large aircraft.

(2) To a contract of conditional sale, when the aircraft involved has not

been registered anywhere prior to the execution of the contract, except as a

new aircraft under a dealer's aircraft registration certificate issued in

accordance with Sec. 47.61 of this chapter.

(c) No person may operate a large civil aircraft of U.S. registry that is

subject to a lease or contract of conditional sale to which paragraph (a) of

this section applies, unless--

(1) The lessee or conditional buyer, or the registered owner if the lessee

is not a citizen of the United States, has mailed a copy of the lease or

contract that complies with the requirements of paragraph (a) of this

section, within 24 hours of its execution, to the Aircraft Registration

Branch, Attn: Technical Section, P.O. Box 25724, Oklahoma City, Oklahoma

73125;

(2) A copy of the lease or contract that complies with the requirements of

paragraph (a) of this section is carried in the aircraft. The copy of the

lease or contract shall be made available for review upon request by the

Administrator, and

(3) The lessee or conditional buyer, or the registered owner if the lessee

is not a citizen of the United States, has notified by telephone or in person

the FAA Flight Standards district office nearest the airport where the flight

will originate. Unless otherwise authorized by that office, the notification

shall be given at least 48 hours before takeoff in the case of the first

flight of that aircraft under that lease or contract and inform the FAA of--

(i) The location of the airport of departure;

(ii) The departure time; and

(iii) The registration number of the aircraft involved.

(d) The copy of the lease or contract furnished to the FAA under paragraph

(c) of this section is commercial or financial information obtained from a

person. It is, therefore, privileged and confidential and will not be made

available by the FAA for public inspection or copying under 5 U.S.C.

552(b)(4) unless recorded with the FAA under part 49 of this chapter.

(e) For the purpose of this section, a lease means any agreement by a

person to furnish an aircraft to another person for compensation or hire,

whether with or without flight crewmembers, other than an agreement for the

sale of an aircraft and a contract of conditional sale under section 101 of

the Federal Aviation Act of 1958. The person furnishing the aircraft is

referred to as the lessor, and the person to whom it is furnished the lessee.

Sec. 91.25  Aviation Safety Reporting Program: Prohibition against use of

reports for enforcement purposes.

The Administrator of the FAA will not use reports submitted to the National

Aeronautics and Space Administration under the Aviation Safety Reporting

Program (or information derived therefrom) in any enforcement action except

information concerning accidents or criminal offenses which are wholly

excluded from the Program.

Secs. 91.27--91.99  [Reserved]

Subpart B--Flight Rules

General

Sec. 91.101  Applicability.

This subpart prescribes flight rules governing the operation of aircraft

within the United States and within 12 nautical miles from the coast of the

United States.

Sec. 91.103  Preflight action.

Each pilot in command shall, before beginning a flight, become familiar

with all available information concerning that flight. This information must

include--

(a) For a flight under IFR or a flight not in the vicinity of an airport,

weather reports and forecasts, fuel requirements, alternatives available if

the planned flight cannot be completed, and any known traffic delays of which

the pilot in command has been advised by ATC;

(b) For any flight, runway lengths at airports of intended use, and the

following takeoff and landing distance information:

(1) For civil aircraft for which an approved Airplane or Rotorcraft Flight

Manual containing takeoff and landing distance data is required, the takeoff

and landing distance data contained therein; and

(2) For civil aircraft other than those specified in paragraph (b)(1) of

this section, other reliable information appropriate to the aircraft,

relating to aircraft performance under expected values of airport elevation

and runway slope, aircraft gross weight, and wind and temperature.

Sec. 91.105  Flight crewmembers at stations.

(a) During takeoff and landing, and while en route, each required flight

crewmember shall--

(1) Be at the crewmember station unless the absence is necessary to perform

duties in connection with the operation of the aircraft or in connection with

physiological needs; and

(2) Keep the safety belt fastened while at the crewmember station.

(b) Each required flight crewmember of a U.S.-registered civil aircraft

shall, during takeoff and landing, keep his or her shoulder harness fastened

while at his or her assigned duty station. This paragraph does not apply if--

(1) The seat at the crewmember's station is not equipped with a shoulder

harness; or

(2) The crewmember would be unable to perform required duties with the

shoulder harness fastened.

Sec. 91.107  Use of safety belts, shoulder harnesses, and child restraint

systems.

(a) Unless otherwise authorized by the Administrator--

(1) No pilot may take off a U.S.-registered civil aircraft (except a free

balloon that incorporates a basket or gondola, or an airship type

certificated before November 2, 1987) unless the pilot in command of that

aircraft ensures that each person on board is briefed on how to fasten and

unfasten that person's safety belt and, if installed, shoulder harness.

(2) No pilot may cause to be moved on the surface, take off, or land a

U.S.-registered civil aircraft (except a free balloon that incorporates a

basket or gondola, or an airship type certificated before November 2, 1987)

unless the pilot in command of that aircraft ensures that each person on

board has been notified to fasten his or her safety belt and, if installed,

his or her shoulder harness.

(3) Except as provided in this paragraph, each person on board a U.S.-

registered civil aircraft (except a free balloon that incorporates a basket

or gondola or an airship type certificated before November 2, 1987) must

occupy an approved seat or berth with a safety belt and, if installed,

shoulder harness, properly secured about him or her during movement on the

surface, takeoff, and landing. For seaplane and float equipped rotorcraft

operations during movement on the surface, the person pushing off the

seaplane or rotorcraft from the dock and the person mooring the seaplane or

rotorcraft at the dock are excepted from the preceding seating and safety

belt requirements. Notwithstanding the preceding requirements of this

paragraph, a person may:

(i) Be held by an adult who is occupying a seat or berth if that person has

not reached his or her second birthday;

(ii) Use the floor of the aircraft as a seat, provided that the person is

on board for the purpose of engaging in sport parachuting; or

(iii) Notwithstanding any other requirement of this chapter, occupy an

approved child restraint system furnished by the operator or one of the

persons described in paragraph (a)(3)(iii)(A) of this section provided that:

(A) The child is accompanied by a parent, guardian, or attendant designated

by the child's parent or guardian to attend to the safety of the child during

the flight;

(B) The approved child restraint system bears one or more labels as

follows:

(1) Seats manufactured to U.S. standards between January 1, 1981, and

February 25, 1985, must bear the label: "This child restraint system conforms

to all applicable Federal motor vehicle safety standards." Vest- and harness-

type child restraint systems manufactured before February 26, 1985, bearing

such a label are not approved for the purposes of this section;

(2) Seats manufactured to U.S. standards on or after February 26, 1985,

must bear two labels:

(i) "This child restraint system conforms to all applicable Federal motor

vehicle safety standards"; and

(ii) "THIS RESTRAINT IS CERTIFIED FOR USE IN MOTOR VEHICLES AND AIRCRAFT"

in red lettering;

(3) Seats that do not qualify under paragraphs (a)(3)(iii)(B)(1) and

(a)(3)(iii)(B)(2) of this section must bear either a label showing approval

of a foreign government or a label showing that the seat was manufactured

under the standards of the United Nations; and

(C) The operator complies with the following requirements:

(1) The restraint system must be properly secured to an approved forward-

facing seat or berth;

(2) The child must be properly secured in the restraint system and must not

exceed the specified weight limit for the restraint system; and

(3) The restraint system must bear the appropriate label(s).

(b) Unless otherwise stated, this section does not apply to operations

conducted under part 121, 125, or 135 of this chapter. Paragraph (a)(3) of

this section does not apply to persons subject to Sec. 91.105.

Sec. 91.109  Flight instruction; Simulated instrument flight and certain

flight tests.

(a) No person may operate a civil aircraft (except a manned free balloon)

that is being used for flight instruction unless that aircraft has fully

functioning dual controls. However, instrument flight instruction may be

given in a single-engine airplane equipped with a single, functioning

throwover control wheel in place of fixed, dual controls of the elevator and

ailerons when--

(1) The instructor has determined that the flight can be conducted safely;

and

(2) The person manipulating the controls has at least a private pilot

certificate with appropriate category and class ratings.

(b) No person may operate a civil aircraft in simulated instrument flight

unless--

(1) The other control seat is occupied by a safety pilot who possesses at

least a private pilot certificate with category and class ratings appropriate

to the aircraft being flown.

(2) The safety pilot has adequate vision forward and to each side of the

aircraft, or a competent observer in the aircraft adequately supplements the

vision of the safety pilot; and

(3) Except in the case of lighter-than-air aircraft, that aircraft is

equipped with fully functioning dual controls. However, simulated instrument

flight may be conducted in a single-engine airplane, equipped with a single,

functioning, throwover control wheel, in place of fixed, dual controls of the

elevator and ailerons, when--

(i) The safety pilot has determined that the flight can be conducted

safely; and

(ii) The person manipulating the controls has at least a private pilot

certificate with appropriate category and class ratings.

(c) No person may operate a civil aircraft that is being used for a flight

test for an airline transport pilot certificate or a class or type rating on

that certificate, or for a part 121 proficiency flight test, unless the pilot

seated at the controls, other than the pilot being checked, is fully

qualified to act as pilot in command of the aircraft.

Sec. 91.111  Operating near other aircraft.

(a) No person may operate an aircraft so close to another aircraft as to

create a collision hazard.

(b) No person may operate an aircraft in formation flight except by

arrangement with the pilot in command of each aircraft in the formation.

(c) No person may operate an aircraft, carrying passengers for hire, in

formation flight.

Sec. 91.113  Right-of-way rules: Except water operations.

(a) Inapplicability. This section does not apply to the operation of an

aircraft on water.

(b) General. When weather conditions permit, regardless of whether an

operation is conducted under instrument flight rules or visual flight rules,

vigilance shall be maintained by each person operating an aircraft so as to

see and avoid other aircraft. When a rule of this section gives another

aircraft the right-of-way, the pilot shall give way to that aircraft and may

not pass over, under, or ahead of it unless well clear.

(c) In distress. An aircraft in distress has the right-of-way over all

other air traffic.

(d) Converging. When aircraft of the same category are converging at

approximately the same altitude (except head-on, or nearly so), the aircraft

to the other's right has the right-of-way. If the aircraft are of different

categories--

(1) A balloon has the right-of-way over any other category of aircraft;

(2) A glider has the right-of-way over an airship, airplane, or rotorcraft;

and

(3) An airship has the right-of-way over an airplane or rotorcraft.

However, an aircraft towing or refueling other aircraft has the right-of-

way over all other engine-driven aircraft.

(e) Approaching head-on. When aircraft are approaching each other head-on,

or nearly so, each pilot of each aircraft shall alter course to the right.

(f) Overtaking. Each aircraft that is being overtaken has the right-of-way

and each pilot of an overtaking aircraft shall alter course to the right to

pass well clear.

(g) Landing. Aircraft, while on final approach to land or while landing,

have the right-of-way over other aircraft in flight or operating on the

surface, except that they shall not take advantage of this rule to force an

aircraft off the runway surface which has already landed and is attempting to

make way for an aircraft on final approach. When two or more aircraft are

approaching an airport for the purpose of landing, the aircraft at the lower

altitude has the right-of-way, but it shall not take advantage of this rule

to cut in front of another which is on final approach to land or to overtake

that aircraft.

Sec. 91.115  Right-of-way rules: Water operations.

(a) General. Each person operating an aircraft on the water shall, insofar

as possible, keep clear of all vessels and avoid impeding their navigation,

and shall give way to any vessel or other aircraft that is given the right-

of-way by any rule of this section.

(b) Crossing. When aircraft, or an aircraft and a vessel, are on crossing

courses, the aircraft or vessel to the other's right has the right-of-way.

(c) Approaching head-on. When aircraft, or an aircraft and a vessel, are

approaching head-on, or nearly so, each shall alter its course to the right

to keep well clear.

(d) Overtaking. Each aircraft or vessel that is being overtaken has the

right-of-way, and the one overtaking shall alter course to keep well clear.

(e) Special circumstances. When aircraft, or an aircraft and a vessel,

approach so as to involve risk of collision, each aircraft or vessel shall

proceed with careful regard to existing circumstances, including the

limitations of the respective craft.

Sec. 91.117  Aircraft speed.

(a) Unless otherwise authorized by the Administrator, no person may

operate an aircraft below 10,000 feet MSL at an indicated airspeed of more

than 250 knots (288 m.p.h.).

(b) Unless otherwise authorized or required by ATC, no person may operate

an aircraft at or below 2,500 feet above the surface within 4 nautical miles

of the primary airport of a Class C or Class D airspace area at an indicated

airspeed of more than 200 knots (230 mph). This paragraph (b) does not apply

to any operations within a Class B airspace area. Such operations shall

comply with paragraph (a) of this section.

(c) No person may operate an aircraft in the airspace underlying a Class B

airspace area designated for an airport or in a VFR corridor designated

through such a Class B airspace area, at an indicated airspeed of more than

200 knots (230 mph).

(d) If the minimum safe airspeed for any particular operation is greater

than the maximum speed prescribed in this section, the aircraft may be

operated at that minimum speed.

Sec. 91.119  Minimum safe altitudes: General.

Except when necessary for takeoff or landing, no person may operate an

aircraft below the following altitudes:

(a) Anywhere. An altitude allowing, if a power unit fails, an emergency

landing without undue hazard to persons or property on the surface.

(b) Over congested areas. Over any congested area of a city, town, or

settlement, or over any open air assembly of persons, an altitude of 1,000

feet above the highest obstacle within a horizontal radius of 2,000 feet of

the aircraft.

(c) Over other than congested areas. An altitude of 500 feet above the

surface, except over open water or sparsely populated areas. In those cases,

the aircraft may not be operated closer than 500 feet to any person, vessel,

vehicle, or structure.

(d) Helicopters. Helicopters may be operated at less than the minimums

prescribed in paragraph (b) or (c) of this section if the operation is

conducted without hazard to persons or property on the surface. In addition,

each person operating a helicopter shall comply with any routes or altitudes

specifically prescribed for helicopters by the Administrator.

Sec. 91.121  Altimeter settings.

(a) Each person operating an aircraft shall maintain the cruising altitude

or flight level of that aircraft, as the case may be, by reference to an

altimeter that is set, when operating--

(1) Below 18,000 feet MSL, to--

(i) The current reported altimeter setting of a station along the route and

within 100 nautical miles of the aircraft;

(ii) If there is no station within the area prescribed in paragraph

(a)(1)(i) of this section, the current reported altimeter setting of an

appropriate available station; or

(iii) In the case of an aircraft not equipped with a radio, the elevation

of the departure airport or an appropriate altimeter setting available before

departure; or

(2) At or above 18,000 feet MSL, to 29.92'' Hg.

(b) The lowest usable flight level is determined by the atmospheric

pressure in the area of operation as shown in the following table:

Lowest

usable

Current altimeter   flight

setting        level

29.92 (or higher)       180

29.91 through 29.42     185

29.41 through 28.92     190

28.91 through 28.42     195

28.41 through 27.92     200

27.91 through 27.42     205

27.41 through 26.92     210

(c) To convert minimum altitude prescribed under Secs. 91.119 and 91.177 to

the minimum flight level, the pilot shall take the flight level equivalent of

the minimum altitude in feet and add the appropriate number of feet specified

below, according to the current reported altimeter setting:

Current altimeter   Adjustment

setting          factor

29.92 (or higher)          None

29.91 through 29.42         500

29.41 through 28.92       1,000

28.91 through 28.42       1,500

28.41 through 27.92       2,000

27.91 through 27.42       2,500

27.41 through 26.92       3,000

Sec. 91.123  Compliance with ATC clearances and instructions.

(a) When an ATC clearance has been obtained, a pilot in command may not

deviate from that clearance, except in an emergency, unless that pilot

obtains an amended clearance. However, except in Class A airspace, this

paragraph does not prohibit that pilot from canceling an IFR flight plan if

the operation is being conducted in VFR weather conditions. When a pilot is

uncertain of an ATC clearance, that pilot must immediately request

clarification from ATC.

(b) Except in an emergency, no person may operate an aircraft contrary to

an ATC instruction in an area in which air traffic control is exercised.

(c) Each pilot in command who, in an emergency, deviates from an ATC

clearance or instruction shall notify ATC of that deviation as soon as

possible.

(d) Each pilot in command who (though not deviating from a rule of this

subpart) is given priority by ATC in an emergency, shall submit a detailed

report of that emergency within 48 hours to the manager of that ATC facility,

if requested by ATC.

(e) Unless otherwise authorized by ATC, no person operating an aircraft may

operate that aircraft according to any clearance or instruction that has been

issued to the pilot of another aircraft for radar air traffic control

purposes.

Sec. 91.125  ATC light signals.

ATC light signals have the meaning shown in the following table:

Meaning with respect to   Meaning with respect to

Color and type of signal  aircraft on the surface      aircraft in flight

Steady green              Cleared for takeoff       Cleared to land.

Flashing green            Cleared to taxi           Return for landing (to

be followed by steady

green at proper time).

Steady red                Stop                      Give way to other

aircraft and continue

circling.

Flashing red              Taxi clear of runway in   Airport unsafe--do not

use                       land.

Flashing white            Return to starting point  Not applicable.

on airport

Alternating red and       Exercise extreme caution  Exercise extreme

green                                               caution.

Sec. 91.126  Operating on or in the vicinity of an airport in Class G

airspace.

(a) General. Unless otherwise authorized or required, each person operating

an aircraft on or in the vicinity of an airport in a Class G airspace area

must comply with the requirements of this section.

(b) Direction of turns. When approaching to land at an airport without an

operating control tower in Class G airspace--

(1) Each pilot of an airplane must make all turns of that airplane to the

left unless the airport displays approved light signals or visual markings

indicating that turns should be made to the right, in which case the pilot

must make all turns to the right; and

(2) Each pilot of a helicopter must avoid the flow of fixed-wing aircraft.

(c) Flap settings. Except when necessary for training or certification, the

pilot in command of a civil turbojet-powered aircraft must use, as a final

flap setting, the minimum certificated landing flap setting set forth in the

approved performance information in the Airplane Flight Manual for the

applicable conditions. However, each pilot in command has the final authority

and responsibility for the safe operation of the pilot's airplane, and may

use a different flap setting for that airplane if the pilot determines that

it is necessary in the interest of safety.

(d) Communications with control towers. Unless otherwise authorized or

required by ATC, no person may operate an aircraft to, from, through, or on

an airport having an operational control tower unless two-way radio

communications are maintained between that aircraft and the control tower.

Communications must be established prior to 4 nautical miles from the

airport, up to and including 2,500 feet AGL. However, if the aircraft radio

fails in flight, the pilot in command may operate that aircraft and land if

weather conditions are at or above basic VFR weather minimums, visual contact

with the tower is maintained, and a clearance to land is received. If the

aircraft radio fails while in flight under IFR, the pilot must comply with

Sec. 91.185.

Sec. 91.127  Operating on or in the vicinity of an airport in Class E

airspace.

(a) Unless otherwise required by part 93 of this chapter or unless

otherwise authorized or required by the ATC facility having jurisdiction over

the Class E airspace area, each person operating an aircraft on or in the

vicinity of an airport in a Class E airspace area must comply with the

requirements of Sec. 91.126.

(b) Departures. Each pilot of an aircraft must comply with any traffic

patterns established for that airport in part 93 of this chapter.

(c) Communications with control towers. Unless otherwise authorized or

required by ATC, no person may operate an aircraft to, from, through, or on

an airport having an operational control tower unless two-way radio

communications are maintained between that aircraft and the control tower.

Communications must be established prior to 4 nautical miles from the

airport, up to and including 2,500 feet AGL. However, if the aircraft radio

fails in flight, the pilot in command may operate that aircraft and land if

weather conditions are at or above basic VFR weather minimums, visual contact

with the tower is maintained, and a clearance to land is received. If the

aircraft radio fails while in flight under IFR, the pilot must comply with

Sec. 91.185.

Sec. 91.129  Operations in Class D airspace.

(a) General. Unless otherwise authorized or required by the ATC facility

having jurisdiction over the Class D airspace area, each person operating an

aircraft in Class D airspace must comply with the applicable provisions of

this section. In addition, each person must comply with Secs. 91.126 and

91.127. For the purpose of this section, the primary airport is the airport

for which the Class D airspace area is designated. A satellite airport is any

other airport within the Class D airspace area.

(b) Deviations. An operator may deviate from any provision of this section

under the provisions of an ATC authorization issued by the ATC facility

having jurisdiction over the airspace concerned. ATC may authorize a

deviation on a continuing basis or for an individual flight, as appropriate.

(c) Communications. Each person operating an aircraft in Class D airspace

must meet the following two-way radio communications requirements:

(1) Arrival or through flight. Each person must establish two-way radio

communications with the ATC facility (including foreign ATC in the case of

foreign airspace designated in the United States) providing air traffic

services prior to entering that airspace and thereafter maintain those

communications while within that airspace.

(2) Departing flight. Each person--

(i) From the primary airport or satellite airport with an operating control

tower must establish and maintain two-way radio communications with the

control tower, and thereafter as instructed by ATC while operating in the

Class D airspace area; or

(ii) From a satellite airport without an operating control tower, must

establish and maintain two-way radio communications with the ATC facility

having jurisdiction over the Class D airspace area as soon as practicable

after departing.

(d) Communications failure. Each person who operates an aircraft in a Class

D airspace area must maintain two-way radio communications with the ATC

facility having jurisdiction over that area.

(1) If the aircraft radio fails in flight under IFR, the pilot must comply

with Sec. 91.185 of the part.

(2) If the aircraft radio fails in flight under VFR, the pilot in command

may operate that aircraft and land if--

(i) Weather conditions are at or above basic VFR weather minimums;

(ii) Visual contact with the tower is maintained; and

(iii) A clearance to land is received.

(e) Minimum Altitudes. When operating to an airport in Class D airspace,

each pilot of--

(1) A large or turbine-powered airplane shall, unless otherwise required

by the applicable distance from cloud criteria, enter the traffic pattern at

an altitude of at least 1,500 feet above the elevation of the airport and

maintain at least 1,500 feet until further descent is required for a safe

landing;

(2) A large or turbine-powered airplane approaching to land on a runway

served by an instrument landing system (ILS), if the airplane is ILS

equipped, shall fly that airplane at an altitude at or above the glide slope

between the outer marker (or point of interception of glide slope, if

compliance with the applicable distance from clouds criteria requires

interception closer in) and the middle marker; and

(3) An airplane approaching to land on a runway served by a visual

approach slope indicator shall maintain an altitude at or above the glide

slope until a lower altitude is necessary for safe landing.

Paragraphs (e)(2) and (e)(3) of this section do not prohibit normal

bracketing maneuvers above or below the glide slope that are conducted for

the purpose of remaining on the glide slope.

(f) Approaches. Except when conducting a circling approach under Part 97 of

this chapter or unless otherwise required by ATC, each pilot must--

(1) Circle the airport to the left, if operating an airplane; or

(2) Avoid the flow of fixed-wing aircraft, if operating a helicopter.

(g) Departures. No person may operate an aircraft departing from an airport

except in compliance with the following:

(1) Each pilot must comply with any departure procedures established for

that airport by the FAA.

(2) Unless otherwise required by the prescribed departure procedure for

that airport or the applicable distance from clouds criteria, each pilot of a

turbine-powered airplane and each pilot of a large airplane must climb to an

altitude of 1,500 feet above the surface as rapidly as practicable.

(h) Noise abatement. Where a formal runway use program has been established

by the FAA, each pilot of a large or turbine-powered airplane assigned a

noise abatement runway by ATC must use that runway. However, consistent with

the final authority of the pilot in command concerning the safe operation of

the aircraft as prescribed in Sec. 91.3(a), ATC may assign a different runway

if requested by the pilot in the interest of safety.

(i) Takeoff, landing, taxi clearance. No person may, at any airport with an

operating control tower, operate an aircraft on a runway or taxiway, or take

off or land an aircraft, unless an appropriate clearance is received from

ATC. A clearance to "taxi to" the takeoff runway assigned to the aircraft is

not a clearance to cross that assigned takeoff runway, or to taxi on that

runway at any point, but is a clearance to cross other runways that intersect

the taxi route to that assigned takeoff runway. A clearance to "taxi to" any

point other than an assigned takeoff runway is clearance to cross all runways

that intersect the taxi route to that point.

Sec. 91.130  Operations in Class C airspace.

(a) General. Unless otherwise authorized by ATC, each aircraft operation in

Class C airspace must be conducted in compliance with this section and Sec.

91.129. For the purpose of this section, the primary airport is the airport

for which the Class C airspace area is designated. A satellite airport is any

other airport within the Class C airspace area.

(b) Traffic patterns. No person may take off or land an aircraft at a

satellite airport within a Class C airspace area except in compliance with

FAA arrival and departure traffic patterns.

(c) Communications. Each person operating an aircraft in Class C airspace

must meet the following two-way radio communications requirements:

(1) Arrival or through flight. Each person must establish two-way radio

communications with the ATC facility (including foreign ATC in the case of

foreign airspace designated in the United States) providing air traffic

services prior to entering that airspace and thereafter maintain those

communications while within that airspace.

(2) Departing flight. Each person--

(i) From the primary airport or satellite airport with an operating control

tower must establish and maintain two-way radio communications with the

control tower, and thereafter as instructed by ATC while operating in the

Class C airspace area; or

(ii) From a satellite airport without an operating control tower, must

establish and maintain two-way radio communications with the ATC facility

having jurisdiction over the Class C airspace area as soon as practicable

after departing.

(d) Equipment requirements. Unless otherwise authorized by the ATC having

jurisdiction over the Class C airspace area, no person may operate an

aircraft within a Class C airspace area designated for an airport unless that

aircraft is equipped with the applicable equipment specified in Sec. 91.215.

(e) Deviations. An operator may deviate from any provision of this section

under the provisions of an ATC authorization issued by the ATC facility

having jurisdiction over the airspace concerned. ATC may authorize a

deviation on a continuing basis or for an individual flight, as appropriate.

Sec. 91.131  Operations in Class B airspace.

(a) Operating rules. No person may operate an aircraft within a Class B

airspace area except in compliance with Sec. 91.129 and the following rules:

(1) The operator must receive an ATC clearance from the ATC facility having

jurisdiction for that area before operating an aircraft in that area.

(2) Unless otherwise authorized by ATC, each person operating a large

turbine engine-powered airplane to or from a primary airport for which a

Class B airspace area is designated must operate at or above the designated

floors of the Class B airspace area while within the lateral limits of that

area.

(3) Any person conducting pilot training operations at an airport within a

Class B airspace area must comply with any procedures established by ATC for

such operations in that area.

(b) Pilot requirements.

(1) No person may take off or land a civil aircraft at an airport within a

Class B airspace area or operate a civil aircraft within a Class B airspace

area unless--

(i) The pilot in command holds at least a private pilot certificate; or

(ii) The aircraft is operated by a student pilot or recreational pilot who

seeks private pilot certification and has met the requirements of Sec. 61.95

of this chapter.

(2) Notwithstanding the provisions of paragraph (b)(1)(ii) of this section,

no person may take off or land a civil aircraft at those airports listed in

section 4 of appendix D of this part unless the pilot in command holds at

least a private pilot certificate.

(c) Communications and navigation equipment requirements. Unless otherwise

authorized by ATC, no person may operate an aircraft within a Class B

airspace area unless that aircraft is equipped with--

(1) For IFR operation. An operable VOR or TACAN receiver; and

(2) For all operations. An operable two-way radio capable of communications

with ATC on appropriate frequencies for that Class B airspace area.

(d) Transponder requirements. No person may operate an aircraft in a Class

B airspace area unless the aircraft is equipped with the applicable operating

transponder and automatic altitude reporting equipment specified in paragraph

(a) of Sec. 91.215, except as provided in paragraph (d) of that section.

Sec. 91.133  Restricted and prohibited areas.

(a) No person may operate an aircraft within a restricted area (designated

in part 73) contrary to the restrictions imposed, or within a prohibited

area, unless that person has the permission of the using or controlling

agency, as appropriate.

(b) Each person conducting, within a restricted area, an aircraft operation

(approved by the using agency) that creates the same hazards as the

operations for which the restricted area was designated may deviate from the

rules of this subpart that are not compatible with the operation of the

aircraft.

Sec. 91.135  Operations in Class A airspace.

Except as provided in paragraph (d) of this section, each person operating

an aircraft in Class A airspace must conduct that operation under instrument

flight rules (IFR) and in compliance with the following:

(a) Clearance. Operations may be conducted only under an ATC clearance

received prior to entering the airspace.

(b) Communications. Unless otherwise authorized by ATC, each aircraft

operating in Class A airspace must be equipped with a two-way radio capable

of communicating with ATC on a frequency assigned by ATC. Each pilot must

maintain two-way radio communications with ATC while operating in Class A

airspace.

(c) Transponder requirement. Unless otherwise authorized by ATC, no person

may operate an aircraft within Class A airspace unless that aircraft is

equipped with the applicable equipment specified in Sec. 91.215.

(d) ATC authorizations. An operator may deviate from any provision of this

section under the provisions of an ATC authorization issued by the ATC

facility having jurisdiction of the airspace concerned. In the case of an

inoperative transponder, ATC may immediately approve an operation within a

Class A airspace area allowing flight to continue, if desired, to the airport

of ultimate destination, including any intermediate stops, or to proceed to a

place where suitable repairs can be made, or both. Requests for deviation

from any provision of this section must be submitted in writing, at least 4

days before the proposed operation. ATC may authorize a deviation on a

continuing basis or for an individual flight.

Sec. 91.137  Temporary flight restrictions.

(a) The Administrator will issue a Notice to Airmen (NOTAM) designating an

area within which temporary flight restrictions apply and specifying the

hazard or condition requiring their imposition, whenever he determines it is

necessary in order to--

(1) Protect persons and property on the surface or in the air from a hazard

associated with an incident on the surface;

(2) Provide a safe environment for the operation of disaster relief

aircraft; or

(3) Prevent an unsafe congestion of sightseeing and other aircraft above an

incident or event which may generate a high degree of public interest.

The Notice to Airmen will specify the hazard or condition that requires the

imposition of temporary flight restrictions.

(b) When a NOTAM has been issued under paragraph (a)(1) of this section, no

person may operate an aircraft within the designated area unless that

aircraft is participating in the hazard relief activities and is being

operated under the direction of the official in charge of on scene emergency

response activities.

(c) When a NOTAM has been issued under paragraph (a)(2) of this section, no

person may operate an aircraft within the designated area unless at least one

of the following conditions are met:

(1) The aircraft is participating in hazard relief activities and is being

operated under the direction of the official in charge of on scene emergency

response activities.

(2) The aircraft is carrying law enforcement officials.

(3) The aircraft is operating under the ATC approved IFR flight plan.

(4) The operation is conducted directly to or from an airport within the

area, or is necessitated by the impracticability of VFR flight above or

around the area due to weather, or terrain; notification is given to the

Flight Service Station (FSS) or ATC facility specified in the NOTAM to

receive advisories concerning disaster relief aircraft operations; and the

operation does not hamper or endanger relief activities and is not conducted

for the purpose of observing the disaster.

(5) The aircraft is carrying properly accredited news representatives, and,

prior to entering the area, a flight plan is filed with the appropriate FAA

or ATC facility specified in the Notice to Airmen and the operation is

conducted above the altitude used by the disaster relief aircraft, unless

otherwise authorized by the official in charge of on scene emergency response

activities.

(d) When a NOTAM has been issued under paragraph (a)(3) of this section, no

person may operate an aircraft within the designated area unless at least one

of the following conditions is met:

(1) The operation is conducted directly to or from an airport within the

area, or is necessitated by the impracticability of VFR flight above or

around the area due to weather or terrain, and the operation is not conducted

for the purpose of observing the incident or event.

(2) The aircraft is operating under an ATC approved IFR flight plan.

(3) The aircraft is carrying incident or event personnel, or law

enforcement officials.

(4) The aircraft is carrying properly accredited news representatives and,

prior to entering that area, a flight plan is filed with the appropriate FSS

or ATC facility specified in the NOTAM.

(e) Flight plans filed and notifications made with an FSS or ATC facility

under this section shall include the following information:

(1) Aircraft identification, type and color.

(2) Radio communications frequencies to be used.

(3) Proposed times of entry of, and exit from, the designated area.

(4) Name of news media or organization and purpose of flight.

(5) Any other information requested by ATC.

Sec. 91.138  Temporary flight restrictions in national disaster areas in

the State of Hawaii.

(a) When the Administrator has determined, pursuant to a request and

justification provided by the Governor of the State of Hawaii, or the

Governor's designee, that an inhabited area within a declared national

disaster area in the State of Hawaii is in need of protection for

humanitarian reasons, the Administrator will issue a Notice to Airmen (NOTAM)

designating an area within which temporary flight restrictions apply. The

Administrator will designate the extent and duration of the temporary flight

restrictions necessary to provide for the protection of persons and property

on the surface.

(b) When a NOTAM has been issued in accordance with this section, no person

may operate an aircraft within the designated airspace unless:

(1) That person has obtained authorization from the official in charge of

associated emergency or disaster relief response activities, and is operating

the aircraft under the conditions of that authorization;

(2) The aircraft is carrying law enforcement officials;

(3) The aircraft is carrying persons involved in an emergency or a

legitimate scientific purpose;

(4) The aircraft is carrying properly accredited newspersons, and that

prior to entering the area, a flight plan is filed with the appropriate FAA

or ATC facility specified in the NOTAM and the operation is conducted in

compliance with the conditions and restrictions established by the official

in charge of on-scene emergency response activities; or,

(5) The aircraft is operating in accordance with an ATC clearance or

instruction.

(c) A NOTAM issued under this section is effective for 90 days or until the

national disaster area designation is terminated, whichever comes first,

unless terminated by notice or extended by the Administrator at the request

of the Governor of the State of Hawaii or the Governor's designee.

Sec. 91.139  Emergency air traffic rules.

(a) This section prescribes a process for utilizing Notices to Airmen

(NOTAMs) to advise of the issuance and operations under emergency air traffic

rules and regulations and designates the official who is authorized to issue

NOTAMs on behalf of the Administrator in certain matters under this section.

(b) Whenever the Administrator determines that an emergency condition

exists, or will exist, relating to the FAA's ability to operate the air

traffic control system and during which normal flight operations under this

chapter cannot be conducted consistent with the required levels of safety and

efficiency--

(1) The Administrator issues an immediately effective air traffic rule or

regulation in response to that emergency condition; and

(2) The Administrator or the Associate Administrator for Air Traffic may

utilize the NOTAM system to provide notification of the issuance of the rule

or regulation.

Those NOTAMs communicate information concerning the rules and regulations

that govern flight operations, the use of navigation facilities, and

designation of that airspace in which the rules and regulations apply.

(c) When a NOTAM has been issued under this section, no person may operate

an aircraft, or other device governed by the regulation concerned, within the

designated airspace except in accordance with the authorizations, terms, and

conditions prescribed in the regulation covered by the NOTAM.

Sec. 91.141  Flight restrictions in the proximity of the Presidential ad

other parties.

No person may operate an aircraft over or in the vicinity of any area to be

visited or traveled by the President, the Vice President, or other public

figures contrary to the restrictions established by the Administrator and

published in a Notice to Airmen (NOTAM).

Effective Date Note: At 54 FR 34291, August 18, 1989, Sec. 91.141 was

added effective August 18, 1990.

Sec. 91.143  Flight limitation in the proximity of space flight operations.

No person may operate any aircraft of U.S. registry, or pilot any aircraft

under the authority of an airman certificate issued by the Federal Aviation

Administration within areas designated in a Notice to Airmen (NOTAM) for

space flight operations except when authorized by ATC, or operated under the

control of the Department of Defense Manager for Space Transportation System

Contingency Support Operations.

Effective Date Note: At 54 FR 34291, August 18, 1989, Sec. 91.143 was

added effective August 18, 1990.

91.144  Temporary Restriction on Flight Operations During Abnormally High

Barometric Pressure Conditions.

(a) Special flight restrictions. When any information indicates that

barometric pressure on the route of flight currently exceeds or will exceed

31 inches of mercury, no person may operate an aircraft or initiate a flight

contrary to the requirements established by the Administrator and published

in a Notice to Airmen issued under this section.

(b) Waivers. The Administrator is authorized to waive any restriction

issued under paragraph (a) of this section to permit emergency supply,

transport, or medical services to be delivered to isolated communities, where

the operation can be conducted with an acceptable level of safety.

Secs. 91.145--91.149  [Reserved]

Visual Flight Rules

Sec. 91.151  Fuel requirements for flight in VFR conditions.

(a) No person may begin a flight in an airplane under VFR conditions unless

(considering wind and forecast weather conditions) there is enough fuel to

fly to the first point of intended landing and, assuming normal cruising

speed--

(1) During the day, to fly after that for at least 30 minutes; or

(2) At night, to fly after that for at least 45 minutes.

(b) No person may begin a flight in a rotorcraft under VFR conditions

unless (considering wind and forecast weather conditions) there is enough

fuel to fly to the first point of intended landing and, assuming normal

cruising speed, to fly after that for at least 20 minutes.

Sec. 91.153  VFR flight plan: Information required.

(a) Information required. Unless otherwise authorized by ATC, each person

filing a VFR flight plan shall include in it the following information:

(1) The aircraft identification number and, if necessary, its radio call

sign.

(2) The type of the aircraft or, in the case of a formation flight, the

type of each aircraft and the number of aircraft in the formation.

(3) The full name and address of the pilot in command or, in the case of a

formation flight, the formation commander.

(4) The point and proposed time of departure.

(5) The proposed route, cruising altitude (or flight level), and true

airspeed at that altitude.

(6) The point of first intended landing and the estimated elapsed time

until over that point.

(7) The amount of fuel on board (in hours).

(8) The number of persons in the aircraft, except where that information is

otherwise readily available to the FAA.

(9) Any other information the pilot in command or ATC believes is necessary

for ATC purposes.

(b) Cancellation. When a flight plan has been activated, the pilot in

command, upon canceling or completing the flight under the flight plan, shall

notify an FAA Flight Service Station or ATC facility.

Sec. 91.155   Basic VFR weather minimums.

(a) Except as provided in paragraph (b) of this section and Sec. 91.157, no

person may operate an aircraft under VFR when the flight visibility is less,

or at a distance from clouds that is less, than that prescribed for the

corresponding altitude and class of airspace in the following table:

Flight

Airspace                visibility        Distance from clouds

Class A                           Not Applicable   Not Applicable.

Class B                           3 statute miles  Clear of Clouds.

Class C                           3 statute miles  500 feet below.

1,000 feet above.

2,000 feet horizontal.

Class D                           3 statute miles  500 feet below.

1,000 feet above.

2,000 feet horizontal.

Class E:

Less than 10,000 feet MSL        3 statute miles  500 feet below.

1,000 feet above.

2,000 feet horizontal

At or above 10,000 feet MSL      5 statute miles  1,000 feet below.

1,000 feet above.

1 statute mile horizontal.

Class G:

1,200 feet or less above the

surface (regardless of MSL

altitude)

Day, except as provided in Sec.

91.155(b)                        1 statute mile   Clear of clouds.

Night, except as provided in

Sec. 91.155(b)                   3 statute miles  500 feet below.

1,000 feet above.

2,000 feet horizontal.

More than 1,200 feet above the

surface but less than 10,000

feet MSL

Day                               1 statute mile   500 feet below.

1,000 feet above.

2,000 feet horizontal.

Night                             3 statute miles  500 feet below.

1,000 feet above.

2,000 feet horizontal.

More than 1,200 feet above the

surface and at or above 10,000

feet MSL                         5 statute miles  1,000 feet below.

1,000 feet above.

1 statute mile horizontal.

(b) Class G Airspace. Notwithstanding the provisions of paragraph (a) of

this section, the following operations may be conducted in Class G airspace

below 1,200 feet above the surface:

(1) Helicopter. A helicopter may be operated clear of clouds if operated at

a speed that allows the pilot adequate opportunity to see any air traffic or

obstruction in time to avoid a collision.

(2) Airplane. When the visibility is less than 3 statute miles but not less

than 1 statute mile during night hours, an airplane may be operated clear of

clouds if operated in an airport traffic pattern within one-half mile of the

runway.

(c) Except as provided in Sec. 91.157, no person may operate an aircraft

beneath the ceiling under VFR within the lateral boundaries of controlled

airspace designated to the surface for an airport when the ceiling is less

than 1,000 feet.

(d) Except as provided in Sec. 91.157 of this part, no person may take off

or land an aircraft, or enter the traffic pattern of an airport, under VFR,

within the lateral boundaries of the surface areas of Class B, Class C, Class

D, or Class E airspace designated for an airport--

(1) Unless ground visibility at that airport is at least 3 statute miles;

or

(2) If ground visibility is not reported at that airport, unless flight

visibility during landing or takeoff, or while operating in the traffic

pattern is at least 3 statute miles.

(e) For the purpose of this section, an aircraft operating at the base

altitude of a Class E airspace area is considered to be within the airspace

directly below that area.

Sec. 91.157  Special VFR weather minimums.

(a) Except as provided in appendix D, section 3, of this part, special VFR

operations may be conducted under the weather minimums and requirements of

this section, instead of those contained in Sec. 91.155, below 10,000 feet

MSL within the airspace contained by the upward extension of the lateral

boundaries of the controlled airspace designated to the surface for an

airport.

(b) Special VFR operations may only be conducted--

(1) With an ATC clearance;

(2) Clear of clouds;

(3) Except for helicopters, when flight visibility is at least 1 statute

mile; and

(4) Except for helicopters, between sunrise and sunset (or in Alaska, when

the sun is 6 deg. or more above the horizon) unless--

(i) The person being granted the ATC clearance meets the applicable

requirements for instrument flight under part 61 of this chapter; and

(ii) The aircraft is equipped as required in Sec. 91.205(d).

(c) No person may take off or land an aircraft (other than a helicopter)

under special VFR--

(1) Unless ground visibility is at least 1 statute mile; or

(2) If ground visibility is not reported, unless flight visibility is at

least 1 statute mile.

Sec. 91.159  VFR cruising altitude or flight level.

Except while holding in a holding pattern of 2 minutes or less, or while

turning, each person operating an aircraft under VFR in level cruising flight

more than 3,000 feet above the surface shall maintain the appropriate

altitude or flight level prescribed below, unless otherwise authorized by

ATC:

(a) When operating below 18,000 feet MSL and--

(1) On a magnetic course of zero degrees through 179 degrees, any odd

thousand foot MSL altitude +500 feet (such as 3,500, 5,500, or 7,500); or

(2) On a magnetic course of 180 degrees through 359 degrees, any even

thousand foot MSL altitude +500 feet (such as 4,500, 6,500, or 8,500).

(b) When operating above 18,000 feet MSL to flight level 290 (inclusive)

and--

(1) On a magnetic course of zero degrees through 179 degrees, any odd

flight level +500 feet (such as 195, 215, or 235); or

(2) On a magnetic course of 180 degrees through 359 degrees, any even

flight level +500 feet (such as 185, 205, or 225).

(c) When operating above flight level 290 and--

(1) On a magnetic course of zero degrees through 179 degrees, any flight

level, at 4,000-foot intervals, beginning at and including flight level 300

(such as flight level 300, 340, or 380); or

(2) On a magnetic course of 180 degrees through 359 degrees, any flight

level, at 4,000-foot intervals, beginning at and including flight level 320

(such as flight level 320, 360, or 400).

Secs. 91.161--91.165  [Reserved]

Instrument Flight Rules

Sec. 91.167  Fuel requirements for flight in IFR conditions.

(a) Except as provided in paragraph (b) of this section, no person may

operate a civil aircraft in IFR conditions unless it carries enough fuel

(considering weather reports and forecasts and weather conditions) to--

(1) Complete the flight to the first airport of intended landing;

(2) Fly from that airport to the alternate airport; and

(3) Fly after that for 45 minutes at normal cruising speed or, for

helicopters, fly after that for 30 minutes at normal cruising speed.

(b) Paragraph (a)(2) of this section does not apply if--

(1) Part 97 of this chapter prescribes a standard instrument approach

procedure for the first airport of intended landing; and

(2) For at least 1 hour before and 1 hour after the estimated time of

arrival at the airport, the weather reports or forecasts or any combination

of them indicate--

(i) The ceiling will be at least 2,000 feet above the airport elevation;

and

(ii) Visibility will be at least 3 statute miles.

Sec. 91.169  IFR flight plan: Information required.

(a) Information required. Unless otherwise authorized by ATC, each person

filing an IFR flight plan shall include in it the following information:

(1) Information required under Sec. 91.153(a).

(2) An alternate airport, except as provided in paragraph (b) of this

section.

(b) Exceptions to applicability of paragraph (a)(2) of this section.

Paragraph (a)(2) of this section does not apply if part 97 of this chapter

prescribes a standard instrument approach procedure for the first airport of

intended landing and, for at least 1 hour before and 1 hour after the

estimated time of arrival, the weather reports or forecasts, or any

combination of them, indicate--

(1) The ceiling will be at least 2,000 feet above the airport elevation;

and

(2) The visibility will be at least 3 statute miles.

(c) IFR alternate airport weather minimums. Unless otherwise authorized by

the Administrator, no person may include an alternate airport in an IFR

flight plan unless current weather forecasts indicate that, at the estimated

time of arrival at the alternate airport, the ceiling and visibility at that

airport will be at or above the following alternate airport weather minimums:

(1) If an instrument approach procedure has been published in part 97 of

this chapter for that airport, the alternate airport minimums specified in

that procedure or, if none are so specified, the following minimums:

(i) Precision approach procedure: Ceiling 600 feet and visibility 2 statute

miles.

(ii) Nonprecision approach procedure: Ceiling 800 feet and visibility 2

statute miles.

(2) If no instrument approach procedure has been published in part 97 of

this chapter for that airport, the ceiling and visibility minimums are those

allowing descent from the MEA, approach, and landing under basic VFR.

(d) Cancellation. When a flight plan has been activated, the pilot in

command, upon canceling or completing the flight under the flight plan, shall

notify an FAA Flight Service Station or ATC facility.

Sec. 91.171  VOR equipment check for IFR operations.

(a) No person may operate a civil aircraft under IFR using the VOR system

of radio navigation unless the VOR equipment of that aircraft--

(1) Is maintained, checked, and inspected under an approved procedure; or

(2) Has been operationally checked within the preceding 30 days, and was

found to be within the limits of the permissible indicated bearing error set

forth in paragraph (b) or (c) of this section.

(b) Except as provided in paragraph (c) of this section, each person

conducting a VOR check under paragraph (a)(2) of this section shall--

(1) Use, at the airport of intended departure, an FAA-operated or approved

test signal or a test signal radiated by a certificated and appropriately

rated radio repair station or, outside the United States, a test signal

operated or approved by an appropriate authority to check the VOR equipment

(the maximum permissible indicated bearing error is plus or minus 4 degrees);

or

(2) Use, at the airport of intended departure, a point on the airport

surface designated as a VOR system checkpoint by the Administrator, or,

outside the United States, by an appropriate authority (the maximum

permissible bearing error is plus or minus 4 degrees);

(3) If neither a test signal nor a designated checkpoint on the surface is

available, use an airborne checkpoint designated by the Adninistrator or,

outside the United States, by an appropriate authority (the maximum

permissible bearing error is plus or minus 6 degrees); or

(4) If no check signal or point is available, while in flight--

(i) Select a VOR radial that lies along the centerline of an established

VOR airway;

(ii) Select a prominent ground point along the selected radial preferably

more than 20 nautical miles from the VOR ground facility and maneuver the

aircraft directly over the point at a reasonably low altitude; and

(iii) Note the VOR bearing indicated by the receiver when over the ground

point (the maximum permissible variation between the published radial and the

indicated bearing is 6 degrees).

(c) If dual system VOR (units independent of each other except for the

antenna) is installed in the aircraft, the person checking the equipment may

check one system against the other in place of the check procedures specified

in paragraph (b) of this section. Both systems shall be tuned to the same VOR

ground facility and note the indicated bearings to that station. The maximum

permissible variation between the two indicated bearings is 4 degrees.

(d) Each person making the VOR operational check, as specified in paragraph

(b) or (c) of this section, shall enter the date, place, bearing error, and

sign the aircraft log or other record. In addition, if a test signal radiated

by a repair station, as specified in paragraph (b)(1) of this section, is

used, an entry must be made in the aircraft log or other record by the repair

station certificate holder or the certificate holder's representative

certifying to the bearing transmitted by the repair station for the check and

the date of transmission.

Sec. 91.173  ATC clearance and flight plan required.

No person may operate an aircraft in controlled airspace under IFR unless

that person has--

(a) Filed an IFR flight plan; and

(b) Received an appropriate ATC clearance.

Sec. 91.175  Takeoff and landing under IFR.

(a) Instrument approaches to civil airports.

Unless otherwise authorized by the Administrator, when an instrument

letdown to a civil airport is necessary, each person operating an aircraft,

except a military aircraft of the United States, shall use a standard

instrument approach procedure prescribed for the airport in part 97 of this

chapter.

(b) Authorized DH or MDA. For the purpose of this section, when the

approach procedure being used provides for and requires the use of a DH or

MDA, the authorized DH or MDA is the highest of the following:

(1) The DH or MDA prescribed by the approach procedure.

(2) The DH or MDA prescribed for the pilot in command.

(3) The DH or MDA for which the aircraft is equipped.

(c) Operation below DH or MDA. Where a DH or MDA is applicable, no pilot

may operate an aircraft, except a military aircraft of the United States, at

any airport below the authorized MDA or continue an approach below the

authorized DH unless--

(1) The aircraft is continuously in a position from which a descent to a

landing on the intended runway can be made at a normal rate of descent using

normal maneuvers, and for operations conducted under part 121 or part 135

unless that descent rate will allow touchdown to occur within the touchdown

zone of the runway of intended landing;

(2) The flight visibility is not less than the visibility prescribed in the

standard instrument approach being used; and

(3) Except for a Category II or Category III approach where any necessary

visual reference requirements are specified by the Administrator, at least

one of the following visual references for the intended runway is distinctly

visible and identifiable to the pilot:

(i) The approach light system, except that the pilot may not descend below

100 feet above the touchdown zone elevation using the approach lights as a

reference unless the red terminating bars or the red side row bars are also

distinctly visible and identifiable.

(ii) The threshold.

(iii) The threshold markings.

(iv) The threshold lights.

(v) The runway end identifier lights.

(vi) The visual approach slope indicator.

(vii) The touchdown zone or touchdown zone markings.

(viii) The touchdown zone lights.

(ix) The runway or runway markings.

(x) The runway lights.

(d) Landing. No pilot operating an aircraft, except a military aircraft of

the United States, may land that aircraft when the flight visibility is less

than the visibility prescribed in the standard instrument approach procedure

being used.

(e) Missed approach procedures. Each pilot operating an aircraft, except a

military aircraft of the United States, shall immediately execute an

appropriate missed approach procedure when either of the following conditions

exist:

(1) Whenever the requirements of paragraph (c) of this section are not met

at either of the following times:

(i) When the aircraft is being operated below MDA; or

(ii) Upon arrival at the missed approach point, including a DH where a DH

is specified and its use is required, and at any time after that until

touchdown.

(2) Whenever an identifiable part of the airport is not distinctly visible

to the pilot during a circling maneuver at or above MDA, unless the inability

to see an identifiable part of the airport results only from a normal bank of

the aircraft during the circling approach.

(f) Civil airport takeoff minimums. Unless otherwise authorized by the

Administrator, no pilot operating an aircraft under parts 121, 125, 127, 129,

or 135 of this chapter may take off from a civil airport under IFR unless

weather conditions are at or above the weather minimum for IFR takeoff

prescribed for that airport under part 97 of this chapter. If takeoff

minimums are not prescribed under part 97 of this chapter for a particular

airport, the following minimums apply to takeoffs under IFR for aircraft

operating under those parts:

(1) For aircraft, other than helicopters, having two engines or less--1

statute mile visibility.

(2) For aircraft having more than two engines-- 1/2  statute mile

visibility.

(3) For helicopters--1/2 statute mile visibility.

(g) Military airports. Unless otherwise prescribed by the Administrator,

each person operating a civil aircraft under IFR into or out of a military

airport shall comply with the instrument approach procedures and the takeoff

and landing minimum prescribed by the military authority having jurisdiction

of that airport.

(h) Comparable values of RVR and ground visibility. (1) Except for Category

II or Category III minimums, if RVR minimums for takeoff or landing are

prescribed in an instrument approach procedure, but RVR is not reported for

the runway of intended operation, the RVR minimum shall be converted to

ground visibility in accordance with the table in paragraph (h)(2) of this

section and shall be the visibility minimum for takeoff or landing on that

runway.

Visibility

(statute

RVR (feet)                      miles)

1,600 . . . . . . . . . . . . . . . . . . . .         1/4

2,400 . . . . . . . . . . . . . . . . . . . .         1/2

3,200 . . . . . . . . . . . . . . . . . . . .         5/8

4,000 . . . . . . . . . . . . . . . . . . . .         3/4

4,500 . . . . . . . . . . . . . . . . . . . .         7/8

5,000 . . . . . . . . . . . . . . . . . . . .           1

6,000 . . . . . . . . . . . . . . . . . . . .       1 1/4

(i) Operations on unpublished routes and use of radar in instrument

approach procedures. When radar is approved at certain locations for ATC

purposes, it may be used not only for surveillance and precision radar

approaches, as applicable, but also may be used in conjunction with

instrument approach procedures predicated on other types of radio

navigational aids. Radar vectors may be authorized to provide course guidance

through the segments of an approach to the final course or fix. When

operating on an unpublished route or while being radar vectored, the pilot,

when an approach clearance is received, shall, in addition to complying with

Sec. 91.177, maintain the last altitude assigned to that pilot until the

aircraft is established on a segment of a published route or instrument

approach procedure unless a different altitude is assigned by ATC. After the

aircraft is so established, published altitudes apply to descent within each

succeeding route or approach segment unless a different altitude is assigned

by ATC. Upon reaching the final approach course or fix, the pilot may either

complete the instrument approach in accordance with a procedure approved for

the facility or continue a surveillance or precision radar approach to a

landing.

(j) Limitation on procedure turns. In the case of a radar vector to a final

approach course or fix, a timed approach from a holding fix, or an approach

for which the procedure specifies "No PT," no pilot may make a procedure turn

unless cleared to do so by ATC.

(k) ILS components. The basic ground components of an ILS are the

localizer, glide slope, outer marker, middle marker, and, when installed for

use with Category II or Category III instrument approach procedures, an inner

marker. A compass locator or precision radar may be substituted for the outer

or middle marker. DME, VOR, or nondirectional beacon fixes authorized in the

standard instrument approach procedure or surveillance radar may be

substituted for the outer marker. Applicability of, and substitution for, the

inner marker for Category II or III approaches is determined by the

appropriate part 97 approach procedure, letter of authorization, or

operations specification pertinent to the operations.

Sec. 91.177  Minimum altitudes for IFR operations.

(a) Operation of aircraft at minimum altitudes. Except when necessary for

takeoff or landing, no person may operate an aircraft under IFR below--

(1) The applicable minimum altitudes prescribed in Parts 95 and 97 of this

chapter; or

(2) If no applicable minimum altitude is prescribed in those parts--

(i) In the case of operations over an area designated as a mountainous area

in part 95, an altitude of 2,000 feet above the highest obstacle within a

horizontal distance of 4 nautical miles from the course to be flown; or

(ii) In any other case, an altitude of 1,000 feet above the highest

obstacle within a horizontal distance of 4 nautical miles from the course to

be flown.

However, if both a MEA and a MOCA are prescribed for a particular route or

route segment, a person may operate an aircraft below the MEA down to, but

not below, the MOCA, when within 22 nautical miles of the VOR concerned

(based on the pilot's reasonable estimate of that distance).

(b) Climb. Climb to a higher minimum IFR altitude shall begin immediately

after passing the point beyond which that minimum altitude applies, except

that when ground obstructions intervene, the point beyond which that higher

minimum altitude applies shall be crossed at or above the applicable MCA.

Sec. 91.179  IFR cruising altitude or flight level.

(a) In controlled airspace. Each person operating an aircraft under IFR in

level cruising flight in controlled airspace shall maintain the altitude or

flight level assigned that aircraft by ATC. However, if the ATC clearance

assigns "VFR conditions on-top," that person shall maintain an altitude or

flight level as prescribed by Sec. 91.159.

(b) In uncontrolled airspace. Except while in a holding pattern of 2

minutes or less or while turning, each person operating an aircraft under IFR

in level cruising flight in uncontrolled airspace shall maintain an

appropriate altitude as follows:

(1) When operating below 18,000 feet MSL and--

(i) On a magnetic course of zero degrees through 179 degrees, any odd

thousand foot MSL altitude (such as 3,000, 5,000, or 7,000); or

(ii) On a magnetic course of 180 degrees through 359 degrees, any even

thousand foot MSL altitude (such as 2,000, 4,000, or 6,000).

(2) When operating at or above 18,000 feet MSL but below flight level 290,

and--

(i) On a magnetic course of zero degrees through 179 degrees, any odd

flight level (such as 190, 210, or 230); or

(ii) On a magnetic course of 180 degrees through 359 degrees, any even

flight level (such as 180, 200, or 220).

(3) When operating at flight level 290 and above, and--

(i) On a magnetic course of zero degrees through 179 degrees, any flight

level, at 4,000-foot intervals, beginning at and including flight level 290

(such as flight level 290, 330, or 370); or

(ii) On a magnetic course of 180 degrees through 359 degrees, any flight

level, at 4,000-foot intervals, beginning at and including flight level 310

(such as flight level 310, 350, or 390).

Sec. 91.181  Course to be flown.

Unless otherwise authorized by ATC, no person may operate an aircraft

within controlled airspace under IFR except as follows:

(a) On a Federal airway, along the centerline of that airway.

(b) On any other route, along the direct course between the navigational

aids or fixes defining that route. However, this section does not prohibit

maneuvering the aircraft to pass well clear of other air traffic or the

maneuvering of the aircraft in VFR conditions to clear the intended flight

path both before and during climb or descent.

Sec. 91.183  IFR radio communications.

The pilot in command of each aircraft operated under IFR in controlled

airspace shall have a continuous watch maintained on the appropriate

frequency and shall report by radio as soon as possible--

(a) The time and altitude of passing each designated reporting point, or

the reporting points specified by ATC, except that while the aircraft is

under radar control, only the passing of those reporting points specifically

requested by ATC need be reported;

(b) Any unforecast weather conditions encountered; and

(c) Any other information relating to the safety of flight.

Sec. 91.185  IFR operations: Two-way radio communications failure.

(a) General. Unless otherwise authorized by ATC, each pilot who has two-way

radio communications failure when operating under IFR shall comply with the

rules of this section.

(b) VFR conditions. If the failure occurs in VFR conditions, or if VFR

conditions are encountered after the failure, each pilot shall continue the

flight under VFR and land as soon as practicable.

(c) IFR conditions. If the failure occurs in IFR conditions, or if

paragraph (b) of this section cannot be complied with, each pilot shall

continue the flight according to the following:

(1) Route. (i) By the route assigned in the last ATC clearance received;

(ii) If being radar vectored, by the direct route from the point of radio

failure to the fix, route, or airway specified in the vector clearance;

(iii) In the absence of an assigned route, by the route that ATC has

advised may be expected in a further clearance; or

(iv) In the absence of an assigned route or a route that ATC has advised

may be expected in a further clearance, by the route filed in the flight

plan.

(2) Altitude. At the highest of the following altitudes or flight levels

for the route segment being flown:

(i) The altitude or flight level assigned in the last ATC clearance

received;

(ii) The minimum altitude (converted, if appropriate, to minimum flight

level as prescribed in Sec. 91.121(c)) for IFR operations; or

(iii) The altitude or flight level ATC has advised may be expected in a

further clearance.

(3) Leave clearance limit. (i) When the clearance limit is a fix from which

an approach begins, commence descent or descent and approach as close as

possible to the expect-further-clearance time if one has been received, or if

one has not been received, as close as possible to the estimated time of

arrival as calculated from the filed or amended (with ATC) estimated time en

route.

(ii) If the clearance limit is not a fix from which an approach begins,

leave the clearance limit at the expect-further-clearance time if one has

been received, or if none has been received, upon arrival over the clearance

limit, and proceed to a fix from which an approach begins and commence

descent or descent and approach as close as possible to the estimated time of

arrival as calculated from the filed or amended (with ATC) estimated time en

route.

Sec. 91.187  Operation under IFR in controlled airspace: Malfunction reports.

(a) The pilot in command of each aircraft operated in controlled airspace

under IFR shall report as soon as practical to ATC any malfunctions of

navigational, approach, or communication equipment occurring in flight.

(b) In each report required by paragraph (a) of this section, the pilot in

command shall include the--

(1) Aircraft identification;

(2) Equipment affected;

(3) Degree to which the capability of the pilot to operate under IFR in the

ATC system is impaired; and

(4) Nature and extent of assistance desired from ATC.

Sec. 91.189  Category II and III operations: General operating rules.

(a) No person may operate a civil aircraft in a Category II or III

operation unless--

(1) The flight crew of the aircraft consists of a pilot in command and a

second in command who hold the appropriate authorizations and ratings

prescribed in Sec. 61.3 of this chapter;

(2) Each flight crewmember has adequate knowledge of, and familiarity with,

the aircraft and the procedures to be used; and

(3) The instrument panel in front of the pilot who is controlling the

aircraft has appropriate instrumentation for the type of flight control

guidance system that is being used.

(b) Unless otherwise authorized by the Administrator, no person may operate

a civil aircraft in a Category II or Category III operation unless each

ground component required for that operation and the related airborne

equipment is installed and operating.

(c) Authorized DH. For the purpose of this section, when the approach

procedure being used provides for and requires the use of a DH, the

authorized DH is the highest of the following:

(1) The DH prescribed by the approach procedure.

(2) The DH prescribed for the pilot in command.

(3) The DH for which the aircraft is equipped.

(d) Unless otherwise authorized by the Administrator, no pilot operating an

aircraft in a Category II or Category III approach that provides and requires

use of a DH may continue the approach below the authorized decision height

unless the following conditions are met:

(1) The aircraft is in a position from which a descent to a landing on the

intended runway can be made at a normal rate of descent using normal

maneuvers, and where that descent rate will allow touchdown to occur within

the touchdown zone of the runway of intended landing.

(2) At least one of the following visual references for the intended runway

is distinctly visible and identifiable to the pilot:

(i) The approach light system, except that the pilot may not descend below

100 feet above the touchdown zone elevation using the approach lights as a

reference unless the red terminating bars or the red side row bars are also

distinctly visible and identifiable.

(ii) The threshold.

(iii) The threshold markings.

(iv) The threshold lights.

(v) The touchdown zone or touchdown zone markings.

(vi) The touchdown zone lights.

(e) Unless otherwise authorized by the Administrator, each pilot operating

an aircraft shall immediately execute an appropriate missed approach

whenever, prior to touchdown, the requirements of paragraph (d) of this

section are not met.

(f) No person operating an aircraft using a Category III approach without

decision height may land that aircraft except in accordance with the

provisions of the letter of authorization issued by the Administrator.

(g) Paragraphs (a) through (f) of this section do not apply to operations

conducted by the holders of certificates issued under part 121, 125, 129, or

135 of this chapter. No person may operate a civil aircraft in a Category II

or Category III operation conducted by the holder of a certificate issued

under part 121, 125, 129, or 135 of this chapter unless the operation is

conducted in accordance with that certificate holder's operations

specifications.

Sec. 91.191  Category II manual.

(a) No person may operate a civil aircraft of United States registry in a

Category II operation unless--

(1) There is available in the aircraft a current, approved Category II

manual for that aircraft;

(2) The operation is conducted in accordance with the procedures,

instructions, and limitations in that manual; and

(3) The instruments and equipment listed in the manual that are required

for a particular Category II operation have been inspected and maintained in

accordance with the maintenance program contained in that manual.

(b) Each operator shall keep a current copy of the approved manual at its

principal base of operations and shall make it available for inspection upon

request of the Administrator.

(c) This section does not apply to operations conducted by the holder of a

certificate issued under part 121 of this chapter.

Sec. 91.193  Certificate of authorization for certain Category II operations.

The Administrator may issue a certificate of authorization authorizing

deviations from the requirements of Secs. 91.189, 91.191, and 91.205(f) for

the operation of small aircraft identified as Category A aircraft in Sec.

97.3 of this chapter in Category II operations if the Administrator finds

that the proposed operation can be safely conducted under the terms of the

certificate. Such authorization does not permit operation of the aircraft

carrying persons or property for compensation or hire.

Secs. 91.195--91.199  [Reserved]

Subpart C--Equipment, Instrument, and Certificate Requirements

Sec. 91.201   Carry-on-baggage.

No pilot in command of an airplane having a seating capacity of more than

19 passengers may permit a passenger to stow his baggage aboard that airplane

except--

(a) In a suitable baggage or cargo storage compartment, or as provided in

Sec. 91.203; or

(b) Under a passenger seat in such a way that it will not slide forward

under crash impacts severe enough to induce the ultimate inertia forces

specified in Sec. 25.561(b)(3) of this chapter, or the requirements of the

regulations under which the airplane was type certificated. After December 4,

1979, restraining devices must also limit sideward motion of under-seat

baggage and be designed to withstand crash impacts severe enough to induce

sideward forces specified in Sec. 25.561(b)(3) of this chapter.

Sec. 91.203  Civil aircraft: Certifications required.

(a) Except as provided in Sec. 91.715, no person may operate a civil

aircraft unless it has within it the following:

(1) An appropriate and current airworthiness certificate. Each U.S.

airworthiness certificate used to comply with this subparagraph (except a

special flight permit, a copy of the applicable operations specifications

issued under Sec. 21.197(c) of this chapter, appropriate sections of the air

carrier manual required by parts 121 and 135 of this chapter containing that

portion of the operations specifications issued under Sec. 21.197(c), or an

authorization under Sec. 91.611) must have on it the registration number

assigned to the aircraft under part 47 of this chapter. However, the

airworthiness certificate need not have on it an assigned special

identification number before 10 days after that number is first affixed to

the aircraft. A revised airworthiness certificate having on it an assigned

special identification number, that has been affixed to an aircraft, may only

be obtained upon application to an FAA Flight Standards district office.

(2) An effective U.S. registration certificate issued to its owner or, for

operation within the United States, the second duplicate copy (pink) of the

Aircraft Registration Application as provided for in Sec. 47.31(b), or a

registration certificate issued under the laws of a foreign country.

(b) No person may operate a civil aircraft unless the airworthiness

certificate required by paragraph (a) of this section or a special flight

authorization issued under Sec. 91.715 is displayed at the cabin or cockpit

entrance so that it is legible to passengers or crew.

(c) No person may operate an aircraft with a fuel tank installed within the

passenger compartment or a baggage compartment unless the installation was

accomplished pursuant to part 43 of this chapter, and a copy of FAA Form 337

authorizing that installation is on board the aircraft.

(d) No person may operate a civil airplane (domestic or foreign) into or

out of an airport in the United States unless it complies with the fuel

venting and exhaust emissions requirements of part 34 of this chapter.

Sec. 91.205  Powered civil aircraft with standard category U.S. airworthiness

certificates: Instrument and equipment requirements.

(a) General. Except as provided in paragraphs (c)(3) and (e) of this

section, no person may operate a powered civil aircraft with a standard

category U.S. airworthiness certificate in any operation described in

paragraphs (b) through (f) of this section unless that aircraft contains the

instruments and equipment specified in those paragraphs (or FAA-approved

equivalents) for that type of operation, and those instruments and items of

equipment are in operable condition.

(b) Visual-flight rules (day). For VFR flight during the day, the following

instruments and equipment are required:

(1) Airspeed indicator.

(2) Altimeter.

(3) Magnetic direction indicator.

(4) Tachometer for each engine.

(5) Oil pressure gauge for each engine using pressure system.

(6) Temperature gauge for each liquid-cooled engine.

(7) Oil temperature gauge for each air-cooled engine.

(8) Manifold pressure gauge for each altitude engine.

(9) Fuel gauge indicating the quantity of fuel in each tank.

(10) Landing gear position indicator, if the aircraft has a retractable

landing gear.

(11) If the aircraft is operated for hire over water and beyond power-off

gliding distance from shore, approved flotation gear readily available to

each occupant and at least one pyrotechnic signaling device. As used in this

section, "shore" means that area of the land adjacent to the water which is

above the high water mark and excludes land areas which are intermittently

under water.

(12) An approved safety belt with an approved metal-to-metal latching

device for each occupant 2 years of age or older.

(13) For small civil airplanes manufactured after July 18, 1978, an

approved shoulder harness for each front seat. The shoulder harness must be

designed to protect the occupant from serious head injury when the occupant

experiences the ultimate inertia forces specified in Sec. 23.561(b)(2) of

this chapter. Each shoulder harness installed at a flight crewmember station

must permit the crewmember, when seated and with the safety belt and shoulder

harness fastened, to perform all functions necessary for flight operations.

For purposes of this paragraph--

(i) The date of manufacture of an airplane is the date the inspection

acceptance records reflect that the airplane is complete and meets the FAA-

approved type design data; and

(ii) A front seat is a seat located at a flight crewmember station or any

seat located alongside such a seat.

(14) An emergency locator transmitter, if required by Sec. 91.207.

(15) For normal, utility, and acrobatic category airplanes with a seating

configuration, excluding pilot seats, of 9 or less, manufactured after

December 12, 1986, a shoulder harness for--

(i) Each front seat that meets the requirements of Sec. 23.785 (g) and (h)

of this chapter in effect on December 12, 1985;

(ii) Each additional seat that meets the requirements of Sec. 23.785(g) of

this chapter in effect on December 12, 1985.

(16) For rotorcraft manufactured after September 16, 1992, a shoulder

harness for each seat that meets the requirements of Sec. 27.2 or Sec. 29.2

of this chapter in effect on September 16, 1991.

(c) Visual flight rules (night). For VFR flight at night, the following

instruments and equipment are required:

(1) Instruments and equipment specified in paragraph (b) of this section.

(2) Approved position lights.

(3) An approved aviation red or aviation white anticollision light system

on all U.S.-registered civil aircraft. Anticollision light systems initially

installed after August 11, 1971, on aircraft for which a type certificate was

issued or applied for before August 11, 1971, must at least meet the

anticollision light standards of part 23, 25, 27, or 29 of this chapter, as

applicable, that were in effect on August 10, 1971, except that the color may

be either aviation red or aviation white. In the event of failure of any

light of the anticollision light system, operations with the aircraft may be

continued to a stop where repairs or replacement can be made.

(4) If the aircraft is operated for hire, one electric landing light.

(5) An adequate source of electrical energy for all installed electrical

and radio equipment.

(6) One spare set of fuses, or three spare fuses of each kind required,

that are accessible to the pilot in flight.

(d) Instrument flight rules. For IFR flight, the following instruments and

equipment are required:

(1) Instruments and equipment specified in paragraph (b) of this section,

and, for night flight, instruments and equipment specified in paragraph (c)

of this section.

(2) Two-way radio communications system and navigational equipment

appropriate to the ground facilities to be used.

(3) Gyroscopic rate-of-turn indicator, except on the following aircraft:

(i) Airplanes with a third attitude instrument system usable through

flight attitudes of 360 degrees of pitch and roll and installed in accordance

with the instrument requirements prescribed in Sec. 121.305(j) of this

chapter; and

(ii) Rotorcraft with a third attitude instrument system usable through

flight attitudes of +/-80 degrees of pitch and +/-120 degrees of roll and

installed in accordance with Sec. 29.1303(g) of this chapter.

(4) Slip-skid indicator.

(5) Sensitive altimeter adjustable for barometric pressure.

(6) A clock displaying hours, minutes, and seconds with a sweep-second

pointer or digital presentation.

(7) Generator or alternator of adequate capacity.

(8) Gyroscopic pitch and bank indicator (artificial horizon).

(9) Gyroscopic direction indicator (directional gyro or equivalent).

(e) Flight at and above 24,000 ft. MSL (FL 240). If VOR navigational

equipment is required under paragraph (d)(2) of this section, no person may

operate a U.S.-registered civil aircraft within the 50 states and the

District of Columbia at or above FL 240 unless that aircraft is equipped with

approved distance measuring equipment (DME). When DME required by this

paragraph fails at and above FL 240, the pilot in command of the aircraft

shall notify ATC immediately, and then may continue operations at and above

FL 240 to the next airport of intended landing at which repairs or

replacement of the equipment can be made.

(f) Category II operations. For Category II operations the instruments and

equipment specified in paragraph (d) of this section and appendix A to this

part are required. This paragraph does not apply to operations conducted by

the holder of a certificate issued under part 121 of this chapter.

Sec. 91.207  Emergency locator transmitters.

(a) Except as provided in paragraphs (e) and (f) of this section, no person

may operate a U.S.-registered civil airplane unless--

(1) There is attached to the airplane an approved automatic type emergency

locator transmitter that is in operable condition for the following

operations, except that after June 21, 1995, an emergency locator transmitter that

meets the requirements of TSO-C91 may  not be used for new installations:

(i) Those operations governed by the supplemental air carrier and

commercial operator rules of parts 121 and 125;

(ii) Charter flights governed by the domestic and flag air carrier rules of

part 121 of this chapter; and

(iii) Operations governed by part 135 of this chapter; or

(2) For operations other than those specified in paragraph (a)(1) of this

section, there must be attached to the airplane an approved personal type or

an approved automatic type emergency locator transmitter that is in operable

condition, except that after June 21, 1995, an emergency locator transmitter that

meets the requirements of TSO-C91 may not be used for new installations .

(b) Each emergency locator transmitter required by paragraph (a) of this

section must be attached to the airplane in such a manner that the

probability of damage to the transmitter in the event of crash impact is

minimized. Fixed and deployable automatic type transmitters must be attached

to the airplane as far aft as practicable.

(c) Batteries used in the emergency locator transmitters required by

paragraphs (a) and (b) of this section must be replaced (or recharged, if the

batteries are rechargeable)--

(1) When the transmitter has been in use for more than 1 cumulative hour;

or

(2) When 50 percent of their useful life (or, for rechargeable batteries,

50 percent of their useful life of charge) has expired, as established by the

transmitter manufacturer under its approval.

The new expiration date for replacing (or recharging) the battery must be

legibly marked on the outside of the transmitter and entered in the aircraft

maintenance record. Paragraph (c)(2) of this section does not apply to

batteries (such as water-activated batteries) that are essentially unaffected

during probable storage intervals.

(d) Each emergency locator transmitter required by paragraph (a) of this

section must be inspected within 12 calendar months after the last inspection

for--

(1) Proper installation;

(2) Battery corrosion;

(3) Operation of the controls and crash sensor; and

(4) The presence of a sufficient signal radiated from its antenna.

(e) Notwithstanding paragraph (a) of this section, a person may--

(1) Ferry a newly acquired airplane from the place where possession of it

was taken to a place where the emergency locator transmitter is to be

installed; and

(2) Ferry an airplane with an inoperative emergency locator transmitter

from a place where repairs or replacements cannot be made to a place where

they can be made.

No person other than required crewmembers may be carried aboard an airplane

being ferried under paragraph (e) of this section.

(f) Paragraph (a) of this section does not apply to--

(1) Turbojet-powered aircraft;

(2) Aircraft while engaged in scheduled flights by scheduled air carriers;

(3) Aircraft while engaged in training operations conducted entirely within

a 50-nautical mile radius of the airport from which such local flight

operations began;

(4) Aircraft while engaged in flight operations incident to design and

testing;

(5) New aircraft while engaged in flight operations incident to their

manufacture, preparation, and delivery;

(6) Aircraft while engaged in flight operations incident to the aerial

application of chemicals and other substances for agricultural purposes;

(7) Aircraft certificated by the Administrator for research and development

purposes;

(8) Aircraft while used for showing compliance with regulations, crew

training, exhibition, air racing, or market surveys;

(9) Aircraft equipped to carry not more than one person; and

(10) An aircraft during any period for which the transmitter has been

temporarily removed for inspection, repair, modification, or replacement,

subject to the following:

(i) No person may operate the aircraft unless the aircraft records contain

an entry which includes the date of initial removal, the make, model, serial

number, and reason for removing the transmitter, and a placard located in

view of the pilot to show "ELT not installed."

(ii) No person may operate the aircraft more than 90 days after the ELT is

initially removed from the aircraft.

Sec. 91.209   Operating in icing conditions.

(a) No pilot may take off an airplane that has--

(1) Frost, snow, or ice adhering to any propeller, windshield, or

powerplant installation, or to an airspeed, altimeter, rate of climb, or

flight attitude instrument system;

(2) Snow or ice adhering to the wings, or stabilizing or control surfaces;

or

(3) Any frost adhering to the wings, or stabilizing or control surfaces,

unless that frost has been polished to make it smooth.

(b) Except for an airplane that has ice protection provisions that meet the

requirements in section 34 of Special Federal Aviation Regulation No. 23, or

those for transport category airplane type certification, no pilot may fly--

(1) Under IFR into known or forecast moderate icing conditions; or

(2) Under VFR into known light or moderate icing conditions; unless the

aircraft has functioning de-icing or anti-icing equipment protecting each

propeller, windshield, wing, stabilizing or control surface, and each

airspeed, altimeter, rate of climb, or flight attitude instrument system.

(c) Except for an airplane that has ice protection provisions that meet the

requirements in section 34 of Special Federal Aviation Regulation No. 23, or

those for transport category airplane type certification, no pilot may fly an

airplane into known or forecast severe icing conditions.

(d) If current weather reports and briefing information relied upon by the

pilot in command indicate that the forecast icing conditions that would

otherwise prohibit the flight will not be encountered during the flight

because of changed weather conditions since the forecast, the restrictions in

paragraphs (b) and (c) of this section based on forecast conditions do not

apply.

Sec. 91.209  Aircraft lights.

No person may, during the period from sunset to sunrise (or, in Alaska,

during the period a prominent unlighted object cannot be seen from a distance

of 3 statute miles or the sun is more than 6 degrees below the horizon)--

(a) Operate an aircraft unless it has lighted position lights;

(b) Park or move an aircraft in, or in dangerous proximity to, a night

flight operations area of an airport unless the aircraft--

(1) Is clearly illuminated;

(2) Has lighted position lights; or

(3) Is in an area which is marked by obstruction lights;

(c) Anchor an aircraft unless the aircraft--

(1) Has lighted anchor lights; or

(2) Is in an area where anchor lights are not required on vessels; or

(d) Operate an aircraft, required by Sec. 91.205(c)(3) to be equipped with

an anticollision light system, unless it has approved and lighted aviation

red or aviation white anticollision lights. However, the anticollision lights

need not be lighted when the pilot in command determines that, because of

operating conditions, it would be in the interest of safety to turn the

lights off.

Sec. 91.211   Flight engineer requirements.

(a) No person may operate the following airplanes without a flight

crewmember holding a current flight engineer certificate:

(1) An airplane for which a type certificate was issued before January 2,

1964, having a maximum certificated takeoff weight of more than 80,000

pounds.

(2) An airplane type certificated after January 1, 1964, for which a flight

engineer is required by the type certification requirements.

(b) No person may serve as a required flight engineer on an airplane

unless, within the preceding 6 calendar months, he has had at least 50 hours

of flight time as a flight engineer on that type airplane, or the

Administrator has checked him on that type airplane and determined that he is

familiar and competent with all essential current information and operating

procedures.

Sec. 91.211  Supplemental oxygen.

(a) General. No person may operate a civil aircraft of U.S. registry--

(1) At cabin pressure altitudes above 12,500 feet (MSL) up to and including

14,000 feet (MSL) unless the required minimum flight crew is provided with

and uses supplemental oxygen for that part of the flight at those altitudes

that is of more than 30 minutes duration;

(2) At cabin pressure altitudes above 14,000 feet (MSL) unless the required

minimum flight crew is provided with and uses supplemental oxygen during the

entire flight time at those altitudes; and

(3) At cabin pressure altitudes above 15,000 feet (MSL) unless each

occupant of the aircraft is provided with supplemental oxygen.

(b) Pressurized cabin aircraft. (1) No person may operate a civil aircraft

of U.S. registry with a pressurized cabin--

(i) At flight altitudes above flight level 250 unless at least a 10-minute

supply of supplemental oxygen, in addition to any oxygen required to satisfy

paragraph (a) of this section, is available for each occupant of the aircraft

for use in the event that a descent is necessitated by loss of cabin

pressurization; and

(ii) At flight altitudes above flight level 350 unless one pilot at the

controls of the airplane is wearing and using an oxygen mask that is secured

and sealed and that either supplies oxygen at all times or automatically

supplies oxygen whenever the cabin pressure altitude of the airplane exceeds

14,000 feet (MSL), except that the one pilot need not wear and use an oxygen

mask while at or below flight level 410 if there are two pilots at the

controls and each pilot has a quick-donning type of oxygen mask that can be

placed on the face with one hand from the ready position within 5 seconds,

supplying oxygen and properly secured and sealed.

(2) Notwithstanding paragraph (b)(1)(ii) of this section, if for any reason

at any time it is necessary for one pilot to leave the controls of the

aircraft when operating at flight altitudes above flight level 350, the

remaining pilot at the controls shall put on and use an oxygen mask until the

other pilot has returned to that crewmember's station.

Sec. 91.213  Inoperative instruments and equipment.

(a) Except as provided in paragraph (d) of this section, no person may take

off an aircraft with inoperative instruments or equipment installed unless

the following conditions are met:

(1) An approved Minimum Equipment List exists for that aircraft.

(2) The aircraft has within it a letter of authorization, issued by the FAA

Flight Standards district office having jurisdiction over the area in which

the operator is located, authorizing operation of the aircraft under the

Minimum Equipment List. The letter of authorization may be obtained by

written request of the airworthiness certificate holder. The Minimum

Equipment List and the letter of authorization constitute a supplemental type

certificate for the aircraft.

(3) The approved Minimum Equipment List must--

(i) Be prepared in accordance with the limitations specified in paragraph

(b) of this section; and

(ii) Provide for the operation of the aircraft with the instruments and

equipment in an inoperable condition.

(4) The aircraft records available to the pilot must include an entry

describing the inoperable instruments and equipment.

(5) The aircraft is operated under all applicable conditions and

limitations contained in the Minimum Equipment List and the letter

authorizing the use of the list.

(b) The following instruments and equipment may not be included in a

Minimum Equipment List:

(1) Instruments and equipment that are either specifically or otherwise

required by the airworthiness requirements under which the aircraft is type

certificated and which are essential for safe operations under all operating

conditions.

(2) Instruments and equipment required by an airworthiness directive to be

in operable condition unless the airworthiness directive provides otherwise.

(3) Instruments and equipment required for specific operations by this

part.

(c) A person authorized to use an approved Minimum Equipment List issued

for a specific aircraft under Part 121, 125, or 135 of this chapter shall use

that Minimum Equipment List in connection with operations conducted with that

aircraft under this part without additional approval requirements.

(d) Except for operations conducted in accordance with paragraph (a) or (c)

of this section, a person may takeoff an aircraft in operations conducted

under this part with inoperative instruments and equipment without an

approved Minimum Equipment List provided--

(1) The flight operation is conducted in a--

(i) Rotorcraft, nonturbine-powered airplane, glider, or lighter-than-air

aircraft for which a master Minimum Equipment List has not been developed; or

(ii) Small rotorcraft, nonturbine-powered small airplane, glider, or

lighter-than-air aircraft for which a Master Minimum Equipment List has been

developed; and

(2) The inoperative instruments and equipment are not--

(i) Part of the VFR-day type certification instruments and equipment

prescribed in the applicable airworthiness regulations under which the

aircraft was type certificated;

(ii) Indicated as required on the aircraft's equipment list, or on the

Kinds of Operations Equipment List for the kind of flight operation being

conducted;

(iii) Required by Sec. 91.205 or any other rule of this part for the

specific kind of flight operation being conducted; or

(iv) Required to be operational by an airworthiness directive; and

(3) The inoperative instruments and equipment are--

(i) Removed from the aircraft, the cockpit control placarded, and the

maintenance recorded in accordance with Sec. 43.9 of this chapter; or

(ii) Deactivated and placarded "Inoperative." If deactivation of the

inoperative instrument or equipment involves maintenance, it must be

accomplished and recorded in accordance with part 43 of this chapter; and

(4) A determination is made by a pilot, who is certificated and

appropriately rated under part 61 of this chapter, or by a person, who is

certificated and appropriately rated to perform maintenance on the aircraft,

that the inoperative instrument or equipment does not constitute a hazard to

the aircraft.

An aircraft with inoperative instruments or equipment as provided in

paragraph (d) of this section is considered to be in a properly altered

condition acceptable to the Administrator.

(e) Notwithstanding any other provision of this section, an aircraft with

inoperable instruments or equipment may be operated under a special flight

permit issued in accordance with Secs. 21.197 and 21.199 of this chapter.

Sec. 91.215  ATC transponder and altitude reporting equipment and use.

(a) All airspace: U.S.-registered civil aircraft. For operations not

conducted under part 121, 127 or 135 of this chapter, ATC transponder

equipment installed must meet the performance and environmental requirements

of any class of TSO-C74b (Mode A) or any class of TSO-C74c (Mode A with

altitude reporting capability) as appropriate, or the appropriate class of

TSO-C112 (Mode S).

(b) All airspace. Unless otherwise authorized or directed by ATC, no person

may operate an aircraft in the airspace described in paragraphs (b)(1)

through (b)(5) of this section, unless that aircraft is equipped with an

operable coded radar beacon transponder having either Mode 3/A 4096 code

capability, replying to Mode 3/A interrogations with the code specified by

ATC, or a Mode S capability, replying to Mode 3/A interrogations with the

code specified by ATC and intermode and Mode S interrogations in accordance

with the applicable provisions specified in TSO C-112, and that aircraft is

equipped with automatic pressure altitude reporting equipment having a Mode C

capability that automatically replies to Mode C interrogations by

transmitting pressure altitude information in 100-foot increments. This

requirement applies--

(1) All aircraft. In Class A, Class B, and Class C airspace areas;

(2) All aircraft. In all airspace within 30 nautical miles of an airport

listed in appendix D, section 1 of this part from the surface upward to

10,000 feet MSL;

(3) Notwithstanding paragraph (b)(2) of this section, any aircraft which

was not originally certificated with an engine-driven electrical system or

which has not subsequently been certified with such a system installed,

balloon or glider may conduct operations in the airspace within 30 nautical

miles of an airport listed in appendix D, section 1 of this part provided

such operations are conducted--

(i) Outside any Class A, Class B, or Class C airspace area; and

(ii) Below the altitude of the ceiling of a Class B or Class C airspace

area designated for an airport or 10,000 feet MSL, whichever is lower; and

(4) All aircraft in all airspace above the ceiling and within the lateral

boundaries of a Class B or Class C airspace area designated for an airport

upward to 10,000 feet MSL; and

(5) All aircraft except any aircraft which was not originally certificated

with an engine-driven electrical system or which has not subsequently been

certified with such a system installed, balloon, or glider----

(i) In all airspace of the 48 contiguous states and the District of

Columbia at and above 10,000 feet MSL, excluding the airspace at and below

2,500 feet above the surface; and

(ii) In the airspace from the surface to 10,000 feet MSL within a 10-

nautical-mile radius of any airport listed in appendix D, section 2 of this

part, excluding the airspace below 1,200 feet outside of the lateral

boundaries of the surface area of the airspace designated for that airport.

(c) Transponder-on operation. While in the airspace as specified in

paragraph (b) of this section or in all controlled airspace, each person

operating an aircraft equipped with an operable ATC transponder maintained in

accordance with Sec. 91.413 of this part shall operate the transponder,

including Mode C equipment if installed, and shall reply on the appropriate

code or as assigned by ATC.

(d) ATC authorized deviations. Requests for ATC authorized deviations

must be made to the ATC facility having jurisdiction over the concerned

airspace within the time periods specified as follows:

(1) For operation of an aircraft with an operating transponder but without

operating automatic pressure altitude reporting equipment having a Mode C

capability, the request may be made at any time.

(2) For operation of an aircraft with an inoperative transponder to the

airport of ultimate destination, including any intermediate stops, or to

proceed to a place where suitable repairs can be made or both, the request

may be made at any time.

(3) For operation of an aircraft that is not equipped with a transponder,

the request must be made at least one hour before the proposed operation.

Sec. 91.217  Data correspondence between automatically reported pressur

altitude data and the pilot's altitude reference.

No person may operate any automatic pressure altitude reporting equipment

associated with a radar beacon transponder--

(a) When deactivation of that equipment is directed by ATC;

(b) Unless, as installed, that equipment was tested and calibrated to

transmit altitude data corresponding within 125 feet (on a 95 percent

probability basis) of the indicated or calibrated datum of the altimeter

normally used to maintain flight altitude, with that altimeter referenced to

29.92 inches of mercury for altitudes from sea level to the maximum operating

altitude of the aircraft; or

(c) Unless the altimeters and digitizers in that equipment meet the

standards of TSO-C10b and TSO-C88, respectively.

Sec. 91.219  Altitude alerting system or device: Turbojet-powered civil

airplanes.

(a) Except as provided in paragraph (d) of this section, no person may

operate a turbojet-powered U.S.-registered civil airplane unless that

airplane is equipped with an approved altitude alerting system or device that

is in operable condition and meets the requirements of paragraph (b) of this

section.

(b) Each altitude alerting system or device required by paragraph (a) of

this section must be able to--

(1) Alert the pilot--

(i) Upon approaching a preselected altitude in either ascent or descent, by

a sequence of both aural and visual signals in sufficient time to establish

level flight at that preselected altitude; or

(ii) Upon approaching a preselected altitude in either ascent or descent,

by a sequence of visual signals in sufficient time to establish level flight

at that preselected altitude, and when deviating above and below that

preselected altitude, by an aural signal;

(2) Provide the required signals from sea level to the highest operating

altitude approved for the airplane in which it is installed;

(3) Preselect altitudes in increments that are commensurate with the

altitudes at which the aircraft is operated;

(4) Be tested without special equipment to determine proper operation of

the alerting signals; and

(5) Accept necessary barometric pressure settings if the system or device

operates on barometric pressure. However, for operation below 3,000 feet AGL,

the system or device need only provide one signal, either visual or aural, to

comply with this paragraph. A radio altimeter may be included to provide the

signal if the operator has an approved procedure for its use to determine DH

or MDA, as appropriate.

(c) Each operator to which this section applies must establish and assign

procedures for the use of the altitude alerting system or device and each

flight crewmember must comply with those procedures assigned to him.

(d) Paragraph (a) of this section does not apply to any operation of an

airplane that has an experimental certificate or to the operation of any

airplane for the following purposes:

(1) Ferrying a newly acquired airplane from the place where possession of

it was taken to a place where the altitude alerting system or device is to be

installed.

(2) Continuing a flight as originally planned, if the altitude alerting

system or device becomes inoperative after the airplane has taken off;

however, the flight may not depart from a place where repair or replacement

can be made.

(3) Ferrying an airplane with any inoperative altitude alerting system or

device from a place where repairs or replacements cannot be made to a place

where it can be made.

(4) Conducting an airworthiness flight test of the airplane.

(5) Ferrying an airplane to a place outside the United States for the

purpose of registering it in a foreign country.

(6) Conducting a sales demonstration of the operation of the airplane.

(7) Training foreign flight crews in the operation of the airplane before

ferrying it to a place outside the United States for the purpose of

registering it in a foreign country.

Sec. 91.221  Traffic alert and collision avoidance system equipment anduse.

(a) All airspace: U.S.-registered civil aircraft. Any traffic alert and

collision avoidance system installed in a U.S.-registered civil aircraft must

be approved by the Administrator.

(b) Traffic alert and collision avoidance system, operation required. Each

person operating an aircraft equipped with an operable traffic alert and

collision avoidance system shall have that system on and operating.

Secs. 91.223--91.299  [Reserved]

Subpart D--Special Flight Operations

Sec. 91.301   Applicability; relation to Part 36.

(a) This subpart prescribes operating noise limits and related requirements

that apply, as follows, to the operation of civil aircraft in the United

States:

(1) Sections 91.303, 91.305, 91.306, and 91.307 apply to civil subsonic

turbojet airplanes with maximum weights of more than 75,000 pounds and--

(i) If U.S. registered, that have standard airworthiness certificates; or

(ii) If foreign registered, that would be required by this chapter to have

a U.S. standard airworthiness certificate in order to conduct the operations

intended for the airplane were it registered in the United States.

Those sections apply to operations to or from airports in the United States

under this part and Parts 121, 123, 129, and 135 of this chapter.

(2) Section 91.308 applies to U.S. operators of civil subsonic turbojet

airplanes covered by this subpart. That section applies to operators

operating to or from airports in the United States under this part and Parts

121, 123, 125, and 135 but not to those operating under Part 129 of this

chapter.

(3) Sections 91.302, 91.309 and 91.311 apply to U.S. registered civil

supersonic airplanes having standard airworthiness certificates, and to

foreign registered civil supersonic airplanes that, if registered in the

United States, would be required by this chapter to have a U.S. standard

airworthiness certificate in order to conduct the operations intended for the

airplane. Those sections apply to operations under this part and under Parts

121, 123, 125, 129, and 135 of this chapter.

(b) Unless otherwise specified, as used in this subpart "Part 36" refers to

14 CFR Part 36, including the noise levels under Appendix C of that part,

notwithstanding the provisions of that part excepting certain airplanes from

the specified noise requirements. For purposes of this subpart, the various

stages of noise levels, the terms used to describe airplanes with respect to

those levels, and the terms "subsonic airplane" and "supersonic airplane"

have the meanings specified under Part 36 of this chapter. For purposes of

this subpart, for subsonic airplanes operated in foreign air commerce in the

United States, the Administrator may accept compliance with the noise

requirements under Annex 16 of the International Civil Aviation Organization

when those requirements have been shown to be substantially compatible with,

and achieve results equivalent to those achievable under, Part 36 for that

airplane. Determinations made under these provisions are subject to the

limitations of Sec. 36.5 of this chapter as if those noise levels were Part

36 noise levels.

Sec. 91.303  Aerobatic flight.

No person may operate an aircraft in aerobatic flight--

(a) Over any congested area of a city, town, or settlement;

(b) Over an open air assembly of persons;

(c) Within the lateral boundaries of the surface areas of Class B, Class C,

Class D, or Class E airspace designated for an airport;

(d) Within 4 nautical miles of the center line of any Federal airway;

(e) Below an altitude of 1,500 feet above the surface; or

(f) When flight visibility is less than 3 statute miles.

Sec. 91.305  Flight test areas.

No person may flight test an aircraft except over open water, or sparsely

populated areas, having light air traffic.

Sec. 91.307  Parachutes and parachuting.

(a) No pilot of a civil aircraft may allow a parachute that is available

for emergency use to be carried in that aircraft unless it is an approved

type and--

(1) If a chair type (canopy in back), it has been packed by a certificated

and appropriately rated parachute rigger within the preceding 120 days; or

(2) If any other type, it has been packed by a certificated and

appropriately rated parachute rigger--

(i) Within the preceding 120 days, if its canopy, shrouds, and harness are

composed exclusively of nylon, rayon, or other similar synthetic fiber or

materials that are substantially resistant to damage from mold, mildew, or

other fungi and other rotting agents propagated in a moist environment; or

(ii) Within the preceding 60 days, if any part of the parachute is composed

of silk, pongee, or other natural fiber, or materials not specified in

paragraph (a)(2)(i) of this section.

(b) Except in an emergency, no pilot in command may allow, and no person

may make, a parachute jump from an aircraft within the United States except

in accordance with Part 105.

(c) Unless each occupant of the aircraft is wearing an approved parachute,

no pilot of a civil aircraft carrying any person (other than a crewmember)

may execute any intentional maneuver that exceeds--

(1) A bank of 60 degrees relative to the horizon; or

(2) A nose-up or nose-down attitude of 30 degrees relative to the horizon.

(d) Paragraph (c) of this section does not apply to--

(1) Flight tests for pilot certification or rating; or

(2) Spins and other flight maneuvers required by the regulations for any

certificate or rating when given by--

(i) A certificated flight instructor; or

(ii) An airline transport pilot instructing in accordance with Sec. 61.169

of this chapter.

(e) For the purposes of this section, "approved parachute" means--

(1) A parachute manufactured under a type certificate or a technical

standard order (C-23 series); or

(2) A personnel-carrying military parachute identified by an NAF, AAF, or

AN drawing number, an AAF order number, or any other military designation or

specification number.

Sec. 91.309  Towing: Gliders.

(a) No person may operate a civil aircraft towing a glider unless--

(1) The pilot in command of the towing aircraft is qualified under Sec.

61.69 of this chapter;

(2) The towing aircraft is equipped with a tow-hitch of a kind, and

installed in a manner, that is approved by the Administrator;

(3) The towline used has breaking strength not less than 80 percent of the

maximum certificated operating weight of the glider and not more than twice

this operating weight. However, the towline used may have a breaking strength

more than twice the maximum certificated operating weight of the glider if--

(i) A safety link is installed at the point of attachment of the towline to

the glider with a breaking strength not less than 80 percent of the maximum

certificated operating weight of the glider and not greater than twice this

operating weight.

(ii) A safety link is installed at the point of attachment of the towline

to the towing aircraft with a breaking strength greater, but not more than 25

percent greater, than that of the safety link at the towed glider end of the

towline and not greater than twice the maximum certificated operating weight

of the glider;

(4) Before conducting any towing operation within the lateral boundaries of

the surface areas of Class B, Class C, Class D, or Class E airspace

designated for an airport, or before making each towing flight within such

controlled airspace if required by ATC, the pilot in command notifies the

control tower. If a control tower does not exist or is not in operation, the

pilot in command must notify the FAA flight service station serving that

controlled airspace before conducting any towing operations in that airspace;

and

(5) The pilots of the towing aircraft and the glider have agreed upon a

general course of action, including takeoff and release signals, airspeeds,

and emergency procedures for each pilot.

(b) No pilot of a civil aircraft may intentionally release a towline, after

release of a glider, in a manner that endangers the life or property of

another.

Sec. 91.311  Towing: Other than under Sec. 91.309.

No pilot of a civil aircraft may tow anything with that aircraft (other

than under Sec. 91.309) except in accordance with the terms of a certificate

of waiver issued by the Administrator.

Sec. 91.313  Restricted category civil aircraft: Operating limitations.

(a) No person may operate a restricted category civil aircraft--

(1) For other than the special purpose for which it is certificated; or

(2) In an operation other than one necessary to accomplish the work

activity directly associated with that special purpose.

(b) For the purpose of paragraph (a) of this section, operating a

restricted category civil aircraft to provide flight crewmember training in a

special purpose operation for which the aircraft is certificated is

considered to be an operation for that special purpose.

(c) No person may operate a restricted category civil aircraft carrying

persons or property for compensation or hire. For the purposes of this

paragraph, a special purpose operation involving the carriage of persons or

material necessary to accomplish that operation, such as crop dusting,

seeding, spraying, and banner towing (including the carrying of required

persons or material to the location of that operation), and operation for the

purpose of providing flight crewmember training in a special purpose

operation, are not considered to be the carriage of persons or property for

compensation or hire.

(d) No person may be carried on a restricted category civil aircraft unless

that person--

(1) Is a flight crewmember;

(2) Is a flight crewmember trainee;

(3) Performs an essential function in connection with a special purpose

operation for which the aircraft is certificated; or

(4) Is necessary to accomplish the work activity directly associated with

that special purpose.

(e) Except when operating in accordance with the terms and conditions of a

certificate of waiver or special operating limitations issued by the

Administrator, no person may operate a restricted category civil aircraft

within the United States--

(1) Over a densely populated area;

(2) In a congested airway; or

(3) Near a busy airport where passenger transport operations are conducted.

(f) This section does not apply to nonpassenger-carrying civil rotorcraft

external-load operations conducted under Part 133 of this chapter.

(g) No person may operate a small restricted-category civil airplane

manufactured after July 18, 1978, unless an approved shoulder harness is

installed for each front seat. The shoulder harness must be designed to

protect each occupant from serious head injury when the occupant experiences

the ultimate inertia forces specified in Sec. 23.561(b)(2) of this chapter.

The shoulder harness installation at each flight crewmember station must

permit the crewmember, when seated and with the safety belt and shoulder

harness fastened, to perform all functions necessary for flight operation.

For purposes of this paragraph--

(1) The date of manufacture of an airplane is the date the inspection

acceptance records reflect that the airplane is complete and meets the FAA-

approved type design data; and

(2) A front seat is a seat located at a flight crewmember station or any

seat located alongside such a seat.

Sec. 91.315  Limited category civil aircraft: Operating limitations.

No person may operate a limited category civil aircraft carrying persons or

property for compensation or hire.

Sec. 91.317  Provisionally certificated civil aircraft: Operating

limitations.

(a) No person may operate a provisionally certificated civil aircraft

unless that person is eligible for a provisional airworthiness certificate

under Sec. 21.213 of this chapter.

(b) No person may operate a provisionally certificated civil aircraft

outside the United States unless that person has specific authority to do so

from the Administrator and each foreign country involved.

(c) Unless otherwise authorized by the Director, Flight Standards Service,

no person may operate a provisionally certificated civil aircraft in air

transportation.

(d) Unless otherwise authorized by the Administrator, no person may operate

a provisionally certificated civil aircraft except--

(1) In direct conjunction with the type or supplemental type certification

of that aircraft;

(2) For training flight crews, including simulated air carrier operations;

(3) Demonstration flight by the manufacturer for prospective purchasers;

(4) Market surveys by the manufacturer;

(5) Flight checking of instruments, accessories, and equipment that do not

affect the basic airworthiness of the aircraft; or

(6) Service testing of the aircraft.

(e) Each person operating a provisionally certificated civil aircraft shall

operate within the prescribed limitations displayed in the aircraft or set

forth in the provisional aircraft flight manual or other appropriate

document. However, when operating in direct conjunction with the type or

supplemental type certification of the aircraft, that person shall operate

under the experimental aircraft limitations of Sec. 21.191 of this chapter

and when flight testing, shall operate under the requirements of Sec. 91.305

of this part.

(f) Each person operating a provisionally certificated civil aircraft shall

establish approved procedures for--

(1) The use and guidance of flight and ground personnel in operating under

this section; and

(2) Operating in and out of airports where takeoffs or approaches over

populated areas are necessary. No person may operate that aircraft except in

compliance with the approved procedures.

(g) Each person operating a provisionally certificated civil aircraft shall

ensure that each flight crewmember is properly certificated and has adequate

knowledge of, and familiarity with, the aircraft and procedures to be used by

that crewmember.

(h) Each person operating a provisionally certificated civil aircraft shall

maintain it as required by applicable regulations and as may be specially

prescribed by the Administrator.

(i) Whenever the manufacturer, or the Administrator, determines that a

change in design, construction, or operation is necessary to ensure safe

operation, no person may operate a provisionally certificated civil aircraft

until that change has been made and approved. Section 21.99 of this chapter

applies to operations under this section.

(j) Each person operating a provisionally certificated civil aircraft--

(1) May carry in that aircraft only persons who have a proper interest in

the operations allowed by this section or who are specifically authorized by

both the manufacturer and the Administrator; and

(2) Shall advise each person carried that the aircraft is provisionally

certificated.

(k) The Administrator may prescribe additional limitations or procedures

that the Administrator considers necessary, including limitations on the

number of persons who may be carried in the aircraft.

Sec. 91.319  Aircraft having experimental certificates: Operating

limitations.

(a) No person may operate an aircraft that has an experimental

certificate--

(1) For other than the purpose for which the certificate was issued; or

(2) Carrying persons or property for compensation or hire.

(b) No person may operate an aircraft that has an experimental certificate

outside of an area assigned by the Administrator until it is shown that--

(1) The aircraft is controllable throughout its normal range of speeds and

throughout all the maneuvers to be executed; and

(2) The aircraft has no hazardous operating characteristics or design

features.

(c) Unless otherwise authorized by the Administrator in special operating

limitations, no person may operate an aircraft that has an experimental

certificate over a densely populated area or in a congested airway. The

Administrator may issue special operating limitations for particular aircraft

to permit takeoffs and landings to be conducted over a densely populated area

or in a congested airway, in accordance with terms and conditions specified

in the authorization in the interest of safety in air commerce.

(d) Each person operating an aircraft that has an experimental certificate

shall--

(1) Advise each person carried of the experimental nature of the aircraft;

(2) Operate under VFR, day only, unless otherwise specifically authorized

by the Administrator; and

(3) Notify the control tower of the experimental nature of the aircraft

when operating the aircraft into or out of airports with operating control

towers.

(e) The Administrator may prescribe additional limitations that the

Administrator considers necessary, including limitations on the persons that

may be carried in the aircraft.

Sec. 91.321  Carriage of candidates in Federal elections.

(a) An aircraft operator, other than one operating an aircraft under the

rules of part 121, 125, or 135 of this chapter, may receive payment for the

carriage of a candidate in a Federal election, an agent of the candidate, or

a person traveling on behalf of the candidate, if--

(1) That operator's primary business is not as an air carrier or commercial

operator;

(2) The carriage is conducted under the rules of this part 91; and

(3) The payment for the carriage is required, and does not exceed the

amount required to be paid, by regulations of the Federal Election Commission

(11 CFR et seq.).

(b) For the purposes of this section, the terms "candidate" and "election"

have the same meaning as that set forth in the regulations of the Federal

Election Commission.

Sec. 91.323  Increased maximum certificated weights for certain airplans

operated in Alaska.

(a) Notwithstanding any other provision of the Federal Aviation

Regulations, the Administrator will approve, as provided in this section, an

increase in the maximum certificated weight of an airplane type certificated

under Aeronautics Bulletin No. 7-A of the U.S. Department of Commerce dated

January 1, 1931, as amended, or under the normal category of part 4a of the

former Civil Air Regulations (14 CFR Part 4a, 1964 ed.) if that airplane is

operated in the State of Alaska by--

(1) An air taxi operator or other air carrier; or

(2) The U.S. Department of Interior in conducting its game and fish law

enforcement activities or its management, fire detection, and fire

suppression activities concerning public lands.

(b) The maximum certificated weight approved under this section may not

exceed--

(1) 12,500 pounds;

(2) 115 percent of the maximum weight listed in the FAA aircraft

specifications;

(3) The weight at which the airplane meets the positive maneuvering load

factor requirement for the normal category specified in Sec. 23.337 of this

chapter; or

(4) The weight at which the airplane meets the climb performance

requirements under which it was type certificated.

(c) In determining the maximum certificated weight, the Administrator

considers the structural soundness of the airplane and the terrain to be

traversed.

(d) The maximum certificated weight determined under this section is added

to the airplane's operation limitations and is identified as the maximum

weight authorized for operations within the State of Alaska.

Sec. 91.325  Primary Category Aircraft: Operating limitations.

(a) No person may operate a primary category aircraft carrying persons or

property for compensation or hire.

(b) No person may operate a primary category aircraft that is maintained by

the pilot-owner under an approved special inspection and maintenance program

except--

(1) The pilot-owner; or

(2) A designee of the pilot-owner, provided that the pilot-owner does not

receive compensation for the use of the aircraft.

Secs. 91.326--91.399  [Reserved]

Subpart E--Maintenance, Preventive Maintenance, and Alterations

Sec. 91.401  Applicability.

(a) This subpart prescribes rules governing the maintenance, preventive

maintenance, and alterations of U.S.-registered civil aircraft operating

within or outside of the United States.

(b) Sections 91.405, 91.409, 91.411, 91.417, and 91.419 of this subpart do

not apply to an aircraft maintained in accordance with a continuous

airworthiness maintenance program as provided in part 121, 127, 129, or Sec.

135.411(a)(2) of this chapter.

(c) Sections 91.405 and 91.409 of this part do not apply to an airplane

inspected in accordance with part 125 of this chapter.

Sec. 91.403  General.

(a) The owner or operator of an aircraft is primarily responsible for

maintaining that aircraft in an airworthy condition, including compliance

with part 39 of this chapter.

(b) No person may perform maintenance, preventive maintenance, or

alterations on an aircraft other than as prescribed in this subpart and other

applicable regulations, including part 43 of this chapter.

(c) No person may operate an aircraft for which a manufacturer's

maintenance manual or instructions for continued airworthiness has been

issued that contains an airworthiness limitations section unless the

mandatory replacement times, inspection intervals, and related procedures

specified in that section or alternative inspection intervals and related

procedures set forth in an operations specification approved by the

Administrator under part 121, 127 or 135 of this chapter or in accordance

with an inspection program approved under Sec. 91.409(e) have been complied

with.

Sec. 91.405  Maintenance required.

Each owner or operator of an aircraft--

(a) Shall have that aircraft inspected as prescribed in subpart E of this

part and shall between required inspections, except as provided in paragraph

(c) of this section, have discrepancies repaired as prescribed in part 43 of

this chapter;

(b) Shall ensure that maintenance personnel make appropriate entries in the

aircraft maintenance records indicating the aircraft has been approved for

return to service;

(c) Shall have any inoperative instrument or item of equipment, permitted

to be inoperative by Sec. 91.213(d)(2) of this part, repaired, replaced,

removed, or inspected at the next required inspection; and

(d) When listed discrepancies include inoperative instruments or equipment,

shall ensure that a placard has been installed as required by Sec. 43.11 of

this chapter.

Sec. 91.407  Operation after maintenance, preventive maintenance, rebuiding,

or alteration.

(a) No person may operate any aircraft that has undergone maintenance,

preventive maintenance, rebuilding, or alteration unless--

(1) It has been approved for return to service by a person authorized under

Sec. 43.7 of this chapter; and

(2) The maintenance record entry required by Sec. 43.9 or Sec. 43.11, as

applicable, of this chapter has been made.

(b) No person may carry any person (other than crewmembers) in an aircraft

that has been maintained, rebuilt, or altered in a manner that may have

appreciably changed its flight characteristics or substantially affected its

operation in flight until an appropriately rated pilot with at least a

private pilot certificate flies the aircraft, makes an operational check of

the maintenance performed or alteration made, and logs the flight in the

aircraft records.

(c) The aircraft does not have to be flown as required by paragraph (b) of

this section if, prior to flight, ground tests, inspection, or both show

conclusively that the maintenance, preventive maintenance, rebuilding, or

alteration has not appreciably changed the flight characteristics or

substantially affected the flight operation of the aircraft.

Sec. 91.409  Inspections.

(a) Except as provided in paragraph (c) of this section, no person may

operate an aircraft unless, within the preceding 12 calendar months, it has

had--

(1) An annual inspection in accordance with part 43 of this chapter and has

been approved for return to service by a person authorized by Sec. 43.7 of

this chapter; or

(2) An inspection for the issuance of an airworthiness certificate in

accordance with part 21 of this chapter.

No inspection performed under paragraph (b) of this section may be

substituted for any inspection required by this paragraph unless it is

performed by a person authorized to perform annual inspections and is entered

as an "annual" inspection in the required maintenance records.

(b) Except as provided in paragraph (c) of this section, no person may

operate an aircraft carrying any person (other than a crewmember) for hire,

and no person may give flight instruction for hire in an aircraft which that

person provides, unless within the preceding 100 hours of time in service the

aircraft has received an annual or 100-hour inspection and been approved for

return to service in accordance with part 43 of this chapter or has received

an inspection for the issuance of an airworthiness certificate in accordance

with part 21 of this chapter. The 100-hour limitation may be exceeded by not

more than 10 hours while en route to reach a place where the inspection can

be done. The excess time used to reach a place where the inspection can be

done must be included in computing the next 100 hours of time in service.

(c) Paragraphs (a) and (b) of this section do not apply to--

(1) An aircraft that carries a special flight permit, a current

experimental certificate, or a provisional airworthiness certificate;

(2) An aircraft inspected in accordance with an approved aircraft

inspection program under part 125, 127, or 135 of this chapter and so

identified by the registration number in the operations specifications of the

certificate holder having the approved inspection program;

(3) An aircraft subject to the requirements of paragraph (d) or (e) of this

section; or

(4) Turbine-powered rotorcraft when the operator elects to inspect that

rotorcraft in accordance with paragraph (e) of this section.

(d) Progressive inspection. Each registered owner or operator of an

aircraft desiring to use a progressive inspection program must submit a

written request to the FAA Flight Standards district office having

jurisdiction over the area in which the applicant is located, and shall

provide--

(1) A certificated mechanic holding an inspection authorization, a

certificated airframe repair station, or the manufacturer of the aircraft to

supervise or conduct the progressive inspection;

(2) A current inspection procedures manual available and readily

understandable to pilot and maintenance personnel containing, in detail--

(i) An explanation of the progressive inspection, including the continuity

of inspection responsibility, the making of reports, and the keeping of

records and technical reference material;

(ii) An inspection schedule, specifying the intervals in hours or days when

routine and detailed inspections will be performed and including instructions

for exceeding an inspection interval by not more than 10 hours while en route

and for changing an inspection interval because of service experience;

(iii) Sample routine and detailed inspection forms and instructions for

their use; and

(iv) Sample reports and records and instructions for their use;

(3) Enough housing and equipment for necessary disassembly and proper

inspection of the aircraft; and

(4) Appropriate current technical information for the aircraft.

The frequency and detail of the progressive inspection shall provide for the

complete inspection of the aircraft within each 12 calendar months and be

consistent with the manufacturer's recommendations, field service experience,

and the kind of operation in which the aircraft is engaged. The progressive

inspection schedule must ensure that the aircraft, at all times, will be

airworthy and will conform to all applicable FAA aircraft specifications,

type certificate data sheets, airworthiness directives, and other approved

data. If the progressive inspection is discontinued, the owner or operator

shall immediately notify the local FAA Flight Standards district office, in

writing, of the discontinuance. After the discontinuance, the first annual

inspection under Sec. 91.409(a)(1) is due within 12 calendar months after the

last complete inspection of the aircraft under the progressive inspection.

The 100-hour inspection under Sec. 91.409(b) is due within 100 hours after

that complete inspection. A complete inspection of the aircraft, for the

purpose of determining when the annual and 100-hour inspections are due,

requires a detailed inspection of the aircraft and all its components in

accordance with the progressive inspection. A routine inspection of the

aircraft and a detailed inspection of several components is not considered to

be a complete inspection.

(e) Large airplanes (to which part 125 is not applicable), turbojet

multiengine airplanes, turbopropeller-powered multiengine airplanes, and

turbine-powered rotorcraft. No person may operate a large airplane, turbojet

multiengine airplane, turbopropeller-powered multiengine airplane, or

turbine-powered rotorcraft unless the replacement times for life-limited

parts specified in the aircraft specifications, type data sheets, or other

documents approved by the Administrator are complied with and the airplane or

turbine-powered rotorcraft, including the airframe, engines, propellers,

rotors, appliances, survival equipment, and emergency equipment, is inspected

in accordance with an inspection program selected under the provisions of

paragraph (f) of this section, except that, the owner or operator of a

turbine-powered rotorcraft may elect to use the inspection provisions of Sec.

91.409(a), (b), (c), or (d) in lieu of an inspection option of Sec.

91.409(f).

(f) Selection of inspection program under paragraph (e) of this section.

The registered owner or operator of each airplane or turbine-powered

rotorcraft described in paragraph (e) of this section must select, identify

in the aircraft maintenance records, and use one of the following programs

for the inspection of the aircraft:

(1) A continuous airworthiness inspection program that is part of a

continuous airworthiness maintenance program currently in use by a person

holding an air carrier operating certificate or an operating certificate

issued under part 121, 127, or 135 of this chapter and operating that make

and model aircraft under part 121 of this chapter or operating that make and

model under part 135 of this chapter and maintaining it under Sec.

135.411(a)(2) of this chapter.

(2) An approved aircraft inspection program approved under Sec. 135.419 of

this chapter and currently in use by a person holding an operating

certificate issued under part 135 of this chapter.

(3) A current inspection program recommended by the manufacturer.

(4) Any other inspection program established by the registered owner or

operator of that airplane or turbine-powered rotorcraft and approved by the

Administrator under paragraph (g) of this section. However, the Administrator

may require revision of this inspection program in accordance with the

provisions of Sec. 91.415.

Each operator shall include in the selected program the name and address of

the person responsible for scheduling the inspections required by the program

and make a copy of that program available to the person performing

inspections on the aircraft and, upon request, to the Administrator.

(g) Inspection program approved under paragraph (e) of this section. Each

operator of an airplane or turbine-powered rotorcraft desiring to establish

or change an approved inspection program under paragraph (f)(4) of this

section must submit the program for approval to the local FAA Flight

Standards district office having jurisdiction over the area in which the

aircraft is based. The program must be in writing and include at least the

following information:

(1) Instructions and procedures for the conduct of inspections for the

particular make and model airplane or turbine-powered rotorcraft, including

necessary tests and checks. The instructions and procedures must set forth in

detail the parts and areas of the airframe, engines, propellers, rotors, and

appliances, including survival and emergency equipment required to be

inspected.

(2) A schedule for performing the inspections that must be performed under

the program expressed in terms of the time in service, calendar time, number

of system operations, or any combination of these.

(h) Changes from one inspection program to another. When an operator

changes from one inspection program under paragraph (f) of this section to

another, the time in service, calendar times, or cycles of operation

accumulated under the previous program must be applied in determining

inspection due times under the new program.

Sec. 91.411  Altimeter system and altitude reporting equipment tests an

inspections.

(a) No person may operate an airplane, or helicopter, in controlled

airspace under IFR unless--

(1) Within the preceding 24 calendar months, each static pressure system,

each altimeter instrument, and each automatic pressure altitude reporting

system has been tested and inspected and found to comply with appendix E of

part 43 of this chapter;

(2) Except for the use of system drain and alternate static pressure

valves, following any opening and closing of the static pressure system, that

system has been tested and inspected and found to comply with paragraph (a),

appendices E and F, of part 43 of this chapter; and

(3) Following installation or maintenance on the automatic pressure

altitude reporting system of the ATC transponder where data correspondence

error could be introduced, the integrated system has been tested, inspected,

and found to comply with paragraph (c), appendix E, of part 43 of this

chapter.

(b) The tests required by paragraph (a) of this section must be conducted

by--

(1) The manufacturer of the airplane, or helicopter, on which the tests and

inspections are to be performed;

(2) A certificated repair station properly equipped to perform those

functions and holding--

(i) An instrument rating, Class I;

(ii) A limited instrument rating appropriate to the make and model of

appliance to be tested;

(iii) A limited rating appropriate to the test to be performed;

(iv) An airframe rating appropriate to the airplane, or helicopter, to be

tested; or

(v) A limited rating for a manufacturer issued for the appliance in

accordance with Sec. 145.101(b)(4) of this chapter; or

(3) A certificated mechanic with an airframe rating (static pressure system

tests and inspections only).

(c) Altimeter and altitude reporting equipment approved under Technical

Standard Orders are considered to be tested and inspected as of the date of

their manufacture.

(d) No person may operate an airplane, or helicopter, in controlled

airspace under IFR at an altitude above the maximum altitude at which all

altimeters and the automatic altitude reporting system of that airplane, or

helicopter, have been tested.

Sec. 91.413  ATC transponder tests and inspections.

(a) No persons may use an ATC transponder that is specified in 91.215(a),

121.345(c), 127.123(b), or Sec. 135.143(c) of this chapter unless, within the

preceding 24 calendar months, the ATC transponder has been tested and

inspected and found to comply with appendix F of part 43 of this chapter; and

(b) Following any installation or maintenance on an ATC transponder where

data correspondence error could be introduced, the integrated system has been

tested, inspected, and found to comply with paragraph (c), appendix E, of

part 43 of this chapter.

(c) The tests and inspections specified in this section must be conducted

by--

(1) A certificated repair station properly equipped to perform those

functions and holding--

(i) A radio rating, Class III;

(ii) A limited radio rating appropriate to the make and model transponder

to be tested;

(iii) A limited rating appropriate to the test to be performed;

(iv) A limited rating for a manufacturer issued for the transponder in

accordance with Sec. 145.101(b)(4) of this chapter; or

(2) A holder of a continuous airworthiness maintenance program as provided

in part 121, 127 or Sec. 135.411(a)(2) of this chapter; or

(3) The manufacturer of the aircraft on which the transponder to be tested

is installed, if the transponder was installed by that manufacturer.

Sec. 91.415  Changes to aircraft inspection programs.

(a) Whenever the Administrator finds that revisions to an approved aircraft

inspection program under Sec. 91.409(f)(4) are necessary for the continued

adequacy of the program, the owner or operator shall, after notification by

the Administrator, make any changes in the program found to be necessary by

the Administrator.

(b) The owner or operator may petition the Administrator to reconsider the

notice to make any changes in a program in accordance with paragraph (a) of

this section.

(c) The petition must be filed with the FAA Flight Standards district

office which requested the change to the program within 30 days after the

certificate holder receives the notice.

(d) Except in the case of an emergency requiring immediate action in the

interest of safety, the filing of the petition stays the notice pending a

decision by the Administrator.

Sec. 91.417  Maintenance records.

(a) Except for work performed in accordance with Secs. 91.411 and 91.413,

each registered owner or operator shall keep the following records for the

periods specified in paragraph (b) of this section:

(1) Records of the maintenance, preventive maintenance, and alteration and

records of the 100-hour, annual, progressive, and other required or approved

inspections, as appropriate, for each aircraft (including the airframe) and

each engine, propeller, rotor, and appliance of an aircraft. The records must

include--

(i) A description (or reference to data acceptable to the Administrator) of

the work performed; and

(ii) The date of completion of the work performed; and

(iii) The signature, and certificate number of the person approving the

aircraft for return to service.

(2) Records containing the following information:

(i) The total time in service of the airframe, each engine, each propeller,

and each rotor.

(ii) The current status of life-limited parts of each airframe, engine,

propeller, rotor, and appliance.

(iii) The time since last overhaul of all items installed on the aircraft

which are required to be overhauled on a specified time basis.

(iv) The current inspection status of the aircraft, including the time

since the last inspection required by the inspection program under which the

aircraft and its appliances are maintained.

(v) The current status of applicable airworthiness directives (AD)

including, for each, the method of compliance, the AD number, and revision

date. If the AD involves recurring action, the time and date when the next

action is required.

(vi) Copies of the forms prescribed by Sec. 43.9(a) of this chapter for

each major alteration to the airframe and currently installed engines,

rotors, propellers, and appliances.

(b) The owner or operator shall retain the following records for the

periods prescribed:

(1) The records specified in paragraph (a)(1) of this section shall be

retained until the work is repeated or superseded by other work or for 1 year

after the work is performed.

(2) The records specified in paragraph (a)(2) of this section shall be

retained and transferred with the aircraft at the time the aircraft is sold.

(3) A list of defects furnished to a registered owner or operator under

Sec. 43.11 of this chapter shall be retained until the defects are repaired

and the aircraft is approved for return to service.

(c) The owner or operator shall make all maintenance records required to be

kept by this section available for inspection by the Administrator or any

authorized representative of the National Transportation Safety Board (NTSB).

In addition, the owner or operator shall present Form 337 described in

paragraph (d) of this section for inspection upon request of any law

enforcement officer.

(d) When a fuel tank is installed within the passenger compartment or a

baggage compartment pursuant to part 43 of this chapter, a copy of FAA Form

337 shall be kept on board the modified aircraft by the owner or operator.

Sec. 91.419  Transfer of maintenance records.

Any owner or operator who sells a U.S.-registered aircraft shall transfer

to the purchaser, at the time of sale, the following records of that

aircraft, in plain language form or in coded form at the election of the

purchaser, if the coded form provides for the preservation and retrieval of

information in a manner acceptable to the Administrator:

(a) The records specified in Sec. 91.417(a)(2).

(b) The records specified in Sec. 91.417(a)(1) which are not included in

the records covered by paragraph (a) of this section, except that the

purchaser may permit the seller to keep physical custody of such records.

However, custody of records by the seller does not relieve the purchaser of

the responsibility under Sec. 91.417(c) to make the records available for

inspection by the Administrator or any authorized representative of the

National Transportation Safety Board (NTSB).

Sec. 91.421  Rebuilt engine maintenance records.

(a) The owner or operator may use a new maintenance record, without

previous operating history, for an aircraft engine rebuilt by the

manufacturer or by an agency approved by the manufacturer.

(b) Each manufacturer or agency that grants zero time to an engine rebuilt

by it shall enter in the new record--

(1) A signed statement of the date the engine was rebuilt;

(2) Each change made as required by airworthiness directives; and

(3) Each change made in compliance with manufacturer's service bulletins,

if the entry is specifically requested in that bulletin.

(c) For the purposes of this section, a rebuilt engine is a used engine

that has been completely disassembled, inspected, repaired as necessary,

reassembled, tested, and approved in the same manner and to the same

tolerances and limits as a new engine with either new or used parts. However,

all parts used in it must conform to the production drawing tolerances and

limits for new parts or be of approved oversized or undersized dimensions for

a new engine.

Secs. 91.423--91.499  [Reserved]

Subpart F--Large and Turbine-Powered Multiengine Airplanes

Sec. 91.501  Applicability.

(a) This subpart prescribes operating rules, in addition to those

prescribed in other subparts of this part, governing the operation of large

and of turbojet-powered multiengine civil airplanes of U.S. registry. The

operating rules in this subpart do not apply to those airplanes when they are

required to be operated under parts 121, 125, 129, 135, and 137 of this

chapter. (Section 91.409 prescribes an inspection program for large and for

turbine-powered (turbojet and turboprop) multiengine airplanes of U.S.

registry when they are operated under this part or part 129 or 137.)

(b) Operations that may be conducted under the rules in this subpart

instead of those in parts 121, 129, 135, and 137 of this chapter when common

carriage is not involved, include--

(1) Ferry or training flights;

(2) Aerial work operations such as aerial photography or survey, or

pipeline patrol, but not including fire fighting operations;

(3) Flights for the demonstration of an airplane to prospective customers

when no charge is made except for those specified in paragraph (d) of this

section;

(4) Flights conducted by the operator of an airplane for his personal

transportation, or the transportation of his guests when no charge,

assessment, or fee is made for the transportation;

(5) Carriage of officials, employees, guests, and property of a company on

an airplane operated by that company, or the parent or a subsidiary of the

company or a subsidiary of the parent, when the carriage is within the scope

of, and incidental to, the business of the company (other than transportation

by air) and no charge, assessment or fee is made for the carriage in excess

of the cost of owning, operating, and maintaining the airplane, except that

no charge of any kind may be made for the carriage of a guest of a company,

when the carriage is not within the scope of, and incidental to, the business

of that company;

(6) The carriage of company officials, employees, and guests of the company

on an airplane operated under a time sharing, interchange, or joint ownership

agreement as defined in paragraph (c) of this section;

(7) The carriage of property (other than mail) on an airplane operated by a

person in the furtherance of a business or employment (other than

transportation by air) when the carriage is within the scope of, and

incidental to, that business or employment and no charge, assessment, or fee

is made for the carriage other than those specified in paragraph (d) of this

section;

(8) The carriage on an airplane of an athletic team, sports group, choral

group, or similar group having a common purpose or objective when there is no

charge, assessment, or fee of any kind made by any person for that carriage;

and

(9) The carriage of persons on an airplane operated by a person in the

furtherance of a business other than transportation by air for the purpose of

selling them land, goods, or property, including franchises or

distributorships, when the carriage is within the scope of, and incidental

to, that business and no charge, assessment, or fee is made for that

carriage.

(c) As used in this section--

(1) A "time sharing agreement" means an arrangement whereby a person leases

his airplane with flight crew to another person, and no charge is made for

the flights conducted under that arrangement other than those specified in

paragraph (d) of this section;

(2) An "interchange agreement" means an arrangement whereby a person leases

his airplane to another person in exchange for equal time, when needed, on

the other person's airplane, and no charge, assessment, or fee is made,

except that a charge may be made not to exceed the difference between the

cost of owning, operating, and maintaining the two airplanes;

(3) A "joint ownership agreement" means an arrangement whereby one of the

registered joint owners of an airplane employs and furnishes the flight crew

for that airplane and each of the registered joint owners pays a share of the

charge specified in the agreement.

(d) The following may be charged, as expenses of a specific flight, for

transportation as authorized by paragraphs (b) (3) and (7) and (c)(1) of this

section:

(1) Fuel, oil, lubricants, and other additives.

(2) Travel expenses of the crew, including food, lodging, and ground

transportation.

(3) Hangar and tie-down costs away from the aircraft's base of operation.

(4) Insurance obtained for the specific flight.

(5) Landing fees, airport taxes, and similar assessments.

(6) Customs, foreign permit, and similar fees directly related to the

flight.

(7) In flight food and beverages.

(8) Passenger ground transportation.

(9) Flight planning and weather contract services.

(10) An additional charge equal to 100 percent of the expenses listed in

paragraph (d)(1) of this section.

Sec. 91.503  Flying equipment and operating information.

(a) The pilot in command of an airplane shall ensure that the following

flying equipment and aeronautical charts and data, in current and appropriate

form, are accessible for each flight at the pilot station of the airplane:

(1) A flashlight having at least two size "D" cells, or the equivalent,

that is in good working order.

(2) A cockpit checklist containing the procedures required by paragraph (b)

of this section.

(3) Pertinent aeronautical charts.

(4) For IFR, VFR over-the-top, or night operations, each pertinent

navigational en route, terminal area, and approach and letdown chart.

(5) In the case of multiengine airplanes, one-engine inoperative climb

performance data.

(b) Each cockpit checklist must contain the following procedures and shall

be used by the flight crewmembers when operating the airplane:

(1) Before starting engines.

(2) Before takeoff.

(3) Cruise.

(4) Before landing.

(5) After landing.

(6) Stopping engines.

(7) Emergencies.

(c) Each emergency cockpit checklist procedure required by paragraph (b)(7)

of this section must contain the following procedures, as appropriate:

(1) Emergency operation of fuel, hydraulic, electrical, and mechanical

systems.

(2) Emergency operation of instruments and controls.

(3) Engine inoperative procedures.

(4) Any other procedures necessary for safety.

(d) The equipment, charts, and data prescribed in this section shall be

used by the pilot in command and other members of the flight crew, when

pertinent.

Sec. 91.505  Familiarity with operating limitations and emergency equipent.

(a) Each pilot in command of an airplane shall, before beginning a flight,

become familiar with the Airplane Flight Manual for that airplane, if one is

required, and with any placards, listings, instrument markings, or any

combination thereof, containing each operating limitation prescribed for that

airplane by the Administrator, including those specified in Sec. 91.9(b).

(b) Each required member of the crew shall, before beginning a flight,

become familiar with the emergency equipment installed on the airplane to

which that crewmember is assigned and with the procedures to be followed for

the use of that equipment in an emergency situation.

Sec. 91.507  Equipment requirements: Over-the-top or night VFR operatios.

No person may operate an airplane over-the-top or at night under VFR unless

that airplane is equipped with the instruments and equipment required for IFR

operations under Sec. 91.205(d) and one electric landing light for night

operations. Each required instrument and item of equipment must be in

operable condition.

Sec. 91.509  Survival equipment for overwater operations.

(a) No person may take off an airplane for a flight over water more than 50

nautical miles from the nearest shore unless that airplane is equipped with a

life preserver or an approved flotation means for each occupant of the

airplane.

(b) No person may take off an airplane for a flight over water more than 30

minutes flying time or 100 nautical miles from the nearest shore unless it

has on board the following survival equipment:

(1) A life preserver, equipped with an approved survivor locator light, for

each occupant of the airplane.

(2) Enough liferafts (each equipped with an approved survival locator

light) of a rated capacity and buoyancy to accommodate the occupants of the

airplane.

(3) At least one pyrotechnic signaling device for each liferaft.

(4) One self-buoyant, water-resistant, portable emergency radio signaling

device that is capable of transmission on the appropriate emergency frequency

or frequencies and not dependent upon the airplane power supply.

(5) A lifeline stored in accordance with Sec. 25.1411(g) of this chapter.

(c) The required liferafts, life preservers, and signaling devices must be

installed in conspicuously marked locations and easily accessible in the

event of a ditching without appreciable time for preparatory procedures.

(d) A survival kit, appropriately equipped for the route to be flown, must

be attached to each required liferaft.

(e) As used in this section, the term shore means that area of the land

adjacent to the water which is above the high water mark and excludes land

areas which are intermittently under water.

Sec. 91.511  Radio equipment for overwater operations.

(a) Except as provided in paragraphs (c) and (d) of this section, no person

may take off an airplane for a flight over water more than 30 minutes flying

time or 100 nautical miles from the nearest shore unless it has at least the

following operable equipment:

(1) Radio communication equipment appropriate to the facilities to be used

and able to transmit to, and receive from, any place on the route, at least

one surface facility:

(i) Two transmitters.

(ii) Two microphones.

(iii) Two headsets or one headset and one speaker.

(iv) Two independent receivers.

(2) Appropriate electronic navigational equipment consisting of at least

two independent electronic navigation units capable of providing the pilot

with the information necessary to navigate the airplane within the airspace

assigned by air traffic control. However, a receiver that can receive both

communications and required navigational signals may be used in place of a

separate communications receiver and a separate navigational signal receiver

or unit.

(b) For the purposes of paragraphs (a)(1)(iv) and (a)(2) of this section, a

receiver or electronic navigation unit is independent if the function of any

part of it does not depend on the functioning of any part of another receiver

or electronic navigation unit.

(c) Notwithstanding the provisions of paragraph (a) of this section, a

person may operate an airplane on which no passengers are carried from a

place where repairs or replacement cannot be made to a place where they can

be made, if not more than one of each of the dual items of radio

communication and navigational equipment specified in paragraphs (a)(1) (i)

through (iv) and (a)(2) of this section malfunctions or becomes inoperative.

(d) Notwithstanding the provisions of paragraph (a) of this section, when

both VHF and HF communications equipment are required for the route and the

airplane has two VHF transmitters and two VHF receivers for communications,

only one HF transmitter and one HF receiver is required for communications.

(e) As used in this section, the term "shore" means that area of the land

adjacent to the water which is above the high-water mark and excludes land

areas which are intermittently under water.

Sec. 91.513  Emergency equipment.

(a) No person may operate an airplane unless it is equipped with the

emergency equipment listed in this section.

(b) Each item of equipment--

(1) Must be inspected in accordance with Sec. 91.409 to ensure its

continued serviceability and immediate readiness for its intended purposes;

(2) Must be readily accessible to the crew;

(3) Must clearly indicate its method of operation; and

(4) When carried in a compartment or container, must have that compartment

or container marked as to contents and date of last inspection.

(c) Hand fire extinguishers must be provided for use in crew, passenger,

and cargo compartments in accordance with the following:

(1) The type and quantity of extinguishing agent must be suitable for the

kinds of fires likely to occur in the compartment where the extinguisher is

intended to be used.

(2) At least one hand fire extinguisher must be provided and located on or

near the flight deck in a place that is readily accessible to the flight

crew.

(3) At least one hand fire extinguisher must be conveniently located in the

passenger compartment of each airplane accommodating more than six but less

than 31 passengers, and at least two hand fire extinguishers must be

conveniently located in the passenger compartment of each airplane

accommodating more than 30 passengers.

(4) Hand fire extinguishers must be installed and secured in such a manner

that they will not interfere with the safe operation of the airplane or

adversely affect the safety of the crew and passengers. They must be readily

accessible and, unless the locations of the fire extinguishers are obvious,

their stowage provisions must be properly identified.

(d) First aid kits for treatment of injuries likely to occur in flight or

in minor accidents must be provided.

(e) Each airplane accommodating more than 19 passengers must be equipped

with a crash axe.

(f) Each passenger-carrying airplane must have a portable battery-powered

megaphone or megaphones readily accessible to the crewmembers assigned to

direct emergency evacuation, installed as follows:

(1) One megaphone on each airplane with a seating capacity of more than 60

but less than 100 passengers, at the most rearward location in the passenger

cabin where it would be readily accessible to a normal flight attendant seat.

However, the Administrator may grant a deviation from the requirements of

this subparagraph if the Administrator finds that a different location would

be more useful for evacuation of persons during an emergency.

(2) On each airplane with a seating capacity of 100 or more passengers, one

megaphone installed at the forward end and one installed at the most rearward

location where it would be readily accessible to a normal flight attendant

seat.

Sec. 91.515  Flight altitude rules.

(a) Notwithstanding Sec. 91.119, and except as provided in paragraph (b) of

this section, no person may operate an airplane under VFR at less than--

(1) One thousand feet above the surface, or 1,000 feet from any mountain,

hill, or other obstruction to flight, for day operations; and

(2) The altitudes prescribed in Sec. 91.177, for night operations.

(b) This section does not apply--

(1) During takeoff or landing;

(2) When a different altitude is authorized by a waiver to this section

under subpart J of this part; or

(3) When a flight is conducted under the special VFR weather minimums of

Sec. 91.157 with an appropriate clearance from ATC.

Sec. 91.517   Passenger information.

(a) Except as provided in paragraph (b) of this section, no person may

operate an airplane carrying passengers unless it is equipped with signs that

are visible to passengers and flight attendants to notify them when smoking

is prohibited and when safety belts must be fastened. The signs must be so

constructed that the crew can turn them on and off. They must be turned on

during airplane movement on the surface, for each takeoff, for each landing,

and when otherwise considered to be necessary by the pilot in command.

(b) The pilot in command of an airplane that is not required, in accordance

with applicable aircraft and equipment requirements of this chapter, to be

equipped as provided in paragraph (a) of this section shall ensure that the

passengers are notified orally each time that it is necessary to fasten their

safety belts and when smoking is prohibited.

(c) If passenger information signs are installed, no passenger or

crewmember may smoke while any "no smoking" sign is lighted nor may any

passenger or crewmember smoke in any lavatory.

(d) Each passenger required by Sec. 91.107(a)(3) to occupy a seat or berth

shall fasten his or her safety belt about him or her and keep it fastened

while any "fasten seat belt" sign is lighted.

(e) Each passenger shall comply with instructions given him or her by

crewmembers regarding compliance with paragraphs (b), (c), and (d) of this

section.

Sec. 91.519  Passenger briefing.

(a) Before each takeoff the pilot in command of an airplane carrying

passengers shall ensure that all passengers have been orally briefed on--

(1) Smoking: Each passenger shall be briefed on when, where, and under what

conditions smoking is prohibited. This briefing shall include a statement, as

appropriate, that the Federal Aviation Regulations require passenger

compliance with lighted passenger information signs and no smoking placards,

prohibit smoking in lavatories, and require compliance with crewmember

instructions with regard to these items;

(2) Use of safety belts and shoulder harnesses: Each passenger shall be

briefed on when, where, and under what conditions it is necessary to have his

or her safety belt and, if installed, his or her shoulder harness fastened

about him or her. This briefing shall include a statement, as appropriate,

that Federal Aviation Regulations require passenger compliance with the

lighted passenger sign and/or crewmember instructions with regard to these

items;

(3) Location and means for opening the passenger entry door and emergency

exits;

(4) Location of survival equipment;

(5) Ditching procedures and the use of flotation equipment required under

Sec. 91.509 for a flight over water; and

(6) The normal and emergency use of oxygen equipment installed on the

airplane.

(b) The oral briefing required by paragraph (a) of this section shall be

given by the pilot in command or a member of the crew, but need not be given

when the pilot in command determines that the passengers are familiar with

the contents of the briefing. It may be supplemented by printed cards for the

use of each passenger containing--

(1) A diagram of, and methods of operating, the emergency exits; and

(2) Other instructions necessary for use of emergency equipment.

(c) Each card used under paragraph (b) must be carried in convenient

locations on the airplane for the use of each passenger and must contain

information that is pertinent only to the type and model airplane on which it

is used.

Sec. 91.521  Shoulder harness.

(a) No person may operate a transport category airplane that was type

certificated after January 1, 1958, unless it is equipped at each seat at a

flight deck station with a combined safety belt and shoulder harness that

meets the applicable requirements specified in Sec. 25.785 of this chapter,

except that--

(1) Shoulder harnesses and combined safety belt and shoulder harnesses that

were approved and installed before March 6, 1980, may continue to be used;

and

(2) Safety belt and shoulder harness restraint systems may be designed to

the inertia load factors established under the certification basis of the

airplane.

(b) No person may operate a transport category airplane unless it is

equipped at each required flight attendant seat in the passenger compartment

with a combined safety belt and shoulder harness that meets the applicable

requirements specified in Sec. 25.785 of this chapter, except that--

(1) Shoulder harnesses and combined safety belt and shoulder harnesses that

were approved and installed before March 6, 1980, may continue to be used;

and

(2) Safety belt and shoulder harness restraint systems may be designed to

the inertia load factors established under the certification basis of the

airplane.

Sec. 91.523  Carry-on baggage.

No pilot in command of an airplane having a seating capacity of more than

19 passengers may permit a passenger to stow baggage aboard that airplane

except--

(a) In a suitable baggage or cargo storage compartment, or as provided in

Sec. 91.525; or

(b) Under a passenger seat in such a way that it will not slide forward

under crash impacts severe enough to induce the ultimate inertia forces

specified in Sec. 25.561(b)(3) of this chapter, or the requirements of the

regulations under which the airplane was type certificated. Restraining

devices must also limit sideward motion of under-seat baggage and be designed

to withstand crash impacts severe enough to induce sideward forces specified

in Sec. 25.561(b)(3) of this chapter.

Sec. 91.525  Carriage of cargo.

(a) No pilot in command may permit cargo to be carried in any airplane

unless--

(1) It is carried in an approved cargo rack, bin, or compartment installed

in the airplane;

(2) It is secured by means approved by the Administrator; or

(3) It is carried in accordance with each of the following:

(i) It is properly secured by a safety belt or other tiedown having enough

strength to eliminate the possibility of shifting under all normally

anticipated flight and ground conditions.

(ii) It is packaged or covered to avoid possible injury to passengers.

(iii) It does not impose any load on seats or on the floor structure that

exceeds the load limitation for those components.

(iv) It is not located in a position that restricts the access to or use of

any required emergency or regular exit, or the use of the aisle between the

crew and the passenger compartment.

(v) It is not carried directly above seated passengers.

(b) When cargo is carried in cargo compartments that are designed to

require the physical entry of a crewmember to extinguish any fire that may

occur during flight, the cargo must be loaded so as to allow a crewmember to

effectively reach all parts of the compartment with the contents of a hand

fire extinguisher.

Sec. 91.527  Operating in icing conditions.

(a) No pilot may take off an airplane that has--

(1) Frost, snow, or ice adhering to any propeller, windshield, or

powerplant installation or to an airspeed, altimeter, rate of climb, or

flight attitude instrument system;

(2) Snow or ice adhering to the wings or stabilizing or control surfaces;

or

(3) Any frost adhering to the wings or stabilizing or control surfaces,

unless that frost has been polished to make it smooth.

(b) Except for an airplane that has ice protection provisions that meet the

requirements in section 34 of Special Federal Aviation Regulation No. 23, or

those for transport category airplane type certification, no pilot may fly--

(1) Under IFR into known or forecast moderate icing conditions; or

(2) Under VFR into known light or moderate icing conditions unless the

aircraft has functioning de-icing or anti-icing equipment protecting each

propeller, windshield, wing, stabilizing or control surface, and each

airspeed, altimeter, rate of climb, or flight attitude instrument system.

(c) Except for an airplane that has ice protection provisions that meet the

requirements in section 34 of Special Federal Aviation Regulation No. 23, or

those for transport category airplane type certification, no pilot may fly an

airplane into known or forecast severe icing conditions.

(d) If current weather reports and briefing information relied upon by the

pilot in command indicate that the forecast icing conditions that would

otherwise prohibit the flight will not be encountered during the flight

because of changed weather conditions since the forecast, the restrictions in

paragraphs (b) and (c) of this section based on forecast conditions do not

apply.

Sec. 91.529  Flight engineer requirements.

(a) No person may operate the following airplanes without a flight

crewmember holding a current flight engineer certificate:

(1) An airplane for which a type certificate was issued before January 2,

1964, having a maximum certificated takeoff weight of more than 80,000

pounds.

(2) An airplane type certificated after January 1, 1964, for which a flight

engineer is required by the type certification requirements.

(b) No person may serve as a required flight engineer on an airplane

unless, within the preceding 6 calendar months, that person has had at least

50 hours of flight time as a flight engineer on that type airplane or has

been checked by the Administrator on that type airplane and is found to be

familiar and competent with all essential current information and operating

procedures.

Sec. 91.531  Second in command requirements.

(a) Except as provided in paragraph (b) of this section, no person may

operate the following airplanes without a pilot who is designated as second

in command of that airplane:

(1) A large airplane, except that a person may operate an airplane

certificated under SFAR 41 without a pilot who is designated as second in

command if that airplane is certificated for operation with one pilot.

(2) A turbojet-powered multiengine airplane for which two pilots are

required under the type certification requirements for that airplane.

(3) A commuter category airplane, except that a person may operate a

commuter category airplane notwithstanding paragraph (a)(1) of this section,

that has a passenger seating configuration, excluding pilot seats, of nine or

less without a pilot who is designated as second in command if that airplane

is type certificated for operations with one pilot.

(b) The Administrator may issue a letter of authorization for the operation

of an airplane without compliance with the requirements of paragraph (a) of

this section if that airplane is designed for and type certificated with only

one pilot station. The authorization contains any conditions that the

Administrator finds necessary for safe operation.

(c) No person may designate a pilot to serve as second in command, nor may

any pilot serve as second in command, of an airplane required under this

section to have two pilots unless that pilot meets the qualifications for

seoond in command prescribed in Sec. 61.55 of this chapter.

Sec. 91.533  Flight attendant requirements.

(a) No person may operate an airplane unless at least the following number

of flight attendants are on board the airplane:

(1) For airplanes having more than 19 but less than 51 passengers on board,

one flight attendant.

(2) For airplanes having more than 50 but less than 101 passengers on

board, two flight attendants.

(3) For airplanes having more than 100 passengers on board, two flight

attendants plus one additional flight attendant for each unit (or part of a

unit) of 50 passengers above 100.

(b) No person may serve as a flight attendant on an airplane when required

by paragraph (a) of this section unless that person has demonstrated to the

pilot in command familiarity with the necessary functions to be performed in

an emergency or a situation requiring emergency evacuation and is capable of

using the emergency equipment installed on that airplane.

Sec. 91.535   Stowage of food, beverage, and passenger service equipment

during aircraft movement on the surface, takeoff, and landing.

(a) No operator may move an aircraft on the surface, take off, or land when

any food, beverage, or tableware furnished by the operator is located at any

passenger seat.

(b) No operator may move an aircraft on the surface, take off, or land

unless each food and beverage tray and seat back tray table is secured in its

stowed position.

(c) No operator may permit an aircraft to move on the surface, take off, or

land unless each passenger serving cart is secured in its stowed position.

(d) No operator may permit an aircraft to move on the surface, take off, or

land unless each movie screen that extends into the aisle is stowed.

(e) Each passenger shall comply with instructions given by a crewmember

with regard to compliance with this section.

Secs. 91.536--91.599  [Reserved]

Subpart G--Additional Equipment and Operating Requirements for Large and

Transport Category Aircraft

Sec. 91.601  Applicability.

This subpart applies to operation of large and transport category U.S.-

registered civil aircraft.

Sec. 91.603  Aural speed warning device.

No person may operate a transport category airplane in air commerce unless

that airplane is equipped with an aural speed warning device that complies

with Sec. 25.1303(c)(1).

Sec. 91.605  Transport category civil airplane weight limitations.

(a) No person may take off any transport category airplane (other than a

turbine-engine-powered airplane certificated after September 30, 1958)

unless--

(1) The takeoff weight does not exceed the authorized maximum takeoff

weight for the elevation of the airport of takeoff;

(2) The elevation of the airport of takeoff is within the altitude range

for which maximum takeoff weights have been determined;

(3) Normal consumption of fuel and oil in flight to the airport of intended

landing will leave a weight on arrival not in excess of the authorized

maximum landing weight for the elevation of that airport; and

(4) The elevations of the airport of intended landing and of all specified

alternate airports are within the altitude range for which the maximum

landing weights have been determined.

(b) No person may operate a turbine-engine-powered transport category

airplane certificated after September 30, 1958, contrary to the Airplane

Flight Manual, or take off that airplane unless--

(1) The takeoff weight does not exceed the takeoff weight specified in the

Airplane Flight Manual for the elevation of the airport and for the ambient

temperature existing at the time of takeoff;

(2) Normal consumption of fuel and oil in flight to the airport of intended

landing and to the alternate airports will leave a weight on arrival not in

excess of the landing weight specified in the Airplane Flight Manual for the

elevation of each of the airports involved and for the ambient temperatures

expected at the time of landing;

(3) The takeoff weight does not exceed the weight shown in the Airplane

Flight Manual to correspond with the minimum distances required for takeoff

considering the elevation of the airport, the runway to be used, the

effective runway gradient, and the ambient temperature and wind component

existing at the time of takeoff; and

(4) Where the takeoff distance includes a clearway, the clearway distance

is not greater than one-half of--

(i) The takeoff run, in the case of airplanes certificated after September

30, 1958, and before August 30, 1959; or

(ii) The runway length, in the case of airplanes certificated after August

29, 1959.

(c) No person may take off a turbine-engine-powered transport category

airplane certificated after August 29, 1959, unless, in addition to the

requirements of paragraph (b) of this section--

(1) The accelerate-stop distance is no greater than the length of the

runway plus the length of the stopway (if present); and

(2) The takeoff distance is no greater than the length of the runway plus

the length of the clearway (if present); and

(3) The takeoff run is no greater than the length of the runway.

Sec. 91.607  Emergency exits for airplanes carrying passengers for hire

(a) Notwithstanding any other provision of this chapter, no person may

operate a large airplane (type certificated under the Civil Air Regulations

effective before April 9, 1957) in passenger-carrying operations for hire,

with more than the number of occupants--

(1) Allowed under Civil Air Regulations Sec. 4b.362 (a), (b), and (c) as in

effect on December 20, 1951; or

(2) Approved under Special Civil Air Regulations SR-387, SR-389, SR-389A,

or SR-389B, or under this section as in effect.

However, an airplane type listed in the following table may be operated with

up to the listed number of occupants (including crewmembers) and the

corresponding number of exits (including emergency exits and doors) approved

for the emergency exit of passengers or with an occupant-exit configuration

approved under paragraph (b) or (c) of this section.

Maximum    Corresponding

number of     number of

occupants       exits

including    authorized

all      for passenger

Airplane type     crewmembers       use

B-307                         61              4

B-377                         96              9

C-46                          67              4

CV-240                        53              6

CV-340 and CV-440             53              6

DC-3                          35              4

DC-3 (Super)                  39              5

DC-4                          86              5

DC-6                          87              7

DC-6B                        112             11

L-18                          17              3

L-049, L-649, L-749           87              7

L-1049 series                 96              9

M-202                         53              6

M-404                         53              7

Viscount 700 series           53              7

(b) Occupants in addition to those authorized under paragraph (a) of this

section may be carried as follows:

(1) For each additional floor-level exit at least 24 inches wide by 48

inches high, with an unobstructed 20-inch-wide access aisleway between the

exit and the main passenger aisle, 12 additional occupants.

(2) For each additional window exit located over a wing that meets the

requirements of the airworthiness standards under which the airplane was type

certificated or that is large enough to inscribe an ellipse 19x26 inches,

eight additional occupants.

(3) For each additional window exit that is not located over a wing but

that otherwise complies with paragraph (b)(2) of this section, five

additional occupants.

(4) For each airplane having a ratio (as computed from the table in

paragraph (a) of this section) of maximum number of occupants to number of

exits greater than 14:1, and for each airplane that does not have at least

one full-size, door-type exit in the side of the fuselage in the rear part of

the cabin, the first additional exit must be a floor-level exit that complies

with paragraph (b)(1) of this section and must be located in the rear part of

the cabin on the opposite side of the fuselage from the main entrance door.

However, no person may operate an airplane under this section carrying more

than 115 occupants unless there is such an exit on each side of the fuselage

in the rear part of the cabin.

(c) No person may eliminate any approved exit except in accordance with the

following:

(1) The previously authorized maximum number of occupants must be reduced

by the same number of additional occupants authorized for that exit under

this section.

(2) Exits must be eliminated in accordance with the following priority

schedule: First, non-over-wing window exits; second, over-wing window exits;

third, floor-level exits located in the forward part of the cabin; and

fourth, floor-level exits located in the rear of the cabin.

(3) At least one exit must be retained on each side of the fuselage

regardless of the number of occupants.

(4) No person may remove any exit that would result in a ratio of maximum

number of occupants to approved exits greater than 14:1.

(d) This section does not relieve any person operating under part 121 of

this chapter from complying with Sec. 121.291.

Sec. 91.609  Flight recorders and cockpit voice recorders.

(a) No holder of an air carrier operating certificate or an operating

certificate may conduct any operation under this part with an aircraft listed

in the holder's operations specifications or current list of aircraft used in

air transportation unless that aircraft complies with any applicable flight

recorder and cockpit voice recorder requirements of the part under which its

certificate is issued except that the operator may--

(1) Ferry an aircraft with an inoperative flight recorder or cockpit voice

recorder from a place where repair or replacement cannot be made to a place

where they can be made;

(2) Continue a flight as originally planned, if the flight recorder or

cockpit voice recorder becomes inoperative after the aircraft has taken off;

(3) Conduct an airworthiness flight test during which the flight recorder

or cockpit voice recorder is turned off to test it or to test any

communications or electrical equipment installed in the aircraft; or

(4) Ferry a newly acquired aircraft from the place where possession of it

is taken to a place where the flight recorder or cockpit voice recorder is to

be installed.

(b) Notwithstanding paragraphs (c) and (e) of this section, an operator

other than the holder of an air carrier or a commercial operator certificate

may--

(1) Ferry an aircraft with an inoperative flight recorder or cockpit voice

recorder from a place where repair or replacement cannot be made to a place

where they can be made;

(2) Continue a flight as originally planned if the flight recorder or

cockpit voice recorder becomes inoperative after the aircraft has taken off;

(3) Conduct an airworthiness flight test during which the flight recorder

or cockpit voice recorder is turned off to test it or to test any

communications or electrical equipment installed in the aircraft;

(4) Ferry a newly acquired aircraft from a place where possession of it was

taken to a place where the flight recorder or cockpit voice recorder is to be

installed; or

(5) Operate an aircraft:

(i) For not more than 15 days while the flight recorder and/or cockpit

voice recorder is inoperative and/or removed for repair provided that the

aircraft maintenance records contain an entry that indicates the date of

failure, and a placard is located in view of the pilot to show that the

flight recorder or cockpit voice recorder is inoperative.

(ii) For not more than an additional 15 days, provided that the

requirements in paragraph (b)(5)(i) are met and that a certificated pilot, or

a certificated person authorized to return an aircraft to service under Sec.

43.7 of this chapter, certifies in the aircraft maintenance records that

additional time is required to complete repairs or obtain a replacement unit.

(c) No person may operate a U.S. civil registered, multiengine, turbine-

powered airplane or rotorcraft having a passenger seating configuration,

excluding any pilot seats of 10 or more that has been manufactured after

October 11, 1991, unless it is equipped with one or more approved flight

recorders that utilize a digital method of recording and storing data and a

method of readily retrieving that data from the storage medium, that are

capable of recording the data specified in appendix E to this part, for an

airplane, or appendix F to this part, for a rotorcraft, of this part within

the range, accuracy, and recording interval specified, and that are capable

of retaining no less than 8 hours of aircraft operation.

(d) Whenever a flight recorder, required by this section, is installed, it

must be operated continuously from the instant the airplane begins the

takeoff roll or the rotorcraft begins lift-off until the airplane has

completed the landing roll or the rotorcraft has landed at its destination.

(e) Unless otherwise authorized by the Administrator, after October 11,

1991, no person may operate a U.S. civil registered multiengine, turbine-

powered airplane or rotorcraft having a passenger seating configuration of

six passengers or more and for which two pilots are required by type

certification or operating rule unless it is equipped with an approved

cockpit voice recorder that:

(1) Is installed in compliance with Sec. 23.1457(a) (1) and (2), (b), (c),

(d), (e), (f), and (g); Sec. 25.1457(a) (1) and (2), (b), (c), (d), (e), (f),

and (g); Sec. 27.1457(a) (1) and (2), (b), (c), (d), (e), (f), and (g); or

Sec. 29.1457(a) (1) and (2), (b), (c), (d), (e), (f), and (g) of this

chapter, as applicable; and

(2) Is operated continuously from the use of the checklist before the

flight to completion of the final checklist at the end of the flight.

(f) In complying with this section, an approved cockpit voice recorder

having an erasure feature may be used, so that at any time during the

operation of the recorder, information recorded more than 15 minutes earlier

may be erased or otherwise obliterated.

(g) In the event of an accident or occurrence requiring immediate

notification to the National Transportation Safety Board under part 830 of

its regulations that results in the termination of the flight, any operator

who has installed approved flight recorders and approved cockpit voice

recorders shall keep the recorded information for at least 60 days or, if

requested by the Administrator or the Board, for a longer period. Information

obtained from the record is used to assist in determining the cause of

accidents or occurrences in connection with the investigation under part 830.

The Administrator does not use the cockpit voice recorder record in any civil

penalty or certificate action.

Sec. 91.611  Authorization for ferry flight with one engine inoperative

(a) General. The holder of an air carrier operating certificate or an

operating certificate issued under Part 125 may conduct a ferry flight of a

four-engine airplane or a turbine-engine-powered airplane equipped with three

engines, with one engine inoperative, to a base for the purpose of repairing

that engine subject to the following:

(1) The airplane model has been test flown and found satisfactory for safe

flight in accordance with paragraph (b) or (c) of this section, as

appropriate. However, each operator who before November 19, 1966, has shown

that a model of airplane with an engine inoperative is satisfactory for safe

flight by a test flight conducted in accordance with performance data

contained in the applicable Airplane Flight Manual under paragraph (a)(2) of

this section need not repeat the test flight for that model.

(2) The approved Airplane Flight Manual contains the following performance

data and the flight is conducted in accordance with that data:

(i) Maximum weight.

(ii) Center of gravity limits.

(iii) Configuration of the inoperative propeller (if applicable).

(iv) Runway length for takeoff (including temperature accountability).

(v) Altitude range.

(vi) Certificate limitations.

(vii) Ranges of operational limits.

(viii) Performance information.

(ix) Operating procedures.

(3) The operator has FAA approved procedures for the safe operation of the

airplane, including specific requirements for--

(i) Limiting the operating weight on any ferry flight to the minimum

necessary for the flight plus the necessary reserve fuel load;

(ii) A limitation that takeoffs must be made from dry runways unless, based

on a showing of actual operating takeoff techniques on wet runways with one

engine inoperative, takeoffs with full controllability from wet runways have

been approved for the specific model aircraft and included in the Airplane

Flight Manual:

(iii) Operations from airports where the runways may require a takeoff or

approach over populated areas; and

(iv) Inspection procedures for determining the operating condition of the

operative engines.

(4) No person may take off an airplane under this section if--

(i) The initial climb is over thickly populated areas; or

(ii) Weather conditions at the takeoff or destination airport are less than

those required for VFR flight.

(5) Persons other than required flight crewmembers shall not be carried

during the flight.

(6) No person may use a flight crewmember for flight under this section

unless that crewmember is thoroughly familiar with the operating procedures

for one-engine inoperative ferry flight contained in the certificate holder's

manual and the limitations and performance information in the Airplane Flight

Manual.

(b) Flight tests: reciprocating-engine-powered airplanes. The airplane

performance of a reciprocating-engine-powered airplane with one engine

inoperative must be determined by flight test as follows:

(1) A speed not less than 1.3 VS1 must be chosen at which the airplane may

be controlled satisfactorily in a climb with the critical engine inoperative

(with its propeller removed or in a configuration desired by the operator and

with all other engines operating at the maximum power determined in paragraph

(b)(3) of this section.

(2) The distance required to accelerate to the speed listed in paragraph

(b)(1) of this section and to climb to 50 feet must be determined with--

(i) The landing gear extended;

(ii) The critical engine inoperative and its propeller removed or in a

configuration desired by the operator; and

(iii) The other engines operating at not more than maximum power

established under paragraph (b)(3) of this section.

(3) The takeoff, flight and landing procedures, such as the approximate

trim settings, method of power application, maximum power, and speed must be

established.

(4) The performance must be determined at a maximum weight not greater than

the weight that allows a rate of climb of at least 400 feet per minute in the

en route configuration set forth in Sec. 25.67(d) of this chapter in effect

on January 31, 1977, at an altitude of 5,000 feet.

(5) The performance must be determined using temperature accountability for

the takeoff field length, computed in accordance with Sec. 25.61 of this

chapter in effect on January 31, 1977.

(c) Flight tests: Turbine-engine-powered airplanes. The airplane

performance of a turbine-engine-powered airplane with one engine inoperative

must be determined by flight tests, including at least three takeoff tests,

in accordance with the following:

(1) Takeoff speeds VR and V2, not less than the corresponding speeds under

which the airplane was type certificated under Sec. 25.107 of this chapter,

must be chosen at which the airplane may be controlled satisfactorily with

the critical engine inoperative (with its propeller removed or in a

configuration desired by the operator, if applicable) and with all other

engines operating at not more than the power selected for type certification

as set forth in Sec. 25.101 of this chapter.

(2) The minimum takeoff field length must be the horizontal distance

required to accelerate and climb to the 35-foot height at V2 speed (including

any additional speed increment obtained in the tests) multiplied by 115

percent and determined with--

(i) The landing gear extended;

(ii) The critical engine inoperative and its propeller removed or in a

configuration desired by the operator (if applicable); and

(iii) The other engine operating at not more than the power selected for

type certification as set forth in Sec. 25.101 of this chapter.

(3) The takeoff, flight, and landing procedures such as the approximate

trim setting, method of power application, maximum power, and speed must be

established. The airplane must be satisfactorily controllable during the

entire takeoff run when operated according to these procedures.

(4) The performance must be determined at a maximum weight not greater than

the weight determined under Sec. 25.121(c) of this chapter but with--

(i) The actual steady gradient of the final takeoff climb requirement not

less than 1.2 percent at the end of the takeoff path with two critical

engines inoperative; and

(ii) The climb speed not less than the two-engine inoperative trim speed

for the actual steady gradient of the final takeoff climb prescribed by

paragraph (c)(4)(i) of this section.

(5) The airplane must be satisfactorily controllable in a climb with two

critical engines inoperative. Climb performance may be shown by calculations

based on, and equal in accuracy to, the results of testing.

(6) The performance must be determined using temperature accountability for

takeoff distance and final takeoff climb computed in accordance with Sec.

25.101 of this chapter.

For the purpose of paragraphs (c)(4) and (5) of this section, "two critical

engines" means two adjacent engines on one side of an airplane with four

engines, and the center engine and one outboard engine on an airplane with

three engines.

Sec. 91.613  Materials for compartment interiors.

No person may operate an airplane that conforms to an amended or

supplemental type certificate issued in accordance with SFAR No. 41 for a

maximum certificated takeoff weight in excess of 12,500 pounds unless within

1 year after issuance of the initial airworthiness certificate under that

SFAR the airplane meets the compartment interior requirements set forth in

Sec. 25.853 (a), (b), (b-1), (b-2), and (b-3) of this chapter in effect on

September 26, 1978.

Secs. 91.615--91.699  [Reserved]

Subpart H--Foreign Aircraft Operations and Operations of U.S.-Registered

Civil Aircraft Outside of the United States

Sec. 91.701  Applicability.

This subpart applies to the operations of civil aircraft of U.S. registry

outside of the United States and the operations of foreign civil aircraft

within the United States.

Sec. 91.703  Operations of civil aircraft of U.S. registry outside of te

United States.

(a) Each person operating a civil aircraft of U.S. registry outside of the

United States shall--

(1) When over the high seas, comply with annex 2 (Rules of the Air) to the

Convention on International Civil Aviation and with Secs. 91.117(c), 91.127,

91.129, and 91.131;

(2) When within a foreign country, comply with the regulations relating to

the flight and maneuver of aircraft there in force;

(3) Except for Secs. 91.307(b), 91.309, 91.323, and 91.711, comply with

this part so far as it is not inconsistent with applicable regulations of the

foreign country where the aircraft is operated or annex 2 of the Convention

on International Civil Aviation; and

(4) When over the North Atlantic within airspace designated as Minimum

Navigation Performance Specifications airspace, comply with Sec. 91.705.

(b) Annex 2 to the Convention on International Civil Aviation, Eighth

Edition--July 1986, with amendments through Amendment 28 effective November

1987, to which reference is made in this part, is incorporated into this part

and made a part hereof as provided in 5 U.S.C. 552 and pursuant to 1 CFR part

51. annex 2 (including a complete historic file of changes thereto) is

available for public inspection at the Rules Docket, AGC-10, Federal Aviation

Administration, 800 Independence Avenue SW., Washington, DC 20591. In

addition, Annex 2 may be purchased from the International Civil Aviation

Organization (Attention: Distribution Officer), P.O. Box 400, Succursale,

Place de L'Aviation Internationale, 1000 Sherbrooke Street West, Montreal,

Quebec, Canada H3A 2R2.

Sec. 91.705  Operations within the North Atlantic Minimum Navigation

Performance Specifications Airspace.

No person may operate a civil aircraft of U.S. registry in North Atlantic

(NAT) airspace designated as Minimum Navigation Performance Specifications

(MNPS) airspace unless--

(a) The aircraft has approved navigation performance capability which

complies with the requirements of Appendix C of this part; and

(b) The operator is authorized by the Administrator to perform such

operations.

(c) The Administrator authorizes deviations from the requirements of this

section in accordance with section 3 of appendix C to this part.

Sec. 91.707  Flights between Mexico or Canada and the United States.

Unless otherwise authorized by ATC, no person may operate a civil aircraft

between Mexico or Canada and the United States without filing an IFR or VFR

flight plan, as appropriate.

Sec. 91.709  Operations to Cuba.

No person may operate a civil aircraft from the United States to Cuba

unless--

(a) Departure is from an international airport of entry designated in Sec.

6.13 of the Air Commerce Regulations of the Bureau of Customs (19 CFR 6.13);

and

(b) In the case of departure from any of the 48 contiguous States or the

District of Columbia, the pilot in command of the aircraft has filed--

(1) A DVFR or IFR flight plan as prescribed in Sec. 99.11 or Sec. 99.13 of

this chapter; and

(2) A written statement, within 1 hour before departure, with the Office of

Immigration and Naturalization Service at the airport of departure,

containing--

(i) All information in the flight plan;

(ii) The name of each occupant of the aircraft;

(iii) The number of occupants of the aircraft; and

(iv) A description of the cargo, if any.

This section does not apply to the operation of aircraft by a scheduled air

carrier over routes authorized in operations specifications issued by the

Administrator.

(Approved by the Office of Management and Budget under OMB control number

2120-0005)

Sec. 91.711  Special rules for foreign civil aircraft.

(a) General. In addition to the other applicable regulations of this part,

each person operating a foreign civil aircraft within the United States shall

comply with this section.

(b) VFR. No person may conduct VFR operations which require two-way radio

communications under this part unless at least one crewmember of that

aircraft is able to conduct two-way radio communications in the English

language and is on duty during that operation.

(c) IFR. No person may operate a foreign civil aircraft under IFR unless--

(1) That aircraft is equipped with--

(i) Radio equipment allowing two-way radio communication with ATC when it

is operated in controlled airspace; and

(ii) Radio navigational equipment appropriate to the navigational

facilities to be used;

(2) Each person piloting the aircraft--

(i) Holds a current United States instrument rating or is authorized by his

foreign airman certificate to pilot under IFR; and

(ii) Is thoroughly familiar with the United States en route, holding, and

letdown procedures; and

(3) At least one crewmember of that aircraft is able to conduct two-way

radiotelephone communications in the English language and that crewmember is

on duty while the aircraft is approaching, operating within, or leaving the

United States.

(d) Over water. Each person operating a foreign civil aircraft over water

off the shores of the United States shall give flight notification or file a

flight plan in accordance with the Supplementary Procedures for the ICAO

region concerned.

(e) Flight at and above FL 240. If VOR navigational equipment is required

under paragraph (c)(1)(ii) of this section, no person may operate a foreign

civil aircraft within the 50 States and the District of Columbia at or above

FL 240, unless the aircraft is equipped with distance measuring equipment

(DME) capable of receiving and indicating distance information from the

VORTAC facilities to be used. When DME required by this paragraph fails at

and above FL 240, the pilot in command of the aircraft shall notify ATC

immediately and may then continue operations at and above FL 240 to the next

airport of intended landing at which repairs or replacement of the equipment

can be made. However, paragraph (e) of this section does not apply to foreign

civil aircraft that are not equipped with DME when operated for the following

purposes and if ATC is notified prior to each takeoff:

(1) Ferry flights to and from a place in the United States where repairs or

alterations are to be made.

(2) Ferry flights to a new country of registry.

(3) Flight of a new aircraft of U.S. manufacture for the purpose of--

(i) Flight testing the aircraft;

(ii) Training foreign flight crews in the operation of the aircraft; or

(iii) Ferrying the aircraft for export delivery outside the United States.

(4) Ferry, demonstration, and test flight of an aircraft brought to the

United States for the purpose of demonstration or testing the whole or any

part thereof.

Sec. 91.713  Operation of civil aircraft of Cuban registry.

No person may operate a civil aircraft of Cuban registry except in

controlled airspace and in accordance with air traffic clearance or air

traffic control instructions that may require use of specific airways or

routes and landings at specific airports.

Sec. 91.715  Special flight authorizations for foreign civil aircraft.

(a) Foreign civil aircraft may be operated without airworthiness

certificates required under Sec. 91.203 if a special flight authorization for

that operation is issued under this section. Application for a special flight

authorization must be made to the Flight Standards Division Manager or

Aircraft Certification Directorate Manager of the FAA region in which the

applicant is located or to the region within which the U.S. point of entry is

located. However, in the case of an aircraft to be operated in the U.S. for

the purpose of demonstration at an airshow, the application may be made to

the Flight Standards Division Manager or Aircraft Certification Directorate

Manager of the FAA region in which the airshow is located.

(b) The Administrator may issue a special flight authorization for a

foreign civil aircraft subject to any conditions and limitations that the

Administrator considers necessary for safe operation in the U.S. airspace.

(c) No person may operate a foreign civil aircraft under a special flight

authorization unless that operation also complies with part 375 of the

Special Regulations of the Department of Transportation (14 CFR part 375).

Secs. 91.717--91.799  [Reserved]

Subpart I--Operating Noise Limits

Sec. 91.801  Applicability: Relation to Part 36.

(a) This subpart prescribes operating noise limits and related requirements

that apply, as follows, to the operation of civil aircraft in the United

States.

(1) Sections 91.803, 91.805, 91.807, 91.809, and 91.811 apply to civil

subsonic turbojet airplanes with maximum weights of more than 75,000 pounds

and--

(i) If U.S. registered, that have standard airworthiness certificates; or

(ii) If foreign registered, that would be required by this chapter to have

a U.S. standard airworthiness certificate in order to conduct the operations

intended for the airplane were it registered in the United States. Those

sections apply to operations to or from airports in the United States under

this part and parts 121, 125, 129, and 135 of this chapter.

(2) Section 91.813 applies to U.S. operators of civil subsonic turbojet

airplanes covered by this subpart. This section applies to operators

operating to or from airports in the United States under this part and parts

121, 125, and 135, but not to those operating under part 129 of this chapter.

(3) Sections 91.803, 91.819, and 91.821 apply to U.S.-registered civil

supersonic airplanes having standard airworthiness certificates and to

foreign-registered civil supersonic airplanes that, if registered in the

United States, would be required by this chapter to have U.S. standard

airworthiness certificates in order to conduct the operations intended for

the airplane. Those sections apply to operations under this part and under

parts 121, 125, 129, and 135 of this chapter.

(b) Unless otherwise specified, as used in this subpart "part 36" refers to

14 CFR part 36, including the noise levels under appendix C of that part,

notwithstanding the provisions of that part excepting certain airplanes from

the specified noise requirements. For purposes of this subpart, the various

stages of noise levels, the terms used to describe airplanes with respect to

those levels, and the terms "subsonic airplane" and "supersonic airplane"

have the meanings specified under part 36 of this chapter. For purposes of

this subpart, for subsonic airplanes operated in foreign air commerce in the

United States, the Administrator may accept compliance with the noise

requirements under annex 16 of the International Civil Aviation Organization

when those requirements have been shown to be substantially compatible with,

and achieve results equivalent to those achievable under, part 36 for that

airplane. Determinations made under these provisions are subject to the

limitations of Sec. 36.5 of this chapter as if those noise levels were part

36 noise levels.

(c) Sections 91.851 through 9l.875 of this subpart prescribe operating

noise limits and related requirements that apply to any civil subsonic

turbojet airplane with a maximum certificated weight of more than 75,000

pounds operating to or from an airport in the 48 contiguous United States and

the District of Columbia under this part, part 121, 125, 129, or 135 of this

chapter on and after September 25, 1991.

Sec. 91.803  Part 125 operators: Designation of applicable regulations.

For airplanes covered by this subpart and operated under part 125 of this

chapter, the following regulations apply as specified:

(a) For each airplane operation to which requirements prescribed under this

subpart applied before November 29, 1980, those requirements of this subpart

continue to apply.

(b) For each subsonic airplane operation to which requirements prescribed

under this subpart did not apply before November 29, 1980, because the

airplane was not operated in the United States under this part or part 121,

129, or 135 of this chapter, the requirements prescribed under Secs. 91.805,

91.809, 91.811, and 91.813 of this subpart apply.

(c) For each supersonic airplane operation to which requirements prescribed

under this subpart did not apply before November 29, 1980, because the

airplane was not operated in the United States under this part or part 121,

129, or 135 of this chapter, the requirements of Secs. 91.819 and 91.821 of

this subpart apply.

(d) For each airplane required to operate under part 125 for which a

deviation under that part is approved to operate, in whole or in part, under

this part or part 121, 129, or 135 of this chapter, notwithstanding the

approval, the requirements prescribed under paragraphs (a), (b), and (c) of

this section continue to apply.

Sec. 91.805  Final compliance: Subsonic airplanes.

Except as provided in Secs. 91.809 and 91.811, on and after January 1,

1985, no person may operate to or from an airport in the United States any

subsonic airplane covered by this subpart unless that airplane has been shown

to comply with Stage 2 or Stage 3 noise levels under part 36 of this chapter.

Sec. 91.807  Phased compliance under Parts 121, 125, and 135: Subsonic

airplanes.

(a) General. Each person operating airplanes under part 121, 125, or 135 of

this chapter, as prescribed under Sec. 91.803 of this subpart, regardless of

the state of registry of the airplane, shall comply with this section with

respect to subsonic airplanes covered by this subpart.

(b) Compliance schedules. Except for airplanes shown to be operated in

foreign air commerce under paragraph (c) of this section or covered by an

exemption (including those issued under Sec. 91.811), airplanes operated by

U.S. operators in air commerce in the United States must be shown to comply

with Stage 2 or Stage 3 noise levels under part 36 of this chapter, in

accordance with the following schedule, or they may not be operated to or

from airports in the United States:

(1) By January 1, 1981--

(i) At least one quarter of the airplanes that have four engines with no

bypass ratio or with a bypass ratio less than two; and

(ii) At least half of the airplanes powered by engines with any other

bypass ratio or by another number of engines.

(2) By January 1, 1983--

(i) At least one-half of the airplanes that have four engines with no

bypass ratio or with a bypass ratio less than two; and

(ii) All airplanes powered by engines with any other bypass ratio or by

another number of engines.

(c) Apportionment of airplanes. For purposes of paragraph (b) of this

section, a person operating airplanes engaged in domestic and foreign air

commerce in the United States may elect not to comply with the phased

schedule with respect to that portion of the airplanes operated by that

person shown, under an approved method of apportionment, to be engaged in

foreign air commerce in the United States.

Sec. 91.809  Replacement airplanes.

A Stage 1 airplane may be operated after the otherwise applicable

compliance dates prescribed under Secs. 91.805 and 91.807 if, under an

approved plan, a replacement airplane has been ordered by the operator under

a binding contract as follows:

(a) For replacement of an airplane powered by two engines, until January 1,

1986, but not after the date specified in the plan, if the contract is

entered into by January 1, 1983, and specifies delivery before January 1,

1986, of a replacement airplane which has been shown to comply with Stage 3

noise levels under part 36 of this chapter.

(b) For replacement of an airplane powered by three engines, until January

1, 1985, but not after the date specified in the plan, if the contract is

entered into by January 1, 1983, and specifies delivery before January 1,

1985, of a replacement airplane which has been shown to comply with Stage 3

noise levels under part 36 of this chapter.

(c) For replacement of any other airplane, until January 1, 1985, but not

after the date specified in the plan, if the contract specifies delivery

before January 1, 1985, of a replacement airplane which--

(1) Has been shown to comply with Stage 2 or Stage 3 noise levels under

part 36 of this chapter prior to issuance of an original standard

airworthiness certificate; or

(2) Has been shown to comply with Stage 3 noise levels under part 36 of

this chapter prior to issuance of a standard airworthiness certificate other

than original issue.

(d) Each operator of a Stage 1 airplane for which approval of a replacement

plan is requested under this section shall submit to the Director, Office of

Environment and Energy, an application constituting the proposed replacement

plan (or revised plan) that contains the information specified under this

paragraph and which is certified (under penalty of 18 U.S.C. 1001) as true

and correct. Each application for approval must provide information

corresponding to that specified in the contract, upon which the FAA may rely

in considering its approval, as follows:

(1) Name and address of the applicant. .

(2) Aircraft type and model and registration number for each airplane to be

replaced under the plan.

(3) Aircraft type and model of each replacement airplane.

(4) Scheduled dates of delivery and introduction into service of each

replacement airplane.

(5) Names and addresses of the parties to the contract and any other

persons who may effectively cancel the contract or otherwise control the

performance of any party.

(6) Information specifying the anticipated disposition of the airplanes to

be replaced.

(7) A statement that the contract represents a legally enforceable, mutual

agreement for delivery of an eligible replacement airplane.

(8) Any other information or documentation requested by the Director,

Office of Environment and Energy, reasonably necessary to determine whether

the plan should be approved.

Sec. 91.811  Service to small communities exemption: Two-engine, subsonc

airplanes.

(a) A Stage l airplane powered by two engines may be operated after the

compliance dates prescribed under Secs. 91.805, 91.807, and 91.809 when, with

respect to that airplane, the Administrator issues an exemption to the

operator from the noise level requirements under this subpart. Each exemption

issued under this section terminates on the earliest of the following dates:

(1) For an exempted airplane sold, or otherwise disposed of, to another

person on or after January 1, 1983, on the date of delivery to that person.

(2) For an exempted airplane with a seating configuration of 100 passenger

seats or less, on January 1, 1988.

(3) For an exempted airplane with a seating configuration of more than 100

passenger seats, on January 1, 1985.

(b) For the purpose of this section, the seating configuration of an

airplane is governed by that shown to exist on December 1, 1979, or an

earlier date established for that airplane by the Administrator.

Sec. 91.813  Compliance plans and status: U.S. operations of subsonic

airplanes.

(a) Each U.S. operator of a civil subsonic airplane covered by this subpart

(regardless of the state of registry) shall submit to the Director, Office of

Environment and Energy, in accordance with this section, the operator's

current compliance status and plan for achieving and maintaining compliance

with the applicable noise level requirements of this subpart. If appropriate,

an operator may substitute for the required plan a notice, certified as true

(under penalty of 18 U.S.C. 1001) by that operator, that no change in the

plan or status of any airplane affected by the plan has occurred since the

date of the plan most recently submitted under this section.

(b) Each compliance plan, including each revised plan, must contain the

information specified under paragraph (c) of this section for each airplane

covered by this section that is operated by the operator. Unless otherwise

approved by the Administrator, compliance plans must provide the required

plan and status information as it exists on the date 30 days before the date

specified for submission of the plan. Plans must be certified by the operator

as true and complete (under penalty of 18 U.S.C. 1001) and be submitted for

each airplane covered by this section on or before 90 days after initially

commencing operation of airplanes covered by this section, whichever is

later, and thereafter--

(1) Thirty days after any change in the operator's fleet or compliance

planning decisions that has a separate or cumulative effect on 10 percent or

more of the airplanes in either class of airplanes covered by Sec. 91.807(b);

and

(2) Thirty days after each compliance date applicable to that airplane

under this subpart, and annually thereafter through 1985, or until any later

date for that airplane prescribed under this subpart, on the anniversary of

that submission date, to show continuous compliance with this subpart.

(c) Each compliance plan submitted under this section must identify the

operator and include information regarding the compliance plan and status for

each airplane covered by the plan as follows:

(1) Name and address of the airplane operator.

(2) Name and telephone number of the person designated by the operator to

be responsible for the preparation of the compliance plan and its submission.

(3) The total number of airplanes covered by this section and in each of

the following classes and subclasses:

(i) For airplanes engaged in domestic air commerce--

(A) Airplanes powered by four turbojet engines with no bypass ratio or with

a bypass ratio less than two;

(B) Airplanes powered by engines with any other bypass ratio or by another

number of engines; and

(C) Airplanes covered by an exemption issued under Sec. 91.811 of this

subpart.

(ii) For airplanes engaged in foreign air commerce under an approved

apportionment plan--

(A) Airplanes powered by four turbojet engines with no bypass ratio or with

a bypass ratio less than two;

(B) Airplanes powered by engines with any other bypass ratio or by another

number of engines; and

(C) Airplanes covered by an exemption issued under Sec. 91.811 of this

subpart.

(4) For each airplane covered by this section--

(i) Aircraft type and model;

(ii) Aircraft registration number;

(iii) Aircraft manufacturer serial number;

(iv) Aircraft powerplant make and model;

(v) Aircraft year of manufacture;

(vi) Whether part 36 noise level compliance has been shown, "Yes/No";

(vii) The appropriate code prescribed under paragraph (c)(5) of this

section which indicates the acoustical technology installed, or to be

installed, on the airplane;

(viii) For airplanes on which acoustical technology has been or will be

applied, following the appropriate code entry, the actual or scheduled month

and year of installation on the airplane;

(ix) For DC-8 and B-707 airplanes operated in domestic U.S. air commerce

which have been or will be retired from service in the United States without

replacement between January 24, 1977, and January 1, 1985, the appropriate

code prescribed under paragraph (c)(5) of this section followed by the actual

or scheduled month and year of retirement of the airplane from service;

(x) For DC-8 and B-707 airplanes operated in foreign air commerce in the

United States which have been or will be retired from service in the United

States without replacement between April 14, 1980, and January 1, 1985, the

appropriate code prescribed under paragraph (c)(5) of this section followed

by the actual or scheduled month and year of retirement of the airplane from

service;

(xi) For airplanes covered by an approved replacement plan under Sec.

91.807(c) of this subpart, the appropriate code prescribed under paragraph

(c)(5) of this section followed by the scheduled month and year for

replacement of the airplane;

(xii) For airplanes designated as "engaged in foreign commerce" in

accordance with an approved method of apportionment under Sec. 91.807(c) of

this subpart, the appropriate code prescribed under paragraph (c)(5) of this

section;

(xiii) For airplanes covered by an exemption issued to the operator

granting relief from noise level requirements of this subpart, the

appropriate code prescribed under paragraph (c)(5) of this section followed

by the actual or scheduled month and year of expiration of the exemption and

the appropriate code and applicable dates which indicate the compliance

strategy planned or implemented for the airplane;

(xiv) For all airplanes covered by this section, the number of spare

shipsets of acoustical components needed for continuous compliance and the

number available on demand to the operator in support of those airplanes; and

(xv) For airplanes for which none of the other codes prescribed under

paragraph (c)(5) of this section describes either the technology applied or

to be applied to the airplane in accordance with the certification

requirements under Parts 21 and 36 of this chapter, or the compliance

strategy or methodology following the code "OTH," enter the date of any

certificate action and attach an addendum to the plan explaining the nature

and the extent of the certificated technology, strategy, or methodology

employed, with reference to the type certificate documentation.

(5) Table of Acoustical Technology/Strategy Codes

Code         Airplane type/model                Certificate technology

A     B-707-120B; B-707-320B/C; B-720B    Quiet nacelles + 1-ring.

B     B-727-100                           Double wall fan duct treatment.

C     B-727-200                           Double wall fan duct treatment

(pre-January 1977 installations

and amended type certificate).

D     B-727-200; B-737-100; B-737-200     Quiet nacelles + double wall fan

duct treatment.

E     B-747-100 (pre-December 1971); B-   Fixed lip inlets + sound absorbing

747-200 (pre-December 1971)         material treatment.

F     DC-8                                New extended inlet and bullet with

treatment + fan duct treatment

areas.

G     DC-9                                P-36 sound absorbing material

treatment kit.

H     BAC-111-200                         Silencer kit (BAC Acoustic Report

522).

I     BAC-111-400                         Silencer kit (BAC Acoustic Report

598).

J     B-707; DC-8                         Reengined with high bypass ratio

turbojet engines + quiet nacelles

(if certificated under stage 3

noise level requirements).

REP--For airplanes covered by an approved replacement plan under Sec.

91.807(c) of this subpart.

EFC--For airplanes designated as "engaged in foreign commerce" in

accordance with an approved method of apportionment under Sec. 91.811 of this

subpart.

RET--For DC-8 and B-707 airplanes operated in domestic U.S. air commerce

and retired from service in the United States without replacement between

January 24, 1977, and January 1, 1985. RFC--For DC-8 and B-707 airplanes

operated by U.S. operators in foreign air commerce in the United States and

retired from service in the United States without replacement between April

14, 1980, and January 1, 1985.

EXD--For airplanes exempted from showing compliance with the noise level

requirements of this subpart.

OTH--For airplanes for which no other prescribed code describes either the

certificated technology applied or to be applied to the airplane, or the

compliance strategy or methodology. (An addendum must explain the nature and

extent of technology, strategy, or methodology and reference the type

certificate documentation.)

Sec. 91.815  Agricultural and fire fighting airplanes: Noise operating

limitations.

(a) This section applies to propeller-driven, small airplanes having

standard airworthiness certificates that are designed for "agricultural

aircraft operations" (as defined in Sec. 137.3 of this chapter, as effective

on January 1, 1966) or for dispensing fire fighting materials.

(b) If the Airplane Flight Manual, or other approved manual material

information, markings, or placards for the airplane indicate that the

airplane has not been shown to comply with the noise limits under part 36 of

this chapter, no person may operate that airplane, except--

(1) To the extent necessary to accomplish the work activity directly

associated with the purpose for which it is designed;

(2) To provide flight crewmember training in the special purpose operation

for which the airplane is designed; and

(3) To conduct "nondispensing aerial work operations" in accordance with

the requirements under Sec. 137.29(c) of this chapter.

Sec. 91.817  Civil aircraft sonic boom.

(a) No person may operate a civil aircraft in the United States at a true

flight Mach number greater than 1 except in compliance with conditions and

limitations in an authorization to exceed Mach 1 issued to the operator under

appendix B of this part.

(b) In addition, no person may operate a civil aircraft for which the

maximum operating limit speed MM0 exceeds a Mach number of 1, to or from an

airport in the United States, unless--

(1) Information available to the flight crew includes flight limitations

that ensure that flights entering or leaving the United States will not cause

a sonic boom to reach the surface within the United States; and

(2) The operator complies with the flight limitations prescribed in

paragraph (b)(1) of this section or complies with conditions and limitations

in an authorization to exceed Mach 1 issued under appendix B of this part.

Sec. 91.819  Civil supersonic airplanes that do not comply with Part 36

(a) Applicability. This section applies to civil supersonic airplanes that

have not been shown to comply with the Stage 2 noise limits of Part 36 in

effect on October 13, 1977, using applicable trade-off provisions, and that

are operated in the United States, after July 31, 1978.

(b) Airport use. Except in an emergency, the following apply to each person

who operates a civil supersonic airplane to or from an airport in the United

States:

(1) Regardless of whether a type design change approval is applied for

under part 21 of this chapter, no person may land or take off an airplane

covered by this section for which the type design is changed, after July 31,

1978, in a manner constituting an "acoustical change" under Sec. 21.93 unless

the acoustical change requirements of part 36 are complied with.

(2) No flight may be scheduled, or otherwise planned, for takeoff or

landing after 10 p.m. and before 7 a.m. local time.

Sec. 91.821  Civil supersonic airplanes: Noise limits.

Except for Concorde airplanes having flight time before January 1, 1980, no

person may operate in the United States, a civil supersonic airplane that

does not comply with Stage 2 noise limits of part 36 in effect on October 13,

1977, using applicable trade-off provisions.

Secs. 91.823--91.849  [Reserved]

Sec. 91.851  Definitions.

For the purposes of Sec. 91.851 through 91.875 of this subpart:

Contiguous United States means the area encompassed by the 48 contiguous

United States and the District of Columbia.

Fleet means those civil subsonic turbojet airplanes with a maximum

certificated weight of more than 75,000 pounds that are listed on an

operator's operations specifications as eligible for operation in the

contiguous United States.

Import means a change in ownership of an airplane from a non-U.S. person to

a U.S. person when the airplane is brought into the United States for

operation.

Operations specifications means an enumeration of airplanes by type, model,

series, and serial number operated by the operator or foreign air carrier on

a given day, regardless of how or whether such airplanes are formally listed

or designated by the operator.

Owner means any person that has indicia of ownership sufficient to register

the airplane in the United States pursuant to part 47 of this chapter.

New entrant means an air carrier or foreign air carrier that, on or before

November 5, 1990, did not conduct operations under part 121, 125, 129, or 135

of this chapter using an airplane covered by this subpart to or from any

airport in the contiguous United States, but that initiates such operation

after that date.

Stage 2 noise levels mean the requirements for Stage 2 noise levels as

defined in part 36 of this chapter in effect on November 5, 1990.

Stage 3 noise levels mean the requirements for Stage 3 noise levels as

defined in part 36 of this chapter in effect on November 5, 1990.

Stage 2 airplane means a civil subsonic turbojet airplane with a maximum

certificated weight of 75,000 pounds or more that complies with Stage 2 noise

levels as defined in part 36 of this chapter.

Stage 3 airplane means a civil subsonic turbojet airplane with a maximum

certificated weight of 75,000 pounds or more that complies with Stage 3 noise

levels as defined in part 36 of this chapter.

Sec. 91.853  Final compliance: Civil subsonic airplanes.

Except as provided in Sec. 91.873, after December 31, 1999, no person shall

operate to or from any airport in the contiguous United States any airplane

subject to Sec. 91.801(c) of this subpart, unless that airplane has been

shown to comply with Stage 3 noise levels.

Sec. 91.855  Entry and nonaddition rule.

No person may operate any airplane subject to Sec. 91.801(c) of this

subpart to or from an airport in the contiguous United States unless one or

more of the following apply:

(a) The airplane complies with Stage 3 noise levels.

(b) The airplane complies with Stage 2 noise levels and was owned by a U.S.

person on and since November 5, 1990. Stage 2 airplanes that meet these

criteria and are leased to foreign airlines are also subject to the return

provisions of paragraph (e) of this section.

(c) The airplane complies with Stage 2 noise levels, is owned by a non-U.S.

person, and is the subject of a binding lease to a U.S. person effective

before and on September 25, 1991. Any such airplane may be operated for the

term of the lease in effect on that date, and any extensions thereof provided

for in that lease.

(d) The airplane complies with Stage 2 noise levels and is operated by a

foreign air carrier.

(e) The airplane complies with Stage 2 noise levels and is operated by a

foreign operator other than for the purpose of foreign air commerce.

(f) The airplane complies with Stage 2 noise levels and--

(1) On November 5, 1990, was owned by:

(i) A corporation, trust, or partnership organized under the laws of the

United States or any State (including individual States, territories,

possessions, and the District of Columbia);

(ii) An individual who is a citizen of the United States; or

(iii) An entity owned or controlled by a corporation, trust, partnership,

or individual described in paragraph (f)(1) (i) or (ii) of this section; and

(2) Enters into the United States not later than 6 months after the

expiration of a lease agreement (including any extensions thereof) between an

owner described in paragraph (f)(1) of this section and a foreign airline.

(g) The airplane complies with Stage 2 noise levels and was purchased by

the importer under a written contract executed before November 5, 1990.

(h) Any Stage 2 airplane described in this section is eligible for

operation in the contiguous United States only as provided under Sec. 91.865

or 91.867.

Sec. 91.857  Airplanes imported to points outside the contiguous United

States.

An operator of a Stage 2 airplane that was imported into a noncontiguous

State, territory, or possession of the United States on or after November 5,

1990, shall:

(a) Include in its operations specifications a statement that such airplane

may not be used to provide air transportation to or from any airport in the

contiguous United States.

(b) Obtain a special flight authorization to operate that airplane into the

contiguous United States for the purpose of maintenance. The special flight

authorization must include a statement indicating that this regulation is the

basis for the authorization.

Sec. 91.859  Modification to meet Stage 3 noise levels.

For an airplane subject to Sec. 91.801(c) of this subpart and otherwise

prohibited from operation to or from an airport in the contiguous United

States by Sec. 91.855, any person may apply for a special flight

authorization for that airplane to operate in the contiguous United States

for the purpose of obtaining modifications to meet Stage 3 noise levels.

Sec. 91.861  Base level.

(a) U.S. Operators. The base level of a U.S. operator is equal to the

number of owned or leased Stage 2 airplanes subject to Sec. 91.801(c) of this

subpart that were listed on that operator's operations specifications for

operations to or from airports in the contiguous United States on any one day

selected by the operator during the period January 1, 1990, through July 1,

1991, plus or minus adjustments made pursuant to paragraphs (a) (1) and (2).

(1) The base level of a U.S. operator shall be increased by a number equal

to the total of the following--

(i) The number of Stage 2 airplanes returned to service in the United

States pursuant to Sec. 91.855(f);

(ii) The number of Stage 2 airplanes purchased pursuant to Sec. 91.855(g);

and

(iii) Any U.S. operator base level acquired with a Stage 2 airplane

transferred from another person under Sec. 91.863.

(2) The base level of a U.S. operator shall be decreased by the amount of

U.S. operator base level transferred with the corresponding number of Stage 2

airplanes to another person under Sec. 91.863.

(b) Foreign air carriers. The base level of a foreign air carrier is equal

to the number of owned or leased Stage 2 airplanes that were listed on that

carrier's U.S. operations specifications on any one day during the period

January 1, 1990, through July 1, 1991, plus or minus any adjustments to the

base levels made pursuant to paragraphs (b) (1) and (2).

(1) The base level of a foreign air carrier shall be increased by the

amount of foreign air carrier base level acquired with a Stage 2 airplane

from another person under Sec. 91.863.

(2) The base level of a foreign air carrier shall be decreased by the

amount of foreign air carrier base level transferred with a Stage 2 airplane

to another person under Sec. 91.863.

(c) New entrants do not have a base level.

Sec. 91.863  Transfers of Stage 2 airplanes with base level.

(a) Stage 2 airplanes may be transferred with or without the corresponding

amount of base level. Base level may not be transferred without the

corresponding number of Stage 2 airplanes.

(b) No portion of a U.S. operator's base level established under Sec.

91.861(a) may be used for operations by a foreign air carrier. No portion of

a foreign air carrier's base level established under Sec. 91.861(b) may be

used for operations by a U.S. operator.

(c) Whenever a transfer of Stage 2 airplanes with base level occurs, the

transferring and acquiring parties shall, within 10 days, jointly submit

written notification of the transfer to the FAA, Office of Environment and

Energy. Such notification shall state:

(1) The names of the transferring and acquiring parties;

(2) The name, address, and telephone number of the individual responsible

for submitting the notification on behalf of the transferring and acquiring

parties;

(3) The total number of Stage 2 airplanes transferred, listed by airplane

type, model, series, and serial number;

(4) The corresponding amount of base level transferred and whether it is

U.S. operator or foreign air carrier base level; and

(5) The effective date of the transaction.

(d) If, taken as a whole, a transaction or series of transactions made

pursuant to this section does not produce an increase or decrease in the

number of Stage 2 airplanes for either the acquiring or transferring

operator, such transaction or series of transactions may not be used to

establish compliance with the requirements of Sec. 91.865.

Sec. 91.865  Phased compliance for operators with base level.

Except as provided in paragraph (a) of this section, each operator that

operates an airplane under part 91, 121, 125, 129, or 135 of this chapter,

regardless of the national registry of the airplane, shall comply with

paragraph (b) or (d) of this section at each interim compliance date with

regard to its subsonic airplane fleet covered by Sec. 91.80l(c) of this

subpart.

(a) This section does not apply to new entrants covered by Sec. 91.867 or

to foreign operators not engaged in foreign air commerce.

(b) Each operator that chooses to comply with this paragraph pursuant to

any interim compliance requirement shall reduce the number of Stage 2

airplanes it operates that are eligible for operation in the contiguous

United States to a maximum of:

(1) After December 31, 1994, 75 percent of the base level held by the

operator;

(2) After December 31, 1996, 50 percent of the base level held by the

operator;

(3) After December 31, 1998, 25 percent of the base level held by the

operator.

(c) Except as provided under Sec. 91.871, the number of Stage 2 airplanes

that must be reduced at each compliance date contained in paragraph (b) of

this section shall be determined by reference to the amount of base level

held by the operator on that compliance date, as calculated under Sec.

91.861.

(d) Each operator that chooses to comply with this paragraph pursuant to

any interim compliance requirement shall operate a fleet that consists of:

(1) After December 31, 1994, not less than 55 percent Stage 3 airplanes;

(2) After December 31, 1996, not less than 65 percent Stage 3 airplanes;

(3) After December 31, 1998, not less than 75 percent Stage 3 airplanes.

(e) Calculations resulting in fractions may be rounded to permit the

continued operation of the next whole number of Stage 2 airplanes.

Sec. 91.867  Phased compliance for new entrants.

(a) New entrant U.S. air carriers.

(1) A new entrant initiating operations under part 121, 125, or 135 of this

chapter on or before December 31, 1994, may initiate service without regard

to the percentage of its fleet composed of Stage 3 airplanes.

(2) After December 31, 1994, at least 25 percent of the fleet of a new

entrant must comply with Stage 3 noise levels.

(3) After December 31, 1996, at least 50 percent of the fleet of a new

entrant must comply with Stage 3 noise levels.

(4) After December 31, 1998, at least 75 percent of the fleet of a new

entrant must comply with Stage 3 noise levels.

(b) New entrant foreign air carriers.

(1) A new entrant foreign air carrier initiating part 129 operations on or

before December 31, 1994, may initiate service without regard to the

percentage of its fleet composed of Stage 3 airplanes.

(2) After December 31, 1994, at least 25 percent of the fleet on U.S.

operations specifications of a new entrant foreign air carrier must comply

with Stage 3 noise levels.

(3) After December 31, 1996, at least 50 percent of the fleet on U.S.

operations specifications of a new entrant foreign air carrier must comply

with Stage 3 noise levels.

(4) After December 31, 1998, at least 75 percent of the fleet on U.S.

operations specifications of a new entrant foreign air carrier must comply

with Stage 3 noise levels.

(c) Calculations resulting in fractions may be rounded to permit the

continued operation of the next whole number of Stage 2 airplanes.

Sec. 91.869  Carry-forward compliance.

(a) Any operator that exceeds the requirements of paragraph (b) of Sec.

91.865 of this part on or before December 31, 1994, or on or before December

31, 1996, may claim a credit that may be applied at a subsequent interim

compliance date.

(b) Any operator that eliminates or modifies more Stage 2 airplanes

pursuant to Sec. 91.865(b) than required as of December 31, 1994, or December

31, 1996, may count the number of additional Stage 2 airplanes reduced as a

credit toward--

(1) The number of Stage 2 airplanes it would otherwise be required to

reduce following a subsequent interim compliance date specified in Sec.

91.865(b); or

(2) The number of Stage 3 airplanes it would otherwise be required to

operate in its fleet following a subsequent interim compliance date to meet

the percentage requirements specified in Sec. 91.865(d).

Sec. 91.871  Waivers from interim compliance requirements.

(a) Any U.S. operator or foreign air carrier subject to the requirements of

Secs. 91.865 or 91.867 of this subpart may request a waiver from any

individual compliance requirement.

(b) Applications must be filed with the Secretary of Transportation at

least 120 days prior to the compliance date from which the waiver is

requested.

(c) Applicants must show that a grant of waiver would be in the public

interest, and must include in its application its plans and activities for

modifying its fleet, including evidence of good faith efforts to comply with

the requirements of Sec. 91.865 or Sec. 91.867. The application should

contain all information the applicant considers relevant, including, as

appropriate, the following:

(1) The applicant's balance sheet and cash flow positions;

(2) The composition of the applicant's current fleet; and

(3) The applicant's delivery position with respect to new airplanes or

noise-abatement equipment.

(d) Waivers will be granted only upon a showing by the applicant that

compliance with the requirements of Secs. 91.865 or 91.867 at a particular

interim compliance date is financially onerous, physically impossible, or

technologically infeasible, or that it would have an adverse effect on

competition or on service to small communities.

(e) The conditions of any waiver granted under this section shall be

determined by the circumstances presented in the application, but in no case

may the term extend beyond the next interim compliance date.

(f) A summary of any request for a waiver under this section will be

published in the Federal Register, and public comment will be invited. Unless

the Secretary finds that circumstances require otherwise, the public comment

period will be at least 14 days.

Sec. 91.873  Waivers from final compliance.

(a) A U.S. air carrier may apply for a waiver from the prohibition

contained in Sec. 91.853 for its remaining Stage 2 airplanes, provided that,

by July 1, 1999, at least 85 percent of the airplanes used by the carrier to

provide service to or from an airport in the contiguous United States will

comply with the Stage 3 noise levels.

(b) An application for the waiver described in paragraph (a) of this

section must be filed with the Secretary of Transportation no later than

January 1, 1999. Such application must include a plan with firm orders for

replacing or modifying all airplanes to comply with Stage 3 noise levels at

the earliest practicable time.

(c) To be eligible to apply for the waiver under this section, a new

entrant U.S. air carrier must initiate service no later than January 1, 1999,

and must comply fully with all provisions of this section.

(d) The Secretary may grant a waiver under this section if the Secretary

finds that granting such waiver is in the public interest. In making such a

finding, the Secretary shall include consideration of the effect of granting

such waiver on competition in the air carrier industry and the effect on

small community air service, and any other information submitted by the

applicant that the Secretary considers relevant.

(e) The term of any waiver granted under this section shall be determined

by the circumstances presented in the application, but in no case will the

waiver permit the operation of any Stage 2 airplane covered by this

subchapter in the contiguous United States after December 31, 2003.

(f) A summary of any request for a waiver under this section will be

published in the Federal Register, and public comment will be invited. Unless

the secretary finds that circumstances require otherwise, the public comment

period will be at least 14 days.

Sec. 91.875  Annual progress reports.

(a) Each operator subject to Sec. 91.865 or Sec. 91.867 of this chapter

shall submit an annual report to the FAA, Office of Environment and Energy,

on the progress it has made toward complying with the requirements of that

section. Such reports shall be submitted no later than 45 days after the end

of a calendar year. All progress reports must provide the information through

the end of the calendar year, be certified by the operator as true and

complete (under penalty of 18 U.S.C. 1001), and include the following

information:

(1) The name and address of the operator;

(2) The name, title, and telephone number of the person designated by the

operator to be responsible for ensuring the accuracy of the information in

the report;

(3) The operator's progress during the reporting period toward compliance

with the requirements of Sec. 91.853, Sec. 91.865 or Sec. 91.867. For

airplanes on U.S. operations specifications, each operator shall identify the

airplanes by type, model, series, and serial number.

(i) Each Stage 2 airplane added or removed from operation or U.S.

operations specifications (grouped separately by those airplanes acquired

with and without base level);

(ii) Each Stage 2 airplane modified to Stage 3 noise levels (identifying

the manufacturer and model of noise abatement retrofit equipment;

(iii) Each Stage 3 airplane on U.S. operations specifications as of the

last day of the reporting period; and

(iv) For each Stage 2 airplane transferred or acquired, the name and

address of the recipient or transferor; and, if base level was transferred,

the person to or from whom base level was transferred or acquired pursuant to

Section 91.863 along with the effective date of each base level transaction,

and the type of base level transferred or acquired.

(b) Each operator subject to Sec. 91.865 or Sec. 91.867 of this chapter

shall submit an initial progress report covering the period from January 1,

1990, through December 31, 199l, and provide:

(1) For each operator subject to Sec. 91.865:

(i) The date used to establish its base level pursuant to Sec. 91.861(a);

and

(ii) a list of those Stage 2 airplanes (by type, model, series and serial

number) in its base level, including adjustments made pursuant to Sec. 91.861

after the date its base level was established.

(2) For each U.S. operator:

(i) A plan to meet the compliance schedules in Sec. 91.865 or Sec. 91.867

and the final compliance date of Sec. 91.853, including the schedule for

delivery of replacement Stage 3 airplanes or the installation of noise

abatement retrofit equipment; and

(ii) A separate list (by type, model, series, and serial number) of those

airplanes included in the operator's base level, pursuant to Sec.

91.861(a)(1) (i) and (ii), under the categories "returned" or "purchased,"

along with the date each was added to its operations specifications.

(c) Each operator subject to Sec. 91.865 or Sec. 91.867 of this chapter

shall submit subsequent annual progress reports covering the calendar year

preceding the report and including any changes in the information provided in

paragraphs (a) and (b) of this section; including the use of any carry-

forward credits pursuant to Sec. 91.869.

(d) An operator may request, in any report, that specific planning data be

considered proprietary.

(e) If an operator's actions during any reporting period cause it to

achieve compliance with Sec. 91.853, the report should include a statement to

that effect. Further progress reports are not required unless there is any

change in the information reported pursuant to paragraph (a) of this section.

(f) For each U.S. operator subject to Sec. 91.865, progress reports

submitted for calendar years 1994, 1996, and 1998, shall also state how the

operator achieved compliance with the requirements of that section, i.e.--

(1) By reducing the number of Stage 2 airplanes in its fleet to no more

than the maximum permitted percentage of its base level under Sec. 91.865(b),

or

(2) By operating a fleet that consists of at least the minimum required

percentage of Stage 3 airplanes under Sec. 91.865(d).

Secs. 91.877--91.899  [Reserved]

Subpart J--Waivers

Sec. 91.901  [Reserved]

Sec. 91.903  Policy and procedures.

(a) The Administrator may issue a certificate of waiver authorizing the

operation of aircraft in deviation from any rule listed in this subpart if

the Administrator finds that the proposed operation can be safely conducted

under the terms of that certificate of waiver.

(b) An application for a certificate of waiver under this part is made on a

form and in a manner prescribed by the Administrator and may be submitted to

any FAA office.

(c) A certificate of waiver is effective as specified in that certificate

of waiver.

Sec. 91.905  List of rules subject to waivers.

Sec.

91.107  Use of safety belts.

91.111  Operating near other aircraft.

91.113  Right-of-way rules: Except water operations.

91.115  Right-of-way rules: Water operations.

91.117  Aircraft speed.

91.119  Minimum safe altitudes: General.

91.121  Altimeter settings.

91.123  Compliance with ATC clearances and instructions.

91.125  ATC light signals.

91.126 Operating on or in the vicinity of an airport in Class G airspace.

91.127 Operating on or in the vicinity of an airport in Class E airspace.

91.129 Operations in Class D airspace.

91.130 Operations in Class C airspace.

91.131 Operations in Class B airspace.

91.133  Restricted and prohibited areas.

91.135 Operations in Class A airspace.

91.137  Temporary flight restrictions.

91.141  Flight restrictions in the proximity of the Presidential and other

parties.

91.143  Flight limitation in the proximity of space flight operations.

91.153  VFR flight plan: Information required.

91.155  Basic VFR weather minimums

91.157  Special VFR weather minimums.

91.159  VFR cruising altitude or flight level.

91.169  IFR flight plan: Information required.

91.173  ATC clearance and flight plan required.

91.175  Takeoff and landing under IFR.

91.177  Minimum altitudes for IFR operations.

91.179  IFR cruising altitude or flight level.

91.181  Course to be flown.

91.183  IFR radio communications.

91.185  IFR operations: Two-way radio communications failure.

91.187  Operation under IFR in controlled airspace: Malfunction reports.

91.209  Aircraft lights.

91.303  Aerobatic flights.

91.305  Flight test areas.

91.311  Towing: Other than under Sec. 91.309.

91.313(e)  Restricted category civil aircraft: Operating limitations.

91.515  Flight altitude rules.

91.705  Operations within the North Atlantic Minimum Navigation Performance

Specifications Airspace.

91.707  Flights between Mexico or Canada and the United States.

91.713  Operation of civil aircraft of Cuban registry.

Secs. 91.907--91.999  [Reserved]

Appendix A to Part 91--Category II Operations: Manual, Instruments, Equipment

and Maintenance

1. Category II Manual--(a) Application for approval. An applicant for

approval of a Category II manual or an amendment to an approved Category II

manual must submit the proposed manual or amendment to the Flight Standards

District Office having jurisdiction of the area in which the applicant is

located. If the application requests an evaluation program, it must include

the following:

(1) The location of the aircraft and the place where the demonstrations are

to be conducted; and

(2) The date the demonstrations are to commence (at least 10 days after

filing the application).

(b) Contents. Each Category II manual must contain--

(1) the registration number, make, and model of the aircraft to which it

applies;

(2) A maintenance program as specified in section 4 of this appendix; and

(3) The procedures and instructions related to recognition of decision

height, use of runway visual range information, approach monitoring, the

decision region (the region between the middle marker and the decision

height), the maximum permissible deviations of the basic ILS indicator within

the decision region, a missed approach, use of airborne low approach

equipment, minimum altitude for the use of the autopilot, instrument and

equipment failure warning systems, instrument failure, and other procedures,

instructions, and limitations that may be found necessary by the

Administrator.

2. Required instruments and equipment. The instruments and equipment listed

in this section must be installed in each aircraft operated in a Category II

operation. This section does not require duplication of instruments and

equipment required by Sec. 91.33 or any other provisions of this chapter.

(a) Group I. (1) Two localizer and glide slope receiving systems. Each

system must provide a basic ILS display and each side of the instrument panel

must have a basic ILS display. However, a single localizer antenna and a

single glide slope antenna may be used.

(2) A communications system that does not affect the operation of at least

one of the ILS systems.

(3) A marker beacon receiver that provides distinctive aural and visual

indications of the outer and the middle marker.

(4) Two gyroscopic pitch and bank indicating systems.

(5) Two gyroscopic direction indicating systems.

(6) Two airspeed indicators.

(7) Two sensitive altimeters adjustable for barometric pressure, each

having a placarded correction for altimeter scale error and for the wheel

height of the aircraft. After June 26, 1979, two sensitive altimeters

adjustable for barometric pressure, having markings at 20-foot intervals and

each having a placarded correction for altimeter scale error and for the

wheel height of the aircraft.

(8) Two vertical speed indicators.

(9) A flight control guidance system that consists of either an automatic

approach coupler or a flight director system. A flight director system must

display computed information as steering command in relation to an ILS

localizer and, on the same instrument, either computed information as pitch

command in relation to an ILS glide slope or basic ILS glide slope

information. An automatic approach coupler must provide at least automatic

steering in relation to an ILS localizer. The flight control guidance system

may be operated from one of the receiving systems required by paragraph (1)

of this paragraph (a).

(10) For Category II operations with decision heights below 150 feet,

either a marker beacon receiver providing aural and visual indications of the

inner marker or a radio altimeter.

(b) Group II. (1) Warning systems for immediate detection by the pilot of

system faults in items (1), (4), (5), and (9) of Group I and, if installed,

for use in Category II operations, the radio altimeter and auto throttle

system.

(2) Dual controls.

(3) An externally vented static pressure system with an alternate static

pressure source.

(4) A windshield wiper or equivalent means of providing adequate cockpit

visibility for a safe visual transition by either pilot to touch down and

roll out.

(5) A heat source for each airspeed system pitot tube installed or an

equivalent means of preventing malfunctioning due to icing of the pitot

system.

3. Instruments and equipment approval--(a) General. The instruments and

equipment required by section 2 of this appendix must be approved as provided

in this section before being used in Category II operations. Before

presenting an aircraft for approval of the instruments and equipment, it must

be shown that, since the beginning of the 12th calendar month before the date

of submission--

(1) The ILS localizer and glide slope equipment were bench checked

according to the manufacturer's instructions and found to meet those

standards specified in RTCA Paper 23-63/DO-117, dated March 14, 1963.

"Standard Adjustment Criteria for Airborne Localizer and Glide Slope

Receivers," which may be obtained from the RTCA Secretariat, 2000 K Street

NW., Washington, D.C. 20006, at cost of 50 cents per copy, payment in cash or

by check or money order payable to the Radio Technical Commission for

Aeronautics;

(2) The altimeters and the static pressure systems were tested and

inspected in accordance with Appendix E to Part 43 of this chapter; and

(3) All other instruments and items of equipment specified in section 2(a)

of this appendix that are listed in the proposed maintenance program were

bench checked and found to meet the manufacturer's specifications.

(b) Flight control guidance system. All components of the flight control

guidance system must be approved as installed by the evaluation program

specified in paragraph (e) of this section if they have not been approved for

Category II operations under applicable type or supplemental type

certification procedures. In addition, subsequent changes to make, model or

design of these components must be approved under this paragraph. Related

systems or devices such as the auto throttle and computed missed approach

guidance system must be approved in the same manner if they are to be used

for Category II operations.

(c) Radio altimeter. A radio altimeter must meet the performance criteria

of this paragraph for original approval and after each subsequent alteration.

(1) It must display to the flight crew clearly and positively the wheel

height of the main landing gear above the terrain.

(2) It must display wheel height above the terrain to an accuracy of plus

or minus 5 feet or 5 percent, whichever is greater, under the following

conditions:

(i) Pitch angles of zero to plus or minus 5 degrees about the mean approach

altitude.

(ii) Roll angles of zero to 20 degrees in either direction.

(iii) Forward velocities from minimum approach speed up to 200 knots.

(iv) Sink rates from zero to 15 feet per second at altitudes from 100 to

200 feet.

(3) Over level ground, it must track the actual altitude of the aircraft

without significant lag or oscillation.

(4) With the aircraft at an altitude of 200 feet or less, any abrupt change

in terrain representing no more than 10 percent of the aircraft's altitude

must not cause the altimeter to unlock, and indicator response to such

changes must not exceed 0.1 second and in addition, if the system unlocks for

greater changes, it must reacquire the signal in less than 1 second.

(5) Systems that contain a push-to-test feature must test the entire system

(with or without an antenna) at a simulated altitude of less than 500 feet.

(6) The system must provide to the flight crew a positive failure warning

display any time there is a loss of power or an absence of ground return

signals within the designed range of operating altitudes.

(d) Other instruments and equipment. All other instruments and items of

equipment required by section 2 of this appendix must be capable of

performing as necessary for Category II operations. Approval is also required

after each subsequent alteration to these instruments and items of equipment.

(e) Evaluation program--(1) Application. Approval by evaluation is

requested as a part of the application for approval of the Category II

manual.

(2) Demonstrations. Unless otherwise authorized by the Administrator, the

evaluation program for each aircraft requires the demonstrations specified in

this paragraph. At least 50 ILS approaches must be flown with at least five

approaches on each of three different ILS facilities and no more than one-

half of the total approaches on any one ILS facility. All approaches shall be

flown under simulated instrument conditions to a 100-foot decision height and

90 percent of the total approaches made must be successful. A successful

approach is one in which--

(i) At the 100-foot decision height, the indicated airspeed and heading are

satisfactory for a normal flare and landing (speed must be plus or minus 5

knots of programed airspeed but may not be less than computed threshold

speed, if auto throttles are used);

(ii) The aircraft, at the 100-foot decision height, is positioned so that

the cockpit is within, and tracking so as to remain within, the lateral

confines of the runway extended;

(iii) Deviation from glide slope after leaving the outer marker does not

exceed 50 percent of full scale deflection as displayed on the ILS indicator;

(iv) No unusual roughness or excessive attitude changes occur after leaving

the middle marker; and

(v) In the case of an aircraft equipped with an approach coupler, the

aircraft is sufficiently in trim when the approach coupler is disconnected at

the decision height to allow for the continuation of a normal approach and

landing.

(3) Records. During the evaluation program the following information must

be maintained by the applicant for the aircraft with respect to each approach

and made available to the Administrator upon request:

(i) Each deficiency in airborne instruments and equipment that prevented

the initiation of an approach.

(ii) The reasons for discontinuing an approach including the altitude above

the runway at which it was discontinued.

(iii) Speed control at the 100-foot decision height if auto throttles are

used.

(iv) Trim condition of the aircraft upon disconnecting the auto coupler

with respect to continuation to flare and landing.

(v) Position of the aircraft at the middle marker and at the decision

height indicated both on a diagram of the basic ILS display and a diagram of

the runway extended to the middle marker. Estimated touch down point must be

indicated on the runway diagram.

(vi) Compatibility of flight director with the auto coupler, if applicable.

(vii) Quality of overall system performance.

(4) Evaluation. A final evaluation of the flight control guidance system is

made upon successful completion of the demonstrations. If no hazardous

tendencies have been displayed or are otherwise known to exist, the system is

approved as installed.

4. Maintenance program. (a) Each maintenance program must contain the

following:

(1) A list of each instrument and item of equipment specified in section 2

of this ppendix that is installed in the aircraft and approved for Category

II operations, including the make and model of those specified in section

2(a).

(2) A schedule that provides for the performance of inspections under

subparagraph (5) of this paragraph within 3 calendar months after the date of

the previous inspection. The inspection must be performed by a person

authorized by Part 43 of this chapter, except that each alternate inspection

may be replaced by a functional flight check. This functional flight check

must be performed by a pilot holding a Category II pilot authorization for

the type aircraft checked.

(3) A schedule that provides for the performance of bench checks for each

listed instrument and item of equipment that is specified in section 2(a)

within 12 calendar months after the date of the previous bench check.

(4) A schedule that provides for the performance of a test and inspection

of each static pressure system in accordance with Appendix E to Part 43 of

this chapter within 12 calendar months after the date of the previous test

and inspection.

(5) The procedures for the performance of the periodic inspections and

functional flight checks to determine the ability of each listed instrument

and item of equipment specified in section 2(a) of this Appendix to perform

as approved for Category II operations including a procedure for recording

functional flight checks.

(6) A procedure for assuring that the pilot is informed of all defects in

listed instruments and items of equipment.

(7) A procedure for assuring that the condition of each listed instrument

and item of equipment upon which maintenance is performed is at least equal

to its Category II approval condition before it is returned to service for

Category II operations.

(8) A procedure for an entry in the maintenance records required by Sec.

43.9 of this chapter that shows the date, airport, and reasons for each

discontinued Category II operation because of a malfunction of a listed

instrument or item of equipment.

(b) Bench check. A bench check required by this section must comply with

this paragraph.

(1) It must be performed by a certificated repair station holding one of

the following ratings as appropriate to the equipment checked:

(i) An instrument rating.

(ii) A radio rating.

(iii) A rating issued under Subpart D of Part 145.

(2) It must consist of removal of an instrument or item of equipment and

performance of the following:

(i) A visual inspection for cleanliness, impending failure, and the need

for lubrication, repair, or replacement of parts;

(ii) Correction of items found by that visual inspection; and

(iii) Calibration to at least the manufacturer's specifications unless

otherwise specified in the approved Category II manual for the aircraft in

which the instrument or item of equipment is installed.

(c) Extensions. After the completion of one maintenance cycle of 12

calendar months a request to extend the period for checks, tests, and

inspections is approved if it is shown that the performance of particular

equipment justifies the requested extension.

[Amdt. 91-39, 32 FR 6906, May 5, 1967; 32 FR 8127, June 7, 1967; Amdt. 91-63,

33 FR 17291, Nov. 22, 1968; Amdt. 91-152, 43 FR 22641, May 25, 1978; Amdt.

91-196, 51 FR 40707, Nov. 7, 1986]

Effective Date Note: At 54 FR 34291, August 18, 1989, Appendix A was

replaced effective August 18, 1990. For the convenience of the user, the new

text is set forth as follows:

Appendix A to Part 91--Category II Operations: Manual, Instruments,

Equipment, and Maintenance

1. Category II Manual

(a) Application for approval. An applicant for approval of a Category II

manual or an amendment to an approved Category II manual must submit the

proposed manual or amendment to the Flight Standards District Office having

jurisdiction of the area in which the applicant is located. If the

application requests an evaluation program, it must include the following:

(1) The location of the aircraft and the place where the demonstrations are

to be conducted; and

(2) The date the demonstrations are to commence (at least 10 days after

filing the application).

(b) Contents. Each Category II manual must contain:

(1) The registration number, make, and model of the aircraft to which it

applies;

(2) A maintenance program as specified in section 4 of this appendix; and

(3) The procedures and instructions related to recognition of decision

height, use of runway visual range information, approach monitoring, the

decision region (the region between the middle marker and the decision

height), the maximum permissible deviations of the basic ILS indicator within

the decision region, a missed approach, use of airborne low approach

equipment, minimum altitude for the use of the autopilot, instrument and

equipment failure warning systems, instrument failure, and other procedures,

instructions, and limitations that may be found necessary by the

Administrator.

2. Required Instruments and Equipment

The instruments and equipment listed in this section must be installed in

each aircraft operated in a Category II operation. This section does not

require duplication of instruments and equipment required by Sec. 91.205 or

any other provisions of this chapter.

(a) Group I. (1) Two localizer and glide slope receiving systems. Each

system must provide a basic ILS display and each side of the instrument panel

must have a basic ILS display. However, a single localizer antenna and a

single glide slope antenna may be used.

(2) A communications system that does not affect the operation of at least

one of the ILS systems.

(3) A marker beacon receiver that provides distinctive aural and visual

indications of the outer and the middle markers.

(4) Two gyroscopic pitch and bank indicating systems.

(5) Two gyroscopic direction indicating systems.

(6) Two airspeed indicators.

(7) Two sensitive altimeters adjustable for barometric pressure, each

having a placarded correction for altimeter scale error and for the wheel

height of the aircraft. After June 26, 1979, two sensitive altimeters

adjustable for barometric pressure, having markings at 20-foot intervals and

each having a placarded correction for altimeter scale error and for the

wheel height of the aircraft.

(8) Two vertical speed indicators.

(9) A flight control guidance system that consists of either an automatic

approach coupler or a flight director system. A flight director system must

display computed information as steering command in relation to an ILS

localizer and, on the same instrument, either computed information as pitch

command in relation to an ILS glide slope or basic ILS glide slope

information. An automatic approach coupler must provide at least automatic

steering in relation to an ILS localizer. The flight control guidance system

may be operated from one of the receiving systems required by subparagraph

(1) of this paragraph.

(10) For Category II operations with decision heights below 150 feet either

a marker beacon receiver providing aural and visual indications of the inner

marker or a radio altimeter.

(b) Group II. (1) Warning systems for immediate detection by the pilot of

system faults in items (1), (4), (5), and (9) of Group I and, if installed

for use in Category III operations, the radio altimeter and autothrottle

system.

(2) Dual controls.

(3) An externally vented static pressure system with an alternate static

pressure source.

(4) A windshield wiper or equivalent means of providing adequate cockpit

visibility for a safe visual transition by either pilot to touchdown and

rollout.

(5) A heat source for each airspeed system pitot tube installed or an

equivalent means of preventing malfunctioning due to icing of the pitot

system.

3. Instruments and Equipment Approval

(a) General. The instruments and equipment required by section 2 of this

appendix must be approved as provided in this section before being used in

Category II operations. Before presenting an aircraft for approval of the

instruments and equipment, it must be shown that since the beginning of the

12th calendar month before the date of submission--

(1) The ILS localizer and glide slope equipment were bench checked

according to the manufacturer's instructions and found to meet those

standards specified in RTCA Paper 23-63/DO-117 dated March 14, 1963,

"Standard Adjustment Criteria for Airborne Localizer and Glide Slope

Receivers," which may be obtained from the RTCA Secretariat, 1425 K St., NW.,

Washington, DC 20005.

(2) The altimeters and the static pressure systems were tested and

inspected in accordance with Appendix E to Part 43 of this chapter; and

(3) All other instruments and items of equipment specified in section 2(a)

of this appendix that are listed in the proposed maintenance program were

bench checked and found to meet the manufacturer's specifications.

(b) Flight control guidance system. All components of the flight control

guidance system must be approved as installed by the evaluation program

specified in paragraph (e) of this section if they have not been approved for

Category III operations under applicable type or supplemental type

certification procedures. In addition, subsequent changes to make, model, or

design of the components must be approved under this paragraph. Related

systems or devices, such as the autothrottle and computed missed approach

guidance system, must be approved in the same manner if they are to be used

for Category II operations.

(c) Radio altimeter. A radio altimeter must meet the performance criteria

of this paragraph for original approval and after each subsequent alteration.

(1) It must display to the flight crew clearly and positively the wheel

height of the main landing gear above the terrain.

(2) It must display wheel height above the terrain to an accuracy of plus

or minus 5 feet or 5 percent, whichever is greater, under the following

conditions:

(i) Pitch angles of zero to plus or minus 5 degrees about the mean approach

attitude.

(ii) Roll angles of zero to 20 degrees in either direction.

(iii) Forward velocities from minimum approach speed up to 200 knots.

(iv) Sink rates from zero to 15 feet per second at altitudes from 100 to

200 feet.

(3) Over level ground, it must track the actual altitude of the aircraft

without significant lag or oscillation.

(4) With the aircraft at an altitude of 200 feet or less, any abrupt change

in terrain representing no more than 10 percent of the aircraft's altitude

must not cause the altimeter to unlock, and indicator response to such

changes must not exceed 0.1 seconds and, in addition, if the system unlocks

for greater changes, it must reacquire the signal in less than 1 second.

(5) Systems that contain a push-to-test feature must test the entire system

(with or without an antenna) at a simulated altitude of less than 500 feet.

(6) The system must provide to the flight crew a positive failure warning

display any time there is a loss of power or an absence of ground return

signals within the designed range of operating altitudes.

(d) Other instruments and equipment. All other instruments and items of

equipment required by Sec. 2 of this appendix must be capable of performing

as necessary for Category II operations. Approval is also required after each

subsequent alteration to these instruments and items of equipment.

(e) Evaluation program--(1) Application. Approval by evaluation is

requested as a part of the application for approval of the Category II

manual.

(2) Demonstrations. Unless otherwise authorized by the Administrator, the

evaluation program for each aircraft requires the demonstrations specified in

this paragraph. At least 50 ILS approaches must be flown with at least five

approaches on each of three different ILS facilities and no more than one

half of the total approaches on any one ILS facility. All approaches shall be

flown under simulated instrument conditions to a 100-foot decision height and

90 percent of the total approaches made must be successful. A successful

approach is one in which--

(i) At the 100-foot decision height, the indicated airspeed and heading are

satisfactory for a normal flare and landing (speed must be plus or minus 5

knots of programmed airspeed, but may not be less than computed threshold

speed if autothrottles are used);

(ii) The aircraft at the 100-foot decision height, is positioned so that

the cockpit is within, and tracking so as to remain within, the lateral

confines of the runway extended;

(iii) Deviation from glide slope after leaving the outer marker does not

exceed 50 percent of full-scale deflection as displayed on the ILS indicator;

(iv) No unusual roughness or excessive attitude changes occur after leaving

the middle marker; and

(v) In the case of an aircraft equipped with an approach coupler, the

aircraft is sufficiently in trim when the approach coupler is disconnected at

the decision height to allow for the continuation of a normal approach and

landing.

(3) Records. During the evaluation program the following information must

be maintained by the applicant for the aircraft with respect to each approach

and made available to the Adninistrator upon request:

(i) Each deficiency in airborne instruments and equipment that prevented

the initiation of an approach.

(ii) The reasons for discontinuing an approach, including the altitude

above the runway at which it was discontinued.

(iii) Speed control at the 100-foot decision height if auto throttles are

used.

(iv) Trim condition of the aircraft upon disconnecting the auto coupler

with respect to continuation to flare and landing.

(v) Position of the aircraft at the middle marker and at the decision

height indicated both on a diagram of the basic ILS display and a diagram of

the runway extended to the middle marker. Estimated touchdown point must be

indicated on the runway diagram.

(vi) Compatibility of flight director with the auto coupler, if applicable.

(vii) Quality of overall system performance.

(4) Evaluation. A final evaluation of the flight control guidance system is

made upon successful completion of the demonstrations. If no hazardous

tendencies have been displayed or are otherwise known to exist, the system is

approved as installed.

4. Maintenance program

(a) Each maintenance program must contain the following:

(1) A list of each instrument and item of equipment specified in Sec. 2 of

this appendix that is installed in the aircraft and approved for Category II

operations, including the make and model of those specified in Sec. 2(a).

(2) A schedule that provides for the performance of inspections under

subparagraph (5) of this paragraph within 3 calendar months after the date of

the previous inspection. The inspection must be performed by a person

authorized by part 43 of this chapter, except that each alternate inspection

may be replaced by a functional flight check. This functional flight check

must be performed by a pilot holding a Category II pilot authorization for

the type aircraft checked.

(3) A schedule that provides for the performance of bench checks for each

listed instrument and item of equipment that is specified in section 2(a)

within 12 calendar months after the date of the previous bench check.

(4) A schedule that provides for the performance of a test and inspection

of each static pressure system in accordance with appendix E to part 43 of

this chapter within 12 calendar months after the date of the previous test

and inspection.

(5) The procedures for the performance of the periodic inspections and

functional flight checks to determine the ability of each listed instrument

and item of equipment specified in section 2(a) of this appendix to perform

as approved for Category II operations including a procedure for recording

functional flight checks.

(6) A procedure for assuring that the pilot is informed of all defects in

listed instruments and items of equipment.

(7) A procedure for assuring that the condition of each listed instrument

and item of equipment upon which maintenance is performed is at least equal

to its Category II approval condition before it is returned to service for

Category II operations.

(8) A procedure for an entry in the maintenance records required by Sec.

43.9 of this chapter that shows the date, airport, and reasons for each

discontinued Category II operation because of a malfunction of a listed

instrument or item of equipment.

(b) Bench check. A bench check required by this section must comply with

this paragraph.

(1) It must be performed by a certificated repair station holding one of

the following ratings as appropriate to the equipment checked:

(i) An instrument rating.

(ii) A radio rating.

(iii) A rating issued under subpart D of part 145 of this chapter.

(2) It must consist of removal of an instrument or item of equipment and

performance of the following:

(i) A visual inspection for cleanliness, impending failure, and the need

for lubrication, repair, or replacement of parts;

(ii) Correction of items found by that visual inspection; and

(iii) Calibration to at least the manufacturer's specifications unless

otherwise specified in the approved Category II manual for the aircraft in

which the instrument or item of equipment is installed.

(c) Extensions. After the completion of one maintenance cycle of 12

calendar months, a request to extend the period for checks, tests, and

inspections is approved if it is shown that the performance of particular

equipment justifies the requested extension.

Appendix B to Part 91--Authorizations to Exceed Mach 1 (Sec. 91.55)

Section 1. Application. (a) An applicant for an authorization to exceed

mach 1 must apply in a form and manner prescribed by the Administrator and

must comply with this appendix.

(b) In addition, each application for an authorization to exceed mach 1

covered by section 2(a) of this appendix must contain all information,

requested by the Administrator, that he deems necessary to assist him in

determining whether the designation of a particular test area, or issuance of

a particular authorization, is a "major Federal action significantly

affecting the quality of the human environment" within the meaning of the

National Environmental Policy Act of 1969 (42 U.S.C. 4321 et seq.), and to

assist him in complying with that Act, and with related Executive orders,

guidelines, and orders, prior to such action.

(c) In addition, each application for an authorization to exceed mach 1

covered by section 2(a) of this appendix must contain--

(1) Information showing that operation at a speed greater than mach 1 is

necessary to accomplish one or more of the purposes specified in section 2(a)

of this appendix, including a showing that the purpose of the test cannot be

safely or properly accomplished by overocean testing;

(2) A description of the test area proposed by the applicant, including an

environmental analysis of that area meeting the requirements of paragraph (b)

of this section; and

(3) Conditions and limitations that will insure that no measurable sonic

boom overpressure will reach the surface outside of the designated test area.

(d) An application is denied if the Administrator finds that such action is

necessary to protect or enhance the environment.

Sec. 2. Issuance. (a) For a flight in a designated test area, an

authorization to exceed mach 1 may be issued when the Administrator has taken

the environmental protective actions specified in section 1(b) of this

appendix, and the applicant shows one or more of the following:

(1) The flight is necessary to show compliance with airworthiness

requirements.

(2) The flight is necessary to determine the sonic boom characteristics of

the airplane, or is necessary to establish means of reducing or eliminating

the effects of sonic boom.

(3) The flight is necessary to demonstrate the conditions and limitations

under which speeds greater than a true flight mach number of 1 will not cause

a measurable sonic boom overpressure to reach the surface.

(b) For a flight outside of a designated test area, an authorization to

exceed mach 1 may be issued if the applicant shows conservatively under

paragraph (a)(3) of this section that--

(1) The flight will not cause a measurable sonic boom overpressure to reach

the surface when the aircraft is operated under conditions and limitations

demonstrated under paragraph (a)(3) of this section; and

(2) Those conditions and limitations represent all foreseeable operating

conditions.

Sec. 3. Duration. (a) An authorization to exceed mach 1 is effective until

it expires or is surrendered, or until it is suspended or terminated by the

Administrator. Such an authorization may be amended or suspended by the

Administrator at any time if he finds that such action is necessary to

protect the environment. Within 30 days of notification of amendment, the

holder of the authorization must request reconsideration or the amendment

becomes final. Within 30 days of notification of suspension, the holder of

the authorization must request reconsideration or the authorization is

automatically terminated. If reconsideration is requested within the 30-day

period, the amendment or suspension continues until the holder shows why, in

his opinion, the authorization should not be amended or terminated. Upon such

showing, the Administrator may terminate or amend the authorization if he

finds that such action is necessary to protect the environment, or he may

reinstate the authorization without amendment if he finds that termination or

amendment is not necessary to protect the environment.

(b) Findings and actions by the Administrator under this section do not

affect any certificate issued under title VI of the Federal Aviation Act of

1958.

(Sec. 307(c), 313(a), 611, Federal Aviation Act of 1958, 49 U.S.C. 1348(c),

1354(a), 1431; sec. 2(b)(2), 6(c), Department of Transportation Act, 49

U.S.C. 1651(b)(2), 1655(c), Title I of the National Environmental Policy Act

of 1969, 42 U.S.C. 4321 et seq., E. O. 11514, Protection and Enhancement of

Environmental Quality, Mar. 5, 1970)

[Doc. No. 10261, Amdt. 91-112, 38 FR 8055, Mar. 28, 1973]

Effective Date Note: At 54 FR 34291, August 18, 1989, Appendix B was

replaced effective August 18, 1990. For the convenience of the user, the new

text is set forth as follows:

Appendix B to Part 91--Authorizations to Exceed Mach 1 (section 91.817)

Section 1. Application

(a) An applicant for an authorization to exceed Mach 1 must apply in a form

and manner prescribed by the Administrator and must comply with this

appendix.

(b) In addition, each application for an authorization to exceed Mach 1

covered by section 2(a) of this appendix must contain all information

requested by the Administrator necessary to assist him in determining whether

the designation of a particular test area or issuance of a particular

authorization is a "major Federal action significantly affecting the quality

of the human environment" within the meaning of the National Environmental

Policy Act of 1969 (42 U.S.C. 4321 et seq.), and to assist him in complying

with that act and with related Executive Orders, guidelines, and orders prior

to such action.

(c) In addition, each application for an authorization to exceed Mach 1

covered by section 2(a) of this appendix must contain--

(1) Information showing that operation at a speed greater than Mach 1 is

necessary to accomplish one or more of the purposes specified in section 2(a)

of this appendix, including a showing that the purpose of the test cannot be

safely or properly accomplished by overocean testing;

(2) A description of the test area proposed by the applicant, including an

environmental analysis of that area meeting the requirements of paragraph (b)

of this section; and

(3) Conditions and limitations that will ensure that no measurable sonic

boom overpressure will reach the surface outside of the designated test area.

(d) An application is denied if the Administrator finds that such action is

necessary to protect or enhance the environment.

Section 2. Issuance

(a) For a flight in a designated test area, an authorization to exceed Mach

1 may be issued when the Administrator has taken the environmental protective

actions specified in section 1(b) of this appendix and the applicant shows

one or more of the following:

(1) The flight is necessary to show compliance with airworthiness

requirements.

(2) The flight is necessary to determine the sonic boom characteristics of

the airplane or to establish means of reducing or eliminating the effects of

sonic boom.

(3) The flight is necessary to demonstrate the conditions and limitations

under which speeds greater than a true flight Mach number of 1 will not cause

a measurable sonic boom overpressure to reach the surface.

(b) For a flight outside of a designated test area, an authorization to

exceed Mach 1 may be issued if the applicant shows conservatively under

paragraph (a)(3) of this section that--

(1) The flight will not cause a measurable sonic boom overpressure to reach

the surface when the aircraft is operated under conditions and limitations

demonstrated under paragraph (a)(3) of this section; and

(2) Those conditions and limitations represent all foreseeable operating

conditions.

Section 3. Duration

(a) An authorization to exceed Mach 1 is effective until it expires or is

surrendered, or until it is suspended or terminated by the Administrator.

Such an authorization may be amended or suspended by the Administrator at any

time if the Administrator finds that such action is necessary to protect the

environment. Within 30 days of notification of amendnent, the holder of the

authorization must request reconsideration or the amendnent becomes final.

Within 30 days of notification of suspension, the holder of the authorization

must request reconsideration or the authorization is automatically

terminated. If reconsideration is requested within the 30-day period, the

amendment or suspension continues until the holder shows why the

authorization should not be amended or terminated. Upon such showing, the

Administrator may terminate or amend the authorization if the Administrator

finds that such action is necessary to protect the environment, or he may

reinstate the authorization without amendment if he finds that termination or

amendnent is not necessary to protect the environment.

(b) Findings and actions by the Administrator under this section do not

affect any certificate issued under Title VI of the Federal Aviation Act of

1958.

Appendix C to Part 91--Operations in the North Atlantic (NAT) Minimum

Navigation Performance Specifications (MNPS) Airspace

Section 1. NAT MNPS airspace is that volume of airspace between FL 275 and

FL 400 extending between latitude 27 degrees north and the North Pole,

bounded in the east by the eastern boundaries of control areas Santa Maria

Oceanic, Shanwick Oceanic, and Reykjavik Oceanic and in the west by the

western boundary of Reykjavik Oceanic Control Area, the western boundary of

Gander Oceanic Control Area, and the western boundary of New York Oceanic

Control Area, excluding the areas west of 60 degrees west and south of 38

degrees 30 minutes north.

Section 2. The navigation performance capability required for aircraft to

be operated in the airspace defined in Sec. 1 of this appendix is as follows:

(a) The standard deviation of lateral track errors shall be less than 6.3

NM (11.7 Km). Standard deviation is a statistical measure of data about a

mean value. The mean is zero nautical miles. The overall form of data is such

that the plus and minus one standard deviation about the mean encompasses

approximately 68 percent of the data and plus or minus two deviations

encompasses approximately 95 percent.

(b) The proportion of the total flight time spent by aircraft 30 NM (55.6

Km) or more off the cleared track shall be less than 5.3 x 10-4 (less than

one hour in 1,887 flight hours).

(c) The proportion of the total flight time spent by aircraft between 50 NM

and 70 NM (92.6 Km and 129.6 Km) off the cleared track shall be less than 13

x 10-5 (less than one hour in 7,693 flight hours).

Section 3. Air traffic control (ATC) may authorize an aircraft operator to

deviate from the requirements of Sec. 91.20 for a specific flight if, at the

time of flight plan filing for that flight, ATC determines that the aircraft

may be provided appropriate separation and that the flight will not interfere

with, or impose a burden upon, the operations of other aircraft which meet

the requirements of Sec. 91.20.

(Secs. 307, 313(a), 601, 604, and 1102, Federal Aviation Act of 1958, as

amended (49 U.S.C. 1348, 1354(a), 1421, 1424, and 1502); sec. 6(c),

Department of Transportation Act (49 U.S.C. 1655(c)))

[Amdt. No. 91-144, 42 FR 64881, Dec. 29, 1977, as amended by Amdt. 91-193, 50

FR 51193, Dec. 13, 1985]

Effective Date Note: At 54 FR 34291, August 18, 1989, Appendix C was

replaced effective August 18, 1990. For the convenience of the user, the new

text is set forth as follows:

Appendix C to Part 91--Operations in the North Atlantic (NAT) Minimum

Navigation Performance Specifications (MNPS) Airspace

Section 1

NAT MNPS airspace is that volume of airspace between FL 275 and FL 400

extending between latitude 27 degrees north and the North Pole, bounded in

the east by the eastern boundaries of control areas Santa Maria Oceanic,

Shanwick Oceanic, and Reykjavik Oceanic and in the west by the western

boundary of Reykjavik Oceanic Control Area, the western boundary of Gander

Oceanic Control Area, and the western boundary of New York Oceanic Control

Area, excluding the areas west of 60 degrees west and south of 38 degrees 30

minutes north.

Section 2

The navigation performance capability required for aircraft to be operated

in the airspace defined in section 1 of this appendix is as follows:

(a) The standard deviation of lateral track errors shall be less than 6.3

NM (11.7 Km). Standard deviation is a statistical measure of data about a

mean value. The mean is zero nautical miles. The overall form of data is such

that the plus and minus 1 standard deviation about the mean encompasses

approximately 68 percent of the data and plus or minus 2 deviations

encompasses approximately 95 percent.

(b) The proportion of the total flight time spent by aircraft 30 NM (55.6

Km) or more off the cleared track shall be less than 5.3 x 10-4 (less than 1

hour in 1,887 flight hours).

(c) The proportion of the total flight time spent by aircraft between 50 NM

and 70 NM (92.6 Km and 129.6 Km) off the cleared track shall be less than 13

x 10-5 (less than 1 hour in 7,693 flight hours.)

Section 3

Air traffic control (ATC) may authorize an aircraft operator to deviate

from the requirements of Sec. 91.705 for a specific flight if, at the time of

flight plan filing for that flight, ATC determines that the aircraft may be

provided appropriate separation and that the flight will not interfere with,

or impose a burden upon, the operations of other aircraft which meet the

requirements of Sec. 91.705.

Appendix D--Airports/Locations: Special Operating Restrictions

Section 1. Locations at which the requirements of Sec. 91.215(b)(2) apply.

The requirements of Sec. 91.215(b)(2) apply below 10,000 feet above the

surface within a 30-nautical-mile radius of each location in the following

list:

Atlanta, GA (The William B. Hartsfield Atlanta International Airport)

Baltimore, MD (Baltimore Washington International Airport)

Boston, MA (General Edward Lawrence Logan International Airport)

Chantilly, VA (Washington Dulles International Airport)

Charlotte, NC (Charlotte/Douglas International Airport)

Chicago, IL Chicago-O'Hare International Airport)

Cleveland, OH (Cleveland-Hopkins International Airport)

Dallas, TX (Dallas/Fort Worth Regional Airport)

Denver, CO (Denver International Airport)

Detroit, MI (Metropolitan Wayne County Airport)

Honolulu, HI (Honolulu International Airport)

Houston, TX (Houston Intercontinental Airport)

Kansas City, KS (Mid-Continent International Airport)

Las Vegas, NV (McCarran International Airport)

Los Angeles, CA (Los Angeles International Airport)

Memphis, TN (Memphis International Airport)

Miami, FL (Miami International Airport)

Minneapolis, MN (Minneapolis-St. Paul International Airport)

Newark, NJ (Newark International Airport)

New Orleans, LA (New Orleans International Airport-Moisant Field)

New York, NY (John F. Kennedy International Airport)

New York, NY (LaGuardia Airport)

Orlando, FL (Orlando International Airport)

Philadelphia, PA (Philadelphia International Airport)

Phoenix, AZ (Phoenix Sky Harbor International Airport)

Pittsburgh, PA (Greater Pittsburgh International Airport)

St. Louis, MO (Lambert-St. Louis International Airport)

Salt Lake City, UT (Salt Lake City International Airport)

San Diego, CA (San Diego International Airport)

San Francisco, CA (San Francisco International Airport)

Seattle, WA (Seattle-Tacoma International Airport)

Tampa, FL (Tampa International Airport)

Washington, DC (Washington National Airport)

Section 2. Airports at which the requirements of Sec. 91.215(b)(5)(ii)

apply.

The requirements of Sec. 91.215(b)(5)(ii) apply to operations in the

vicinity of each of the following airports:

Billings, MT (Logan International Airport)

Section 3. Locations at which fixed-wing Special VFR operations are

prohibited.

The Special VFR weather minimums of Sec. 91.157 do not apply to the

following airports:

Atlanta, GA (The William B. Hartsfield Atlanta International Airport)

Baltimore, MD (Baltimore/Washington International Airport)

Boston, MA (General Edward Lawrence Logan International Airport)

Buffalo, NY (Greater Buffalo International Airport)

Chicago, IL (Chicago-O'Hare International Airport)

Cleveland, OH (Cleveland-Hopkins International Airport)

Columbus, OH (Port Columbus International Airport)

Covington, KY (Greater Cincinnati International Airport)

Dallas, TX (Dallas/Fort Worth Regional Airport)

Dallas, TX (Love Field)

Denver, CO (Denver International Airport)

Detroit, MI (Metropolitan Wayne County Airport)

Honolulu, HI (Honolulu International Airport)

Houston, TX (Houston Intercontinental Airport)

Indianapolis, IN (Indianapolis International Airport)

Los Angeles, CA (Los Angeles International Airport)

Louisville, KY (Standiford Field)

Memphis, TN (Memphis International Airport)

Miami, FL (Miami International Airport)

Minneapolis, MN (Minneapolis-St. Paul International Airport)

Newark, NJ (Newark International Airport)

New York, NY (John F. Kennedy International Airport)

New York, NY (LaGuardia Airport)

New Orleans, LA (New Orleans International Airport-Moisant Field)

Philadelphia, PA (Philadelphia International Airport)

Pittsburgh, PA (Greater Pittsburgh International Airport)

Portland, OR (Portland International Airport)

San Francisco, CA (San Francisco International Airport)

Seattle, WA (Seattle-Tacoma International Airport)

St. Louis, MO (Lambert-St. Louis International Airport)

Tampa, FL (Tampa International Airport)

Washington, DC (Washington National Airport)

Section 4. Locations at which solo student pilot activity is not permitted.

Pursuant to Sec. 91.131(b)(2), solo student pilot operations are not

permitted at any of the following airports.

Atlanta, GA (The William B. Hartsfield Atlanta International Airport)

Boston, MA (General Edward Lawrence Logan International Airport)

Chicago, IL (Chicago-O'Hare International Airport)

Dallas, TX (Dallas/Fort Worth Regional Airport)

Los Angeles, CA (Los Angeles International Airport)

Miami, FL (Miami International Airport)

Newark, NJ (Newark International Airport)

New York, NY (John F. Kennedy International Airport)

New York, NY (LaGuardia Airport)

San Francisco, CA (San Francisco International Airport)

Washington, DC (Washington National Airport)

Andrews Air Force Base, MD

[Amdt. No. 91-227, 56 FR 65661, Dec. 17, 1991, as amended by Amdt. 91-235,

58 FR 51968, Oct. 5, 1993; 58 FR 57549, Oct. 26, 1993; Amdt. 91-236, 59 FR

2918, Jan. 19, 1994; Amdt. 91-237, 59 FR 6547, Feb. 11, 1994; Amdt. 91-238,

59 FR 10958, Mar. 9, 1994; Amdt. 91-241, 59 FR 24916, May 13, 1994; 59 FR

29716, June 9, 1994]

Appendix E to Part 91--Airplane Flight Recorder Specifications

Installed

system /1/

minimum

accuracy (to     Sampling

recovered    interval (per  Resolution/4/

Parameters         Range          data)         second)       read out

Relative Time     8 hr minimum   +/-0.125% per  1              1 sec.

(From Recorded                   hour

on Prior to

Takeoff)

Indicated         Vso to VD      +/-5% or +/-   1              1% /3/

Airspeed          (KIAS)         10 kts.,

whichever is

greater.

Resolution 2

kts. below

175 KIAS

Altitude          -1,000 ft. to  +/-100 to +/-  11             25 to 150 ft.

max cert.      700 ft. (see

alt. of A/C    Table 1, TSO

C51-a)

Magnetic Heading  360 deg.       +/-5 deg.      1              1 deg.

Vertical          -3g to +6g     +/-0.2g in     4 (or 1 per    0.03g.

Acceleration                     addition to    second where

+/-0.3g        peaks, ref.

maximum        to 1g are

datum          recorded)

Longitudinal      +/-1.0g        +/-1.5% max.   2              0.01g.

Acceleration                     range

excluding

datum error

of +/-5%

Pitch Attitude    100% of        +/-2 deg.      1              0.8 deg.

usable

Roll Attitude     +/-60 deg. or  +/-2 deg.      1              0.8 deg.

100% of

usable

range,

whichever is

greater

Stabilizer Trim   Full Range     +/-3% unless   1              1% /3/

Position, or                     higher

uniquely

required

Pitch Control

Position

Engine Power,     Full Range     +/-3% unless   1              1% /3/

Each Engine:                     higher

uniquely

required

Fan or N1       Maximum Range  +/-5%          1              1% /3/

Speed or EPR

or Cockpit

indications

Used for

Aircraft

Certification

OR

Prop. speed                                   1 (prop        1% /3/

and Torque                                    Speed)

(Sample Once/                                 1 (torque)1% /3/

Sec as Close

together as

Practicable)

Altitude Rate/2/  +/-8,000 fpm   +/-10%.        1              250 fpm.

(need depends                    Resolution                    below 12,000

on altitude                      250 fpm

resolution)                      below 12,000

ft.

indicated

Angle of Attack/2/ -20 deg. to    +/-2 deg.     1              0.8% /3/

(need depends     40 deg. or

on altitude       100% of

resolution)       usable range

Radio             On/Off                        1

Transmitter

Keying

(Discrete)

TE Flaps          Each discrete                 1

(Discrete or      position (U,

Analog)           D, T/O, AAP)

OR

LE Flaps          Analog 0-100%  +/-3%          1              1% /3/

(Discrete or      range

Analog)

Each discrete                 1

position (U,

D, T/O, AAP)

OR

Thrust Reverser,  Analog 0-100%  +/-3 deg.      1              1% /3/

Each Engine       range

(Discrete)        Stowed or

full reverse

Spoiler/          Stowed or out                 1

Speedbrake

(Discrete)

Autopilot         Engaged or                    1

Engaged           Disengaged

(Discrete)

/1/ When data sources are aircraft instruments (except altimeters) of

acceptable quality to fly the aircraft the recording system excluding these

sensors (but including all other characteristics of the recording system)

shall contribute no more than half of the values in this column.

/2/ If data from the altitude encoding altimeter (100 ft. resolution) is

used, then either one of these parameters should also be recorded. If

however, altitude is recorded at a minimum resolution of 25 feet, then

these two parameters can be omitted.

/3/ Per cent of full range.

/4/ This column applies to aircraft manufactured after October 11, 1991.

Appendix F to Part 91--Helicopter Flight Recorder

Specifications

Installed

system /1/

minimum

accuracy (to     Sampling

recovered    interval (per  Resolution /3/

Parameters        Range          data)         second)       read out

Relative Time  4 hr minimum   +/-0.125% per  1              1 sec.

(From                         hour

Recorded on

Prior to

Takeoff)

Indicated      VM in to VD    +/-5% or +/-   1              1 kt.

Airspeed       (KIAS)         10 kts.,

(minimum       whichever is

airspeed       greater

signal

attainable

with

installed

pilot-static

system)

Altitude       -1,000 ft. to  +/-100 to +/-  1              25 to 150 ft.

20,000 ft.     700 ft. (see

pressure       Table 1, TSO

altitude       C51-a)

Magnetic       360 deg.       +/-5 deg.      1              1 deg.

Heading

Vertical       -3g to +6g     +/-0.2g in     4 (or 1 per    0.05g.

Acceleration                  addition to    second where

+/-0.3g        peaks, ref.

maximum        to 1g are

datum          recorded)

Longitudinal   +/-1.0g        +/-1.5% max.   2              0.03g.

Acceleration                  range

excluding

datum error

of +/-

Pitch          100% of        +/-2 deg.      1              0.8 deg.

Attitude       usable range

Roll Attitude  +/-60 or 100%  +/-2 deg.      1              0.8 deg.

of usable

range,

whichever is

greater

Altitude Rate  +/-8,000 fpm   +/-10%         1              250 fpm below

Resolution                    12,000.

250 fpm

below 12,000

ft.

indicated

Engine Power,

Each Engine

Main Rotor     Maximum Range  +/-5%          1              1% /2/.

Speed

Free or Power  Maximum Range  +/-5%          1              1% /2/.

Turbine

Engine Torque  Maximum Range  +/-5%          1              1% /2/.

Flight

Control

Hydraulic

Pressure

Primary        High/Low                      1

(Discrete)

Secondary--if  High/Low                      1

applicable

(Discrete)

Radio          On/Off                        1

Transmitter

Keying

(Discrete)

Autopilot      Engaged or                    1

Engaged        Disengaged

(Discrete)

SAS Status-    Engaged or                    1

Engaged        Disengaged

(Discrete)

SAS Fault      Fault/OK                      1

Status

(Discrete)

Flight

Controls

Collective     Full range     +/-3%          2              1% /2/.

Pedal          Full range     +/-3%          2              1% /2/.

Position

Lat. Cyclic    Full range     +/-3%          2              1% /2/.

Long. Cyclic   Full range     +/-3%          2              1% /2/.

Controllable   Full range     +/-3%          2              1% /2/.

Stabilator

Position

/1/ When data sources are aircraft instruments (except altimeters) of

acceptable quality to fly the aircraft the recording system excluding

these sensors (but including all other characteristics of the recording

system) shall contribute no more than half of the values in this column.

/2/ Per cent of full range.

/3/ This column applies to aircraft manufactured after October 11, 1991.