PART 97--STANDARD INSTRUMENT APPROACH PROCEDURES

Subpart A--General

Sec.

97.1  Applicability.

97.3  Symbols and terms used in procedures.

97.5  Bearings; courses; headings; radials; miles.

Subpart B--Procedures

97.10  General.

Subpart C--TERPS Procedures

97.20  General.

Authority: 49 U.S.C. 1348, 1354(a), 1421, and 1510; 49 U.S.C. 106(g)

(Revised, Pub. L. 97-449, January 12, 1983; and 14 CFR 11.49(b)(2)).

Source: Docket No. 1580, Amdt. 1-1, 28 FR 6719, June 29, 1963, unless

otherwise noted.

Cross Reference: For changes in isogenic values based on the U.S. Coast and

Geodetic Survey for Epoch 1965, which minutely affect the Standard Instrument

Approach Procedures of this part, see 30 FR 7598, June 11, 1965.

Subpart A--General

Sec. 97.1   Applicability.

This part prescribes standard instrument approach procedures for instrument

letdown to airports in the United States and the weather minimums that apply

to takeoffs and landings under IFR at those airports.

Sec. 97.3   Symbols and terms used in procedures.

As used in the standard terminal instrument procedures prescribed in this

part--

(a) "A" means alternate airport weather minimum.

(b) "Aircraft approach category" means a grouping of aircraft based on a

speed of 1.3 Vso (at maximum certificated landing weight). Vso and the

maximum certificated landing weight are those values as established for the

aircraft by the certificating authority of the country of registry. The

categories are as follows:

(1) Category A: Speed less than 91 knots.

(2) Category B: Speed 91 knots or more but less than 121 knots.

(3) Category C: Speed 121 knots or more but less than 141 knots.

(4) Category D: Speed 141 knots or more but less than 166 knots.

(5) Category E: Speed 166 knots or more.

(c) Approach procedure segments for which altitudes (all altitudes

prescribed are minimum altitudes unless otherwise specified) or courses, or

both, are prescribed in procedures, are as follows:

(1) "Initial approach" is the segment between the intial approach fix and

the intermediate fix or the point where the aircraft is established on the

intermediate course or final approach course.

(2) "Initial approach altitude" means the altitude (or altitudes, in High

Altitude Procedures) prescribed for the initial approach segment of an

instrument approach.

(3) "Intermediate approach" is the segment between the intermediate fix or

point and the final approach fix.

(4) "Final approach" is the segment between the final approach fix or point

and the runway, airport, or missed-approach point.

(5) "Missed approach" is the segment between the missed-approach point, or

point of arrival at decision height, and the missed-approach fix at the

prescribed altitude.

(d) "C" means circling landing minimum, a statement of ceiling and

visibility values, or minimum descent altitude and visibility, required for

the circle-to-land maneuver.

(d-1) "Copter procedures" means helicopter procedures, with applicable

minimums as prescribed in Sec. 97.35 of this part. Helicopters may also use

other procedures prescribed in Subpart C of this part and may use the

Category A minimum descent altitude (MDA) or decision height (DH). The

required visibility minimum may be reduced to one-half the published

visibility minimum for Category A aircraft, but in no case may it be reduced

to less than one-quarter mile or 1,200 feet RVR.

(e) "Ceiling minimum" means the minimum ceiling, expressed in feet above

the surface of the airport, required for takeoff or required for designating

an airport as an alternate airport.

(f) "D" means day.

(g) "FAF" means final approach fix.

(h) "HAA" means height above airport.

(h-1) "HAL" means height above a designated helicopter landing area used

for helicopter instrument approach procedures.

(i) "HAT" means height above touchdown.

(j) "MAP" means missed approach point.

(k) "More than 65 knots" means an aircraft that has a stalling speed of

more than 65 knots (as established in an approved flight manual) at maximum

certificated landing weight with full flaps, landing gear extended, and power

off.

(l) "MSA" means minimum safe altitude, an emergency altitude expressed in

feet above mean sea level, which provides 1,000 feet clearance over all

obstructions in that sector within 25 miles of the facility on which the

procedure is based (LOM in ILS procedures).

(m) "N" means night.

(n) "NA" means not authorized.

(o) "NOPT" means no procedure turn required (altitude prescribed applies

only if procedure turn is not executed).

(o-1) "Point in space approach" means a helicopter instrument approach

procedure to a missed approach point that is more than 2,600 feet from an

associated helicopter landing area.

(p) "Procedure turn" means the maneuver prescribed when it is necessary to

reverse direction to establish the aircraft on an intermediate or final

approach course. The outbound course, direction of turn, distance within

which the turn must be completed, and minimum altitude are specified in the

procedure. However, the point at which the turn may be commenced, and the

type and rate of turn, is left to the discretion of the pilot.

(q) "RA" means radio altimeter setting height.

(r) "RVV" means runway visibility value.

(s) "S" means straight-in landing minimum, a statement of ceiling and

visibility, minimum descent altitude and visibility, or decision height and

visibility, required for a straight-in landing on a specified runway. The

number appearing with the "S" indicates the runway to which the minimum

applies. If a straight-in minimum is not prescribed in the procedure, the

circling minimum specified applies to a straight-in landing.

(t) "Shuttle" means a shuttle, or race-track-type, pattern with 2-minute

legs prescribed in lieu of a procedure turn.

(u) "65 knots or less" means an aircraft that has a stalling speed of 65

knots or less (as established in an approved flight manual) at maximum

certificated landing weight with full flaps, landing gear extended, and power

off.

(v) "T" means takeoff minimum.

(w) "TDZ" means touchdown zone.

(x) "Visibility minimum" means the minimum visibility specified for

approach, or landing, or takeoff, expressed in statute miles, or in feet

where RVR is reported.

(Secs. 307(c), 313(a), 601, Federal Aviation Act of 1958, as amended (49

U.S.C. 1348(c), 1354(a), 1421); sec. 6(c), Department of Transportation Act

(49 U.S.C. 1655(c)))

[Amdt. 97-561, 32 FR 13911, Oct. 6, 1967, as amended by Amdt. 97-803, 37 FR

6287, Mar. 28, 1972; Amdt. 97-1133, 44 FR 15659, Mar. 15, 1979]

Sec. 97.5   Bearings; courses; headings; radials; miles.

(a) All bearings, courses, headings, and radials in this part are magnetic.

(b) RVR values are stated in feet. Other visibility values are stated in

statute miles. All other mileages are stated in nautical miles.

[Amdt. 97-561, 32 FR 13912, Oct. 6, 1967]

Subpart B--Procedures

Editorial Note: The procedures set forth in this subpart were formerly

carried as Secs. 609.100 through 609.500 of this title and were transferred

to Part 97 as Secs. 97.11 through 97.19, respectively, but are not carried in

the Code of Federal Regulations. For Federal Register citations affecting

these procedures, see the List of CFR Sections Affected in the Finding Aids

section of this volume.

Sec. 97.10   General.

This subpart prescribes standard instrument approach procedures other than

those based on the criteria contained in the U.S. Standard for Terminal

Instrument Approach Procedures (TERPs). Standard instrument approach

procedures adopted by the FAA and described on FAA Form 3139 are incorporated

into this part and made a part hereof as provided in 5 U.S.C. 552(a)(1) and

pursuant to 1 CFR Part 20. The incorporated standard instrument approach

procedures are available for examination at the Rules Docket and at the

National Flight Data Center, Federal Aviation Administration, 800

Independence Avenue SW., Washington, D.C. 20590. Copies of SIAPs adopted in a

particular FAA Region are also available for examination at the headquarters

of that region. Moreover, copies of SIAPs originating in a particular Flight

Inspection District Office are available for examination at that office.

Based on the information contained on FAA Form 3139, standard instrument

approach procedures are portrayed on charts prepared for the use of pilots by

the U.S. Coast and Geodetic Survey and other publishers of aeronautical

charts.

[Amdt. 97-969, 35 FR 5609, Apr. 7, 1970]

Subpart C--TERPS Procedures

Source: Docket No. 8130, Amdt. 97-561, 32 FR 13912, Oct. 6, 1967, unless

otherwise noted.

Editorial Note: The procedures for Secs. 97.21 through 97.35, respectively,

are not carried in the Code of Federal Regulations. For Federal Register

citations affecting these procedures, see the List of CFR Sections Affected

in the Finding Aids section of this volume.

Sec. 97.20   General.

This subpart prescribes standard instrument approach procedures based on

the criteria contained in the U.S. Standard for Terminal Instrument Approach

Procedures (TERPs). The standard instrument approach procedures adopted by

the FAA and described on FAA Form 8260-3, 8260-4, or 8260-5 are incorporated

into this part and made a part hereof as provided in 5 U.S.C. 552(a)(1) and

pursuant to 1 CFR Part 20. The incorporated standard instrument approach

procedures are available for examination at the Rules Docket and at the

National Flight Data Center, Federal Aviation Administration, 800

Independence Avenue SW., Washington, D.C. 20590. Copies of SIAPs adopted in a

particular FAA Region are also available for examination at the headquarters

of that Region. Moreover, copies of SIAPs originating in a particular Flight

Inspection District Office are available for examination at that Office.

Based on the information contained on FAA Form 8260-3, 8260-4, and 8260-5,

standard instrument approach procedures are portrayed on charts prepared for

the use of pilots by the U.S. Coast and Geodetic Survey and other publishers

of aeronautical charts.

(Sec. 6(c) Department of Transportation Act. 49 U.S.C. 1655(c) and 5 U.S.C.

552(a)(1))

[Amdt. 97-969, 35 FR 5609, Apr. 7, 1970]