#                              MEMO#

#TO#:       Howard

Frank

#FROM#:     Bob Groppo

#RE#:       CP/M CBIOS for the new Decision 1

#DATE#:     4/19/83

Th� followin� � configuration� wil� b� require� fo� th� Decsio� � �

in the ne� configuration:

64K CP/M Version 2.2 Cbios revision E.31          1 of 2

BOOT:   A,D: DJ/DMA 5"  B,C: HD/DMA M10  E,F: DJ/DMA 8"

MOVCPM10: A,B: HD/DMA M10  C,D: DJ/DMA 5"  E,F: DJ/DMA 8"

Multio 9600 Console;   1200 (CTS) List;  Centronics Punch

==>> BOOT DISKETTE <<==

64K CP/M Version 2.2 Cbios revision E.31          1 of 2

BOOT:   A,D: DJ/DMA 5"  B,C: HD/DMA M16  E,F: DJ/DMA 8"

MOVCPM10: A,B: HD/DMA M16  C,D: DJ/DMA 5"  E,F: DJ/DMA 8"

Multio 9600 Console;  1200 (CTS) List;  Centronics Punch

==>> BOOT DISKETTE <<==

The "Boot" disk should have only the following files:

PIP.COM        STAT.COM       SYSGEN.COM          MOVCPM.COM

MOVCPM10.COM   FORMATMW.COM   FORMATDJ.COM        BAUD.COM

DDT.COM        CHECK.COM      MBASIC.COM          BAUD.DOC

FILES.DOC      FORMATMW.DOC   FORMATDJ.DOC        AUTO.COM

Al� listing� (.ASM� an� rarel� use� file� shoul� b� o� Dis� �� o� �

2��  Thi� wil� mak� installin� th� softwar� ont� th� har� dis� �

much simpler.

W� shoul� als� suppl� � CENTRONIC� drive� fo� th� PUNC� devic� t� �

g� alon� wit� th� adapte� boar� whic� shoul� loo� lik� this:

.pa�;********************************************************

;*  Centronics print routine.			        *

;*  11/12/82  BJG					*

;********************************************************

BASE	EQU	48H

GROUP	EQU	BASE + 7

CLOCK	EQU	BASE + 2

STROBE	EQU	BASE

DATA	EQU	BASE + 1

STATUS  EQU	BASE

BUSY	EQU	20H		;PRINTER BUSY FLAG

; MULT I/O PRINTER RESTORE SUBROUTINE (VIA GROUP PORT)

; EXECUTE ONCE AT SYSTEM COLD BOOT

MRESTR:	MVI A,0H

OUT GROUP

MVI A,0C0H

OUT STROBE		;TURN DATA STROBE OFF

MVI A,30H

OUT GROUP		;TURN RESTORE ON

MVI A,20H		;TURN RESTORE OFF

OUT GROUP

;  WB I/O PRINTER RESTORE ROUTINE

;  EXECUTE ONCE AT SYSTEM COLD BOOT

WRESTR: MVI A,0FFH

OUT 48H			;TURN STROBES OFF

MVI A,0C0H

OUT CLOCK		;TURN ON DRIVERS

MVI A,07FH		;SEND RESTORE

OUT STROBE

MVI A,0FFH

OUT STROBE

;  ENTER ROUTINE WITH CHARACTER TO PRINT IN "C"

;  CP/M LST PRINT ROUTINE

PRINT:	IN STATUS		;CHECK IF PRINTER READY

ANI BUSY

JZ PRINT		;LOOP TILL READY

.pa�MCHECK: IN 049H			;READ MOTHERBOARD SWITCH

CPI 0FFH		;FF MEANS MULT I/O

JZ PMULT

MOV A,C

OUT DATA		;DATA VALID

MVI A,0BFH		;STROBE LOW

OUT STROBE

XTHL			;STALL 10 uS

XTHL

MVI A,0FFH		;STROBE HIGH

OUT STROBE

RET

PMULT:  MOV A,C

OUT STROBE

MVI A,0BFH		;STROBE LOW

OUT DATA

XTHL			;STALL 10 uS

XTHL

MVI A,0FFH		;STROBE HIGH

OUT DATA

RET

END

W� shoul� als� suppl� � simpl� utilit� t� swa� th� PUNC� and LIS� �

devic� betwee� th� seria� por� an� centronic� por� t� mak� i� �

easier on the customer when he runs the application packages.

I� w� ha� a� AUTO.CO� comman� lik� i� th� Micr� Decision�� w� �

coul� execut� th� abov� progra� o� powe� u� t� se� whic� device�

will normally be the LIST device.

W� nee� th� abov� softwar� complete� b� NC� fo� releas� wit� th� �

new computer.  The new documentation relies on these changes.#####################################################################################################