MORSE v1.11

-----------

MORSE is a little CW training program. It is primarily written for

Soundblaster Pro, but it will probably work with other Creative cards

(SB16, AWE32) as well. However, I'm not 100% sure. Give it a try, as

I use to say :).

Please see CHANGES.DOC for the latest changes/additions.

IMPORTANT

---------

The configuration file format has changed in this version. Therefore,

if you have any configuration files from v1.10 (or previous version)

you want to use, you have to convert them to the new format with the

CFGCONV utility before loading them into v1.11.

--------------------------------------------------------------------------

Commands

--------

1   Send groups of 5 with random characters.

2   Send variable length groups with random characters.

3   Send text from an ASCII file.

C   Select which characters to include when using 1 and 2 above.

To toggle a single character, just press the corresponding key

(As I am from Sweden, the program also includes the Swedish

characters �, � and �).

You can toggle whole groups of characters as well:

Alt-L   Toggle letters

Alt-N   Toggle numbers

Alt-X   Toggle other characters and special codes

A clarification of the characters used for special codes

--------------------------------------------------------

I'm not sure if the characters used in the program for special

codes are standard, but here's a list of what they stand for:

*  Repetition

=  Break

~  Attention

�  AS  (Wait)

+  End of transmission    (dit-dah-dit-dah-dit)

@  End of QSO             (dit-dit-dit-dah-dit-dah)

M   Set min/max length for variable length groups.

Valid values range from 1 to 80.

W   Set speed in WPM.

Valid values range from 5 to 999. 500 seems to be a good

speed to start with for beginners (just kidding).

F   Set tone frequency for CW and interference tone (in Hz).

Valid values range from 10 to 6213. Just for fun... :)

T   Set CW tone attack/release.

Valid values range from 0 to 15. This command lets you set

the attack and release times of the CW tone. It acts almost

like some sort of "weight" setting, however, the relations

between the dit's and dah's don't change. Only the audio

characteristics of the tone.

Note: When you set attack or release to low values, the tone

gets a strange "breathy" character. It is unfortunately un-

avoidable, so for the most natural tone, stick with values

from about 9 and up.

(After all, it wasn't me that engineered the FM chip :))

I   Toggle interference tone on/off.

H   Show/hide sent characters.

B   Set type of background noise:

0 = None   1 = Standard radio hiss :)   2 = Pile-up

V   Set volume levels.

Valid values range from 0 to 15. The mixer of Soundblaster

Pro is kind of strange; as an example, if you enter a value

of 14, it will change it to 15. *In spite of* the hardware

registers having 4 bits for the value. It seems that it only

uses three bits internally, as the volume only changes on

every other value.

S   Save configuration

The following parameters are saved:

WPM

Character selection

Minimum & maximum group length

Volume levels

Noise type

Tone frequencies

CW tone attack/release

Interference tone setting (on/off)

"MORSE.CFG" is automatically read upon start of the program.

L   Load a configuration file.

D   Dir, list files in current directory.

J   Jump to DOS.

Q   Quit.

---------------------------------------------------------------------------

The program is rather self-explanatory on the whole, so I think you'll

get an understanding of it rather quickly.

Good luck!

Mats Petersson SM5SXL                 mats.petersson@mbox301.swipnet.se