INTERRUPTS, RASTERS, BORDERS AND OTHER STUFF by Stefan Posthuma

With  my hands still shaking a little from a heavy Andes  Attack

session  (I reached level 14 with 350000 or so points!),  I  feel

ready  to tell you something about the things that make life   on

the ST so colorful.  In fact,  they are the essential element  of

every super-duper demo that is being created nowadays.  They  are

called: rasters.

First of all,  let me tell you what rasters are and before that,

I will inform you about the basis of a raster: the interrupt.

Let's say you write a program and you want to do something which

has  to  be done with specific time-intervals,  like  blinking  a

cursor  or  playing  some music.  You can go  through  a  lot  of

trouble  and  call  your routines very often  and  time  them  or

something so they will be executed regularly,  or you can program

them on interrupt.

When you move your mouse, the keyboard processor will notice it,

process the movement and then send the information to the  68000.

It sounds simple,  but how does one processor 'send' something to

the other? Well, it simply 'interrupts' the processor it wants to

tell  something.  In  case of the mouse  movement,  the  keyboard

processor will generate an interrupt which will put the 68000  in

interrupt mode.

Now the 68000 has 7 interrupt levels.  This means that there are

7  different  interrupts  possible.   Also  this  means  that  an

interrupt can interrupt an interrupt when it has a higher level!

The  different levels are used for the  keyboard-processor,  the

vertical blank interrupt etc. More on this later.

Immediately  after it has received the signal,  the  68000  will

first complete its current instruction.  After that,  the  status

register  and  the  program counter will be  put  on  stack,  the

processor  will enter supervisor mode and will jump to a  certain

memory  location.  These  locations are set  on  fixed  addresses

somewhere  in the first 500 bytes of memory or so.  The  keyboard

processor will create a level 6 interrupt.  The 68000 will  fetch

the value of memory location $118 and jump to the address  formed

by  that value.  Then it will execute the  instructions  normally

from that address. When it reaches an RTE instruction, the status

register  and program counter will be fetched from stack and  the

processor  goes  back  to user mode and  program  execution  will

continue  just like nothing happened.  As long as  the  interrupt

routine does not corrupt the stack or changes any registers  used

by  the normal program,  the normal program will not 'know'  that

there has been an interrupt.

Still with me?  If you're not, don't worry too much about it, it

is  not  totally essential for the understanding of the  rest  of

this article.

Now  TOS  has a nice feature if you want to  use  an  interrupt-

driven routine.  The level 4 interrupt is the VBL.  As you  might

know,  the  screen  is painted by an electron beam.  On  a  color

monitor in 50Hz mode,  the screen is drawn 50 times per second by

this electron beam.  It starts at the top left-hand corner of the

screen  and  draws from left to right.  When it has  reached  the

right edge of the screen,  it will jump back to the left and draw

the next line.  When it has reached the bottom of the screen,  it

will jump back to the top of the screen and start all over again.

At this point,  the level 4 interrupt will be generated. The time

needed for the electron beam to go back to the top of the  screen

is  long enough for the 68000 to do some work.  When you  are  in

50Hz  mode,  the  level 4 interrupt will be created  50  times  a

second!

At location $4CE (a legal one),  is a list of 8  addresses.  The

first few ones are used by TOS,  but there are always a couple of

them free to use.  (They will be zero).  When you have a routine,

just put its address in one of the free locations and it will be

called 50 times a second! Make sure your routine is short because

it will slow down other operations.

Back  to the story about the electron beam.  Let's say  you  can

track  it  and  you can do something at specific  places  on  the

screen.  If you can somehow see when the beam is in the middle of

the  screen  and you switch color palette  just  then,  the  rest

of the screen will be painted using the new palette and you  will

have 32 colors on the screen!  (This is the basic principle, make

sure you understand it!!)

So how can we track the electron beam then?

Well,  it all comes down to timer B.  Timer B is a nice  feature

from  the MFP 68901 processor and it counts  screen  lines!  Each

time a new screen line is drawn,  the timer B data register  will

be increased so it will contain the number of the scanline drawn,

starting  at the first one after the upper  border.  Even  better

about timer B is that when you put a value in it,  let's say  10,

10  scanlines later an interrupt will be  generated!  Well,  this

means that we have found the way. This is the way it works!

Ok, practical now. Let's create a raster.

First of all, we need a safe way to trigger timer B. Why not use

the  level 4 interrupt which occurs each time the  electron  beam

reaches the bottom of the screen. Here we set the value for timer

B and in that scanline, the interrupt will occur! So if we want a

raster on scanline 100,  we activate timer B with a value of  100

in the level 4 interrupt and wait for the timer B interrupt.

Uh oh....strange things are happening....

MFP 68901: Hi guys! Interrupt time!

68000    : No way, I'm tired...leave me alone!

MFP 68901: Ah, come one, just one little interrupt...

68000    : Forget it, go away!

MFP 68901: Ok, have it your way.

68000    : Fine.

MFP 68901: OVERRIDE!!!!!!

68000    : OUCH!!!

MMU      : Hey? What's going on? Who is overriding there without

filing the proper request at my address bus?

MFP 68901: I am and you can't stop me.

68000    : Groan...

MMU      : Stop it right now or I'll have to get involved here.

68000    : Yeah, stop this idiot!

MFP 68901: Fools!

MMU      : SYNCHRONISE!!

Shifter  : AAAAAAHHHHH! My borders!!!

MMU      : Ooops, wrong interrupt level.

MFP 68901: Wow! no borders, let's throw some rasters in there!

Shifter  : NO!!! AAAAAAAAAAAAAGGGRRLLL!!

68000    : Hey guys, what about me?

MMU, MFP 68901: We're having fun.  In fact,  we haven't had this

much fun since the Union Demo.

Shifter  : WHAT???!! UNION DEMO??!!!? AAAAAAAHAHAAAAAHAAHAAA!!!

68000    : Nurse!!

MMU      : Yeah,  border  here,  synchro there.....I'm having  a

blast.

MFP 68901: Ok, what does this address do?

Shifter  : AHAHAAAAAAAAAGGRRRLGGALAAAHAAHAHHH!!!!

68000    : Hey, look out!!

Gemdos critial error handler: PANIC!

Exception vector: ZAP!

68000, MMU, MFP 68901: Uh oh.....

Shifter  : UNION DEMO!!!  RAAAAAAHHH! LOST BOYS!! GROOOOOWWWLL!!

CAREBEARS!!! AAAAAAAAAAHHAHAHAHAHHAHAHGGGGGLLLlllpp..

ST       : CRASH!!

Stefan   : F.CK!!

Good thing I press CONTROL-V regularly.  I was about to tell you

exactly how to program a Timer B raster interrupt. First thing we

do  is to install the new vectors so the 68000 will jump  to  our

routines when interrupted.  The VBL vector is $70 and the Timer B

vector  is  $120.  Now before we start messing  around  with  the

vectors,  we  have to put them in a safe place so we can  restore

them to their normal values at the end of the program.  Also,  we

must make sure that any other VBL routines (e.g. music) are still

called  so  we must jump to the old value of the level  4  vector

after our level 4 interrupt.  The following source fragments  are

originally by TEX.  All I know about rasters I learned from them.

Thanks guys!

Note: supervisor mode must be on before you can mess with any of

these registers.

hblon:   move.l  $120,oldtb      ; save old timer B vector

move.l  $70,old4        ; save old VBL vector

move.l  $70,new4b+2     ; now,  the new VBL routine will

jump  to the old VBL after  it

is done.

move.b  $fffffa07,old07 ; Timer B enable

move.b  $fffffa09,old09 ; Timer C enable

move.b  $fffffa0f,old0f ; Timer B in-service

move.b  $fffffa11,old11 ; Timer C in-service

move.b  $fffffa1b,old1b ; Timer B control

and.b   #$df,$fffa09    ; disable Timer C

and.b   #$fe,$fffa07    ; disable Timer B

move.l  #newtb,$120     ; new Timer B vector

move.l  #new4,$70       ; new VBL vector

or.b    #1,$fffffa07    ; enable Timer B

or.b    #1,$fffffa13    ; set Timber B mask

rts

You can see that timer C will be switched off.  This to  prevent

it from messing things up.  Now we have installed two vectors for

the VBL and the Timer B and we have set some registers of the MFP

68901 to allow Timer B interrupts and to disable Timer C.

The thing we need now are the two routines to handle the VBL and

the Timer B we installed.

new4:           clr.b   $fffffa1b       ; disable Timer B

move.b  100,$fffffa21   ; set Timber B offset

move.b  #8,$fffffa1b    ; enable Timer B

move.w  #$0,$ff8240     ; background = black

new4b:          jmp     $12345678       ; the old VBL vector will

have    replaced    the

$12345678  so   control

will  be passed  on  to

the old VBL routine.

The  important value here is the '100'.  This will mean that  in

the 100th scanline of the screen,  a Timer B interrupt occurs and

the vector at $120 will be called.

newtb:          clr.b   $fffffa1b.w     ; timer stop

movem.l a0/d0,-(sp)     ; save A0 and D0

move.w  #$fa21,a0       ; timer B data

move.b  (a0),d0         ; get value of Timer B

wait:           cmp.b   (a0),d0         ; wait one scanline

beq.s   wait

move.w  #$700,$ff8240   ; background red

movem.l (sp)+,a0/d0     ; get registers back

bclr    #0,$fffffa0f.w  ; end of interrupt

rte

First  of all,  you have to stop the timer to prevent any  other

interrupts from it.  After that,  you HAVE to push any  registers

you use on the stack. This is of vital importance, else somewhere

in  the middle of the normal program,  registers will be  changed

and  things  go  utterly wrong.  Then we load the  Timer  B  data

register into A0. Reading this register will give you the current

value of Timber B,  i.e. the current scanline. After that we read

the value of it in D0 and wait until it changes.  In other words,

we wait for the beginning of the next scanline.

Why?

Well,  when an interrupt occurs,  the 68000 has to complete  its

current instruction. After that, registers (PC and ST) have to be

pushed on stack etc.  All this takes a little while and when  the

actual interrupt routine is called,  the electron beam is already

in the left border,  drawing visible screen area.  If you  change

colors now,  this will be visible.  Also,  the interrupt will not

always  occur  at exactly the same place,  so you will  see  some

nervous  flickering which is of course not tolerable if you  want

to write a demo.

So  we  wait  until  the beginning  of  the  next  scanline  and

immediately after, change screen color. The electron beam is then

still  drawing the non-visible parts of the left border and  your

raster will be nice and stable.

To restore the vectors, use the following code:

hbloff:     move.w  sr,-(sp)            ; save status register

move.w  #$2700,sr           ; disable all interrupts

move.b  old07(pc),$fffffa07 ; restore all registers

move.b  old09(pc),$fffffa09

move.b  old0f(pc),$fffffa0f

move.b  old11(pc),$fffffa11

move.b  old1b(pc),$fffffa1b

move.l  oldtb,$120          ; old Timer B back

move.l  old4,$70            ; old VBL back

move.w  (sp)+,sr            ; restore status register

rts

OK,  these  are the raw basics for raster programming.  Now  you

know  how to create a raster, let your imagination run  free  and

create demos!

I  will  give you one more example,  let's say we  want  another

raster  at screen line 150.  This means we will have two Timer  B

interrupts,  one at line 100 and another one at line 150.  In the

VBL, you set the one for line 100 and in the Timer B interrupt at

line 100, you set the one for line 150. Easy!

So  in the new4 routine,  you add 1 line of code that  sets  the

vector for the first Timer B.

move.l  newtb1,$120

In the newtb1 routine, you have to do three extra things:

- set the new value for Timer B

- set the new Timer B vector

- enable Timer B again.

The  new value is set in scanlines from the current one.  So  if

you  want a raster at scanline 150,  you have to set the Timer  B

value to 150-100 = 50.

move.l  #$fa21,a0

move.b  #50,(a0)        ; this sets the new Timer B value

move.l  #newtb2,$120    ; set the new Timer B vector

move.b  #8,$fffffa1b    ; allow Timer B again

Now you have to create a routine called newtb2 that handles  the

next raster.  It is identical to the newtb1 routine (without  the

new changes).

Of course,  you can do more things with rasters than just change

background color.  By changing more colors, and changing not only

one scanline,  but some more,  you can create the typical 'round'

rasters that are to be found in so many demos.  Just take a  look

at  the color demos in this folder.

Also,  if  you  use a table of values for timer B instead  of  a

fixed number, you can create moving rasters.

The last purpose of raster interrupts is to get rid of  borders.

Now  it has been explained already in the wizard  series,  but  I

will tell you once again. This is how border-obliteration works:

The MMU (Memory Management Unit or GLUE chip) tells the  Shifter

(the video chip responsible for drawing the screen) exactly  when

to display screen memory and when to display border. There is one

thing that the MMU does not really like and that is the  changing

of screen frequency (register $ff820a). When you do this, it will

'forget' the Shifter for a tiny while.

Now  if you change the frequency at a certain point in the  last

scanline of the screen,  the MMU will forget to tell the  Shifter

that  the lower border has started and the Shifter  will  happily

continue displaying screen memory instead of border.

The only thing special about the new screen memory is that it is

shifted one plane to the left.  There is 16 bytes missing at  the

lower-right  corner of the screen.  But that is not  annoying  at

all.

So you simply generate a Timer B interrupt at scanline 199, wait

a  while,  change screen frequency (50-60Hz),  wait for the  next

scanline,  wait some more and change the frequency back (60-50Hz)

and the border will be gone!  Timer B will also happily  continue

counting scanlines, so rasters in the lower border are no trouble

whatsoever!  (Check out the color demo!).  For source material

about this, check ST NEWS 4.2

Removing the upper border is achieved in the same way. But there

is  a  serious problem here.  There are two versions of  the  MMU

around and they both require a different position where you  have

to change frequency.  It wouldn't be that much of a problem (just

change  frequency  at both points and you are home free)  if  the

screen  address wouldn't start at different points on the  screen

then. This  would mean that your graphics would be all  misplaced

on one ST while it works perfectly well on another.

There  must be a way to check this.  I think it can be  done  by

generating  a  Timer  B  interrupt  at  a  certain  position  and

examining  the  video  address counter  ($ff8205-$ff8209)  but  I

haven't tried this yet.

Removing the right and the left border is a little (a LOT  more)

harder.  It  still has to do with changing frequency,  but  other

problems arise here.  Now getting rid of the right border is  not

too  difficult  (I  already managed it).  You have  to  create  a

raster,  wait  a  while until the electron beam it at  a  precise

point and change frequency.  Do it at exactly the right spot  and

the right border will be gone.  The problem lies in the  exactly.

Remember the story about the flicker?  Well, even if you wait for

the next scanline,  you never can be sure of the exact  position.

So you need a way to synchronise the raster so it happens at  the

exact spot every time.  There is a way but it is a little hard to

explain.

With this technique,  you can also create 'scrolling' rasters. I

did this in the 4.4 demo (really nice one...I'm still working  on

it)  and there is an ST NEWS logo in the lower,  left  and  right

border  which scrolls and bounces and is made up entirely out  of

rasters. It is REALLY nice.

Also,  you  have open the border over and over again  for  every

scanline.  This  means  you spend a lot of time waiting  for  the

right border and if you open the entire right border,  you  won't

have any time left.

The left border is still a mystery to me. After a lot of trying,

I finally gave up.  The screen did the most crazy things but  the

left border just remained.

One  final remark about the full-screen demo by Level 16  (UNION

DEMO!!!).  Andreas  told  me  how  he did it  and  it  is  really

brilliantly  solved.  I mean you spend all time waiting  for  the

border  so  there is no time left for the things  he  does  (like

music, a sprite, color animation and rasters). But he did it.

It has to do with filling in the code for the music etc.  in the

'gaps'  (normally  a lot of NOP's) where he waits  for  the  next

border.  It  must  have  been one hell of a job  to  get  it  all

synchronised.  Also,  it is just one big list of instructions, no

loops or whatever (one jump I guess to the beginning). So this is

why  it  is so big and this is why the assembler  source  is  2Mb

large.  Now  this  is just guessing (but I think I am  right)  so

Andreas might laugh a lot after reading this.

Check out the source in the RASTERS folder.  'RASTERS.S' it  is

called.