*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*

|                                                                           |

*                            DOC sur le BLITTER ATARI                       *

|                                                                           |

*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*

Well, since Mr. Good hasn't said anything about this, I will

assume that it' s safe to post this (particularly since this

won' t do you too much good  if you don' t actually have the

blitter).  This is the  manual that  I received on disk from

Atari which covers the details of the blitter chip  and  how

it works.  Enjoy!

P.S:   It is already formatted for 66 lines per page output, right

justified, and stuff like that.  For those of you who need

different margins and such, I apologize,  this is the only

copy I have.

****

****

****

******

** ** **

**  **  **

**    **    **

User Manual for the Atari ST

Bit-Block Transfer Processor (BLiTTER)

The Atari Corporation

Sunnyvale, California

15 June 1987

TABLE OF CONTENTS

Introduction ......................................    1

Bit-Block Transfers ..........................    2

Bit-Block Transfer ...........................    3

Functional Description ............................    3

Programming Model .................................    5

Register Map .................................    5

Bit-Block Addresses ..........................    5

Source X Increment ......................    6

Source Y Increment ......................    6

Destination Address .....................    6

Destination X Increment .................    6

Destination Y Increment .................    6

X Count .................................    7

Y Count .................................    7

Bit-Block Alignments .........................    7

Endmask 1, 2, 3 .........................    7

Skew ....................................    7

FXSR ....................................    7

NSFR ....................................    8

Logic Operations .............................    8

Logic Operations ........................    8

Halftone Operations ..........................    8

Halftone RAM ............................    8

Line Number .............................    8

Smudge ..................................    9

Halftone Operations .....................    9

Bus Accesses .................................    9

Hog .....................................    9

Busy ....................................    9

Appendix A -- Programming Example .................    10

Appendix B -- References ..........................    17

THE SCOPE  OF THIS  DOCUMENT is limited to a functional description of the

Atari ST BLiTTER.   This document  is not  a data  sheet for  system inte-

gration, rather  it is  a user  manual for  system programming.   For more

information, please refer to the texts listed at the end of this document.

*** INTRODUCTION

The Atari ST Bit-Block Transfer Processor  (BLiTTER) is  a hardware imple-

mentation of  the bit-block  transfer (BitBlt)  algorithm.   BitBlt can be

simply described as a procedure that moves bit-aligned data from  a source

location to  a destination  location through a given logic operation.  The

BitBlt primitive can be used to perform such operations as:

o  Area seed filling

o  Rotation by recursive subdivision

o  Slice and smear magnification

o  Brush line drawing using Bresenham DDA

o  Text transformations (eg. bold, italic, outline)

o  Text scrolling

o  Window updating

o  Pattern filling

And general memory-to-memory block copying [1].

The heart of BitBlt was first  formally defined  by Newman  and Sproull in

their  description  of  the  function  RasterOp [2].  As defined, RasterOp

performed its block transfers on a bit-by-bit basis  and was  limited to a

small  subset  of  possible  source  and destination Boolean combinations.

Enhancements to  RasterOp such  as processing  bits in  parallel or intro-

ducing  a  halftone  pattern  into  the  transfer  were  literally left as

exercises for the reader.

In an effort to improve the functionality and performance of  the original

algorithm,  the   prescribed  enhancements   were  incorporated  into  the

definition of RasterOp and implemented in  hardware  as the  RasterOp Chip

[3].    However  the  RasterOp  Chip  lacked the two-dimensionality of the

original function and suffered from a performance bottleneck caused by the

loading and  reloading of  source, destination,  and halftone data (ie. it

could not DMA).

While efforts were being made to improve the performance of  RasterOp, the

formal definition  of RasterOp was further refined and became the basis of

the BitBlt copyLoop primitive in  the  Smalltalk-80  graphics  kernel [4].

Because of  its comprehensive  interface definition,  the BitBlt primitive

was inefficient and required special-case optimizations  that violated its

general-purpose  nature.    Clearly  a  hardware solution was necessary to

increase the performance of  the BitBlt  copyLoop without  sacrificing its

functionality.

The Atari ST BLiTTER is a hardware solution to the performance problems of

BitBlt.  The BLiTTER is a  DMA  device  that  implements  the  full BitBlt

copyLoop definition  with the addition  of a few minor extensions.  Single

word or multi-word increments and decrements are provided for transfers to

destinations in Atari ST video display memory.  A center mask, which would

otherwise be  a constant  all ones,  is   also provided  for an additional

Atari BLiTTER doc - page 1

level of texture.  The remainder of this document is directly based on the

original functional description of the Atari ST BLiTTER.

*** BIT-BLOCK TRANSFERS

As previously stated, a bit-block transfer can be described as a procedure

that  moves  bit-aligned  data  from  a  source  location to a destination

location through  a  given  logic  operation.    There  are  sixteen logic

combination rules  associated with  the merging  of source and destination

data.  Note that  this  set  contains  all  possible  combinations between

source and  destination.   The following  table contains  the valid BitBlt

combination rules:

LOGIC OPERATIONS

(~s&~d)|(~s&d)|(s&~d)|(s&d)

_______________________________________

|    |                                  |

MSB LSB   | OP | COMBINATION RULE                 |

|    |                                  |

0 0 0 0   | 0  | all zeros                        |

0 0 0 1   | 1  | source AND destination           |

0 0 1 0   | 2  | source AND NOT destination       |

0 0 1 1   | 3  | source                           |

0 1 0 0   | 4  | NOT source AND destination       |

0 1 0 1   | 5  | destination                      |

0 1 1 0   | 6  | source XOR destination           |

0 1 1 1   | 7  | source OR destination            |

1 0 0 0   | 8  | NOT source AND NOT destination   |

1 0 0 1   | 9  | NOT source XOR destination       |

1 0 1 0   | A  | NOT destination                  |

1 0 1 1   | B  | source OR NOT destination        |

1 1 0 0   | C  | NOT source                       |

1 1 0 1   | D  | NOT source OR destination        |

1 1 1 0   | E  | NOT source OR NOT destination    |

1 1 1 1   | F  | all ones                         |

|____|__________________________________|

Adjustments to block extents and  several  other  transfer  parameters are

determined prior  to the  invocation of  the actual block transfer.  These

adjustments and parameters include clipping, skew, end masks, and overlap.

Clipping.  The  source  and  destination  block  extents  are  adjusted to

conform  with  a  specified  clipping  rectangle.    Since both source and

destination blocks are of equal dimension, the destination block extent is

clipped to  the extent of the source block (or vice versa).  Note that the

block transfer need not be performed if the resultant extent is zero.

Skew.  The source-to-destination horizontal bit skew is calculated.

End Masks.  The left and  right partial  word masks  are determined.   The

masks are merged if the destination is one word in width.

Overlap.  The block locations are checked for possible overlap in order to

avoid the destruction of source data before it is transferred.

Atari BLiTTER doc - page 2

In  non-overlapping  transfers  the  source  block  scanning  direction is

inconsequential and  can by default be from upper left to lower right.  In

overlapping transfers the source scanning  direction  is  also  from upper

left to  lower right if the source-to-destination transfer direction is up

and/or to the left  (ie.  source  address  is  greater  than  or  equal to

destination address).   However,  if the overlapping source-to-destination

transfer direction is down and/or to the right (ie. source address is less

than  destination  address),  then  the  source data is scanned from lower

right to upper left.

After  the  transfer  parameters  are  determined  the  bit-block transfer

operation can  be invoked,  transferring source to destination through the

logic operation (HALFTONE and HOP will be described in the next section):

*** BIT-BLOCK TRANSFER

_________  _____________           ________________

|         ||             |         |                |

|  SOURCE ||  SOURCE     |         |  DESTINATION   |

|_________||_____________|         |________________|

|________________|<< SKEW |                  |

|                              |

______________      ___|____       ________________   |

|              |    |        |     |                |  |

|   HALFTONE   |----|  HOP   |-----|    LOGIC OP    |--|

|______________|    |________|     |________________|  |

|         |

____|____     |

|         |    |

| ENDMASK |____|

|_________|

|

_________|_________

|                   |

|  NEW DESTINATION  |

|___________________|

*** FUNCTIONAL DESCRIPTION

Please refer to the  bit-block transfer  diagram in  the previous section.

To understand  how the  components of a block transfer work, let's look at

the simplest possible transfer.  Take  the case  where we  wish to  fill a

block of  memory with either all zeros or all ones (OP = 0 or OP = F).  In

this case only the LOGIC OP block, which generates the ones or  zeros, and

the ENDMASK  block are  in the  data path.   If  the end mask contains all

ones, the BLiTTER will simply write one word after the other to  the dest-

ination address without ever reading the destination.

As  the  writes  take  place  the  destination  address  will  be adjusted

according to the values in  the  DESTINATION  X  INCREMENT,  DESTINATION Y

INCREMENT, X  COUNT, and  Y COUNT  registers.   These registers define the

size and shape of the  block  to  be  transferred.    The  X  and  Y COUNT

registers define  the size  of the  block.  The X COUNT register specifies

the number  of  word-size  writes  required  to  update  one  line  of the

destination.   The Y COUNT register specifies the number of these lines in

the  block.  The  DESTINATION  X  INCREMENT  register  is  a  signed  (2's

Atari BLiTTER doc - page 3

complement) 16-bit  quantity which  is added to the destination address to

calculate the address of the next destination word  of the  line.   On the

last write  of the  line the DESTINATION Y INCREMENT is added to calculate

the address of the first word of the next line.

The end mask determines  which bits  of the  destination word  will be up-

dated.   Bits of  the destination which correspond to ones in the end mask

will be updated.  Bits of the destination which correspond to zeros in the

end mask  will remain unchanged.  Note that if any bits of the destination

are to be left unchanged, a  read-modify-write is  required.   In order to

improve  performance  a  read  will  only  be performed if it is required.

There are three ENDMASK registers numbered 1 through 3.  ENDMASK 1 is used

only for the first write of the line.  ENDMASK 3 is used only for the last

write of the line.  ENDMASK 2 is used for all other writes.

Now let's consider a more complicated  case,  suppose  we  want  to  XOR a

destination block  with a  16 x  16 halftone  pattern.   First we load the

HALFTONE RAM with the halftone pattern.   Select  halftone only  using the

HOP register and select source XOR destination using the OP register.  The

LINE NUMBER register is used to specify which of the 16 words  of HALFTONE

RAM is  used for  the current  line.  This register will be incremented or

decremented at the end  of each  line according  to the  sign of  the DES-

TINATION Y  INCREMENT register.  Set the DESTINATION X and Y INCREMENT and

X and Y COUNT registers to the appropriate values and  start the transfer.

This same  procedure can be followed to do the combination using any logic

operation by simply changing the value in the OP register.   Similarly the

combination can  be performed using a source block instead of the HALFTONE

RAM or using the logical AND  of a  source block  and the  HALFTONE RAM by

changing the  value of  the HOP register.  A source block is the same size

as the  destination block  but may  have different  increments and address

defined by the SOURCE X and Y INCREMENT and SOURCE ADDRESS registers.

Finally, let's look at the case when the source and destination blocks are

not bit-aligned.  In this case we may  need to  read the  first two source

words into  the 32-bit source buffer and use the 16 bits that line up with

the appropriate bits of  the destination,  as specified  by the  SKEW reg-

ister.  When the next source word is read, the lower 16 bits of the source

buffer is transferred to the upper 16 bits  and the  lower is  replaced by

the new  data.   This process  is reversed  when the  source is being read

from the right to the left (SOURCE X INCREMENT negative).  Since there are

cases when  it may  be necessary for an extra source read  to be performed

at the beginning of each line to "prime" the source buffer and  cases when

it may  not be  necessary due  to the  choice of  end mask, a bit has been

provided which forces the extra read.   The  FXSR (aka.  pre-fetch) bit in

the SKEW register indicates, when set, that an extra source read should be

performed at the beginning of each  line  to  "prime"  the  source buffer.

Similarly the  NFSR (aka  post-flush) bit, when set, will prevent the last

source read of the  line.   This read  may not  be necessary  with certain

combinations of end masks and skews.  If the read is suppressed, the lower

to upper half buffer transfer still occurs.   Also in  this case,  a read-

modify-write cycle  is performed  on the destination for the last write of

each line regardless of the value of the corresponding ENDMASK register.

Atari BLiTTER doc - page 4

*** PROGRAMMING MODEL

The BLiTTER contains a set of registers that  specify bit-block addresses,

bit-block  alignments,  logic  and  halftone operations, and bus accesses.

The  register  set-up  time  remains  practically  constant  and  is large

relative to  small block transfers, whereas large bit-blocks are dominated

by the execution time of the transfer itself.

** REGISTER MAP

The following is a map of  the BLiTTER  programmable registers  (note that

all unused bits read back as zeros):

FF 8A00   |oooooooo||oooooooo|     HALFTONE RAM

FF 8A02   |oooooooo||oooooooo|

FF 8A04   |oooooooo||oooooooo|

:        ::        :

FF 8A1E   |oooooooo||oooooooo|

FF 8A20   |oooooooo||ooooooo-|     SOURCE X INCREMENT

FF 8A22   |oooooooo||ooooooo-|     SOURCE Y INCREMENT

FF 8A24   |--------||oooooooo|     SOURCE ADDRESS

FF 8A26   |oooooooo||ooooooo-|

FF 8A28   |oooooooo||oooooooo|     ENDMASK 1

FF 8A2A   |oooooooo||oooooooo|     ENDMASK 2

FF 8A2C   |oooooooo||oooooooo|     ENDMASK 3

FF 8A2E   |oooooooo||ooooooo-|     DESTINATION X INCREMENT

FF 8A30   |oooooooo||ooooooo-|     DESTINATION Y INCREMENT

FF 8A32   |--------||oooooooo|     DESTINATION ADDRESS

FF 8A34   |oooooooo||ooooooo-|

FF 8A36   |oooooooo||oooooooo|     X COUNT

FF 8A38   |oooooooo||oooooooo|     Y COUNT

FF 8A3A   |------oo|               HOP

FF 8A3B   |----oooo|               OP

FF 8A3C   |ooo-oooo|

||| |__|_____________ LINE NUMBER

|||__________________ SMUDGE

||__________________ HOG

|___________________ BUSY

FF 8A3D   |oo--oooo|

||  |__|_____________ SKEW

||___________________ NFSR

|____________________ FXSR

Atari BLiTTER doc - page 5

** BIT-BLOCK ADDRESSES

This  subsection  describes  registers  that  specify  bit-block  origins,

address increments, and extents.

SOURCE ADDRESS

This 23-bit register contains the  current  address  of  the  source field

(only word  addresses may  be specified).  It may be accessed using either

word or longword instructions.  The value read back is  always the address

of the  next word to be used in a source operation.  It will be updated by

the amounts specified in the SOURCE X INCREMENT and the SOURCE Y INCREMENT

registers as the transfer progresses.

SOURCE X INCREMENT

This is  a signed  15-bit register,  the least significant bit is ignored,

specifying the offset in bytes to the address of the  next source  word in

the  current  line.    This  value  will be sign-extended and added to the

SOURCE ADDRESS register at the end of a source word fetch, whenever  the X

COUNT register  does not  contain a value of one.  If the X COUNT register

is  loaded  with  a  value  of  one  this  register  is  not  used.   Byte

instructions can not be used to read or write this register.

SOURCE Y INCREMENT

This is  a signed  15-bit register,  the least significant bit is ignored,

specifying the offset in bytes to the address of the first source  word in

the next  line.   This value will be sign-extended and added to the SOURCE

ADDRESS register at the end of  the last  source word  fetch of  each line

(when the  X COUNT  register contains  a value  of one).   If  the X COUNT

register is loaded with a value of one this register  is used exclusively.

Byte instructions can not be used to read or write this register.

DESTINATION ADDRESS

This 23-bit register contains the current address of the destination field

(only word addresses may be specified).  It  may be  accessed using either

word or long-word instructions.  The value read back is always the address

of the next word  to be  modified in  the destination  field.   It will be

updated by  the amounts  specified in  the DESTINATION X INCREMENT and the

DESTINATION Y INCREMENT registers as the transfer progresses.

DESTINATION X INCREMENT

This is a signed 15-bit register,  the least  significant bit  is ignored,

specifying the offset in bytes to the address of the next destination word

in the current line.  This value will  be sign-extended  and added  to the

DESTINATION  ADDRESS  register  at  the  end  of a destination word write,

whenever the X COUNT register does not contain a value of one.   If  the X

COUNT register  is loaded  with a  value of one this register is not used.

Byte instructions can not be used to read or write this register.

DESTINATION Y INCREMENT

This is a signed 15-bit register,  the least  significant bit  is ignored,

specifying the  offset in  bytes to  the address  of the first destination

Atari BLiTTER doc - page 6

word in the next line.  This value will be sign-extended and  added to the

DESTINATION ADDRESS register at the end of the last destination word write

of each line (when the X COUNT register contains a value of one).   If the

X  COUNT  register  is  loaded  with  a value of one this register is used

exclusively.  Byte instructions cannot be used on this register.

X COUNT

This 16-bit register  specifies  the  number  of  words  contained  in one

destination line.   The  minimum number  is one  and the  maximum is 65536

designated by zero.  Byte instructions can not  be used  to read  or write

this register.   Reading  this register  returns the number of destination

words yet to be  written in  the current  line, NOT  necessarily the value

initially  written  to  the  register.    Each  time a destination word is

written the value will be decremented until it reaches zero, at which time

it will be returned to its initial value.

Y COUNT

This  16-bit  register  specifies  the  number of lines in the destination

field.  The minimum number is one and the maximum  is 65536  designated by

zero.   Byte instructions  can not be used to read or write this register.

Reading this register returns the number  of destination  lines yet  to be

written,  NOT  necessarily  the  value  initially written to the register.

Each time a destination line is  completed the  value will  be decremented

until it reaches zero, at which time the tranfer is complete.

** BIT-BLOCK ALIGNMENTS

This subsection  describes registers  that   specify  bit-block end masks,

source-to-destination skew, and source data fetching.

ENDMASK 1, 2, 3

These 16-bit registers are used to mask destination  writes.   Bits of the

destination word  which correspond to ones in the current ENDMASK register

will be modified.  Bits of the destination word which  correspond to zeros

in  the  current  ENDMASK    register  will remain unchanged.  The current

ENDMASK register is determined by position in the line.  ENDMASK 1 is used

only for  the first  write of a line.  ENDMASK 3 is used only for the last

write of a line.  ENDMASK 2 is used in all other cases.  In the  case of a

one word  line ENDMASK  1 is  used.   Byte instructions can not be used to

read or write these registers.

SKEW

The least significant four bits  of  the  byte-wide  register  at  FF 8A3D

specify the  source skew.   This is the amount the data in the source data

latch is shifted right before being  combined with  the halftone  mask and

destination data.

FXSR

FXSR stands  for Force  eXtra Source Read.  When this bit is set one extra

source read is performed at the  start  of  each  line  to  initialize the

remainder portion source data latch.

Atari BLiTTER doc - page 7

NFSR

NFSR stands  for No  Final Source  Read.   When this  bit is  set the last

source read of each line is not performed.  Note  that use  of this and/or

the FXSR  bit the  requires an  adjustment to  the SOURCE  Y INCREMENT and

SOURCE ADDRESS registers.

** LOGIC OPERATIONS

This subsection describes registers that specify the logic combinations of

source and destination bit-block data.

The  least  significant  four  bits  of  the byte-wide register at FF 8A3B

specify the source/destination combination rule according to the following

table:

_______________________________________

|    |                                  |

| OP | COMBINATION RULE                 |

|    |                                  |

| 0  | all zeros                        |

| 1  | source AND destination           |

| 2  | source AND NOT destination       |

| 3  | source                           |

| 4  | NOT source AND destination

tination       |

| 5  | destination                      |

| 6  | source XOR destination           |

| 7  | source OR destination            |

| 8  | NOT source AND NOT destination

on   |

| 9  | NOT source XOR destination       |

| A  | NOT destination                  |

| B  | source OR NOT destination        |

| C  | NOT source                       |

| D  | NOT source OR destination        |

| E  | NOT source OR NOT destination    |

| F  | all ones                         |

|____|__________________________________|

** HALFTONE OPERATIONS

This  subsection  describes  registers  that  specify the halftone pattern

memory, halftone word index, and combinations of source and halftone data.

HALFTONE RAM

This RAM holds a 16x16 halftone pattern mask.  Each word is  valid for one

line of the destination field and is repeated every 16 lines.  The current

word is pointed to by the  value  in  the  LINE  NUMBER  register.   These

registers  may   be  read,   but  can  not  be  accessed  using  byte-wide

instructions.

LINE NUMBER

The least significant four bits  of  the  byte-wide  register  at  FF 8A3C

specify the  current halftone  mask.   The current value times two plus FF

Atari BLiTTER doc - page 8

8A00 gives the address of  the  current  halftone  mask.    This  value is

incremented or  decremented at  the end of each line and will wrap through

zero.   The sign  of the  DESTINATION Y  INCREMENT determines  if the line

number is  incremented or decremented (increment if positive, decrement if

negative).

SMUDGE

The SMUDGE bit, when set, causes  the least  significant four  bits of the

skewed  source  data  to  be  used  as the address of the current halftone

pattern.  Note that the halftone operation is  still valid  when SMUDGE is

set.

HALFTONE OPERATIONS

The  least  significant  two  bits  of  the  byte-wide register at FF 8A3A

specify the  source/halftone combination  rule according  to the following

table:

_____________________________

|    |                        |

| HOP| COMBINATION RULE       |

|    |                        |

| 0  | all ones               |

| 1  | halftone               |

| 2  | source                 |

| 3  | source AND halftone    |

|____|________________________|

** BUS ACCESSES

This subsection  describes registers  that specify  bus access control and

BLiTTER start/status.

HOG

The HOG bit, when cleared, causes the processor  and the  blitter to share

the bus equally.  In this mode each will get 64 bus cycles while the other

is halted.  When set, the bit will cause the processor to  be halted until

the transfer  is complete.  In either case the BLiTTER will yield to other

DMA devices.  Bus arbitration may  allow the  processor to  execute one or

more  instructions  even  in  hog  mode.  Therefore, don't assume that the

instruction following the one  which sets  the BUSY  bit will  be executed

only  after  the  transfer  is  complete.    The BUSY bit may be polled to

achieve this kind of synchronization.

BUSY

The BUSY bit is set after all the other registers have been initialized to

begin the  transfer operation.   It  will remain set until the transfer is

complete.  The interrupt line  is  a  duplicate  of  this  bit.    See the

Programming Example for more details on how to use the BUSY bit.

Atari BLiTTER doc - page 9

Appendix A -- Programming Example

In order to maintain software compatibility with new or upgraded Atari STs

equipped with the BLiTTER, software developers need only follow guidelines

set forth  by the  VDI and "LINE A" documents.  Revised TOS ROMs will work

in concert with the  BLiTTER, enhancing  the performance  of many  VDI and

"LINE A"  operations.  This occurs in a manner transparent to an executing

program.  Thus no special actions need be taken to utilize the performance

advantages of the BLiTTER.

As a  rule of  thumb, never  make a  VDI or  "LINE A"  call from within an

interrupt context since unpredictable and potentially catastrophic results

will  occur   should  one  BLiTTER  operation  interrupt  another  BLiTTER

operation.

The following program has  not been  optimized and  is presented  here for

exemplary purposes only.

* (c) 1987 Atari Corporation

*    All Rights Reserved.

* BLiTTER BASE ADDRESS

BLiTTER   equ  $FF8A00

* BLiTTER REGISTER OFFSETS

Halftone  equ  0

Src_Xinc  equ  32

Src_Yinc  equ  34

Src_Addr  equ  36

Endmask1  equ  40

Endmask2  equ  42

Endmask3  equ  44

Dst_Xinc  equ  46

Dst_Yinc  equ  48

Dst_Addr  equ  50

X_Count   equ  54

Y_Count   equ  56

HOP       equ  58

OP        equ  59

Line_Num  equ  60

Skew      equ  61

* BLiTTER REGISTER FLAGS

fHOP_Source    equ  1

fHOP_Halftone  equ  0

fSkewFXSR      equ  7

fSkewNFSR      equ  6

fLineBusy      equ  7

fLineHog       equ  6

fLineSmudge    equ  5

Atari Blitter doc - Appendix A  page A.10

* BLiTTER REGISTER MASKS

mHOP_Source    equ  $02

mHOP_Halftone  equ  $01

mSkewFXSR      equ  $80

mSkewNFSR      equ  $40

mLineBusy      equ  $80

mLineHog       equ  $40

mLineSmudge    equ  $20

*         E n D m A s K   d A t A

*

* These tables are referenced by PC relative instructions.  Thus,

* the labels on these tables must remain within 128 bytes of the

* referencing instructions forever.  Amen.

*

* 0: Destination  1: Source   <<< Invert right end mask data >>>

lf_endmask:

dc.w $FFFF

rt_endmask:

dc.w $7FFF

dc.w $3FFF

dc.w $1FFF

dc.w $0FFF

dc.w $07FF

dc.w $03FF

dc.w $01FF

dc.w $00FF

dc.w $007F

dc.w $003F

dc.w $001F

dc.w $000F

dc.w $0007

dc.w $0003

dc.w $0001

dc.w $0000

* TiTLE:  BLiT_iT

*

* PuRPoSE:

*    Transfer a rectangular block of pixels located at an

*    arbitrary X,Y position in the source memory form to

*    another arbitrary X,Y position in the destination memory

*    form using replace mode (boolean operator 3).

*    The source and destination rectangles should not overlap.

*

* iN:

*    a4   pointer to 34 byte input parameter block

*

* Note: This routine must be executed in supervisor mode as

*    access is made to hardware registers in the protected region

*    of the memory map.

*

Atari Blitter doc - Appendix A  page A.11

*

*    I n p u t   p a r a m e t e r   b l o c k   o f f s e t s

SRC_FORM  equ  0    ; Base address of source memory form .l

SRC_NXWD  equ  4    ; Offset between words in source plane .w

SRC_NXLN  equ  6    ; Source form width .w

SRC_NXPL  equ  8    ; Offset between source planes .w

SRC_XMIN  equ  10   ; Source blt rectangle minimum X .w

SRC_YMIN  equ  12   ; Source blt rectangle minimum Y .w

DST_FORM  equ  14   ; Base address of destination memory form .l

DST_NXWD  equ  18   ; Offset between words in destination plane.w

DST_NXLN  equ  20   ; Destination form width .w

DST_NXPL  equ  22   ; Offset between destination planes .w

DST_XMIN  equ  24   ; Destination blt rectangle minimum X .w

DST_YMIN  equ  26   ; Destination blt rectangle minimum Y .w

WIDTH     equ  28   ; Width of blt rectangle .w

HEIGHT    equ  30   ; Height of blt rectangle .w

PLANES    equ  32   ; Number of planes to blt .w

BLiT_iT:

lea  BLiTTER,a5          ; a5-> BLiTTER register block

*

* Calculate Xmax coordinates from Xmin coordinates and width

*

move.w    WIDTH(a4),d6

subq.w    #1,d6               ; d6<- width-1

move.w    SRC_XMIN(a4),d0     ; d0<- src Xmin

move.w    d0,d1

add.w     d6,d1               ; d1<- src Xmax=src Xmin+width-1

move.w    DST_XMIN(a4),d2     ; d2<- dst Xmin

move.w    d2,d3

add.w     d6,d3               ; d3<- dst Xmax=dstXmin+width-1

*

* Endmasks are derived from source Xmin mod 16 and source Xmax

*    mod 16

*

moveq.l   #$0F,d6   ; d6<- mod 16 mask

move.w    d2,d4          ; d4<- DST_XMIN

and.w     d6,d4          ; d4<- DST_XMIN mod 16

add.w     d4,d4          ; d4<- offset into left end mask tbl

move.w    lf_endmask(pc,d4.w),d4        ; d4<- left endmask

move.w    d3,d5          ; d5<- DST_XMAX

and.w     d6,d5          ; d5<- DST_XMAX mod 16

add.w     d5,d5          ; d5<- offset into right end mask tbl

move.w    rt_endmask(pc,d5.w),d5   ; d5<-inverted right end mask

not.w     d5                       ; d5<- right end mask

Atari Blitter doc - Appendix A  page A.12

*

* Skew value is (destination Xmin mod 16 - source Xmin mod 16)

* && 0x000F.  Three discriminators are used to determine the

* states of FXSR and NFSR flags:

*

*    bit 0     0: Source Xmin mod 16 =< Destination Xmin mod 16

*              1: Source Xmin mod 16 >  Destination Xmin mod 16

*

*    bit 1     0: SrcXmax/16-SrcXmin/16 <> DstXmax/16-DstXmin/16

*                   Source span              Destination span

*              1: SrcXmax/16-SrcXmin/16 == DstXmax/16-DstXmin/16

*

*    bit 2     0: multiple word Destination span

*         1: single word Destination span

*

*    These flags form an offset into a skew flag table yielding

*    correct FXSR and NFSR flag states for the given source and

*    destination alignments

*

move.w    d2,d7     ; d7<- Dst Xmin

and.w     d6,d7     ; d7<- Dst Xmin mod16

and.w     d0,d6     ; d6<- Src Xmin mod16

sub.w     d6,d7     ; d7<- Dst Xmin mod16-Src Xmin mod16

*                        ; if Sx&F > Dx&F then cy:1 else cy:0

clr.w     d6        ; d6<- initial skew flag table index

addx.w    d6,d6     ; d6[bit0]<- intraword alignment flag

lsr.w     #4,d0     ; d0<- word offset to src Xmin

lsr.w     #4,d1     ; d1<- word offset to src Xmax

sub.w     d0,d1     ; d1<- Src span - 1

lsr.w     #4,d2     ; d2<- word offset to dst Xmin

lsr.w     #4,d3     ; d3<- word offset to dst Xmax

sub.w     d2,d3     ; d3<- Dst span - 1

bne       set_endmasks   ; 2nd discriminator is one word dst

* When destination spans a single word, both end masks are merged

* into Endmask1.  The other end masks will be ignored by the BLiTTER

and.w     d5,d4          ; d4<- single word end mask

addq.w    #4,d6          ; d6[bit2]:1 => single word dst

set_endmasks:

move.w    d4,Endmask1(a5)     ; left end mask

move.w    #$FFFF,Endmask2(a5) ; center end mask

move.w    d5,Endmask3(a5)     ; right end mask

cmp.w     d1,d3          ; the last discriminator is the

bne       set_count      ; equality of src and dst spans

addq.w    #2,d6          ; d6[bit1]:1 => equal spans

Atari Blitter doc - Appendix A  page A.13

set_count:

move.w    d3,d4

addq.w    #1,d4          ; d4<- number of words in dst line

move.w    d4,X_Count(a5) ; set value in BLiTTER

* Calculate Source starting address:

*

*   Source Form address              +

*  (Source Ymin * Source Form Width) +

* ((Source Xmin/16) * Source Xinc)

move.l    SRC_FORM(a4),a0     ; a0-> start of Src form

move.w    SRC_YMIN(a4),d4     ; d4<- offset in lines to Src Ymin

move.w    SRC_NXLN(a4),d5     ; d5<- length of Src form line

mulu      d5,d4               ; d4<- byte offset to (0, Ymin)

add.l     d4,a0               ; a0-> (0, Ymin)

move.w    SRC_NXWD(a4),d4;    d4<- offset between consecutive

move.w    d4,Src_Xinc(a5)     ;      words in Src plane

mulu      d4,d0          ; d0<- offset to word containing Xmin

add.l     d0,a0          ; a0-> 1st src word (Xmin, Ymin)

* Src_Yinc is the offset in bytes from the last word of one Source

* line to the first word of the next Source line

mulu      d4,d1               ; d1<- width of src line in bytes

sub.w     d1,d5               ; d5<- value added to ptr at end

move.w    d5,Src_Yinc(a5)     ; of line to reach start of next

* Calculate Destination starting address

move.l    DST_FORM(a4),a1     ; a1-> start of dst form

move.w    DST_YMIN(a4),d4     ; d4<- offset in lines to dst Ymin

move.w    DST_NXLN(a4),d5     ; d5<- width of dst form

mulu      d5,d4     ; d4<- byte offset to (0, Ymin)

add.l     d4,a1     ; a1-> dst (0, Ymin)

move.w    DST_NXWD(a4),d4     ; d4<- offset between consecutive

move.w    d4,Dst_Xinc(a5)     ;  words in dst plane

mulu      d4,d2               ; d2<- DST_NXWD * (DST_XMIN/16)

add.l     d2,a1               ; a1-> 1st dst word (Xmin, Ymin)

* Calculate Destination Yinc

mulu      d4,d3               ; d3<- width of dst line - DST_NXWD

sub.w     d3,d5               ; d5<- value added to dst ptr at

move.w    d5,Dst_Yinc(a5)     ;  end of line to reach next line

* The low nibble of the difference in Source and Destination alignment

* is the skew value.  Use the skew flag index to reference FXSR and

* NFSR states in skew flag table.

and.b     #$0F,d7                  ; d7<- isolated skew count

or.b      skew_flags(pc,d6.w),d7 ; d7<- necessary flags and skew

Atari Blitter doc - Appendix A  page A.14

move.b    d7,Skew(a5)              ; load Skew register

move.b    #mHOP_Source,HOP(a5)     ; set HOP to source only

move.b    #3,OP(a5)           ; set OP to "replace" mode

lea       Line_Num(a5),a2     ; fast refer to Line_Num register

move.b    #fLineBusy,d2       ; fast refer to LineBusy flag

move.w    PLANES(a4),d7       ; d7 <- plane counter

bra  begin

*    T h e   s e t t i n g   o f   s k e w   f l a g s

*

*

* QUALIFIERS   ACTIONS           BITBLT DIRECTION: LEFT -> RIGHT

*

* equal Sx&F>

* spans Dx&F FXSR NFSR

*

* 0     0     0    1 |..ssssssssssssss|ssssssssssssss..|

*

|......dddddddddd|dddddddddddddddd|dd..............|

*

*    0                     1                      1                  0

|......ssssssssss|ssssssssssssssss|ss..............|

*                    |..dddddddddddddd|dddddddddddddd..|

*

* 1     0     0    0 |..ssssssssssssss|ssssssssssssss..|

*                    |...ddddddddddddd|ddddddddddddddd.|

*

* 1     1     1    1 |...sssssssssssss|sssssssssssssss.|

*                    |..dddddddddddddd|dddddddddddddd..|

skew_flags:

dc.b mSkewNFSR           ; Source span < Destination span

dc.b mSkewFXSR           ; Source span > Destination span

dc.b 0                   ; Spans equal Shift Source right

dc.b mSkewNFSR+mSkewFXSR ; Spans equal Shift Source left

* When Destination span is but a single word ...

dc.b 0         ; Implies a Source span of no words

dc.b mSkewFXSR ; Source span of two words

dc.b 0         ; Skew flags aren't set if Source and

dc.b 0         ; Destination spans are both one word

next_plane:

move.l    a0,Src_Addr(a5)     ; load Source pointer to this plane

move.l    a1,Dst_Addr(a5)     ; load Dest ptr to this plane

move.w    HEIGHT(a4),Y_Count(a5)   ; load the line count

move.b    #mLineBusy,(a2)     ; <<< start the BLiTTER >>>

add.w     SRC_NXPL(a4),a0     ; a0-> start of next src plane

add.w     DST_NXPL(a4),a1     ; a1-> start of next dst plane

Atari Blitter doc - Appendix A  page A.15

* The BLiTTER is usually operated with the HOG flag cleared.

* In this mode the BLiTTER and the ST's cpu share the bus equally,

* each taking 64 bus cycles while the other is halted.  This mode

* allows interrupts to be fielded by the cpu while an extensive

* BitBlt is being processed by the BLiTTER.  There is a drawback in

* that BitBlts in this shared mode may take twice as long as BitBlts

* executed in hog mode.  Ninety percent of hog mode performance is

* achieved while retaining robust interrupt handling via a method

* of prematurely restarting the BLiTTER.  When control is returned

* to the cpu by the BLiTTER, the cpu immediately resets the BUSY

* flag, restarting the BLiTTER after just 7 bus cycles rather than

* after the usual 64 cycles.  Interrupts pending will be serviced

* before the restart code regains control.  If the BUSY flag is

* reset when the Y_Count is zero, the flag will remain clear

* indicating BLiTTER completion and the BLiTTER won't be restarted.

*

* (Interrupt service routines may explicitly halt the BLiTTER

* during execution time critical sections by clearing the BUSY flag.

* The original BUSY flag state must be restored however, before

* termination of the interrupt service routine.)

restart:

bset.b    d2,(a2)        ; Restart BLiTTER and test the BUSY

nop                      ; flag state.  The "nop" is executed

bne  restart             ; prior to the BLiTTER restarting.

*                             ; Quit if the BUSY flag was clear.

begin:

dbra d7,next_plane

rts

Atari Blitter doc - Appendix A  page A.16

Appendix B -- References

[1]  Rob Pike, Leo  Guibas,  and  Dan  Ingalls, 'SIGGRAPH'84

Course Notes:  Bitmap Graphics', AT&T Bell Laboratories

1984.

[2]  William  Newman  and  Robert  Sproull,  'Principles  of

Interactive   Computer   Graphics',  McGraw-Hill  1979,

Chapter 18.

[3]  John Atwood, '16160 RasterOp Chip Data  Sheet', Silicon

Compilers   1984.      See   also   'VL16160   RasterOp

Graphics/Boolean Operation ALU', VLSI Technology 1986.

[4]  Adele Goldberg and David  Robson,  'Smalltalk-80:   The

Language  and its Implementation', Addison-Wesley 1983,

Chapter 18.

Atari Blitter doc - Appendix B  B.17