02200000000801

1

2

9[...............................................................]0

THE SECRET OF THE FISH by Udo of TEX

On our visit to Holland (see the article in the ST NEWS Volume  3

Issue  2) Richard urged me to write an article about the  'fish'.

If you don't know anything about the 'fish' get your hand on  our

"B.I.G.  Demo"  and view our second Psych-O-Screen.  Perhaps  you

will  see a fish in the bottom right corner of the  screen  where

usually the border appears. If you can't see any fish you are one

of  the few poor guys who have an ATARI SC1224  monitor.  But  at

least  you must see some 'air bubbles'.  The fish was painted  to

explain the presence of the air bubbles, and the air bubbles were

only painted to hide a useful error of the video chip. And now we

have  reached the theme of this article:  Fooling the video  chip

not to make the lower border.

1) How to switch off the lower border:

To  irritate  the  video shifter you  must  switch  the  video

frequency  from  50 to 60 Hz in the last line of  the  screen.

This  will irritate the video chip so it forgets to  draw  the

last  2  Bytes  of the screen memory (this  causes  the  right

border   to  start  16  pixels  earlier  and  to   hide   this

malfunction the air bubbles were painted in the screen). After

displaying  200 lines the MMU is checking if 32000  Byte  have

been  displayed.  But due to the 2 Byte error there are  still

some left.  If switch back to 50 Hz in the left border of  the

next  line,  the MMU will continue sending memory data to  the

shifter  without stopping.  It depends on the monitor and  the

ST's  video  transistor but in the best case you will  see  64

more  lines  of video data till the monitor starts  his  blank

gap.  I  will now explain a routine doing this,  which  is  my

modified RIP (raster interrupt program) from ST NEWS Volume  2

Issue 6.

A complete source is somewhere on this disk (in the "PROGRAMS"

folder, ED).

First  we need a level 4 routine which is called at every  VBL

(that means 50 times per second).

new4:

clr.b  $fffffa1b.w              ;stop timer

move.b #199,$fffffa21.w         ;set data register

move.b #8,$fffffa1b.w           ;start timer

new4b:

jmp    $000000

This  rout sets the timer c data register to 199 so  our  main

program  will  be interrupted to handle  the  border  removing

properly.

Now the Timer C handler:

newtb:

clr.b   $fffffa1b.w             ;timer stop

movem.l d0/d3-d7/a0-a6,-(sp)

move.w  #$fa21,a4

move.b  #180,(a4)               ;set counter

move.b  #8,-6(a4)

This  will  install  a new Timer C data so we  are  no  longer

disturbed by the Timer C, but it is still running.

moveq   #0,d0                   ;wait till middle of line

mulu    d0,d0

mulu    d0,d0

move.b  #0,$ffff820a.w          ;on to 60 Hz

We  now  spent a lot of time multiplying nonsense to  let  the

electron  beam  run  in  the middle of  the  screen  and  then

switched to 60 Hz.

move.b  (a4),d0                 ;wait for right border

wait:

cmp.b   (a4),d0

beq     wait

As the Timer C is still running,  we use this loop to wait for

the right border.

moveq   #0,d0                   ;wait on left border

mulu    d0,d0

mulu    d0,d0

move.b  #2,$ffff820a.w          ;back to 50 Hz

With this commands we spent some time waiting for the HBL then

we switched back to 50 Hz.

movem.l (sp)+,d0/d3-d7/a0-a6

bclr    #0,$fffffa0f.w          ;end of interrupt

rte

Now we have ended the Interruptrout.

This was my explanation of the border removing for those who  had

no time or no patience to read it in the scroller of the  "B.I.G.

Demo".  The only disadvantage of the program is that in the worst

case  you can see only 8 lines of the border graphic (on  a  very

bad  SC1224  monitor).  But  I  hope you  can  use  this  program

efficiently.

I you have any questions or suggestions write to:

TEX

P.O. Box 1322

D-6702 Bad Drkheim

West Germany

Udo (-me-) of TEX