DBGRID Version 1.0

A GRID SQUARE AND DISTANCE/BEARING CALCULATOR PROGRAM FOR RADIO AMATEURS

Copyright (c) 1998 by Earl W. Cunningham, K6SE

All Rights Reserved

INTRODUCTION

There are a number of amateur radio awards and contests based on the

Maidenhead Locator System, a system where the surface of the Earth is

divided into areas known as "grid squares".  Membership in the ARRL's VHF/UHF

Century Club (VUCC) is an esteemed award for those amateurs who can confirm

contact with 100 or more 1-degree by 2-degree grid squares above 50 mHz.

Rather than the traditional "599" or "59" report exchange used in most

contests, some contests require that you exchange a 4- or 6-character grid

square with the station contacted.  Furthermore, the point scoring in some of

these contests is based on the distance between grid squares.

The Maidenhead Locater System was conceived by a group of European VHF radio

amateurs who met in Maidenhead, England in 1980.  The system was devised as a

method to simplify identification a transmitter location anywhere on the

surface of the Earth.

A grid square identifier consists of four characters -- two uppercase letters

followed by two numerals -- such as "DM04".  The letters identify a "field"

which measures 10 degrees in latitude by 20 degrees in longitude.  324 fields

("AA" to "RR"), cover the Earth's entire surface.  Each field is further

divided into 100 "squares" numbered 00 to 99.  Each square measures 1 degree in

latitude by 2 degrees in longitude.  The four-character identifier is known as

a "grid square".

Each grid square can be further subdivided into 576 "subsquares" measuring

2.5 minutes (1/24 degree) in latitude by 5 minutes (1/12 degree) in longitude

by adding two lowercase letters ("aa" to "xx") to the end of the grid square

identifer.  A six-character grid square identifier, such as "DM04vp" describes

an area about 9 km (5.6 miles) by 4.5 km (2.8 miles) at the Earth's equator.

DBGRID is a program which, when given the latitude and longitude of any

location, will calculate the six-character grid square of that location.  It

will also, when given a grid square (or subsquare) identifer, show you the

latitude and longitude of the grid square (or subsquare).  It will also very

accurately calculate the distance and beam heading between any two grid squares

(or subsquares) or between any two latitude/longitude locations.  The name

"DBGRID" is derived from Distance/Bearing/Grid, which describes its features.

There are a number of programs available which do the same job as DBGRID,

however all of them appear to have shortcomings.  Some calculate invalid grid

squares (such as "SS99yy") in the polar regions.  Some will allow invalid

latitudes, longitudes and grid squares to be entered.  Most calculate distances

from corners of grid squares or subsquares rather than from their centers. Most

are inaccurate in their distance and bearing calculations because they use

standard spherical trigonometry while, in fact, the Earth is not a perfect

sphere.  Some will "crash" when trying to compute zero or antipodal distances

or distances between locations having the same longitude.  DBGRID has none of

these shortcomings.

DBGRID accurately calculates grid square identifiers, even at the north and

south poles.  It will not allow any invalid parameters to be entered.  It

calculates distances between grid squares and subsquares from the centers of

those areas rather than from their corners.  Best of all, DBGRID is extremely

accurate in its distance and bearing calculations because it does not neglect

the fact that the Earth is not a perfect sphere, but is ellipsoidal.

Only two other programs are available that accurately calculate distances

between two points on the surface of the ellipsoidal Earth.  "BD" is currently

used as the "standard" by the ARRL for calculating long distance/record-

breaking QSOs on VHF/UHF/microwave frequencies.  It is available for download

from the ARRL BBS.  BD, however, appears to have a slight error of up to 50

meters or more over large distances.  It is assumed that this is because BD

uses older data than is currently available for the Earth's dimensions.

The other accurate program, "INVERSE", is used by the United States National

Geodetic System (NGS).  It is available for download from their Web site.

INVERSE offers the user the capability of "plugging in" different values for

the dimensions of the Earth, so it will never be outdated.  However, it is not

designed for amateur radio use and will not do any grid square calculations.

DBGRID uses Earth dimensions published by the World Geodetic System known as

"WGS 84".  WGS 84 was first published in 1984, but it has been updated several

times since then.  DBGRID uses the most recent data (revision G873) effective

29 September 1996, which is:

Equatorial radius: 6,378,137.0000 meters

Polar radius: 6,356,752.3142 meters

Flattening: 1/298.2572235630

DBGRID has been compared on numerous paths with the U.S. government program

(INVERSE) and distance and bearing results agree exactly in all cases.  Even

though the algorithms used by the two programs differ in appearance they

obviously agree mathematically.  DBGRID uses the algorithm developed by P.D.

Thomas of the NRL and published by the U.S. Naval Oceanographic Office in 1970.

His FORTRAN code, translated into Pascal, is used as the basis for distance and

bearing calculations in DBGRID (DBGRID was written using Turbo Pascal).

USING DBGRID

DBGRID.EXE is an MS-DOS program and will run on any IBM/compatible computer.

It will also run on those computers in a "window" from Windows NT/95/98.  It

will use your computer's math co-processor for mathematical calculations, but

if you have an older machine without a math co-processor, the program will

emulate an 80x87 and still do highly accurate calculations.

Before you run DBGRID.EXE, try to determine the latitude and longitude of

your QTH as accurately as possible.  There are several ways to do this:

1) Use a Global Positioning System (GPS) receiver.

2) Use a U.S. Geological Survey (USGS) map.

3) Your local library or town hall may have the information.

The first time that you run DBGRID.EXE you will be presented with a prompt on

the screen which says:

"Your grid square needs to be calculated to continue"

The program will then ask you to select the format you desire to use to enter

your latitude and longitude coordinates, i.e., degrees/minutes/seconds format

or decimal degrees.  Choose the desired format and enter your coordinates when

prompted.  The program will calculate your six-character grid square and store

it, along with your latitude and longitude, in a small disk file named "DB.DAT"

in the same directory as DBGRID.EXE.  Once the data is stored, the program will

not ask for it the next time you run DBGRID.  The stored data may be changed at

any time in the future, if you so desire, by selecting option number 8 from

DBGRID's main menu.  This menu will appear as soon as you enter the requested

data, as follows:

GRID SQUARE/DISTANCE BEARING CALCULATOR

1) Distance & Bearing from Your Grid Square to Another Grid Square

2) Distance & Bearing from Your Lat/Long to Another Lat/Long

3) Distance & Bearing Between Any Two Grid Squares

4) Distance & Bearing Between Any Two Latitude/Longitude Points

5) Find a Grid Square from Latitude & Longitude

6) Find the Latitude & Longitude of a Grid Square

7) Change Lat/Long Format (it is now set for Decimal Degrees)

8) Set Your Stored Location

X) Exit Program

Select an Option (1-8 or X):

Option number 1 computes the distance and bearing from your stored 4- or

6-character grid square to any other 4- or 6-character grid square.

Option number 2 computes the distance and bearing from your stored latitude

and longitude to any other latitude and longitude.

Option number 3 computes the distance and bearing between any two grid

squares.  Either 4-character or 6-character grid identifiers may be used for

either end of the path.

Option number 4 computes the distance and bearing between any two latitude/

longitude points on the Earth's surface.

Option number 5 will tell you the 6-character grid square identifier for a

latitude and longitude that you provide.

Option number 6 will tell you the latitude and longitude of any 4-character

or 6-character grid square.

Option number 7 sets the data entry and display of latitudes and longituides

to either degrees/minutes/seconds format or decimal degrees format.  The

program defaults to decimal degrees on startup.  When option number 7 is

selected, "it is now set for Decimal Degrees" will toggle to "it is now set for

Deg/Min/Sec".  Use this option to set the desired format.

Option number 8 may be used if desired to change your stored latitude,

longitude and grid square.

Pressing "X" will exit the program and return you to DOS.

For menu option numbers 1, 3 and 6, the convention for grid square

identifiers is that the field ID (first two characters) should be uppercase

letters and the subsquare ID (last two characters in a 6-character grid) should

be lowercase letters.  When entering a grid square identifier, either uppercase

or lowercase characters may be typed.  The program will convert the first two

characters to uppercase and (if six characters) the last two to lowercase.  The

program ensures that no invalid grid square identifier can be entered.  Only 4-

or 6-character IDs are accepted.  If you try to enter an invalid character a

beep will sound.  The backspace key may be used to correct entry errors and you

may abort at any time while entering grid square identifiers by pressing the

<Esc> key.

Distance calculations for grid squares is always the distance between the

centers of the 4- or 6-character grid squares.

Prior to selecting menu option numbers 2, 4, 5, or 8, check menu option

number 7 to see if decimal degrees or degrees/minutes/seconds format is the

current setting.  You might want to toggle to the opposite setting  prior to

proceeding.

To enter a latitude which is south of the equator, precede it with a minus

(-) sign.  A plus (+) sign may be used for northern latitudes, but that is not

really necessary.

To enter an east longitude, precede it with a minus (-) sign.  A plus (+)

sign may be used for west longitudes, but it is not necessary.  It was decided

to use minus for east and plus for west in DBGRID because it was found that

most other programs of this type use that convention, including the government

program, INVERSE.

DBGRID ensures that invalid latitudes and longitudes cannot be entered, i.e.,

if you try to enter a latitude value greater than 90 degrees or a longitude

value greater than 180 degrees, it will be ignored by the program.  When in the

degrees/minutes/seconds mode, the program will ignore any numeral greater than

"5" for the first digit of minutes or seconds -- "59" is the highest number it

is possible to enter in these cases.  A beep will sound if the key for any

invalid character is pressed.

Small numbers may be preceded by zeroes, if desired -- i.e., 1 degree

longitude may be entered as "1", "01", or "001".

Pressing the <Enter> key without entering a latitude or longitude will

automatically set that data to zero.  The backspace key may be used to correct

any data entry errors prior to pressing the <Enter> key for that line.

Pressing the <Esc> key will abort the operation at any point and return you

to the main menu.

DBGRID is distributed as freeware for non-commercial use only.  If you have

any problems running the program or detect any "bugs" please report them to the

author at:

E-mail:

k6se@juno.com

Postal mail:

Earl W. Cunningham, K6SE

41041 27th St. West

Palmdale, CA  93551

USA