Chapter 1

Introduction

1.1  What is TNCTERM?

TNCTERM is a complete TNC operating environment.  It consists of a set

of programs that provide the amateur radio operator with automated ter-

minal node control, logging, and message generation.  TNC.EXE, the pri-

mary TNC program, is a 'C' program which provides the user with a menu

oriented interface to the TNC.  It also has links to the other programs

which provide the operator with a full set of data base functions.

. TNCED.EXE     - full screen editor

. LOGBOOK.EXE   - ISAM based logbook database program

. COUNTRIES.EXE - callsign/country lookup program with great circle

distance & bearing computation

. HAMGRAM.EXE   - NTS message generation and database

1.2 Flexibility

-  TNC may be configured to function with either the COM1 or COM2

serial ports at speeds from 300 to 9600 baud.

-  Fully configurable color selection, terminal screen layout, and

file location are just a few of the adjustable parameters.

-  Provides all of the common file handling functions of the popular

communications programs including ASCII file transmit and re-

ceive, screen buffer captures,

-  Supports all modes, CW, AMTOR, RTTY, PACKET (VHF/HF).

1.3 Ease of Use

-  Pull down menus provide complete access to all internal and

external functions.

-  Fully scrollable receive buffer of 800 lines.  Receive buffer

may be written to disk file of users choice.

-  Receive function operates independent of transmit buffer.  Create

outgoing text while the other op' is transmitting.

-  Removes operator from the cryptic TNC protocol language.  All

change of mode transmit/receive, etc are handled by TNCTERM.

Operator makes selections by pulldown menu selection.

1.4 Distribution of TNCTERM

TNCTERM is commercial software.  Clermont Computer Consultants retains

sole distibution and resale rights to all programs contained on the dis-

tribution disk including:

TNC.EXE

TNCED.EXE

TNCEDINS.EXE

HAMLOG.EXE

HAMGRAM.EXE

COUNTRY.EXE.

1.5  The most recent version of this software is always available from

the author at:

Clermont Computer Consultants

122 N. Ravenwood Drive

Cape May Court House, NJ  08210

(609) 624 0076.

The cost for the software on either 5 1/4" or 3 1/2" diskette is

$35.00.  Please send check or money order drawn on U.S. dollars with

your order.

1.6  Disclaimer

W1HKJ, David Freese and Clermont Computer Consultants make no  warran-

ty, expressed or implied, concerning this program's merchantability or

fitness for any particular use.  In no event is David Freese or  Cler-

mont Computer Consultants liable to you for any damages resulting from

your use of the program.

Chapter 2

Getting Started with TNCTERM

2.1 System Requirements

TNCTERM has the following minimal requirements:

-  IBM  PC, XT, AT, PS-2 or compatible with at least 320 K bytes  of

memory.

-  DOS 2.0 or later

-  Floppy disk drive, 360K, 1.2M, 720K, 1.44M, or a hard drive.

-  Either COM1 or COM2 port available for serial communications.

-  the  Kantronics All Mode Terminal Node Controller, KAM or

the  MFJ Multi-Mode Data Controller, MFJ-1278 or

the  PK232

2.2 TNCTERM installation

TNCTERM is distributed with the following files:

TNC.EXE        Executable file which directly interfaces to the

Terminal Node Controller.

TNCTERM.KAM    Configuration file for using TNCTERM with the

Kantronics All Mode TNC.

TNCTERM.MFJ    Configuration file for using TNCTERM with the

MFJ-1278 Multi-Mode Data Controller

TNCTERM.AEA    Configuration file for using TNCTERM with the

AEA PK-232 TNC.

COUNTRY.EXE    Stand alone prefix/country lookup program

COUNTRY.LST    ASCII text file containing the Prefix/Country list

as found in the 1991 Call Book.  This file is used

by COUNTRY.EXE.

TNCED.EXE      Full screen editor, colors and keyboard reconfig-

urable.  Edits file up to maximum of available

computer memory.

TNCEDINS.EXE   Installation program for TNCED.EXE.  Allows color,

and keyboard reconfiguration.

LOGBOOK.EXE    ISAM based database for keeping amateur radio log.

Includes both date and callsign indexing of the

database.

HAMGRAM.EXE    dBase-III compatible database of National Traffic

System messages.  Builds NTS messages with proper

pre and postambles, and word count.

2.3  First-time Setup of TNCTERM

HARD DISK   I suggest you create a directory called \HAM.  Copy all of

the files to this directory and include the directory name in your

path definition.

FLOPPY DISK Simply make a working copy of the distribution disk to use

as a working disk.  If you have a two floppy drive system you may

modify the configuration file to direct all log and other data files

to the second disk.  See the configuration file specification below.

2.3.1 CONFIG.SYS - no changes to your config.sys file are necessary.

2.3.2 AUTOEXEC.BAT - Add the directory name of your TNCTERM files to

your path statement.

2.3.3  Before  executing the program, you should prepare the TNCTERM.CFG

file for your specific installation.  Three preconfigured files are

included with the distribution disk, TNCTERM.KAM, TNCTERM.MFJ,

TNCTERM.AEA.  Copy the appropriate file to a new file called TNCTERM.CFG.

2.3.4  The TNCTERM.DEF file contains several user configurable parameters

and some others that you should only change if you need to adapt the

program to some other TNC.  A line which begins with a semicolon is

ignored by TNC.EXE and can be used for comments in the configuration file.

Each configurable parameter is specified by a line such as:

TNCPORT=1

The TNC.EXE program will search for the string 'TNCPORT=' and expects

to find the parameter immediately following the equals sign.  Do not put

spaces around the '=' sign.

RS-232 serial port definitions:

TNCPORT=1                1 = COM1, 2 = COM2

TNCPARITY=0              0 = NONE, 1 = ODD, 2 = EVEN

TNCSTOPBITS=1            1 or 2 STOPBITS

TNCWORDLEN=8             7 or 8 bit WORDLEN

TNCBAUD=9600             300,600,1200,2400,4800,9600

Child process definitions:

TNCLOGPROG=hamlog.exe

TNCLOGBOOK=hamlog

TNCEDIT=tnced.exe

TNCCOUNTRIES=country.exe

TNCCOUNTRYLIST=country.lst

TNCNTSPROG=hamgram.exe

If you have a logbook program that you prefer to use, it may be specified

on the TNCLOGPROG= line.  If that program expects to be passed a parameter

it may be specified on the TNCLOGBOOK= line.  If no drive or path is given

then TNCTERM will expect to find them in the default directory.

2.4 Initial Set Up of TNCTERM.

Turn the TNC and your computer off.  Connect the two using the config-

ured  communications  port.  Connect the TNC to your  HF/VHF  gear  in

accordance  with  the TNC instruction manual.  I  recommend  beginning

with  the VHF/UHF equipment.

Turn your computer on and boot it using DOS 3.0 or later.

Change to the drive and directory containing TNCTERM programs and execute

TNCTERM.EXE.

2.5 FIRST TIME USE OF KAM OR MFJ-1278

Turn  the  TNC on and wait for a readable message to  appear  on  your

screen.  Immediately press the asterisk '*'.  The TNC will now be  set

to the same baud as the computer.  Follow the instructions

for your TNC to make this a permanent setup.

2.5.1 TIME ZONE Environment Variable

Enter the following either in your AUTOEXEC.BAT file or from the DOS

command prompt:

SET TIMEZONE=4      ( a range of -11 to +11 allowed)

If the TIMEZONE variable is missing from the environment or its value is

0 then no correction is made to GMT.

Chapter 3

Main Display & Operating Modes

3.1  Main Display

3.1.1   The  main  display consists of a transmit  window,  a  receive

window and a status bar.

����������������������������������������������������������Ŀ

�                                                          �

�                                                          �

�                                                          �

�                     Receive window                       �

�                                                          �

�                                                          �

�                                                          �

�                                                          �

����������������������������������������������������������Ĵ

�                        Status Bar                        �

����������������������������������������������������������Ĵ

�                                                          �

�                                                          �

�                     Transmit Window                      �

�                                                          �

�                                                          �

������������������������������������������������������������

3.1.2   Receive Window - This multi-line window will contain the  most

recently  received  data.  It contains the most recently received

text in an 800 line text buffer.

3.1.3   Status Bar - The right hand portion of the status bar shows the

present date time properly converted to Greenwich Mean Time.  The

left hand portion of the status bar indicates whether the capture or

receive file mode is active, and if a text file is being transmitted.

3.1.4  Transmit Window - The transmit window is a multi-line  scrolla-

ble  area for entering transmitted text.  When transmit data is  taken

from  external files it is  shown sequentially in the area.

3.1.5  Pull Down Menus -  All program operations are available through

a pull down menu system.  The menu system is activated by pressing the

F10 key.  Menu selection is accomplished by use of the arrow and Enter

keys.  Some menu selections will open submenus for further selection.

3.2  Operating Modes

3.2.1   TNCTERM  properly interfaces with the TNC in  the  CW,  AMTOR,

RTTY, ASCII, HF-PACKET, and VHF-PACKET operating modes.  It provides a

layer of isolation between the operator and the TNC control  language.

This isolation is particularly beneficial in CW, AMTOR, ASCII and RTTY

modes as it allows the operator to concentrate on the QSO and not  the

control  language.   It is frustrating to have to look  up  a  desired

control  language sequence in the middle of a QSO.  Mode selection  is

make on the MODE pull down menu.

Chapter 4

Menu System

4.1  A pull-down / pop-up menu system is used to access all function

of the program.  In addition, certain functions can be quickly brought

up by alternate hot keys.  On the menu system these are shown by the

<K> symbol where K is the access character.

4.2  The menu system is activated by pressing function key <F10>.

The top line of the display contains the names of each menu section

and the leftmost menu will be active.  The screen appears as:

Modes  Files  Buffers  Accessory  Setup  DosApp

������Ŀ

�Ascii �

�Baudot�

�Cw    �

�Fax   �

�Packet�

��������

4.3  You can move between vertical menu items by pressing the bold

character or by using the up and down arrow keys.  Movement between

horizontal menus is obtained by use of the left and right arrow keys

only.  The selected item is indicated by reverse field.  Press

<ENTER> to activate that function.

4.4  For example, to activate morse code, Press <C> or move the reverse

field indicator using the up/down arrow keys.  Press <ENTER>.  A new

pull down menu will appear next to the Cw entry, indicating that further

selection is required:

Modes  Files  Buffers  Accessory  Setup  DosApp

������Ŀ

�Ascii �

�Baudot��������Ŀ

�Cw    ��HF     �

�Fax   ��VHF(mcw�

�Packet����������

��������

4.5  The other menus are:

Modes  Files  Buffers  Accessory  Setup  DosApp

�����������Ŀ

�Capture <C>�

�Receive <R>�

�Transmit   �

�ErasRcvBuff�

�SaveRcvBuff�

�ViewRcvBuff�

�������������

Modes  Files  Buffers  Accessory  Setup  DosApp

���������Ŀ

�LoadBuffr�

�ModBuffer�

�XmtBuffer�

�����������

Modes  Files  Buffers  Accessory  Setup  DosApp

�����������Ŀ

�Country    �

�LogBook <L>�

�NTSmsgs    �

�������������

Modes  Files  Buffers  Accessory  Setup  DosApp

���������Ŀ

�Baud     �

�cwWPM    �

�Date/Time�

���������ĳ

�Colors   �

�LoadClrs �

�SaveClrs �

�����������

Modes  Files  Buffers  Accessory  Setup  DosApp

���������Ŀ

�Dos      �

�Editor   �

�eXit  <X>�

�����������

Chapter 5

Transmit Buffers

5.1   Ten transmit buffers are available for quick insertion into  the

transmit data stream.  These buffers are read from and written  to the

file TNCTERM.BFR unless otherwise specified.  Transmit buffer mainten-

ance is found under the Buffer pull down menu.

5.2   Transmitting  a buffer - You may select a transmit buffer by one

of two methods.

(1) Using the pull down menus, select XmtBuffer.  The entire set

of 10 buffers will be displayed.  Using the UP and DWN arrow

keys, select the desired buffer and press ENTER.  If you change

your mind you may abort the selection by pressing ESC.

(2) Select a transmit buffer by  using  the alternate key combined

with a number key.  For example: buffer  number 3 is inserted

into the transmit stream by pressing alt<3>.

5.3  Viewing and Modifying Buffers - Access the buffers by selecting

ModBuffer on the Buffer pull down menu.  Specify the name of the buffer

file.  The default editor will be called with the specified file.  Change

the text as desired and exit the editor using <alt>X (if using TNCED).

The ten buffers will be updated on return to TNC.

5.4 Loading a new set of buffers.  Select the LoadBuffer function on the

Buffer pull down menu.  Specify the name of the desired buffer file.  If

the file is found the ten buffers will be overwritten with the file data.

5.5  Modifying the buffer file directly - Each buffer file is a simple

text  file  containing ten lines where each line does  not  exceed  80

characters.   You may modify the file using a text editor.  Make  sure

that the editor treats the file as a non-document.

NOTE:  FILE SELECTION

At any file name prompt you may enter a sequence of wildcards in the

file name.  For example:

*.TXT       ..\TEXT\BUFFERS.?91      c:\WP\BUFFER?.WP

When a wildcard is entered a pick list will be displayed showing all

matching file names.  The actual list may be longer than the display

so you can scroll past the end of the display.  Move to the desired

file using the UP and DOWN arrow keys and select it by pressing <ENTER>.

If the file is not found, press <ESC> to return to the file name

prompt.  If only one matching file is found, its name will be entered

into the file name prompt and no pick list will be displayed.

Chapter 6

File Transmission

6.1  Plaintext (ASCII) files can be transmitted in any mode.  This mode

is entered on the Files menu.  You will be prompted to enter the name

of the file for transmission.  Include the drive and path if necessary.

Characters will begin transmission immediately and are displayed in the

transmit window.  Tranmsission can be terminated by pressing <ESC>.

Chapter 7

Receive Buffer

7.1  The receive buffer in TNC consists of 800 lines of 80  characters

each.  The receive window only shows the last several lines  in the

buffer.  Access to the entire receive buffer can be made by pressing

the Page-Up key or via the Files pull down menu.  The receive window

will  then  show the  receive  buffer beginning with the first line of

text  received.  Movement  through the buffer is made with the up and

down arrow  keys.  When   you  are  finished  press  <ESC>  to  return

to the normal transmit/receive screen.

7.2  Saving the Receive Buffer - Access the save receive buffer function

on the Files pull down menu.  You will be prompted for a filename in

which to save the buffer.  The default is TNCTERM.BFR.

Chapter 8

Data Base Use

8.1  CWTERM uses and maintains three distinct data base files:

DEFAULT NAME             USE

LOGBOOK.DB     ISAM database file provides full log  keep-

ing functions using a pop up menu.

HAMGRAM.DBF    DBASE-III  compatible file containing  a  complete

message generation and storage system.

COUNTRY.LST    ASCII text file containing cross reference data of

Prefix  to  Country.  Much faster than  using  the

callsign book.

8.2  LOG BOOK DATA FILE

LOGBOOK.DB  is the default log book file.  You may alter the log  name

in the CWTERM.CFG configuration file.  This file is a ISAM database with

the following field definitions:

FIELD NAME     TYPE      SIZE

DATE           DATE      8

TIME           CHAR      4

CALLSIGN       CHAR      15

RSTIN          CHAR      3

RSTOUT         CHAR      3

FREQ           NUMBER    8:4

MODE           CHAR      3

COMMENT        CHAR      29

QSLSENT        DATE      8

QSLRCVD        DATE      8

8.2.1   LOGBOOK INDEX FILES - A single composite index file is maintain-

ed.  Both a DATE-TIME and CALLSIGN index are contained in this single

IDX file.

8.2.2   Accessing the Logbook Data  -  The logging function is accessed

on the Log/NTS pull down menu or by pressing <alt>L. The following form

will overwrite the lowest lines of the screen.

Date  �Time�  Callsign     �IN �OUT� Freq   �MOD� Comments       �RecNbr

��������������������������������������������������������������������������������

�    �               �   �   �        �   �

��������������������������������������������������������������������������������

Add Bottom Delete Edit Find Index Next QSL Prev Top  eXit       [DATETIME]

8.2.3  A new database will always show a RecNbr of 0 and blank fields

with the exception of Freq which will be 0.0000. Each function is invoked

by the first capital letter in the command word.  To record a logbook

entry use the Add function.  To modify an existing entry use the Edit

function.  When done either Adding or Modifying an entry return to the

command mode by pressing ESC to discard the changes or ctl_END to save

them to the database.  For additions to the database the Date and Time

will be entered from the system date-time corrected to GMT in accordance

with the environment variable TIMEZONE.

IMPORTANT!!  ESC     discards the ADD or EDIT record

ctl_END saves the record to the database.

8.2.4  Pressing <I> for index will toggle the index between callsign

and date-time.  The date-time is the default index and the last entry

in the log book will be the one shown on initial entry.

8.2.5  The <F>ind function performs the search based on the callsign

index.  I usually use this to find a previously worked callsign.

My log now has over 2000 entries covering a period of three years.

Finding a previously worked station is instantaneous!  The find may

be for an entire callsign or a partial.  For example:  entering W4

for the search string will put you at the first W4 callsign in the

logbook.  If a W4xxx entry does not exist, then the next record in order

after W4 will be displayed.

8.2.6  <D>elete a record opens a dialog box for confirmation of the

delete request.  A delete record in an ISAM database is permanent!

The database and both index files will be modified to reflect the

removal of all 'deleted' records.

8.2.7  Return to the calling program (TNC or COMMAND.COM) BY pressing

X for eXit.

8.2.8  Separate LOGBOOK maintenance.  The program LOGBOOK.EXE can be

executed from the DOS command line.  Multiple logbooks can be maintained

and are accessed by specifying them on the command line extension:

C:\HAM\LOGBOOK LOGBOOK  to access the CWTERM logbook

C:\HAM\LOGBOK CONTEST   to access the logbook named CONTEST

8.3  NTS MESSAGE DATA FILE

8.3.1 CWTERM maintains a data base of NTS messages which have been

transmitted.  These messages are in a DBASE-III compatible file with

the following data dictionary:

DATA FIELD     TYPE      SIZE

MSG_NBR        C         4

PRECEDENCE     C         1

HANDLING       C         1

HANDL_EXT      C         5

STATION        C         10

ORIGIN         C         15

FILE_TIME      C         4

FILE_MONTH     C         3

FILE_DAY       C         2

ADDRESSEE      C         25

ADDR1          C         25

ADDR2          C         25

CITY           C         25

STATE          C         2

ZIP            C         5

PHONE          C         13

TEXT_1         C         65

TEXT_2         C         65

TEXT_3         C         65

TEXT_4         C         65

TEXT_6         C         65

SIGN           C         25

SIGN_ADR1      C         25

SIGN_ADR2      C         25

SIGN_CITY      C         25

SIGN_ST        C         2

SIGN_ZIP       C         5

SIGN_PHN       C         13

8.3.1  NTS Data Base Access - You access the NTS data base via the Log/NTS

menu. The transmit/receive screen will be replaced with a data entry form

which relates directly to the NTS message format.  The last message sent

(or a blank message for a new data base) will be in the fields.

8.3.2 Data entry and movement through the data base is very similar to

making log book entries.  The main difference is that prompts and

selectable data entries are provided.  The casual preparation of an

NTS message does not require much of a learning curve.  The word count

is performed automatically.

8.3.3   Data Entry Format:

�Ĵ 1 of  1 ��ĴHAMGRAM.DBF�Ĵ04/14/90���������������������������Ŀ

� Nbr  P   Handling Inst  OrigSta  Place of Origin  Time Mon Day  �

�����������������������������������������������������������������Ĵ

�            HX                                                   �

�����������������������������������������������������������������Ĵ

�    To:                             Phone #                      �

�                                                                 �

�                                                                 �

�                                                                 �

�����������������������������������������������������������������Ĵ

�                                                                 �

�                                                                 �

�                                                                 �

�                                                                 �

�                                                                 �

�����������������������������������������������������������������Ĵ

�   From:                            Phone #                      �

�                                                                 �

�                                          ����������������������Ĵ

�                                          �Word Count :          �

�������������������������������������������������������������������

N)ext P)rev T)op b(O)t G)o S)rch E)dit A)dd D)el B)row Pa(C)k W)rite Q)uit

<F1> for HELP

8.3.4  Writing the message to an ASCII file.  Pressing the <W> select

key will write the message in NTS format to a file name MSGnnnnn where

nnnnn is the assigned message number in the data entry form.

8.3.5  Return to the transmit receive screen by pressing <Q> for quit.

8.3.6  The file MSGnnnnn may now be sent as a plain text file trans-

mission.

8.4 COUNTRY

8.4.1 COUNTRY.EXE is a fast retrieval program for locating countries

when the callsign prefix is known.

8.4.1 The file COUNTRY.LST is an ASCII file with the following format:

;Prefix Country/State/City      �sDD.MM�sDDD.MM

;{Local}Cape May NJ             � 38.57�  74.50

A2      BOTSWANA                �-22.00� -24.00

A3      TONGA IS                �-20.00� 175.00

A4X     OMAN                    � 22.00� -58.00

A5      BHUTAN                  � 27.30� -90.06

Comment and special data lines begin with a semicolon.  The program

will default to computation of distance in nautical miles.  To change

to another standard delete the appropriate leading semi-colon from

the appropriate one of the lines which specify statue miles or kilo-

meters:

;;{Earth major axis st.miles     � 3906.5

;;{Earth major axis in kilometrs}� 6387.388

Each line of data must conform to the specified format.

Prefix Country/State/City      �sDD.MM�sDDD.MM

Your own latitude and longitude is specified on the ";local" line as

shown above.

The entry for Cape May NJ, the authors QTH is:

;{Local}Cape May NJ             � 38.57�  74.50

The entry for Cedar Rapids would be:

;{Local}IOWA, CEDAR RAPIDS      � 41.58�  91.40

8.4.2  Accessing Prefix/Country list - Select Countries from the

Accessory pull down menu to call up the COUNTRY.EXE program.  You may

also access this program from the COMMAND.COM prompt.

8.4.3  Quick location of a prefix can be obtained by pressing the

first letter of the prefix.

8.4.4 Search by name of country is obtained by pressing CONTROL-S.

This opens a request window for the search string.  The search string

is not case sensitive, for example INDIA, india, and India will all

locate the same entry.  The light bar will be placed on the first

entry found containing that string, or left in place if not found.

8.4.5 The file can be maintained using a non-document editor.  Be

certain to observe the limitation of line size and format.  You

should also maintain the alpha numeric sort of entries.  Adding or

deleting lines will not effect the program operation.

Chapter 9

Capturing Incoming Text

9.1  Two methods are available for capturing incoming text.  The

receive buffer may be saved to a text file as previously described.

The incoming text may also be sent to a file as it arrives.   This is

accomplished by pressing alt<C>.  The capture file is alwasy named

TNCTERM.LOG and will be in the default directory.

9.2 Incoming text will continue to be sent to the file until the next

alt<C> is pressed.  The file will be closed.

Chapter 10

Modifying Program Defaults

10.1  SCREEN COLORS

10.1.1  Screen colors may be changed and saved to a default file called

TNCTERM.CLR.  The program will read and use this file is in the default

directory when the program is first executed.

10.1.2  COLOR SELECTION SCREEN   The color selection screen is available

as a choice on the SetUp menu.  The function keys are used to change

the foreground and background colors of each screen area used by TCN.EXE.

The colors selected by any one function key will rotate continuously

through the available choices as the key is pressed.

10.1.3  SAVING and LOADING a color file can be performed from the SetUp

menu.  The default file will be TNCTERM.CLR but can be any file name

of your choice.

10.2 CW Speed

10.2.1 Select cwWPM from the Setup menu.  Enter the desired WPM into

the prompt window.

10.3 RTTY Baud

10.3.1 Select Baud from the Setup menu.  Enter the desired baud into

the prompt window.

10.4 Date/Time

10.5 Selecting Date/Time from the Setup menu will cause the proper

transmission of the DAYTIME string to the terminal node controller.

You should be sure that your are at the cmd: prompt when selecting

this function.  This will force the terminal node controller to be

keeping the same date/time as the computer.

Chapter 11

DOS Applications

11.1  You may temporarily exit TNC by selecting Dos from the DosApp

menu.  The program will bring up a temporary COMMAND.COM from which

you can execute DOS commands or other programs.  You may be constrained

by the amount of memory available.  Try checking by using the MEM

command.  On a 286 laptop computer with 640 K RAM there is 397 K of

memory available at this temporary DOS prompt.  If you specify a

different COMSPEC file in your autoexec.bat be certain that it is

a full specification including drive and directory.  The program will

call up your specified command file if it is found.

Return to TNC by entering 'EXIT' at the dos prompt.

11.2  EDITOR  The editor specified in the TNCTERM.DEF file can be

invoked with a specified edit file by selecting Editor from the

DosApp menu.

11.3  PROGRAM TERMINATION - The program can be terminated in two ways -

by selecting the eXit line on the DosApp menu, or by pressing the

<alt>X key.  In either case you will be required to enter a 'y', or

'Y' to confirm the termination.

Chapter 12

TNCED Editor

TNCED is a full screen editor that is distributed with a Word Star (tm)

compatible keyboard interface.  It is capable of editing multiple files

with a tiled window for each file.  The total size of all files and

temporary buffers cannot exceed the size of available memory.  A 640 K

computer under DOS 4.0 can handle a single file of approximately 400,000

characters.

The program is invoked from the command line as:

TNCED file1 (file2 file3 file4 ...)

to open and edit multiple files.

The program is less than 60K in size and therefore is a convenient

editor to use when invoked as a child process from another program.

This is the way it is used when called by CWTERM.

The program is completely configurable by the install program:

TNCEDINST.EXE.

Keyboard primary and secondary control sequences can be changed to

suit the individual user (emulate NE or QED or any other editor of

your choice).  They can be modified for your individual taste.  As

distributed the primary control sequences are WordStar (tm) compat-

ible.

GRAPHICS EDITING

The editor can be put into a graphics mode by pressing either <F10>

or <alt G>.  You return to the text mode by pressing the same key.

The top line of the screen will show the present graphics characters

available.  Press <G> a few times to observe the full set of graphics

characters available.  Pressing <A> toggles between automatic and manual

selection of cursor movement.  When the AUTO is shown on the top line the

direction will change each time a corner is inserted into the text.  Nor-

mal motion is clock-wise.  You can force the cursor movement to change

direction by pressing the <D>.

The <SPACE>, cursor keys, HOME, END, PageUp, and PageDn all behave the

same as when in a text mode.  The <INSERT> key is disabled and the editor

will only perform in a type over mode.

You insert graphic characters by the use of the numeric keypad.  For

example the following set of graphics corresponds directly to the numeric

keypad as:

GRAPHICS        KEYPAD

�   �   �     7   8   9

�   �   �     4   5   6

�   �   �     1   2   3

�   �        0   .

The distribution copy of TNCED is installed for B&W displays with a

standard set of control key sequences.

The arrow keys are specified as Lft, Rgt, Up and Dn.

The control key is specified by the carat (^).  For example the key

sequence control page up would be:

<^PgUp>.

�����������������������������������������������������������������������Ŀ

� STANDARD SET - CONTROL FUNCTION  �  PRIMARY         �  SECONDARY      �

�����������������������������������������������������������������������Ĵ

� Character left                   �  <Lft>           �  <^S>           �

� Character right                  �  <Rgt>           �  <^D>           �

� Word left                        �  <^Lft>          �  <^A>           �

� Word right                       �  <^Rgt>          �  <^F>           �

� Line up                          �  <Up>            �  <^E>           �

� Line down                        �  <Dn>            �  <^X>           �

� Scroll up                        �  <^W>            �                 �

� Scroll down                      �  <^Z>            �                 �

� Page down                        �  <PgDn>          �  <^C>           �

� Page up                          �  <PgUp>          �  <^R>           �

�����������������������������������������������������������������������Ĵ

� Top of window                    �  <^PgUp>         �  <^Q><^R>       �

� Bottom of window                 �  <^PgDn>         �  <^Q><^C>       �

� Cursor to left side              �  <Home>          �  <^Q><^S>       �

� Cursor to right side             �  <End>           �  <^Q><^D>       �

� Top of screen                    �  <^Home>         �  <^Q><^E>       �

� Bottom of screen                 �  <^End>          �  <^Q><^X>       �

� Go to line                       �  <^J><^L>        �  <alt N>        �

� Go to column                     �  <^J><^C>        �                 �

� Previous cursor position         �  <^Q><^P>        �                 �

�������������������������������������������������������������������������

�����������������������������������������������������������������������Ŀ

� STANDARD SET - CONTROL FUNCTION  �  PRIMARY         �  SECONDARY      �

�����������������������������������������������������������������������Ĵ

� Abort Command                    �  <^U>            �                 �

�����������������������������������������������������������������������Ĵ

� Undo last deletion               �  <^Q><^U>        �  <alt U>        �

� Restore line                     �  <^Q><^L>        �                 �

� Tab                              �  <^I>            �                 �

� Insert control char              �  <^P> char       �                 �

� New line                         �  <^M>            �                 �

� Insert line                      �  <^N>            �                 �

� Delete current character         �  <Del>           �  <^G>           �

� Delete left character            �  <^H>            �  <^Bks>         �

� Delete right word                �  <^T>            �                 �

� Delete line right                �  <^Q><^Y>        �                 �

� Delete line                      �  <^Y>            �                 �

�����������������������������������������������������������������������Ĵ

� Find pattern                     �  <^Q><^F>        �  <alt F>        �

� Find and replace                 �  <^Q><^A>        �  <alt R>        �

� Find next                        �  <^L>            �                 �

�����������������������������������������������������������������������Ĵ

� Graphics toggle                  �  <F10>           �  <alt G>        �

�������������������������������������������������������������������������

�����������������������������������������������������������������������Ŀ

� STANDARD SET - CONTROL FUNCTION  �  PRIMARY         �  SECONDARY      �

�����������������������������������������������������������������������Ĵ

� Log drive/path                   �  <^J><^D>        �                 �

� Show TNCED version               �  <^J><^V>        �                 �

� Show available memory            �  <^J><^R>        �  <alt M>        �

� Toggle inster mode               �  <^V>            �  <Ins>          �

� Toggle autoindent mode           �  <^Q><^I>        �  <alt I>        �

� Set undo limit                   �  <^J><^U>        �                 �

� Set default extension            �  <^J><^E>        �                 �

�������������������������������������������������������������������������

�����������������������������������������������������������������������Ŀ

� STANDARD SET - CONTROL FUNCTION  �  PRIMARY         �  SECONDARY      �

�����������������������������������������������������������������������Ĵ

� Abandon file                     �  <^K><^Q>        �  <alt Q>        �

� Read file into window            �  <^K><^R>        �                 �

� Save and continue edit           �  <^K><^S>        �                 �

� Save to file                     �  <^K><^N>        �                 �

� Save and exit to DOS             �  <^K><^X>        �  <alt X>        �

�����������������������������������������������������������������������Ĵ

� Add Window                       �  <^O><^A>        �  <alt W><alt W> �

� Resize current window            �  <^O><^S>        �  <alt W><alt S> �

� Next Window                      �  <^O><^N>        �  <alt W><alt N> �

� Previous Window                  �  <^O><^P>        �  <alt W><alt P> �

�����������������������������������������������������������������������Ĵ

� Begin block                      �  <^K><^B>        �  <alt B><alt B> �

� End block                        �  <^K><^K>        �  <alt B><alt E> �

� Top of block                     �  <^Q><^B>        �                 �

� Bottom of block                  �  <^Q><^K>        �                 �

� Copy block                       �  <^K><^C>        �  <alt B><alt C> �

� Move block                       �  <^K><^V>        �  <alt B><alt M> �

� Delete block                     �  <^K><^Y>        �  <alt B><alt D> �

� Toggle (hide) block display      �  <^K><^H>        �  <alt B><alt H> �

� Mark curent word                 �  <^K><^T>        �                 �

� Write block to file              �  <^K><^W>        �                 �

�������������������������������������������������������������������������

�����������������������������������������������������������������������Ŀ

� STANDARD SET - CONTROL FUNCTION  �  PRIMARY         �  SECONDARY      �

�����������������������������������������������������������������������Ĵ

� Set marker 0                     �  <^K>0           �                 �

� Set marker 1                     �  <^K>1           �                 �

� Set marker 2                     �  <^K>2           �                 �

� Set marker 3                     �  <^K>3           �                 �

� Set marker 4                     �  <^K>4           �                 �

� Set marker 5                     �  <^K>5           �                 �

� Set marker 6                     �  <^K>6           �                 �

� Set marker 7                     �  <^K>7           �                 �

� Set marker 8                     �  <^K>8           �                 �

� Set marker 9                     �  <^K>9           �                 �

�����������������������������������������������������������������������Ĵ

� Hide/Unhide marker               �  <^K><^M>        � <alt B><alt H>  �

�����������������������������������������������������������������������Ĵ

� Jump to marker 0                 �  <^Q>0           �                 �

� Jump to marker 1                 �  <^Q>1           �                 �

� Jump to marker 2                 �  <^Q>2           �                 �

� Jump to marker 3                 �  <^Q>3           �                 �

� Jump to marker 4                 �  <^Q>4           �                 �

� Jump to marker 5                 �  <^Q>5           �                 �

� Jump to marker 6                 �  <^Q>6           �                 �

� Jump to marker 7                 �  <^Q>7           �                 �

� Jump to marker 8                 �  <^Q>8           �                 �

� Jump to marker 9                 �  <^Q>9           �                 �

�������������������������������������������������������������������������