This file contains ONLY PART of the

NTIA Report 83-127

titled

"Estimating the Performance of

Telecommunications Systems

Using the Ionospheric

Transmission Channel"

furthermore, no guarantee

is made to the accuracy of this copy.

TABLE OF CONTENTS

Page

LIST OF FIGURES                                          ix

LIST OF TABLES                                           xii

LIST OF CONTROL CARDS                                    xiii

LIST OF SYMBOLS                                          xv

1.    INTRODUCTION                                       1

2.    PRIMARY PROGRAM RELATED REQUIREMENTS               6

2.1   Computer Implementation Requirements         6

2.2   Files Used by IONCAP                         7

2.3   Program Operation Review                     8

3.    INPUT  DATA REQUIREMENTS                           10

3.1   General Data Requirements                    10

3.2   Input Language Definition                    11

3.3   Description of the User-Defined Input Data   11

3.3.1    Program  Control,  Execution, and

Termination Control Cards           12

3.3.2    Diurnal, Month, and Solar Activity

Control Cards                       17

3.3.3    System Configuration Control Cards  21

3.3.4    User-Defined Data Base and System

Override Cards                      25

3.3.5    Input,  Output,  and  Comment

Control  Cards                      30

3.3.6    Procedure  Related  Control Cards   32

3.4   Input Data Sheet                             35

3.5   Input Data Card Formats                      35

4.    OUTPUT   OPTIONS                                   35

4.1   Ionospheric Parameters Output Options,

METHOD = 1 or 2                            36

4.2   MUF Output Options, METHOD = 3 through 12    37

4.3   Systems Performance Output Options,

METHOD = 16 through 24                      38

4.4   Antenna Output Options,

METHOD = 13, 14, or 15                      41

5.    APPLICATIONS                                       41

5.1   Ionospheric Parameters Applications          41

5.2   MUF Applications                             42

5.3   System Performance Applications              42

7.    SPECIFIC INPUT EXAMPLES                           58

7.1   Ionospheric Parameter Example                58

7.2   Antenna Pattern Example                     59

7.3   MUF Example                                 59

7.4   System Performance Example                  59

7.5   User-Selected Output Example                60

7.6   LUF Example                                 61

7.7   Outgraph Example                            62

7.8   Procedure Example                           63

7.9   External Antenna File Example               64

8.    ACKNOWLEDGMENTS                                  65

9.    REFERENCES                                      198

LIST OF TABLES

Page

Table  2.    Valid Name Identifiers                   137

Table  3.    Available Output Methods                 138

Table  4.    Required SNR's for Radiotelephone

Service                                 139

Table  5.    Required SNR's for Radioteletype

Service                                 140

Table  6.    Typical Values of Ground Electrical

Characteristics                         141

Table  7.    Output Combinations for the OUTGRAPH

Card                                    142

Table  8.    Header Line Options for the TOPLINES

Card                                    143

Table  9.    System Performance Output Line

Options for the BOTLINES Card           144

Table  10.   System Performance Output Line

Options as Preset by the METHOD Number  145

LIST OF CONTROL CARDS

Page

Control  Card       METHOD card                           155

Control  Card  2.   EXECUTE card                          156

Control  Card  3.   QUIT card                             157

Control  Card  4.   TIME card                             158

Control  Card  5.   MONTH card                            159

Control  Card  6.   MONTHLOOP card                        160

Control  Card  7.   SUNSPOT card                          161

Control  Card  8.   NEXT card                             162

Control  Card  9.   LABEL card                            163

Control  Card  10.  CIRCUIT card                          164

Control  Card  11.  SYSTEM card                           165

Control  Card  12.  FREQUENCY card                        166

Control  Card  13.  SAMPLE card                           167

Control  Card  14.  EFVAR card                            168

Control  Card  15.  ESVAR card                            169

Control  Card  16.  EDP card                              170

Control  Card  20.  FPROB card                            174

Control  Card  21.  COMMENT card                          175

Control  Card  22.  AUXIN card                            176

Control  Card  23.  AUXOUT card                           177

Control  Card  24.  ANTOUT card                           178

Control  Card  25.  OUTGRAPH card                         179

Control  Card  26.  TOPLINES card                         180

Control  Card  27.  BOTLINES card                         181

Control  Card  30.  PROCEDURE card                        184

Control  Card  31.  END card                              185

Control  Card  32.  Terminated rhombic card               186

Control  Card  33.  Vertical monopole card                187

Control  Card  34.  Horizontal dipole card                188

Control  Card  35.  Horizontal Yagi card                  189

Control  Card  36.  Vertical log periodic card            190

Control  Card  37.  Curtain card                          191

Control  Card  38.  Sloping vee card                      192

Control  Card  39.  Inverted L card                       193

Control  Card  40.  Sloping rhombic card                  194

Control  Card  41.  Interlaced rhombic card               195

Control  Card  42.  Constant gain card                    196

Control  Card  43.  Antenna pattern read from file        197

xiv

PREFACE

The Institute for Telecommunication Sciences and its predecessors in the

U.S. Department of Commerce have been collecting ionospheric data and

developing methods to use these data in the prediction of the expected

performance of high frequency (HF) skywave systems since the start of World

War II.

Much of these data and the techniques for using the data are stored for

use by computers. Also several "standard" output formats have emerged to

assist in the planning and operation of high-frequency systems using skywaves.

This report describes the use of the latest developed method the "Ionospheric

Communications Analysis and Prediction Program" (IONCAP). The input and output

characteristics in this report relate to IONCAP Version 78.03. The version

number was generated to historically document the IONCAP program as it

currently exists and to facilitate a means of identifying subsequent versions

of the program. The version number 78.03 indicates that this version of IONCAP

was developed in 1978 and is the third version of the IONCAP program. The

earlier versions of the program, 77.01 and 77.02, were developmental versions

and were not distributed.

For many years, numerous organizations have been employing the HF spectrum

to communicate over long distances. It was recognized in the late thirties

that these communication systems were subject to marked variations in

performance. The effective operation of long-distance HF systems Increased in

proportion to the ability to predict variations in the ionosphere, since such

an ability permitted the selection of optimum frequencies, antennas, and other

circuit parameters. A worldwide network of ionosondes was established to

measure ionospheric parameters. Worldwide noise measurement records were taken

and observed variations in signal and amplitudes were recorded over various HF

paths. The results of this research established that most variations in HF

system performance were directly related to changes in the ionosphere, which

in turn are affected in a complex manner by solar activity, seasonal and

diurnal variations, as well as latitude and longitude. By 1948 a treatise of

ionospheric radio propagation was published by the Central Radio Propagation

Laboratory (CRPL) of the National Bureau of Standards. This document (CRPL,

1948) outlined the-state of the art in HF propagation. Manual techniques were

given for analyzing HF circuits of short, intermediate, and long distances.

Because the manual methods were laborious and time consuming, various

organizations developed computer programs to analyze HF circuit performance.

All these programs were based on manual methods for short or intermediate

distances and used various numeric representations of the ionospheric data.

The program described here is a direct descendant of these programsUse of the

Ionospheric Communications Analysis and Prediction Program (IONCAP) is

described in this report. The Ionospheric Communications Analysis and

Prediction Program (IONCAP) is in modular form and coded in simple FORTRAN,

following as much as possible the ANSI 66 standard. The modular form allows

any subsection to be replaced without affecting the rest of the program. As

much as possible, table look-up techniques are used to reduce computer run

time, to facilitate the modular structure, and to assist in the detection of

errors in any subsection. In particular, iterative., search procedures are

eliminated as these tend to become unstable under some ionospheric conditions.

The program is divided into seven largely independent sections:

1.    input subroutines,

2.    path geometry subroutines,

3.    antenna subroutines,

4.    ionospheric parameter subroutines,

5.    maximum usable frequency subroutines,

6.    system performance subroutines, and

7.    output subroutines.

The input subroutines handle the various input options. There are three

inputs: card images, a long-term data tape image, and an antenna tape image.

The card images contain the circuit parameters and control run options., The

long-term data tape contains numeric coefficients for ionospheric parameters

and for atmospheric noise as well as tables of parameters needed for circuit

performance. The antenna tape contains optional antenna patterns which can

either be generated by. the IONCAP program or obtained from some other source.

Simplification of the input card images, greater input flexibility, plus

extended and newly developed, input features improve this section over

previous models.

The path geometry subroutines determine the circuit geometry, select

areas to sample the ionosphere, and evaluate the magnetic field at these

sample areas. There are no significant improvements in this section over

previous models.

The antenna subroutines process antenna data input cards, calculate

antenna gains, and output antenna patterns. The program has the simple sub-

routine from ITSA-1 (Lucas and Haydon, 1966) for the basic antenna models.

These assume the antennas are associated with existing systems that have been

properly designed.

The ionospheric parameter subroutines evaluate the ionospheric

parameters needed by the program. Previous programs assumed an implicit two

parabola ionosphere.                iv

An explicit electron density profile is used in this program. The profile

consists of a D-E region starting at 70 km, an F2 region, an F1 ledge, and an

E-F valley. [A method of including the top side is given in a Radio Science

article (Haydon and Lucas, 1968).] Other electron density profile sub-routines

may be substituted and will not affect the operation of the program.

Observation indicates that absorption equations using the secant law require

modification when frequencies do not traverse the entire absorbing-region,

i.e., with reflection heights lower than 90 km. An empirical modification to

the secant law is included in this program.

The maximum usable frequency (MUF) subroutine is a direct determination

of the junction frequency based on an electron density profile derived from

monthly median parameters of the ionosphere rather than an iterative search. A

corrected form of Martyn's theorem (Martyn, 1959) is used. The E, F1, and F2

layer MUFs are considered. There is also a separate sporadic-E MUF. The

program also includes the classical nomogram method as an Option (Lucas and

Haydon, 1961).

The system performance subroutines evaluate all the usual circuit

performance parameters. There are two basic subroutines one for shorter

distances and one for long distances (greater than 10,000 km). The models for

the shorter distances are replacements for previous computer programs. The

long-distance models have not previously been incorporated into a computer

program. The short-distance models correspond to the manual method given by

Haydon et al. (1969). A manual method somewhat like the long-distance models

is given in NBS Report 462, (CRPL, 1948). The short-distance model evaluates

all possible ray paths for the circuit, including high and low angle modes; E,

F1, and F2 modes; above the MUF modes; and sporadic-E modes. losses include

regular D-E absorption (CCIR-252 loss), deviative losses, and

sporadic-E-losses. The CCIR-252 loss is basically for F2 modes. For E layer

modes an adjustment of the absorption is required, and for frequencies which

have low reflection heights (less than 90 km) a further correction to the

frequency dependence is added. The noise at the receiver site is evaluated and

combined with signal statistics to estimate the signal-to-noise statistics.

An extension of the single-hop model to long paths would lead to the

expectation that failure of propagation at any of the reflection areas would

cause propagation to fail altogether. Empirically, however, it has been found

that propagation does not fail until the ionosphere either fails to launch a

skywave or does not permit skywave reception; i.e., these are control areas

about 2,000 km from each end of the path. The long-distance model evaluates a

skywave launch capability at the transmitter and a skywave intercept

capability at the receiver, using an antenna-gain-minus-ionosphere-loss

function at each end of the path.                  v

Losses are the same as for the short paths at each end of the path with a loss

per kilometer function used to fill in the path. Noise and signal statistics

are the same for the short-distance or the long-distance paths.

The output subroutines generate all the output options as line printer

images which can be printed, written to magnetic tape, or saved on disk. The

available output options and the corresponding input required to generate the

output is described in this report.

The HF communications systems performance program has been extensively

revised. the chief limitations to further improvements appear to be the lack

of better statistical knowledge of the ionospheric and noise parameters and

their interrelations. There are six areas of expected improvement incorporated

into this revision of the program:

1.   A more complete description of the ionosphere.

2. Loss equations modified to include E-mode adjustments, sporadic-E

effects, over-the-MUF losses, and losses for low reflection heights.

3. Revision of the ray path geometry calculations by an empirical

adjustment of Martyn's theorem.

4. Revision of the loss statistics to include the effects of the

sporadic-E layer and over-the-MUF modes.

5. The development of a long-distance model by using the single-hop models

to select an efficient launch path at the transmitter, to select an efficient

intercept path at the receiver, and to evaluate the ionosphere losses between

the launch and intercept areas of the ionosphere.

6.   Revisions of the antenna gain models.

Much of the work completed is an incorporation of the combined efforts of

various laboratories, both government and private, and both domestic and

foreign. Although this program is coded so that revision of any subpart is

relatively easy, it is difficult to join so many diverse submodels while

maintaining consistency and continuity of the entire program. The whole in

this case is much more than a sum of the parts. The use of the program with a

description of-input and output options is described in this report. The

underlying assumptions and the mathematical-physical models are described in a

companion report.                       vi

ESTIMATING THE PERFORMANCE OF TELECOMMUNICATION

SYSTEMS USING THE IONOSPHERIC TRANSMISSION CHANNEL

Ionospheric Communications Analysis and

Prediction Program User's Manual

Larry R. Teters, John L. Lloyd,*

George W. Haydon,** and Donald L. Lucas***

This report describes the operation and use of the Ionospheric

Communications Analysis and Prediction Program (IONCAP). The computer program

is an integrated system of subroutines designed to predict high-frequency (HF)

skywave system performance and analyze ionospheric parameters. These

computer-aided predictions may be used in the planning and operation of

high-frequency communication systems using skywaves.

This report contains instructions for the use of IONCAP. A description of

the input data requirements, including data definition, organization, and

instructions for setup of the various analysis tasks, is presented. Procedures

and formats are given for preparing the input data and executing the program.

The various outputs are presented and described with an interpretation of the

analysis results.

INTRODUCTION

For many years, numerous organizations have been employing the HF spectrum to

communicate over long distances. It was recognized in the late 1930's that

these communication systems were subject to marked variations in performance,

and it was hypothesized that most of these variations were directly related to

changes in the ionosphere. Considerable effort was made in the United States,

as well as in, other countries, to investigate ionospheric parameters and

determine their effect on radio waves and the associated reliability of HF

circuits. The effective operation of long-distance HF systems increased in

proportion to the ability to predict variations in the ionosphere, since such

an ability permitted the selection of optimum frequencies, antenna systems,

and other circuit parameters. With the encouragement provided by these

findings, it was decided that more basic ionospheric data were necessary in

order to develop adequate models to anticipate ionospheric conditions

affecting HF propagation.

*The authors are with the Institute for Telecommunication Sciences, National

Telecommunications and Information Administration, U.S. Department of

Commerce, Boulder, Colorado 80303.

**The author was formerly with the Institute for Telecommunication Sciences.

***The author is with the National Telecommunications and Information

Administration, Office of Spectrum Management, Boulder, Colorado 80303.

A worldwide network of vertical incidence ionosondes was established to

measure values of parameters such as foE, foF1, foEs, foF2, and h'F. Worldwide

noise measurement records were started and steps were taken to record observed

variations in signal amplitudes over various HF paths. The results of this

research established that ionized regions ranging from approximately 80 to 600

km above the earth's surface provide the medium of transmission for

electromagnetic energy in the HF spectrum (3 to 30 megahertz (MHz)). Most

variations in HF system performance are directly related to changes in these

ionized regions, which in turn are affected in a complex manner by solar

activity, seasonal and diurnal variations, and latitude and longitude.

The Radio Propagation Unit of the U.S. Army Signal Corps provided a great

deal of information and guidance on the phenomena of HF propagation in 1945

(RPU, 1945). By 1948, a treatise of ionospheric radio propagation was

published by the Central Radio Propagation Laboratory (CRPL) of the National

Bureau of Standards. This document (CRPL, 1948) outlined the state of the art

in HF propagation. Techniques were included for: (1) predicting the maximum

usable frequencies (MUF); (2) determining the MUF for any path at any time,

taking into account the various possible modes of propagation by combining

theory and operational experiences; and (3) estimating skywave field

strengths. The model used to make the MUF predictions employed the "two

control point" method and assumed the ionosphere to be concentric with

reflection occurring only from the regular E and F2 layers.

Laitinen and Haydon (1950) of the U.S. Amy Signal Radio Propagation Agency

furthered the science of predicting HF system performance by developing

empirical ionospheric absorption equations and combining them with the

theoretical ground loss, free-space loss, and antenna gain factors so that

expected field strengths could be estimated for radio signals reflected from

the E and F2 regions, considering the effect of solar activity, and seasonal

and diurnal variations.

The accumulated techniques and methods presented in the above cited

literature and a number of other studies were then combined to establish

effective manual methods for predicting the expected performance of HF

communication systems; however, these methods were laborious and time

consuming even when only estimates of the MUF and optimum traffic frequency

(FOT) were needed.

To alleviate this problem, electronic computer routines were developed by

such organizations as Stanford Research Institute (1957), and the Central

Radio (page 2) Propagation Laboratory (Lucas and Haydon, 1961). All of these

routines were based upon the established manual prediction methods. The latter

program gave the first computerized technique that incorporated the numerical

coefficient representation of the ionospheric characteristics (Jones and

Gallet, 1962). However, only the expected MUF and FOT were predicted.

In 1962, CRPL issued a report on a computer routine that used the then

most recent improvements in the theory of performance predictions, combining

the more predictable ionospheric characteristics with circuit parameters to

calculate expected HF system performance; e.g., MUF-FOT, system loss,

reliability, etc. (Lucas and Haydon, 1962).

By 1960, it was recognized that high-frequency radar systems could detect

aircraft at considerable distance, and consideration was given to the idea of

applying the already developed computer programs used to predict the

ionospheric effect on HF communication to HF radar systems (Headrick and

Skolnik, 1974). At the outset, simple revisions were made to the HF prediction

program, including doubling the ionospheric losses associated with a given

point-to-point path.

The predicted signal-to-noise ratios obtained by these modifications were

compared with actual backscatter amplitudes, and the results were encouraging.

However, there were obvious deficiencies in the skywave model communication

that appeared immediately, when only simple modification of the model was used

to predict the backscatter amplitudes; e.g.:

1. Mirror reflection for the E-F2 layers was not satisfactory.

2. The F1 layer was not included as a separate layer.

3. The sporadic-E layer was not included.

4. Ionospheric tilts and their effects on antenna requirements and

absorption were not included.

5. Fading characteristics of adjacent frequencies were not adequately

included.

6. Takeoff angles were not dependent on frequency.

7. Predicted losses were in error when transmission via the lower layers

was nearly specular.

It was not possible to revise the prediction model to resolve all of the

above deficiencies in one effort; however, the development of the parabolic

distribution of electron density model established in ITSA-1 (Lucas and

Haydon, 1966) was a first step. In this model, the electron density profile at

each sample area was assumed to be adequately represented by two parabolic

layers; i.e., the E and F2 layers. The height of maximum ionization, layer

thickness, and electron density were derived from locations as defined by the

classical "two control point" method previously used in the calculation of the

upper limit of frequencies and transmission loss and at the path midpoint. 3

This work was continued in two separate paths, one for communication

analysis and predictions, reported in ITS-78 (Barghausen et al., 1969) and

another for analysis and prediction of over-the-horizon (OTH) radar systems

reported in NRL Tech. Reports 2226 and 2500 (Headrick et al., 1971; Lucas et

al., 1972). The IONCAP program is intended to bring together the two branches

and especially to make the techniques developed for HF skywave radar coverage

available to HF communication users. Radar techniques based on well-defined

ray paths are applied to the single-hop model; however, multihop paths exhibit

different behavior patterns (Whale, 1969). Available ionospheric parameters,

the use of these parameters to predict electron density profiles, and the use

of the profiles to predict the geometry and losses associated with skywave

propagation are discussed in an unpublished document entitled "Estimating the

performance of telecommunication systems using the ionospheric transmission

channel--Techniques for analyzing ionospheric effects upon HF systems," by J.

L. Lloyd, D. L. Lucas, G. W. Haydon, and L. R. Teters.

The stimulus for the development of IONCAP comes mainly from two

sources: (1) the radar backscatter data, which results in the revision of the

single-path model, and (2) the computation of efficient launch at intercept

path. For the single hops, this revision was based on the radar analysis model

developed jointly by ITS and the Naval Research Laboratory (Headrick et al.,

1971). It includes a complete electron density profile (D, E, F1, and F2

regions) as well as the sporadic-E layer. Both the low-angle and high-angle

rays are included. It is recognized that ray treatment is not complete, indeed

it is inappropriate at times, so "over-the-MUF" propagation is included as

well as Es propagation. For short-distance propagation, the stochastic factors

affecting the behavior of the ionosphere frustrate the prediction of discrete

multihop modes and make the extension of the short-distance techniques both

inappropriate and inaccurate. The method developed for long-distance

propagation is similar to those developed at CRPL (1948) and those developed

by Harnischmacher and Rawer at Zentralsetelle fur Funkberatung, and at SPIM

(Harnischmacher, 1960). They also include methods developed at the Radio

Research Centre (Whale, 1969), Auckland, New Zealand; e.g., such concepts as

"azimuth diversity" rather than great circle propagation alone, and coverage

by stochastic "scattering" rather than following an individual ray through an

absorbing region. Both the short-distance and the long-distance models are

used out to the limit of short-distance hop propagation; beyond this range

only the long-distance models are used. (page 4) The basic theoretical guides

used were the monograph by Davies (1969) and those by Bremmer (1949) and by

Budden (1966).

The output representations of ionospheric communication predictions

presented in this report include the historically accepted and proven outputs

of ITS-78 (Barghausen et al., 1969).

In the initial planning or in the modification of many communication

system, there may be an appreciable delay between the circuit planning and the

actual circuit construction or modification. This is of particular importance

for high frequency circuits which have marked time and geographic variations

in optimum frequency, required power, and system performance. Predictions of

ionospheric characteristics and techniques for using these characteristics

are, however, available and may be used to anticipate the performance of HF

communication circuits and thereby provide the lead time for necessary

equipment selection, frequency selection, and frequency and time-sharing

arrangements.

High-frequency radio communication depends upon the ability of the

ionosphere to return the radio signals back to earth. Prediction of ionization

levels in the various regions of the ionosphere is, therefore, essential to

any prediction of HF skywave circuit performance. The maximum frequency

returned from the ionosphere usually establishes the upper limit of the useful

HF range. The degree of ionization in the various regions is useful in

estimating probable modes, and the transmission loss for these modes is

combined with the antenna performance and transmitter power available to

estimate the expected HF skywave signal available anywhere at any time.

The expected skywave signal may be compared with the expected radio noise

environment to predict the likelihood that the circuit will operate

satisfactorily. This likelihood may be used to select optimum frequencies,

proper antennas, required transmitter power, optimum time of operation, and

broadcast coverage as a function of time and/or frequency.

This user's manual is subdivided into several sections. Section 2

describes the primary program-related requirements, including a program

operation review, computer implementation requirements, and data files used by

IONCAP. The input language definition, input data description, input card

image formats, and input requirements (including data organization, maximum

system size, restrictions, and default values) are presented in Section 3. The

various IONCAP task options and control parameters are also discussed in

Section 3. Section 4 describes the output options and possible program

termination error messages. Several applications for system performance

predictions are presented in Section 5 along with a sample test case. (page 5)

The geometry of the internal antenna package, ITSA-1, and typical antenna

patterns generated are presented in Figures 43 through 65. Section 5 also

contains a summary of the tabular and graphical outputs available with the

IONCAP version 78.03 computer program. The version number was generated to

historically document the IONCAP program as it currently exists and to

facilitate a means of identifying subsequent versions of the program. The

version number 78.03 identifies that this version of IONCAP was developed in

1978 and is the third version of the program. The earlier versions of the

program, 77.01 and 77.02, were developmental versions and were not

distributed. Section 6 briefly describes the data base file used with the

program. Section 7 contains input examples to assist the user in constructing

IONCAP input. (page 6)

2.2  Files Used by IONCAP

The primary input to the program is in the form of card images which are

read from LU5 (logical unit number 5). The LU5 could be a copy of the system

input file (card reader), a permanent file previously stored on disk or tape,

or a working file previously generated by another program prior to executing

IONCAP. The user may specify, as an input option, the use of the auxiliary

input file LU15. This essentially allows the user to have card images from two

different files used as input in the same run.

The primary output from the program is in the form of line printer images

which are written on LU6. The LU6 could then be copied to the system output

file (line printer), or a permanent file can be created and saved on tape or

disk, or it can be used as input to some other program. The user may specify,

as an input option, to use the auxiliary output file LU16. This essentially

allows the user to have line printer images on two different files in the same

run (see the AUXOUT control card discussed in subsection 3.3.5

The ionospheric long-term data base file (LU2) contains yearly, seasonal,

and monthly information essential to the ionospheric predictions generated by

IONCAP. A complete description of this data base file can be found in Section

6. These data are not user-defined input.

The COMMON-MUFS file (LU20) is created, if specified, as a task option by

the user. This file contains all of the variables contained in COMMON /MUFS/

which contains MUF, LUF, FOT, and other variables. It allows the user the

flexibility of creating his own output format specification by writing a

computer program which uses this file as its input. Therefore, LU20 can be

stored as a permanent file for later use or can be used as a working file

should a user output program be executed after IONCAP terminates execution

(see Section 5). Use of this option generally requires programming interface.

The antenna output file (LU25) is created if specified by the user. This

file contains antenna patterns created by the IONCAP antenna package (ITSA-1).

These antenna patterns can be saved as a permanent file and used as input to

the IONCAP program at a later date or possibly used as input to some other

analysis program. This is discussed in Section 3.4. The antenna input file

(LU26) is a permanent file used to read antenna patterns that have previously

been created by IONCAP or by some other antenna package or routine. If

specified by the user, the antenna patterns are read from this file instead of

being computed by the IONCAP program (see ANTENNA control card subsection).

(page 7)

Use of this option to input antenna patterns not generated by IONCAP generally

requires the user to create a software interface to IONCAP. The procedure file

(LU35) is a scratch file created by the IONCAP program to read and write input

procedures, if any are created by the user. (Refer to Section 3.3.6 for

procedural definition and use.) Note that this is a local scratch file which

is manipulated by the IONCAP program itself and should not be tampered with by

the user or programmer. The debug output file (LU61) was used during the

development of the IONCAP program. It contains printed output which was used

to verify and debug the-IONCAP program. This file is not available to the

user.

2.3  Program Operation Review

The first step in using the IONCAP program is to assemble the appropriate

datafor the system configuration to be analyzed. These data are then prepared

on punched card images which are read as input directly to the computer

program or perhaps stored on a disk or magnetic tape file for reading as input

to the program at a later date.

Each input card image contains a name identifier that is used as a label

to identify the specific type of data on that particular card image. A listing

of all valid name identifiers is given in Table 2, and a complete description

of each name identifier and the associated input data is given in Section 3.3.

The user may also create his own name identifier by defining an input

procedure. The input procedure definition, discussed in Section 3.3.6, allows

the user to represent several card images by a single name identifier. When

reference is made to the procedure name during program execution, the program

essentially replaces the procedure name with the card images constituting the

procedure definition.

Program execution begins by examining the entire input file for any user-

defined procedures. If any are located, the procedure names are stored in a

table of valid name identifiers and the actual procedure definitions are

written on a scratch file for later reference. This allows the procedure

definition to occur anywhere in the input file, and thus, a procedure need not

be defined before a reference is made to it. (Computer programmer types may

see parallels to this technique in other applications.)

The computer program then checks the name identifier of each card image on

the input file with the internal table of valid name identifiers. If an

invalid name identifier is located, the program flags the card image in error

and continues to examine the name identifier on each of the remaining card

images.                         (page 8)

Errors in the name identifier are most often caused by (1) a spelling

error in the identifier name or (2) a reference to an undefined procedure. If

any name identifier errors are located, a message is printed and program

execution is terminated. This is henceforth referred to as Phase I of the

IONCAP execution.

At this time, there is little or no diagnostic check of the data contained

on any card image. Only the name identifier is examined. Thus, program

execution will continue if there are no name identifier errors even though

there may possibly be errors in the actual input data. It is, therefore,

essential for the user to examine each card image to insure that it is punched

correctly and that it contains the necessary data before attempting to execute

the IONCAP program. If no name identifier errors are located on the input

file, the program rewinds the user-defined input file and begins to read and

process the data on each card image to define a system configuration. Once the

configuration is complete, the analysis is performed. The analysis portion of

IONCAP is henceforth referenced to as Phase 2. A flow diagram of the basic

IONCAP control is given in Figure 2.

The IONCAP program performs four basic analysis tasks in Phase 2. These

tasks are discussed in detail in the IONCAP theoretical report and summarized

below:

(1) Ionospheric Parameters. The ionosphere is predicted using parameters which

describe four ionospheric regions: E, F1, F2, and Es. For each sample area,

the location, time of day, and all ionospheric parameters are derived. These

may be used to find an electron density profile, which may be integrated to

construct a predicted ionogram. These options are specified by methods l and

2, which are briefly described in Table 3.

(2) Antenna Patterns. The user may precalculate the antenna gain pattern

needed for the system performance predictions. These options are specified by

methods 13, 14, and 15, which are briefly described in Table 3. If the pattern

is precalculated, then the antenna gain is computed for all frequencies (1-30

MHz) and elevation angles. If the pattern is not precalculated, then the gain

value is determined for a particular frequency and elevation angle as needed.

(3) Maximum usable frequency (MUF). The maximum frequency at which a skywave

mode exists can be predicted. The 10% (FOT), 50% (MUF), and 90% (HPF) levels

are calculated for each of the four ionospheric regions predicted. These

numbers are a description of the state of the ionosphere between two locations

on the earth and not a statement on the actual performance of any operational

communications circuit. These options are specified by methods 3 to 12, which

are briefly described in Table 3. 9

(4) Systems Performance. A comprehensive prediction of radio system

performance parameters (up to 22) is provided. Emphasis is upon the

statistical performance over a period of a month. A search to find the lowest

usable high frequency (LUF) is provided. These options are specified by

methods 16 to 29, which are briefly described in Table 3.

3.  INPUT DATA REQUIREMENTS

This section describes the general data requirements and the input

language definition. It also includes a complete description of all

user-defined input data and the corresponding input card image formats. The

minimum input requirements and default values for each analysis and prediction

method are presented, as are the data organization and input restrictions.

3.1 General Data Requirements

The data requirements of the IONCAP program consist of a fixed data base

and user-defined input data. Much of the basic data required to use the IONCAP

program is stored on the ionospheric long-term data base file. This data base

includes geographic and time variations of the ionosphere and of atmospheric

noise levels, the relationship between the ionospheric characteristics and the

propagation path geometry and signal attenuation, and the theoretical

performance of common antenna systems. The data base is fixed to the user and

is not part of the user-defined input data. A complete description of the

ionospheric long-term data base is contained in Section 6. However, the user

may execute the IONCAP program without the data base file by specifying

certain user-defined data (see Section 3.3.4). This, however, generally

requires specific information concerning the ionosphere.

The user-defined input data consist of card images and perhaps an optional

antenna file and the input depends primarily on the complexity of the analysis

or system performance prediction requested (see Section 3.4). As an example,

to determine the upper useful frequency limits, only the geographic location

of the circuit terminals and the time are required. For more complex

predictions such as the expected reliability of communications, additional

user-defined-input-data are required. This data would include frequency of

operation, the type of antenna used, the transmitter power, the type of

service required, and the man-made noise environment, among others. (page 10)

A complete description of all user-defined data is presented in Section

3.3, and the required user-defined input data for each analysis or prediction

method are summarized in Section 3.4. The input required for each run option

is given in Table 13. The details of each card format are given in Section

3.5.

3.2  Input Language Definition

The name identifiers mentioned in Section 2.3 constitute an input language

which is the primary communication link between the user and the actual

program analysis. The language provides the user with the means of describing

complex physical configurations in somewhat familiar terms. It also provides a

means of controlling the type and the extent of the analysis to be performed

as well as the type and amount of output desired. Desirable characteristics of

the input language include (1) format simplicity of the input statement and

(2) quasi-order independence of the input data for a particular system

configuration. Thus, the input card images can often be presented to the

program in any order (with some restrictions) to describe the topology of a

system configuration, the type of analysis to be performed, and the output

desired. Once these characteristics for a particular configuration have been

defined, the program can be executed. Another desirable characteristic is the

ability to alter certain characteristics of the configuration without having

to redefine all other characteristics. This allows the user to alter certain

parameters in the system configuration while keeping other parameters fixed.

The benefit of this is that it allows the user to execute the program for

several configurations that have common characteristics in the same run

without redefining all characteristics of the system configuration.

3.3  Description of the User-defined Input Data

The user-defined input data are in the form of card images and perhaps an

optional antenna data file. As mentioned in Section 2.3, each input card image

contains a name identifier that is used as a label to identify the specific

type of data on that particular card image. A listing of all valid name

identifiers is presented in Table 2, and a complete description of each name

identifier and the associated data is given in the following subsections along

with card formats and examples. Each input card image will henceforth be

called a control card with the name identifier used as the name of the control

card. Formats for each control card are given in Section 3.5. All cards start

with a 10 column identifier. All data fields are five columns long. Tables 14

and 15 contain all card formats and comments concerning these cards. (page 11)

3.3.1  Program Control, Execution, and Termination Control Cards

The following control cards define the analysis task to be performed,

indicate when the task will be performed for the current system configuration,

and indicate how the program will reach a normal termination.

METHOD control card

The METHOD card defines the analysis task to be performed for a particular

system configuration and also specifies the initial page number for the output

file. The two parameters on the METHOD card are (see Control Card 1.

References to the control cards contain an example of each card along with a

table of the contents of the card):

(a) METHOD is the parameter which controls the type of program analysis

and predictions performed. At the present time there are 30 task options

available. Many of these tasks differ only in the representation of the output

and require the same (or nearly the same) computations. The basic tasks

consist of (1) tabulation of ionospheric parameters and ionograms from the

ionospheric data base file, (2) MUF-FOT calculation and graphs, (3)

computation and tabulation of antenna patterns, (4) system performance

predictions, and (5) LUF calculation and graphs. A brief description of each

method available is given in Table 3, and a presentation of the output

generated by each method is given in Section 4.

(b) NPAGO is the parameter that controls the initial page number on

the output file. The initial output page is set to this value if NPAGO is

positive. If the variable is left blank, negative or zero, then the initial

page on the output file is set-to one.

EXECUTE control card

The EXECUTE card causes the program to perform the indicated analysis task

for the currently defined system configuration. This requires the user to

specify the program control card and the system configuration cards prior to

the EXECUTE card (see Control Card 2). There is only one parameter on the

EXECUTE card described as follows:

KRUN is the parameter that indicates the layers at the ionospheric

parameters that are calculated. If KRUN is less than or equal to zero, then

the parameters of all layers

(E, F1, F2, and Es) are calculated; if KRUN = 1 then the parameters of

the E layer, F1 layer, and F2 layers are calculated; if KRUN = 2, the

parameters for the Es layer only are calculated; if KRUN is greater than or

equal to three, then no ionospheric parameters are calculated. (page 12)

This allows the user to skip the calculation of ionospheric parameters and

use the last set previously calculated or a set of ionospheric parameters

entered with the input cards. The use of the KRUN option may require

additional consideration beyond that presented here. It is also often possible

to cause program analysis to be performed by having two consecutive control

cards with the same name identifier. The control cards that allow this are:

METHOD

CIRCUIT

EXECUTE

Thus, if the user desires to have both a tabulation of two circuits, he may

accomplish this by having two consecutive CIRCUIT cards on the input file. The

remainder of this system configuration is unchanged if this technique is used.

QUIT control card

The QUIT card causes termination of the IONCAP program. It must,

therefore, be the last physical control card on the user-defined input file.

This card is essential to the program since it is used when examining the name

identifiers on the control cards in Phase 1, and as a default termination of a

month-sunspot loop (to be discussed in NEXT control card subsection) as well

as the program termination indicator in Phase 2. There are no data parameters

on the QUIT control card (see Control Card 3).

3.3.2  Diurnal, Month, and Solar Activity Control Cards

The following control cards define the time of day, the month, year, and

sunspot number of the solar activity period of interest for which the analysis

and predictions are performed.

It should be noted that the months and sunspots specified on the following

control cards constitute a loop. That is to say, the analysis and predictions

are performed for every linear combination of months and sunspots indicated.

Separate control cards are, therefore, not required for each month and/or

solar activity period of interest. The month-sunspot (or sunspot-month) loop

is discussed in the NEXT control card subsection, with examples given in

Figures 3 through 8. Thus, several months can be executed with one sunspot

number and/or several sunspots can be executed a single month. This also

demonstrates a procedure example. In order to do selected month, or

combination of months, with individual sunspot numbers, use the PROCEDURE

card. For example, the set below will do the calculations for all seasons of

1976 for each of the circuits defined in the procedure called NORTH. (page 13)

See Section 3.3.6 for a further discussion of the PROCEDURE card. The fixed

field formats of each card are presented on the corresponding control card.

PROCEDURE  NORTH

CIRCUIT  cards representing necessary communication paths

EXECUTE

END

Control cards defining system configuration:

MONTH      1976      4

SUNSPOT    12.6

NORTH

MONTH      1976      6

SUNSPOT    12.2

NORTH

MONTH      1976      9

SUNSPOT    14.3

NORTH

MONTH      1976     12

SUNSPOT    14.8

NORTH

QUIT

TIME control card

The TIME card indicates the time of day for which the analysis and

predictions are to be performed. The hours can be either universal time or

local mean time at the transmitter, depending on an input parameter. There are

four parameters on the TIME control card, described as follows (see Control

Card 4):

(a) IHRO indicates the starting hour in universal time or in local mean

time at the transmitter.

(b) IHRE indicates the ending hour in universal time or in local mean

time at the transmitter.

(c) IHRS indicates the hourly increment. The hourly increment is added to

the starting hour to determine the next hour. This incremental process

continues until the ending hour is reached.

(d) ITIM indicates that he specified time is universal time or local mean

time. If ITIM is negative, then the specified times are local mean times (LMT)

at the transmitter, otherwise the specified times are universal time (UT).

(page 14)

MONTH control card

The MONTH card indicates the year and months for which the analysis and

prediction are to be performed. The year is used for identification purposes

only and has no effect on the program calculations. June and December are

typical of seasonal extremes for long-range planning. The parameters on the

MONTH control card are as follows (see Control Card 5):

(a) NYEAR indicates the year, but has no effect on the program

calculations.

(b) MONTHS is an array of up to 12 months where 1 represents January, 2

represents February, etc. The program analysis and predictions are performed

for each of the months the user specifies. The program assumes there are no

additional months to process when a blank or a zero month is encountered. The

desired months can be specified in any order. This, for example, allows the

user to perform the analysis and predictions for December before January if he

so desires. Predictions for each month specified on the MONTH card are

performed for each sunspot number specified on the SUNSPOT card.

MONTHLOOP control card

The MONTHLOOP card is similar to the MONTH card in that it indicates the

year and the months for which the analysis and predictions are to be

performed. The difference between the two cards is that the user specifies the

individual months desired on the MONTH card, and he specifies a range of

months desired on the MONTHLOOP card. As previously mentioned, the year is

used for identification purposes only and has no effect on the program

calculations. The four parameters on the MONTHLOOP card are as follows (see

Control Card 6):

(a) NYEAR indicates the year, but has no effect on the program

calculations.

(b) MINIT indicates the starting month where 1 represents January, 2

represents February (c) MFINAL indicates the ending month where 1 represents

January, 2 represents February

(d) MINC indicates the monthly increment. The monthly increment is added

to the starting month to determine the next month. This incrementing process

continues until the ending month is reached. The MONTHLOOP card, along with a

corresponding sunspot number, requires the program to read the ionospheric

long-term data base file to obtain months and seasonal information.

Predictions for each month specified on the MONTHLOOP card are performed for

each sunspot number specified on the SUNSPOT card. (page 15)

SUNSPOT control card

The SUNSPOT card indicates the sunspot numbers of the solar activity

period of interest and is the 12-month running average for each of the months

specified. sunspot number of 10 is typical for low solar activity and sunspot

numbers between 110 and 130 are typical of high solar activity. NOTE: monthly

or daily solar activity indices are not used, only the expected 12-month

running average is used.

The parameter on the SUNSPOT card is as follows (see Control Card 7):

SUNSP is an array of up to 12 sunspot numbers. The program analysis and

predictions are performed for each of the solar activity periods the user

specifies and for each month specified. The desired sunspot numbers can be

specified in any order with the one restriction, that, if a zero sunspot

number is desired, it must be the first one indicated on the control card.

This restriction is necessary since the program assumes that there are no

additional sunspot numbers to process when a blank or zero sunspot number is

encountered. The program uses a "look-ahead" procedure which allows the first

specified sunspot number to be blank or zero.

NEXT control card

The NEXT control card is the termination of a month-sunspot (or sunspot-

month) loop. Each of the control cards that occur after the MONTH (or

MONTHLOOP) card and the SUNSPOT card, but before the NEXT card, is processed

for every combination of months and sunspot numbers specified. Thus, the body

of the loop consists of all the control cards after the MONTH-SUNSPOT cards up

to the NEXT card. A month-sunspot loop is generated if the MONTH (or

MONTHLOOP) card precedes the SUNSPOT card. A typical deck structure for a

month-sunspot loop is given in Figure 3, and the effect of a typical

month-sunspot loop is demonstrated in Figure 4. A sunspot-month loop is

generated if the SUNSPOT card precedes the MONTH (or MONTHLOOP) card. A

typical deck structure for a sunspot-month loop is given in Figure 5, and the

effect of a typical sunspot-month loop is demonstrated in Figure 6. Several

month-sunspot (or sunspot-month) loops may occur in the same run, as

demonstrated in Figure 7. However, nesting of month-sunspot loops is not

permissible. The QUIT card acts as the terminator to a month-sunspot (or

sunspot- month) loop if no NEXT control card is present, as demonstrated in

Figure 8. There are no parameters on the NEXT control card (see Control Card

8). This card is not necessary if only one MONTH-SUNSPOT loop combination is

defined.                    (page 16)

3.3.3   System Configuration Control Cards

The following control cards identify and define the system configuration

to be analyzed. This includes a label to identify the system coordinates, the

location of the transmitter and the receiver, and the actual system parameters

necessary to define the system configuration.

LABEL control card

The LABEL card contains alphanumeric information used to describe the

system location on both the input and output. A recommended usage of the LABEL

card is to specify a label for each transmitter-receiver configuration used,

where the alphanumeric information describes the location of the coordinates

given on the CIRCUIT control card. The parameters on the LABEL card are as

follows (see Control Card 9):

(a) ITRAN is an array of 20 alphanumeric characters used to describe the

transmitter location.

(b) IRCVR is an array of 20 alphanumeric characters used to describe the

receiver location. It should be noted that the information on the LABEL card

is used for identification purposes only. Thus, the user is allowed a maximum

of 40 alphanumeric characters to describe the transmitter-receiver location

and need not be concerned with which characters are stored in ITRAN and which

are stored in IRCVR.

CIRCUIT control card

The CIRCUIT card contains the geographic coordinates of the transmitter

and receiver and a variable to indicate the user's choice between shorter or

longer great circle paths from the transmitter to the receiver. Normally the

short circle path is desired by the user.  On the CIRCUIT control card, the

symbol N denotes northern hemisphere, S denotes southern  hemisphere, W

denotes  western hemisphere, and E denotes eastern hemisphere (the directions

may be spelled out if desired; such as NORTH). The parameters on the CIRCUIT

card  are  as  follows  (see Control Card 10):

(a) TLATD indicates the latitude at the transmitter in degrees.

(b) ITLAT indicates the northern (N) or southern (S) hemisphere at the

transmitter.

(c) TLONGD indicates the longitude at the transmitter in degrees.

(d) ITLONG indicates the eastern (E) or western (W) hemisphere at the

transmitter.

(e) RLATD indicates the latitude at the receiver in degrees.

(page 17)

(f) IRLAT indicates the northern (N) or southern (S) hemisphere at the

receiver.

(g) RLONGD indicates the longitude at the receiver in degrees.

(h) IRLONG indicates the eastern (E) or western (W) hemisphere at the

receiver.

(i) NPSL indicates the user's choice between shorter or longer great

circle paths from the transmitter to the receiver. The longer great circle

path is used if the user specifies NPSL = 1, otherwise the shorter great

circle path is used.  Often desired is the shortest distance between

transmitter and receiver.  This is accomplished by leaving the variable NPSL

blank, therefore, selecting the shorter great circle path  by default.

SYSTEM control card

The SYSTEM card includes parameters necessary to define the system

configuration. This includes the transmitter power, man-made noise level,

minimum take off angle, required circuit reliabilities, required

signal-to-noise ratio, the maximum difference in delayed signal power for

multipath, and the maximum difference in delay time for multipath. The

parameters defined on the SYSTEM card are described below (see Control Card

11):

(a) PWR indicates the transmitter power in kilowatts. Note that this is

the power delivered to the transmitting antenna. (The program internally

converts this to decibels (dB) above a kilowatt as needed for computations.)

(b) XNOISE indicates the expected man-made noise level at the receiver in

dBW (decibels below 1W) in a 1 Hz bandwidth at 3 MHz. XNOISE should be input

as a positive value unless the user desires to designate the receiving

location area as industrial, residential, rural, or remote unpopulous by

specifying:

XNOISE = -1. for industrial (the program then uses -125. dBW)

XNOISE = -2. for residential (the program then uses -136. dBW)

XNOISE = -3. for rural (the program then uses -148. dBW)

XNOISE = -4. for remote unpopulous (the program then uses -164. dBW)

Typical values of man-made noise relative to population of the receiving area

are given in Figure 9. In remote unpopulous areas, cosmic noise will normally

dominate over man-made noise.

(c) AMIND indicates the minimum takeoff angle in degrees. The value is

normally very small unless antenna performance is expected to be so poor at

low angles that these angles should not be used in the estimation of upper

useful frequencies, or if the horizon is so obstructed that low takeoff and

reception angles appear unlikely. The program uses a default value of 3

degrees if this parameter is not set by the user. A value of 0.001 will

effectively give the full range of possible radiation angles and should be

specified by the user if the 3 degrees default is not required. 18

(d) XLUFP indicates the required circuit reliability, which is an estimate

of the percentage of days within the month that the signal quality will be

acceptable, and should be specified for calculation of the LUF or time

availability for service probability. (NOTE: XLUFP is expressed as a

percentage.) The program uses a default value of 90 if this parameter is not

set by the user.

(e) RSN indicates the required signal-to-noise and is the ratio of the

hourly median signal power in the occupied bandwidth relative to the hourly

median noise in a 1 Hz bandwidth, which is necessary to provide the type and

quality of service required (expressed in decibels). Table 4 and Table 5 show

typical required S/N ratios for radiotelephone service and radioteletype

service. NOTE: The S/N values in this report and in the computer model are the

ratio of signal in occupied bandwidth to noise in a 1 Hz bandwidth.

(f) The user may choose the multipath computations. The multipath

probability is an estimate of the likelihood that two or more skywave modes

will exist within the specified power tolerance and outside the time delay

tolerance.

PMP indicates the maximum difference in delayed signal power in decibels

between skywave modes to permit satisfactory system performance in the

presence of multiple signals. If PMP is blank or zero, multipath is not

considered.

(g) DMPX indicates the maximum difference in delay time in milliseconds

between skywave propagation modes to permit satisfactory system performance in

the presence of multiple signals. The program uses a default value of .85 if

this parameter is not set by the user.

FREQUENCY control card

The FREQUENCY complement control card contains up to 11 user-defined

frequencies that are used in the calculation. The FREQUENCY card is not

necessary with the analysis and prediction options dealing only with

ionospheric parameters, (page 19) ionograms, MUFs, antennas, or LUFs. System

performance requires the FREQUENCY card. The parameter on the FREQUENCY card

is as described below (see Control Card 12):

FREL is the array of up to 11 user-defined frequencies in megahertz. If

the first frequency on the control card is left blank, then the FOT is

inserted as the first frequency. If the entire FREQUENCY card is left blank,

then the 11 frequencies used in the calculation are computed for each hour by

the program. The computed frequency range then begins at 2 MHz and ends at the

HPF on a non-linear scale based on the E-MUF and F-MUF values. It should be

noted that, in this manner, a different frequency complement is generated for

each hour.

ANTENNA control card

The ANTENNA control card defines whether the antenna is transmitting or

receiving, specifies the type of the antenna, the ground conductivity, and

dielectric constants in addition to the other parameters necessary to compute

the necessary antenna patterns. Typical values for selected ground types are

given in Table 6 where typical good ground is low hills with unforested rich

soil or flat wet coastal regions; fair ground is medium hills or forested

heavy soil; and typical poor ground includes rocky steep hills, sandy dry

coastal regions, and city industrial areas. The ground constants are optional

on each antenna card. If not specified by the user, they will be calculated

with the land mass map from the ionospheric long-term data base file. The

ground constants will be set to poor ground as a default if they are not

specified by the user and the land mass map is not available. It is

permissible for the user to specify up to three transmitter antennas and three

receiver antennas to cover the frequency range considered for each system

configuration. It is also possible to indicate that the antenna is to be input

from an external antenna file (LU25) instead of being calculated by the

program, The input parameters required vary with antenna type. The basic

ANTENNA card parameters are described below, and the geometric description of

each available antenna type is given in Figures 43 through 65, Table 16, and

Control Cards 32 through 43.

(a) IAT indicates whether the antenna is transmitting or receiving, IAT =

1 for transmitting, IAT = 2 for receiving.

(b) IANTR indicates the number which corresponds to the antenna type

desired. The user has the option to input the antenna pattern from the

optional external antenna file as discussed in Section 3.4. (page 20)

(c) AETA indicates the antenna bearing in degrees from east of north.

Negative denotes the off azimuth angle from the circuit path. Positive denotes

the main beam direction.

(d) ASIG indicates the ground conductivity in mhos per meter.

(e) AEPS indicates the relative dielectric constant.

The following parameters describe various characteristics dependent on the

antenna type specified. The user should refer to Section 3.4 for the

description of the following antenna parameters and the antenna

characteristics required for each antenna type.

(f) AND

(g) ANL The definition and description of these parameters

(h) ANH depend on the antenna type specified.

(i) AEX

(j) AFQB indicates the ending frequency when the user specifies more than

one transmitting type or receiving antenna type for the frequency range

considered. The next antenna type used for the transmitter or receiver would

then begin at 1 MHz greater than the ending frequency specified here.

(k) IAIN indicates the antenna number (up to three) when the user

specifies more than one transmitter or receiver antenna for the frequency

range considered. This parameter is also used as a control parameter to

specify the location on the optional external antenna file if one is used.

This usage is discussed in Section 3.4.

3.3.4 User-defined Data Base and System Override Cards

The following control cards allow the user to specify an external data

base consisting of (1) the geographical and geomagnetic parameters relating to

a specified sample area, (2) the critical frequency, semi-thickness, and

height of maximum ionization for the E, F1, and F2 layers, (3) lower, median,

and upper decile values of the critical frequency and the virtual height of

reflection, and (4) the true height and electron density. The ionospheric

long-term data base file is not necessary when these ionospheric parameters

are defined by the user. The user may also override internal program

characteristics such as (1) the integration scheme used for the E and F2

layers and (2) the critical frequency multipliers to adjust the heights of the

ionospheric layers. These values can be obtained using METHOD equal to 1 in a

previous run. Use of the control cards in this section require in-depth

information of ionospheric parameters and would be avoided by most users.

These control cards are used to alter the use of predefined ionospheric data.

(page 21)

SAMPLE control card

The SAMPLE control card contains the following geographic and geomagnetic

parameters relating to a specified sample area. A maximum of five sample areas

may be defined by the user (see Control Card 13).

(a) I indicates the specific sample area number, 1 to 5.

(b) SLAT indicates the geographic latitude of the sample area in degrees.

(c) ISLAT indicates the geographic latitude of the sample area is in the

northern or southern hemisphere (N or S).

(d) SLONG indicates the geographic longitude of the sample area in

degrees.

(e) ISLONG indicates the geographic longitude of the sample area is in the

eastern or western hemisphere (E or W).

(f) SGLAT indicates the geomagnetic latitude in degrees.

(g) ISGLAT indicates the geomagnetic latitude is in the northern or

southern hemisphere (N or S).

(h) RD(I) indicates the distance from the sample area to the transmitter

in kilometers.

(i) GYZ(I) indicates the gyrofrequency in megahertz.

(j) CLCK(I) indicates the local mean time at the sample area.

(k) GMDIP(I) indicates the geomagnetic dip angle at the sample area.

(l) ARTIC(I) indicates the auroral loss at the sample area.

(m) SIGPAT(I) indicates the ground conductivity at the sample area.

(n) EPSPAT(I) indicates the relative dielectric constant at the sample

area.

EFVAR control card

The EFVAR control card allows the user to indicate the critical frequency,

semi-thickness, and the height of maximum ionization for the E, F1, and F2

layers for a specified sample area. The user may specify the following

parameters for each of the sample areas defined on the SAMPLE control cards

(see Control Card 14).

(a) I indicates the specific sample area number, 1 to 5.

(b) FI(1,l) indicates the critical frequency for the E layer, foE, in

megahertz for the specified sample area.

(c) YI(1,I) indicates the semi-thickness for E layer, ymE, in kilometers

for the specified sample area.

(d) HI(1,I) indicates the height of maximum ionization for the E layer,

hmE, in kilometers for the specified sample area.

(e) FI(2,I) indicates the critical frequency for the F1 layer, foF1, in

megahertz for the specified sample area. 22

(f) YI(2,I) indicates the semi-thickness for the F1 layer, ymF1, in

kilometers for the specified sample area.

(g) HI(2,I) indicates the height of maximum ionization for the F1 layer,

hmF1, in kilometers for the specified sample area.

(h) FI(3,I) indicates the critical frequency for the F2 layer, foF2, in

megahertz for the specified sample area.

(i) YI(3,I) indicates the semi-thickness for the F2 layer, ymF2, in

kilometers for the specified sample area.

(j) HI(3,I) indicates the height of maximum ionization for the F2 layer,

hmF2, in kilometers for the specified sample area.

ESVAR control card

The ESVAR control card allows the user to indicate lower, median, and upper

decile values for the Es layer and the virtual height of reflection for a

specified sample area. The user may specify the following parameters for each

of the sample areas defined on the SAMPLE control cards (see Control Card 15).

(a) I indicates the specific sample area number, 1 to 5.

(b) FS(I,I) indicates the lower decile critical frequency for the Es

layer, foEs, in megahertz for the specified sample area (example lowest

value = 2 MHz).

(c) FS(2,I) indicates the median decile critical frequency for the Es

layer, foEs, in megahertz for the specified sample area (example median

value = 4 MHz).

(d) FS(3,I) indicates the upper decile critical frequency for the Es

layer, foEs, in megahertz for the specified sample area (example highest

value = 6 MHz).

(e) HS(I) indicates the virtual height of reflection for the Es layer in

kilometers for the specified sample area (example vertical height = 110

kilometers).

EDP control card

The EDP control card allows the user to read and use an external electron

density profile for a specified sample area. Current implementation

restrictions prohibit defining more than one sample area if an external

electron density profile is used. The user may specify the following

parameters for one sample area (see Control Card 16). A temporary parameter is

used to switch to the internal electron density profile if desired.

(page 23)

(a) JSAMP indicates the specific sample area number (=1).

(b) ITEMP is a temporary parameter which causes the internal electron

density profile to be used if the user specifies the word "OFF." Thus, this

parameter acts as a switch to "turn off" the external electron density profile

and "turn on" the internal electron density profile. If not "OFF," then the

EDP control card is followed by eight cards which contain:

HTR is the array of 50 that indicates the true heights in kilometers,

(four cards, see Control Card 17).

FNSQ is the array of 50 that indicates plasma frequency squared in

(MHz)2 (four cards, see Control Card 18).

INTEGRATE control card

The INTEGRATE control card allows the user to specify model segment

integration for the F2 layer instead of the slower Gaussian integration. Thus,

the user can request fast integration for the E layer and for the F2 layer

when the F1 is not present. The D-E layer is preset using calculations from

the Gaussian integration. A temporary parameter is used to allow the user to

alternate between the model-segment and Gaussian integration. The program

performs the slow Gaussian integration as a default if no INTEGRATE card is

present.

(a) INTEG is the temporary parameter used to specify the integration

method. If the user specifies "-l" on the INTEGRATE control card, then

Gaussian integration is performed. If the user specifies a value greater than

or equal to zero, then model-segment integration is performed. If "OFF" is

specified (variable ITEMP), the program returns to the fast integration (see

Control Card 19).

FPROB control card

The FPROB control card allows the user to adjust the heights of the E, F1,

F2, and Es ionospheric layers. The predicted critical frequencies are

multiplied by the parameters on the FPROB card to raise or lower each

corresponding ionospheric layer. If the user-defined parameter is greater than

one, the critical frequencies used by the program will be larger than the

predicted critical frequencies. If the user-defined parameter is less than

one, the critical frequencies used by the program will be smaller than the

predicted critical frequencies. The user may remove the F1 and Es layers

entirely by specifying the value "0.0" for the critical frequency multiplier

for either layer. The user should not, however, specify the value "0.0" for

the critical frequency multiplier for the E or F2 layers. (page 24)

The following default values are used if (1) the critical frequency

multipliers are not specified by the user or (2) the value specified for the E

or F2 layer is negative or zero. The program uses the default value of 1.0 for

the critical frequency multipliers for the E, F1, and F2 layers. Thus, the

actual predicted critical frequency values are used by the program and the

corresponding ionospheric layer ionization is neither raised nor lowered. The

default value of the critical frequency multiplier for the Es layer is 0.7 to

allow for median losses. A temporary parameter is specified if the user

desires to use the default values for the actual frequency multipliers rather

than the values previously specified on the FPROB card. If the user indicates

"OFF" on a subsequent FPROB control card, the default critical frequency

multiplier values are used. The user may also redefine the critical frequency

multipliers previously defined by providing a subsequent FPROB control card.

The following parameters are on the FPROB control card (see Control Card 20):

(a) PSC(1) indicates the critical frequency multiplier for the E layer.

(b) PSC(2) indicates the critical frequency multiplier for the F1 layer.

(c) PSC(3) indicates the critical frequency multiplier for the F2 layer.

(d) PSC(4) indicates the critical frequency multiplier for the Es layer.

3.3.5  Input, Output, and Comment Control Cards

The following control cards allow the user to place comments in the input

file, use auxiliary input and output files, and identify additional

input/output options available to the user. These control cards are used to

(1) create an external antenna pattern file and (2) allow the user to request

the outputs of various program options without recomputing parameters.

COMMENT control card

The COMMENT control card permits the user to place comments anywhere in

the user-defined input file. This may benefit the user in specifying the

system configuration in terms of descriptive text, and any number of comment

cards may be used. During program execution, the comment cards are listed

along with the other control cards, but otherwise they have no effect on

execution of the program (see Control Card 21).

AUXIN control card

The AUXIN control card allows the user to read all, or a portion, of the

user-defined data from an auxiliary input file. This may be useful if the user

has a portion of the system configuration stored on a permanent file or

working file generated by some program before running IONCAP. The user may

then use the primary input file to read part of the input data and the

auxiliary input file to read the remainder. (page 25) Thus, in essence, it is

possible to specify the user-defined data from two files within the same run.

A temporary alphanumeric parameter on the AUXIN control card is used to

specify when the auxiliary input file is used (see Control Card 22). If the

temporary parameter is left blank, the control cards that follow the AUXIN

card are read from the auxiliary input file. This process continues until the

user specifies a second AUXIN control card with the temporary parameter set to

"OFF." The value "OFF" indicates that the control cards that follow the AUXIN

card are once again read from the primary input file. It is, therefore,

possible for the user to alternate between the primary and auxiliary input

files several times in one run as demonstrated in Figure 10. If there is no

AUXIN control card specified, then all control cards and input form the

primary input file.

AUXOUT control card

The AUXOUT control card allows the user to write all or a portion of the

program generated output to the auxiliary output file. Therefore, it is

possible to write on two files within the same run. This may be desirable if a

portion of the generated output is to be saved as a permanent file or if two

separate reports are generated. A temporary alphanumeric parameter on the

AUXOUT control card is used to specify when the auxiliary output file is used

(see Control Card 23). If the temporary parameter is left blank, the output

generated by the program that follows the AUXOUT card is written to the

auxiliary output file. This continues until the user specifies a second AUXOUT

control card with the temporary parameter set to "OFF." The value "OFF"

indicates that the program generated output that follows the AUXOUT card is

once again written to the primary output file. It is, therefore, possible for

the user to alternate between the primary and auxiliary output files several

times in one run as demonstrated in Figure 11. If there is no AUXOUT control

card specified, then all program generated printed output is written to the

primary output file.

ANTOUT control card

The ANTOUT control card allows the user to write antenna patterns

generated by the IONCAP program to a separate binary file if the program

option to generate antenna patterns is used (methods 13, 14, 15). This file

could then be saved as a permanent file for use at a later date with the

IONCAP program, or possibly with some other analysis program. This may be a

desirable capability for the user who plans to use the same antennas for

several sets of read data or to use the same antenna configuration for several

periodic runs. (page 26) The user could generate and save the antenna patterns

on a permanent file, then read the antennas from the permanent file when

running each of his data sets. It is also possible for the file to be used as

a working file for direct input to some other analysis program. The process of

reading the antenna file is discussed in Section 6.

A temporary alphanumeric parameter on the ANTOUT control card is used to

specify when antenna patterns are output to the antenna file (see Control Card

24). If the temporary parameter is left blank, then all antenna patterns

generated following the ANTOUT card are written to the file. If the temporary

parameter is set to "OFF," then none of the antenna patterns generated

following the ANTOUT card are written to the file. It is often more efficient

to generate the desired antenna patterns before performing other program

analysis or predictions. This process is demonstrated in Figure 12, where

transmitter/receiver antenna pattern pairs are generated.

The user should be aware that the antenna patterns are generated and

written on the antenna file only if methods 13, 14, or 15 are specified. Thus,

if the user has an ANTOUT control card but does not specify methods 13, 14, or

15, no antenna patterns are generated for output and consequently none are

written to the antenna file. Also, should the user specify methods 13, 14, or

15 but not include an ANTOUT control card, then the antenna patterns are

generated and written to the primary (or auxiliary) output file along with any

other program generated output, but the patterns are not written to the binary

antenna file. A method of using antenna patterns generated by the IONCAP

program or by other means available to the user, is described in Section 6.3.

OUTGRAPH control card

The OUTGRAPH control card allows the user to specify the output of several

methods without recomputing the variables. This is of particular use if

several line printer graphs or other output forms are needed for various

combinations of variables. A temporary alphanumeric parameter on the OUTGRAPH

control card is used to indicate if additional output is requested for the

current system configuration (see Control Card 25). If the word "OFF" is

specified, then no additional output is desired. If the user desires

additional output, he simply indicates the method numbers (up to 12) of the

output desired on the OUTGRAPH card. As an example, if method 7 is run, the

output from methods 8, 9, and 10 can be obtained without any additional

computing as is demonstrated in Figure 13.

KTOUT is an array of up to 12 method numbers of the desired output.

(page 27)

The user should be aware that the particular program option specified on

the METHOD card dictates the output that can be requested on the OUTGRAPH

card. There are two limitations as to the output available with certain

program analysis options. (1) The user may not request output on the OUTGRAPH

card that requires computations which were not performed during the program

analysis. As an example, the user may not request output for methods which

require system performance if only the MUF was computed. (2) Several of the

analysis options are performed within loops where the variables are not saved

throughout the duration of the loop. Such analysis options cannot be requested

on the OUTGRAPH card since the loops are terminated before the output

requested on the OUTGRAPH card is generated. This implies that the user cannot

always obtain the additional output by using the OUTGRAPH card. It is,

therefore, optimal to run the program analysis options (specified on the

METHOD card) that are performed within the loops and specify any additional

desired output on the OUTGRAPH card.

Should the user specify output for a method on the OUTGRAPH card which

violates one of the above restrictions, the program ignores the particular

request and attempts to process any additional user requests for output. Thus,

as an example, if the user should specify output for methods which require

system performance and only the MUF was computed, the program would ignore the

requests. Table 7 indicates the output available for methods specified on the

OUTGRAPH card for each of the program analysis task options and also indicates

the optimal combinations. The user may request LUF output (methods 26-29) if

system performance has been calculated by methods 16-25. The LUF obtained

would not be the computed LUF but would be the first frequency in the

frequency complement that has a computed reliability greater than or equal to

the required circuit reliability. If none if the reliabilities are as large as

the required reliability, the frequency with the largest reliability is chosen

as the LUF. A designator is printed to indicate the reliability. The user

would compute the actual LUF by specifying methods 26-29 rather than

specifying this on the OUTGRAPH control card. The OUTGRAPH card acts as a

software implemented "sense switch." Once the user specifies a desired output

on an OUTGRAPH card, the user "enables" the software implemented "sense

switch" and will continue to obtain the specified output options (when

permissible) for every program analysis task he performs. This process

continues until the user specifies either (1) an additional OUTGRAPH card or

(2) an OUTGRAPH card with the word "OFF" specified, which disables the

software implementation "sense switch." Should the user obtain much more

output than he believes he has requested when using an OUTGRAPH card and when

processing several system configurations, it is probable that he needs an

OUTGRAPH "OFF" card somewhere (usually following an EXECUTE card). (page 28)

Should the user obtain less output than he has requested on an OUTGRAPH card,

it is caused by an invalid user request that the program has ignored. The

OUTGRAPH card may be specified anywhere before the EXECUTE card to request

additional output options; however, the additional output will be generated

after the current task option has been completed. If no OUTGRAPH card appears,

the user receives only the output of the task option indicated on the METHOD

card.

The output requested on the OUTGRAPH control card may be written to the

auxiliary output file by specifying a negative method on the OUTGRAPH card.

TOPLINES control card

The TOPLINES control card allows the user to select the output header

lines he desires to print when running method 23. The information output, for

the most part, consists of the user-defined system configuration. This control

card, along with the BOTLINES control card, allows the user to indicate the

specific output lines he desires and thus together they constitute a

user-defined output option. The user can select the output desired rather than

those defined by other method numbers. There is one parameter array on the

TOPLINES card. This option may be useful if the user does not desire the exact

output generated by the task options implemented (see Control Card 26).

LINTP is an array of up to 8 user-selected header lines.

Table 8 describes the header lines available to the user, along with the

corresponding values of the parameter LINTP. The user may redefine the header

lines desired at any point in the input deck by specifying an additional

TOPLINES card which overrides the previous specified lines. The user may also

cancel the header lines previously selected by indicating the word "OFF" as

the value of a temporary parameter on the TOPLINES card. The program has one

default header line which consists of the program task option (METHOD), the

program name and version number, as well as the page number. If the user does

not include a TOPLINES card or specifies TOPLINES "OFF," the default header

line is the only one printed when method 23 is indicated. The TOPLINES card

has no effect on any program task option other than method 23. BOTLINES

control card

The BOTLINES control card allows the user to select the desired parameters

to print when running method 23. The information output consists of the

current values of the variables the user indicates. (page 29) This control

card, along with the TOPLINES control card, allows the user to indicate the

specific output lines desired, and thus constitutes a user-defined output

option. The user may, therefore, request the desired output rather than those

specified by other method numbers. This may be useful if the user does not

desire the exact output generated by the other task options. There is one

parameter array on the BOTLINES card (see Control Card 27).

LINBD is an array of up to 14 user-selected variables.

Table 9 describes the variables available to the user along with the

corresponding values of the parameter LINBD. The user may redefine the

variables desired at any point by specifying an additional BOTLINES card which

overrides the previous specified variables. The user also may cancel the

variables previously selected by indicating the word "OFF" as the value of a

temporary parameter on the BOTLINES card. The program outputs only the

frequency complement as the default if the user does not include a BOTLINES

card or specifies BOTLINES "OFF" when method 23 is indicated. The BOTLINES

card has no effect on any program task option other than method 23. Table 10

describes the line numbers preset by using methods other than 23.

FREEFORM control card

NOTE: The freeform processor has not been developed for the IONCAP 78.03

version. Should the user specify a FREEFORM control card, the program will

output a message to that effect and terminate program execution. Therefore,

the user-defined control cards must be formatted card images, which are

described in Section 3.4.

DEBUG control card

The DEBUG control card was implemented during the development of the

IONCAP program as an aid in the validation of the program code. The use of the

DEBUG card generated line printer output of temporary variables, iterative

values of variables, and other information used to confirm the program

analysis and predictions. Therefore, the DEBUG control card acts as a "do

nothing" statement when specified by the user. The DEBUG control card is

ignored, and the program processes the next control card (see Control Card

29).

3.3.6.   Procedure Related Control Cards

The general purpose of an input procedure is to allow the user the

capability of referencing a set of control cards by a single name identifier.

In other words, it allows the user to reference several control cards by using

a single control card. (page 30) This could be useful if the set of control

cards is to be referenced multiple times. The methodology and general program

flow of creating an input procedure was discussed in Section 2.3. The

following control cards describe the definition and usage of an input

procedure. An input procedure may be defined anywhere in the user-defined

input file since the procedure file is created during Phase 1, which occurs

before execution of the analysis phase of the program. This allows the user to

define a procedure after a reference is made to it if he so desires. It is,

however, optimal to define the input procedures at the beginning of the

user-defined input.

PROCEDURE control card

The PROCEDURE control card identifies the name of an input procedure and

thus constitutes the beginning of the procedure definition. The user-defined

procedure name is the only parameter on the PROCEDURE card (see Control Card

30). This temporary parameter is, as previously discussed, compared with and

added to the table of reserved name identifiers that constitute the set of

valid input control cards. The user may specify any name identifier for his

procedure except those reserved control card identifiers which were given in

Table 2. The program ignores the procedure definition and continues to process

the remaining control cards should the user attempt to define a procedure with

a reserved name identifier. All control cards following the PROCEDURE control

card and preceding the END control card constitute the body of the procedure.

Should the user specify two input procedures with the same name, the first

definition takes preference and the other definition is ignored. END control

card. The END control card terminates an input procedure definition. Each

user-defined input procedure must be terminated with an END control card since

it is used (1) as a delimiter when creating the procedure file during the

first phase of program execution and (2) indicates that the control cards

following it are to be read from either the primary or auxiliary input file

instead of the procedure file when encountered during the analysis phase of

program execution. Thus, the END card indicates that the control cards

following are not on the procedure file (see Control Card 31).

"USER DEFINED PROCEDURE NAME" control card

The "USER DEFINED PROCEDURE NAME" control card replaces the reference to

the procedure name with the actual control cards which constitute the

procedure (page 31) The characters on this card are to correspond to those

used as a parameter on the procedure card. When the "USER DEFINED PROCEDURE

NAME" is used as a control card, the procedure file is searched to locate the

specified procedure. The control cards on the procedure file that occur before

the END card are then processed as if they had occurred on the primary or

auxiliary input files. A procedure file structure example is given in Figure

14.

3.4  Input Data Sheet

The required input data can be tabulated before data cards need to be

punched by completing data sheets as given in Tables 11 and 12. The data

required depend on the complexity of the computations requested by the user.

The relationship between the input data and the output options is shown in

Table 13. The principal options are ionospheric parameters, methods 1 and 2;

antenna patterns, only methods 13, 14, and 15; MUFs, methods 3 to 12; or

system performance and LUF, methods 16 to 29. See Section 4 for a description

of the output options. These data sheets are formatted for the convenience of

the card punch operator in the preparation of the input data cards, as

described in Section 3.3, and may also be used when requesting predictions

from ITS. Often the user who has gained some experience running the IONCAP

program will not need to prepare the data sheets, presented to assist in the

preparation of these data sheets.

TABLE 11:

1. General Information. Lines 1 and 2 are self-explanatory and provide the

identification of the person or organization requesting the prediction.

1(A) Year - If predictions are required for a specific time, the year

should be specified. If predictions are for long-term planning, the general

level of solar activity for the time period may be specified instead of the

year; e.g., high, medium, low. In either case, the 12-month running average

sunspot number (SSN) may be given. If the sunspot number is not given and

predictions are requested from ITS, an appropriate number will be chosen. SSN

10 is typical for low solar activity and SSN 110-130 is typical for high solar

activity. Monthly or daily solar activity indices are never used, only the

expected 12-month running average.

1(B) Months - Circle the month or months for which predictions are

required. June and December are typical of seasonal extremes for long-range

planning.

1(C) Transmitter Site Electrical Characteristics - Specify the ground

conductivity in mhos per meter and the relative dielectric constant at the

transmitting site. (page 32) Typical values for selected ground types are

given in Table 6. If the electrical characteristics are unknown, the site may

be described as sea water, or as good, fair, or poor ground. Typical good

ground is low hills with unforested, rich soil or flat, wet coastal regions;

fair ground is medium hills or forested heavy soil; and typical poor ground

includes rocky, steep hills, sandy, dry coastal regions, and city industrial

areas.

1(D) Receiver Site Electrical Characteristics The same as (C), above, but

for the receiver site.

2. Program Control Information.

2(A) Type Tabulation (Method) Desired - Table 3 summarizes the major

outputs available for each tabulation, and Section 4 describes these outputs.

Select the outputs required, and then choose the tabulation which provides

this output. The tabulation number identification is also the identification

of the "METHOD." Sample tabulations and detailed descriptions are shown in

Section 4.

2(B) Beginning Hour of Calculation - The first hour for which predictions

are required expressed in two digits; i.e., fractional hours may not be

specified.

2(C) Ending Hour of Calculation - The last hour for which predictions are

required.

2(D) Increment Hour of Calculation - The hourly increment of the

computations. For graphs requiring the calculation of LUFs, it is recommended

that the beginning hour be 02, the ending hour 24, and the increment 2. The

hours are UT or LMT at the transmitter site dependent on specification by the

user.

2(E) Antenna Pattern Tabulation - In all methods that require antenna

characteristics, a tabulation of antenna gains is available. Indicate the

tabulations desired.

2(F) Great Circle Path - Except for very long paths, only the short great

circle is usually considered. If both long and short paths are required,

separate computations are necessary.

3. Frequency Complement. The frequencies that should be considered. A choice

of up to 11 frequencies is possible. See the FREQUENCY control card for

additional information.

4. Circuit and System Performance Parameters.

4(A) Transmitter Location - The name and geographic coordinates expressed

in degrees.

4(B) Receiver Location - As in 4(A) above.

4(C) Minimum Takeoff Angle - Normally 3.0 unless antenna performance is

expected to be so poor at low angles that these angles should not be used in

the estimation of upper useful frequency limits or if the horizon is so

obstructed that low takeoff and reception angles appear unlikely. (page 33)

4(D) Transmitter Power - The power delivered to the transmitting antenna

expressed in kilowatts.

4(E) Required S/N Ratio - The ratio in decibels of the-hourly median

signal power in the occupied bandwidth relative to the hourly median noise in

a 1 Hz bandwidth which is necessary to provide the type and quality of service

required. Tables 4 and 5 show typical required S/N ratios. If the required S/N

ratio is unknown, the required service may be given. Both the type and quality

must be specified.

4(F) Man-Made Noise at the Receiver Site Either the type of area

(industrial, residential, rural, or remote) or the expected noise in a 1 Hz

bandwidth at 3 MHz may be specified. Industrial area man-made noise is -125.

dBW, residential is-136. dBW, rural is -148. dBW, and remote is -164. dBW. In

remote unpopulous areas, cosmic noise will normally dominate over man-made

noise. Specify this as a positive number unless one of the above defaults is

desired.

4(G) Circuit Reliability Required - When a lowest usable frequency (LUF)

or service probability computation is required, a minimum acceptable circuit

reliability must be specified. A typical value is 90 percent.

4(H) Multipath Computation Required - Multipath probability is an estimate

of the likelihood that two (or more) skywave propagation modes will exist

within specified power and time delay tolerances.

4(I) Minimum Tolerable Power Ratio - The minimum tolerable power ratio

between skywave modes at the receiver input terminal to permit satisfactory

system performance in the presence of multiple signals.

4(J) Maximum Tolerable Time Delay - The maximum tolerable difference in

delay times between skywave propagation modes (with power levels within the

4(I) criteria above) to permit satisfactory system performance in the presence

of multiple signals.

TABLE 12:

Table 12, an extension of Table 11, is designed to record the required

antenna information.

5. Antenna Parameters. Indicate the data required for the 11 antenna types.

Figures 43 through 65 show the geometry involved to assist in the completion

of Section 5 of Table 12 and also reflect the antenna gain patterns

corresponding to the sample input data cards in Section 3.5. If an antenna

other than the types shown is being used, its performance may sometimes be

approximated by selecting an antenna with a similar pattern. (page 34) Some

simple antennas (e.g., dipoles and verticals), permit the inclusion of

additional gain to assist in this approximation. If this is not adequate, the

program will allow reading of externally computed (or measured) patterns. This

option may, however, require in-depth knowledge of the program and usually a

software interface to IONCAP.

6. Transmitting Antennas. Tabulate the type of transmitting antenna or

antennas to be used. The beginning and ending frequency for each antenna must

be included in all cases, and these frequencies must not overlap. Up to three

antennas may be used for the transmitter and/or for the receiver.

7. Receiving antennas. Tabulate similar information for the receiving

antennas. All comments in (6) above apply.

3.5  Input Data Card Formats

This section lists the details of all the data cards valid for the IONCAP

input file. All card formats and comments are given in Tables 14 and 15. This

is followed by detailed control card diagrams for each card type, except the

antenna cards. Table 16 gives examples of all the antenna cards where the

control card diagrams (control cards 32-43) give the details of each antenna

card. Section 3.3 describes the use of the cards, and Section 4 describes the

output. Section 7 contains specific input examples.

Each card format consists of an alphanumeric name, up to 10 characters,

followed by up to 14 data fields of five columns each. Each floating point

variable may have a decimal inserted, while the integer variables must be

right justified. Floating variables are read in an F5.1 format, except for the

geographic latitudes and longitudes and the frequency card (CIRCUIT and SAMPLE

cards) which are F5.2 format. Thus "12345" in the field is the number 1234.5

for all variables except the latitude or longitude variables which become

123.45. The hemisphere is specified by immediately following the number by

"NORTH." "SOUTH," "EAST," or "WEST" or "E," "W" as appropriate.

4.  OUTPUT OPTIONS

There are 30 output options that may be specified by the user (Table 3).

These are divided into four subsets:

(1) ionospheric descriptions, METHOD = 1 or 2;

(2) antenna patterns, METHOD 13, 14, or 15;

(3) MUF predictions, METHOD 3 through 12; and

(4) LUF and system performance predictions, METHOD = 16 through 29.

(page 35)

The input required for each of the four run option subsets is summarized

in Table 13. All the output figures described in the next three subsections

were generated using the data cards listed in Figure 15. Each individual

antenna type is described and an example given in the last subsection 4.4. The

first line of all the output options gives the method number, program version,

and a page number, the start of which is specified by the user.

4.1  Ionospheric Parameters Output Options, METHOD = 1 or 2

There are two outputs, a list of ionospheric parameters (Figure 16) and an

ionogram (Figure 17). The data on Figure 16 are divided into a set of header

lines and a body of output lines.

Line 2. The month, year, and sunspot number are given on the second line.

Line 3. The first 40 characters on the third line are those specified by the

user on the LABEL card.

Line 4. The fourth line is the location of the transmitter and of the

receiver. The azimuths, transmitter to receiver, and receiver to transmitter,

are given in degrees east of north. The circuit distance is given in both

nautical miles and kilometers.

Line 5. The semi-thickness of the E layer, YE, the height of maximum

ionization HE, and the reflection height of the Es layer, HS, are given in

kilometers.

The body of the figure consists of the result of sampling the ionosphere

at one, three, or five areas along the path. (There is only one sample area

for this specific example.) Columns 1 and 2 give the sample area location.

Columns 3 and 4 give the local time at the sample area and universal time

(same for all sample areas). Column 5 is the median E critical frequency, foE,

in megahertz. Columns 6, 7, and 8 are the F1-layer median critical frequency,

foF1, in megahertz; semi thickness, ymF1, in kilometers; and height of maximum

ionization, hmF1, in kilometers. Column 9 is one-half the gyrofrequency, fH.

Columns 10, 11, and 12 are F2 zero MUF, (foF2 + 112 fH), in megahertz; ymF2 in

kilometers; and hmF2 in kilometers. Columns 13, 14, and 15 are lower, median,

and upper decile from the maps of the minimum of foE or foEs, in megahertz.

Note that a FPROB card was used to adjust the numbers to the predicted median

(1.0 on all data fields). The normal (default) value is 0.7 of that predicted.

Column 16 is the F2 M(3000) factor. Column 17 is the virtual height, hpF2, in

kilometers. Column 18 is the ratio of hmF2 and ymF2. Columns 19 and 20 are the

zenith angle and the maximum zenith angle at which the F1 layer exists, both

in degrees. Column 21 is the geomagnetic latitude. (page 36)

The ionogram output option, method 2, is in Figure 17. Up to three

ionograms are constructed from the up to five sample areas along the circuit

path. Lines 2, 3, and 4 are the same as for Figure 16. Line 5 gives the

universal time, GMT, the local time at the sample area, the sample area

location, the distances in kilometers from the transmitter at which the E and

F parameters were taken, the type of integration used, and the manner in which

the Fl layer was added to the electron density profile. The graph is vertical

sounding frequency (in megahertz) versus true and virtual heights (in

kilometers) where true and virtual height are represented by "." and "x"

respectively in the graph. The Es layer is given by a line at the Es

reflection height. The character "U" is used until the 90 percent value of

foEs is exceeded; "M" is used until the 50 percent value is exceeded; and "L"

is used until the 10 percent value is exceeded. The parameters for each layer

are given in the upper left corner. The table on the right gives the sounding

frequencies and the true and virtual heights.

4.2  MUF Output Options, METHOD = 3 through 12

The maximum usable frequency (MUF) output options include all mode

information as well as the distributions of the MUF for each layer. First note

that methods 3, 4, 5, and 6 refer to the MUFs for the E(F1) and F2 layers

using the old nomogram method. This assumes a virtual height of about 300 km.

The results of this model are not always valid in the 4000-10000 km range. The

other MUF output comes from a complete electron density profile. All

information possible from the MUF calculation are given in Figure 18. Lines 2,

3, and 4 are as described for Figure 16. Line 5 is the minimum radiation angle

used to determine the MUF. The body of the figure is composed of two lines for

each hour, consisting of MUF information for each layer. Column 1 is universal

time. Column 2 is local time at the transmitter. The next seven columns are

for the E layer on one line and the F2 layer on the following. Column 3 is the

10 percent value of the MUF; i.e., the FOT, MHz. The MUF during a month is

expected to exceed the FOT 90 percent of the days, and to be less than the FOT

10 percent of the days. Column 4 is the 50 percent value of the MUF in

megahertz. Column 5 is the 90 percent value of the MUF; i.e., the HPF in

megahertz. Column 6 is the radiation angle in degrees. Column 7 is the virtual

height of reflection in kilometers. Column 8 is the true height of reflection

in kilometers. Column 9 is the equivalent vertical frequency in megahertz.

Columns 10 to 16 repeat the same information for the Fl layer on the first

line and the Es layer on the second line. Note that, if the Fl layer is

missing, the E values are used. Figures 19, 20, 21, and 22 are diurnal plots

and tables of some of these parameters that have proved useful to analysis in

the past.                   (page 37)

4.3 System Performance Output Options, METHOD = 16 through 24

The prediction of the performance of a communication system operating

between two points on the earth's surface is the main output of the IONCAP

program. There are two principle forms of output: first, a table of up to 22

performance variables, and, second, a table or graph of the lowest usable high

frequency (LUF). The program has been written in such a way that the user may

specify the system performance lines desired. The header identification lines

can also be selected. Table 8 shows the eight toplines available; Table 9

shows the 22 system performance lines available; Table 10 shows the line types

specified by using the METHOD card. Table 7 shows the diurnal graphs that may

be combined with the line selections.

Figures 23, 24, and 25 show a typical run with system performance output

(method 16 with precalculation of antenna patterns). Figures 23 and 24 are

tables of antenna gain in decibels at frequencies from 2 to 30 MHz and

radiation angles from 0 to 90 degrees. These patterns and the input for each

antenna type are described in detail in Section 4.4.

In METHOD 16, the header line information is largely input data supplied

by the user.

Line 0: This appears on each output page and includes the method, program

version, and page number. User selected header lines are:

Line 1: Month, year, and sunspot number.

Line 2: Label as supplied by user, and headings for next line.

Line 3: Transmitter location, receiver location, the azimuth of the

transmitter to the receiver in degrees east of north; the azimuth of the

receiver to the transmitter in degrees east of north; path distance in

nautical miles and kilometers.

Line 4: Minimum radiation angle in degrees.

Line 5: Antenna subroutine used (defining the antenna module used in IONCAP).

Line 6: Transmitter antenna line; up to three may be printed. Information

varies with antenna type (see Section 3.4). Typically, this is frequency

range, antenna type, height of antenna above ground in meters (- is

wavelengths), length of active part, an angle associated with the antenna

structure in degrees, and the off main beam azimuth in degrees. Note, lengths

specified should be "electrical" lengths.

Line 6A: Receiver antenna line. Up to three may be printed. The data are

specified similar to the transmitter antenna line. (page 38)

Line 7: System line which has: transmitter power in kilowatts, man-made noise

level at 3 MHz in dBW, required reliability and required signal-to-noise ratio

in decibels.

Line 8: Multipath line which has required power tolerance in decibels and

required delay time tolerance in milliseconds.

The system performance lines are repeated each desired hour until the page

is filled. The process is repeated if additional pages are necessary. The

columns are time (universal) and up to 12 frequencies in megahertz; the first

is the MUF and the remainder the frequency complement. Figures 25 to 32 give

the various system performance output defined by methods 16 to 23 as indicated

in Table 3. All possible output lines for system performance are as follows

and shown in Figure 29.

Line 0, FREQ: Time and frequency line as associated with each column (default

line always printed). The first four lines always refer to the most reliable

mode (MRM). The system performance parameter usually comes from the sum of all

six modes. If no decision can be made, the selection of MRM is based on number

of hops and then on SNR. All distribution refers to a diurnal period of an

hour and a yearly period of a month.

Line 1, MODE: For the short path model, number of hop and mode type. E is E

layer; Fl is Fl layer; F2 is F2 layer; ES is Es layer; N is a one hop Es with

n Fl or F2 hops. (MRM) For the long path model (Figure 30), the mode at the

transmitter end and the mode at the receiver end are given. The user can force

the long path model by specifying method 21 regardless of the actual path

length.

Line 2, ANGLE: The radiation angle in degrees; for long paths (Figure 30), two

lines are given: the first for the transmitter end, and the second for the

receiver end. (MRM)

Line 3, DELAY: Time delay in milliseconds. (MRM)

Line 4, V HITE: Virtual height in kilometers. (MRM)

Line 5, F DAYS: The probability that the operating frequency will exceed the

predicted MUF.

Line 6, LOSS: Median system loss in decibels for the most reliable mode.

Line 7, DBU: Median field strength expected at the receiver location in

decibels above 1 microvolt per meter.

Line 8, S DBW: Median signal power expected at the receiver input terminals in

decibels above a watt. (page 39)

Line 9, N DBW: Median noise power expected at the receiver in decibels above a

watt.

Line 10, SNR: Median signal-to-noise-ratio in decibels.

Line 11, RPWRG: Required combination of transmitter power and antenna gains

needed to achieve the required reliability in decibels.

Line 12, REL: Reliability. The probability that the SNR exceeds the required

SNR. Note this applies to all days of the month and includes the effect of all

mode types: E, (Fl), F2, Es, and over-the-MUF modes.

Line 13, MPROB: The probability of an additional mode within the multipath

tolerances (short paths only).

Line 14, S PRB: Service probability. The probability that the required

reliability will be met.

Line 15, SIG LW: Lower decile signal power (field strength and loss) increment

in decibels. Line 16, SIG UP: Upper decile signal power (field strength and

loss) increment decibels.

Lines 17, 18, 19, and 20 are experimental over-the-MUF to VHF scatter

parameters an should be ignored by most users.

Line 17, VHFDBU: VHF median field strength in decibels above 1 microvolt per

meter.

Line 18, VHF LW: VHF lower decile increment in decibels.

Line 19, VHF UP: VHF upper decile increment in decibels.

Line 20, VHFMOD: VHF mode type.

Line 21, SNR LW: Lower decile SNR increment in decibels.

Line 22, SNR UP: Upper decile SNR increment in decibels.

A composite table of reliabilities only is given in Figure 33 which comes

from the user-selecting method 24. A similar table for any one variable can be

selected by running method 23 and selecting the desired line. However, a

frequency line will be printed each hour. If information for each mode is

required, method 25 will produce the data in Figures 34 and 35. For each hour,

the ionospheric data, as in method 1, is printed. For each frequency, the mode

and signal parameters are printed for each mode up to six. The last column is

the selected most reliable mode. The same information is provided for the MUF.

Methods 26, 27, 28, and 29 will calculate the LUF, the lowest frequency

having a specified circuit reliability, usually 0.90. Figure 36 is a table of

LUF values with the MUF distribution and the median Es layer MUF. Figures 37,

38, and 39 are available diurnal plots of MUF prediction along with the LUF.

Figures 40 and 41 provide MUF and system performance predictions for a long

path (greater than 10000 km).

4.4  Antenna Output Options, METHOD = 13, 14, or 15

The detailed description of each antenna pattern and the required input

definitions are given in this section. The gain subroutines used in the IONCAP

are approximate models using the "one-tem" theory and assume that the antenna

parameters are within the design limits of each antenna and that the operating

frequency is such that the antenna is close to resonance. These antenna models

are appropriate when used in a propagation model where other uncertainties

overshadow the uncertainties in the gain due to the antenna mode limitations.

However, it is not appropriate to use these models to design antennas or to

evaluate their performance outside of their design limits or far from

resonance. Also, they will not typically produce accurate results for

full-wavelength or multiple-full-wavelength antennas again due to the

limitations of one-term theory. The submodule described here also assumes the

main beams are pointed along the circuit path. The input data card images in

Figure 42 were used to generate the patterns of this section. Figures 43 to 65

are pairs for each pattern, the first giving the structure of the antenna and

the second giving a sample pattern.

5.  APPLICATIONS

The primary application of the IONCAP program is to use the system

performance options to select a frequency complement. While intended mainly

for program test and evaluation procedures, the other output options, if used

with some interpretation, may provide the analyst with enough information to

solve a particular problem. Some possible applications will be discussed in

the same order as the program complexity; i.e., as

(1) ionospheric descriptions,

(2) MUF predictions,

(3) system performance predictions, and

(4) antenna applications

5.1  Ionospheric Parameters Applications

The ionospheric description output consists of a table of parameters

(Figure 16) and a graph and table of ionograms (Figure 17). The parameters are

the output of the long-term world maps of the ionospheric parameters. The

accuracy of the maps were determined when they were generated. The IONCAP

theoretical report has details and references to these maps. The ionogram

output includes the vertical sounding frequency and the virtual height, so

that this ionogram may be compared to measured ionograms. (page 41) Note,

however, a monthly median may not be characteristic of that for a given day.

An example of a predicted ionogram superimposed on a record of a month's

measurements is given in Figure 66. The arrow points to the selected median

ionogram, while the circles are from the IONCAP program. The effect of the

changes of critical frequencies from the maps may be studied using the FPROB

card (Control Card 20) or by use of the EFVAR and ESVAR cards (Control Cards

14 and 15).

5.2  MUF Applications

In the absence of any other criteria for the planning of a system using HF

skywave, the first, and simplest, criteria is an estimate of the frequency

having efficient ionospheric reflections. Normally, an estimate of the

frequencies expected to have efficient ionospheric support 90 percent of the

time, FOT, and those having efficient ionospheric support 50 percent of the

time, MUF, are adequate estimates of upper frequency limits for system

planning. Figures 18 through 22 show the possible MUF outputs. Note that

Figure 18 (METHOD=7) gives all mode information and MUF distribution for each

layer. These MUF calculations are only a description of the state of the

ionosphere and do not include any system parameters. They should not be

confused with the maximum operation frequency (MOF) for transmission between

two points on an existing circuit. A full system performance calculation

should be made to estimate the MOF or to compare with observed MOFs. If a full

system performance computation is generated, the user should examine the

frequency complement predictions rather than the MUF.

5.3  System Performance Applications

5.3.1  Selecting an Optimum Frequency

The complexities of propagation, the diversity of service requirements,

and the fluctuation of spectrum congestion preclude any clear simple criteria

for the selection of optimum frequencies. An adequate signal-to-noise ratio at

the receiver for the specified type and quality of service is often a useful

criterion. In general, within the HF spectrum, radio noise tends to decrease

as frequency is increased. During the daylight hours when HF power

requirements are highest, the propagation loss tends to decrease as frequency

is increased. Since the noise normally decreases and signal normally increases

with frequency, it is a general rule for HF skywave circuits that the higher

the frequency the better the signal-to-noise ratio until frequency is

increased to a point where reflection from the ionosphere becomes improbable.

A first approximation to the optimum frequency in the absence of interference

may, therefore, be made by estimating the highest frequency having an

efficient ionospheric reflection consistent with the circuit reliability

required; i.e., the MUF calculations described in Section 5.2 above. (page 42)

Since there is normally limited flexibility in the selection of

frequencies and since the optimum frequency based on the probable upper useful

frequency limit has marked diurnal, seasonal, and other variations, it is

desirable to establish the probable useful range of frequencies. The FOT as

shown is based on a 90 percent probability of efficient ionospheric support

and may be used as an estimate of the probable upper frequency limit; a

corresponding lower useful frequency limit may be estimated by considering the

probability that the available signal-to-noise ratio will be adequate. Since

noise normally increases as frequency decreases and signal normally decreases

as frequency decreases, there is usually a frequency below which the

probability of an adequate signal-to-noise ratio is unacceptable. This

probability is often set at 90 percent and the corresponding frequency is

known as the lowest useful frequency (LUF). These limits; i.e., FOT and LUF,

are shown in Figure 36 and graphically displayed in Figure 37. (page 43)

6.3  Antenna Patterns Stored on File

It is possible, but not mandatory, to execute the IONCAP program using

antenna patterns that have been previously computed by (a) IONCAP using the

ANTOUT control card discussed in Section 3.3.5, (b) some other analysis

program that has been modified to create and write antenna patterns in the

format expected by IONCAP or (c) to use field measurements and interpolation

to create and write antenna patterns in the format expected by IONCAP. This

use may require some in-depth knowledge of the working of IONCAP.

The antenna pattern input file (denoted by LU26) is stored on magnetic

tape or disk as an internal binary file. The user should assign the local file

name TAPE26 to the permanent tape or disk file prior to the execution of the

IONCAP program. It is, of course, assumed that the user has previously created

and stored the permanent file by one of the methods described above. The user

would then specify antenna type 18 on the ANTENNA control card to indicate

that the antenna pattern is to be input from the antenna file. The location of

the desired antenna pattern on the antenna file is specified using variable

IAIN for antenna type 18. (Note: This variable indicates from one to three

antennas to cover the frequency range specified for the transmitter and

receiver antennas for all antenna types except antenna type 18.)

It is necessary to specify the location of the desired antenna pattern on

the antenna file since several antenna patterns could be written on the same

antenna file. Locating the desired antenna pattern depends on whether the

antenna pattern is beyond or before the current location on the antenna file.

(page 57) If the desired antenna pattern is beyond the current position on the

antenna file, the variable IAIN indicates the number of antenna patterns to

skip forward from the present position on the antenna file before reading the

desired pattern. Thus, in this case, IAIN must be set to a positive number or

zero by the user. For example, IAIN = 0 indicates that no antenna patterns are

to be skipped from the current position, which means the next antenna pattern

on the file is read. If IAIN = 3, then three antenna patterns are to be

skipped from the current position before the desired antenna pattern is read.

If the desired antenna pattern is before the current location on the antenna

file, the variable IAIN indicates that the antenna pattern should be rewound

before searching for the desired pattern. This is accomplished by setting IAIN

negative. The negative number causes the file to be rewound, and the pattern

read is specified by the absolute value of IAIN. For example, IAIN = -1

indicates that the antenna file is rewound before reading the desired antenna

pattern and that the first pattern on the file is the antenna pattern read. If

IAIN = -3, then the antenna file is rewound before reading the desired antenna

pattern and the third pattern on the file is the antenna pattern read.

7.  SPECIFIC INPUT EXAMPLES

This section is intended to demonstrate specific input options of the

IONCAP program to reinforce information provided in earlier sections. Specific

input requirements necessary to demonstrate particular IONCAP capabilities are

discussed. References to corresponding output for each example is also

presented.

7.1  Ionospheric Parameter Example

The input required to generate the ionospheric parameters for the control

points of a particular circuit consists of the following control: METHOD,

MONTH, SUNSPOT, TIME, LABEL, CIRCUIT, EXECUTE and QUIT. The input necessary to

generate the similar output represented in Figure 16 consists of the

following:

METHOD        1

MONTH      1970    1

SUNSPOT    100.

TIME          1   24    1   -1

LABEL     BOULDER, COLORADO TO ST.  LOUIS, MO.

CIRCUIT   40.03N    105.3W    38.67N    90.25W

EXECUTE

QUIT

(page 58)

It should be noted that the sporadic-E critical frequency has been multiplied

by .7 to allow for median losses in this example by program default. Figure

16, discussed earlier, is a presentation of the ionospheric parameters for

this circuit without the sporadic-E critical frequency reduced.

7.2  Antenna Pattern Example

The input required to generate an antenna pattern for both a transmitter

and receiver antenna consists of the following control cards: METHOD, ANTENNA,

EXECUTE, and QUIT. The specific input necessary to generate the output

represented in Figures 23 and 24 consists of the following:

METHOD          15

ANTENNA          1     2       .001   4.      -.5

ANTENNA          2     2       .001   4.     -.25

EXECUTE

QUIT

To generate the pattern of a transmitter antenna, only use method 13 and for a

receiver antenna, only use method 14.

7.3  MUF Example

The input required to generate MUF predictions consists of the following

control cards: METHOD, MONTH, SUNSPOT, TIME, LABEL, CIRCUIT, EXECUTE, and

QUIT. The specific input necessary to generate the output represented in

Figure 18 consists of the following:

METHOD 7

MONTH         1970     1

SUNSPOT       100.

TIME            12  24  12   -1

LABEL      BOULDER, COLORADO TO St. LOUIS, MO.

CIRCUIT    40.03N      105.3W     38.67N     90.25W

EXECUTE

QUIT

7.4  System Performance Example

The input required to generate system performance predictions consists of

the following control cards: METHOD, MONTH, SUNSPOT, TIME, LABEL, CIRCUIT,

ANTENNA, FREQUENCY, EXECUTE, and QUIT. The input necessary to generate output

similar to the output represented in Figure 25 consists of the following:

(page 59)

METHOD          16

MONTH         1970     1

SUNSPOT       100.

TIME            12    24  12   -1

LABEL       BOULDER, COLORADO TO ST.  LOUIS, MO.

CIRCUIT    40.03N      105.3W      38.67N   90.25W

ANTENNA          1     2      .001    4.      -.5

ANTENNA          2     2      .001    4.     -.25

FREQUENCY      2.0   3.0  5.0  7.5  10.0  12.5  15.0 17.5 20.0 25.0 30.0

EXECUTE

QUIT

It should be noted that the above input does not cause the transmitter and

receiver antenna patterns to be precomputed as was the situation in Figure 25.

(This is indicated by the absence of method 15 card to compute the gain

pattern i this example.) The above input causes the antenna gain values to be

computed as needed for specific frequencies and elevation angles. This

indicates that the gain value for 7.5 MHz needed in Figure 25 was interpolated

between 7 MHz and 8 MHz. The gain value for 7.5 MHz in the above example is

computed as the frequency 7.5 MHz is processed. It should further be noted

that this value would be recomputed for each hour specified in the above

example. Thus, the gain is computed at each frequency and for each hour as

needed.

7.5  User-Selected Output Example

The input required to generate user-selected system performance

predictions consists of the following control cards: METHOD, TOPLINES,

BOTLINES, MONTH, SUNSPOT, TIME, LABEL, CIRCUIT, ANTENNA, FREQUENCY, EXECUTE,

and QUIT. The input necessary to generate output similar to the output

represented in Figure 32 consists of the following:

METHOD          23

TOPLINES         1     2   3     4     5    6     7

BOTLINES         1     2   4    10    11   12

MONTH         1970     1

SUNSPOT       100.

TIME            12    24  12    -1

LABEL      BOULDER, COLORADO    TO ST.  LOUIS, MO.

CIRCUIT     40.03N     105.3W     38.67N   90.25W

ANTENNA          1     2     .001    4.      -.5

ANTENNA          2     2       .001     4.  -.25

FREQUENCY     2.0  3.0   5.0  7.5 10.0 12.5 15.0  17.5 20.0 25.0 30.0

EXECUTE

QUIT

The header lines specified on the TOPLINES are given in Table 8 and are listed

below:

(1) month, year, and sunspot number

(2) alphanumeric information on the LABEL card

(3) transmitter and receiver information (coordinates)

(4) minimum takeoff angle

(5) transmitter antenna information

(6) receiver antenna information

(7) transmitter power, man-made noise, required reliability, and required

SNR.

The output predictions specified on the BOTLINES card are given in Table 9 and

listed below:

(1) number of hops and mode type

(2) radiation angle and transmitter

(4) virtual height of most reliable mode

(10) median SNR

(11) required power gain for most reliable mode

(12) reliability.

It should be noted that the above input does not cause the antenna pattern to

be precomputed as was the case in Figure 32. (This is indicated by the absence

of the method 15 card to compute the gain tables in this example.) The above

input would compute the gain value of each frequency for each hour as needed.

This has been discussed in Section 7.4.

7.6  LUF Example

The input required to generate LUF predictions consists of the following

control cards: METHOD, MONTH, SUNSPOT, TIME, LABEL, CIRCUIT, ANTENNA, EXECUTE,

and QUIT. The input necessary to generate output similar to the output

represented in Figure 36 consists of the following: (page 61)

METHOD          26

MONTH         1970     1

SUNSPOT       100.

TIME            2     24    2    -1

LABEL       BOULDER,  COLORADO  TO  ST.  LOUIS, MO.

CIRCUIT     40.03N     105.3W      38.67N   90.25W

ANTENNA         1      2      .001    4.        -.5

ANTENNA         2      2      .001    4.       -.25

EXECUTE

QUIT

It should be noted that the above input does not cause the antenna pattern to

be precomputed as was the situation in Figure 36. (This is indicated above by

the absence of the method 15 card to compute the gain tables in this example.)

The above input would cause the antenna gain to be computed for the LUF rather

than interpolate in the gain table as was the case in Figure 36.

7.7  Outgraph Example

The input required to generate the MUF predictions given in Figures 18

through 22 consists of the following control cards: METHOD, OUTGRAPH, MONTH,

SUNSPOT, TIME, LABEL, CIRCUIT, EXECUTE, and QUIT. The specific input necessary

to generate the output represented in Figures 18 through 22 consists of the

following:

METHOD           7

OUTGRAPH         8      9  10    11

MONTH        1970       1

SUNSPOT      100.

TIME             1     24   1    -1

LABEL      BOULDER, COLORADO TO ST.  LOUIS, MO.

CIRCUIT      40.03N     105.3W     38.67N     90.25W

EXECUTE

QUIT

The method card defines the predictions to be generated and the output form

generated by that method. Therefore, the predictions generated consist of the

MUF-FOT table for each ionospheric layer since method 7 was specified in this

example. The user may select additional output providing that the computations

for the output have already been generated. Therefore, in this case, the user

can select the outputs of methods 8 through 11 consisting of the MUF-FOT

graph, HPF-MUF-FOT graph, MUF-FOT-ANG graph, and MUF-FOT-Es graph, as is

indicated in Table 7. (page 62)

7.8  Procedure Example

The user may desire to have predictions for several circuits at several

months and sunspot numbers, as was discussed in Section 3.3.2. The following

input procedure will generate MUF predictions for several circuits at several

month and sunspot numbers. The output generated would be similar to the output

represented in Figure 19.

PROCEDURE CCIR

LABEL      TOKYO TO AKITO

CIRCUIT    35.7N      139.5E     39.73N    140.1E

EXECUTE

LABEL      BRACKNELL  TO   LUECHOW

CIRCUIT    52.OON     1.20W      53.OON    11.20E

EXECUTE

LABEL      SHANNON TO LUECHOW

CIRCUIT    52.7N      8.9W       53.OON    11.20E

EXECUTE

END

METHOD         8

TIME           1    24    1    -1

MONTH       1971     1

SUNSPOT     80.4

CCIR

MONTH       1971     4

SUNSPOT     70.9

CCIR

QUIT

The result of the above input is MUF-FOT graphs for each of the three

specified circuits at each of the two months indicated. Thus, six output pages

would be generated. It should be noted, as was presented in Section 3.3.2 and

3.3.6, that the procedure name, in this case CCIR, is placed in the input deck

to allow the circuits predefined by the name CCIR to be placed in the position

whenever the name CCIR is used. (page 63)

7.9  External Antenna File Example

The user may wish to save the antenna gain table of his antennas for use

on a separate run. This may be particular significant if the same antennas are

used for several communication paths, or the predictions are generated with

the same antennas each month, or the user has a specific antenna pattern

created by another program, or possibly the actual measure d antenna gain

values. To create the antenna file corresponding to the patterns given in

Figures 23 and 24, the following input should be used:

METHOD          15

ANTOUT

ANTENNA          1    2      .001    4.      -.5

ANTENNA          2    2      .001    4.     -.25

EXECUTE

QUIT

The user could then store the patterns generated on magnetic tape or disk for

reference at a later time. See the discussion of the ANTOUT card in Sections

3.3.5 and 6.3. Use of the antenna patterns previously generated and stored

externally, can be done using the following input, which will also generate

the system performance output represented in Figure 25.

METHOD          16

MONTH         1970    1

SUNSPOT       100.

TIME            12   24  12    -1

LABEL     BOULDER, COLORADO TO ST.  LOUIS, MO.

CIRCUIT     40.03N    105.3W     38.67N    90.25W

ANTENNA          1   18

ANTENNA          2   18

FREQUENCY   2.0  3.0  5.0  7.5 10.0 12.5 15.0 17.5 20.0 25.0 30.0

EXECUTE

QUIT

The difference in this example and the example presented in Section 7.4 is

that the antenna patterns are read from a file instead of being computed.

Section 6.3 explains this procedure in depth. (page 64)

8.  ACKNOWLEDGMENTS

The authors wish to express appreciation to Mr. Miles Merkel of the

U.S. Army Communications Command; Mr. James M. Headrick of the Naval Research

Laboratory; and Mr. Warren G. Richards of International Communications Agency,

Voice of America, for funding which contributed to the development of the

IONCAP program. The authors wish to express appreciation to the many

individuals who developed earlier ionospheric models at ITS and elsewhere

which contributed to the development of IONCAP. The authors wish to express

appreciation to Dr. Rayner K. Rosich and Mr. James S. Washburn for their

valuable contributions and suggestions to the report and to Mr. Duane C.

Hyovalti for his technical review and valuable suggestions.

Table 2.  Valid Name Identifiers

IDENTIFIER          DESCRIPTION OF INPUT PARAMETERS

METHOD        Program run option and beginning page number

MONTH         Year and a list of up to 12 months

MONTHLOOP     Year and months specified in a loop

SUNSPOT       List of sunspots (all months are run for each)

CIRCUIT       Transmitter-receiver locations

SYSTEM        Power, noise, min. angle, req. reliability, SNR,

Time delay and power increment for multipath

TIME          Time of day loop (and indicator for LMT or UT)

ANTENNA       Transmitter or receiver, antenna type and parameters

FREQUENCY     Operating frequencies

LABEL         Alphanumeric label for identification

INTEGRATE     GE. 0 will do a fast integration when no F1 is present

EXECUTE       Execute program with parameters currently set

SAMPLE        Optional geophysical samples (for a specified area)

EFVAR         Optional E, F1 and F2 parameters (for a specified area)

ESVAR         Optional Es parameters (for a specified area)

EDP           True heights and electron density (for a specified area)

AUXIN         Read input card images from an alternate file

AUXOUT        Write program output to an alternate file

ANTOUT        Write antenna patterns on a file

OUTGRAPH      Request output of several methods

COMMENT       Comment card in user defined input

FREEFORM      Input is freeform card images (NOT IMPLEMENTED)

PROCEDURE     Definition of an input procedure

END           Termination of an input procedure definition

NEXT          End of month/sunspot loop

QUIT          Termination of program execution

FPROB         Critical frequency multipliers

TOPLINES      User specified heading lines (for method 23)

BOTLINES      User specified output lines (for method 23)

DEBUG         Write debug output used for program development

(USER DEFINED PROCEDURE NAME) Replace procedure name with its definition

137

Table 3.  Available Output Methods

METHOD         DESCRIPTION OF METHOD

1          Ionospheric parameters

2          Ionograms

3          MUF-FOT lines (nomogram)

4          MUF-FOT graph

5          HPF-MUF-FOT graph

6          MUF-FOT-Es graph

7          FOT-MUF table (full ionosphere)

8          MUF-FOT graph

9          HPF-MUF-FOT graph

10          MUF-FOT-ANG graph

11          MUF-FOT-Es graph

12          MUF by magnetic indices, K (not implemented)

13          Transmitter antenna pattern

14          Receiver antenna pattern

15          Both transmitter and receiver antenna patterns

16          System performance (S.P.)

17          Condensed system performance, reliability

18          Condensed system performance, service probability

19          Propagation path geometry

20          Complete system performance (C.S.P.)

21          Forced long path model (C.S.P.)

22          Forced short path model (C.S.P.)

23          User selected output lines (set by TOPLINES and  BOTLINES)

24          MUF-REL table

25          All modes table

26          MUF-LUF-FOT table (nomogram)

27          FOT-LUF graph

28          MUF-FOT-LUF graph

29          MUF-LUF graph

Table 4.  Required SNR's for Radiotelephone Service

GRADE OF  SERVICE*

Radio Telephone

Description         Operator-to-Operator**      Good Commercial Quality

(Diversity)  No   Dual  No   Dual

6A3                                 51    48   75    70

Double Sideband - AM

3A3 Single Sideband - AM

3A3a (reduced carrier)            49    46   73    68

3A3j (suppressed carrier)         48    45   72    67

6A3 Independent Sideband - AM

6A3b (2-voice channels)       50    47    74   69

9A3b (3-voice channels)       50    47    74   69

12A3b (4-voice channels)      51    48    75   70

*Required Signal-to-Noise Ratio in occupied bandwidth relative

to noise in a 1 Hz bandwidth (dB)

**For 90% intelligibility of related words

Table 6.  Typical Values of Ground Electrical Characteristics

GROUND TYPE               CONDUCTIVITY   DIELECTRIC CONSTANT

Sea Water                    5.0 mhos/m        80

Good Ground                  0.01              10

Poor Ground and Sea Ice      0.001              4

Polar Ice Cap                0.0001             1

Fresh Water                  0.002             80

Perfect Ground                 -2              -2

Free Space                     -1              -1

Table 7.  Output Combinations for the OUTGRAPH Card

METHOD CARD             OUTGRAPH CARD

PROGRAM TASK      ADDITIONAL OUTPUT AVAILABLE FOR THE SPECIFIED

OPTION*           TASK OPTION (SPECIFIED ON OUTGRAPH CARD)

3                    4      5     6

4                    3      5     6

5                    3      4     6

6                    3      4     5

7                    8  to  11

8                    9  to  11

9                    8, 10, 11

10                    8, 9,  11

11                    8  to  10

16  TO  25            8  to  11,  26 to 29**

26                    8  to  11,  27 to 29

27                    8  to  11,  26, 28, 29

28                    8  to  11,  26 to 29

29                    8  to  11,  26 to 28

*Optimal program task options for additional output.

**LUF values set by first frequency in the frequency

complement which has a computed reliability that is greater

than or equal to the required system reliability. If none of

the computed reliabilities are at least as large as that

required, the frequency with the greatest reliability is chosen

and a designator is printed to indicate the reliability. (Note

that this occurs only when the user specifies 26 to 29 on the

OUTGRAPH card and 16 to 25 on the METHOD card. The actual

computed values for LUF are printed when the user specifies 26

to 29 on the OUTGRAPH card and 26 to 29 on the METHOD card.)

142

Table 8.  Header Line Options for the TOPLINES Card

LINTP               DESCRIPTION OF HEADER LINE

1     Month, year, and sunspot number

2     Alphanumeric information on the label card

3 Transmitter and receiver information consisting of

coordinates, azimuth, and great circle distance (in

kilometers and nautical miles)

4     Minimum take-off angle (in degrees)

5 Transmitter antenna information consisting of the

operating frequency range, antenna type (in mnemonics),

height*, length*, off azimuth angle, conductivity,

dielectric constant as well as other antenna

characteristics

6 Receiver antenna information consisting of the operating

frequency range, antenna type (in mnemonics), height*,

length*, off azimuth angle, conductivity, dielectric

constant as well as other antenna characteristics

7 Power, 3 MHz man-made noise, required reliability and

required SNR

8 Multipath power tolerance and delay time tolerance

*Height and length given in meters if positive and wavelength

if negative.

Table 9.  System Performance Output Line Options  BOTLINES

LINBD  OUTPUT MNEMONIC             VARIABLE DESCRIPTION

1     MODE*            Number of hops for MUF and mode type

MODE**           Mode type at transmitter end and at receiver end

2     ANGLE*           Radiation angle at transmitter, degrees

ANGLE**          Radiation angle at transmitter and receiver end, degrees

3     DELAY            Time delay most reliable mode (MRM), milliseconds

4     V HITE           Virtual height MRM, kilometers

5     F DAYS           Probability operating frequency exceeds predicted MUF

6     LOSS             Median system loss, MRM, dB

7     DBU              Field strength, median, dBu

8     S DBW            Signal power, median, dbw

9     N DBW            Noise strength, median, dBw

10     SNR              Median SNR, dB

11     RPWRG            Required power gain for MRM,  dB

12     REL              Reliability

13     MPROB            Multipath probability (short paths only)

14     SPRB             Service probability

15     SIGLW            Signal (loss), lower decile,   dB

16     SIGUP            Signal (loss), upper decile,   dB

17     VHFDBU           VHF field strength, median,   dBu

18     VHFLW            VHF field strength, lower decile, dB

19     VHFUP            VHF field strength, upper decile, dB

20     VHFMOD           VHF mode type

21     SNRLW            SNR, lower decile, dB xmtr antenna gain

22     SNRUP            SNR, upper decile, dB rcvr antenna gain

*if short path

**if  long path

Table 10. System Performance Output Line Options as Preset by Method Number

METHOD     LINES                 DESCRIPTION

16        1 to 13                Detailed system performance

17        1, 2, 5, 7, 10, 12     Condensed S. P. with reliability

18        19 29,59 79 10, 14     Condensed S. P. with service  probability

19        1, 2, 3, 4, 5          Propagation path geometry

20        1 to 22 (all)          Complete S. P. output

21        1 to 16                Force long path submodule

22        1 to 16                Force short path submodule

METHOD Control Card

Columns     Name        Format  Description of Input Data

1-10                   A10     "METHOD" control card identifier

11-15     Method        I5      Program task option, 1 to 30

16-20     NPAGO         I5      Starting page number on output file.

(Note if NPAGO <= 0 then initial page is

set to default of one.)

Control Card  1. METHOD card            155

EXECUTE Control Card (3.3.1)

Columns     Name        Format  Description of Input Data

1-10                   A10     "EXECUTE" control card identifier

11-15       KRUN        I5      3 = no indices calculation; 2 = Es indices

only; 1 = E, F1, F2 indices only; 0 = E,

F1, F2, Es indices, (default is KRUN = 0

which indicates indices are calculate

for each ionospheric layer)

Control Card  2. EXECUTE card           156

QUIT Control Card (3.3.1)

Columns     Name        Format      Description of Input Data

1-10                             A10       "QUIT" control card identifier

NOTE: THIS CARD MUST BE PRESENT AND MUST BE THE LAST CARD ON THE INPUT FILE.

Control Card  3.   QUIT card            157

TIME Control Card (3.3.2)

Columns     Name        Format  Description Input Data

1-10                   A10     "TIME" control card identifier

11-15       IHRO        I5      Start time, hours

16-20       IHRE        I5      Stop time, hours

21-25       IHRS        I5      Hour increment, hours

26-30       ITIM        I5      If negative LMT at transmitter, otherwise UT

Control Card  4. TIME card              158

MONTH  Control  Card   (3.3.2)

Columns     Name        Format  Description of Input Data

1-10                   A10     "MONTH" control card identifier

11-15     NYEAR         I5      Year

16-20     MONTHS(1)     I5      an array of up to 12 months where January is

21-25     MONTHS(2)     I5      represented by  1, February is represented

26-30     MONTHS(3)     I5      by 2, etc.

31-35     MONTHS(4)     I5

36-40     MONTHS(5)     I5

41-45     MONTHS(6)     I5

46-50     MONTHS(7)     I5

51-55     MONTHS(8)     I5

56-60     MONTHS(9)     I5

61-65     MONTHS(10)    I5

66-70     MONTHS(11)    I5

71-75     MONTHS(12)    I5

Control Card  5.     MONTH card         159

MONTHLOOP Control Card (3.3.2)

Columns Name    Format  Description of Input Data

1-10           AI0     "MONTHLOOP" control card identifier

11-15   NYEAR   I5      Year

16-20   MINIT   I5      Starting month num; 1 is Jan, 2 is Feb.

21-25   MFINAL  I5      Last month number

26-30   MINC    I5      Increment

Control Card  6.     MONTHLOOP card     160

SUNSPOT Control Card (3.3.2)

Columns     Name        Format  Description of Input Data

1-10                   A10     "SUNSPOT" control card identifier

11-15     SUNSP(1)      F5.1    An array of up to

16-20     SUNSP(2)      F5.1    12 sunspot numbers.

21-25     SUNSP(3)      F5.1

26-30     SUNSP(4)      F5.1

31-35     SUNSP(5)      F5.1

36-40     SUNSP(6)      F5.1

41-45     SUNSP(7)      F5.1

46-50     SUNSP(8)      F5.1

51-55     SUNSP(9)      F5.1

56-60     SUNSP(10)     F5.1

61-65     SUNSP(ll)     F5.1

66-70     SUNSP(12)     F5.1

May be negative; only the first sunspot number.  SUNSP(1), may be 0.0.

Control Card  7.   SUNSPOT card         161

NEXT Control Card (3.3.2)

Columns     Name    Format  Description of Input Data

1-10                A10     "NEXT" control card identifier

Indicates end of a

month-sunspot loop

Control Card  8.   next card        162

LABEL Control  Card  (3.3.3)

Columns     Name        Format  Description of Input Data

1-10                   A10     "LABEL" control card identifier

11-20     ITRAN(1)      A10     For output identification only

21-30     ITRAN(2)      A10     For output identification only

31-40     IRCVR(1)      A10     For output identification only

41-50     IRCVR(2)      A10     For output identification only

Control Card  9.  LABEL card            163

CIRCUIT Control Card (3.3.3)

Columns     Name        Format  Description of Input Data

1-10                   A10     "CIRCUIT" control card identifier

11-15     TLATD         F5.2    Latitude of transmitter, degrees

16     ITLAT         A1      north, S = south

21-25     TLONGD        F5.2    Longitude of transmitter, degrees

26     ITLONG        A1      E = east, W = west

31-35     RLATD         F5.2    Latitude of receiver,  degrees

36     IRLAT         A1      N = north, S = south

41-45     RLONGD        F5.2    Longitude of receiver, degrees

46     IRLONG        A1      E = east, W = west

51-55     NPSL          I5      Use long great circle path if = 1

(default is NPSL = 0 which specifies that

the shortest distance between transmitter

and receiver is used as circuit path)

Control Card 10. CIRCUIT card           164

SYSTEM Control Card (3.3.3)

Columns     Name        Format  Description of Input Data

1-10                   A10     "SYSTEM" control card identifier

11-15      PWR          F5.2    Transmitter power, kilowatts

16-20      XNOISE       F5.0    man-made noise in dB below a watt (-dBW);

-l is industrial (125),-2 is residential (136),

-3 is rural (148), -4 is remote (164)*

21-25      AMIND        F5.2    minimum radiation angle, degrees

26-30      XLUFP        F5.0    Required reliability, percent

31-35      RSN          F5.2    Required SNR. dB

36-40      PMP          F5.2    Multipath power tolerance. dB

41-45      DMPX         F5.2    Multipath time delay-increment, milliseconds

*The user may indicate other man-made noise values by

specifying a positive value in this field.

Control Card 11.   SYSTEM card          165

FREQUENCY  Control  Card   (3.3.3)

Columns     Name        Format  Description in Input Data

1-10                   A10     "FREQUENCY" control card identifier

11-15      FREL(1)      F5.2    Array of up to 11 frequencies, MHz,

16-20      FREL(2)      F5.2    if FREL(1) is zero, then the

21-25      FREL(3)      F5.2    FOT replaces FREL(1)

26-30      FREL(4)      F5.2

31-35      FREL(5)      F5.2

36-40      FREL(6)      F5.2

41-45      FREL(7)      F5.2

46-50      FREL(8)      F5.2

51-55      FREL(9)      F5.2

56-60      FREL(10)     F5.2

61-65      FREL(11)     F5.2

Up to eleven values on this card are used.  A non-positive value terminates

the frequency loop. If card Is blank a complement Is calculated for each hour.

Control Card 12. FREQUENCY card         166

SAMPLE  Control   Card   (3.3,4)

Columns     Name        Format  Description in Input Data

1-10                   A10     "SAMPLE" control card identifier

11-15      I            I5      Sample area 1 to 5

16-20      SLAT         F5.2    Geographic latitude, degrees

21      ISLAT        A1      N = North, S = South

26-30      SLONG        F5.2    Geographic longitude, degrees

31      ISLONG       A1      E = East, W = West

36-40      SGLAT        F5.2    Geomagnetic latitude, degrees

41      ISGLAT       A1      N = North, S = South

46-50      RD(I)        F5.0    Distance from transmitter, km

51-55      GYZ(I)       F5.2    Gyrofrequency, MHz

56-60      CLCK(I)      F5.2    Local mean time

61-65      GMDIP(I)     F5.2    Geomagnetic dip angle

66-70      ARTIC(I)     F5.2    Auroral loss, dB

71-75      SIGPAT(I)    F5.2    Ground conductivity,  mhos/m

76-80      EPSPAT(I)    F5.2    Relative dielectric constant

Control Card 13. SAMPLE card        167

EFVAR Control Card  (3.3.4)

Columns     Name        Format  Description in Input Data

1-10                   A10     "EFVAR" control card identifier

11-15      I            I5      Sample area 1 to 5

16-20      FI(1,I)      F5.2    foE, MHz

21-25      YI(1,I)      F5.1    ymE, km

26-30      HI(1,I)      F5.1    hmE, km

31-35      FI(2,I)      F5.2    foF1, MHz

36-40      YI(2,I)      F5.1    ymF1, km

41-45      HI(2,I)      F5.1    hmF1, km

46-50      FI(3,I)      F5.2    foF2, MHz

51-55      YI(3,I)      F5.1    ymF2, km

56-60      HI(3,I)      F5.1    hmF2, km

Control Card 14.  EFVAR card        168

ESVAR Control Card  (3.3.4)

Columns     Name        Format  Description of Input Data

1-10                   A10     "ESVAR" control card identifier

11-10      I            I5      Sample area 1 to 5

16-20      FS(1,I)      F5.2    foEs lower decile, MHz=lowest value, e.g., 2.

21-25      FS(2,I)      F5.2    foEs median, e.g., 4., MHz

26-30      FS(3,I)      F5.1    foEs upper decile, MHz=highest value, e.g., 6.

31-35      HS(I)        F5.1    Virtual height of reflection, km, e.g., 110

Control Card 15.        ESVAR card  169

EDP    Control  Card (3.3.4)

Columns     Name        Format  Description of Input Data

1-10       AI0         "EDP" control card identifier

1-15       JSAMP       I5      Must = 1 (limitation on

current implementation)

16-25       ITEMP       A10     if "OFF" return to use

calculated EDP

If not "OFF" followed by 8

cards; first four contain

heights, second four contain

square of plasma frequency

Control Card 16.        EDP card  170

Data for external electron density profile (3.3.4)

Columns     Name        Format  Description of Input Data

1-5       HTR(1)      F5.2    True heights, kilometers

6-10       HTR(2)      F5.2

Control Card 17. Electron Density, true height 171

Data for external electron density profile (3.3.4)

Columns     Name        Format  Description of Input Data

1-5       FNSQ(1)     F5.2    Plasma frequency (MHz) squared

6-10       FNSQ(2)     F5.2

Control Card 18. Electron Density, square of plasma frequency

INTEGRATE Control Card (3.3.4)

Columns Name  Format  Description of Input Data

1-10         A10     "INTEGRATE" control card identifier

11-15   INTEG I5      > 0 indicates fast integration;

< 0 indicates Gaussian integration

-OR-

11-20   ITEMP A10     "OFF" indicates return to fast

integration

Control Card 19.   Integrate    173

FPROB  Control  Card   (3.3.4)

Columns     Name        Format  Description of Input Data

1-10                   A10     "FPROB" control card identifier

11-15      PSC(1)       F5.1    Multiplier for foE > 0

16-20      PSC(2)       F5.1    Multiplier for foF1

21-25      PSC(3)       F5.1    Multiplier for foF2 > 0

26-30      PSC(4)       F5.1    Multiplier for foEs

-OR-

11-20      ITEMP        A10     "OFF" returns to default values of

1.0, 1.0, 1.0, 0.7

The value for foE and foF2 must be positive.

The F1 layer can be removed by setting the

multiplier for foF1 to 0. The Es layer can

be removed as a separate layer in the

same manner.

Control Card 20.      FPROB card    174

COMMENT control Card (3.3.5)

Columns     Name        Format  Description of Input Data

1-10                   A10     "COMMENT" control card identifier

11-80                           User inserts description of the input file or

other remarks and comments; not internally

read by program

Control Card 21. COMMENT card       175

AUXIN Control Card (3.3.5)

Columns     Name        Format  Description of Input Data

1-10                   A10     "AUXIN" control card identifier

11-20      ITEMP        A10     If = "OFF" return to user defined input

file LU5 otherwise program  uses LUl5

as user defined input file

Control Card 22. AUXIN card          176

AUXOUT Control Card (3.3.5)

Columns     Name        Format  Description of Input Data

1-10                   A10     "AUXOUT" control card identifier

11-20      ITEMP        A10     If = "OFF" return line printer output to

default LU6, otherwise program uses line

printer image output to LU16.

Control Card 23. AUXOUT card         177

ANTOUT Control Card (3.3.5)

Columns     Name        Format  Description of Input Data

1-10                   A10     "ANTOUT" control card identifier

11-20      ITEMP        A10     If = "OFF" do not output any antenna patterns,

otherwise pattern will be output to LU25.

Control Card 24.   ANTOUT card       178

OUTGRAPH Control Card (3.3.5)

Columns     Name        Format  Description of Input Data

1-10                   A10     "OUTGRAPH" control card identifier

11-15      KTOUT(1)     I5      Indicates additional methods" of desired

16-20      KTOUT(2)     I5      graphical output (specify 1 to 12 methods)

21-25      KTOUT(3)     I5

26-30      KTOUT(4)     I5      (See Table 7 for valid output combinations.)

31-35      KTOUT(5)     I5

36-40      KTOUT(6)     I5

41-45      KTOUT(7)     I5

46-50      KTOUT(8)     I5

51-55      KTOUT(9)     I5

56-60      KTOUT(10)    I5

61-65      KTOUT(11)    I5

66-70      KTOUT(12)    I5

-OR-

11-20      ITEMP        A10     if "OFF", stop additional output

Control Card 25.     OUTGRAPH card  179

TOPLINES Control Card (3.3.5)

Columns     Name        Format  Description of Input Data

1-10                   A10     "TOPLINES" control card identifier

11-15      LINTP(1)     I5      Header line number; 1 to 8 (refer to table 8

16-20      LINTP(2)     I5      for description) Program will print those

21-25      LINTP(3)     I5      header lines specified when method 23 is used.

26-30      LINTP(4)     I5

31-35      LINTP(5)     I5

36-40      LINTP(6)     I5

41-45      LINTP(7)     I5

46-50      LINTP(8)     I5

51-55      LINTP(9)     I5

56-60      LINTP(10)    I5

61-65      LINTP(11)    I5

66-70      LINTP(12)    I5

71-75      LINTP(13)    I5

76-80      LINTP(14)    I5

11-20      ITEMP        A10     "OFF" indicates to discontinue printing the,

header line(s) specified on a previous

"TOPLINES" card

Control Card 26. TOPLINES card      180

BOTLINES Control Card (3.3.5)

Columns     Name        Format  Description of Input Data

1-10                   A10     "BOTLINES" control card identifier to output,

11-15     LINBD(1)      I5      Line number, 1 to 22 (indicate parameter see

16-20     LINBD(2)      I5      Table 9) when method 23 is used

21-25     LINBD(3)      I5

26-30     LINBD(4)      I5

31-35     LINBD(5)      I5

36-40     LINBD(6)      I5

41-45     LINBD(7)      I5

46-50     LINBD(8)      I5

51-55     LINBD(9)      I5

56-60     LINBD(10)     I5

61-65     LINBD(ll)     I5

66-70     LINBD(12)     I5

71-75     LINBD(13)     I5

76-80     LINBD(14)     I5

11-20     ITEMP         A10     "OFF" indicates to discontinue printing the

parameter(s) specified on a previous

"BOTLINES" card

Control Card 27.   BOTLINES card    181

PROCEDURE Control Card (3.3.6)

Columns     Name        Format  Description of Input Data

1-10                   A10     "PROCEDURE" control card identifier user

11-20      ITEMP        A10     defined procedure name. Procedure definition.

follows this card with a set of control cards

terminate with an END card. During program

execution, when a control card is encountered

with ITEMP in columns 1-10 the entire set will

be inserted.

Control Card 30. PROCEDURE card     184

END Control Card (3.3.6)

Columns     Name        Format  Description of Input Data

1-10                   A10     "END" control card identifier

Indicates end of a procedure definition (all

cards between PROCEDURE control cards and

END card constitute procedure definition)

Control Card 31.   END card         185

ANTENNA   CARD,   TERMINATED   RHOMBIC

Columns Name Format  Description of  Input Data

1-10        A10     "ANTENNA"

11-15  IAT   I5      1 for transmitter, 2 for receiver

16-20  IANTR I5      Antenna type = 1

21-25  AETA  F5.1    Bearing of antenna, degrees, E of N

26-30  ASIG  F5.1    Ground conductivity, mhos/m

31-35  AEPS  F5.1    Relative dielectric constant

36-40  AND   F5.1    Tilt angle, degrees

(1/2 the large interior angle)

41-45  ANL   F5.1    Antenna leg length, meters

(wavelengths if negative)

46-50  ANH   F5.1    Antenna height, meters

(wavelengths if negative)

51-55                Not used for this antenna

56-60                Not used for this antenna

61-65                Not used for this antenna

66-70                Not used for this antenna

71-75  AFQB  F5.1    Ending frequency when more than one

antenna is used

76-80  IAIN  I5      Antenna number indicator (each

transmitter or receiver antenna can be

defined by up to three different antennas

over the frequency range; default is one,

if IAIN is left blank.

Control Card 32. Terminated rhombic card                 186

ANTENNA CARD, VERTICAL MONOPOLE

Columns Name    Format  Description of Input Data

1-10           A10     "ANTENNA" 2

11-15   IAT     I5      1 for transmitter, 2 for receiver

16-20   IANTR   I5      antenna type = 2

21-25                   Not used for  this antenna

26-30   ASIG    F5.1    ground conductivity, mhos/m

31-35   AEPS    F5.1    relative dielectric constant

36-40                   Not used for this antenna

41-45   ANL     F5.1    antenna height, meters

(wavelengths if negative)

46-50   ANH     F5.1    gain above a dipole, dB

51-55                   Not used for this antenna

56-60                   Not used for this antenna

61-65                   Not used for this antenna

66-70                   Not used for this antenna

71-75   AFQB    F5.1    Ending frequency when more than one

antenna is used

76-80   IAIN    I5      Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,  if IAIN is

left blank.

Control Card 33. Vertical monopole card                        187

ANTENNA CARD, HORIZONTAL DIPOLE

Columns Name    Format  Description of Input Data

1-10           A10     "ANTENNA"   3

11-15   IAT     I5      1 for transmitter, 2 for receiver

16-20   IANTR   I5      antenna type = 3

21-25   AETA    F5.1    bearing, degrees E of N

26-30   ASIG    F5.1    ground conductivity, mhos/m

31-35   AEPS    F5.1    relative dielectric constant

36-40                   Not used for this antenna

41-45   ANL     F5.1    antenna length, meters

(wavelengths if negative)

46-50   ANH     F5.1    antenna height, meters

(wavelengths if negative)

51-55   AEX (1) F5.1    gain above 1/2 wavelength horizontal

dipole, dB

56-60                   Not used for this antenna

61-65                   Not used for this antenna

66-70                   Not used for this antenna

71-75   AFQB    F5.1    Ending frequency when more than one

antenna is used

76-80   IAIN    I5      Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,

if IAIN is left blank.

Control Card 34. Horizontal  dipole card                    188

ANTENNA CARD, HORIZONTAL YAGI

Columns Name Format  Description of Input Data

1-10         A10    "ANTENNA"   4

1-15  IAT    I5     1 for transmitter, 2 for receiver

16-20  IANTR  I5     antenna type = 4

21-25  AETA   F5.1   bearing of antenna, degrees E of N

26-30  ASIG   F5.1   ground conductivity, mhos/m

31-35  AEPS   F5.1   relative dielectric constant

36-40                Not used for this antenna.

41-45  ANL    F5.1   antenna length, meters

(wavelengths if negative)

46-50  ANH    F5.1   antenna height, meters

(wavelengths if negative)

51-55  AEX(1) F5.1   gain above 1/2 wavelength horizontal

dipole, dB

56-60                Not used for this antenna.

61-65                Not used for this antenna.

66-70                Not used for this antenna.

71-75  AFQB   F5.1   Ending frequency when more than one

antenna is used

76-80  IAIN   I5     Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,

if IAIN is left blank.

Control Card 35. Horizontal Yagi card                        189

ANTENNA CARD, VERTICAL LOG PERIODIC ARRAY OF MONOPOLES

Columns Name    Format  Description of Input Data

1-10           A10     "ANTENNA"   5

11-15   IAT     I5      1 for transmitter, 2 for receiver

16-20   IANTR   I5      antenna type = 5

21-25                   Not  used  for  this  antenna.

26-30   ASIG    F5.1    ground conductivity, mhos/m

31-35   AEPS    F5.1    relative dielectric constant

36-40                   Not used for this antenna.

41-45   ANL     F5.1    antenna height, must be 1/4

wavelength (-.25)

46-50                   Not used for this antenna.

51-55   AEX (1) F5.1    gain above a 1/4 wavelength vertical

56-60                   Not used for this antenna.

61-65                   Not used for this antenna.

66-70                   Not used for this antenna.

71-75   AFQB    F5.1    Ending frequency when more than one

antenna is used

76-80   IAIN    I5      Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,

if IAIN is left blank.

NOTE: The vertical log periodic array of monopoles is

estimated from a 1/4 wave vertical monopole plus additional

gain.

Control Card 36. Vertical log periodic card                   190

ANTENNA CARD, CURTAIN

Columns Name    Format  Description of Input Data

1-10           A10     "ANTENNA"   6

11-15   IAT     I5      1 for transmitter, 2 for receiver

16-20   IANTR   I5      antenna type = 6

21-25   AETA    F5.1    antenna bearing degrees E of N

26-30   ASIG    F5.1    ground conductivity, mhos/m

31-35   AEPS    F5.1    relative dielectric constant

36-40   AND     F5.1    number of bays

41-45   ANL     F5.1    antenna element length, meters

46-50   ANH     F5.1    antenna height to first element,

meters

51-55   AEX (1) F5.1    number of elements per bay

56-60   AEX (2) F5.1    distance between elements centers

meters

61-65   AEX (3) F5.1    vertical spacing of elements, meters

66-70   AEX (4) F5.1    distance from screen, meters

71-75   AFQB    F5.1    ending frequency when more than one

antenna is used

76-80   IAIN    I5      Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,

if IAIN is left blank.

Control Card 37. Curtain Antenna                 191

ANTENNA CARD, TERMINATED SLOPING VEE

Columns Name    Format  Description of Input Data

1-10           A10     "ANTENNA"   7

11-15   IAT     15      1 for transmitter, 2 for receiver

16-20   IANTR   I5      antenna type = 7

21-25   AETA    F5.1    bearing of antenna degrees E of N

26-30   ASIG    F5.1    ground conductivity mhos/m

31-35   AEPS    F5.1    relative dielectric constant

36-40   AND     F5.1    1/2 apex angle in plane of wires

41-45   ANL     F5.1    antenna  leg  length,  meters

46-50   ANH     F5.1    antenna height, meters

51-55   AEX (1) F5.1    terminated height

56-60                   Not used for this antenna.

61-65                   Not used for this antenna.

66-70                   Not used for this antenna.

71-75   AFQB    F5.1    ending frequency  when more than one

antenna is used

76-80   IAIN    I5      Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,

if IAIN is left blank.

Control Card 38. Sloping vee card                     192

ANTENNA  CARD,  INVERTED   L

Columns Name    Format  Description of Input Data

1-10           A10     "ANTENNA"    8

11-15   IAT     I5      1 for transmitter, 2 for receiver

16-20   IANTR   I5      antenna type = 8

21-25   AETA    F5.1    bearing of antenna degrees E of N

26-30   ASIG    F5.1    ground conductivity mhos/m

31-35   AEPS    F5.1    relative dielectric constant

36-40                   Not used for this antenna.

41-45   ANL     F5.1    antenna length,  meters

46-50   ANH     F5.1    antenna height,  meters

51-55                   Not used for this antenna.

56-60                   Not used for this antenna.

61-65                   Not used for this antenna.

66-70                   Not used for this antenna.

71-75   AFQB    F5.1    ending frequency  when more than one

antenna is used

76-80   IAIN    I5      Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,

if IAIN is left blank.

Control Card 39. Inverted L card                     193

ANTENNA CARD, TERMINATED SLOPING RHOMBIC

Columns Name Format  Description of Input Data

1-10          A10   "ANTENNA"   9

11-15  IAT     I5    1 for transmitter, 2 for receiver

16-20  IANTR   I5    antenna =  9

21-25  AETA    F5.1  antenna bearing degrees E of N

26-30  ASIG    F5.1  ground conductivity, mhos/m

31-35  AEPS    F5.1  relative dielectric constant

36-40  AND     F5.1  1/2 large interior angle in plane of wire

41-45  ANL     F5.1  leg length, meters

46-50  ANH     F5.1  antenna height meters

51-55  AEX (1) F5.1  terminate height, meters

56-60                Not used for this antenna,

61-65                Not used for this antenna,

66-70                Not used for this antenna,

71-75  AFQB    F5.1  ending frequency  when more than one

antenna is used

76-80  IAIN    I5    Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,

if IAIN is left blank.

Control Card 40. Sloping rhombic card                    194

ANTENNA CARD, INTERLACED RHOMBIC

Columns Name  Format Description of Input Data

1-10         A10    "ANTENNA"   10

11-15  IAT    I5     1  for transmitter, 2 for receiver

16-20  IANTR  I5     antenna type = 10

21-25  AETA   F5.1   antenna bearing degrees E of N

26-30  ASIG   F5.l   ground conductivitys mhos/m

31-35  AEPS   F5.1   relative dielectric constant

36-40  AND    F5.1   1/2 large interior angle in plane of

wire, degrees

41-45  ANL    F5.1   leq length, meters

46-50  ANH    F5.1   lower antenna height, meters

51-55  AEX(1) F5.1   vertical  displacement, meters

56-60  AEX(2) F5.1   horizontal  feed point displacement,

meters

61-65                Not used for this antenna.

66-70                Not used for this antenna.

71-75  AFQB    F5.1  ending frequency  when more than one

antenna is used

76-80  IAIN    I5    Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,

if IAIN is left blank.

Control Card 41. Interlaced rhombic card                 195

ANTENNA CARD, CONSTANT GAIN

Columns Name    Format  Description of Input Data

1-10           A10     "ANTENNA" 12

11-15   IAT     I5      1 for transmitter, 2 for receiver

16-20   IANTR   I5      antenna type = 12

21-25   AETA    F5.1    gain above isotropic, dB

26-30   ASIG    F5.1    ground conductivity, mhos/m

31-35   AFPS    F5.1    relative dielectric constant

36-40                   Not used for this antenna.

41-45                   Not used for this antenna.

46-50   ANH     F5.1    antenna efficiency, dB

51-55                   Not used for this antenna.

56-60                   Not used for this antenna.

61-65                   Not used for this antenna.

66-70                   Not used for this antenna.

71-75   AFQB    F5.1    ending frequency  when more than one

antenna is used

76-80   IAIN    I5      Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,

if IAIN is left blank.

Control Card 42. Constant gain card                      196

ANTENNA  CARD, FROM FILE

Columns Name  Format Description of Input Data

1-10         A10    "ANTENNA" 18

11-15   IAT   I5     1 for transmitter, 2 for receiver

16-20   IANTR I5     antenna type = 18

21-25

26-30

31-35

36-40                Columns 21-55 Not used for this antenna,

41-45

46-50

51-55

56-60                Antenna type 18 (If  IAI N >= 0, then

61-65                IAIN indicates the number of antennas to

66-70                skip forward from present position on the

71-75                antenna file (LU26) before reading the

desired pattern (b) IF IAIN < 0, then

the antenna file is rewound before

searching for the antenna pattern.

76-80   IAIN  I5     Antenna number indicator (each

transmitter or receiver antenna can

be defined by up to three different

antennas over the frequency range;

default is one,

if IAIN is left blank.

Control Card 43. Antenna pattern read from file     197

9.  REFERENCES

Baker, H. C., and A. H. Lagrone (1962), Digital computation of the mutual

impedance between thin dipoles, Proc. IRE Trans. Ant. & Prop., AP-10 (2),

172-178.

Barghausen, A. F., J. W. Finney, L. L. Proctor, and L. D. Schultz (1969),

Predicting long-term operational parameters of high-frequency skywave

telecommunication systems, ESSA Technical Report ERL 110-ITS78, Washington, D.

C., May.

Beckmann, B. (1958), Concerning the relationship of the field intensity to the

limits of the transmission frequency range, NTZ 11, 523, 528.

Bibl, K., A. Paul, and K. Rawer (1961), Absorption in the D and E regions and

its time variation, J. Atmospheric and Terrest. Phys. 23, 244-259, December.

Bremmer, H. (1949), Terrestrial Radio Waves (Elsevier Publishing Co., New

York, New York).

Budden, K. G. (1966), Radio Waves in the Ionosphere (Cambridge University

Press, Cambridge, England).

CCIR (1964), World distribution and characteristics of atmospheric radio

noise, Rept. 322, Documents of the Xth Plenary Assembly, Geneva, 1963.

CCIR (1966), Atlas of ionospheric characteristics, Rept. 340-1, Oslo, ITU,

Geneva, Switzerland.

CCIR (1970), CCIR interim method for estimating skywave field strength and

transmission loss at frequencies between the approximate limits of 2 and 30

MHz, CCIR Report 252-2, ITU, Geneva, Switzerland.

Central Radio Propagation Laboratory (1948), Ionospheric radio propagation,

NBS Circular 462.

Croft, T. A. (1967), HF radio focusing caused by the electron distribution

between ionospheric layers, J. Geophys, Res., 72, No. 9, 2343-2355.

Davies, K. (1965), Ionospheric Radio Propagation, NBS Monograph 80 (U.S.

Government Printing Office, Washington, D.C. 20402).

Davies, K. (1969), Ionospheric Radio Waves (Blaisdell Publishing Co., Waltham,

Mass)

Davis, R. M., and N. L. Groome (1965), The effect of auroral zone absorption

on high frequency system loss, NBS Report 8810, Boulder, Colorado.

Fejer, J. A. (1961), The absorption of short radio waves in the ionospheric D

and E regions, J. A. and T. P. 23, 260-274.

Finney, J. B. (1963), Ray tracing development for propagation studies, NBS

Report 7697.

George, P. L. (1971), The calculation of ionospheric absorption in HF radio

propagation prediction, WRE-Technical Note-A207(AP). Department of Supply,

Weapons Research Establishment, South Australia.

198

Harnischmacher, E. (1960), A calculation method of ionospheric propagation

conditions for very high and antipode distance, in Electromagnetic Wave

Propagation, (Academic Press).

Haselgrove, J. (1954), Ray theory and a new method for ray tracing, (Report of

Conference on Physics of the Ionosphere, London Physical Society).

Haydon, G. W., M. Leftin, and R. K. Rosich (1976), Predicting the Performance

of High Frequency Skywave Telecommunication Systems, OT Report 76-102,

Boulder, Colorado.

Haydon, G. W., and D. L. Lucas (1968), Predicting ionosphere electron density

profiles, Radio Science 3, No. 13, pp. 111-119.

Haydon, G. W., D. L. Lucas, and R. A. Hanson (1969), Technical considerations

in the selection of optimum frequencies for high frequency skywave

communication services, ESSA Tech. Report ERL 113-ITS 81, U. S. Dept. of

Commerce, Boulder, Colorado. This report is a reprint of NBS Report 7249,

originally issued November, 1962.

Headrick, J. M., and M. I. Skolnik (1974), Over-the-Horizon Radar in the HF

Band, Proc. IEEE 62, No. 6, 664-673.

Headrick, J. M., J. M. Thomason, D. L. Lucas, S. R. McCammon, R. A. Hanson,

and J. L. Lloyd (1971), Virtual path tracing for HF radar including an

ionospheric model, NRL Memo Report 2226, Naval Research Laboratory,

Washington, D.C.

Jasik, H. (1961), Antenna Engineering Handbook (McGraw-Hill, New York).

Jones, W. B., and R. M. Gallet (1962), The representation of diurnal and

geographic variations of ionospheric data by numerical methods, Radio Sci. (J.

Res. NBS), No. 4, 419-438.

Jones, W. B., and F. G. Stewart (1970), A numerical method for global mapping

of plasma frequency, Radio Sci. No. 5. 773-784.

Jordan, E. C., and K. G. Balmain (1968), Electromagnetic Waves and Radiating

Systems, Prentice-Hall, Englewood Cliffs, N. J.

Kelley, L. (1945), Calculation of skywave field intensities, maximum usable

frequencies, U. S. Army Signal Corps Tech. Report No. 6.

King, R. W. P., and T. T. Wu (1965), The cylindrical antenna with arbitrary

driving point, IEEE Trans. on Ant. & Prop., AP-13, 710-718.

Laitinen, P. 0. (1957), Linear communication antennas, Tech. Report No. 7,

revised, October, U. S. Signal Radio Propagation Agency, Fort Monmouth, N.J.

Laitinen, P. O., and G. W. Haydon (1950), Analysis and prediction of skywave

field intensities in the high frequency band, U. S. Army Signal Radio

Propagation Agency Tech. Report No. 9, Rev. (RPN 203).

199

Leftin, M. (1976), Numerical representation of monthly median critical

frequencies of the regular E region (foE), OT Report 76-88, Boulder, Colorado.

Leftin, M., S. M. Ostrow, and C. Preston (1968), Numerical maps of foEs for

solar cycle minimum and maximum, ESSA Technical Report ERL 73-ITS 63, Boulder,

CO.

Lejay, P., and D. Lepechinsky (1950), Field intensity at the receiver as a

function of distance, Nature 165, 306.

Lucas, D. L., and J. D. Harper (1965), A numerical representation of CCIR

Report 233 high frequency (3-30 Mc/s) atmospheric radio noise data, NBS Tech.

Note No. 318, U. S. Department of Commerce, Boulder, Colorado.

Lucas, D. L., and G. W. Haydon (1961), MUF-FOT predictions by electronic

computers, NBS Report 6789. U. S. Department of Commerce, Boulder, Colorado

80303.

Lucas, D. L., and G. W. Haydon (1962), Predicting the performance of band 7

communications systems using electronic computer, NBS Report 7619.

Lucas, D. L., and G. W. Haydon (1966), Predicting statistical performance

indexes for high frequency telecommunications systems, ESSA Tech Report IER

1-ITSA 1, U. S. Department of Commerce, Boulder, Colorado 80303.

Lucas, D. L., J. L. Lloyd, J. M. Headrick, and J. F. Thomason (1972), Computer

techniques for planning and management of OTH radars, NRL Memo Report 2500,

Naval Research Laboratory, Washington, D. C.

Ma, M. T. (1974), Theory and Application of Antenna Arrays (John Wiley & Sons,

New York).

Ma, M. T., and L. C. Walters (1967), Computed radiation patterns of

log-periodic antennas over lossy plane ground, ESSA Tech. Report IER 54-ITSA

52, U. S. Department of Commerce, Boulder, Colorado.

Ma, M. T., and L. C. Walters (1969), Power gains for antennas over lossy plane

ground, ESSA Tech. Report ERL 104-ITS 74, U. S. Department of Commerce,

Boulder, Colorado. See also Ma (1974).

Martyn, D. F. (1959), The normal F region of the ionosphere, Proc. IRE 47, No.

2, 147-155.

Pedersen, P. 0. (1927), The Propagation of Radio Waves, Danmarks

Naturvidenskabelige Samfund, Copenhagen.

Philips, M. L. (1962), Evaluation of effective Es reflectivity of obscuration

because of Es ionization, External Tech. Memo No. 14, Electro-Physics Labs.

Piggott, W. R. (1953), The reflection and absorption of radio waves in the

ionosphere, Proc. IEEE 100, Part III, No. 64, pp. 61-72.

Piggott, W. R., and K. Rawer (1972), U.R.S.I. Handbook of ionogram

interpretation and reduction, World Data Center A for Solar-Terrestrial

Physics, Report UAG-23, NOAA, Boulder, Colorado.

200

Ramo, S., and J. R. Whinnery (1960), Fields and waves in modern radio, John

Wiley and Sons, New York.

Ratcliffe, J. A. (1951), A quick method for analyzing ionospheric records, J.

Geophys. Res. 56, 463-485.

Rawer, K. (ed.) (1976), Manual on Ionospheric Absorption Measurements, Report

UAG-57, NOAA, Boulder, Colorado.

Rice, P. L., A. G. Longley, K. A. Norton, and A. P. Barsis (1965),

Transmission Loss Predictions for Tropospheric Communication Circuits, NBS

Technical Note, No. 101, U. S. Department of Commerce, Boulder, Colorado.

Rosich, R. K., and W. B. Jones (1973), The numerical representation of the

critical frequency of the F1 region of the ionosphere, OT Report 73-22,

Boulder, Colorado.

Schelkunoff, S. A., and H. F. Friis (1952), Antennas - theory and practice,

John Wiley and Sons, New York.

Schultz, L. D., and R. M. Gallet (1970), A survey and analysis of normal

ionospheric absorption measurements obtained from pulse reflections, ESSA

Professional Paper 4.

Shimazaki, T. (1955), Worldwide daily variations in the height of the maximum

electron density of the ionospheric F2 layer, J. Radio Res. Labs., Japan, 2,

No. 7, 86-97.

Spogen, Leo R., J. L. Lloyd, and E. P. Moore, Bell Aerosystems Company,

Arizona Operations, "HF and LF Propagation Models for Interference

Prediction," Technical Report No. RADC-TR-67-396, August 1967.

Stanford Research Institute (1957), The application of digital computing

techniques to the problem of interference on HF communication circuits and

networks, Special Report 1, SRI Project 2124, Menlo Park, California.

Whale, H. A. (1969), Effects of ionospheric scattering in very -long distance

radio communications (Plenum Press, New York, New York).

Wheeler, J. L. (1966), Transmission loss for ionospheric propagation above the

standard MUF, Radio Science, 1, No. 11, 1303-1308.

201