OPTIMIZING ANTENNA DESIGNS WITH AO

Copyright 1995 by Brian Beezley, K6STI

All Rights Reserved

Optimizing Antenna Designs With AO.............1

Pattern-Quality Measure........................1

Design Tradeoffs...............................2

Omnidirectional Antennas.......................4

Optimization Method............................5

Running The Optimizer..........................6

Variables Menu.................................7

Frequency Menu.................................8

Sources Menu...................................8

Save Menu......................................9

Optimizer Screen..............................10

Optimizing Tips...............................11

Index.........................................16

---- OPTIMIZING ANTENNA DESIGNS WITH AO ------------------------

AO contains a design optimizer that uses the method of

steepest descent.  This mathematical technique is simple,

effective, and nearly always stable.  Although it may not always

yield the very best of all possible designs, it can greatly

improve the performance of almost any antenna.  The optimizing

technique and its limitations are described later in detail.

The optimizer iteratively varies antenna dimensions to

improve the design objective.  You specify which dimensions to

vary and the nature of the objective.  Dimensions include the

coordinates of wire ends, source magnitudes and phases, and RLC

load values.  The objective is a weighted combination of forward

gain, generalized F/B, SWR, Zr (input resistance), and mag(Zi)

(input reactance magnitude).

The optimizer does not generate an optimal design in one

step.  Instead, it creates a new design that is incrementally

better than the current one.  The new design then is substituted

for the current design and the process repeated.  The algorithm

converges to an optimal design as iteration proceeds.  The

optimizer quits when it decides that no further improvement is

possible.  You may interrupt the iteration process whenever you

like.

You specify which dimensions the optimizer can vary.

Each dimension to be varied must be represented in the antenna

file by a symbol.  The optimizer varies these symbol values to

change the antenna design.  You may use up to 50 symbols in the

antenna file; up to 20 may be specified as optimizer variables.

Use the Variables Menu to specify which symbols are variables.

Be sure to read the sections in AO.DOC on symbolic dimensions,

symbolic expressions, and shifts and rotates before attempting

to optimize a design.

You specify the optimizing objective with the Tradeoffs

Menu.  It's not possible to simultaneously optimize every

desirable antenna characteristic (for example, maximum forward

gain never coincides with maximum F/B).  With tradeoffs, you

tell the optimizer how to balance conflicting design goals.  See

the Design Tradeoffs section for details.

AO can optimize a design at up to ten frequencies

simultaneously.

---- PATTERN-QUALITY MEASURE -----------------------------------

AO uses a generalized notion of front-to-back power

ratio (F/B) as a measure of pattern quality.  Conventional F/B

is the ratio of forward power (at 0 degrees azimuth) to power

radiated in the opposite direction (at 180 degrees).  AO's

generalized F/B is the ratio of forward power to power radiated

within a specified region to the rear of the antenna.  Antenna

designs with good conventional F/B often have large backlobes at

1

angles other than 180 degrees (for example, see BEVERAGE.ANT).

Much better patterns result when designs are optimized for F/B

over an angular region.  The F/B region begins at 180 degrees

azimuth and extends forward to limits set by you.

The F/B region may be defined in the azimuth or

elevation planes, or it may be defined over a solid-angle

region.  The latter can be used to minimize rear power radiated

at angles off the principal planes.  Use the Tradeoffs Menu to

specify the limits of the F/B region.  You may specify the

resolution at which the pattern is sampled.  Use fine resolution

for complex patterns with many small lobes.  Use coarse

resolution for smooth patterns to speed calculation.  The

resolution value is the elevation-angle increment.  Azimuth-

angle resolution automatically varies with elevation angle

(fewer azimuth points are needed to cover the radiation sphere

as elevation angle increases).

AO provides two methods for characterizing power within

the F/B region.  The first method simply averages power over the

region.  When F/B is optimized using this method, total power

within the F/B region is minimized.  This is particularly useful

with EME arrays to minimize total earth noise on receive.  The

second method uses an L4 norm.  This mathematical technique is

like that used to compute RMS power, but the fourth power and

root are used instead of the second.  F/B computed using the L4

norm is approximately the ratio of forward power to the power of

the peak lobe within the F/B region.  When F/B is optimized

using peak power, backlobe amplitudes are minimized while

maintaining roughly equal amplitude.  This method is

particularly useful with HF antennas since it minimizes worst-

case interference from signals arriving at random rear angles.

---- DESIGN TRADEOFFS ------------------------------------------

Maximizing antenna forward gain yields the strongest

transmit signal in the forward direction.  On receive, it

produces the best signal-to-noise ratio for omnidirectional

noise.  However, it also results in a narrow main beam, a large

backlobe, multiple sidelobes, low input impedance, and small SWR

bandwidth.  These undesirable side-effects cannot be ameliorated

without giving up some forward gain.  Similarly, antennas

designed for good patterns sacrifice gain, while those designed

for wide SWR bandwidth give up gain and pattern quality.

Because of the inherent tradeoffs in antenna design, AO

permits a combination of forward gain, F/B, SWR, and input

impedance to be optimized.  You specify the combination of

parameters and their relative importance with the Tradeoffs

Menu.

The parameter tradeoffs are expressed in percent,

indicating relative importance.  For example, to maximize

forward gain regardless of the other parameters, set the gain

tradeoff to 100%.  To maximize gain but also take F/B into

2

account, set the F/B tradeoff to some nonzero value, say 50%.

This tells AO to weigh both forward gain and F/B when

optimizing, and in this case, to give them roughly equal

importance.  AO does this by forming a weighted sum of the

forward gain and F/B values and then maximizing the sum.

When optimization is allowed to run to completion, the

final design will be optimal in the following sense:  Any small

design change that yields a gain improvement of X dB will

necessarily reduce F/B by at least Y dB, where Y is related to X

by the ratio of the optimization summing weights.  In other

words, there will not be any free gain or free F/B to be had,

and to improve one parameter, the other must be sacrificed by a

factor proportional to its relative importance.

SWR may be minimized with the SWR tradeoff.  The SWR

value may refer to a fixed impedance (for example, 50 ohms) or

to the impedance of the antenna at a particular frequency.  See

the Sources Menu section for details.

Zr may be raised with the Zr tradeoff.  The optimizer

does not target a specific Zr value; it just tries to make Zr

higher.  Zr can fall to very low values when optimizing for

maximum forward gain.  Higher Zr eases matching-network design,

minimizes conductivity losses, and broadbands an antenna.

Any antenna may be resonated with a Zi tradeoff.  The

optimizer tries to minimize the magnitude (absolute value) of

input reactance.  Be sure to allow the driven element of

parasitic arrays to vary when resonating a design; otherwise,

parasitic-element lengths will be compromised to achieve

resonance.  AO minimizes mag(Zi) only at SWR reference

frequencies or at frequencies that use a fixed SWR-reference

impedance.  This restriction lets you simultaneously minimize

SWR over multiple bands while resonating the antenna at one

frequency within each band.  See the Sources Menu section for

details.

Vary the tradeoff percentages according to the

importance you place on each design parameter.  The values

themselves are not important; it is their effect that counts.

For example, to emphasize F/B, simply increase the F/B tradeoff

value.  If too much gain is being sacrificed for SWR, raise the

gain tradeoff or lower the SWR tradeoff.  By adjusting the

tradeoff percentages, you can home-in on a design meeting your

requirements.  The design will be optimal in the sense that an

optimal tradeoff will have been made among the performance

parameters according to the importance assigned to each.

You can tell AO that when a parameter reaches a certain

level, it is good enough.  AO will not try to increase the

parameter beyond the specified limit.  For example, if you tell

AO that 20 dB F/B is good enough (set F/B OK > 20 dB in the

Tradeoffs Menu), whenever F/B at an individual frequency goes

above 20 dB, AO will not regard it as an improvement.  This

frees AO to improve other objectives, like forward gain.  Good-

3

enough thresholds work independently at each frequency, so

they're effective at flattening response across a band.  Use the

tradeoff percentages to proportion optimizer driving power

according your design priorities.  Use the good-enough

thresholds to dynamically redirect that power once an objective

has been met.

Forward gain is always computed at the default elevation

angle.  This is the "El Angle for Az Pattern" in the Options

Menu (for convenience, this menu is available both from the main

screen and the optimizer screen).  Use the F/B Az and F/B El

parameters in the Tradeoffs Menu to set the boundaries of the

F/B region.  For antennas over ground, enter 0 for F/B El to

compute F/B at the default elevation angle.  F/B Res is the

sampling resolution for the F/B region.  When the pattern is

smooth, you can speed calculation by using a resolution of 10

degrees or more.  The F/B Res value is the elevation-angle

increment.  It's also the azimuth-angle increment at 0 degrees

elevation.

AO computes F/B over +/- azimuth and +/- elevation

angles for free-space models unless X-axis pattern symmetry is

selected from the Options Menu.  F/B calculation runs four times

faster when you use this option for antennas with symmetrical

patterns (twice as fast for a planar F/B region).  For antennas

over ground, F/B calculation runs twice as fast when you invoke

this option for models with symmetrical azimuth patterns.  Be

sure to take advantage of X-axis pattern symmetry whenever

possible.

You can select peak or average F/B response.  When you

select peak F/B, AO computes F/B internally using an L4 norm

instead of the true peak.  However, true-peak F/B is displayed

because it's more meaningful.  It's possible for L4-norm F/B to

increase while true-peak F/B drops slightly.  If F/B

occasionally drops while optimizing peak F/B with a 100% F/B

tradeoff, this is the reason (the optimizer never decreases the

value of its objective, which is L4-norm F/B in this case).

Average F/B is displayed when selected.

F/B computed during regular analysis is conventional F/B

(rear power is computed at the single point at 180 degrees

azimuth).  This F/B value normally is not the same as that

computed by the optimizer.

When ellipticity is selected, "Ellip" or "Ell" appears

in place of forward gain in the Tradeoffs Menu and elsewhere on

the optimizer screen.

---- OMNIDIRECTIONAL ANTENNAS ----------------------------------

So far, it's been assumed that you're optimizing a

directional antenna.  AO also can optimize designs that are

omnidirectional in azimuth.  The last item in the Tradeoffs Menu

4

lets you define gain as omnidirectional gain instead of forward

gain.

When you select omnidirectional gain, gain is averaged

over 360 degrees azimuth over the selected elevation range.  F/B

is not used.  Instead, AO calculates Delta-G, the variation in

gain over the selected region.  Average or peak variation can be

selected in the Tradeoffs Menu.  When the Delta-G tradeoff is

nonzero, AO tries to minimize gain variation.

You optimize omnidirectional antennas in the same way

that you optimize directional designs.  Whenever Delta-G El is

less than 90 degrees, so that the Delta-G region is not the

entire radiation sphere, it makes sense to use a nonzero gain

tradeoff.  AO will try to focus power uniformly over the

selected region by taking power from the zenith and nadir.  You

can trade average gain for gain ripple by adjusting the gain and

Delta-G tradeoffs.

Average Delta-G is computed using the absolute value of

the dB difference between the gain at an angle and the average

gain over the Delta-G region.  Peak Delta-G applies an L4 norm

to the difference.  To provide a more meaningful number, true-

peak Delta-G is displayed when peak is selected, even though the

L4 norm is used internally.  Peak-to-peak gain ripple is

approximately double the peak value.

You can substitute ellipticity for gain using the

Options Menu.  Delta-E is then displayed in place of Delta-G.

The omnidirectional-optimization feature of AO normally

is not useful with naturally omnidirectional antennas like

simple vertical radiators.  However, to create a vertical design

with uniform gain over a range of elevation angles, you can

define a Delta-G region and optimize for maximum omnidirectional

gain.

---- OPTIMIZATION METHOD ---------------------------------------

AO uses a steepest-descent optimization technique.  This

simple, hill-climbing method (steepest ascent, when maximizing)

works as follows:

First, each variable is changed individually by a very

small amount (with all other variables unchanged) to calculate

the sensitivity of the design objective to that variable.  The

objective is the weighted combination of gain, F/B, SWR, Zr, and

Zi that you've defined with the tradeoffs.  The set of element

sensitivities is called a gradient.  The gradient is an

n-dimensional vector, where n is the number of variables.  The

gradient vector points in the direction that maximizes the

objective.  After the gradient is calculated, an iteration step

is taken in which all variables are updated simultaneously along

the gradient vector, each in proportion to its respective

sensitivity and all in proportion to a step size.  If the step

5

size is not too large, the update will yield a new design with

incrementally higher performance.

Because it takes a long time to calculate the gradient,

the algorithm keeps iterating variables along the same gradient

vector until there is no further improvement.  A new gradient is

then calculated and the process repeated.  The algorithm

automatically adjusts iteration-step size for quickest

convergence.  The optimizer quits when the step size has been

automatically reduced many times in succession, indicating that

further improvement is not possible with small perturbations of

antenna dimensions.

This optimization technique is quite stable and

efficient, locating optimal dimensions for an antenna without

wild-goose chases in a reasonable number of iterations.

However, the method does not guarantee that the global optimum

is found (nor does any other optimization technique, except for

exhaustive search over all possible antenna geometries, which is

impractical).  The global optimum is the very best set of

antenna dimensions possible, where "best" is defined by the

tradeoffs.  The global optimum is missed whenever the algorithm

converges to a local optimum, where small movement in any

direction yields inferior results.

Optimization using perturbation techniques is inherently

nearsighted.  If you're maximizing forward gain, for example,

and there's a small bump in the gain curve for a certain set of

antenna dimensions, the design may wind up on top of the bump

because the algorithm cannot see beyond the immediate vicinity

to the highest point of the overall curve.  If you imagine

trying to climb to the very top of a mountain range with

multiple peaks in dense fog, you can visualize the problem.

Fortunately, local optima are encountered infrequently

with most antenna designs.  They usually occur only when a

design undergoes radical change (for example, when the starting

design is grossly different from the optimal design).  If you

believe that your design has converged to a local optimum, you

can modify antenna dimensions manually and continue automatic

optimization in a different region of the performance space.

To avoid terminating optimization prematurely, AO

continues to iterate even when very small performance

improvements are being made.  Sometimes this slowly inches a

design into a new region of the performance space where

significant improvement becomes possible.  However, whenever you

think further improvement is unlikely, you can interrupt the

optimizer by pressing Esc.

---- RUNNING THE OPTIMIZER -------------------------------------

Press Z from the Main Menu to bring up the Optimizer

Menu.  Before you optimize, you must select variables with the

Variables Menu.  You also should check all settings in the

6

Frequency, Tradeoffs, and Sources menus.  When everything is

set, press Z to start the optimizer.  If you start it with

upper-case Z, the optimizer beeps twice when done if allowed to

run to completion.  Press Esc at any time to interrupt it.

Press Z to restart it.  Press Esc to exit the optimizer and

return to the Main Menu.  (If the Optimizer Menu covers screen

information, pressing Esc hides the menu so you can see the

whole screen.  Press Esc once more to return to the Main Menu or

any other key to bring back the Optimizer Menu.)

Whenever you return to the Main Menu after optimizing,

the current model is the optimized design.  You can return to

the Main Menu to manually edit a design or generate a plot, and

then continue optimization.  (All sources are activated when you

return to the Main Menu.  You may have to disable some to

analyze or plot a multiband design.)

Once the optimizer changes a design, a file named

OUT.ANT becomes the current antenna file.  OUT.ANT is the same

as the original antenna file except that symbol definitions

reflect optimized values.  AO never modifies the original

antenna file (but you can overwrite it with the Save command).

OUT.ANT becomes the current antenna file only after the

optimizer changes the design (or you save the design to a file).

When in doubt, check the top line of the RUN file to determine

the current antenna file.

When you quit AO, the optimized design is in OUT.ANT.

You must rename OUT.ANT to save it.  If you don't, it will be

overwritten the next time you optimize a design.

AO uses the same segmentation density for optimization

as for normal analysis.  You can speed optimization considerably

by using fewer segments when accuracy permits.

---- VARIABLES MENU --------------------------------------------

Use the Variables Menu to tell AO which antenna-file

symbols to vary to optimize the design.  Dependent symbols

(those defined in terms of other symbols) cannot be optimized

and are not listed in the Variables Menu.  Only the first 20

independent symbols in the antenna file can be varied by the

optimizer.  If you use lots of symbols, you may have to

rearrange them to get the ones you want to vary within the first

20.

For maximum speed, always vary the fewest possible

number of symbols.  For example, it's not necessary to vary

driven-element length when optimizing Yagis or cubical quads

unless you want to resonate the design.  (Conversely, you

shouldn't attempt to resonate these designs without allowing

driven-element length to vary.)

Sometimes you must use one variable to represent more

than one dimension.  For example, side half-length must control

7

loop size for cubical quads; to preserve shape and symmetry,

only one variable can be used.  Here's another example:  To

maintain pattern symmetry in a four-square array, the two middle

elements must use the same variables for source magnitude and

phase.

Use the * key to fix or vary all listed symbols.

While you can define symbols for impedance and Laplace

Transform loads, AO optimizes RLC loads only.

---- FREQUENCY MENU --------------------------------------------

Use the Frequency Menu to select up to ten optimization

frequencies.  AO optimizes simultaneously at all frequencies

(this means that the objective function at all frequencies is

taken into account when new variable values are determined).  AO

weighs all frequencies equally.  Unless automatic segmentation

is disabled, the design is automatically resegmented at each

frequency.

For maximum speed, optimize at the fewest possible

number of frequencies.  To generate a broadband design without

optimizing at multiple frequencies, try Jim McCook, W6YA,'s

trick of specifying a relatively high Zr tradeoff value.  This

shortcut won't produce a design as good as one optimized over

the intended frequency range, but it's very handy for quick-and-

dirty optimization, particularly when combined with sparse

segmentation.

---- SOURCES MENU ----------------------------------------------

The Sources Menu lets you select which sources are

active at each frequency.  It also lets you select the SWR

reference for each frequency.

Frequencies are listed vertically and are labeled F1

through F10.  The labels are used only to identify frequencies

along the top of the menu.  S1 through S16 identify sources in

the order of appearance in the antenna file.  Sources beyond the

first 16 do not appear in the menu and are always active at all

frequencies.

Most antennas excite all sources at each frequency.  AO

defaults to this condition.  However, multiband antennas often

have a separate driven element for each band.  For these models,

define all possible sources in the antenna file.  Then use the

Sources Menu to disable some of the sources at each frequency.

Use the arrow keys to move the [ ] cursor to a rectangular block

symbol.  The block represents activation of a source.  Press the

spacebar to toggle the source on or off.  AO beeps and won't let

you leave the Sources Menu unless there is at least one active

source defined for each frequency.

8

When AO models an inactive source, it shorts it.

However, the actual impedance at the source location depends on

the matching network and on the impedance, length, and far-end

termination of the feedline.  Keep this fact in mind whenever

you notice significant current in the unfed driven elements of

multiband designs.

SWR can be referred to a fixed impedance (that defaults

to 50 ohms) or to the antenna input impedance at a particular

frequency.  For antennas fed directly without a matching

network, a fixed-impedance SWR reference often is appropriate.

This is the AO default.  When optimizing a model across a

frequency band, you can tell AO to use the complex conjugate of

input impedance at one frequency as the SWR reference impedance.

Effectively, the antenna is assumed to be matched at this

frequency (the reference frequency), where the SWR will be 1.

The SWR at other frequencies indicates the design bandwidth.  AO

assumes that a bandwidth-independent matching network is used.

This assumption is very good for single-band matching networks

like gamma, T, and hairpin matches.

To change the SWR reference, use the arrow keys to move

the cursor (which becomes < > in the SWR-reference part of the

menu) to the desired frequency and press the spacebar.  The

diamond, which indicates the active SWR reference, moves to the

cursor location.  Each frequency has one SWR reference, so

there's just one diamond per line (several block symbols are

possible).  When optimizing multiband antennas, you'll probably

want a separate SWR reference for each band.  For example, you

might optimize a dual-band Yagi at three frequencies on 20

meters and three on 15.  You can select the middle 20-meter

frequency as the SWR reference for each 20-meter frequency and

the middle 15-meter frequency as the SWR reference for each 15-

meter frequency.  AO beeps and won't let you leave the Sources

Menu if more than one source is active at any SWR-reference

frequency (this includes sources beyond the first 16; these

sources are always active at all frequencies).

The Sources Menu may seem complicated, but it provides

complete generality for elaborate models.  Fortunately, you

never need to use the Sources Menu when optimizing antennas at a

single frequency (as long as you want to excite all sources

defined in the antenna file).

---- SAVE MENU -------------------------------------------------

You can save the current design in a file with the Save

Menu.  It's not necessary to enter the .ANT file extension.  You

can specify a drive or path if necessary.  If you press just

Enter, the last file saved is updated and overwritten (if none

has been saved, SAVE.ANT is written).

It's a good idea to save interesting intermediate

designs while optimizing.  This lets you come back later and

investigate promising design-path forks.  You can update and

9

overwrite the input file by specifying it as the output

filename.  You can print the current design by entering PRN or

LPT.

---- OPTIMIZER SCREEN ------------------------------------------

AO lists gain (or ellipticity), F/B (or Delta-G or

Delta-E), Zr, Zi, and SWR for the starting model at all

frequencies along the top of the screen.  As optimization

proceeds, current values are tabulated in a second block below.

Whenever you restart the optimizer, current values become

starting values and go on top, while new values appear below.

To save computation, F/B is not listed when the F/B

tradeoff is 0%.  (However, Delta-G and Delta-E are listed

because their values may be computed quickly.)  Zr, Zi, and SWR

are not listed at frequencies with more than one active source.

This eliminates ambiguity about which source the numbers apply

to.  Gain values in dBdc or dBic are mislabeled as dBd or dBi on

the optimizer screen.

Below optimization results, AO lists the name and

current value of each variable (the first eight characters of

long variable names are listed).  The column labeled with a

delta symbol gives the magnitude of the most recent change for

each variable.  This number is the change as a percentage of the

largest variable of the same type.  There are 11 types of

variables, including wire coordinates that affect wire lengths,

coordinates that don't, source magnitudes, source phases,

resistive loads, etc.  Delta is displayed in thousandths of a

percent.  Values greater than 9999 are displayed as 9999.  This

number shows how strongly each variable affects the design

objective.  You can speed optimization by fixing variables with

consistently low delta.  Such variables won't improve the design

very much.

The bottom line shows the status of the optimizer.

Model count is incremented each time AO recomputes wire

currents.  For multifrequency models, the current frequency is

given next.  The current operation is shown at the center of the

status line.  When computing the gradient, AO shows the name of

the variable being analyzed.  When taking an iteration step, AO

displays the step size.  This number scales the individual

gradient components to calculate new variable values.  The step

size begins at 1 (1000 for insensitive models) and is

automatically adjusted for best convergence.  AO increases the

step size when it can for faster convergence.  It lowers it

whenever a trial design reduces the value of the design

objective.  By watching the step size change, you can get a

feeling for optimization progress.  Whenever an error occurs

during optimization, "Error" appears next on the status line.

See the Optimizing Tips section for more information on errors.

A set of bar graphs for the various analysis steps

appears in the lower-right screen corner.  When the optimizer

10

modifies source magnitude or phase, it doesn't need to refill

and refactor the mutual-impedance matrix.  Two steps that

normally are very fast, file parse and matrix solve, have no

bargraphs.  When optimizing just source magnitudes and phases

with no F/B tradeoff, the optimization rate is limited by these

two invisible steps.

No optimization results are recorded in the RUN file.

---- OPTIMIZING TIPS -------------------------------------------

Tip #1:  Don't Walk Away

If you feed AO a starting design, press Z, walk away,

come back sometime later, and build what's in OUT.ANT, the

design probably will work well and you'll be happy.  You'll be

happy until later, when you finally spend some time fooling

around with AO.  Then, chances are good you'll be disappointed

with your antenna and frustrated by the whole experience.

Despite sophisticated optimization algorithms, antenna design

really can't be fully automated--it can't take into account

intangibles that involve you.

What do you really want out of a design?  You want the

best possible antenna, of course!  But what do you mean by best?

That depends on a variety of factors whose relative importance

is known only to yourself.  OK, so you translate your design

preferences into tradeoff values, press Z, and walk away.  Fine,

except that while you're gone, AO happens to iterate across a

remarkable antenna geometry for which all backlobes are 40 dB

down.  This once-in-a-lifetime design is passed up because it

costs 0.1 dB in forward gain, you told AO to optimize for 90%

gain and 10% F/B, and the numbers just don't work out (or you

told it that 25 dB F/B was good enough).

One intangible that you cannot communicate to AO is your

valuation of unexpected discoveries in the performance space.

You might be perfectly happy with a result, but if AO comes

across a novel design that isn't quite as good according to your

tradeoffs, it will pass it up.  AO isn't smart enough to know

that you might be interested in unique designs that don't quite

meet your stated criteria.

The example above exaggerates for emphasis.  As

presented, it's not likely to occur.  Yet, in milder form, such

things do happen.  The point is this:  Without letting AO run

and watching what happens, you have no idea what constitutes a

reasonable design or a good design.  You don't know the limits

of the performance space.  Unless you have a lot of experience

with antennas, you really don't know what a particular

configuration is capable of.  Therefore, you don't know how to

set the tradeoffs and good-enough thresholds wisely.

So the first tip is this:  Don't run AO unattended until

you've gained some experience with it.  Stick around to see what

11

it comes up with.  If your computer is slow and watching AO run

is unbearable, come back from time to time to see what it's up

to.  Experiment freely with tradeoff values and good-enough

thresholds.  Try different starting geometries.  Don't make up

your mind about what you want until you see what's really

possible.

Tip #2:  Start With A Good Design

You can make AO's job a lot easier, and the optimization

a lot quicker, by starting with a good design.  A reasonable

design also makes landing on a local optimum less likely and

lowers the chance of optimizer instability.

It can be a lot of fun to throw a design with arbitrary

dimensions at AO and watch it sort things out.  You can get a

good idea of how the algorithm works by doing this since the

dimension changes are easy to follow.  But it's not a good way

to arrive at a final design if you're serious about the results.

AO does not do a global search of the performance space.

If you feed it a 3-element, 20-meter Yagi with a 2-foot boom and

tell it to go for maximum forward gain, the chances of it

lengthening the boom properly to 25-30 feet without encountering

a local optimum are remote.  (After writing this, I decided to

try it.  For the starting design I used, AO increased the

boomlength 25% and stopped with a 30-inch boom).

Tip #3:  Don't Let Errors Scare You

Occasionally the optimizer will change a variable so

drastically that invalid antenna geometry results and "Error" is

displayed on the status line.  AO continues to optimize after

most errors.  The error message is intended to give you greater

insight into what's happening; it's not intended to alarm you.

Although AO tries its best to keep variables within

reasonable bounds (like non-negative resistive loads, for

example), most of the time it simply can't tell whether the

geometry of a trial design is valid without actually trying it.

When an error is detected, AO goes through a backstep procedure

to recover the last valid geometry and try again with reduced

step size.  This usually puts the optimizer back on track.  When

optimization ends, the total number of errors is displayed on

the status line if nonzero.  Be suspicious of models with high

error counts.

If AO runs out of pulses and can't recover from this

error with two attempts, the "Too Many Pulses" error message is

displayed and optimization halts.  This may simply indicate that

a valid design has expanded beyond the capacity of AO to model

it.  It may also indicate that the geometry has become scrambled

and a wire length has blown up beyond reason.

12

Whenever you run out of pulses during optimization, it's

best to try to understand why AO wants more.  For example, if

you optimize a long-wire antenna for maximum near-endfire gain,

AO will increase wire length until its capacity is exhausted.

This is natural, since the longer the wire, the more gain

available.  In this case, the error indicates a modeling

oversight rather than a design problem.

When you're just fooling around with AO and it's too

much trouble to analyze the cause of an error, simply edit the

antenna file to correct the geometry and continue optimization

without varying wild dimensions.

Some errors could be eliminated by providing a menu in

which you specify valid bounds for each variable.  However, I

decided that AO's setup procedure was complicated enough without

requiring you to deal with yet another menu.  Errors occur

seldom enough that I thought it better to risk frightening you

with an error message than to overburden you.

Whenever errors are reported, it's a good idea to check

the geometry display and the antenna-file image in the RUN file

for problems.

Tip #4:  Don't Accept Unreasonable Results

AO models reality, but not necessarily correctly.  It

does a very good job with antennas, but you shouldn't believe

everything it tells you.  It has its limitations.  See the

Algorithm Limitations and Corrections section in AO.DOC for

details.

Sometimes the segmentation density AO starts with

becomes inadequate as optimization proceeds.  This can happen,

for example, when wires migrate very close.  At other times,

despite internal checks, AO may fold one wire on top of another.

The usual result in these cases is unrealistically high forward

gain.  If results begin to look funny, stop the optimizer and

inspect antenna geometry with the geometry display.  These kinds

of things usually occur only when you hand AO an unreasonable

problem, like the 2-foot Yagi mentioned earlier.

Occasionally I get telephone calls that go something

like this:  The caller, who has fastidiously modeled some

simple, trick antenna, wants to know if the reported 12 dBd gain

is actually real so he can decide whether to commence immediate

commercial production of his breakthrough product.

Don't abandon common sense, and don't let an unruly AO

intimidate you.

13

Tip #5:  Be Wary of Narrowband Optimization

It can be exhilarating to optimize an antenna at a spot

frequency.  Remarkable gains and impressive patterns often are

possible.  Matched SWR is always 1.  But before you build a

narrowband design, take the trouble to examine its frequency

response.  Whenever you specify just one frequency, the

optimizer takes you quite literally:  It is totally oblivious to

antenna response anywhere else.  Unless you use lots of

segments, optimize an unusually insensitive geometry, mount the

antenna entirely in the clear, and construct it with great

workmanship and precision, you're asking for trouble with a

narrowband design.

AO isn't perfect and neither are you.  For anything

other than a paper design, it's prudent to back off and trade

some performance for realizability and bandwidth.  Most errors,

whether caused by modeling, environment, or construction,

manifest themselves as a frequency offset between predicted and

actual performance.  Wideband designs are naturally robust to

frequency offsets.

Experience is everything when attempting to judge the

practicality of a design.  But you may find this rule helpful:

If predicted antenna performance would make you nervous if it

hadn't been generated by a computer, don't trust the design.

Tip #6:  Have Patience

The computational power of affordable computers

increases dramatically each year and this trend shows no sign of

slowing.  Yet even the fastest of today's computers may take a

long time to optimize a complex design over multiple

frequencies.

Experience with the YO Yagi Optimizer program, which

runs hundreds of times faster than AO and from which AO's

optimizer was derived, has shown that complex Yagi models

sometimes require thousands or even tens of thousands of

iterations to converge to an optimal design.  For production

runs in which you want the very best results, you may be

shortchanging yourself if you press Esc when the design seems to

have settled after just a few hundred iterations.

You'll need experience with AO before you can

confidently pull the plug on the optimizer.  Sometimes a design

can languish on a stable and apparently optimal plateau for

hundreds of iterations, only to break out into a new region of

the performance space where rapid improvement occurs.  This

situation doesn't happen often, but you should be aware that

it's possible.

If you're just playing around with AO, it's of little

value to let it grind out the last .01 dB of forward gain.  You

can learn more by turning it loose on a different problem.  But

14

for a design you're going to construct, you really owe it to

yourself to let AO run to completion.  After all, it's easy to

let it run overnight.  (Tom Ring, WA2PHW, holds the current

record with a four-day optimization of a long-boom, dual-band,

VHF Yagi.)

Tip #7:  Adjusting Variable Sensitivity

Occasionally you may encounter a model that is

unresponsive or hypersensitive to a certain variable.  You may

feel that the variable should get more or less attention than

its gradient component warrants.  Since you have a global view

of the problem while AO's knowledge is strictly local, your

judgment may be superior.

You can increase sensitivity to a variable by

multiplying it by a constant.  For example, rather than using

the symbol x directly, set y = 2 * x and use y instead.  In this

case, results will be twice as sensitive to changes in x.  You

can increase sensitivity as x increases with y = x * x.

Similarly, you can divide by a constant to lower

sensitivity uniformly, for example, set y = x / 2.  You can use

y = SQR(x) to lower sensitivity as x increases.

15

INDEX

* key  8

180 degrees azimuth  4

50 ohms  9

9999  10

< >  9

[ ]  8

Absolute value  3

Algorithm  6

Analysis steps  10

Angular region  2

ANT file extension  9

Antenna file  7

AO isn't perfect and neither are you  14

Average F/B  4

Averages power  2

Backlobe amplitudes  2

Backlobes  1

Backstep procedure  12

Bandwidth  9

Bar graphs  10

Beep  7, 8, 9

Best convergence  10

Best possible antenna  11

Best results  14

Best set of antenna dimensions  6

BEVERAGE.ANT  1

Block  8

Blown up beyond reason  12

Bottom line  10

Bounds  12

Break out  14

Broadband  3, 8

Change for each variable  10

Characterizing power  2

Complex conjugate  9

Conductivity losses  3

Conflicting design goals  1

Conventional F/B  1, 4

Converge  1

Convergence  6

Coordinates that affect wire lengths  10

Current model  7

Current operation  10

DBdc  10

DBic  10

Default elevation angle  4

Delta symbol  10

Delta-E  5, 10

Delta-G  5, 10

16

Dependent symbols  7

Design Tradeoffs  2

Design-path forks  9

Diamond  9

Dimensions  1

Don't abandon common sense  13

Drive  9

Driven element  3, 8

Dual-band Yagi  9

Earth noise  2

Elevation-angle increment  2

Ellipticity  5

EME arrays  2

Enter  9

Error  10, 12, 13

Esc  7

Exhilarating  14

F/B  1, 2

F/B Az  4

F/B El  4

F/B region  2, 4

F/B Res  4

F/B tradeoff  2

F1  8

Faster convergence  10

File parse  11

Fixed impedance  3, 9

Flattening response across a band  4

Fooling around with AO  11, 13

Forward gain  2, 5

Forward power  1

Four-square array  8

Free gain or free F/B  3

Frequency Menu  8

Front-to-back  1

Gain ripple  5

Gain tradeoff  2

Gain variation  5

Gamma  9

Generalized F/B  1

Global optimum  6

Global search  12

Global view  15

Good-enough thresholds  3

Gradient  5, 10

Hairpin match  9

HF antennas  2

Hill-climbing method  5

Home-in on a design  3

Hypersensitive  15

17

Impedance  3

Impressive patterns  14

Improve the design  10

Independent symbols  7

Input impedance  2, 9

Input reactance  3

Input reactance magnitude  1

Input resistance  1

Intangibles  11

Intermediate designs  9

Interrupt  6

Invalid antenna geometry  12

Invisible steps  11

Iteration  1, 5, 14

L4 norm  2, 4, 5

Limit  2, 3, 11

Limitations  13

Limits of the F/B region  2

Local optimum  6, 12

LPT  10

Mag(Zi)  1

Magnitude  3, 10

Matching network  9

Matching-network design  3

Matrix solve  11

McCook  8

Method of steepest descent  1

Minimize SWR over multiple bands  3

Model count  10

Multiband  7, 8, 9

Multifrequency models  10

Multiple peaks  6

Narrowband design  14

Naturally omnidirectional antennas  5

Objective  1

Objective function  8

OK >  3

Omnidirectional Antennas  4

Omnidirectional gain  4

Omnidirectional noise  2

Once-in-a-lifetime design  11

Optimal design  1

Optimal dimensions  6

Optimal plateau  14

Optimal tradeoff  3

Optimization Method  5

Optimization progress  10

Optimization results  11

Optimization summing weights  3

Optimized values  7

Optimizer instability  12

Optimizer Menu  6

18

Optimizer Screen  10

Optimizer variables  1

Optimizing Antenna Designs With AO  1

Optimizing objective  1

Optimizing Tips  11

OUT.ANT  7

Overwrite the input file  9

Overwritten  7, 9

Paper design  14

Parameter tradeoffs  2

Parasitic arrays  3

Path  9

Pattern-Quality Measure  1

Peak F/B  4

Peak lobe  2

Peak power  2

Peak-to-peak gain ripple  5

Percent  2, 10

Performance space  6, 11, 12, 14

Perturbations of antenna dimensions  6

Power  2

Principal planes  2

Print the current design  10

PRN  10

Pull the plug on the optimizer  14

Quads  8

Quick-and-dirty optimization  8

Radiation sphere  2

Random rear angles  2

Receive  2

Record  15

Rectangular block symbol  8

Region  1

Relative importance  2

Remarkable gains  14

Rename OUT.ANT  7

Resegment  8

Resolution  2

Resonate  3, 7

Ring  15

RLC load values  1

RLC loads  8

RMS power  2

RUN file  11, 13

Running The Optimizer  6

S1  8

Save Menu  9

Save the current design  9

SAVE.ANT  9

Segmentation density  7

Sensitivity to a variable  15

Shifts and rotates  1

19

Signal-to-noise ratio  2

Solid-angle region  2

Source magnitudes and phases  1, 11

Sources  8

Sources Menu  8

Spacebar  8

Sparse segmentation  8

Speed  2, 4, 7, 8, 10

Spot frequency  14

SQR  15

Status  10

Steepest-descent optimization  5

Step size  5, 6, 10, 12

SWR  2, 3

SWR bandwidth  2

SWR reference  3, 8

Symbol  1, 7

Symbolic dimensions  1

Symbolic expressions  1

T  9

Terminating optimization  6

Tip #1:  Don't Walk Away  11

Tip #2:  Start With A Good Design  12

Tip #3:  Don't Let Errors Scare You  12

Tip #4:  Don't Accept Unreasonable Results  13

Tip #5:  Be Wary of Narrowband Optimization  14

Tip #6:  Have Patience  14

Tip #7:  Adjusting Variable Sensitivity  15

Total power  2

Tradeoff percentages  3

Tradeoffs  2

Tradeoffs Menu  2

Transmit  2

Trick  8

Trick antenna  13

True-peak Delta-G  5

True-peak F/B  4

Unattended  11

Undesirable side-effects  2

Unexpected discoveries  11

Uniform gain over a range of elevation angles  5

Unique designs  11

Unrealistically high forward gain  13

Unresponsive  15

Update  5, 9

Upper-case Z  7

Valid geometry  12

Variable of the same type  10

Variables  1

Variables Menu  7

Variation in gain  5

Vertical radiators  5

20

W6YA  8

WA2PHW  15

Weighted  1, 3, 5

What do you really want out of a design?  11

Wild dimensions  13

Wild-goose chase  6

Wire ends  1

Worst-case interference  2

X-axis pattern symmetry  4

YO Yagi Optimizer  14

Z  6

Zi tradeoff  3

Zr tradeoff  3

21