I cannot provide a correct switch settings for a GT4000. I do not have one.

The switches according to the manual:

(; how archaic: typed in instead of a scanned picture ;)

0=OFF 1=ON

1 2 3 4 5 6 7 8

SW1: b b b s p p l l	b=Baud rate   s=stop bit  p=parity   l=line printer

1 0 0 1 0 0 0 0	Factory default

0 0 0		19200 bps

1 0 0		 9600 bps (factory)

0 1 0		 4800 bps

1 1 0		 2400 bps

0 0 1		 1200 bps

1 0 1		  600 bps

0 1 1		  300 bps

1 1 1		    -

0		1 stopbit

1		2 stopbits (factory)

x 0	parity disabled (factory)

0 1	parity odd

1 1	parity even

0 0	EPSON 24 bit color printer (factory)

1 x	undefined

x 1	undefined

1 2 3 4 5 6 7 8

SW2: h h g g g c c 0	h=halftoning mode   g=gamma corr.   c=color corr.

1 0 0 0 0 1 1 0	Factory default

0 0		disabled

1 0		halftoning mode A (factory)

0 1		halftoning mode B

1 1		halftoning mode C

0 0 0		CRT Display A (factory)

1 0 0		CRT Display B

0 1 0		Printer Output A

1 1 0		Printer Output B

0 0 1		Printer Output C

1 x 1		undefined

x 1 1		undefined

0 0	Impact dot matrix printer

1 0	Thermal transfer printer

0 1	Ink jet printer

1 1	CRT display (factory)

0	Always 0