This describes how to setup the device driver of TOSHIBA ATAPI CD-ROM Drive.

This device driver is for exclusive use of TOSHIBA ATAPI CD-ROM drives.

How to setup the CD-ROM device driver manualy ?

1. Copy the CD-ROM device driver to the system disk.

2. Add the following line to CONFIG.SYS of the system disk.

LASTDRIVE=Z

DEVICE=<drive>:<path>\TOSHVxxx.SYS <option switch>

Option switches

/D:<device name>     Specifying the device name.

Must match the device name of MSCDEX.EXE.

/C:<turn stop time>  It specifies the time which stop the drive motor

when not using CD-ROM drive.

The following value can be specified.

/C:1      125 ms

/C:2      250 ms

/C:3      500 ms

/C:4        1 s

/C:5        2 s

/C:6        4 s

/C:7        8 s

/C:8       16 s

/C:9       32 s     (Default)

/C:A        1 min

/C:B        2 min

/C:C        4 min

/C:D        8 min

/C:E       16 min

/C:F       32 min

/V:                  Display the detailed information on power up.

/B:                  Driver enables the DMA transfer. (See Note)

[For example]

Device driver copied to C:\CDROM. Specifying TOSCD001 in the device

name and specifying 1 minute in the turn stop time.

DEVICE=C:\CDROM\CDROMDRV.SYS /D:TOSCD001 /C:A

3. Add the following line to AUTOEXEC.BAT.

C:\DOS\MSCDEX.EXE /D:<device name>

4. Restart computer.

Note:   There is the following restrictive item for (/B) option switch.

1. Driver supports DMA transfer only for Intel Triton and Triton2 PCI Chip

Set.

2. If doing DMA transfer with Intel Triton2 PCI chip set, use EMM386.EXE

version below 4.49. (EMM386.EXE Version4.49 should be in MS-DOS 6.22.)

3. If application buffer address (store CD-ROM data) is not word align,

the driver will not transfer data via DMA. (Use PIO transfer)