*************************************

*                                   *

*              S U B T              *

*  ON THE MVS/XA OPERATING SYSTEM   *

*                                   *

*************************************

John S. Ward

03/18/93

EJECT

SUBJECT: NEW SUBT MACRO FOR SUBMITTING JCL FROM         DATE:02/01/93

ISPF/PDF EDIT

Due to the new CPU time batch restrictions on the MVS system, a new

ISREDIT MACRO called SUBT has been written.  It works very much like

the SUB command for submitting a JCL program from the edit panel in

ISPF/PDF.  The purpose of SUBT is to simplify the task of specifying

a CPU time limit for a job you submit directly out of edit.  With the

new batch time restrictions if time is not specified on the job card

of a batch job, the system will set time to the maximum value and your

job will not run until after-hours.

The way SUBT accomplishes this is by building a job card for you and

temporarily inserting it on top of your code.  For this reason your

job stream can not already contain a job card.  Unlike the SUB command

which checks for the existence of a job card before creating one, SUBT

will just insert another one.  So if you already have a job card on

your JCL run stream, make sure you have coded the time parameter and

use the Sub command as usual.

When entered from the command line while in edit mode, SUBT builds

your job card for you with a default time of 5 CPU seconds.  A job with

a time of 5 seconds or less has the maximum chance of beginning

execution quickly.  See the member BATCHCPU in SYS.MVS.UPDATE for more

information on coding the time parameter for the batch job time

control.  SUBT will also set the jobname on your job card to your

userid plus 'X' as the default, but you can override both of these

defaults if you wish.

The syntax for the SUBT is as follows:

SUBT           - A job card will be created with your userid plus

'X' and a time value of 5 seconds.  (both defaults)

SUBT B         - A job card will be created with your userid plus

'B' and a time value of 5 seconds.  (time default)

SUBT AA,45     - A job card will be created with your userid plus

'AA' and a time value of 45 seconds.

After you have entered the SUBT command, you will either be returned

to the EDIT panel (where you see the contents of your job stream) or

back to the member selection list in EDIT.  The reason for this is that

when the SUBT command temporarily inserts a job card on top of your

JCL run stream, the changed dataset bit is set.  If you have not made

any changes to the member before the SUBT command, then you will be

brought back to the member selection list.  In this way you will not

be rewriting your member every time you use the SUBT command.  If you

have made changes to the member before using the SUBT command, you will

be returned to the EDIT panel as normal.

NOTE: The Subt command is only supported for interactive use only.

This command is not to be used in the batch environment.