����������������������������������������������������������

COMPARISON BETWEEN DOS FORMAT, FDFORMAT 1.8 AND 2M/2MF 2.0

����������������������������������������������������������

Note: If you are a DISKCOPY or FORMAT programs

programmer,  read the note at the end of

this file as soon as possible.

2M is not only the highest capacity formatter at this moment,  but

also 2M diskettes are the faster ones.  This test will show it to you.

Note that some absolute results may slightly vary from one computer to

another, but the relation between formatter programs doesn't vary.  If

you decide to use 2M, you will realize that default formats (820/1476K

and 984/1804/3608K) are also as reliable as DOS standard 360K/1.2M and

720K/1.44/2.88M).

1. - MAXIMUM STORAGE AFTER FORMAT.

����������������������������������

Physical limits are 1,025,000 for 5�-DD (with 82 tracks)  and 1,708,224

bytes in 5�-HD.  In 3�-DD  (high density controller card at 300 Kbit/sec)

the limit is 1,230,000 bytes; in 3�-HD, 2,050,000 bytes.

�����������������������������������������Ŀ

�   Double Density   �   High Density     �

�����������������������������������������������������������������������Ŀ

� FORMAT (40/80t)      �   368,640   (360K) � 1,228,800 (1,200K) �      �

� FDFORMAT (82t)       �   839,680   (820K) � 1,511,424 (1,476K) � 5.25 �

� 2MF /F (82t)         �   839,680   (820K) � 1,511,424 (1,476K) �      �

� 2MF /M (82t)         �   923,648   (902K) � 1,595,392 (1,558K) � (5�) �

�����������������������������������������������������������������������Ĵ

� FORMAT (40/80t)      �   737,280   (720K) � 1,474,560 (1,440K) �      �

� FDFORMAT (82t)       �   839,680   (820K) � 1,763,328 (1,722K) � 3.5  �

� 2MF /F (82t)         � 1,007,616   (984K) � 1,847,296 (1,804K) �      �

� 2MF /M (82t)         � 1,091,584 (1,066K) � 1,931,254 (1,886K) � (3�) �

�������������������������������������������������������������������������

��������������������Ŀ

� Extra High Density �

��������������������������������������������������Ŀ

� FORMAT (40/80t)      � 2,949,120 (2,880K) �      �

� 2MF /F (82t)         � 3,694,592 (3,608K) � 3.5  �

� 2MF /M (82t)         � 3,862,528 (3,772K) � (3�) �

����������������������������������������������������

2. - PERFORMANCE AT DOS LEVEL.

������������������������������

In order to compare the real performance of different diskette formats,

here are the results of a simple test.  To simplify,  only 3.5-HD diskette

results are shown.  The TEST directory of hard disk contained two files of

256 Kb, 3 files of 128 Kb, 4 files of 64 Kb,  5 files of 32 Kb, 6 files of

16 Kb and 1 file of 15.5 Kb, totalling 1423.5 Kb on 21 files.

Write test:   COPY C:\TEST\*.* B:

Read test:    COPY /B B:\*.* NUL

����������������������������������������������������������������������������Ŀ

� Disk Size      �  1.44   �  1.44   �  1.64   �  1.72   �  1.80   �  1.88   �

����������������������������������������������������������������������������Ĵ

� Formatter      �  FORMAT �FDFORMAT �FDFORMAT �FDFORMAT �   2MF   �   2MF   �

����������������������������������������������������������������������������Ĵ

� Write time     � 1:17.27 � 1:06.72 � 1:00.74 � 1:27.05 � 1:15.30 � 1:23.93 �

����������������������������������������������������������������������������Ĵ

� Read time      � 0:59.82 � 0:48.50 � 0:44.11 � 1:05.69 � 0:43.78 � 0:54.16 �

����������������������������������������������������������������������������Ĵ

� Free space left�       0 �       0 � 203,776 � 287,744 � 370,688 � 454,656 �

����������������������������������������������������������������������������Ĵ

� Write (Kb/sec) �  18.42  �  21.34  �  23.44  �  16.35  �  18.90  �  16.96  �

����������������������������������������������������������������������������Ĵ

� Read (Kb/sec)  �  23.80  �  29.35  �  32.27  �  21.67  �  32.51  �  26.28  �

����������������������������������������������������������������������������Ĵ

� Kb/sec average �  21.11  �  25.35  �  27.86  �  19.01  �  25.71  �  21.62  �

� Relative Index � 100.00  � 120.09  � 131.98  �  90.05  � 121.79  � 102.42  �

������������������������������������������������������������������������������

Notes:

- The test was executed on MS-DOS 6.2 with only 2M and FDREAD in memory.

- /B switch is necessary to COPY binary files onto character device as NUL.

- Reading from hard disk takes 5.5 sec (measured with the command COPY /B

C:\TEST\*.* NUL), this time was subtracted from write test's time.

- Diskette drive motor was on when COPY started.

- FDFORMAT diskettes were formatted with /X:2 and /Y:3 switches.

3. - PERFORMANCE AT FULL-TRACK ACCESS LEVEL.

��������������������������������������������

This results are obtained with the 2M-FDTR (Floppy Data Transfer Rate)

utility, which reads/writes a complete diskette in  whole-track  blocks,

always taking care on never using buffers across 64K DMA frontier (which

will results in less performance).  All  statistics shown are in Kb/sec.

The physical limits are:  36,62 Kb/sec at 300 Kbps  (5�-DD and 2M 3�-DD)

and 61,03 Kb/sec at 500 Kbps (5�-HD and 3�-HD).  This  results  are only

possible on AT computers.

��������������������������������������������������Ŀ

� Double Density �  High Density  �  Extra High D. �

��������������������������������������������������Ĵ

�   Read  Write  �   Read  Write  �   Read  Write  �

�������������������������������������������������������������������������Ŀ

� FORMAT        �  18.16  18.16  �  30.13  30.13  �                �      �

� FDFORMAT  (*) �  22.11  22.12  �  39.73  39.73  �                � 5.25 �

� FDFORMAT (**) �  25.00  25.00  �  25.26  25.23  �                �      �

� 2MF /F        �  25.04  25.00  �  46.33  46.33  �                � (5�) �

� 2MF /M        �  16.49  16.49  �  28.50  28.47  �                �      �

�������������������������������������������������������������������������Ĵ

� FORMAT        �  15.05  15.05  �  30.14  30.14  �  ??.??  ??.??  �      �

� FDFORMAT  (*) �  19.32  19.32  �  39.58  39.53  �   --     --    � 3.5  �

� FDFORMAT (**) �  21.78  21.75  �  24.79  24.79  �   --     --    �      �

� 2MF /F        �  25.72  25.76  �  48.49  48.50  �  ??.??  ??.??  � (3�) �

� 2MF /M        �  16.25  16.25  �  28.74  28.77  �  ??.??  ??.??  �      �

���������������������������������������������������������������������������

(*) DOS standard formats (360-720-1.2-1.44) and best /X and /Y.

(**) Maximum capacity formats (820-1.48-1.72) and best /X and /Y.

4. - FORMAT TIME.

�����������������

All results are been obtained in a 386-25 computer.

��������������������������������������������������Ŀ

� Double Density �  High Density  �  Extra High D. �

�������������������������������������������������������������������������Ŀ

� FORMAT        �      0:37      �      1:13      �                �      �

� FDFORMAT  (*) �      0:42      �      1:24      �                � 5.25 �

� FDFORMAT (**) �      1:28      �      1:52      �                �      �

� 2MF /F        �      1:24      �      1:30      �                � (5�) �

� 2MF /M        �      2:33      �      2:34      �                �      �

�������������������������������������������������������������������������Ĵ

� FORMAT        �      1:24      �      1:34      �      ?:??      �      �

� FDFORMAT  (*) �      1:38      �      1:42      �       --       � 3.5  �

� FDFORMAT (**) �      1:46      �      2:17      �       --       �      �

� 2MF /F        �      1:37      �      1:45      �      ?:??      � (3�) �

� 2MF /M        �      2:46      �      3:19      �      ?:??      �      �

���������������������������������������������������������������������������

(*) DOS standard formats (360-720-1.2-1.44) and best /X and /Y.

(**) Maximum capacity formats (820-1.48-1.72) and best /X and /Y.

5. - PROGRAMMERS NOTE.

����������������������

The /X and /Y switches of Christoph H. Hochst�tter FDFORMAT used in all

the above tests are listed here, and are the best options in most cases in

the AT systems (not in PC/XT):

���������������������Ŀ

�    /X        /Y     �

�����������������������������Ĵ

� 5�-DD �     1    �    3     �

� 5�-HD �     2    �    3     �

� 3�-DD �     1    �    2     �

� 3�-HD �     2    �    3     �

�������������������������������

Too  many  disk  copy  programs I have tested until now don't get the

maximun speed possible. When a 1.44M diskette is formatted with a correct

sector sliding (using /X:2 and /Y:3 FDFORMAT options for example) a whole

diskette must be read or write in 36 seconds, at sector access level.  If

the read/write buffer cross a 64K DMA frontier,  the access is divided in

some phases and many tracks needs more than one disk revolution.  Some of

the copy programs  also write the tracks in a temporary file on hard disk

just when read it from diskette, instead of wait until all memory becomes

full.  So,  these  programs are all slow because each track requires more

than one disk revolution to be accesed. Also while using the DOS standard

(incorrect)  sector sliding of /X:0 and /Y:0,  a whole disk read or write

time is never above 48 seconds in 1.44M media.

The sector sliding consist in a different numbering order of sectors in

the tracks. Example with /X:2 (in head changes) and /Y:3 (track changes):

Cyl 0  Side 0:   1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16 17 18

Cyl 0  Side 1:  17 18  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16

Cyl 1  Side 0:  14 15 16 17 18  1  2  3  4  5  6  7  8  9 10 11 12 13

Cyl 1  Side 1:  12 13 14 15 16 17 18  1  2  3  4  5  6  7  8  9 10 11

When  diskette  drive changes the side or cylinder active, some time is

spent ...  and  may be sufficient to can not read or write the next track

when drive head begins the operation (needed wait to next revolution).

The memory is also divided in 64K areas by the DMA controller.  No disk

operations are possible involving two of this areas.  For example, in the

direction 1FE3:0000 a sector can not be read or write, because the memory

directions involved are from 1FE3:0000 to 1FE3:01FF  (from absolute 1FE30

to absolute 2002F, in DMA pages 1 and 2). When DOS is loaded in computer,

the BIOS INT 13h is fixed,  maybe  using an internal buffer...  but  disk

performace get down, because disk operation is divided in some phases.

Remember: use the correct sector sliding in your disk formats and never

lets your program to cross any DMA frontier, if possible.  If  you  think

that is impossible to read or write 1.44M  FDFORMAT /X:2 /Y:3 diskette in

only 36 seconds ... or 1.80M 2M diskette in 37 seconds... execute 2M-FDTR

on it!.