------------------------------------------------------------------------

PCHECK  printer check, reset            Ver 1.1 (c) 1997 Horst Schaeffer

------------------------------------------------------------------------

PCHECK checks if the printer is ready. Optional reset.

Will only work for parallel printer ports.

Syntax: PCHECK [options]

Option /R  printer (hardware) reset

/N  (n=1..4) printer device number (default=1)

Errorlevel 0     printer is ready

else  not ready

With Option /R the printer may return "not ready" because it usually

takes a few seconds until the reset is done.

If you care:

A non-zero errorlevel represents the status bit pattern:

bit         decimal             status

0              1               time out

3              8               I/O error

4             16               NOT online

5             32               paper out

7            128               NOT ready

The errorlevel is the sum of the decimal values for bits set.

There is no general rule, which combination will be reported.

----

PCHECK is freeware by Horst Schaeffer - no warranties of any kind

Contact -  FIDO: 2:2480/13.75, E-Mail: horst@confusion.rmc.de

= 17 MAY 1995