A B M D E M O

-------------

INTRODUCTION

ABMDEMO is a program written to demonstrate the operation of the

Asynchronous Balanced Mode (ABM) version of the HDLC communications

protocol. ABM is the mode of HDLC that is used in many areas

of communications. It is the basis for LAP-B in X.25, IS8802 in

LANs, LAP-D in ISDN, etc.

ABMDEMO simulates the two ends of the HDLC link and produces a

display showing the transmission taking place, the identities of

the frames, and the key state variables at each end of the link.

It has been used at Monash University to aid in the teaching of

communications protocols.

The program runs under MSDOS 3.3 or later, and does not use a graphics card.

DISTRIBUTION

This distribution contains the executable, plus some sample control

files and some batch files to demonstrate their use.

OPERATION

To run ABMDEMO, the invocation is:

abmdemo [-ccontrolfile] [-llogfile]

where controlfile is a file of parameters for the simulation, and

logfile is the name of a file in which a copy of the screen

output is stored for later perusal. If either is missing, the

user is prompted for the file names, if any. If there is no

control file the run details are requested in a series of prompts.

The following are the run parameters:

For each of the 'A' and 'B' DTEs in turn:

(a) "f" if there are to be a fixed number of I frames in the

simulation, or "c" if it is to be a continuous random

generation of I frames

(b)	if the above was "f", the number of I frames to send. If it

was "c", the % of time the link is to be busy.

(c)	"f" if the frames are to be of a fixed length (in "clock ticks")

or "r" if the frames are to be of a random length.

(d)	the frame length in clock ticks (4 <= L <= 15). In the "r" case

it is the average length.

(e)	the probability of an I frame encountering an error

during transmission

Finally, specify the number of clock ticks in the run.

Here is a sample controlfile:

f 2 f 5 40

f 0 f 4 0

60

This file would result in a run of 60 clock ticks. DTE A would have

2 I frames, each of length 5 ticks, and there would be a 40% chance of

an I frame being received in error. DTE B has no frames.

The result of this simulation run would be:

A B M D E M O  HDLC SIMULATOR

D T E  A                        D T E  B

R  V  V    F   N N                C T         F   N N        V  V  R

T  S  R    R   S R                L I         R   S R        S  R  T

C          A                      O C         A                    C

M                      C K         M

E                      K           E

1  1  0    I   0 0 P/F=0   ooo     0                         0  0  0

1  1  0                    OOO     1                         0  0  0

1  1  0                    OOO     2                         0  0  0

1  1  0                    OOO     3                         0  0  0

1  1  0                    OOO     4                         0  0  0

2  2  0    I   1 0 P/F=0   ooo     5   ooo    RR    1 P/F=0  0  1  0

2  2  0                    XXX     6   OOO                   0  1  0

2  2  0                    XXX     7   OOO                   0  1  0

2  2  0                    XXX     8   OOO                   0  1  0

1  2  0                    XXX     9                         0  1  0

1  2  0                           10                         0  1  0

[At this point frame 0 has been acknowledged, but frame 1 was corrupted

so DTE B ignores it. In this example, DTE A's "ack timer" runs out and

triggers a recover sequence. Look for the RR P/F = 1.]

1  2  0                           11                         0  1  0

1  2  0                           12                         0  1  0

1  2  0                           13                         0  1  0

[some clock ticks deleted to save space]

1  2  0                           37                         0  1  0

1  2  0    RR    0 P/F=1   ooo    38                         0  1  0

1  2  0                    OOO    39                         0  1  0

1  2  0                    OOO    40                         0  1  0

1  2  0                    OOO    41                         0  1  0

1  2  0                           42   ooo    RR    1 P/F=1  0  1  0

1  2  0                           43   OOO                   0  1  0

1  2  0                           44   OOO                   0  1  0

1  2  0                           45   OOO                   0  1  0

1  2  0    I   1 0 P/F=0   ooo    46                         0  1  0

1  2  0                    OOO    47                         0  1  0

1  2  0                    OOO    48                         0  1  0

1  2  0                    OOO    49                         0  1  0

1  2  0                    OOO    50                         0  1  0

1  2  0                           51   ooo    RR    2 P/F=0  0  2  0

1  2  0                           52   OOO                   0  2  0

1  2  0                           53   OOO                   0  2  0

1  2  0                           54   OOO                   0  2  0

0  2  0                           55                         0  2  0

AUTHOR

Jim Breen

Department of Robotics & Digital Technology

Monash University

Melbourne, Australia

jwb@capek.rdt.monash.edu.au