@include "common.mac"

@chapter "Grundlagen zur Sprache"

@snode "Assembling your first program"

This chapter describes some basics of machine language programming,

but beginners should may read a good book about that.

To get your first program started you should enter the directory

SRC and start 'Make.bat'. The next step is to include the directory

BIN to your search path. If you start 'Install.bat' latter won't be

done. It is a good idea to update the PATH definition in your

AUTOEXEC.BAT.

If TMA is executable from everywhere get into one of the example

directories and assemble the sources. For instance:

CD EXAMPLES\DATALAND

TMA DATALAND.ASM

Some example sources need to be assembled to EXE files. If this is

the case, an error message is omitted. Use the -e switch to force

EXE generation:

TMA -e RELIEF.ASM

Have fun!

@endnode

@sndode "Programming syntax"

We have three sorts of instructions when programming in assembly.

mnemonics	 This are the instructions your CPU will work on.

data definitions They define data areas, lists and structures, the

places where you can store your data.

directives	 Assembler commands. They tell TMA how to handle

the source and how to generate code.

Mnemonics, the names of real CPU instructions, usually need operands

which indicate what type of data to use for the specified operation.

MOV DX,10	; Load register DX with decimal 10

Values may be stored in registers or memory. To reserve memory

space, use the DB/DW/DD etc instructions:

DB ?		;Reserve a byte

DW ?		;Reserve a word

You can continue a line in the next one by using the backslash '\'.

As far as I know this is also possible in Borland's TASM when in

IDEAL mode.

db"Hello !"\

,10,13,0

NOTE: DON'T USE THE BACKSLASH *WITHIN* OPERANDS!

@endnode

@snode "Syntax of effective addresses"

Nearly every x86 instruction that needs operands, uses effective

adresses (I'll call the EAs).

With EAs you can:

- Load memory with register contents or vice versa.

- Load memory or a register with a constant value.

Their syntax is not too complex at all and they don't differ much

from instruction to instruction.

Usually you'd write

MOV AX,word ptr[0]

to load AX with the contents of memory address 0, but you may also

do that a smarter way. TMA recognizes that AX is a WORD register

and you may leave off explicit notations for transfer sizes:

MOV AX,[0]

If you have to indicate the transfer size, because you don't use

registers or the syntax is ambiguous, a single letter is enough to

tell TMA what to do. This is very compatible to A386:

MOV B[1000H],200  ;BYTE transfer

PUSH D[0]	  ;Pushes a DWORD on stack.

At this time there're three possible abbrevviations:

byte ptr  [...] -> B[...]

word ptr  [...] -> W[...]

dword ptr [...] -> D[...]

NOTE: When using PUSH or POP instructions, a WORD transfer is assumed.

x86 processors know EAs for 16bit and for 32bit addresses and their

syntaxes vary:

Effective Adresses (16 bit)

A 16bit-EAs may contain:

- 1 base register, BX or BP together with

- 1 index register, SI or DI

- 1 constant displacement, signed BYTE or signed WORD

- a base or index register on its own.

For instance:

MOV AX,[BX]

MOV AX,[BP]

MOV AX,[SI]

MOV AX,[DI]

MOV AX,[BX+SI]

MOV AX,[BX+DI]

MOV AX,[BP+SI]

MOV AX,[BP+DI]

MOV AX,[BX+SI+100]

MOV AX,[DI-100]

MOV AX,[100]

Effective Addresses (32 bit)

A 32bit-EA may contain:

- any 32bit register except SP as base or index

- a scaling factor for the index register (2,4 or 8)

- a constant displacement of signed WORD or signed DWORD

MOV [ECX+EBX],EAX

MOV AX,[EAX+EBX*4+100]

MOV AX,[EAX+100]

LEA EAX,[EAX+EAX*8]	;Multiplies EAX by 9

@endnode

@snode "Labels"

TMA knows 2 classes of labels for code and data. The label type

is recognized by its syntax.

When defining a code label, let a ':' follow.

When referring to a code label, it's address is returned.

When referring to a data label, the contents of it's address

are returned; you don't have to put them in braces.

If you need the contents of a code label address, put it braces.

MOV AX,[codelabel]

If you need the address of a data label, use the OFFSET directive or

its abbrevviation OFS.

MOV AX,OFFSET datalabel

MOV AX,OFS datalabel

@endnode

@snode "Global und local code labels"

When working on large sources it's quite difficult to name labels

a reasonable way, especially if you just need to know that they are

there. For this reason, local labels, are implemented. They may be

defined more than once with the same name.

A86/A386 compatible local code labels

To be compatible to A86/A386 a local label must start with a single

letter followed by a number. You must explicitly indicate forward

jumps when reffering to them.

l1:	JMP >a2

a1:	JMP l1

a2:	JMP a1

NOTE: Even crossed jumps are allowed. A86/A386 will omit error messages

on them:

l1:	JMP >l1

JMP l1

l1:

REMARK: This is about future extensions for compatibility to TASM and MASM

but I'm not quite sure how they're really used. Tell me.

/*

TASM-Kompatible Sprungmarken :

Innerhalb von Prozeduren, die mit Hilfe des PROC-Befehls definiert

wurden, knnen lokale Labels im Klartext angegeben werden. Allerdings

mssen sie mit einem Klammeraffen '@' beginnen, und sind aus

anderen Prozeduren nicht erreichbar.

Um dem Assembler die Benutzung von lokalen Labels zu signalisieren,

ist der LOCALS-Befehl zu benutzen, welcher durch NOLOCALS wieder

aufgehoben werden kann.

LOCALS

tasmlocals PROC

JMP @@local1

JMP @@local2

@@local1:

@@local2:

ENDP

MASM-kompatible Sprungmarken :

Auch MASM verlangt eine eigene Notation fr lokale Labels.

Man beginnt sie dafr mit einem Dollarzeichen '$'.

masmlocals PROC

JMP $local1

JMP $local2

$local1:

$local2:

ENDP

Einzeldefinition

Auch ein Label in blicher Schreibweise kann mit dem Befehl LOCAL

als lokal definiert werden.

callxms MACRO

local1	LOCAL

CALLF XmsDriver

OR AX,AX

JE local1

JMP XmsError

local1: ENDM

*/ noch nicht funktionsfhig

@endnode

@snode "Programs and procedures"

TMA is very useful for beginners. Red tape is not needed at all

while still having full control of the generated code.

At last there's no problem generating COM files.

Here's an example (EXAMPLES\SNIPPETS\HELLO.ASM):

MOV DX,msg

MOV AH,9

INT 21H

MOV AX,4C00h

INT 21H

msg:	DB"Hello world!",10,13,"$"

@endnode

@endchp