STOBIN.DOC

1989-9-24

STOBIN is a small program for converting files in Motorola

S-record format into memory image files.  The Motorola freeware

cross-assemblers for Motorola's microcontroller chips (MCU's) do

not produce executable files.  They produce ASCII files which

indicate what the executable file should contain.  The format

used in the ASCII files is called the Motorola S-record format.

If you want to program a Motorola microcontroller via the

"bootstrap mode" available on some of them, you'll need to

convert the S-record output from the assembler into binary, burn

the binary version of the program into an (E)PROM, and use the

(E)PROM and a suitable programming circuit to program your

microcontroller.  Motorola usually prints a programmer schematic

in the data sheet for any part which has "bootstrap mode".

This version of STOBIN is a "quick and dirty" program

which I wrote for my own use, so it lacks the usual niceties.  It

requires MS-DOS version 3.0 or higher, it does not read the

command line (it prompts you for all input), it won't let you

tell it what to name the output file, and it will never write

over an old file.  If there's already a file with the name the

program wants to use, the program simply quits with the error

message "Attempt to create output file failed."  The program

names the output file *.BIN, where * is the name (without the

three character extension) of the first input file.

One nicety that this STOBIN does have:  it allows you to

build a binary file from any number of S-record files.  However,

it does not check for address conflicts among the input files.

Each memory location in the output file will contain the value

called out in the most recent input file which had a value for

that location.

To use STOBIN, just enter STOBIN.  The program will ask

you for the name of an input file.  Enter the name of the first

S-record file you wish to use (with path and drive, if needed).

Next the program will ask you what size of memory chip you're

going to program.  Enter the size in BYTES (not kbytes) of your

(E)PROM (for instance, enter 2048 for a 2k EPROM).  Next, the

program will ask you whether it should fill unused memory

locations with ones or zeroes.  If your microcontroller data

sheet recommends programming unused locations with $FF or ones,

press 1 (don't press the ENTER key).  If you microcontroller data

sheet recommends programming unused locations with $00 or zeroes,

enter 0 (don't press the ENTER key).  If you don't know whether

to use ones or zeroes, pick your favorite.  You won't do any harm

by picking the "wrong" one (programming will simply go a tad

slower, and if you reprogram your chip many many times you'll

wear it out a little faster).  Finally, the program will ask you

for the name of the next input file you want to use.  If you

don't want to use any more files, just press the ENTER key.

Otherwise, enter the name of the next file you want to use, and

the prompt will reappear so you can use another file.  When

you're done using input files, just press the ENTER key at the

"next file" prompt.