"Sector Access" FAT Save or Restore:

VERY IMPORTANT NOTES:

I have provided some safety measures to protect areas (Root Directory and

Data) beyond the FAT, in case you make a mistake replacing the FAT with a

different FAT file/s (for another PC).  I will NOT accept any responsibility

for any other mistake that you make.  As one of my many tests, I "blanked"

(filled with 0s) a complete FAT on a HD.  Without the FAT ("blanked"), the PC

had no OS (or anything else) when I turned it on.  After I "Restored" the FAT

using a Startup floppy disk, SA (Sector Access), and the FAT (file) I saved to

a floppy/s, everything was normal again.

FAT PURPOSE:

A FAT (File Allocation Table) is a "map" of the locations of all your files.

If this became corrupted due to a virus or other cause, it would be a MAJOR

DISASTER.  NOTE: The "starting point" for each file is in the directory.

FAT SAVE (BackUp):

If you have more than one PC, each Startup floppy disk and the disk/s used

to save the FAT, SHOULD BE CLEARLY MARKED in order to identify the PC they

belong to.  Make sure you have a copy of SA on a floppy too.  NOTE: The FAT

backup file/s does not have to be on the same disk with SA.

Copy (Save/BackUp) the FAT (as a file/s) to a Startup floppy disk or

separate disk/s (if you have a large FAT).  My FAT BackUp for one of my PCs

with 1.1 Gig FAT16 Hard drive is about 134K.  If your HD is several "Gigs", it

is possible you will need more than one floppy disk.  SA will prompt you for

the next disk in such an event.  NOTE: A one FAT file will be named

"FAT-BU.000".  IF the FAT is too large to fit on one floppy, "FAT-BU.000" is

renamed to "FAT-BU.001" and you will be prompted to insert the next disk for

"FAT-BU.002", etc.  IMPORTANT NOTE: It won't do any good if the FAT file is

stored on the HD and a HD FAT problem occurs.  Also, it is a GOOD IDEA to

backup the HD's Boot, as well as the MBR.

CORRUPTED FAT:

If the FAT has been corrupted, it can prevent the OS (Operating System),

such as Windows or Dos, from being installed when you turn on your PC.  This

will depend on what part of the FAT is "damaged".  Also, access to files will

be lost or they may appear corrupted due to corrupted cluster "pointers"

pointing to the wrong clusters.

CORRUPT FAT CHECK TIP:

If you have some access to your HD, after you used a Startup disk, assuming

you have a copy of "SCANDISK.EXE" on the floppy (normally located in the Dos

directory or I believe in the "Command" sub-directory to Windows 95/98),

perform a SCANDISK on the C: drive (assuming that's your HD).  If numerous

errors are reported, don't "Fix" anything.  Get a spare floppy disk/s, and

launch SA (located on a floppy), copy the current (C:) HD FAT to the spare

floppy disk/s as a precaution.  Replace the FAT with the backup FAT and

perform a SCANDISK again.  If the results are far fewer errors, it is very

likely that the FAT was corrupted.  NOTE: After replacing the FAT with it's

backup, some errors reported by SCANDISK are due to files added or relocated

(not by a "Move") after the FAT was saved.

RESTORING THE FAT:

If your HD can not access the proper data, and it is not caused by a

hardware problem, it is possible that the FAT may be corrupted.

1. With your PC turned off, insert a Startup disk and turn on the PC.

When ready, insert the disk with SA if it is not on the Startup disk.  As

insurance, using SA (Sector Access) replace the MBR and Boot Sectors with the

BackUp, in case they are corrupted (assuming you saved them).

2. Using SA, proceed to Replace the FAT with the FAT backup.  If you make a

mistake, and insert the wrong disk, SA will let you know and you can switch

disks and start again.  NOTE: SA has no way of knowing "FAT-BU.000",

"FAT-BU.001", etc. belongs to the PC you are restoring or another PC.  It is

up to you to store them on a disk/s with proper identification.

LOST FILES AFTER RESTORING A FAT:

Files added to your HD after a FAT save, will be lost after a FAT is

"Restored".  This is a small consequence compared to a major loss.  Periodic

FAT Saves is recommended if you add or relocate (not by a "Move") many files

on your HD often.

IMPORTANT NOTE:

After a FAT "Restore", ALWAYS perform a "SCANDISK" and "Fix" loose ends.

IMPORTANT "DEFRAG" TIP:

If you "DEFRAG" your HD, when finished, save the FAT (replace the older FAT

file backup).  "DEFRAG" rearranges files and the FAT is changed to "point" to

the new locations.