Microsoft Windows Display Driver Compatibility Program

Procedural & Test Suite

Information

Copyright © 1992 Microsoft Corporation

Version 1.09

March 5, 1992

Contents

µDriver Compatibility Testing Program	3

Program Objectives	3

Windows 3.1 Driver Library (WDL)	3

WDL Distribution	3

Getting a Driver into the WDL	4

WDL Listing & Driver Update Testing Procedures	4

Custom Driver:	4

Chipset Vendor Driver:	4

Who to Contact for More Information	5

Display Drivers - Criteria for Submission	5

What You Must Submit	5

When to Submit Drivers to Microsoft	6

Fees for Driver Testing	7

Overview	8

Test Suite Objective	8

Tests Included	8

ISV Applications Included	8

Display Sets	8

Using The Tests	9

Description of the Tests	10

DTA (Display Test Application)	10

ResView (Resource Viewer)	11

DIBView (DIB Viewer)	12

DispTest (Display Test)	13

DISPTEST.EXE Command Line Options:	13

WinWord (Microsoft Word For Windows 1.1a)	15

PowerPoint (Microsoft PowerPoint 2.00e)	16

PageMaker (Aldus PageMaker 4.0)	17

Executing the DCT Shell	18

Starting and the Windows DCT Shell	18

Appendix A	19

List of Required and Optional Tests	19

Appendix B	21

Optional Tests Section	21

Appendix C	23

Glossary of Terms	23

# Driver Compatibility Testing Program

The Display Driver Compatibility Test Suite that you have received in this DDK, is part of a new program that was launched by Microsoft in conjunction with the release of Windows 3.1.  In this DDK you will find the information and materials needed to participate in this program.  This document contains information about the program details and how you, as a hardware vendor, may participate in it.

## Program Objectives

There are several objectives for the Microsoft Driver Compatibility Testing & Distribution Program (DTP).

1.	Improve the quality of Windows Device Drivers by providing test suites to hardware vendors for use in testing their drivers.

2.	Increase the quantity of device drivers available to Windows customers.

3.	Make device drivers more readily available to Windows Customers.

4.	Give hardware vendors more methods for distributing their new and updated device drivers to Windows Customers.

## Windows 3.1 Driver Library (WDL)

The Windows 3.1 Driver Library is the replacement of the Windows 3.0 Supplemental Driver Library.  The important difference between the 3.0 SDL and the 3.1 WDL is that the 3.1 WDL will be a dynamic library of drivers that will continually grow and change.  Drivers that successfully pass through one of the driver programs will be distributed in the WDL.  The WDL will contain the following driver types:

*	Printers

*	Displays

*	Networks

*	Audio

This program is likely to grow to include other driver types in the future.

## WDL Distribution

Drivers will be distributed mainly via electronic methods:  Compuserve, GENie, Microsoft PSS Online Support, Microsoft PSS BBS, User Group BBS's, etc.  We believe that electronic distribution is the most effective way to make sure that Windows customers always receive the most current drivers.  We realize that not every customer will have a modem, so the WDL will be available on diskette through Microsoft fulfillment.

## Getting a Driver into the WDL

In order to have a driver distributed via the Windows 3.1 Driver Library it must pass the requirements set in the Driver Testing Program.  There are no deadlines for participation because the Driver Testing Program will continue indefinitely into the future.  Drivers will be released for distribution in the WDL immediately after they have passed compatibility tests on site at Microsoft.  (Contingent upon the signing of a Device Driver Code Licensing agreement).

The compatibility tests used by Microsoft for testing drivers are included in the DDK.  These tests are included so that you can thoroughly test and debug your drivers PRIOR TO submitting them to Microsoft for testing.

## WDL Listing & Driver Update Testing Procedures

Once your driver/adapter card has successfully passed through the Compatibility Testing Program it will be distributed via the WDL.   How your adapter card is listed is dependent upon the type of driver you are using with the adapter card.

### Custom Driver:

If you are distributing your own customized driver with your display adapter card, your driver file and card name will be listed on the WDL directory list.  The driver file that you provide must have a unique name in this case.

If you should update the driver and want to distribute the updated driver via the WDL you will need to submit the updated driver for testing.  In most cases you will not need to supply additional hardware, a Driver Testing Program Application form, or a Device Driver Code Licensing agreement for driver update testing.  (This is subject to change on a case by case basis.)  Everything else on the list of items needed for a driver test is always required.  (See list below.)

### Chipset Vendor Driver:

If you distribute the unmodified driver, VDD and Grabber files from your chipset vendor with your display adapter card, your device will be listed on the WDL directory list with the chipset vendor's driver file.

If this driver is upgraded, your card will automatically be retested  at Microsoft, using the upgraded file(s) provided to us by the chipset vendor.  We will assume that you will be distributing the updated driver with your card.  If your card passes testing with the updated driver you will remain a listed adapter card on the WDL.  If your card does not pass testing with the updated driver you will be notified immediately.  Your adapter card will be removed from the list of devices supported by this driver until the problem is corrected.

## Who to Contact for More Information

If after reading this document you have other questions about the display driver programs you can contact the following group at Microsoft:

Microsoft Hardware Vendor Relations (HVR) Group

Email:	internet:ihv@microsoft.com

compuserve:>internet!ihv@microsoft.com

FAX:	(206) 93MSFAX

The HVR Group can answer questions on the driver testing & distribution program procedures.  They can also check the status of your driver in the Microsoft testing process once you have submitted it.

## Display Drivers - Criteria for Submission

In order to distribute a display driver on the Windows 3.1 Driver Library it must meet the following criteria:

1.	Must be version stamped according to the instructions provided in the Windows 3.1 SDK (Software Developer's Kit).

2.	The driver setup disk should include ALL files needed for setting up your driver. There should be no references made to other disks (i.e., Windows 3.0, Windows 3.1, or other WDL disks) during the installation process for your driver.

3.	Must provide support for a mouse in a Windowed MS-DOS application. (Information available in the DDK manual.)

4.	Must provide font selection in an MS-DOS VM. (Information available in the DDK manual.)

5.	Must provide TrueType support. (Information available in the DDK manual.)

6.	Pass the display driver compatibility test suite that is included int he DDK. This test suite must be run on each board/mode combination that is supported by your driver.

## What You Must Submit

You must submit the following items to Microsoft in order to have your driver distributed in the WDL. Testing will not be started on your driver(s)/adapter card(s) until ALL of these items are received. Omission of any item will cause a delay in the testing process!

1.	A completed and signed Display Driver Testing Program Application form. This form will include basic information about your submissions to the testing program. The Display Driver Testing Program Application form (PROGAPP.DOC) is a Word for Windows document. This document is found on your DCT diskettes and is installed with the rest of the display test documentation. *Note:  Please submit this form in hardcopy to Microsoft.

2.	Driver setup diskette. This diskette should be precisely what you distribute with your video adapter card. *Note:  You must submit this diskette even if you are distributing an unmodified chipset vendor's driver with your card.  (We check the installation process that you supply the customer.)

3.	Two of each video adapter card supported by this driver.  We must have ALL cards that you sell which are supported by this driver.  These cards are not returned to you after the testing process is completed.  One goes into our product support lab, the other into our testing lab for support and troubleshooting purposes.  If a card ships with a variety of memory configurations (i.e., 1 MB, 2 MB or 4MB), you should test each of your drivers with the minimum required memory configuration for the resolution being tested. When you submit your adapter boards to Microsoft, submit fully populated boards (maximum memory configuration) to allow the same adapter to be used to test all of your drivers' resolutions. The adapter boards you submit must be shipping products.  NO BETA adapter boards will be accepted.

4.	Any special equipment that is required to use your adapter card(s).  For example if a special monitor is required you will need to send two of them to Microsoft.

5.	Test result log files.  The DDK display driver test suite will create a set of log files (one per display set.  A display set is a card/driver/mode combination.  More specific information is provided in the display test documentation.)

You will be submitting these log files to Microsoft on a diskette.  The process for putting the log file onto diskette is automated for you.  This process will place each successive log file in a separate directory on the diskette.  The directories will be named automatically.  The process tells you, as it is copying the log file to diskette, which directory the log file is being placed in.  It is REQUIRED that for each display set's log file, you document both the diskette label name and directory name where the corresponding log file has been placed.  There is space provided for you to record this information on the Display Driver Compatibility Testing Program Application Form.

Note:  It is possible that you may submit enough log files to fill multiple diskettes.  This is perfectly acceptable.

More information on log files and how to prepare your diskettes is presented in the documented:  CTSHELL.DOC.

6.	BETA Test Verification forms.  Five of your customers must fill out a BETA Test Verification form to demonstrate that you have adequately beta tested your driver.  You must have 5 BETA Test Verification Forms per driver.  For example, if you are submitting 2 drivers, you would need to submit 10 BETA Test Verification forms  (5 for each driver.)  Each beta tester must test your driver/card combination for a MINIMUM of 2 weeks to be considered a valid beta tester by Microsoft.  The Beta Test Verification Form (BETADISP.DOC) is a Word for Windows document.  This document is found on your DCT diskettes and is installed with the rest of the display test documentation. *Note:  Please submit these forms in hardcopy to Microsoft.

## When to Submit Drivers to Microsoft

You may submit your display drivers for testing at any time.  Once drivers/display adapters are received by Microsoft, they will be tested on a first come first serve basis.

## Where to Send Your Materials

When you have gathered the items listed above you will send the entire package to the following address:

Windows 3.1 Display Driver Testing Team

One Microsoft Way

Building 3/2

Redmond, WA  98052-6399

## Fees for Driver Testing

There are currently no fees for display driver compatibility testing. This is subject to change in the future.

*Note:	All aspects of the Driver Testing Program are subject to change at the discretion of Microsoft without prior notice. Please contact the Microsoft Hardware Vendor Relations (HVR) Group (as described above) for the most current information.

# Overview

## Test Suite Objective

The Microsoft Windows Display Driver Compatibility Test  Suite (DCT) provides a set of tests for verifying the robustness and correct operation of display drivers and their supporting files (Virtual Display Device, Standard Mode Grabber, and Enhanced Mode Grabber).  These tests can be used as stand-alone tests during the development and debugging stages of the development cycle, or they may be launched from the CT shell.  The CT shell is a Windows application that launches the tests, keeps track of test results, and provides an on-screen problem report form.  You will run the Display Driver Compatibility Tests from the CT shell when you are ready to submit your driver for compatibility testing.  The log file(s) generated will communicate test results to the Microsoft Windows testing group.

## Tests Included

The DCT includes several tests.  Some of these tests were designed as test applications, while others are data files for popular Windows applications.  While you are encouraged to use as many of the test files as possible, not all tests are required for participation in the Windows DCTP.  Appendix A lists which tests are required and which are optional, as well as what application each is intended for.

## ISV Applications Included

The DCT includes minimal versions of Aldus PageMaker 4.0, Microsoft PowerPoint 2.00e, and Microsoft Word For Windows 1.1a.  This software is included with the permission of the owners and is included in this package for testing purposes only. Any other use of this software violates the license agreement and international copyright law.

## Display Sets

Testing of display drivers will be in terms of "Display Sets".  A Display Set is defined as one Windows driver and its support files, being tested on one display adapter, at one resolution and color capability, in both Standard and Enhanced modes of Windows.  All display sets must be tested before any log files will be evaluated by Microsoft.  For example, Zippy Incorporated produces two display adapters, the Z-200 and the Z-250.  Both are based on their proprietary Z-20 chip.  The Z-200 is available only with 256K and supports two modes: 640x480x16 colors and 800x600x16 colors.  The Z-250 is a more advanced card and is available with either 256K or 512K.  With 256K, it supports the same modes as the Z-200.  With 512K, it also supports modes of 640x480x256 colors and 1024x768x16 colors.  This is the list of all 6 display sets to be tested with this driver:

Adapter

Memory

Resolution

Colors

Modes

Z-200

256K

640x480

16

Standard,Enhanced

800x600

16

Standard,Enhanced

Z-250

256K

640x480

16

Standard,Enhanced

800x600

16

Standard,Enhanced

512K

640x480

256

Standard,Enhanced

1024x768

16

Standard,Enhanced

Notice that even though the Z-200 and Z-250 have common modes, the fact that different display hardware is being used requires that both be tested as separate display sets.  The modes common to the Z-250 with 256K and the Z-250 with 512K, however, are not considered as different display sets, because the display hardware is identical, with the exception of on-board memory.  In this situation, the tests should be performed on the board with less memory.

## Using The Tests

To execute the tests, please perform the following steps:

1.	Install the DCT on each test machine to be used.  You should include both 286 and 386 platforms in your testing.  Both platforms will be used internally by Microsoft.

2.	The tests may be launched directly from the program manager.  Most of tests will not be automated.  Refer to the documentation for each test for specific instructions.

3.	The tests may also be launched from the shell.  Tests launched from the shell have some automation, and may be a good starting point.

4.	When you receive the finalized DCT (including additional tests), you will receive precise instructions on what test results should be submitted to Microsoft.

# Description of the Tests

This section of the document describes the individual tests which are included in the Windows DCT.  These instructions are supplied as a supplement to the on-screen instructions that the tests provide while running.

## DTA (Display Test Application)

Mode: Standard, Enhanced

Description:

This application (DTA.EXE) allows the test engineer to test GDI's primitive drawing functions.  At the driver level, these functions are all translated into calls to Output().  Parameters such as drawing objects, colors, background modes, and Raster Operations (ROPs) can be controlled through the use interface, allowing the test engineer to create a specific set of conditions for testing.  A more thorough description may be found in DTA.DOC, which was also installed by the setup program.

DCT  Instructions:

When the program is started from the shell, a script is used to automate the test scenarios.  There are actually four separate scripts that involve DTA, each running independently of the others. Each script has specific objectives:

DTA Pen Tests:	Verify correct pen realization and manipulation

DTA Brush Tests:	Verify correct brush realization and manipulation

DTA Primitives Tests:	Verify correct output of graphics primitives generated by the driver

DTA Miscellaneous Tests:	Verify correct operation of ROP3 codes, background modes, and bitmap conversion

Generally speaking, DTA will use the driver to draw an object on the screen, then use its understanding of GDI to simulate what the driver should have placed on the screen.  The test engineer is then asked whether the two objects match.  It is important to note that DTA uses a fixed template for its simulations, so some variation in line styles (specifically, scaling and where the line styles begin and end) may be seen at this point.  The other parameters that could cause confusion (documented in DTA.DOC), are controlled by the script, so the match should be very easy to determine.  It may be useful to run two instances of ZOOMIN.EXE (included in the SDK) to facilitate detailed screen comparisons.

If a failure is indicated, the test engineer will be prompted to complete a problem report form indicating the specific difficulty.  The parameters used for drawing will be automatically saved to the log file, so there is no need for the test engineer to include this data in the problem report form.

At the end of each test, DTA will indicate whether the test result will be recorded as Pass or Fail, along with the reasons for failure if failure is indicated.  DTA will report failure if a problem report was generated during the test.  Please note that reporting an error does not automatically mean that the driver under test will fail Microsoft's internal testing.  Each problem report will be evaluated separately when the driver is submitted.

## ResView (Resource Viewer)

Mode: Standard, Enhanced

Description:

All display drivers must provide the basic resources used on the display.  These bitmaps include icons, cursors, and bitmaps.  With the advent of drivers capable of operating at different resolutions (and containing multiple sets of resources), it is important to guarantee that the correct size resources are used in all cases, and that all drivers provide users with a consistent appearance.  ResView (RESVIEW.EXE) is designed to do exactly that.  Please refer to RESVIEW.DOC for more information on using ResView.

DCT Instructions:

When invoked from the shell, ResView runs automatically, loading each of the required resources and asking the test engineer if the resource is correctly displayed.  Errors are automatically reported if a resource is not found.  Verification can be done by running ResView on a system with a comparable Microsoft display driver (VGA or 8514), and comparing the displays, or by comparing the results to the bitmaps included in the documentation.

## DIBView (DIB Viewer)

Mode: Standard, Enhanced

Description:

All display drivers must be able to manipulate a variety of bitmap formats.  In addition to the native bitmap format of the display, the driver must also understand 4 OS/2 1.1 (PM) Device-Independent Bitmap (DIB) Formats (1, 4, 8, and 24 bits per pixel, or bpp), 4 Windows DIB Formats (1, 4, 8, and 24bpp), and 2 Run-Length Encoded (RLE) formats (RLE4, RLE8).  DibView (DIBVIEW.EXE) is designed to allow the test engineer to manipulate the different bitmap formats, and exercise SetDIBits, GetDIBits, StretchDIBits, SetDIBitsToDevice, BitBlt, and StretchBlt.  For more specific information about DIBView, refer to DIBVIEW.DOC, which was installed by the setup program.

DCT Instructions:

The shell includes 4 automated tests using DIBView.  Each of these tests has a specific objective:

DibView Bitmap Format Tests:	Exercises the driver's ability to convert DIBs between formats using GetDIBits.

DibView Bitmap Stretch Tests:	Exercises the driver's ability to stretch bitmaps using StretchBlt and StretchDIBits

DibView Bitmap Scroll Tests:	Exercises the driver's ability to move/scroll bitmaps using BitBlt, SetDIBits, and SetDIBitsToDevice

DibView Palette Tests:	Exercises the driver's palette management using AnimatePalette, CreatePalette, GetPaletteEntries, RealizePalette, and SelectPalette.

When DIBView prompts the test engineer if the image is correct, the test engineer should confirm that the image is recognizable and that there are no gaps or skewed sections.  During the stretch tests, the images will be distorted to fit within the window.  During the scroll tests, the images should be undistorted, but only a portion of the image will be generally visible.  24 bpp drivers should never experience loss of color resolution, 8 bpp drivers should only experience loss of color resolution on 24 bpp DIBs, and 16 color drivers should experience loss of color resolution on any DIB containing more than 16 colors.  Within these guidelines, all colors should be clear and appropriate

The palette tests load 4 8bpp bitmaps, then shift the focus between the DIBs.  On a palette device, the DIB receiving the focus should remap the palette to its color list.  This will temporarily alter the appearance of the other DIBs, which then adapt their logical palettes to the current physical palette.  The net result is some color deterioration in the DIBs that do not have the focus.  Please note that on non-palette devices (i.e., 4 bpp and 24 bpp), the DIBs should not change at all when the focus is transferred.

## DispTest (Display Test)

Mode: Standard, Enhanced

Description:

This DOS application (DISPTEST.EXE) will test the display drivers, the VDD (enhanced mode only) and clipboard grabber.  It is capable of switching the video display to different modes and configurations of vertical scan line values, text rows and text columns.  It will also display colors so that they can be tested for color mapping when putting the application in a window in Enhanced Mode Windows.   Also, it will test the Prnt-Scrn to Clipboard functionality.  In Standard mode, only text can be copied to the clipboard.  In Enhanced mode, both text and graphics are supported.

DCT Instructions:

When the program is executed from within the shell, simply follow the directions provided in the instruction box at the top left of the displayed screens.  For more detailed instructions press the F1 key and a help page will be brought up.

The actions you may be asked to perform are as follows:

1.	Press a key to continue: Verify the contents of the screen to make sure the text and graphic areas look as they should (i.e., no extraneous lines or malformed characters.)

2.	Put application in a window (Enhanced mode only): Press Alt-Enter to force the application into a window so that you can view it under windows and verify that the contents of the window look correct.  See the comments under "Press Prnt-Screen, Check Clipboard", below.

3.	Put application in full screen mode (Enhanced mode only): Press Alt-Enter to return the application to full screen view.

4.	Press Prnt-Screen, Check Clipboard:  When the clipboard checking is turned off, the test engineer must verify the results of the clipboard copying themselves.  Press the Prnt-Scrn key and then open Clipboard and verify that the contents look correct (Note: in most cases the proper display form will be OEM_TEXT format.)  The screen should be recognizably the same as what was previously displayed, although the grabber should map the colors to their closest counterpart in Windows.  (see Note about known bugs, above)

5.	Other windowing instructions, where errors are expected: perform as the instructions state, but  expect to have Windows report an error due to it's inability to display a certain mode in a window.

NOTE: If at some point during the test an error message comes up which tells the test engineer that the current display mode is not displayable in a window and it then proceeds to iconize the program, the test engineer may have to take the following steps to continue the test:

1.	Bring up the system menu for the program by selecting its icon,

2.	Choose Settings from the system menu,

3.	Select Full Screen mode, in the Settings Dialog box, select OK.

4.	Return to the desk top and double click on the program icon.

### DISPTEST.EXE Command Line Options:

If running DISPTEST.EXE from DOS the following command line options are available:

/C - Turns on the automatic clipboard comparison routines.

/Dn - Before the normal test begins this will cause the test to attempt to switch into all 			modes:

n = 1: 1-7, 13-19

n = 2: That were detected possible by /P (see below)

/H or /? - will bring up a list of available command line options.

/I<Screen Type> - This will ignore the display type which was automatically determined and allow the program to read from any section in the .ini file that is identical to the <Screen Type> parameter. (i.e., <Screen Type> = [VGA], [EGA],... brackets are required.)  Any section may be added to the .ini file such as [TESTGROUP1] and then /I[TESTGROUP1] could be used in the command line.

/L[Filename] - which will specify a log file other that the default, or if left blank it will log to the default DCT log file.

/N - Turns off the "inverting" effect around the instructions.  Provided for convenience on slow

displays.  This will cause the /C option to fail due to the difference caused in the resulting screen contents.

/P - Before the test begins this will do a check to see what video modes the BIOS detects are possible for the given display type.  Note: This is not accurate on most displays and performs mainly as a regression item.

/Svmmssii - This switch allows the program to ignore the .ini file, and video detection and display one single screen then exit.  The parameter is as follows:

v 	- video type (see .ini file header for info, this parameter limits the following parameters to only what options are available for that particular video type.)

mm 	- must be two digits, stands for the mode.

ss 	- stands for subscreen number (see .ini file header for info.)

ii 	- stands for instruction type number (mostly unimportant for single screen test, but see .ini file header for description of options.)

/T<Filename> - which will choose a specific .ini file to run from other than the default.

/V - Display the video type code that disptest.exe has detected your video display as being.

## WinWord (Microsoft Word For Windows 1.1a)

Mode: Standard, Enhanced

Description:

WinWord (WINWORD.EXE) is one of the retail applications used for testing.  It is extremely font-intensive, and includes a near-WYSIWYG display capability.  The main test consists of a WinWord macro that enumerates all the fonts supported by the document.  This macro requires that the "Apple LaserWriter Plus" printer driver be configured to print to FILE.  This is needed because the printer selected can drastically change the screen fonts available.

DCT Instructions:

When run from either the Program Manager or the DCT shell, a macro template called NORMAL.DOT is automatically loaded.  Begin execution of the macro by pressing Ctrl+D.  Each font will be sampled on the screen, with a MessageBox identifying the font after each one.  All fonts should be displayed without extraneous lines or characters.  On Windows 3.1, all True Type Fonts (Arial, Times New Roman, Courier New, and Symbol) should flow smoothly from smallest to largest.  Lines should all be evenly spaced, and should not overlap one another.  If an error is indicated, the test engineer will be prompted to complete a problem report.  Please be as specific as possible in this report, including the font name, attributes (Bold, Underline, etc.), and point size that is causing the problem.

## PowerPoint (Microsoft PowerPoint 2.00e)

Mode: Standard, Enhanced

Description:

PowerPoint (POWERPNT.EXE) is a graphics presentation package for the Windows environment.  It makes extensive use of most graphic functions, including metafiles and palettes (where applicable), and performs the operations to both memory and display.  It also exercises BitBlt, which is one of the most difficult pieces of code to develop in a display driver.

DCT Instructions:

When loaded from the Program Manager or DCT shell, the test file DCT_TEST.PPT will be loaded.  To begin the test, select File.Slide Show, and set slide advance to manual, then press OK.  The first slide will be displayed.  To advance a slide, click the left mouse button.  To go back, click the right mouse button.  (Allow a second or two for the slide to change).  When all slides have been displayed, the normal display will be restored.  You may then exit the program normally.  After exiting, the test engineer will be asked if any errors were encountered.  If errors are indicated, a problem report form will be presented.  Please provide as much relevant information as possible.

The test engineer should look for anomalies like a skewed screen, a hung machine, or palettes not being updated (Note: When switching slides, PowerPoint realizes the palette of the new slide before erasing the old one.  As a result, the old slide will be momentarily displayed with incorrect colors just before the shift occurs.  This is only evident on palette devices, and is correct behavior.)  Aberrations should be investigated.  If unsure if it is a driver bug, compare the behavior with a Microsoft driver of similar color capabilities.

## PageMaker (Aldus PageMaker 4.0)

Mode: Standard, Enhanced

Description:

PageMaker (PM4.EXE) is a desktop publishing package designed for the Windows environment.  It makes extensive use of fonts to perform justification, rotation, and other advanced formatting, as well as sophisticated mixture of text and graphics.  For the context of the DCT, its main interest is its font manipulation abilities.  This is most important when testing a display driver that has intrinsic font capabilities or supports font caching.

DCT Instructions:

When loaded from the Program Manager or CT Shell, the test file DCT_TEST.PM4 is loaded.  This test is largely manual, and requires the test engineer to manually reformat the text and verify that it is correctly displayed.

To select a different font or size, perform the following steps:

1)	In the main window, click on the text (A mouse is required for this test).

2)	Press Ctrl+E to activate an editing window.,

3)	Press Ctrl+A to select all text in the editing window.

4)	Select a font from the Type.Font menu and/or a size from the Type.Size menu.

5)	Close the editing window.  The document will be reformatted when it is repainted.

To rotate the text, perform the following steps:

1)	In the main window, click on the text.

2)	Select a rotation from the Element.Text rotation menu.

3)	Click on OK--the text will be refreshed when it is repainted.

A good selection of fonts should be tested, including TrueType fonts in a variety of sizes and styles, device fonts in a variety of sizes, and GDI fonts in several sizes.  Each type of font should be rotated in all four directions, and the orientation on the page should match the orientation shown in the text rotation dialog.

If the test was run from the script, the test engineer will be asked (when PageMaker is exited) if any errors were observed.  If the test engineer answers yes, a problem report will be presented.  Please provide as much information as possible, including the currently selected printer driver, as this will influence the on-screen representation of device fonts.

# Executing the DCT Shell

## Starting and the Windows DCT Shell

The CT Shell is intended for use primarily after all other tests have been run and the driver is thoroughly debugged.  It can also be used to automate the tests as they will be automated when the log files to be submitted to Microsoft are generated.  The shell provides services to facilitate test execution, log file generation and submission, and which tests have yet to be run.  To launch the DCT Shell, double-click on the "DCT Shell" that the setup program created in the "Windows DCT Tools" group in Program manager.  This will start the CT Shell.  Please refer to CTSHELL.DOC (installed with the DCT) for more detailed instructions on using the CT shell.

# Appendix A

## List of Required and Optional Tests

This list includes all tests included with the DCT, the application required for each, and whether or not the required application is included with the DCT.  Please note that Microsoft reserves the right to use any or all of these files for internal testing, and that any display driver failing one or more of these tests may be removed from the DCTP.  Optional test files should all be displayed clearly, not cause any faults, and not corrupt the screen.  The text should be clear and legible.  Note that some optional test files will generate warnings while being loaded.  You may safely ignore these warnings, as they are caused by the data files themselves.

Test File

Required/ Optional

Target Application

Target Application Included?

DISPTEST.INI

Required

DispTest

Yes

DTA.EXE

Required

DTA

Yes

RESVIEW.EXE

Required

ResView

Yes

BCHSIGN.DIB

Required

DibView

Yes

BCHSIGN2.DIB

Required

DibView

Yes

BUCHART3.RLE

Required

DibView

Yes

CHESS.BMP

Required

DibView

Yes

CLOUDS2.DIB

Required

DibView

Yes

COAST2.DIB

Required

DibView

Yes

KIMBA.RLE

Required

DibView

Yes

KRIS1.RLE

Required

DibView

Yes

LTHOUSE1.DIB

Required

DibView

Yes

OREGON2.RLE

Required

DibView

Yes

PATH.RLE

Required

DibView

Yes

POLICE.DIB

Required

DibView

Yes

SAMCAT.DIB

Required

DibView

Yes

SAMCAT3.DIB

Required

DibView

Yes

SEG1BPP.DIB

Required

DibView

Yes

SEG4BPP.DIB

Required

DibView

Yes

SEG8BPP.DIB

Required

DibView

Yes

TEST.BMP

Required

DibView

Yes

WOOD1.DIB

Required

DibView

Yes

3PATTERN.XLC

Optional

Microsoft Excel 3.0

No

3PIE.XLC

Optional

Microsoft Excel 3.0

No

3_3DLINE.XLC

Optional

Microsoft Excel 3.0

No

EXCEL_02.XLS

Optional

Microsoft Excel 3.0

No

EXCEL_07.XLS

Optional

Microsoft Excel 3.0

No

EXCEL_11.XLS

Optional

Microsoft Excel 3.0

No

EXCEL_16.XLC

Optional

Microsoft Excel 3.0

No

COREL1.CDR

Optional

Corel Draw! 2.0

No

COREL2.CDR

Optional

Corel Draw! 2.0

No

COREL3.CDR

Optional

Corel Draw! 2.0

No

COREL4.CDR

Optional

Corel Draw! 2.0

No

GENOA_1.CDR

Optional

Corel Draw! 2.0

No

GENOA_2.CDR

Optional

Corel Draw! 2.0

No

TRAIN.CDR

Optional

Corel Draw! 2.0

No

CARDFILE.CRD

Optional

Microsoft Cardfile 3.1

Yes

NOTEPAD.TXT

Optional

Microsoft Notepad 3.1

Yes

PROJ1.MPP

Optional

Microsoft Project 1.0

No

CAPABIL.PM4

Optional

Aldus Pagemaker 4.0

Yes

COLOR3.PM4

Optional

Aldus Pagemaker 4.0

Yes

COLOR07.PM4

Optional

Aldus Pagemaker 4.0

Yes

DCT_TEST.PM4

Required

Aldus Pagemaker 4.0

Yes

PGMKR_07.PM4

Optional

Aldus Pagemaker 4.0

Yes

PGMKR_10.PM4

Optional

Aldus Pagemaker 4.0

Yes

PM4TEST1.PM4

Optional

Aldus Pagemaker 4.0

Yes

XLCHART.PM4

Optional

Aldus Pagemaker 4.0

Yes

DCT_TEST.PPT

Required

Microsoft PowerPoint 2.00e

Yes

DEMO3.PPT

Optional

Microsoft PowerPoint 2.00e

Yes

INSPIRE.PPT

Optional

Microsoft PowerPoint 2.00e

Yes

PRSPP16.PPT

Optional

Microsoft PowerPoint 2.00e

Yes

SHADING.PPT

Optional

Microsoft PowerPoint 2.00e

Yes

TASTEPS.PPT

Optional

Microsoft PowerPoint 2.00e

Yes

ACTS.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

ACTSL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

ARIAL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

ARIALL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

ARTTEST.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

ARTTESTL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

AUTO2.DOT

Optional

Microsoft Word for Windows 1.1a

Yes

AUTOT2.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

BIG.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

BIGL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

BOXES.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

BOXESL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

COURIER.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

COURIERL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

DCT_FONT.DOT

Required

Microsoft Word for Windows 1.1a

Yes

DEMO.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

INTLTEST.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

SPINAKER.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

SYMBOL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

SYMBOLL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

TABLETT.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

TABLETTL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

TESTMACS.DOT

Optional

Microsoft Word for Windows 1.1a

Yes

TIMESNR.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

TIMESNRL.DOC

Optional

Microsoft Word for Windows 1.1a

Yes

# Appendix B

## Optional Tests Section

This section contains additional tests that are not required for submission of drivers to Microsoft.  However, Microsoft reserves the right to use any portion of these tests for internal testing, and any driver failing these tests may be returned to you for further development.

1)	In Enhanced mode, run two instances of DispTest.  Alternate between the two Virtual Machines (VMs) using Alt-Tab.  Try switching between them when both are in different modes.  Everything should work correctly.

2)	In Standard mode, run a graphical DOS Application (DispTest will work fine) and press PrntScrn while the display is in graphics mode.  Then check the clipboard--nothing should have changed.

3)	In Enhanced mode, run a DOS-based CAD application and tell it to draw a complex drawing.  While it is drawing, use Alt-Enter to toggle between full screen and windowed modes.

4)	In both Standard and Enhanced modes, make sure that Alt-Tab, Alt-Esc, and Ctrl-Esc can be used to shift from a DOS box back to Windows and from Windows back to the DOS box.

5)	In Enhanced mode, Print from DIBView while alternating between Windows and a DOS VM.  Make sure that the printed output is complete and correct.

6)	Create some files whose bits are exact multiples of 65536, then load these files as background bitmaps via control panel  Some sample dimenstions of files that meet this criterion are:

Bits Per Pixel

Height (Pixels)

Width (Pixels)

1

512

1024

4

256

512

8

256

256*

24

128

170**

*	256color.bmp, included with Windows, has these dimensions.

**	Since each scanline must be DWORD aligned, each line will contain 512 bytes.

7)	Use Stress.exe (included with the SDK) to restrict the amount of resources available for Windows.  In particular, limit the global memory to less than 200 KB while operating testing.

8)	Run PRNTTEST.EXE (included with the PCT), select a printer that does not enumerate its own brushes, but relies on the Brute of the display driver.  Verify that all brushes and pens are correctly enumerated.  (The Windows 3.0 8514 code failed to enumerate hatched brushes in the Brute).

9)	Run Microsoft Money. Create an account and enter an initial balance of $500. Create payments of $100, $150, and $125. Mark the checks for $150 and $120 as cleared (use Ctrl+M to toggle clear state). Balance the account, entering $225 as the ending balance and the current date as the statement date. Press Continue, then Finish. You should see a message that you have successfully balanced your account. Some existing 3.0 drivers will incorrectly cause the screen to go black instead of displaying the message box.

# Appendix C

## Glossary of Terms

This glossary lists some terms which are used within the Windows DCT as well as in the DOS environment in general.  In providing these terms the goal is to enable the tester to better understand the tests being performed and perhaps generate further testing ideas.

DCT - Microsoft Windows 3.1 Display Driver Compatibility Tests

DCTP - Microsoft Windows 3.1 Driver Compatibility Test Program

Device Virtualization - The process of intercepting I/O operations where the operating system creates the illusion of a separate device for each virtual machine while only one physical device is present in the system.

Display Set - One display adapter and its Windows driver being tested at a fixed resolution and color capability in both Standard and Enhanced modes.

DOS Box - An instance of COMMAND.COM launched from within Windows.  When running Enhanced (386) mode, each instance has its own Virtual Machine (VM).

Enhanced Mode Grabber (.3GR) - This is the support file used by the system to paint the display of a DOS application that is being run in a window.  The grabber translates display output calls into Windows calls that are then displayed in a window.  Used only in enhanced (386) mode.

Instance - In this document this term refers to a single occurrence of a process or program.  For example, the second instance of Write.exe is the second copy of Write run in the system.  This is not to be confused with the similar term Instance Data.

Standard mode Grabber (.2GR) - This is the support file used by Windows to communicate with DOS applications when Windows is running in Standard (286) mode.  It is responsible for saving and restoring video displays when the system focus is being transferred between Windows and Non-Windows applications.

Vendor - This general term is used throughout the document to refer to IHVs, ISVs,  and OEMs.

Virtual Display Device (VDD) - A device driver used by Windows in Enhanced (386) mode to virtualize all I/O with the display hardware.  The VDD is device-specific.

Virtual Machine - An environment available on 80386 based machines which allows program to run as though they are the only programs in the system.  Each Enhanced mode DOS Box has its own address space (1MB), I/O port space, and interrupt vector table.  Multiple virtual machines can be running simultaneously, each under the illusion that it is in complete control of the computer.