Microsoft Windows 3.1

Network Compatibility Test

Copyright 1990,1992 (c) Microsoft Corporation

Microsoft Confidential

Version 3.1

Revision 1.0

March 1992

Contents

Overview

µ4

Objective	4

Windows 3.1 Driver Library (WDL)	4

WDL Distribution	4

Getting a Driver into the WDL	4

WDL Information	5

System Requirements	6

Requirements	6

Setting Up Windows 3.1	7

Setting Up	7

What's New	7

Administrative Setup	7

Quick Install Option	7

Workstation Setup Recommendations	8

Setting Up the NCT	9

Setting Up the Files	9

Setting Up the NCT Variables	10

Executing the NCT	14

Starting the Windows NCT Application	14

Features and Manipulating Tests	15

Viewing All Tests	15

Adding a test to the Tests to be Run list box	16

Deleting a test from the Tests to be Run list box	16

Pausing and Aborting Tests	16

The Statistics Box	17

Options	17

Running the Tests	17

Running Each Test	17

Manual Input	17

Problems Running Tests	19

Test Crashing Upon Execution	19

Missing and Not Found Executables and Libraries	19

When In Doubt	20

The Problem Report Form	20

The Wrap-Up: Completing the NCT	21

Automated Tests	23

Applet Network Test	23

DMA Test	23

DOS Multi-VM Test	24

Enh Net Lg File Test	24

Enh Net Sm File Test	25

FM Test - Drive A	25

FM Test - Drive C	26

FM Test - Network	26

Int 13 Test	26

Int 13 VM Test	27

NetBIOS VM Test	27

Network Stress Test	27

Std Net Lg File Test	28

Std Net Sm File Test	28

Task Swapper	28

WinNet API Test	29

WinNet Prt API Test	29

Win NetBIOS Test	30

XMS TEST	30

Recommended Manual Tests	31

Application Compatibility Test	31

Int 13 DOS Test	32

Mail/Message Test	32

Mouse Driver Test	33

Network Error Conditions	34

Network Multiple DOS VM Test	35

Setup Test	378

Additional Manual Tests	41

EMM386 Test	41

RAMDrive Test	41

Swapfile Test	42

XMS DOS Test	43

Advanced Tests	44

Virtual COMM Device	44

Virtual Display Device	45

Command Line Parameters	46

Appendix A	48

Glossary of Terms	48

Overview

OBJECTIVE

The Network Driver Compatibility Test that you have received in this DDK, is part of a new program that was launched by Microsoft in conjunction with the release of Windows 3.1.  In this DDK you will find the information and materials needed to participate in this program.

There are several objectives for the Microsoft Driver Compatibility Testing & Distribution Program (DTP)

1.	Improve the quality of Windows Device Drivers by providing test suites to vendors for use in testing their drivers.

2.	Increase the quantity of device drivers available to Windows customers.

3.	Make device drivers more readily available to Windows customers.

4.	Give vendors more methods for distibuting their new and updated device drivers to Windows customers.

WINDOWS 3.1 DRIVER LIBRARY (WDL)

The Windows 3.1 Driver Library is the replacement of the Windows 3.0 Supplemental Driver Library.  The important difference between the 3.0 SDL and the 3.1 WDL is that the 3.1 WDL will be a dynamic library of drivers that will continually grow and change.  Drivers that successfully pass through one of the driver programs will be distributed in the WDL.  The WDL will contain the following driver types:

*	Networks

*	Printers

*	Displays

*	Audio

The program is likely to grow to include other driver types in the future.

WDL DISTRIBUTION

Drivers will be distributed mainly via electronic methods:  Compuserve, GENie, Microsoft PSS Online Support, Microsoft PSS BBS, and User Group BBS's.  We believe that electronic distribution is the most effective way to make sure that Windows customers always receive the most current drivers.  We realize that not every customer has a modem, so the WDL will be available on diskette through Microsoft fullfillment.

GETTING A DRIVER INTO THE WDL

In order to have a driver distibuted via the Windows 3.1 Driver Library it must pass the requirements set in the Driver Testing Program.  There are no deadlines for participation because the Driver Testing Program will continue indefinitely into the future.  Drivers will be released for distribution in the WDL immediately after they have passed compatibility tests on site at Microsoft.  (Contingent upon the signing of a Device Driver Code Licensing agreement).

The compatibility tests used by Microsoft for testing drivers are included in the DDK.  These tests are included so that you can thoroughly test and debug your drivers prior to submitting them to Microsoft for testing.

WDL INFORMATION

For more information on how to get your driver distributed via the WDL please contact Microsoft Product Support Services.

System Requirements

REQUIREMENTS

To install and complete the Windows 3.1 NCT on your hardware you need to have:

1.	A computer system (PC) that meets the Microsoft Windows hardware requirements.

2.	General understanding of the Windows user interface.

3.	Installed Windows 3.1 onto your network according to the instructions in the Setting Up Windows 3.1 section of this document.

4.	Approximately 2.8 Mb of free space on the hard disk drive.

5.	The NCT tests takes approximately eight hours to complete.  The Network Stress Test is designed to run approximately 8 hours but can be run overnight.

6.	Two formatted high density floppy diskettes.  One diskette is sent back to Microsoft with the completed test information and the other diskette is used during the NCT Test.

7.	Your network should be configured with at least one network printer.

8.	To run the Windows NCT under Windows 3.1 you need 2MB of RAM for both Standard and Enhanced Mode.  This means that you may need to disable some device drivers and TSR programs on some systems.

9.	The DOS directory in the path.

10.	All network popup services should be disabled.

Setting Up Windows 3.1

SETTING UP

After reading this section, install Windows 3.1 using the Setup Test found in the  Recommended Manual Tests section of this document.

WHAT'S NEW

The installation of Windows in a network environment is comprised of two parts:  installing Windows onto a network server, and installing Windows for each workstation.  New to Windows 3.1 are two setup features:  the Administrative Setup Option and the Quick Install Option.

ADMINISTRATIVE SETUP

The Administrative Setup Option (setup /a) is used by system administrators to install Windows onto a network server.  This option copies, expands, renames, and marks all files as "read only" from the Windows distribution diskettes to the network server.  Once the Windows files are installed on the server, individual workstations can be configured for Windows by running the setup program from this shared network directory.

QUICK INSTALL OPTION

The Quick Install Option is available when setup is run for each individual workstation.  The Quick Install Option requires minimal user interaction.  This option automatically uses the System Information settings that Windows detects for your hardware environment and make the necessary changes to CONFIG.SYS and AUTOEXEC.BAT files.  Users are still allowed to install printer drivers with this option.

If you are installing Windows 3.1 onto a workstation which has Windows 3.0 installed, the Quick Install Option gives you the option of upgrading over the existing Windows 3.0 files, or installing Windows 3.1 into another directory.

If you are installing Windows 3.1 onto a workstation which does not have Windows 3.0 installed, the Quick Install Option does not prompt you for a destination directory, but automatically installs the Windows 3.1 files into the C:\WINDOWS directory.

WORKSTATION SETUP RECOMMENDATIONS

1.	Backup your hard drive.  This step is recommended prior to installing any software for testing.

2.	Disk space depends on the setup parameters chosen.

3.	Run Setup /n from the Windows 3.1 shared directory on your server.  Follow the instructions in Setup.  We encourage you to install and upgrade over your existing Windows 3.0 directory, but recommend that you backup your group files (*.GRP) and initialization files (*.INI) before doing so.

4.	Try various combinations of Setup parameters (/n or no parameters) and options (Quick Install or Full Install).  A matrix of all the combinations are provided in the Recommended Manual Tests section - Setup Test.

5.	After installing Windows 3.1, we recommend that you run Swapfile to set up a permanent paging file on your hard disk (if applicable).  The procedures for using the Swapfile utility are described in the chapter Optimizing Windows of the Microsoft Windows User's Guide version 3.1.

Setting Up the NCT

SETTING UP THE FILES

Perform the following steps to install the Windows NCT:

1.	Create a directory for the NCT on the hard drive.  For example, to create the directory NCT31 on your C: drive use the DOS make directory command ( <Enter> represents pressing the Enter or Return key):

MD  C:\NCT31 <Enter>

2.	Insert the first NCT disk in the floppy drive.

3.	If Windows is running, exit Windows to DOS.  From outside of Windows, run the Install batch file from the floppy drive and specify the NCT directory you created in step one and the directory on your hard drive in which Windows 3.1 is installed.  For example, if the NCT disk is in the A: drive and your Windows 3.1 directory is C:\WINDOWS you would perform the following:

A: <Enter>

INSTALL  C:\NCT31  C:\WINDOWS<Enter>

4.	The Install batch file automatically copies the files from the NCT diskette onto your hard drive.  Note that the files on the NCT diskette are compressed and therefore need to be expanded by the Install batch file before they can be run by DOS or Windows.  In the rare case that you need to expand a file off of a NCT diskette, simply change drives to the floppy containing the NCT diskette and issue the following command:

EXPAND <FILE.EXT> [D:][\DIR]

Where FILE.EXT is the name of the file you wish to expand, D is the drive you want to expand the file to, and DIR is the directory you want the expanded file in.

Make sure that you have enough free disk space before running Install.  If the Expand utility is unable to create files when expanding it can result in zero length files being created.

Note:	The directory would need to have ATTRIB -R run on all the files before they can be deleted and a reinstall attempted.

5.	It is easier and more convenient to invoke the NCT shell if the application is added as a Program Manager Program Item.  To do this, perform the following while in Windows:

a.	Activate the group in which you would like the NCT Shell program item to exist.  This can be done by clicking once in that group with the mouse.

b.	From the Program Manager File menu select New...

c.	The New Program Object dialog comes up with Program Item selected.  Simply press Enter to select OK and the Program Item setting.

d.	The Program Item Properties dialog comes up to be filled in.  For Description, the current field, type:

Windows 3.1 NCT

e.	Tab to the Command Line field and enter the name of the NCT directory that you created in Step 1 and append the name of the NCT shell executable, WINNCT.EXE.  Following the Install examples above, the Command Line field would look like this:

C:\NCT31\WINNCT.EXE

It is not necessary to fill in the remaining fields in this dialog for the NCT.  Simply press Enter to select OK and complete the installation.

SETTING UP THE NCT VARIABLES

Since each network is setup differently, you need to provide configuration definitions of your networking environment prior to running Windows 3.1 and the NCT.  Some examples of network configuration definitions are:  share or volume names, username, network printers and network directories.  This information is kept in the file WINPLAY.INI which is located in the Windows directory specified when you ran the INSTALL batch file.

Using your favorite text editor, open the file WINPLAY.INI.  The first section is marked [ScriptVars].  There are 27 variable names in this section which need to be defined.  The definition for each variable is described below.

Note1:	WinPlay.ini is a text file.  Be sure to save WinPlay.ini as a text file when using an editor or word processor.

Note2:	If you are using Digital Equipment's DECNet network you cannot specify shares on your server that require passwords.

Note3:	For drive letter specifications, do not place a colon after the drive letter.

Note4:	For directory path specifications, a preceding backslash "\" is required.

UserName

The username you use to logon to your network.

NCTDriveLtr

The drive letter where you installed the NCT files.

Note:	If you install the NCT files onto your network, you must make sure that the specified network drive is redirected prior to starting the NCT tests.

NCTPath

The path where you installed the NCT files.

NCTFlopLtr

The drive letter of a floppy drive on your machine.  This variable has already been assigned a default value of "A".  If you have a multi-floppy drive machine and want the NCT tests to write files to a floppy drive other than floppy drive A, then change this variable to represent the desired floppy drive letter.

Note:	If you are running the NCT on a diskless workstation, set this variable to the same value as the NCTDrive1 variable.

AutoDelay

This variable is assigned a delay time (in seconds) for certain NCT tests.  It has already been assigned a default value of 5.  It is recommended that you use this default value.

RemIOLoc

The drive letters of the local drives you wish to include in certain NCT tests.  Certain NCT tests read/write variable file sizes to the local drives.

Note:	A space is required between each individual drive letter.  We recommend that you assign the variable RemIOLoc to one floppy drive and to one local hard disk partition if available.

ShortPrnDelay and LongPrnDelay

These variables have been assigned a default delay time, in seconds, for certain NCT printer tests.  It is recommended that you use these default values.

NetDrive

NetDriveX	X = 1, 2, and 3

The drive letter which is not currently used for network redirection.  The three drives you specify are used by the NCT for network redirection.

NetPath

NetPathX	X = 1, 2, and 3

A network server name and share, or a network server name and volume name.  The three server/share names or server/volume names are used by the NCT to connect to the specified server share or server volume.  You must have Read/Write access on the shares/volume specified.

NetPwd

NetPwdX	X = 1, 2, and 3

The password needed to attach to a particular server share if necessary.  This variable is used primarily for Lan Manager based networks.  For NetWare Networks, or Lan Manager shares that do not require passwords, leave this variable blank.

NetDir

NetDirX	X = 1, 2, and 3

The directory name which the NCT creates on your server.  This variable has already been assigned a default value.  If you prefer to change the variable(s), you must specify a directory name which does not exist on the server share or volume specified by the other variables in a set.

Note:	The directory assigned must be a a single-level directory specification such as \test1.  This variable should be unique for each copy of the NCT running on the same network.  It is recommended that you change the variables to reflect your name, such as  JohnE.111, JohnE.222, and JohnE.333.

NetPrt

NetPrtX	X = 1 (for LPT1), 2 (for LPT2), and 3 (for LPT3)

The printer share or print queue for each of the LPTX ports.  The NCT uses these variables to redirect the LPTX ports to the network printer share or print queue specified.  All 3 variables must be assigned values which represents the network printers on your network.  If you have only one network printer attached to your server, then assign each of the variables to the same network printer share or print queue.

Note:	The NCT only prints to the LPT ports specified in Control Panel/Printers dialog box.  Make sure that you have defined the correct printer drivers for each of the network printers you specify for NetPrtX.

NetPrtPwd

NetPrtPwdX	X = 1 (for LPT1), 2 (for LPT2), and 3 (for LPT3)

The password needed to attach to a particular server printer share.  This variable is used primarily for Lan Manager based networks.  For NetWare Networks, or Lan Manager printer shares that do not require passwords, leave this variable blank.

Examples

Lan Manager or MSNet Based Networks

Two Lan Manager or MSNet based networks, named Server1 and Server2.

Server1 is configured as follows:

a.	One share, named Share1, which does not require a password.

b.	One network printer share, named printer1, which requires a password (p1).

Server 2 is configured as follows:

a.	Two shares, named Share2 and Share3 - both requiring passwords (hot & cold).

b.	Two network printer shares, named printer2 and printer3.  Both printer shares do not require passwords.

The workstation configuration does not use drives p, q, and r for any network redirection.  Assign the following variables as:

NetDrive1=p

NetPath1=\\Server1\Share1

NetPwd1=

NetDir1=\NCT31Tst.301

NetPrt1=\\Server1\Printer1

NetPrtPwd1=P1

NetDrive2=q

NetPath2=\\Server2\Share2

NetPwd2=hot

NetDir2=\NCT31Tst.302

NetPrt2=\\Server2\Printer2

NetPrtPwd2=

NetDrive3=r

NetPath3=\\Server2\Share3

NetPwd3=cold

NetDir3=\NCT31Tst.303

NetPrt3=\\Server2\Printer3

NetPrtPwd3=

NetWare Network

A NetWare server, named Server1, configured as follows:

a.	Three network volumes:  SYS, OPS, and MANF

b.	The workstation does not have read/write access to the root of volume SYS.  Volume SYS has one directory named "TEST" in which the workstation does have read/write access.  Configure the variables so that the NCT test will use this directory.

c.	Three network printers with the following print queue assignments: Printer1, Printer2 and Printer3

The workstation configuration does not currently map drives p, q, and r for any network redirection.  Assign the following variables as:

NetDrive1=p

NetPath1=Server1\SYS:TEST

NetPwd1=

NetDir1=\NCT31Tst.301

NetPrt1=Server1/Printer1

NetPrtPwd1=

NetDrive2=q

NetPath2=Server1\OPS:

NetPwd2=

NetDir2=\NCT31Tst.302

NetPrt2=Server1/Printer2

NetPrtPwd2=

NetDrive3=r

NetPath3=Server1\MANF:

NetPwd3=

NetDir3=\NCT31Tst.303

NetPrt3=Server1/Printer3

NetPrtPwd3=

Note:	In example above, drive p will be mapped to SERVER1\SYS:TEST and the subdirectory NCT31TST.301 would be created in the TEST directory on volume SYS.

For NetWare, it is valid to specify <SERVER>/<VOL>:<DIR-PATH> for the variable NetPathX= where <DIR-PATH> is a valid directory in which you have read/write access.  Thus, this would be a valid variable definition:

NetPath1=SERVER1\SYS:TEST1\TEST2\TEST3\TEST4

Executing the NCT

STARTING THE WINDOWS NCT APPLICATION

To start the Windows 3.1 NCT application, first run Windows in the desired mode.  It is recommended that the order of executing the different Windows modes during testing be as follows:

Standard Mode	Win /s

Enhanced Mode	Win /e

Please see the System Requirements section above for information on memory requirements for the Windows NCT .

Note:	Please keep one formatted floppy disk in the floppy disk drive while running the NCT.  You must use the drive specified for the NCTFlopLtr variable.

Invoke the Windows 3.1 NCT by double clicking on its icon in the Program Manager or by selecting it and pressing Enter.  The Welcome to Windows 3.1 NCT dialog is displayed.

Fill in the information requested for each field.  The Contact is the individual within your organization that Microsoft contacts for information about the Network Compatibility Program and results of the NCT.  The NCT only prompts you once for this information -- the first time you run the NCT Shell on each system.  Selecting Close closes this dialog and brings up the NCT Shell application window.

The status bar at the bottom of the application window provides instructions and information about the active button or selected item.  You can use the mouse to manipulate the NCT buttons, use the Alt key in conjunction with the accelerator keys, or use the tab key to move between them and then press Enter or the Space bar to select what is highlighted.

FEATURES AND MANIPULATING TESTS

Viewing All Tests

Select the Choose button, and the application window expands to show the List of All Tests list box to the right.

.

Select Choose again to return the window to its previous size.  Both the Tests to be Run list box and the List of All Tests list box display an icon for each test and the test name.  In addition, the test name is colored according to its status, using the following scheme:

green	=	Passed

yellow	=	Running now

red	=	Failed

white	=	Not yet run

pink	=	Disabled or Not Applicable (N/A)

Adding a Test to the Tests to be Run List Box

Tests can be added to the Tests to be Run list box by either dragging tests from the List of All Tests, or by pressing the Insert key while a test from List of All Tests is selected.  This feature should be utilized when it is necessary to regress tests in other modes or rerun certain tests after making changes to the system configuration.

Deleting a Test from the Tests to Run List Box

Tests can be deleted from the list of running tests by either dragging the test on the Black Hole, dragging the Black Hole on the test, or pressing the Delete key while a test is selected.  This feature should be utilized when it is necessary to regress tests in other modes or rerun certain tests after making changes to the system configuration

Pausing and Aborting Tests

After the Go button is pushed, it changes into a Pause button.  Pushing the Pause button inserts a pause into the sequence of running tests which takes effect as soon as the currently executing test has finished.  Selecting the Abort button invokes a message box which warns that current results will be erased.  In addition, it prompts you to verify that the abort should be executed.  If Yes is selected, then all tests are cleared from the Tests to Run list box for the current mode and all current statistics are cleared from the result file and the Statistics group box. If you select No, tests are cleared from the Test to Run list box but statistics are not changed. If Cancel is selected, nothing is changed.

Using the keyboard, you can active the Pause command by pressing Shift+Alt+F7, and Abort by pressing Shift+Alt+F8.

The Statistics Box

The Statistics box gives an overview of the number of tests that have passed, failed, not been run, are currently running or are disabled for each mode of Windows.  Select the button for a particular mode to view the statistics.  The mode selected in the Statistics box must match the current Windows mode in order for test execution to proceed..

Options

Under the Options menu are options that allow changing the way the NCT tests are run, Stress Mode, and the way the NCT Shell Displays information.  Disabling Options/Display/View Icons option enables more tests to be viewed at once in a list box.  Enabling the Options/Display/Use Colors, you can optionally specify to use text instead of color status indicators.  Enabling Options/Stress Mode causes tests to be repeatedly performed on your system.  In this mode, any tests added to the Tests to be Run list will be executed without waiting for any other test to finish.  Upon termination, a test will remain in the Tests to be Run list and will be immediately reexecuted.  The tests will continuously recycle until regression mode is stopped or they are removed by selecting Abort.  To disable Stress Mode, deselect Stress Mode from the Options menu.

RUNNING THE TESTS

Running Each Test

Go is the active button and pressing Enter activates this command and begins the tests.  The Go command runs all tests in the Tests to be Run list box sequentially.  If there are no tests in this list, the Go command automatically adds all of the uncompleted tests into the Tests to be Run list box and executes them.  We suggest you start each test individually, as follows:

1.	Select the Choose button.

2.	Highlight the first test that has not been run in the List of All Tests.

3.	Press the Insert key or drag the test to the Tests to be Run list box.

4.	Press Enter or select the Go button.

5.	This will run the selected test through completion.

6.	When the test has ended, repeat steps 1-5 until all of the tests have been run.

As the tests are launched from the Windows NCT you are prompted with instructions to execute.  Many tests verify their results internally after prompting you to perform a set of commands or inputs.

Manual Input

Some tests use a generalized instruction system under which you are presented with an Instructions window and prompted for manual input and test verification in the Test Guide window.  Both of these windows are "top-most" windows which reside above all other windows.  They can be moved and minimized like normal windows so that you can view all of the screen.

You should read the entire body of text in the Instruction box carefully before proceeding to the next step of the test.  In some cases not all the text is displayed in the Instructions box at once.  Verify that you have read all the instructions for a test by scrolling to the bottom of the text (using the scroll bars or arrow keys).

In addition to presenting you with a list of instructions, the tests may execute commands and spawn applications.  An example of this type of test is the Int 13 VM Test which presents instructions to switch between the VMs while they are executing.  After you select Continue the test completes.

When such a test is finished, an Evaluate the Test dialog (seen below) is displayed.  In this dialog, you enter comments describing any test failures or problems that were encountered

You should select one of the Possible Results buttons based on the test's outcome:

Abort - clears any actions performed thus far in the test sequence.  An Aborted test is represented as a Not Run test in the Statistics list box and it is necessary to run this test at another time.

Disable - represents that the test was not able to be performed because of system configuration problems or limitations.

Fail - states that the test did not run according to specifications or a problem occurred while executing.  If a test fails or you encounter a problem while executing a test, fill out a problem report via the Problem Report Form dialog in the Windows NCT Shell.  For a detailed description of this procedure please see the Problem Report Form section of this document.

Pass - represents that the test ran with no problems and according to the specifications.

When all the tests for the current Windows mode have been executed a message appears.  When this occurs exit Windows, restart Windows in a new mode and run the Windows NCT again.  The NCT then executes the tests geared for that particular Windows mode.  When the NCT has been run in two Windows modes (see the Outline of the Procedure section above for details) the NCT is complete.

Upon completion of the final mode, instead of exiting Windows, select the Wrap Up command from the Windows NCT File menu.  This copies the NCT configuration and result files to a floppy diskette to be sent to the Microsoft Windows Testing organization.  Please see the section entitled The Wrap Up: Completing the NCT of this document for more details.

If you wish to verify your systems beyond the standard NCT test suite, see the Advanced Tests section in this document.

PROBLEMS RUNNING TESTS

This section touches on some possible problems that may occur while running the Windows NCT tests and ways to address, correct, or work around them if that becomes necessary.

Test Crashing Upon Execution

If during the execution of the tests a test crashes Windows and you are forced to reboot, that test is left with a status of Not Run.  When you start the Windows NCT after this has occurred you should highlight the test in the Tests to be Run list box and drag the test over the Black Hole or press delete so the test is not run again.  Select the test from the List of All Tests and drag the test over the Black Hole or choose Test/Mark Test as Failed from the menus which will mark the test as Failed.  Pease fill out a Problem Report explaining the position the test was in when the crash occurred and anything else that may help us track the problem.

Note:	This is a last resort and should only be used after attempts to run the particular test have failed and crashed the system.

Missing and Not Found Executables and Libraries

In very low memory conditions when loading of executable modules and libraries can fail it may appear that programs are not being located correctly.  Particularly in real mode, this can be a sign of low memory and can be rectified by removing unused TSRs, DOS device drivers, and network software, rebooting, and restarting Windows.

Another root to the same symptom can be an application Working Directory, set via the File/Properties command in the Program Manager, as described in the setup section at the beginning of this document.  If this field is not set to the Windows NCT directory in which it was originally installed, libraries and executables for tests can not be found.

When In Doubt

When in doubt, or when it appears that test execution has gone a stray, close down the Windows NCT, exit Windows, and restart the Windows NCT anew.

THE PROBLEM REPORT FORM

The problem report feature is the preferred way to make detailed notes of problems that arise during testing.    The Windows 3.1 NCT provides an on-screen window that presents the appropriate fields and information to be filled out and then stores the information on your hard disk.

To invoke this dialog, select the Problem Report command from the File menu in the NCT Shell.  A dialog showing a new report is presented (see below).  You may fill in the fields with the relevant information or view a previously prepared report by changing the number in the edit field under Select Report To Edit, or by using the scroll bar.  The problem reports that you fill in are stored on the hard disk until the NCT has been fully completed.  At that point a wrap up procedure is executed and the problem reports, along with other result log files are copied to a floppy diskette.

.

You are required to enter information into the Title and Technician fields.  There are three check boxes which represent the Windows mode in which the problem occurs.  Select one or more of these boxes.

The Configuration text box automatically pulls in the configuration file that describes the current system the report is being entered on.  It should not be necessary for you to modify this text.  You should verify that the Configuration information reflects the system configuration at the time the problem was discovered.  For the Problem Description field, enter detailed, specific information about the problem or failure that occurred.  The Steps to Reproduce field should provide the steps necessary for someone not familiar with the problem to be able to reproduce it.  When all the fields are filled in, press the Enter key or select Done.  Please see the example above.

The Report menu for the Problem Report form has four commands:  Open..., Save As..., Write to Text File..., and Restore.  These provide a set of utility commands to facilitate the moving of problem report information to different machines and to put it in a form that can be printed out.  The Open... command allows you to open a set of problem reports that were composed on another system.  The Save As... command saves the set of problem reports that you have composed on your system to another area or media.

The intended use of the Open... and Save As... commands is as follows.  A set of problem reports are composed on one system.  You select Save As... to save the set of reports to a file on a floppy diskette drive.  The floppy diskette is transferred to a second machine which has the Windows NCT installed on it.  On the second machine from the Report menu select Open... and select the file on the floppy drive.  Use the Save As... command to save this set of problem reports on second system's hard drive.

Note:	When importing a problem report file from another system via Open..., if you attempt to create a new problem report for this file the configuration information used for that particular report is that of the current system, not the system from which the file came.  Be very careful to verify that the configuration information reflects the system on which the problem was found.

Although the Windows NCT provides mechanisms for copying problem report files, it should be emphasized that problem reports should be submitted with the NCT results for the machine on which the problem was discovered.  For more information see the section entitled The Wrap-Up: Completing the NCT, below.

The Write to Text File... command can be used to write a problem report out to a text file on disk.  This text file can then be sent to a printer or loaded into a text editor such as the Windows Notepad.

The Restore command restores the current problem report form information to its prior state.

THE WRAP UP: COMPLETING THE NCT

When you have completed the NCT, run the Wrap Up procedure to copy the NCT data files to a floppy diskette to be sent back to Microsoft for recording.  The NCT is completed when the Windows NCT has been run in all the modes which your hardware supports.  For a 286 based machine with the required memory this is standard mode (Win /s).  For a 386 or 486 based machine with the required memory this is standard, and enhanced mode (Win /e).  Execution of all the NCT tests in both modes can be verified by selecting each button in the Statistics box in the Windows NCT application window and verifying that for each mode that your hardware supports the Not Run fields is set to 0 (zero).

Invoke the Wrap Up procedure by selecting Wrap Up from the Windows NCT File menu.  The following dialog is presented:

:

Insert a blank, formatted high density floppy diskette in the drive and enter the correct drive in the Drive edit field.  Select Start to begin transferring files.

This completes the Windows 3.1 NCT.  All tests in each supported mode must be run in order to effectively evaluate the network's compatibility.

Automated Tests

This section of the document describes the individual tests which are executed from the Windows NCT.  These instructions are supplied as a supplement to the on-screen instructions that the tests provide while running.  They are presented here in the order that they are executed by the Windows NCT.

Each test description below has a Mode section describing which mode the test is intended to be executed in, an Objective section giving a description of what the test should accomplish, an Estimated Time to give the user an idea of how long it takes to run the test, an Overview to give a general explanation of the areas covered by the test, and a Procedure section which details the instructions required to complete the test.

Note:	Please keep one formatted floppy disk in the floppy disk drive while running the NCT.  You must use the drive specified for the NCTFlopLtr variable.

APPLET NETWORK TEST

Mode:	Standard and Enhanced

Objective:	To test Applet operations in a network environment.

Est. Time:	25 min.

Overview:	The Applet Network Test tests the following functional areas:

1.	Network drive connections and disconnections

2.	Network file open and file save operations.

3.	Network printing.

4.	Running all of the Windows applets simultaneously.

Procedure:	There is no interaction required.

DMA TEST

Mode:	Enhanced (or Standard on 286)

Objective:	The Direct Memory Access application tests the capability of DOS to format a floppy disk and copy files between your hard drive and floppy drive while in a DOS Prompt (VM).

Est. Time:	5 min.

Overview:	The test requires you to Alt+Tab, hold down the Alt key and press the Tab key, several times during the formatting and copying to truly test the functionality of Direct Memory Access.

Procedure:

Note:	The DOS command FORMAT must be in your path for this test to execute properly.

1.	When the program is executed from within the shell, it spawns a DOS Prompt (VM) which runs the DOS Format command.

2.	Format then asks you to place a blank disk in the drive.

3.	After the disk is in Drive A:, press Enter.

4.	Format formats the disk for use in the test.  Please use a disk which is capable of the highest density for that drive, 1.2Mb, 1.44Mb, etc....

5.	While the disk is being formatted, press Alt+Tab so that you are returned to Windows.

6.	Then return to the DOS application using Alt+Tab, and repeat this three or four times during the formatting process.

7.	When the formatting is done, Format may ask you for a volume label, just press Enter.

8.	When it asks if you would like to format another, answer no.

9.	At this point the program begins copying a file between your new floppy and the hard drive.

10.	Switch, press Alt+Tab, between the DOS PROMPT (VM) and Windows a few times.

11.	When all the copying is done the test logs out and returns to the NCT.

DOS MULTI-VM TEST

Mode:	Enhanced

Objective:	The objective of the Multiple DOS VM test is to ensure that Windows can properly handle and coordinate simultaneous tasks in multiple DOS Virtual Machines.

Est. Time:	30 min.

Overview:	The DOS Multiple VM Test is an automated script.  The automated script brings up three (3) DOS VMs and automatically runs a series of tests within each DOS VM.

The test covers these functional areas:

1.	DOS utilities

2.	NetBIOS functions

3.	File Input/Output

Procedure:	There is no interaction required.

Note:	This test will attempt to start 3 DOS VM's.  If the amount of memory on the machine does not allow this, the test will attempt to clear an "Insufficient Memory" dialog box and log this procedure.

ENH NET LG FILE TEST

Mode:	Enhanced

Objective:	The Enhanced Network Large File Test is designed to confirm the compatibility of the network software and Windows 3.1.

Est. Time:	25 min.

Overview:	The test creates a file in the test directory and then copies the file between the local and network drives, increasing the file size on each copy.  This continuous copying ensures that Windows can create and copy files on the network drive independent of file size.

Procedure:	There is no interaction required.

ENH NET SM FILE TEST

Mode:	Enhanced

Objective:	The Enhanced Network Small File Test is designed to confirm the compatibility of the network software and Windows 3.1.

Est. Time:	15 min.

Overview:	The test creates a file in the test directory and then copies the file between the local and network drives, increasing the file size on each copy.  This continuous copying ensures that Windows can create and copy files on the network drive independent of file size.

Procedure:	There is no interaction required.

FM TEST - DRIVE A

Mode:	Standard and Enhanced

Objective:	File Manager Test for the floppy drive verifies compatibility with the Windows File Manager and floppy drive A.

Est. Time:	15 min.

Overview:	File Manager Test for Floppy Drive A runs a series of tests that verify the ability to format floppies, create, delete and copy files.

Procedure:	When the test starts running, place a blank or unneeded floppy disk of the highest capacity that the drive is capable of formatting (1.2M or 1.44, etc..) in the drive, then click OK.

Note1:	This test is only applicable if you have a floppy drive.

FM TEST - DRIVE C

Mode:	Standard and Enhanced

Objective:	File Manager Test for the hard drive verifies compatibility with the Windows File Manager, DOS and hard drive C.

Est. Time:	10 min.

Overview:	File Manager Test for Drive C runs a series of tests that verify the ability to create, delete and copy files.

Procedure:	There is no interaction required.

FM TEST - NETWORK

Mode:	Standard and Enhanced

Objective:	To test File Manager operations in a network environment.

Est. Time:	15 min.

Overview:	The File Manager Network Test tests the following functional areas:

1.	Network drive connections and disconnections

2,	Network file copy and file move operations.

3.	Network file attributes.

4.	Network directory creation and deletion.

Procedure:	There is no interaction required.

INT 13 TEST

Mode:	Enhanced (or Standard on 286)

Objective:	This test performs reads and writes to the hard drive using the Int 13 routines for drive access.

Est. Time:	5 min. (depending on drive size)

Overview:	This test is a performance test and should be run once from within the NCT and once from outside of Windows, see the section Recommended Manual Tests for instructions on running the test outside of Windows.  Results from both are recorded so that the performance of Windows on your hardware is known.

Procedure:	There is no interaction required.

INT13 VM TEST

Mode:	Enhanced

Objective:	The Int13 VM test launches two versions of the Int13 test in two different DOS VMs.

Est. Time:	5 min. (depending on drive size)

Overview:	This test is not a performance test but a stress of the virtualization of the hard drive done by the Windows system.

Procedure:

1.	This test launches two copies of the INT13 test.

2.	When these have both started, use Alt+Tab to switch between them several times during their run.

3.	After both copies have completed their run, select Continue.

4.	Both of the VMs finish running and prompt you to select any key.  For the tests to exit successfully, you must activate each VM, using Alt+Tab, and select any key.

NETBIOS VM TEST

Mode:	Standard and Enhanced

Objective:	The VM NetBIOS Test is designed to exercise the NetBIOS implementation within a Windows VM.

Est. Time:	10 min.

Overview:	This test runs only if NetBIOS support is detected on the system.  The network and NetBIOS support must be loaded before starting Windows.

Procedure:	There is no user interaction required.

NETWORK STRESS TEST

Mode:	Enhanced

Objective:	The objective of the Network Stress Test is to ensure that Windows can coordinate tasks in multiple DOS Virtual Machines simultaneously over a long period of time.

Est. Time:	4 hours.  It is recommended that you run this test overnight.

Overview:	The Network Stress Test brings up two (2) DOS VMs and automatically runs a series of network tests within each DOS VM.  The test also executes a series of Windows tasks while the DOS VMs are running in the background.

Procedure:	There is no user interaction required.

STD NET LG FILE TEST

Mode:	Standard

Objective:	The Standard Network Large File Test is designed to confirm the compatibility of the network software and Windows 3.1.

Est. Time:	25 min.

Overview:	The test creates a file in the test directory and then copies the file between the local and network drives, increasing the file size on each copy.  This continuous copying ensures that Windows can create and copy files on the network drive independent of file size.

Procedure:	There is no interaction required.

STD NET SM FILE TEST

Mode:	Standard

Objective:	The Standard Network Small File Test is designed to confirm the compatibility of the network software and Windows 3.1.

Est. Time:	15 min.

Overview:	The test creates a file in the test directory and then copies the file between the local and network drives, increasing the file size on each copy.  This continuous copying ensures that Windows can create and copy files on the network drive independent of file size.

Procedure:	There is no interaction required.

TASK SWAPPER TEST

Mode:	Standard

Objective:	To ensure that Windows can properly handle and coordinate multiple DOS batch files running in Standard Mode.

Est. Time:	20 min.

Overview:	The test connects the Network drives, creates two DOS batch files, and starts the DOS batch files.  Use Alt+Tab to switch between the batch files and Windows.

Procedure:

1.	The first batch file started is designed to stress the network connection with various DOS commands.

2.	The second batch file performs a series of network file IO.

3.	The second batch file also runs a NetBIOS test.  While the NetBIOS test is running, any swapping between batch files and Windows is disabled, and this part of the test is designed to make sure that this option is disabled.

4.	While both batch files are running, and before the NetBIOS test starts, you should swap back and forth, using Alt+Tab, between the batch files a number of times.

5.	Once the Net BIOS test has completed, you should swap to the first batch file to let it complete.

6.	Once both batch files have completed, follow the directions in the Instruction dialog box to complete the test.

WINNET API TEST

Mode:	Standard and Enhanced

Objective:	The WinNet API Test tests the APIs in the Network Driver.

Est. Time:	10 min.

Overview:	Tests the non-interactive APIs of the network driver. It tests retrieving the user name, connecting to disk/printer shares, and cancelling connections.

Procedure:	There is no interaction required.

Note:	The WinNet API Test performs operations on servers that support Long File Name (LFN) files.  If your network includes a server that supports LFNs (such as an OS/2 HPFS server), you must set up the first two Network drives in the winplay.ini, NetPath1 and NetPath2, to point to these servers.  The script will skip the LFN portion of the test if it does not detect LFN support from the first two servers in the winplay.ini.

WINNET PRT API TEST

Mode:	Standard and Enhanced

Objective:	The WinNet Prt API Test tests the Network Printer APIs.

Est. Time:	10 min.

Overview:	Tests the non-interactive APIs of the network printer driver. It tests retrieving the print queue name, connecting to printer shares, and cancelling connections.

Procedure:	There is no interaction required.

WIN NETBIOS TEST

Mode:	Standard and Enhanced

Objective:	The Windows NetBIOS Test is designed to exercise your NetBIOS implementation from within a Windows application.

Est. Time:	10 min.

Overview:	This test runs only if NetBIOS support is detected on the system.  The network and NetBIOS support must be loaded before starting Windows.

Procedure:	There is no interaction required.

XMS TEST

Mode:	Enhanced (or Standard on 286)

Objective:	This application tests Himem.sys and its ability to correctly manipulate the hardware A20 line and memory addressing.

Est. Time:	2 min.

Overview:	The XMS Test runs in a DOS Box insideof Windows.

Procedure:	There is no interaction required.

Recommended Manual Tests

The following tests are included as recommended manual tests and need to be run in order to fully complete the NCT.

APPLICATION COMPATIBILITY TEST

Mode:	Standard and Enhanced

Objective:	The Application Compatibility Test is designed to ensure network functionality and compatibility with existing Windows/DOS applications.

Est. Time:	Testing should take approximately 20 to 45 minutes depending upon the application.

Overview:	The functionality consists of opening and editing files from the local hard disk and the network hard disk and printing the files to the printer.

Procedure:

The Application Checklist in Appendix B contains most of the various Windows and DOS applications, please mark the applications that you have, fill in as much information as possible, and return this sheet to Microsoft along with all of the NCT reports.

The following are guidelines to follow when testing your applications to make sure that they are compatible with Windows 3.1 and your network.  If you can run the application from the network try the suggestions under Running the Application from the Network to make sure you can run simultaneous instances of the application across the network.

TESTING THE APPLICATION

1.	Open a file from a local drive, make changes to the file and save the file.  Print the file to a network printer and to a local printer.

2.	Open a file from a network drive, make changes to the file and save the file.  Print the file to a network printer and to a local printer.

3.	Open a file from a floppy drive, make changes to the file and save the file.  Print the file to a network printer and to a local printer.

4.	Disconnect the printer and try to print.  Does the application give you a coherent error message?

5.	Can you run simultaneous instances of the application?

RUNNING THE APPLICATION FROM THE NETWORK

1.	Start the application on Workstation 1 and start the same application on Workstation 2.

2.	Open a file from the network drive on Workstation 1 and open the same file from

Workstation 2.

3.	From Workstation 2 modify the file and print the file to a network printer.

4.	From Workstation 1 save the file to the network drive and exit the application.

5.	Workstation 2 should save the file to the network drive and exit the application.

6.	Start the application from Workstation 1 again, open the previous file, and verify that all

changes have been saved to the file correctly

INT13 DOS TEST

Mode:	Enhanced (or Standard on 286)

Objective:	This test performs reads and writes to the hard drive using the Int 13 routines for drive access.

Est. Time:	5 min.

Overview:	This test is a performance test and should be run from outside of Windows.  Results from both this test and Int 13 Test are recorded so that the performance of Windows on your hardware is known.  While the test should not change any data on the drive, important data should be backed up prior to the test in case of failure.

Procedure:

Exit Windows and go to the NCT Directory.  Execute the INT13RUN batch file.  When this program has completed, restart Windows.

MAIL/MESSAGE TEST

Mode:	Standard and Enhanced

Objective:	Test the compatibility of the network's mail/message services with Windows 3.1.

Est. Time:	10 min.

Overview:	The test executes a call to the DOS Prompt and starts the mail/message service.

Procedure:	Follow the procedures outlined below.

WINDOWS TO DOS PROMPT

1.	Start a DOS Prompt from within Windows and start the mail/message service.

2.	Alt+Tab back to Windows.

3.	Start a second DOS Prompt from within Windows and attempt to start the mail/message service

again.

4.	Close the second DOS Prompt.

5.	Alt+Tab back to the first DOS Prompt with the mail/message service.

SENDING MESSAGES FROM THE SERVICE

1.	From the mail/message service send a mail/message to another workstation.

2.	From the mail/message service send a mail/message to the entire network, including your own

workstation.

RECEIVING MESSAGES FROM WITHIN WINDOWS

1.	Alt+Tab back to Windows.

2.	Have another workstation send a mail/message to your workstation.

3.	Alt+Tab back to the mail/message service.

4.	Send a mail/message back to the other workstation.

5.	Alt+Tab back to Windows.

ERROR CONDITIONS IN WINDOWS

1.	Access the menu to run an executable from Program Manager, press Alt+File and Run.

2.	Enter A: with no floppy in Drive A to produce an Int 24 error condition.

3.	Have another workstation send a mail/message to your workstation while the error condition

exists.  Verify that the mail/message was received.

4.	Cancel the error condition.

PRINTING MESSAGES

1.	Alt+Tab back to the mail/message service.

2.	Print a mail/message to a printer.

3.	Exit out of the mail/message service and type EXIT to return to Windows.

MOUSE DRIVER TEST

Mode:	Enhanced (or Standard on 286)

Objective:	The following tests are designed to verify the compatibility of your mouse with the Windows mouse driver and the Windows virtual mouse driver.  If you notice any results which are different from those specified in the result section of this test, please provide the correct PASS/FAIL result along with a comment describing what you saw.

Est. Time:	20 min.

Overview:	The Mouse Driver Test tests the compatibility of your mouse in Windows and at the DOS Prompt.

Procedure:

MOUSE TEST ONE

1.	Verify that your Config.sys contains a line similar to the following:

'DEVICE=[PATH]MOUSE.SYS'

where [PATH] = the path where a current mouse driver is located.

If this line is not present, add it to your config.sys file and reboot the machine so that the mouse device driver is loaded.

2.	Run Windows and open up a DOS Box.

3.	Run an application in the DOS Box which uses the mouse.

4.	Alt+Tab between the DOS Box and a Windows application and verify that the mouse continues to work correctly in both.

Note:	Be sure to use the mouse buttons, and move the mouse in both the DOS Box and in the Windows application.  Watch for the mouse freezing, buttons which fail to responding, sluggish responses, multiple cursors, or any other incorrect response from the mouse.

MOUSE TEST TWO

In Control Panel, change the mouse setting for Tracking Speed.  Increase or decrease the setting, then click OK to save the change.  Verify that the mouse tracking has responded correctly.

Repeat this with several different speed settings, both increased and decreased, to verify that the tracking speed responds correctly.

In 386 enhanced mode, these settings should not effect any mouse behavior in DOS Prompts that are opened prior to changing setting, so you may wish to open a DOS Box as in the first mouse test and verify that the effects of changing Control Panel does not effect the application running in the DOS Prompt.

MOUSE TEST THREE

In Control Panel, change the mouse setting for Double Click Speed.  Increase or decrease the setting, then click OK to save the change.  Verify that the mouse double click speed has responded correctly.  This can be verified by attempting to select the mouse settings dialog by double clicking on the icon, then closing it, and attempting to choose it by double clicking slower or faster.

Repeat this with several different speed settings, both increased and decreased to verify that the double click speed setting responds correctly.

In 386 enhanced mode, these settings should not effect any DOS Prompts that are opened prior to changing settings, so you may wish to open a DOS Prompt as in the first mouse test and verifying that the effects of changing Control Panel's settings do not effect the application running in the DOS Prompt.

MOUSE TEST FOUR

In Control Panel, swap the mouse button mapping so that the left button is supposed to signify the right button and vice versa.  Then click OK to save the change.  Then verify that the mouse button mapping has responded correctly.  This can be verified by opening the mouse settings dialog and clicking the left and right buttons repeatedly, verifying that the mouse in the setting box indicates that they have been switched.

Switch the mapping back to it's original state and close Control Panel.

In 386 enhanced mode, these settings should not effect any DOS boxes that are opened prior to changing setting, so you may wish to open a DOS Prompt as in the first mouse test and verifying

that the effects of changing Control Panel's settings do not effect the application running in the DOS Prompt.

NETWORK ERROR CONDITIONS

Mode:	Standard and Enhanced

Objective:	The objective of this test is to check how windows 3.1 handles error conditions in a network environment.

Est. Time:	30 min.

Overview:	The Network Error Condition test covers several errors a user may receive while running windows 3.1 in a network environment.  We want to verify that messages received by the user are explicit and that the user is able to recover after each error.

Procedure:

Error conditions should be run after all your other windows tests have been run.  Results of each error condition should be logged on the Network Error Conditions worksheet in Appendix B and sent back to Microsoft.

FULL SETUP

The following Error Conditions should be run on a FULL SETUP. (Setup from the server to your local drive)

Network not started

1.	Attempt to connect to a network drive from File Manager.

2.	From a DOS box attempt to connect to a network drive.

3.	From a DOS box attempt to start a network.

Start network

1.	From Control Panel / Networks attempt to log off network while running Windows.

NET SETUP

The following Error Conditions should be run on a NET  SETUP. (Setup /n from the server to your local drive)

Network not started

1.	Attempt to start Windows.

Network started

1.	From a DOS box attempt to log off network.

2.	From a DOS box attempt to disconnect  network drive which you are running Windows

from.

3.	From a DOS box attempt to redirect / map to an already redirected / mapped drive.

4.	From a DOS box attempt to redirect / map the drive you are running Windows from.

5.	From a DOS box attempt to redirect / map a  drive to a nonexistent directory.

6.	From a DOS box attempt to connect to a network drive with an incorrect password.

7.	From a DOS box attempt to redirect / map to an invalid printer.

8.	From File Manager attempt to disconnect the network drive which you are running Windows

from.

9.	From File Manager attempt to redirect a drive to an invalid  path.

10.	From File Manager attempt to connect to a network drive with an incorrect password.

11.	From File Manager attempt to delete a nonexistent file.

12.	From File Manager attempt to  copy a file using an invalid filename.

13.	From Control Panel / Printer attempt to redirect / map to an invalid printer.

14.	From Control Panel / Printer attempt to redirect / map to an already redirected printer.

15.	From Notepad attempt to Print without printer drivers loaded.

16.	Disconnect the printer while printing.

17.	Disconnect the network cable from the workstation net card while in a Windows session.

18.	Reboot the server while the workstation running .

NETWORK MULTIPLE DOS VM TEST

Mode:	Enhanced

Objective:	The objective of the Network Multiple DOS VM Manual test is to ensure that Windows properly handles and coordinates network specific tasks in multiple DOS Virtual Machines (VM).

Est. Time:	30 min.

Overview:	The DOS Multiple VM Test is a manual test.  The test has you bring up two (2) DOS VMs and run a series of network specific tests in each VM.

The test covers these functional areas:

1.	IPX Specific applications (NetWare ONLY)

2.	DOS applications

3.	Network Specific commands

4.	Printing

Procedure:

1.	Bring up Windows 3.1 in enhanced mode (win /3).  From Program Manager, run (Alt-File-Run) the PIF files DOSVM4.pif and DOSVM5.pif.  Each PIF file brings up a windowed, background mode DOS VM.  We reference DOSVM4.pif's VM as DOS VM #4 and DOSVM5.pif's VM as DOS VM #5.

2.	As you work in DOS VM #4 and DOS VM #5, you should switch, Alt+Tab, between them several times, at any time during the test, to ensure that each DOS VM is still active.

3.	Make DOS VM #4 the active VM.  Make any necessary network drive and printer connections at this time.

4.	Start a DOS application (such as Word 5.5, Wordperfect 5.1, Microsoft PC Works...etc).  You may have to switch DOS VM #4 to full screen mode if your DOS application does not run in windowed mode.

Please verify that the following operations work with your DOS application:

a.	Create a new file.

b.	Save the file to a network drive.

c.	Print the file to a network printer.

d.	Exit the application.

e.	Start the application again, and open the file.

f.	Save the file to a local drive.

Once you verify the above basic operations work, leave the DOS application running in  DOS VM #4 and proceed with step #4.  If you had to switch to full screen mode earlier, switch back to windowed mode, Alt+Enter.  You can minimize DOS VM #4 if you prefer.

5.	Make DOS VM #5 the active VM. In this VM, we want to test commands that are specific to your network.  Try to perform the following types of operations in DOS VM #5.

a.	Printer redirection/maintenance/and print queue management.

b.	Network drive redirection and maintenance.

c.	User security/registration/permissions/access.

d.	Network specific commands which you use (such as NetWare's NCOPY command).

e.	If you are running NetWare v3.1 (or above), run the RCONSOLE command in this DOS VM.  Once RCONSOLE is running, try to switch, Alt+Tab, to DOS VM #4.  Verify that the DOS application is still running (try printing your file to network printer).  Return to DOS VM #5.

6.	Additional tests which you may want to perform in DOS VM #5 are the network drive connection tests:

a.	Use your network's command to redirect a drive to the network.

b.	Run File Manager from Program Manager. Does File Manager display the redirected drive as a network drive?  Can you display the files on that network drive?

c.	Bring up the networks disk connection dialog box (select Disk/Net Connections).  Does File Manager display your redirected drive?

d.	Exit File Manager.

e.	Switch, Alt+Tab, to DOS VM #4 (your DOS application).  Try to save your file to the redirected drive.  What happens?  Does DOS VM #4 recognize the network redirection made in DOS VM #5?

f.	Make DOS VM #5 active.  Use your network's command to disconnect the network drive connection which was made earlier in this VM (step 1).

7.	Additional tests which you may want to perform in DOS VM #5 are the printer redirection tests:

a.	Use your network's command to redirect a parallel port to a network printer.

b.	Run Control Panel from Program Manager.  Run the printers applet.  Double-click the Connect button (Control+C, Enter) to display printer connections.

c.	Is your printer redirection displayed in the dialog box?

d.	Switch, Alt+Tab, back to DOS VM #4 (your DOS application).  Try to print your file to the redirected network printer.  Does DOS VM #4 recognize the printer redirection made in DOS VM #5?

e.	Return to ControlPanel/Printers/Connections dialog box.  Disconnect the printer redirected earlier (Step #1). Close the connections dialog box and the printers dialog box.  Close Control Panel.

f.	Return to DOS VM #5.  Try to print a file to the parallel port that was redirected in Step #1.  Do you receive an error?  How did the VM handle the error?

7.	Make DOS VM #5 active.  Close DOS VM #5 by typing "exit" at the DOS prompt.

8.	Make DOS VM #4 active.  Close your DOS application.  Close DOS VM #4 by typing "exit" at the DOS prompt.

Note:  Any problems encountered during the manual portion of this test should be reported using the Report Form in the Windows NCT shell.

SETUP TEST

Mode:	Not Applicable

Objective:	The objective of the Setup Test is to ensure that Windows installs properly onto workstations

in a networking environment.

Est. Time:	1 hour

Overview:	The setup test is designed to test the following functional areas:

1.	Administrative installation option (Setup /a)

2.	Two new setup options (Quick Install and Custom Install)

3.	Three ways to setup Windows 3.1 files in a network environment

a.	Setup /n to local drive

b.	Setup /n to network drive

c.	Setup to local drive (setup with no parameters)

4.	Setup's detection of workstations system information (such as display, network..etc).

5.	Setup's detection of applications (on local and network drives) and installing correct PIF files.

6.	Setup's detection and upgrading of certain files (such as Himem.sys)

7.	Setup's ability to modify Config.sys and Autoexec.bat correctly.

Notes:	Try setting up Windows onto as many network workstations over the network as possible - simultaneously (section 2: Installing Windows 3.1 onto a Network Workstation).  It is not necessary to have each workstation running the same setup parameter and option.  Rather try to vary with each workstation, the setup parameters and options (outlined below).

Procedure:

INSTALLING WINDOWS 3.1 ONTO A SERVER

Requirements

1.	Note how much disk space is available on server disk:	______Mb.

2.	If the available space is less than 16Mb, please stop and free additional disk space.  Proceed when the space requirement has been fulfilled.

Install Windows 3.1 files onto network server.

1.	Run Setup /a from Windows 3.1 disk #1.

2.	Specify a network disk/directory as the destination.

Installation Verification

1.	Once Setup /a is complete, verify the following information:

a.	None of the files end with the extension "_" (underscore)

b.	Make sure the attributes of the files that were expanded onto the server are read-only.

INSTALLING WINDOWS 3.1 ONTO WORKSTATIONS

Requirements

1.	Workstation network software loaded and user logged into network.

2.	Workstation must have access to the directory where the Windows 3.1 files are installed.

Setup Windows 3.1 onto network workstations

1.	There are now six possible ways to setup Windows 3.1 onto your network workstation.  Outlined in the matrix below are the six possible combinations:

Setup Options

Quick Install

Custom Install

Setup /n to local drive

Setup Configuration #1

Setup Configuration #2

Setup

Parameters

Setup /n to network drive

Setup Configuration #3

Setup Configuration #4

Setup (no parameters) to local drive

Setup Configuration #5

Setup Configuration #6

For Example:

a.	Setup Configuration #1 signifies that you run setup with the /n parameter.  Once setup begins, choose the "Quick Install" option.  When prompted for a destination directory, choose a drive/path on the local drive (users are prompted only if installing on a workstation which has Windows, otherwise setup installs Windows 3.1 into c:\windows).

b.	Setup Configuration #4 signifies that you run setup with the /n parameter.  Once setup begins, choose the "Custom Install" option.  When prompted for a destination directory, choose a drive/path on the network.

2.	Choose one of the six ways to setup Windows 3.1 onto your network workstation.  (please reference the matrix which outlines the six setup configurations - page 38)  Please record the setup configuration number you have decided to test:  # __________.

3.	Run SETUP from the Windows 3.1 network directory with the appropriate setup parameters and options.   The Windows 3.1 network directory is the directory specified when installing the Windows 3.1 files onto the server drive.  The following tasks should be performed if prompted:

a.	Install the appropriate printer driver(s) for all network printers.

b.	Applications should be searched for on both local drive(s) and network drive(s).

c.	On-line documentation should be reviewed during the setup process.

SETUP VERIFICATION

1.	Run the DOS maintenance mode of Setup.  This is accomplished by running setup (without parameters) from the Windows 3.1 destination directory (the directory you specified as the destination for your Windows 3.1 files during Setup, such as c:\win31).  Verify that the System Information components (network, display, mouse, keyboard...etc) are correct.  This is particularly important to verify that the "Quick Install" setup option properly detects your system information.

2.	Verify that any files requiring an upgrade, such as HIMEM.SYS, SMARTDRV.SYS, RAMDRIVE.SYS, EMM386.SYS, and EGA.SYS, are copied to the correct location on the workstation and that modifications to CONFIG.SYS and AUTOEXEC.BAT were appropriate and written properly.

3.	Verify that Windows 3.1 searches and detects applications on the local drives as well as network drives. Verify that the proper PIF file was installed.

ADDITIONAL SETUP TESTS

Outlined below are a few additional setup tests.  The tests outline error conditions which may occur.  You may wish to take some additional time to investigate how Windows 3.1 handles these conditions.

1.	Once you have Windows installed on a network workstation, start windows without the network software loaded (stand-alone PC).  What happens if you setup the workstation using the different setup parameters/options?

2.	During the administrative setup (setup /a) or during setup for a network workstation (setup /n), cause a server and/or workstation error, such as breaking the network connection.

3.	Setup a network workstation (Custom Install Option).  Change the default system information to an incorrect network or display.  After setup is completed, change the incorrect system settings to the correct settings using the DOS maintenance mode of setup.

4.	Setup a network workstation (Custom Install Option).  Change the system information "Network" from the default setting to "No Network Installed".  After setup is completed, does windows start up?

Note:	The following test case may cause your workstation to hang.  Please choose a workstation in which you don't mind rebooting/resetting.

5.	Send a message or mail to the workstation while it is running setup (setup /a or setup /n).  Try sending messages or mail at different times during setup.  How does setup handle messages or mail?

Additional Manual Tests

The following tests included in the NCT are optional.

EMM386 TEST

Mode:	Enhanced

Overview:	The purpose of the EMM386 test is to ensure that your system is compatible with the expanded memory manager shipped with the Windows package.

Procedure:

Note:	This test is only to be performed on 386 and 486 based machines.

If you have already installed EMM386.exe or you have already performed the steps below then

start at number 5.  Otherwise, please perform the following steps:

1.	Select Exit Windows.

2.	Install EMM386.exe by adding the following line to the config.sys file after the line device=himem.sys:

device=c:\windows\EMM386.exe 64

Note:	This example assumes that your Windows directory is C:\WINDOWS.  It also assumes that you are running on a system with two megabytes of RAM or less.  If you have more RAM increase this amount (64) proportionately.

3.	Reboot your system.

4.	Run Windows in enhanced mode and run the Windows NCT.

5.	Go to the Program Manager and run a DOS Prompt.  Use the Exit command to return to Windows.  Verify that no hangs, crashes, or strange behavior occur.

6.	Exit Windows now and remove the device=EMM386.exe line from the CONFIG.SYS and reboot your system.

RAMDRIVE TEST

Mode:	Enhanced (or Standard on 286)

Objective:	The following tests are designed to verify the compatibility of the Windows virtual disk device, RAMDrive, with your system.

Est. Time:	15 min.

Overview:	The tests below verify the device by executing applications and manipulating files on the virtual disk.

Procedure:

1.	In your Config.sys, install the Ramdrive.sys provided with the Windows package to create a logical disk drive (for more information consult the Windows User's Guide).

A Ramdrive of 128Kb should suffice for this test.  Thus your Config.sys looks something like this:

device=c:\windows\himem.sys

...

device=c:\windows\ramdrive.sys 128 /e

Note:	The /e option installs Ramdrive in extended memory. Use the /a option instead of /e to install Ramdrive in expanded memory.

2.	Reboot your machine.

3.	Using File Manager, copy Calendar.exe to the logical drive created by  Ramdrive.

4.	Execute Calendar.exe from the logical drive, create a file, and save it to the logical drive.

5.	Run a second instance of Calendar.exe from the logical drive and load the .CAL file which you have created.

6.	Verify that you are able to run Calendar.exe, save the file, and reload it in a second instance of Calendar.exe.

7.	Exit Windows and remove the device=RamDrive.exe line from the CONFIG.SYS and reboot your system.

SWAPFILE TEST

Mode:	Standard and Enhanced

Objective:	The Swapfile test verifies that you are able to install and operate Windows with a permanent Windows swapfile installed.

Est. Time:	10 min.

Overview:	Setup a permanent Swapfile and verify that Windows loads properly.

Procedure:

Notes:	This test is designed for 386 and 486 based machines only.  This Swapfile test is not capable of running on a diskless workstation.

1.	Run Windows in enhanced mode.

2.	Do not run the Windows NCT.  Create a Swapfile by selecting Control Panel/386 Enhanced/Virtual Memory.

3.	Create a permanent swap file of any size on any available disk partition.

4. 	Exit Windows and restart Windows in Enhanced Mode.

5.	Verify that Windows is able to start and that you are able to complete the remaining tests in the NCT test suite.

DO NOT remove the permanent swap file until the tests are complete.  The permanent swap file can be deleted by running Windows in enhanced mode, selecting Control Panel/386 Enhanced/Virtual Memory and removing the swapfile.

If you are unable to create a swap file for any reason please note it in the results for this test.

XMS DOS TEST

Mode:	Enhanced (or Standard on 286)

Objective:	This application tests Himem.sys and its ability to correctly manipulate the hardware A20 line and memory addressing.

Est. Time:	2 min.

Overview:	This application runs outside of Windows.

Procedure:

Exit Windows and go to the NCT Directory.  Execute the XMSRUN batch file.  When this program has completed its execution, restart Windows and the Windows NCT.

Advanced Tests

This section contains information on additional testing that you can do on your hardware to provide increased coverage of Windows compatibility.  Also discussed here is more information about the programs which are shipped with the NCT so that you can use these tools to generate new test cases.

It is not required that these extra tests be run.  They are provided here for you if you are interested in verifying Windows compatibility with your system beyond the standard compatibility test suite.

VIRTUAL COMM DEVICE

Mode:	Enhanced (or Standard on 286)

Objective:	To test the identification of the communications ports by the VCD and then to force communication (serial) port device contention between two command VMs.

Est. Time:

Overview:	This test discusses the 386 Enhanced mode device virtualization of the communications port by the Virtual Comm Device (VCD).  Windows 3.1 supports up to four communication ports.  This test can be performed on a COM1 or COM2 device.  You should run this test on any serial port higher than two (i.e. Com3 or Com4) that is supported on your system.

Procedure:

TEST ONE

The VCDevice communications port test application checks to see how many serial ports you have on your machine and prompts you for the port number you wish to use.  VCDevice displays the current status of the port and sends and receives data from it.  The message "This VM is the Com3 Port Owner" indicates that the current VM is able to access the Com3 port.

Run Control Panel and select the Windows 386 (Enhanced) icon.  Select the Com device which you are testing from the Device list box and set its Idle time to 6 (in sec.s).  Save the settings by selecting OK.

Run VCDevice.pif and select a Com port to test.  Without closing the first instance, run a second instance of VCDevice.pif and select the same port number to test.  In the second instance of VCDevice after you have entered the port number to use you receive the Device Conflict dialog stating that contention has occurred between the two instances of VCDevice.  Select OK, to give the first instance of VCDevice port ownership.

Verify that the second VCDevice's status line reads:

"This VM is NOT the owner of the COMx port."

Go back to the first instance and verify that it is still the owner.

TEST TWO

Continuing from Test One, close the first instance of VCDevice, the COMx port owner (this is done by simply pressing any non-Windows key on the keyboard).  Now return to the second instance.

Verify that the second instance status line reads:

"This VM is the COMx port owner!"

TEST THREE

The following test only works if there is a debugging terminal or printer attached to the Com port which you are testing.  Also in this test the Autoexec.bat file is specified.  Please note that any text file can be used in its place.

Run VCDevice.pif and select a port to test.  Run File Manager and perform File/Copy... C:\AUTOEXEC.BAT to COMx.  Where COMx is the same port that you specified in VCDevice.pif.  Verify that you receive the 386 Enhanced mode Device Conflict message

"'Windows' is attempting to use the COMx device, which

'Serial Port Test Application.' is currently using.  Do you want 'Windows' to take control of the device?"

Select "Y" <Enter>.

Verify that Windows is given COMx port ownership by the fact that it completes the copy of the text file to Comx.  Also return to VCDevice.pif and verify that after about six seconds its status line now reads

"This VM is the COMx Port Owner!"

ADDITIONAL TESTS

Additional tests for the VCD might include serial communications programs running in a DOS VM with various settings at varying baud rates up to 9600.  In addition, running a Windows communication program at the same time from the Windows VM; verifying contention, etc.

VIRTUAL DISPLAY DEVICE

Mode:	Enhanced (or Standard on 286)

Objective:	This test discusses the 386 Enhanced mode device virtualization of the display by the Virtual Display Device (VDD).

Overview:	Windows 3.1 supports a variety of video modes in a DOS VM and in a windowed DOS Prompt.  There are restrictions to what the current VDDs can support.however.  DispTest is the DOS display test program that comes with the NCT.  It has been written specifically for enhanced mode Windows and exercising the VDD.

Procedure:

For additional testing of the VDD run more then one instance of DispTest at a time.  Change video modes in each VM and move between each one verifying that the screen is not corrupted.  Execute the test with various PIF settings.

Although DispTest was written for enhanced mode Windows it can still be used to exercise the "grabber" under standard and real mode Windows.  Changing into various modes, moving information the Clipboard and verifying it, etc.

COMMAND LINE PARAMETERS

DISPTEST.EXE

If running DISPTEST.EXE from DOS, the following command line options are available:

/Dn	Before the normal test begins this causes the test to attempt to switch into all modes:

n = 1: 1-7, 13-19

n = 2: That were detected possible by /P (see below)

/H or /?	Brings up a list of available command line options.

/I<screen type>	This ignores the display type which was automatically determined and allow the program to read from any section in the .ini file that is identical to the <Screen Type> parameter. (i.e. <Screen Type> = [VGA], [EGA],... brackets are required.)  Any section may be added to the .ini file such as [TESTGROUP1] and then /I[TESTGROUP1] could be used in the command line.

/L<filename>	Specifies a log file other that the default, or if left blank it logs to the default NCT log file.

/N	Turns off the "inverting" effect around the instructions.  Provided for convenience on slow displays.  This causes the /C option to fail due to the difference caused in the resulting screen contents.

/P	Before the test begins, this does a check to see what video modes the BIOS detects are possible for the given display type.  Note: This is not accurate on most displays and performs mainly as a regression item.

/Svmmssii	This switch allows the program to ignore the .ini file, and video detection and display one single screen then exit.  The parameter is as follows:

v	video type (see .ini file header for info, this parameter limits the following parameters to only what options are available for that particular video type.)

mm	must be two digits, stands for the mode.

ss	stands for subscreen number (see .ini file header for info.)

ii	stands for instruction type number (mostly unimportant for single screen test, but see .ini file header for description of options.)

/T<filename>	Chooses a specific .ini file to run from other than the default.

/V	Display the video type code that disptest.exe has detected your video display as being.

DMATEST.EXE

If running DMATEST.EXE from DOS, the following command line options are available:

/Cn	n is number of times to perform copy (inner loop of test.)

/D<filename>	 Allows you to specify the filename of the destination file, otherwise it uses a default name.

/Fc	 c is the drive letter to test.  (Only 'A' and 'B' are allowed, and it defaults to 'A')

/H or /?	 Brings up a list of available command line options.

/I	 Performs only drive type detection, reports what it found, then exits.

/L<filename>	 Specifies the log file you wish DMATEST to log to.  If /L without a filename is specified or if /L is not used, it uses the default NCT log file.

/N	Allows you to run the test without formatting the disk.  It performs only the copying as a test.

/P%options%	Allows you to pass options to the DOS Format command.  For example, you could use this to specify the density with which you wish to format the disk, if it is not the default for the drive.  The %'s are required to mark what is to be passed in, and if a " is needed in the string to pass, use a ' instead and it is be translated.

/Rn	n is number of times to repeat format (outer loop of test.)

/S<filename>	Allows you to specify the source file to copy during the copy test.  A temporary file is created from this file, then used for the copying so that the original is never other than to create the temporary file.

INT13TST.EXE

If running INT13TST.EXE from DOS the following command line options are available:

/Bn	n is the starting location on the disk.  (0 <= n <= 10) where (n=0) specifies start at logical sector 0, and (n=10) specifies start at the last sector minus the amount of sectors that are to be read.  (n = 1,9 are in tenths of disks total sectors).

/C	Turns on the current sector display, so that you can monitor its progress through the disk.  This slows down the operation and cause the test results to be less accurate.

/Dn	n is 1 or 2.  (n=1) specifies that the primary hard drive is to be used and (n=2) is the secondary.  Note: this is the physical drive number, this test ignores partitions and any other logical drive specifications.

/H or /?	Brings up a list of available command line options.

/I	Drive information only.  This displays the drive information on the specified hard drive, then exits.

/L<filename>	Specifies the log file to use.  /L with no filename uses a default log file name, and no /L logs to the NCT log files.

/Rn	Specifies a random seed which is used to determine sector order to operate on.  Normally sectors are read/written in sequential order and this turns on a random order.  If /R is used with no seed, 1024 is used.  This option should be used to verify that caching is not causing a invalid time during the test.

/Sn	Specifies the number of sectors to operate on.

/W	Turns on writing.  The default options only read the sectors while this one reads a sector then writes the contents right back to the sector.

XMSTEST.EXE

If running XMSTEST.EXE from DOS the following command line options are available:

/A 	Disable the standard A20 tests.  - Default execution runs these tests automatically.

/C:n 	Cycle the chosen tests n times.  Default value is 1.

/D 	Test A20 Local Disable.

/E 	Test A20 Local Enable.

/I	Tests the A20 Local Enable and Disable to verify that they do NOT enable interrupts during their execution.  This is a know fault in some Phoenix BIOS routines.

/H 	Lists program options.

/M 	Disables test for Conventional/Extended memory moves. - Default options test this.

/P	Tests Extended Memory writes with pausing.

/R	Reallocate memory block test.

/S:n	Random seed value for Write Test.  Default value is 255

/U	Test UMB allocation.

/V[V...]	Turns up verbosity level, (more Vs, higher verbosity).  Default is 0.

/W	Disables test for Extended Write and Verify.  - Default options test this.

Appendix A

GLOSSARY OF TERMS

This glossary lists some terms which are used in the Windows Network Compatibility test.  Some of the terms are specific to the Windows environment and are included to assist you with any unfamiliar terms.

DOS PROMPT  A virtual machine running Command.com.  Also known as DOS box.

ENHANCED MODE  Only available on 80386/80486-based computers with a minimum of 2 MB of memory.  Enhanced mode allows for the multitasking of non-Windows applications.

EXPANDED MEMORY  Bank-switched memory made available to DOS applications running on 8086/8088-based PCs, or on 80286/80386/80486-based computers in real mode, for which expanded memory boards and accompanying Expanded Memory Manager (EMM) has been added.

EXTENDED MEMORY  Memory in 80286/80386/80486-based computers which is located above the 1MB address boundary.

IHV  Independent hardware vendor.

ISV  Independent software vendor.

HIMEM.SYS  A device driver which facilitates the execution of code in extended memory ( memory  above 1 Megabyte).

MODE  Refers to the three different operating modes for running Windows:  Standard and Enhanced.

OEM  Original equipment manufacturer.

SMARTDRV.EXE  A device driver which creates a disk cache in extended or expanded memory.

STANDARD MODE  Available on all  80286/80386/80486-based computers with at least 1MB of memory.  Standard mode provides access to extended memory and also lets you switch among non-Windows applications.

VIRTUAL MACHINE  An environment available on 80386 based machines which allows programs to run as though they are the only programs in the system.  Each DOS PROMPT (VM) has its own address space (1MB), I/O port space, and interrupt vector table.  Multiple virtual machines can be run simultaneously, each under the illusion that it is in complete control of the computer.