TABLE OF CONTENTS

cd32mpeg.device/CMD_FLUSH

cd32mpeg.device/CMD_RESET

cd32mpeg.device/CMD_WRITE

cd32mpeg.device/GetSCR

cd32mpeg.device/MPEGCMD_GETDEVINFO

cd32mpeg.device/MPEGCMD_GETVIDEOPARAMS

cd32mpeg.device/MPEGCMD_PAUSE

cd32mpeg.device/MPEGCMD_PLAY

cd32mpeg.device/MPEGCMD_PLAYLSN

cd32mpeg.device/MPEGCMD_READFRAMEYUV

cd32mpeg.device/MPEGCMD_SEARCH

cd32mpeg.device/MPEGCMD_SEEKLSN

cd32mpeg.device/MPEGCMD_SETAUDIOPARAMS

cd32mpeg.device/MPEGCMD_SETBORDER

cd32mpeg.device/MPEGCMD_SETVIDEOPARAMS

cd32mpeg.device/MPEGCMD_SETWINDOW

cd32mpeg.device/MPEGCMD_SINGLESTEP

cd32mpeg.device/MPEGCMD_SLOWMOTION

cd32mpeg.device/SetSCR

cd32mpeg.device/CMD_FLUSH                           cd32mpeg.device/CMD_FLUSH

NAME

CMD_FLUSH -- cancel all pending CMD_WRITE requests

FUNCION

CMD_FLUSH aborts all CMD_WRITE requests in progress or queued.

This call will also flush any decoder buffers.  However, the current

play state will not be changed.  For the best visual results, the

currently executing MPEGCMD_PLAY command should be aborted, or the

device should be in a paused state.  Otherwise, undesirable video

or audio artifacts may occur due to decoder underdlow.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to reset

io_Command   - command number for CMD_FLUSH

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

OUTPUTS

None

cd32mpeg.device/CMD_RESET                           cd32mpeg.device/CMD_RESET

NAME

CMD_RESET -- restore device to a known state

FUNCTION

CMD_RESET reset the hardware and/or software of the mpeg device

driver to a known state, cancels all pending I/O requests and resets

all error conditions.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to reset

io_Command   - command number for CMD_RESET

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

OUTPUTS

None

cd32mpeg.device/CMD_WRITE                           cd32mpeg.device/CMD_WRITE

NAME

CMD_WRITE -- normal I/O entry point

FUNCTION

CMD_WRITE is used to send data to the MPEG device driver when playing

back an MPEG stream using the MPEGCMD_PLAY command.

Depending on the capabilities of the MPEG device driver, the data

may be packs from an ISO System stream, or may be raw video and/or

audio streams.

The amount of data the needs to be written to the device depends on

the playback mode being used.  For normal rate playback, it would be

a good idea to keep a large amount of data queued up in order to

avoid video or audio glitches.

When sending MPEG system streams to the device, each block of data

sent by the CMD_WRITE command MUST correspond to exactly one pack

of data.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to queue data for

io_Command   - command number for CMD_WRITE

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

io_Data      - Pointer to a data buffer that contains valid data

io_Length    - Amount of data pointed to by io_Data.

iomr_PTSxxxx - A timestamp associated with this data segment.  This

is typically the PTS value if this data segment was

part of a system stream.

iomr_MPEGFlags - Status information regarding this data:

MPEGFLAGF_VALIDCLK - io_PTSxxxx contains a valid

timestamp associated with

this data.

OUTPUTS

io_Error     - If non-zero, then an error of some kind occurred.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

NOTES

For efficient operation, it will be best to keep a certain minimum

amount of data queued up at any given time, otherwise gaps or glitches

may appear in the audio and/or video output.   Approximately 64k for

video and 16k for audio is a good starting point.

cd32mpeg.device/GetSCR                                 cd32mpeg.device/GetSCR

NAME

GetSCR -- Get the current System Clock Reference value.

SYNOPSIS

clockValue = GetSCR()

D0

ULONG GetSCR(struct Unit *unit);

FUNCTION

This call is used to get the MPEG System Clock Reference from a MPEG

device driver.  This is usually used for synchronizing one device

driver to another.

INPUTS

unit	     - The value in the io_Unit field of the IO request

returned by OpenDevice.

OUTPUTS

clockValue   - The lower 32 bits of the SCR.  The MPEG clock is

specified to be 33 bits, but we only use 32 bits of

that clock.

cd32mpeg.device/MPEGCMD_GETDEVINFO         cd32mpeg.device/MPEGCMD_GETDEVINFO

NAME

MPEGCMD_GETDEVINFO -- Determine the capabilities of an MPEG device

FUNCTION

MPEGCMD_GETDEVINFO is used to determine what types of operations a

MPEG device is capable of, as well as getting a short description of

that board.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to begin playback on

io_Command   - command number for CMD_GETDEVINFO

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

io_Data      - Pointer to a MPEGDevInfo structure to be filled in

by the device.

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

NOTES

Best results will be acheived if you queue up some data before

sending this command.  See cd32mpeg.device/CMD_WRITE for more details.

cd32mpeg.device/MPEGCMD_GETVIDEOPARAMS cd32mpeg.device/MPEGCMD_GETVIDEOPARAMS

NAME

MPEGCMD_GETVIDEOPARAMS -- Get settings for video playback/recording

FUNCTION

MPEGCMD_GETVIDEOPARAMS may be used to determine what the current

settings are for playback and/or recording.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to get parameters from

io_Command   - command number for MPEGCMD_GETVIDEOPARAMS

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

io_Data      - Pointer to a MPEGVideoParams structure.

io_Length    - Sizeof(struct MPEGVideoParams)

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

cd32mpeg.device/MPEGCMD_PAUSE                   cd32mpeg.device/MPEGCMD_PAUSE

NAME

MPEGCMD_PAUSE -- Set the pause mode of the device

FUNCTION

MPEGCMD_PAUSE is used to temporarily suspend the playback or recording

process of an MPEG stream, the intention being to resume playback at

some later time.  All state information as well as any unused buffers

will be maintained by the device driver.  The last displayed video

frame will remain on the screen, and any audio output will be

suspended.

To enable pause, io_Arg1 should be set to a non-zero value.  To

resume normal/slow motion playback, io_Arg1 should be cleared.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to begin playback on

io_Command   - command number for MPEGCMD_PLAY

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

iomr_StreamType  - Set to one of MPEGSTREAM_VIDEO, MPEGSTREAM_AUDIO or

MPEGSTREAM_SYSTEM depending on the stream type.

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

cd32mpeg.device/MPEGCMD_PLAY                     cd32mpeg.device/MPEGCMD_PLAY

NAME

MPEGCMD_PLAY -- Start decoding a MPEG stream

FUNCTION

MPEGCMD_PLAY tells the MPEG device driver to begin standard rate MPEG

playback.

The MPEG stream data must be supplied via the CMD_WRITE device

command.  If you are playing back MPEG from a CD-ROM, you may wish to

use the MPEGCMD_PLAYLSN command.

This command currently will run forever until it is aborted.  Future

versions of the device driver may automatically detect the end of an

MPEG stream.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to begin playback on

io_Command   - command number for MPEGCMD_PLAY

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

iomr_StreamType  - Set to one of MPEGSTREAM_VIDEO, MPEGSTREAM_AUDIO

or MPEGSTREAM_SYSTEM depending on the stream type.

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

NOTES

Best results will be acheived if you queue up some data before

sending this command.  See cd32mpeg.device/CMD_WRITE for more details.

cd32mpeg.device/MPEGCMD_PLAYLSN               cd32mpeg.device/MPEGCMD_PLAYLSN

NAME

MPEGCMD_PLAYLSN -- Start playing a MPEG stream from CD.

FUNCTION

MPEGCMD_PLAYLSN tells the MPEG device driver to begin standard rate

MPEG playback from CD using cd.device.

The command will not return until it has played io_Length number of

sectors or either "end" of the stream is reached.  Play may also be

aborted using AbortIO().

Due to the nature of MPEG video, you MUST supply the position of

the start of the MPEG stream you are playing in iomr_Arg2.  This is

because the current decoder must read the MPEG Video Sequence

Header to dertmine parameters such as picture size, frame rate, etc.

Once this has been done, the device will seek to the position given

in iomr_Offset.

If the current Scan speed setting is less than zero, then the device

will start scanning from the end of the sequence once it has located

the sequence header.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to begin playback on

io_Command   - command number for MPEGCMD_PLAY

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

iomr_StreamType  - Set to one of MPEGSTREAM_VIDEO, MPEGSTREAM_AUDIO

or MPEGSTREAM_SYSTEM depending on the stream type.

iomr_Data    - unused.

iomr_Offset  - Logical sector number to start playback at.

iomr_Length  - Number of sectors to play

iomr_Arg1    - Sector size to use with cd.device

iomr_Arg2    - The real beginning of the stream on the disk.

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

cd32mpeg.device/MPEGCMD_READFRAMEYUV     cd32mpeg.device/MPEGCMD_READFRAMEYUV

NAME

MPEGCMD_READFRAMEYUV -- Read a single video frame in YUV format

FUNCTION

MPEGCMD_READFRAMEYUV may be used to read out the currently displayed

video frame.  The data is supplied in LCrCb format according to MPEG

specifications.

The io_Data field of the IO request should point to a

MPEGFrameStore structure filled in as follows:

mfs_Width    - The width of the video stream

mfs_Height   - The height of the video stream

mfs_Luma     - Pointer to an array of UBYTE's to store the Luminance

information for the current frame.  Must be at least

mfs_Width*mfs_Height bytes in size.

mfs_Cr	     - Pointer to an array of UBYTE's to store the Cr Chroma

information for the current frame.  Must be at least

mfs_Width*mfs_Height/4 bytes in size.

mfs_Cb	     - Pointer to an array of UBYTE's to store the Cb Chroma

information for the current frame.  Must be at least

mfs_Width*mfs_Height/4 bytes in size.

Note that the Cr and Cb components of an MPEG frame have half the

horizontal and vertical resolution of the Luminance data.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to set parameters for

io_Command   - command number for MPEGCMD_SETAUDIOPARAMS

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

io_Data      - Pointer to a MPEGFrameStore structure.

io_Length    - Sizeof(struct MPEGFrameStore)

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

NOTES

Not all devices can support all features listed in the include files,

so please use the CMD_GETDEVINFO command to determine what the board

is actually capable of doing.

cd32mpeg.device/MPEGCMD_SEARCH                 cd32mpeg.device/MPEGCMD_SEARCH

NAME

MPEGCMD_SEARCH -- Set forward or reverse search mode and/or search

for a video I-frame.

FUNCTION

MPEGCMD_SEARCH is used to either set the forward or reverse search

mode, or to do a one-shot search for a video I-frame.

The iomr_Arg1 field of the IO request should be set to a non-zero

value to enable search mode.  If you are playing an MPEG stream

using the MPEGCMD_PLAYLSN command, iomr_Arg1 should be set to the

number of sectors to skip after each I-frame is found.  This value

may be positive or negative.  If you are using the MPEGCMD_PLAY

command, this value should be a 1.

If you would like the device to search for the next I-frame and then

enter pause mode, you may set the MPEGF_ONESHOT flag in the

iomr_MPEGFlags field of the IO request structure.  In this case, the

IO request will not come back until the frame has been found and the

device has entered the paused state.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to step

io_Command   - command number for MPEGCMD_SEARCH

iomr_StreamType  - Set to MPEGSTREAM_SYSTEM or MPEGSTREAM_VIDEO.

io_Flags     - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

cd32mpeg.device/MPEGCMD_SEEKLSN               cd32mpeg.device/MPEGCMD_SEEKLSN

NAME

MPEGCMD_PLAY -- Seek to an arbitrary position with an MPEG stream.

FUNCTION

MPEGCMD_SEEKLSN tells the MPEG device driver to seek to an arbitrary

position withing the stream when playing from CD.

This command may only be used while there is a currently executing

MPEGCMD_PLAYLSN command.  Also, the fields of the IOMPEGReq structure

must be set up exactly as if you were issuing the MPEGCMD_SEEKLSN

command.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to begin playback on

io_Command   - command number for MPEGCMD_PLAY

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

iomr_StreamType  - Set to one of MPEGSTREAM_VIDEO, MPEGSTREAM_AUDIO

or MPEGSTREAM_SYSTEM depending on the stream type.

iomr_Data    - unused.

iomr_Offset  - Logical sector number to seek to.

iomr_Length  - Number of sectors to play

iomr_Arg1    - Sector size to use with cd.device

iomr_Arg2    - The real beginning of the stream on the disk.

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

cd32mpeg.device/MPEGCMD_SETAUDIOPARAMS cd32mpeg.device/MPEGCMD_SETAUDIOPARAMS

NAME

MPEGCMD_SETAUDIOPARAMS -- Set parameters for playback/recording

FUNCTION

MPEGCMD_SETPARAMS allows you to set such things as DAC attenuation,

sample rage, target bitrate for decoding, etc.

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to set parameters for

io_Command   - command number for MPEGCMD_SETAUDIOPARAMS

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

io_Data      - Pointer to a MPEGAudioParams structure.

io_Length    - Sizeof(struct MPEGAudioParams)

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

NOTES

Not all devices can support all features listed in the include files,

so please use the CMD_GETDEVINFO command to determine what the board

is actually capable of doing.

cd32mpeg.device/MPEGCMD_SETBORDER           cd32mpeg.device/MPEGCMD_SETBORDER

NAME

MPEGCMD_SETBORDER -- Set display position and border color

FUNCTION

MPEGCMD_SETBORDER is used to set the display position and border color

for the decoded video stream.

The io_Data field of the IO request must point to a MPEGBorderParams

structure.   That structure should be set up as follows:

mbp_LeftBorder  - Offset from left of display (Hi-Res pixels)

mbp_TopBorder	- Offset from top of display  (NI scanlines)

mbp_BorderRed	- 8-bit red value for the border color

mbp_BorderGreen - 8-bit green value for the border color

mbp_BorderBlue  - 8-bit blue value for the border color

Values of 0 for mbp_LeftBorder and/or mbp_TopBorder mean that the

video should be centered horizontally and/or vertically centered

on the display.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to step

io_Command   - command number for MPEGCMD_SETBORDER

io_Data	     - pointer to a MPEGBorderParams structure.

io_Length    - sizeof(struct MPEGWindowParams)

io_Flags     - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

cd32mpeg.device/MPEGCMD_SETVIDEOPARAMS cd32mpeg.device/MPEGCMD_SETVIDEOPARAMS

NAME

MPEGCMD_SETVIDEOPARAMS -- Set parameters for playback/recording

FUNCTION

MPEGCMD_SETVIDEOPARAMS allows you to set such things as picture size,

display type (NTSC/PAL), etc.

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to set parameters for

io_Command   - command number for MPEGCMD_SETVIDEOPARAMS

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

io_Data      - Pointer to a MPEGVideoParams structure.

io_Length    - Sizeof(struct MPEGVideoParams)

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

NOTES

Not all devices can support all features listed in the include files,

so please use the MPEGCMD_GETDEVINFO command to determine what the

device is actually capable of doing.

The mvp_DisplayType parameter will be ignored during normal playback.

If you need to explicitly set what type of screen the MPEG will be

displayed on, you must set it before playback begins.

cd32mpeg.device/MPEGCMD_SETWINDOW           cd32mpeg.device/MPEGCMD_SETWINDOW

NAME

MPEGCMD_SETWINDOW -- Set portion of MPEG video to display

FUNCTION

MPEGCMD_SETWINDOW is used to set the portion of a MPEG video stream

that will be displayed.  The io_Data parameter must point to a

MPEGWindowParams structure that contains offset/size information.

The fields in the MPEGWindowParams structure are used as follows:

mwp_XOffset - X Offset into the decoded picture for display start

mwp_YOffset - Y Offset into the decoded picture for display start

mwp_Width - How wide the displayed portion should be

mwp_Height - How high the displayed portion should be

mwp_XOffset and mwp_Width are specified in Amiga Hi-Res pixels.

mwp_YOffset and mwp_Height are specified in non-interlaced scan

lines.

Note: The CL450 is limited to positioning the decode window on

lo-res pixel boundaries, so mwp_XOffset will be scaled down by

two.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to step

io_Command   - command number for MPEGCMD_SETWINDOW

io_Data	     - pointer to a MPEGWindowParams structure.

io_Length    - sizeof(struct MPEGWindowParams)

io_Flags     - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

cd32mpeg.device/MPEGCMD_SINGLESTEP         cd32mpeg.device/MPEGCMD_SINGLESTEP

NAME

MPEGCMD_STEP -- Step one video frame

FUNCTION

MPEGCMD_STEP may be used with mpeg device drivers that support it to

single step through successive frames of an MPEG movie.  This command

will cause the device to enable pause mode if it wasn't already.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit      - unit number to step

io_Command   - command number for CMD_STEP

io_Flags     - flags, must be cleared if not use:

IOF_QUICK - (CLEAR) reply I/O request

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

cd32mpeg.device/MPEGCMD_SLOWMOTION         cd32mpeg.device/MPEGCMD_SLOWMOTION

NAME

MPEGCMD_SLOWMOTION -- Set slow motion playback mode.

FUNCTION

MPEGCMD_SLOWMOTION is used to set or clear slow motion playbakc.

For this command, iomr_Arg1 is used to set the slow-motion frame

rate.  The display framerate will be equal to (stream frame rate/

iomr_Arg1).  So a value of 2 would play the stream at half speed,

4 would be quarter speed, etc.  A value of 0 will clear the slow

motion mode.  A value of 1 will cause video to play at the normal

rate, but no audio decoding will occur.

INPUTS

mn_ReplyPort - pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device    - pointer to device node, must be set by (or copied

from I/O block set by) OpenDevice function

io_Unit      - unit number to step

io_Command   - command number for MPEGCMD_SLOWMOTION

iomr_Arg1    - see above.

io_Flags     - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

OUTPUTS

io_Error     - If non-zero, then an error of some kind occured.

See <exec/errors.h> and <devices/mpeg.h> for more

details.

cd32mpeg.device/SetSCR                                 cd32mpeg.device/SetSCR

NAME

SetSCR -- Set the current time

SYNOPSIS

success = SetSCR(unitPtr, clockValue)

D0               D0

BOOL SetSCR(struct Unit *unit, ULONG clockValue);

FUNCTION

This call is used to set the MPEG System Clock Reference for a MPEG

device driver.  This is usually used for synchronizing one device

driver to another.

INPUTS

unit	     - The value in the io_Unit field of the IO request

returned by OpenDevice().

clockValue   - The lower 32 bits of the SCR.  The MPEG clock is

specified to be 33 bits, but we only use 32 bits of

that clock.

OUTPUTS

success	     - Whether or not the clock was set.

BUGS

The current hardware has a limitation in that you can only use

this function to synchronize a raw video stream to another source.