TimerInt

NAME

TimerInt -- lowlevel.library/AddTimerInt(), RemTimerInt(),

StartTimerInt(), StopTimerInt() Test Program

SYNOPSIS

CLI:

TimerInt

Workbench:

TimerInt

FUNCTION

TimerInt is a part of a series of test programs for lowlevel.library.

The purpose of TimerInt is to test the lowlevel.library/AddTimerInt(),

RemTimerInt(), StartTimerInt(), and StopTimerInt() functions by

installing a timer interrupt handler and starting and stopping

one-shot and continuous timers.

To set the timer interval, use the Interval integer gadget.

To select the timer mode (one-shot or continuous), use the Mode

cycle gadget.

To start the timer interrupt, select the Start button gadget.

Note that to run a one-shot timer multiple times, the Start button

gadget must be selected for each run.

To stop the timer interrupt, select the Stop button gadget.

The Deviation number gadget displays the maximum deviation (in

microseconds) between the timer interrupt and independent timings

performed using the system clock.

To terminate TimerInt, select the close gadget of the window.

INPUTS

None

RESULT

RETURN_OK (0)       -   Success

RETURN_WARN (5)     -   Warning

RETURN_FAIL (20)    -   Failure

EXAMPLE

NOTES

To ensure that the specified user data for the keyboard interrupt

handler is correctly received, the keyboard interrupt handler

compares the interrupt data against a magic cookie. If any calls

to interrupt routine had incorrect interrupt data, the user is

notified when the program exits and a warning (RETURN_WARN (5))

is returned by KBInt.

BUGS

If timer interrupt deviates from timing with timer.device by one second

or more, an incorrect deviation will be reported because the deviation

is measured in microseconds only.

SEE ALSO