Parsing MIDI

~~~~~~~~~~~~

First off, I suggest that you send for a copy of the MIDI specification

from the International MIDI Association (5316 W. 57th St., Los Angeles, CA

90056, U.S.A). The cost for the basic specification is $40.00 for non-members.

In the mean time, however, here is a quick summary of some of the most

important aspects of MIDI:

MIDI is primarily a _gestural_ language, in other words it does not

represent sounds or even music, but rather a series of actions by a performer,

such as hitting a key, pressing a pedal, etc. The instrument can interpret

the messages however it chooses (for example, many instruments have a feature

which allows the musical keyboard to be remapped to a set of non-standard

musical frequencies, allowing non-western musical styles).

MIDI data comes in a stream of bytes. Bytes which have the high bit (bit 7)

set are _status bytes_. Bytes which have the high bit cleared are _data bytes_.

Each midi message consists of a status byte followed by zero or more

data bytes. The number of data bytes depends on the status byte.

Midi messages are organized into the following general classes:

Channel Messages (status bytes $80-$EF)

System Common Messages (status bytes $F0-$F7)

System Real-Time Messages (status bytes $F8-$FF)

Channel messages are messages which are sent to specific MIDI "channels".

Each channel message has a 4-bit channel number which is stored in bits 0-3

of the status byte for that message. MIDI instruments generally listen to one

or more channels, and any messages which have a channel number which the

instrument is not listening to is ignored. In addition, a MIDI instrument

may assign instruments or other state information on a channel-by-channel

basis, for example channel #1 might be a trumpet sound, channel #2 an organ,

etc.

System Common messages and System Real-time messages generally cause

global changes to the instrument. The difference between the two is that

the latter are generally very short and tend to come rapidly.

The include file "mididefs.h" supplied with CAMD lists all the types

of status bytes (the status bytes for each type of channel message is defined

without the channel number -- you must OR in the channel number into the

status byte. So for example, the "Note On" messages is listed as 0x90, but

can actually be any value between 0x90 and 0x9F, depending on the channel).

Running Status: In order to increase the limited bandwidth of MIDI

devices, a form of simple compression called running status is used.

Basically, what this means is that you can omit a status byte if the

previous status byte was exactly the same. So for example, if you wanted

to turn on two notes on the same channel, instead of sending the

byte stream:

0x92 0x41 0x12

0x92 0x52 0x44

You could send:

0x90 0x41 0x12 0x52 0x44

Note that only Channel messages may use running status. This is because

channel messages are all fixed in length, while many types of system messages

are variable length (They continue until the next status byte).

Fortunately, if you're writing an application that uses the CAMD library,

you don't need to worry about running status or the number of data bytes

because CAMD automatically handles running status on both input and output,

and knows how long each message should be. But there may be times when you

need to deal with MIDI streams directly.

The most important MIDI messages to support are "note on" and "note off".

All other messages can simply be ignored for a minimalist implementation.

A "note on" message consists of three bytes:

status byte (0x90 plus the channel number)

pitch       (in other words, the keyboard key that was hit, 0-127,

where 60 is middle C)

velocity    (How hard you hit the key, 1-127. 0 is a special case.

Velocity usually affects how loudly the note is played)

A "Note Off" message has exactly the same format, except that the

status byte is (0x80 | channel number) instead of (0x90 | channel number).

Most instruments support _polyphony_, which means that multiple note-on

messages can be sent to turn on several notes at once, and then later

note-off messages can be sent to turn them off again. Each note-off will

turn off one note of the given pitch. The velocity field for note-offs is

usually ignored in most instruments, but can be used for special effects

such as decay rate.

There are two additional ways to turn off a note.

The first of these is to send a note-on with a velocity of zero. This

is exactly equivalent to sending a note-off with a velocity of 64. The

reason for this is to take greater advantage of running-status compression.

This allows many note-on's and note-off's to be sent without changing the

status byte.

The second is the "All notes off" command. This is a special command

supported by some instruments, and is detailed in the MIDI specification.

You don't need to support it for a first cut implementation. The reason

for this command is simple: Since the MIDI note-on and note-off messages

are seperate messages, it is possible for notes to become "stuck" if the

MIDI cable is unplugged (or other transmission problem occurs, such as

crashing the computer) between the note-on and the note-off command.

Since the note-off command never arrived, the note will continue to play

(annoying everyone in the room) even if the cable is plugged back in.

(Note that unlike serial cables, it is electically safe to abuse MIDI cables

this way). "All Notes Off" is a conveniant way to silence everything on a

channel.