TABLE OF CONTENTS

input.device/AddHandler

input.device/RemHandler

input.device/Reset

input.device/SetMPort

input.device/SetMTrig

input.device/SetMType

input.device/SetPeriod

input.device/SetThresh

input.device/Start

input.device/WriteEvent

input.device/AddHandler                 input.device/AddHandler

NAME

AddHandler - add an input handler to the device

FUNCTION

Add a function to the list of functions called to handle

input events generated by this device.  The function is called

as

newInputEvents = Handler(inputEvents, handlerData);

D0			     A0		  A1

IO REQUEST

io_Message	mn_ReplyPort set

io_Device	preset by OpenDevice

io_Unit		preset by OpenDevice

io_Command	IND_ADDHANDLER

io_Data		a pointer to an interrupt structure.

is_Data 	the handlerData pointer described above

is_Code	the Handler function address

NOTES

The interrupt structure is kept by the input device until a

RemHandler command is satisfied for it.

input.device/RemHandler                 input.device/RemHandler

NAME

RemHandler - remove an input handler from the device

FUNCTION

Remove a function previously added to the list of handler

functions.

IO REQUEST

io_Message	mn_ReplyPort set

io_Device	preset by OpenDevice

io_Unit		preset by OpenDevice

io_Command	IND_REMHANDLER

io_Data		a pointer to the interrupt structure.

NOTES

This command is not immediate

input.device/Reset                           input.device/Reset

NAME

Reset - reset the input device

FUNCTION

Reset resets the input device without destroying handles

to the open device.

IO REQUEST

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Unit		preset by the call to OpenDevice

io_Command	CMD_RESET

io_Flags	IOB_QUICK set if quick I/O is possible

input.device/SetMPort                     input.device/SetMPort

NAME

SetMPort - set the current mouse port

FUNCTION

This command sets the gameport port at which the mouse is

connected.

IO REQUEST

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Unit		preset by the call to OpenDevice

io_Command	IND_SETMPORT

io_Flags	IOB_QUICK set if quick I/O is possible

io_Length	1

io_Data		a pointer to a byte that is either 0 or 1,

indicating that mouse input should be obtained

from either the left or right controller port,

respectively.

input.device/SetMTrig                     input.device/SetMTrig

NAME

SetMTrig - set the conditions for a mouse port report

FUNCTION

This command sets what conditions must be met by a mouse

before a pending Read request will be satisfied.  The trigger

specification is that used by the gameport device.

IO REQUEST

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Unit		preset by the call to OpenDevice

io_Command	IND_SETMTRIG

io_Flags	IOB_QUICK set if quick I/O is possible

io_Length	sizeof(gameportTrigger)

io_Data		a structure of type GameportTrigger, which

has the following elements

gpt_Keys -

GPTB_DOWNKEYS set if button down transitions

trigger a report, and GPTB_UPKEYS set if button up

transitions trigger a report

gpt_Timeout	-

a time which, if exceeded, triggers a report;

measured in vertical blank units (60/sec)

gpt_XDelta	-

a distance in x which, if exceeded, triggers a

report

gpt_YDelta	-

a distance in x which, if exceeded, triggers a

report

input.device/SetMType                     input.device/SetMType

NAME

SetMType - set the current mouse port controller type

FUNCTION

This command sets the type of device at the mouse port, so

the signals at the port may be properly interpreted.

IO REQUEST

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Unit		preset by the call to OpenDevice

io_Command	IND_SETMTYPE

io_Flags	IOB_QUICK set if quick I/O is possible

io_Length	1

io_Data		the address of the byte variable describing

the controller type, as per the equates in

the gameport include file

input.device/SetPeriod                   input.device/SetPeriod

NAME

SetPeriod - set the key repeat period

FUNCTION

This command sets the period at which a repeating key repeats.

This command always executes immediately.

IO REQUEST - a timerequest

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Unit		preset by the call to OpenDevice

io_Command	IND_SETPERIOD

io_Flags	IOB_QUICK set if quick I/O is possible

io_tv_Secs	the repeat period seconds

io_tv_Micro	the repeat period microseconds

input.device/SetThresh                   input.device/SetThresh

NAME

SetThresh - set the key repeat threshold

FUNCTION

This command sets the time that a key must be held down before

it can repeat.  The repeatability of a key may be restricted

(as, for example, are the shift keys).

This command always executes immediately.

IO REQUEST - a timerequest

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Unit		preset by the call to OpenDevice

io_Command	IND_SETTHRESH

io_Flags	IOB_QUICK set if quick I/O is possible

io_tv_Secs	the threshold seconds

io_tv_Micro	the threshold microseconds

input.device/Start                           input.device/Start

NAME

Start - restart after stop

FUNCTION

Start restarts the unit after a stop command.

IO REQUEST

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Unit		preset by the call to OpenDevice

io_Command	CMD_START

io_Flags	IOB_QUICK set if quick I/O is possible

input.device/WriteEvent                 input.device/WriteEvent

NAME

WriteEvent - propagate input event(s) to all handlers

FUNCTION

IO REQUEST

io_Message	mn_ReplyPort set if quick I/O is not possible

io_Device	preset by the call to OpenDevice

io_Unit		preset by the call to OpenDevice

io_Command	IND_WRITEEVENT

io_Flags	IOB_QUICK set if quick I/O is possible

io_Length	the size of the io_Data area in bytes: there

are sizeof(inputEvent) bytes per input event.

io_Data		a buffer area with input events(s).  The

fields of the input event are:

ie_NextEvent

links the events together, the last event

has a zero ie_NextEvent.

ie_Class

ie_SubClass

ie_Code

ie_Qualifier

ie_X, ie_Y

ie_TimeStamp

as desired

NOTES

The contents of the input event(s) are destroyed.