TABLE OF CONTENTS

disk.resource/AllocUnit

disk.resource/FreeUnit

disk.resource/GetUnit

disk.resource/GetUnitID

disk.resource/GiveUnit

disk.resource/AllocUnit                     disk.resource/AllocUnit

NAME

AllocUnit - allocate a unit of the disk

SYNOPSIS

Success = AllocUnit( unitNum ), DRResource

D0     	             D0         A6

FUNCTION

This routine allocates one of the units of the disk.  It should

be called before trying to use the disk (via GetUnit).

INPUTS

unitNum -- a legal unit number (zero through three)

RESULTS

Success -- nonzero if successful.  zero on failure.

EXCEPTIONS

SEE ALSO

BUGS

disk.resource/FreeUnit                       disk.resource/FreeUnit

NAME

FreeUnit - deallocate the disk

SYNOPSIS

FreeUnit( unitNum ), DRResource

D0         A6

FUNCTION

This routine deallocates one of the units of the disk.  It should

be called when done with the disk.  Do not call it if you did

no successfully allocate the disk (there is no protection -- you

will probably crash the disk system).

INPUTS

unitNum -- a legal unit number (zero through three)

RESULTS

EXCEPTIONS

SEE ALSO

BUGS

disk.resource/GetUnit                         disk.resource/GetUnit

NAME

GetUnit - allocate the disk for a driver

SYNOPSIS

lastDriver = GetUnit( unitPointer ), DRResource

D0		      A1             A6

FUNCTION

This routine allocates the disk to a driver.  It is either

immediately available, or the request is saved until the disk

is available.  When it is available, your unitPointer is

sent back to you (via ReplyMsg).  You may then reattempt the

GetUnit.

Allocating the disk allows you to use the disk's resources.

Remember however that there are four units to the disk; you are

only one of them.  Please be polite to the other units (by never

selecting them, and by not leaving interrupts enabled, etc.).

When you are done, please leave the disk in the following state:

dmacon dma bit ON

dsklen dma bit OFF (write a #DSKDMAOFF to dsklen)

adkcon disk bits -- any way you want

entena:disk sync and disk block interrupts -- Both DISABLED

CIA resource index interrupt -- DISABLED

8520 outputs -- doesn't matter, because all bits will be

set to inactive by the resource.

8520 data direction regs -- restore to original state.

INPUTS

unitPtr - a pointer you your disk resource unit structure.

Note that the message filed of the structure MUST

be a valid message, ready to be replied to.

RESULTS

lastDriver - if the disk is not busy, then the last unit

to use the disk is returned.  This may be used to

see if a driver needs to reset device registers.

(If you were the last user, then no one has changed

any of the registers.  If someone else has used it,

then any allowable changes may have been made).  If the

disk is busy, then a null is returned.

EXCEPTIONS

SEE ALSO

BUGS

disk.resource/GetUnitID                     disk.resource/GetUnitID

NAME

GetUnitID - find out what type of disk is out there

SYNOPSIS

idtype = GetUnitID( unitNum ), DRResource

D0	            D0         A6

FUNCTION

INPUTS

RESULTS

idtype -- the type of the disk drive.  Standard types are

defined in the resource include file.

EXCEPTIONS

SEE ALSO

BUGS

disk.resource/GiveUnit                       disk.resource/GiveUnit

NAME

GiveUnit - Free the disk back up

SYNOPSIS

GiveUnit(), DRResource

A6

FUNCTION

This routine frees the disk after a driver is done with it.

If others are waiting, it will notify them.

INPUTS

RESULTS

EXCEPTIONS

SEE ALSO

BUGS