TABLE OF CONTENTS

serial.device/command/Break

serial.device/command/Clear

serial.device/command/Flush

serial.device/command/Query

serial.device/command/Read

serial.device/command/Reset

serial.device/command/SetParams

serial.device/command/Start

serial.device/command/Stop

serial.device/command/Write

serial.device/function/AbortIO

serial.device/function/BeginIO

serial.device/function/CloseDevice

serial.device/function/OpenDevice

serial.device/command/Break                     serial.device/command/Break

NAME

Break -- send a break signal over the serial line

FUNCTION

This command sends a break signal (serial line held low

for an extended period) out the serial port. This is accom-

plished by setting the UARTBRK bit of reg ADKCON. After a

duration (user specifiable via setparams, default 250000

microseconds) the bit is reset and the signal discontinued.

If the QUEUEDBRK bit of io_SerFlags is set in the io_Request

block, the request is placed at the back of the write-request

queue and executed in turn. If the QUEUEDBRK bit is not set,

the break is started immediately, control returns to the

caller, and the timer discontinues the signal after the

duration is completed. Be aware that calling BREAK may

affect other commands such as ABORT, FLUSH, STOP, START, etc...

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      SDCMD_BREAK

io_Flags        set/reset IO_QUICK per above description

RESULTS

Error -- if the Break succeded, then Error will be null.

If the Break failed, then the Error will be non-zero.

serial.device/command/Clear                     serial.device/command/Clear

NAME

Clear -- clear the serial port buffers

FUNCTION

This command resets the serial port's read buffer pointers.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_CLEAR

RESULTS

Error -- If the Clear succeded, then io_Error will be null.

If the Clear failed, then the io_Error will be non-zero.

serial.device/command/Flush                     serial.device/command/Flush

NAME

Flush -- clear all queued I/O requests for the serial port

FUNCTION

This command purges the read and write request queues for the

serial device. Flush will not affect active requests.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_FLUSH

RESULTS

Error -- if the Flush succeded, then io_Error will be null.

If the Flush failed, then the io_Error will be non-zero.

serial.device/command/Query                     serial.device/command/Query

NAME

Query -- query serial port/line status

FUNCTION

This command return the status of the serial port lines and

registers. The number of unread bytes in the serial device's

read buffer is shown in io_Actual.

The break send & received flags are cleared by a query, and

whenever a read IORequest is returned with a error

in io_Error.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       preset by OpenDevice

io_Unit         preset by OpenDevice

io_Command      SDCMD_QUERY

RESULTS

io_Status        BIT  ACTIVE  FUNCTION

LSB       0    ---    reserved

1    ---    reserved

2    high   parallel "sel" on the A1000

On the A500 & A2000, "sel" is also

connected to the serial port's

"Ring Indicator".  Be cautious when

making cables.

3    low    Data Set Ready

4    low    Clear To Send

5    low    Carrier Detect

6    low    Ready To Send

7    low    Data Terminal Ready

MSB       8    high   hardware overrun

9    high   break sent (most recent output)

10    high   break received (as latest input)

11    high   transmit x-OFFed

12    high   receive x-OFFed

13-15    ---    reserved

io_Actual       set to count of unread input characters

io_Error -- Query will always succeded.

serial.device/command/Read                       serial.device/command/Read

NAME

Read -- read input from serial port

FUNCTION

This command causes a stream of characters to be read in from

the serial port buffer.  The number of characters is specified

in io_Length.

The Query function can be used to check how many characters

are currently waiting in the serial port buffer.  If more characters

are requested than are currently available, the ioRequest

will be queued until it can be satisfied.

The best way to handle reads is to first Query to get the number

of characters currently in the buffer.  Then post a read request

for that number of characters (or the maximum size of your buffer).

If zero characters are in the buffer, post a request

for 1 character.  When at least one is ready, the device will return

it.  Now start over with another Query.

Before the program exits, it must be sure to AbortIO() then WaitIO()

any outstanding ioRequests.

IO REQUEST

io_Message      A mn_ReplyPort is required

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_READ

io_Flags        If the IOB_QUICK bit is set, read will try

to complete the IO quickly

io_Length       number of characters to receive.

io_Data         pointer to buffer

RESULTS

Error -- if the Read succeded, then io_Error will be null.

If the Read failed, then io_Error will be non-zero.

io_Error will indicate problems such as parity mismatch,

break, and buffer overrun.

SEE ALSO

serial.device/command/Write

serial.device/command/Query

serial.device/command/SetParams

BUGS

Having multiple outstanding read IORequests at any one time will

probably fail.

Old documentation mentioned a mode where io_Length was set to -1.

If you want a NULL terminated read, use the io_TermArray instead.

serial.device/command/Reset                     serial.device/command/Reset

NAME

Reset -- reinitializes the serial port

FUNCTION

This command resets the serial port to its freshly initialized

condition. It aborts all I/O requests both queued and current,

relinquishes the current buffer, obtains a new default sized

buffer, and sets the port's flags and parameters to their

boot-up time default values. The functions places the reset

parameter values in the ioRequest block.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_RESET

RESULTS

Error -- if the Reset succeded, then Error will be null.

If the Reset failed, then the Error will be non-zero.

serial.device/command/SetParams             serial.device/command/SetParams

NAME

SetParams -- change parameters for the serial port

FUNCTION

This command allows the caller to change parameters for the

serial device. Except for xON-xOFF enable/disable, it will

reject a setparams call if any reads or writes are active

or pending.

Note specifically:

1. Valid input for io_Baud is between 112 and 292000 baud inclusive;

asynchronous i/o above 32KB (especially on a busy system) may

be ambitious.  Note that a request for 110 baud should be rounded

to 112 baud... the hardware minimum.

2. The EOFMODE and QUEUEDBRK bits of io_SerFlags can be set/reset

in the io_Rqst block without a call to SetParams. The SHARED

and 7WIRE bits of io_SerFlags can be used in OpenDevice calls.

ALL OTHER PARAMETERS CAN ONLY BE CHANGED BY THE SetParams

COMMAND.

3. RBufLen must be at least 64.

4. If not used, io_ExtFlags MUST be set to zero.

5. xON-xOFF is by default enabled. The XDISABLED bit is the only

parameter that can be changed via a SetParams call while the

device is active. Note that this will return the value

SerErr_DevBusy in the io_Error field.

6. If trying to run MIDI, you should set the RAD_BOOGIE bit of

io_SerFlags to eliminate unneeded overhead. Specifically, this skips

checks for parity, x-OFF handling, character lengths other than

8 bits, and testing for a break signal. Setting RAD_BOOGIE will

also set the XDISABLED bit.

Note that writing data (that's already in MIDI format) at MIDI rates

is easily accomplished. Using this driver alone for MIDI reads may,

however, may not be reliable, due to MIDI timestamping requirements,

and possibility of overruns in a busy multitasking and/or display

intensive environment.

7. If you select mark or space parity (see io_ExtFlags in serial.h),

this will cause the SERB_PARTY_ON bit to be set, and the setting

of SERB_PARTY_ODD to be ignored.

8. For best results, set the RAD_BOOGIE flag whenever possible.  See

#6 for details.

9. Note that at this time parity is *not* calculated for the xON-xOFF

characters.  If you have a system that is picky about the parity of

these, you must set your own xON-xOFF characters in io_CtlChar.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       preset by OpenDevice

io_Unit         preset by OpenDevice

io_Command      SDCMD_SETPARAMS (0x0B)

NOTE that the following fields are filled in by Open

to reflect the serial device's current configuration.

io_CtlChar      a longword containing byte values for the

xON,xOFF,INQ,ACK fields (respectively)

(INQ/ACK not used at this time)

io_RBufLen      length in bytes of input buffer

NOTE that any change in buffer size causes the

current buffer to be deallocated and a new,

correctly sized one to be allocated. Thusly,

the CONTENTS OF THE OLD BUFFER ARE LOST.

io_ExtFlags     additional serial flags (bitdefs in devices/serial.h)

mark & space parity may be specified here.

io_Baud         baud rate for reads AND writes. (See 1 above)

io_BrkTime      duration of break signal in MICROseconds

io_TermArray    ASCII descending-ordered 8-byte array of

termination characters. If less than 8 chars

used, fill out array w/lowest valid value.

Terminators are checked only if EOFMODE bit of

io_Serflags is set. (e.g. x512F040303030303 )

io_ReadLen      number of bits in read word (1-8) not including parity

io_WriteLen     number of bits in write word (1-8) "      "       "

io_StopBits     number of stop bits (0, 1 or 2)

io_SerFlags     see devices/serial.h for bit equates, NOTE that x00

yields exclusive access, xON/OFF-enabled, no

parity checking, 3-wire protocol and TermArray

inactive.

RESULTS

Error -- if the SetParams succeded, then Error will be null.

If the SetParams failed, then the Error will be non-zero.

SEE ALSO

serial.device/function/OpenDevice

serial.device/function/BeginIO

serial.device/command/Start                     serial.device/command/Start

NAME

Start -- restart paused I/O over the serial port

FUNCTION

This function restarts all current I/O on the serial port by

sending an xON to the "other side", and submitting a "logical

xON" to "our side", if/when appropriate to current activity.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_START

RESULTS

SEE ALSO

serial.device/command/Stop

serial.device/command/Stop                       serial.device/command/Stop

NAME

Stop -- pause all current I/O over the serial port

FUNCTION

This command halts all current I/O on the serial port by

sending an xOFF to the "other side", and submitting a "logical

xOFF" to "our side", if/when appropriate to current activity.

IO REQUEST

io_Message      mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_STOP

RESULTS

SEE ALSO

serial.device/command/Start

serial.device/command/Write                     serial.device/command/Write

NAME

Write -- send output to serial port

FUNCTION

This command causes a stream of characters to be written out

the serial port. The number of characters is specified in

io_Length, unless -1 is used, in which case output is sent until

a null(0x00) is encountered.

IO REQUEST

io_Message      must have mn_ReplyPort initialized

io_Device       set by OpenDevice

io_Unit         set by OpenDevice

io_Command      CMD_WRITE

io_Flags        Set IOF_QUICK to try quick I/O

io_Length       number of characters to transmit, or if set

to -1 transmit until null encountered in buffer

io_Data         pointer to block of data to transmit

RESULTS

Error -- if the Write succeded, then io_Error will be null.

If the Write failed, then the io_Error will be non-zero.

SEE ALSO

serial.device/command/setParams

serial.device/command/Read

serial.device/function/AbortIO               serial.device/function/AbortIO

NAME

AbortIO(ioRequest) -- abort an I/O request

A1

FUNCTION

This is a direct function call to the serial.device, and an exec

call.  Most programs will simply call exec.

This function attempts to aborts a specified read or write request.

If the request is active, it is stopped immediately. If the request is

queued, it is painlessly removed.

After AbortIO(), you must generally do a WaitIO().

INPUTS

iORequest  -- pointer to the IORequest Block that is to be aborted.

RESULTS

io_Error -- if the Abort succeded, then io_Error will be #IOERR_ABORTED

(-2) and the request will be flagged as aborted (bit 5 of

io_Flags is set).  If the Abort failed, then the Error will be zero.

SEE ALSO

serial.device/function/BeginIO

serial.device/function/BeginIO               serial.device/function/BeginIO

NAME

BeginIO(ioRequest),deviceNode -- start up an I/O process

A1        A6

FUNCTION

This is a direct function call to the device.  It is intended for

more advanced programmers.  See exec's DoIO() and SendIO() for

the normal method of calling devices.

This function initiates a I/O request made to the serial

device. Other than read or write, the functions are performed

synchronously, and do not depend on any interrupt handling

logic (or it's associated discontinuities), and hence should

be performed as IO_QUICK.

With some exceptions, reads and writes are merely initiated by

BeginIO, and thusly return to the caller as begun, not completed.

Completion is signalled via the standard ReplyMsg routine.

Multiple requests are handled via FIFO queueing.

One exception to this non-QUICK handling of reads and writes

is for READS when:

- IO_QUICK bit is set

- There are no pending read requests

- There is already enough data in the input buffer to satisfy

this I/O Request immediately.

In this case, the IO_QUICK flag is not cleared, and the request

is completed by the time it returns to the caller. There is no

ReplyMsg or signal bit activity in this case.

INPUTS

ioRequest  -- pointer to an I/O Request Block of size

io_ExtSerSize (see serial.i for size/definition),

containing a valid command in io_Command to process,

as well as the command's other required parameters.

deviceNode -- pointer to the "serial.device", as found in

the IO_DEVICE of the ioRequest.

RESULTS

io_Error -- if the BeginIO succeded, then Error will be null.

If the BeginIO failed, then the Error will be non-zero.

I/O errors won't be reported until the io completes.

SEE ALSO

devices/serial.h

serial.device/function/CloseDevice       serial.device/function/CloseDevice

NAME

CloseDevice -- close the serial port

SYNOPSIS

CloseDevice(deviceNode)

A1

FUNCTION

This is an exec call that terminates communication with the

serial device.  Upon closing, the device's input buffer is freed.

Note that all IORequests MUST be complete before closing.

If any are pending, your program must AbortIO() then WaitIO()

to complete them.

INPUTS

deviceNode - pointer to the device node, set by Open

SEE ALSO

serial.device/function/Open

serial.device/function/OpenDevice         serial.device/function/OpenDevice

NAME

OpenDevice -- Request an opening of the serial device.

SYNOPSIS

error = OpenDevice(SERIALNAME, unit, ioRequest, flags)

D0                 A0          D0    A1         D0

FUNCTION

This is an exec call.  Exec will search for the serial.device, and

if found, will pass this call on to the device.

Unless the shared-access bit (bit 5 of io_SerFlags) is set,

exclusive use is granted and no other access to that unit is

allowed until the owner closes it.  All the serial-specific fields

in the ioRequest are initialized to their most recent values (or

the Preferences default, for the first time open).

If support of 7-wire handshaking (i.e. RS232-C CTS/RTS protocol)

is required, set the 7WIRE bit in io_SerFlags before opening

the serial device.

INPUTS

SERIALNAME	- pointer to literal string "serial.device"

unit      	- Must be zero, or a user setable unit number.

(This field is used by multiple port controllers)

Zero specifies the built-in serial port.

ioRequest 	- pointer to an ioRequest block of size io_ExtSerSize

to be initialized by the serial.device.

(see devices/serial.h for the definition)

NOTE use of io_SerFlags (see FUNCTION above)

IMPORTANT: The ioRequest block MUST be of size io_ExtSerSize !

flags     	- Must be zero for future compatibility

RESULTS

D0 	  - same as io_Error

io_Error  - If the Open succeded, then io_Error will be null.

If the Open failed, then io_Error will be non-zero.

io_Device - A pointer to whatever device will handle the calls

for this unit.  This pointer may be different depending

on what unit is requested.

BUGS

If 7-wire handshaking is specified, this enables a timeout "feature".

If the device holds off the computer for more than about 30-60

seconds, the device will return the write request with the error

SerErr_TimerErr.  Don't depend on this, however.  If you want a timeout,

set up the timer.device and wait for either timer, or serial IO to

complete.

SEE ALSO

exec/CloseDevice

devices/serial.h