TABLE OF CONTENTS

narrator.device/AbortIO

narrator.device/OpenDevice

narrator.device/CMD_FLUSH

narrator.device/CMD_READ

narrator.device/CMD_RESET

narrator.device/CMD_START

narrator.device/CMD_STOP

narrator.device/CMD_WRITE

narrator.device/CloseDevice

narrator.device/AbortIO				    narrator.device/AbortIO

NAME

AbortIO - Abort an IO request

SYNOPSIS

AbortIO(IORequest)

A1

FUNCTION

Exec library call to abort a specified READ or WRITE request.

The IORequest may be in the queue or currently active.  If

currently active, the request is immediately stopped and then

removed.

INPUTS

Pointer to the IORequest block to be aborted.

RESULTS

io_Error field in the IORequest block set to #IOERR_ABORTED.

SEE ALSO

narrator.device/CloseDevice			  narrator.device/CloseDevice

NAME

CloseDevice - terminates access to the narrator device

SYNOPSIS

CloseDevice(IORequest)

A1

FUNCTION

Close invalidates the IO_UNIT and IO_DEVICE fields in the

IORequest block, preventing subsequent IO until another

OpenDevice.  CloseDevice also reduces the open count.  If

the count goes to 0 and the expunge bit is set, the device

is expunged.  If the open count goes to zero and the delayed

expunge bit is not set, CloseDevice sets the expunge bit.

INPUTS

A valid IORequest block with its io_Message structure, and

io_Device and io_Unit fields properly initialized.  These

fields are initialized by OpenDevice.

RESULTS

CloseDevice invalidates the unit and device pointers in the

IORequest block.

SEE ALSO

narrator.device/CMD_FLUSH			narrator.device/CMD_FLUSH

NAME

CMD_FLUSH - Aborts all inprogress and queued requests

SYNOPSIS

Standard device command.

FUNCTION

Aborts all inprogress and queued speech requests.

INPUTS

Valid IORequest block with the io_Command field set to CMD_FLUSH.

A valid IORequest block is one with its io_Message structure, and

io_Device and io_Unit fields properly initialized.  The easiest

way to insure proper initialization is to make a copy of the

IORequest block after a successful OpenDevice call.

RESULTS

io_Error in IORequest block set to 0

SEE ALSO

Exec input/output documentation.

narrator.device/CMD_Read			narrator.device/CMD_Read

NAME

CMD_READ - Query the narrator device for mouth shape or other

synchronization events.

SYNOPSIS

Standard device command.

FUNCTION

Currently, there are three events which the user can inquire

about from the narrator device.  These are: mouth shape changes,

start of word, and start of syllable.  Each read request returns

information about any or all of these events as determined by

the bits set in the sync field of the read IORequest block.  In

the case of mouth shape changes, each shape returned is guaranteed

to be different from the previously returned shape to allow

updating to be done only when necessary.  Each read request is

associated with a write request	by information contained in the

IORequest block used to open the device.  Since the first field

in the read IORequest block is a write IORequest structure, this

association is easily made by copying the write IORequest block

(after the OpenDevice call) into the voice field of the read

IORequest block.  If there is no write in progress or in the

device input queue with the same pseudo unit number as the read

request, the read will be returned to the user with an error.  This

is also how the user knows that the write request has finished and

that s/he should not issue any more reads.  Note that in this case

the mouth shapes may not be different from previously returned values.

INPUTS

mouth_rb IORequest block with the voice field (a narrator_rb

structure) copied from the associated write request with the

following fields modified:

io_Message - Pointer to message port for read request

io_Command - CMD_READ

io_Error   - Clear before issuing first read

width      - 0

height     - 0

RESULTS

As long as the speech is in progress, each read returns the

following information in the mouth_rb IORequest block.

If mouth shape changes are requested the following fields are

modified:

width  - Contains mouth width value in arbitrary units

height - Contains mouth height value in arbitrary units

shape  - Compressed form of mouth shapes (internal use only)

******	NEW FOR V37 NARRATOR

If word synchronization is requested:

sync   - Bit NDB_WORDSYNC is set

If syllable synchronization is requested:

sync   - Bit NDB_SYLSYNC is set

Note that any or all of the above fields can be set and it is

the user's responsibility to check for all possibilities.

SEE ALSO

CMD_WRITE

Exec input/output documentation.

narrator.device/CMD_RESET			    narrator.device/CMD_RESET

NAME

CMD_RESET - Reset the device to a known state

SYNOPSIS

Standard device command.

FUNCTION

Resets the device as though it has just be initialized.

Aborts all read/write requests whether active of enqueued.

Restarts device if it has been stopped.

INPUTS

Valid IORequest block with the io_Command field set to CMD_RESET.

A valid IORequest block is one with its io_Message structure, and

io_Device and io_Unit fields properly initialized.  The easiest

way to insure proper initialization is to make a copy of the

IORequest block after a successful OpenDevice call.

RESULTS

SEE ALSO

Exec input/output documentation.

narrator.device/CMD_START			    narrator.device/CMD_START

NAME

CMD_START - Restarts the device after a CMD_STOP command

SYNOPSIS

Standard device command.

FUNCTION

CMD_START restarts the currently active speech (if any)	and

allows queued requests to start.

INPUTS

Valid IORequest block with the io_Command field set to CMD_START

A valid IORequest block is one with its io_Message structure, and

io_Device and io_Unit fields properly initialized.  The easiest

way to insure proper initialization is to make a copy of the

IORequest block after a successful OpenDevice call.

RESULTS

io_Error set to 0.

SEE ALSO

Exec input/output documentation.

narrator.device/CMD_STOP			    narrator.device/CMD_STOP

NAME

CMD_STOP  - Stops the device.

SYNOPSIS

Standard device command.

FUNCTION

CMD_STOP halts the currently active speech (if any) and	prevents

any queued requests from starting.

INPUTS

Valid IORequest block with the io_Command field set to CMD_STOP.

A valid IORequest block is one with its io_Message structure, and

io_Device and io_Unit fields properly initialized.  The easiest

way to insure proper initialization is to make a copy of the

IORequest block after a successful OpenDevice call.

RESULTS

io_Error set to 0.

SEE ALSO

Exec input/output documentation.

narrator.device/CMD_WRITE			    narrator.device/CMD_WRITE

NAME

CMD_WRITE - Send speech request to the narrator device

SYNOPSIS

Standard device command.

FUNCTION

Sends a phonetic string to the narrator device to be spoken

and, optionally, is used to direct the narrator device to

return mouth shape changes, and word and syllable sync events

in response to read requests from the user.  The phonetic string

consists of ASCII characters representing the individual phonemes.

Refer to the narrator device chapter of the libraries and devices

volume of the ROM Kernel Manual for detailed information.

INPUTS

User IORequest block (struct narrator_rb as defined in .h file).

The OpenDevice call will initialize the IORequest block to a

"standard male" voice.  If you want to change any parms, do so

after the OpenDevice call and before the DoIO (or SendIO/WaitIO).

For a complete description of the narrator_rb structure, see the

narrator.h or .i include file.  Note that the OpenDevice call does

not initialize all the fields needed by the narrator device.  The

IORequest fields which must be set by the user before issuing the

write request are:

io_Command - Set to CMD_WRITE

io_Data    - Pointer to phonetic string

io_Length  - Length of phonetic string

ch_masks   - Array of audio channel selection masks (see audio

device documentation for description of this field)

nm_masks   - Number of audio channel selection masks

****** NEW FOR V37 NARRATOR

flags - The bit NDB_NEWIORB must be set in the flags field if

any of the new features of the V37 narrator are used

In addition to producing synthetic speech, the narrator device

also provides features for synchronizing the speech to animation

or other user defined events.  There are three types of events

that the user can request.  They are mouth shape changes, start of

new word, and start of new syllable.  Mouth shape changes are

requested by setting the mouths field of the IORequest block to a

non-zero value.  Word and syllable sync events are requested by

setting the NDB_WORDSYNC and/or NDB_SYLSYNC bits in the flags field

of the IORequest block.  Note that word and syllable sync only work

in V37 and later versions of the narrator device.

RESULTS

The narrator device range checks and performs other validity

checks for all input parms.  If any input is in error, the device

sets the io_Error field of the IORequest block to an appropriate

value (see include files for error codes).  If everything is in

order, the narrator device will produce the speech and clear the

io_Error field.  The io_Actual field is set to the length of the

input string that was actually processed.  If the return code

indicates a phoneme error (ND_PhonErr), io_Actual is the NEGATIVE

of the position in the input string where the error occured.

SEE ALSO

Read command.

Audio device documentation.

Exec input/output documentation.

narrator.device/OpenDevice			    narrator.device/OpenDevice

NAME

OpenDevice - opens the narrator device.

SYNOPSIS

error = OpenDevice("narrator.device",  unit, IORequest, flags);

D0                       A0            D0     A1         D1

FUNCTION

The OpenDevice routine grants access to the narrator device.

OpenDevice checks the unit number, and if non-zero, returns an

error (ND_UnitErr).  If this is the first time the driver has

been opened, OpenDevice will attempt to open the audio device

and allocate the driver's static buffers.  If either of these

operations fail, an error is returned.  See the .h and .i

include files for possible error return codes.  Next, OpenDevice

(done for all opens, not just the first one) initializes various

fields in the user's IORequest block (see below).  If users wish

to use non-default values for these parms, the values must be set

after the open is done.  OpenDevice also assigns a pseudo unit

number to the IORB for use in synchronizing read and write requests.

See the read command for more details.	Finally, OpenDevice stores

the device node pointer in the IORequest block and clears the

delayed expunge bit.

***** NEW FOR V37 NARRATOR *****

Several new fields in the IORequest block have been added for V37

narrator.  These fields are initialized when the device is opened

if the NDB_NEWIORB bit is set in the flags field of the user's

IORequest block.  Note that NDB_NEWIORB is set in the IORequest

block, NOT in the "flags" input parm to the OpenDevice call.

INPUTS

device     - "narrator.device"

unit       - 0

IORequest  - Pointer to the user's IORequest block

flags	   - 0

RESULTS

The narrator device will initialize the IORequest block as follows

(assume IORB points to the IOrequest block):

IORB->rate = 150;		/* Speaking rate in words/minute */

IORB->pitch = 110;		/* Baseline pitch in Hertz     	*/

IORB->mode = NATURALF0;		/* Pitch (F0) mode		*/

IORB->sex = MALE;		/* Sex of voice			*/

IORB->volume = 64		/* Volume, full on		*/

IORB->sampfreq = 22200		/* Audio sampling freq 		*/

IORB->mouths = 0		/* Don't generate sync events	*/

and if the NDB_NEWIORB bit is set:

IORB->F0enthusiasm = 0		/* F0 excursion factor		*/

IORB->F0perturb = 32		/* F0 perturbation (in 32nds)	*/

IORB->F1adj = 0			/* F1 adjustment in 5% steps	*/

IORB->F2adj = 0 		/* F2 adjustment in 5% steps	*/

IORB->F3adj = 0			/* F3 adjustment in 5% steps	*/

IORB->A1adj = 0			/* A1 adjustment in decibels	*/

IORB->A2adj = 0			/* A2 adjustment in decibels	*/

IORB->A3adj = 0		 	/* A3 adjustment in decibels	*/

IORB->articulate = 100		/* Transition time multiplier	*/

IORB->centralize = 0		/* Degree of vowel centralization */

IORB->centphon = ""		/* Pointer to central ASCII phon  */

IORB->AVbias = 0		/* AV bias			*/

IORB->AFbias = 0		/* AF bias			*/

IORB->priority = 100		/* Priority while speaking	*/

SEE ALSO

The include files contain the complete IORequest block definition,

default settings, and error return codes.

Exec input/output documentation.