TABLE OF CONTENTS

disk.resource/AllocUnit

disk.resource/FreeUnit

disk.resource/GetUnit

disk.resource/GetUnitID

disk.resource/GiveUnit

disk.resource/ReadUnitID

disk.resource/AllocUnit                               disk.resource/AllocUnit

NAME

AllocUnit - allocate a unit of the disk

SYNOPSIS

Success = AllocUnit( unitNum ), DRResource

D0     	             D0         A6

BOOL AllocUnit(LONG);

FUNCTION

This routine allocates one of the units of the disk.  It should

be called before trying to use the disk (via GetUnit).

In reality, it is perfectly fine to use GetUnit/GiveUnit if AllocUnit

fails.  Do NOT call FreeUnit if AllocUnit did not succeed.  This

has been the case for all revisions of disk.resource.

INPUTS

unitNum -- a legal unit number (zero through three)

RESULTS

Success -- nonzero if successful.  zero on failure.

EXCEPTIONS

SEE ALSO

BUGS

disk.resource/FreeUnit                                 disk.resource/FreeUnit

NAME

FreeUnit - deallocate the disk

SYNOPSIS

FreeUnit( unitNum ), DRResource

D0         A6

void FreeUnit(LONG);

FUNCTION

This routine deallocates one of the units of the disk.  It should

be called when done with the disk.  Do not call it if you did

no successfully allocate the disk (there is no protection -- you

will probably crash the disk system).

INPUTS

unitNum -- a legal unit number (zero through three)

RESULTS

EXCEPTIONS

SEE ALSO

FreeUnit()

BUGS

Doesn't check if you own the unit, or even if anyone owns it.

disk.resource/GetUnit                                   disk.resource/GetUnit

NAME

GetUnit - allocate the disk for a driver

SYNOPSIS

lastDriver = GetUnit( unitPointer ), DRResource

D0		      A1             A6

struct DiscResourceUnit *GetUnit(struct DiscResourceUnit *);

FUNCTION

This routine allocates the disk to a driver.  It is either

immediately available, or the request is saved until the disk

is available.  When it is available, your unitPointer is

sent back to you (via ReplyMsg).  You may then reattempt the

GetUnit.

Allocating the disk allows you to use the disk's resources.

Remember however that there are four units to the disk; you are

only one of them.  Please be polite to the other units (by never

selecting them, and by not leaving interrupts enabled, etc.).

When you are done, please leave the disk in the following state:

dmacon dma bit ON

dsklen dma bit OFF (write a #DSKDMAOFF to dsklen)

adkcon disk bits -- any way you want

entena:disk sync and disk block interrupts -- Both DISABLED

CIA resource index interrupt -- DISABLED

8520 outputs -- doesn't matter, because all bits will be

set to inactive by the resource.

8520 data direction regs -- restore to original state.

NOTE: GetUnit() does NOT turn on the interrupts for you.

You must use AbleICR (for the index interrupt) or intena

(for the diskbyte and diskblock interrupts) to turn them

on.  You should turn them off before calling GiveUnit,

as stated above.

INPUTS

unitPtr - a pointer you your disk resource unit structure.

Note that the message filed of the structure MUST

be a valid message, ready to be replied to.  Make sure

ln_Name points to a null-terminated string, preferably

one that identifies your program.

You need to set up the three interrupt structures,

in particular the IS_DATA and IS_CODE fields.  Set them

to NULL if you don't need that interrupt.  Also, set

the ln_Type of the interrupt structure to NT_INTERRUPT.

WARNING: don't turn on a disk resource interrupt unless

the IS_CODE for that interrupt points to executable code!

IS_CODE will be called with IS_DATA in A1 when the

interrupt occurs.  Preserve all regs but D0/D1/A0/A1.

Do not make assumptions about A0.

RESULTS

lastDriver - if the disk is not busy, then the last unit

to use the disk is returned.  This may be used to

see if a driver needs to reset device registers.

(If you were the last user, then no one has changed

any of the registers.  If someone else has used it,

then any allowable changes may have been made).  If the

disk is busy, then a null is returned.

EXCEPTIONS

SEE ALSO

GiveUnit()

BUGS

disk.resource/GetUnitID                               disk.resource/GetUnitID

NAME

GetUnitID - find out what type of disk is out there

SYNOPSIS

idtype = GetUnitID( unitNum ), DRResource

D0	            D0         A6

LONG GetUnitID(LONG);

FUNCTION

Gets the drive ID for a given unit.  Note that this value may

change if someone calls ReadUnitID, and the drive id changes.

INPUTS

unitNum -- a legal unit number (zero through three)

RESULTS

idtype -- the type of the disk drive.  Standard types are

defined in the resource include file.

EXCEPTIONS

SEE ALSO

ReadUnitID()

BUGS

disk.resource/GiveUnit                                 disk.resource/GiveUnit

NAME

GiveUnit - Free the disk back up

SYNOPSIS

GiveUnit(), DRResource

A6

void GiveUnit();

FUNCTION

This routine frees the disk after a driver is done with it.

If others are waiting, it will notify them.

INPUTS

RESULTS

EXCEPTIONS

SEE ALSO

GetUnit()

BUGS

In pre-V36, GiveUnit didn't check if you owned the unit.  A patch

for this was part of 1.3.1 SetPatch.  Fixed in V36.

disk.resource/ReadUnitID                             disk.resource/ReadUnitID

NAME

ReadUnitID - reread and return the type of drive (V37)

SYNOPSIS

idtype = ReadUnitID( unitNum ), DRResource

D0	               D0         A6

ULONG ReadUnitID(LONG);

FUNCTION

Rereads the drive id for a specific unit (for handling drives

that change ID according to what sort of disk is in them.  You

MUST have done a GetUnit before calling this function!

INPUTS

unitNum -- a legal unit number (zero through three)

RESULTS

idtype -- the type of the disk drive.  Standard types are

defined in the resource include file.

EXCEPTIONS

SEE ALSO

GetUnitID()

BUGS