Flash 1.4

Using the 'flash' utility:

FLASH is a tool for copying disk-like media to PCMCIA FlashROM cards.

Currently only the Intel FlashROM programming algorithim is supported.

In the future, algorithims for EEPROM, and Epson FlashROM may be

supported.  The Flash tool verifies that the FlashROM card you

insert returns the Intel identifier byte before it erases, or writes

to the card.

To create a readable disk, it makes the most sense to let a filesystem

figure out the proper disk structure for files, so FLASH does not

try to do this.   Rather, you can copy your files to a formatted

floppy disk, disk-like card, or ramdrive (RAD:) of a size which

will fit on your flashrom card.

Warning: When you use Flash to put a disk image on a FlashROM card,

an exact duplicate disk is created.  Do not access the original disk

or the new FlashROM duplicate via DOS while both are mounted - it

will confuse DOS and cause problems.

To duplicate one of these, do the following:

1.) Select SOURCE (trackdisk unit 0, carddisk unit 0, or ramdrive unit 0).

trackdisk unit 0 -

Refers to the built-in floppy drive.

Advantages: No need for lots of card swapping.

Disadvantages: Relatively slow (but not intolerable), and limited

size (880K for standard density Amiga floppies).  Will not

duplicate MS-DOS disks (requires a future option for mfm.device).

carddisk unit 0 -

Refers to use of the PCMCIA card slot, and a disk-like formatted card.

Advantages: Allows for quick proto-typing on an SRAM card

up to the maximum of 4 megabytes.  Can read MS-DOS like

cards.

Disadvantages: Painful because card swapping is required.

The SRAM card must have a battery backup so it can be removed

while the FlashROM card is being programmed.

ramdrive unit 0 -

Refers to use of ramdrive.device (RAD:).

Advantages: Allows for quick duplication, and modifications

of RAD: contents.  No card swapping required.

Disadvantages: Requires the system have sufficient RAM to

create a ramdrive disk as large as the FlashROM.  See

special note below regarding the 'Install' option.

Binary File -

This will copy a binary file to FlashROM, but also requires that

the binary file already contain an appropriate Card Information Structure.

Such a file may be created, for example, by doing a binary save of

the memory contents of an overlapped-type PCMCIA RAM card.

If Flash recognizes the RAM card information which is created by

the AmigaXIP loader tool "loadc", it will convert the device tuple

to a flashrom tuple before flashing.

When copying from disklike media, the Flash tool will create an

appropriate disk-like Card Information Structure for you (based

on disk geometry, or in the case of the ramdrive, from the mountlist

entry).

2.) Select FlashROM speed.  250ns is the recommended default.  Consult

FlashROM documentation before selecting faster speeds.

3.) Select FlashROM total size.  There is a maximum of 4 megabytes,

which is all the A600 can support.

4.) Select FlashROM zone size.  Consult FlashROM documentation for this

information.  Simply put, zone size is the number of bytes which will

'potentially' be erased when the ERASE command is sent.  In actual

use, the ERASE command may be sent many times until the entire zone

is completely erased.

5.) Select type of card.  Currently only Intel FlashROM is supported.

In the future, EEPROM, and other flash rom types may be supported.

6.) There is generally no reason to turn off the "Erase Enable" option.

Doing so is only useful if you want to be warned that a FlashROM is

not already blank during the duplication process.  However you

may still use the "Full Erase" option to completely erase the card.

7.) Enable "Full Verify" for second pass verification.  While not

generally needed (since each byte is verified during writes),

the second pass is useful if you want the extra sense of security.

8.) The "Install" option is only available when duplicating from

the ramdrive.  This is required because it is not possible to

use the "C:Install" program to make the ramdrive bootable (the

ramdrive uses bootpoint booting, not bootblock booting like

floppies, and disk-like cards).  When enabled, a 2.0 style bootblock

is written to the first 2 disk blocks of the card.  The 3 byte

DOS type identifier is left unchanged.

9.) Select an action:

Check Blank - checks every zone of the card to see if it

is blank (every byte is $FF).

Full Erase - Erases every zone of the card.  For Intel

FlashROM, this includes writing 0's to the card, followed

by erase.

Program - Checks each zone of the card for blank state,

erases the zone if it is not blank (unless you've

disabled this feature), and prompts you as needed for

the source, and destination media.

10.) The "Abort" gadget is enabled during any destructive operations,

such as Erase and Write operations.  It is currently not enabled during

read, and verify operations.  Aborting brings up a requester verifying

that you really want to abort.

11.) To quit, click on the window close gadget.