@DATABASE AmigaMail

@NODE MAIN "Developer Support Package / sana2device.doc"

@TOC "VIII-51/readme/MAIN"

TABLE OF CONTENTS

@{" sana2.device/AbortIO " link VIII-51-1}

@{" sana2.device/CloseDevice " link VIII-51-2}

@{" sana2.device/CMD_CLEAR " link VIII-51-3}

@{" sana2.device/CMD_FLUSH " link VIII-51-4}

@{" sana2.device/CMD_INVALID " link VIII-51-5}

@{" sana2.device/CMD_READ " link VIII-51-6}

@{" sana2.device/CMD_RESET " link VIII-51-7}

@{" sana2.device/CMD_START " link VIII-51-8}

@{" sana2.device/CMD_STOP " link VIII-51-9}

@{" sana2.device/CMD_UPDATE " link VIII-51-10}

@{" sana2.device/CMD_WRITE " link VIII-51-11}

@{" sana2.device/OpenDevice " link VIII-51-12}

@{" sana2.device/S2_ADDMULTICASTADDRESS " link VIII-51-13}

@{" sana2.device/S2_BROADCAST " link VIII-51-14}

@{" sana2.device/S2_CONFIGINTERFACE " link VIII-51-15}

@{" sana2.device/S2_DELMULTICASTADDRESS " link VIII-51-16}

@{" sana2.device/S2_DEVICEQUERY " link VIII-51-17}

@{" sana2.device/S2_GETGLOBALSTATS " link VIII-51-18}

@{" sana2.device/S2_GETSPECIALSTATS " link VIII-51-19}

@{" sana2.device/S2_GETSTATIONADDRESS " link VIII-51-20}

@{" sana2.device/S2_GETTYPESTATS " link VIII-51-21}

@{" sana2.device/S2_MULTICAST " link VIII-51-22}

@{" sana2.device/S2_OFFLINE " link VIII-51-23}

@{" sana2.device/S2_ONEVENT " link VIII-51-24}

@{" sana2.device/S2_ONLINE " link VIII-51-25}

@{" sana2.device/S2_READORPHAN " link VIII-51-26}

@{" sana2.device/S2_TRACKTYPE " link VIII-51-27}

@{" sana2.device/S2_UNTRACKTYPE " link VIII-51-28}

@ENDNODE

@NODE VIII-51-1 "sana2.device/AbortIO"

NAME

AbortIO -- Remove an existing device request.

SYNOPSIS

error = AbortIO(Sana2Req)

D0              A1

LONG AbortIO(struct IOSana2Req *);

FUNCTION

This is an exec.library call.

This function aborts an ioRequest. If the request is active, it may or

may not be aborted. If the request is queued it is removed. The

request will be returned in the same way as if it had normally

completed.  You must WaitIO() after AbortIO() for the request to

return.

INPUTS

Sana2Req        - Sana2Req to be aborted.

RESULTS

error           - Zero if the request was aborted, non-zero otherwise.

io_Error in Sana2Req will be set to IOERR_ABORTED

if it was aborted.

NOTES

SEE ALSO

exec.library/AbortIO(), exec.library/WaitIO()

BUGS

@ENDNODE

@NODE VIII-51-2 "sana2.device/CloseDevice"

NAME

CloseDevice -- Close the device.

SYNOPSIS

CloseDevice(Sana2Req)

A1

void CloseDevice(struct IOSana2Req *);

FUNCTION

This function is called by exec.library CloseDevice().

This function performs whatever cleanup is required at device closes.

Note that all IORequests MUST be complete before closing. If any are

pending, your program must AbortIO() then WaitIO() each outstanding

IORequest to complete them.

INPUTS

Sana2Req        - Pointer to IOSana2Req initialized by OpenDevice().

NOTES

SEE ALSO

exec.library/CloseDevice(), exec.library/OpenDevice()

BUGS

@ENDNODE

@NODE VIII-51-3 "sana2.device/CMD_CLEAR"

NAME

Clear -- Clear internal network interface read buffers.

FUNCTION

There are no device internal buffers, so CMD_CLEAR does not apply to

this class of device.

IO REQUEST

ios2_Command    - CMD_CLEAR.

RESULTS

ios2_Error      - IOERR_NOCMD.

NOTES

SEE ALSO

BUGS

@ENDNODE

@NODE VIII-51-4 "sana2.device/CMD_FLUSH"

NAME

Flush -- Clear all queued I/O requests for the SANA-II device.

FUNCTION

This command aborts all I/O requests in both the read and write

request queues of the device.  All pending I/O requests are

returned with an error message (IOERR_ABORTED).  CMD_FLUSH does not

affect active requests.

IO REQUEST

ios2_Command    - CMD_FLUSH.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

NOTES

SEE ALSO

BUGS

@ENDNODE

@NODE VIII-51-5 "sana2.device/CMD_INVALID"

NAME

Invalid -- Return with error IOERR_NOCMD.

FUNCTION

This command causes device driver to reply with an error IOERR_NOCMD

as defined in <exec/errors.h> indicating the command is not supported.

IO REQUEST

ios2_Command    - CMD_INVALID.

RESULTS

ios2_Error      - IOERR_NOCMD.

NOTES

SEE ALSO

BUGS

Not known to be useful.

@ENDNODE

@NODE VIII-51-6 "sana2.device/CMD_READ"

NAME

Read -- Get a packet from the network.

FUNCTION

Get the next packet available of the requested packet type. The data

copied (via a call to the requestor-provided CopyToBuffer function)

into ios2_Data is normally the Data Link Layer packet data only. If

bit SANA2IOB_RAW is set in ios2_Flags, then the entire physical frame

will be returned.

Unlike most Exec devices, SANA-II device drivers do not have internal

buffers.  If you wish to read data from a SANA-II device you should

have multiple CMD_READ requests pending at any given time.  The

functions provided by you the requestor will be used for any incoming

packets of the type you've requested.  If no read requests are

outstanding for a type which comes in and no read_orphan requests are

outstanding, the packet will be lost.

IO REQUEST

ios2_Command    - CMD_READ

ios2_Flags      - Supported flags are:

SANA2IOB_RAW

SANA2IOB_QUICK

ios2_PacketType - Packet type desired.

ios2_Data       - Abstract data structure to hold packet data.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

ios2_Flags      - The following flags may be returned:

SANA2IOB_RAW

SANA2IOB_BCAST

SANA2IOB_MCAST

ios2_SrcAddr    - Source interface address of packet.

ios2_DstAddr    - Destination interface address of packet.

ios2_DataLength - Length of packet data.

ios2_Data       - Abstract data structure which packet data is

contained in.

NOTES

The driver may not directly examine or modify anything pointed to by

ios2_Data.  It *must* use the requester-provided functions to access

this data.

SEE ALSO

S2_READORPHAN, CMD_WRITE, any_protocol/CopyToBuffer

BUGS

@ENDNODE

@NODE VIII-51-7 "sana2.device/CMD_RESET"

NAME

Reset -- Reset the network interface to initialized state.

FUNCTION

Currently, SANA-II devices can only be configured once (with

CMD_CONFIGINTERFACE) and cannot be re-configured, hence,

CMD_RESET does not apply to this class of device.

IO REQUEST

ios2_Command    - CMD_RESET.

RESULTS

ios2_Error      - IOERR_NOCMD.

NOTES

SEE ALSO

BUGS

@ENDNODE

@NODE VIII-51-8 "sana2.device/CMD_START"

NAME

Start -- Restart device operation.

FUNCTION

There is no way for the driver to keep queuing requests without

servicing them, so CMD_STOP does not apply to this class of device.

S2_OFFLINE and S2_ONLINE do perform a similar function to CMD_STOP

and CMD_START

IO REQUEST

ios2_Command    - CMD_START.

RESULTS

ios2_Error      - IOERR_NOCMD.

NOTES

SEE ALSO

S2_ONLINE, S2_OFFLINE

BUGS

@ENDNODE

@NODE VIII-51-9 "sana2.device/CMD_STOP"

NAME

Stop -- Pause device operation.

FUNCTION

There is no way for the driver to keep queuing requests without

servicing them, so CMD_STOP does not apply to this class of device.

S2_OFFLINE and S2_ONLINE do perform a similar function to CMD_STOP

and CMD_START

IO REQUEST

ios2_Command    - CMD_STOP.

RESULTS

ios2_Error      - IOERR_NOCMD.

NOTES

SEE ALSO

S2_ONLINE, S2_OFFLINE

BUGS

@ENDNODE

@NODE VIII-51-10 "sana2.device/CMD_UPDATE"

NAME

Update -- Force packets out to device.

FUNCTION

Since there are no device internal buffers, CMD_UPDATE does not

apply to this class of device.

IO REQUEST

ios2_Command    - CMD_UPDATE.

RESULTS

ios2_Error      - IOERR_NOCMD.

NOTES

SEE ALSO

BUGS

@ENDNODE

@NODE VIII-51-11 "sana2.device/CMD_WRITE"

NAME

Write -- Send packet to the network.

FUNCTION

This command causes the packet to be sent to the specified network

interface. Normally, appropriate packet header and trailer information

will be added to the packet data when it is sent.  If bit SANA2IOB_RAW

is set in io_Flags, then the ios2_Data is assumed to contain an entire

physical frame and will be sent (copied to the wire via

CopyFromBuffer() unmodified.

Note that the device should not check to see if the destination

address is on the local hardware.  Network protocols should realize

that the packet has a local destination long before it gets to a

SANA-II driver.

IO REQUEST

ios2_Command    - CMD_WRITE.

ios2_Flags      - Supported flags are:

SANA2IOB_RAW

SANA2IOB_QUICK

ios2_PacketType - Packet type to send.

ios2_DstAddr    - Destination interface address for this packet.

ios2_DataLength - Length of the Data to be sent.

ios2_Data       - Abstract data structure which packet data is

contained in.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

The driver may not directly examine or modify anything pointed to by

ios2_Data.  It *must* use the requester-provided functions to access

this data.

SEE ALSO

CMD_READ, S2_BROADCAST, S2_MULTICAST, any_protocol/CopyFromBuffer

BUGS

@ENDNODE

@NODE VIII-51-12 "sana2.device/OpenDevice"

NAME

Open -- Request an opening of the network device.

SYNOPSIS

error = OpenDevice(unit, IOSana2Req, flags)

D0                 D0    A1          D1

BYTE OpenDevice(ULONG, struct IOSana2Req *, ULONG);

FUNCTION

This function is called by exec.library OpenDevice().

This function performs whatever initialization is required per

device open and initializes the Sana2Req for use by the

device.

INPUTS

unit            - Device unit to open.

Sana2Req        - Pointer to IOSana2Req structure to be initialized by

the sana2.device.

flags           - Supported flags are:

SANA2OPB_MINE

SANA2OPB_PROM

ios2_BufferManagement   - A pointer to a tag list containing

pointers to buffer management functions.

RESULTS

error           - same as io_Error

io_Error        - Zero if successful; non-zero otherwise.

io_Device       - A pointer to whatever device will handle the calls

for this unit.  This pointer may be different

depending on what unit is requested.

ios2_BufferManagement   - A pointer to device internal information

used to call buffer management functions.

NOTES

SEE ALSO

exec.library/OpenDevice(), exec.library/CloseDevice()

BUGS

@ENDNODE

@NODE VIII-51-13 "sana2.device/S2_ADDMULTICASTADDRESS"

NAME

AddMulticastAddress -- Enable an interface multicast address.

FUNCTION

This command causes the device driver to enable multicast packet

reception for the requested address.

IO REQUEST

ios2_Command    - S2_ADDMULTICASTADDRESS.

ios2_SrcAddr    - Multicast address to enable.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

Multicast addresses are added globally -- anyone using the device

may receive packets as a result of any multicast address which has

been added for the device.

Since multicast addresses are not "bound" to a particular packet type,

each enabled multicast address has an "enabled" count associated with

it so that if two protocols add the same multicast address and later

one removes it, it is still enabled until the second removes it.

SEE ALSO

S2_MULTICAST, S2_DELMULTICASTADDRESS

BUGS

@ENDNODE

@NODE VIII-51-14 "sana2.device/S2_BROADCAST"

NAME

Broadcast -- Broadcast a packet on network.

FUNCTION

This command works the same as CMD_WRITE except that it also performs

whatever special processing of the packet is required to do a

broadcast send. The actual broadcast mechanism is neccessarily

network/interface/device specific.

IO REQUEST

ios2_Command    - S2_BROADCAST.

ios2_Flags      - Supported flags are:

SANA2IOB_RAW

SANA2IOB_QUICK

ios2_PacketType - Packet type to send.

ios2_DataLength - Length of the Data to be sent.

ios2_Data       - Abstract data structure which packet data is

contained in.

RESULTS

ios2_DstAddr    - The contents of this field are to be

considered trash upon return of the IOReq.

ios2_Error      - Zero if successful; non-zero otherwise.

This command can fail for many reasons and

is not supported by all networks and/or

network interfaces.

ios2_WireError  - More specific error number.

NOTES

The DstAddr field may be trashed by the driver because this function

may be implemented by filling DstAddr with a broadcast address and

internally calling CMD_WRITE.

SEE ALSO

CMD_WRITE, S2_MULTICAST

BUGS

@ENDNODE

@NODE VIII-51-15 "sana2.device/S2_CONFIGINTERFACE"

NAME

ConfigInterface -- Configure the network interface.

FUNCTION

This command causes the device driver to initialize the interface

hardware and to set the network interface address to the address in

ios2_SrcAddr. This command can only be executed once and, if

successful, will leave the driver and network interface fully

operational and the network interface in ios2_SrcAddr.

To set the interface address to the factory address, the network

management software must use GetStationAddress first and then call

ConfigInterface with the result. If there is no factory address then

the network software must pick an address to use.

Until this command is executed the device will not listen for any

packets on the hardware.

IO REQUEST

ios2_Command    - S2_CONFIGINTERFACE.

ios2_Flags      - Supported flags are:

SANA2IOB_QUICK

ios2_SrcAddr    - Address for this interface.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

ios2_SrcAddr    - Address of this interface as configured.

NOTES

Some networks have the interfaces choose a currently unused interface

address each time the interface is initialized. The caller must check

ios2_SrcAddr for the actual interface address after configuring the

interface.

SEE ALSO

S2_GETSTATIONADDRESS

BUGS

@ENDNODE

@NODE VIII-51-16 "sana2.device/S2_DELMULTICASTADDRESS"

NAME

DelMultiCastAddress -- Disable an interface multicast address.

FUNCTION

This command causes device driver to disable multicast packet

reception for the requested address.

It is an error to disable a multicast address that is not enabled.

IO REQUEST

ios2_Command    - S2_DELMULTICASTADDRESS

ios2_SrcAddr    - Multicast address to disable.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

Multicast addresses are added globally -- anyone using the device

may receive packets as a result of any multicast address which has

been added for the device.

Since multicast addresses are not "bound" to a particular packet type,

each enabled multicast address has an "enabled" count associated with

it so that if two protocols add the same multicast address and later

one removes it, it is still enabled until the second removes it.

SEE ALSO

S2_ADDMULTICASTADDRESS

BUGS

@ENDNODE

@NODE VIII-51-17 "sana2.device/S2_DEVICEQUERY"

NAME

DeviceQuery -- Return parameters for this network interface.

FUNCTION

This command causes the device driver to report information about the

device. Up to SizeAvailable bytes of the information is copied

into a buffer pointed to by ios2_StatData. The format of the data is

as follows:

struct Sana2DeviceQuery

{

/*

** Standard information

*/

ULONG SizeAvailble; /* bytes available */

ULONG SizeSupplied; /* bytes supplied */

LONG  DevQueryFormat;   /* this is type 0 */

LONG  DeviceLevel;      /* this document is level 0 */

/*

** Common information

*/

UWORD AddrFieldSize;    /* address size in bits */

ULONG MTU;              /* maximum packet data size */

LONG  bps;              /* line rate (bits/sec) */

LONG  HardwareType;     /* what the wire is */

/*

** Format specific information

*/

};

The SizeAvailable specifies the number of bytes that the caller

is prepared to accomodate, including the standard information fields.

SizeSupplied is the number of bytes actually supplied,

including the standard information fields, which will not exceed

SizeAvailable.

<devices/sana2.h> includes constants for these values.  If your

hardware does not have a number assigned to it, you must contact

CATS to get a hardware number.

IO REQUEST

ios2_Command    - S2_DEVICEQUERY.

ios2_StatData   - Pointer to Sana2DeviceQuery structure to fill in.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

SEE ALSO

BUGS

@ENDNODE

@NODE VIII-51-18 "sana2.device/S2_GETGLOBALSTATS"

NAME

GetGlobalStats -- Get interface accumulated statistics.

FUNCTION

This command causes the device driver to retrieve various global

runtime statistics for this network interface. The format of the

data returned is as follows:

struct Sana2DeviceStats

{

ULONG PacketsReceived;

ULONG PacketsSent;

ULONG BadData;

ULONG Overruns;

ULONG UnknownTypesReceived;

ULONG Reconfigurations;

timeval LastStart;

};

IO REQUEST

ios2_Command    - S2_GETGLOBALSTATS.

ios2_StatData   - Pointer to Sana2DeviceStats structure to fill.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

SEE ALSO

S2_GETSPECIALSTATS

BUGS

@ENDNODE

@NODE VIII-51-19 "sana2.device/S2_GETSPECIALSTATS"

NAME

GetSpecialStats -- Get network type specific statistics.

FUNCTION

This function returns statistics which are specific to the type of

network medium this driver controls. For example, this command could

return statistics common to all Ethernets which are not common to all

network mediums in general.

The supplied Sana2SpecialStatData structure is given below:

struct Sana2SpecialStatData

{

ULONG RecordCountMax;

ULONG RecordCountSupplied;

struct Sana2StatRecord[RecordCountMax];

};

The format of the data returned is:

struct Sana2StatRecord

{

ULONG Type;     /* commodore registered */

LONG Count;     /* the stat itself */

char *String;   /* null terminated */

};

The RecordCountMax field specifies the number of records that the

caller is prepared to accomodate.

RecordCountSupplied is the number of record actually supplied which

will not exceed RecordCountMax.

IO REQUEST

ios2_Command    - S2_GETSPECIALSTATS.

ios2_StatData   - Pointer to a Sana2SpecialStatData structure to fill.

RecordCountMax must be initialized.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

Commodore will maintain registered statistic Types.

SEE ALSO

S2_GETGLOBALSTATS, <devices/sana2specialstats.h>

BUGS

@ENDNODE

@NODE VIII-51-20 "sana2.device/S2_GETSTATIONADDRESS"

NAME

GetStationAddress -- Get default and interface address.

FUNCTION

This command causes the device driver to copy the current interface

address into ios2_SrcAddr, and to copy the factory default station

address (if any) into ios2_DstAddr.

IO REQUEST

ios2_Command    - S2_GETSTATIONADDRESS.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

ios2_SrcAddr    - Current interface address.

ios2_DstAddr    - Default interface address (if any).

NOTES

SEE ALSO

S2_CONFIGINTERFACE

BUGS

@ENDNODE

@NODE VIII-51-21 "sana2.device/S2_GETTYPESTATS"

NAME

GetTypeStats -- Get accumulated type specific statistics.

FUNCTION

This command causes the device driver to retrieve various packet type

specific runtime statistics for this network interface. The format of

the data returned is as follows:

struct Sana2TypeStatData

{

LONG PacketsSent;

LONG PacketsReceived;

LONG BytesSent;

LONG BytesReceived;

LONG PacketsDropped;

};

IO REQUEST

ios2_Command    - S2_GETTYPESTATS.

ios2_PacketType - Packet type of interest.

ios2_StatData   - Pointer to TypeStatData structure to fill in.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

Statistics for a particular packet type are only available while that

packet type is being ``tracked''.

SEE ALSO

S2_TRACKTYPE, S2_UNTRACKTYPE

BUGS

@ENDNODE

@NODE VIII-51-22 "sana2.device/S2_MULTICAST"

NAME

Multicast -- Multicast a packet on network.

FUNCTION

This command works the same as CMD_WRITE except that it also performs

whatever special processing of the packet is required to do a

multicast send. The actual multicast mechanism is neccessarily

network/interface/device specific.

IO REQUEST

ios2_Command    - S2_MULTICAST.

ios2_Flags      - Supported flags are:

SANA2IOB_RAW

SANA2IOB_QUICK

ios2_PacketType - Packet type to send.

ios2_DstAddr    - Destination interface address for this packet.

ios2_DataLength - Length of the Data to be sent.

ios2_Data       - Abstract data structure which packet data is

contained in.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

This command can fail for many reasons and

is not supported by all networks and/or

network interfaces.

ios2_WireError  - More specific error number.

NOTES

The address supplied in ios2_DstAddr will be sanity checked (if

possible) by the driver. If the supplied address fails this sanity

check, the multicast request will fail immediately with ios2_Error

set to S2WERR_BAD_MULTICAST.

Another Amiga will not receive a multicast packet unless it has had

the particular multicast address being used S2_ADDMULTICASTADRESS'd.

SEE ALSO

CMD_WRITE, S2_BROADCAST, S2_ADDMULTICASTADDRESS

BUGS

@ENDNODE

@NODE VIII-51-23 "sana2.device/S2_OFFLINE"

NAME

Offline -- Remove interface from service.

FUNCTION

This command removes a network interface from service.

IO REQUEST

ios2_Command    - S2_OFFLINE.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

Aborts all pending reads and writes with ios2_Error set to

S2ERR_OUTOFSERVICE.

While the interface is offline, all read, writes and any other

command that touches interface hardware will be rejected with

ios2_Error set to S2ERR_OUTOFSERVICE.

This command is intended to permit a network interface to be

tested on an otherwise live system.

SEE ALSO

S2_ONLINE

BUGS

@ENDNODE

@NODE VIII-51-24 "sana2.device/S2_ONEVENT"

NAME

OnEvent -- Return when specified event occures.

FUNCTION

This command returns when a particular event condition has occured

on the network or this network interface.

IO REQUEST

ios2_Command    - S2_ONEVENT.

ios2_Flags      - Supported flags are:

SANA2IOB_QUICK

ios2_WireError  - Mask of event(s) to wait for

(from <devices/sana2.h>).

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - Mask of events that occured.

NOTES

If this device driver does not understand the specified event

condition(s) then the command returns immediately with

ios2_Req.io_Error set to S2_ERR_NOT_SUPPORTED and ios2_WireError

S2WERR_BAD_EVENT.  A successful return will have ios2_Error set to

zero ios2_WireError set to the event number.

All pending requests for a particular event will be returned when

that event occurs.

All event types that cover a particular condition are returned when

that condition occures. For instance, if an error is returned by

a buffer management function during receive processing, events of

types S2EVENT_ERROR, S2EVENT_RX and S2EVENT_BUFF would be returned if

pending.

Types ONLINE and OFFLINE return immediately if the device is

already in the state to be waited for.

SEE ALSO

BUGS

@ENDNODE

@NODE VIII-51-25 "sana2.device/S2_ONLINE"

NAME

Online -- Put a network interface back in service.

FUNCTION

This command places an offline network interface back into service.

IO REQUEST

ios2_Command    - S2_ONLINE.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

This command is responsible for putting the network interface

hardware back into a known state (as close as possible to the

state before S2_OFFLINE) and resets the unit global and special

statistics.

SEE ALSO

S2_OFFLINE

BUGS

@ENDNODE

@NODE VIII-51-26 "sana2.device/S2_READORPHAN"

NAME

ReadOrphan -- Get a packet for which there is no reader.

FUNCTION

Get the next packet available that does not satisfy any then-pending

CMD_READ requests. The data returned in the ios2_Data structure is

normally the Data Link Layer packet type field and the packet data. If

bit SANA2IOB_RAW is set in ios2_Flags, then the entire Data Link Layer

packet, including both header and trailer information, will be

returned.

IO REQUEST

ios2_Command    - CMD_READORPHAN.

ios2_Flags      - Supported flags are:

SANA2IOB_RAW

SANA2IOB_QUICK

ios2_DataLength - Length of the Data to be sent.

ios2_Data       - Abstract data structure which packet data is

contained in.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

ios2_Flags      - The following flags may be returned:

SANA2IOB_RAW

SANA2IOB_BCAST

SANA2IOB_MCAST

ios2_SrcAddr    - Source interface address of packet.

ios2_DstAddr    - Destination interface address of packet.

ios2_DataLength - Length of the Data to be sent.

ios2_Data       - Abstract data structure which packet data is

contained in.

NOTES

This is intended for debugging and management tools.  Protocols should

not use this.

As with 802.3 packets on an ethernet, to determine which protocol

family the returned packet belongs to you may have to specify

SANA2IOB_RAW to get the entire data link layer wrapper (which is where

the protocol type may be kept). Notice this necessarily means that

this cannot be done in a network interface independent fashion.  The

driver will, however, fill in the PacketType field to the best of its

ability.

SEE ALSO

CMD_READ, CMD_WRITE

BUGS

@ENDNODE

@NODE VIII-51-27 "sana2.device/S2_TRACKTYPE"

NAME

TrackType -- Accumulate statistics about a packet type.

FUNCTION

This command causes the device driver to accumulate statistics about

a particular packet type. Packet type statistics, for the particular

packet type, are zeroed by this command.

IO REQUEST

ios2_Command    - S2_TRACKTYPE.

ios2_PacketType - Packet type of interest.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

SEE ALSO

S2_UNTRACKTYPE, S2_GETTYPESTATS

BUGS

@ENDNODE

@NODE VIII-51-28 "sana2.device/S2_UNTRACKTYPE"

NAME

UntrackType -- End statistics about a packet type.

FUNCTION

This command causes the device driver to stop accumulating

statistics about a particular packet type.

IO REQUEST

ios2_Command    - S2_UNTRACKTYPE.

ios2_PacketType - Packet type of interest.

RESULTS

ios2_Error      - Zero if successful; non-zero otherwise.

ios2_WireError  - More specific error number.

NOTES

SEE ALSO

S2_TRACKTYPE, S2_GETTYPESTATS

BUGS

@ENDNODE