TABLE OF CONTENTS

cgxvideo.library/--background--

cgxvideo.library/AttachVLayerTagList

cgxvideo.library/CreateVLayerHandleTagList

cgxvideo.library/DeleteVLayerHandle

cgxvideo.library/DetachVLayer

cgxvideo.library/GetVLayerAttr

cgxvideo.library/LockVLayer

cgxvideo.library/SetVLayerAttrTagList

cgxvideo.library/UnLockVLayer

cgxvideo.library/--background--                cgxvideo.library/--background--

PURPOSE

cgxvideo.library allows you to use gfx card's video overlay features

in your applications. This library is part of the CyberGraphX V3/V4

package. As of now, only the CyberVision64/3D graphics board supports

video overlay features.

No calls have been added for the V4 (v42) release. You can open v42

of cgxvideo.library to be sure that you open on a v4 CyberGraphX

system but v41 is required to allow your application to run with

CyberGraphX V3 also.

SUPPORTED FORMATS

Currently, cgxvideo.library supports 4 different color space formats,

which all use 2 bytes per pixel. The data can be passed through

unscaled or scaled up horizontally and vertically by an arbitrary

amount. YCbCr/YUV data is color space converted and all data is

converted to RGB-24 (8.8.8) format.

SRCFMT_YUV16

This format displays data that is in YUV-16 (4.2.2) video data format.

Input range is 0-255 for every component. Every pixel data word

contains at least one Y value in the highbyte, followed by one U value

in the lowbyte for each even and one V value in the lowbyte for each

odd data word. Since the SRCFMT_YCbCr16 format is more common on

various graphics cards, it is recommended to avoid this format and use

SRCFMT_YCbCr16 instead.

Pixel 0           Pixel 1

YYYYYYYYUUUUUUUU YYYYYYYYVVVVVVVV ........

7654321076543210 7654321076543210

SRCFMT_YCbCr16

This format displays data that is in YCbCr-16 (4.2.2) video format.

The Y-, U, and V-data are in the range specified by CCIR-601-2.

Y ranges from 16 (black) to 235 (white). U and V range from 16

to 240 (128 means 0 or no color, so the range is -112 to 112

actually).

Pixel 0           Pixel 1

YYYYYYYYUUUUUUUU YYYYYYYYVVVVVVVV ........

7654321076543210 7654321076543210

SRCFMT_RGB15PC

Source data uses 2 bytes/pixel (5.5.5). Each component uses 5 Bits,

the most significant bit is unused. For historical reasons, this format

is byte swapped.

Pixel 0           Pixel 1

GGGBBBBB0RRRRRGG GGGBBBBB0RRRRRGG ........

21043210-4321043 21043210-4321043

SRCFMT_RGB16PC

Source data uses 2 bytes/pixel (5.6.5). red and blue component uses

5 Bits, green component uses 6bits. For historical reasons, this format

is byte swapped.

Pixel 0           Pixel 1

GGGBBBBBRRRRRGGG GGGBBBBBRRRRRGGG ........

2104321043210543 2104321043210543

EXAMPLE

{

APTR VLHandle;

// init code goes here ......

.

.

.

if(VLHandle = CreateVLayerHandleTags(screen, VOA_SrcType,SRCFMT_YUV,

VOA_SrcWidth, 160,

VOA_SrcHeight, 120,

TAG_DONE))

{

if(!AttachVLayerTags(VLHandle, window, TAG_DONE))

{

if(LockVLayer(VLHandle))

{

UWORD *srcdata;

srcdata = GetVLayerAttr(VLHandle, VOA_BaseAddress);

// modify source data here

.

.

.

UnLockVLayer(VLHandle);

}

else

{

// Unable to lock VLayer

}

DetachVLayer(VLHandle);

}

else

{

// Unable to attach vlayer to window

}

DeleteVLayerHandle(VLHandle);

}

}

cgxvideo.library/AttachVLayerTagList      cgxvideo.library/AttachVLayerTagList

NAME

AttachVLayerTagList -- attach a created video layer to a given window

AttachVLayerTags -- varargs stub for AttachVLayerTagList

SYNOPSIS

result = AttachVLayerTagList( VLayerHandle, Window, TagItems)

D0                              A0           A1      A2

ULONG AttachVLayerTagList(struct VLayerHandle *,struct Window *,

struct TagItem *)

result = AttachVLayerTags(VLayerHandle, Window,

Tag1, ... );

ULONG AttachVLayerTags(struct VLayerHandle *,struct Window *,Tag Tag1,

...);

FUNCTION

Attaches the previously created videolayer handle to a given window.

As a result, the video overlay should now be linked to the window.

If the window is moved or resized, the overlay is moved or resized,

too.

INPUTS

VLayerHandle - pointer to a previously created videolayer handle

Window - pointer to a window that the videolayer should be attached to

TagItems - pointer to an optional tag list

TAGS

Tags available are:

VOA_LeftIndent (ULONG) - additional offset from the left window border

VOA_RightIndent (ULONG) - additional offset from the right window border

VOA_TopIndent (ULONG) - additional offset from the top window border

VOA_BottomIndent (ULONG) - additional offset from the bottom window border

RESULT

result - 0 if videolayer could be attached to the window

SEE ALSO

DetachVLayer()

cgxvideo.library/CreateVLayerHandleTagList  cgxvideo/CreateVLayerHandleTagList

NAME

CreateVLayerHandleTagList -- create a video layer handle

CreateVLayerHandleTags -- varargs stub for CreateVLayerHandleTagList

SYNOPSIS

VLayerHandle = CreateVLayerHandleTagList( Screen, TagItems)

D0                                           A0     A1

struct VLayerHandle *CreateVLayerHandleTagList(struct Screen *,

struct TagItem *)

VLayerHandle = CreateVLayerHandleTags(Screen, Tag1, ... );

struct VLayerHandle *CreateVLayerHandleTags(struct Screen *,Tag Tag1,

...);

FUNCTION

Creates a video layer handle for the given screen ?

INPUTS

VLayerHandle - pointer to a previously created videolayer handle

Window - pointer to a window that the videolayer should be attached to

TagItems - pointer to an optional tag list

TAGS

Tags available are:

VOA_SrcType (ULONG) - specifies source type that is used for video

overlay data

Currently supported formats:

SRCFMT_YUV16 (not recommended, use YCbCr16 instead)

SRCFMT_YCbCr16

SRCFMT_RGB15PC

SRCFMT_RGB16PC

VOA_SrcWidth (ULONG) - source width in pixel units

VOA_SrcHeight (ULONG) -  source height in pixel units

VOA_Error (ULONG *) - If you specify VOA_Error with ti_Data pointing

to an ULONG, you will get more detailed information

if the creation of the video layer handle fails

VOA_UseColorKey (BOOL) - If you specify VOA_UseColorKey as TRUE, color

keying is enabled for the video layer. A

certain color key is generated then and the

stream data is only visible where this color

could be found.

VOA_UseBackFill (BOOL) - If you specify VOA_UseBackFill as TRUE automatic

backfilling for the videolayer is enabled. This

option is only available if color keying is

enabled.

RESULT

VLayerHandle - pointer to the created videolayer handle or 0

SEE ALSO

DeleteVLayerHandle()

cgxvideo.library/DeleteVLayerHandle        cgxvideo.library/DeleteVLayerHandle

NAME

DeleteVLayerHandle -- delete a created video layer handle

SYNOPSIS

DeleteVLayerHandle( VLayerHandle )

A0

void DeleteVLayerHandle(struct VLayerHandle *)

FUNCTION

Deletes a created video layer handle

INPUTS

VLayerHandle - pointer to a previously created videolayer handle

RESULT

none

SEE ALSO

CreateVLayerHandle()

cgxvideo.library/DetachVLayer                    cgxvideo.library/DetachVLayer

NAME

DetachVLayer -- detach a video layer from a window

SYNOPSIS

result = DetachVLayer( VLayerHandle)

D0                          A0

ULONG DetachVLayer(struct VLayerHandle *)

result = DetachVLayer(VLayerHandle);

FUNCTION

Detaches a videolayer from a given window. As a result, the video

overlay should now be unlinked from the window and the original

contents of the window are visible now.

INPUTS

VLayerHandle - pointer to a previously created videolayer handle

RESULT

result - 0 if videolayer could be detached from the window

SEE ALSO

AttachVLayerTagList()

cgxvideo.library/GetVLayerAttr                  cgxvideo.library/GetVLayerAttr

NAME

GetVLayerAttr -- get a certain attribute from a video layer

SYNOPSIS

value = GetVLayerAttr( VLayerHandle, AttrNum)

D0                       A0           D0

ULONG GetVLayerAttr(struct VLayerHandle *, ULONG AttrNum)

FUNCTION

Gets a certain attribute from a given video layer. You have to call

LockVLayer() to make sure that the result is valid !

INPUTS

VLayerHandle - pointer to a previously created videolayer handle

AttrNum - attribute that you want to get

ATTRIBUTES

Attributes available are:

VOA_BaseAddress - if this attribute is specified the base address for

the source data is returned

VOA_ColorKeyPen - returns the pen number used for color keying. If color

keying is not enabled, -1 is returned

VOA_ColorKey -    returns the 24 bit color value used for color keying.

If color keying is not enabled, -1 is returned.

RESULT

value - the value for the given attribute

SEE ALSO

SetVLayerAttr()

cgxvideo.library/LockVLayer                        cgxvideo.library/LockVLayer

NAME

LockVLayer -- locks the specified video layer to allow access to

source data

SYNOPSIS

result = LockVLayer( VLayerHandle )

D0                      A0

ULONG LockVLayer(struct VLayerHandle *)

FUNCTION

Locks the specified video layer to allow access to source data. Make

sure that you don't keep that lock for too long. It is only allowed

to keep it for a short time.

INPUTS

VLayerHandle - pointer to a previously created videolayer handle

RESULT

result - TRUE if video layer could be locked, FALSE otherwise

SEE ALSO

UnlockVLayer()

cgxvideo.library/SetVLayerAttrTagList    cgxvideo.library/SetVLayerAttrTagList

NAME

SetVLayerAttrTagList -- set certain attributes for a video layer

SetVLayerAttrTags -- varargs stub for SetVLayerAttrTagList

SYNOPSIS

SetVLayerAttrTagList( VLayerHandle, TagItems)

A0           A1

void SetVLayerAttrTagList(struct VLayerHandle *, struct TagItem *)

SetVLayerAttrTags(VLayerHandle, Tag1, ... );

void SetVLayerAttrTags(struct VLayerHandle *,,Tag Tag1, ...);

FUNCTION

Sets certain attributes for a given video layer

INPUTS

VLayerHandle - pointer to a previously created videolayer handle

TagItems - pointer to a tag list which contains attributes to be

modified

TAGS

Tags available are:

VOA_LeftIndent (ULONG) - additional offset from the left window

border

VOA_RightIndent (ULONG) - additional offset from the right window

border

VOA_TopIndent (ULONG) - additional offset from the top window border

VOA_BottomIndent (ULONG) - additional offset from the bottom window

border

RESULT

none

SEE ALSO

GetVLayerAttr()

cgxvideo.library/UnLockVLayer                    cgxvideo.library/UnLockVLayer

NAME

UnLockVLayer -- Unlocks a previously locked video layer

SYNOPSIS

UnLockVLayer( VLayerHandle )

A0

void UnLockVLayer(struct VLayerHandle *)

FUNCTION

Unlocks a previouly locked video layer

INPUTS

VLayerHandle - pointer to a previously created videolayer handle

RESULT

none

SEE ALSO

LockVLayer()