Callbak

v1.0

(c) 1996 Mark Pyne, Overflow Software

Introduction:

-------------

The purpose of this program, is to  allow a sysop to be able to share

a phone line for  two or three purposes. It allows  you to share  Voice, Data

and Fax with one phone line, and one modem. You define yourself what software

you wish to be run if a  data/fax call comes in, the  software can be Windows

or DOS based.

Callbak works using the  callback method, which means you  set either

Data/Fax or  Voice to be the default type of call, so if a caller rings for a

user definable  number of rings then it either loads your chosen software, or

shows as a  voice caller  depending on which  you set as  default. However if

the caller  is not  what the default  caller is set  to be, i.e. Default Call

is DATA, then  they must ring  once, hang up and  then 'Callback' within  the

user defined time limit and  the software will  just allow the phone  to keep

ringing with  a convenient  message, so that you know that it is a voice call

and can pickup the handset. It can be set either way around, so  you can have

Voice as  the default type of call  and Data as the 'Callback' option, or the

other way around. It is entirely upto you.

Configuration:

--------------

Whilst Callbak is  very configurable to  the  user, it has  also been

designed to be simple to setup, and example batch  files and detailed command

line help is all available with the program.

As  Callbak  uses  command line  configuration  options, rather  than

tedious configuration  files, you  must  edit the  CALLBACK.BAT to suit  your

setup  and  within a couple  of minutes, you can  have it all  up and running

smoothly. If  you have any  difficulty  getting the  program to  work, PLEASE

read through these steps very  carefully, and if it still  does not work then

my details are at the end of this document.

Step 1. Use a TEXT Editor to edit the file CALLBACK.BAT

Step 2.1 If your Com Port is installed on a standard IRQ and Base Address,

or you have no idea what I am talking about, then  continue here,

otherwise refer to Step 2.2.

Adjust the fourth line to suit, the example one looks like this:

CALLBAK 3 12 4 B A

In plain english, this is what the options are:

CALLBAK ComPort CallbackReset NumRingsBeforeDefault

DefCallType PhoneType

Step 2.2 If your Com Port  Base address or  IRQ is not  standard, then you

need to adjust the third line in CALLBACK.BAT to  suit your setup

like so, otherwise goto step 2.3. The example one follows:

CALLBAK 3 12 4 B A 3e8 5

In plain english, this is what the options are:

CALLBAK ComPort CallbackReset NumRingsBeforeDefault

DefCallType PhoneType ComBaseAddress ComIRQ

Step 2.3 Please note that ALL options must  have a number, other  than the

forth and  fifth options which  MUST have either the  letter A or

the letter B. These options mean the following:

ComPort is the ComPort which your modem is installed on.

-------

CallBackReset is the number  of seconds Callbak  waits  after

------------- detecting and losing  a call, before  resetting

to a  first  call again. The  caller must  ring

once, hang  up and  then callback  WITHIN  this

timelimit for it to be  classed as a  Callback.

So for example, if your default  caller is  set

as DATA, and  this  is set  to 20,  then  if  a

caller  rings, and  hangs  up he / she  has  20

seconds to  callback  and be passed  through to

voice. If they ring after 20  seconds then they

are  classed  as a  new caller, and  can either

ring once and hang up, and then have another 20

seconds  to  come through  as a voice  call, or

keep ringing  the number of  rings you set (see

next paragraph) to go through as a DATA Caller.

NumRingsBeforeDefault is the  Number of  Rings  it waits  for

--------------------- before passing through to your  default

type of  call. So for  example, if  you

have your Default  Type of Call  set as

DATA and  this  set to  4, then when  a

caller  rings, they  must  let it  ring

4 times before Callbak  exits and  runs

the defined program (step 3)

DefCallType is the Default Type of Call for a continuous ring

----------- as opposed to a Callback. The value can either be

A or  B. If it  is  set to  A, then Voice  is the

default  type  of  call, which  means  that  if a

caller rings in, and lets it  ring for the number

of rings you  set in  NumRingsBeforeDefault  then

the call is  classed as a  Voice Caller and  will

come up and tell you. If however you set it to B,

then  DATA  is the  default  type  of call, which

means that if a  caller rings  for the  number of

rings set in NumRingsBeforeDefault, then they are

classed as a  DATA Caller  and go through to  the

software  which you  setup later on in the  batch

file (step 3).

PhoneType is the setting which sets the  type of Phone system

--------- in use in your  country. This alters the timing and

listening delays which are very important. There is

currently  2  settings, A and B.  A for the  United

Kingdom and B for /some/ parts of the U.S.A. As all

countries vary, the only thing I can really suggest

is to try both  settings. If  your machine  detects

the difference between a Callback and  a continuous

ring, then you  have the right  setting. If neither

options  work, then  your Country/State is  using a

different timing  option, please write to me  and I

will get  your Phone  System included  in the  next

release (see end of document for support  details.)

BaseAddress is the base address of the Com Port. Please note,

----------- this should ONLY be used, if the program fails to

communicate with your  Com Port properly, or  you

are  using a  non-standard base  address for your

Com Port, otherwise LEAVE THIS BLANK!

ComIRQ is the IRQ of the Com Port.  Please note, this  should

------ ONLY be  used, if  the  program  fails to  communicate

with  your  Com Port  properly, or  you  are  using  a

non-standard  IRQ for  your Com Port, otherwise  LEAVE

THIS BLANK!

Step 3. Adjust the line  which suits, the  BBS/FAX Software which you  are

planning on using with Callbak, underneath the :DATA  label in the

CALLBACK.BAT batch file.

You will need to remove the REM from the appropriate  program, and

then adjust  the paths and command-line parameters. It may even be

that your  program is  not listed, but using the  samples in their

you should be able to get it going, failing that then feel free to

contact me, tell me the program name and EXE  filename, and I will

do my best to help.

For example, the one I use, is to load  Camelot (Excalibur) and it

looks like this:

start /w c:\excal\camelot.exe -p7 -q

The  START is a useful program  which comes  with Windows 95 / NT,

which allows you to start a Windows  Application from DOS  and the

/W is  a very important  feature, which stops Callbak from loading

before the user has gone. So, the start program runs  and executes

the CAMELOT program with the  -p7 and  -q command-line parameters.

The -p7  means  PORT ID 7 (as setup in Arthur Maintenance,  Ports)

and the -q means that after the caller has logged off, the program

exits and will end up reloading Callbak. You must also remember to

unrem the PROGRKIL line from the batch file. The  PROGRKIL program

is very  important  and  MUST be loaded  BEFORE  your  BBS program

(Camelot in the above example).

Another  example, is  RemoteAccess which  is run  through a  batch

file, this must be started like this:

CALL c:\ra\ra.bat

It HAS to be started with  'CALL' because you are  running a Batch

file from within a Batch  file. Your RA.BAT file  also needs to be

modified so  that once a user logs off, rather than  going back to

the beginning to reload RA, the batch file must change back to the

directory  that   CALLBACK.BAT   is  located  in,  and  then  just

terminate/finish, so  that control is given back  to CALLBACK.BAT.

Generally  speaking, your  are  best not to use  PROGRKIL with DOS

Programs that run under batch files, although it is up to you.

Step 4. Adjust the  PROGRKIL line  in the  CALLBACK.BAT. You need  to make

sure, that  this is  loading  BEFORE your  BBS Software, and  that

the commandline, is the  name of your BBS  softwares window title.

So for  example my  Excalibur  window for  Node 3, is  reported as

"COM3 - No User (Waiting Call)", so the command-line for  PROGRKIL

in CALLBACK.BAT would  be "PROGRKIL COM3 - No User (Waiting Call)"

in my case. The  PROGRKIL is a  program which kills  the  selected

program, if it is still active in ten minutes. This works, because

BBS softwares  Windows Title  is different  when  there is a  user

online to when there is not a user online. This means, that if the

modems  do not  connect properly, after  ten minutes  CallBak will

reload, however if theres caller online after 10 minutes then they

stay on the BBS, but when they leave, CallBak will reload (as long

as you have told the  BBS software to  exit after the  caller logs

off, this is done by -q  in Excalibur, and just  letting the batch

file finish  in cases of  DOS  BBS Software, and  so on for  other

software.)

Step 5. If you are running under Windows 3.1/ 3.11/ 95/ NT, then you  MUST

edit your SYSTEM.INI. Under the section  [386Enh], you need to add

the line  "ComxAutoAssign=0" (without the  quotes), and alter  the

x to your Com Port number, so if the modem is on Com2 for example,

then you  would add  "Com2AutoAssign=0". This line is  required if

you want to  be able to run both Dos  and Windows applications  on

the same Com Port, without rebooting.

Step 6. Configuration is now complete, all you need  to do is run CALLBACK

to  start  the  program  (restart  the  machine if  you  added the

ComxAutoAssign=0 line in SYSTEM.INI).

Compatability:

--------------

Callbak has been fully tested under the following  operating systems,

modems, and fossil drivers by either myself or the team of Beta Testers.

* Operating Systems

- Windows 95 v4.00.950a

- MSDos v6.2

- MSDos v6.22

* Fossil Drivers

- WinFossil 95 v1.12

- None

- X00 v1.53

* BBS Software / Mailers

- Beemail Wildbee 0.93.212

- FrontDoor v2.12sw

- Excalibur v1.52

- RemoteAccess v2.50+

* Modems

- US Robotics 33k6 External Voice Mail Sportster

- US Robotics 33k6 External V.Everything Courier

- Supra 33k6 External Plug and Play

- Unbranded 33k6 Internal Voice Mail

- Unbranded 33k6 External Voice Mail

- Trust 14k4 Internal

* Countries Phone Systems

- United Kingdom

- United States of America (Various States)

- Italy

Problems:

---------

List below is common  problems that  people encounter, along with the

solutions. If your  problem is  not listed, or  the answer does not solve  it

then please  contact me on any  method listed at  the bottom of the  document

under the 'Support' header.

Q) When CallBak switches to my Windows software (Excalibur/Beemail/etc), it

cannot  open  the  Com Port. Excalibur  says, [(***3) - No user (Waiting

Call))].

A) You need to edit your  SYSTEM.INI, under the section  [386Enh], you need

to add the line  "ComxAutoAssign=0" (without the  quotes), and alter the

x to your Com Port number, so if the modem is on Com 2 for example, then

you would add  "Com2AutoAssign=0". Be sure to reboot your machine after.

Q) When running PROGRKIL I get a message  saying that Windows  is required,

and/or it loads windows.

A) Yes, PROGRKIL is only needed for users who are using Windows, whether it

be for DOS or Windows BBS software is irelevent. If you are getting this

message, then I assume you are running under a DOS system, and so should

edit  your  CALLBACK.BAT  and  remove all  references  to  the  PROGRKIL

program.

Q) The display says Caller Gone when the phones still ringing.

A) The wrong phone setting has been chosen for your Phone Network. You need

to alter the 'PhoneType'  setting in the  command-line  in CALLBACK.BAT.

If both  A  and  B  have  the same  effect, then  please  contact me via

E-Mail, and  I  will  add the  specifications  needed  for  it  to  work

correctly in your country and send you the latest version.

Registration:

-------------

This product is  shareware, and  you may  use it for upto 30  days to

evaluate it. After  this time, if  you  still wish to continue  using Callbak

then you are required to register Callbak for the equivalent to 10 UK Pounds.

Registration costs 10 and entitles you to ALL future updates. Please

consider  registering  this product, as 10  for a  lifetime  program is much

cheaper  than the  installation of  an  extra line, and the  quarterly rental

charges.

There is a file called REGSITE.TXT included  with this archive, which

contains all the Registration sites  around the world where you  may register

this program. Please fill in  REGFORM.TXT, and  then send it to your  nearest

registration site that is listed in REGSITE.TXT. If there is no  registration

site for your  country, then please send it  to myself at the  United Kingdom

address.

Support:

--------

If you experience any problems at all using this package, then please

get in contact with me, and I will help you as much as possible. I will  help

those who  have and who  have not  registered  Callbak! and ideas for  future

releases are welcomed. Questions/Help  should be sent to me  using one of the

below methods:

E-Mail:  software@overflow.com

overflow@ndirect.co.uk

Fidonet:  2:2503/313,  2:2503/315

BBS: +44 (0)116 2711846 (RemoteAccess + Excalibur)

+44 (0)116 2714017 (RemoteAccess + Excalibur)

Voice: +44 (0)116 2718335