.pl

000000 00   00 0000000

0 00 0 00   00  00   0

00   00   00  00

00   0000000  0000

00   00   00  00

00   00   00  00   0

0000  00   00 0000000

000000   00000  00   00   0000  00   00   00000

00 000 00   00  00 00     00   000  00  00   0

00  00 00   00   000      00   0000 00 00

00000  00   00   000      00   0000000 00

00  00 00   00  00 00     00   00 0000 00  000

00 000 00   00 00   00    00   00  000  00  00

000000   00000  00   00   0000  00   00   00000

000000   00000   00000  000  00

00 000 00   00 00   00  00  00

00  00 00   00 00   00  00 00

00000  00   00 00   00  0000

00  00 00   00 00   00  0000

00 000 00   00 00   00  00 00

000000   00000   00000  000  00

Courtesy of M & M Enterprises

(c) 1990

_______________________________________________________________________

Copyright 1990

Table of Contents

_______________________________________________________________________

Chapter 1 Introduction . . . . . . . . . . . . . . . . . 1

Chapter 2 Tutorials

Electronic Toll Fraud Devices (Lex Luthor). . . . . 2

Boxing Basics (Dr. Pepper). . . . . . . . . . . . . 5

The Blue Box and Ma Bell (Herb Friedman). . . . . . 7

How Ma Bell Catches Blue Boxers (Lex Luthor). . . .14

Dealing With The Rate & Route Operator

(Fred Steinbeck). . . . . . . . . . . . .17

Step by Step Switching Notes (Phantom Phreaker) . .18

Verification (Fred Steinbeck) . . . . . . . . . . .23

Chapter 3 International Dialing

International Dialing (Dr. Jimmy & Mr. Jim) . . . .26

Overseas Boxing (King Blotto) . . . . . . . . . . .28

The History of British Phreaking (Lex Luthor) . . .29

Chapter 4 The Boxes

Aqua Box (Kopy Kat) . . . . . . . . . . . . . . . .34

Beige Box (The Exterminator). . . . . . . . . . . .36

Beige Box (Cardiac Arrest). . . . . . . . . . . . .39

Beige Box (Shooting Shark). . . . . . . . . . . . .44

Black Box (Uncle Joe) . . . . . . . . . . . . . . .45

Black Box (from Sherwood Forest). . . . . . . . . .46

Blue Box (Jester Sluggo). . . . . . . . . . . . . .48

Brown Box (Author Unknown). . . . . . . . . . . . .56

CheeseBox (Sir Knight). . . . . . . . . . . . . . .57

Clear Box (The Bit of the Postman). . . . . . . . .58

Gold Box (Dr. Revenge). . . . . . . . . . . . . . .59

Green Box (BIOC Agent 006). . . . . . . . . . . . .61

Mauve Box (Captain Generic) . . . . . . . . . . . .61

Olive Box (Arnold). . . . . . . . . . . . . . . . .63

Pearl Box (Dr. D-Code). . . . . . . . . . . . . . .64

Pearl Box (Dispater). . . . . . . . . . . . . . . .66

Pink Box (Baba O'Riley) . . . . . . . . . . . . . .68

Red Box (Pink Panther). . . . . . . . . . . . . . .69

Silver Box (Silent Screen). . . . . . . . . . . . .71

Silver Box & White Box (Explorer) . . . . . . . . .73

Switch Box (Autopsy Saw). . . . . . . . . . . . . .76

Tan Box (Samurai Cat) . . . . . . . . . . . . . . .79

Urine Box (Wolfgang von Albatros) . . . . . . . . .80

White Box (Uncle Joe) . . . . . . . . . . . . . . .82

Appendices

A    Miscellaneous Boxing Notes. . . . . . . . . . . . .84

B    Line Signals in CCIT Systems (Sherwood Forest II) .88

C    Various Tone Frequencies. . . . . . . . . . . . . .89

D    Trunking Off of 911 (The Conflict). . . . . . . . .90

E    International Dialing Codes . . . . . . . . . . . .91

_______________________________________________________________________

M & M Enterprises

_______________________________________________________________________

_______________________________________________________________________

Copyright 1990

Introduction

_______________________________________________________________________

Chapter 1

Introduction

------------

Well folks, here is another offering from the never idle

keyboards of M & M Enterprises. I hope that at least one or two

of the items herein can be of use.

This document was created for one main purpose..to clean up

my records. Having 500 pages of numerous texts on boxing, some

one or two pages in length and others much longer, I decided to

shift through all of them and get rid of the duplicates and

obviously useless ones. This got the pile down to 200 pages. Now

came the fun part..editing the remainder. Some were done in 40

columns, others in all CAPS, and some could be combined with a

paragraph or two from other similiar files. What is left is

contained in this far shorter document.

I have attempted to correct some of the obvious errors as

far as electronics design, but since my experience is solely in

the Blue Box area, you will have to take those articles at face

value. Good luck and keep the faith- Blue Boxing is not totally

dead!

_______________________________________________________________________

M & M Enterprises

2                                               The Boxing Book

_______________________________________________________________________

Chapter 2

Tutorials

---------

ELECTRONIC TOLL FRAUD DEVICES by LEX LUTHOR

-----------------------------

This file is designed to identify various kinds of ETF

(Electronic Toll Fraud) devices and to describe their operation,

according to a booklet put out by Bell entitled: THE INVESTIGA#

TION AND PROSECUTION OF ELECTRONIC TOLL FRAUD DEVICES. (For

official use only).

There are several different types of electronic equipment

which may be generally classified as ETF devices. The most sig#

nificant is the 'blue box'. The characteristics of each type of

device are discussed below.

* Cheese Box *

--------------

Its design may be crude or very sophisticated. Its size

varies; one was found the size of a half-dollar.

A Cheese Box is used most often by bookmakers or betters to

place wagers without detection from a remote location. The device

inter-connects 2 phone lines, each having different #'s but each

terminating at the same location. In effect, there are two phones

at the same location which are linked together through a Cheese

Box. It is usually found in an unoccupied apartment connected to

a phone jack or connecting block. The bookmaker, at some remote

location, dials one of the numbers and stays on the line. Various

betters dial the other number but are automatically connected

with the bookmaker by means of the Cheese Box inter-connection.

If, in addition to a Cheese Box, a Black Box is included in the

arrangement, the combined equipment would permit toll-free call#

ing on either line to the other line.

If a police raid were conducted at the terminating point of

the conversation --the location of the Cheese Box-- there would

be no evidence of gambling activity. This device is sometime

difficult to identify. Law enforcement officials have been ad#

vised that when unusual devices are found associated with tele#

phone connections, the phone company security representative

should be contacted to assist in identification. (This probably

would be good for a BBS, especially with the Black Box set up,

and if you ever decide to take the board down, you wouldn't have

to change your phone #. It also makes it so you yourself cannot

be traced. I am not sure about calling out from one though).

_______________________________________________________________________

Copyright 1990

Tutorials                                                     3

_______________________________________________________________________

* BLUE BOX *

------------

The 'Blue Box' was so named because of the color of the

first one found. The design and hardware used in the Blue Box is

fairly sophisticated, and its size varies from a large piece of

apparatus to a miniaturized unit that is approximately the size

of a 'king size' package of cigarettes. The Blue Box contains 12

or 13 buttons or switches that emit multi-frequency tones charac#

teristic of the tones used in the normal operation of the tele#

phone toll (long distance) switching network. The Blue Box ena#

bles its user to originate fraudulent ('free') toll calls by

circumventing toll billing equipment. The Blue Box may be direct#

ly connected to a phone line, or it may be acoustically coupled

to a telephone handset by placing the Blue Box's speaker next to

the transmitter or the telephone handset. The operation of a Blue

Box will be discussed in more detail below.

To understand the nature of a fraudulent Blue Box call, it

is necessary to understand the basic operation of the Direct

Distance Dialing (DDD) telephone network. When a DDD call is

properly originated, the calling number is identified as an

integral part of establishing the connection. This may be done

either automatically or, in some cases, by an operator asking the

calling party for his telephone number. This information is

entered on a tape in the Automatic Message Accounting (AMA)

office. This tape also contains the number assigned to the trunk

line over which the call is to be sent. The information relating

to the call contained on the tape includes: called number, call#

ing number, time of call. The time of disconnect at the end of

the call is also recorded.

Although the tape contains info with respect to many differ#

ent calls, the various data entries with respect to a single call

are eventually correlated to provide billing info for use by your

Bell accounting department.

The typical Blue Box user usually dials a number that will

route the call into the telephone network without charge. For

example, the user will very often call a well-known INWATTS (toll

free) customer's number. The Blue Box user, after gaining this

access to the network and, in effect, 'seizing' control and

complete dominion over the line, operates a key on the Blue Box

which emits a 2600 Hz tone. This tone causes the switching equip#

ment to release the connection to the INWATTS customer's line.

The 2600 Hz tone is a signal that the calling party has hung up.

The Blue Box simulates this condition. However, in fact the local

trunk on the calling party's end is still connected to the toll

network. The Blue Box user now operates the 'KP' (Key Pulse) key

on the Blue Box to notify the toll switching equipment that

switching signals are about to be emitted. The user then pushes

the 'number' buttons on the Blue Box corresponding to the tele#

phone # being called. After doing so, he/she operates the 'ST'

(Start) key to indicate to the switching equipment that the

signaling is complete. If the call is completed, only the portion

_______________________________________________________________________

M & M Enterprises

4                                               The Boxing Book

_______________________________________________________________________

of the original call prior to the emission of the 2600 Hz tone is

recorded on the AMA tape. The tones emitted by the Blue Box are

not recorded on the AMA tape. Therefore, because the original

call to the INWATTS # is toll-free, no billing is rendered in

connection with the call.

Although the above is a description of a typical Blue Box

operation using a common method of entry into the network, the

operation of a Blue Box may vary in any one or all of the follow#

ing respects:

(a) The Blue Box may include a rotary dial to apply the 2600

Hz tone and the switching signals. This type of Blue Box is

called a 'dial pulser' or 'rotary SF' Blue Box.

(b) Entrance into the DDD toll network may be effected by a

pretext call to other toll-free # such as Universal Directo#

ry Assistance (555-1212) or any # in the INWATTS network,

either inter-state or intra-state, working or non-working.

(c) Entrance into the DDD toll network may also be in the form

of 'short haul' calling. A 'short haul' call is a call to

any # which will result in a lesser amount of toll charges

than the charges for the call to be completed by the Blue

Box. For example, a call to Birmingham from Atlanta may cost

$.80 for the first 3 minutes while a call from Atlanta to

Los Angeles is $1.85 for 3 minutes. Thus, a short haul, 3

minute call to Birmingham from Atlanta, switched by use of a

Blue Box to Los Angeles, would result in a net fraud of

$2.65 for a 3 minute call.

(d) A Blue Box may be wired into the telephone line or acous#

tically connected to the handset. The Blue Box may even be

built inside a regular Touch-Tone phone, using the phone's

pushbuttons for the Blue Box's signaling tones.

(e) A magnetic tape recording may be used to record the Blue

Box tones representative of specific phone #'s. Such a tape

recording could be used in lieu of a Blue Box to fraudulent#

ly place calls to the phone #'s recorded on the magnetic

tape.

All Blue Boxes, except 'dial pulse' or 'rotary SF' Blue

Boxes, must have the following 4 common operating capabilities:

(a) It must have signaling capability in the form of a 2600 Hz

tone. The tone is used by the toll network to indicate,

either by its presence or its absence, an 'on hook' (idle)

or 'off hook' (busy) condition of the trunk.

(b) The Blue Box must have a 'KP' tone that unlocks or readies

the multi-frequency receiver at the called end to receive

the tones corresponding to the called phone #.

_______________________________________________________________________

Copyright 1990

Tutorials                                                     5

_______________________________________________________________________

(c) The typical Blue Box must be able to emit MF tones which

are used to transmit phone #'s over the toll network. Each

digit of a phone # is represented by a combination of 2

tones. For example, the digit 2 is transmitted by a combina#

tion of 700 Hz and 1100 Hz.

(d) The Blue Box must have an 'ST' key which consists of a

combination of 2 tones that tell the equipment at the called

end that all digits have been sent and that the equipment

should start switching the call to the called number.

The 'dial pulser' or 'rotary SF' Blue Box requires only a

dial with a signaling capability to produce a 2600 Hz tone.

* BLACK BOX *

-------------

This ETF device is so-named because of the color of the

first one found. It varies in size and usually has one or two

switches or buttons. Attached to the telephone line of a called

party, the Black Box provides toll-free calling *to* that party's

line. A Black Box user informs other persons beforehand that they

will not be charged for any calls placed to him. The user then

operates the device causing a 'non-charge' condition ('no answer'

or 'disconnect')  to be recorded on the telephone company's bill#

ing equipment. A Black Box is relatively simple to construct and

is much less sophisticated than a Blue Box.

* RED BOX *

-----------

This device is coupled acoustically to the handset transmit#

ter of a single slot coin telephone. The device emits signals

identical to those tones emitted when coins are deposited. Thus,

local or toll calls may be placed without the actual deposit of

coins.

BOXING BASICS By Dr. Pepper

-------------

What I will provide here in a summary of the three most

important boxes, some of the ways Ma Bell tries to catch boxers,

and some suggestions on avoiding being caught.

The most famous box, the blue box, is essentially like a

portable touch-tone pad, except the tones are not touch-tone

(DTMF)  -- they are the trunk signaling frequencies (MF). A good

box also contains the supervision control frequency, 2600 Hz

(SF).

_______________________________________________________________________

M & M Enterprises

6                                               The Boxing Book

_______________________________________________________________________

Hackers use the box by gaining access to a DDD (or other)

network sender (it's like a dial tone), usually by making a 800

or 555 call. A much better access is if there are local tandems

in your area used for non-accounted calls but allowing non-local

outgoing calls.

Phreaks use blue boxes because it allows the introduction of

network dial codes other than area code and number and opened an

incredibly complex maze to be explored and toyed with. Other

people used blue boxes to complete calls without paying. This

made AT&T intent on stopping the access; to them the phreaks

tying up tandems from Kalamazoo to Moscow were a nuisance, and

the others were thieves.

One of the most successful means of catching blue boxers was

and is the feature in the accounting program which calls atten#

tion to any number which shows a large number of 800 or 555

calls. (We know of telecommunications managers on our side who

have been called by AT&T to find out why so and so - an

employee-- makes so many calls to the company 800 number). Other

means include analyzing trunk trouble reports (if your box

doesn't treat the trunk the same way AT&T's equipment does, it

can in some cases generate trouble reports).

Once the suspicion is there, your local Telco puts a 'pen

recorder' on your line, and everything you dial -rotary, touch-

tone, or MF gets recorded on paper - this paper will be used as

evidence against you. Eventually the U.S. Government prosecutes

you for 'interstate fraud by wire' -- an extremely broad law.

In order to eliminate blue boxing, which relies on the use

of the MF signals on the same circuit that you talk on, Ma Bell

is converting to a new system, Common Channel Interoffice Signal#

ing (CCIS). There are many benefits of CCIS other than eliminat#

ing blue boxing, but it will eventually eliminate blue boxing

because it sends the network signaling information on data links

between signal transfer points (STPS) associated with various

switching machines.

It will take many years for CCIS to be universally used, but

it is going in fast. As long as there is one non-CCIS link in the

network, the phreaks will find it and ply their hobby.

To avoid getting caught: don't use your box from the same

place repeatedly. Don't complete calls to friends and sit and

gab-- if no 'signs and signals' are transmitted (you don't pass

information -- you don't communicate) you have not committed

fraud by wire. You may get charged with possession, if state laws

where you are caught make boxes illegal.

The second famous box is the black box. With the black box,

you receive calls without the caller being charged. This is

useless for hackers -- only the freephone people are interested.

This works in step-by-step (SXS) offices, #1 and #5 Crossbar, and

some non-Bell offices. Bell's ESS offices are too clever for this

_______________________________________________________________________

Copyright 1990

Tutorials                                                     7

_______________________________________________________________________

trick- your phone isn't connected to the incoming trunk unless

you are off-hook.

Ma Bell catches blank boxers by examining trunk records. Why

was the trunk in use for so long, but not off-hook? In #5 Cross#

bar offices, there will also be funny account entries -- origi#

nate followed by disconnect -- a long time later -- with no

answer in between. ESS offices can generate this data when audits

are done ( a random occurrence).

The best way to avoid being caught is to stay away from this

one. At least keep your calls short -- the length of 15-20 rings

or so.

The final famous box is the red box. This is the electronic

equivalent of the chime people used to take to a pay phone so

that the operator thinks money is being deposited. The gongs were

easy to record or obtain and operators could be easily fooled.

The newer phones with the beeps made it necessary to come up with

something more precise.

People get caught when an operator gets suspicious and

calls security, or when the accounting info for the phone says

there isn't enough money in the coin box. Bell security then

stakes out the pay phone -- and zap. Never use the same pay

phone twice.

THE BLUE BOX AND MA BELL by Herb Friedman,

------------------------

Communications Editor

Radio Electronics Magazine November 1987

Before the breakup of AT&T, Ma Bell was everyone's favorite

enemy. So it was not surprising that so many people worked so

hard and so successfully at perfecting various means of making

free and untraceable telephone calls. Whether it was a "Red Box"

used by Joe and Jane College to call home, or a "Blue Box" used

by organized crime to lay off untraceable bets, the technology

that provided the finest telephone system in the world contained

the seeds of it's own destruction.

The fact of the matter is that the Blue Box was so effective

at making untraceable calls that there is no estimate as to how

many calls were made or who made them. No one knows for certain

whether Ma Bell lost revenues of $100, $100-million, or $1-bil#

lion on the Blue Box. Blue Boxes were so effective at making

free, untraceable calls that Ma Bell didn't want anyone to know

about them, and for many years denied their existence. They even

went as far as strong-arming a major consumer science magazine

into killing an article that had already been prepared on the

Blue and Red boxes. Further, the police records of a major city

_______________________________________________________________________

M & M Enterprises

8                                               The Boxing Book

_______________________________________________________________________

contain a report concerning a break-in at the residence of the

author of that article. The only item missing following the

break-in was the folder containing copies of the earliest Blue-

Box designs and a Bell-System booklet that described how sub#

scriber billing was done by the AMA machine--a booklet that Ma

Bell denied ever existed. Since the AMA (Automatic Message Ac#

counting) machine was the means whereby Ma Bell eventually

tracked down both the Blue and Red Boxes, we'll take time out to

explain it. Besides, knowing how the AMA machine works will help

you to better understand "phone phreaking."

WHO MADE THE CALL

-----------------

Back in the early days of the telephone, a customer's bill#

ing was originated in a mechanical counting device, which was

usually called a "register" or a "meter." Each subscriber's line

was connected to a meter that was part of a wall of meters. The

meter clicked off the message units, and once a month someone

simply wrote down the meter's reading, which was later interpo#

lated into message-unit billing for those subscriber's who were

charged by the message unit. (Flat rate subscriber's could make

unlimited calls only within a designated geographic area. The

meter clicked off message units for calls outside that area.)

Because eventually there were too many meters to read individual#

ly, and because more subscribers started questioning their month#

ly bills, the local telephone companies turned to photography. A

photograph of a large number of meters served as an incontestable

record of their reading at a given date and time, and was much

easier to convert to customer billing by the accounting depart#

ment.

As you might imagine, even with photographs billing was

cumbersome and did not reflect the latest technical developments.

A meter didn't provide any indication of what the subscriber was

doing with the telephone, nor did it indicate how the average

subscriber made calls or the efficiency of the information serv#

ice (how fast the operators could handle requests). So the meters

were replaced by the AMA machine. One machine handled up to

20,000 subscribers. It produced a punched tape for a 24-hour

period that showed, among other things, the time a phone was

picked up (went off-hook), the number dialed, the time the called

party answered, and the time the originating phone was hung up

(placed on-hook).

One other point, which will answer some questions that

you're certain to think of as we discuss the Red and Blue boxes:

Ma Bell did not want persons outside their system to know about

the AMA machine. The reason? Almost everyone had complaints

--usually unjustified-- about their billing. Had the public been

aware of the AMA machine, they would have asked for a monthly

list of their telephone calls. It wasn't that Ma Bell feared

errors in billing; rather, they were fearful of being buried

under an avalanche of paperwork and customer complaints. Also,

the public believed their telephone calls were personal and

_______________________________________________________________________

Copyright 1990

Tutorials                                                     9

_______________________________________________________________________

untraceable, and Ma Bell didn't want to admit that they knew

about the who, when, and where of every call. And so Ma Bell

always insisted that billing was based on a meter unit that

simply "clicked" for each message unit; that there was no record,

other than for long-distance calls, as to who called whom. Long

distance was handled by, and the billing information was done by

an operator, so there was a written record Ma Bell could not

deny.

The secrecy surrounding the AMA machine was so pervasive

that local, state, and even federal police were told that local

calls made by criminals were untraceable, and that people who

made obscene telephone calls could not be tracked down unless the

person receiving the calls could keep the caller on the line for

some 30 to 50 minutes so the connections could be physically

traced by technicians. Imagine asking a woman or child to put up

with almost an hours worth of the most horrendous obscenities in

the hope someone could trace the line. Yet in areas where the AMA

machine had replaced meters, it would have been a simple, though

perhaps time-consuming task, to track down the numbers called by

any telephone during a 24-hour period. But Ma Bell wanted the AMA

machine kept as secret as possible, and so many a criminal was

not caught, and many a woman was harried by the obscene calls of

a potential rapist, because existence of the AMA machine was

denied.

As a sidelight as to the secrecy surrounding the AMA ma#

chine, someone at Ma Bell or the local operating company decided

to put the squeeze on the author of the article on Blue Boxes,

and reported to the Treasury Department that he was, in fact,

manufacturing them for organized crime--the going rate in the mid

1960's was supposedly $20,000 a box. (Perhaps Ma Bell figured the

author would get the obvious message: Forget about the Blue Box

and the AMA machine or you'll spend lots of time, and much money

on lawyer's fees to get out of the hassles it will cause.) The

author was suddenly visited by his place of employment by a

Treasury agent. Fortunately, it took just a few minutes to con#

vince the agent that the author was really just that, and not a

technical wizard working for the mob. But one conversation led to

another, and the Treasury agent was astounded to learn about the

AMA machine. (Wow! Can an author whose story is squelched spill

his guts.) According to the Treasury agent, his department had

been told that it was impossible to get a record of local calls

made by gangsters: The Treasury department had never been in#

formed of the existence of automatic message accounting. Needless

to say, the agent left with his own copy of the Bell System

publication about the AMA machine, and the author had an appoint#

ment with the local Treasury Bureau director to fill him in on

the AMA Machine. That information eventually ended up with Sena#

tor Dodd, who was conducting a congressional investigation into,

among other things, telephone company surveillance of subscriber

lines--which was a common practice for which there was detailed

instructions, Ma Bell's own switching equipment ("crossbar")

manual.

_______________________________________________________________________

M & M Enterprises

10                                               The Boxing Book

_______________________________________________________________________

THE BLUE BOX

------------

The Blue Box permitted free telephone calls because it used

Ma Bell's own internal frequency-sensitive circuits. When direct

long-distance dialing was introduced, the crossbar equipment knew

a long-distance call was being dialed by the three-digit area

code. The crossbar then converted the dial pulses to the CCITT

tone groups, that are used for international and trunkline sig#

nalling. (Not that those do not correspond to Touch-Tone frequen#

cies.) The tone groups represent more than just numbers; among

other things there are tone groups identified as KP (prime) and

ST (start)--keep them in mind. When a subscriber dialed an area

code and a telephone number on a rotary-dial telephone, the

crossbar automatically connected the subscriber's telephone to a

long-distance truck, converted the dial pulses to CCITT tones

sent out on the long-distance trunk that set up or selected the

routing and caused electro-mechanical equipment in the target

city to dial the called telephone.

Operator-assisted long-distance calls worked the same way.

The operator simply logged into a long-distance trunk and pushed

the appropriate buttons, which generated the same tones as direct

dial equipment. The button sequence was KP (which activated the

long-distance equipment), then the complete area code and tele#

phone number. At the target city, the connection was made to the

called number but ringing did not occur until the operator there

pressed the ST button. The sequence of events of early Blue Boxes

went like this: The caller dialed information in a distant city,

which caused his AMA machine to record a free call to informa#

tion. When the information operator answered, he pressed the KP

key on the Blue Box, which disconnected the operator and gave him

access to a long-distance trunk. He then dialed the desired

number and ended with an ST, which caused the target phone to

ring. For as long as the conversation took place, the AMA machine

indicated a free call to an information operator. The technique

required a long-distance information operator because the local

operator, not being on a long-distance trunk, was accessed

through local wire switching, not the CCITT tones.

CALL ANYWHERE

-------------

Now imagine the possibilities. Assume the Blue Box user was

in Philadelphia. He would call Chicago information, disconnect

from the operator with a KP tone, and then dial anywhere that was

on direct-dialing service: Los Angeles, Dallas, or anywhere in

the world the Blue Boxer could get the international codes.

The legend often told of one Blue Boxer who, in the 1960's,

lived in New York and had a girlfriend at a college near Boston.

Now back in the 1960's, making a telephone call to a college town

on the weekend was even more difficult than it is today to make a

call from New York to Florida on a reduced-rate holiday using one

of the cut-rate long-distance carriers. So our Blue Boxer got on

_______________________________________________________________________

Copyright 1990

Tutorials                                                     11

_______________________________________________________________________

an international operator's circuit to Rome, Blue Boxed through

to a Hamburg operator, and asked Hamburg to patch through to

Boston. The Hamburg operator thought the call originated in Rome

and inquired as to the "operator's" good English, to which the

Blue Boxer replied that he was an expatriate hired to handle

calls by American tourists back to their homeland. Every weekend,

while the Northeast was strangled by reduced-rate long-distance

calls, our Blue Boxer had no trouble sending his voice almost

7,000 miles for free.

VACUUM TUBES

------------

Assembly plans for Blue Boxes were sold through classified

advertisements in the electronic-hobbyist magazines. One of the

earliest designs was a two-tube portable model that used a 1.5-

volt "A" battery for the filaments and a 125-volt "B" battery for

the high-voltage (B+) power supply. It consisted of two phase-

shift oscillators sharing a common speaker that mixed the tones

from both oscillators. The user placed the speaker over the

telephone handset's transmitter and simply pressed the buttons

that corresponded to the desired CCITT tones. It was just that

simple.

Actually, it was even easier then it reads because Blue

Boxers discovered they did not need the operator. If they dialed

an active telephone located in certain nearby, but different,

area codes, they could Blue Box just as if they had Blue Boxed

through an information operator's circuit. The subscriber whose

line was blue Box conversation was short, the "dead" phone sud#

denly came to life the next time it was picked up. Using a list

of "distant" numbers, a Blue Boxer would never hassle plain to

the telephone company. The difference between Blue Boxing off a

subscriber rather than an information operator was that the Blue

Boxer's AMA tape indicated a real long-distance telephone call

--perhaps costing 15 or 25 cents--instead of a freebie. Of

course, that is the reason why when Ma Bell finally decided to go

public with "assisted" newspaper articles about the Blue Box

users they had apprehended, it was usually about some college kid

or "phone phreak." One never read of a mobster being caught.

Greed and stupidity were the reasons why the kid's were caught.

It was the transistor that led to Ma Bell going public with the

Blue Box. By using transistors and RC phase-shift networks for

the oscillators, a portable Blue Box could be made inexpensively,

and small enough to be used unobtrusively from a public tele#

phone. The college crowd in the many technical schools went crazy

with the portable Blue Box; they could call the folks back home,

their friends, or get a free network (the Alberta and Carolina

connections--which could be a topic for a whole separate article)

and never pay a dime to Ma Bell. Unlike the mobsters who were

willing to pay a small long-distance charge when Blue Boxing, the

kids wanted it, wanted it all free, and so they used the informa#

tion operator routing, and would often talk "free-of-charge" for

hours on end.

_______________________________________________________________________

M & M Enterprises

12                                               The Boxing Book

_______________________________________________________________________

Ma Bell finally realized that Blue Boxing was costing them

big bucks, and decided a few articles on the criminal penalties

might scare the Blue Boxers enough to cease and desist. But who

did Ma Bell catch?  The college kids and the greedies. When Ma

Bell decided to catch the Blue Boxers she simply examined the AMA

tapes for calls to an information operator that were excessively

long. No one talked to an operator for 5, 10, 30 minutes, or

several hours. Once a long call to an operator appeared several

times on an AMA tape, Ma Bell simply monitored the line and the

Blue Boxer was caught. (Now do you understand why we opened with

an explanation of the AMA machine?) If the Blue Boxer worked from

a telephone booth, Ma Bell simply monitored the booth. Ma Bell

might not have known who originated the call, but she did know

who got the call, and getting that party to spill their guts was

no problem. The mob and a few Blue Box hobbyists (maybe even

thousands) knew of the AMA machine, and so they used a real

telephone number for the KP skip. Their AMA tapes looked perfect#

ly legitimate. Even if Ma Bell had told the authorities they

could provide a list of direct-dialed calls made by local mob#

sters, the AMA tapes would never show who was called through a

Blue Box. For example, if a bookmaker in New York wanted to lay

off some action in Chicago, he could make a legitimate call to a

phone in New Jersey and then Blue Box to Chicago. Of course,

automatic tone monitoring, computerized billing, and ESS (Elec#

tronic Switching Systems) now make that all virtually impossible.

but that's the way it was.

You might wonder how Ma Bell discovered the tricks of the

Blue Boxers. Simple, they hired the perpetrators as consultants.

While the initial newspaper articles detailed the potential jail

penalties for apprehended Blue Boxers, except for Ma Bell employ#

ees who assisted a Blue Boxer, it is almost impossible to find an

article on the resolution of the cases because most hobbyist Blue

Boxers got suspended sentences and/or probation if they assisted

Ma Bell in developing anti-Blue Box techniques. It is asserted,

although it can't be easily proven, that cooperating ex-Blue

Boxers were paid as consultants. (If you can't beat them, hire

them to work for you.)

Should you get any ideas about Blue Boxing, keep in mind

that modern switching equipment has the capacity to recognize

unauthorized tones. It's the reason why a local office can leave

their subscriber Touch-Tone circuits active, almost inviting you

to use the Touch-Tone service. A few days after you use an unau#

thorized Touch-Tone service, the business office will call and

inquire whether you'd like to pay for the service or have it

disconnected. The very same central-office equipment that knows

you're using Touch-Tone frequencies knows if your line is origi#

nating CCITT signals.

_______________________________________________________________________

Copyright 1990

Tutorials                                                     13

_______________________________________________________________________

THE RED BOX

-----------

The Red Box was primarily used by the college crowd to avoid

charges when frequent calls were made between two particular

locations, say the college and a student's home. Unlike the

somewhat complex circuitry of the Blue Box, a Red Box was nothing

more than a modified telephone; in some instances nothing more

than a capacitor, a momentary switch, and a battery. As you

recall from our discussion of the Blue Box, a telephone circuit

is really established before the target phone ever rings, and the

circuit is capable of carrying an AC signal in either direction.

When the caller hears the ringing in their handset, nothing is

happening at the receiving end because the ringing signal he

hears is really a tone generator at his local telephone office.

The target (called) telephone actually gets it 20 pulses-per-

second ringing voltage when the person who dialed hears nothing

--in the "dead" spaces between hearing the ringing tone. When the

called phone is answered and taken off hook, the telephone com#

pletes a local-office DC loop that is the signal to stop the

ringing voltage. About three seconds later the DC loop results in

a signal being sent all the way back to the caller's AMA machine

that the called telephone was answered. Keep that three-second

AMA delay in mind.

Now as we said earlier, the circuit can actually carry AC

before the DC loop is closed. The Red Box is simply a device that

provides a telephone with a local battery so that the phone can

generate an AC signal without having a DC connection to the

telephone line. The earliest of the Red Boxes was the surplus

military field telephone, of which there were thousands upon

thousands in the marketplace during the 1950's and 1960's. The

field telephone was a portable telephone unit having a manual

ringer worked by a crank--just like the telephone Grandpa used on

the farm--and two D-cells. A selector switch set up the unit so

that it could be connected to a combat switchboard, with the DC

power supplied by the switchboard. But if a combat unit wasn't

connected to a switchboard, and the Lieutenant yelled "Take a

wire," the signalman threw a switch on his field telephone that

switched in the local batteries. To prevent the possibility of

having both ends of the circuit feeding battery current into the

line in opposite polarity--thereby resulting in silence--the

output from the field telephone when running from its internal

batteries was only the AC representing the voice input, not

modulated DC.

PRESS ONCE TO TALK

------------------

The Red Box was used at the receiving end; let's assume it's

the old homestead. The call was originated by Junior (or Sis) at

their college 1000 miles away from home. Joe gave the family one

ring and then hung up, which told them that he's calling. Pop set

up the Red Box. Then Junior redialed the old homestead. Pop

lifted the handset when the phone rang. Then Pop closed a momen#

_______________________________________________________________________

M & M Enterprises

14                                               The Boxing Book

_______________________________________________________________________

tary switch for about a half-second, which caused the local

telephone office to silence the ringing signal. When Pop released

the switch, the folks can talk to Junior without Junior getting

charged because his AMA tape did not show his call was answered

(the DC loop must be closed for at least three-seconds for the

AMA tape to show Junior's call was answered). All the AMA tape

showed is that Junior let the phone ring at the old homestead for

almost 30 minutes; a length of time that no Bell Operating Compa#

ny is likely to believe twice!

A modern Red Box is simply a conventional telephone that's

been modified to emulate the vintage 1940 military field tele#

phone. Aside from the fact that the operating companies can now

nail every Red Box user because all modern billing equipment

shows the AMA information concerning the length of time a caller

let the target phone ring, it's use has often put severe psycho#

logical strain on the users.

There are no hard facts concerning how many Red Boxes were

in use, or how much money Ma Bell lost, but one thing is known:

she had little difficulty in closing down Red Boxes in virtually

all instances where the old folks were involved because Mom and

Pop usually would not tolerate what to them was stealing. If you

as a reader have any ideas about using a Red Box, bear in mind

that the AMA machine (or it's equivalent) will get you every

time, even if you use a phone booth, because the record will show

the number being called, and as with the Blue Box, the people on

the receiving end will spill their guts to the cops.

HOW BELL CATCHES BLUE BOXERS By LEX LUTHOR AND THE LEGION OF DOOM

----------------------------

UPDATED: 25-JUNE-84

This article describes the investigative procedures used by

the security department of Ma Bell. Most of the file will talk

about blue box investigations because of the frequency of the

blue box cases referred to law enforcement officials for prosecu#

tion.

The security department may first discover evidence of

boxing activity from looking at calling patterns to particular

numbers. Such analyses may reveal abnormal calling patterns which

possibly are the result of box activity (such as 122 hour calls

to dir. assist). Also, cases of suspected boxing are referred to

the security department from the various operating departments of

Bell, from other telephone companies , or from law enforcement

officials. In some instances, detection and identification of a

calling station originating suspected blue box tones can be

provided by use of a special non-monitoring test equipment.

_______________________________________________________________________

Copyright 1990

Tutorials                                                     15

_______________________________________________________________________

If initial indications are that there is a good possibility

that a blue box is being used on a particular phone line, the

security department determines certain information about the

line. The name of the subscriber to that line is identified, and

an inventory is made of the line and station equipment being

provided to him. A discreet background investigation (record) is

conducted to establish the subscriber's identity. After this

preliminary data is gathered, blue box detection units are in#

stalled on the suspected line to establish "probable cause" for

further investigation. If the "probable cause" equipment indi#

cates repeated blue box activity on the line, other equipment is

then installed to document such activity.

The "probable cause" equipment ascertains the presence of

multi-frequency tones on the subscribers end of the line which

would not be present in normal usage. The "probable cause" device

now being used by some Bell central offices register each and

every application of 2600hz tones in single-frequency (sf) sig#

naling and or 2600hz tone followed by kp tones used in multi-

frequency (mf) signaling. As previously stated, such tones should

not normally be present on the line.

If "probable cause" is established, other detection, identi#

fication and documentation equipment is installed. The primary

equipment now being used is the dialed number recorder (DNR),

coupled with an auxiliary tape recorder. The DNR is activated

when the suspect subscriber's phone goes "off-hook" and prints on

paper tape the following information concerning the call: the

date and time of the call and the digits dialed over the suspects

line. Moreover, the DNR records on the paper tape an indicator of

the presence of 2600hz tones on the line and the presence of

multi-frequency signaling tones on the subscriber's line. The

auxiliary tape recorder is activated *only* after the presence of

2600hz tone on the line is detected by the DNR (indicating the

use of a blue box). Once the tape recorder is activated, it

records the tones being emitted by the blue box, other signaling

tones, and the ringing cycle on the called end . It also records

a minimum amount of ensuing conversation for the purpose of:

(1) establishing that the fraudulent call was consummated

(2) establishing the identity of the fraudulent caller.

The timing duration of the tape recorder is pre-set. A time

of one-minute (including pulsing, ringing and conversation) is

the standard setting; however, if the blue box user is suspected

of making overseas calls, the timing may be set for 2 minutes

because of the greater time required by the blue box user to

complete the call. Upon termination of the call, the DNR automat#

ically prints the time of termination and the date. It should be

pointed out that the presence of 2600hz tones *plus* multi-fre#

quency signaling tones on a subscriber's line positively estab#

lishes that a blue box is being used to place a fraudulent call

because such tones are not normally originated from a subscribers

line.

_______________________________________________________________________

M & M Enterprises

16                                               The Boxing Book

_______________________________________________________________________

Once the raw data described above is gathered, the security

department collects and formulates the data into legally admissi#

ble evidence of criminal activity. Such evidence will establish:

(1) That a fraudulent call was placed by means of an ETF

device,

(2) That conversation ensued,

(3) That the fraudulent call was placed by an identified

individual, and

(4) That such call was not billed to the subscriber number

from which the blue box call originated.

The evidence which is then available consists of documents

and also of expert witness testimony by telephone company person#

nel concerning the contents of those documents, the operation of

the blue box, and the operation of the detection equipment.

(note- similar techniques are used in the investigation of other

forms of ETF.)

Presentation of Evidence to Prosecutors

---------------------------------------

The evidence accumulated by the security department is

carefully reviewed by the legal department for the purpose of

determining whether sufficient evidence exists to warrant the

presentation of the evidence to law enforcement officials. If the

evidence does warrant such action, it is presented under appro#

priate circumstances to the proper law enforcement officials. In

all cases where prosecution is recommended, a professionally

investigated and documented summary of the case will be prepared

and presented by the security department to the prosecutor's

office. Each case recommended for prosecution will be prepared as

completely as possible, usually necessitating little or no pre-

trial investigation for the prosecutor. The summary of the case

will include the following:

(a) A background of the case with details of the defendant's

activities and a summary of all pertinent investigative

steps and interviews conducted in the course of the investi#

gation.

(b) Identification of witnesses.

_______________________________________________________________________

Copyright 1990

Tutorials                                                     17

_______________________________________________________________________

DEALING WITH THE RATE & ROUTE OPERATOR By Fred Steinbeck

--------------------------------------

It seems that fewer and fewer people have blue boxes these

days, and that is really too bad. Blue boxes, while not all that

great for making free calls (since the TPC can tell when the call

was made, as well as where it was to and from), are really a lot

of fun to play with. Short of becoming a real live TSPS operator,

they are about the only way you can really play with the network.

For the few of you with blue boxes, here are some of the

phrases which may make life easier when dealing with the rate &

route (R&R) operators. To get the R&R operator, you send a KP +

141 + ST. In some areas, you may need to put another NPA before

the 141 (i.e. KP + 213 + 141 + ST), if you have no local R&R

operator.

The R&R operator has a myriad of information, and all it

takes to get this data is mumbling cryptic phrases. There are

basically four special phrases to give the R&R operators. They

are NUMBERS route, DIRECTORY route, OPERATOR route, and PLACE

NAME.

To get an area code for a city, one can call the R&R opera#

tor and ask for the numbers route. For example, to find the area

code for Carson City, Nevada, we'd ask the R&R operator for

'Carson City, Nevada, numbers route, please' and get the answer,

'702 plus', meaning that 702 plus 7 digits gets us there.

Sometimes directory assistance isn't just NPA + 131. The way

to get these routings is to call R&R and ask for 'Anaheim, Cali#

fornia, directory route, please'. Of course, she'd tell us it was

714 plus, which means 714 + 131 gets us the D.A. operator there.

This is sort of a pointless example, but I couldn't come up with

a better one on short notice.

Let's say you wanted to find out how to get the inward

operator for Sacramento, California. The first six digits of a

number in that city will be required (the NPA and an NXX). For

example, let us use 916 756. We would call R&R, and when the

operator answered, say '916 756, operator route, please'. The

operator would say, '916 plus 001 plus'. This means that 916 +

001 + 121 will get you the inward operator for Sacramento.

Do you know the city which corresponds to 503 640? The R&R

operator does, and will tell you that it is Hillsboro, Oregon, if

you sweetly ask for 'Place name, 503 640, please'.

For example, let's say you need the directory route for

Sveg, Sweden. Simply call R&R, and ask for 'International, Baden,

Switzerland. TSPS directory route, please'. In response to this,

you'd get, 'Directory to Sveg, Sweden. Country code 46 plus

1170'. So you'd route yourself to an international sender, and

send 46 + 1170 to get the D.A. operator in Sweden.

_______________________________________________________________________

M & M Enterprises

18                                               The Boxing Book

_______________________________________________________________________

Inward operator routings to various countries are obtained

the same way 'International, London, England, TSPS inward route,

please', and get 'country code 44 plus 121'. Therefore, 44 plus

121 gets you inward for London.

Inwards can get you language assistance if you don't speak

the language. Tell the foreign inward, 'United States calling.

Language assistance in completing a call to (called party) at

(called number)'.

R&R operators are people too, so always be polite, make sure

use of them, and dial with care.

Note: As a result of the break-up, R&R is now KP+800+141+1212+ST

STEP BY STEP SWITCHING NOTES By PHANTOM PHREAKER

----------------------------

The following research was done on a class 5 Step By Step

switching system. Items mentioned in this article are not guaran#

teed to work with your particular office. The following interest#

ing topics about Step By Step switching are for informational and

educational purposes only. This article is aimed at people who

wish to learn more about telephone switching systems.

I realize step-by-step switching is dwindling every day,

with many electromechanical SxS offices being replaced with newer

electronic/digital switches and Remote Switching Systems (RSS's).

However, rural areas of the U.S. still use Step, so if you are

ever in an area served by a SxS CO you may be able to use this

information.

ANI Failure/ONI

---------------

To understand this technique, you must understand how ANI

functions in the Step-by-Step switching system. Your CO sends

ANI, with your number, in MF or DP to receivers that collect the

ANI information and store it, along with the called number, on

the appropriate form of AMA tape. ANI outpulsing in MF can use

either LAMA (Local Automatic Message Accounting) or CAMA (Cen#

tralized Automatic Message Accounting). ANI sent in DP type

signaling can also be used, but is rare. DP vs MF trunk signaling

is similar to the difference between DTMF and pulse dialing,

except on a trunk. DP signaling sends all information in short

bursts of 2600Hz tones.

Causing ANIF's/ONI is an easy task in SxS (and some versions

of Xbar), because the customer's link to the CO will allow the

customer to input MF tones to influence a calls completion. This

can be done by dialing a long distance number and listening to

the clicks that follow. After the first click when you are done

_______________________________________________________________________

Copyright 1990

Tutorials                                                     19

_______________________________________________________________________

dialing, you will hear a few more. They will be timed very close

to one another, and the last click occurs right before the called

telephone rings. The number and speed of the clicks probably

varies. Basically what these clicks are is the Toll Office that

serves your CO setting up a route for your call. In order to

abuse this knowledge, you need access to a MF source, whether it

be a blue box, a computer with a good sound chip, tape recording,

etc. Right before you hear the series of clicks, send one of the

following sequences in MF:

KP+1 (Repeatedly) For Automatic Number Identification Failure (ANIF)

-or-

KP+2 (Repeatedly) For Operator Number Identification (ONI)

(Note:these will not work if your CO uses DP signalling.)

Play these tones into the phone at a sufficient volume so

that they 'drown out' the series of clicks. Do not send an ST

signal, as you are not actually dialing on a trunk. You must send

these MF sequences quickly for this method to work correctly.

After you have played your 'routing' a few times, you will hear a

TSPS operator intercept your call and ask for the number you are

calling FROM.

When an ANIF is recognized, the call is cut through to a

TSPS site that serves your area. Now, you can give the operator

any number in your exchange and she will enter the billing infor#

mation manually, and put the call through. The toll charges will

appear on the customer who owns the number you gave. You can also

accomplish a similar feat by merely flashing the switchhook

during the series of clicks. This will send DC pulses that scram#

ble the ANI outpulsing and cause your call to be sent to a TSPS

operator before the dialed number. Be sure to stop sending the MF

'routing' after the operator attaches or she may know that some#

thing's up. Use this method sparingly and with caution. It would

also be a good idea not to use the same number for billing more

than one time. Don't use this method in excess, because a toll

office report will list the number of ANI failures for a specific

time period.

The ONI method works better because it is assumed ONI is

needed to identify a caller's DN upon a multi-party line. Too

many ANI failures will generate a report upon a security/mainte#

nance TTY, so if you plan on using this method, use the ONI

method instead of just ANI Failure.

The basic idea behind the ANIF is to scramble your ANI

information by using MF (or the switchhook) to send your LD call

to a TSPS operator for Operator Number Identification (ONI) due

to ANI Failure. The idea behind the ONI method is that you are

fooling the switch into thinking you are calling from a multi-

party line and ONI is needed to identify your DN.

_______________________________________________________________________

M & M Enterprises

20                                               The Boxing Book

_______________________________________________________________________

Test numbers

------------

Some other interesting things in the Step By Step system can

be found by dialing test numbers. Test numbers in SxS switching

systems are usually hidden in the XX99 area, as opposed to 99XX,

which is common for other types of switching systems. These types

of numbers are possibly physical limitations of a SxS switch, and

thus a milliwatt tone or other test numbers will be placed there,

because a normal DN can't be assigned such a number. However,

these XX99 numbers are usually listed in COSMOS as test numbers.

Another interesting note about XX99 numbers is that they seem (at

least in some offices) to be on the same circuit. (That is, if

one person calls an XX99 number and receives a test tone, and

another person calls any other XX99 number in that same prefix,

the second caller will receive a busy signal).

Here we must examine the last four digits of a telephone

number in detail.

XXXX=WXYZ             W=Thousands digit

X=Hundreds digit

Y=Tens digit

Z=Units digit

Dialing your prefix followed by an XX99 may result in a busy

signal test number, a network overflow (reorder), milliwatt

tones, or other type of error messages encountered when dialing.

Not every XX99 number is a test number, but many are. Try

looking for these in a known Step by Step office.

The numbers that return a busy signal are the ones that

incoming callers are connected to when the Sleeve lead of the

called Directory Number is in a voltage present state, which

means the line is in use or off-hook. More about this in the next

topic.

Busy signal conferencing

------------------------

Another interesting feature of the Step-By-Step system is

the way busy tones (60 IPM) are generated. In ESS and DMS central

offices, busy signals that are sent by the terminating switch are

computer generated and sound very even and clear with no signal

irregularity. In SxS, all calls to a particular DN are sent to

the same busy signal termination number, which can be reached

most of the time by a POTS number. These busy tones are not

computer generated and the voice path is not cut-off.

You can take advantage of this and possibly have a 'busy

signal conference'. This can be achieved by having several people

dial the same busy DN that is served by a Step office, or by

dialing an always-busy termination number. When you are connected

to the busy signal, you will also be able to hear anyone else who

_______________________________________________________________________

Copyright 1990

Tutorials                                                     21

_______________________________________________________________________

has dialed the same busy number. Connection quality is very poor

however, so this is not a good way to communicate.

As an added bonus, answering supervision is not returned on

busy numbers, and thus the call will be toll-free for all parties

involved. However, you must be using AT&T as your inter-LATA

carrier if the call to the busy number is an inter-LATA call for

you. So if your IC is US Sprint, you must first dial the AT&T

Carrier Access Code (10ATT) before the busy number. If your IC

doesn't detect answer supervision, and begins billing immediately

or after a certain amount of time, then you will be billed for

the length of the call.

Temporarily 'freezing' a line

-----------------------------

A SxS switching system that operates on the direct control

principle is controlled directly by what the subscriber dials.

Jamming a line on SxS to prevent service is possible by simply

flashing the switchhook a number of times. Or you may find after

several aborted dialing attempts, the line will freeze until it

is reset, either manually or by some time-out mechanism. Usually

the time the line is out of action is only a few minutes. The

line will return a busy signal to all callers, and the subscriber

who has a 'dead' phone will not even hear sidetone. This happens

when one of the elements in the switch train gets jammed. The

switch train consists of the linefinder, which sends a dial tone

to the subscriber who lifted his telephone, and places voltage on

the S (Sleeve) lead as to mark that given DN as busy. Next in the

switch train are the selectors. The selectors are what receive

the digits you dial and move accordingly. The last step in the

switch train is the connector. The connector is what connects

calls that are intraoffice, and sends calls to a Toll office when

necessary. Other types of devices can be used in the switch

train, such as Digit Absorbing Selectors, where needed.

Toll/Operator assisted dialing

------------------------------

You may be able to dial 1/0+ numbers with your prefix in#

cluded in some areas. You can dial any call that you could nor#

mally reach by dialing 1+ or 0+. For example, to dial an operator

assisted call to a number in Chicago, you could dial NXX + 0312 +

555 + 1000 where NXX is your prefix, and you would receive the

usual TSPS bong tone, and the number you dialed, 312 + 555 +

1000, would show up on the TSPS consoles LED readout board. You

can also use a 1 in place of the 0 in the above example to put

the call through as a normal toll call.

This method does not bypass any type of billing, so don't

get your hopes up high.

The reason this works is twofold. The first reason is that

the thousandths digit in many SxS offices determines the type of

_______________________________________________________________________

M & M Enterprises

22                                               The Boxing Book

_______________________________________________________________________

call. A 0 or a 1 in place of another number (which would repre#

sent a local call) is handled accordingly. The other reason is

due to a Digit Absorbing Selector that can be installed in some

SxS offices to 'absorb' the prefix on intraoffice calls when it

is not needed to process the call. A DAS can absorb either two or

three digits, depending on whether the CO needs any prefix

digit(s) for intraoffice call completion.

Hunting prefixes

----------------

SxS switches may also translate an improperly dialed local

call and send it to the right area over interoffice trunks. Take

for instance, you need to make a local call to 492-1000. You

could dial 292-1000 and reach the exact same number, provided

that there is no 292 prefix within your local calling area.

However, only the first digit of a prefix may be modified or the

call will not go through correctly unless you happen to have

dialed a valid local prefix. You also cannot use a 1 or a 0 in

place of the first prefix digit, because the switch would inter#

pret that as either dialing a toll or an operator assisted call.

Trunks

------

Step by Step switching system incoming and outgoing trunks

are very likely to use In-band supervisory signalling. This means

you could possibly use numbers served by a SxS CO to blue box off

of. But, some older step areas may not use MF signaling, but DP

signalling. DP signaling uses short bursts of 600Hz to transfer

information as opposed to Multi-Frequency tones. In DP signaling,

there are no KP or ST equivalents. Boxing may be accomplished

from DP trunks by sending short bursts of 2600Hz (2 bursts would

be the digit 2). Acceptable pulse rates are 7.5 to 12 pulses per

second, but the normal rate is 10 pulses per second. A pulse

consists of an 'on hook' (2600Hz) tone and an off-hook (no tone).

So, at 10 pulses per second, a digit might be .04 seconds of tone

and .06 seconds of silence. DP is rarely used today, but some

direct-control Step offices still use it. Common Control Step

offices are much more likely to use MF trunk signalling.

As said at the start of this file, some of the things men#

tioned here may have no practical use, but are being exposed to

the public and to those who did not know about any one of the

procedures mentioned here previously.

References and acknowledgements

-------------------------------

Basic Telephone Switching Systems-By David Talley, Hayden

publishers

No. 1 AMARC-Bell System Technical Journal

Mark Tabas for information about CAMA and DP, The Marauder,

and Doom Prophet.

_______________________________________________________________________

Copyright 1990

Tutorials                                                     23

_______________________________________________________________________

Verification By Fred Steinbeck From TAP issue # 88  10-83

------------

There has been a great deal of controversy in the realm of

phreakdom over a mysterious subject known under a number of

different names, including "Verification", "Autoverification",

"Verify", "Autoverify", "Verify Busy", and even "VFY BY". All of

these names basically mean the same thing: the ability to listen

to another person's telephone line from any telephone in the

direct-dialable world.

Needless to say, Bell System is very tight lipped about

knowledge regarding verification. Indeed, the infamous book

'Notes  on long distance dialing' ('68 edition) says, "Care must

be taken to insure that the customer never gains verification

capabilities." With a printed policy like that, you can imagine

what their real-world policy is like! Even their own rate and

route operators will not give verification on routing codes (at

least in my experience), one even responding, "What?! You must be

crazy! We don't give those out!" Before you get too far into this

article, I will state simply: I don't know how to verify. Howev#

er, I have been fooling with various things related to it, and

collecting information on it for some time now. Therefore, while

I can't do it (yet), I may be able to point some other bright

TAPer on the right track, and perhaps he or she will show us all

how. If you have knowledge not covered in this article, but don't

want to write an article on your own, please send your ideas,

comments, or information to Project Verify, C/O TAP.

Verify has also been called "Autoverify", and I have no idea

why. This is not, to my knowledge, a Bell System term (at least

I've never seen it in any manuals). As far as I know, there is

verify, which means being able to listen to speech (kind of; see

below) on a line, and there is the "Emergency Interrupt which

allows you to take part in the conversation taking place on the

line in question. It has been suggested that "Autoverify" is the

same as an emergency interrupt , but I tend to disagree with this

idea. It should be noted that the verification circuitry does not

actually let an operator listen to a conversation without making

a beep on the line every so often. Instead, she will hear en#

crypted speech. However, I believe with the proper methods,

verify can be converted to an emergency interrupt.

Verification is normally done either by your normal "0"

(TSPS) operator, if the call is in your home NPA (HNPA), or by an

inward operator (IO). If the call is outside your HNPA, your

normal operator will call the IO for the NPA,and say, "Verify

Busy" or "Emergency Interrupt" please, 555 1212."  The IO will

perform whatever magic he or she must, and then report back. If

the call is in your HNPA, though, the "0" operator can do the

verification herself by using the "VFY BY" key on her keyshelf.

However, in some areas, the operator uses a routing code to

accomplish verification, and this is the loop hole we shall at#

tack.

_______________________________________________________________________

M & M Enterprises

24                                               The Boxing Book

_______________________________________________________________________

It follows that if a IO or "0" operator can do it, so can

we, with a blue box. Now, courtesy of Robert Allen (who brought

it to my attention) and Susan Thunder (who apparently discovered

it), here is what used to work for getting operators to hook you

into conversations with other people (i.e.,let you listen to them

till you hung up): You'd call the operator and say "Operator,

TSPS Maintenance Engineer Calling. Ring forward to 001 + NPA +

7d, ring back to my number, hit ring forward, no AMA, and then

position release.

This creates some problems, and you must be familiar with

the TSPS console (by dialing "0"), you are on the "back", or

incoming part of a loop. When she places a call for you, the call

goes out on the "forward", or outgoing part of the loop. If an

operator wants to make a call, she punches KP FWD (keypulse

forward), the number, and ST. Ring FWD puts a 90 volt ringing

signal across the forward part of the line (and may dial the

number as well).  The problem arises from the fact that I don't

know if Ring FWD will actually dial a call, and if there is some

other subtle difference between it and KP FWD.

Let us assume ringing forward makes a call from the TSPS

console to whatever number is given. Ring back causes your phone

to ring (it is assumed you hung up after giving her your instruc#

tions; if you didn't you'd hear an annoying 90 volts across the

earpiece.) "No AMA" means "no automatic message accounting", so

nobody gets billed for the call, although it will show up on a

tape somewhere. "Position Release" removes the operator from the

circuit, and allows her to receive other calls. This leaves an

unaccounted-for ring forward.

The verification circuit, as you know, likes to encrypt

conversation, which is something we don't want. Well, the second

Ring FWD sends another 90 volts crashing against the verify

circuitry, which Juda Gerad thinks removes the voice encryption

from the line, puts the operator (and you) in circuit, and puts a

beep tone on the line every five seconds. This seems to make

sense, and I am inclined to agree with him.

The bit about "..001 + NPA + 7D" causes the thought "MF

routing code" to spring immediately to mind. Now, the above trick

was supposed to work in the 213 NPA. I have tried both "KP + 001

+ 213 + 7D + ST", and some other area codes. I generally get

nothing, a reorder signal, or a tandem recording.

Here's some food for thought: On an official Telco sheet I

have, labeled " 213 NPA MF Routing Codes", 001 is listed as "VFY

BY", or verify busy for the 213 NPA. 002 is listed for the 805

NPA. Ma Bell likes to have standardized routing codes, such

logical, then, that 001 would be a sort of "standard" verify

code, and other prefixes would be tacked on at 002,003, etc.

However, I have heard from a retired operator that verification

codes are different from area to area, and are not always nice

numbers like 001, 002. Ah, well, a guy can hope, can't he?

_______________________________________________________________________

Copyright 1990

Tutorials                                                     25

_______________________________________________________________________

Some suggestions for future attacks on this dilemma: Every#

one call your operators and subtly ask questions. I have found

they tend to give information out easier if you ask for something

that you would ordinarily have to be a company employee to know

about, such as rate steps, operator routings, etc.

Casually let slip that you used to be (or still are) an

operator, or that you work for company security. Also, you might

want to blue box some codes like 001 followed by your NPA and the

last 7D of a busy number. If you get a sort of "whispery noise",

try blasting the line with a ringing signal (you might piggyback

another line onto yours and call the piggyback to generate the 90

volts) and see if that does anything.

_______________________________________________________________________

M & M Enterprises

26                                               The Boxing Book

_______________________________________________________________________

Chapter 3

International Dialing

---------------------

INTERNATIONAL DIALING by Dr Jimmy and Mr Jim

---------------------

The Phun of International Calling

---------------------------------

-or-

How you can be an diplomat for the United States

------------------------------------------------

There are several ways to complete the overseas connection.

One is to use extenders (800 #s) with international calling

capability, this is easy for most people since it can be done

from fortress phones (payphones), what a mind blower, calling a

overseas payphone from a payphone. But 800 #s can be traced.

Another bummer is that they can not be called from the

Lyons' Teleconferencing Network, well not directly that is.

Another alternative is to use a PBX or PABX system which will

dial an outside line, well the 800 extenders are also PBXs usual#

ly, but PBXs which do not have an NPA of 800 are only good for

local calls can be called in from the teleconference, but the

connection is not that great. I know, because on a recent tele#

conference we called a girl in London and the American Embassy in

Iran, and a couple of Australian payphones. Another problem with

bringing PBXs and extenders into the teleconference is getting

rid of them if the phone answers busy or doesn't answer. You have

to blast 2600Hz on the teleconference, this can trunk many people

on line. Unfortunately at the present time, international calling

and Lyons' teleconferencing do not go well together. This fact

shattered this phreak's dream of a worldwide teleconference.

Enough about that. Here are some ways to calling interna#

tionally with a Blue Box or anything that generates MF tones.

First of all, seize a tandem in the usual way and perform this

sequence of events:

KP + NPA + ZZZ + ST

Where KP and ST are the standard tones you all familiar

with, and NPA, ZZZ are the Number Planning Area and number of the

overseas sender respectively. (They will be listed later.) Now

you should here a slight clunk-cheep or CLUNK, depending on where

you are. Then a continuous carrier is heard. This is what you

want. Now do this sequence of events:

KP + XCC + SC + LN + ST

Where X is a 1 for cable connection

0 for satellite connection

_______________________________________________________________________

Copyright 1990

27                                        International Dialing

_______________________________________________________________________

CC is the two or three digit country code

SC is the City Code of the city you wish to call

LN is the Local Number, this can be from three to eight digits.

Also since you are using a MFer you can reach inward opera#

tors. I do not know if this can be done from an PBX or extender.

The list of some the country codes and city codes are listed

at the end of this file, or are available from the phone book

(medium to large cities with direct international calling avail#

able through 011, usually are listed in the front few pages.)

OR you can do it as the Doctor does, make the phone company

operators work for their money, use Rate and Route.

To call up Rate and Route, get on your favorite trunk and

beep this:

KP + 800 + 141 + 1212 + ST

Now you may note, this has a NPA of 800, but it cannot be

directly dialed through DTMF or rotary since it is an invalid

exchange to you the subscriber.

When you call this up, you will be connected with a TSPS

position and if all the positions are busy, it will give you a

recording: 'Please hold on. All positions are busy at the moment

and your request will be answered as soon as possible.' Wait, it

really does answer.

When you are hooked up to the Rate and Route operator ask

for:

TSPS International Inward Operators Route to ....., or

TSPS International Directory Assistance Route to .....

where ..... is the city and country NAME, not code. and she/he

will say: "Right. That's  county code CC, city code SC, and then

the number is #####."

And these numbers do vary. London information is:

44 + 1 + 986 + 3611

and Sydney, Australia is:

61 + 2 + 0143

I am not completely certain, but I believe when you MF into

a non-English speaking country, Inward and D. A. Ops speak Eng#

lish because I believe they know it is coming from the United

States. At least this happened with me when I called German D. A.

I have not called any country, other than Germany, which

does not have English as its main language.

_______________________________________________________________________

M & M Enterprises

28                                               The Boxing Book

_______________________________________________________________________

A list of the overseas senders:

Location       NPA         Number

--------       ---         ------

White Plains, NY     914          182

New York City        212          183

Pittsburgh, PA       412          184

Orlando, FL          305          185

Oakland, CA          415          186

Denver, CO           303          187

Overseas Boxing By King Blotto

---------------

The miracles of a blue box can't unfold, until one takes

full advantage of it by calling overseas. There has been much

speculation about boxing overseas, and alot of B.S.

First off, there is no such thing as KP2! Or 11 or 12 tones.

This is just somebody's ramblings! There is a KP special tone,

but more on that some other day. Anyways, the correct procedure

to calling international via blue box is:

KP011CCCS <- where CCC is the country code. And it

must be 3 digits long. If the code is

only 2 (e.g. England), then a '0' must

prefix it.

Second sequence is :

KP01CCXXXxxxx <- where CC is the 2 digit city code

with a '0' used as a prefix if less than

two digits. XXX-xxxx is the local number.

Don't forget the ST tone at the end.

Anyways, an example call to England, to the Kelshire Hilton

in London would go:

KP011044ST

KP01016035342ST

You should already know how to seize a tandem. But, as a

tip, you should use something that is used to being used for long

periods of time:

_______________________________________________________________________

Copyright 1990

29                                        International Dialing

_______________________________________________________________________

The History Of British Phreaking by Lex Luthor

--------------------------------

(With thanks to Peter McIvers for the list of frequencies

mentioned later in this file.)

NOTE: the British Post Office, is the U.S. equivalent of Ma

Bell.

In Britain, phreaking goes back to the early fifties, when

the technique of 'Toll A drop back' was discovered. Toll A was an

exchange near St. Paul's which routed calls between London and

nearby non-London exchanges. The trick was to dial an unallocated

number, and then depress the receiver-rest for 1/2 second. This

flashing initiated the 'clear forward' signal, leaving the caller

with an open line into the Toll A exchange. He could then dial

018, which forwarded him to the trunk exchange- at that time, the

first long distance exchange in Britain- and follow it with the

code for the distant exchange to which he would be connected at

no extra charge.

The signals needed to control the UK network today were

published in the "Institution of Post Office Engineers Journal"

and reprinted in the Sunday Times (15 Oct. 1972).

The signaling system they use: signaling system No. 3 uses

pairs of frequencies selected from 6 tones separated by 120Hz.

With that info, the phreaks made "Bleepers" or as they are called

here in the U.S. "Blue Box", but they do utilize different MF

tones then the U.S., thus, your U.S. blue box that you smuggled

into the UK will not work, unless you change the frequencies.

In the  early seventies, a simpler system based on different

numbers of pulses with the same frequency (2280Hz) was used. For

more info on that, try to get ahold of: Atkinson's "Telephony and

Systems Technology".

The following are timing and the frequencies for boxing in

the UK and other foreign countries. Special thanks to Peter

McIvers for the following info:

British "bleeper" boxes have the very same layout as U.S.

blue boxes. The frequencies are different, though. They use two

sets of frequencies, forward and backward. Forward signals are

sent out by the bleeper box; the backward signals may be ignored

(it's sort of like using full duplex). The frequencies are as

follows:

U.S.:

UK:	     700    900    1100   1300	 1500	1700

Forward:    1380   1500    1620   1740	 1860	1980 Hz

Backward:   1140   1020    900	   780	  660	 540 Hz

_______________________________________________________________________

M & M Enterprises

30                                               The Boxing Book

_______________________________________________________________________

For example, change the 900 Hz potentiometers in your box to

1500 Hz. All numbers 1-0 (10) are in the same order as in an

American box. The ones after this are their codes for operator

11, operator 12, spare 13, spare 14, and 15.

One of these is KP, one (probably 15) is Star; it won't be

too hard to figure out. The signals should carry -11.5dBm +/- 1dB

onto the line; the frequencies should be within +/- 4Hz (as is

the British equipment). Also, the 1VF system is still in opera#

tion in parts of the U.K. This would encode all signals 1 to 16

as binary numbers; for instance, a five is 0101. There are six

intervals per digit, each 50ms long for a total of 300ms. First

is a start pulse of 2280 for 50ms. Then, using the example of

five (0101), there is a 50ms pause, a 50ms pulse of 2280, a 50ms

pause, and a 50ms pulse of 2280. Finally, there is a 50ms pause

that signals the end of the digit. The frequency tolerance on the

2280 Hz is +/- 0.3%; it is sent at -6 +/- 1dBm.  An idle line is

signaled by the presence of a 3825Hz tone for more than 650ms.

This must be within 4Hz.

France uses the same box codes as the US, with an additional

1900Hz acknowledgement signal, at -8.7 +/- 1dBm per frequency.

Spain uses a 2 out of 5 mf code (same frequencies as US),

with a 1700 Hz acknowledge signal.

Other places using the 1VF system are:

Australia, 2280Hz +/-6Hz, 35ms/digit at -6dB.

Germany, France: same as Australia; also, some 1VF systems in the

UK.

Switzerland: same as Australia, only it uses 3000Hz, not 2280.

Sweden: same as above, but at 2400Hz.

Spain: some parts use 1VF with 2500Hz.

There is one other major system: the 2VF system. In this

system, each digit is 35ms long. The number is encoded in binary

as with the 1VF system. Using the example of five (0101), here's

how the American 2VF system was sent:

2400 pulse, pause, 2040 pulse, pause, 2400 pulse, pause, 2040

pulse, pause. The digits and pauses are all 35ms long, for a

total of 280ms per digit. Other countries are still using a

similar high/low pair with the same timings. Some parts of Italy

use the 1VF system with 2040Hz; some use the 2VF system with 2040

and 2400 (same as original US) Hz. The Netherlands uses a 2VF

system with 2400 and 2500 Hz pulses. With the 2VF system, all

frequencies should be within 2Hz.

_______________________________________________________________________

Copyright 1990

31                                        International Dialing

_______________________________________________________________________

Also, here are some specs for American phone equipment:

Dial Tone: 350+440Hz, -17.5 to -14.5 dBm/tone.

Off-Hook (ROH): 1400+2060+2450+2600(!) on/off 5 times per second

Busy: 480+620Hz; slow busy: 0.5 +/- 0.05 sec = 1 period

(about twice a second), at -28.5 to -22.5 dBm/tone.

Ring: 440+480 Hz at -23.5 to -20.5 dBm/tone.

A ring is modulated at 20 +/- 3Hz, 2sec on, 4sec off.

Call waiting: 440Hz, on 1 second.

Recorder Connection: 1400Hz, beeps every 15minutes.

Multiparty line ring: same frequency and modulation as ring, but

1sec on, 2sec off (twice as fast).

Now, back to British Phreaking:In the early days of British

phreaking, the Cambridge University Titan Computer was used to

record and circulate numbers found by the exhaustive dialing of

local networks. These numbers were used to create a chain of

links from local exchange to local exchange across the country,

bypassing the trunk circuits. Because the internal routing codes

in the UK network are not the same as those dialed by the caller,

the phreaks had to discover them by 'probe and listen' techniques

or more commonly known in the U.S.--SCANNING. What they did was

put in likely signals and listened to find out if they succeeded.

The results of scanning were circulated to other phreaks. Discov#

ering each other took time at first, but eventually the phreaks

became organized. The "TAP" of Britain was called "Undercurrents"

which enabled British phreaks to share the info on new numbers,

equipment etc.

To understand what the British phreaks did, think of the

phone network in three layers of lines: Local, trunk, and inter#

national. In the UK, Subscriber Trunk Dialing (STD), is the

mechanism which takes a call from the local lines and (legiti#

mately) elevates it to a trunk or international level. The UK

phreaks figured that a call at trunk level can be routed through

any number of exchanges, provided that the right routing codes

were found and used correctly. They also had to discover how to

get from local to trunk level either without being charged (which

they did with a bleeper box) or without using (STD). Chaining has

already been mentioned but it requires long strings of digits and

speech gets more and more faint as the chain grows, just like it

does when you stack trunks back and forth across the U.S. The way

the security reps snagged the phreaks was to put a simple 'print#

ermeter' or as we call it: a pen register on the suspects line,

which shows every digit dialed from the subscribers line.

_______________________________________________________________________

M & M Enterprises

32                                               The Boxing Book

_______________________________________________________________________

The British prefer to get onto the trunks rather than chain#

ing. One way was to discover where local calls use the trunks

between neighboring exchanges, start a call and stay on the trunk

instead of returning to the local level on reaching the distant

switch. This again required exhaustive dialing and made more work

for Titan; it also revealed 'fiddles', which were inserted by

Post Office Engineers. What fiddling means is that the engineers

rewired the exchanges for their own benefit. The equipment is

modified to give access to a trunk with out being charged, an

operation which is pretty easy in Step by Step (SXS) electrome#

chanical exchanges, which were installed in Britain even in the

1970s (NOTE: I know of a back door into the Canadian system on a

4A CO., so if you are on SXS or a 4A, try scanning 3 digit ex#

changes, i.e.,: dial 999,998,997 etc. and listen for the beep-

kerchink, if there are no 3 digit codes which allow direct access

to a tandem in your local exchange and bypasses the AMA so you

won't be billed, not have to blast 2600 every time you wish to

box a call.

A famous British 'fiddler' revealed in the early 1970s

worked by dialing 173. The caller then added the trunk code of 1

and the subscribers local number. At that time, most engineering

test services began with 17X, so the engineers could hide their

fiddles in the nest of service wires. When security reps started

searching, the fiddles were concealed by tones signaling: 'number

unobtainable' or 'equipment engaged' which switched off after a

delay. The necessary relays are small and easily hidden.

There was another side to phreaking In the UK in the six#

ties. Before STD was widespread, many 'ordinary' people were

driven to occasional phreaking from sheer frustration at the

inefficient operator controlled trunk system. This came to a head

during a strike about 1961 when operators could not be reached.

Nothing complicated was needed. Many operators had been in the

habit of repeating the codes as they dialed the requested numbers

so people soon learnt the numbers they called frequently. The

only 'trick' was to know which exchanges could be dialed through

to pass on the trunk number. Callers also needed a pretty quiet

place to do it, since timing relative to clicks was important.

The most famous trial of British phreaks was called the Old

Baily trial. Which started on 3 Oct. 1973. What they phreaks did

was to dial a spare number at a local call rate but involving a

trunk to another exchange. Then they send a 'clear forward' to

their local exchange, indicating to it that the call is finished;

but the distant exchange doesn't realize because the caller's

phone is still Off the hook. They now have an open line into the

distant trunk exchange and sends to it a 'seize' signal: '1'

which puts him onto its outgoing lines. Now, if they know the

codes, the world is open to them. All other exchanges trust his

local exchange to handle the billing; they just interpret the

tones they hear.

_______________________________________________________________________

Copyright 1990

33                                        International Dialing

_______________________________________________________________________

Meanwhile, the local exchange collects only for a local

call. The investigators discovered the phreaks holding a confer#

ence somewhere in England surrounded by various phone equipment

and bleeper boxes, also printouts listing 'secret' Post Office

codes. (They probably got them from trashing?) The judge said:

"some take to heroin, some take to telephones" for them phone

phreaking was not a crime but a hobby to be shared with fellow

enthusiasts and discussed with the Post Office openly over dinner

and by mail. Their approach and attitude to the worlds largest

computer, the global telephone system, was that of scientist s

conducting experiments or programmers and engineers testing

programs and systems. The judge appeared to agree, and even asked

them for phreaking codes to use from his local exchange!!!

_______________________________________________________________________

M & M Enterprises

34                                               The Boxing Book

_______________________________________________________________________

Chapter 4

The Boxes

---------

The Aqua Box by Kopy Kat

------------

For a long time, it was impossible to escape from the lock

in trace, this box does offer an escape route with simple direc#

tions to it. This box is quite a simple concept, and almost any

phreaker with basic electronics knowledge can construct and use

it.

The Lock in Trace

-----------------

A lock in trace is a device used by the F.B.I. to lock into

the phone users location so that he can not hang up while a trace

is in progress. For those of you who are not familiar with the

concept of `Locking In', then here's a brief explanation.

The F.B.I. can tap into a conversation, sort of like a

three-way call phone line. All the phone connections are held

open by a certain voltage of electricity, that is why you some#

times get static and a faint connection when you are calling far

away, because the electricity has trouble keeping the line up.

What the lock in trace does is cut into the line and generate

that same voltage straight into the lines. That way, when you try

to hang up, voltage is retained. Your phone will ring like some#

one was calling you even after you hang up. (If you have call-

waiting, you should understand better about that.)

So, in order to eliminate the tracer, you must lower the

voltage on your phone line.  You should know that every time

someone else picks up the phone line, then the voltage decreases

a little. In the first steps of planning this out, K.K. suggested

getting about hundred phones all hooked into the same line that

could all be taken off the hook at the same time. That would

greatly decrease the voltage level. That is also why most three-

way connections that are using Bell service three way calling

become quite faint after a while.

By now, you should understand the basic idea. You have to

drain all of the power out of the line so the voltage can not be

kept up.  A rather sudden draining of power would quickly defeat

the F.B.I's voltage machine, because it was only built to sustain

the exact voltage necessary to keep the line open.

For now, imagine this: One of the normal Radio Shack genera#

tors that you can go pick up one end of the cord that hooks into

the central box has a phone jack on it and the other has an

electrical plug. This way, you can "Flash" voltage through the

line, but cannot drain it. So, some modifications have to be

done.

_______________________________________________________________________

Copyright 1990

35                                                    The Boxes

_______________________________________________________________________

The Aqua Box

------------

Materials Needed-

----------------

A BEOS (Basic Electrical Output Socket), like a small lamp

type connection, where you just have a simple plug and wire

that would plug into a light bulb.

One of the cords mentioned above, if you can't find one then

construct your own...  Same voltage connection, but the

restrainer must be built in. (I.E. the Central Box).

Two phone jacks (one for the modem, one for if you are being

traced, to plug the aqua box into).

Some creativity and easy work.

NOTICE:  No phones have to be destroyed/modified to make

this box, so don't go out and buy a new phone for it!

All right, this is a very simple procedure. If you have the

BEOS, it could drain into anything, a radio, or whatever.

1) Take the connection cord. Examine the plug at the end. It

should have only two prongs, if it has three, still, do not

fear. Make sure the electrical appliance is turned off

unless you want to get fried the quick way while making this

thing. Most plugs will have a hard plastic design on the top

of them to prevent you from getting in at the electrical

wires inside. Well, get a nice sharp blade and remove it. If

you want to keep the plug, ( I don't see why..) then just

cut the top off. When you look inside, low and behold, you

will see that at the base of the prongs there are a few

wires connecting in. Those wires conduct the power into the

appliance. So, you carefully unwrap those from the sides and

pull them out until they are about an inch ahead of the

prongs. If you don't want to keep the jack, then just rip

the prongs out. If you are, cover the prongs with insulation

tape so they will not connect with the wires when the power

is being drained from the line.

2) Do the same thing with the prongs on the other plug, so you

have the wires evenly connected. Now wrap the end of the

wires around each other. If you happen to have the other end

of the voltage cord hooked into the phone, stop reading now,

you're too stupid to continue. After you've wrapped the

wires around each other, then cover the whole thing with the

plugs with insulating tape. Then, if you build your own

control Box or if you bought one, then cram all the wires

into that and reclose it. That box is your ticket out of

this.

_______________________________________________________________________

M & M Enterprises

36                                               The Boxing Book

_______________________________________________________________________

3) Re-Check everything to make sure it's all in place. This is

a pretty flimsy connection, but on later models when you get

more experienced at it then you can solder away at it and

form the whole device into one big box, with some kind of

cheap MATTEL hand-held game inside to be the power connect#

er.

In order to use this, just keep this box handy. Plug it into

the jack if you want, but it will slightly lower the voltage so

it isn't connected. When you plug it in, if you see sparks,

unplug it and restart the whole thing. But if it just seems fine,

just leave it.

Using it!!

----------

Now, so you have the whole thing plugged in and all. Do not

use this unless the situation is desperate! When the trace has

gone on, don't panic, unplug your phone, and turn on the appli#

ance that it was hooked to. It will need energy to turn itself

on, and here's a great source. Just plug it into the phone jack

and your troubles are over! The voltage to keep a phone line open

is pretty small and a simple light bulb should drain it all and

you will no longer need to worry about the system attempting to

invade upon your privacy.

Introducing the Beige Box by The Exterminator & The Terminal Man

-------------------------    (5/17/85)

INTRODUCTION

------------

Have you ever wanted a lineman's handset? Surely every

phreak has at least once considered the phun that he could have

with one. After searching unlocked phone company trucks for one

for months, we had an idea. We could build one. We did, and named

it the "Beige Box, " simply because that is the color of ours. In

the following file we will give complete instructions for the

construction and use of a Beige Box.

CONSTRUCTION

------------

The construction is very simple. First, you must understand

the concept of the device. In a modular jack, there are four

wires. These are red, green, yellow, and black. For a single line

telephone, however, only two matter: the red (ring) and green

(tip). The yellow and black are not necessary for this project. A

lineman's handset only has two clips on it: the ring and the tip.

_______________________________________________________________________

Copyright 1990

37                                                    The Boxes

_______________________________________________________________________

You will need a phone (we recommend a touch-tone) with a

modular plug, a modular jack, and two large alligator clips

(preferably red and green, respectively). Take the modular jack

and look at the bottom of its casing. There should be a gray jack

with four wires (red, green, yellow, and black) leading out of

it. To the end of the red wire attach a red alligator clip. To

the end of the green wire attach a green alligator clip. The

yellow and black wires can be removed, although I would only set

them aside so that you can use the modular jack in future

projects. Now insert your telephone's modular plug into the

modular jack. That's it.

This particular model is nice because it can be easily made,

is inexpensive, uses common parts that are readily available, is

small, is lightweight, and does not require the destruction of a

phone.

BEIGE BOX USES

--------------

There are many uses for the Beige Box. However, before you

can use it, you must know how to attach it to its output device.

This device can be any of several Bell switching apparatus that

include terminal sets (i.e., remote switching centers, bridging

heads, cans, etc.). To open most Bell Telephone switching appara#

tus, you must have a 7/16 inch hex driver. This piece of equip#

ment can be picked up at your local hardware store. With your

7/16 hex driver, turn the security bolt(s) approximately 1/8 of

an inch counter-clockwise and open. If your output device is

locked, then you must have some knowledge of destroying and/or

picking locks. However, we have never encountered a locked output

device. Once you have opened your output device, you should see a

mass of wires connected to terminals. On most of your output

devices, the terminals should be labeled "T" (Tip - if not la#

beled, it is usually on the left) and "R" (Ring - if not labeled,

it is usually on the right). Remember: Ring - red -  right. The

"Three R's" -- a simple way to remember which is which.

Now you must attach the red alligator clip (ring) to the "R"

(ring) terminal. Attach the green alligator clip (tip) to the "T"

(tip) terminal. NOTE: If instead of a dial tone you hear nothing,

re-adjust the alligator clips so that they are not touching each

other or other terminals. Also make sure that they are firmly

attached. By this time you should hear a dial tone. Dial ANI and

find out the number that you are using (you wouldn't want to use

your own).

_______________________________________________________________________

M & M Enterprises

38                                               The Boxing Book

_______________________________________________________________________

Here are some practical applications:

o  Eavesdropping

o  Long distance, static-free phone calls to friends

o  Dialing direct to Alliance Conferencing (also static-free)

o  Phucking people over

o  Bothering the operator at little risk to yourself

o  Blue Boxing with a greatly reduced chance of getting caught

o  Anything at all that you want, since you are an extension

on that line

Eavesdropping:

-------------

To be most effective, first attach the Beige Box and then

your phone. This eliminates static caused by connecting the box,

therefore reducing the potential suspicion of your victim. When

eavesdropping, it is always best to be neither seen nor heard. If

you hear someone dialing out, do not panic; but rather hang up,

wait, and pick up the receiver again. The person will either have

hung up or tried to complete their call again. If the latter is

true, then listen in, and perhaps you will find information

worthy of blackmail! If you would like to know who you are lis#

tening to, after dialing ANI, pull a CN/A on the number.

Dialing Long Distance:

---------------------

This section is self-explanatory, but don't forget to dial a

"1" before the NPA.

Dialing Direct to Alliance Conferencing:

---------------------------------------

Simply dial 0-700-456-1000 and you will get instructions

from there. I prefer this method over PBXs, since PBXs often have

poor reception and are more difficult to come by.

Phucking People Over:

--------------------

This is a very large topic for discussion. Just by using the

other functions described, you can create a large phone bill for

the person (they will not have to pay it, but it is a hassle for

them). In addition, since you are an extension of the person's

line, leave your phone off hook, and they will not be able to

make or receive calls. This can be extremely nasty because no one

would suspect the cause of the problem.

_______________________________________________________________________

Copyright 1990

39                                                    The Boxes

_______________________________________________________________________

Bothering the Operator:

----------------------

This is also self-explanatory and can provide hours of

entertainment. Simply ask or say things to her that are offensive

and you would not like traced to your line. This also corresponds

with the previous described section, Phucking People Over. After

all, guess who's line it gets traced to?

Blue Boxing:

-----------

See a file on Blue Boxing for more details. This is an

especially nice feature if you live in an ESS-equipped prefix,

since the calls are, once again, not traced to your line.

POTENTIAL RISKS OF BEIGE BOXING

-------------------------------

Overuse of the Beige Box may cause suspicions within the

Gestapo, and result in legal problems. Therefore, I would recom#

mend that you:

o  Use more than one output device

o  Choose a secluded spot to do your Beige Boxing

o  Keep a low profile (i.e., do not post under your real name

on a public BBS concerning your accomplishments)

o  In order to make sure that the enemy has not been inside

your output device, I recommend that you place a piece of

transparent tape over the opening of your output device.

Therefore, if it is opened in your absence, the tape will be

displaced and you will be aware of the fact that someone has

been intruding upon your territory.

Beige Boxing By Cardiac Arrest [09/26/89]

------------

Introduction

------------

Well, I KNOW that nearly everybody and their brother knows

how to beige box, but what magazine is complete without a file as

basic as that. Anyways, if you know how to beige box, and consid#

er yourself master beiger, skip this and go on to the next file.

Otherwise, I'll try to help beginners and maybe give some experi#

enced boxers food for thought.

_______________________________________________________________________

M & M Enterprises

40                                               The Boxing Book

_______________________________________________________________________

What IS Beige Boxing

--------------------

If you've ever paid any attention to the phone company,

you've definitely seen a guy in funny Ma Bell overalls running

around with a funny-looking telephone with alligator clips coming

out the bottom. That's the Ma Bell version of the "beige box",

called a Lineman's Handset. There are literally TONS of uses for

a beige boxes, and they are simple to make, so it's usually a

good introduction to the phreaking world.

The Purpose Of This File

------------------------

If even one person reads this file and learns something,

I've accomplished what I set out to do (how cliche, right?). But

seriously, I'm going to attempt to provide several easy methods

of beige boxing. Some experienced beigers will definitely see

some familiar designs, but they might also see a new twist or

two. I'll also include (hopefully) easy but complete directions

of some of the possibilities for use.

Back To Reality

---------------

Ok, on with the file. There are about as many beige box

designs as there are uses, and with both, new ideas are always

popping up. The designs in this file are by no means the best

designs. I HOPE that they're some of the easiest, but who am I to

say.

Method #1 (Generic, Phone Destroying, Design)

---------------------------------------------

Required Materials

1  Telephone that you wont miss (it'll be a permanent beige

box)

2  Alligator clips

1  Telephone cord

1  Screwdriver

1  Pair of wire cutters

1  Soldering iron

Solder

Construction

1.  Open up the telephone with the screwdriver. I can't give

exact directions, because different models vary, but if

you can't find the screws, try checking under the plastic

plate that holds the phone number of the location.

2.  Look at the modular jack (the thing the phone cord plugs

into). Find the red and green wires. These are the ones

you want. Trace these wires with your finger to the screw

_______________________________________________________________________

Copyright 1990

41                                                    The Boxes

_______________________________________________________________________

that holds them down. Connect your phone cord to these

screws, either by soldering them, or by wrapping them

around the screw and tightening it down.

3.  Run the telephone cord out the modular jack's hole. If

you can't squeeze it through the jack, take the wire

cutters the cut the wires leading to  it, and yank it

out. That should leave plenty of room.

4.  Re-assemble your phone.

5.  At the end of the telephone cord hanging out of the

phone, connect the alligator clips to the same wires

hooked up to the screws inside the housing of the phone.

You can connect them either by soldering, or by splicing

the wire to them (twisting them around the hole and

praying that it holds).

Method #2 (A spin-off of #1, but less permanent)

------------------------------------------------

Required Materials

1  Telephone (Don't worry, you wont wreck this one)

1  Telephone cord (You can use one of the springy ones that

you always tangle up when you're on the phone)

2  Alligator clips

1  Pair of wire cutters

1  Soldering iron

Solder

Construction

1.  Cut the modular plug (the thing that plugs into the wall

or telephone set) off ONE end of the telephone cord.

2.  Find the red and green wires and connect the alligator

clips to these by soldering or splicing them.

3.  Connect the other end (the that still has a plug) to a

telephone.

Method #3 (Similar to #2, but using a wall jack vice a cord)

------------------------------------------------------------

Required Materials

1  Telephone (This wont get wrecked, either)

1  Modular telephone wall jack (This WILL get wrecked)

2  Alligator clips

1  Pair of wire cutters

1  Soldering iron

Solder

_______________________________________________________________________

M & M Enterprises

42                                               The Boxing Book

_______________________________________________________________________

Construction

1.  Look on the back of the wall jack. You should see the

typical red and green wires going into the back of the

jack. Leave the end going into the jack alone, but trace

them to where the go into the plate holding the jack. Cut

them here (being sure, as I said, to leave the jack end

alone).

2.  Hook the alligator clips up to the red/green wires.

3.  Plug the phone into the wall jack.

Testing Your Box

----------------

Ok, now that you've got one of the boxes described above (or

a different one), you're ready to go. Go outside, and on the side

of your house, you should be able to find a small, approximately

3" X 3", puke-green box, with a bolt in the middle of it. Take a

wrench (I'm not sure what the size is, but a 10mm wrench works

for me, and that's all I really care about. But be careful, since

it's not exact, you might strip it) and take off the bolt. You'll

probably have to clear out some cobwebs, since it hasn't been

used in a while. Inside the box, you should see four screws (one

on each corner) with the typical red/green wires connected to

them.  (If you have two phone lines, the bottom screws will have

black/yellow wires, if you have one phone line, the bottom won't

have any). You can probably guess what happens from here--Hook

the alligator clips up to the screws. You should get a dial tone.

If you didn't, make sure the connection is clean, that you're

hooked up to the right terminals (screws), etc. If you still

don't get one, you're screwed. That means there's something wrong

with your box. If you do get a dial tone, you're probably guess#

ing what you can do from here.

Where Can You Use The Beige Box

-------------------------------

You can use the beige box on several pieces of equipment.

You can go to you're best friend's house and use it like I de#

scribed. You can open up one of those ugly green boxes about 3'

high in the back yard of every couple houses. Inside you'll see

pretty much the same thing as at individual houses, only there's

several houses running through the box, not just yours. I have

heard that you can use a beige inside a Ma Bell manhole, but I

crawled down one (not fun) and there was a huge plastic tube. You

can see the telephone wires inside, but I have no idea how to get

to them. There are definitely more uses, but these are the ones

I've been exposed to.

_______________________________________________________________________

Copyright 1990

43                                                    The Boxes

_______________________________________________________________________

The Box Of Many Uses

--------------------

As I've mentioned, there are TONS of uses for beige boxes,

and the ones I explain are merely the ones I've had some fun

with. It's all basically the same, but there are some interesting

twists.

Conferences

-----------

Definitely one of the funniest. It's easier to do than ex#

plain, but I'll give it a shot. First, call up a conference

service (I'll list them in a second). From here, you'll pretty

much get instructions (at least on the ones I've used). Basical#

ly, you call up your buddies, tell them what's going on, and hit

a key (usually *) and they get put into the conference. From

there, you and all your friends can all talk to each other, trade

codes, etc. Get the idea?  (You can even call foreign numbers. On

our conference, we voiced a user from Italy and called a hotel in

Madrid for someone to practice Spanish).

Conference Services :

0-700-456-1000     0-700-456-1001     0-700-456-1002

0-700-456-1003     0-700-456-1004     0-700-456-2000

0-700-456-2001     0-700-456-2002     0-700-456-2003

0-700-456-2004

Tapping

-------

If you hook up your beige box, and hear voices, the rightful

owner of the line is obviously using it.  Well, that's about all

there is to phone tapping.  Just shut up and listen.

L/D Calling

-----------

Hey, it's not YOUR bill, so go ahead and call your pal in

France.  Maybe voice verify some users on your BBS.

Conclusion

----------

That's about it. I won't pretend to be an expert on beige

boxes, so I won't say that these are the limits, or that these

are the best methods. I'm just trying to provide a non-technical

introduction to phreaking.

_______________________________________________________________________

M & M Enterprises

44                                               The Boxing Book

_______________________________________________________________________

Introduction To Beige Boxing By Shooting Shark [1/13/85]

----------------------------

Beige Boxes are amazingly simple to make. They are just test

sets, which are used by telephone linemen for connecting to any

phone line. And test sets are just ordinary telephones. They are

called beige boxes because they are a combination of a brown box,

a device for listening into phone lines, and a white box, which

is a portable touch-tone generator, sold by Radio Shack and AT&T.

(AT&T actually colors theirs white, which I find hilarious.)

Beige Boxing is easy, fun, and offers dialing options often

unavailable through LDC ports or extenders - you are physically

using an unsuspecting person's phone line for making calls any#

where - even to the 0-700 and 900 SACs (Special Area Codes) and

976- numbers. The only reason I am writing this file is to make

this enjoyable hobby aware to phreaks who want something differ#

ent to try.

Building The Beige Box

----------------------

Take any phone (a touch-tone phone is preferred) and strip

off the modular plug down to four wires. Connect heavy-duty

alligator clamps to the red and green wires (these are the only

wires you really need to make phone calls).

Using The Box

-------------

Drive to a downtown location. Go behind a business, and look

for any metal boxes on the wall that say General Electric, AT&T,

the name of your BOC etc. Open the box. You may find wires of

four colors leading to bolts. Just find any two places where a

red and green wire terminate and connect the alligator clips of

your Beige Box to these wires. If you don't get a dial tone, try

another combination of whatever red and green wire endings you

find (If the box has non-standard wire coloring, you are in for a

bit more work). Once you have a hookup where you have a dial

tone, you're ready to go. Note - if you get a connection where

you hear talking, HANG UP and note the connection as a good one.

That line is just being used now. If you are daring, speak into

the mouthpiece, and tell them that you are a telephone repairman,

and that they should not use the phone for an hour. This gives

you time to call Dial-A-Muppet, set up your conference, etc!

Another good source of phone wiring is an apartment complex

- all the connections for the various apartments will be in one

box, giving you many choices! This can be a little more danger#

ous. If it is in a quiet area, you may find yourself being stared

at from a second-story window by an old woman in curlers who

simply wondered why there was a conversation going on by her

trash can. One possible remedy to this is to get a hat that has

the emblem of your local BOC - If anybody asks you what you're

doing, cheerfully say "Telephone maintenance!". A fellow phreak

named David Letterman successfully ran a tap from the box of his

_______________________________________________________________________

Copyright 1990

45                                                    The Boxes

_______________________________________________________________________

own apartment building through his window and to his telephone.

He knew the family who's phone bill he was adding to, and swore

that he knew nothing about it when their bill with $50 worth of

"unplaced calls" came. Pacific Bell reimbursed them, and he

removed the tap soon after. If you want to do this, you may even

consider connecting the guts of a cordless phone to the phone box

and keeping the handset in your home. That way, if someone goes

nosing around the tampered-with phone box, the source of the tap

will be untraceable.

One good thing to do is to call your local ANI # and the

number of the line you are borrowing will be read off to you. If

the robot voice seems to have trouble reciting the number, you

have a spare line, and you're only ripping off the phone company!

If not, you can then call that # from another location to find

out who it is, or perhaps to tell them that you are the phone

company and that they should not use their phone for a few hours.

In fact, if you are talking with someone on a line and the owner

of the line picks up his/her phone to make a call, tell them that

you are the telco, etc. etc, we're doing some testing, please

don't use your phone, etc. Works all the time!

BLACK BOX PLANS by Uncle Joe

---------------

Parts:

-----

SPST toggle switch

10K Ohm 1/2 Watt 10% resistor

Procedure:

---------

Cut two 6 inch lengths of wire

Attach these to the switch terminals

Open phone

Locate F and RR terminals in the phone

Wrap the resistor between these 2 terminals making sure it

only touches 2 terminals

Connect 1 wire from the switch to the RR terminal

Disconnect the green wire from its terminal

Attach this green wire to the remaining switch wire

Bring the switch out of the phone and close it up

_______________________________________________________________________

M & M Enterprises

46                                               The Boxing Book

_______________________________________________________________________

Switch to a position where you get a normal dial tone and mark

this NORMAL. Mark the other position FREE.

Instructions for use:

--------------------

When the phone rings, lift and drop the receiver as quickly as

possible

Make absolutely sure the phone has stopped ringing

Flip the switch to FREE

Pick up the phone and talk normally

Schematic:

blue wire ---|

|--- F

white wire --|    |

resistor

|

R R --- switch ---

|

green wire ------------------------

How to build a BLACK BOX From Sherwood Forest

------------------------

To all who dare --

What is a BLACK BOX? A BLACK BOX is a device that is hooked

up to your phone that fixes your phone so that when you get a

call, the caller doesn't get charged for the call. This is good

for calls up to 1/2 hour, after 1/2  hour the Phone Co. gets

suspicious, and then you can guess what happens.

The way it works:

----------------

What this little beauty does is keep the line voltage from

dropping to 10v when you answer your phone. The line is instead

kept at 36v and it will make the phone think that it is still

ringing while your talking. The reason for the 1/2 hour time

limit is that the Phone CO. thinks that something is wrong after

1/2 an hour of ringing.

All parts are available Radio Shack. Using the least possi#

ble parts and arrangement, the cost is $0.98! And that is parts

for two of them! Talk about a deal! If you want to splurge then

you can get a small PC board, and a switch. There are two sche#

matics for this box, one is for most normal phones. The second

one is for phones that don't work with the first. It was made for

use with a Bell Trimline touch tone phone.

_______________________________________________________________________

Copyright 1990

47                                                    The Boxes

_______________________________________________________________________

**  Schematic 1 for most phones  **

**         LED ON: BOX ON        **

FROM >--------------------GREEN->  TO

LINE >--|   1.8k  LED  |---RED--> PHONE

|--/</</<--|>--|

|              |

------>/<-------

SPST

Parts:  1 1.8k 1/2 watt resistor

1 1.5v LED

1 SPST switch

You  may just have two wires which  you connect together for

the switch.

**  Schematic 2 for all phones  **

**        LED ON: BOX OFF       **

FROM >---------------GREEN->  TO

LINE >-------      ---RED--> PHONE

|  LED |

-->/<--|>--

|         |

---/</</---

1.8k

Parts:  1 1.8k 1/2 watt resistor

1 1.5v LED

1 DPST switch

Here is the PC board layout that I recommend using. It is

neat and is very easy to hook up.

Schematic #1        Schematic #2

**************     ****************

*            *     *  -------     *

* --<LED>--- *     *  |     |     *

* |        | *     *  | <SWITCH>  *

* RESISTOR | *     *  | |      |  *

*        | | *     *  | |      /R *

* -------- | *     *  | |      \E *

* |        | *     *  | <LED>| /S *

* --SWITCH-- *     *  |      | \I *

*  |      |  *     *  |      | /S *

L *  |      |  * F L *  |      | |  * F

I>RED-      -RED>O I>RED-      ---RED>O

N>-----GREEN---->N N>-----GREEN------>N

E *            * E E *              * E

**************     ****************

_______________________________________________________________________

M & M Enterprises

48                                               The Boxing Book

_______________________________________________________________________

Once you have hooked up all the parts, you must figure out

what set of wires go to the line and which go to the phone. This

is because of the fact that LED's must be put in, in a certain

direction. Depending on which way you put the LED is what con#

trols what wires are for the line & phone.

How to find out:

---------------

Hook up the box in one direction using one set of wires for

line and the other for phone.

*NOTE*  For Model I switch should be OFF.

*NOTE*  For Model 2 switch should be set to side connecting

the LED.

Once you have hooked it up, then pick up the phone and see

if the LED is on. If it is, the LED will be lit. If is doesn't

light then switch the wires and try again. Once you know which

are which then label them.

*NOTE*  - If neither directions worked then your switch was

in the wrong position. Now label the switch in its current posi#

tion as BOX ON.

How to use it:

-------------

When the box is *ON* then you may only receive calls. Your

phone will ring like normal and the LED on the box will flash. If

you answer the phone now, then the LED will light and the caller

will not be charged

Hang up the phone after you are done talking like normal.

You will not be able to get a dial-tone or call when the box is

on, so turn the box *OFF* for normal calls. I don't recommend

that you leave it on all the time, as you don't want it to answer

when Ma Bell calls!

Building Your Own Blue Box By Jester Sluggo [11/27/86]

--------------------------

This Blue Box is based on the Exar 2207 Voltage Controlled

Oscillator. There are other ways to build Blue Boxes, some being

better and some not as good, but I chose to do it this way. My

reason for doing so: because at the time I started this project,

about the only schematic available on BBS's was the one written

by Mr. America and Nickie Halflinger. Those plans soon (in about

90 seconds) became very vague in their context with a couple in-

consistencies, but I decided to "rough it out" using those plans

(based on the Exar 2207 VCO) and build the Blue Box using that as

my guide. During the construction of the Blue Box, I decided to

type-up a "more complete and clear" set of Blue Box schematics

than the file that I based mine on, in order to help others who

_______________________________________________________________________

Copyright 1990

49                                                    The Boxes

_______________________________________________________________________

may be trying/thinking of building a Blue Box. I hope these help.

Note:  You should get a copy of the Mr. America/Nickie

Halflinger Blue Box plans. Those plans may be of help to anyone

who may have difficulty understanding these plans. Also, these

plans currently do not support CCITT.

Why should I build a Blue Box?

------------------------------

Many of you may have that question, and here's my answer.

Blue Boxing was the origin of phreaking (excluding whistling).

Without the advent of Blue Boxes, I feel that some of the ad#

vances in the telecommunications industry would've taken longer

to develop (The need to stop the phone phreaks forced AT+T Bell

Laboratories to "step up" their development to stop those

thieves!).

There is no harm in building a Blue Box (except the knowl#

edge you will gain in the field of electronics). Although there

are software programs (Soft Blue Boxes) available for many mi#

cro's that will produce the Blue Box Multi-Frequency (MF) tones,

they are not as portable as an actual Blue Box (you can't carry

your computer to a telephone, so you must use it from home which

could possibly lead to danger).

Many phreaks are announcing the end of the Blue Box Era, but

due to discoveries I have made (even on ESS 1A and possibly ESS

5), I do not believe this to be true. Although many people con#

sider Blue Boxing "a pain in the ass", I consider Blue Boxing to

be "phreaking in its' purest form". There is much to learn on the

current phone network that has not been written about, and Blue

Boxes are necessary for some of these discoveries. The gift of

free phone calls tends to be a bonus.

Note: Blue Boxes also make great Christmas gifts!

Items needed to construct a Blue Box.

-------------------------------------

Here is the list of items you will need and where you can

get them. It may be a good idea to gather some of the key parts

(the chips, and especially the potentiometers, they took about 6

months to back order through Digi-key. Also, basic electronics

tools will be necessary, and you might want to test the circuit

on a bread board, then wire-wrap the final project. Also, you

will need a box of some sort to put it in (like the blue plastic

kind at Radio Shack that cost around $5.00).

Note: An oscilloscope should be used when tuning in the

potentiometers because the Bell system allows

only a 7-10% tolerance in the precision of the

frequencies.

_______________________________________________________________________

M & M Enterprises

50                                               The Boxing Book

_______________________________________________________________________

Qty.  Item                 Part No.    Place

-------------------------------------------------

1  | 4 x 4 Keypad       |           | Digi-Key

6  | Inverter Chip      | 74C04     |

32 | Potentiometer      |           |

1  | 4-16 Converter Chip| 74LS154   |

1  | 16 Key Decoder     | 74C922    |

2  | 2207 VCO           | XR2207CP  | Exar Corp.

3  | .01 uf Capacitor   | 272-1051  | Radio Shack

5  | .1 uf Capacitor    | 272-135   | Radio Shack

2  | 1.5K Ohm Resistor  |           | Radio Shack

2  | 1.0K Ohm Resistor  |           | Radio Shack

1  | Speaker            |           | From an old Autovon phone

1  | 9 Volt Battery     |           | Anywhere

The resistors should be a +/- 5% tolerance.

The speaker can be from a regular telephone (mine just hap#

pened to be from an old Autovon phone). But make sure that

you remove the diode.

The Potentiometers should have a 100K Ohm range (but you may

want to make the calculations yourself to double check).

The 9-volt battery can be obtained for free if you use your

Radio Shack Free Battery Club card.

The Exar 2207 VCO can be found if you call the Exar Corp.

located in Sunnyvale, CA. Call them, and tell them the state

you live in, and they'll give the name and phone number to

the distributor that is located closest to you. The 2207

will vary from about $3.00 for the silicon-grade (which is

the one you'll want to use) to about $12.00 for the high-

grade Military chip.

Note:  When you call Exar, you may want to ask them to send

you the spec-sheets that gives greater detail as to the

operation and construction of the chip.

_______________________________________________________________________

Copyright 1990

51                                                    The Boxes

_______________________________________________________________________

Schematic Diagram

-----------------

+--------------+            +-------------+

|  1  2  3  A  |            |  Figure #1  |

|  4  5  6  B  |            +-------------+

|  7  8  9  C  |            | Logic Side  |

|  *  0  #  D  |            +-------------+

++-+-+-+-+-+-+-+

1 | 3 | 5 | 7 |           (VCC)

| 2 | 4 | 6 | 8           (+5 Volts)    +----+

| | | < u | | |             [+]         |   _|_

| | | | | | | |              |          |   \_/GND

+--+-+-+-+-+-+-+-+----+      +--+----------+---+

|  2 | 11| 10| 7 |    |      |  14         7   |

(.01C)  |  | 3 | 4 | 8 | 1  12+------+1                |

+--||---+5                  13+------+2   (*74C04*)    |

_|_      |                     |      |                 |

\_/GND   |     (*74C922*)      |      +-----------------+

+--||-+6                    |

|(.1C)|                     |

_|_    |                    |

\_/GND |   9  17 16 15 14  18|

+--+--+--+--+--+---+--+                          +---+

|  |  |  |  |   |                             |   |

_|_ A  B  C  D   |                            _|_  |

GND\_/ |  |  |  |  [+] (VCC)      [+] (VCC)      \_/  |

|  |  |  |      (+5 volts)  |  (+5 volts) GND  |

|  |  |  |                  |                  |

-------+--+--+--+------------------+----------------- |

|      23 22 21 20                 24             18+-+

+-----+12                                                 | |

|     |                 (*74LS154*)                     19+-+

_|_    |                                                   |

\_/GND |  1  2  3  4  5  6  7  8  9 10 11 13 14 15 16 17   |

+--+--+--+--+--+--+--+--+--+-+--+--+--+--+--+--+----+

1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16

|  |  |  |  |  |  |  |  | |  |  |  |  |  |  |

|

+-------------------------------+

|       +--+       +------->

|       |  |       |(Connects to Fig. 2)

+--+-------+--+-------+---+

|  3--|>o--4  5--|>o--6   |

|   (Invtr.)   (Invtr.)   |

+---------------+7                        |

_|_              |        (*74C04*)        |

GND\_/   (VCC) [+]--+14                       |

(+5 volts)   |                         |

+-------------------------+

_______________________________________________________________________

M & M Enterprises

52                                               The Boxing Book

_______________________________________________________________________

+-------------+                                _

|  Figure #2  |                               / |

+---+-------------+----+        +----------------+  |

| Tone Generation Side |       _|_               |  | SPKR

+----------------------+    GND\_/    +---+--+---+  |

|   |       \_|

|   |

|   |  +---------------+

+-------+                    |   |  |               |

|      _|_                   |   +--+14             |

|      \_/GND                |      |  (Repeat of)  |

|                            |      |    (First)    |

----- (.1C)                    |      |   (Circuit)   |

-----                          |      |               |

|                            |      | (*XR2207CP*)  |

|       +-----------------+  |   +--+6              |

|       |                 |  |   |  |               |

[+]-----+-------+1              14+--+   |  +---------------+

(VCC)            |                 |      +--------------------+

(+9 Volts)  +----+2                |                           |

|    |               12+---------------------+     |

(.01C) -----  |                 |                    _|_    |

-----  |  (*XR2207CP*)   |                    \_/GND |

|    |                 |       1.5K Ohms           |

+----+3              11+---+---\/\Rx/\/---+--+     |

|                 |   |              | _|_    |

|                 |   +---\/\Rx/\/---+ \_/GND |

|                 |       1.0K Ohms           |

|               10+----+                      |

+-------------+6               9+----+---+                  |

|             |                8+----+   |                  |

|             |                 |      ----- (.1C)          |

|             +-----------------+      -----                |

+---------+                             _|_      +----------+

|         | Pot.                     GND\_/ Pot. |          |

|        \/\/\/\/--+-----------------------\/\/\/\/         |

|         1400 Hz. |                        1600 Hz.        |

+---------+        |                             +----------+

|         | Pot.   |                        Pot. |          |

|        \/\/\/\/--+----------------+------\/\/\/\/         |

|         1500 Hz. |                |       900 Hz.         |

|                  |                |                       |

|     14 more      |                |       14 More         |

|   Potentiometers |                |     Potentiometers    |

|     in this      |                |       in this         |

|   area left out  |                |     area left out     |

|   for simplicity |                |     for simplicity    |

|                  |                |                       |

|                  |                |                       |

|

(Connects)  |

<-------------+

(Figure 1)

_______________________________________________________________________

Copyright 1990

53                                                    The Boxes

_______________________________________________________________________

Multiplex Keypad System

-----------------------

First, the multiplex pattern used in the 4x4 keypad layout.

I suggest that keys 0-9 be used as the Blue Box's 0-9 keys, and

then you can assign A-D, *, # keys to your comfort (i.e., * = Kp,

# = St, D = 2600, and A-C as Kp1, Kp2 or however you want).

Note: On your 2600 Hz. key (The D key in example above)

it may be a good idea to tune in a second

potentiometer to 3700 Hz. (Pink Noise).

Keypad      Key Assignments   Multiplex Pattern

+---------+   +-------------+    +------------+

| 1 2 3 A |   | 1  2  3  4  |    | 1  2  3  A |----Y1=8   X1=3

| 4 5 6 B |   | 5  6  7  8  |    | 4  5  6  B |----Y2=1   X2=5

| 7 8 9 C |   | 9  10 11 12 |    | 7  8  9  C |----Y3=2   X3=6

| * 0 # D |   | 13 14 15 16 |    | *  0  #  D |----Y4=4   X4=7

+---------+   +-------------+    +------------+

|  |  |  |

X1 X2 X3 X4

Schematic Help

--------------

This is the Key to the diagrams in the schematic. I hope

that they help more then they might hurt.

_|_

\_/GND   is the Ground symbol

| |

---| |--   is the Capacitor symbol

| |     (.1C)  stands for a .1 uf Capacitor

(.01C) stands for a .01 uf Capacitor

|

-----

-----     is another Capacitor symbol

|

--\/\Rx/\/-- is the Resistor symbol (The 1.5K Ohm and 1.0K Ohm

Resistors are at +/- 5% )

---+

|

\/\/\/\/-- is the Potentiometer symbol (The frequencies I

supplied above are just examples.)

--|>o--     is the Inverter symbol

_______________________________________________________________________

M & M Enterprises

54                                               The Boxing Book

_______________________________________________________________________

Blue Box Frequencies

--------------------

This section is taken directly from Mark Tabas's "Better

Homes and Blue Boxing" file Part 1.

Frequencies (Hz)  Domestic  Int'l

----------------------------------

700+900            1        1

700+1100           2        2

900+1100           3        3

700+1300           4        4

900+1300           5        5

1100+1300           6        6

700+1500           7        7

900+1500           8        8

1100+1500           9        9

1300+1500           0        0

700+1700          ST3p     Code 11

900+1700          STp      Code 12

1100+1700          KP       KP1

1300+1700          ST2p     KP2

1500+1700          ST       ST

2600+3700      *Trunking Frequency*

Note: For any further information about the uses or duration

of the frequencies, read the Mark Tabas files.

Now that you have built your blue box here's the correct way

to use it. Call any long distance information # through 1 or more

extenders. Enter a 2600 Hz tone just before information answers.

This will disconnect them and will place you in outward operator

mode. Enter a KP tone or KP2 if you're placing an international

call. This enables the dialing sequence. Enter the # you're

dialing using Multi Freq. tones. Use the format XXX-XXX-XXXX.

Enter an ST tone. This stops the dialing sequence and places

the call.

Before you do any of this it is important to realize that

AT&T doesn't look favorably upon people who use blue boxes. It's

a federal crime to use one and it is very easy to get caught if

you're not careful. Only you can decide whether or not you're

willing to risk putting yourself into very deep trouble.

As to use, the first thing you need to understand is that

there are two(2) types of toll completing trunks: INWARD and

OUTWARD. The names are reference to the office that is switching

the call (the toll center that serves the watts line you called)

and each type of trunk has a different class of service. From an

INWARD toll completing trunk, you  can reach the different serv#

ice operators, the toll test board, and the inward operator. Some

offices also allow remote testing and it is in these offices that

_______________________________________________________________________

Copyright 1990

55                                                    The Boxes

_______________________________________________________________________

you can access the outward toll completing trunks. The outward

trunks allow you to make verification (emergency) calls, do serv#

ice monitoring(tapping), stack trunks(busy out all trunks between

LA and NYC), enable and disable TSPS positions, and in some

cases(on some 4A's) issue temporary rerouting instructions(send

all calls from LA to NYC via Miami, Boston, or any other class 5

office or offices). Both type of trunks allow you to place a

'standard' call with a box.

In some offices, mostly the small ones with a toll test

board that is unattended at night and on weekends, you can get an

outward toll completing trunk as well as performing other test

and routing functions. You do this by using three digit codes

that are invalid exchanges(not of the pattern nnx;see note 1=).

During the Sixties, the codes used were fairly standard and

consistent; however, when the boxes became popular and the

phreaks started doing things like routing all calls from Dallas

to Ft. Worth via Washington D.C., Mother started changing the

test codes on a random(as far as I know ) basis. What I would

suggest is that everybody interested in doing this sort of thing

pick out a nice quiet little office somewhere and work on discov#

ering the codes acceptable to that office.

Each numbering plan area (NPA, also known as area code) has

an office designated as its master office. This office controls

all of the other toll offices in the area as well as serving as a

concentration point for most out of area calls. To access the

services of a non-master office you need it's 'city  code', this

is a three(3) digit code that is of the form 0xx, and is sent

after the area code;see note 2 =. As an example, the 'city code'

for Canton, Ohio is 042; thus to reach the inward operator in

Canton, you would send 'KP-216 -042-121-ST' where as if you

wanted the inward operator in Cleveland, you would send 'KP-216-

121- ST'. The reason this is necessary is that the operator in

Cleveland can't verify a number in Canton, so if you want to

verify someone in Canton you need the city code. Also, most area

master offices have  dedicated data trunks to the network control

center and thus don't accept test and rerouting commands over the

switched network.

Note 1:  the normal format for telephone numbers is as

follows: nyn/nnx-xxxx.  where n=any digit except 1 and 0; y=0 or

1, and x= any digit. Yes, I know that in some area codes the nnx

format has changed to nxx. This is a new occurrence and only

occur where there has been an outrageous population increase in

the last few years and all of the funny exchanges are connected

directly to master offices and thus don't conflict with the 'city

code' format.

_______________________________________________________________________

M & M Enterprises

56                                               The Boxing Book

_______________________________________________________________________

Now, if you're wondering about what to call from an operator

trunk, here are some goodies to help you out:

0-700-456-1000 Teleconference Line (free, because you

are the operator!)

xxx+101    - toll  switching

xxx+121    - local operator

xxx+131    - information

xxx+141    - rate & route

xxx+181    - coin refund operator

xxx+11501  - mobile operator

xxx+11521  - mobile operator

xxx+11511  - conference operator (800-544-6363)

Here are some examples of what you may find if you dialed KP

+ 213 + PXX + ST:

PXX      Effect

---------------------------

101      LA 2 Toll switching

117      Disconnect

121      213 Inward Op.

131      Local Information

162      DA (555-1212)

163      Information

167      818 DA

181      Dial Assistance

182-188   Satellite Senders

(kp+Country+city+#+St)

025      Spanish Translator!

Conclusion

----------

This is just one way to build a Blue Box. If you choose this

way, then I hope this file is adequate enough to aid you in the

construction. Although these are not the best plans, they do

work. This file does not tell you how to use it or what to do

once it's built. For that information I suggest that you read the

Mark Tabas's "Better Homes and Blue Boxing" files.

Brown Box Plans

---------------

Introduction

------------

This is a fairly simple modification that can be made to any

phone. All it does is allow you to take any 2 lines in your house

and create a party line. So far I have not heard of Any problems

with it from my friends that have set one up and I have not had

any either. There is one thing that you will notice when you are

one of the two people who is called by a person with this box.

The other person0will sound a little bit faint. I could overcome

_______________________________________________________________________

Copyright 1990

57                                                    The Boxes

_______________________________________________________________________

this with some amplifiers but then there wouldn't be very many of

these boxes made.

I think that the convenience of having two people on line at

any one time will make up for the minor volume loss.

Phone Modification Instructions

-------------------------------

Here is the diagram:

*    =    -    +    KEY:

*    =    -    +    ___________________________

*    =    -    +    | PART           | SYMBOL |

*    =    -    +    ---------------------------

*    =    -    +    |BLACK WIRE      |   *    |

*    ==_/_-    +    |YELLOW WIRE     |   =    |

*******_/_++++++    |RED WIRE        |   +    |

|              |    |GREEN WIRE      |   -    |

|              |    |SPDT SWITCH     |  _/_   |

|              |    |                |  _/_   |

|_____PHONE____|    |VERTICAL WIRE   |   |    |

|HORIZONTAL WIRE |   _    |

---------------------------

In some houses the black and yellow are already wired, in

others you will have to go out to your box and rewire it. A good

way to figure out which line is which is to take the phone you

are looking for off the hook. Then you only need to take the red

and green wires entering your phone and hook them to the differ#

ent pairs of red and green going into the house. You can't hurt

anything in the phone or telephone by probing. When you find the

pair that you want take the black from your line and attach it to

the red of the other line then take the yellow and attach it to

the green line. Now you are all set to go. For people with rotary

phones you can have one person call you then place the second

call out to the other person. Though not a phreakers tool, the

brown box can be phun.

MAKING YOUR PHONE INTO A CHEESEBOX By SIR KNIGHT

----------------------------------

A Cheesebox (named for the type of box the first one was

found in) is a type of box which will, in effect, make your

telephone a pay-phone. This is a simple, modernized, and easy way

of doing it.

Inside info: these were first used by bookies many years ago

as a way of making calls to people without being called by the

cops or having their numbers traced and/or tapped.

_______________________________________________________________________

M & M Enterprises

58                                               The Boxing Book

_______________________________________________________________________

How to Make a Modern Cheese Box

-------------------------------

Ingredients:

------------

1 Call Forwarding Service on the line

1 Set of Red Box Tones

The number to your prefix's Intercept Operator (do some

scanning for this)

How to Use:

----------

After you find the number to the Intercept Operator in your

prefix, use your call-forwarding and forward all calls to her.

This will make your phone stay off the hook (actually, now it

waits for a quarter to be dropped in). You now have a Cheese Box.

In order to call out on this line: you must use your red box

tones and generate the quarter dropping in, then you can make

phone calls to people. As far as I know, this is fairly safe, and

they do not check much, although I am not sure, I think you can

even make credit-card calls from a Cheese Box phone and not get

traced.

The Clear Box By The Bit of The Postmen.(c) 1984 by 2600 Magazine

-------------

The clear box is a new device which has just been invented

that can be used throughout Canada and rural United States. The

clear box works on "PostPay" payphones (fortress phones). Those

are the payphones that don't require payment until after the

connection is established. You pick up the phone, get a dial

tone, dial your number, and then insert your money after the

person answers. If you don't deposit the money, then you can not

speak to the person on the other end, because your mouth piece is

cut off, but not the ear piece (obviously these phones are nice

for free calls to weather or time or other such recordings).

All you must do is to go to your nearby Radio Shack, or

electronics store, and get a four-transistor amplifier and a

telephone suction cup induction pick-up. The induction pick-up

would be hooked up as it normally would to record a conversation,

except that it would be plugged into the output of the amplifier

and a microphone would be hooked to the input. So when the party

that is being called answers, the caller could speak through the

little microphone instead. His voice then goes through the ampli#

fier and out the induction coil, and into the back of the receiv#

er where it would then be broadcast through the phone lines and

the other party would be able to hear the caller.

The Clear Box thus 'clears up' the problem of not being

heard. Luckily, the line will not be cut-off after a certain

_______________________________________________________________________

Copyright 1990

59                                                    The Boxes

_______________________________________________________________________

amount of time because it will wait forever for the coins to be

put in. The biggest advantage for all of us about this new clear

box is the fact that this type of payphone will most likely

become very common. Due to a few things:

1st, it is a cheap way of getting the DTF, dial-tone-first

service,

2nd, it doesn't require any special equipment (for the phone

company). This payphone will work on any phone line. Usually a

payphone line is different, but this is a regular phone line and

it is set up so the phone does all the charging, not the company.

GOLD BOX PLANS By DR. REVENGE

--------------

What It Does:

-------------

When you put a Gold Box on two phone lines, it lets anyone

who calls one of the lines call out on the other. So when MCI or

Sprint traces the line it will tell them that you're calling from

the line you hooked the Gold Box up to.

This is great for hooking up to pay phones. Just think,

phreaking without the heart attacks when you hear a click and the

phun you can have with a Red Box and AT&T calling cards. Just

don't use a Blue Box, call the President and say you want to kill

him, government computers, or anything that they trace by detect#

ing a tone in the line. But you can still call the operator and

ask for her bra size or if she knows what a trunk tone is!

How to build it:

----------------

You will need the following:

two   10k ohm resistors

three 1.4k ohm resistors

two   2N3904 transistors

two   photo-cells,

two   red LED's (the more light produced the better),

a box that won't let light in

red and green wire.

Light from the #1 LED must shine directly on the photocell

#1. The Gold Box I made needed the top of the LED's to touch the

photocell for it to work. The same applies to the #2 photocell &

LED.

_______________________________________________________________________

M & M Enterprises

60                                               The Boxing Book

_______________________________________________________________________

#1

:-Photocell--:

:            :

:            :Base

:          -----

:          Transistor

:   #1     -----

:  +LED-    : :

:  -I(--    : :Collector

RED1--<     >:--: :-------:-----GREEN2

-I(-- :            ----------:

#2   :                      :

LED   :-/\/\/-/\/\/-/\/\/-/\/\/

10K    10K   1.4K 1.4K

Resistors

#2

-Photocell-----------------

:                         :

:Base                     :

-----                       :

Transistor                  :

------                      :

: :Emitter                 :

GREEN1- --------------------------RED2

:   :

/\/\/

1.4K Resistor

The 1.4k resistor is variable and if the second part of the

Gold Box is skipped, it will still work, but when someone picks

the phone up they hear a faint dial tone in the background and

might report it to the Gestapo, er, AT&T. 1.4k will give you

good reception with little risk of a Gestapo agent at your door.

Now that you have built it, take 2 green wires of the same

length. Strip the ends, twist 2 ends together and connect them to

Green1 and place a piece of tape on it with line #1 writing on

it. Continue the process with red1, only use red wire. Repeat

with Red2 and Green2, but change to line #2.

How to Install:

---------------

You will need to find two phone lines that are close togeth#

er. Label one of the phones lines line #1. Cut the phone lines

and take the outer coating off it. There should be 4 wires. Cut

the yellow and black wires off & strip the red and green wires

for both lines.

Line #1 should be in two pieces. Take the green wire off one

end and connect it to one of the green wires on the Gold Box.

Take the other half of line #1 and hook the free green wire to

the green wire on the phone line. Repeat the process with Red1 &

the other line.

_______________________________________________________________________

Copyright 1990

61                                                    The Boxes

_______________________________________________________________________

All you need to do now is to write down the phone numbers of

the place you hooked it up at and go home & call it. You should

get a dial tone! If not, try changing the emitter with  collec#

tor.

THE GREEN BOX By BIOC Agent 006

-------------

Paying the initial rate in order to use a red box (on cer#

tain fortresses) left a sour taste in many red boxer's mouths

thus the GREEN BOX was invented. The green box generates useful

tones such as COIN COLLECT, COIN RETURN, and RINGBACK. These are

the tones that ACTS or the TSPS operator would send to the CO

when appropriate. Unfortunately, the green box cannot be used at

a fortress station but it must be used by the CALLED party. Here

are the tones:

COIN COLLECT	700 + 1100 Hz

COIN RETURN       1100 + 1700 Hz

RINGBACK		700 + 1700 Hz

Before the called party sends any of these tones, an opera#

tor released signal should be sent to alert the MF detectors at

the CO. This can be accomplished by sending 900 + 1500 Hz or a

single 2600 Hz wink (90 ms) followed by a 60 ms gap and then the

appropriate signal for at least 900 ms.  Also, do not forget that

the initial rate is collected shortly before the 3 minute period

is up.

Incidentally, once the above MF tones for collecting and

returning coins reach the CO, they are converted into an appro#

priate DC pulse (-130 volts for return & +130 volts for collect).

This pulse is then sent down the tip to the fortress. This causes

the coin relay to either return or collect the coins.

The alleged "T-Network" takes advantage of this information.

When a pulse for COIN COLLECT (+130 VDC) is sent down the line,

it must be grounded somewhere. This is usually either the yellow

or black wire. Thus, if the wires are exposed, these wires can be

cut to prevent the pulse from being grounded. When the three

minute initial period is almost up, make sure that the black &

yellow wires are severed; then hang up, wait about 15 seconds in

case of a second pulse, reconnect the wires, pick up the phone,

hang up again, and if all goes well it should be "JACKPOT" time.

How to Build Your Own Mauve Box by Captain Generic With Help from

-------------------------------    The Genetic Mishap. [11/24/86]

Please bear with me, as the construction of this box will

seem rather silly. This box was found in a construction site. Or

rather, it found us at a construction site. We were using a $5

Radio Shack phone out of a semi-completed office building. One

_______________________________________________________________________

M & M Enterprises

62                                               The Boxing Book

_______________________________________________________________________

afternoon during a holiday, and immediately following a storm, we

found our bus in shambles. To our surprise, we also found that

one of the phone connections we were tapping had been draped

through a murky puddle. The fact that surprised us even more was

that this line still worked and now possessed some great capabil#

ities.

Materials:

----------

2 Tupperware or similar 8oz containers

1 small bag earth (dirt) (12oz)

1 pint water

2 lantern batteries

1 nine volt battery

1 battery clip

2 SPST switches

4 ounces of iron shavings

2 polar magnets

5 feet wire

1 set soldering equipment

This is the part you won't believe. Take the Tupperware

containers, and fill them with a mixture of the earth and the

iron shavings. Make sure that the mixture is well done.

(*NOTE* for best results, use the sand in fine ash trays.)

Cut the red and green wires and splice the switches into

them. From the switches, solder wire to the magnets. Connect the

red to the + (positive) side of one magnet, and the green to the

- (negative) side of the second magnet. From the other poles of

the magnet, solder wires the battery & clip. Make sure the +

(positive) and - (negative) are correct. Set the nine-volt bat#

tery between the two Tupperware containers and place the battery

end of the two magnets into the Tupperware. Now connect wire to

the two poles of the lantern battery, and place them in the same

containers as the poles of the magnets/9-volt battery. You are

almost done. Finally, add just enough water to the two pots, and

let them sit in the sun and bake like bricks. At this point, you

have a MAUVE BOX.

Explaining and Using What You Have:

-----------------------------------

The red and green wires have been placed into a magnetic

field which is being charged continually by a lantern battery.

(It is necessary to change this battery every one to one and a

half months.) This will literally pull in the nearest phone

conversation. (Don't try this in a big apartment or dorm.) When

the 9-volt battery is connected, this will now create enough

current for the poles of the magnets to reverse themselves

(perhaps you're seen Mr. Wizard do this. It's just like with the

soap). At this point, you have a phone transmitting to one (if

not more) of the nearest phones. (Again, if you're in a dorm,

_______________________________________________________________________

Copyright 1990

63                                                    The Boxes

_______________________________________________________________________

don't try this.) I suppose this just accomplishes what a tap

would do, but with a MAUVE BOX, your fingerprints never will show

on a terminal or on someones telephone lines.

Notes and Addendum:

-------------------

This will only work with a touch-tone phone connected to a

phone line. When the switches are pulled, it's off your line and

into the air. This is named a MAUVE BOX, because this is the most

disgusting box, and I find mauve to be the single most disgusting

color I know of. Also, this file is for information purposes

only. This is not to be used in an illegal manner.

Olive Box Plans By Arnold

---------------

This is a relatively new box, and all it basically does is

serve as a phone ringer. You have two choices for ringers, a

piezoelectric transducer (ringer), or a standard 8 ohm speaker.

The speaker has a more pleasant tone to it, but either will do

fine. This circuit can also be used in conjunction with a rust

box to control an external something or other when the phone

rings. Just connect the 8 ohm speaker output to the inputs on the

rust box, and control the pot to tune it to light the light

(which can be replaced by a relay for external controlling) when

the phone rings.

______________

|              |        ^

NC --|-- 5      4 --|-----/\/\/------->G

|              |      / R2

G<----)|----|-- 6      3 --|-- NC

| C3    |      U1      |

-------|-- 7      2 --|---------- --- -- - > TO RINGER

|              |

----|-- 8      1 --|--

|    |______________|  |

|                       ---/\/\/----|(----- L1

|                           R1      C1

------------------------------------------ L2

a. Main ringer TTL circuit

_

FROM PIN 2 < - -- --- ----------| |_| |------------->G

P1

b. Piezoelectric transducer

_______________________________________________________________________

M & M Enterprises

64                                               The Boxing Book

_______________________________________________________________________

FROM PIN 2 < - -- --- ---------|(---------.  .-------|  |/ |

>||<       |S1|  |

>||<     --|  |  |

>||<    |  |__|\ |

G<---------.>||<.---        \|

T1

c. Electro magnetic transducer

Parts List

----------

U1 - Texas Instruments TCM1506

T1 - 4000:8 ohm audio transformer

S1 - 8 ohm speaker

R1 - 2.2k resistor

R2 - External variable resistor; adjusts timing frequency

C1 - .47uF capacitor

C2 - .1uF capacitor

C3 - 10uF capacitor

L1 - Tip

L2 - Ring

L1 and L2 are the phone line.

Shift Rate:

-----------

This is the formula for determining the shift rate:

1                   1

SR = --------------------- = ------------ = 6.25 Hz

(DSR(1/f1)+DSR(1/f2))    128     128

----  + ----

1714    1500

DSR = Shift Divider Rate ratio = 128

f1 = High Output Frequency    = 1714

f2 = Low Output Frequency     = 1500

How to make a Pearl Box By Dr. D-Code

-----------------------

The Pearl Box:Definition

------------------------

This is a box that may substitute for many boxes which

produce tones in hertz. The Pearl Box when operated correctly can

produce tones from 1-9999hz. As you can see, 2600, 1633, 1336 and

other crucial tones are obviously in its sound spectrum.

_______________________________________________________________________

Copyright 1990

65                                                    The Boxes

_______________________________________________________________________

Materials:

----------

C1, C2 .5mf or .5uf ceramic disk capacitors

Q1     NPN transistor (2N2222 works best)

S1     Normally open momentary SPST switch

S2     SPST toggle switch

B1     Standard 9-Volt battery

R1     Single turn, 50k potentiometer

R2       "     "    100k potentiometer

R3       "     "    500k potentiometer

R4       "     "    1meg potentiometer

SPKR   Standard 8-ohm speaker

T1     Mini transformer (8-ohm works best)

Misc.  Wire, solder, soldering iron, PC board or perfboard,

box to contain the completed unit, battery clip

Instructions for building The Pearl Box:

----------------------------------------

Since the instruction are EXTREMELY difficult to explain in

words, you will be given a schematic instead. It will be quite

difficult to follow but try it any way.

(Schematic for The Pearl Box)

+-------------+-------+---------+

|             |        \        +--S1----

C1            C2        \           SPKR

|             |          +      +--------

+             |      ----+T1    |

|\            +----------+------+

|  b  c-------|          |

|   Q1                   |

|     e-----S2---+       |

|                |       |

|               B1       |

|                |       |

|                +-------+

|R1   R2   R3  R4|

/\/\ /\/\ /\/\/\/\

+--+ +--+ +--+

Now that you are probably thoroughly confused, let me ex#

plain a few minor details. The potentiometer area is rigged so

that the left pole is connected to the center pole of the poten#

tiometer next to it. The middle terminal of T1 is connected to

the piece of wire that runs down to the end of the battery.

Correct operation of The Pearl Box:

-----------------------------------

You may want to get some dry-transfer decals at Radio Shack

to make this job a lot easier. Also, some knobs for the tops of

the potentiometers may be useful too. Use the decals to calibrate

_______________________________________________________________________

M & M Enterprises

66                                               The Boxing Book

_______________________________________________________________________

the knobs. R1 is the knob for the ones place, R2 is for the tens

place, R3 if for the hundreds place and R4 is for the thousands

place. S1 is for producing the all the tones and S2 is for

power.

Step 1: Turn on the power and adjust the knobs for the desired

tone. (Example: For 2600hz- R1 = 0: R2 = 0: R3 = 6: R 4 = 2)

Step 2: Hit the pushbutton switch and you have the tone. If

you don't have a tone, then recheck all connections and

schematic.

PEARL BOX Schematic by Dispater [7/1/89]

-------------------

Introduction:

-------------

After reading the earlier renditions of schematics for the

Pearl Box, I decided that there was an easier and cheaper way of

doing the same thing with an IC and parts you probably have just

laying around the house.

What Is A Pearl Box and Why Do I Want One?

------------------------------------------

A Pearl Box is a tone generating device that is used to make

a wide range of single tones. Therefore, it would be very easy to

modify this basic design to make a Blue Box by making 2 Pearl

Boxes and joining them together in some fashion.

A Pearl Box can be used to create any tone you wish that

other boxes may not. It also has a tone sweep option that can be

used for numerous things like detecting different types of phone

tapping devices.

Parts List:

-----------

CD4049 RCA integrated circuit

.1 uF disk capacitor

1 uF 16V electrolytic capacitor

1K resistor

10M resistor

1meg pot

1N914 diode

Some SPST momentary push-button switches

1 SPDT toggle switch

9 Volt battery & clip

_______________________________________________________________________

Copyright 1990

67                                                    The Boxes

_______________________________________________________________________

State-of-the-Art-Text Schematic:

+ 16V  1uF -

_____________________________||_____

|        |     |              ||     |           _

|   _______________________          |__________| |/| 8ohms

____|__|_____:__|__:__|_       |          __________| | |

| 9 10 11 12 13 14 15 16 |      |         |          |_|\|

|        CD4049UBE       |      |         |

|_1__2__3__4__5__6__7__8_|      |         |          _

|  |  |__|  |__|  |  |__________________|_________[-]

|  |  |           |                               [b]

|  |__________________________|                   [a]

|     |           |           |                   [t]

|     |    1N914  |           |                   [t]

|___________|/|___________________________________[+]

|     |\|   |           |

|           |           |

|    10M    |           |

|___/\/\/\__|           |

|           |           |

|_____||____|           |  <-- These 2 wires to

||    |           |      the center pole of

.1uF   50V |           |      the switch)

____________________|           |___________________________

|                  __[Toggle Switch]________                |

|                  |                        |          ___  |

|                  |                        |          o o  |

|                  |                        | /\/\/\___| |__+

|_/\/\/\____/\/\/\ |                        |    ^          |

1K         ^   |                        +____|     ___  |

|___|                        |          o o  |

| /\/\/\___| |__|

(pot side)              (push-button  |  ^

side)  |__|

Explanation:

------------

The 2 wires that lead from the main part of the circuit

should be connected to the center poles on the toggle switch.

Put the 2 wires to the pot on one side and the 2 wires going to

the push-buttons to the other side. That way you can switch

between tone sweep and the favorite tones you like (the push-

button side).

To keep tones that you want to use frequently like 1850 Hz

then all you have to do is put in a variable resistor and adjust

it to where you have the correct tone, then just put a push-

button switch on the line. You can link them together in a chain,

etc. There are many other good modifications to make to the box

so have fun and be smart.

_______________________________________________________________________

M & M Enterprises

68                                               The Boxing Book

_______________________________________________________________________

How To Build A Pink Box By Baba O'Riley

-----------------------

The function of a "Pink Box" is to add a hold button that

allows music or any thing else to be played into the telephone

while the person is on hold. This Modification can either be done

right in the telephone as a separate box.

Materials Needed

----------------

Some Bell wire or Phone wire

SPST momentary switch         RS # 275-1547

470 ohm resistor              RS # 271-019

1 LED (Approx 5V)             RS # 276-041

SCR, 2N5061  (Transistor)

Audio Transformer (Ratio 10K:600)

RCA phono Jack                RS # 274-346

Screw drivers, Soldering Irons, Ser, Etc.

Construction

------------

1. Open the wall box and locate the RED and GREEN wires.

2. Take a piece of RED wire and strip the end and attach it to

the red lead on the wall box. Do the same for the Green.

3. Connect the GREEN wire to the ANODE of the LED.

4. Connect the CATHODE side of the LED to the UPPER pin of the

primary side of the transformer.  See diagram below.

______

To one pole of phono ---!Top !--- To CATHODE of LED

-!View!-        Primary side

To other phono pole  ---!____!--- To pole of trans. & one pole

of switch

5. Connect the pin directly across from the transformer to one

pole of the phono jack.

6. Connect the RED wire to one side of the resistor and to the

"C pole" of the transistor.

7. Connect the open pin of the switch the other side of the

resistor and to the "G pole" of the transistor.

_______________________________________________________________________

Copyright 1990

69                                                    The Boxes

_______________________________________________________________________

Wiring Diagram

--------------

RCA Jack    X-former    LED

_____      C   A

Pole or Jack --/---| Top |---/--(*)--\------GREEN wire

-|View |- Primary   --|---RED wire

Pole of Jack --/---|_____|---/-+       (O)

|        |

|     [--+-----Pole of Switch

|

|--------/-----Pole of Switch

Key to Symbols

--------------

|, --, /   Connection or wire

_/    C pole of transistor    --(*)--  LED

[_)--  G pole of transistor

|

I     A pole of transistor   (O)  Resis

|

_____

---| Top |---

-| View|- Primary    Transformer

---|_____|---

Use of the Pink Box

-------------------

Hook the RED and GREEN wires up to the appropriate terminals

and hook the RCA jack to the output on your stereo. Turn on your

stereo at a good volume. Now call a friend. To test the Box, Hold

down the switch and hang up the phone. The LED should light up

and your friend should hear music, If not then start over. The

hold is shut off if you pick up a phone on that line or your end

hangs up.

Red Boxes Revived By Pink Panther

-----------------

Probably most of the information I am about to tell you, you

probably already know or have it stored somewhere. But I have

seen quite a lot of questions on the subject lately, and thought

to explain a couple of things.

Blue boxing has been dead for quite some time since every#

thing went to ESS, and the same with black boxing. The latest

form of boxing is red and green boxing. They both deal with

_______________________________________________________________________

M & M Enterprises

70                                               The Boxing Book

_______________________________________________________________________

fortress phones and can only be used with a fortress phone.

With a red box, you dial a number at a fortress, insert a

nickel, which is the ground check, and play the tape. It will

emulate coins being dropped into the fortress. Since there is

also questions on what are and how to get these tones, I've

created a simple step process:

1)  Obtain a recorder that you can directly hook into a phone

line. If you use a regular recorder, you will need some

modification on it. If you have an answering machine, then

you have it made.

2)  Find a fortress, and follow the metal pipe (usually metal)

from the fortress to where ever it ends up. At some point on

the pipe, there will be a small box which is held together

by two screws. Unscrew the box.

3)  You now should find two bolts with wires connected to

them. The wires are 22 gauge (which is fairly thin wire).

If you see thicker wires, such as 12 gauge wires, these are

220 volt AC lines, usually connected to the light in the

phone booth. Do not touch the AC lines, unless you are

stupid. Connect the tape recorder to the proper bolts, which

means the 22 gauge wire.

4)  Now dial a long distance phone number, and you will get a

recording to insert some money. Insert about $6.00 in quar#

ters, then hang up and your money will be returned. The

tones should have been recorded with a normal tape with no

dolby.

5)  Obtain a recorder with a built in speaker, or rip apart a

phone set and obtain the earpiece. If there is a diode

across the earpiece, remove it. Connect the earpiece to the

output of the recorder. (I recommend using an earpiece

rather than a built in speaker).

6)  To test your tones, dial 0-959-1230 from a fortress, and

you should get 'Coin Test . Please Deposit .' Play back the

tones you recorded and if everything goes well, you should

hear 'Quarter' everytime a tone is played. Remember you only

recorded quarter tones. You can record any tones you want by

inserting different coins at the recording stage. If you are

having problems, try adjusting the volume.

7)  To use, dial a non-local number, insert a real nickel, and

play the tones. Make sure you have enough tones on the

recorder to complete the call.

Now I will explain a little about what exactly happens when

you deposit coins. When you deposit a coin, it goes through a

series of tests, determining what type of coin it is. It will be

deposited in various coin slots within the fortress itself if

everything goes right. But before it is deposited in the right

_______________________________________________________________________

Copyright 1990

71                                                    The Boxes

_______________________________________________________________________

slot it will cause a wheel to be turned. A nickel will turn the

wheel once, a dime twice, and quarter five times. This will cause

a frequency to be generated which is sent to a operator or com#

puter. A capacitor is placed across the speech circuit while

these tones are generated so that the customer does not here

them. Here are the tones and PPS (pules per second):

Nickel:  1 beep   5-8.5 PPS

(1700hz + 2200hz for 66ms)

Dime:  2 beeps  5-8.5 PPS

(1700hz + 2200hz for 66ms on, 66ms off,

then 66ms on)

Quarter:  5 beeps  12-17 PPS

(1700hz + 2200hz for 33ms on and

33ms off, repeated a total of 5 times)

SILVER BOX PLANS By Silent Screen, Telcom 7, Floppy Disk, and

----------------     Anonymous

Well, now that I have you here, I may as well bore you with

a little history of silver boxes. One day, some engineer over at

Bell Labs got the bright idea to sing to his computer, and found

that the computer responded by dialing a phone number. Well, sort

of. Anyway, the 10 digits (old style, with the # and * blocked

out) touch tone pad was born. Lo and behold, though, 2 sets of

people decided that 10 buttons just wasn't enough. They needed

more. The two people of whom I am speaking are of course Ma Bell

and Pa Autovon (the military phone network). So, Ma decided to

add an extra column to her phones, this one merging the standard

row tones with a 1633 Hz tone (to provide the mystical DTMF).

Bell named these buttons A,B,C and D. Aren't you glad we have

such imaginative people at the Bell System? The army named them

Flash, Flash Overide, Priority, and Priority Interrupt, or is it

Flash Interrupt and Priority Overide? Anyway, these are used as

varying degrees of priority during wartime and wargame activities

so generals can call their secretaries very quickly. Bell's use

of A,B,C and D is not so clear. However, the last button D has an

interesting property.

On about 50% of the information lines in the country it will

give you a pulsing dial tone. You can then enter commands to what

appears to be a test system for 4A boxes.

Anyway, the mod. WARNING: Read these instructions several

times before you actually try them. Make sure you know how to

solder. This could permanently damage your phone!

Unscrew the 2 large screws on the base of your phone. Take

the cover off and place it and the screws in a safe place. Now,

loosen but don't remove the screws on the sides of the touch tone

keypad. These are on the sides and attach it to its mounting

brackets. Now carefully remove the pad from its brackets without

ripping any wires. You will notice a plastic cover on the pad.

Separate the 2 halves and get them out of your way. Don't destroy

_______________________________________________________________________

M & M Enterprises

72                                               The Boxing Book

_______________________________________________________________________

them, they'll be needed. Just move them down the wire harness

that goes thru them.

Now look at the top of the pad so the 1-2-3 row is facing

away and the *-0-# row is toward you. Turn over the pad. You

should see a mass of wires, gold plated contacts, discrete compo#

nents and 2 large donut shaped black things. These are coils that

generate the frequencies. Ma Bell was always one for standards so

the coils she manufactures are capable of generating all 4 pri#

mary tones. She only gives you connections to 3 of them, though.

You're about to make your own connections to the 4th and make the

3rd column of keys "bank switched" between normal and 4th row.

Cut 3 lengths of wire of different colors about 2 ft. long.

(Better overkill than underkill.) I used blue, gray and brown,

but these colors are arbitrary. But I will be referring to these

colors in this message. Look at the coil on the left with the 5

solder contacts facing you rather than perpendicular to you.

Count over 4 contacts from the left or 2 from the right and

solder a wire to the 4th post from the left. This is the 1633 Hz

output. Solder the other end of this wire to the left pole of the

smallest PDT switch you can find. This is the point of no return

now. Take a look at the bottom edge of the keypad. You should see

a row of 3 gold plated contacts to the right of 2 very large

capacitors. Look at the one on the left. This one controls the

rightmost bank of keys on the phone.

Gently separate the 2 touching connectors; They are soldered

together. Spread them apart. Solder the brown wire to the top

contact (the one furthest from you) and solder it to the right

pole of the SPDT. Now, take the blue wire and solder it to the

bottom closest contact. Solder the other end of this wire to the

center pole of the SPDT. You have now completed your mods.

When the switch is in one position you will get normal

tones. In the other you'll get 1633 tones. Some finesse can be

added by passing the 3 wires thru the plastic cover and in thru

the gap in the case of your phone under the place where you hang

it up. Then solder the wires to the switch in the proper places

and you can glue the switch to the wall on the inside of that

small alcove in the slack wire back into the phone. This was just

to make sure you read the whole thing before trying it.

Now call out-of-state directory assistance (XXX-555-1212)

using normal tones. Switch quickly to 1633 and press down on the

# key (now the D key). If you are on an old 4A switchbox you will

get a pulsing dial tone. You can then switch back to normal and

try dialing different #'s. Two of the most interesting are 6 and

7. These often form a loop-around type connection and 2 people

can call in, one using 6 and the other 7, and talk in this man#

ner.

NOTE: You will not receive a pulsing tone until the operator

actually picks up on the line. If you hear ringing keep pressing.

The tone must be on at the same time the operator gives her

"beep." If you hear her cursing at stupid people with stuck

_______________________________________________________________________

Copyright 1990

73                                                    The Boxes

_______________________________________________________________________

buttons chances are this one doesn't work. Try out of the way

states like Montana or Wyoming.

Combined Silver Box and White Box By Explorer & Cat 033

---------------------------------

These are the instructions on how to build a combined Silver

Box and White box. Most of the parts required can be found at

your nearest Radio Shack. The only real important part required

not sold at Radio Shack is the keypad. This can be picked up at

any electronic store. These instructions are for a 12 button

keypad. A toggle switch is added for the Silver Box application.

if you have a 16 button keypad, then the toggle switch isn't

needed.

The tones made by a touch tone telephone are not single

tones. They are a combination of two tones, making "DTMF" (Dual

Tone Multi-Frequency). The normal telephone dials 12 different

signals but is capable of dialing 16 different signals.

The power required by a keypad is about 25 volts, but they

will work with as little as 15, thereby allowing the use of two 9

volt batteries. This design allows you to use a standard 8ohm

speaker instead of a telephone speaker. To accomplish this we use

a matching transformer.

Parts Required:

---------------

1)  5089 dtmf tone encoder (rs 276-1301)

2)  audio transformer (rs 273-1380)

3)  tv colorburst crystal 3.579545 mhz (rs 272-1310)

4)  9v battery straps x2 (rs 270-325)

5)  9v battery x2 (rs 23-464)

6)  wire wrap socket (rs 276-1994)

7)  switch dpdt (rs 275-663)

8)  keypad (cardinal electronics)

9)  speaker 8ohm (rs 40-245)

10) circuit board (optional)

11) connecting wire or wire wrap

12) box (optional)

How to Construct it:

--------------------

A few construction notes. I suggest that you solder and tape

all connections.

[1]  Connect the red wire of the transformer to either termi#

nal on the speaker.

[2]  Connect the white wire from the transformer to the other

terminal on the speaker.

_______________________________________________________________________

M & M Enterprises

74                                               The Boxing Book

_______________________________________________________________________

[3]  Connect the red (positive) wire of one battery clip to

the black wire of the other battery clip.

[4]  Connect the remaining red wire on the battery clip to pin

#1 on the IC.

[5]  Connect the crystal to pin #7 and pin #8 on the IC.

[6]  Connect the blue wire from the transformer to pin #16 on

the IC.

[7]  Connect the green wire from the transformer and the other

black wire from the battery clip to pin #6.

[8]  Connect one side of the switch to pin #9. Connect the

other side of the switch to pin #5 (only for 12 button

pads).

[9]  Since keypads may differ, try to get a pinout of the pad

you are using. The pad I have has 8 pinouts. One is a ground

(this goes to pin #6 on the IC). 1,4,7,* keys connect to pin

#3. 2,5,8,0 keys connect to pin #4. 3,6,9,# keys connect to

the center of the switch. 1,2,3 keys connect to pin #14.

4,5,6 keys connect to pin #13. 7,8,9 keys connect to pin

#12. *,0,# keys connect to pin #11.

[10] Check all connections to make sure none are touching each

other.

[11] Connect the two nine volt batteries and your keypad is

complete.

Note: the remaining pins on the IC are not used. The black

wire on the transformer is also not used. When none of the but#

tons are pressed, this unit uses no power, thereby eliminating

the need for a power switch.

_______________________________________________________________________

Copyright 1990

75                                                    The Boxes

_______________________________________________________________________

CRYSTAL (3.579545 MHZ)

+-#-+

| +-+         +--------------------------- +18V --+

| |           |                                   |

| | +---------|----------+--- <- XFMR (GREEN)    ---

+----------------+        |                        -

|8 7 6 5 4 3 2 1 |       ---

|                /        -

|               | NOTCH

|  1 1 1 1 1 1 1 \         \   XFMR (RED)

|9 0 1 2 3 4 5 6 |  SPKR-> |]============

+----------------+         /   XFMR (WHITE)

|

+----------- <- XFMR (BLUE)

IC   IC

#3   #4    +--------------------------+

|    |    |                          |

[1]  [2]  [3] -> IC #14    IC #5 <-+  |  +-> IC #9

[4]  [5]  [6] -> IC #13            +--+--+

[7]  [8]  [9] -> IC #12              / DPDT SWITCH

[*]  [0]  [#] -> IC #11

KEYPAD

KEY      FREQ. #1    FREQ. #2

---      --------    --------

1          697        1209

2          697        1336

3          697        1477

A          697        1633

4          770        1209

5          770        1336

6          770        1477

B          770        1633

7          852        1209

8          852        1336

9          852        1477

C          852        1633

*          941        1209

0          941        1336

#          941        1477

D          941        1633

All frequencies are measured in Hertz.

_______________________________________________________________________

M & M Enterprises

76                                               The Boxing Book

_______________________________________________________________________

Usage of a Silver Box

---------------------

First off, a Silver Box adds the four extra tones to your

phone that Ma Bell never told you about. They go like this:

1  2  3  A

4  5  6  B

7  8  9  C

*  0  #  D

There used to be many places where you could use Silver Box

tones, but now you can only use it in places where Ma Bell hasn't

gotten around to replacing her old switchboxes, (4a), or on

AUTOVON. AUTOVON is a special military phone line, like a mass of

private lines hooked together. To see if a specific area has the

old 4a switchboxes try this test:

Call up the directory assistance by using xxx-555-1212,

(where xxx is the area code you are checking out. Now hold down

the "D" key as soon as you finish dialing. (If you are using the

toggle switch type then flip the switch and hold down the "#"

key.) The phone should start ringing and the operator will an#

swer.

If the operator screams at you to stop pressing your keypad,

then hang up and try a different areacode (this one does not

work.)

If you get a pulsing tone then you got a good line! Now try

out different numbers. 6 and 7 usually form a loop line and two

people with Silver Boxes can talk. Some of these are watched, so

don't say anything you wouldn't want your mother to hear!

SWITCHBOX PLANS by Autopsy Saw

---------------

This tutorial is notable in that it is totally and absolute#

ly within legal bounds. It's only object is to guide you in

building a device with which you can control, on many different

levels, your home phone lines. It is designed primarily for two

phone numbers, although I suspect that provisions for more than

two can be easily added. With it, you can put one or both phone

lines on hold with visible indicators of each lines status;

conference call with two people; change a phone from line #1 to

line #2; and lastly, make one phone line physically dead to

outside world. This is good, for instance, if you don't want to

receive any calls, or if you want to stop someone from talking on

that line really quickly.

The circuit is relatively simple to build and I believe it

is much more economical than buying something like Radio Shack's

line controller. It will require some knowledge of electronics

_______________________________________________________________________

Copyright 1990

77                                                    The Boxes

_______________________________________________________________________

and the phone line in general, but I don't think that should be a

problem. I have personally found many uses for the controller,

and have put in into a pleasant "project box" so it is not an

eyesore.

Parts List

----------

Two pushbutton (on-off) switches for hold

Two LED's for line status indication

One DPDT switch for conferencing

One SPST switch for line shut off

Another DPDT switch for phone line switching

Wire

2 modular phone plugs with at least 10 feet of wire each

Two 1.7K ohm resistors

*OPTIONAL* - Project Box

Construction

------------

We will take the construction of the switchbox-controller in

parts. First, the hold switches and status indicators. The fol#

lowing is a diagram of the essential circuit:

Pushbutton #1    LED

--------------------------|x|-----0---

Phone Line #1     1.7K Resistor      |

-----------------/\/\/\/\/------------

Some word of explanation is in order. First of all, the

second phone line hold is constructed the same as the first.

Basically, there are two wires that come out of your phone line,

the red and the green. You have to experiment to find out which

one is connected to the pushbutton switch and which one to the

Resistor. To do this, just connect it in any way and then turn

the switch on. If the LED lights up, you connected it right,

otherwise reverse the connections. Got it? Good, now do the same

for line #2, following exactly the same procedure.

Now, let's do an easy one. This will be the line blackout

switch. First, decide which line you wish to black out. Actually,

you can do it to both lines if you wish, just get an extra SPST

switch. Now, connect the switch across the line as follows:

--------------------------------|-------|

x                      | Hold  |

|x| <---Switch          |Circuit|

x                      |       |

--------------------------------|-------|

Don't worry that your line can't handle both circuits, it

can. To test out, pick up a phone on the line you have connected

the switch to and flip it on. There should be dead silence. Now,

turn it off. You should hear a dial tone. You have to be an

_______________________________________________________________________

M & M Enterprises

78                                               The Boxing Book

_______________________________________________________________________

absolute idiot if this doesn't work.

OK, let's see what we have so far. If everything is working

you should now have two hold circuits and a blackout switch

connected. Now it really gets tough!

Next on the agenda is the conference switch. I have a few

precautions for you on this one. First, you have to know what

you're doing, and second, it doesn't work that well. The reason

it doesn't is because I am naturally very lazy and didn't feel

like adding the needed phone transformer. But, it is still good

for many uses, limited only by your imagination (to use a

cliche').

First, take the double pole double throw switch into your

hand. It should look like this on the back:

---------

- 1   2 -

- 3   4 -

- 5   6 -

---------

* Note - Numbers correspond to actual pins.

Now connect two wires to pins 3 & 4 and two wires to pins 1

& 2. The wires going to pins one and two are connected across the

first phone line and the wires from three and four go across the

second phone line. Test it out: Pick up your phone (either line)

and wait for dial tone. Then throw on the conference switch. You

should get a second dial tone. Now dial 555-1212. You should now

have two D.A.'s on the line, asking each other "What City?". If

not, then reverse the connections to phone line #1 or #2, it

doesn't matter. Now it should work. If not, then recheck EVERY#

THING! You messed up!

Now for the last section - the phone line switcher. With

this, you can connect a phone directly to the switchbox and have

it operate from line #1 or line #2, depending on the position of

the last DPDT switch. Let me emphasize that this is optional.

Actually, all the circuits are - they are independent of each

other and each can be built separately or together.

Now, referring to the diagram of the DPDT switch above:

Connect two wires to pins 1 & 2. Then connect these to phone line

#1. Now connect two wires to 5 & 6 and connect these to phone

line # 2. Lastly, connect two wires to 3 & 4 and connect the ends

of these to a phone. That's it! Now, pick up the phone and you

should hear a dial tone. Throw the switch and you should hear

another dial tone. In some areas you may dial '958' to hear

exactly what number you are currently connected to, but that

should be a problem. If you really can't figure it out, then

either trace the circuit back or call one of the numbers and see

if you get a busy signal or if it rings.

_______________________________________________________________________

Copyright 1990

79                                                    The Boxes

_______________________________________________________________________

Finally - Put the whole mess inside a project case, unless

you like the site of wires all over the place. This also should#

n't be a problem if you know even the basics of hardware con#

struction.

The Tan Box by Samurai Cat and Tarkin Darklighter

-----------

Introduction

------------

The Tan box allows you to make recordings from a phone line,

and it will only record once the victim's phone is picked up.

Basically, this is a modified linesman's handset, or "Beige

Box." A Beige Box consists of a speaker and two wires, a red

(ring) and a green (tip). The Tan Box also works on this princi#

ple.

Parts:

------

A small cassette recorder, preferably a micro cassette

recorder. (It MUST have both a microphone jack and a remote

jack)

A single line recording control. (Radio Shack part number 43-

228)

2 alligator clips (Radio Shack number 270-374)

A plastic box to contain the above parts (also available at

Radio Shack)

Tan or Black spray paint

7/16 inch hex driver

Construction:

-------------

1.  Take the recording control and cut the modular plug off.

Make sure you leave enough wire!

2.  Strip the red and green wires. (The yellow and black wires

are not necessary, and can be removed.)

3.  Drill a hole on the end of the plastic box (enough for two

wires to go through).

4.  Put the recording control in the box and run the red and

green wires through the hole. Attach one alligator clip to

each of the wires.

5.  Put the micro cassette recorder in the box and plug the

remote and microphone wires from the recording control into

it.

_______________________________________________________________________

M & M Enterprises

80                                               The Boxing Book

_______________________________________________________________________

6.  Close the box.

7.  Spray paint the box and wires to make it look like it is

supposed to be where it is. (BE SURE YOU LABEL THE WIRES

BEFORE YOU PAINT THEM!)

Installation

------------

You can either hook it up to the box on the side of their

house or a bridging head. Use the 7/16" hex driver to open either

of these. (On older houses, you may not have to use a hex driv#

er.) Attach the red wire to the right terminal and the green to

the left. (Remember: Red-Ring-Right) If you attached the box to a

bridging head, there may be room to leave it inside. If you

attached it to the side of a house, run the wires out of their

box and close it. Attach the Tan Box to the side of their house.

Make sure you come back and get the box pretty soon, or you

may find it gone! Micro cassette recorders are not cheap!

Be careful, wiretapping is a felony (2-20 years, a $10,000

fine, or both), and various other laws may be broken (i.e.,

recording without the other party knowing it, etc.)

Ways to avoid being Beige/Tan Boxed

-----------------------------------

A tap detector may be purchased at Radio Shack (of course).

If you do detect a tap, find the box, and you will have just

gotten a new tape recorder!

Urine Box Plans By Wolfgang von Albatros [3/2/86]

---------------

Now at last, Underground_Elite brings you these ultrasophis#

ticated box plans for, well, uh, I guess we just kind of bring

them for you. After many months of keeping this puppy secret, we

here at Underground_Elite have finally come to the consensus that

the public oughtn't be kept in the dark from the wrath of Ma

Bell, that great telephone company in the sky.

The Urine Box (tm) basically creates a capacitative disturb#

ance between the ring and tip wires in another's telephone head#

set. I have utilized this to the distinct advantage of the opera#

tor of the Urine box and to the disadvantage of the other party

involved, hopefully you won't get caught.

Without further a doo, wire this schematic together. Copper

wire (or gold) will work best due to its lessened resistance. It

might be a good idea to keep the wiring cooled, to further reduce

current loss.

_______________________________________________________________________

Copyright 1990

81                                                    The Boxes

_______________________________________________________________________

47k ohms  ---------------         .01 uF

|\    |----/\/\/------| SPST switch |---|------)|-------|

| \   |               ---------------   |               |

|  |--|                                 |               | +

|  |--------<-|-----|---/\/\/-----------|           ----------

| /       PVP diode |   470k ohms                   | 12 VDC |

|/                  |        _                      ----------

|       /         .005 uF           | -

SPKR 20W            |--/\/\/\/\/--------)|----<-|-------|

|    _/ VR1 (see note1)   PVP diode |

|                                   |

to RING                              to TIP

(your phone)                         (your phone)

Note1 - VR1 is a three position variable resistor, enabling

settings equivalent to 100k ohms, 210k ohms, and 320k ohms

(all 110k apart).

Note2 - The diodes are silicon-germanium based bipolar RF

reducers to limit current polarity reversal. If these are

placed backwards the box will not work as intended.

Note3 - The speaker will emit low tones when the SPST is

closed (ie the box is "ON") and when the settings of VR1 are

moved to higher resistance the tone level will increase.

Note4 - Place setting evaluation symbols to represent the

lowest resistance setting for VR1 to be "STEAM", the second

(210) to be "MELT", the third (320) to be "VAPORIZE".

Usage of box.

-------------

Use in place of your normal telephone receiver. Do not

attempt usage as a regular phone. Call victim by sending tones

through speaker with tape recorded noise. Use touch tone sounds

as pulse requires electric disruption of line, not just audio.

After answer (listen in on SPKR 1) set VR1 to desired strength

and let her go. Victim will feel unpleasant sensations in his/her

head and will undergo a series of high intensity seizures, not

unlike epilepsy, and jointly proportional to VR-1 setting and

time induced.

Settings. (The Fun Part)

------------------------

STEAM. A short -poof- of water vapor condenses within the

victim's earpiece, rendering the telephone short circuited

and the victim quite dazed.

MELT.  Causes structural phase change of receiver (usually

reinforced urea formaldehyde plastic material - high tech).

This has the unpleasant bonus of removing the victim's hand.

VAPORIZE. If the victim is particularly nasty, this setting

will remove his upper anatomy (if box is enabled properly,

that is!). Continued usage of this setting is generally not

_______________________________________________________________________

M & M Enterprises

82                                               The Boxing Book

_______________________________________________________________________

recommended, as death or other serious injury will result.

White Box Plans by Uncle Joe & Blacksmith

---------------

This article will tell you how to change a normal touch tone

keypad and convert it to a portable unit. In addition, we give

the touch-tone frequencies. First of all, the tones made by a

touch tone telephone are not single tones, they are a combination

of two tones, making "DTMF" (dual tone multi-frequency). The

normal tone telephone dials 12 different signals, but is capable

of dialing 16 different signals.

The power required by a keypad is about 25 volts, but they

will work with as little as 15, thereby allowing the use of two

9-volt radio batteries. As you may have guessed, they are also

designed to operate with a telephone type speaker (and phone

line), and not the standard 8-ohm speaker which needs to be used

for adequate volume. To accomplish this, we use a matching trans#

former, this is one of those miniature ones available at Radio

Shack. Enough of the theory, now for the circuit.

Material Required:

------------------

A touch tone keypad

A miniature 1000 to 8 ohm transformer (R.S. # 273-1380)

A standard 8-ohm speaker

Two 9-volt radio batteries

Two 9-volt battery clips

A case to put it all in (optional)

A few construction notes, I suggest that you solder and tape

all connections. It is also important to read this entire bulle#

tin before attempting to construct this.

First, connect the RED wire of the transformer to either

terminal on the speaker. Now connect the WHITE wire from the

transformer to the other terminal on the speaker. Next, connect

the RED (positive) wire of one battery clip to the black wire of

the other battery clip.  Now connect the remaining RED wire on

the second battery clip to the GREEN wire from the touch tone

pad. Connect the BLUE wire from the touch tone pad to the ORANGE-

and-BLACK striped wire from the touch tone pad. To these two

wires, now connect the remaining black lead from first battery

clip. You have now finished the power connection to the keypad.

Connect the BLACK wire from the keypad to the BLUE wire on the

transformer. Next connect the RED-and-GREEN striped wire from the

keypad to the GREEN wire on the transformer. The BLACK wire on

the transformer should not be connected to anything, along with

quite a few wires from the keypad.  The connection of the keypad

is now complete.

Next, all you have to do is connect two nine volt batteries

_______________________________________________________________________

Copyright 1990

83                                                    The Boxes

_______________________________________________________________________

to the battery clips, and you'll be ready to go. You may want to

mount it in a case for easy portability.

Note that the silver box modification CAN be made to this

unit, allowing complete remote phreaking. When none of the but#

tons are pressed, this unit uses NO power, thereby eliminating

the need for a power switch, and extending the life of the bat#

teries.

The following are the frequency combinations generated by

each button on the keypad:

KEY  FREQ. #1  FREQ. #2

---  --------  --------

1      697      1209

2      697      1336

3      697      1477

A      697      1633

4      770      1209

5      770      1336

6      770      1477

B      770      1633

7      852      1209

8      852      1336

9      852      1477

C      852      1633

*      941      1209

0      941      1336

#      941      1477

D      941      1633

All frequencies are measured in Hertz.  Note that A,B,C and

D are not normally present (except for silver boxes).

_______________________________________________________________________

M & M Enterprises

84                                               The Boxing Book

_______________________________________________________________________

Appendix A

Miscellaneous Boxing Notes From Near and Far

--------------------------------------------

Boxes that do not work on ESS systems:

Black

Blue (will work off of 800's and various numbers)

-----------------------------------------------------------------

Red boxing is perfectly possible in an ESS switching system.

In fact, it's actually a little easier, due to the fact that most

fortress phones in the ESS system are DTF (dial-tone first). In

other words, you can pick up a fortress phone, and there will be

a dial-tone 'before' you insert any money at all. From there you

simply blow your red box tones into the mouthpiece and you're all

set.

In Step by Step systems, and most versions of Crossbar, you

are required to insert money before you get a dialtone (you need

the dialtone, to send your red box tones, as the mouthpiece is

actually 'disconnected' until money is inserted (earpiece also).

So here's where the 'Green Box' tones came in, after inserting

your initial dime to initiate dialtone, thus enabling the red box

tones to be accepted, after you finished the call, one would have

the 'called' party blow the Green Box, 'Coin Return' tones, and

have the original dime refunded. Note-> It -HAS- to be the called

phone that sends the coin return tone. You can't send them your#

self (from the fortress).

In a CCITT (out band) system, all switching signal are sent

on a separate line other than the voice line, thus 'disabling'

boxing.

-----------------------------------------------------------------

Multi-Frequency Tones

A    697Hz + 1633Hz     B    770Hz + 1633Hz

C    852Hz + 1633Hz     D    941Hz + 1633Hz

1    700Hz +  900Hz     2    700Hz + 1100Hz

3    900Hz + 1100Hz     4    700Hz + 1300Hz

5    900Hz + 1300Hz     6   1100Hz + 1300Hz

7    700Hz + 1500Hz     8    900Hz + 1500Hz

9   1100Hz + 1500Hz     0   1300Hz + 1500Hz

KP  1100Hz + 1700Hz     ST  1500Hz + 1700Hz

*11   700Hz + 1700Hz    *12   900Hz + 1700Hz

*KP2 1300Hz + 1700Hz     SF  2600Hz

* = Used on CCITT System 5, for special international calling.

Green Box Tones

Coin Collect 700Hz + 1100Hz

Coin Return 1100Hz + 1700Hz

_______________________________________________________________________

Copyright 1990

85                                                   Appendix A

_______________________________________________________________________

Ringback     700Hz + 1700Hz

Money Tones

Nickels  1 time  1700Hz + 2200Hz for 66msec.

Dimes    2 times 1700Hz + 2200Hz for 66msec each

Quarters 5 times 1700Hz + 2200Hz for 33msec each

----------------------------------------------------------------

To do a verification of a number using a blue box, do the

following:

1. Seize a trunk, wait for beep-chunk

2. Dial (MF) KP+Your NPA

3. Dial a two digit area identifier. These are one of the

following: 00, 11, 22, 33, 44, 55, 66, 77, 88, 99. Which one

you use depends on the NPA you give it. You'll have to find

one that works.

4. Then dial NPA+###+####+ST where this is the number you wish

to verify. You'll hear the trunks clicking a bit, then a

beep and you'll be on the line. The voices will be scram#

bled, though. The verify lasts about 20-40 seconds.

------------------------------------------------------------------

To box an overseas call:

1. Seize a trunk with 2600Hz

2. Dial KP+NPA+SENDER+ST

Where SENDER is an overseas routing station, such as :

182 - White Plains, NY

184 - New York, NY

185 - Pittsburgh, PA

186 - Orlando, FL

3. Dial KP+X+CC+CI.C+LN+ST

Where CC = Country Code

CI.C     = City Code

LN       = Local Number

X is a number sometimes required to be entered for some

stations. 0 gives a satellite, and 1 links via cable (better

connection).

Try KP+914+182+ST

then KP+34+1+254+5400+ST

(Hotel in Spain)

KP+000+0000+ST will tell you what switching station you are

routed through.

_______________________________________________________________________

M & M Enterprises

86                                               The Boxing Book

_______________________________________________________________________

-----------------------------------------------------------------

Table of Proposed Standard Audible Tones in North

America (From CCITT Document AP III-84)

Use       Frequency in Hz   Power at CO   Timing

350 440 480 620

===============================================

Dial Tone  X   X             -13 DB     continuous

Busy Tone          X   X     -24 DB     .5 sec on/off

Reorder            X   X     -24 DB     .2 sec on .3 sec off

Audible        X   X         -16 DB     2 sec on/4 sec off

Ringing

High Tone          X         -16 DB     varies with use

Pre-exemption  X       X     -18 DB     single 200/500 msec

Tone

Call Waiting   X             -13 DB     single 500 msec

Tone

=============================================================

High Tone is used in many ways. For example:

1. Spurts of tone to indicate specific orders to operators

in the manual service (order tones).

2. To inform operators of lines that are temporarily out of

service.

3. To alert customers that their services are in a permanent

off-hook condition.

Pre-exemption Tones are used in certain private switched

networks which may interconnect with national networks.

Frequency limits are +/- 0.5 % of nominal.

----------------------------------------------------------------

TASI Locking Frequency

TASI (time assignment speech interpolation) is used on

satellite trunks, and basically allows more than one person to

use a trunk by putting them on while the other person isn't

talking. Of course, you'd never hear the other person talking on

your trunk. When you start to talk, however, the TASI controller

has to find an open trunk for you. Because of this, some of your

speech is lost (because of the delay in finding a trunk). This is

_______________________________________________________________________

Copyright 1990

87                                                   Appendix A

_______________________________________________________________________

called clipping. Not much of a problem for speech, but can really

mess up data transmissions. A TASI locking frequency (1850 Hz)

sent before transmission will keep the TASI from putting anyone

else on your trunk, or you on anyone elses.

_______________________________________________________________________

M & M Enterprises

88                                               The Boxing Book

_______________________________________________________________________

Appendix B

LINE SIGNALS IN CCITT SYSTEMS Courtesy of Sherwood Forest II

-----------------------------

You will be given an entry followed by 4 pieces of informa#

tion on that entry. The titles for those 4 pieces of information

are: Direction, CCITT #3 (1VF), CCITT #4 (2VF), and CCITT #5

(2VF). The individual pieces of information will be, for the most

part, abbreviated to the use of symbols, the symbols table will

follow the entries at the end of this section.

SEIZE TERMINAL:  -->,X,PX,X

SEIZE TRANSIT:  -->,N/A,PY,N/A

START PULSING-TERMINAL:  <--,X,X,Y

START PULSING-TRANSIT:  <--,N/A,Y,N/A

END OF PULSING (ST)*:-->,250MS,xSxSxSx,1500+1700HZ

BUSY:  <--,XX,PX,Y

ACKNOWLEDGE:  -->,N/A,N/A,X

ANSWER:  <--,XSX,PY,X

CLEAR BACK (ON HOOK):  <--,XX,PX,Y

RING FORWARD:  -->,XSX,PYY,Y(850+OR-200MS)

CLEAR FORWARD (DISCONNECT):  -->,XXSXX,PXX,X+Y

DISCONNECT ACKNOWLEDGE:  <--,XXSXX,PYY,X+Y

SYMBOL TABLE

X = 2280+OR-6HZ,120+OR-30MS

XX = 2280+OR-6HZ,600+OR-120MS

S=100+OR-20MS SILENCE

<-- OR --> = DIRECTION

Those were for CCITT #3 (1VF) (Second item after each signal)

X = 2040+OR-6HZ,100+OR-20MS

Y = 2400+OR-6HZ,100+OR-20MS

XX = 350+OR-70MS

YY = 350+OR-70MS

S = 35+OR-7MS;x:2040HZ,35MS

P = (2040HZ,2400HZ),150+30MS

Those were for CCITT #4 (2VF)

X = 2400+OR-6HZ

Y = 2600+OR-6HZ

Those were for CCITT #5 (2VF)

N/A = Not Applicable

* = Combination #15 of Address Code

_______________________________________________________________________

Copyright 1990

89                                                   Appendix C

_______________________________________________________________________

Appendix C

Tone Frequencies of Various Boxes:

Dual Tone Multi-Frequency (Touch-Tone)

Key    Freq. #1   Freq. #2     ^    Key    Freq. #1   Freq. #2

---    --------   --------     |    ---    --------   --------

1      697        1209        |     7      852        1209

2      697        1336        |     8      852        1336

3      697        1477        |     9      852        1477

A      697        1633        |     C      852        1663

4      770        1209        |     *      941        1203

5      770        1336        |     0      941        1336

6      770        1477        |     #      941        1477

B      770        1633        v     D      941        1633

Multi-Frequency (Blue box/operator tones)

Key    Freq. #1   Freq. #2     ^    Key    Freq. #1   Freq. #2

---    --------   --------     |    ---    --------   --------

1       700        900        |     9      1100       1500

2       700       1100        |    10      1300       1500

3       900       1100        |    11       700       1700

4       700       1300        |    12       900       1700

5       900       1300        |    KP      1100       1700

6      1100       1300        |    KP2     1300       1700

7       700       1500        |    ST      1500       1700

8       900       1500        |  Disconnect      2600

Red Box tones:  pay phone change tones

$.05    1700Hz+2200Hz for 66 ms

$.10    1700Hz+2200Hz for 66ms on, 66ms off, 66ms on

$.25    1700Hz+2200Hz for 33ms on, 33ms off (repeated 5 times)

Green Box tones:  return change put in pay phone

40ms 2600Hz, 60ms silence, 900ms 700+1100 (BB 2)

90ms 2600Hz, 160ms silence, [BB 2 90 on, 90 off repeated 4 times]

KP + 011 + country + national # + ST  = international calling

KP + NPA + 121 + ST              = Inward Operator

KP + NPA + 131 + ST              = Directory Assistance

KP + NPA + 101 + ST              = Toll Switching

KP + NPA + 141 + ST              = Rate & Route

KP + NPA + 181 + ST              = Coin Refund Operator

KP + NPA + 11501 + ST            = Mobile Operator

KP + NPA + 11521 + ST            = Mobile Operator

KP + NPA + 11511 + ST            = Conference Operator

KP + NPA + PFX + SUFX + ST       = Free call

TASI Locking Frequency:

(Time Assignment Speech Interpolation, used on satellites for

data transmission):  1850Hz before transmission

_______________________________________________________________________

M & M Enterprises

90                                               The Boxing Book

_______________________________________________________________________

Appendix D

Trunking Off of 911 by The Conflict

-------------------    Modified by Maxwell Smart

We are not sure how useful this will be anyone, since the

majority of these users do not possess blue boxes. Oh well, c'est

la vie, and on with the fun. Essentially, the blue box generates

tones which are important to telephone companies. The most impor#

tant tone being the 2600 Hz. This is used for seizing and giving

up trunks. Well, an interesting concept came up one day when a

few of the guys were screwing around at a local fortress phone.

We decided to call 911 and blow a 2600. The results were very

bizarre. The effects of trunking a 911 can be tremendously damag#

ing! See, the 2600 blew 911 off its assigned trunk position.

Although we could not seize this for outdial, we could play

operator. We played with the inwards for a time, then we hung up

and called back 911. A recording came up saying that 911 was not

available in this area and to call the police/ambulance/firehouse

(like the one Richmond residents get). That was very funny since

only minutes ago, the 911 was functioning fine. Well, we cruised

from this local and hit another phone.

We DTMF'd 911 and got the same recording. It seems that the

area's 911 was disabled for nearly 3 days. They reassigned the

trunk position I guess 2 days after, then they had to resetup the

equipment.

_______________________________________________________________________

Copyright 1990

91                                                   Appendix E

_______________________________________________________________________

Appendix E

International Dialing Codes (Country + Routing)

-----------------------------------------------

To dial international calls:

International Access Code + Country code + Routing code

Example :

To call Frankfurt, Germany, you would do the following:

011 + 49 + 611 + (# wanted) + # sign(octothrope)

The # sign at the end is to tell Bell that you are done

entering in all the needed info.

Here is the list of Country Codes, listed next to the

country, and the routing codes listed next to the city.

Algeria-213          American Samoa- 684    Andorra- 33

-------              --------------         -------

all points- 078

Argentina- 54        Australia- 61          Austria- 43

---------            ---------              -------

Buenos Aires- 1      Melbourne- 3           Innsbruck- 5222

Sydney- 2              Vienna- 222

Bahrain- 973         Belgium- 32            Belize- 501

-------              -------                ------

no routing needed    Antwerp- 31            no routing needed

Brussels- 2

Bolivia- 591         Brazil- 591/(55)       Brunei- 673

-------              ------                 ------

La Paz- 2            Brasilia-61

Rio de Janeiro- 21

Sao Paulo- 11

Cameron- 237         Chile- 56/(506)        China- 86

--------             -----                  -----

Santiago- 2            Tainan- 62

Valparaiso- 31         Taipei- 2

Colombia- 56/(57)    Costa Rica- 506        Cyprus- 357

--------             ----- ----             ------

none needed          no routing needed      Nicosia- 21

Czechoslovakia- 42   Denmark- 45            Ecuador- 593

--------------       -------                -------

Aalborg- 8             Cuenca- 4

Copenhagen 1 or 2      Quito- 2

_______________________________________________________________________

M & M Enterprises

92                                               The Boxing Book

_______________________________________________________________________

Egypt- 20            El Salvador- 503       Ethiopia- 251

-----                ----------             --------

no routing needed

Fiji- 679            Finland- 358           France- 33

----                 -------                ------

none needed                                 Bordeaux- 56

Marseille- 91

Nice- 93

Paris- 1

French Antilles- 596 French Polynesia- 689  Gabon- 241

---------------      ----------------       -----

Germany- 49          German. Rep- 37        Gibraltar- 350

-------              ------- ---            ---------

Berlin- 30

Bonn- 228

Frankfurt- 661

Munich- 89

Greece- 30           Guam- 671              Guantanamo Bay

------               ----                   (US Navy Base)- 53

Athens- 1            no routing needed      --------------

Rhodes- 241

Guatemala- 502       Guyana- 592            Haiti- 509

---------            ------                 -----

Guatemala City- 2    Georgetown- 02         Port Au Prince- 1

Honduras- 504        Hong Kong- 852         Hungary- 36

--------             ---- ----              -------

no routing needed    Hong Kong- 5

Kowloon- 3

Iceland- 354         India- 91              Indonesia- 62

-------              -----                  ---------

Jakarta- 21

Iran- 98             Iraq- 964              Ireland- 353

----                 ----                   -------

Teheran- 21          Baghdad- 1             Dublin- 1

Galway- 91

Israel- 978          Italy- 39              Ivory Coast- 225

------               -----                  ----- -----

Haifa- 4             Florence- 55           no routing needed

Jerusalem- 2         Naples- 81

Tel Aviv- 3          Rome- 6

Venice- 41

Japan- 81            Jordan- 962            Kenya- 254

-----                ------                 -----

Hiroshima- 822                              Nairobi- 2

Tokyo- 3

_______________________________________________________________________

Copyright 1990

93                                                   Appendix E

_______________________________________________________________________

Yokohama- 45

Korea- 82            Kuwait- 965            Liberia- 231

-----                ------                 -------

Pusan- 51            no routing needed      none needed

Seoul- 2

Lesotho- 266         Libya- 218             Liechtenstein- 41

-------              -----                  -------------

Tripoli- 21            All points- 75

Luxembourg- 352      Malawi- 265            Malaysia- 60

----------           ------                 --------

no routing needed                           Kuala Lumpur- 3

Monaco- 33           Morocco- 212           Nambia- 264

------               -------                ------

All points- 93

Netherlands- 31      Netherlands Antilles- 599

-----------          ----------- --------

Amsterdam- 20

Rotterdam- 10

The Hague- 70

New Caledonia- 687   New Zealand- 64        Nicaragua- 505

--- ---------        --- -------            ---------

no routing needed    Auckland- 9            Managua- 2

Wellinton- 4

Nigeria- 234         Norway- 47             Oman- 968

-------              ------                 ----

Lagos- 1             Bergen- 5

Oslo- 2

Pakistan- 92         Panama- 507            Papua New Guinea-675

--------             ------                 ----- --- ------

none needed            no routing needed

Paraguay- 595        Peru- 51               Philippines- 63

--------             ----                   ------------

Asuncion- 21         Arequipa- 542          Manila- 2

Lima- 14

Poland- 48           Portugal- 351          Qatar- 974

------               --------               -----

Lisbon- 19

Rumania- 40          St. Pierre & Miquelon- 508

-------              --- ------ - --------

Bucuresti- 0

Saipan- 670          San Marino- 39         Saudi Arabia- 966

------               --- ------             ----- ------

All points- 541        Riyadh- 1

_______________________________________________________________________

M & M Enterprises

94                                               The Boxing Book

_______________________________________________________________________

Senegal- 221         Singapore- 65          South Africa- 27

-------              ---------              ----- ------

no routing needed                           Cape Town- 21

Pretoria- 12

Spain- 34            Sri Lanka- 94          Suriname- 597

-----                --- -----              --------

Barcelona- 3         Colombo- 1             no routing needed

Canary Is.- 28

Madrid- 1

Seville- 54

Swaziland- 268       Sweden- 46             Switzerland- 41

---------            ------                 -----------

Goteborg- 31           Berne- 31

Stockholm- 8           Geneva- 22

Lucerne- 41

Zurich- 1

Tahiti- 689          Taiwan- 886            Tanzania- 255

------               ------                 --------

none needed

Thailand- 66         Tunisia- 216           Turkey- 90

--------             -------                ------

Bangkok- 2           Tunis- 1               Istanbul- 11

Uganda- 256          United Arab            United Kingdom- 44

------               Emirates- 971          ------ -------

--------               Belfast- 232

Abu Dhabi- 2           Cardiff- 222

Ajman- 6               Edinburgh- 31

Al Ain- 3              Glasgow- 41

Aweir- 49              Liverpool- 51

Dubai- 4               London- 1

Fujairah- 91

Jebel Dhana- 5

Sharjah- 6

Umm-Al-Quwain- 6

Uruguay- 598         USSR- 7                Vatican City- 39

-------              ----                   ------- ----

Kiev- 044              All points- 6

Leningrad- 812

Minsk- 017

Moscow- 095

Tallinn- 0142

Venezuela- 58        Yemen Arab Rep.- 967   Yugoslavia- 38

---------            ----- ---- ----        ----------

Caracas- 2                                  Belgrade- 11

Maracaibo- 61                               Zagreb- 41

Zambia- 260          Zimbabwe- 263

------               --------

_______________________________________________________________________

Copyright 1990

##########################################################################################